3GPP TSG RAN WG3 Meeting #129	R3-255833

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	10.2

Source:	Samsung, Jio Platforms, Cybercore, Nokia, ZTE

Title:	(TP to SON BLCR for TS 38.300) MRO for CHO with candidate SCGs

Document for:	Approval

# Introduction

This contribution provides TP to BLCR for TS 38.300 on MRO for CHO with candidate SCG(s).

# TP to BLCR for TS 38.300 on CHO with candidate SCG(s)

#### 15.5.2.6	PSCell addition/change failure

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21].

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure has been caused by a different node, the serving M-NG-RAN node sends SCG Failure Indication message to that relevant M-NG-RAN node. If it is an associated SN which should perform optimization, that M-NG-RAN node sends SCG FAILURE INFORMATION REPORT message to the SN as defined in TS 37.340 [21].