3GPP TSG-RAN WG3 Meeting #129	R3-255806

Bengaluru, India, August 25th–29th, 2025

Agenda Item:	17.3

Source:	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms, Huawei

Title:	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Document for:	Other

Information

Implementing the agreement from RAN3#129 meeting.

Agreement 1: For the class 1 procedures:

Change the procedure and message name from “UL WUS Configuration Provision” to “OD-SIB1 Configuration Provision”

Change from NR CGI at NG-RAN node1 (Naming FFS) to NES Cell ID

Change from NR CGI at NG-RAN node2 (Naming FFS) to Cell A ID

Please all TP using the name above for the procedure, messages

Agreement 2: For the class 2 procedures:

UL WUS CONFIGURATION PROVISION STATUS UPDATE => OD-SIB1 Configuration Provision Status Update

Please all TP using the name above for the procedure, messages

Agreements 3: the structure of the class 2 message

In the class 2 message “OD-SIB1 Configuration Provision Status Update”, it is agreed to include “NES Cell ID” as Mandatory presented.

Cell A ID is optionally presented.

Agreement 4: to capture in the class 1 procedure that:

If Cell A ID IE is included in the provision request, then Cell A gNB shall, if support, broadcast the OD-SIB1 configuration in the indicated Cell A.

Agreements 5: to capture in the Class 2 procedure text:

If Cell A ID IE is present, this message indicates that the indicated Cell A has stopped the broadcasting.

If Cell A ID IE is not present, then all the Cell A(s) have stopped the broadcasting for the given NES Cell.

In the response and failure message, include Optional Cell A ID and specify that the Cell A gNB may send back the Cell A ID only if it is included in the request message.

Text Proposal for BL CR for TS 38.423

*************** Start of TP ***************

- 8.x.x	OD-SIB1 Configuration Provision

- 8.x.x.1	General

The purpose of the OD-SIB1 Configuration Provision procedure is to enable an NG-RAN node1 to provide OD-SIB1 configuration information to an NG-RAN node2 and to request the NG-RAN node2 to transmit the provided OD-SIB1 configuration information.

The procedure is also used to enable the NG-RAN node1 to request the NG-RAN node2 to stop transmission of the OD-SIB1 configuration.

The procedure uses non-UE -associated signaling.

- 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: OD-SIB1 Configuration Provision procedure, successful operation

If the start is included in the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, broadcast the provided OD-SIB1 Configuration in SIBx and reply with the OD-SIB1 CONFIGURATION PROVISION RESPONSE message. The NG-RAN node2 stores the OD-SIB1 Configuration information.

If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, broadcast the OD-SIB1 Configuration in the cell indicated by the Cell A ID IE.

If the start is included in the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message and the NES Cell ID IE indicates a cell for which the NG-RAN node2 is already broadcasting the OD-SIB1 Configuration:

-	If the request is for the existing Cell A ID(s), the NG-RAN node2 shall, if supported, stop the ongoing broadcasting and broadcast the newly provided OD-SIB1 Configuration in SIBx. The NG-RAN node2 replace the stored OD-SIB1 Configuration information with the newly received OD-SIB1 Configuration information.

-	If the request is for a new Cell A ID, the NG-RAN node2 shall, if supported, start broadcasting the provided OD-SIB1 Configuration in SIBx in the indicated cell.

If the stop is included in the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall stop broadcasting the OD-SIB1 Configuration, for the given NES Cell ID, in SIBx. The NG-RAN node2 removes the stored OD-SIB1 Configuration information.

The NG-RAN node2 includes the Cell A ID IE in the OD-SIB1 CONFIGURATION PROVISION RESPONSE message if it is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message.

- 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: OD-SIB1 Configuration Provision procedure, unsuccessful operation

If the NG-RAN node2 cannot broadcast the provided OD-SIB1 Configuration in SI, or a failure occurs during OD-SIB1 Configuration Provision, the NG-RAN node2 shall send the OD-SIB1 CONFIGURATION PROVISION FAILURE message. The message shall contain the Cause IE with an appropriate value.

If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message and the NG-RAN node2 cannot broadcast the provided OD-SIB1 Configuration in SIBx of the cell indicated in the Cell A ID IE, or a failure occurs during OD-SIB1 Configuration Provision, the NG-RAN node2 shall send the OD-SIB1 CONFIGURATION PROVISION FAILURE message. The message shall contain the Cause IE with an appropriate value.

The NG-RAN node2 includes the Cell A ID IE in the OD-SIB1 CONFIGURATION PROVISION FAILURE message if it is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message.

- 8.x.x.4	Abnormal Conditions

Void.

- 8.x.y	OD-SIB1 Configuration Provision Status Update

- 8.x.y.1	General

This procedure is initiated by an NG-RAN node2 to inform a neighbouring NG-RAN node1 about a previously admitted OD-SIB1 configuration provision being stopped.

The procedure uses non-UE associated signalling.

- 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: OD-SIB1 Configuration Provision Status Update procedure, successful operation

The NG-RAN node2 shall indicate that a previously admitted OD-SIB1 configuration provision in the OD-SIB1 CONFIGURATION PROVISION STATUS message is being stopped. The NG-RAN node2 removes the stored OD-SIB1 Configuration information after signalling this message.

If the Cell A ID IE is present, the NG-RAN node1 considers that the indicated Cell has stopped the broadcasting of OD-SIB1 Configuration in SIBx.

If the Cell A ID IE is not present, the NG-RAN node1 considers that all the Cell A(s) in NG-RAN node2 have stopped the broadcasting for the given NES Cell in NG-RAN node1

- 8.x.y.3	Unsuccessful Operation

Not applicable.

- 8.x.y.4	Abnormal Conditions

Void.

*************** Next change ***************

- 9.x.x.x	OD-SIB1 CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request to start or to stop the provisioning of an OD-SIB1 configuration.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >>OD-SIB1 Configuration | M |  | OCTET STRING | Includes the OD-SIB1-Config IE as specified in SIBxx in TS 38.331 [10]. | – |  |
| >>NES Cell ID | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 | – |  |
| >>Cell A ID | O |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 requested to broadcast the OD-SIB1 configuration. | – |  |
| >Stop |  |  |  |  |  |  |
| >>NES Cell ID | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

- 9.1.3.y	OD-SIB1 CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the OD-SIB1 configuration will be provided by the NG-RAN node2 as requested by the NG-RAN node1.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NES Cell ID | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 | YES | reject |
| Cell A ID | O |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 requested to broadcast the OD-SIB1 configuration. | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

- 9.1.3.z	OD-SIB1 CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the OD-SIB1 configuration cannot be provided by the NG-RAN node2 as requested by the NG-RAN node1.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| NES Cel IDl | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 which supports OD-SIB1On-Demand SIB1 | YES | ignore |
| Cell A ID | O |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 requested to broadcast the OD-SIB1OD-SIB1 configuration. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

- 9.1.3.t	OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted OD-SIB1 configuration provision is being stopped.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NES Cell ID | M |  |  | Indicates the NR cell of the NG-RAN node1 | YES | reject |
| Cell A ID | O |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 requested to broadcast the OD-SIB1 configuration. | YES | ignore |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the status of the OD-SIB1 configuration transmission. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

*************** Next change ***************

- 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextConfirm,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart,

HandoverSuccess,

ConditionalHandoverCancel,

EarlyStatusTransfer,

FailureIndication,

HandoverReport,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

AccessAndMobilityIndication,

CellTrafficTrace,

RANMulticastGroupPaging,

ScgFailureInformationReport,

ScgFailureTransfer,

F1CTrafficTransfer,

IABTransportMigrationManagementRequest,

IABTransportMigrationManagementResponse,

IABTransportMigrationManagementReject,

IABTransportMigrationModificationRequest,

IABTransportMigrationModificationResponse,

IABResourceCoordinationRequest,

IABResourceCoordinationResponse,

CPCCancel,

PartialUEContextTransfer,

PartialUEContextTransferAcknowledge,

PartialUEContextTransferFailure,

RachIndication,

DataCollectionRequest,

DataCollectionResponse,

DataCollectionFailure,

DataCollectionUpdate,

ULWUSConfigurationProvisionRequest,

ULWUSConfigurationProvisionResponse,

ULWUSConfigurationProvisionFailure,

ULWUSConfigurationProvisionStatus

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-cellTrafficTrace,

id-RANMulticastGroupPaging,

id-scgFailureInformationReport,

id-scgFailureTransfer,

id-f1CTrafficTransfer,

id-iABTransportMigrationManagement,

id-iABTransportMigrationModification,

id-iABResourceCoordination,

id-retrieveUEContextConfirm,

id-cPCCancel,

id-partialUEContextTransfer,

id-rachIndication,

id-dataCollectionReportingInitiation,

id-dataCollectionReporting,

id-uLWUSConfigurationProvision,

id-uLWUSConfigurationProvisionStatus

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

XnAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnRemoval												|

xnSetup													|

nGRANnodeConfigurationUpdate							|

e-UTRA-NR-CellResourceCoordination						|

cellActivation											|

reset													|

resourceStatusReportingInitiation						|

mobilitySettingsChange									|

iABTransportMigrationManagement							|

iABTransportMigrationModification						|

iABResourceCoordination									|

partialUEContextTransfer								|

dataCollectionReportingInitiation						|

uLWUSConfigurationProvision,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport				|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				|

cellTrafficTrace						|

rANMulticastGroupPaging					|

scgFailureInformationReport				|

scgFailureTransfer						|

f1CTrafficTransfer						|

retrieveUEContextConfirm				|

cPCCancel								|

rachIndication							|

dataCollectionReporting					|

uLWUSConfigurationProvisionStatus,

...

}

*************** Next change ***************

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

sNStatusTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-sNStatusTransfer

CRITICALITY				ignore

}

*************** Next change ***************

dataCollectionReportingInitiation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				DataCollectionRequest

SUCCESSFUL OUTCOME				DataCollectionResponse

UNSUCCESSFUL OUTCOME			DataCollectionFailure

PROCEDURE CODE					id-dataCollectionReportingInitiation

CRITICALITY						reject

}

dataCollectionReporting	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataCollectionUpdate

PROCEDURE CODE			id-dataCollectionReporting

CRITICALITY				ignore

}

uLWUSConfigurationProvision		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ULWUSConfigurationProvisionRequest

SUCCESSFUL OUTCOME				ULWUSConfigurationProvisionResponse

UNSUCCESSFUL OUTCOME			ULWUSConfigurationProvisionFailure

PROCEDURE CODE					id-uLWUSConfigurationProvision

CRITICALITY						reject

}

uLWUSConfigurationProvisionStatus	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULWUSConfigurationProvisionStatus

PROCEDURE CODE			id-uLWUSConfigurationProvisionStatus

CRITICALITY				ignore

}

END

-- ASN1STOP

*************** Next change ***************

- 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

*************** Next change ***************

id-S-CPAC-Request,

id-sk-Counter,

id-Source-M-NG-RANnodeID,

id-SourceSN-to-TargetSN-QMCInfo,

id-RegistrationRequestForDataCollection,

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection,

id-NodeAssociatedInfoResult,

id-SLPositioning-Ranging-Services-Info,

id-PDUSessionsListToBeReleased-UPError,

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-RequestType,

id-OD-SIB1-Cell,

id-ProvisionStatus,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells,

maxnoofSSBAreas

FROM XnAP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

*************** Next change ***************

-- **************************************************************

--

#### -- DATA COLLECTION UPDATE

--

-- **************************************************************

DataCollectionUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionUpdate-IEs}},

...

}

DataCollectionUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-CellMeasurementResultForDataCollection-List	CRITICALITY ignore	TYPE CellMeasurementResultForDataCollection-List		PRESENCE optional }|

{ ID id-UEAssociatedInfoResult-List					CRITICALITY ignore	TYPE UEAssociatedInfoResult-List							PRESENCE optional }|

{ ID id-NodeAssociatedInfoResult					CRITICALITY ignore	TYPE NodeAssociatedInfoResult								PRESENCE optional },

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION REQUEST

--

-- **************************************************************

ULWUSConfigurationProvisionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ULWUSConfigurationProvisionRequest-IEs}},

...

}

ULWUSConfigurationProvisionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RequestType								CRITICALITY reject	TYPE RequestTypeChoice								PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

RequestTypeChoice ::= CHOICE {

Start							ULWUSConfigurationInformation,

Stop							NR-CGI,

choice-extension				ProtocolIE-Single-Container { {RequestTypeChoice-ExtIEs} }

}

RequestTypeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ULWUSConfigurationInformation ::= SEQUENCE {

uLWUSConfiguration					OCTET STRING,

oD-SIB1-Cell						NR-CGI,

assisting-Cell						NR-CGI						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ULWUSConfigurationInformation-ExtIEs} }	OPTIONAL,

...

}

ULWUSConfigurationInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION RESPONSE

--

-- **************************************************************

ULWUSConfigurationProvisionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ULWUSConfigurationProvisionResponse-IEs}},

...

}

ULWUSConfigurationProvisionResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-OD-SIB1-Cell							CRITICALITY reject	TYPE NR-CGI									PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION FAILURE

--

-- **************************************************************

ULWUSConfigurationProvisionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ ULWUSConfigurationProvisionFailure-IEs}},

...

}

ULWUSConfigurationProvisionFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-OD-SIB1-Cell							CRITICALITY ignore	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION STATUS

--

-- **************************************************************

ULWUSConfigurationProvisionStatus ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ULWUSConfigurationProvisionStatus-IEs}},

...

}

ULWUSConfigurationProvisionStatus-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-OD-SIB1-Cell							CRITICALITY ignore	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-ProvisionStatus							CRITICALITY ignore	TYPE ProvisionStatus							PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

ProvisionStatus ::= ENUMERATED {stopped, ...}

END

-- ASN1STOP

*************** Next change ***************

- 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication											ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-cellTrafficTrace												ProcedureCode ::= 38

id-RANMulticastGroupPaging										ProcedureCode ::= 39

id-scgFailureInformationReport									ProcedureCode ::= 40

id-ProcedureCode41-NotToBeUsed									ProcedureCode ::= 41

id-scgFailureTransfer											ProcedureCode ::= 42

id-f1CTrafficTransfer											ProcedureCode ::= 43

id-iABTransportMigrationManagement								ProcedureCode ::= 44

id-iABTransportMigrationModification							ProcedureCode ::= 45

id-iABResourceCoordination										ProcedureCode ::= 46

id-retrieveUEContextConfirm										ProcedureCode ::= 47

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

id-rachIndication												ProcedureCode ::= 50

id-dataCollectionReportingInitiation							ProcedureCode ::= 51

id-dataCollectionReporting										ProcedureCode ::= 52

id-uLWUSConfigurationProvision									ProcedureCode ::= 5x

id-uLWUSConfigurationProvisionStatus							ProcedureCode ::= 5y

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

*************** Next change ***************

id-CPAC-Preparation-Type																			ProtocolIE-ID ::= 468

id-UserPlaneFailureIndication																		ProtocolIE-ID ::= 469

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-RequestType																						ProtocolIE-ID ::= 4xx

id-OD-SIB1-Cell																						ProtocolIE-ID ::= 4xx+1

id-ProvisionStatus																					ProtocolIE-ID ::= 4xx+3

END

-- ASN1STOP

*************** Start of TP ***************