3GPP TSG-RAN3 Meeting #130	R3-258195

Dallas, US, Nov 17 - 21, 2025

Agenda Item:	10.5.1

Source: 	Qualcomm Incorporated, JIO Platforms

Title: 	(pCR to TR 38.760-3) Discussion on 6G AI/ML Use Cases

Document for:	Discussion/Decision

# Introduction

TSG RAN#109 approved on updated SI on 6G Radio in RP-252912 [1]. The SID contains the following objectives for AI/ML study in RAN3 scope.

RP-252912 - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services

In RAN3#129bis, we agreed to study AI/ML based optimization of Energy Savings and Mobility Optimization

RAN3#129bis High-level principles for AI/ML RAN in 6G: The design of AI/ML algorithms and models for RAN3 led use cases are implementation specific and out of RAN3 scope The following use cases will be studied in RAN3: - AI/ML based Network energy saving - AI/ML based mobility optimization Other use cases can be discussed in future meetings based on contributions.

In this paper, we discuss our views on AI/ML use cases and general principles for 6G.

# Discussion

## General Principles for NG-RAN AI/ML

Native AI/ML support in 6G RAN features is one of the key objectives of 6G. AI/ML use cases started in the later release of 5G, and not many use cases were addressed in 5G for AI based optimization. 6G Native AI/ML should enable optimization of the Network and UE performance also considering optimized resource consumption and signalling.  6G AI/ML use cases should maintain balance between performance gains vs added complexity and with the rapid advancements of AI/ML algorithms/models, 6G RAN is expected to see considerable improved performance from day 1.

5G AI/ML functional framework consisted of Data collection, AI/ML Training, AI/ML Inference and AI/ML feedback. For any AI/ML use case input data is the key for Model Training. Varied data collected over an extended area and over an extended period of time will enable training AI model for different scenarios. Initial AI/ML model training can be non real-time and does not need data collection in real-time. But offline training models should be allowed to be fine tuned in real time to improve model performance by using real-time data as well.

AI/ML model Training can be real-time and non-real-time, hence training data may need to be collected both in real-time and non-real-time procedures.

5G AI/ML framework considered AI/ML Training located in RAN or OAM for 5G AI/ML use cases. Initial AI/ML Training is not real-time and needs varied inputs from different sources. It is beneficial for Model Training to happen in a centralized location for offline model training which is trusted and can enforce user privacy protection of the collected data, rather than individual RAN nodes.

In 5G, NG-RAN AI/ML framework, AI/ML training is located in RAN or OAM. It is beneficial to support Model Training in a centralized location.

For 6G AI/ML use cases, it should be possible to support initial model training at a centralized Node.

Both initial Model training and fine tuning should occur in a location that is trusted and can enforce user privacy protection.

For 5G AI/ML use cases, location of AI/ML inference was restricted to NG-RAN for non-split architecture and gNB-CU for split architecture. Though certain use cases like AI/ML based CCO necessitated the AI/ML inference to be located in the gNB-DU for non-split architecture, it was not agreed citing AI/ML functional framework doesn’t support this. For RAN1/2 use cases like BM and CSI compression, it is logical run inference in 5G DU but this was not discussed and not supported, thus causing ineffective usage of AI/ML models on network side. However, in 6G, the AI/ML functional framework should be flexible to accommodate different use cases and allow location on inference in any RAN NF based on the use case. Similar to 5G, the AI/ML models and algorithm should be left to implementation and should be out of the 3GPP  scope.

In 5G, the location of AI/ML inference was restricted to only gNB-CU in a split architecture, irrespective of the use cases supported.

6G RAN AI/ML use cases should support flexible deployment of AI/ML inference in any RAN network function depending on the AI/ML use case.

For 5G AI/ML use cases, input data for training and feedback was restricted among RAN nodes only. The inference output was only exchanged between the RAN nodes. RAN3 AI/ML started earlier than RAN1 and RAN2 AI/ML use cases. Hence collaboration of input and output for AI/ML use cases across RAN1/2/3 was not possible in 5G. For AI/ML use cases like Mobility, it is beneficial to get input data from UE and also provide inference to UE.

In 5G, input data for AI/ML use cases and the inference output was restricted to be supported only between the RAN nodes.

6G should support input data for RAN AI/ML use cases from all sources including UE, RAN, CN and 3rd party sources.

For 5G AI/ML use cases, different sources of input data and feedback data was proposed and specified like Xn based data collection from neighbouring nodes, MDT data from UE, SON and QoE data from UE etc. All these different sources of data are collected using different mechanisms and used for AI/ML training and feedback purposes. Since 5G AI/ML use cases was built on top of existing 5G architecture, the use cases did not introduce drastic changes to 5G architecture and procedures and worked around with existing mechanisms.

However in 6G, it is good to re-look at the way in which data is collected from different sources in different ways and if there is a possibility to simply the data collection for AI/ML use cases. MDT and QoE (if agreed to be supported in 6G) is triggered from OAM (MCE and TCE). However, SON data collection is triggered by RAN. Data collection from neighbouring nodes are triggered by RAN. Below table shows data collected for AI/ML based on 5G AI/ML use cases.

| Types of data | Triggered By | Producer | Consumer | Transfer Mechanism |
| --- | --- | --- | --- | --- |
| L1 Measurement Report | RAN | UE | RAN | UCI |
| L3 Measurement Report | RAN | UE | RAN | RRC |
| RAN AI/ML Data | RAN | RAN | RAN | Xn Data Collection Procedure |
| MDT | OAM | UE | OAM, RAN (TCE) | RRC, Trace Job |
| SON (UE and NW) Data | RAN | UE, RAN | RAN, OAM (only RLF and RCEF) | RRC, Trace Job |
| QoE Data | OAM | UE | RAN, OAM (MCE) | RRC, Trace Job |

For 5G AI/ML use cases, different sources of input/feedback data was proposed and specified like MDT, SON, QoE, Xn based data collection. MDT and QoE data collection are triggered from OAM (TCE and MCE) whereas SON (UE and NW reports) and Xn based data collection are triggered by RAN.

Since initial training and fine tuning can happen in centralised location like OAM and in RAN nodes, the data collected from different sources should be made available to both OAM and RAN.

Since AI/ML initial training and fine tuning can happen in centralised and distributed location (e.g OAM,  RAN nodes), it is beneficial to support accessibility of collected data across different network functions where AI/ML training is supported (e,g RAN, OAM).

Study if MDT and QoE (if agreed to be supported in 6G) data collection triggered via OAM (TCE and MCE servers) can be merged with AI/ML data collection and support accessibility of this data to Network function where AI/ML Training happens.

Study if SON (NW and UE reports) data collection triggered from RAN can be merged with AI/ML data collection and support accessibility of this data to Network function where AI/ML Training happens.

## AI/ML Use Cases

In RAN3#129bis based on popular support, it was agreed to study the following use cases for 6G -

- AI/ML based Mobility Optimization

- AI/ML based Network Energy Savings

Since the 6G Architecture and Interfaces are unclear, RAN3 can only discuss the input, output and feedback data for these use cases.

RAN3 to study only the input, output and feedback data for the AI/ML use cases and not the interface impacts.

For both of the above use cases, training and inference can be located in OAM, to have a centralised AI/ML optimization for the features in the RAN nodes in a particular network deployment. Also distributed AI/ML optimization should be supported in RAN nodes.

For AI/ML based Network Energy Savings and Mobility Optimization use case the AI/ML Training can be located in RAN or in OAM.

For AI/ML based Network Energy Savings and Mobility Optimization, the AI/ML Inference can be located in OAM or in RAN, depending on the use case latency requirements for inference.

Network Energy Savings

AI/ML based Energy Savings was studied in 5G, but due to lack of consensus, only exchange of Energy cost among the RAN nodes was specified without establishing clear benefits. AI/ML based Network Energy Savings was not addressed in 5G. In 6G, it is beneficial to study widely and commercially deployed use cases like Cell/Carrier switch on and off, beam steering and switch on/off for AI/ML based Energy Savings.

5G NG-RAN AI/ML Network Energy Savings use case only addressed exchange of Energy Cost among RAN nodes without any AI/ML based optimization and without any clear performance benefits. Do not consider Energy Cost for Energy Savings AI use case in 6G.

For AI/ML based Network Energy Savings consider Cell and Carrier activation/deactivation for energy savings. Study the input, output and feedback data for AI/ML based Cell and Carrier Activation/deactivation.

For AI/ML based Network Energy Savings consider activation/deactivation of beams and beam steering. Study input, output and feedback data for AI/ML based beam activation and deactivation.

For AI/ML based Cell, Carrier, Beam activation/deactivation study RRM impacts in RAN (e.g Handover of UEs, Release of UEs etc).

Mobility Optimization

Measurements and Measurement events are key functions of Handover. AI/ML based optimization of the measurements will benefit RAN for early HO preparation, resource allocation and avoid HO failures. RAN2 studied and are specifying Measurement and event prediction at the UE. It will be beneficial to support similar prediction on the Network side. In 5G only UE trajectory Prediction was addressed as a part of Mobility optimization.

5G NG-RAN AI/ML Mobility Optimization use case addressed exchange of UE Trajectory Prediction among the RAN nodes for UEs in Connected Mode. RAN2 studied prediction of measurements and events by the UE for AI/ML based mobility in Rel19.

For AI/ML based Mobility Optimization study Measurement and Measurement Event Prediction in the RAN node for Legacy L3 Handover. Consider Rel19 RAN2 AI/ML based mobility SI inputs from TR 38.744 as the baseline for RAN3 study.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	AI/ML model Training can be real-time and non-real-time, hence training data may need to be collected both in real-time and non-real-time procedures.

Observation 2.	In 5G, NG-RAN AI/ML framework, AI/ML training is located in RAN or OAM. It is beneficial to support Model Training in a centralized location.

Observation 3.	In 5G, the location of AI/ML inference was restricted to only gNB-CU in a split architecture, irrespective of the use cases supported.

Observation 4.	In 5G, input data for AI/ML use cases and the inference output was restricted to be supported only between the RAN nodes.

Observation 5.	For 5G AI/ML use cases, different sources of input/feedback data was proposed and specified like MDT, SON, QoE, Xn based data collection. MDT and QoE data collection are triggered from OAM (TCE and MCE) whereas SON (UE and NW reports) and Xn based data collection are triggered by RAN.

Observation 6.	Since AI/ML initial training and fine tuning can happen in centralised and distributed location (e.g OAM,  RAN nodes), it is beneficial to support accessibility of collected data across different network functions where AI/ML training is supported (e,g RAN, OAM).

Observation 7.	5G NG-RAN AI/ML Network Energy Savings use case only addressed exchange of Energy Cost among RAN nodes without any AI/ML based optimization and without any clear performance benefits. Do not consider Energy Cost for Energy Savings AI use case in 6G.

Observation 8.	5G NG-RAN AI/ML Mobility Optimization use case addressed exchange of UE Trajectory Prediction among the RAN nodes for UEs in Connected Mode. RAN2 studied prediction of measurements and events by the UE for AI/ML based mobility in Rel19.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	For 6G AI/ML use cases, it should be possible to support initial model training at a centralized Node.

Proposal 2:	Both initial Model training and fine tuning should occur in a location that is trusted and can enforce user privacy protection.

Proposal 3:	6G RAN AI/ML use cases should support flexible deployment of AI/ML inference in any RAN network function depending on the AI/ML use case.

Proposal 4:	6G should support input data for RAN AI/ML use cases from all sources including UE, RAN, CN and 3rd party sources.

Proposal 5:	Study if MDT and QoE (if agreed to be supported in 6G) data collection triggered via OAM (TCE and MCE servers) can be merged with AI/ML data collection and support accessibility of this data to Network function where AI/ML Training happens.

Proposal 6:	Study if SON (NW and UE reports) data collection triggered from RAN can be merged with AI/ML data collection and support accessibility of this data to Network function where AI/ML Training happens.

Proposal 7:	RAN3 to study only the input, output and feedback data for the AI/ML use cases and not the interface impacts.

Proposal 8:	For AI/ML based Network Energy Savings and Mobility Optimization use case the AI/ML Training can be located in RAN or in OAM.

Proposal 9:	For AI/ML based Network Energy Savings and Mobility Optimization, the AI/ML Inference can be located in OAM or in RAN, depending on the use case latency requirements for inference.

Proposal 10:	For AI/ML based Network Energy Savings consider Cell and Carrier activation/deactivation for energy savings. Study the input, output and feedback data for AI/ML based Cell and Carrier Activation/deactivation.

Proposal 11:	For AI/ML based Network Energy Savings consider activation/deactivation of beams and beam steering. Study input, output and feedback data for AI/ML based beam activation and deactivation.

Proposal 12:	For AI/ML based Cell, Carrier, Beam activation/deactivation study RRM impacts in RAN (e.g Handover of UEs, Release of UEs etc).

Proposal 13:	For AI/ML based Mobility Optimization study Measurement and Measurement Event Prediction in the RAN node for Legacy L3 Handover. Consider Rel19 RAN2 AI/ML based mobility SI inputs from TR 38.744 as the baseline for RAN3 study.

# Annex (TP to TR 38.760)

7		AI/ML for NG-RAN

Artificial Intelligence (AI) and Machine Learning (ML) techniques can be utilised in RAN to optimize and to improve Network performance and User experience. Network can collect data from different sources to enable varied AI/ML use cases to enhance Network Performance.

7.1		AI/ML Use Cases

7.1.2		General Principles

AI/ML algorithms and models are out of 3GPP scope.

RAN AI/ML use cases should support initial model training at a centralized node. Initial model training and fine tuning should occur in a location that is trusted and can enforce user privacy protection. RAN AI/ML use cases should support flexible deployment of AI/ML inference in any RAN network function depending on the use case requirement. Input data for RAN AI/ML use cases can be collected from different sources (including UE, RAN, CN and 3rd party sources).

7.1.3		AI/ML based Energy Savings

AI/ML techniques can be used to improvise the Energy Savings in the Network. The following use cases can be studied for AI/ML based Network Energy Savings.

- Cell and Carrier activation/deactivation

- Beam activation/deactivation

7.1.4		AI/ML based Mobility Optimization

AI/ML techniques can be used to benefit RAN for early HO preparation, optimal resource allocation, avoid HO failures etc. The following use cases can be studied for AI/ML based Mobility Optimization for Normal L3 Measurement based Handover.

- Measurements and Measurement Events Prediction

# References

[1] RP-252912, Revised SID: Study on 6G Radio

[2] TR 38.760, Study on 6G Radio RAN3 aspects