3GPP TSG-RAN WG3 Meeting #130						R3-258251

Dallas, USA, 17 – 21 Nov. 2025

Agenda item:		10.3.2

Title:		(pCR to TR38.760-3) Transport Layer protocol consideration for 6G RAN

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the 6G SI [1], the following objects should be discussed by RAN3 that:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

This contribution will discuss the RAN-CN interface protocol stack.

# 2. Discussion

2.1 STCP vs QUIC for 6G RAN-CN interface

SCTP has been extensively and long-term used as a transport layer in 4G and 5G RAN interfaces, e.g., 4G S1-C, 4G X2-C, 5G NG-C, 5G Xn-C, and 5G F1-C, providing highly stable and reliable transportation of control signaling. At the same time, we can observe some drawbacks of SCTP in 5G systems. For instance, it lacks a built-in security mechanism; using IPSec for SCTP to compensate for this introducing overhead cost and reduces transmission efficiency. Furthermore, SCTP has a relatively high connection establishment latency due to its multi-way handshake. Additionally, the complex upgrades and migrations for the SCTP supporting system increase the maintenance cost. Furthermore, even though SCTP can handle multiple streams of data at once, a delay in one stream can still block the others which then cause Head-of-line blocking, so the congestion control is also not perfectly resolved.

Observation 1: SCTP transport layer provides stable and reliable transportation of control signalling for e.g. RAN-CN interface in the 4G/5G system, but it also has some drawbacks in terms of security, connection establishment latency, migration, and congestion control.

In the 6G, the new requirements are proposed, like cloud-native deployment, and service-based architecture proposed in the RANP meetings. Therefore, it is a chance to re-evaluate whether the SCTP as a transport layer protocol is suitable for 6G requirements.

Observation 2: The 6G raises new requirements and brings new challenges to the SCTP transport layer.

The new transport layer protocol QUIC (Quick UDP Internet Connections) has several attractive features for the 6G RAN-CN interface. It provides built-in security by default, and 0-RTT/1-RTT handshake directly reduces connection setup time. And QUIC has a better solution for congestion control because it supports multiple streams, and packet loss in one stream does not block others. Its use of a connection ID (instead of IP/port) enables better connection migration and software/hardware upgrading which is friendly to could-native deployment. Also, being implemented over UDP makes QUIC implemented over UDP, which makes it more flexible in data transportation and ideal for cloud RAN deployment.

Observation 3: Compared to SCTP, QUIC has several attractive features for 6G RAN-CN interface in terms of security, latency, congestion control, migration, and transportation flexibility.

However, it introduces a backward compatibility issue, since 6G nodes with QUIC are unable to communicate directly with existing 5G nodes with SCTP (e.g., 5G RAN to 6G CN, 6G RAN to 5G CN), potentially increasing the complexity of interworking transportation. Additionally, QUIC was introduced in CN, but it is still a new technology for the RAN, which would have more workload in RAN3 compared to SCTP. Furthermore, SCTP has successful deployment in the current 5G/4G network system, while the practical performance of QUIC has not been verified in RAN.

Observation 4: Introducing QUIC in the 6G RAN-CN interface will have backward compatibility issues, more workload in RAN3, and unverified performance.

Proposal 1: RAN3 is suggested to study whether to keep using SCTP or to adopt the new QUIC technology.

# 3. Conclusion

Observation 1: SCTP transport layer provides stable data transportation for RAN-CN interface in the 4G/5G system, but it also has some drawbacks in terms of security, throughput, latency, migration, and congestion control.

Observation 2: The 6G raises new requirements and brings new challenges to the SCTP transport layer.

Observation 3: Compared to SCTP, QUIC has several attractive features for 6G RAN-CN interface in terms of security, latency, congestion control, migration, and transportation flexibility.

Observation 4: Introducing QUIC in the 6G RAN-CN interface will have backward on compatibility issues, more workload in RAN3, and unverified performance.

Proposal 1: RAN3 is suggested to study whether to keep using SCTP or to adopt the new QUIC technology.

# 4. References

[1] RP-252912, Revised SID: Study on 6G Radio

# 5. Annex (pCR to TR38.760-3)

Start of Change

## 6.1		RAN-CN Interface

### 6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

### 6.1.2	RAN-CN Interface Functions

RAN-CN control plane interface supports following functions:

-	UE context management	: The functionality to manage UE context between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress;

-	PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress.

### 6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

The 6G RAN-CN interface for control plane has the following Options:

  - The SCTP/IP based interface

  - The QUIC based interface

### 6.1.4	Evaluation of RAN-CN Interface Options

Editor’s note: This chapter includes evaluation and comparison of RAN-CN interface options described in clause 6.1.3.

6.1.4.1 SCTP/IP based control interface

Advantages:

  - Highly stable and reliable transportation of control signaling.

  - Applied in 4G/5G for a long time.

Disadvantages:

  - It lacks a built-in security mechanism.

  - High connection establishment latency.

  - Complex upgrades and migration.

  - Head-of-line blocking is not perfectly resolved.

6.1.4.2 QUIC based control interface

Advantages:

  - built-in security.

  - No head-of-line blocking issue.

  - better connection migration and software/hardware upgrading.

  - friendly to could-native deployment.

Disadvantages:

  - backward compatibility to 4G/5G system.

  - practical performance of QUIC has not been verified in RAN.

## 6.2	RAN Internal Interfaces

Editor’s Note: This chapter shall comprehensively address study on CU-DU and CP-UP aspects including functional split considering interim milestone TSG#112 (June/2026) to take a decision during TSG#115: March 2027

Editor’s Note: This chapter should also address the interfaces between 6G RAN nodes (equivalent to Xn).

### 6.2.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN Internal Interfaces

End Change