3GPP TSG-RAN WG3 Meeting #130	R3-258322

Dallas, US, 17 - 21 November 2025

Title:	Correction on the Rel-19 set ID assignment in inter-CU LTM

Agenda Item:	9.2.5

Source:	Huawei, Nokia, Jio Platforms, CATT, China Telecom, Lenovo

Document for:	Discussion

1	Introduction

The Rel-19 Mobility Enhancement WI was completed at last meeting. The definition and usage of the Rel-19 set ID list (the proposedLTM-NoSecurityChangeID-List IE) need further clarification which relates to the following magical words in ASN.1.

LTMHandoverInformationRequest ::= SEQUENCE { proposedLTM-NoSecurityChangeID-List LTM-NoSecurityChangeID-List OPTIONAL, -- This IE may need to be refined. targetNG-RANnodeUEXnAPID NG-RANnodeUEXnAPID OPTIONAL, referenceConfiguration ReferenceConfiguration OPTIONAL, lTMConfigurationIDMappingList LTMConfigurationIDMappingList OPTIONAL, cSIResourceConfiguration CSIResourceConfiguration OPTIONAL, requestForCSI-RSResourceConfigForLayer1Measurements RequestForCSI-RSResourceConfigForLayer1Measurements OPTIONAL, requestForCSI-RSResourceConfigForEarlyCSIAcquisition RequestForCSI-RSResourceConfigForEarlyCSIAcquisition OPTIONAL, iE-Extensions ProtocolExtensionContainer { { LTMHandoverInformationRequest-ExtIEs} } OPTIONAL, ... }

This contribution provides further clarifications and analysis on the Rel-19 set ID assignment.

2	Discussion

2.1	Background

In Rel-19 Mobility Enhancement WI, RAN2 introduced a parameter named as LTM no security change ID (also called Rel-19 set ID) for each candidate cell to enable the UE determine whether security key change is needed or not in inter-CU LTM. RAN2 further clarified in the LS [ 1] that The LTM no security change ID  can also be used to determine  security key change in intra-CU LTM. The relevant text in the LS [1] is quoted as below.

The term “R19 set ID” refers to the “LTM no security change ID” configured to the UE for serving cell and candidate cells separately. RAN2 assumes that even for intra-CU LTM, the network is allowed to configure different R19 set IDs for different cells (including serving and/or intra-CU candidate cells) if the network wants to trigger security key update. From UE perspective, security key update is performed when the R19 set IDs are different between the serving cell and the target cell, whether it is intra-CU or inter-CU LTM can be transparent to the UE.

The key requirement of the Rel-19 set ID assignment is that candidate cells that need security key change in LTM have to be assigned with different Rel-19 set IDs with each other. While among the candidate cells in a single candidate gNB, the candidate cells that does not need security key change in LTM should be assigned with the same Rel-19 set ID.

With above guidance, RAN3 discussed the Rel-19 set ID assignment for candidate cells during LTM preparation. The discussed two solutions are:

- Option 1: The source gNB assigns a proposed Rel-19 set ID list for each candidate gNB and sends it in the HANDOVER REQUEST message, then candidate gNB assigns the Rel-19 set IDs for its own candidate cells and feedback to the source gNB in the HANDOVER REQUEST ACKNOWLEDGE message.

- Option 2: The source gNB assigns the R19 set ID for each candidate cell in the HANDOVER REQUEST message. The candidate gNB may update the R19 set ID to the one of its other candidate cells received earlier based on its security key change policy and reply the updated R19 set ID to the source gNB in the HANDOVER REQUEST ACKNOWLEDGE message.

Option 1 is adopted and captured into the specification currently for the sake of WI completion.

2.2	Rel-19 set ID assignment

LTM preparation is performed by a per candidate cell Handover Request procedure. Which means if there are multiple candidate cells in a candidate gNB, the source gNB needs to trigger several Handover Request procedures towards the same candidate gNB.

The intended option 1 discussed in the WI phase includes the following steps:

- The source gNB only includes the Proposed LTM No Security Change ID List IE once which is in the very first HANDOVER REQUEST message, containing the proposed LTM No Security Change IDs assigned to all the candidate cells in a certain gNB.

- In the response HANDOVER REQUEST ACKNOLEDGE message, the candidate gNB includes the selected LTM No Security Change ID for a concerned candidate cell.

- For any additional new candidate cell preparation on the same candidate gNB, the source gNB needs to include a extended Proposed LTM No Security Change ID List IE with new Rel-19 Set IDs in the HANDOVER REQUEST message for the new candidate ells.

The existing specification implementation for option 1 only includes an optional Proposed LTM No Security Change ID List IE in HANDOVER REQUEST message, a procedural text for the optional IE, and a mandatory IE in HANDOVER REQUEST ACKNOLEDGE message.

[…] If the Proposed LTM No Security Change ID List IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the LTM No Security Change ID IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell, and act as specified in TS 38.300 [9]. […] In Handover Request: >Proposed LTM No Security Change ID List; O; 9.2.3.xx15; –. […] In Handover Request Ack: >LTM No Security Change ID; M; INTEGER(1..9,…); Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10].; –.

The existing specification implementation has ambiguities compared to the intended option 1 in 4 aspects:

- The existing procedural text is  problematic, as it means that the source gNB needs to include the Proposed LTM No Security Change ID List IE in every HANDOVER REQUEST message. This contradicts the intended solution discussed at last meeting.

- In the list, the source gNB should include as many Proposed LTM No Security Change IDs as the number of candidate cells in the candidate gNB. This may restrict that the source gNB implementation has to determine the number of candidate cells in each candidate gNB before the very first Handover Request procedure. Such kind of restriction is missing either in stage 3 or in stage 2 which may cause potential IOT issues.

- For new candidate cells added after, how the source NB to encode the proposed Rel-19 set ID list is ambiguous.

- There is no any content in stage 2 to support the stage 3 text“act as specified in TS 38.300 [9]” for Rel-19 set ID assignment.

The expected stage 2 impact for TS 38.300 may look like:

#### 9.2.3.5 L1/L2 Triggered Mobility ##### 9.2.3.5.1 General […] LTM No Security Change ID Assignment in inter-CU LTM The source gNB includes the proposed LTM No Security Change ID list in the very first HANDOVER REQUEST message to a candidate gNB. The number of the LTM No Security Change ID in the list equals to the number of candidate cells to be prepared in that candidate gNB. The candidate gNB selects the LTM No Security Change ID from the proposed LTM No Security Change ID list for each candidate cell and returns the selected LTM No Security Change ID in the corresponding HANDOVER REQUEST ACKNOLEDGE message associated to each candidate cell. For any new candidate cell added after, the source gNB should include the LTM No Security Change IDs of previous prepared candidate cells in the proposed LTM No Security Change ID list in the HANDOVER REQUEST message for the new candidate cell.

Proposal 1: To clarify above implicit implementation requirements on the Rel-19 set ID list assignment in either stage 2 or stage 3 specification to avoid any IOT issue.

If proposal 1 is not agreeable due to large specification impact, it is proposed to reconsider the option 2 which only contains limited specification impact as shown below (Procedural text can be removed if the IE is mandatory in both the request and acknowledge message):

[…] […] In Handover Request: >Proposed LTM No Security Change ID; M; 9.2.3.xx15; –. […] In Handover Request Ack: >Accepted LTM No Security Change ID; M; INTEGER(1..9,…); Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10].; –.

Proposal 2: To reconsider option 2 as illustrated above if proposal 1 is not agreeable.

3.	Conclusion

This contribution discusses the Rel-19 set ID assignment and proposes:

Proposal 1: To clarify the ambiguities on the Rel-19 set ID list implementation in either stage 2 or stage 3 specification to avoid any IOT issue.

Proposal 2: To reconsider option 2 as illustrated above if proposal 1 is not agreeable.

The CR for proposal 1 as option 1, and the CR for proposal 2 as option 2 are provided in [2] and [3].

4 	Reference

[1] R2-2503012 	LS on RAN2 agreements for security key handling in inter-CU LTM

[2] R3-258323	Correction on the Rel-19 set ID assignment in inter-CU LTM (Option 1)

[3] R3-258324	Correction on the Rel-19 set ID assignment in inter-CU LTM (Option 2)