3GPP TSG-RAN WG3 Meeting #130	R3-258330

Dallas, US, 17- 21 November, 2025

Title:	Consideration on RAN internal function split and interfaces

Agenda Item:	10.4

Source:	Huawei

Document for:	Discussion

- Introduction

The approved 6G SID [1] contains contents regarding RAN architecture, which includes the following bullet:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

To be continued...

And, based on the discussion about the “RAN internal split” topic in RAN3#129 bis, the Chair notes record the following to be continued issue:

In this contribution, we provide our views on the RAN internal functional split and interfaces.

- Discussion

  - Introducing “Xn-like” interface between 6G RAN nodes

6G RAN node

“Xn-like” interface

6G RAN node

Figure 1 Xn-like interface between 6G RAN nodes.

For the RAN interfaces, the “Xn-like” interface between 6G RAN nodes should be introduced for supporting the UE’s mobility, as well as some other features (e.g., dual connectivity, energy saving, resource coordination, etc.) if the necessity can be justified. In addition, the new services (e.g., AI, Sensing) in 6G may also have new requirements on coordination between 6G RAN nodes.

--------------Excerpt from TS 38.420--------------

The general principles for the specification of the Xn interface are as follows:

-	the Xn interface is open;

-	the Xn interface supports the exchange of signalling information between two NG-RAN nodes, and the forwarding of PDUs to the respective tunnel endpoints;

-	from a logical standpoint, the Xn is a point-to-point interface between two NG-RAN nodes. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the two NG-RAN nodes.

Regarding the general principles of such “Xn-like” interface for 6G, we think it can be similar as the principles for Xn interface in 5G, which are excerpted as below [2].

Proposal 1: For RAN internal interfaces, “Xn-like” interface between 6G RAN nodes should be supported, taking the new service requirements into account. RAN3 to reuse the general principles of the Xn interface for 6G “Xn-like” interface as much as possible, in particular:

-	the “Xn-like” interface shall support the exchange of signalling information between two endpoints, and the forwarding of PDUs to the respective endpoints;

-	from a logical standpoint, the “Xn-like” is a point-to-point interface between two endpoints. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

  - RAN internal functional split and interfaces

To study the RAN internal function split, it is worth to review the design of 5G RAN architecture first. The 5G RAN architecture supports CU-DU split, and CP-UP separation. F1 interface was introduced between gNB-CU and gNB-DU, and the E1 interface was designed between gNB-CU-CP and gNB-CU-UP.

Based on the discussion in the last RAN3 meeting, almost all companies have common understanding on the necessity of learning lessons from 5G RAN internal split. But different companies have quite diversified lessons from 5G CU-DU split, e.g., some companies observed only advantages and want to continue promoting in 6G, while some other companies identified many pain points and hope the motivation can be carefully justified for 6G.

The guidance from Chair notes of RAN3#129bis meeting [3] indicates that we should focus on the to be continued issue “Capture pain points and benefits (and potentially derived requirements) of 5G HLS”. This is important to derive the answer for whether we still need RAN internal functional split. In the following part, we try to give our view on the pain points and benefits of 5G high layer split, i.e., 5G CU-DU split.

    - Lessons from the 5G CU-DU split

Pain points of 5G CU-DU split

According to the continuous evolvement of 5G for several releases (from Release 15 to Release 19), the observed pain points of 5G CU-DU split are

- 1). Increased latency for both control plane and user plane. The 5G RAN CU-DU split will increase latency for both control plane and user plane handling when compared to the non-HLS RAN, considering the additional processing delay plus transport latency (especially for non-ideal backhaul) over the F1 interface.

- 2). Increased complexity for control plane.

  - 2.1) Inefficient RRC procedures. In 5G CU-DU split RAN architecture, the RRC anchor at network side is gNB-CU, while multiple lower layer configuration for the air interface is generated by the gNB-DU which requires additional signalling between gNB-CU and gNB-DU.

  - 2.2) Inefficient UE context management. In 5G CU-DU split RAN architecture, both the gNB-CU and the gNB-DU need to maintain UE context. Once the UE context needs some update, it will result in many F1AP procedures between the gNB-CU and the gNB-DU.

- 3) User plane performance impact. Apart from the increased latency for user plane data transmission as introduced in 1), the CU-DU split will face more challenge for user plane handling.

  - 3.1) Hard for implementing UP optimization mechanisms over F1-U. As detailed analysed in [4], 5G introduced many solutions for user plane to optimize the performance, some of them (e.g., unnecessary RLC retransmission avoidance, timely RLC retransmission/polling) require tight coordination between the PDCP layer and the RLC layer. Such solutions work well for the non-HLS RAN, but for CU-DU split RAN architecture, the coordination between PDCP and RLC layer at the RAN side needs to be conducted via the F1-U interface. The additional latency and complexity are inevitable for such scenario. Then, the performance gain of these solutions will be largely impacted.

  - 3.2) Slow flow control. Since the gNB-CU owns PDCP layer, but the radio resource is scheduled by the gNB-DU, the flow control in F1 interface relies on the report of Downlink Data Delivery Status (DDDS) from the gNB-DU. Considering the latency of the F1-U interface and when to generate the DDDS is up to gNB-DU implementation, such flow control may not be conducted in time.

- 4) Less efficient for launching new feature. Normally, to support a new feature in the CU-DU split RAN node, the operator should enhance the F1 interface (F1-C and/or F1-U) and upgrade of both gNB-CU and gNB-DU accordingly. Consequently, this may set more obstacles for supporting new feature.

- 5) Increased cost

  - 5.1) Increased OPEX. Considering that the gNB-CU and gNB-DU needs separate operation and management, the OPEX of split RAN is more than non-HLS RAN.

  - 5.2) Increased CAPEX: Transport network for F1. For the operators which do not have any existing transport network for F1 interface, the cost for deploying new transport network will increase the CAPEX.

- 6) Additional cost to meet high requirement for CU resiliency. Since the CU can connect to multiple DUs, the unavailable of CU will cause service interruption for large area, and the consequence will be unacceptable. The complexity and cost for achieving such high resiliency requirement at RAN side are not negligible.

Observation 1-1: The pain point of 5G CU-DU split includes:

1) Increased latency for both control plane and user plane

2) Increased complexity for control plane

2.1) Inefficient RRC procedures

2.2) Inefficient UE context management

3) User plane performance impact.

3.1) Hard for implementing UP optimization mechanisms over F1-U

3.2) Slow flow control

4) Less efficient for launching new feature

5) Increased cost

5.1) Increased OPEX

5.2) Increased CAPEX: Transport network for F1

6) Additional cost to meet high requirement for CU resiliency

Benefits of 5G CU-DU split claimed by proponents

During the study phase of 5G CU-DU split, the claimed benefits of CU-DU split from proponents include the following:

- Flexible CU/DU deployment. CU-DU split architecture can support flexible CU/DU deployment, which will be beneficial for meeting various requirements for multiple traffics.

- Centralized coordination. The CU/DU split architecture can gain from centralized coordination e.g., mobility and load management, etc.

- Introducing opportunities for more vendors. The standardized F1 interface can facilitate mutli-vendor supply chain eco-system. For example, the vendor of gNB-DU can be different from the vendor of gNB-CU. But, as far as we know, the 5G CU-DU split RAN has not been commonly deployed in commercial case, the multi-vendor CU-DU RAN is even rarer.

Observation 1-2: About 5G CU-DU split, the benefits claimed by proponents during 5G study phase include

1) Flexible CU/DU deployment

2) Centralized coordination

3) Introducing opportunities for more vendors

    - Summary

Based on the above analysis, it is recommended that RAN3 to evaluate whether to have high layer split (HLS) based RAN internal functional split in 6G.

Proposal 1: RAN3 to take the above analysis into account when evaluating whether to have HLS in 6G.

- Conclusion

This paper investigated the interface between 6G RAN nodes, and the RAN internal functional split, the following observations and proposals are derived:

Xn-like interface between RAN nodes

Proposal 1: For RAN internal interfaces, “Xn-like” interface between 6G RAN nodes should be supported, taking the new service requirements into account. RAN3 to reuse the general principles of the Xn interface for 6G “Xn-like” interface as much as possible, in particular:

-	the “Xn-like” interface shall support the exchange of signalling information between two endpoints, and the forwarding of PDUs to the respective endpoints;

-	from a logical standpoint, the “Xn-like” is a point-to-point interface between two endpoints. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

RAN internal split: CU-DU split

Observation 1-1: The pain point of 5G CU-DU split includes:

1) Increased latency for both control plane and user plane

2) Increased complexity for control plane

2.1) Inefficient for RRC procedures

2.2) Inefficient UE context management

3) User plane performance impact.

3.1) Hard for implementing UP optimization mechanisms over F1-U

3.2) Slow flow control

4) Less efficient for launching new feature

5) Increased cost

5.1) Increased OPEX

5.2) Increased CAPEX: Transport network for F1

6) Additional cost to meet high requirement for CU resiliency

Observation 1-2: About 5G CU-DU split, the benefits claimed by proponents during 5G study phase include

1) Flexible CU/DU deployment

2) Centralized coordination

3) Introducing opportunities for more vendors

Proposal 2: RAN3 to take the above analysis into account when evaluating whether to have HLS in 6G.

- Reference

- RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

- 3GPP TS 38.420, NG-RAN; Xn general aspects and principles.

- Chair notes of RAN3 #129bis meeting.

- R3-256711, OPPO, Discussion on 6G RAN internal interfaces, 3GPP RAN3#129bis meeting.

Annex: Text Proposal for 6G TR 38.760-3

Start of Change

6	RAN Architecture and Interfaces

Editor’s note: The aim of this section is to describe all aspects related to the Interface between gNB to CN.

------------------------------unrelated parts are skipped ----------------------------

6.2       RAN internal functional split and interfaces

Editor’s Note: This chapter should comprehensively address the CU-DU and CP-UP aspects considering interim milestone.

Editor’s Note: This chapter should also address the interfaces between 6G NBs (equivalent to Xn)

6.2.X	Interface between RAN nodes

The “Xn-like” interface is applicable for the connection between two RAN nodes.

6G RAN node

“Xn-like” interface

6G RAN node

Figure 6.2.X-1  Xn-like interface between 6G RAN nodes.

6.2.X.1	General principles of interface between RAN nodes

The general principles for the specification of interface between RAN nodes are as follows:

-	the “Xn-like“ interface shall support the exchange of signalling information between two endpoints, and the forwarding of PDUs to the respective endpoints;

-	from a logical standpoint, the “Xn-like“ is a point-to-point interface between two endpoints. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.