3GPP TSG-RAN WG3 Meeting #130	R3-258361

Dallas, US, 17 – 21 November 2025

Agenda item:	10.2.2

Source: 	Lenovo

Title: 	6G RAN Logical Architecture

Document for:	pCR

1. Introduction

The general principles and requirements of 6G RAN architecture was discussed at the RAN3#129bis meeting with the following agreements [1].

## 5.1 General Principles All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles. The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS] ## 5.2 Deployment Scenarios The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914. - FFS on the implications of this requirement on 6G RAN architecture. - FFS whether all deployment scenarios of TR 38.914 can be supported.

In this contribution, we discuss the 6G RAN logical architecture.

2. Discussion

Functions hold by 6G RAN are related to what architecture/protocol should be supported by the 6G RAN architecture. To initiate the discussions on the number of functionalities to be supported by the 6G, it would be useful to take the 5G gNB functions as a reference at first stage.

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling);

-	IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data;

-	Selection of an AMF at UE attachment when no routing to an AMF can be determined from the information provided by the UE;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Support of UEs in RRC_INACTIVE state;

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Dual Connectivity;

-	Tight interworking between NR and E-UTRA;

-	Maintain security and radio configuration for User Plane CIoT 5GS Optimisation.

Among the functions, the DC was discussed without agreements. RAN plenary also agreed to make a decision on DC by September 2026. Therefore, these functions can be added later to avoid occupying too much time of technical study. For the tight interworking between different RATs and the UP CIoT optimization, there is no need to cover them at the very beginning. For NAS messages distribution, there may be some enhancements e.g., to support distributed NAS, which depends on SA2’s discussion.

Proposal 1: NR functions are considered as a reference to 6GR, while the DC, tight interworking between 6GR and other RAT (e.g. NR or E-UTRA), and UP CIoT optimization are not considered at the beginning.

Motivated by the relatively recent uptake by many operators of 5GC, it is reasonable to assume that the 6G CN is an evolution of the 5GC to reduce the investments. In line with the assumption, the interfaces related to the 6G RAN can be the evolution of the interfaces related to the 5G RAN, which have been successfully deployed. For example,

-	The 6G RAN is connected to the 6G CN via an NG-C like interface for control plane, e.g., by means of the evolution of NG-C interface.

-	The 6G RAN is connected to the 6G CN via an NG-U like interface for user plane, e.g., by means of the evolution of NG-U interface.

-	The different 6G RAN nodes (e.g., 6G BS) are interconnected with each other via an Xn like interface, e.g., by means of the evolution of Xn interface.

The 6G network are expected to include new service types beyond traditional communication, such as data collection, sensing and AI. The characteristics of those new services are quite different from legacy functions. Service-based interface (SBI) between the RAN and CN for the new functions supporting the new service types, which can avoid too many P2P interface definitions and unnecessary transparent forwarding among network entities, thus enable efficient coordination between the RAN and CN. In addition, the RAN can more directly expose information to the CN.

Figure 1 shows an example of the overall 6G RAN architecture.

Figure 1: Overall 6G RAN Architecture

Proposal 2: 6G RAN can inherit the following interfaces, with potential enhancements:

-	The 6G RAN node is connected to the 6G CN via an NG-C like interface for control plane, e.g., by means of the evolution of NG-C interface. The 6G RAN node is also connected to the 6G CN via a SBI interface to support the new services such as AI, ISAC and etc.

-	The 6G RAN node is connected to the 6G CN via an NG-U like interface for user plane, e.g., by means of the evolution of NG-U interface.

-	The different 6G RAN nodes (e.g., 6G BS) are interconnected with each other via an Xn like interface, e.g., by means of the evolution of Xn interface.-

Compared with the 5G RAN, the 6G RAN should support some new services, e.g., AI, sensing.

It is highly probable that 6G-RAN nodes and devices will be equipped with AI-related capabilities. For the AI service, it typically requires high computing demand to guarantee the quality, low latency to support the real-time communication, and high security on the exposed AI data. To satisfy these requirements, the 6G RAN can provide new functions to support the AI services. For example, the new function supports the AI computing resource management, online AI model training, AI model inference, AI data collection and exposure, etc.

Proposal 3: The 6G RAN architecture should support AI functions, e.g., computing resource management, model training, model inference, data collection and exposure.

For the sensing service, it typically requires data privacy, low latency and flexible deployment (e.g., BS mono-static, BS-BS bi-static, BS-UE bi-static, UE-BS bi-static, UE mono-static and UE-UE bi-static). To satisfy these requirements, the 6G RAN can provide a new function to support the sensing service. For example, the new function supports the sensing measurement configuration, sensing datacollection to obtain the cloud of points or sensing results, etc.

Proposal 4: The 6G RAN architecture should support sensing functions, e.g., sensing measurement configuration, sensing data collection.

For these new RAN functions, there are two possible deployment options.

-	Option 1: New RAN functions are integrated into 6G BS. With this option, the 6G BS provides the new RAN functions, and no new node in the 6G RAN is introduced. The benefit is to keep the 6G BS as a single point towards the 6G CN and other 6G BS, while the drawback is the low efficiency of deployment and collaboration between different 6G BSs for support new services.

-	Option 2: The new RAN functions are deployed independently from the 6G BS providing communication service. With this option, one or more RAN nodes will be introduced in the 6G RAN architecture. The benefit is a unified RAN node for easy deployment, while the drawback is to introduce new interface between the 6G BS and the new RAN node.

Proposal 5: RAN3 to discuss the following options to support the new RAN functions related to AI service and sensing service.

-	Option 1: The 6G BS provides the new RAN functions.

-	Option 2: Introduce a node different from the 6G BS in the 6G RAN to provide the new RAN functions.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: NR functions are considered as a reference to 6GR, while the DC, tight interworking between 6GR and other RAT (e.g. NR or E-UTRA), and UP CIoT optimization are not considered at the beginning.

Proposal 2: 6G RAN can inherit the following interfaces, with potential enhancements:

-	The 6G RAN node is connected to the 6G CN via an NG-C like interface for control plane, e.g., by means of the evolution of NG-C interface. The 6G RAN node is also connected to the 6G CN via a SBI interface to support the new services such as AI, ISAC and etc.

-	The 6G RAN node is connected to the 6G CN via an NG-U like interface for user plane, e.g., by means of the evolution of NG-U interface.

-	The different 6G RAN nodes (e.g., 6G BS) are interconnected with each other via an Xn like interface, e.g., by means of the evolution of Xn interface.

Proposal 3: The 6G RAN architecture should support AI functions, e.g., computing resource management, model training, model inference, data collection and exposure.

Proposal 4: The 6G RAN architecture should support sensing functions, e.g., sensing measurement configuration, sensing data collection.

Proposal 5: RAN3 to discuss the following options to support the new RAN functions related to AI service and sensing service.

-	Option 1: The 6G BS provides the new RAN functions.

-	Option 2: Introduce a node different from the 6G BS in the 6G RAN to provide the new RAN functions.

4. Reference

- TR 38.760-3 v0.1.0: Study on 6G radio RAN3 aspects.

# 5. Text Proposal

pCR for TR 38.760-3.

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

5.3	Overall RAN Architecture

Editor’s note: The logical architecture can be captured here and should include the entire architecture including possible influence from existing and new services.

A 6G RAN node is either:

-	a 6G BS providing the 6GR user plane and control plane protocol terminations towards the UE; or

-	an 6g-gNB providing the NR user plane and control plane protocol terminations towards the UE.

The logical nodes in 6G RAN are interconnected with each other via an Xn like interface, which is the evolution of Xn interface.

The logical node in 6G RAN is connected to the 6G CN via an NG-U like interface for user plane, which is the evolution of NG-U interface.

The logical node in 6G RAN is connected to the 6G CN via an NG-C like interface for control plane, which is the evolution of NG-C interface.

The logical node in 6G RAN is also connected to the 6G CN via an SBI interface to support the new services such as AI, ISAC and etc.

The 6G RAN architecture is illustrated in Figure 5.3-1.

Figure 5.3-1: 6G RAN architecture

5.3.1	RAN-CN Functional Split

Editor’s note: The aim of this section is to describe functions split between RAN and CN.

5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN.

Functions similar to NR as listed in TS 38.300:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink and downlink;

-	IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data;

-	Selection of 6G CN node at UE attachment when no routing to a 6G CN node can be determined from the information provided by the UE;

-	Routing of User Plane data towards 6G CN;

-	Routing of Control Plane information towards 6G CN;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information;

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Support of UEs in RRC_INACTIVE state;

-	Distribution function for NAS messages;

-	Radio access network sharing.

Functions specific for 6G RAN:

-	AI service support

-	This function provides the capability for 6G RAN to support AI service, such as computing resource management, model training, model inference, data collection and exposure.

-	Sensing service support

-	This function provides the capability for 6G RAN to support sensing service, such as sensing measurement configuration, sensing data collection.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>