3GPP TSG-RAN WG3 #130	R3-258366

Dallas, USA, 17th -21st November 2025

Agenda Item:	10.5.1

Source:	Lenovo

Title: 		Consideration on 6G AI use cases

Document for:	Discussion and Decision

# Introduction

In the last RAN3 meeting, RAN3 agreed to study the AIML based network energy saving, and AIML based mobility optimization use cases. In this paper, we discuss the initial consideration on AIML based NES and AIML based mobility as well as other possible use cases that can be studied.

The design of AI/ML algorithms and models for RAN3 led use cases are implementation specific and out of RAN3 scope. The following use cases will be studied in RAN3: - AI/ML based Network energy saving - AI/ML based mobility optimization Other use cases can be discussed in future meetings based on contributions.

# Discussion

## AIML based Network Energy Saving

6G SID - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

According to 6G SID, the design of non-AIML based network energy saving will be studied separately. There is also a general guidance that the AIML based use case discussion needs coordination with related design areas.

At this early stage, basic non-AIML based NES mechanisms have not been studied, and it is also unclear what measurement metric will be specified and available at RAN that can be used as input for AIML based NES. Thus, we believe it is beneficial for RAN3 to study aspects that is AIML specific and form/capture high level goals/principles with respect to AIML based NES.

The input for AIML based NES depends on the specification/definition of RAN available measurements as well as the non-AIML based NES design, which will be discussed separately.

The general goal for the AIML based NES use case can be that the AIML based network energy saving use case aims to reduce overall RAN energy consumption globally or within a concerned region. For example, the energy consumption reduced in one RAN node shouldn’t bring higher energy consumption increase in another RAN node.

RAN3 captures the goal of AIML based NES use case is to reduce the overall RAN energy consumption globally or within a concerned region.

In addition, one aspect that RAN3 didn’t consider when discussing the AIML based network energy saving in 5GA is the energy consumption caused by AIML operation itself (e.g., training/inference). It is certainly not the intention/goal if the additional energy consumption due to AIML training/inference is much higher than the reduced energy consumption at the RF TX/RX chain.

Therefore, RAN3 is suggested to study the approaches to evaluate the energy consumption due to the operation of AIML (e.g., training/inference) in the AIML based NES.

For AIML based NES, RAN3 is suggested to consider the energy consumption due to the AIML operation (e.g., training/inference) and study the approaches to evaluate.

## AIML based Mobility Optimization

The similar situation exists for the AIML based mobility optimization use case. The UE mobility framework is under study and led by RAN2. The input can be used for AIML based mobility optimization from RAN3 point view will depend on the general mobility framework design.

6G SID - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

The input for AIML based Mobility Optimization depends on the general mobility framework and RRM framework design, which will be discussed separately and led by RAN2.

Therefore, at this early stage, RAN3 is suggested to clarify the general goal for the AIML based Mobility Optimization use case. In our understanding, the goal of this use case is to reduce the UE performance degradation during mobility.

RAN3 captures the goal of AIML based mobility optimization use case is to reduce the UE performance degradation due to mobility.

Another aspect we noticed is that in 5GA AIML based mobility optimization discussion, RAN3 didn’t consider the AIML capability at UE side which may leads to suboptimal design.

- RAN2-led AI for Mobility: UE/RAN can predict RRM measurement result (e.g., of serving and neighbor cells) to select the appropriate target cell and trigger the handover proactively.

- RAN3-led AI based Mobility Optimization: RAN can predict the future UE trajectory to select the appropriate target cell.

In particular, RAN assumed the RAN can predict the UE trajectory while didn’t consider the UE side AIML capability to predict UE’s trajectory (which could be even more precise).

The UE side AIML capability (e.g., RRM prediction, trajectory prediction) can be also considered in RAN3 AIML based mobility optimization study.

## Other use cases

Besides the AIML based NES and AIML based mobility optimization use cases agreed last time, we believe the following AI for RAN use cases can be studied as well. In the meanwhile, we believe the AIML based Network Slicing can be considered as part of the AIML based load balancing with finer granularity.

- AILM based load balancing to improve:

  - Currently the load balancing decisions relying on the current/past-state cell load status are insufficient. The traffic load and resource status of the network changes rapidly, especially in the scenarios with high-mobility and large number of connections, which may lead to ping-pong handover between different cells, cell overload and degradation of user service quality.

  - It is difficult to guarantee the overall network and service performance when performing load balancing. For the load balancing, the UEs in the congested cell may be offloaded to the target cell, by means of handover procedure or adapting handover configuration. For example, if the Ues with time-varying traffic load are offloaded to the target cell, the target cell may be overloaded with new-arrival heavy traffic. It is difficult to determine whether the service performance after the offloading action meets the desired targets.

- AIML based Network Slicing to improve:

  - The fulfilment of target requirements derived from the SLA of each supported network slice.

  - AI/ML function can analyze metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements

- AIML based Coverage and Capacity Optimization to improve:

  - The reactive CCO approach used in non-AIML based solution: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

  - With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

RAN3 is suggested to also study the following AI for RAN use cases

  - AI based load balancing including different granularities, e.g., cell/slicing

  - AI based CCO

In 6G, the RAN is envisioned not just as a conduit for wireless connectivity but as an intelligent, programmable platform that can offer AI inference, training, or general-purpose computing resources to external entities—such as:

- Cloud or centralized AI orchestrators

- Core Network Entity

- Third-party applications (e.g., AR/VR, autonomous vehicles, smart factories)

This is often referred to as "RAN-as-a-Service", where the RAN exposes its computational and AI resources through standardized APIs.

An example is illustrated as below, assuming a 6G RAN is already running its RAN functionality applications and has spare AI/computing resources available, it can expose those resources to the Operator’s AI Orchestration system. Based on operational needs, the Operator AI Orchestration can then configure and deploy its own AI applications (e.g., joint training of an AIML model) onto the RAN’s unused AI/computing resources (e.g., GPU, NPU). For example, by using Docker containerization, these applications can run independently without interfering with each other.

RAN3 studies the 6GRAN-AI-as-Service use case, wherein 6G RAN can expose its spare AI/computing resources to a different node.

# Conclusion

Based on the discussion above, we observe:

Observation 1	The input for AIML based NES depends on the specification/definition of RAN available measurements as well as the non-AIML based NES design, which will be discussed separately.

Observation 2	The input for AIML based Mobility Optimization depends on the general mobility framework and RRM framework design, which will be discussed separately and led by RAN2.

Based on the discussion above, we propose:

Proposal 1	RAN3 captures the goal of AIML based NES use case is to reduce the overall RAN energy consumption globally or within a concerned region.

Proposal 2	For AIML based NES, RAN3 is suggested to consider the energy consumption due to the AIML operation (e.g., training/inference) and study the approaches to evaluate.

Proposal 3	RAN3 captures the goal of AIML based mobility optimization use case is to reduce the UE performance degradation due to mobility.

Proposal 4	The UE side AIML capability (e.g., RRM prediction, trajectory prediction) can be also considered in RAN3 AIML based mobility optimization study.

Proposal 5	RAN3 is suggested to also study the following AI for RAN use cases

a.	AI based load balancing including different granularities, e.g., cell/slicing

b.	AI based CCO

Proposal 6	RAN3 studies the 6GRAN-AI-as-Service use case, wherein 6G RAN can expose its spare AI/computing resources to a different node.

# TP to TR38.760

***************************** Change Starts **********************************

## 7.1	AI/ML use cases

Editor’s note: Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

### 7.1.1 AIML based Network Energy Saving

AIML based NES use case aims to reduce the overall RAN energy consumption globally or within a concerned region.

### 7.1.2 AIML based Mobility Optimization

AIML based mobility optimization use case aims to reduce the UE performance degradation due to mobility.

***************************** Change Ends **********************************