3GPP TSG-RAN WG3 Meeting #130	R3-258543

# Dallas, Texas, U.S:, 17th - 21st November 2025

Agenda Item:	10.3.1

Source:	Ericsson

Title:	On General Principles and Requirements for the 6G RAN-CN interface

Document for:	Discussion and Decision

# 1	Introduction

In [1] a new Study Item was approved to study aspects of the 6G RAN.

This document discusses key aspects of the RAN-CN interface specification work for 6G.

# 2	Discussion

## 2.1	6G RAN-CN interface properties

Discussions on 6G RAN and 6G RAN interfaces include generally used terms denoting architectural items whose definitions would be beneficial to revisit. The respective definitions necessary to define a logical (RAN) architecture are provided below:

Logical entity: an architectural item hosting a clearly defined set of functions.

Application Protocol: a set of rules defining the way how a logical entity communicates with another logical entity by means of protocol data units and defining how a logical entity behaves upon reception of a protocol data unit (e.g. execute a function, store data, reply with a respective defined protocol data unit).

Logical interface: an architectural item via which a logical entity interacts with another logical entity by means of exchanging protocol data.

NOTE:	In the past we have followed certain criteria for whether interactions among logical entities are specified in stage-2 or stage-3, but that is for further detailed discussion.

Proposal 1:	Capture the above definitions of Logical Entity, Application Protocol and Logical Interface in the TR 38.760-3.

For Agenda Item 10.2 on architectural principles, we have provided functional reasons for assuming an architecture, where functions dealing with configuration and handling of the UE shall not be distributed among multiple logical entities, as this not only comes at the cost of complexity, sub-optimal configuration decisions and resulting impacts on the overall performance, but also reduces greatly the degrees of freedom with which RAN internal functionalities are realised, based on deployment and use-cases.

The 3GPP system architecture evolving across several generations has been defining a single termination point for the RAN-CN application protocol, both, at the RAN and the CN side. For legacy features to be supported in 6G, we believe that a reasonable starting point would be to assume equivalent architectural assumptions for the 6G RAN-CN interface as compared to 5G. Such assumption seems to be justified given the stability of 5GS (which builds on stability of 4G/EPS). This approach would avoid expensive and time-consuming specification and implementation work with uncertain result from system-stability point of view.

Proposal 2:	Agree on the assumption that for 5G functionalities also supported in 6G, 6G RAN-CN application protocol functions assume the same RAN-CN functional split as for 5G and that 6G RAN-CN Application Protocol functions are supported and executed at a single logical entity in the 6G RAN and the 6G CN.

Further, it might be worth to note that the resources offered by a RAN entity to realise a certain user requested service, are "localised resources", i.e. within a certain geographical area (supplied by physical cell resources) there is one, unique, RAN entity that is able to provide such "localised resource", and that there is one, unique, RAN entity that provides RAN communication resources to a certain user.

Observation 3:	For provision of necessary system resources for a user, the CN communicates with a unique, single logical RAN entity that provides radio resources for serving the user.

Such general principle is independent of the actual method and framework by which the 6G RAN-CN Application protocol is specified.

We have seen multiple documents at RAN3#129-bis that have identified the applicability of the interface principle that was established already in previous generations, see TS 38.410, clause 4.2. NG interface general principles

-	from a logical standpoint, the NG is a point-to-point interface between an NG-RAN node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the NG-RAN and 5GC;

And we agree, given the discussion above, that such definition is still applicable for 6G, irrespective of the method/framework for defining the RAN-CN AP.

Proposal 4: Agree to capture in TR 38.760-3 the following principle and definition: 
"From a logical point of view, the 6G RAN-CN interface is a Point-to-Point Interface between a 6G RAN logical entity and a 6G CN logical entity".

"A Point-to-Point interface is a logical, standardized control plane interface between two logical entities. It enables the establishment of a logical association and the exchange of protocol data between the logical entities constituting such point-to-point interface in a secure way.
The realization of a logical point-to-point interface does not require a direct transport connection between the logical entities constituting it and allows flexible deployment scenarios including virtualized and cloud environments."

## 2.2	General considerations and principles

State of the art "logical node/entity" implementation/deployment

As already considered and discussed for 5G, modern realisations of what is called a "logical node" moved away from "monolithic" implementations. Such design mindset of "monolithic" implementations allowed interface specification work to consider all node-functions to be built in a single "box", without considering what lies "behind" the assumed interface termination point.

Note:	We are using the term "logical node" in the discussion part in order to refer to terminology used in pre-6G generations. Usage of this term shouldn’t create inconsistencies/contradictions with the terminology suggested in section 2.1 for the sake of future discussions in the 6G RAN study.
Respective proposals will adapt the suggested terms.

Even more valid in 2025, roughly 10 years after the start of 5G discussions, "logical nodes" may be realised in a  way where nearly all RAN functions can be supported by COTS (commercial-off-the-shelf) computing platforms, while some other functions run on specialised hardware, and, depending on the function realised, in a highly scalable way.

Discussions on the design of RAN interfaces always followed the principle that, e.g., the RAN-CN (and RAN-RAN) interface should not be affected by such implementation choices. We always used to say that "logical node internal properties are implementation specific", but it can be shown that, for 6G, certain aspects of the current 5G framework for the major NG-RAN interfaces will have to be revisited  with respect to their suitability in terms of scalability, their ability to perform well in a cloud-environment, their ability to support a modular design of RAN protocols and functions.

We have been using the terms "scalability", modularity and "cloud environment" which should be clarified first.

Realisations of "logical nodes" may require the ability to add or remove computing resources for certain functions, as needed, depending on e.g. traffic fluctuation over different time scales and respective communication load, e.g. during rush hours, at fair-grounds/stadiums or considering network densification. The ability to dynamically adjust to traffic load may be made possible by realising functions in a data centre, and distributing processing load across multiple computing resources. The ability to add or remove, i.e. to "scale-in" and "scale-out" depending on the current traffic load should not directly affect basic application protocol function specification work, i.e. basic application protocol functions like e.g. UE mobility, UE resource control.

Further, we should continue assuming that a logical node's ability to expand/deflate processing power should neither be visible to the end user, nor shall it affect inter-vendor operability.

The means applied for node-internal distribution of internal threads, and the distribution of functions on diverse compute platforms should be transparent to other nodes interacting via standardised, open interfaces.

The standard should support a modular design of RAN protocols and functions by means of ensuring interoperable communication between such modules, and by not creating obstacles to enabling flexible modularisation.

Based on the above we would like to establish the following fundamental principles:

Proposal 5:	The 6G RAN-CN interface design shall enable realisation of 6G RAN and 6G CN entities' modular functions in a cloud environment while neither affecting inter-vendor operability nor being visible to end users.

Proposal 6:	How 6G RAN-CN interface traffic is distributed to RAN/CN entity-internal modular functionalities shall be transparent to the peer node.

## 2.3	Modularisation in 6G RAN-CN application protocol design

One aspect of RAN-CN application protocol specification is the ability to cluster node- and protocol functionalities depending on individual implementation and deployment choices. An implementation choice could be to realise a group of functionalities within a "module" and to be processed on a specific processing platform. This is not a new aspect, e.g. TS 38.410 §4.2 states that "the NG Application Protocol supports modular procedures design". The question is whether such "support" has been utilised in an advantageous way.

A positive example: 
Looking at the NGAP function "Paging", it can be easily seen that such function can be well "modularised", as it does obviously not have any interaction with other functions and because it is dedicated to a single functionality.

A not so-so-positive example:
Looking at the NGAP function "Path Switch", many can remember discussions in the 4G/5G past where due to optimisation reasons, a multitude of functions have been added to the "pure" switch of C-/U-Plane following an Xn handover execution. This is an example of a non-modular design, where one procedure serves many functionalities and where modifying or distributing one of the functionalities associated to the procedure has a cascade effect on all other functionalities.

Looking back to past discussions, the main argument of cropping protocol functions within several Elementary Procedures was seen in the reduction of the number of messages exchanged. Having a modularised design in mind, it becomes clear that such approach would create node-internal functional interdependencies and by that reducing the degrees of freedom in modularisation choices.

Note that considering modularisation during application protocol design is totally independent of architectural choices, it is independent of discussing protocol stacks, etc. It affects the (RAN) application protocol design only.

Also note that modular application protocol design allows maintenance and introduction of new functions in a modular way, which enables scalability on a node-internal functional level.

Proposal 7:	The 6G RAN-CN control plane interface design shall consider expected 6G logical entity realisations applying modularisation by

-	defining dedicated procedures for different protocol functions

-	minimising interactions between the entity-internal functions

-	avoiding serving multiple functionalities with a single procedure as much as possible.

## 2.4	End-to-end aspects of 6G RAN-CN application protocol specification

### 2.4.1	6G RAN-CN Interface Signalling Transport

TS 38.413 Clause 6, "Services Expected from Signalling Transport" states the following:

The signalling connection shall provide in sequence delivery of NGAP messages. NGAP shall be notified if the signalling connection breaks.

Following this statement, NGAP does not contain any protocol functions that probe the TNLA availability, it is able to contain class 2 procedures (w/o reception confirmation on AP level) and there is no need for an AP-level mechanism for guaranteed and in-sequence message delivery.

Having logical node realisations with modular, distributed, parallelised processing in mind we can imagine an example implementation where the interface signalling TNL termination point is followed by a load balancer (for initiating messages) that further is followed by an entity that distributes incoming traffic to actual processing resources, each processing resource potentially responsible for very specific tasks, according to implementation and deployment choices on how to group functions, as shown in the figure below:

Figure 2.3-1: Potential implementation of a logical RAN node.

The current RAN interface 5G protocol stack takes care of guaranteed/in-sequence delivery only up to the TNL termination point, as shown on the left side. If a message gets lost inside the node, e.g., on the way from the TNL termination point to the actual processing entity, as shown on the very right side of the figure above, there is no mechanism in the current system to recover from this situation. In other words, the signalling transport on NG-C is not able to cater for losses/duplication etc. caused by node internal failures, and the 5G RAN application protocol specifications do not contain any functions to cope with such effects.

To take it to an extreme, 5G RAN interface specifications assume that all functions processing AP messages are concentrated at the TNL termination point of the interface termination point, which is unrealistic and contradicts the common expectation of today's node realisations.

One approach to alleviate the problem arising from above example implementation is to de-couple the application protocol (AP) from the signalling transport underneath, i.e. in principle to expect from the signalling TNL only delivery of messages.

A "TNL agnostic" design of the application protocol would consequently adhere to an "end-to-end" principle, i.e. the AP design would need to provide protocol functions like application-level reliability and connectivity check instead of delegating such features to the signalling transport.

Proposal 8:	6G RAN-CN (and RAN-RAN) application protocol design shall be decoupled from the signalling transport, i.e. it shall be "signalling TNL agnostic", i.e. it shall not expect any other function from the signalling transport than the delivery of SDUs.

### 2.4.2	First details how 6G RAN-CN Application Protocol could follow a "signalling TNL agnostic" design approach

RAN APs support addressing of signalling-association instances for e.g. UE-associated signalling by means of AP IDs (see section 6.2 in TS 38.401). Such signalling associations are in principle also used explicitly e.g. for measurement related signalling (Measurement IDs on Xn), see also TS 38.401 Clause 6.2.

To apply this approach to a modularised cloud environment, the scope of such identifiers may need some discussions, but it becomes evident, that means are missing to route AP messages to the proper functional modules' instance without decoding the full AP message, i.e. "beyond" the signalling TNL termination point.

Proposal 9:	6G RAN-CN control plane interface transports information that enables efficient routing of AP messages supporting 6G logical entity realisations in a modularised cloud environment.

One consideration we would like to bring forward already at this stage of discussions is as follows:

It might be worth to consider efficiency of routing by introducing a routing header (functionally) outside  the application message, but yet encoded as and transmitted with the application message. Taking into consideration that the distribution entity at the logical node may need to serve several protocols/interfaces, such routing header could consist of a protocol identification (in 5G terms e.g. "NGAP", "XnAP"), a protocol function discriminator (in RAN3 AP terms e.g. the Elementary Procedure code) and functional module instance identifications on sending and receiving (source/target) side.

If such outside-header is of fixed length, it could be easily masked at the distributing entity at the receiving side. This would enable decoding of the outside-header only, for example by functions that would route the message based on the outside-header content. Such functions would not need to spend processing cycles decoding the full application message.

While the allocation of Protocol IDs and Protocol Function Discriminator IDs needs explicit specification, source and target identification for the functional module instance could be achieved by simply defining a numerical identifier. The allocation of source/target instance identification at source/target side being would be up to the source/target entity to decide/organise, while the target/source side receiving such identification would not need to interpret it. Also mapping of protocol functions (procedures) to node internal functional modules reflects the implementation/deployment choice.

# 3	Conclusion and Proposals

We have discussed key aspects of the 6G RAN-CN interface specification work, which leads to the following proposals:

Proposal 1:	Capture the following definitions of Logical Entity, Application Protocol and Logical Interface in the TR 38.760-3:

Logical entity: an architectural item hosting a clearly defined set of functions.

Application Protocol: a set of rules defining the way how a logical entity communicates with another logical entity by means of protocol data units and defining how a logical entity behaves upon reception of a protocol data unit (e.g. execute a function, store data, reply with a respective defined protocol data unit).

Logical interface: an architectural item via which a logical entity interacts with another logical entity by means of exchanging protocol data.

Proposal 2:	Agree on the assumption that for 5G functionalities also supported in 6G, 6G RAN-CN application protocol functions assume the same RAN-CN functional split as for 5G and that 6G RAN-CN Application Protocol functions are supported and executed at a single logical entity in the 6G RAN and the 6G CN.

Observation 3:	For provision of necessary system resources for a user, the CN communicates with a unique, single logical RAN entity that provides radio resources for serving the user.

Proposal 4: Agree to capture in TR 38.760-3 the following principle and definition: 
"From a logical point of view, the 6G RAN-CN interface is a Point-to-Point Interface between a 6G RAN logical entity and a 6G CN logical entity".

"A Point-to-Point interface is a logical, standardized control plane interface between two logical entities. It enables the establishment of a logical association and the exchange of protocol data between the logical entities constituting such point-to-point interface in a secure way.
The realization of a logical point-to-point interface does not require a direct transport connection between the logical entities constituting it and allows flexible deployment scenarios including virtualized and cloud environments."

Proposal 5:	The 6G RAN-CN interface design shall enable realisation of 6G RAN and 6G CN entities' modular functions in a cloud environment while neither affecting inter-vendor operability nor being visible to end users.

Proposal 6:	How 6G RAN-CN interface traffic is distributed to RAN/CN entity-internal modular functionalities shall be transparent to the peer node.

Proposal 7:	The 6G RAN-CN control plane interface design shall consider expected 6G logical entity realisations applying modularisation by

-	defining dedicated procedures for different protocol functions

-	minimising interactions between the entity-internal functions

-	avoiding serving multiple functionalities with a single procedure as much as possible.

Proposal 8:	6G RAN-CN (and RAN-RAN) application protocol design shall be decoupled from the signalling transport, i.e. it shall be "signalling TNL agnostic", i.e. it shall not expect any other function from the signalling transport than the delivery of SDUs.

Proposal 9:	6G RAN-CN control plane interface transports information that enables efficient routing of AP messages supporting 6G logical entity realisations in a modularised cloud environment.

We tried to derive definitions and requirements suitable to be captured in the RAN3 internal TR in R3-258544 and finally propose:

Final Proposal: Discuss whether definitions and requirements in R3-258544 are suitable and agreeable to be included in the RAN3 internal TR.

# 4	References

[1]	RP-252912 "Revised SID: Study on 6G Radio" latest version as of RAN#109