3GPP TSG-RAN3 #130                                                                              R3-258573

# Dallas, USA, November 17-21, 2025

Agenda item:	10.4

Source:	Rakuten, Samsung, NTT DOCOMO, Verizon, Qualcomm, FiberCop

Title:	Insights from a commercial deployment of multi-vendor standardized F1 interface

Document for:	Discussion and Decision

# Introduction

In RAN#108 meeting [1], the 6G WG SID was approved with the following objectives on RAN architecture:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In RAN3#129-bis meeting, 6G RAN internal functional split was discussed and the following was captured in the Chair’s notes for scope of discussion in the next meeting.

Capture pain points and benefits (and potentially derived requirements) of 5G HLS To be continued...

In this paper, we will present Rakuten’s experience with deploying multi-vendor standardized F1 interface in the commercial deployments in Japan and outside. In the process, we will address the questions which were brought to us in the previous meeting by various participating companies.

# Rakuten’s Deployment

Rakuten Mobile’s network, starting as a green-field deployment in Japan, provides one of the most interesting real-world cases of a CU–DU split architecture with multi-vendor RAN. In its 4G network, the split used a proprietary midhaul interface; and when rolling out the 5G network, the operator moved to standardized interfaces (including the 3GPP F1 interface and O-RAN compliant units). Rakuten’s decision to adopt the paradigm of a split, cloud-native radio access network (RAN) rather than a traditional monolithic baseband design from day one represents one of the most significant architectural departures in mobile-network engineering. Rakuten now has widescale deployment of 4G and 5G sites both in terms of macro and small cells. It has embraced the true spirit of the Open RAN architecture and has collaborated with multiple vendors to adapt to various deployment use cases.

Rakuten Mobile's deployment of a fully virtualized, cloud-native Open RAN network has had a significant impact on the Japanese telecom industry, influencing the adoption of CU/DU split architectures by the incumbent operators. Since its full-scale launch in April 2020, Rakuten has built a nationwide 5G network based on Open RAN standards, deploying over 17,000 outdoor base stations as of May 2024. It has partnered with vendors for enterprise small cells, femto cells, 5G Sub6 small cells, 5G mmWave macros and indoor small cells. There are upwards of 150K+ small cells with third-party DUs in Rakuten deployment in Japan and currently under trial in Europe as well. With this, Rakuten Mobile has demonstrated that RAN disaggregation can be achieved successfully, leading to a more diversified and resilient network infrastructure.

2.1 Background

From the beginning of its operations in Japan, Rakuten has sought to build a fully virtualized and software-driven network. To achieve the vision, Rakuten’s RAN solution employs a functionally split architecture with separated logical elements, namely Central Units (CU), Distributed Units (DU), and Radio Units (RU), that can be deployed at different locations and interconnected via IP-based links.

Traditional RAN solutions are built as monolithic baseband units (BBUs), where all protocol stack layers, from the packet data convergence protocol (PDCP) down to the physical (PHY) layer, are integrated on a single proprietary hardware platform co-located with the radio. This design provides tight control over timing but creates a vertically integrated ecosystem which is dominated by a few vendors. It results in high site costs, limited scalability, and strong vendor dependence.

In contrast, a split or virtualized RAN (vRAN) separates functions according to their latency sensitivity. Upper-layer tasks (PDCP, RRC) are hosted in centralized data centers as virtual network functions (VNFs) or containers, while real-time layers (RLC, MAC, PHY) are hosted on edge servers near the radios. This virtualization paradigm provides significant deployment flexibility, addressing the limitations of the traditional RAN architecture and, more importantly, eliminating vendor lock-in within each mobile network.

2.2 Strategic Motivation for Rakuten’s Split Design Adoption

Cloud-native virtualization and centralization

Rakuten’s design embraced cloud-native virtualization, running CU and DU functions as software instances on COTS servers in data centers rather than proprietary boxes at every site. The split deployment allows multiple DUs to connect to a single CU. By centralizing the CU functions, this design facilitates the pooling of computing resources across many cells, known as C-RAN pooling gains, thereby reducing total baseband capacity requirements. By placing CU functions in regional or central data centers, Rakuten was able to consolidate control-plane and non-real-time user-plane functions, which are located in the upper layers of the protocol stack and less sensitive to latency, onto commodity servers such as RDCs rather than relying on specialized DSP hardware. Real-time processing (PHY/MAC layer) remains at the DU near the cell site, reducing latency for user-plane data while still allowing centralized management.

Vendor disaggregation and openness

Monolithic RANs tie software tightly to proprietary hardware, leading to vendor lock-in. By splitting the CU and DU and using a disaggregated RAN with standardized (or proprietary) interfaces, Rakuten is able to mix and match components from multiple vendors. Rakuten deliberately pursued hardware–software disaggregation by adopting a functionally split architecture that enabled CU and DU components from different vendors to operate within a multi-vendor model. The design created a foundation for what would later mature into full Open RAN interoperability. The modular approach reduced dependency on a single vendor and allowed Rakuten to negotiate better business terms and conditions, integrate innovative hardware faster, and gradually expand the vendor ecosystem as the standards evolved. It also helped mitigate supply-chain risks, which is critical for a greenfield operator building a nationwide network.

Automation and operational agility

Running CU and DU functions in a cloud environment allowed Rakuten to apply DevOps methodologies - continuous integration and deployment (CI/CD), automated scaling, and “zero-touch” configuration. For instance, in Rakuten’s network, new 5G small cells could be commissioned in the order of minutes rather than hours or days. Moreover, network software updates could be performed centrally, with orchestration managed through Rakuten’s cloud platform. Such agility is unattainable in a hardware-bound monolithic system.

Cost optimization

Deploying a nationwide RAN is capital-intensive. Monolithic systems require proprietary hardware at every site, leading to high CAPEX and maintenance costs. Replacing proprietary BBUs (which require specialized hardware requirements at each site) with virtualized software on COTS servers lowered capital expenditure and enabled elastic resource allocation. CU resources can be shared across multiple DUs or scaled dynamically depending on traffic. As the network traffic demand fluctuates over time and across locations, compute resources could be dynamically reallocated, minimizing idle capacity. Furthermore, reducing power consumption, particularly critical for macro sites, can deliver substantial energy savings. Rakuten has reported significant reduction in total cost of ownership (TCO) compared with legacy architectures. Since Rakuten has started as a green field network, part of cost savings also stems from equipment footprint reduction, hardware virtualization, multi-vendor sourcing, and automation.

Scalability and rapid deployment

Because CU and DU capacities can scale independently, new cells or enterprise small-cell networks can be added without redesigning the entire baseband infrastructure. The split architecture enables efficient scaling, particularly in dense urban areas. The flexibility led to improved QoS management and network elasticity, which is critical for a greenfield operator competing with established MNOs. This elasticity supported Rakuten’s accelerated rollout, where nationwide 4G coverage was achieved within only two years.

Path to 5G and beyond

Although the initial LTE deployment used a proprietary CU–DU interface in 4G days, its structure reflected the 3GPP Option 2 split, which was later standardized as the F1 interface for 5G. The same architectural principles thus facilitated a seamless migration to 5G Open RAN using standardized, interoperable interfaces. Over time, as 5G and Open RAN matured, Rakuten transitioned to standardized interfaces (F1 in 3GPP, O-RAN splits), achieving full multi-vendor interoperability. By adopting a split design from the outset, the network was inherently more modular and adaptable.

Observation 1: There are several strategic reasons which led Rakuten to adopt the split architecture in its deployment in both 4G (with proprietary midhaul) and 5G (with standardized F1 interface) networks.

2.3 Integration Challenges and Mitigation

Rakuten’s choice of a split architecture introduced non-trivial engineering challenges.

Multi-vendor integration and proprietary midhaul (4G phase): When the CU–DU interface was not standardized (for LTE), Rakuten used a proprietary midhaul between the CU and DUs. Integrating with multiple hardware vendors (radios, DUs) meant aligning interface semantics, transport behavior, flow control, synchronization and timing. The lack of a defined standard made this process complex. Rakuten mitigated this complexity by working closely with its CU, DU and hardware vendors to develop and test the interface, adopt COTS hardware, define internal interoperability procedures and aggressively automate commissioning. This approach was planned as a stepping-stone for migrating to a standardized F1 interface.

F1-U interface and flow control: Although 3GPP defines F1-U, the standard leaves implementation flexibility in areas such as QoS handling, GTP-U extension headers, buffering, and flow control. One of the key integration challenges in a multi-vendor environment lies in the lack of fully harmonized flow-control and buffering mechanisms across the two vendors’ DU and CU implementations. Since different vendors may apply different strategies for things like PDCP/GTP-U packet burst handling, buffer thresholds, or back-pressure signaling, when the DU from vendor A and the CU from vendor B communicate over F1-U, the mismatched assumptions can lead to unexpected jitter, packet drops, or throughput fluctuations. To mitigate this, Rakuten enforced joint interoperability testing, standardized test-cases for flow-control scenarios, and explicit agreement on buffer/threshold/QoS behavior for F1-U between vendors. Rakuten used dedicated PoCs and quality-assurance systems to validate end-to-end flows and developed automated verification environments for multi-vendor RAN components.

Midhaul latency for eMBB: The latency between CU and DU, midhaul latency, doesn’t always need to be as tight as early assumptions suggested. Extensive testing and field experience from Rakuten Mobile’s virtualized 4G and 5G deployments demonstrated that CU–DU midhaul latency can be significantly higher than traditionally assumed without degrading eMBB service performance. Early C-RAN designs targeted < 5ms one-way latency between CU and DU; however, Rakuten’s measurements showed that latencies up to 20ms were operationally acceptable for broadband traffic [7]. Since the time-critical HARQ and MAC scheduling loops are contained entirely within the DU, the CU’s higher-layer functions are tolerant of longer round-trip delays. This finding enabled Rakuten to centralize CU resources in regional data centers, simplifying architecture, reducing the number of edge sites, and dramatically lowering TCO. The relaxed midhaul latency tolerance also simplified multi-vendor integration and made cloud-hosted CU deployments a viable option for standard mobile broadband services.

Edge-native user plane: In a split RAN architecture, the operator can decide where to anchor the user plane based on its need. It can either be centralized at the CU or distributed closer to the DU. For most eMBB traffic, 10–20ms midhaul latency is acceptable, however, for URLLC, gaming, or low-latency enterprise applications, the additional CU–DU transport delay can exceed acceptable round-trip budgets (often 5–10ms end-to-end). Rakuten has shown that by placing the user plane closer to the DU dramatically reduces transport latency. This approach allows the system to meet stringent low-latency requirements for applications such as URLLC, AR/VR, and edge analytics, while retaining centralized control-plane management for coordination and policy enforcement.

SoC-Based Distributed Units: System-on-Chip (SoC) based DU designs have emerged as a cost-effective solution for implementing the latency-critical Layer 1/2 functions in 4G and 5G RANs. By integrating baseband accelerators and general-purpose cores onto a single silicon die, SoCs eliminate the need for expensive x86 servers or FPGA accelerator cards at each DU site. This integration lowers bill-of-materials cost, reduces power consumption, and enables compact, passively cooled form factors suitable for small cells and indoor deployments. Rakuten Mobile leveraged SoC-based DUs from some vendors to achieve large-scale CAPEX reduction in its dense small-cell layer, while retaining x86/FPGA-based DUs from other vendors for macro sites requiring higher throughput and flexibility. The standardized CU–DU interface allows both hardware types to coexist under the same CU domain, enabling a unified operational model and faster product evolution across the ecosystem.

Automation, commissioning and scaling: Deploying thousands of sites, often with multiple vendors, required advanced automation and orchestration. Achieving that required standardized software, containerized CU/DU, automated workflows, and multi‐vendor plug-in orchestration. This challenge is addressed in the Rakuten deployment framework by building a cloud-native vRAN infrastructure, adopting Kubernetes/CNF, and rigorous automation protocols.

Vendor supply chain, cost pressures and ecosystem immaturity: At the time Rakuten began commercial operations, Open RAN and multi-vendor splits were relatively new; many vendors had limited product maturity, and integration risk was high. Rakuten mitigated this by limiting number of Stock Keeping Unit (SKUs), adopting standard hardware, working closely with vendors, building internal integration capability and gradually expanding the multi‐vendor ecosystem as standards matured. By limiting the number of SKUs, operators reduce complexity in deployment, testing, integration, and inventory management. A smaller number of SKUs mean fewer unique configurations to validate across the network, which is critical in a multi-vendor Open RAN environment like the one deployed by Rakuten Mobile.

Performance, reliability and user-experience concerns: There was risk that a disaggregated, virtualized RAN might under-perform compared to legacy monolithic RAN. Rakuten had to demonstrate that throughput, latency, reliability would be comparable. They achieved measurable performance successes: for example, Rakuten hit 1.77 Gbps in mmWave tests on its 5G network in Japan, which matched the theoretical maximum for the given configuration. Rakuten has consistently monitored critical KPIs, tuned software, leveraged automation/self-healing and refined orchestration to maintain performance.

Bottom line is that there are several challenges to rolling out a new network whether monolith or disaggregated. However, the perception of performance impact (in terms of compromised throughput or additional latency) with a split architecture is just subjective and has not been observed in the Rakuten network. At the start of 5G, perhaps the technology was not mature, and the risk of moving from monolithic architecture was too much for operators to take on. However, almost a decade later, there are several operators around the world now who have successfully deployed the split RAN architecture and are keen to realize the benefits of it in 6G as well.

Observation 2: There were several challenges which must be overcome with the F1 interface in a multi-vendor deployment use case. Many of these are “early pain points” and integration and IoT testing becomes a part of the roll out process, irrespective of architecture choices. Rakuten has established best practices to handle these in a manner which can be carried over to 6G also.

# RAN CU-DU split Benefits

Despite the challenges, Rakuten achieved significant benefits from its CU–DU split and multi-vendor architecture. The following are some of the benefits of the split architecture which were observed in Rakuten’s 4G and 5G networks and will remain crucial for 6G going forward:

Enabling open eco-system: In an open environment, multi-vendor hardware and software can coexist on a common platform. This breaks proprietary vendor lock-in and allows the Mobile Network Operators (MNOs) to configure their networks using a cherry-picking approach, selecting the most competitive components from different vendors as shown in Figure 1, which reduces the total cost of ownership.

Figure 1: Open eco-system enabled by the RAN CU-DU split

Diverse deployment scenarios: The CU-DU split provides flexibility to support a variety of deployment scenarios, allowing the MNOs to tailor their network architecture to specific use cases, traffic patterns, and cost considerations. The choice of deployment scenario also depends on factors such as service requirements (e.g., latency), available transport network resources, and the desired level of centralization. The functional entities, RU, DU, CU-UP, and CU-CP, can be physically placed at various tiers in the network. These tiers include Cell site, Edge site, Regional Site and Central Site as. The choice of different options, from a fully distributed to a fully centralized RAN architecture, can address different requirements, offering the associated benefits (such as edge computing, high throughput at high mobility or centralized processing and simplified distributed coverage), within constraints (such as cost or transport requirements).

Figure 2: Deployment scenarios with disaggregated architecture

Single CU software stack: Having a single CU domain residing at an edge server or an RDC for multiple macro and small cells allows consistent handling of handover, mobility, scheduling, and interference coordination. For example, a user moving from an indoor femto to an outdoor macro cell remains under the same CU context, there is no inter-CU signaling or context transfer delay. This kind of deployment reduces latency during mobility and simplifies QoS and bearer management across the entire coverage layer. Moreover, this option simplifies the interface with the core network, where only one NG-C interface is needed. Bug fixes, performance optimizations, and new features are implemented centrally rather than per cell type. This unification shortens release cycles and accelerates product maturity.

Enhanced operational management: The split architecture supports reducing the operational cost. Consolidating CUs in a central location allows for simplified network management, maintenance, and software updates across many cell sites. This architecture reduces the need for frequent and costly on-site visits. Moreover, it brings down the cost of power consumption. Centralizing the power-intensive processing equipment reduces overall energy consumption compared to powering baseband units at every cell site.

High availability and resiliency: The CU-DU split architecture enhances high availability and resiliency by decoupling failures, enabling centralized redundancy, and facilitating rapid recovery. In this model, if a Distributed Unit (DU) fails, the impact is isolated to that specific cell's coverage area, allowing the Centralized Unit (CU) to manage mobility and handovers to a neighboring, functioning cell, minimizing service disruption. Centralizing CUs in a data center allows for robust redundancy through server virtualization, where a failing CU instance can be quickly relocated or restarted on a healthy server without requiring physical hardware replacement at a cell site.

Energy efficiency: In times of low network traffic, the virtualized functions of the CU and DU can be reallocated onto fewer servers, allowing other hardware to be put into a low power "sleep" state, dramatically reducing energy consumption. Also, by aggregating the baseband processing of multiple DUs, the CU can handle varying loads more effectively. This provides a high resource multiplexing gain, ensuring that processing power is used more efficiently across the network rather than being siloed in individual, often underutilized, base stations. This is particularly advantageous in scenarios with disparate traffic loads across different geographical areas.

Observation 3: Rakuten has observed numerous advantages of the RAN CU-DU split-based architecture which includes enabling open ecosystem with multi-vendor support, diverse deployment options to meet operator’s unique needs, network virtualization and cloud deployment options with the associated benefits.

# Adoption of disaggregation in commercial deployments

One of the most important points of discussion is the current state of adoption of the CU-DU split with open F1 interface in the commercial deployments world-wide. The adoption varies significantly across regions due to differences in market structure, infrastructure, spectrum policy, and strategic priorities. There are operators around the world, and many of them in Japan, who have successfully deployed this option in their commercial networks. Given this, it is critical to examine what technical and business reasons have been a factor for CU/DU split commercial adoption.

Most of the MNOs in Japan have conducted successful trials and commercially deployed the 3GPP split RAN. Japan has extremely dense urban environments, making Centralized RAN (C-RAN) and split architecture more attractive to manage resources efficiently. A centralized baseband unit pool can serve many nearby radio units with low latency fronthaul. The availability of extensive fiber coverage enables high throughput and low latency applications. Japanese operators (including Rakuten Mobile) are early adopters of Open RAN, which is based on 3GPP split interfaces (especially F1, E1, and Open Fronthaul). This supports multi-vendor interoperability, virtualization, and cloud-native deployment, which align with their strategies. There is strong governmental and industrial alignment in Japan to push for next-gen architectures, which favors adopting the 3GPP disaggregated options over the traditional monolithic architecture.

In USA, some of the legacy operators have successfully adopted the split RAN option in their deployment, as mentioned in the RAN3#129-bis meeting. Moreover, other operators have conducted trials and are interested in this option going forward in 6G. In China, the operators have traditionally favored the integrated architectures with domestic vendors, but are keen on exploring the split architecture option, especially for private networks and enterprise deployment scenarios. In India also, major operators are conducting trials with the standardized F1 interface and consider this to be a viable option for private and enterprise networks. Many of the European operators have invested heavily in traditional, integrated RAN and often multi-band macro cell deployments, making rapid transformation to split architectures less financially viable. Moreover, many operators (especially in Europe, the US, Australia, etc.) are devoting budget, engineering effort and planning to replacing or phasing out their incumbent equipment because of security/regulatory reasons arising out of geopolitical events. This has significantly impacted the MNO plans of network architecture evolution.

We have summarized the various benefits of the split RAN architecture in the previous section. The following is a summary of some of the factors which might be a hurdle for operators from adopting or experimenting with the standardized F1 interface.

Low penetration of 5G networks: By the end of 2024, 5G accounted only for 30% of the mobile connections in Europe [6]. And only a small fraction of this is standalone deployment. This is also indicative of the low adoption rates of the split architecture in Europe, where majority of the operation is still 4G, for which 3GPP has not defined a CU-DU interface.

Vendor lock-in: Some vendors believe a closed implementation on the CU-DU interface or a monolithic architecture can optimize network operations and guarantee performance. However, the proprietary implementations on the F1 interface limits interoperability, making it hard to swap out the incumbent components. Due to the limited product portfolio of CU/DU split by the established vendors, the MNOs are often discouraged from trying out the disaggregation option.

Disaggregated RAN isn’t plug-and-play: Multi-vendor integration across CU and DU introduces complexity which require interoperability validation and custom tuning and performance optimization across vendors. We have discussed some of these challenges in Section 2 of this document.

Perception of risk: Operators are risk-averse with infrastructure investments. Traditional monolithic RAN systems are field-proven and well-supported. Disaggregated RAN components - especially from new vendors - lack the same track record. This perception was also aided by traditional vendors who did not offer mature products with standardized F1 interfaces. However, this is rapidly changing with the growing ecosystem of supporting vendors and with more operators adopting the Open RAN deployment.

Limited immediate ROI: The business case for disaggregated RAN is strongest at scale or in greenfield builds (e.g., Rakuten Mobile). Brownfield operators often do not see a clear ROI from moving to CU/DU split with open F1 interface unless they are already overhauling legacy networks or targeting edge-native use cases.

These issues discussed above are often a barrier to entry for new vendors. And all this combined has resulted in slow adoption of the disaggregated architecture.

Observation 4: There are many reasons for slow adoption of the CU/DU split option with open interfaces. These include additional testing and customization requirements, limited immediate ROI, perception of risk and strategic lock in by incumbent vendors. These issues are often a barrier to entry for new vendors and deter the operators from trying out the disaggregated option.

# Why should the RAN split (CU-DU split) be carried forward to 6G

In 6G, where demands are even more stringent than in 5G, evolving the RAN architecture (including dynamic functional splits, more edge cloud, possibly non-terrestrial, AI/ML in the loop, etc.) is thought necessary by many in this industry. Based on Rakuten’s successful deployment with 4G and 5G split architecture deployment, we list the following reasons justifying why it is important to continue to support these RAN split going forward with the 6G standardization activities.

Foundation for virtualization: 6G is envisioned as a fully cloud-native, intelligent, and highly automated network. The CU-DU split provides the necessary architectural foundation for a virtualized RAN that can be deployed on cloud platforms, a requirement for future 6G networks.

Statistical Multiplexing gains: In a monolithic RAN, each baseband unit (BBU) at a cell site is a self-contained system dedicated to that site’s traffic. Its compute, memory, and processing capacity are dimensioned for peak load, even though, in practice, traffic varies over time (e.g., high during day, low at night). Since compute resources cannot be shared across sites, much of the capacity remains underutilized. With a CU–DU split, multiple cells (DUs) are connected to the same CU pool at the RDC/CDC, their traffic fluctuations statistically average out where some cells may be busy while others are idle. This allows compute resources (vCPUs, memory, I/O) to be dynamically allocated where needed, achieving statistical multiplexing and significantly improving utilization efficiency. Analytical and simulation studies on centralized RAN architectures suggest that enlarging the pooling domain can improve compute-resource utilization by approximately 10–20 %, depending on traffic correlation and latency constraints [5].

Enhanced AI/ML integration: The centralization of the CU enables a central point to embed Artificial Intelligence (AI) and Machine Learning (ML) functionalities. The AI/ML entity can collect data from multiple network nodes to perform real-time optimization, interference management, and dynamic resource allocation, a core feature of future 6G networks. Logical nodes responsible for higher-layer functionalities, particularly the CU, are ideal to host AI/ML models and entities that collects data. This entity can be responsible for training the AIML models which are resource and computation intensive. The comparatively lightweight and less complex task inferencing can then be performed at the DUs.

Addressing higher traffic volumes: 6G will push traffic volumes far beyond what 5G handles today, requiring even more advanced resource management and coordination. The resource pooling and intelligent load balancing capabilities inherent in the CU-DU split will be essential for managing this explosive growth.

Maximizing network openness and agility: Continuing the Open RAN philosophy enabled by the CU-DU split will prevent vendor lock-in and encourage a more agile and innovative ecosystem for 6G development. This ensures the network can evolve rapidly to support new technologies and business models.

Support for diverse use cases: 6G will support a wider range of services with varying requirements for latency, bandwidth, and reliability, including non-terrestrial networks (NTN) like satellites. The flexible nature of the CU-DU split, with different possible functional splits, allows tailored deployments to meet these diverse needs.

- Highly dynamic geo-distributed computing: The CU-DU split is the basis for a future 6G network where computing resources are dynamically distributed across the network. This will support applications with ultra-low latency requirements, such as holographic communication and real-time augmented reality, by placing the compute power where it's most needed.

- Integrated Sensing and Communication (ISAC): 6G is expected to combine communication and sensing into a single technology. The split architecture would allow for the centralized processing of massive amounts of sensor data collected from the distributed DUs, enabling real-time environmental monitoring, accurate positioning, and enhanced awareness of the network environment.

- Seamless integration with NTN: The CU-DU split is critical for integrating satellite and High-Altitude Platform Station (HAPS) into the overall network. By keeping the lightweight DU on the satellite and the more intensive CU on the ground, the network can provide ubiquitous coverage in rural and remote areas while managing the unique constraints of NTN, such as resource limitations and mobility.

- Multi-TRP (Transmission and Reception Point): In 6G, the split architecture will enable more sophisticated multi-TRP operations. By centralizing the control plane in the CU, operators can orchestrate complex transmission and reception schemes involving multiple DUs to improve reliability and throughput.

- Advanced beamforming: By processing higher-layer signaling centrally, the CU can provide more sophisticated coordination for beamforming and signal processing, especially in massive MIMO deployments.

- Network Slicing and Multi Service Handling: It will be easier to configure logical splits and resource allocation per service type (URLLC, eMBB, IoT) using CU/DU architecture.

Observation 5: It is imperative that the support of CU-DU split with open F1 interface carries forward to the 6G design and standardization.

We have presented data from Rakuten’s deployment in Japan, highlighting the strategy behind the adoption of the split architecture in 4G (pre-standard) and 5G (standard-compliant) network. We have given a peak of the challenges and how we addressed them. Despite this, we observed many benefits of this architecture choice which we’d like to continue to see in 6G. Furthermore, we have also presented the technical and business reasons why 5G did not show significant adoption of these architectural features. We believe that there are several solutions to address the challenges, and a certain mindset shift is required by the operators and incumbent vendors in ensuring wider adoption of the split architecture. We have summarized the reasons why it is imperative to support the RAN split (CU-DU split) in 6G.  Based on all this, we propose that RAN3 take cognizance of the merits of the CU-DU split with open F1 interface and recommend it for 6G architecture.

Proposal 1: We propose that RAN3 takes cognizance of the merits of the CU-DU split with open F1 interface and recommend it for 6G architecture.

# Conclusion

We have the following observations:

Observation 1: There are several strategic reasons which has led Rakuten to adopt the split architecture in its deployment in both 4G (with proprietary midhaul) and 5G (with standardized F1 interface).

Observation 2: There were several challenges which must be overcome with the F1 interface in a multi-vendor deployment use case. Many of these are “early pain points” and integration and IoT testing becomes a part of the roll out process, irrespective of architecture choices. Rakuten has established best practices to handle these in a manner which can be carried over to 6G also.

Observation 3: Rakuten has observed numerous advantages of the RAN CU-DU split-based architecture which includes enabling open ecosystem with multi-vendor support, diverse deployment options to meet operator’s unique needs, network virtualization and cloud deployment options with the associated benefits.

Observation 4: There are many reasons for slow adoption of the CU/DU split option with open interfaces. These include additional testing and customization requirements, limited immediate ROI, perception of risk and strategic lock in by incumbent vendors. These issues are often a barrier to entry for new vendors and deter the operators from trying out the disaggregated option.

Observation 5: It is imperative that the support of CU-DU split with open F1 interface carries forward to the 6G design and standardization.

We have the following proposals:

Proposal 1: We propose that RAN3 takes cognizance of the merits of the CU-DU split with open F1 interface and recommend it for 6G architecture.

# 7		Reference

[1] Chair Notes RAN#108

[2] RP-252880: Moderator's summary for RAN led 6G SI

[3] Chair Notes RAN#109

[4] Chair Notes RAN3#129-bis

[5] NGMN Alliance, “vRAN and Cloud-RAN Architecture White Paper,” (2022)

[6] The Mobile Economy Europe 2025, GSMA Report

[7] Field presentation by Altiostar & Rakuten Symphony, TIP 2021-23