3GPP TSG-RAN WG3 Meeting #130	R3-258582

Dallas, US, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1653 | Rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NG-RAN node UE XnAP IDs transfer after LTM failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Offinno, Jio Platforms, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As based on RAN2 design, fast LTM recovery can be performed only in the candidate cells with the same LTM No Security Change ID as the source cell ( case 1: CELL SWITCH NOTIFICATION is not sent to the target gNB) or with the same LTM No Security Change ID as the target cell (case 2: CELL SWITCH NOTIFICATION is sent to the target gNB). For case 1, there is no need to transfer the last Target NG-RAN node UE XnAP IDs of all candidate cells to the new serving gNBs, as the fast LTM recovery cell is in the source gNB. If the UE reconnects to another candidate gNB, the correct procedure should be RRCReestablishment. In this case, XnAP Retrieval UE Context procedure is applied. And the LTM preparation in the candidate gNBs has to be cancelled or released. For case 2, there is no need to transfer the last Target NG-RAN node UE XnAP IDs of all candidate cells to the new serving gNBs once again either, as the CELL SWITCH NOTIFICATION has been already sent to the new serving gNBs in which contains the last Target NG-RAN node UE XnAP IDs. Scenario (case 2 abovementioned): LTM Configuration Update from source gNB to candidate gNB after LTM failure In inter-CU LTM, it was agreed in WI phase that “The old source gNB can deliver the old target UE XnAP ID(s) to the new serving gNB via Cell Switch Notification and LTM Configuration Update message.” In consequence, the last Target NG-RAN node UE XnAP ID IE for all candidate cells is included in the LTM CONFIGURATION UPDATE message. Transferring the last Target NG-RAN node UE XnAP IDs in the LTM CONFIGURATION UPDATE message was agreed for the LTM failure case. In which it assumes that a HANDOVER SUCCESS will be sent to the source gNB from the candidate gNB serving the new target cell where the UE performs fast LTM recovery. However, such assumption is not correct. As based on RAN2 design, fast LTM recovery can be performed only in the candidate cells with the same LTM No Security Change ID as the target cell, i.e., a candidate cell different from the target cell in the target gNB. As the CELL SWITCH NOTIFICATION has been already sent to the target gNB in which it contains the last Target NG-RAN node UE XnAP IDs, there is no need to transfer the last Target NG-RAN node UE XnAP IDs of all candidate cells to the target gNB once again by the LTM CONFIGURATION UPDATE message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following change is made to the XnAP for inter-CU LTM. - Remove the Last Target NG-RAN node UE XnAP ID IE for all candidate cells in the LTM CONFIGURATION UPDATE message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redundant IEs for unsupported function exit in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1, 9.3.5, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 9.1.5.1	LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| LTM Updates to Candidate Node Information |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration | O |  | OCTET STRING | Includes the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| LTM Updates to Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >LTM Updates to Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Early Sync Information | O |  | 9.2.3.218 | Indicates the consolidated early synchronization information to be used for subsequent LTM. | – |  |
| >>LTM CFRA Resource Information | O |  | 9.2.3.232 |  | – |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate cell identified by the Candidate Cell ID IE. | – |  |
| >>AS Security Information | O |  | 9.2.3.50 |  | – |  |
| >>LTM No Security Change ID | O |  | INTEGER(1..9,…) | Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10] | – |  |
| >>Data Forwarding Info for LTM List |  | 0..1 |  |  | – |  |
| >>>Data Forwarding Info for LTM Item |  | 1..<maxnoofPDUSessions> |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>>Data Forwarding Info from target NG-RAN node | M |  | 9.2.1.16 |  | – |  |
| LTM UE Security Information | O |  | 9.2.3.230 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<< Next Change >>

LTMUpdatesToCandidateCellInformation-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMUpdatesToCandidateCellInformation-Item

LTMUpdatesToCandidateCellInformation-Item ::= SEQUENCE {

candidateCellID							NR-CGI,

earlySyncInformation					EarlySyncInformation				OPTIONAL,

lTMCFRAResourceInformation				LTMCFRAResourceInformation 			OPTIONAL,

ueBasedTAMeasurementConfiguration		OCTET STRING 						OPTIONAL,

aSSecurityInformation					AS-SecurityInformation				OPTIONAL,

lTM-NoSecurityChangeID					LTM-NoSecurityChangeID				OPTIONAL,

lTMUEAssociation						LTMUEAssociationInformation-List	OPTIONAL,

dataForwardingInfoForLTM-List			DataForwardingInfoForLTM-List		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMUpdatesToCandidateCellInformation-Item-ExtIEs} } OPTIONAL,

...

}

LTMUpdatesToCandidateCellInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<< End of Changes >>