3GPP TSG-RAN WG3 Meeting #130 	R3-258655

Dallas, US, 17 – 21 November 2025

Agenda item:		12.2.2

Source:	LG Electronics Inc.

Title:	Discussion on open issues for intra-CU LTM use case

Document for:		Other

1. Introduction

In the last RAN3 meeting, for the intra-CU LTM use case, the following were agreed:

Prioritize normal intra-CU LTM scenarios. For CU-DU split architecture, the following two scenarios are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU. RAN3 to study the following - AI/ML assisted candidate cell selection for LTM Handover Preparation Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

Also, there were open issues as follows:

RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM. For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM. Further discuss whether to study addition of beam prediction in UE Trajectory prediction. Further discuss predicting TA value in the next meeting. Further discuss prediction of validity time of a measured TA value.

In this contribution, we focus on some open issues for the intra-CU LTM use case and provide our view.

2. Discussion

2.1 Addition of beam prediction in UE trajectory prediction

In the last RAN3 meeting, the following was agreed:

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

Our understanding of this agreement is to utilize AI/ML-based prediction information to select an appropriate beam at the time of handover execution to reduce beam selection failure. The main beam-related problems that cause handover failure are typically beam mismatch during early synchronization, target beam selection error, and SP-CSI-RS activation timing problem. These aspects can be addressed at the handover preparation stage.

Objective 1: Beam-related problems that lead to handover failure can be mitigated at the handover preparation stage.

Considering the above agreement, candidate beam handling can actively secure a wider set of candidate beams and select the most likely beam among them with AI/ML. In other words, beam selection can be improved without combining the beam with the trajectory. On the other hand, beam-trajectory coupling may require an evaluation methodology for beam prediction and may increase evaluation complexity, such as higher trajectory report granularity.

Objective 2: Without combining beam prediction with UE trajectory prediction, beam selection can be improved, and unnecessary increase in evaluation complexity can be avoided.

Based on the above observations, the following proposal is suggested:

Proposal 1: The addition of beam prediction in UE trajectory prediction may not be necessary within the scope of this study.

2.2 Prediction of TA value

During the offline discussion at RAN3#129-bis, some companies stated that they do not support TA value prediction because incorrect prediction of TA value can lead to LTM handover failures. However, the risk of incorrectness is not unique to TA prediction. Mobility decisions based on measurements already require validation. In addition, existing Rel-18 AI/ML mobility principles define data collection and feedback loops for evaluating prediction accuracy as highlighted in yellow as below [1]:

### 16.20.3 Data Collection and Reporting The following information can be configured to be reported by an NG-RAN node: - Predicted resource status information, including predicted radio resource status per slice; - UE performance feedback, including per-slice UE performance; - Measured UE trajectory; - Energy Cost (EC); - Predicted slice available capacity. The collection and reporting of the above information are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure. The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover. Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node. …

The same mechanisms can be reused to validate and adjust the TA prediction. Post-switch feedback (e.g., actual TA value of the target cell) can be collected to evaluate prediction accuracy and improve subsequent model behaviour, allowing self-correction over time.

Observation 3: Incorrectness risk is not unique to TA value prediction and can be mitigated or bounded using post-switch feedback mechanisms already in the Rel-18 AI/ML mobility optimization use case.

It is noted that early TA acquisition is identified as a major contributor to interruption time and service discontinuity during LTM procedures, and AI/ML-based prediction aims to reduce such interruptions rather than increase them. Also, TA prediction is not envisioned as a mandatory replacement of legacy early TA acquisition. Fallback mechanisms remain available at the implementation level. In other words, it can be used as auxiliary information to improve the success rate, and in case of uncertainty, a fallback can be performed.

Observation 4: TA value prediction is a means to reduce delay factors and can be operated as an assistive function.

Furthermore, during early synchronization, the signalling burden across F1 interfaces (e.g., between the gNB-CU and the source gNB-DU, as well as between the gNB-CU and the candidate gNB-DUs) increases as the number of candidate cells grows. Predicted TA values can alleviate this burden, providing trade-off benefits while keeping prediction risks manageable.

Observation 5: TA value prediction has the advantage of reducing F1 signalling burden.

Based on the above observations, the following proposal is suggested:

Proposal 2: TA value prediction for candidate gNB-DUs or cells using AI/ML functionality within the NG-RAN node should be considered.

If the TA value prediction for candidate gNBs or cells is considered, evaluating the prediction accuracy may be necessary. For example, if the TA value prediction was successful, the UE performs the RACH-less LTM with the target candidate cell. If the node that predicted the TA value is aware that the UE performed the RACH-less LTM based on feedback information from the target candidate cell, it can identify that there was no issue with the TA value prediction. If the TA value prediction was not done properly, the UE performs the RACH-based LTM with the target candidate cell. When it is known that the UE performed the RACH-based LTM, the node that predicted TA value can recognize that the prediction was incorrect.

Proposal 3: A procedure for evaluating the prediction accuracy should be considered if Proposal 2 is agreeable.

2.3 Inference in gNB-DU

In the last RAN3 meeting, on whether to consider the beam prediction, the following was agreed:

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

The gNB-DU has access to the L1 measurements, which contain rich and frequently updated information that is relevant for beam prediction. If the gNB-DU transmits the L1 measurements to the gNB-CU to enable the gNB-CU to perform the beam prediction, it may introduce significant signalling overhead and increase the burden on the F1 interface. Additionally, considering the TA value prediction, the gNB-DU may perform TA value prediction based on L1 measurements, while the gNB-CU may perform TA value prediction based on L3 measurements.

Proposal 4: The model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

- Conclusion

In this contribution, we focused on some open issues for the intra-CU LTM use case and provided our view. The following observations and proposals are made:

Addition of beam prediction in UE trajectory prediction

Objective 1: Beam-related problems that lead to handover failure can be mitigated at the handover preparation stage.

Objective 2: Without combining beam prediction with UE trajectory prediction, beam selection can be improved, and unnecessary increase in evaluation complexity can be avoided.

Proposal 1: The addition of beam prediction in UE trajectory prediction may not be necessary within the scope of this study.

Prediction of TA value

Observation 3: Incorrectness risk is not unique to TA value prediction and can be mitigated or bounded using post-switch feedback mechanisms already in the Rel-18 AI/ML mobility optimization use case.

Observation 4: TA value prediction is a means to reduce delay factors and can be operated as an assistive function.

Observation 5: TA value prediction has the advantage of reducing F1 signalling burden.

Proposal 2: TA value prediction for candidate gNB-DUs or cells using AI/ML functionality within the NG-RAN node should be considered.

Proposal 3: A procedure for evaluating the prediction accuracy should be considered if Proposal 2 is agreeable.

Inference in gNB-DU

Proposal 4: The model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

Annex TP for TR 38.745

Proposal 5: It is proposed to agree the TP proposed for TR 38.745.

- References

- TS 38.300 v19.0.0

Annex TP for TR 38.745

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

# 4	Use cases and Solutions

## 4.1	AI/ML based Intra-CU LTM

### 4.1.1	Use case description

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [x].

Intra-CU LTM is specified in TS38.401[x].

AI/ML can be used to optimise Intra-CU LTM procedures, e.g., to enhance Network and UE performance, optimize resource allocation and reduce mobility failures.

### 4.1.2	Solutions and Standard impacts

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

For CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2		Input data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as input data:

From the UE:

- UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.3		Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

- Candidate cell and beam for LTM HO Preparation

- Predicted TA value

#### 4.1.2.4		Feedback of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM, the following feedback can be considered to be collected from gNB-DUs:

- Whether the UE performed the RACH-less LTM or the RACH-based LTM with the target candidate cell

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>