3GPP TSG-RAN WG3 #130																	          R3-258664

Dallas, US, 17th-21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1669 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME |  | Radio Access Network |  |  |  |  | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Corrections on LTM Early Sync resource update |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  |  |  |  |  | Date: |  |  | 2025-11-07 |  |  |  |  |
| Category: | F |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-19 |  |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |  |  |  |
| Reason for change: |  | - A source gNB-DU can use PRACH resources allocated for LTM early sync by a candidate DU for multiple UEs. However, the source gNB-DU is not informed when the available PRACH resources for LTM early sync are released or added. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce two new class 2 procedures, one from candidate gNB-DU to gNB-CU, and the other from gNB-CU to source gNB-DU, to notify that the PRACH resources previously allocated for LTM early sync are updated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The source gNB-DU has no clear understanding of the available PRACH resources allocated for early sync by other candidate DU, which may lead to early sync failures or interfere with other UEs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.2.x (new), 8.2.y (new), 9.2.1.x (new), 9.2.1.y (new), 9.4.3, 9.4.4, 9.4.5,9 .4.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | X |  |  | Other core specifications |  |  |  |  |  |  | TS 38.423 CR 1661 |  |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

- 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

(skip unchanged)

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |
| DU-CU Cell Switch Notification | DU-CU CELL SWITCH NOTIFICATION |
| CU-DU Cell Switch Notification | CU-DU CELL SWITCH NOTIFICATION |
| DU-CU TA Information Transfer | DU-CU TA INFORMATION TRANSFER |
| CU-DU TA Information Transfer | CU-DU TA INFORMATION TRANSFER |
| QoE Information Transfer Control | QOE INFORMATION TRANSFER CONTROL |
| RACH Indication | RACH INDICATION |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| Mobile IAB F1 Setup Triggering | MIAB F1 SETUP TRIGGERING |
| Mobile IAB F1 Setup Outcome Notification | MIAB F1 SETUP OUTCOME NOTIFICATION |
| Broadcast Transport Resource Request | BROADCAST TRANSPORT RESOURCE REQUEST |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| DU-CU Access And Mobility Indication | DU-CU ACCESS AND MOBILITY INDICATION |
| CU-DU Mobility Initiation | CU-DU MOBILITY INITIATION REQUEST |
| CLI Indication | CLI INDICATION |
| DU-CU Early RACH Resources Update | DU-CU EARLY RACH RESOURCES UPDATE |
| CU-DU Early RACH Resources Update | CU-DU EARLY RACH RESOURCES UPDATE |

<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>

8.2.x	DU-CU Early RACH Resources Update

8.2.x.1	General

The purpose of the DU-CU Early RACH Resources Update procedure is to to enable the gNB-DU to send the update of Early RACH Resources information to the gNB-CU. The procedure uses non-UE-associated signalling.

8.2.x.2	Successful Operation

Figure 8.2.x.2-1: DU-CU Early RACH Resources Update procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU Early RACH Resources Update message.

Upon reception of the DU-CU Early RACH Resources Update message, the gNB-CU shall, if supported, shall take the information into account.

8.2.y	CU-DU Early RACH Resources Update

8.2.y.1	General

The purpose of the CU-DU Early RACH Resources Update procedure is to to enable the gNB-CU to send the update of Early RACH Resources information to the gNB-DU. The procedure uses non-UE-associated signalling.

8.2.y.2	Successful Operation

Figure 8.2.x.2-1: CU-DU Early RACH Resources Update procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU Early RACH Resources Update message.

Upon reception of the CU-DU Early RACH Resources Update message, the gNB-DU shall, if supported, shall take the information into account.

<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>

9.2.1.x		DU-CU EARLY RACH RESOURCES UPDATE

This message is sent by the gNB-DU to inform the gNB-CU about the update of Early RACH Resources information.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DU to CU Early RACH Resources To Release List |  |  |  |  |  |  |
| >DU to CU Early RACH Resources To Release Item |  | 1..< maxnoofEarlyRACHResources> |  |  | – |  |
| >>gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| >>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 | For intra-gNB LTM case, this IE may contain the Early RACH Resources Requester ID. | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |  |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [35]. |  |  |
| >>Preamble Index List | M |  | 9.3.1.329 |  |  |  |
| DU to CU Early RACH Resources To Add List |  |  |  |  |  |  |
| >DU to CU Early RACH Resources To Add List Item |  | 1..< maxnoofEarlyRACHResources> |  |  | – |  |
| >>gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| >>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 | For intra-gNB LTM case, this IE may contain the Early RACH Resources Requester ID. | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |  |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [35]. |  |  |
| >>Preamble Index List | M |  | 9.3.1.329 |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>

9.2.1.y		CU-DU EARLY RACH RESOURCES UPDATE

This message is sent by the gNB-CU to inform the gNB-DU about the update of Early RACH Resources information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CU to DU Early RACH Resources To Release List |  |  |  |  |  |  |
| >CU to DU Early RACH Resources To Release List Item |  | 1..< maxnoofEarlyRACHResources> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |  |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [35]. |  |  |
| >>Preamble Index List | M |  | 9.3.1.329 |  |  |  |
| CU to DU Early RACH Resources To Add List |  |  |  |  |  |  |
| >CU to DU Early RACH Resources To Add List Item |  | 1..< maxnoofEarlyRACHResources> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |  |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [35]. |  |  |
| >>Preamble Index List | M |  | 9.3.1.329 |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of ASN.1 changes >>>>>>>>>>>>>>>>>>>>

9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM F1AP-CommonDataTypes

Reset,

ResetAcknowledge,

F1SetupRequest,

(skip)

MulticastContextNotificationConfirm,

MulticastContextNotificationRefuse,

MulticastCommonConfigurationRequest,

MulticastCommonConfigurationResponse,

MulticastCommonConfigurationRefuse,

BroadcastTransportResourceRequest,

DUCUAccessAndMobilityIndication,

DUCUAccessAndMobilityIndication,

SRSInformationReservationNotification,

CUDUMobilityInitiationRequest,

CLI-Indication,

DUCUCSIRSCoordinationRequest,

DUCUCSIRSCoordinationResponse,

CUDUCSIRSCoordinationRequest,

CUDUCSIRSCoordinationResponse,

DUCUEarlyRACHResourcesUpdate,

CUDUEarlyRACHResourcesUpdate

FROM F1AP-PDU-Contents

id-Reset,

id-F1Setup,

id-gNBDUConfigurationUpdate,

id-gNBCUConfigurationUpdate,

id-UEContextSetup,

id-UEContextRelease,

id-UEContextModification,

id-UEContextModificationRequired,

id-DUCUAccessAndMobilityIndication,

id-ErrorIndication,

id-UEContextReleaseRequest,

id-DLRRCMessageTransfer,

id-ULRRCMessageTransfer,

id-GNBDUResourceCoordination,

id-privateMessage,

id-UEInactivityNotification,

id-InitialULRRCMessageTransfer,

id-SystemInformationDeliveryCommand,

id-Paging,

id-Notify,

id-WriteReplaceWarning,

id-PWSCancel,

id-PWSRestartIndication,

id-PWSFailureIndication,

id-GNBDUStatusIndication,

id-RRCDeliveryReport,

id-F1Removal,

id-NetworkAccessRateReduction,

id-TraceStart,

id-DeactivateTrace,

id-DUCURadioInformationTransfer,

id-CUDURadioInformationTransfer,

id-BAPMappingConfiguration,

id-GNBDUResourceConfiguration,

id-IABTNLAddressAllocation,

id-IABUPConfigurationUpdate,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-accessAndMobilityIndication,

id-ReferenceTimeInformationReportingControl,

id-ReferenceTimeInformationReport,

id-accessSuccess,

id-cellTrafficTrace,

id-PositioningMeasurementExchange,

id-PositioningAssistanceInformationControl,

id-PositioningAssistanceInformationFeedback,

id-PositioningMeasurementReport,

id-PositioningMeasurementAbort,

id-PositioningMeasurementFailureIndication,

id-PositioningMeasurementUpdate,

id-TRPInformationExchange,

id-PositioningInformationExchange,

id-PositioningActivation,

id-PositioningDeactivation,

id-PositioningInformationUpdate,

id-E-CIDMeasurementInitiation,

id-E-CIDMeasurementFailureIndication,

id-E-CIDMeasurementReport,

id-E-CIDMeasurementTermination,

id-BroadcastContextSetup,

id-BroadcastContextRelease,

id-BroadcastContextReleaseRequest,

id-BroadcastContextModification,

id-MulticastGroupPaging,

id-MulticastContextSetup,

id-MulticastContextRelease,

id-MulticastContextReleaseRequest,

id-MulticastContextModification,

id-MulticastDistributionSetup,

id-MulticastDistributionRelease,

id-PDCMeasurementInitiation,

id-PDCMeasurementTerminationCommand,

id-PDCMeasurementFailureIndication,

id-PDCMeasurementReport,

id-pRSConfigurationExchange,

id-measurementPreconfiguration,

id-measurementActivation,

id-QoEInformationTransfer,

id-PosSystemInformationDeliveryCommand,

id-DUCUCellSwitchNotification,

id-CUDUCellSwitchNotification,

id-DUCUTAInformationTransfer,

id-CUDUTAInformationTransfer,

id-QoEInformationTransferControl,

id-RachIndication,

id-TimingSynchronisationStatus,

id-TimingSynchronisationStatusReport,

id-MIABF1SetupTriggering,

id-MIABF1SetupOutcomeNotification,

id-MulticastContextNotification,

id-MulticastCommonConfiguration,

id-BroadcastTransportResourceRequest,

id-SRSInformationReservationNotification,

id-CUDUMobilityInitiationRequest,

id-CLI-Indication,

id-DUCUCSIRSCoordination,

id-CUDUCSIRSCoordination,

id-DUCUEarlyRACHResourcesUpdate,

id-CUDUEarlyRACHResourcesUpdate

FROM F1AP-Constants

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

##### skip unchanged #####

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

##### skip unchanged #####

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

multicastGroupPaging						|

broadcastContextReleaseRequest				|

multicastContextReleaseRequest				|

pDCMeasurementReport						|

pDCMeasurementTerminationCommand			|

pDCMeasurementFailureIndication				|

measurementActivation						|

qoEInformationTransfer						|

posSystemInformationDelivery					|

dUCUCellSwitchNotification					|

cUDUCellSwitchNotification					|

dUCUTAInformationTransfer					|

cUDUTAInformationTransfer					|

qoEInformationTransferControl					|

rachIndication								|

timingSynchronisationStatusReport				|

mIABF1SetupTriggering						|

mIABF1SetupOutcomeNotification				|

broadcastTransportResourceRequest			|

dUCUAccessAndMobilityIndication			|

sRSInformationReservationNotification		|

cUDUMobilityInitiation						|

cLI-Indication								|

dUCUEarlyRACHResourcesUpdate			    |

cUDUEarlyRACHResourcesUpdate	         	,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

##### skip unchanged #####

dUCUCSIRSCoordination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCUCSIRSCoordinationRequest

SUCCESSFUL OUTCOME		DUCUCSIRSCoordinationResponse

PROCEDURE CODE			id-DUCUCSIRSCoordination

CRITICALITY				reject

}

cUDUCSIRSCoordination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDUCSIRSCoordinationRequest

SUCCESSFUL OUTCOME		CUDUCSIRSCoordinationResponse

PROCEDURE CODE			id-CUDUCSIRSCoordination

CRITICALITY				reject

}

dUCUEarlyRACHResourcesUpdate	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCUEarlyRACHResourcesUpdate

PROCEDURE CODE			id-DUCUEarlyRACHResourcesUpdate

CRITICALITY				ignore

}

cUDUEarlyRACHResourcesUpdate	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		cUDUEarlyRACHResourcesUpdate

PROCEDURE CODE			id-cUDUEarlyRACHResourcesUpdate

CRITICALITY				ignore

}

END

-- ASN1STOP

9.4.4	PDU Definitions

##### skip unchanged #####

LastVisitedLTMCells,

OnDemand-SIB1-Cell,

LTMSecurityInformation,

LTMInformationSCGAdd,

LTMInformationSCGMod,

TARemainingInfoList,

CSI-RSCoordinationRequestList,

CSI-RSCoordinationResultList,

Future-Coverage-Modification-Notification,

Predicted-CCO-Assistance-Information,

Neighbour-Future-Coverage-Modification-Notification,

EarlyRACHResourcesToRelease-List

EarlyRACHResourcesToAdd-List

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

##### skip unchanged #####

id-TARemainingInfoList,

id-CSI-RSCoordinationRequestList,

id-CSI-RSCoordinationResultList,

id-Future-Coverage-Modification-Notification,

id-Predicted-CCO-Assistance-Information,

id-NeighbourFutureCoverageModNotification,

id-EarlyRACHResourcesToRelease-List,

id-EarlyRACHResourcesToAdd-List,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofMRBsforUE,

maxnoofServingCellMOs

FROM F1AP-Constants

##### skip unchanged #####

-- **************************************************************

--

#### -- DU-CU EARLY RACH RESOURCES UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU EARLY RACH RESOURCES UPDATE

--

-- **************************************************************

DUCUEARLYRACHRESOURCESUPDATE  ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DUCUEARLYRACHRESOURCESUPDATEIEs}},

...

}

DUCUEARLYRACHRESOURCESUPDATEIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					  CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-EarlyRACHResourcesToRelease-List		CRITICALITY ignore	TYPE DUtoCUEarlyRACHResourcesToRelease-List				PRESENCE optional }|

{ ID id-EarlyRACHResourcesToAdd-List		CRITICALITY ignore	TYPE DUtoCUEarlyRACHResourcesToAdd-List				PRESENCE optional }

...

}

-- **************************************************************

--

#### -- CU-DU EARLY RACH RESOURCES UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- CU-DU EARLY RACH RESOURCES UPDATE

--

-- **************************************************************

CUDUEARLYRACHRESOURCESUPDATE  ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUEARLYRACHRESOURCESUPDATEIEs}},

...

}

CUDUEARLYRACHRESOURCESUPDATEIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					  CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-EarlyRACHResourcesToRelease-List		CRITICALITY ignore	TYPE CUtoDUEarlyRACHResourcesToRelease-List				PRESENCE optional }|

{ ID id-EarlyRACHResourcesToAdd-List		CRITICALITY ignore	TYPE CUtoDUEarlyRACHResourcesToAdd-List				PRESENCE optional }

...

}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

##### skip unchanged #####

#### -- C

####### skip unchanged #######

CUtoDUEarlyRACHResourcesToRelease-List ::= SEQUENCE (SIZE(1..maxnoofEarlyRACHResources)) OF EarlyRACHResourcesToRelease-Item

CUtoDUEarlyRACHResourcesToRelease-Item ::= SEQUENCE {

candidateCellID					NR-CGI,

preambleIndexList				PreambleIndexList,

rA-RNTI							RA-RNTI,

ie-Extension 					ProtocolExtensionContainer { { CUtoDUEarlyRACHResourcesToRelease-Item-ExtIEs} } OPTIONAL,

...

}

CUtoDUEarlyRACHResourcesToRelease-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CUtoDUEarlyRACHResourcesToAdd-Item ::= SEQUENCE {

candidateCellID					NR-CGI,

preambleIndexList				PreambleIndexList,

rA-RNTI							RA-RNTI,

ie-Extension 					ProtocolExtensionContainer { { CUtoDUEarlyRACHResourcesToAdd-Item-ExtIEs} } OPTIONAL,

...

}

CUtoDUEarlyRACHResourcesToAdd-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

####### skip unchanged #######

#### -- D

####### skip unchanged #######

DUtoCUEarlyRACHResourcesToRelease-List ::= SEQUENCE (SIZE(1..maxnoofEarlyRACHResources)) OF EarlyRACHResourcesToRelease-Item

DUtoCUEarlyRACHResourcesToRelease-Item ::= SEQUENCE {

gNB-ID 		                    GlobalGNB-ID,

sourceGNB-DU-ID 		        GNB-DU-ID,

candidateCellID					NR-CGI,

preambleIndexList				PreambleIndexList,

rA-RNTI							RA-RNTI,

ie-Extension 					ProtocolExtensionContainer { { DUtoCUEarlyRACHResourcesToRelease-Item-ExtIEs} } OPTIONAL,

...

}

DUtoCUEarlyRACHResourcesToRelease-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DUtoCUEarlyRACHResourcesToAdd-Item ::= SEQUENCE {

gNB-ID 		                    GlobalGNB-ID,

sourceGNB-DU-ID 		        GNB-DU-ID,

candidateCellID					NR-CGI,

preambleIndexList				PreambleIndexList,

rA-RNTI							RA-RNTI,

ie-Extension 					ProtocolExtensionContainer { { DUtoCUEarlyRACHResourcesToAdd-Item-ExtIEs} } OPTIONAL,

...

}

DUtoCUEarlyRACHResourcesToAdd-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

##### skip unchanged #####

id-DUCUTAInformationTransfer				ProcedureCode ::= 84

id-CUDUTAInformationTransfer				ProcedureCode ::= 85

id-QoEInformationTransferControl			ProcedureCode ::= 86

id-RachIndication							ProcedureCode ::= 87

id-TimingSynchronisationStatus				ProcedureCode ::= 88

id-TimingSynchronisationStatusReport		ProcedureCode ::= 89

id-MIABF1SetupTriggering					ProcedureCode ::= 90

id-MIABF1SetupOutcomeNotification			ProcedureCode ::= 91

id-MulticastContextNotification 			ProcedureCode ::= 92

id-MulticastCommonConfiguration				ProcedureCode ::= 93

id-BroadcastTransportResourceRequest		ProcedureCode ::= 94

id-DUCUAccessAndMobilityIndication			ProcedureCode ::= 95

id-SRSInformationReservationNotification	ProcedureCode ::= 96

id-CUDUMobilityInitiationRequest			ProcedureCode ::= 97

id-CLI-Indication							ProcedureCode ::= 98

id-DUCUCSIRSCoordination					ProcedureCode ::= 99

id-CUDUCSIRSCoordination					ProcedureCode ::= 100

id-DUCUEarlyRACHResourcesUpdate    		    ProcedureCode ::= XX  -- to be assigned by MCC

id-CUDUEarlyRACHResourcesUpdate    		    ProcedureCode ::= XX  -- to be assigned by MCC

<<<<<<<<<<<<<<<<<<<< End of ASN.1 changes >>>>>>>>>>>>>>>>>>>>