3GPP TSG-RAN WG3 Meeting #130	R3-258679

Dallas, US, 17 – 21 November 2025

Agenda Item:	10.2.1

Source:	ZTE Corporation

Title:	Discuss on MRO for inter CU LTM

Document for:	Approval

Introduction

In the RAN3#129bis meeting, the following progress was made.

1. Use cases At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.: BFR shortly after successful LTM cell switch, failure due to wrong beam and outdated TA; LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell; Near failure case: SHR successful HO Successful case: ping/pong Potential new cases will not be precluded. 2. Solution options Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable. For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as base line. For inter-CU LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line. For the connection failure cases: - After receiving a failure indication, the last serving gNB/gNB-CU performs initial analysis. - L3 based inter-CU LTM is within the scope of the WID

This contribution provides our views focus on MRO for inter-CU LTM.

Discussion

LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell

At last meeting ,the following is confirmed by the meeting.

For the connection failure cases:

-	After receiving a failure indication, the last serving gNB/gNB-CU performs initial analysis.

L3 based inter-CU LTM is within the scope of the WID

LTM is a cell switch procedure where a gNB receives L1 or L3 measurement report(s) from a UE, and based on these reports, the gNB may trigger a cell switch command via a MAC CE. When L3 measurements are involved, the UE typically sends a MeasurementReport message (L3 measurement result) to the gNB. The Source gNB determines to initiate LTM configuration and eventually decide the LTM execution based on L3 measurement results.

Unlike L1 measurements, which are generally based on beam quality, L3 based measurements are evaluated based on cell quality. This cell quality is derived by the UE performing measurements on multiple beams (at least one) of a cell and averaging the measurement results.

For split architectural , we have the following analysis table:

For L1 based LTM:

|  | Source DU | Source CU | Target DU | Target CU |
| --- | --- | --- | --- | --- |
| Improper L1 cell switch triggering | Possible: The DU, responsible for L1 filtering and processing, applies inappropriate filtering or misinterprets L1 measurement results (e.g., beam RSRP)3, leading to premature or delayed triggering. | N/A | N/A | N/A |
| Wrong target cell selection | N/A | Possible for wrong candidate cell list prepared. | Possible for filtering candidate cell list: Based on its internal network resource load, policy, or capability assessment, the Target CU decide whether to accept each candidate cell proposed by the Source CU. | Possible for filtering candidate cell list: The Target DU may not accept every candidate cell proposed by the Source CU. |

Proposal 1: For inter gNB L1 based LTM, the last serving gNB/gNB-CU is responsible for performing initial analysis.

- For failures caused by improper triggering , the source CU forwards the RLF report to the DU responsible for the failure.

- For failures caused by incorrect candidate LTM cell selection, the source CU performs MRO optimization itself or forwards the RLF report to the target gNB responsible for the failure.

For L3 based LTM:

|  | Source DU | Source CU | Target DU | Target CU |
| --- | --- | --- | --- | --- |
| Improper L3 cell switch triggering | Possible If the Source DU delays or fails to timely send the MAC CE after receiving the instruction from the CU due to its own scheduling or execution logic error, the timing of the LTM handover (HO) may be inappropriate, especially in a fast-changing radio environment. | Possible When the LTM is triggered based on L3 measurements, an incorrect judgment of the trigger timing by the Source CU leads it to send the CU-DU MOBILITY INITIATION REQUEST message to the Source DU, requesting cell handover or early synchronization. | N/A | N/A |
| Wrong target cell selection | N/A | Possible for wrong candidate cell list prepared. | Possible for filtering candidate cell list: Based on its internal network resource load, policy, or capability assessment, the Target CU decide whether to accept each candidate cell proposed by the Source CU. | Possible for filtering candidate cell list: The Target DU may not accept every candidate cell proposed by the Source CU. |

Proposal 2: For inter gNB L3 based LTM, the last serving gNB/gNB-CU is responsible for performing initial analysis.

- For failures caused by improper triggering, the source CU may performs MRO optimization or forwards the RLF report to the DU responsible for the failure.

- For failures caused by incorrect candidate LTM cell selection, the source CU performs MRO optimization itself or forwards the RLF report to the target gNB responsible for the failure.

Which node response for Target Beam Selection Failure in L3 based LTM

The DU is responsible for making the final LTM Cell switch decision and generating the Cell Switch Command MAC CE. This command must include the optimal TCI state ID (beam indication) for the Target Cell. If the Selected Measurement Quantities provided by the CU are inaccurate or point to a suboptimal SSB or CSI-RS, the DU may select a compromised Target Beam. A switch to a poorly selected beam can immediately lead to low radio link quality, resulting in Beam Failure Detection (BFD) or a required Beam Failure Recovery (BFR) post-switch.

When serving gNB/gNB-CU performs initial analysis, how can gNB-CU figure out Which node response for optimization?

Proposal 3: RAN3 to discuss which node response the optimization of wrong beam selection during L3 based LTM due to misappropriate assistant information provided by the CU.

BFR shortly after successful LTM cell switch & failure due to wrong beam and outdated TA

Summary of Rel-19 Beam Failure Recovery shortly after successful LTM Cell Switch (Case 1):

Successful LTM Cell Switch Definition: A successful LTM cell switch refers to either the successful completion of the Random Access (RA) procedure (for RACH-based LTM) or the successful reception of the RRCReconfigurationComplete message (for RACH-less LTM).

Failure Detection (Near Failure): The target gNB-DU identifies this near failure by detecting that the time gap between the successful LTM cell switch and the subsequent BFR in the same cell is smaller than a configured threshold (e.g. store_UE_cntxt). The specific timing value for this detection is up to implementation and does not need to be mentioned in the specification.

Similar as ACCESS AND MOBILITY message in F1AP, the corresponding XnAP message can be reused.

Proposal 4: To use the existing XnAP ACCESS AND MOBILITY INDICATION message to transmit the recovery beam information for Rel-19Case 1&2&3(e.g BFR shortly after successful LTM cell switch).

Conclusion

In this contribution, we provide the following observations and proposal:

Proposal 1: For inter gNB L1 based LTM, the last serving gNB/gNB-CU is responsible for performing initial analysis.

- For failures caused by improper triggering , the source CU forwards the RLF report to the DU responsible for the failure.

- For failures caused by incorrect candidate LTM cell selection, the source CU performs MRO optimization itself or forwards the RLF report to the target gNB responsible for the failure.

Proposal 2: For inter gNB L3 based LTM, the last serving gNB/gNB-CU is responsible for performing initial analysis.

- For failures caused by improper triggering, the source CU may performs MRO optimization or forwards the RLF report to the DU responsible for the failure.

- For failures caused by incorrect candidate LTM cell selection, the source CU performs MRO optimization itself or forwards the RLF report to the target gNB responsible for the failure.

Proposal 3: RAN3 to discuss which node response the optimization of wrong beam selection during L3 based LTM due to misappropriate assistant information provided by the CU.

Proposal 4: To use the existing XnAP ACCESS AND MOBILITY INDICATION message to transmit the recovery beam information for Rel-19Case 1&2&3(e.g BFR shortly after successful LTM cell switch).

Reference

[1] RP-251869