3GPP TSG-RAN WG3 Meeting #130	R3-258775

# Dallas, US, 17th - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1662 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE Based TA Measurement ID assignment for inter-CU (SCG) LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, ZTE, LG Electronics, Huawei, Nokia, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For inter-CU (SCG) LTM, the source gNB may propose the list of IDs to the target gNB to be used when allocating UE Based TA Measurement ID. This is harmonized to the way assigning Rel-19 Set ID among gNBs. Therefore, some new IEs related to the UE Based Measurement ID assignment are needed in handover procedure and SN addition/modification procedure for inter-CU (SCG) LTM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the Proposed LTM UE Based TA Measurement ID List IE in the HANDOVER REQUEST and in the S-NODE ADDITION/MODIFICATION REQUEST message. - Introduce the UE Based TA Measurement Configuration IE in the HANDOVER REQUEST ACKNOWLEDGE and in the S-NODE ADDITION/MODIFICATION REQUEST ACKNOWLEDGE message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE Based TA Measurement function may not work well for inter-CU (SCG) LTM scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2; 8.3.1.2; 8.3.3.2; 9.1.1.1; 9.1.1.2; 9.1.2.1; 9.1.2.5; 9.2.3.243; (new) 9.2.3.xxx; 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1. Update the part related to the UE Based TA Measurement ID, add co-source. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start Change >>>>>>>>>>>>>>>>>>>>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

If the procedure concerns an LTM, parallel transactions are allowed only when using the same source UE AP ID. Possible parallel requests are identified by the target cell ID.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Target NG-RAN node UE XnAP ID IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node shall remove the existing prepared conditional HO identified by the Target NG-RAN node UE XnAP ID IE and the Target Cell Global ID IE. It is up to the implementation of the target NG-RAN node when to remove the HO information.

<skip unchanged part>

If the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the LTM information for the UE in the target NG-RAN node and include the LTM Handover Information Request Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Reference Configuration IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for generating the LTM configuration. If the Request for CSI-RS Resource Configuration for L1 Measurements IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for L1 Measurements IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for Early CSI Acquisition IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the CSI Resource Configuration IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration included in the LTM Candidate Configuration IE for the requested candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Proposed LTM No Security Change ID List IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the LTM No Security Change ID IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell, and act as specified in TS 38.300 [9].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the Target NG-RAN node UE XnAP ID IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, consider that the source NG-RAN node requests LTM preparation for an already established UE association.

If the Early Sync Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the early synchronization information for the UE in the target NG-RAN node.

If the Early Sync Information Response IE is contained in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, store it and use it as defined in TS 38.300 [9].

If the LTM CFRA Resource Information IE is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [35].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the target NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, and the Geographical Area IE contains the MDT PLMN List IE, the target NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

If the Proposed LTM UE Based TA Measurement ID List IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the LTM UE Based TA Measurement Configuration IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell, and act as specified in TS 38.300 [9].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

<skip unchanged part>

If the LTM Candidate PSCell Addition Information Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use the information to prepare for LTM candidate PSCell(s) configuration, and include the LTM Candidate PSCell Addition Information Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the CSI Resource Configuration IE is contained in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration.

If the SCG Reference Configuration Request IE set to "request" is contained in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, provide the SCG reference configuration for LTM.

If the LTM Configuration ID Mapping List IE is contained in in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider this as the mapping information for the suggested LTM candidate PSCell(s).

If the LTM Information SN Addition IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the S-NG-RAN node Addition Preparation procedure has been triggered as part of an MCG LTM.

If the Proposed LTM UE Based TA Measurement ID List IE is contained in the LTM Candidate PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use this information for preparing LTM candidate PSCells, as described in TS 37.340 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE MODIFICATION REQUEST message, it shall start the timer TXnDCprep.

<skip unchanged part>

If the LTM Candidate PSCell Information Update Request IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the included information provides the SCG LTM related information of the prepared candidate PSCells and may include the LTM Candidate PSCell Information Update Acknowledge IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, as described in TS 37.340 [8].

If the CSI Resource Configuration IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration.

If the LTM Configuration ID Mapping List IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider this as the mapping information for the prepared LTM candidate PSCell(s).

If the Proposed LTM No Security Change IDs IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use this information for preparing LTM candidate PSCells, as described in TS 37.340 [8].

If the LTM SCG Security Configuration IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use this information to apply security during SCG LTM.

If the LTM Candidate PSCell To be Cancelled List IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the request concerns the cancellation of LTM candidate PSCell(s) indicated, and may provide the updated information for SCG LTM in the LTM Candidate PSCell Information Update Acknowledge IE of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

If the LTM Information SN Modification IE set to "intra-MN-LTM" is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure has been triggered as part of an MCG LTM.

If the LTM Inter-SN Execution Notification IE set to "executed" is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the UE has been successfully accessed to another candidate SN and may stop data transmission to the UE. If the Data Forwarding and Offloading Info from source NG-RAN node IE within the PDU Session Resource Modification Info – SN terminated IE is also included for some PDU session in the PDU Session Resources To Be Modified List IE of the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node may include the Data Forwarding Info from target NG-RAN node IE within the PDU Session Resource Modification Response Info – SN terminated IE of the corresponding PDU sessions in the PDU Session Resources Admitted To Be Modified List IE of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to provide the new data forwarding address information for inter-SN SCG LTM.

If the Proposed LTM UE Based TA Measurement ID List IE is contained in the LTM Candidate PSCell Information Update Request IE included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use this information for preparing LTM candidate PSCells, as described in TS 37.340 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Handover Information Request | O |  |  |  | YES | reject |
| >Proposed LTM No Security Change ID List | O |  | LTM No Security Change ID List 9.2.3.231 |  | – |  |
| >Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | – |  |
| >Reference Configuration | O |  | OCTET STRING | Includes the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >Request for CSI-RS Resource Configuration for L1 Measurements | O |  | ENUMERATED (true, …) |  | – |  |
| >Request for CSI-RS Resource Configuration for Early CSI Acquisition | O |  | ENUMERATED (true, …) |  | – |  |
| >Proposed LTM UE Based TA Measurement ID List | O |  | LTM UE Based TA Measurement ID List 9.2.3.xxx | Indicates the LTM UE Based TA Measurement IDs to be assigned during the preparation of candidate cells. | – |  |
| Early Sync Information Request | O |  | 9.2.3.217 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Handover Information Request Acknowledge | O |  |  |  | YES | ignore |
| >SSB Information | M |  | 9.2.3.228 |  | – |  |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. | – |  |
| >LTM Candidate Configuration | M |  | OCTET STRING | Includes the ltm-CandidateConfig as defined in TS 38.331 [10]. | – |  |
| >LTM No Security Change ID | M |  | INTEGER(1..9,…) | Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10]. | – |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | – |  |
| >CSI-RS Resource Configuration for L1 Measurements | O |  | 9.2.3.224 |  | – |  |
| >CSI-RS Resource Configuration for Early CSI Acquisition | O |  | 9.2.3.224 |  | – |  |
| >LTM CFRA Resource Information | O |  | 9.2.3.232 |  | – |  |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the candidate cell. | – |  |
| Early Sync Information Response | O |  | Early Sync Information 9.2.3.218 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.1.2	Messages for Dual Connectivity Procedures

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 | This IE is ignored if the S-CPAC Request Information IE is present in the Conditional PSCell Addition Information Request IE or the LTM Candidate PSCell Addition Information Request IE is present. | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Candidate PSCell Addition Information Request | O |  |  |  | YES | reject |
| >LTM Request Indication | M |  | ENUMERATED (request, ...) |  | – |  |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the candidate SN may prepare. | – |  |
| >Suggested LTM Candidate PSCell List | M |  | LTM Candidate PSCell Request List 9.2.3.242 |  | – |  |
| >Proposed LTM No Security Change IDs | M |  | LTM No Security Change ID List 9.2.3.231 | Indicates the LTM No Security Change IDs to be assigned during the preparation of candidate PSCells. | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >SCG Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for LTM is requested. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >Proposed LTM UE Based TA Measurement ID List | O |  | LTM UE Based TA Measurement ID List 9.2.3.xxx | Indicates the LTM UE Based TA Measurement IDs to be assigned during the preparation of candidate PSCells. | – |  |
| LTM Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Candidate PSCell Information Update Request | O |  |  |  | YES | ignore |
| >Multiple S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>LTM Candidate PSCell Prepared List | O |  | 9.2.3.243 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >Proposed LTM No Security Change IDs | O |  | LTM No Security Change ID List 9.2.3.231 | Indicates the LTM No Security Change IDs to be assigned during the preparation of candidate PSCells. | – |  |
| >LTM SCG Security Configuration | O |  | 9.2.3.241 |  | – |  |
| >LTM Candidate PSCell To be Cancelled List | O |  | LTM Candidate PSCell Request List 9.2.3.242 |  | – |  |
| >Proposed LTM UE Based TA Measurement ID List | O |  | LTM UE Based TA Measurement ID List 9.2.3.xxx | Indicates the LTM UE Based TA Measurement IDs to be assigned during the preparation of candidate PSCells. | – |  |
| LTM Information SN Modification | O |  |  |  | YES | ignore |
| >LTM Reconfiguration | M |  | ENUMERATED (intra-MN-LTM, ...) |  | – |  |
| LTM Inter-SN Execution Notification | O |  | ENUMERATED (executed, …) | Indicates that inter-SN SCG LTM was executed. | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.243	LTM Candidate PSCell Prepared List

This IE provides the information related to the prepared LTM candidate PSCell(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTM Candidate PSCell Prepared List |  | 1 |  |  |
| >LTM Candidate PSCell Prepared Item |  | 1 .. < maxnoofLTMCells> |  |  |
| >>PSCell ID | M |  | NR CGI 9.2.2.7 |  |
| >>LTM No Security Change ID | M |  | INTEGER (0.. 9, …) | Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10] |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. |
| >>Early Sync Information | O |  | 9.2.3.218 |  |
| >>LTM CFRA Resource Information | O |  | 9.2.3.232 |  |
| >>SSB Information | O |  | 9.2.3.228 |  |
| >>CSI-RS Resource Configuration for Layer 1 Measurements | O |  | CSI-RS Resource Configuration 9.2.3.224 |  |
| >>CSI-RS Resource Configuration for Early CSI Acquisition | O |  | CSI-RS Resource Configuration 9.2.3.224 |  |
| >>Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate PSCell identified by the PSCell ID IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.xxx	LTM UE Based TA Measurement ID List

This IE contains the UE Based TA Measurement Configuration information for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTM UE Based TA Measurement ID Item |  | 1.. <maxnoofLTMCells> |  |  |
| >UE Based TA Measurement Configuration | M |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

===================<skip unchanged part>====================

#### -- L

===================<skip unchanged part>====================

LTMHandoverInformationRequest ::= SEQUENCE {

proposedLTM-NoSecurityChangeID-List		LTM-NoSecurityChangeID-List		OPTIONAL,				-- This IE may need to be refined.

targetNG-RANnodeUEXnAPID				NG-RANnodeUEXnAPID				OPTIONAL,

referenceConfiguration					ReferenceConfiguration			OPTIONAL,

lTMConfigurationIDMappingList			LTMConfigurationIDMappingList	OPTIONAL,

cSIResourceConfiguration				CSIResourceConfiguration		OPTIONAL,

requestForCSI-RSResourceConfigForLayer1Measurements		RequestForCSI-RSResourceConfigForLayer1Measurements		OPTIONAL,

requestForCSI-RSResourceConfigForEarlyCSIAcquisition	RequestForCSI-RSResourceConfigForEarlyCSIAcquisition		OPTIONAL,

proposedLTM-UEBasedTAMeasurementID-List	LTM-UEBasedTAMeasurementID-List	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMHandoverInformationRequest-ExtIEs} } OPTIONAL,

...

}

LTMHandoverInformationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMConfigurationIDMappingList ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMConfigurationIDMapping-Item

LTMConfigurationIDMapping-Item::= SEQUENCE{

lTMCellID			NR-CGI,

lTMConfigurationID	LTMConfigurationID,

iE-Extensions		ProtocolExtensionContainer {{ LTMConfigurationIDMapping-Item-ExtIEs}}		OPTIONAL

}

LTMConfigurationIDMapping-Item-ExtIEs 	XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMConfigurationID ::=  INTEGER (1..8)

LTM-NoSecurityChangeID-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTM-NoSecurityChangeID

LTM-NoSecurityChangeID ::= INTEGER (1..9,...)

LTMHandoverInformationRequestAcknowledge ::= SEQUENCE {

sSBInformation								SSBInformation,

tCIStateConfigurationList					OCTET STRING,

lTMCandidateConfiguration					OCTET STRING,

lTM-NoSecurityChangeID						LTM-NoSecurityChangeID,

completeCandidateConfigurationIndicator		CompleteCandidateConfigurationIndicator 	OPTIONAL,

cSI-RSResourceConfigurationForLayer1Measurements		CSI-RSResourceConfiguration		OPTIONAL,

cSI-RSResourceConfigurationForEarlyCSIAcquisition		CSI-RSResourceConfiguration		OPTIONAL,

lTMCFRAResourceInformation								LTMCFRAResourceInformation 		OPTIONAL,

ueBasedTAMeasurementConfiguration	OCTET STRING	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMHandoverInformationAcknowledge-ExtIEs} } OPTIONAL,

...

}

LTMHandoverInformationAcknowledge-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMCellSwitchInformation	::= SEQUENCE {

jointorDLTCIStateID				JointorDLTCIStateID,

uLTCIStateID					ULTCIStateID 				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { LTMCellSwitchInformation-ExtIEs } }	OPTIONAL,

...

}

LTMCellSwitchInformation-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

===================<skip unchanged part>====================

LTMPSCellInformation-AddReq ::= SEQUENCE {

lTM-RequestIndication					ENUMERATED {request, ...},

maxNrofPSCellsToPrepare					MaxNrofPSCellsToPrepare,

suggestedLTMCandidatePSCellList			LTM-PSCell-Request-List,

proposedLTM-NoSecurityChangeID-List		LTM-NoSecurityChangeID-List,

cSI-ResourceConfiguration				CSIResourceConfiguration													OPTIONAL,

sCG-ReferenceConfigRequest				ENUMERATED {request, ...} 													OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList												OPTIONAL,

proposedLTM-UEBasedTAMeasurementID-List	LTM-UEBasedTAMeasurementID-List												OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-AddReq-ExtIEs} }			OPTIONAL,

...

}

LTMPSCellInformation-AddReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-AddReqAck ::= SEQUENCE {

lTM-CandidatePSCellPreparedList		LTM-PSCell-Prepared-List												OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { LTMPSCellInformation-AddReqAck-ExtIEs} }			OPTIONAL,

...

}

LTMPSCellInformation-AddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-UpdateReq ::= SEQUENCE {

multipleSN-List							MultipleCandidateSN-List,

cSI-ResourceConfiguration				CSIResourceConfiguration											OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList										OPTIONAL,

proposedLTM-NoSecurityChangeID-List		LTM-NoSecurityChangeID-List											OPTIONAL,

lTM-SCG-SecurityConfiguration			LTM-SCG-SecurityConfiguration-List									OPTIONAL,

lTM-CandidatePSCellToBeCancelled		LTM-PSCell-Request-List												OPTIONAL,

-- reference to the LTM-PSCell-Request-List to be refined

proposedLTM-UEBasedTAMeasurementID-List	LTM-UEBasedTAMeasurementID-List										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-UpdateReq-ExtIEs} }				OPTIONAL,

...

}

LTMPSCellInformation-UpdateReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTMPSCellInformation-UpdateReqAck ::= SEQUENCE {

lTM-CandidatePSCellPreparedList			LTM-PSCell-Prepared-List											OPTIONAL,

cSI-ResourceConfiguration				CSIResourceConfiguration											OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-UpdateReqAck-ExtIEs} }			OPTIONAL,

...

}

LTMPSCellInformation-UpdateReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LTM-PSCell-Prepared-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTM-PSCell-Prepared-Item

LTM-PSCell-Prepared-Item ::= SEQUENCE {

pscell-id											NR-CGI,

lTM-NoSecurityChangeID								LTM-NoSecurityChangeID,

tCI-StatesConfigurationsList						OCTET STRING											OPTIONAL,

earlySyncInformation								EarlySyncInformation									OPTIONAL,

lTMCFRAResourceInformation							LTMCFRAResourceInformation 								OPTIONAL,

sSBInformation										SSBInformation											OPTIONAL,

cSI-RSResourceConfigurationForLayer1Measurements	CSI-RSResourceConfiguration								OPTIONAL,

cSI-RSResourceConfigurationForEarlyCSIAcquisition	CSI-RSResourceConfiguration								OPTIONAL,

complete-CandidateConfigurationIndicator			ENUMERATED {complete, ...}								OPTIONAL,

ueBasedTAMeasurementConfiguration					OCTET STRING											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTM-PSCell-Prepared-Item-ExtIEs} } 						OPTIONAL,

...

}

LTM-PSCell-Prepared-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTM-UEBasedTAMeasurementID-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTM-UEBasedTAMeasurementID-Item

LTM-UEBasedTAMeasurementID-Item ::= SEQUENCE {

ueBasedTAMeasurementConfiguration				OCTET STRING,

iE-Extensions							ProtocolExtensionContainer { { LTM-UEBasedTAMeasurementID-Item-ExtIEs} } OPTIONAL,

...

}

LTM-UEBasedTAMeasurementID-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>