3GPP TSG-RAN WG3 #130	R3-258808

Dallas, U.S.A., 17th – 21st November 2025

Agenda Item:	9.2.8

Source:	Ericsson (Moderator)

Title: 	Summary of offline discussions: Rel-19 Network Energy Saving Correction

Document for:	Discussion and Approval

# Introduction

CB: # 11_FolderName

- description of Cell A ID is absent and related O&M requirements, further discuss based on 8150, 8301 & 8504

- check 8453, refine wording of the 1st change, fix SIBxx, consider further semantics clarification if agreeable

- fix criticalities, if needed, based on 8280, 8402, 8170, 8374

- check 38.401 cleanup, based on 8171, 8302, 8502, 8614

(Ericsson - moderator)

# For Chairman

Description of Cell A ID is absent and related O&M requirements, further discuss based on 8150, 8301 & 8504

- The common understanding is that the current XnAP specification coves both cases ( with or without Cell ID presented).

- We agree no updates in st3 and st2 is needed.

Check 8453, refine wording of the 1st change, fix SIBxx, consider further semantics clarification if agreeable

Fix criticalities, if needed, based on 8280, 8402, 8170, 8374

XnAP R3-258453 is revised to R3-258809 and agreed

- These documents should be marked as “Merged” R3-258280, R3-258374, R3-258402

- R3-258170 is noted,

Agreements:

- The first two changes in R3-258453 are agreead. The third change in the semantice is not needed.

- For NES procedures, all optional IEs related to NES operation have criticality “ignore”; All mandatory IEs related to NES operation have critical “reject”, in response/ failure; and status update.

- The criticality change in 9.1.3.33 in R3-258716 should follow the above agreements on criticality.

Check 38.401 cleanup, based on 8171, 8302, 8502, 8614

R3-258302 is revised to R3-258810 and agreed

- This CR is collecting all the agreeable contents from the 38.401 CRs submitted to this meeting.

- These documents should be marked as “Merged”: R3-258171, R3-258502, R3-258614

# 3 Discussion

3.1 XnAP R3-258453

check 8453, refine wording of the 1st change, fix SIBxx, consider further semantics clarification if agreeable

- Refine wording of the 1st change:

Upon reception of the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message with the Provision Status IE set to "stopped", the NG-RAN node1 shall consider that the NG-RAN node2 has stopped the OD-SIB1 broadcasting for the previously admitted OD-SIB1 configuration for the cell identified by the NES Cell ID IE. If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message, the NG-RAN node1 shall consider that a previously admitted OD-SIB1 configuration for the cell identified by the NES Cell ID IE is no longer transmitted in the cell identified by the Cell A ID IE. The NG-RAN node2 removes the stored OD-SIB1 configuration information that is no longer valid.

What need to be revised?

The changes in the CR are agreed

- fix SIBxx to SIB26: Agreeable

- Consider further semantics clarification if agreeable: Agree not to include

3.2 When Cell A ID is absent

description of Cell A ID is absent and related O&M requirements, further discusses based on 8150, 8301 & 8504

XnAP CR: 8150, 8301

TS 38.300 draft CR:  8504

ZTE: if we don’t mention the “absence” of the IE in St3, then nothing is needed.

RAK: ok with both st3 and st2

The common understanding that the current XnAP specification coves both cases ( with or without Cell ID).

We agree no updates in st3 and st2 is needed.

3.3 Criticalities related

fix criticalities, if needed, based on 8280, 8402, 8170, 8374

The below are the proposals

#### 9.1.3.31	OD-SIB1 CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the OD-SIB1 configuration will be provided by the NG-RAN node2 as requested by the NG-RAN node1.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NES Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | ignore |
| Cell A ID | O |  | NR CGI 9.2.2.7 |  | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.32	OD-SIB1 CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the OD-SIB1 configuration cannot be provided by the NG-RAN node2 as requested by the NG-RAN node1.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| NES Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Cell A ID | O |  | NR CGI 9.2.2.7 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.33	OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted OD-SIB1 configuration provision is being stopped.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NES Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Cell A ID | O |  | NR CGI 9.2.2.7 |  | YES | ignore |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the status of the OD-SIB1 configuration transmission. | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

Agreement:

- For NES procedures, all optional IEs related to NES operation have criticality “ignore”; All mandatory IEs related to NES operation have critical “reject”, in response/ failure; and status update.

3.4  Check 38.401

- check 38.401 cleanup, based on 8171, 8302, 8502, 8614

Ask revision for R3-258302, all the CR are merged when the contents are correct.

# 4 Moderator Summary

Capture the agreements in Chapter 2.