3GPP TSG-RAN WG3 Meeting # 94	R3-163118

Reno, USA, Nov. 14th – 18th, 2016

Agenda item:		10.3.3

Source:	LG Electronics Inc.

Title:	Text Proposal on procedures for Xn interface

Document for:		Discussion

1. Text Proposal

----------------Start of the First Change---------------

7.3.1.2	Xn Interface Functions

The Xn-C interface supports the following functions:

-	Xn interface management and error handling procedures to manage the Xn-C interface;

-	Error indication;

-	Setting up the Xn;

-	Resetting the Xn;

-	Updating the Xn configuration data;

-	Xn removal.

-	UE connected mode mobility management: procedures to manage the UE mobility for connected mode between nodes in the New RAN;

-	Handover preparation;

-	Handover cancellation.

-	Dual connectivity: procedures to enable usage of additional resources in a secondary node in the New RAN.

The Xn-U interface supports the following functions:

-	Data forwarding

-	Flow control

7.3.1.3	Xn Interface Procedures

To support the functions listed in Section 7.3.1.2, the Xn interface should support the following procedures. The procedures are classified in different categories.

Xn Interface Management and Error Handling Procedures:

-	Xn Setup: to establish an Xn interface between the New RAN nodes

-	Xn Reset: to reset the Xn interface

-	Error Indication: to report detected errors in one incoming message

-	New RAN Configuration Update: to update the configuration for the New RAN nodes over Xn interface

-	Xn Removal: to remove the signaling connection between two New RAN nodes in a controlled manner

UE connected mode Mobility Management Procedures:

-	Handover Preparation: to establish necessary resources in a New RAN node for an incoming handover

-	Handover Cancellation: to cancel an ongoing handover preparation or an already prepared Xn handover

-	UE Context Release: to indicate to the source new RAN that radio and control plane resources for the associated UE context are allowed to be released

Dual Connectivity Procedures supporting option 4/4a and 7/7a (whether to be applied to DC between two gNBs is FFS):

-	Secondary Node Addition

-	Secondary Node Modification (Master node initiated)

-	Secondary Node Modification (Secondary node initiated)

-	Secondary Node Release (Master node initiated)

-	Secondary Node Release (Secondary node initiated)

Editor’s Note:	 Work in other RAN WGs on inter-RAT DC for New RAN, on 5G intra-system mobility and NR specifics needs to be revisited in order to understand whether DC signalling schemes as specified for E-UTRA can be re-used for 5G. It is however expected, that role definitions from E-UTRAN (master node, secondary node) can still be applied for options 4/4a and 7/7a.

----------------End of the First Change---------------