3GPP TSG-RAN3 Meeting # 97bis	R3-174172

Prague, Czech, Oct 9-13, 2017

Agenda item:		10.10.2.1

Source:	Huawei

Title:					pCR on gNB CU&DU update procedure to 38.401

Document for:		Approval

1.	Introduction

This pCR tries to capture the following agreements to 38.401:

- To introduce gNB-DU update function

- To introduce gNB-CU update function

# Text proposal for TS 38.401 V0.3.0

Beginning of Text Proposal to TS 38.401

10.3.2.2	F1-C functions

10.3.2.2.1	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an  error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and gNB-CU.

The F1 setup  function allows to exchange  application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. The F1 setup is initiated by the gNB-DU.

The gNB-CU Configuration Update and gNB-DU update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, and activate or deactivate the cells.

Editor’s note: The space of NR Cell ID, and how/whether to map them in the gNB-CU are FFS.

End of Text Proposal to TS 38.401