# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document establishes the minimum RF requirements for NR User Equipment (UE) Interworking operation with other radios. This includes but is not limited to additional requirements for carrier aggregation or NR dual connectivity between Range 1 and Range 2 and additional requirements due to NR non-standalone (NSA) operation mode with E-UTRA.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[4]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

[5]	3GPP TS 38.521-3: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[6]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[7]	3GPP TS 36.211: "E-UTRA; Physical channels and modulation"

[8]	3GPP TS 36.331: " Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification"

[9]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[10]	3GPP TS 38.213: "NR; Physical layer procedures for control"

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities"

[12]	3GPP TS 38.133: "NR; Requirements for support of radio resource management"

[13]	3GPP TS 38.211: "NR; Physical channels and modulation".

[14]	3GPP TS 38.214: "NR; Physical layer procedures for data"

[15]	3GPP TS 38.133: "NR; Requirements for support of radio resource management"

[16]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for CA or DC operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for CA or DC operation, for serving cell c

BWE-UTRA_Channel	Channel bandwidth of E-UTRA carrier

BWE-UTRA_Channel_CA	Channel bandwidth of E-UTRA sub-block which is composed of intra-band contiguous CA E-UTRA carriers

BWNR_Channel	Channel bandwidth of NR carrier

BWNR_Channel_CA	Channel bandwidth of NR sub-block which is composed of intra-band contiguous CA NR carriers

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

EN-DCACLR	The ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW

E-UTRAACLR	E-UTRA ACLR

FC	RF reference frequency for the carrier center on the channel raster

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FOOB	The boundary between the NR out of band emission and spurious emission domains

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resource blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=1jNRBj*2μj  for carrier 1 to j, where μ is defined in TS 38.211 [13]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj  for carrier j, where μ is defined in TS 38.211 [13]

PCMAX	The configured maximum UE output power

RBstart	Indicates the lowest RB index of transmitted resource blocks

Wgap	The sub-block gap between the two sub-blocks

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BCS	Bandwidth Combination Set

CA	Carrier Aggregation

CC	Component Carrier

DC	Dual Connectivity

EIRP	Equivalent Isotropically Radiated Power

EN-DC	E-UTRA/NR DC

EVM	Error Vector Magnitude

FDM	Frequency Division Multiplexing

FR	Frequency Range

ENBW	The aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block

ITS	Intelligent Transportation System

ITU-R	Radiocommunication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

MCG	Master Cell Group

NR	New Radio

NS	Network Signalling

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

OOB	Out-of-band

OOBE	Out-of-band emission

OTA	Over The Air

PRB	Physical Resource Block

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

Rx	Receiver

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SL	Sidelink

SUL	Supplementary uplink

TDM	Time Division Multiplex

Tx	Transmitter

UE	User Equipment

UL MIMO	Up Link Multiple Antenna transmission

ULSUP	Uplink sharing from UE perspective

# 4	General

## 4.1	Relationship between minimum requirements and test requirements

The present document is interwork specification for NR UE, covering RF characteristics and minimum performance requirements. Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification 3GPP TS 38.521-3 [5].

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification TS 38.521-3 [5] defines test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the shared risk principle.

The shared risk principle is defined in Recommendation ITU-R M.1545 [6].

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

d)	Terminal that supports EN-DC or NE-DC configuration shall meet E-UTRA requirements as specified in TS 36.101 [4] and NR requirements as in TS 38.101-1 [2] and TS 38.101-2 [3] unless otherwise specified in this specification

e)	All the requirements for intra-band contiguous and non-contiguous EN-DC or NE-DC apply under the assumption of the same uplink-downlink and special subframe configurations in the E-UTRA and slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the NR for the EN-DC or NE-DC, a time offset between the two RATs configurations may be required.

f)	For EN-DC or NE-DC  combinations with CA configurations for E-UTRA and/or NR, all the requirements for E-UTRA and/or NR all the requirements for E-UTRA and/or NR intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the PSCell and SCells for NR and the same uplink-downlink and special subframe configurations in Pcell and SCells for E-UTRA.

A terminal which supports an EN-DC or NE-DC configuration shall support:

If any subsets of the EN-DC or NE-DC configuration do not specify its own bandwidth combination sets in 5.3B, then the terminal shall support the same E-UTRA bandwidth combination sets it signals the support for in E-UTRA CA configuration part of E-UTRA – NR DC and shall support the same NR bandwidth combination sets it signals the support for in NR CA configuration part of E-UTRA – NR DC.

Else if one of the subsets of the EN-DC or NE-DC configuration specify its own bandwidth combination sets in 5.3B, then the terminal shall support a product set of channel bandwidth for each band specified by E-UTRA bandwidth combination sets, NR bandwidth combination sets, and EN-DC or NE-DC bandwidth combination sets it singnals the support.A terminal which supports an inter-band EN-DC or NE-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC or NE-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.

A terminal which supports NE-DC configurations shall meet the minimum requirements for corresponding EN-DC configuration, unless otherwise specified.

For CA or DC configurations, which include FR2 intra-band CA combinations with multiple FR2 sub-blocks, where at least one of the sub-blocks is a contiguous CA combination :

-	if the field partialFR2-FallbackRX-Req is not present, the UE shall meet all applicable UE RF requirements for the highest order CA configuration and all associated fallback CA configurations;

-	if the field partialFR2-FallbackRX-Req is present, for each FR2 intra-band CA configuration with multiple sub-blocks that the UE indicates support for explicitly in UE capability signalling: the in-gap UE RF requirements in clauses 7.5A, 7.5B, 7.6A, 7.6B apply as the equivalent requirements for the associated fallback FR2 intra-band CA configurations with the same number of sub-blocks, where at least one of the sub-blocks consists of a contiguous CA configuration. The UE shall meet all applicable UE RF requirements for fallback CA configurations with a lesser number of sub-blocks;

-	regardless of the field partialFR2-FallbackRX-Req, the UE shall meet all DL out-of-gap requirements for all lower order fallback CA configurations.

Terminal that supports inter-band NR-DC between FR1 and FR2 configuration shall meet the requirements for corresponding CA configuration (suffix A), unless otherwise specified.

## 4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) between FR1 and FR2 |
| B | Dual-Connectivity (DC) with and without SUL including UL sharing from UE perspective, inter-band NR DC between FR1 and FR2 |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specifications are identified as described in Table 5.1-1. Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered, unless otherwise stated.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

The present specification covers band combinations including

-	at least one FR1 operating band and one FR2 operating band for carrier aggregation and dual connectivity operations;

-	at least one E-UTRA operating band for dual connectivity operations.

## 5.2	Operating bands

NR is designed to operate in FR1 operating bands defined in TS 38.101-1 [2] and FR2 operating bands defined in TS 38.101-2 [3]. E-UTRA is designed to operate in operating bands defined in TS 36.101 [4].

## 5.2A	Operating bands for CA

### 5.2A.1	Inter-band CA between FR1 and FR2

NR carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1 and Table 5.2A.1-2. The band combinations include at least one FR1 operating band and one FR2 operating band.

Operating bands for CA including Band n90 are defined by the corresponding operating bands for CA including Band n41 with Band n90 replacing Band n41. For brevity the said operating bands for CA including Band n90 are not listed in the tables below but are covered by this specification.

If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pairin the higher order band combination.

Table 5.2A.1-1: Band combinations for inter-band CA between FR1 and FR2 (two bands)

| NR CA Band |  | NR Band |
| --- | --- | --- |
| CA_n1-n2571 |  | n1, n257 |
| CA_n1-n2581 |  | n1, n258 |
| CA_n2-n2601 |  | n2, n260 |
| CA_n2-n2611 |  | n2, n261 |
| CA_n3-n2571 |  | n3, n257 |
| CA_n3-n2581 |  | n3, n258 |
| CA_n5-n2581 |  | n5, n258 |
| CA_n5-n2601 |  | n5, n260 |
| CA_n5-n2611 |  | n5, n261 |
| CA_n7-n2581 |  | n7, n258 |
| CA_n7-n2571 |  | n7, n257 |
| CA_n8-n2571 |  | n8, n257 |
| CA_n7-n2601 |  | n7, n260 |
| CA_n8-n2581 |  | n8, n258 |
| CA_n12-n2601 |  | n12, n260 |
| CA_n14-n2601 |  | n14, n260 |
| CA_n30-n2601 |  | n30, n260 |
| CA_n25-n2571 |  | n25, n257 |
| CA_n25-n2581 |  | n25, n258 |
| CA_n25-n2601 |  | n25, n260 |
| CA_n25-n2611 |  | n25, n261 |
| CA_n26-n2581 |  | n26, n258 |
| CA_n28-n2571 |  | n28, n257 |
| CA_n28-n2581 |  | n28, n258 |
| CA_n34-n2581 |  | n34, n258 |
| CA_n38-n2571 |  | n38, n257 |
| CA_n38-n2581 |  | n38, n258 |
| CA_n39-n2571 |  | n39, n257 |
| CA_n39-n2581 |  | n39, n258 |
| CA_n40-n2571 |  | n40, n257 |
| CA_n40-n2581 |  | n40, n258 |
| CA_n41-n2571 |  | n41, n257 |
| CA_n41-n2581 |  | n41, n258 |
| CA_n41-n2601 |  | n41, n260 |
| CA_n41-n2611 |  | n41, n261 |
| CA_n48-n2601 |  | n48, n260 |
| CA_n48-n2611 |  | n48, n261 |
| CA_n48-n2631 |  | n48, n263 |
| CA_n66-n2571 |  | n66, n257 |
| CA_n66-n2581 |  | n66, n258 |
| CA_n66-n260 |  | n66, n260 |
| CA_n66-n261 |  | n66, n261 |
| CA_n71-n2571 |  | n71, n257 |
| CA_n71-n2601 |  | n71, n260 |
| CA_n71-n2611 |  | n71, n261 |
| CA_n77-n2571 |  | n77, n257 |
| CA_n77-n2581 |  | n77, n258 |
| CA_n77-n2571 |  | n77, n257 |
| CA_n77-n2591 |  | n77, n259 |
| CA_n77-n2601 |  | n77, n260 |
| CA_n77-n2611 |  | n77, n261 |
| CA_n78-n2571 |  | n78, n257 |
| CA_n78-n2581 |  | n78, n258 |
| CA_n78-n2591 |  | n78, n259 |
| CA_n79-n2571 |  | n79, n257 |
| CA_n79-n2581 |  | n79, n258 |
| CA_n79-n2591 |  | n79, n2591 |
| CA_n105-n2571 | n105, n257 |  |
| CA_n105-n2581 | n105, n258 |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |

Table 5.2A.1-2: Band combinations for inter-band CA between FR1 and FR2 (three bands)

| NR CA Band | NR Band |
| --- | --- |
| CA_n1-n3-n257 | n1, n3, n257 |
| CA_n1-n3-n258 | n1, n3, n258 |
| CA_n1-n8-n257 | n1, n8, n257 |
| CA_n1-n28-n2571 | n1, n28, n257 |
| CA_n1-n28-n258 | n1, n28, n258 |
| CA_n1-n41-n2571 | n1, n41, n257 |
| CA_n1-n77-n2571 | n1, n77, n257 |
| CA_n1-n78-n25711 | n1, n78, n257 |
| CA_n1-n79-n2571 | n1, n79, n257 |
| CA_n2-n5-n260 | n2, n5, n260 |
| CA_n2-n5-n261 | n2, n5, n261 |
| CA_n2-n66-n260 | n2, n66, n260 |
| CA_n2-n66-n261 | n2, n66, n261 |
| CA_n2-n77-n260 | n2, n77, n260 |
| CA_n2-n77-n261 | n2, n77, n261 |
| CA_n3-n8-n257 | n3, n8, n257 |
| CA_n3-n28-n2571 | n3, n28, n257 |
| CA_n3-n28-n258 | n3, n28, n258 |
| CA_n3-n41-n257 | n3, n41, n257 |
| CA_n3-n77-n2571 | n3, n77, n257 |
| CA_n3-n78-n2571 | n3, n78, n257 |
| CA_n3-n79-n2571 | n3, n79, n257 |
| CA_n5-n66-n260 | n5, n66, n260 |
| CA_n5-n66-n261 | n5, n66, n261 |
| CA_n5-n77-n260 | n5, n77, n260 |
| CA_n5-n77-n261 | n5, n77, n261 |
| CA_n7-n25-n257 | n7, n25, n257 |
| CA_n7-n25-n260 | n7, n25, n260 |
| CA_n7-n66-n257 | n7, n66, n257 |
| CA_n7-n71-n257 | n7, n71, n257 |
| CA_n7-n78-n258 | n7, n78, n258 |
| CA_n8-n77-n257 | n8, n77, n257 |
| CA_n8-n78-n2571 | n8, n78, n257 |
| CA_n28-n41-n257 | n28, n41, n257 |
| CA_n28-n77-n2571 | n28, n77, n257 |
| CA_n28-n78-n2571 | n28, n78, n257 |
| CA_n28-n79-n2571 | n28, n79, n257 |
| CA_n39-n40-n258 | n39, n40, n258 |
| CA_n39-n41-n258 | n39, n41, n258 |
| CA_n40-n41-n258 | n40, n41, n258 |
| CA_n41-n66-n260 | n41, n66, n260 |
| CA_n41-n77-n257 | n41, n77, n257 |
| CA_n41-n78-n257 | n41, n78, n257 |
| CA_n41-n79-n257 | n41, n79, n257 |
| CA_n41-n79-n258 | n41, n79, n258 |
| CA_n66-n77-n260 | n66, n77, n260 |
| CA_n66-n77-n261 | n66, n77, n261 |
| CA_n77-n79-n257 | n77, n79, n257 |
| CA_n77-n79-n258 | n77, n79, n258 |
| CA_n77-n79-n259 | n77, n79, n259 |
| CA_n77-n257-n2591 | n77, n257, n259 |
| CA_n78-n79-n257 | n78, n79, n257 |
| CA_n78-n79-n259 | n78, n79, n259 |
| CA_n78-n257-n2591 | n78, n257, n259 |
| CA_n79-n257-n2591 | n79, n257, n259 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |

Table 5.2A.1-3: Band combinations for inter-band CA between FR1 and FR2 (four bands)

| NR CA Band | NR Band |
| --- | --- |
| CA_n1-n3-n8-n257 | n1, n3, n8, n257 |
| CA_n1-n3-n28-n257 | n1, n3, n28, n257 |
| CA_n1-n3-n41-n257 | n1, n3, n41, n257 |
| CA_n1-n3-n77-n257 | n1, n3, n77, n257 |
| CA_n1-n3-n79-n257 | n1, n3, n79, n257 |
| CA_n1-n8-n77-n257 | n1, n8, n77, n257 |
| CA_n1-n8-n78-n2571 | n1, n8, n78, n257 |
| CA_n1-n28-n41-n257 | n1, n28, n41, n257 |
| CA_n1-n28-n77-n257 | n1, n28, n77, n257 |
| CA_n1-n28-n79-n257 | n1, n28, n79, n257 |
| CA_n1-n41-n77-n257 | n1, n41, n77, n257 |
| CA_n1-n41-n79-n257 | n1, n41, n79, n257 |
| CA_n1-n77-n79-n257 | n1, n77, n79, n257 |
| CA_n1-n78-n79-n257 | n1, n78, n79, n257 |
| CA_n2-n5-n48-n260 | n2, n5, n48, n260 |
| CA_n2-n5-n48-n261 | n2, n5, n48, n261 |
| CA_n2-n5-n66-n260 | n2, n5, n66, n260 |
| CA_n2-n5-n66-n261 | n2, n5, n66, n261 |
| CA_n2-n5-n77-n260 | n2, n5, n77, n260 |
| CA_n2-n5-n77-n261 | n2, n5, n77, n261 |
| CA_n2-n48-n66-n260 | n2, n48, n66, n260 |
| CA_n2-n48-n66-n261 | n2, n48, n66, n261 |
| CA_n2-n66-n77-n260 | n2, n66, n77, n260 |
| CA_n2-n66-n77-n261 | n2, n66, n77, n261 |
| CA_n3-n7-n78-n258 | n3, n7, n78, n258 |
| CA_n3-n8-n77-n257 | n3, n8, n77, n257 |
| CA_n3-n28-n41-n257 | n3, n28, n41, n257 |
| CA_n3-n28-n77-n2571 | n3, n28, n77, n257 |
| CA_n3-n28-n78-n2571 | n3, n28, n78, n257 |
| CA_n3-n41-n77-n257 | n3, n41, n77, n257 |
| CA_n3-n41-n79-n257 | n3, n41, n79, n257 |
| CA_n3-n77-n79-n257 | n3, n77, n79, n257 |
| CA_n3-n28-n79-n257 | n3, n28, n79, n257 |
| CA_n28-n41-n77-n257 | n28, n41, n77, n257 |
| CA_n28-n41-n79-n257 | n28, n41, n79, n257 |
| CA_n5-n48-n66-n260 | n5, n48, n66, n260 |
| CA_n5-n48-n66-n261 | n5, n48, n66, n261 |
| CA_n5-n66-n77-n260 | n5, n66, n77, n260 |
| CA_n5-n66-n77-n261 | n5, n66, n77, n261 |
| CA_n28-n77-n79-n257 | n28, n77, n79, n257 |
| CA_n28-n78-n79-n257 | n28, n78, n79, n257 |
| CA_n41-n77-n79-n257 | n41, n77, n79, n257 |
| CA_n77-n79-n257-n259 | n77, n79, n257, n259 |
| CA_n78-n79-n257-n259 | n78, n79, n257, n259 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |

Table 5.2A.1-4: Band combinations for inter-band CA between FR1 and FR2 (five bands)

| NR CA Band | NR Band |
| --- | --- |
| CA_n1-n3-n8-n77-n257 | n1, n3, n8, n77, n257 |
| CA_n1-n3-n28-n41-n257 | n1, n3, n28, n41, n257 |
| CA_n1-n3-n28-n77-n257 | n1, n3, n28, n77, n257 |
| CA_n1-n3-n28-n79-n257 | n1, n3, n28, n79, n257 |
| CA_n1-n3-n41-n77-n257 | n1, n3, n41, n77, n257 |
| CA_n1-n3-n41-n79-n257 | n1, n3, n41, n79, n257 |
| CA_n1-n3-n77-n79-n257 | n1, n3, n77, n79, n257 |
| CA_n1-n28-n41-n77-n257 | n1, n28, n41, n77, n257 |
| CA_n1-n28-n41-n79-n257 | n1, n28, n41, n79, n257 |
| CA_n1-n28-n77-n79-n257 | n1, n28, n77, n79, n257 |
| CA_n1-n41-n77-n79-n257 | n1, n41, n77, n79, n257 |
| CA_n3-n28-n41-n77-n257 | n3, n28, n41, n77, n257 |
| CA_n3-n28-n41-n79-n257 | n3, n28, n41, n79, n257 |
| CA_n3-n41-n77-n79-n257 | n3, n41, n77, n79, n257 |
| CA_n28-n41-n77-n79-n257 | n28, n41, n77, n79, n257 |

## 5.2B	Operating bands for DC

### 5.2B.1	General

The operating bands are specified in clause 5.5B for operation with EN-DC, NGEN-DC, NE-DC or NR-DC configured.

### 5.2B.2	Void

### 5.2B.3	Void

### 5.2B.4	Void

### 5.2B.5	Void

### 5.2B.6	Void

### 5.2B.7	Void

## 5.2E	Operating bands for V2X

### 5.2E.1	Intra-band V2X bands

NR V2X operation is designed to operate with E-UTRA sidelink in TDM mode on the operating bands combinations listed in Table 5.2E.1-1.

Table 5.2E.1-1: Intra-band V2X operating bands

| E-UTRA V2X-NR V2X Band Combination | E-UTRA or NR Band | Interface |
| --- | --- | --- |
| V2X_47_n471 | 47 | PC5 |
|  | n47 | PC5 |
| NOTE 1: Only single switched SL is supported. |  |  |

### 5.2E.2	Inter-band V2X bands

NR V2X operation is designed to operate concurrent with E-UTRA uplink/downlink on the operating bands combinations listed in Table 5.2E.2-1.

Table 5.2E.2-1: Inter-band con-current V2X operating bands

| E-UTRA-NR V2X Band Combination | E-UTRA or NR Band | Interface |
| --- | --- | --- |
| V2X_1_n47 | 1 | Uu |
|  | n47 | PC5 |
| V2X_n1_47 | n1 | Uu |
|  | 47 | PC5 |
| V2X_8_n47 | 8 | Uu |
|  | n47 | PC5 |
| V2X_5_n47 | 5 | Uu |
|  | n47 | PC5 |
| V2X_n5_47 | n5 | Uu |
|  | 47 | PC5 |
| V2X_n8_47 | n8 | Uu |
|  | 47 | PC5 |
| V2X_3_n47 | 3 | Uu |
|  | n47 | PC5 |
| V2X_20_n38 | 20 | Uu |
| V2X_n3_47 | n3 | Uu |
|  | 47 | PC5 |
| V2X_n34_47 | n34 | Uu |
|  | 47 | PC5 |
| V2X_34_n47 | 34 | Uu |
|  | n47 | PC5 |
|  | n38 | PC5 |
| V2X_n39_47 | 47 | PC5 |
|  | n39 | Uu |
| V2X_39_n47 | 39 | Uu |
|  | n47 | PC5 |
| V2X_n40_47 | 47 | PC5 |
|  | n40 | Uu |
| V2X_40_n47 | 40 | Uu |
|  | n47 | PC5 |
| V2X_n71_47 | 47 | PC5 |
|  | n71 | Uu |
| V2X_41_n47 | 41 | Uu |
|  | n47 | PC5 |
| V2X_n71_47 | 47 | PC5 |
|  | n71 | Uu |
| V2X_n71-(n)471 | 47 | PC5 |
|  | n47 | PC5 |
|  | n71 | Uu |
| V2X_n78_47 | 47 | PC5 |
|  | n78 | Uu |
| V2X_n79_47 | 47 | PC5 |
|  | n79 | Uu |
| NOTE 1: Only single switched SL in ITS band is supported. |  |  |

## 5.3	UE Channel bandwidth

## 5.3A	UE Channel bandwidth for CA

### 5.3A.1	Inter-band CA between FR1 and FR2

For inter-band NR CA between FR1 and FR2, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class as specified in clause 5.3A.5 of TS 38.101-1 [2] and clause 5.3A.4 of TS 38.101-2 [3] independently.

## 5.3B	UE Channel bandwidth for DC

### 5.3B.0	General

For intra-band contiguous EN-DC, the aggregated channel bandwidth is sum of the individual NR and E-UTRA channel bandwidths assuming nominal EN-DC channel with 0 kHz offset spacing as specified in clause 5.4.

ENBW = BWNR_Channel + BWE-UTRA_Channel

In the case where the NR sub-block and/or the E-UTRA sub-block itself is composed of intra-band contiguous CA carriers, the EN-DC aggregated channel bandwidth is the sum of the aggregated channel bandwidths of the NR and E-UTRA sub-blocks assuming nominal EN-DC channel spacing between the NR sub-block and E-UTRA sub-block.

ENBW = BWNR_Channel_CA + BWE-UTRA_Channel_CA

Intra-band contiguous EN-DC configurations are defined using intra-band contiguous EN-DC bandwidth class notation DC_(n)Xyz where the first EN-DC bandwidth class letter y indicates the number of contiguous E-UTRA carriers and the second EN-DC bandwidth class letter z indicates the number of contiguous NR carriers for the EN-DC combination of E-UTRA Band X and NR Band nX. Applicable contiguous intra-band EN-DC bandwidth classes are listed in Table 5.3B.0-1.

Table 5.3B.0-1: Intra-band contiguous EN-DC bandwidth classes

| Intra-band contiguous EN-DC bandwidth class | Number of contiguous CC |  |
| --- | --- | --- |
|  | E-UTRA | NR |
| AA | 1 | 1 |
| AB | 1 | 2 |
| CA | 2 | 1 |
| DA | 3 | 1 |

The UE channel bandwidths for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said UE channel bandwidths for band combinations with Band n90 are not listed in the tables below but are covered by this specification.

### 5.3B.1	Intra-band EN-DC in FR1

#### 5.3B.1.1	General

The requirements for intra-band EN-DC in this specification are defined for EN-DC configurations with associated bandwidth combination sets.

For each EN-DC configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

#### 5.3B.1.2	BCS for Intra-band contiguous EN-DC

For intra-band contiguous EN-DC, an EN-DC configuration is consisting of an E-UTRA band and a corresponding NR band having the same frequency range which supports an intra-band contiguous EN-DC bandwidth class.

Bandwidth combination sets for intra-band contiguous EN-DC are specified in Table 5.3B.1.2-1. The EN-DC configurations and bandwidth combination sets in Table 5.3B.1.2-1 also apply to higher order EN-DC combinations that include inter-band and intra-band EN-DC on the downlink and inter-band EN-DC on the uplink. If no BCS is reported in the UE capabilities for an intra-band combination the default is that the UE supports BCS0.

Table 5.3B.1.2-1: EN-DC configurations and bandwidth combination sets defined for intra-band contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)3AA | DC_(n)3AA4 | 5, 10, 15, 20 | 5,10,15,20, 25, 30 |  | 50 | 0 |
|  |  |  | 5,10,15,20, 25, 30 | 5, 10, 15, 20 |  |  |
| DC_(n)5AA | DC_(n)5AA4 | 5, 10 | 5, 10, 15, 20 |  | 25 | 0 |
|  |  |  | 5, 10, 15, 20 | 5, 10 |  |  |
| DC_(n)7AA | DC_(n)7AA4 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
|  |  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |
| DC_(n)12AA | DC_(n)12AA4 | 5, 10 | 5, 10 |  | 15 | 0 |
|  |  |  | 5, 10 | 5, 10 |  |  |
| DC_(n)38AA | DC_(n)38AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)41AA | DC_(n)41AA | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 20, 30, 40, 50, 60, 80,100 | 10 |  |  |
| DC_(n)41AB | DC_(n)41AA | 10 | 20+20 |  | 70 | 0 |
|  |  |  | 20+20 | 10 |  |  |
|  |  | 20 | 10+20 |  |  |  |
|  |  |  | 10+20 | 20 |  |  |
|  |  | 20 | 20+30 |  |  |  |
|  |  |  | 20+30 | 20 |  |  |
| DC_(n)41CA | DC_(n)41AA | 20+20 | 40, 60, 80,100 |  | 140 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 40, 50, 60, 80,100 |  | 140 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 140 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20+20 |  |  |
|  |  | 10+20 | 10, 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 10+20 |  |  |
| DC_(n)41DA | DC_(n)41AA | 20+20+20 | 40, 60, 80,100 |  | 160 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 40, 50, 60, 80,100 |  | 160 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 30, 40, 50, 60, 80,100 |  | 160 | 2 |
|  |  |  | 30, 40, 50, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+15 | 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 30, 40, 50, 60, 80,100 | 20+20+15 |  |  |
| DC_(n)48AA | DC_(n)48AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)48CA | DC_(n)48AA4 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)48DA | DC_(n)48AA4 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)66AA | DC_(n)66AA4 | 5, 10, 15, 20 | 5,10,15,20, 25, 30, 40 |  | 60 | 0 |
|  |  |  | 5,10,15,20, 25, 30, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)71AA | DC_(n)71AA3 | 15 | 5 |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |
|  |  | 5 | 5, 10, 15 |  |  |  |
|  |  |  | 5 | 15 |  |  |
|  |  |  | 5, 10 | 10 |  |  |
|  |  |  | 5, 10, 15 | 5 |  |  |
|  |  | 5 | 5,10,15,20 |  | 253 | 1 |
|  |  | 10 | 5,10,15 |  |  |  |
|  |  | 15 | 5,10 |  |  |  |
|  |  |  | 5,10,15,20 | 5 |  |  |
|  |  |  | 5,10,15 | 10 |  |  |
|  |  |  | 5,10 | 15 |  |  |
|  | - | 5 | 5, 10, 15, 20 |  | 356 | 2 |
|  |  | 10 | 10, 15, 20 |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |
|  |  |  | 5, 10, 15, 20 | 5 |  |  |
|  |  |  | 10, 15, 20 | 10 |  |  |
|  |  |  | 15, 20 | 15 |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: For maximum DL aggregated bandwidth of 25 MHz the asymmetric UL and DL channel bandwidth combination of Table 5.3.6-1 in TS 38.101-1 [2] is used with a maximum UL contiguous aggregated bandwidth of 20 MHz. Furthermore, a restriction is imposed on bandwidth combinations so that only a subset of BCS1 is allowed to be used on the uplink, and this subset is equivalent to BCS0. NOTE4: Only single switched UL is supported. NOTE5: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 6: Bandwidth Combination Set 2 only applies to intra-band EN-DC with uplink in n71 but not in Band 71, and paired with another E-UTRA band. |  |  |  |  |  |  |

#### 5.3B.1.3	BCS for Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC, an EN-DC configuration is consisting of an E-UTRA band and a corresponding NR band having the same frequency range which supports E-UTRA and NR carriers, where E-UTRA configuration is indicated by using E-UTRA CA bandwidth class as defined in TS 36.101 [4] and NR configuration is indicated by using NR CA bandwidth class as defined in TS 38.101-1 [2].

Requirements for intra-band non-contiguous EN-DC are defined for the EN-DC configurations and bandwidth combination sets specified in Table 5.3B.1.3-1. The EN-DC configurations and bandwidth combination sets in Table 5.3B.1.3-1 also apply to higher order EN-DC combinations that include inter-band and intra-band EN-DC on the downlink and inter-band EN-DC on the uplink.  If no BCS is reported in the UE capabilities for an intra-band combination the default is that the UE supports BCS0.

Table 5.3B.1.3-1: EN-DC configurations and bandwidth combination sets defined for intra-band non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_1A_n1A | DC_1A_n1A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_2A_n2A | DC_2A_n2A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_3A_n3A | DC_3A_n3A |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30 |  |  |  |
| DC_5A_n5A | DC_5A_n5A2 | 5, 10 | 5, 10, 15 |  | 20 | 0 |
| DC_7A_n7A3 | DC_7A_n7A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_40A_n40A | DC_40A_n40A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20 |  |  |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 20, 30, 40, 50, 60, 80,100 | 10 |  |  |
| DC_41A_n41A | DC_41A_n41A | 20 | 40, 60, 80,100 |  | 120 | 0 |
| DC_41C_n41A | DC_41A_n41A | 20+20 | 40, 60, 80,100 |  | 140 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 40, 50, 60, 80,100 |  | 140 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20 |  |  |
| DC_41D_n41A | DC_41A_n41A | 20+20+20 | 40, 60, 80,100 |  | 160 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 40, 50, 60, 80,100 |  | 160 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20+20 |  |  |
| DC_48A_n48A | DC_48A_n48A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_48A-48A_n48A | DC_48A_n48A2 | CA_48A-48A _BCS0 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | CA_48A-48A _BCS0 |  |  |
| DC_48C_n48A | DC_48A_n48A2 | CA_48C _BCS0 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | CA_48C _BCS0 |  |  |
| DC_48D_n48A | DC_48A_n48A2 | CA_48D _BCS0 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | CA_48D _BCS0 |  |  |
| DC_66A_n66A | DC_66A_n66A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30, 40 |  | 60 | 1 |
|  |  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20 |  |  |
| DC_66B_n66A | DC_66A_n66A2 | CA_66B_BCS0 | 5, 10, 15, 20, 40 |  | 50 | 0 |
|  |  | CA_66B_BCS0 | 5, 10, 15, 20, 25, 30, 40 |  | 60 | 1 |
| DC_66A-66A_n66A | DC_66A_n66A2 | CA_66A-66A _BCS0 | 5, 10, 15, 20, 40 |  | 70 | 0 |
|  |  | CA_66A-66A _BCS0 | 5, 10, 15, 20, 25, 30, 40 |  | 80 | 1 |
|  |  |  | 5, 10, 15, 20, 25, 30, 40 | CA_66A-66A _BCS0 |  |  |
| DC_71A_n71A | DC_71A_n71A2 | 15 | 5 |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |
|  |  | 5 | 5, 10, 15 |  |  |  |
|  |  |  | 5 | 15 |  |  |
|  |  |  | 5, 10 | 10 |  |  |
|  |  |  | 5, 10, 15 | 5 |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 30 | 1 |
|  |  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |  |  |  |

Table 5.3B.1.3-2: EN-DC configurations and bandwidth combination sets defined for mixed intra-band contiguous and non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)48CA5 | DC_48A_n48A2 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)48DA5 | DC_48A_n48A2 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_48A-(n)48AA5 | DC_(n)48AA2 DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 5: The UE supporting these configurations indicates ‘both’ by IE intraBandENDC-Support. |  |  |  |  |  |  |

### 5.3B.1a	Intra-band NE-DC in FR1

#### 5.3B.1a.1	General

The requirements for intra-band NE-DC in this specification are defined for NE-DC configurations with associated bandwidth combination sets.

For each NE-DC configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

#### 5.3B.1a.2	BCS for Intra-band contiguous NE-DC

For intra-band contiguous NE-DC, an NE-DC configuration is a single operating band supporting an intra-band contiguous NE-DC bandwidth class.

Bandwidth combination sets for intra-band contiguous NE-DC are specified in Table 5.3B.1a.2-1. The NE-DC configurations and bandwidth combination sets in Table 5.3B.1a.2-1 also apply to higher order NE-DC combinations that include inter-band and intra-band EN-DC on the downlink and inter-band NE-DC on the uplink. If no BCS is reported in the UE capabilities for an intra-band combination the default is that the UE supports BCS0.

Table 5.3B.1a.2-1: NE-DC configurations and bandwidth combination sets defined for intra-band contiguous NE-DC

| NR – E-UTRA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink NE-DC configuration | Uplink NE-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_3(n)AA | DC_3(n)AA1 | 5, 10, 15, 20 | 5,10,15, 20, 25, 30 |  | 50 | 0 |
|  |  |  | 5,10,15, 20, 25, 30 | 5, 10, 15, 20 |  |  |
| NOTE 1: Only single switched UL is supported. |  |  |  |  |  |  |

## 5.3C	Void

## 5.3D	Void

## 5.3E	UE Channel bandwidth for V2X

### 5.3E.0	General

The requirements specified in clause 5.3B are applicable to NR V2X UE.

### 5.3E.1	Intra-band contiguous V2X in FR1

For intra-band contiguous E-UTRA NR V2X UE, an EN-DC bandwidth class in Table 5.3B.0-1 are considered to specify the V2X transmission/reception configurations.

Bandwidth combination sets and V2X transmission/reception configurations for intra-band contiguous V2X UE are specified in Table 5.3E.1-1.

Table 5.3E.1-1: E-UTRA-NR V2X configurations and bandwidth combination sets for intra-band contiguous V2X UE

| V2X configuration | SL transmisison band | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| V2X_(n)47AA | E-UTRA Band 47 or NR band n47 | 10 | 10,20,30,40 | 60 | 0 |
|  |  | 20 | 10,20,30,40 |  |  |

### 5.3E.2	Intra-band non-contiguous V2X in FR1

For intra-band non-contiguous E-UTRA NR V2X UE, an EN-DC bandwidth class in Table 5.3B.0-1 are considered to specify the V2X transmission/reception configurations.

Bandwidth combination sets and SL transmission/reception configurations for intra-band non-contiguous V2X are specified in Table 5.3E.2-1.

Table 5.3E.2-1: E-UTRA-NR V2X configurations and bandwidth combination sets for intra-band non-contiguous V2X UE

| V2X configuration | SL transmisison band | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| V2X_47A_n47A | E-UTRA Band 47 or NR band n47 | 10 | 10,20,30,40 | 60 | 0 |
|  |  | 20 | 10,20,30,40 |  |  |

### 5.3E.3	Inter-band V2X in FR1

For inter-band E-UTRA NR V2X UE, the each channel bandwidth for inter-band V2X operations in FR1 is specified in TS 36.101 [4] and TS 38.101-1 [2], respectively.

## 5.4	Void

## 5.4A	Channel arrangement for CA

The channel arrangement for CA operations in FR1 and FR2 as specified in TS 38.101-1 [2] and TS 38.101-2 [3], respectively.

## 5.4B	Channel arrangement for DC

### 5.4B.0	General

The channel arrangement for intra-band EN-DC operations in FR1 is specified in TS 36.101 [4] and TS 38.101-1 [2] , respectively.

### 5.4B.1	Channel spacing for intra-band EN-DC carriers

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between E-UTRA carrier and an adjacent NR carrier for intra-band contiguous EN-DC is defined as following:

-	For NR operating bands with 100 kHz channel raster,

Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2

-	For NR operating bands with 15 kHz channel raster,

-	Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-5kHz, 0kHz, 5kHz} for ∆FRaster equals to 15 kHz

-	Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-10 kHz, 0 kHz, 10 kHz} for ∆FRaster equals to 30 kHz

where BWE-UTRA_Channel and BWNR_Channel are the channel bandwidths of the E-UTRA and NR carriers, ∆FRaster is the  band dependent channel raster granularity defined in TS38.101-1[2]. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario.

For intra-band non-contiguous EN-DC the channel spacing between E-UTRA and NR carriers shall be larger than the nominal channel spacing defined in this clause.

## 5.5	Configuration

## 5.5A	Configuration for CA

#### 5.5A.1	Inter-band CA configurations between FR1 and FR2

The configurations for operating bands for CA including Band n41 also apply for the corresponding operating bands for CA with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for operating bands for CA with Band n90 are not listed in the tables below but are covered by this specification.

The configuration tables for CA describe Bandwidth Combination Sets. Bandwidth Combination Set 4 and 5 contains all possible defined channel bandwidths for each FR1 band in the combination. The fact that BCS4 and BCS5 contains all channel bandwidths for each FR1 band does not alter if a bandwidth is mandatory or optional for a given band. Bandwidths that are identified as optional in Table 5.3.5-1 of TS 38.101-1 [2] for a given release are still optional for UEs that support BCS4 or BCS5, where the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities. Note that the minimum bandwidth is indicated only in BCS5 and BCS5 shall not be indicated together with BCS4 for a CA configuration. For inter-band CA combinations including intra-band CA and with BCS4 or BCS5 in the following configuration tables, the Bandwidth Combination Sets for the FR1 intra-band CA are BCS4 or BCS5, respectively, and the Bandwidth Combination Sets for the FR2 intra-band CA are BCS0.

Table 5.5A.1-1a: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n257A | CA_n1A-n257A | n1 | 5, 10, 15, 20 |  | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n257D | CA_n257D CA_n1A-n257A/D | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257D |  |  |
| CA_n1A-n257E | - | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257E |  |  |
| CA_n1A-n257F | - | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257F |  |  |
| CA_n1A-n257G | CA_n257G CA_n1A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n257H | CA_n257G/H CA_n1A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n257I | CA_n257G/H/I CA_n1A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n257J | CA_n257G/H/I/J CA_n1A-n257A/G/H/I/J | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257J |  |  |
| CA_n1A-n257K | CA_n257G/H/I/J/K CA_n1A-n257A/G/H/I/J/K | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257K |  |  |
| CA_n1A-n257L | CA_n257G/H/I/J/K CA_n1A-n257A/G/H/I/J/K | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257L |  |  |
| CA_n1A-n257M | CA_n257G/H/I/J/K CA_n1A-n257A/G/H/I/J/K | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257M |  |  |
| CA_n1A-n257(2A) | CA_n1A-n257A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  | n257 | CA_n257(2A) |  |  |
| CA_n1A-n257(2G) | CA_n1A-n257A/G | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  | n257 | CA_n257(2G) |  |  |
| CA_n1A-n257(A-G) | CA_n1A-n257A/G | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  | n257 | CA_n257(A-G) |  |  |
| CA_n1A-n258A | CA_n1A-n258A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n1A-n258B | CA_n1A-n258A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n1A-n258C | CA_n1A-n258A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n1A-n258D | CA_n1A-n258A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258D |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n1A-n258E | CA_n1A-n258A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258E |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n1A-n258F | CA_n1A-n258A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258F |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n1A-n258G | CA_n1A-n258A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258G |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n1A-n258H | CA_n1A-n258A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258H |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n1A-n258I | CA_n1A-n258A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258I |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n1A-n258J | CA_n1A-n258A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258J |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n1A-n258K | CA_n1A-n258A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258K |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n1A-n258L | CA_n1A-n258A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258L |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n1A-n258M | CA_n1A-n258A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258M |  |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n1A-n258(2A) | CA_n1A-n258A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  | n258 | CA_n258(2A) |  |  |
| CA_n1A-n258(2G) | CA_n1A-n258A/G | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  | n258 | CA_n258(2G) |  |  |
| CA_n1A-n258(A-G) | CA_n1A-n258A/G | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  | n258 | CA_n258(A-G) |  |  |

Table 5.5A.1-1b: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n2A-n260A | CA_n2A-n260A | n2 |  | 5, 10, 15, 20 | 0 |
|  |  | n260 |  | 50, 100, 200, 400 |  |
| CA_n2A-n260G | CA_n2A-n260A/G | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260G |  |  |
| CA_n2A-n260H | CA_n2A-n260A/G/H | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260H |  |  |
| CA_n2A-n260I | CA_n2A-n260A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260I |  |  |
| CA_n2A-n260J | CA_n2A-n260A/G/H/I/J | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260J |  |  |
| CA_n2A-n260K | CA_n2A-n260A/G/H/I/J/K | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260K |  |  |
| CA_n2A-n260L | CA_n2A-n260A/G/H/I/J/K/L | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260L |  |  |
| CA_n2A-n260M | CA_n2A-n260A/G/H/I/J/K/L/M | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260M |  |  |
| CA_n2A-n260R2 | CA_n2A-n260A/R2 | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260R2 |  |  |
| CA_n2A-n260R3 | CA_n2A-n260A/R2/R3 | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260R3 |  |  |
| CA_n2A-n260R4 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260R4 |  |  |
| CA_n2A-n260R5 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260R5 |  |  |
| CA_n2A-n260R6 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260R6 |  |  |
| CA_n2A-n260R7 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260R7 |  |  |
| CA_n2A-n260R8 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260R8 |  |  |
| CA_n2A-n260R9 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260R9 |  |  |
| CA_n2A-n260R10 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n260 | CA_n260R10 |  |  |
| CA_n2(2A)-n260A | CA_n2A-n260A | n2 | CA_n2(2A) |  | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n2(2A)-n260G | CA_n2A-n260A/G | n2 | CA_n2(2A) |  | 0 |
|  |  | n260 | CA_n260G |  |  |
| CA_n2(2A)-n260H | CA_n2A-n260A/G/H | n2 | CA_n2(2A) |  | 0 |
|  |  | n260 | CA_n260H |  |  |
| CA_n2(2A)-n260I | CA_n2A-n260A/G/H/I | n2 | CA_n2(2A) |  | 0 |
|  |  | n260 | CA_n260I |  |  |
| CA_n2(2A)-n260J | CA_n2A-n260A/G/H/I/J | n2 | CA_n2(2A) |  | 0 |
|  |  | n260 | CA_n260J |  |  |
| CA_n2(2A)-n260K | CA_n2A-n260A/G/H/I/J/K | n2 | CA_n2(2A) |  | 0 |
|  |  | n260 | CA_n260K |  |  |
| CA_n2(2A)-n260L | CA_n2A-n260A/G/H/I/J/K/L | n2 | CA_n2(2A) |  | 0 |
|  |  | n260 | CA_n260L |  |  |
| CA_n2(2A)-n260M | CA_n2A-n260A/G/H/I/J/K/L/M | n2 | CA_n2(2A) |  | 0 |
|  |  | n260 | CA_n260M |  |  |
| CA_n2A-n261A | CA_n2A-n261A | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n2A-n261G | CA_n2A-n261A/G | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261G |  |  |
| CA_n2A-n261H | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261H |  |  |
| CA_n2A-n261I | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261I |  |  |
| CA_n2A-n261J | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261J |  |  |
| CA_n2A-n261K | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261K |  |  |
| CA_n2A-n261L | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261L |  |  |
| CA_n2A-n261M | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261M |  |  |
| CA_n2A-n261(2A) | CA_n2A-n261A | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(2A) |  |  |
| CA_n2A-n261(2G) | CA_n2A-n261A/G | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(2G) |  |  |
| CA_n2A-n261(2H) | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(2H) |  |  |
| CA_n2A-n261(2I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(2I) |  |  |
| CA_n2A-n261(3A) | CA_n2A-n261A | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(3A) |  |  |
| CA_n2A-n261(4A) | CA_n2A-n261A | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(4A) |  |  |
| CA_n2A-n261(A-G) | CA_n2A-n261A/G | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(A-G) |  |  |
| CA_n2A-n261(A-H) | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(A-H) |  |  |
| CA_n2A-n261(A-I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(A-I) |  |  |
| CA_n2A-n261(A-J) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(A-J) |  |  |
| CA_n2A-n261(A-K) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(A-K) |  |  |
| CA_n2A-n261(A-L) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(A-L) |  |  |
| CA_n2A-n261(G-H) | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(G-H) |  |  |
| CA_n2A-n261(H-I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(H-I) |  |  |
| CA_n2A-n261(G-I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(G-I) |  |  |
| CA_n2A-n261(A-G-H) | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n2A-n261(A-G-I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n2A-n261(2A-H) | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n2A-n261(2A-G) | CA_n2A-n261A/G | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n2A-n261(2A-I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n2A-n261(A-2G) | CA_n2A-n261A/G | n2 | 5, 10, 15, 20 |  | 0 |
|  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n1A-n258R2 | CA_n1A-n258A/R2 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  | 0 |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n1A-n258R3 | CA_n1A-n258A/R2/R3 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  | 0 |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n1A-n258R4 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  | 0 |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n1A-n258R5 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  | 0 |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n1A-n258R6 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  | 0 |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n1A-n258R7 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  | 0 |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n1A-n258R8 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  | 0 |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n1A-n258R9 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  | 0 |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n1A-n258R10 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 |  | 0 |
|  |  | n258 | CA_n258R10 |  |  |

Table 5.5A.1-1c: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n257A | CA_n3A-n257A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n257D | CA_n3A-n257A/D | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257D |  |
| CA_n3A-n257G | CA_n3A-n257A/G | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n3A-n257H | CA_n3A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n3A-n257I | CA_n3A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n3A-n257J | CA_n3A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n3A-n257K | CA_n3A-n257A/G/H/I/J | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n3A-n257L | CA_n3A-n257A/G/H/I/J/K | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n3A-n257M | CA_n3A-n257A/G/H/I/J/K/L | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n3A-n257(2A) | CA_n3A-n257A/(2A) | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257(2A) |  |
| CA_n3A-n257(2G) | CA_n3A-n257A/G/(2G) | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257(2G) |  |
|  | CA_n3A-n257A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n257 | CA_n257(2G) |  |
| CA_n3A-n257(A-G) | CA_n3A-n257A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | CA_n257(A-G) |  |
| CA_n3(2A)-n257A | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3(2A)-n257G | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n3(2A)-n257H | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n3(2A)-n257I | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n3(2A)-n257J | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n3(2A)-n257K | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n3(2A)-n257L | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n3(2A)-n257M | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n3B-n257A | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3B-n257G | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n3B-n257H | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n3B-n257I | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n3B-n257J | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n3B-n257K | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n3B-n257L | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n3B-n257M | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n3A-n258A | CA_n3A-n258A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3A-n258B | CA_n3A-n258A CA_n3A-n258B | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n3A-n258B | CA_n3A-n258A/B | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n3A-n258C | CA_n3A-n258A/B/C | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n3A-n258D | CA_n3A-n258A/D | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n3A-n258E | CA_n3A-n258A/D/E | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n3A-n258F | CA_n3A-n258A/D/E/F | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n3A-n258G | CA_n3A-n258A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n3A-n258H | CA_n3A-n258A/G/H | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n3A-n258I | CA_n3A-n258A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n3A-n258J | CA_n3A-n258A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n3A-n258K | CA_n3A-n258A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n3A-n258L | CA_n3A-n258A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n3A-n258M | CA_n3A-n258A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n3A-n258R2 | CA_n3A-n258A/R2 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R2 |  |
| CA_n3A-n258R3 | CA_n3A-n258A/R2/R3 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R3 |  |
| CA_n3A-n258R4 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R4 |  |
| CA_n3A-n258R5 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R5 |  |
| CA_n3A-n258R6 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R6 |  |
| CA_n3A-n258R7 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R7 |  |
| CA_n3A-n258R8 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R8 |  |
| CA_n3A-n258R9 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R9 |  |
| CA_n3A-n258R10 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R10 |  |
| CA_n3A-n258(2A) | CA_n3A-n258A/(2A) | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n3A-n258(2G) | CA_n3A-n258A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n3A-n258(A-G) | CA_n3A-n258A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n3(2A)-n257A | CA_n3A-n257A | n3 | CA_n3(2A) | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3(2A)-n257G | CA_n3A-n257A CA_n3A-n257G | n3 | CA_n3(2A) | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n3(2A)-n257H | CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H | n3 | CA_n3(2A) | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n3(2A)-n257I | CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I | n3 | CA_n3(2A) | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n3(2A)-n258A | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3(2A)-n258G | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n3(2A)-n258H | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n3(2A)-n258I | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n3(2A)-n258J | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n3(2A)-n258K | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n3(2A)-n258L | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n3(2A)-n258M | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n3B-n258A | CA_n3A-n258A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3B-n258B | CA_n3A-n258A/B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3B-n258C | CA_n3A-n258A/B/C | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3B-n258D | CA_n3A-n258A/D | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3B-n258E | CA_n3A-n258A/D/E | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3B-n258F | CA_n3A-n258A/D/E/F | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3B-n258G | CA_n3A-n258A/G | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n3B-n258H | CA_n3A-n258A/G/H | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n3B-n258I | CA_n3A-n258A/G/H/I | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n3B-n258J | CA_n3A-n258A/G/H/I | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n3B-n258K | CA_n3A-n258A/G/H/I | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n3B-n258L | CA_n3A-n258A/G/H/I | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n3B-n258M | CA_n3A-n258A/G/H/I | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n3B-n258R2 | CA_n3B-n258A/R2 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R2 |  |
| CA_n3B-n258R3 | CA_n3B-n258A/R2/R3 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R3 |  |
| CA_n3B-n258R4 | CA_n3B-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R4 |  |
| CA_n3B-n258R5 | CA_n3B-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R5 |  |
| CA_n3B-n258R6 | CA_n3B-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R6 |  |
| CA_n3B-n258R7 | CA_n3B-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R7 |  |
| CA_n3B-n258R8 | CA_n3B-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R8 |  |
| CA_n3B-n258R9 | CA_n3B-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R9 |  |
| CA_n3B-n258R10 | CA_n3B-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R10 |  |

Table 5.5A.1-1d: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration |  |  | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n258A |  |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n258 |  | 50, 100, 200, 400 |  |  |
| CA_n5A-n258B |  |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n258 |  | CA_n258B |  |  |
| CA_n5A-n258C |  |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n258 |  | CA_n258C |  |  |
| CA_n5A-n258D |  |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n258 |  | CA_n258D |  |  |
| CA_n5A-n258E |  |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n258 |  | CA_n258E |  |  |
| CA_n5A-n258F |  |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n258 |  | CA_n258F |  |  |
| CA_n5A-n258G |  |  | CA_n5A-n258A/G | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n258 |  |  | CA_n258G |  |
| CA_n5A-n258H |  |  | CA_n5A-n258A/G/H | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n258 |  |  | CA_n258H |  |
| CA_n5A-n258I |  |  | CA_n5A-n258A/G/H/I | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n258 |  |  | CA_n258I |  |
| CA_n5A-n258J |  |  | CA_n5A-n258A/G/H/I | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n258 |  |  | CA_n258J |  |
| CA_n5A-n258K |  |  | CA_n5A-n258A/G/H/I | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n258 |  |  | CA_n258K |  |
| CA_n5A-n258L |  |  | CA_n5A-n258A/G/H/I | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n258 |  |  | CA_n258L |  |
| CA_n5A-n258M |  |  | CA_n5A-n258A/G/H/I | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n258 |  |  | CA_n258M |  |
| CA_n5A-n260G |  |  | CA_n5A-n260A/G | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260G |  |
| CA_n5A-n260H |  |  | CA_n5A-n260A/G/H | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260H |  |
| CA_n5A-n260I |  |  | CA_n5A-n260A/G/H/I | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260I |  |
| CA_n5A-n260J |  |  | CA_n5A-n260A/G/H/I/J | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260J |  |
| CA_n5A-n260K |  |  | CA_n5A-n260A/G/H/I/J/K | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260K |  |
| CA_n5A-n260L |  |  | CA_n5A-n260A/G/H/I/J/K/L | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260L |  |
| CA_n5A-n260M |  |  | CA_n5A-n260A/G/H/I/J/K/L/M | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260M |  |
| CA_n5A-n260R2 |  |  | CA_n5A-n260A/R2 | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260R2 |  |
| CA_n5A-n260R3 |  |  | CA_n5A-n260A/R2/R3 | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260R3 |  |
| CA_n5A-n260R4 |  |  | CA_n5A-n260A/R2/R3/R4 | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260R4 |  |
| CA_n5A-n260R5 |  |  | CA_n5A-n260A/R2/R3/R4 | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260R5 |  |
| CA_n5A-n260R6 |  |  | CA_n5A-n260A/R2/R3/R4 | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260R6 |  |
| CA_n5A-n260R7 |  |  | CA_n5A-n260A/R2/R3/R4 | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260R7 |  |
| CA_n5A-n260R8 |  |  | CA_n5A-n260A/R2/R3/R4 | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260R8 |  |
| CA_n5A-n260R9 |  |  | CA_n5A-n260A/R2/R3/R4 | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260R9 |  |
| CA_n5A-n260R10 |  |  | CA_n5A-n260A/R2/R3/R4 | n5 |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  | n260 |  |  | CA_n260R10 |  |
| CA_n5A-n260(2A) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(2A) |  |  |
| CA_n5A-n260(3A) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(3A) |  |  |
| CA_n5A-n260(4A) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(4A) |  |  |
| CA_n5A-n260(5A) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(5A) |  |  |
| CA_n5A-n260(6A) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(6A) |  |  |
| CA_n5A-n260(7A) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(7A) |  |  |
| CA_n5A-n260(8A) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(8A) |  |  |
| CA_n5A-n260(2G) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(2G) |  |  |
| CA_n5A-n260(2H) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(2H) |  |  |
| CA_n5A-n260(A-G) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(A-G) |  |  |
| CA_n5A-n260(2A-G) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(2A-G) |  |  |
| CA_n5A-n260(A-H) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(A-H) |  |  |
| CA_n5A-n260(2A-2G) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(2A-2G) |  |  |
| CA_n5A-n260(3A-G) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(3A-G) |  |  |
| CA_n5A-n260(A-2G) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(A-2G) |  |  |
| CA_n5A-n260(G-H) |  |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n260 |  | CA_n260(G-H) |  |  |
| CA_n5A-n261A |  |  | CA_n5A-n261A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 |  | 50, 100, 200, 400 |  |  |
| CA_n5A-n261(2A) |  |  | CA_n5A-n261A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 |  | CA_n261(2A) |  |  |
| CA_n5A-n261(3A) |  |  | CA_n5A-n261A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 |  | CA_n261(3A) |  |  |
| CA_n5A-n261(4A) |  |  | CA_n5A-n261A | n5 |  | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 |  | CA_n261(4A) |  |  |
| CA_n5A-n261G | CA_n5A-n261A/G |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261G |  |  |
| CA_n5A-n261H | CA_n5A-n261A/G/H |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261H |  |  |
| CA_n5A-n261I | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261I |  |  |
| CA_n5A-n261J | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261J |  |  |
| CA_n5A-n261K | CA_n5A-n261A/G/H/ |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261K |  |  |
| CA_n5A-n261L | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261L |  |  |
| CA_n5A-n261M | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261M |  |  |
| CA_n5A-n261O | CA_n5A-n261A |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261O |  |  |
| CA_n5A-n261P | CA_n5A-n261A |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261P |  |  |
| CA_n5A-n261Q | CA_n5A-n261A |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261Q |  |  |
| CA_n5A-n261(2G) | CA_n5A-n261A/G |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(2G) |  |  |
| CA_n5A-n261(2H) | CA_n5A-n261A/G/H |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(2H) |  |  |
| CA_n5A-n261(2I) | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(2I) |  |  |
| CA_n5A-n261(A-G) | CA_n5A-n261A/G |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(A-G) |  |  |
| CA_n5A-n261(A-H) | CA_n5A-n261A/G/H |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |
| CA_n5A-n261(A-I) | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(A-I) |  |  |
| CA_n5A-n261(A-J) | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(A-J) |  |  |
| CA_n5A-n261(A-K) | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(A-K) |  |  |
| CA_n5A-n261(A-L) | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(A-L) |  |  |
| CA_n5A-n261(G-H) | CA_n5A-n261A/G/H |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |
| CA_n5A-n261(G-J) | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(G-J) |  |  |
| CA_n5A-n261(H-I) | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |
| CA_n5A-n261(G-I) | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |
| CA_n5A-n261(A-G-H) | CA_n5A-n261A/G/H |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n5A-n261(A-G-I) | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n5A-n261(2A-H) | CA_n5A-n261A/G/H |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n5A-n261(2A-G) | CA_n5A-n261A/G |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n5A-n261(3A-G) | CA_n5A-n261A/G |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(3A-G) |  |  |
| CA_n5A-n261(2A-I) | CA_n5A-n261A/G/H/I |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n5A-n261(A-2G) | CA_n5A-n261A/G |  |  | n5 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |

Table 5.5A.1-1e: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n257A | CA_n7A-n257A | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n7 |  | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n257 |  | See n257 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n7A-n257G | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257G |  |  |  |
|  | CA_n7A-n257A/G | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n7A-n257H | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257H |  |  |  |
|  | CA_n7A-n257A/G/H | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n7A-n257I | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257I |  |  |  |
|  | CA_n7A-n257A/G/H/I | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257I |  |  |  |
| CA_n7A-n257J | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257J |  |  |  |
|  | CA_n7A-n257A/G/H/I/J | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257J |  |  |  |
| CA_n7A-n257K | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257K |  |  |  |
|  | CA_n7A-n257A/G/H/I/J/K | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257K |  |  |  |
| CA_n7A-n257L | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257L |  |  |  |
|  | CA_n7A-n257A/G/H/I/J/K/L | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257L |  |  |  |
| CA_n7A-n257M | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257M |  |  |  |
|  | CA_n7A-n257A/G/H/I/J/K/L/M | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257M |  |  |  |
| CA_n7A-n258A | CA_n7A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |  |
| CA_n7A-n258B | CA_n7A-n258A CA_n7A-n258B | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258B |  |  |  |
| CA_n7A-n258C | CA_n7A-n258A/B/C | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258C |  |  |  |
| CA_n7A-n258D | CA_n7A-n258A/D | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258D |  |  |  |
| CA_n7A-n258E | CA_n7A-n258A/D/E | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258E |  |  |  |
| CA_n7A-n258F | CA_n7A-n258A/D/E/F | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258F |  |  |  |
| CA_n7A-n258G | CA_n7A-n258A/G | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258G |  |  |  |
| CA_n7A-n258H | CA_n7A-n258A/G/H | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258H |  |  |  |
| CA_n7A-n258I | CA_n7A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258I |  |  |  |
| CA_n7A-n258J | CA_n7A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258J |  |  |  |
| CA_n7A-n258K | CA_n7A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258K |  |  |  |
| CA_n7A-n258L | CA_n7A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258L |  |  |  |
| CA_n7A-n258M | CA_n7A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258M |  |  |  |
| CA_n7A-n258R2 | CA_n7A-n258A/R2 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R2 |  |  |  |
| CA_n7A-n258R3 | CA_n7A-n258A/R2/R3 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R3 |  |  |  |
| CA_n7A-n258R4 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R4 |  |  |  |
| CA_n7A-n258R5 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R5 |  |  |  |
| CA_n7A-n258R6 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R6 |  |  |  |
| CA_n7A-n258R7 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R7 |  |  |  |
| CA_n7A-n258R8 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R8 |  |  |  |
| CA_n7A-n258R9 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R9 |  |  |  |
| CA_n7A-n258R10 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R10 |  |  |  |
| CA_n7B-n258A | CA_n7A-n258A | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |  |
| CA_n7B-n258B | CA_n7A-n258A/B | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258B |  |  |  |
| CA_n7B-n258C | CA_n7A-n258A/B/C | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258C |  |  |  |
| CA_n7B-n258D | CA_n7A-n258A/D | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258D |  |  |  |
| CA_n7B-n258E | CA_n7A-n258A/D/E | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258E |  |  |  |
| CA_n7B-n258F | CA_n7A-n258A/D/E/F | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258F |  |  |  |
| CA_n7B-n258G | CA_n7A-n258A/G | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258G |  |  |  |
| CA_n7B-n258H | CA_n7A-n258A/G/H | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258H |  |  |  |
| CA_n7B-n258I | CA_n7A-n258A/G/H/I | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258I |  |  |  |
| CA_n7B-n258J | CA_n7A-n258A/G/H/I | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258J |  |  |  |
| CA_n7B-n258K | CA_n7A-n258A/G/H/I | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258K |  |  |  |
| CA_n7B-n258L | CA_n7A-n258A/G/H/I | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258L |  |  |  |
| CA_n7B-n258M | CA_n7A-n258A/G/H/I | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258M |  |  |  |
| CA_n7B-n258R2 | CA_n7B-n258A/R2 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R2 |  |  |  |
| CA_n7B-n258R3 | CA_n7B-n258A/R2/R3 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R3 |  |  |  |
| CA_n7B-n258R4 | CA_n7B-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R4 |  |  |  |
| CA_n7B-n258R5 | CA_n7B-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R5 |  |  |  |
| CA_n7B-n258R6 | CA_n7B-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R6 |  |  |  |
| CA_n7B-n258R7 | CA_n7B-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R7 |  |  |  |
| CA_n7B-n258R8 | CA_n7B-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R8 |  |  |  |
| CA_n7B-n258R9 | CA_n7B-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R9 |  |  |  |
| CA_n7B-n258R10 | CA_n7B-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R10 |  |  |  |
| CA_n7A-n260A | CA_n7A-n260A | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n7A-n260G | CA_n7A-n260A/G | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260G |  |  |  |
| CA_n7A-n260H | CA_n7A-n260A/G/H | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260H |  |  |  |
| CA_n7A-n260I | CA_n7A-n260A/G/H/I | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260I |  |  |  |
| CA_n7A-n260J | CA_n7A-n260A/G/H/I/J | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260J |  |  |  |
| CA_n7A-n260K | CA_n7A-n260A/G/H/I/J/K | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260K |  |  |  |
| CA_n7A-n260L | CA_n7A-n260A/G/H/I/J/K/L | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260L |  |  |  |
| CA_n7A-n260M | CA_n7A-n260A/G/H/I/J/K/L/M | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260M |  |  |  |
| CA_n8A-n257A | CA_n8A-n257A | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |  |
| CA_n8A-n257D | - | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257D |  |  |  |
| CA_n8A-n257E | - | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257E |  |  |  |
| CA_n8A-n257F | - | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257F |  |  |  |
| CA_n8A-n257G | CA_n257G CA_n8A-n257A/G | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n8A-n257H | CA_n257G/H CA_n8A-n257A/G/H | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n8A-n257I | CA_n257G/H/I CA_n8A-n257A/G/H/I | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257I |  |  |  |
| CA_n8A-n257J | CA_n257G/H/I/J CA_n8A-n257A/G/H/I/J | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257J |  |  |  |
| CA_n8A-n257K | CA_n257G/H/I/J/K CA_n8A-n257A/G/H/I/J/K | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257K |  |  |  |
| CA_n8A-n257L | CA_n8A-n257A/G/H/I/J/K | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257L |  |  |  |
| CA_n8A-n257M | CA_n8A-n257A/G/H/IJ/K | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257M |  |  |  |
| CA_n8A-n258A | CA_n8A-n258A | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |  |
| CA_n8A-n258B |  | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  |  | 0 |  |  |  |
|  |  | n258 |  | CA_n258B |  |  |
| CA_n8A-n258C | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258C |  |  |
| CA_n8A-n258D | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258D |  |  |
| CA_n8A-n258E | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258E |  |  |
| CA_n8A-n258F | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258F |  |  |
| CA_n8A-n258G | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258G |  |  |
| CA_n8A-n258H | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258H |  |  |
| CA_n8A-n258I | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258I |  |  |
| CA_n8A-n258J | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258J |  |  |
| CA_n8A-n258K | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258K |  |  |
| CA_n8A-n258L | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258L |  |  |
| CA_n8A-n258M | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258M |  |  |

Table 5.5A.1-1f: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n12A-n258A | CA_n12A-n258A | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n12A-n260A | CA_n12A-n260A | n12 |  | 5, 10, 15 | 0 |
|  |  | n260 |  | 50, 100, 200, 400 |  |
| CA_n12A-n260G | CA_n12A-n260A CA_n12A-n260G | n12 |  | 5, 10, 15 | 0 |
|  |  | n260 |  | CA_n260G |  |
| CA_n12A-n260G | CA_n12A-n260A/G | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260G |  |  |
| CA_n12A-n260H | CA_n12A-n260A/G/H | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260H |  |  |
| CA_n12A-n260I | CA_n12A-n260A/G/H/I | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260I |  |  |
| CA_n12A-n260J | CA_n12A-n260A/G/H/I/J | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260J |  |  |
| CA_n12A-n260K | CA_n12A-n260A/G/H/I/J/K | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260K |  |  |
| CA_n12A-n260L | CA_n12A-n260A/G/H/I/J/K/L | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260L |  |  |
| CA_n12A-n260M | CA_n12A-n260A/G/H/I/J/K/L/M | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260M |  |  |
| CA_n12A-n261A | CA_n12A-n261A | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n14A-n260A | CA_n14A-n260A | n14 | 5, 10 |  | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n14A-n260G | CA_n14A-n260A/G | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260G |  |  |
| CA_n14A-n260H | CA_n14A-n260A/G/H | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260H |  |  |
| CA_n14A-n260I | CA_n14A-n260A/G/H/I | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260I |  |  |
| CA_n14A-n260J | CA_n14A-n260A/G/H/I/J | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260J |  |  |
| CA_n14A-n260K | CA_n14A-n260A/G/H/I/J/K | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260K |  |  |
| CA_n14A-n260L | CA_n14A-n260A/G/H/I/J/K/L | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260L |  |  |
| CA_n14A-n260M | CA_n14A-n260A/G/H/I/J/K/L/M | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260M |  |  |
| CA_n18A-n257A | CA_n18A-n257A | n18 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n18A-n257G | CA_n18A-n257A CA_n18A-n257G | n18 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n18A-n257H | CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H | n18 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n18A-n257I | CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n18A-n257I | n18 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257I |  |

Table 5.5A.1-1g: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- |
| CA_n25A-n257A | CA_n25A-n257A | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n257 | See n257 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n257G | CA_n25A-n257A/G | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257G |  |
| CA_n25A-n257H | CA_n25A-n257A/G/H | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257H |  |
| CA_n25A-n257I | CA_n25A-n257A/G/H/I | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257I |  |
| CA_n25A-n257J | CA_n25A-n257A/G/H/I/J | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257J |  |
| CA_n25A-n257K | CA_n25A-n257A/G/H/I/J/K | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257K |  |
| CA_n25A-n257L | CA_n25A-n257A/G/H/I/J/K/L | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257L |  |
| CA_n25A-n257M | CA_n25A-n257A/G/H/I/J/K/L/M | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257M |  |
| CA_n25A-n258A | CA_n25A-n258A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n25A-n258(2A) | CA_n25A-n258A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n25A-n258(3A) | CA_n25A-n258A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n25A-n258(4A) | CA_n25A-n258A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n25A-n258(5A) | CA_n25A-n258A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258(5A) |  |
| CA_n25A-n258G | CA_n25A-n258A/G | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n25A-n258(2G) | CA_n25A-n258A/G | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n25A-n258H | CA_n25A-n258A/G/H | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n25A-n258(A-G) | CA_n25A-n258A/G | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n25A-n258(A-H) | CA_n25A-n258A/G/H | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n25A-n258(G-H) | CA_n25A-n258A/G/H | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n25A-n260A | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n260(2A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260(2A) |  |  |
| CA_n25A-n260(3A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260(3A) |  |  |
| CA_n25A-n260(4A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260(4A) |  |  |
| CA_n25A-n260(5A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260(5A) |  |  |
| CA_n25A-n260(6A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260(6A) |  |  |
| CA_n25A-n260(7A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260(7A) |  |  |
| CA_n25A-n260(8A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260(8A) |  |  |
| CA_n25A-n260G | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260G |  |  |
|  | CA_n25A-n260A/G | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n260 | CA_260G |  |  |
| CA_n25A-n260H | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260H |  |  |
|  | CA_n25A-n260A/G/H | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n260 | CA_260H |  |  |
| CA_n25A-n260I | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260I |  |  |
|  | CA_n25A-n260A/G/H/I | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n260 | CA_260I |  |  |
| CA_n25A-n260J | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260J |  |  |
|  | CA_n25A-n260A/G/H/I/J | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n260 | CA_260J |  |  |
| CA_n25A-n260K | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260K |  |  |
|  | CA_n25A-n260A/G/H/I/J/K | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n260 | CA_260K |  |  |
| CA_n25A-n260L | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260L |  |  |
|  | CA_n25A-n260A/G/H/I/J/K/L | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n260 | CA_260L |  |  |
| CA_n25A-n260M | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n260 | CA_n260M |  |  |
|  | CA_n25A-n260A/G/H/I/J/K/L/M | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n260 | CA_260M |  |  |
| CA_n25A-n261A | CA_n25A-n261A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n261 | 50, 100, 200, 400 |  |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n261 | See n261 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n261(2A) | CA_n25A-n261A | n25 | 5, 10, 15, 20 | 0 |  |
|  |  | n261 | CA_n261(2A) |  |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  | n261 | CA_n261(2A) |  |  |
| CA_n26A-n258A | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n26A-n258B | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n26A-n258C | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n26A-n258D | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n26A-n258E | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n26A-n258F | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n26A-n258G | CA_n26A-n258A/G | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |  |
|  |  | n258 | CA_n258G |  |  |  |
| CA_n26A-n258H | CA_n26A-n258A/G/H | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |  |
|  |  | n258 | CA_n258H |  |  |  |
| CA_n26A-n258I | CA_n26A-n258A/G/H/I | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |  |
|  |  | n258 | CA_n258I |  |  |  |
| CA_n26A-n258J | CA_n26A-n258A/G/H/I | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |  |
|  |  | n258 | CA_n258J |  |  |  |
| CA_n26A-n258K | CA_n26A-n258A/G/H/I | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |  |
|  |  | n258 | CA_n258K |  |  |  |
| CA_n26A-n258L | CA_n26A-n258A/G/H/I | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |  |
|  |  | n258 | CA_n258L |  |  |  |
| CA_n26A-n258M | CA_n26A-n258A/G/H/I | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |  |
|  |  | n258 | CA_n258M |  |  |  |
| CA_n26(2A)-n258A | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |  |
| CA_n26(2A)-n258B | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n26(2A)-n258C | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n26(2A)-n258D | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n26(2A)-n258E | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n26(2A)-n258F | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n26(2A)-n258G | CA_n26A-n258A/G | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n26(2A)-n258H | CA_n26A-n258A/G/H | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n26(2A)-n258I | CA_n26A-n258A/G/H/I | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n26(2A)-n258J | CA_n26A-n258A/G/H/I | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n26(2A)-n258K | CA_n26A-n258A/G/H/I | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n26(2A)-n258L | CA_n26A-n258A/G/H/I | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n26(2A)-n258M | CA_n26A-n258A/G/H/I | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n26A-n258R2 | CA_n26A-n258A/R2 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R2 |  |
| CA_n26A-n258R3 | CA_n26A-n258A/R2/R3 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R3 |  |
| CA_n26A-n258R4 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R4 |  |
| CA_n26A-n258R5 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R5 |  |
| CA_n26A-n258R6 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R6 |  |
| CA_n26A-n258R7 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R7 |  |
| CA_n26A-n258R8 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R8 |  |
| CA_n26A-n258R9 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R9 |  |
| CA_n26A-n258R10 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R10 |  |

Table 5.5A.1-1h: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- |
| CA_n28A-n257A | CA_n28A-n257A | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n28A-n257D | CA_n28A-n257A/D | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257D |  |  |
| CA_n28A-n257G | CA_n257G CA_n28A-n257A/G | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n28A-n257H | CA_n257G/H CA_n28A-n257A/G/H | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n28A-n257I | CA_n257G/H/I CA_n28A-n257A/G/H/I | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n28A-n258A | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n28A-n258B | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n28A-n258C | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n28A-n258D | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n28A-n258E | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n28A-n258F | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n28A-n258G | CA_n28A-n258A/G | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n28A-n258H | CA_n28A-n258A/G/H | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n28A-n258I | CA_n28A-n258A/G/H/I | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n28A-n258J | CA_n28A-n258A/G/H/I | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n28A-n258K | CA_n28A-n258A/G/H/I | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n28A-n258L | CA_n28A-n258A/G/H/I | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n28A-n258M | CA_n28A-n258A/G/H/I | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n28A-n258R2 | CA_n28A-n258A/R2 | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n28A-n258R3 | CA_n28A-n258A/R2/R3 | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n28A-n258R4 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n28A-n258R5 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n28A-n258R6 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n28A-n258R7 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n28A-n258R8 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n28A-n258R9 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n28A-n258R10 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n30A-n260A | CA_n30A-n260A | n30 | 5, 10 | 0 |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n30A-n260G | CA_n30A-n260A/G | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n30A-n260H | CA_n30A-n260A/G/H | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n30A-n260I | CA_n30A-n260A/G/H/I | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n30A-n260J | CA_n30A-n260A/G/H/I/J | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n30A-n260K | CA_n30A-n260A/G/H/I/J/K | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n30A-n260L | CA_n30A-n260A/G/H/I/J/K/L | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n30A-n260M | CA_n30A-n260A/G/H/I/J/K/L/M | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n34A-n258A | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n34A-n258B | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n34A-n258C | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n34A-n258D | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n34A-n258E | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n34A-n258F | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n34A-n258G | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n34A-n258H | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n34A-n258I | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n34A-n258J | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n34A-n258K | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n34A-n258L | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n34A-n258M | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n38A-n257A | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n38A-n257G | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n38A-n257H | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n38A-n257I | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n38A-n257J | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257J |  |  |
| CA_n38A-n257K | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257K |  |  |
| CA_n38A-n257L | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257L |  |  |
| CA_n38A-n257M | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257M |  |  |
| CA_n38A-n258A | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n38A-n258G | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n38A-n258H | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n38A-n258I | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n38A-n258J | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n38A-n258K | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n38A-n258L | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n38A-n258M | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n39A-n258A | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n39A-n258B | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n39A-n258C | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n39A-n258D | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n39A-n258E | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n39A-n258F | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n39A-n258G | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n39A-n258H | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n39A-n258I | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n39A-n258J | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n39A-n258K | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n39A-n258L | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n39A-n258M | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n28A-n257D | CA_n28A-n257A CA_n28A-n257D | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257D |  |  |
| CA_n28A-n257G | CA_n257G CA_n28A-n257A CA_n28A-n257G | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n28A-n257H | CA_n257G CA_n257H CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n28A-n257I | CA_n257G CA_n257H CA_n257I CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I | n28 | 5, 10, 15, 20 | 0 |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n28A-n258A | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n28A-n258B | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n28A-n258C | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n28A-n258D | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n28A-n258E | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n28A-n258F | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n28A-n258G | CA_n28A-n258A CA_n28A-n258G | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n28A-n258H | CA_n28A-n258A CA_n28A-n258G CA_n28A-n258H | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n28A-n258I | CA_n28A-n258A CA_n28A-n258G CA_n28A-n258H CA_n28A-n258I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n28A-n258J | CA_n28A-n258A CA_n28A-n258G CA_n28A-n258H CA_n28A-n258I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n28A-n258K | CA_n28A-n258A CA_n28A-n258G CA_n28A-n258H CA_n28A-n258I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n28A-n258L | CA_n28A-n258A CA_n28A-n258G CA_n28A-n258H CA_n28A-n258I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n28A-n258M | CA_n28A-n258A CA_n28A-n258G CA_n28A-n258H CA_n28A-n258I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n30A-n260A | CA_n30A-n260A | n30 | 5, 10 | 0 |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n30A-n260G | CA_n30A-n260A CA_n30A-n260G | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n30A-n260H | CA_n30A-n260A CA_n30A-n260G CA_n30A-n260H | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n30A-n260I | CA_n30A-n260A CA_n30A-n260G CA_n30A-n260H CA_n30A-n260I | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n30A-n260J | CA_n30A-n260A CA_n30A-n260G CA_n30A-n260H CA_n30A-n260I CA_n30A-n260J | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n30A-n260K | CA_n30A-n260A CA_n30A-n260G CA_n30A-n260H CA_n30A-n260I CA_n30A-n260J CA_n30A-n260K | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n30A-n260L | CA_n30A-n260A CA_n30A-n260G CA_n30A-n260H CA_n30A-n260I CA_n30A-n260J CA_n30A-n260K CA_n30A-n260L | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n30A-n260M | CA_n30A-n260A CA_n30A-n260G CA_n30A-n260H CA_n30A-n260I CA_n30A-n260J CA_n30A-n260K CA_n30A-n260L CA_n30A-n260M | n30 | 5, 10 | 0 |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n34A-n258A | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n34A-n258B | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n34A-n258C | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n34A-n258D | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n34A-n258E | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n34A-n258F | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n34A-n258G | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n34A-n258H | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n34A-n258I | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n34A-n258J | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n34A-n258K | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n34A-n258L | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n34A-n258M | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n38A-n257A | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n38A-n257G | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n38A-n257H | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n38A-n257I | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n38A-n257J | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257J |  |  |
| CA_n38A-n257K | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257K |  |  |
| CA_n38A-n257L | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257L |  |  |
| CA_n38A-n257M | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n257 | CA_n257M |  |  |
| CA_n38A-n258A | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n38A-n258G | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n38A-n258H | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n38A-n258I | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n38A-n258J | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n38A-n258K | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n38A-n258L | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n38A-n258M | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n39A-n258A | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |

Table 5.5A.1-1i: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n257A | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n40A-n257D | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257D |  |
| CA_n40A-n257E | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257E |  |
| CA_n40A-n257F | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257F |  |
| CA_n40A-n257G | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n40A-n257H | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n40A-n257I | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n40A-n257J | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n40A-n257K | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n40A-n257L | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n40A-n257M | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n40B-n257A | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n40B-n257D | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257D |  |
| CA_n40B-n257E | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257E |  |
| CA_n40B-n257F | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257F |  |
| CA_n40B-n257G | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n40B-n257H | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n40B-n257I | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n40B-n257J | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n40B-n257K | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n40B-n257L | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n40B-n257M | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n40A-n258A | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n40A-n258D | CA_n40A-n258A | n40 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n40A-n258E | CA_n40A-n258A | n40 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n40A-n258F | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n40A-n258G | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n40A-n258H | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n40A-n258I | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n40A-n258J | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n40A-n258K | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n40A-n258L | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n40A-n258M | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258M |  |

Table 5.5A.1-1j: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n257A | CA_n41A-n257A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n41A-n257G | CA_n257G CA_n41A-n257A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n41A-n257H | CA_n257G/H CA_n41A-n257A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n41A-n257I | CA_n257G/H/I CA_n41A-n257A/G/H/I | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n41A-n258A | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | See n258 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n258B | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n41A-n258C | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n41A-n258D | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n41A-n258E | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n41A-n258F | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n41A-n258G | CA_n41A-n258A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258G |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n258 | CA_n258G |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258G |  |
| CA_n41A-n258H | CA_n41A-n258A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258H |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n258 | CA_n258H |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258H |  |
| CA_n41A-n258I | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n41A-n258J | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n41A-n258K | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n41A-n258L | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n41A-n258M | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n41A-n258(2A) | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(2A) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n41A-n258(3A) | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n41A-n258(4A) | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n41A-n258(5A) | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(5A) |  |
| CA_n41A-n258(2G) | CA_n41A-n258A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(2G) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n41A-n258(A-G) | CA_n41A-n258A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n41A-n258(A-H) | CA_n41A-n258A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n41A-n258(G-H) | CA_n41A-n258A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n41C-n258A | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | See n258 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n258(2A) | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258(2A) |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n41C-n258(3A) | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n41C-n258(4A) | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n41C-n258(5A) | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258(5A) |  |
| CA_n41C-n258G | CA_n41A-n258A/G | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258G |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258G |  |
| CA_n41C-n258(2G) | CA_n41A-n258A/G | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258(2G) |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n41C-n258H | CA_n41A-n258A/G/H | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258H |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258H |  |
| CA_n41C-n258(A-G) | CA_n41A-n258A/G | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n41C-n258(A-H) | CA_n41A-n258A/G/H | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n41C-n258(G-H) | CA_n41A-n258A/G/H | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n41(2A)-n258A | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n41(2A)-n258(2A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(2A) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n41(2A)-n258(3A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n41(2A)-n258(4A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n41(2A)-n258(5A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(5A) |  |
| CA_n41(2A)-n258G | CA_n41A-n258A/G | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258G |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258G |  |
| CA_n41(2A)-n258(2G) | CA_n41A-n258A/G | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(2G) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n41(2A)-n258H | CA_n41A-n258A/G/H | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258H |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258H |  |
| CA_n41(2A)-n258(A-G) | CA_n41A-n258A/G | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n41(2A)-n258(A-H) | CA_n41A-n258A/G/H | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n41(2A)-n258(G-H) | CA_n41A-n258A/G/H | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n41A-n260A | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n260(2A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n41A-n260(3A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n41A-n260(4A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n41A-n260(5A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n41A-n260(6A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n41A-n260(7A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n41A-n260(8A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n41A-n260G | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260G |  |
|  | CA_n41A-n260A/G | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n41A-n260H | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260H |  |
|  | CA_n41A-n260A/G/H | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n41A-n260I | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260I |  |
|  | CA_n41A-n260A/G/H/I | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n41A-n260J | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260J |  |
|  | CA_n41A-n260A/G/H/I/J | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n41A-n260K | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260K |  |
|  | CA_n41A-n260A/G/H/I/J/K | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n41A-n260L | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260L |  |
|  | CA_n41A-n260A/G/H/I/J/K/L | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n41A-n260M | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260M |  |
|  | CA_n41A-n260A/G/H/I/J/K/L/M | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n41(2A)-n260A | CA_n41A-n260A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n260(2A) | CA_n41A-n260A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n41(2A)-n260(3A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n41(2A)-n260(4A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n41(2A)-n260(5A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n41(2A)-n260(6A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n41(2A)-n260(7A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n41(2A)-n260(8A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n41(2A)-n260G | CA_n41A-n260A/G | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260G |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n41(2A)-n260H | CA_n41A-n260A/G/H | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260H |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n41(2A)-n260I | CA_n41A-n260A/G/H/I | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260I |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n41(2A)-n260J | CA_n41A-n260A/G/H/I/J | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260J |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n41(2A)-n260K | CA_n41A-n260A/G/H/I/J/K | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260K |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n41(2A)-n260L | CA_n41A-n260A/G/H/I/J/K/L | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260L |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n41(2A)-n260M | CA_n41A-n260A/G/H/I/J/K/L/M | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260M |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n41C-n260A | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n260(2A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n41C-n260(3A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n41C-n260(4A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n41C-n260(5A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n41C-n260(6A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n41C-n260(7A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n41C-n260(8A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n41C-n260G | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260G |  |
|  | CA_n41A-n260A/G | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n41C-n260H | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260H |  |
|  | CA_n41A-n260A/G/H | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n41C-n260I | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260I |  |
|  | CA_n41A-n260A/G/H/I | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n41C-n260J | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260J |  |
|  | CA_n41A-n260A/G/H/I/J | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n41C-n260K | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260K |  |
|  | CA_n41A-n260A/G/H/I/J/K | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n41C-n260L | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260L |  |
|  | CA_n41A-n260A/G/H/I/J/K/L | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n41C-n260M | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260M |  |
|  | CA_n41A-n260A/G/H/I/J/K/L/M | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n41(2A)-n257A | CA_n41A-n257A | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n41(2A)-n257G | CA_n41A-n257A CA_n41A-n257G | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n41(2A)-n257H | CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n41(2A)-n257I | CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H | n41 | CA_n41(2A) BCS1 | 0 |
|  | CA_n41A-n257I | n257 | CA_n257I |  |
| CA_n41(2A)-n258A | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n41(2A)-n258(2A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(2A) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n41(2A)-n258(3A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n41(2A)-n258(4A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n41(2A)-n258(5A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | See n258 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n258G | CA_n41A-n258A CA_n41A-n258G | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258G |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258G |  |
| CA_n41(2A)-n258(2G) | CA_n41A-n258A CA_n41A-n258G | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(2G) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n41(2A)-n258H | CA_n41A-n258A CA_n41A-n258G CA_n41A-n258H | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258H |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258H |  |
| CA_n41(2A)-n258(A-G) | CA_n41A-n258A CA_n41A-n258G | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n41(2A)-n258(A-H) | CA_n41A-n258A CA_n41A-n258G CA_n41A-n258H | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n41(2A)-n258(G-H) | CA_n41A-n258A CA_n41A-n258G CA_n41A-n258H | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n41A-n260A | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n260(2A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n41A-n260(3A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n41A-n260(4A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n41A-n260(5A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n41A-n260(6A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n41A-n260(7A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n41A-n260(8A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n41A-n260G | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260G |  |
|  | CA_n41A-n260A CA_n41A-n260G | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n41A-n260H | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260H |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n41A-n260I | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260I |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n41A-n260J | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260J |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n41A-n260K | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260K |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J CA_n41A-n260K | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n41A-n260L | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260L |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J CA_n41A-n260K CA_n41A-n260L | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n41A-n260M | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260M |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J CA_n41A-n260K CA_n41A-n260L CA_n41A-n260M | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n41(2A)-n260A | CA_n41A-n260A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n260(2A) | CA_n41A-n260A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n41(2A)-n260(3A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n41(2A)-n260(4A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n41(2A)-n260(5A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n41(2A)-n260(6A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n41(2A)-n260(7A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n41(2A)-n260(8A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n41(2A)-n260G | CA_n41A-n260A CA_n41A-n260G | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260G |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n41(2A)-n260H | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260H |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n41(2A)-n260I | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260I |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n41(2A)-n260J | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260J |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n41(2A)-n260K | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J CA_n41A-n260K | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260K |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n41(2A)-n260L | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J CA_n41A-n260K CA_n41A-n260L | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260L |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n41(2A)-n260M | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J CA_n41A-n260K CA_n41A-n260L CA_n41A-n260M | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260M |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n41C-n260A | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n260(2A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n41C-n260(3A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n41C-n260(4A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n41C-n260(5A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n41C-n260(6A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n41C-n260(7A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n41C-n260(8A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n41C-n260G | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260G |  |
|  | CA_n41A-n260A CA_n41A-n260G | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n41C-n260H | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260H |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n41C-n260I | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260I |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n41C-n260J | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260J |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n41C-n260K | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260K |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J CA_n41A-n260K | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n41C-n260L | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260L |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J CA_n41A-n260K CA_n41A-n260L | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n41C-n260M | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260M |  |
|  | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n41A-n260J CA_n41A-n260K CA_n41A-n260L CA_n41A-n260M | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n41A-n261A | CA_n41A-n261A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n261(2A) | CA_n41A-n261A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n41C-n261A | CA_n41A-n261A | n41 | CA_n41C | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n261 | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n261A | CA_n41A-n261A | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n261 | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n261(2A) | CA_n41A-n261A | n41 | CA_n41C | 0 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n41(2A)-n261(2A) | CA_n41A-n261A | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n261 | CA_n261(2A) |  |

Table 5.5A.1-1k: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration |  | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48A-n260A |  | CA_n48A-n260A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48A-n260G | CA_n48A-n260A/G |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260G |  |  |  |
| CA_n48A-n260H | CA_n48A-n260A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260H |  |  |  |
| CA_n48A-n260I | CA_n48A-n260A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260I |  |  |  |
| CA_n48A-n260J | CA_n48A-n260A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260J |  |  |  |
| CA_n48A-n260K | CA_n48A-n260A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260K |  |  |  |
| CA_n48A-n260L | CA_n48A-n260A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260L |  |  |  |
| CA_n48A-n260M | CA_n48A-n260A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260M |  |  |  |
| CA_n48A-n260R2 | CA_n48A-n260A/R2 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R2 |  |  |  |
| CA_n48A-n260R3 | CA_n48A-n260A/R2/R3 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R3 |  |  |  |
| CA_n48A-n260R4 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R4 |  |  |  |
| CA_n48A-n260R5 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R5 |  |  |  |
| CA_n48A-n260R6 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R6 |  |  |  |
| CA_n48A-n260R7 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R7 |  |  |  |
| CA_n48A-n260R8 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R8 |  |  |  |
| CA_n48A-n260R9 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R9 |  |  |  |
| CA_n48A-n260R10 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R10 |  |  |  |
| CA_n48(2A)-n260A | CA_n48A-n260A |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |  |
| CA_n48(2A)-n260G | CA_n48A-n260A/G |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260G |  |  |  |
| CA_n48(2A)-n260H | CA_n48A-n260A/G/H |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260H |  |  |  |
| CA_n48(2A)-n260I | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260I |  |  |  |
| CA_n48(2A)-n260J | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260J |  |  |  |
| CA_n48(2A)-n260K | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260K |  |  |  |
| CA_n48(2A)-n260L | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260L |  |  |  |
| CA_n48(2A)-n260M | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260M |  |  |  |
| CA_n48B-n260A | CA_n48A-n260A |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |  |
| CA_n48B-n260G | CA_n48A-n260A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260G |  |  |  |
| CA_n48B-n260H | CA_n48A-n260A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260H |  |  |  |
| CA_n48B-n260I | CA_n48A-n260A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260I |  |  |  |
| CA_n48B-n260J | CA_n48A-n260A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260J |  |  |  |
| CA_n48B-n260K | CA_n48A-n260A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260K |  |  |  |
| CA_n48B-n260L | CA_n48A-n260A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260L |  |  |  |
| CA_n48B-n260M | CA_n48A-n260A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260M |  |  |  |
| CA_n48(A-B)-n260A | CA_n48A-n260A |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |  |
| CA_n48(A-B)-n260G | CA_n48A-n260A/G |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260G |  |  |  |
| CA_n48(A-B)-n260H | CA_n48A-n260A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260H |  |  |  |
| CA_n48(A-B)-n260I | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260I |  |  |  |
| CA_n48(A-B)-n260J | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260J |  |  |  |
| CA_n48(A-B)-n260K | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260K |  |  |  |
| CA_n48(A-B)-n260L | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260L |  |  |  |
| CA_n48(A-B)-n260M | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260M |  |  |  |
| CA_n48A-n261A | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |  |
| CA_n48A-n261G | CA_n48A-n261A/G |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261G |  |  |  |
| CA_n48A-n261H | CA_n48A-n261A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261H |  |  |  |
| CA_n48A-n261I | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261I |  |  |  |
| CA_n48A-n261J | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261J |  |  |  |
| CA_n48A-n261K | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261K |  |  |  |
| CA_n48A-n261L | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261L |  |  |  |
| CA_n48A-n261M | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261M |  |  |  |
| CA_n48A-n261(2A) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
| CA_n48A-n261(2G) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
| CA_n48A-n261(2I) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2I) |  |  |  |
| CA_n48A-n261(2H) | CA_n48A-n261A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
| CA_n48A-n261(3A) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
| CA_n48A-n261(4A) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(4A) |  |  |  |
| CA_n48A-n261(A-G) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
| CA_n48A-n261(A-G-H) | CA_n48A-n261A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
| CA_n48A-n261(A-G-I) | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
| CA_n48A-n261(A-H) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
| CA_n48A-n261(A-I) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
| CA_n48A-n261(G-H) | CA_n48A-n261A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
| CA_n48A-n261(H-I) | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
| CA_n48A-n261(G-I) | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
| CA_n48A-n261(2A-G) | CA_n48A-n261A/G |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
| CA_n48A-n261(2A-H) | CA_n48A-n261A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
| CA_n48A-n261(2A-I) | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
| CA_n48A-n261(A-2G) | CA_n48A-n261A/G |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
| CA_n48(2A)-n261A | CA_n48A-n261A |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |  |
| CA_n48(2A)-n261G | CA_n48A-n261A/G |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261G |  |  |  |
| CA_n48(2A)-n261H | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261H |  |  |  |
| CA_n48(2A)-n261I | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261I |  |  |  |
| CA_n48(2A)-n261J | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261J |  |  |  |
| CA_n48(2A)-n261K | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261K |  |  |  |
| CA_n48(2A)-n261L | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261L |  |  |  |
| CA_n48(2A)-n261M | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261M |  |  |  |
| CA_n48(2A)-n261(G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
| CA_n48(2A)-n261(2H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
| CA_n48(2A)-n261(G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
| CA_n48(2A)-n261(A-G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
| CA_n48(2A)-n261(H-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
| CA_n48(2A)-n261(2A-G) | CA_n48A-n261A/G |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
| CA_n48(2A)-n261(2A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
| CA_n48(2A)-n261(2A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
| CA_n48(2A)-n261(2A) | CA_n48A-n261A |  | n48 | CA_n48(2A) BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
| CA_n48(2A)-n261(3A) | CA_n48A-n261A |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
| CA_n48(2A)-n261(2G) | CA_n48A-n261A/G |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
| CA_n48(2A)-n261(A-2G) | CA_n48A-n261A/G |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
| CA_n48(2A)-n261(A-G) | CA_n48A-n261A/G |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
| CA_n48(2A)-n261(A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
| CA_n48(2A)-n261(A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
| CA_n48(2A)-n261(A-G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
| CA_n48B-n261A | CA_n48A-n261A |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |  |
| CA_n48B-n261G | CA_n48A-n261A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261G |  |  |  |
| CA_n48B-n261H | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261H |  |  |  |
| CA_n48B-n261I | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261I |  |  |  |
| CA_n48B-n261J | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261J |  |  |  |
| CA_n48B-n261K | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261K |  |  |  |
| CA_n48B-n261L | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261L |  |  |  |
| CA_n48B-n261M | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261M |  |  |  |
| CA_n48B-n261(G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
| CA_n48B-n261(2H) | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
| CA_n48B-n261(G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
| CA_n48B-n261(A-G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
| CA_n48B-n261(H-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
| CA_n48B-n261(2A-G) | CA_n48A-n261A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
| CA_n48B-n261(2A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
| CA_n48B-n261(2A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
| CA_n48B-n261(2A) | CA_n48A-n261A |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
| CA_n48B-n261(3A) | CA_n48A-n261A |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
| CA_n48B-n261(2G) | CA_n48A-n261A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
| CA_n48B-n261(A-2G) | CA_n48A-n261A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
| CA_n48B-n261(A-G) | CA_n48A-n261A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
| CA_n48B-n261(A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
| CA_n48B-n261(A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
| CA_n48B-n261(A-G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
| CA_n48(A-B)-n261A | CA_n48A-n261A |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |  |
| CA_n48(A-B)-n261G | CA_n48A-n261A/G |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261G |  |  |  |
| CA_n48(A-B)-n261H | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261H |  |  |  |
| CA_n48(A-B)-n261I | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261I |  |  |  |
| CA_n48(A-B)-n261J | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261J |  |  |  |
| CA_n48(A-B)-n261K | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261K |  |  |  |
| CA_n48(A-B)-n261L | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261L |  |  |  |
| CA_n48(A-B)-n261M | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261M |  |  |  |
| CA_n48(A-B)-n261(A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
| CA_n48(A-B)-n261(G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
| CA_n48(A-B)-n261(2G) | CA_n48A-n261A/G |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
| CA_n48(A-B)-n261(2H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
| CA_n48(A-B)-n261(A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
| CA_n48(A-B)-n261(G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
| CA_n48(A-B)-n261(2A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
| CA_n48(A-B)-n261(A-2G) | CA_n48A-n261A/G |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
| CA_n48(A-B)-n261(A-G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
| CA_n48(A-B)-n261(H-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
| CA_n48(A-B)-n261(2A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
| CA_n48(A-B)-n261(A-G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
| CA_n48A-n260G |  | CA_n48A-n260A CA_n48A-n260G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260G |  |  |
| CA_n48A-n260H |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260H |  |  |
| CA_n48A-n260I |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260I |  |  |
| CA_n48A-n260J |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260J |  |  |
| CA_n48A-n260K |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260K |  |  |
| CA_n48A-n260L |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260L |  |  |
| CA_n48A-n260M |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260M |  |  |
| CA_n48A-n260R2 |  | CA_n48A-n260A CA_n48A-n260R2 | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260R2 |  |  |
| CA_n48A-n260R3 |  | CA_n48A-n260A CA_n48A-n260R2 CA_n48A-n260R3 | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260R3 |  |  |
| CA_n48A-n260R4 |  | CA_n48A-n260A CA_n48A-n260R2 CA_n48A-n260R3 CA_n48A-n260R4 | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260R4 |  |  |
| CA_n48A-n260R5 |  | CA_n48A-n260A CA_n48A-n260R2 CA_n48A-n260R3 CA_n48A-n260R4 | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260R5 |  |  |
| CA_n48A-n260R6 |  | CA_n48A-n260A CA_n48A-n260R2 CA_n48A-n260R3 CA_n48A-n260R4 | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260R6 |  |  |
| CA_n48A-n260R7 |  | CA_n48A-n260A CA_n48A-n260R2 CA_n48A-n260R3 CA_n48A-n260R4 | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260R7 |  |  |
| CA_n48A-n260R8 |  | CA_n48A-n260A CA_n48A-n260R2 CA_n48A-n260R3 CA_n48A-n260R4 | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260R8 |  |  |
| CA_n48A-n260R9 |  | CA_n48A-n260A CA_n48A-n260R2 CA_n48A-n260R3 CA_n48A-n260R4 | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260R9 |  |  |
| CA_n48A-n260R10 |  | CA_n48A-n260A CA_n48A-n260R2 CA_n48A-n260R3 CA_n48A-n260R4 | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | CA_n260R10 |  |  |
| CA_n48(2A)-n260A |  | CA_n48A-n260A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48(2A)-n260G |  | CA_n48A-n260A CA_n48A-n260G | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n260 |  | CA_n260G |  |  |
| CA_n48(2A)-n260H |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n260 |  | CA_n260H |  |  |
| CA_n48(2A)-n260I |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n260 |  | CA_n260I |  |  |
| CA_n48(2A)-n260J |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n260 |  | CA_n260J |  |  |
| CA_n48(2A)-n260K |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n260 |  | CA_n260K |  |  |
| CA_n48(2A)-n260L |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n260 |  | CA_n260L |  |  |
| CA_n48(2A)-n260M |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n260 |  | CA_n260M |  |  |
| CA_n48(3A)-n260A |  | CA_n48A-n260A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48(3A)-n260G |  | CA_n48A-n260A CA_n48A-n260G | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260G |  |  |
| CA_n48(3A)-n260H |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260H |  |  |
| CA_n48(3A)-n260I |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260I |  |  |
| CA_n48(3A)-n260J |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260J |  |  |
| CA_n48(3A)-n260K |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260K |  |  |
| CA_n48(3A)-n260L |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260L |  |  |
| CA_n48(3A)-n260M |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260M |  |  |
| CA_n48(4A)-n260A |  | CA_n48A-n260A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48(4A)-n260G |  | CA_n48A-n260A CA_n48A-n260G | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260G |  |  |
| CA_n48(4A)-n260H |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260H |  |  |
| CA_n48(4A)-n260I |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260I |  |  |
| CA_n48(4A)-n260J |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260J |  |  |
| CA_n48(4A)-n260K |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260K |  |  |
| CA_n48(3A)-n260L |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260L |  |  |
| CA_n48(4A)-n260M |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260M |  |  |
| CA_n48B-n260A |  | CA_n48A-n260A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48B-n260G |  | CA_n48A-n260A CA_n48A-n260G | n48 |  | CA_n48B |  | 0 |
|  |  |  | n260 |  | CA_n260G |  |  |
| CA_n48B-n260H |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H | n48 |  | CA_n48B |  | 0 |
|  |  |  | n260 |  | CA_n260H |  |  |
| CA_n48B-n260I |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48B |  | 0 |
|  |  |  | n260 |  | CA_n260I |  |  |
| CA_n48B-n260J |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48B |  | 0 |
|  |  |  | n260 |  | CA_n260J |  |  |
| CA_n48B-n260K |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48B |  | 0 |
|  |  |  | n260 |  | CA_n260K |  |  |
| CA_n48B-n260L |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48B |  | 0 |
|  |  |  | n260 |  | CA_n260L |  |  |
| CA_n48B-n260M |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48B |  | 0 |
|  |  |  | n260 |  | CA_n260M |  |  |
| CA_n48C-n260A |  | CA_n48A-n260A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48C-n260G |  | CA_n48A-n260A CA_n48A-n260G | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260G |  |  |
| CA_n48C-n260H |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260H |  |  |
| CA_n48C-n260I |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260I |  |  |
| CA_n48C-n260J |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260J |  |  |
| CA_n48C-n260K |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260K |  |  |
| CA_n48C-n260L |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260L |  |  |
| CA_n48C-n260M |  | CA_n48A-n260A CA_n48A-n260G CA_n48A-n260H CA_n48A-n260I | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260M |  |  |
| CA_n48A-n263A |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48A-n263G |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48A-n263H |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48A-n263I |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48A-n263J |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48A-n263K |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48A-n263L |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48A-n263M |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48(2A)-n263A |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48(2A)-n263G |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48(2A)-n263H |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48(2A)-n263I |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48(2A)-n263J |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48(2A)-n263K |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48(2A)-n263L |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48(2A)-n263M |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48B-n263A |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48B-n263G |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48B-n263H |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48B-n263I |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48B-n263J |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48B-n263K |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48B-n263L |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48B-n263M |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48(A-B)-n263A |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48(A-B)-n263G |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48(A-B)-n263H |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48(A-B)-n263I |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48(A-B)-n263J |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48(A-B)-n263K |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48(A-B)-n263L |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48(A-B)-n263M |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48C-n263A |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48C-n263G |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48C-n263H |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48C-n263I |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48C-n263J |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48C-n263K |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48C-n263L |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48C-n263M |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48(3A)-n263A |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48(3A)-n263G |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48(3A)-n263H |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48(3A)-n263I |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48(3A)-n263J |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48(3A)-n263K |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48(3A)-n263L |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48(3A)-n263M |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48(4A)-n263A |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48(4A)-n263G |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48(4A)-n263H |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48(4A)-n263I |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48(4A)-n263J |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48(4A)-n263K |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48(4A)-n263L |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48(4A)-n263M |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |

Table 5.5A.1-1l: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n257A | CA_n66A-n257A | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n66A-n257G | CA_n66A-n257A/G | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | CA_n257G |  |
| CA_n66A-n257H | CA_n66A-n257A/G/H | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | CA_n257H |  |
| CA_n66A-n257I | CA_n66A-n257A/G/H/I | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | CA_n257I |  |
| CA_n66A-n257G | CA_n66A-n257A CA_n66A-n257G | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | CA_n257G |  |
| CA_n66A-n257H | CA_n66A-n257A CA_n66A-n257G CA_n66A-n257H | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | CA_n257H |  |
| CA_n66A-n257I | CA_n66A-n257A CA_n66A-n257G CA_n66A-n257H CA_n66A-n257I | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | CA_n257I |  |
| CA_n66A-n258A | CA_n66A-n258A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n66A-n258(2A) | CA_n66A-n258A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n66A-n258(3A) | CA_n66A-n258A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n66A-n258(4A) | CA_n66A-n258A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n66A-n258(5A) | CA_n66A-n258A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n258 | CA_n258(5A) |  |
| CA_n66A-n258G | CA_n66A-n258A/G | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n66A-n258(2G) | CA_n66A-n258A/G | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n66A-n258H | CA_n66A-n258A/G/H | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n66A-n258(A-G) | CA_n66A-n258A/G | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n66A-n258(A-H) | CA_n66A-n258A/G/H | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n66A-n258(G-H) | CA_n66A-n258A/G/H | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n66A-n260A | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n66 | See n66 channel bandwidths in 1 Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in 1 Table 5.3.5-1 |  |
| CA_n66A-n260(2A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n66A-n260(3A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n66A-n260(4A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n66A-n260(5A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n66A-n260(6A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n66A-n260(7A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n66A-n260(8A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n66A-n260G | CA_n66A-n260A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260G |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260G |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n66A-n260H | CA_n66A-n260A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260H |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260H |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n66A-n260I | CA_n66A-n260A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260I |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260I |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n66A-n260J | CA_n66A-n260A/G/H/I/J | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260J |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260J |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n66A-n260K | CA_n66A-n260A/G/H/I/J/K | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260K |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260K |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n66A-n260L | CA_n66A-n260A/G/H/I/J/K/L | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260L |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260L |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n66A-n260M | CA_n66A-n260A/G/H/I/J/K/L/M | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260M |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260M |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n66A-n260R2 | CA_n66A-n260A/R2 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R2 |  |
| CA_n66A-n260R3 | CA_n66A-n260A/R2/R3 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R3 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R3 |  |
| CA_n66A-n260R4 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R4 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R4 |  |
| CA_n66A-n260R5 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R5 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R5 |  |
| CA_n66A-n260R6 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R6 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R6 |  |
| CA_n66A-n260R7 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R7 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R7 |  |
| CA_n66A-n260R8 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R8 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R8 |  |
| CA_n66A-n260R9 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R9 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R9 |  |
| CA_n66A-n260R10 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R10 |  |
| CA_n66(2A)-n260A | CA_n66A-n260A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n66(2A)-n260G | CA_n66A-n260A/G | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n66(2A)-n260H | CA_n66A-n260A/G/H | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n66(2A)-n260I | CA_n66A-n260A/G/H/I | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n66(2A)-n260J | CA_n66A-n260A/G/H/I/J | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n66(2A)-n260K | CA_n66A-n260A/G/H/I/J/K | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n66(2A)-n260L | CA_n66A-n260A/G/H/I/J/K/L | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n66(2A)-n260M | CA_n66A-n260A/G/H/I/J/K/L/M | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n66A-n261A | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n261(2A) | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n66A-n261(3A) | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(3A) |  |
| CA_n66A-n261(4A) | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(4A) |  |
| CA_n66A-n261G | CA_n66A-n261A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261G |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261G |  |
| CA_n66A-n261H | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261H |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261H |  |
| CA_n66A-n261I | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261I |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261I |  |
| CA_n66A-n261J | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261J |  |
|  | CA_n66A-n261A/G/H/I/J | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261J |  |
| CA_n66A-n261K | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261K |  |
|  | CA_n66A-n261A/G/H/I/J/K | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261K |  |
| CA_n66A-n261L | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261L |  |
|  | CA_n66A-n261A/G/H/I/J/K/L | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261L |  |
| CA_n66A-n261M | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261M |  |
|  | CA_n66A-n261A/G/H/I/J/K/L/M | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261M |  |
| CA_n66A-n261O | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261O |  |
| CA_n66A-n261P | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261P |  |
| CA_n66A-n261Q | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261Q |  |
| CA_n66A-n261(2G) | CA_n66A-n261A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2G) |  |
| CA_n66A-n261(2H) | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2H) |  |
| CA_n66A-n261(2I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2I) |  |
| CA_n66A-n261(A-G) | CA_n66A-n261A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-G) |  |
| CA_n66A-n261(A-H) | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-H) |  |
| CA_n66A-n261(A-I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-I) |  |
| CA_n66A-n261(A-J) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-J) |  |
| CA_n66A-n261(A-K) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-K) |  |
| CA_n66A-n261(A-L) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-L) |  |
| CA_n66A-n261(G-H) | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(G-H) |  |
| CA_n66A-n261(H-I) | CA_n66A-n261A/G/H/ | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(H-I) |  |
| CA_n66A-n261(G-I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(G-I) |  |
| CA_n66A-n261(A-G-H) | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-G-H) |  |
| CA_n66A-n261(A-G-I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-G-I) |  |
| CA_n66A-n261(2A-H) | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2A-H) |  |
| CA_n66A-n261(2A-G) | CA_n66A-n261A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2A-G) |  |
| CA_n66A-n261(2A-I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2A-I) |  |
| CA_n66A-n261(A-2G) | CA_n66A-n261A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-2G) |  |
| CA_n71A-n257A | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
|  | CA_n71A-n257A | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257A |  |
| CA_n71A-n257G | CA_n71A-n257A/G | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257G |  |
| CA_n71A-n257H | CA_n71A-n257A/G/H | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257H |  |
| CA_n71A-n257I | CA_n71A-n257A/G/H/I | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257I |  |
| CA_n71A-n260A | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n71A-n260(2A) | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n71A-n260(3A) | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n71A-n260(4A) | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n71A-n261A | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
| CA_n71A-n261(2A) | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(2A) |  |

Table 5.5A.1-1m: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n77A-n257A | CA_n77A-n257A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n77A-n257D | CA_n77A-n257A/D | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n77A-n257E | CA_n77A-n257A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257E |  |
| CA_n77A-n257F | CA_n77A-n257A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257F |  |
| CA_n77A-n257G | CA_n257G CA_n77A-n257A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n77A-n257H | CA_n257G/H CA_n77A-n257A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n77A-n257I | CA_n257G/H/I CA_n77A-n257A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n77A-n257J | CA_n257G/H/I/J CA_n77A-n257A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n77A-n257K | CA_n257G/H/I/J/K CA_n77A-n257A/G/H/I/J/K | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n77A-n257L | CA_n257G/H/I/J/K/L CA_n77A-n257A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n77A-n257M | CA_n257G/H/I/J/K/L/M CA_n77A-n257A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n77A-n257(2A) | CA_n77A-n257A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257(2A) |  |
| CA_n77A-n257(2G) | CA_n77A-n257A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257(2G) |  |
| CA_n77A-n257(A-G) | CA_n77A-n257A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257(A-G) |  |
| CA_n77C-n257A | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n77C-n257D | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n77C-n257E | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257E |  |
| CA_n77C-n257F | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257F |  |
| CA_n77C-n257G | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n77C-n257H | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n77C-n257I | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n77C-n257J | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n77C-n257K | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n77C-n257L | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n77C-n257M | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n77(2A)-n257A | CA_n77A-n257A | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n77(2A)-n257D | CA_n77A-n257A/D | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n77(2A)-n257G | CA_n77A-n257A/G | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n77(2A)-n257H | CA_n77A-n257A/G/H | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n77(2A)-n257I | CA_n77A-n257A/G/H/I | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n77(2A)-n257J | CA_n77A-n257A/G/H/I/J | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n77(2A)-n257K | CA_n77A-n257A/G/H/I/J/K | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n77(2A)-n257L | CA_n77A-n257A/G/H/I/J/K/L | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n77(2A)-n257M | CA_n77A-n257A/G/H/I/J/K/L/M | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n77(3A)-n257A | CA_n77A-n257A | n77 |  | CA_n77(3A) | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n77(3A)-n257D | CA_n77A-n257A/D | n77 |  | CA_n77(3A) | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n77(3A)-n257G | CA_n77A-n257A/G | n77 |  | CA_n77(3A) | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n77(3A)-n257H | CA_n77A-n257A/G/H | n77 |  | CA_n77(3A) | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n77(3A)-n257I | CA_n77A-n257A/G/H/I | n77 |  | CA_n77(3A) | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n77A-n258A | CA_n77A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n77A-n258D | CA_n77A-n258A/D | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258D |  |
| CA_n77A-n258G | CA_n77A-n258A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n77A-n258H | CA_n77A-n258A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n77A-n258I | CA_n77A-n258A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n77A-n258J | CA_n77A-n258A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n77(2A)-n258A | CA_n77A-n258A | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n77(2A)-n258D | CA_n77A-n258A/D | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258D |  |
| CA_n77(2A)-n258G | CA_n77A-n258A/G | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n77(2A)-n258H | CA_n77A-n258A/G/H | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n77(2A)-n258I | CA_n77A-n258A/G/H/I | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n77(2A)-n258J | CA_n77A-n258A/G/H/I/J | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n77(3A)-n258A | CA_n77A-n258A | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n77(3A)-n258D | CA_n77A-n258A/D | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | CA_n258D |  |
| CA_n77(3A)-n258G | CA_n77A-n258A/G | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n77(3A)-n258H | CA_n77A-n258A/G/H | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n77(3A)-n258I | CA_n77A-n258A/G/H/I | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n77(3A)-n258J | CA_n77A-n258A/G/H/I/J | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n77A-n258(2A) | CA_n77A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258(2A) |  |
| CA_n77A-n258(2G) | CA_n77A-n258A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258(2G) |  |
| CA_n77A-n258(3A) | CA_n77A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258(3A) |  |
| CA_n77A-n258(4A) | CA_n77A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258(4A) |  |
| CA_n77A-n258(5A) | CA_n77A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258(5A) |  |
| CA_n77A-n258(A-G) | CA_n77A-n258A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258(A-G) |  |
| CA_n77A-n259A | CA_n77A-n259A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n259 |  | 50, 100, 200, 400 |  |
| CA_n77A-n259G | CA_n259G CA_n77A-n259A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259G |  |
| CA_n77A-n259H | CA_n259G/H CA_n77A-n259A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259H |  |
| CA_n77A-n259I | CA_n259G/H/I CA_n77A-n259A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259I |  |
| CA_n77A-n259J | CA_n259G/H/I/J CA_n77A-n259A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259J |  |
| CA_n77A-n259K | CA_n259G/H/I/J/K CA_n77A-n259A/G/H/I/J/K | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259K |  |
| CA_n77A-n259L | CA_n259G/H/I/J/K/L CA_n77A-n259A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259L |  |
| CA_n77A-n259M | CA_n259G/H/I/J/K/L/M CA_n77A-n259A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259M |  |
| CA_n77A-n260A | CA_n77A-n260A | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | 50, 100, 200, 400 |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77A-n260G | CA_n77A-n260A/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260G |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260G |  |
| CA_n77A-n260H | CA_n77A-n260A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260H |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260H |  |
| CA_n77A-n260I | CA_n77A-n260A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260I |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260I |  |
| CA_n77A-n260J | CA_n77A-n260A/G/H/I/J | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260J |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260J |  |
| CA_n77A-n260K | CA_n77A-n260A/G/H/I/J/K | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260K |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260K |  |
| CA_n77A-n260L | CA_n77A-n260A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260L |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260L |  |
| CA_n77A-n260M | CA_n77A-n260A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260M |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260M |  |
| CA_n77A-n260R2 | CA_n77A-n260A/R2 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R2 |  |
| CA_n77A-n260R3 | CA_n77A-n260A/R2/R3 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R3 |  |
| CA_n77A-n260R4 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R4 |  |
| CA_n77A-n260R5 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R5 |  |
| CA_n77A-n260R6 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R6 |  |
| CA_n77A-n260R7 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R7 |  |
| CA_n77A-n260R8 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R8 |  |
| CA_n77A-n260R9 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R9 |  |
| CA_n77A-n260R10 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R10 |  |
| CA_n77C-n260A | CA_n77A-n260A | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | 50, 100, 200, 400 |  |
| CA_n77C-n260G | CA_n77A-n260A/G | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260G |  |
| CA_n77C-n260H | CA_n77A-n260A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260H |  |
| CA_n77C-n260I | CA_n77A-n260A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260I |  |
| CA_n77C-n260J | CA_n77A-n260A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260J |  |
| CA_n77C-n260K | CA_n77A-n260A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260K |  |
| CA_n77C-n260L | CA_n77A-n260A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260L |  |
| CA_n77C-n260M | CA_n77A-n260A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260M |  |
| CA_n77(2A)-n260A | CA_n77(2A) CA_n77A-n260A | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | 50, 100, 200, 400 |  |
| CA_n77(2A)-n260G | CA_n77(2A) CA_n77A-n260A/G | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260G |  |
| CA_n77(2A)-n260H | CA_n77(2A) CA_n77A-n260A/G/H | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260H |  |
| CA_n77(2A)-n260I | CA_n77(2A) CA_n77A-n260A/G/H/I | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260I |  |
| CA_n77(2A)-n260J | CA_n77(2A) CA_n77A-n260A/G/H/I/J | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260J |  |
| CA_n77(2A)-n260K | CA_n77(2A) CA_n77A-n260A/G/H/I/J/K | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260K |  |
| CA_n77(2A)-n260L | CA_n77(2A) CA_n77A-n260A/G/H/I/J/K/L | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260L |  |
| CA_n77(2A)-n260M | CA_n77(2A) CA_n77A-n260A/G/H/I/J/K/L/M | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260M |  |
| CA_n77A-n261A | CA_n77A-n261A | n77 |  | 10, 15, 20, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | 50, 100, 200, 400 |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 |  | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77A-n261D | CA_n77A-n261A/D | n77 |  | 10, 15, 20, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261D |  |
| CA_n77A-n261G | CA_n77A-n261A/G | n77 |  | 10, 15, 20, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261G |  |
| CA_n77A-n261H | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261H |  |
| CA_n77A-n261I | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261I |  |
| CA_n77A-n261J | CA_n77A-n261A/G/H/I/J | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261J |  |
| CA_n77A-n261K | CA_n77A-n261A/G/H/I/J/K | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261K |  |
| CA_n77A-n261L | CA_n77A-n261A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261L |  |
| CA_n77A-n261M | CA_n77A-n261A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261M |  |
| CA_n77A-n261(2A) | CA_n77A-n261A | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2A) |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 |  | CA_n261(2A) |  |
| CA_n77A-n261(2G) | CA_n77A-n261A/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2G) |  |
| CA_n77A-n261(2H) | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2H) |  |
| CA_n77A-n261(2I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2I) |  |
| CA_n77A-n261(3A) | CA_n77A-n261A | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(3A) |  |
| CA_n77A-n261(4A) | CA_n77A-n261A | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(4A) |  |
| CA_n77A-n261(A-G) | CA_n77A-n261A/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-G) |  |
| CA_n77A-n261(A-H) | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-H) |  |
| CA_n77A-n261(A-I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-I) |  |
| CA_n77A-n261(G-H) | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(G-H) |  |
| CA_n77A-n261(G-I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(G-I) |  |
| CA_n77A-n261(H-I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(H-I) |  |
| CA_n77A-n261(A-J) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-J) |  |
| CA_n77A-n261(A-K) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-K) |  |
| CA_n77A-n261(A-L) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-L) |  |
| CA_n77A-n261(A-G-H) | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-G-H) |  |
| CA_n77A-n261(A-G-I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-G-I) |  |
| CA_n77A-n261(2A-H) | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2A-H) |  |
| CA_n77A-n261(2A-G) | CA_n77A-n261A/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2A-G) |  |
| CA_n77A-n261(2A-I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2A-I) |  |
| CA_n77A-n261(A-2G) | CA_n77A-n261A/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-2G) |  |
| CA_n77C-n261A | CA_n77A-n261A | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | 50, 100, 200, 400 |  |
| CA_n77C-n261G | CA_n77A-n261A/G | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261G |  |
| CA_n77C-n261H | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261H |  |
| CA_n77C-n261I | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261I |  |
| CA_n77C-n261J | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261J |  |
| CA_n77C-n261K | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261K |  |
| CA_n77C-n261L | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261L |  |
| CA_n77C-n261M | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261M |  |
| CA_n77C-n261(G-H) | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(G-H) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(G-H) |  |
| CA_n77C-n261(2H) | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2H) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2H) |  |
| CA_n77C-n261(G-I) | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(G-I) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(G-I) |  |
| CA_n77C-n261(A-G-H) | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-G-H) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-G-H) |  |
| CA_n77C-n261(H-I) | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(H-I) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(H-I) |  |
| CA_n77C-n261(2A-G) | CA_n77A-n261A/G | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2A-G) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2A-G) |  |
| CA_n77C-n261(2A-H) | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2A-H) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2A-H) |  |
| CA_n77C-n261(2A-I) | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2A-I) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2A-I) |  |
| CA_n77C-n261(2A) | CA_n77A-n261A | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2A) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2A) |  |
| CA_n77C-n261(3A) | CA_n77A-n261A | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(3A) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(3A) |  |
| CA_n77C-n261(2G) | CA_n77A-n261A/G | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2G) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2G) |  |
| CA_n77C-n261(A-2G) | CA_n77A-n261A/G | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-2G) |  |
|  |  | n48 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-2G) |  |
| CA_n77C-n261(A-G) | CA_n77A-n261A/G | n48 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-G) |  |
|  |  | n48 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-G) |  |
| CA_n77C-n261(A-H) | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-H) |  |
|  |  | n48 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-H) |  |
| CA_n77C-n261(A-I) | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-I) |  |
|  |  | n48 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-I) |  |
| CA_n77C-n261(A-G-I) | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-G-I) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-G-I) |  |
| CA_n77(2A)-n257E | CA_n77A-n257A | n77 | CA_n77(2A) |  | 0 |
|  |  | n257 | CA_n257E |  |  |
| CA_n77(2A)-n257F | CA_n77A-n257A | n77 | CA_n77(2A) |  | 0 |
|  |  | n257 | CA_n257F |  |  |

Table 5.5A.1-1n: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n78A-n257A | CA_n78A-n257A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n78A-n257D | CA_n78A-n257A/D | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n78A-n257D | CA_n78A-n257A CA_n78A-n257D | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n78A-n257E | CA_n78A-n257A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n78A-n257F | CA_n78A-n257A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n78C-n257A | CA_n78A-n257A | n78 |  | CA_n78C | 0 |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n78C-n257D | CA_n78A-n257A | n78 |  | CA_n78C | 0 |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n78C-n257E | CA_n78A-n257A | n78 |  | CA_n78C | 0 |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n78C-n257F | CA_n78A-n257A | n78 |  | CA_n78C | 0 |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n78C-n257G | CA_n78A-n257A/G | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257G |  |  |
| CA_n78C-n257H | CA_n78A-n257A/G/H | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257H |  |  |
| CA_n78C-n257I | CA_n78A-n257A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257I |  |  |
| CA_n78C-n257J | CA_n78A-n257A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257J |  |  |
| CA_n78C-n257K | CA_n78A-n257A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257K |  |  |
| CA_n78C-n257L | CA_n78A-n257A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257L |  |  |
| CA_n78C-n257M | CA_n78A-n257A/G/H/M | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257M |  |  |
| CA_n78A-n257G | CA_n257G CA_n78A-n257A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257G |  |  |
| CA_n78A-n257H | CA_n257G/H CA_n78A-n257A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257H |  |  |
| CA_n78A-n257I | CA_n257G/H/I CA_n78A-n257A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257I |  |  |
| CA_n78A-n257J | CA_n257G/H/I/J CA_n78A-n257A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257J |  |  |
| CA_n78A-n257K | CA_n257G/H/I/J/K CA_n78A-n257A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257K |  |  |
| CA_n78A-n257L | CA_n257G/H/I CA_n78A-n257A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257L |  |  |
| CA_n78A-n257M | CA_n257G/H/I CA_n78A-n257A/G/H/I | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257M |  |  |
| CA_n78A-n257(2A) | CA_n78A-n257A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257(2A) |  |  |
| CA_n78A-n257(A-G) | CA_n78A-n257A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257(A-G) |  |  |
| CA_n78A-n257(2G) | CA_n78A-n257A/G/(2G) | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257(2G) |  |  |
| CA_n78(2A)-n257A | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n78(2A)-n257D | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257D |  |  |
| CA_n78(2A)-n257E | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257E |  |  |
| CA_n78(2A)-n257F | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257F |  |  |
| CA_n78(2A)-n257G | CA_n78A-n257A/G | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257G |  |  |
| CA_n78(2A)-n257H | CA_n78A-n257A/G/H | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257H |  |  |
| CA_n78(2A)-n257I | CA_n78A-n257A/G/H/I | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257I |  |  |
| CA_n78(2A)-n257J | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257J |  |  |
| CA_n78(2A)-n257K | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257K |  |  |
| CA_n78(2A)-n257L | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257L |  |  |
| CA_n78(2A)-n257M | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257M |  |  |
| CA_n78A-n258A | CA_n78A-n258A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n78A-n258B | CA_n78A-n258A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258B |  |  |
| CA_n78A-n258C | CA_n78A-n258A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258C |  |  |
| CA_n78A-n258D | CA_n78A-n258A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258D |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258D |  |  |
| CA_n78A-n258E | CA_n78A-n258A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258E |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258E |  |  |
| CA_n78A-n258F | CA_n78A-n258A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258F |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258F |  |  |
| CA_n78A-n258G | CA_n78A-n258A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258G |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258G |  |  |
| CA_n78A-n258H | CA_n78A-n258A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n258 | CA_n258H |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258H |  |  |
| CA_n78A-n258I | CA_n78A-n258A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258I |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258I |  |  |
| CA_n78A-n258J | CA_n78A-n258A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258J |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258J |  |  |
| CA_n78A-n258K | CA_n78A-n258A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258K |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258K |  |  |
| CA_n78A-n258L | CA_n78A-n258A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258L |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258L |  |  |
| CA_n78A-n258M | CA_n78A-n258A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258M |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258M |  |  |
| CA_n78A-n258R2 | CA_n78A-n258A/R2 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n78A-n258R3 | CA_n78A-n258A/R2/R3 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n78A-n258R4 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n78A-n258R5 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n78A-n258R6 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n78A-n258R7 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n78A-n258R8 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n78A-n258R9 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n78A-n258R10 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n78A-n258(2A) | CA_n78A-n258A/(2A) | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258(2A) |  |  |
| CA_n78A-n258(2G) | CA_n78A-n258A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258(2G) |  |  |
| CA_n78A-n258(A-G) | CA_n78A-n258A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258(A-G) |  |  |
| CA_n78B-n258A | CA_n78A-n258A | n78 | CA_n78B |  | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n78B-n258B | CA_n78A-n258A | n78 | CA_n78B |  | 0 |
|  |  | n258 | CA_n258B |  |  |
| CA_n78C-n258A | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n78C-n258B | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258B |  |  |
| CA_n78C-n258C | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258C |  |  |
| CA_n78C-n258D | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258D |  |  |
| CA_n78C-n258E | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258E |  |  |
| CA_n78C-n258F | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258F |  |  |
| CA_n78C-n258G | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258G |  |  |
| CA_n78C-n258H | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258H |  |  |
| CA_n78C-n258I | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258I |  |  |
| CA_n78C-n258J | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258J |  |  |
| CA_n78C-n258K | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258K |  |  |
| CA_n78C-n258L | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258L |  |  |
| CA_n78C-n258M | CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258M |  |  |
| CA_n78A-n259A | CA_n78A-n259A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n259 | 50, 100, 200, 400 |  |  |
| CA_n78A-n259G | CA_n259G CA_n78A-n259A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259G |  |  |
| CA_n78A-n259H | CA_n259G/H CA_n78A-n259A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259H |  |  |
| CA_n78A-n259I | CA_n259G/H/I CA_n78A-n259A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259I |  |  |
| CA_n78A-n259J | CA_n259G/H/I/J CA_n78A-n259A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259J |  |  |
| CA_n78A-n259K | CA_n259G/H/I/J/K CA_n78A-n259A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259K |  |  |
| CA_n78A-n259L | CA_n259G/H/I/J/K/L CA_n78A-n259A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259L |  |  |
| CA_n78A-n259M | CA_n259G/H/I/J/K/L/M CA_n78A-n259A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259M |  |  |

Table 5.5A.1-1o: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n79A-n257A | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n79A-n257D | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n79A-n257E | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257E |  |
| CA_n79A-n257F | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257F |  |
| CA_n79A-n257G | CA_n257G CA_n79A-n257A/G | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n79A-n257H | CA_n257G/H CA_n79A-n257A/G/H | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n79A-n257I | CA_n257G/H/I CA_n79A-n257A/G/H/I | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n79A-n257J | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n79A-n257K | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n79A-n257L | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n79A-n257M | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n79C-n257A | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n79C-n257D | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n79C-n257E | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257E |  |
| CA_n79C-n257F | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257F |  |
| CA_n79C-n257G | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n79C-n257H | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n79C-n257I | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n79C-n257J | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n79C-n257K | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n79C-n257L | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n79C-n257M | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n79A-n258A | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n79A-n258B | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258B |  |
| CA_n79A-n258C | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258C |  |
| CA_n79A-n258D | CA_n79A-n258A/D | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258D |  |
| CA_n79A-n258E | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258E |  |
| CA_n79A-n258F | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258F |  |
| CA_n79A-n258G | CA_n79A-n258A/G | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n79A-n258H | CA_n79A-n258A/G/H | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n79A-n258I | CA_n79A-n258A/G/H/I | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n79A-n258J | CA_n79A-n258A/G/H/I/J | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n79A-n258D | CA_n79A-n258A CA_n79A-n258D | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258D |  |
| CA_n79A-n258E | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258E |  |
| CA_n79A-n258F | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258F |  |
| CA_n79A-n258G | CA_n79A-n258A CA_n79A-n258G | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n79A-n258H | CA_n79A-n258A CA_n79A-n258G CA_n79A-n258H | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n79A-n258I | CA_n79A-n258A CA_n79A-n258G CA_n79A-n258H CA_n79A-n258I | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n79A-n258J | CA_n79A-n258A CA_n79A-n258G CA_n79A-n258H CA_n79A-n258I CA_n79A-n258J | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n79A-n258K | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258K |  |
| CA_n79A-n258L | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258L |  |
| CA_n79A-n258M | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258M |  |
| CA_n79C-n258A | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n79C-n258G | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n79C-n258H | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n79C-n258I | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n79C-n258J | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n79C-n258K | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258K |  |
| CA_n79C-n258L | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258L |  |
| CA_n79C-n258M | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258M |  |
| CA_n79A-n259A | CA_n79A-n259A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | 50, 100, 200, 400 |  |
| CA_n79A-n259G | CA_n259G CA_n79A-n259A CA_n79A-n259G | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259G |  |
| CA_n79A-n259G | CA_n259G CA_n79A-n259A/G | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259G |  |  |
| CA_n79A-n259H | CA_n259G/H CA_n79A-n259A/G/H | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259H |  |  |
| CA_n79A-n259I | CA_n259G/H/I CA_n79A-n259A/G/H/I | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259I |  |  |
| CA_n79A-n259J | CA_n259G/H/I/J CA_n79A-n259A/G/H/I/J | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259J |  |  |
| CA_n79A-n259K | CA_n259G/H/I/J/K CA_n79A-n259A/G/H/I/J/K | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259K |  |  |
| CA_n79A-n259L | CA_n259G/H/I/J/K/L CA_n79A-n259A/G/H/I/J/K/L | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259L |  |  |
| CA_n79A-n259M | CA_n259G/H/I/J/K/L/M CA_n79A-n259A/G/H/I/J/K/L/M | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259M |  |  |

Table 5.5A.1-1p: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n105A-n257A | CA_n105A-n257A | n105 | 5, 10,15, 20, 25, 30, 35 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n105A-n258A | CA_n105A-n258A | n105 | 5, 10,15, 20, 25, 30, 35 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |

The following notes are applied to the above tables:

NOTE 1:	This UE channel bandwidth is optional in this release of the specification. (From Table 5.3.5-1 of 38.101-1)

NOTE 2:	The CA configurations are given in Table 5.5A.1-1 of either TS 38.101-1 or TS 38.101-2 where unless otherwise stated BCS0 is referred to.

NOTE 3: 	The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively.

NOTE 4:	This UE channel bandwidth is optional in this release of the specification.

NOTE 5:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a SCell part of DC or CA configuration (In Table 5.3.5-1 in 38.101-1).

NOTE 6: 	The delimiter “/” is only used in the uplink configurations for the sake of simplicity. For example, CA_nxA-nyA/B/C denotes CA_nxA-nyA, CA_nxA-nyB and CA_nxA-nyC, where nx and ny are two NR bands, ny is a FR2 band and A, B and C are the corresponding bandwidth classes respectively.

Table 5.5A.1-2: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (three bands)

| NR CA configuration |  | Uplink configuration |  | NR Band | Channel bandwidth (MHz) (NOTE 1) |  | Bandwidth combination set |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n257A |  | CA_n1A-n3A CA_n1A-n257A CA_n3A-n257A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n3A-n257G |  | CA_n1A-n3A CA_n1A-n257A/G CA_n3A-n257A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n1A-n3A-n257H |  | CA_n1A-n3A CA_n1A-n257A/G/H CA_n3A-n257A/G/H CA_n3A-n257G |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n1A-n3A-n257I |  | CA_n1A-n3A CA_n1A-n257A/G/H/I CA_n3A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n1A-n3A-n257J |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n1A-n3A-n257K |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n1A-n3A-n257L |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n1A-n3A-n257M |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n1A-n3A-n258A |  | CA_n1A-n3A CA_n1A-n258A CA_n3A-n258A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n3A-n258D |  | CA_n1A-n3A CA_n1A-n258A CA_n3A-n258A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n1A-n3A-n258G |  | CA_n1A-n3A CA_n1A-n258A/G CA_n3A-n258A/G |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n258 | CA_n258G |  |  |  |  |
| CA_n1A-n3A-n258H |  | CA_n1A-n3A CA_n1A-n258A/G/H CA_n3A-n258A/G/H |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n258 | CA_n258H |  |  |  |  |
| CA_n1A-n3A-n258I |  | CA_n1A-n3A CA_n1A-n258A/G/H/I CA_n3A-n258A/G/H/I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n258 | CA_n258I |  |  |  |  |
| CA_n1A-n3A-n258J |  | CA_n1A-n3A CA_n1A-n258A/G/H/I CA_n3A-n258A/G/H/I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |  |
|  |  |  |  | n258 | CA_n258J |  |  |  |  |
| CA_n1A-n8A-n257A |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n8A-n257D |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n1A-n8A-n257E |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257E |  |  |  |  |
| CA_n1A-n8A-n257F |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257F |  |  |  |  |
| CA_n1A-n8A-n257G |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n1A-n8A-n257H |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n1A-n8A-n257I |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n1A-n8A-n257J |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n1A-n8A-n257K |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n1A-n8A-n257L |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n1A-n8A-n257M |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n1A-n18A-n257A |  | CA_n1A-n18A CA_n1A-n257A CA_n18A-n257A |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n18 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n18A-n257G |  | CA_n1A-n18A CA_n1A-n257A CA_n1A-n257G CA_n18A-n257A CA_n18A-n257G |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n18 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n1A-n18A-n257H |  | CA_n1A-n18A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n18 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n1A-n18A-n257I |  | CA_n1A-n18A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n18A-n257I |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n18 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n1A-n28A-n257A |  | CA_n1A-n28A CA_n1A-n257A CA_n28A-n257A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n28A-n257G |  | CA_n257G CA_n1A-n28A CA_n1A-n257A/G CA_n28A-n257A/G |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n1A-n28A-n257H |  | CA_n257G/H CA_n1A-n28A CA_n1A-n257A/G/H CA_n28A-n257A/G/H |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n1A-n28A-n257I |  | CA_n257G/H/I CA_n1A-n28A CA_n1A-n257A/G/H/I CA_n28A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n1A-n28A-n258A |  | CA_n1A-n28A CA_n1A-n258A CA_n28A-n258A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n28A-n258D |  | CA_n1A-n28A CA_n1A-n258A CA_n28A-n258A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n1A-n28A-n258G |  | CA_n1A-n28A CA_n1A-n258A/G CA_n28A-n258A/G |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | CA_n258G |  |  |  |  |
| CA_n1A-n28A-n258H |  | CA_n1A-n28A CA_n1A-n258A/G/H CA_n28A-n258A/G/H CA_n1A-n258H CA_n28A-n258H |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | CA_n258H |  |  |  |  |
| CA_n1A-n28A-n258I |  | CA_n1A-n28A CA_n1A-n258A/G/H/I CA_n28A-n258A/G/H CA_n1A-n258H/I CA_n28A-n258I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | CA_n258I |  |  |  |  |
| CA_n1A-n28A-n258J |  | CA_n1A-n28A CA_n1A-n258A/G/H CA_n28A-n258A/G CA_n1A-n258H CA_n28A-n258H CA_n1A-n258I CA_n28A-n258I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | CA_n258J |  |  |  |  |
| CA_n1A-n40A-n258A |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n40A-n258D |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n1A-n40A-n258E |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50,60 |  |  |  |  |
|  |  |  |  | n258 | CA_n258E |  |  |  |  |
| CA_n1A-n40A-n258F |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |  |  |
|  |  |  |  | n258 | CA_n258F |  |  |  |  |
| CA_n1A-n40A-n258G |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |  |  |
|  |  |  |  | n258 | CA_n258G |  |  |  |  |
| CA_n1A-n40A-n258H |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |  |  |
|  |  |  |  | n258 | CA_n258H |  |  |  |  |
| CA_n1A-n40A-n258I |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |  |  |
|  |  |  |  | n258 | CA_n258I |  |  |  |  |
| CA_n1A-n40A-n258J |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |  |  |
|  |  |  |  | n258 | CA_n258J |  |  |  |  |
| CA_n1A-n40A-n258K |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |  |  |
|  |  |  |  | n258 | CA_n258K |  |  |  |  |
| CA_n1A-n40A-n258L |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |  |  |
|  |  |  |  | n258 | CA_n258L |  |  |  |  |
| CA_n1A-n40A-n258M |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |  |  |
|  |  |  |  | n258 | CA_n258M |  |  |  |  |
| CA_n1A-n41A-n257A |  | CA_n1A-n41A CA_n1A-n257A CA_n41A-n257A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n41A-n257G |  | CA_n257G CA_n1A-n41A CA_n1A-n257A/G CA_n41A-n257A/G |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n1A-n41A-n257H |  | CA_n257G/H CA_n1A-n41A CA_n1A-n257A/G/H CA_n41A-n257A/G/H |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n1A-n41A-n257I |  | CA_n257G/H/I CA_n1A-n41A CA_n1A-n257A/G/H/I CA_n41A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n1A-n77A-n257A |  | CA_n1A-n77A CA_n1A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n77A-n257G |  | CA_n257G CA_n1A-n77A CA_n1A-n257A/G CA_n77A-n257A/G |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n1A-n77A-n257H |  | CA_n257G/H CA_n1A-n77A CA_n1A-n257A/G/H CA_n77A-n257A/G/H |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n1A-n77A-n257I |  | CA_n257G/H/I CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n1A-n77A-n257J |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n1A-n77A-n257K |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n1A-n77A-n257L |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n1A-n77A-n257M |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n1A-n77(2A)-n257A |  | CA_n1A-n77A CA_n1A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n77(2A)-n257G |  |  | CA_n1A-n77A |  | n1 |  |  | 5, 10, 15, 20 |  |  | 0 |  |  |
|  |  |  | CA_n1A-n257A/G |  | n77 |  |  | CA_n77(2A) |  |  |  |  |  |
|  |  |  | CA_n1A-n257G |  | n257 |  |  | CA_n257G |  |  |  |  |  |
| CA_n1A-n77(2A)-n257H |  |  | CA_n77A-n257A/G |  | n1 |  |  | 5, 10, 15, 20 |  |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n1A-n77(2A)-n257I |  | CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n1A-n77(2A)-n257J |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n1A-n77(2A)-n257K |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n1A-n77(2A)-n257L |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n1A-n77(2A)-n257M |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n1A-n77(3A)-n257A |  | CA_n1A-n77A CA_n1A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(3A) |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n77(3A)-n257G |  |  | CA_n1A-n77A |  | n1 |  |  | 5, 10, 15, 20 |  |  | 0 |  |  |
|  |  |  | CA_n1A-n257A/G |  | n77 |  |  | CA_n77(3A) |  |  |  |  |  |
|  |  |  | CA_n1A-n257G |  | n257 |  |  | CA_n257G |  |  |  |  |  |
| CA_n1A-n77(3A)-n257H |  |  | CA_n77A-n257A/G |  | n1 |  |  | 5, 10, 15, 20 |  |  | 0 |  |  |
|  |  |  | CA_n77A-n257G- |  | n77 |  |  | CA_n77(3A) |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | CA_n257H |  |  |  |  |  |
| CA_n1A-n77(3A)-n257I |  |  |  |  | n1 |  |  | 5, 10, 15, 20 |  |  | 0 |  |  |
|  |  |  | CA_n1A-n77A |  | n77 |  |  | CA_n77(3A) |  |  |  |  |  |
|  |  |  | CA_n1A-n257A/G/H |  | n257 |  |  | CA_n257I |  |  |  |  |  |
| CA_n1A-n78A-n257A |  | CA_n1A-n78A CA_n1A-n257A CA_n78A-n257A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n78A-n257D |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n1A-n78A-n257E |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257E |  |  |  |  |
| CA_n1A-n78A-n257F |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257F |  |  |  |  |
| CA_n1A-n78A-n257G |  | CA_n257G CA_n1A-n78A CA_n1A-n257A/G CA_n78A-n257A/G |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n1A-n78A-n257H |  | CA_n257G/H CA_n257H CA_n1A-n257A/G/H CA_n78A-n257A/G/H |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n1A-n78A-n257I |  | CA_n257G/H/I CA_n1A-n78A CA_n1A-n257A/G/H/I CA_n78A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n1A-n78A-n257J |  | CA_n257G/H/I/J CA_n1A-n78A CA_n1A-n257A/G/H/I/J CA_n78A-n257A/G/H/I/J |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n1A-n78A-n257K |  | CA_n257G/H/I/J/K CA_n1A-n78A CA_n1A-n257A/G/H/I/J/K CA_n78A-n257A/G/H/I/J/K |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n1A-n78A-n257L |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n1A-n78A-n257M |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n1A-n78A-n258A |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n78A-n258D |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n1A-n78A-n258E |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258E |  |  |  |  |
| CA_n1A-n78A-n258F |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258F |  |  |  |  |
| CA_n1A-n78A-n258G |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258G |  |  |  |  |
| CA_n1A-n78A-n258H |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258H |  |  |  |  |
| CA_n1A-n78A-n258I |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258I |  |  |  |  |
| CA_n1A-n78A-n258J |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258J |  |  |  |  |
| CA_n1A-n78A-n258K |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258K |  |  |  |  |
| CA_n1A-n78A-n258L |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258L |  |  |  |  |
| CA_n1A-n78A-n258M |  | - |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258M |  |  |  |  |
| CA_n1A-n79A-n257A |  | CA_n1A-n79A CA_n1A-n257A CA_n79A-n257A |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n1A-n79A-n257G |  | CA_n257G CA_n1A-n79A CA_n1A-n257A/G CA_n79A-n257A/G |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n1A-n79A-n257H |  | CA_n257G/H CA_n1A-n79A CA_n1A-n257A/G/H CA_n79A-n257A/G/H |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n1A-n79A-n257I |  | CA_n257G/H/I CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n2A-n5A-n260A |  | CA_n2A-n5A CA_n2A-n260A CA_n5A-n260A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n5A-n260G |  | CA_n2A-n5A CA_n2A-n260A/G CA_n5A-n260A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n2A-n5A-n260H |  | CA_n2A-n5A CA_n2A-n260A/G/H CA_n5A-n260A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n2A-n5A-n260I |  | CA_n2A-n5A CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n2A-n5A-n260J |  | CA_n2A-n5A CA_n2A-n260A/G/H/I/J CA_n5A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n2A-n5A-n260K |  | CA_n2A-n5A CA_n2A-n260A/G/H/I/J/K CA_n5A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n2A-n5A-n260L |  | CA_n2A-n5A CA_n2A-n260A/G/H/I/J/K/L CA_n5A-n260A/G/H/I/J/K/L |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n2A-n5A-n260M |  | CA_n2A-n5A CA_n2A-n260A/G/H/I/J/K/L/M CA_n5A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n2A-n5A-n261A |  | CA_n2A-n5A CA_n2A-n261A CA_n5A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n5A-n261G |  | CA_n2A-n5A CA_n2A-n261A/G CA_n5A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n2A-n5A-n261H |  | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n2A-n5A-n261I |  | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n2A-n5A-n261J |  | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n2A-n5A-n261K |  | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n2A-n5A-n261L |  | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n2A-n5A-n261M |  | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n2A-n5A-n261(2G) |  | CA_n2A-n5A CA_n2A-n261A/G CA_n5A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n2A-n5A-n261(G-H) |  | CA_n2A-n5A CA_n2A-n261A CA_n2A-n261G CA_n2A-n261H CA_n5A-n261A CA_n5A-n261G CA_n5A-n261H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n2A-n5A-n261(A-G-H) |  | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n2A-n5A-n261(G-I) |  | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n2A-n5A-n261(2H) |  | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n2A-n5A-n261(A-G-I) |  | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n2A-n5A-n261(H-I) |  | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n2A-n5A-n261(2A-G) |  | CA_n2A-n5A CA_n2A-n261A/G CA_n5A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-G) |  |  |  |  |
| CA_n2A-n5A-n261(2A-H) |  | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n2A-n5A-n261(2A-I) |  | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n2A-n5A-n261(2A) |  | CA_n2A-n5A CA_n2A-n261A CA_n5A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A) |  |  |  |  |
| CA_n2A-n5A-n261(3A) |  | CA_n2A-n5A CA_n2A-n261A CA_n5A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(3A) |  |  |  |  |
| CA_n2A-n5A-n261(A-G) |  | CA_n2A-n5A CA_n2A-n261A/G CA_n5A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G) |  |  |  |  |
| CA_n2A-n5A-n261(A-2G) |  | CA_n2A-n5A CA_n2A-n261A/G CA_n5A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n2A-n5A-n261(A-H) |  | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n2A-n5A-n261(A-I) |  | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-I) |  |  |  |  |
| CA_n2A-n12A-n260A |  | CA_n2A-n12A CA_n2A-n260A CA_n12A-n260A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n12 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n12A-n260G |  | CA_n2A-n12A CA_n2A-n260A/G CA_n12A-n260A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n12 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n2A-n12A-n260H |  | CA_n2A-n12A CA_n2A-n260A/G/H CA_n12A-n260A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n12 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n2A-n12A-n260I |  | CA_n2A-n12A CA_n2A-n260A/G/H/I CA_n12A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n12 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n2A-n12A-n260J |  | CA_n2A-n12A CA_n2A-n260A/G/H/I/J CA_n12A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n12 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n2A-n12A-n260K |  | CA_n2A-n12A CA_n2A-n260A/G/H/I/J/K CA_n12A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n12 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n2A-n12A-n260L |  |  |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n12 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n2A-n12A-n260M |  | CA_n2A-n12A CA_n2A-n260A/G/H/I/J/K/L/M CA_n12A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n12 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n2A-n14A-n260A |  | CA_n2A-n14A CA_n2A-n260A CA_n14A-n260A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n14 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n14A-n260G |  | CA_n2A-n14A CA_n2A-n260A/G CA_n14A-n260A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n14 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n2A-n14A-n260H |  | CA_n2A-n14A CA_n2A-n260A/G/H CA_n14A-n260A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n14 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n2A-n14A-n260I |  | CA_n2A-n14A CA_n2A-n260A/G/H/I CA_n14A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n14 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n2A-n14A-n260J |  | CA_n2A-n14A CA_n2A-n260A/G/H/I/J CA_n14A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n14 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n2A-n14A-n260K |  | CA_n2A-n14A CA_n2A-n260A/G/H/I/J/K CA_n14A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n14 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n2A-n14A-n260L |  | CA_n2A-n14A CA_n2A-n260A/G/H/I/J/K/L CA_n14A-n260A/G/H/I/J/K/L |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n14 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n2A-n14A-n260M |  | CA_n2A-n14A CA_n2A-n260A/G/H/I/J/K/L/M CA_n14A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n14 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n2A-n30A-n260A |  | CA_n2A-n30A CA_n2A-n260A CA_n30A-n260A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n30A-n260G |  | CA_n2A-n30A CA_n2A-n260A/G CA_n30A-n260A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n2A-n30A-n260H |  | CA_n2A-n30A CA_n2A-n260A/G/H CA_n30A-n260A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n2A-n30A-n260I |  | CA_n2A-n30A CA_n2A-n260A/G/H/I CA_n30A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n2A-n30A-n260J |  | CA_n2A-n30A CA_n2A-n260A/G/H/I/J CA_n30A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n2A-n30A-n260K |  | CA_n2A-n30A CA_n2A-n260A/G/H/I/J/K CA_n30A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n2A-n30A-n260L |  | CA_n2A-n30A CA_n2A-n260A/G/H/I/J/K/L CA_n30A-n260A/G/H/I/J/K/L |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n2A-n30A-n260M |  | CA_n2A-n30A CA_n2A-n260A/G/H/I/J/K/L/M CA_n30A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n2A-n48A-n260A |  | CA_n2A-n260A CA_n48A-n260A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n48A-n260G |  | CA_n2A-n260A/G CA_n48A-n260A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n2A-n48A-n260H |  | CA_n2A-n260A/G/H CA_n48A-n260A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n2A-n48A-n260I |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n2A-n48A-n260J |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n2A-n48A-n260K |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n2A-n48A-n260L |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n2A-n48A-n260M |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n2A-n48(2A)-n260A |  | CA_n2A-n260A CA_n48A-n260A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n48(2A)-n260G |  | CA_n2A-n260A/G CA_n48A-n260A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n2A-n48(2A)-n260H |  | CA_n2A-n260A/G/H CA_n48A-n260A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n2A-n48(2A)-n260I |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n2A-n48(2A)-n260J |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n2A-n48(2A)-n260K |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n2A-n48(2A)-n260L |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n2A-n48(2A)-n260M |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n2A-n48B-n260A |  | CA_n2A-n260A CA_n48A-n260A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n48B-n260G |  | CA_n2A-n260A/G CA_n48A-n260A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n2A-n48B-n260H |  | CA_n2A-n260A/G/H CA_n48A-n260A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n2A-n48B-n260I |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n2A-n48B-n260J |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n2A-n48B-n260K |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n2A-n48B-n260L |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n2A-n48B-n260M |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n2A-n48A-n261A |  | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n48A-n261G |  | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n2A-n48A-n261H |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n2A-n48A-n261I |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n2A-n48A-n261J |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n2A-n48A-n261K |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n2A-n48A-n261L |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n2A-n48A-n261M |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n2A-n48A-n261(A-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n2A-n48A-n261(G-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n2A-n48A-n261(2A-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n2A-n48A-n261(A-2G) |  | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n2A-n48A-n261(A-G-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n2A-n48A-n261(2G) |  | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n2A-n48A-n261(2H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n2A-n48A-n261(G-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n2A-n48A-n261(H-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n2A-n48A-n261(2A-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n2A-n48A-n261(A-G-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n2A-n48(2A)-n261A |  | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n48(2A)-n261G |  | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n2A-n48(2A)-n261H |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n2A-n48(2A)-n261I |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n2A-n48(2A)-n261J |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n2A-n48(2A)-n261K |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n2A-n48(2A)-n261L |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n2A-n48(2A)-n261M |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n2A-n48(2A)-n261(A-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n2A-n48(2A)-n261(G-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n2A-n48(2A)-n261(2A-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n2A-n48(2A)-n261(A-2G) |  | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n2A-n48(2A)-n261(A-G-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n2A-n48(2A)-n261(2G) |  | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n2A-n48(2A)-n261(2H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n2A-n48(2A)-n261(G-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n2A-n48(2A)-n261(H-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n2A-n48(2A)-n261(2A-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n2A-n48(2A)-n261(A-G-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n2A-n48B-n261A |  | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n48B-n261G |  | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n2A-n48B-n261H |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n2A-n48B-n261I |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n2A-n48B-n261J |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n2A-n48B-n261K |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n2A-n48B-n261L |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n2A-n48B-n261M |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n2A-n48B-n261(A-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n2A-n48B-n261(G-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n2A-n48B-n261(2A-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n2A-n48B-n261(A-2G) |  | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n2A-n48B-n261(A-G-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n2A-n48B-n261(2G) |  | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n2A-n48B-n261(2H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n2A-n48B-n261(G-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n2A-n48B-n261(H-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n2A-n48B-n261(2A-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n2A-n48B-n261(A-G-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n2A-n66A-n260A |  | CA_n2A-n66A CA_n2A-n260A CA_n66A-n260A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n66A-n260G |  | CA_n2A-n66A CA_n2A-n260A/G CA_n66A-n260A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n2A-n66A-n260H |  | CA_n2A-n66A CA_n2A-n260A/G/H CA_n66A-n260A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n2A-n66A-n260I |  | CA_n2A-n66A CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n2A-n66A-n260J |  | CA_n2A-n66A CA_n2A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n2A-n66A-n260K |  | CA_n2A-n66A CA_n2A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n2A-n66A-n260L |  | CA_n2A-n66A CA_n2A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n2A-n66A-n260M |  | CA_n2A-n66A CA_n2A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n2A-n66A-n261A |  | CA_n2A-n66A CA_n2A-n261A CA_n66A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n66A-n261G |  | CA_n2A-n66A CA_n2A-n261A/G CA_n66A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n2A-n66A-n261H |  | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n2A-n66A-n261I |  | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n2A-n66A-n261J |  | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n2A-n66A-n261K |  | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n2A-n66A-n261L |  | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n2A-n66A-n261M |  | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n2A-n66A-n261(2G) |  | CA_n2A-n66A CA_n2A-n261A/G CA_n66A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n2A-n66A-n261(G-H) |  | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n2A-n66A-n261(A-G-H) |  | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n2A-n66A-n261(G-I) |  | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n2A-n66A-n261(2H) |  | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n2A-n66A-n261(A-G-I) |  | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n2A-n66A-n261(H-I) |  | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n2A-n66A-n261(2A-G) |  | CA_n2A-n66A CA_n2A-n261A/G CA_n66A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-G) |  |  |  |  |
| CA_n2A-n66A-n261(2A-H) |  | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n2A-n66A-n261(2A-I) |  | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n2A-n66A-n261(2A) |  | CA_n2A-n66A CA_n2A-n261A CA_n66A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A) |  |  |  |  |
| CA_n2A-n66A-n261(3A) |  | CA_n2A-n66A CA_n2A-n261A CA_n66A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(3A) |  |  |  |  |
| CA_n2A-n66A-n261(A-2G) |  | CA_n2A-n66A CA_n2A-n261A/G CA_n66A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n2A-n66A-n261(A-G) |  | CA_n2A-n66A CA_n2A-n261A/G CA_n66A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G) |  |  |  |  |
| CA_n2A-n66A-n261(A-H) |  | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n2A-n66A-n261(A-I) |  | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-I) |  |  |  |  |
| CA_n2A-n77A-n260A |  | CA_n2A-n77A CA_n77A-n260A CA_n2A-n260A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n77A-n260G |  | CA_n2A-n77A CA_n2A-n260A/G CA_n77A-n260A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n2A-n77A-n260H |  | CA_n2A-n77A CA_n2A-n260A/G/H CA_n77A-n260A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n2A-n77A-n260I |  | CA_n2A-n77A CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n2A-n77A-n260J |  | CA_n2A-n77A CA_n2A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n2A-n77A-n260K |  | CA_n2A-n77A CA_n2A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n2A-n77A-n260L |  | CA_n2A-n77A CA_n2A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n2A-n77A-n260M |  | CA_n2A-n77A CA_n2A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n2A-n77C-n260A |  | CA_n2A-n260A CA_n77A-n260A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260A |  |  |  |  |
| CA_n2A-n77C-n260G |  | CA_n2A-n260A/G CA_n77A-n260A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n2A-n77C-n260H |  | CA_n2A-n260A/G/H CA_n77A-n260A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n2A-n77C-n260I |  | CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n2A-n77C-n260J |  | CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n2A-n77C-n260K |  | CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n2A-n77C-n260L |  | CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n2A-n77C-n260M |  | CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n2A-n77A-n261A |  | CA_n77A-n261A CA_n2A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n2A-n77A-n261G |  | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n2A-n77A-n261H |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n2A-n77A-n261I |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n2A-n77A-n261J |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n2A-n77A-n261K |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n2A-n77A-n261L |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n2A-n77A-n261M |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n2A-n77A-n261(A-H) |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n2A-n77A-n261(G-H) |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n2A-n77A-n261(2A-H) |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n2A-n77A-n261(A-2G) |  | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n2A-n77A-n261(A-G-H) |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n2A-n77A-n261(A-I) |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-I) |  |  |  |  |
| CA_n2A-n77A-n261(G-I) |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n2A-n77A-n261(2G) |  | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n2A-n77A-n261(2H) |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n2A-n77A-n261(2A-I) |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n2A-n77A-n261(A-G-I) |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n2A-n77A-n261(H-I) |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n2A-n77C-n261A |  | CA_n2A-n261A CA_n77A-n261A |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261A |  |  |  |  |
| CA_n2A-n77C-n261G |  | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n2A-n77C-n261H |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n2A-n77C-n261I |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n2A-n77C-n261J |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n2A-n77C-n261K |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n2A-n77C-n261L |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n2A-n77C-n261M |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n2A-n77C-n261(A-H) |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n2A-n77C-n261(G-H) |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n2A-n77C-n261(2A-H) |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n2A-n77C-n261(A-2G) |  | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n2A-n77C-n261(A-G-H) |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n2A-n77C-n261(A-I) |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-I) |  |  |  |  |
| CA_n2A-n77C-n261(G-I) |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n2A-n77C-n261(2G) |  | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n2A-n77C-n261(2H) |  | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n2A-n77C-n261(H-I) |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n2A-n77C-n261(2A-I) |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n2A-n77C-n261(A-G-I) |  | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n3A-n7A-n257A |  | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n7A-n257G |  | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3A-n7A-n257H |  | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3A-n7A-n257I |  | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n3A-n7A-n257J |  | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n3A-n7A-n257K |  | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n3A-n7A-n257L |  | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n3A-n7A-n257M |  | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n3B-n7A-n257A |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3B-n7A-n257G |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3B-n7A-n257H |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3B-n7A-n257I |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n3B-n7A-n257J |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n3B-n7A-n257K |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n3B-n7A-n257L |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n3B-n7A-n257M |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n3A-n7A-n258A |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n7A-n258B |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258B |  |  |  |  |
| CA_n3A-n7A-n258C |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258C |  |  |  |  |
| CA_n3A-n7A-n258D |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n3A-n7A-n258E |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258E |  |  |  |  |
| CA_n3A-n7A-n258F |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258F |  |  |  |  |
| CA_n3A-n7A-n258G |  | CA_n3A-n258A/G CA_n7A-n258A/G CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258G |  |  |  |  |
| CA_n3A-n7A-n258H |  | CA_n3A-n258A/G/H CA_n7A-n258A/G/H CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258H |  |  |  |  |
| CA_n3A-n7A-n258I |  | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258I |  |  |  |  |
| CA_n3A-n7A-n258J |  | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258J |  |  |  |  |
| CA_n3A-n7A-n258K |  | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258K |  |  |  |  |
| CA_n3A-n7A-n258L |  | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258L |  |  |  |  |
| CA_n3A-n7A-n258M |  | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258M |  |  |  |  |
| CA_n3A-n7B-n258A |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n7B-n258B |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258B |  |  |  |  |
| CA_n3A-n7B-n258C |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258C |  |  |  |  |
| CA_n3A-n7B-n258D |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n3A-n7B-n258E |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258E |  |  |  |  |
| CA_n3A-n7B-n258F |  | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258F |  |  |  |  |
| CA_n3A-n7B-n258G |  | CA_n3A-n258A/G CA_n7A-n258A/G CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258G |  |  |  |  |
| CA_n3A-n7B-n258H |  | CA_n3A-n258A/G/H CA_n7A-n258A/G/H CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258H |  |  |  |  |
| CA_n3A-n7B-n258I |  | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258I |  |  |  |  |
| CA_n3A-n7B-n258J |  | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258J |  |  |  |  |
| CA_n3A-n7B-n258K |  | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258K |  |  |  |  |
| CA_n3A-n7B-n258L |  | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258L |  |  |  |  |
| CA_n3A-n7B-n258M |  | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n7 | CA_n7B |  |  |  |  |
|  |  |  |  | n258 | CA_n258M |  |  |  |  |
| CA_n3B-n7A-n258A |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3B-n7A-n258G |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258G |  |  |  |  |
| CA_n3B-n7A-n258H |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258H |  |  |  |  |
| CA_n3B-n7A-n258I |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258I |  |  |  |  |
| CA_n3B-n7A-n258J |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258J |  |  |  |  |
| CA_n3B-n7A-n258K |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258K |  |  |  |  |
| CA_n3B-n7A-n258L |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258L |  |  |  |  |
| CA_n3B-n7A-n258M |  | - |  | n3 | CA_n3B_BCS0 |  | 0 |  |  |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |  |  |
|  |  |  |  | n258 | CA_n258M |  |  |  |  |
| CA_n3A-n8A-n257A |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n8A-n257G |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3A-n8A-n257H |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3A-n8A-n257I |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n3A-n8A-n257J |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n3A-n8A-n257K |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n3A-n8A-n257L |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n3A-n8A-n257M |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n3A-n18A-n257A |  | CA_n3A-n18A CA_n3A-n257A CA_n18A-n257A |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n18 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n18A-n257G |  | CA_n3A-n18A CA_n3A-n257A CA_n3A-n257G CA_n18A-n257A CA_n18A-n257G |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n18 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3A-n18A-n257H |  | CA_n3A-n18A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n18 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3A-n18A-n257I |  | CA_n3A-n18A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n18A-n257I |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n18 | 5, 10, 15 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| 2CA_n3A-n28A-n257A |  | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n28A CA_n3A-n257A CA_n28A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n28A-n257D |  | CA_n3A-n28A CA_n3A-n257A/D CA_n28A-n257A/D |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n3A-n28A-n257G |  | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n28A CA_n3A-n257A/G CA_n28A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3A-n28A-n257H |  | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n28A CA_n3A-n257A/G/H CA_n28A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3A-n28A-n257I |  | CA_n3A-n28A CA_n3A-n257A/G/H/I CA_n28A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n3A-n28A-n258A |  | CA_n3A-n28A CA_n3A-n258A CA_n28A-n258A |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n28A-n258D |  | CA_n3A-n28A CA_n3A-n258A CA_n28A-n258A |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n3A-n28A-n258G |  | CA_n3A-n28A CA_n3A-n258A CA_n28A-n258A CA_n3A-n258G CA_n28A-n258G |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | CA_n258G |  |  |  |  |
| CA_n3A-n28A-n258H |  | CA_n3A-n28A CA_n3A-n258A CA_n28A-n258A CA_n3A-n258G CA_n28A-n258G CA_n3A-n258H CA_n28A-n258H |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | CA_n258H |  |  |  |  |
| CA_n3A-n28A-n258I |  | CA_n3A-n28A CA_n3A-n258A CA_n3A-n258G CA_n3A-n258H CA_n3A-n258I CA_n28A-n258A CA_n28A-n258G CA_n28A-n258H CA_n28A-n258I |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | CA_n258I |  |  |  |  |
| CA_n3A-n28A-n258J |  | CA_n3A-n28A CA_n3A-n258A CA_n28A-n258A CA_n3A-n258G CA_n28A-n258G CA_n3A-n258H CA_n28A-n258H CA_n3A-n258I CA_n28A-n258I |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |  |
|  |  |  |  | n258 | CA_n258J |  |  |  |  |
| CA_n3A-n41A-n257A |  | CA_n3A-n41A CA_n3A-n257A CA_n41A-n257A |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n41A-n257G |  | CA_n3A-n41A CA_n3A-n257A/G CA_n41A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3A-n41A-n257H |  | CA_n3A-n41A CA_n3A-n257A/G/H CA_n41A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3A-n41A-n257I |  | CA_n3A-n41A CA_n3A-n257A/G/H/I CA_n41A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n3A-n77A-n257A |  | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n77A-n257D |  | CA_n3A-n77A CA_n3A-n257A/D CA_n77A-n257A/D |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n3A-n77A-n257G |  | CA_n3A-n77A CA_n3A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3A-n77A-n257H |  | CA_n3A-n77A CA_n3A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3A-n77A-n257I |  | CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n3A-n77A-n257J |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n3A-n77A-n257K |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n3A-n77A-n257L |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n3A-n77A-n257M |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n3A-n77(2A)-n257A |  | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n77(2A)-n257D |  | CA_n3A-n77A CA_n3A-n257A/D CA_n77A-n257A/D |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n3A-n77(2A)-n257G |  | CA_n3A-n77A CA_n3A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3A-n77(2A)-n257H |  | CA_n3A-n77A CA_n3A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3A-n77(2A)-n257I |  | CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n3A-n77(2A)-n257J |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n3A-n77(2A)-n257K |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n3A-n77(2A)-n257L |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n3A-n77(2A)-n257M |  | - |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n3A-n77(3A)-n257A |  | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(3A) |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n77(3A)-n257D |  | CA_n3A-n77A CA_n3A-n257A/D CA_n77A-n257A/D |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(3A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n3A-n77(3A)-n257G |  | CA_n3A-n77A CA_n3A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(3A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3A-n77(3A)-n257H |  | CA_n3A-n77A CA_n3A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(3A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3A-n77(3A)-n257I |  | CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(3A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n3A-n78A-n257A |  | CA_n3A-n78A CA_n3A-n257A CA_n78A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n78A-n257D |  | CA_n3A-n78A CA_n3A-n257A/D CA_n78A-n257A/D |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n3A-n78A-n257G |  | CA_n3A-n78A CA_n3A-n257A/G CA_n78A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3A-n78A-n257H |  | CA_n3A-n78A CA_n3A-n257A/G/H CA_n78A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3A-n78A-n257I |  | CA_n3A-n78A CA_n3A-n257A/G/H/I CA_n78A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n3A-n78A-n258A |  | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n78A-n258B |  | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258B |  |  |  |  |
| CA_n3A-n78A-n258C |  | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258C |  |  |  |  |
| CA_n3A-n78A-n258D |  | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n3A-n78A-n258E |  | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258E |  |  |  |  |
| CA_n3A-n78A-n258F |  | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258F |  |  |  |  |
| CA_n3A-n78A-n258G |  | CA_n3A-n258A/G CA_n78A-n258A/G CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258G |  |  |  |  |
| CA_n3A-n78A-n258H |  | CA_n3A-n258A/G/H CA_n78A-n258A/G/H CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258H |  |  |  |  |
| CA_n3A-n78A-n258I |  | CA_n3A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258I |  |  |  |  |
| CA_n3A-n78A-n258J |  | CA_n3A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258J |  |  |  |  |
| CA_n3A-n78A-n258K |  | CA_n3A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258K |  |  |  |  |
| CA_n3A-n78A-n258L |  | CA_n3A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258L |  |  |  |  |
| CA_n3A-n78A-n258M |  | CA_n3A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258M |  |  |  |  |
| CA_n3A-n79A-n257A |  | CA_n3A-n79A CA_n3A-n257A CA_n79A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n3A-n79A-n257G |  | CA_n257G CA_n3A-n79A CA_n3A-n257A/G CA_n79A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n3A-n79A-n257H |  | CA_n257G/H CA_n3A-n79A CA_n3A-n257A/G/H CA_n79A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n3A-n79A-n257I |  | CA_n257G/H/I CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n5A-n30A-n260A |  | CA_n5A-n30A CA_n5A-n260A CA_n30A-n260A |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n30A-n260G |  | CA_n5A-n30A CA_n5A-n260A/G CA_n30A-n260A/G |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n5A-n30A-n260H |  | CA_n5A-n30A CA_n5A-n260A/G/H CA_n30A-n260A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n5A-n30A-n260I |  | CA_n5A-n30A CA_n5A-n260A/G/H/I CA_n30A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n5A-n30A-n260J |  | CA_n5A-n30A CA_n5A-n260A/G/H/I/J CA_n30A-n260A/G/H/I/J |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n5A-n30A-n260K |  | CA_n5A-n30A CA_n5A-n260A/G/H/I/J/K CA_n30A-n260A/G/H/I/J/K |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n5A-n30A-n260L |  | CA_n5A-n30A CA_n5A-n260A/G/H/I/J/K/L CA_n30A-n260A/G/H/I/J/K/L |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n5A-n30A-n260M |  | CA_n5A-n30A CA_n5A-n260A/G/H/I/J/K/L/M CA_n30A-n260A/G/H/I/J/K/L/M |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n5A-n48A-n260A |  | CA_n5A-n260A CA_n48A-n260A |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n48A-n260G |  | CA_n5A-n260A/G CA_n48A-n260A/G |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n5A-n48A-n260H |  | CA_n5A-n260A/G/H CA_n48A-n260A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n5A-n48A-n260I |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n5A-n48A-n260J |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n5A-n48A-n260K |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n5A-n48A-n260L |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n5A-n48A-n260M |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n5A-n48(2A)-n260A |  | CA_n5A-n260A CA_n48A-n260A |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n48(2A)-n260G |  | CA_n5A-n260A/G CA_n48A-n260A/G |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n5A-n48(2A)-n260H |  | CA_n5A-n260A/G/H CA_n48A-n260A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n5A-n48(2A)-n260I |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n5A-n48(2A)-n260J |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n5A-n48(2A)-n260K |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n5A-n48(2A)-n260L |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n5A-n48(2A)-n260M |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I ` |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n5A-n48B-n260A |  | CA_n5A-n260A CA_n48A-n260A |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n48B-n260G |  | CA_n5A-n260A/G CA_n48A-n260A/G |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n5A-n48B-n260H |  | CA_n5A-n260A/G/H CA_n48A-n260A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n5A-n48B-n260I |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n5A-n48B-n260J |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n5A-n48B-n260K |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n5A-n48B-n260L |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n5A-n48B-n260M |  | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n5A-n48A-n261A |  | CA_n5A-n261A CA_n48A-n261A |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n48A-n261G |  | CA_n5A-n261A/G CA_n48A-n261A/G |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n5A-n48A-n261H |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n5A-n48A-n261I |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n5A-n48A-n261J |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n5A-n48A-n261K |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n5A-n48A-n261L |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n5A-n48A-n261M |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n5A-n48A-n261(A-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n5A-n48A-n261(G-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n5A-n48A-n261(2A-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n5A-n48A-n261(A-2G) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n5A-n48A-n261(A-G-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n5A-n48A-n261(2G) |  | CA_n5A-n261A/G CA_n48A-n261A/G |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n5A-n48A-n261(2H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n5A-n48A-n261(G-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n5A-n48A-n261(H-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n5A-n48A-n261(2A-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n5A-n48A-n261(A-G-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n5A-n48(2A)-n261A |  | CA_n5A-n261A CA_n48A-n261A |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n48(2A)-n261G |  | CA_n5A-n261A/G CA_n48A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n5A-n48(2A)-n261H |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n5A-n48(2A)-n261I |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n5A-n48(2A)-n261J |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n5A-n48(2A)-n261K |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n5A-n48(2A)-n261L |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n5A-n48(2A)-n261M |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n5A-n48(2A)-n261(A-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n5A-n48(2A)-n261(G-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n5A-n48(2A)-n261(2A-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n5A-n48(2A)-n261(A-2G) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n5A-n48(2A)-n261(A-G-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n5A-n48(2A)-n261(2G) |  | CA_n5A-n261A/G CA_n48A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n5A-n48(2A)-n261(2H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n5A-n48(2A)-n261(G-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n5A-n48(2A)-n261(H-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n5A-n48(2A)-n261(2A-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n5A-n48(2A)-n261(A-G-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n5A-n48B-n261A |  | CA_n5A-n261A CA_n48A-n261A |  | n5 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n48B-n261G |  | CA_n5A-n261A/G CA_n48A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n5A-n48B-n261H |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n5A-n48B-n261I |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n5A-n48B-n261J |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n5A-n48B-n261K |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n5A-n48B-n261L |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n5A-n48B-n261M |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n5A-n48B-n261(A-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n5A-n48B-n261(G-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n5A-n48B-n261(2A-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n5A-n48B-n261(A-2G) |  | CA_n5A-n261A/G CA_n48A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n5A-n48B-n261(A-G-H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n5A-n48B-n261(2G) |  | CA_n5A-n261A/G CA_n48A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n5A-n48B-n261(2H) |  | CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n5A-n48B-n261(G-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n5A-n48B-n261(H-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n5A-n48B-n261(2A-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n5A-n48B-n261(A-G-I) |  | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n48 | CA_n48B |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n5A-n66A-n260A |  | CA_n5A-n66A CA_n5A-n260A CA_n66A-n260A |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n66A-n260G |  | CA_n5A-n66A CA_n5A-n260A/G CA_n66A-n260A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n5A-n66A-n260H |  | CA_n5A-n66A CA_n5A-n260A/G/H CA_n66A-n260A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n5A-n66A-n260I |  | CA_n5A-n66A CA_n5A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n5A-n66A-n260J |  | CA_n5A-n66A CA_n5A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n5A-n66A-n260K |  | CA_n5A-n66A CA_n5A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n5A-n66A-n260L |  | CA_n5A-n66A CA_n5A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n5A-n66A-n260M |  | CA_n5A-n66A CA_n5A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n5A-n66A-n261A |  | CA_n5A-n66A CA_n5A-n261A CA_n66A-n261A |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n66A-n261G |  | CA_n5A-n66A CA_n5A-n261A/G CA_n66A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n5A-n66A-n261H |  | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n5A-n66A-n261I |  | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n5A-n66A-n261J |  | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n5A-n66A-n261K |  | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n5A-n66A-n261L |  | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n5A-n66A-n261M |  | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n5A-n66A-n261(2G) |  | CA_n5A-n66A CA_n5A-n261A/G CA_n66A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n5A-n66A-n261(G-H) |  | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n5A-n66A-n261(A-G-H) |  | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n5A-n66A-n261(G-I) |  | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n5A-n66A-n261(2H) |  | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n5A-n66A-n261(A-G-I) |  | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n5A-n66A-n261(H-I) |  | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n5A-n66A-n261(2A-G) |  | CA_n5A-n66A CA_n5A-n261A/G CA_n66A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-G) |  |  |  |  |
| CA_n5A-n66A-n261(2A-H) |  | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n5A-n66A-n261(2A-I) |  | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n5A-n66A-n261(2A) |  | CA_n5A-n66A CA_n5A-n261A CA_n66A-n261A |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A) |  |  |  |  |
| CA_n5A-n66A-n261(3A) |  | CA_n5A-n66A CA_n5A-n261A CA_n66A-n261A |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(3A) |  |  |  |  |
| CA_n5A-n66A-n261(A-G) |  | CA_n5A-n66A CA_n5A-n261A/G CA_n66A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G) |  |  |  |  |
| CA_n5A-n66A-n261(A-2G) |  | CA_n5A-n66A CA_n5A-n261A/G CA_n66A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n5A-n66A-n261(A-H) |  | CA_n5A-n66A CA_n5A-n261A/G/I CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n5A-n66A-n261(A-I) |  | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-I) |  |  |  |  |
| CA_n5A-n77A-n260A |  | CA_n5A-n77A CA_n77A-n260A CA_n5A-n260A |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n77A-n260G |  | CA_n5A-n77A CA_n5A-n260A/G CA_n77A-n260A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n5A-n77A-n260H |  | CA_n5A-n77A CA_n5A-n260A/G/H CA_n77A-n260A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n5A-n77A-n260I |  | CA_n5A-n77A CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n5A-n77A-n260J |  | CA_n5A-n77A CA_n5A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n5A-n77A-n260K |  | CA_n5A-n77A CA_n5A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n5A-n77A-n260L |  | CA_n5A-n77A CA_n5A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n5A-n77A-n260M |  | CA_n5A-n77A CA_n5A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n5A-n77C-n260G |  | CA_n5A-n260A/G CA_n77A-n260A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n5A-n77C-n260H |  | CA_n5A-n260A/G/H CA_n77A-n260A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n5A-n77C-n260I |  | CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n5A-n77C-n260J |  | CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n5A-n77C-n260K |  | CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n5A-n77C-n260L |  | CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n5A-n77C-n260M |  | CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n5A-n77A-n261A |  | CA_n77A-n261A CA_n5A-n261A |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | 50, 100, 200, 400 |  |  |  |  |
| CA_n5A-n77A-n261G |  | CA_n5A-n261A/G CA_n77A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n5A-n77A-n261H |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n5A-n77A-n261I |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n5A-n77A-n261J |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n5A-n77A-n261K |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n5A-n77A-n261L |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n5A-n77A-n261M |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n5A-n77A-n261(A-H) |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n5A-n77A-n261(G-H) |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n5A-n77A-n261(2A-H) |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n5A-n77A-n261(A-2G) |  | CA_n5A-n261A/G CA_n77A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n5A-n77A-n261(A-G-H) |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n5A-n77A-n261(A-I) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-I) |  |  |  |  |
| CA_n5A-n77A-n261(G-I) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n5A-n77A-n261(2G) |  | CA_n5A-n261A/G CA_n77A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n5A-n77A-n261(2H) |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n5A-n77A-n261(2A-I) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n5A-n77A-n261(A-G-I) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n5A-n77A-n261(H-I) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n5A-n77C-n261A |  | CA_n5A-n261A CA_n77A-n261A |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261A |  |  |  |  |
| CA_n5A-n77C-n261G |  | CA_n5A-n261A/G CA_n77A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261G |  |  |  |  |
| CA_n5A-n77C-n261H |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261H |  |  |  |  |
| CA_n5A-n77C-n261I |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261I |  |  |  |  |
| CA_n5A-n77C-n261J |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261J |  |  |  |  |
| CA_n5A-n77C-n261K |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261K |  |  |  |  |
| CA_n5A-n77C-n261L |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261L |  |  |  |  |
| CA_n5A-n77C-n261M |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261M |  |  |  |  |
| CA_n5A-n77C-n261(A-H) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-H) |  |  |  |  |
| CA_n5A-n77C-n261(G-H) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-H) |  |  |  |  |
| CA_n5A-n77C-n261(2A-H) |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-H) |  |  |  |  |
| CA_n5A-n77C-n261(A-2G) |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-2G) |  |  |  |  |
| CA_n5A-n77C-n261(A-G-H) |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-H) |  |  |  |  |
| CA_n5A-n77C-n261(A-I) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-I) |  |  |  |  |
| CA_n5A-n77C-n261(G-I) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(G-I) |  |  |  |  |
| CA_n5A-n77C-n261(2G) |  | CA_n5A-n261A/G CA_n77A-n261A/G |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2G) |  |  |  |  |
| CA_n5A-n77C-n261(2H) |  | CA_n5A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2H) |  |  |  |  |
| CA_n5A-n77C-n261(H-I) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(H-I) |  |  |  |  |
| CA_n5A-n77C-n261(2A-I) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(2A-I) |  |  |  |  |
| CA_n5A-n77C-n261(A-G-I) |  | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |  |
|  |  |  |  | n261 | CA_n261(A-G-I) |  |  |  |  |
| CA_n7A-n25A-n257A |  | CA_n7A-n257A CA_n25A-n257A |  | n7 | See n7 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n257 | See n257 channel bandwidths in 38.101-2 Table 5.3.5-1 |  |  |  |  |
| CA_n7A-n25A-n257G |  | CA_n7A-n257A/G CA_n25A-n257A/G |  | n7 | See n7 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n7A-n25A-n257H |  | CA_n7A-n257A/G/H CA_n25A-n257A/G/H |  | n7 | See n7 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n7A-n25A-n257I |  | CA_n7A-n257A/G/H/I CA_n25A-n257A/G/H/I |  | n7 | See n7 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n7A-n25A-n257J |  | CA_n7A-n257A/G/H/I/J CA_n25A-n257A/G/H/I/J |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n7A-n25A-n257K |  | CA_n7A-n257A/G/H/I/J/K CA_n25A-n257A/G/H/I/J/K |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n7A-n25A-n257L |  | CA_n7A-n257A/G/H/I/J/K/L CA_n25A-n257A/G/H/I/J/K/L |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n7A-n25A-n257M |  | CA_n7A-n257A/G/H/I/J/K/L/M CA_n25A-n257A/G/H/I/J/K/L/M |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n7A-n25A-n260A |  | CA_n7A-n260A CA_n25A-n260A |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
| CA_n7A-n25A-n260G |  | CA_n7A-n260A/G CA_n25A-n260A/G |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n7A-n25A-n260H |  | CA_n7A-n260A/G/H CA_n25A-n260A/G/H |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n7A-n25A-n260I |  | CA_n7A-n260A/G/H/I CA_n25A-n260A/G/H/I |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n7A-n25A-n260J |  | CA_n7A-n260A/G/H/I/J CA_n25A-n260A/G/H/I/J |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260J |  |  |  |  |
| CA_n7A-n25A-n260K |  | CA_n7A-n260A/G/H/I/J/K CA_n25A-n260A/G/H/I/J/K |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260K |  |  |  |  |
| CA_n7A-n25A-n260L |  | CA_n7A-n260A/G/H/I/J/K/L CA_n25A-n260A/G/H/I/J/K/L |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260L |  |  |  |  |
| CA_n7A-n25A-n260M |  | CA_n7A-n260A/G/H/I/J/K/L/M CA_n25A-n260A/G/H/I/J/K/L/M |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |  |
|  |  |  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |  |  |
|  |  |  |  | n260 | CA_n260M |  |  |  |  |
| CA_n7A-n66A-n257A |  | CA_n7A-n257A CA_n66A-n257A |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 4 and 5 |  |  |
|  |  |  |  | n66 | 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n7A-n66A-n257G |  | CA_n7A-n257A/G CA_n66A-n257A/G | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 4 and 5 |  |
|  |  |  | n66 |  | 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n7A-n66A-n257H |  | CA_n7A-n257A/G/H CA_n66A-n257A/G/H | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 4 and 5 |  |
|  |  |  | n66 |  | 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n7A-n66A-n257I |  | CA_n7A-n257A/G/H/I CA_n66A-n257A/G/H/I | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 4 and 5 |  |
|  |  |  | n66 |  | 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n7A-n71A-n257A |  | CA_n7A-n257A CA_n71A-n257A | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 4 and 5 |  |
|  |  |  | n71 |  | 5, 10, 15, 20 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n7A-n71A-n257G |  | CA_n7A-n257A/G CA_n71A-n257A/G | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 4 and 5 |  |
|  |  |  | n71 |  | 5, 10, 15, 20 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n7A-n71A-n257H |  | CA_n7A-n257A/G/H CA_n71A-n257A/G/H | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 4 and 5 |  |
|  |  |  | n71 |  | 5, 10, 15, 20 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n7A-n71A-n257I |  | CA_n7A-n257A/G/H/I CA_n71A-n257G/H/I | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 4 and 5 |  |
|  |  |  | n71 |  | 5, 10, 15, 20 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n7A-n78A-n258A |  | CA_n7A-n78A CA_n7A-n258A CA_n78A-n258A | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n7A-n78A-n258B |  | CA_n7A-n78A CA_n7A-n258A/B CA_n78A-n258A/B | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258B |  |  |  |  |
| CA_n7A-n78A-n258C |  | CA_n7A-n78A CA_n7A-n258A/B/C CA_n78A-n258A/B/C | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258C |  |  |  |  |
| CA_n7A-n78A-n258D |  | CA_n7A-n78A CA_n7A-n258A/D CA_n78A-n258A/D | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258D |  |  |  |  |
| CA_n7A-n78A-n258E |  | CA_n7A-n78A CA_n7A-n258A/D/E CA_n78A-n258A/D/E | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258E |  |  |  |  |
| CA_n7A-n78A-n258F |  | CA_n7A-n78A CA_n7A-n258A/D/E/F CA_n78A-n258A/D/E/F | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258F |  |  |  |  |
| CA_n7A-n78A-n258G |  | CA_n7A-n78A CA_n7A-n258A/G CA_n78A-n258A/G | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258G |  |  |  |  |
| CA_n7A-n78A-n258H |  | CA_n7A-n78A CA_n7A-n258A/G/H CA_n78A-n258G/H | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258H |  |  |  |  |
| CA_n7A-n78A-n258I |  | CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258I |  |  |  |  |
| CA_n7A-n78A-n258J |  | CA_n7A-n78A CA_n7A-n258A/G/H/I/J CA_n78A-n258A/G/H/I/J | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258J |  |  |  |  |
| CA_n7A-n78A-n258K |  | CA_n7A-n78A CA_n7A-n258A/G/H/I/J/K CA_n78A-n258A/G/H/I/J/K | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258K |  |  |  |  |
| CA_n7A-n78A-n258L |  | CA_n7A-n78A CA_n7A-n258A/G/H/I/J/K/L CA_n78A-n258A/G/H/I/J/K/L | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258L |  |  |  |  |
| CA_n7A-n78A-n258M |  | CA_n7A-n78A CA_n7A-n258A/G/H/I/J/K/L/M CA_n78A-n258A/G/H/I/J/K/L/M | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258M |  |  |  |  |
| CA_n7B-n78A-n258A |  | CA_n7B-n78A CA_n7B-n258A CA_n78A-n258A | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n7B-n78A-n258B |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/B CA_n78A-n258A/B | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258B |  |  |  |  |
| CA_n7B-n78A-n258C |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/B/C CA_n78A-n258A/B/C | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258C |  |  |  |  |
| CA_n7B-n78A-n258D |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/D CA_n78A-n258A/D | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258D |  |  |  |  |
| CA_n7B-n78A-n258E |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/D/E CA_n78A-n258A/D/E | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258E |  |  |  |  |
| CA_n7B-n78A-n258F |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/D/E/F CA_n78A-n258A/D/E/F | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258F |  |  |  |  |
| CA_n7B-n78A-n258G |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/G CA_n78A-n258A/G | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258G |  |  |  |  |
| CA_n7B-n78A-n258H |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/G/H CA_n78A-n258G/H | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258H |  |  |  |  |
| CA_n7B-n78A-n258I |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258I |  |  |  |  |
| CA_n7B-n78A-n258J |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/G/H/I/J CA_n78A-n258A/G/H/I/J | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258J |  |  |  |  |
| CA_n7B-n78A-n258K |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/G/H/I/J/K CA_n78A-n258A/G/H/I/J/K | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258K |  |  |  |  |
| CA_n7B-n78A-n258L |  | CA_n7B CA_n7B-n258A/G/H/I/J/K/L CA_n78A-n258A/G/H/I/J/K/L | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258L |  |  |  |  |
| CA_n7B-n78A-n258M |  | CA_n7B CA_n7B-n78A CA_n7B-n258A/G/H/I/J/K/L/M CA_n78A-n258A/G/H/I/J/K/L/M | n7 |  | CA_n7B |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258M |  |  |  |  |
| CA_n8A-n77A-n257A |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n8A-n77A-n257G |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n8A-n77A-n257H |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n8A-n77A-n257I |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n8A-n77A-n257J |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n8A-n77A-n257K |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n8A-n77A-n257L |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n8A-n77A-n257M |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n8A-n77(2A)-n257A |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n8A-n77(2A)-n257G |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n8A-n77(2A)-n257H |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n8A-n77(2A)-n257I |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n8A-n77(2A)-n257J |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n8A-n77(2A)-n257K |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n8A-n77(2A)-n257L |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n8A-n77(2A)-n257M |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n8A-n78A-n257A |  | CA_n8A-n78A CA_n8A-n257A CA_n78A-n257A |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n8A-n78A-n257D |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n8A-n78A-n257E |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257E |  |  |  |  |
| CA_n8A-n78A-n257F |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257F |  |  |  |  |
| CA_n8A-n78A-n257G |  | CA_n257G CA_n8A-n78A CA_n8A-n257A/G CA_n78A-n257A/G |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n8A-n78A-n257H |  | CA_n257G/H CA_n8A-n78A CA_n8A-n257A/G/H CA_n78A-n257A/G/H |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n8A-n78A-n257I |  | CA_n257G/H/I CA_n8A-n78A CA_n8A-n257A/G/H/I CA_n78A-n257A/G/H/I |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n8A-n78A-n257J |  | CA_n257G/H/I/J CA_n8A-n78A CA_n8A-n257A/G/H/I/J CA_n78A-n257A/G/H/I/J |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n8A-n78A-n257K |  | CA_n257G/H/I/J/K CA_n8A-n78A CA_n8A-n257A/G/H/I/J/K CA_n78A-n257A/G/H/I/J/K |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n8A-n78A-n257L |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n8A-n78A-n257M |  | - |  | n8 | 5, 10, 15, 20 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n12A-n30A-n260A |  | CA_n12A-n30A CA_n12A-n260A CA_n30A-n260A |  | n12 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n12A-n30A-n260G |  | CA_n12A-n30A CA_n12A-n260A CA_n30A-n260A CA_n12A-n260G CA_n30A-n260G |  | n12 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n30 | 5, 10 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n12A-n30A-n260G |  | CA_n12A-n30A CA_n12A-n260A/G CA_n30A-n260A/G | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n12A-n30A-n260H |  | CA_n12A-n30A CA_n12A-n260A/G/H CA_n30A-n260A/G/H | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n12A-n30A-n260I |  | CA_n12A-n30A CA_n12A-n260A/G/H/I CA_n30A-n260A/G/H/I | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n12A-n30A-n260J |  | CA_n12A-n30A CA_n12A-n260A/G/H/I/J CA_n30A-n260A/G/H/I/J | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n12A-n30A-n260K |  | CA_n12A-n30A CA_n12A-n260A/G/H/I/J/K CA_n30A-n260A/G/H/I/J/K | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n12A-n30A-n260L |  | CA_n12A-n30A CA_n12A-n260A/G/H/I/J/K/L CA_n30A-n260A/G/H/I/J/K/L | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n12A-n30A-n260M |  | CA_n12A-n30A CA_n12A-n260A/G/H/I/J/K/L/M CA_n30A-n260A/G/H/I/J/K/L/M | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n12A-n66A-n260A |  | CA_n12A-n66A CA_n12A-n260A CA_n66A-n260A | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n12A-n66A-n260G |  | CA_n12A-n66A CA_n12A-n260A/G CA_n66A-n260A/G | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n12A-n66A-n260H |  | CA_n12A-n66A CA_n12A-n260A/G/H CA_n66A-n260A/G/H | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n12A-n66A-n260I |  | CA_n12A-n66A CA_n12A-n260A/G/H/I CA_n66A-n260A/G/H/I | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n12A-n66A-n260J |  | CA_n12A-n66A CA_n12A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n12A-n66A-n260K |  | CA_n12A-n66A CA_n12A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n12A-n66A-n260L |  | CA_n12A-n66A CA_n12A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n12A-n66A-n260M |  | CA_n12A-n66A CA_n12A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n12A-n77A-n260A |  | CA_n12A-n77A CA_n12A-n260A CA_n77A-n260A | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n12A-n77A-n260G |  | CA_n12A-n77A CA_n12A-n260A/G CA_n77A-n260A/G | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n12A-n77A-n260H |  | CA_n12A-n77A CA_n12A-n260A/G/H CA_n77A-n260A/G/H | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n12A-n77A-n260I |  | CA_n12A-n77A CA_n12A-n260A/G/H/I CA_n77A-n260A/G/H/I | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n12A-n77A-n260J |  | CA_n12A-n77A CA_n12A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n12A-n77A-n260K |  | CA_n12A-n77A CA_n12A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n12A-n77A-n260L |  | CA_n12A-n77A CA_n12A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n12A-n77A-n260M |  | CA_n12A-n77A CA_n12A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M | n12 |  | 5, 10, 15 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n14A-n30A-n260A |  | CA_n14A-n30A CA_n14A-n260A CA_n30A-n260A | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n14A-n30A-n260G |  | CA_n14A-n30A CA_n14A-n260A/G CA_n30A-n260A/G | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n14A-n30A-n260H |  | CA_n14A-n30A CA_n14A-n260A/G/H CA_n30A-n260A/G/H | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n14A-n30A-n260I |  | CA_n14A-n30A CA_n14A-n260A/G/H/I CA_n30A-n260A/G/H/I | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n14A-n30A-n260J |  | CA_n14A-n30A CA_n14A-n260A/G/H/I/J CA_n30A-n260A/G/H/I/J | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n14A-n30A-n260K |  | CA_n14A-n30A CA_n14A-n260A/G/H/I/J/K CA_n30A-n260A/G/H/I/J/K | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n14A-n30A-n260L |  | CA_n14A-n30A CA_n14A-n260A/G/H/I/J/K/L CA_n30A-n260A/G/H/I/J/K/L | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n14A-n30A-n260M |  | CA_n14A-n30A CA_n14A-n260A/G/H/I/J/K/L/M CA_n30A-n260A/G/H/I/J/K/L/M | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n30 |  | 5, 10 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n14A-n66A-n260A |  | CA_n14A-n66A CA_n14A-n260A CA_n66A-n260A | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n14A-n66A-n260G |  | CA_n14A-n66A CA_n14A-n260A/G CA_n66A-n260A/G | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n14A-n66A-n260H |  | CA_n14A-n66A CA_n14A-n260A/G/H CA_n66A-n260A/G/H | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n14A-n66A-n260I |  | CA_n14A-n66A CA_n14A-n260A/G/H/I CA_n66A-n260A/G/H/I | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n14A-n66A-n260J |  | CA_n14A-n66A CA_n14A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n14A-n66A-n260K |  | CA_n14A-n66A CA_n14A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n14A-n66A-n260L |  | CA_n14A-n66A CA_n14A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n14A-n66A-n260M |  | CA_n14A-n66A CA_n14A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n14A-n77A-n260A |  | CA_n14A-n77A CA_n14A-n260A CA_n77A-n260A | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n14A-n77A-n260G |  | CA_n14A-n77A CA_n14A-n260A/G CA_n77A-n260A/G | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n14A-n77A-n260H |  | CA_n14A-n77A CA_n14A-n260A/G/H CA_n77A-n260A/G/H | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n14A-n77A-n260I |  | CA_n14A-n77A CA_n14A-n260A/G/H/I CA_n77A-n260A/G/H/I | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n14A-n77A-n260J |  | CA_n14A-n77A CA_n14A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n14A-n77A-n260K |  | CA_n14A-n77A CA_n14A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n14A-n77A-n260L |  | CA_n14A-n77A CA_n14A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n14A-n77A-n260M |  | CA_n14A-n77A CA_n14A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M | n14 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n18A-n28A-n257A |  | CA_n18A-n28A CA_n18A-n257A CA_n28A-n257A |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n18A-n28A-n257G |  | CA_n18A-n28A CA_n18A-n257A CA_n18A-n257G CA_n28A-n257A CA_n28A-n257G |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n18A-n28A-n257H |  | CA_n18A-n28A CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n18A-n28A-n257I |  | CA_n18A-n28A CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n18A-n257I CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n28 | 5, 10 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n18A-n41A-n257A |  | CA_n18A-n41A CA_n18A-n257A CA_n41A-n257A |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n18A-n41A-n257G |  | CA_n18A-n41A CA_n18A-n257A CA_n18A-n257G CA_n41A-n257A CA_n41A-n257G |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n18A-n41A-n257H |  | CA_n18A-n41A CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n18A-n41A-n257I |  | CA_n18A-n41A CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n18A-n257I CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n18A-n77A-n257A |  | CA_n18A-n77A CA_n18A-n257A CA_n77A-n257A |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n18A-n77A-n257G |  | CA_n18A-n77A CA_n18A-n257A CA_n18A-n257G CA_n77A-n257A CA_n77A-n257G |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n18A-n77A-n257H |  | CA_n18A-n77A CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n18A-n77A-n257I |  | CA_n18A-n77A CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n18A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n18A-n77(2A)-n257A |  | CA_n18A-n77A CA_n18A-n257A CA_n77A-n257A |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n18A-n77(2A)-n257G |  | CA_n18A-n77A CA_n18A-n257A CA_n18A-n257G CA_n77A-n257A CA_n77A-n257G |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n18A-n77(2A)-n257H |  | CA_n18A-n77A CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n18A-n77(2A)-n257I |  | CA_n18A-n77A CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n18A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77(2A) |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n18A-n78A-n257A |  | CA_n18A-n78A CA_n18A-n257A CA_n78A-n257A |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n18A-n78A-n257G |  | CA_n18A-n78A CA_n18A-n257A CA_n18A-n257G CA_n78A-n257A CA_n78A-n257G |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n18A-n78A-n257H |  | CA_n18A-n78A CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n18A-n78A-n257I |  | CA_n18A-n78A CA_n18A-n257A CA_n18A-n257G CA_n18A-n257H CA_n18A-n257I CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I |  | n18 | 5, 10, 15 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n25A-n41A-n260A |  | CA_n25A-n260A CA_n41A-n260A |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n25A-n41A-n260G |  | CA_n25A-n260A CA_n41A-n260A |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260G |  |  |  |  |
| CA_n25A-n41A-n260H |  | CA_n25A-n260A CA_n41A-n260A |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260H |  |  |  |  |
| CA_n25A-n41A-n260I |  | CA_n25A-n260A CA_n41A-n260A |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260I |  |  |  |  |
| CA_n25A-n41A-n260(2A) |  | CA_n25A-n260A CA_n41A-n260A |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n260 | CA_n260(2A) |  |  |  |  |
| CA_n26A-n78A-n258A |  | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A |  | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n26A-n78A-n258B |  | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A |  | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258B |  |  |  |  |
| CA_n26A-n78A-n258C |  | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A |  | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258C |  |  |  |  |
| CA_n26A-n78A-n258D |  | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A |  | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n26A-n78A-n258E |  | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A |  | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258E |  |  |  |  |
| CA_n26A-n78A-n258F |  | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A |  | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258F |  |  |  |  |
| CA_n26A-n78A-n258G | CA_n26A-n258A/G CA_n78A-n258A/G CA_n26A-n78A |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258G |  |  |  |  |
| CA_n26A-n78A-n258H | CA_n26A-n258A/G/H CA_n78A-n258A/G/H CA_n26A-n78A |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258H |  |  |  |  |
| CA_n26A-n78A-n258I | CA_n26A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n26A-n78A |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258I |  |  |  |  |
| CA_n26A-n78A-n258J | CA_n26A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n26A-n78A |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258J |  |  |  |  |
| CA_n26A-n78A-n258K | CA_n26A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n26A-n78A |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258K |  |  |  |  |
| CA_n26A-n78A-n258L | CA_n26A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n26A-n78A |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258L |  |  |  |  |
| CA_n26A-n78A-n258M | CA_n26A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n26A-n78A |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258M |  |  |  |  |
| CA_n28A-n41A-n257A | CA_n28A-n41A CA_n28A-n257A CA_n41A-n257A |  | n28 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n28A-n41A-n257G | CA_n28A-n41A CA_n28A-n257A/G CA_n41A-n257A/G |  | n28 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n28A-n41A-n257H | CA_n28A-n41A CA_n28A-n257A/G/H CA_n41A-n257A/G/H |  | n28 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n28A-n41A-n257I | CA_n28A-n41A CA_n28A-n257A/G/H/I CA_n41A-n257A/G/H/I |  | n28 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n28A-n77A-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n28A-n77A-n257D | CA_n28A-n77A CA_n28A-n257A/D CA_n77A-n257A/D |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257D |  |  |  |  |
| CA_n28A-n77A-n257G | CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n28A-n77A-n257H | CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n28A-n77A-n257I | CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n28A-n77(2A)-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n28A-n77(2A)-n257D | CA_n28A-n77A CA_n28A-n257A/D CA_n77A-n257A/D |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257D |  |  |  |  |
| CA_n28A-n77(2A)-n257G | CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n28A-n77(2A)-n257H | CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n28A-n77(2A)-n257I | CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n28A-n77(3A)-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(3A) |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n28A-n77(3A)-n257D | CA_n28A-n77A CA_n28A-n257A/D CA_n77A-n257A/D |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(3A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257D |  |  |  |  |
| CA_n28A-n77(3A)-n257G | CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(3A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n28A-n77(3A)-n257H | CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(3A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n28A-n77(3A)-n257I | CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(3A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n28A-n78A-n257A | CA_n28A-n78A CA_n28A-n257A CA_n78A-n257A |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n28A-n78A-n257D | CA_n28A-n78A CA_n28A-n257A/D CA_n78A-n257A/D |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257D |  |  |  |  |
| CA_n28A-n78A-n257G | CA_n28A-n78A CA_n28A-n257A/G CA_n78A-n257A/G |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n28A-n78A-n257H | CA_n28A-n78A CA_n28A-n257A/G/H CA_n78A-n257A/G/H |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n28A-n78A-n257I | CA_n28A-n78A CA_n28A-n257A/G/H/I CA_n78A-n257A/G/H/I |  | n28 |  | 5, 10, 15, 20 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n28A-n78A-n258A |  | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A |  | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n28A-n78A-n258B |  | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A |  | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258B |  |  |  |  |
| CA_n28A-n78A-n258C |  | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A |  | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258C |  |  |  |  |
| CA_n28A-n78A-n258D |  | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A |  | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n28A-n78A-n258E |  | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A |  | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258E |  |  |  |  |
| CA_n28A-n78A-n258F |  | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A |  | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258F |  |  |  |  |
| CA_n28A-n78A-n258G | CA_n28A-n258A/G CA_n78A-n258A/G CA_n28A-n78A |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258G |  |  |  |  |
| CA_n28A-n78A-n258H | CA_n28A-n258A/G CA_n78A-n258A/G CA_n28A-n78A |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258H |  |  |  |  |
| CA_n28A-n78A-n258I | CA_n28A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n28A-n78A |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258I |  |  |  |  |
| CA_n28A-n78A-n258J | CA_n28A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n28A-n78A |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258J |  |  |  |  |
| CA_n28A-n78A-n258K | CA_n28A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n28A-n78A |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258K |  |  |  |  |
| CA_n28A-n78A-n258L | CA_n28A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n28A-n78A |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258L |  |  |  |  |
| CA_n28A-n78A-n258M | CA_n28A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n28A-n78A |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258M |  |  |  |  |
| CA_n28A-n79A-n257A | CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A |  | n28 |  | 5, 10, 15, 20, 30 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n28A-n79A-n257G | CA_n257G CA_n28A-n79A CA_n28A-n257A/G CA_n79A-n257A/G |  | n28 |  | 5, 10, 15, 20, 30 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n28A-n79A-n257H | CA_n257G/H CA_n28A-n79A CA_n28A-n257A/G/H CA_n79A-n257A/G/H |  | n28 |  | 5, 10, 15, 20, 30 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n28A-n79A-n257I | CA_n257G/H/I CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n28 |  | 5, 10, 15, 20, 30 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n30A-n66A-n260A | CA_n30A-n66A CA_n30A-n260A CA_n66A-n260A |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n30A-n66A-n260G | CA_n30A-n66A CA_n30A-n260A/G CA_n66A-n260A/G |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n30A-n66A-n260H | CA_n30A-n66A CA_n30A-n260A/G/H CA_n66A-n260A/G/H |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n30A-n66A-n260I | CA_n30A-n66A CA_n30A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n30A-n66A-n260J | CA_n30A-n66A CA_n30A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n30A-n66A-n260K | CA_n30A-n66A CA_n30A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n30A-n66A-n260L | CA_n30A-n66A CA_n30A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n30A-n66A-n260M | CA_n30A-n66A CA_n30A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n30A-n77A-n260A | CA_n30A-n77A CA_n30A-n260A CA_n77A-n260A |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n30A-n77A-n260G | CA_n30A-n77A CA_n30A-n260A/G CA_n77A-n260A/G |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n30A-n77A-n260H | CA_n30A-n77A CA_n30A-n260A/G/H CA_n77A-n260A/G/H |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n30A-n77A-n260I | CA_n30A-n77A CA_n30A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n30A-n77A-n260J | CA_n30A-n77A CA_n30A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n30A-n77A-n260K | CA_n30A-n77A CA_n30A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n30A-n77A-n260L | CA_n30A-n77A CA_n30A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n30A-n77A-n260M | CA_n30A-n77A CA_n30A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M |  | n30 |  | 5, 10 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n39A-n40A-n258A | CA_n39A-n40A CA_n39A-n258A CA_n40A-n258A |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n39A-n41A-n258A | CA_n39A-n41A CA_n39A-n258A CA_n41A-n258A |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n258 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n40A-n41A-n258A |  | CA_n40A-n41A CA_n40A-n258A CA_n41A-n258A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n40A-n77A-n257A |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n40A-n77A-n257D |  | CA_n40A-n77A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n40A-n77A-n257E |  | CA_n40A-n77A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257E |  |  |  |  |
| CA_n40A-n77A-n257F |  | CA_n40A-n77A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257F |  |  |  |  |
| CA_n40A-n77A-n257G |  | CA_n40A-n77A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n40A-n77A-n257H |  | CA_n40A-n77A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n40A-n77A-n257I |  | CA_n40A-n77A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n40A-n77A-n257J |  | CA_n40A-n77A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n40A-n77A-n257K |  | CA_n40A-n77A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n40A-n77A-n257L |  | CA_n40A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n40A-n77A-n257M |  | CA_n40A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n40A-n77C-n257A |  | CA_n40A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n40A-n77C-n257D |  | CA_n40A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n40A-n77C-n257E |  | CA_n40A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257E |  |  |  |  |
| CA_n40A-n77C-n257F |  | CA_n40A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257F |  |  |  |  |
| CA_n40A-n77C-n257G |  | CA_n40A-n77A CA_n77A-n257A-n257A CA_n40A-n257A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n40A-n77C-n257H |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n40A-n77C-n257I |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n40A-n77C-n257J |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n40A-n77C-n257K |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n40A-n77C-n257L |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n40A-n77C-n257M |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n40B-n77A-n257A |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n40B-n77A-n257D |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n40B-n77A-n257E |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257E |  |  |  |  |
| CA_n40B-n77A-n257F |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257F |  |  |  |  |
| CA_n40B-n77A-n257G |  | CA_n40A-n77ACA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n40B-n77A-n257H |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n40B-n77A-n257I |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n40B-n77A-n257J |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n40B-n77A-n257K |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n40B-n77A-n257L |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n40B-n77A-n257M |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n40B-n77C-n257A |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n40B-n77C-n257D |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257D |  |  |  |  |
| CA_n40B-n77C-n257E |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257E |  |  |  |  |
| CA_n40B-n77C-n257F |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257F |  |  |  |  |
| CA_n40B-n77C-n257G |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n40B-n77C-n257H |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n40B-n77C-n257I |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n40B-n77C-n257J |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257J |  |  |  |  |
| CA_n40B-n77C-n257K |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257K |  |  |  |  |
| CA_n40B-n77C-n257L |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257L |  |  |  |  |
| CA_n40B-n77C-n257M |  | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B |  | 0 |  |  |
|  |  |  |  | n77 | CA_n77C |  |  |  |  |
|  |  |  |  | n257 | CA_n257M |  |  |  |  |
| CA_n40A-n78A-n257A |  | CA_n40A CA_n78A CA_n40A-n257A CA_n78A-n257A |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257A |  |  |  |  |
| CA_n40A-n78A-n257D | CA_n40A CA_n78A CA_n40A-n257A/D CA_n78A-n257A/D |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257D |  |  |  |  |
| CA_n40A-n78A-n257E | CA_n40A CA_n78A CA_n78A-n257A/D/E CA_n40A-n257A/D/E |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257E |  |  |  |  |
| CA_n40A-n78A-n257F | CA_n40A CA_n78A CA_n78A-n257A/D/E/F CA_n40A-n257A/D/E/F |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257F |  |  |  |  |
| CA_n40A-n78A-n257G | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G CA_n40A-n257A/D/E/F/G |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n40A-n78A-n257H | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H CA_n40A-n257A/D/E/F/G/H |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n40A-n78A-n257I | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I CA_n40A-n257A/D/E/F/G/H/I |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n40A-n78A-n257J | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J CA_n40A-n257A/D/E/F/G/H/I/J |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257J |  |  |  |  |
| CA_n40A-n78A-n257K | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K CA_n40A-n257A/D/E/F/G/H/I/J/K |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257K |  |  |  |  |
| CA_n40A-n78A-n257L | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K/L CA_n40A-n257A/D/E/F/G/H/I/J/K/L |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257L |  |  |  |  |
| CA_n40A-n78A-n257M | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K/L/M CA_n40A-n257A/D/E/F/G/H/I/J/K/L/M |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257M |  |  |  |  |
| CA_n40A-n78C-n257A | CA_n40A CA_n78A CA_n40A-n257A CA_n78A-n257A |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257A |  |  |  |  |
| CA_n40A-n78C-n257D | CA_n40A CA_n78A CA_n40A-n257A/D CA_n78A-n257A/D |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257D |  |  |  |  |
| CA_n40A-n78C-n257E | CA_n40A CA_n78A CA_n78A-n257A/D/E CA_n40A-n257A/D/E |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257E |  |  |  |  |
| CA_n40A-n78C-n257F | CA_n40A CA_n78A CA_n78A-n257A/D/E/F CA_n40A-n257A/D/E/F |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257F |  |  |  |  |
| CA_n40A-n78C-n257G | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G CA_n40A-n257A/D/E/F/G |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n40A-n78C-n257H | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H CA_n40A-n257A/D/E/F/G/H |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n40A-n78C-n257I | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I CA_n40A-n257A/D/E/F/G/H/I |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n40A-n78C-n257J | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J CA_n40A-n257A/D/E/F/G/H/I/J |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257J |  |  |  |  |
| CA_n40A-n78C-n257K | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K CA_n40A-n257A/D/E/F/G/H/I/J/K |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257K |  |  |  |  |
| CA_n40A-n78C-n257L | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K/L CA_n40A-n257A/D/E/F/G/H/I/J/K/L |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257L |  |  |  |  |
| CA_n40A-n78C-n257M | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K/L/M CA_n40A-n257A/D/E/F/G/H/I/J/K/L/M |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C |  |  |  |  |
|  |  |  | n257 |  | CA_n257M |  |  |  |  |
| CA_n40A-n78(2A)-n257A | CA_n40A CA_n78A CA_n40A-n257A CA_n78A-n257A |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n40A-n78(2A)-n257D | CA_n40A CA_n78A CA_n40A-n257A/D CA_n78A-n257A/D |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257D |  |  |  |  |
| CA_n40A-n78(2A)-n257E | CA_n40A CA_n78A CA_n78A-n257A/D/E CA_n40A-n257A/D/E |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257E |  |  |  |  |
| CA_n40A-n78(2A)-n257F | CA_n40A CA_n78A CA_n78A-n257A/D/E/F CA_n40A-n257A/D/E/F |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257F |  |  |  |  |
| CA_n40A-n78(2A)-n257G | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G CA_n40A-n257A/D/E/F/G |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n40A-n78(2A)-n257H | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H CA_n40A-n257A/D/E/F/G/H |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n40A-n78(2A)-n257I | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I CA_n40A-n257A/D/E/F/G/H/I |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n40A-n78(2A)-n257J | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J CA_n40A-n257A/D/E/F/G/H/I/J |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257J |  |  |  |  |
| CA_n40A-n78(2A)-n257K | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K CA_n40A-n257A/D/E/F/G/H/I/J/K |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257K |  |  |  |  |
| CA_n40A-n78(2A)-n257L | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K/L CA_n40A-n257A/D/E/F/G/H/I/J/K/L |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257L |  |  |  |  |
| CA_n40A-n78(2A)-n257M | CA_n40A CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K/L/M CA_n40A-n257A/D/E/F/G/H/I/J/K/L/M |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257M |  |  |  |  |
| CA_n40B-n78A-n257A | CA_n40B CA_n78A CA_n40B-n257A CA_n78A-n257A |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n40B-n78A-n257D | CA_n40B CA_n78A CA_n40B-n257A/D CA_n78A-n257A/D |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257D |  |  |  |  |
| CA_n40B-n78A-n257E | CA_n40B CA_n78A CA_n78A-n257A/D/E CA_n40B-n257A/D/E |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257E |  |  |  |  |
| CA_n40B-n78A-n257F | CA_n40B CA_n78A CA_n78A-n257A/D/E/F CA_n40B-n257A/D/E/F |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257F |  |  |  |  |
| CA_n40B-n78A-n257G | CA_n78A CA_n78A-n257A/D/E/F/G CA_n40B-n257A/D/E/F/G |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n40B-n78A-n257H | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H CA_n40B-n257A/D/E/F/G/H |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n40B-n78A-n257I | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H/I CA_n40B-n257A/D/E/F/G/H/I |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n40B-n78A-n257J | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J CA_n40B-n257A/D/E/F/G/H/I/J |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257J |  |  |  |  |
| CA_n40B-n78A-n257K | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K CA_n40B-n257A/D/E/F/G/H/I/J/K |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257K |  |  |  |  |
| CA_n40B-n78A-n257L | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K/L CA_n40B-n257A/D/E/F/G/H/I/J/K/L |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257L |  |  |  |  |
| CA_n40B-n78A-n257M | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K/L/M CA_n40B-n257A/D/E/F/G/H/I/J/K/L/M |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257M |  |  |  |  |
| CA_n40B-n78(2A)-n257A | CA_n40B CA_n78A CA_n40B-n257A CA_n78A-n257A |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n40B-n78(2A)-n257D | CA_n40B CA_n78A CA_n40B-n257A/D CA_n78A-n257A/D |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257D |  |  |  |  |
| CA_n40B-n78(2A)-n257E | CA_n40B CA_n78A CA_n78A-n257A/D/E CA_n40B-n257A/D/E |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257E |  |  |  |  |
| CA_n40B-n78(2A)-n257F | CA_n40B CA_n78A CA_n78A-n257A/D/E/F CA_n40B-n257A/D/E/F |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257F |  |  |  |  |
| CA_n40B-n78(2A)-n257G | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G CA_n40B-n257A/D/E/F/G |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n40B-n78(2A)-n257H | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H CA_n40B-n257A/D/E/F/G/H |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n40B-n78(2A)-n257I | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H/I CA_n40B-n257A/D/E/F/G/H/I |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n40B-n78(2A)-n257J | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J CA_n40B-n257A/D/E/F/G/H/I/J |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257J |  |  |  |  |
| CA_n40B-n78(2A)-n257K | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K CA_n40B-n257A/D/E/F/G/H/I/J/K |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257K |  |  |  |  |
| CA_n40B-n78(2A)-n257L | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K/L CA_n40B-n257A/D/E/F/G/H/I/J/K/L |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257L |  |  |  |  |
| CA_n40B-n78(2A)-n257M | CA_n40B CA_n78A CA_n78A-n257A/D/E/F/G/H/I/J/K/L/M CA_n40B-n257A/D/E/F/G/H/I/J/K/L/M |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n257 |  | CA_n257M |  |  |  |  |
| CA_n40B-n78C-n257A | CA_n40B CA_n78C CA_n40B-n257A CA_n78C-n257A |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n40B-n78C-n257D | CA_n40B CA_n78C CA_n40B-n257A/D CA_n78C-n257A/D |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | CA_n257D |  |  |  |  |
| CA_n40B-n78C-n257E | CA_n40B CA_n78C CA_n78C-n257A/D/E CA_n40B-n257A/D/E |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | CA_n257E |  |  |  |  |
| CA_n40B-n78C-n257F | CA_n40B CA_n78C CA_n78C-n257A/D/E/F CA_n40B-n257A/D/E/F |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | CA_n257F |  |  |  |  |
| CA_n40B-n78C-n257G | CA_n40B CA_n78C CA_n78C-n257A/D/E/F/G CA_n40B-n257A/D/E/F/G |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n40B-n78C-n257H | CA_n40B CA_n78C CA_n78C-n257A/D/E/F/G/H CA_n40B-n257A/D/E/F/G/H |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n40B-n78C-n257I | CA_n40B CA_n78C CA_n78C-n257A/D/E/F/G/H/I CA_n40B-n257A/D/E/F/G/H/I |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n40B-n78C-n257J | CA_n40B CA_n78C CA_n78C-n257A/D/E/F/G/H/I/J CA_n40B-n257A/D/E/F/G/H/I/J |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | CA_n257J |  |  |  |  |
| CA_n40B-n78C-n257K | CA_n40B CA_n78C CA_n78C-n257A/D/E/F/G/H/I/J/K CA_n40B-n257A/D/E/F/G/H/I/J/K |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | CA_n257K |  |  |  |  |
| CA_n40B-n78C-n257L | CA_n40B CA_n78C CA_n78C-n257A/D/E/F/G/H/I/J/K/L CA_n40B-n257A/D/E/F/G/H/I/J/K/L |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | CA_n257L |  |  |  |  |
| CA_n40B-n78C-n257M | CA_n40B CA_n78C CA_n78C-n257A/D/E/F/G/H/I/J/K/L/M CA_n40B-n257A/D/E/F/G/H/I/J/K/L/M |  | n40 |  | CA_n40B_BCS1 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78C_BCS1 |  |  |  |  |
|  |  |  | n257 |  | CA_n257M |  |  |  |  |
| CA_n40A-n78A-n258A |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 100 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n40A-n78A-n258D |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258D |  |  |  |  |
| CA_n40A-n78A-n258E |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258E |  |  |  |  |
| CA_n40A-n78A-n258F |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258F |  |  |  |  |
| CA_n40A-n78A-n258G |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258G |  |  |  |  |
| CA_n40A-n78A-n258H |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258H |  |  |  |  |
| CA_n40A-n78A-n258I |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258I |  |  |  |  |
| CA_n40A-n78A-n258J |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258J |  |  |  |  |
| CA_n40A-n78A-n258K |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258K |  |  |  |  |
| CA_n40A-n78A-n258L |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258L |  |  |  |  |
| CA_n40A-n78A-n258M |  | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |  |  |
|  |  |  |  | n258 | CA_n258M |  |  |  |  |
| CA_n40A-n79A-n258A |  | CA_n40A-n79A CA_n79A-n258A CA_n40A-n258A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n258 | 50, 100, 200, 400 |  |  |  |  |
| CA_n41A-n66A-n260A |  | CA_n41A-n260A CA_n66A-n260A |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | 50, 100, 200, 400 |  |  |  |  |
| CA_n41A-n66A-n260(2A) |  | CA_n41A-n260A CA_n66A-n260A |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |  |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  |  | n260 | CA_n260(2A) |  |  |  |  |
| CA_n41A-n66A-n260G | CA_n41A-n260A/G CA_n66A-n260A/G |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n41A-n66A-n260H | CA_n41A-n260A/G/H CA_n66A-n260A/G/H |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n41A-n66A-n260I | CA_n41A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n41A-n77A-n257A | CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n41A-n77A-n257G | CA_n257G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n41A-n77A-n257H | CA_n257G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n41A-n77A-n257I | CA_n257G/H/I CA_n41A-n77A/G/H/I CA_n77A-n257A/G/H/I |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n41A-n77(2A)-n257A | CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n41A-n77(2A)-n257G | CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n41A-n77(2A)-n257H | CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n41A-n77(2A)-n257I | CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n41A-n77(3A)-n257A | CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(3A) |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n41A-n77(3A)-n257G | CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(3A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n41A-n77(3A)-n257H | CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(3A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n41A-n77(3A)-n257I | CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(3A) |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n41A-n78A-n257A |  | - |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n41A-n78A-n257G |  | - |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n41A-n78A-n257H |  | - |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257H |  |  |  |  |
| CA_n41A-n78A-n257I |  | - |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |  |  |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257I |  |  |  |  |
| CA_n41A-n79A-n257A |  | CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n257 | 50, 100, 200, 400 |  |  |  |  |
| CA_n41A-n79A-n257G |  | CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n79A-n257A CA_n79A-n257G |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |  |  |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  |  | n257 | CA_n257G |  |  |  |  |
| CA_n41A-n79A-n257G | CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n41A-n79A-n257H | CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n41A-n79A-n257I | CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n41A-n79A-n258A | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48A-n66A-n260A | CA_n48A-n260A CA_n66A-n260A |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48A-n66A-n260G | CA_n48A-n260A/G CA_n66A-n260A/G |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n48A-n66A-n260H | CA_n48A-n260A/G/H CA_n66A-n260A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n48A-n66A-n260I | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n48A-n66A-n260J | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n48A-n66A-n260K | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n48A-n66A-n260L | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n48A-n66A-n260M | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n48B-n66A-n260A | CA_n48A-n260A CA_n66A-n260A |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48B-n66A-n260G | CA_n48A-n260A/G CA_n66A-n260A/G |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n48B-n66A-n260H | CA_n48A-n260A/G/H CA_n66A-n260A/G/H |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n48B-n66A-n260I | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n48B-n66A-n260J | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n48B-n66A-n260K | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n48B-n66A-n260L | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n48B-n66A-n260M | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n48(2A)-n66A-n260A | CA_n48A-n260A CA_n66A-n260A |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48(2A)-n66A-n260G | CA_n48A-n260A/G CA_n66A-n260A/G |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n48(2A)-n66A-n260H | CA_n48A-n260A/G/H CA_n66A-n260A/G/H |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n48(2A)-n66A-n260I | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n48(2A)-n66A-n260J | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n48(2A)-n66A-n260K | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n48(2A)-n66A-n260L | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n48(2A)-n66A-n260M | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n48A-n66A-n261A | CA_n48A-n261A CA_n66A-n261A |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48A-n66A-n261G | CA_n48A-n261A/G CA_n66A-n261A/G |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261G |  |  |  |  |
| CA_n48A-n66A-n261H | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261H |  |  |  |  |
| CA_n48A-n66A-n261I | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261I |  |  |  |  |
| CA_n48A-n66A-n261J | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261J |  |  |  |  |
| CA_n48A-n66A-n261K | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261K |  |  |  |  |
| CA_n48A-n66A-n261L | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261L |  |  |  |  |
| CA_n48A-n66A-n261M | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261M |  |  |  |  |
| CA_n48A-n66A-n261(A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-H) |  |  |  |  |
| CA_n48A-n66A-n261(G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-H) |  |  |  |  |
| CA_n48A-n66A-n261(2A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-H) |  |  |  |  |
| CA_n48A-n66A-n261(A-2G) | CA_n48A-n261A/G CA_n66A-n261A/G |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-2G) |  |  |  |  |
| CA_n48A-n66A-n261(A-G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-H) |  |  |  |  |
| CA_n48A-n66A-n261(2G) | CA_n48A-n261A/G CA_n66A-n261A/G |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2G) |  |  |  |  |
| CA_n48A-n66A-n261(2H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2H) |  |  |  |  |
| CA_n48A-n66A-n261(2A-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-I) |  |  |  |  |
| CA_n48A-n66A-n261(A-G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-I) |  |  |  |  |
| CA_n48A-n66A-n261(G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-I) |  |  |  |  |
| CA_n48A-n66A-n261(H-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(H-I) |  |  |  |  |
| CA_n48B-n66A-n261A | CA_n48A-n261A CA_n66A-n261A |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48B-n66A-n261G | CA_n48A-n261A/G CA_n66A-n261A/G |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261G |  |  |  |  |
| CA_n48B-n66A-n261H | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261H |  |  |  |  |
| CA_n48B-n66A-n261I | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261I |  |  |  |  |
| CA_n48B-n66A-n261J | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261J |  |  |  |  |
| CA_n48B-n66A-n261K | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261K |  |  |  |  |
| CA_n48B-n66A-n261L | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261L |  |  |  |  |
| CA_n48B-n66A-n261M | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261M |  |  |  |  |
| CA_n48B-n66A-n261(A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-H) |  |  |  |  |
| CA_n48B-n66A-n261(G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-H) |  |  |  |  |
| CA_n48B-n66A-n261(2A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-H) |  |  |  |  |
| CA_n48B-n66A-n261(A-2G) | CA_n48A-n261A/G CA_n66A-n261A/G |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-2G) |  |  |  |  |
| CA_n48B-n66A-n261(A-G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-H) |  |  |  |  |
| CA_n48B-n66A-n261(2G) | CA_n48A-n261A/G CA_n66A-n261A/G |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2G) |  |  |  |  |
| CA_n48B-n66A-n261(2H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2H) |  |  |  |  |
| CA_n48B-n66A-n261(G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-I) |  |  |  |  |
| CA_n48B-n66A-n261(H-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(H-I) |  |  |  |  |
| CA_n48B-n66A-n261(2A-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-I) |  |  |  |  |
| CA_n48B-n66A-n261(A-G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48B_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-I) |  |  |  |  |
| CA_n48(2A)-n66A-n261A | CA_n48A-n261A CA_n66A-n261A |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48(2A)-n66A-n261G | CA_n48A-n261A/G CA_n66A-n261A/G |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261G |  |  |  |  |
| CA_n48(2A)-n66A-n261H | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261H |  |  |  |  |
| CA_n48(2A)-n66A-n261I | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261I |  |  |  |  |
| CA_n48(2A)-n66A-n261J | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261J |  |  |  |  |
| CA_n48(2A)-n66A-n261K | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261K |  |  |  |  |
| CA_n48(2A)-n66A-n261L | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261L |  |  |  |  |
| CA_n48(2A)-n66A-n261M | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261M |  |  |  |  |
| CA_n48(2A)-n66A-n261(A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-H) |  |  |  |  |
| CA_n48(2A)-n66A-n261(G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-H) |  |  |  |  |
| CA_n48(2A)-n66A-n261(2A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-H) |  |  |  |  |
| CA_n48(2A)-n66A-n261(A-2G) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-2G) |  |  |  |  |
| CA_n48(2A)-n66A-n261(A-G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-H) |  |  |  |  |
| CA_n48(2A)-n66A-n261(2G) | CA_n48A-n261A/G CA_n66A-n261A/G |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2G) |  |  |  |  |
| CA_n48(2A)-n66A-n261(2H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2H) |  |  |  |  |
| CA_n48(2A)-n66A-n261(G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-I) |  |  |  |  |
| CA_n48(2A)-n66A-n261(H-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(H-I) |  |  |  |  |
| CA_n48(2A)-n66A-n261(2A-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-I) |  |  |  |  |
| CA_n48(2A)-n66A-n261(A-G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n48 |  | CA_n48(2A)_BCS1 |  |  | 0 |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-I) |  |  |  |  |
| CA_n48A-n77A-n260A | CA_n48A-n260A CA_n77A-n260A |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48A-n77A-n260G | CA_n48A-n260A/G CA_n77A-n260A/G |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n48A-n77A-n260H | CA_n48A-n260A/G/H CA_n77A-n260A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n48A-n77A-n260I | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n48A-n77A-n260J | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n48A-n77A-n260K | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n48A-n77A-n260L | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n48A-n77A-n260M | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n48A-n77C-n260A | CA_n48A-n260A CA_n77A-n260A |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48A-n77C-n260G | CA_n48A-n260A/G CA_n77A-n260A/G |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n48A-n77C-n260H | CA_n48A-n260A/G/H CA_n77A-n260A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n48A-n77C-n260I | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n48A-n77C-n260J | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n48A-n77C-n260K | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n48A-n77C-n260L | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n48A-n77C-n260M | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n48A-n77A-n261A | CA_n48A-n261A CA_n77A-n261A |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48A-n77A-n261G | CA_n48A-n261A/G CA_n77A-n261A/G |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261G |  |  |  |  |
| CA_n48A-n77A-n261H | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261H |  |  |  |  |
| CA_n48A-n77A-n261I | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261I |  |  |  |  |
| CA_n48A-n77A-n261J | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261J |  |  |  |  |
| CA_n48A-n77A-n261K | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261K |  |  |  |  |
| CA_n48A-n77A-n261L | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261L |  |  |  |  |
| CA_n48A-n77A-n261M | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261M |  |  |  |  |
| CA_n48A-n77A-n261(A-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-H) |  |  |  |  |
| CA_n48A-n77A-n261(G-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-H) |  |  |  |  |
| CA_n48A-n77A-n261(2G) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2G) |  |  |  |  |
| CA_n48A-n77A-n261(2H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2H) |  |  |  |  |
| CA_n48A-n77A-n261(2A-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-H) |  |  |  |  |
| CA_n48A-n77A-n261(A-2G) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-2G) |  |  |  |  |
| CA_n48A-n77A-n261(A-G-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-H) |  |  |  |  |
| CA_n48A-n77A-n261(G-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-I) |  |  |  |  |
| CA_n48A-n77A-n261(H-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(H-I) |  |  |  |  |
| CA_n48A-n77A-n261(2A-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-I) |  |  |  |  |
| CA_n48A-n77A-n261(A-G-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-I) |  |  |  |  |
| CA_n48A-n77C-n261A | CA_n48A-n261A CA_n77A-n261A |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n48A-n77C-n261G | CA_n48A-n261A/G CA_n77A-n261A/G |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261G |  |  |  |  |
| CA_n48A-n77C-n261H | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261H |  |  |  |  |
| CA_n48A-n77C-n261I | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261I |  |  |  |  |
| CA_n48A-n77C-n261J | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261J |  |  |  |  |
| CA_n48A-n77C-n261K | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261K |  |  |  |  |
| CA_n48A-n77C-n261L | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261L |  |  |  |  |
| CA_n48A-n77C-n261M | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261M |  |  |  |  |
| CA_n48A-n77C-n261(A-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-H) |  |  |  |  |
| CA_n48A-n77C-n261(G-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-H) |  |  |  |  |
| CA_n48A-n77C-n261(2G) | CA_n48A-n261A/G CA_n77A-n261A/G |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2G) |  |  |  |  |
| CA_n48A-n77C-n261(2H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2H) |  |  |  |  |
| CA_n48A-n77C-n261(2A-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-H) |  |  |  |  |
| CA_n48A-n77C-n261(A-2G) | CA_n48A-n261A/G CA_n77A-n261A/G |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-2G) |  |  |  |  |
| CA_n48A-n77C-n261(A-G-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-H) |  |  |  |  |
| CA_n48A-n77C-n261(G-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-I) |  |  |  |  |
| CA_n48A-n77C-n261(H-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(H-I) |  |  |  |  |
| CA_n48A-n77C-n261(2A-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-I) |  |  |  |  |
| CA_n48A-n77C-n261(A-G-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-I) |  |  |  |  |
| CA_n66A-n77A-n260A | CA_n66A-n77A CA_n77A-n260A CA_n66A-n260A |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n66A-n77A-n260G | CA_n66A-n77A CA_n66A-n260A/G CA_n77A-n260A/G |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n66A-n77A-n260H | CA_n66A-n77A CA_n66A-n260A/G/H CA_n77A-n260A/G/H |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n66A-n77A-n260I | CA_n66A-n77A CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n66A-n77A-n260J | CA_n66A-n77A CA_n66A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n66A-n77A-n260K | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n66A-n77A-n260L | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n66A-n77A-n260M | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n66A-n77(2A)-n260A | CA_n66A-n77A CA_n66A-n260A CA_n77(2A) CA_n77A-n260A |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |  |  |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n66A-n77(2A)-n260G | CA_n66A-n77A CA_n66A-n260A/G CA_n77(2A) CA_n77A-n260A/G |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n66A-n77(2A)-n260H | CA_n66A-n77A CA_n66A-n260A/G/H CA_n77(2A) CA_n77A-n260A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n66A-n77(2A)-n260I | CA_n66A-n77A CA_n66A-n260A/G/H/I CA_n77(2A) CA_n77A-n260A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n66A-n77(2A)-n260J | CA_n66A-n77A CA_n66A-n260A/G/H/I/J CA_n77(2A) CA_n77A-n260A/G/H/I/J |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n66A-n77(2A)-n260K | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K CA_n77(2A) CA_n77A-n260A/G/H/I/J/K |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n66A-n77(2A)-n260L | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K/L CA_n77(2A) CA_n77A-n260A/G/H/I/J/K/L |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n66A-n77(2A)-n260M | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K/L/M CA_n77(2A) CA_n77A-n260A/G/H/I/J/K/L/M |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77(2A) |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n66A-n77C-n260A | CA_n66A-n260A CA_n77A-n260A |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260A |  |  |  |  |
| CA_n66A-n77C-n260G | CA_n66A-n260A/G CA_n77A-n260A/G |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260G |  |  |  |  |
| CA_n66A-n77C-n260H | CA_n66A-n260A/G/H CA_n77A-n260A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260H |  |  |  |  |
| CA_n66A-n77C-n260I | CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260I |  |  |  |  |
| CA_n66A-n77C-n260J | CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260J |  |  |  |  |
| CA_n66A-n77C-n260K | CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260K |  |  |  |  |
| CA_n66A-n77C-n260L | CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260L |  |  |  |  |
| CA_n66A-n77C-n260M | CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n260 |  | CA_n260M |  |  |  |  |
| CA_n66A-n77A-n261A | CA_n77A-n261A CA_n66A-n261A |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n66A-n77A-n261G | CA_n66A-n261A/G CA_n77A-n261A/G |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261G |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261G |  |  |  |  |
| CA_n66A-n77A-n261H | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261H |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261H |  |  |  |  |
| CA_n66A-n77A-n261I | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261I |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261I |  |  |  |  |
| CA_n66A-n77A-n261J | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261J |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261J |  |  |  |  |
| CA_n66A-n77A-n261K | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261K |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261K |  |  |  |  |
| CA_n66A-n77A-n261L | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261L |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261L |  |  |  |  |
| CA_n66A-n77A-n261M | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261M |  |  |  |  |
|  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261M |  |  |  |  |
| CA_n66A-n77A-n261(A-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-H) |  |  |  |  |
| CA_n66A-n77A-n261(G-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-H) |  |  |  |  |
| CA_n66A-n77A-n261(2A-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-H) |  |  |  |  |
| CA_n66A-n77A-n261(A-2G) | CA_n66A-n261A/G CA_n77A-n261A/G |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-2G) |  |  |  |  |
| CA_n66A-n77A-n261(A-G-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-H) |  |  |  |  |
| CA_n66A-n77A-n261(A-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-I) |  |  |  |  |
| CA_n66A-n77A-n261(G-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-I) |  |  |  |  |
| CA_n66A-n77A-n261(2G) | CA_n66A-n261A/G CA_n77A-n261A/G |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2G) |  |  |  |  |
| CA_n66A-n77A-n261(2H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2H) |  |  |  |  |
| CA_n66A-n77A-n261(2A-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-I) |  |  |  |  |
| CA_n66A-n77A-n261(A-G-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-I) |  |  |  |  |
| CA_n66A-n77A-n261(H-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(H-I) |  |  |  |  |
| CA_n66A-n77C-n261A | CA_n66A-n261A CA_n77A-n261A |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261A |  |  |  |  |
| CA_n66A-n77C-n261G | CA_n66A-n261A/G CA_n77A-n261A/G |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261G |  |  |  |  |
| CA_n66A-n77C-n261H | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261H |  |  |  |  |
| CA_n66A-n77C-n261I | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261I |  |  |  |  |
| CA_n66A-n77C-n261J | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261J |  |  |  |  |
| CA_n66A-n77C-n261K | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261K |  |  |  |  |
| CA_n66A-n77C-n261L | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261L |  |  |  |  |
| CA_n66A-n77C-n261M | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261M |  |  |  |  |
| CA_n66A-n77C-n261(A-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-H) |  |  |  |  |
| CA_n66A-n77C-n261(G-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-H) |  |  |  |  |
| CA_n66A-n77C-n261(2A-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-H) |  |  |  |  |
| CA_n66A-n77C-n261(A-2G) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-2G) |  |  |  |  |
| CA_n66A-n77C-n261(A-G-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-H) |  |  |  |  |
| CA_n66A-n77C-n261(A-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-I) |  |  |  |  |
| CA_n66A-n77C-n261(G-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(G-I) |  |  |  |  |
| CA_n66A-n77C-n261(2G) | CA_n66A-n261A/G CA_n77A-n261A/G |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2G) |  |  |  |  |
| CA_n66A-n77C-n261(2H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2H) |  |  |  |  |
| CA_n66A-n77C-n261(H-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(H-I) |  |  |  |  |
| CA_n66A-n77C-n261(2A-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(2A-I) |  |  |  |  |
| CA_n66A-n77C-n261(A-G-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |  |
|  |  |  | n77 |  | CA_n77C_BCS1 |  |  |  |  |
|  |  |  | n261 |  | CA_n261(A-G-I) |  |  |  |  |
| CA_n77A-n79A-n257A | CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n77A-n79A-n257G | CA_n257G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n77A-n79A-n257H | CA_n257G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n77A-n79A-n257I | CA_n257G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n77(2A)-n79A-n257A | CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n77 |  | CA_n77(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n77(2A)-n79A-n257G | CA_n257G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n77 |  | CA_n77(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n77(2A)-n79A-n257H | CA_n257G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H |  | n77 |  | CA_n77(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n77(2A)-n79A-n257I | CA_n257G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n77 |  | CA_n77(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n77(3A)-n79A-n257A | CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n77 |  | CA_n77(3A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n77(3A)-n79A-n257G | CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n77 |  | CA_n77(3A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n77(3A)-n79A-n257H | CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H |  | n77 |  | CA_n77(3A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n77(3A)-n79A-n257I | CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n77 |  | CA_n77(3A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n77A-n79A-n258A | CA_n77A-n79A CA_n77A-n258A CA_n79A-n258A |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n77A-n79A-n258D | CA_n77A-n79A CA_n77A-n258A/D CA_n79A-n258A/D |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258D |  |  |  |  |
| CA_n77A-n79A-n258G | CA_n77A-n79A CA_n77A-n258A/G CA_n79A-n258A/G |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258G |  |  |  |  |
| CA_n77A-n79A-n258H | CA_n77A-n79A CA_n77A-n258A/G/H CA_n79A-n258A/G/H |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258H |  |  |  |  |
| CA_n77A-n79A-n258I | CA_n77A-n79A CA_n77A-n258A/G/H/I CA_n79A-n258A/G/H/I |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258I |  |  |  |  |
| CA_n77A-n79A-n258J | CA_n77A-n79A CA_n77A-n258A/G/H/I/J CA_n79A-n258A/G/H/I/J |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258J |  |  |  |  |
| CA_n77(2A)-n79A-n258A | CA_n77A-n79A CA_n77A-n258A CA_n79A-n258A |  | n77 |  | CA_n77(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n77(2A)-n79A-n258D | CA_n77A-n79A CA_n77A-n258A/D CA_n79A-n258A/D |  | n77 |  | CA_n77(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258D |  |  |  |  |
| CA_n77(2A)-n79A-n258G | CA_n77A-n79A CA_n77A-n258A/G CA_n79A-n258A/G |  | n77 |  | CA_n77(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258G |  |  |  |  |
| CA_n77(2A)-n79A-n258H | CA_n77A-n79A CA_n77A-n258A/G/H CA_n79A-n258A/G/H |  | n77 |  | CA_n77(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258H |  |  |  |  |
| CA_n77(2A)-n79A-n258I | CA_n77A-n79A CA_n77A-n258A/G/H/I CA_n79A-n258A/G/H/I |  | n77 |  | CA_n77(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258I |  |  |  |  |
| CA_n77(2A)-n79A-n258J | CA_n77A-n79A CA_n77A-n258A/G/H/I/J CA_n79A-n258A/G/H/I/J |  | n77 |  | CA_n77(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n258 |  | CA_n258J |  |  |  |  |
| CA_n77A-n79A-n259A | CA_n77A-n79A CA_n77A-n259A CA_n79A-n259A |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n77A-n79A-n259G | CA_n259G CA_n77A-n79A CA_n77A-n259A/G CA_n79A-n259A/G |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n77A-n79A-n259H | CA_n259G/H CA_n77A-n79A CA_n77A-n259A/G/H CA_n79A-n259A/G/H |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n77A-n79A-n259I | CA_n259G/H/I CA_n77A-n79A CA_n77A-n259A/G/H/I CA_n79A-n259A/G/H/I |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n77A-n79A-n259J | CA_n259G/H/I/J CA_n77A-n79A CA_n77A-n259A/G/H/I/J CA_n79A-n259A/G/H/I/J |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n77A-n79A-n259K | CA_n259G/H/I/J/K CA_n77A-n79A CA_n77A-n259A/G/H/I/J/K CA_n79A-n259A/G/H/I/J/K |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n77A-n79A-n259L | CA_n259G/H/I/J/K/L CA_n77A-n79A CA_n77A-n259A/G/H/I/J/K/L CA_n79A-n259A/G/H/I/J/K/L |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n77A-n79A-n259M | CA_n259G/H/I/J/K/L/M CA_n77A-n79A CA_n77A-n259A/G/H/I/J/K/L/M CA_n79A-n259A/G/H/I/J/K/L/M |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n77A-n257A-n259A | CA_n77A-n257A CA_n77A-n259A |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n77A-n257A-n259G | CA_n259G CA_n77A-n257A CA_n77A-n259A/G |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n77A-n257A-n259H | CA_n259G/H CA_n77A-n257A CA_n77A-n259A/G/H |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n77A-n257A-n259I | CA_n259G/H/I CA_n77A-n257A CA_n77A-n259A/G/H/I |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n77A-n257A-n259J | CA_n259G/H/I/J CA_n77A-n257A CA_n77A-n259A/G/H/I/J |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n77A-n257A-n259K | CA_n259G/H/I/J/K CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n77A-n257A-n259L | CA_n259G/H/I/J/K/L CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K/L |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n77A-n257A-n259M | CA_n259G/H/I/J/K/L/M CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K/L/M |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n77A-n257G-n259A | CA_n257G CA_n77A-n257A/G CA_n77A-n259A |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n77A-n257G-n259G | CA_n257G CA_n259G CA_n77A-n257A/G CA_n77A-n259A/G |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n77A-n257G-n259H | CA_n257G CA_n259G/H CA_n77A-n257A/G CA_n77A-n259A/G/H |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n77A-n257G-n259I | CA_n257G CA_n259G/H/I CA_n77A-n257A/G CA_n77A-n259A/G/H/I |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n77A-n257G-n259J | CA_n257G CA_n259G/H/I/J CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n77A-n257G-n259K | CA_n257G CA_n259G/H/I/J/K CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n77A-n257G-n259L | CA_n257G CA_n259G/H/I/J/K/L CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K/L |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n77A-n257G-n259M | CA_n257G CA_n259G/H/I/J/K/L/M CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K/L/M |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n77A-n257H-n259A | CA_n257G/H CA_n77A-n257A/G/H CA_n77A-n259A |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n77A-n257H-n259G | CA_n257G/H CA_n259G CA_n77A-n257A/G/H CA_n77A-n259A/G |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n77A-n257H-n259H | CA_n257G/H CA_n259G/H CA_n77A-n257A/G/H CA_n77A-n259A/G/H |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n77A-n257H-n259I | CA_n257G/H CA_n259G/H/I CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n77A-n257H-n259J | CA_n257G/H CA_n259G/H/I/J CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n77A-n257H-n259K | CA_n257G/H CA_n259G/H/I/J/K CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n77A-n257H-n259L | CA_n257G/H CA_n259G/H/I/J/K/L CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K/L |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n77A-n257H-n259M | CA_n257G/H CA_n259G/H/I/J/K/L/M CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K/L/M |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n77A-n257I-n259A | CA_n257G/H/I CA_n77A-n257A/G/H/I CA_n77A-n259A |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n77A-n257I-n259G | CA_n257G/H/I CA_n259G CA_n77A-n257A/G/H/I CA_n77A-n259A/G |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n77A-n257I-n259H | CA_n257G/H/I CA_n259G/H CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n77A-n257I-n259I | CA_n257G/H/I CA_n259G/H/I CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n77A-n257I-n259J | CA_n257G/H/I CA_n259G/H/I/J CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n77A-n257I-n259K | CA_n257G/H/I CA_n259G/H/I/J/K CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n77A-n257I-n259L | CA_n257G/H/I CA_n259G/H/I/J/K/L CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K/L |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n77A-n257I-n259M | CA_n257G/H/I CA_n259G/H/I/J/K/L/M CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K/L/M |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n78A-n79A-n257A | CA_n78A-n79A CA_n78A-n257A CA_n79A-n257A |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n78A-n79A-n257G | CA_n257G CA_n78A-n79A CA_n78A-n257A/G CA_n79A-n257A/G |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n78A-n79A-n257H | CA_n257G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n79A-n257A/G/H |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n78A-n79A-n257I | CA_n257G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n78(2A)-n79A-n257A | CA_n78A-n79A CA_n78A-n257A CA_n79A-n257A |  | n78 |  | CA_n78(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n78(2A)-n79A-n257G | CA_n257G CA_n78A-n79A CA_n78A-n257A/G CA_n79A-n257A/G |  | n78 |  | CA_n78(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
| CA_n78(2A)-n79A-n257H | CA_n257G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n79A-n257A/G/H |  | n78 |  | CA_n78(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
| CA_n78(2A)-n79A-n257I | CA_n257G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n78 |  | CA_n78(2A) |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
| CA_n78A-n79A-n259A | CA_n78A-n79A CA_n78A-n259A CA_n79A-n259A |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n78A-n79A-n259G | CA_n259G CA_n78A-n79A CA_n78A-n259A/G CA_n79A-n259A/G |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n78A-n79A-n259H | CA_n259G/H CA_n78A-n79A CA_n78A-n259A/G/H CA_n79A-n259A/G/H |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n78A-n79A-n259I | CA_n259G/H/I CA_n78A-n79A CA_n78A-n259A/G/H/I CA_n79A-n259A/G/H/I |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n78A-n79A-n259J | CA_n259G/H/I/J CA_n78A-n79A CA_n78A-n259A/G/H/I/J CA_n79A-n259A/G/H/I/J |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n78A-n79A-n259K | CA_n259G/H/I/J/K CA_n78A-n79A CA_n78A-n259A/G/H/I/J/K CA_n79A-n259A/G/H/I/J/K |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n78A-n79A-n259L | CA_n259G/H/I/J/K/L CA_n78A-n79A CA_n78A-n259A/G/H/I/J/K/L CA_n79A-n259A/G/H/I/J/K/L |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n78A-n79A-n259M | CA_n259G/H/I/J/K/L/M CA_n78A-n79A CA_n78A-n259A/G/H/I/J/K/L/M CA_n79A-n259A/G/H/I/J/K/L/M |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n78A-n257A-n259A | CA_n78A-n257A CA_n78A-n259A |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n78A-n257A-n259G | CA_n259G CA_n78A-n257A CA_n78A-n259A/G |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n78A-n257A-n259H | CA_n259G/H CA_n78A-n257A CA_n78A-n259A/G/H |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n78A-n257A-n259I | CA_n259G/H/I CA_n78A-n257A CA_n78A-n259A/G/H/I |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n78A-n257A-n259J | CA_n259G/H/I/J CA_n78A-n257A CA_n78A-n259A/G/H/I/J |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n78A-n257A-n259K | CA_n259G/H/I/J/K CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n78A-n257A-n259L | CA_n259G/H/I/J/K/L CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K/L |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n78A-n257A-n259M | CA_n259G/H/I/J/K/L/M CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K/L/M |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n78A-n257G-n259A | CA_n257G CA_n78A-n257A/G CA_n78A-n259A |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n78A-n257G-n259G | CA_n257G CA_n259G CA_n78A-n257A/G CA_n78A-n259A/G |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n78A-n257G-n259H | CA_n257G CA_n259G/H CA_n78A-n257A/G CA_n78A-n259A/G/H |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n78A-n257G-n259I | CA_n257G CA_n259G/H/I CA_n78A-n257A/G CA_n78A-n259A/G/H/I |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n78A-n257G-n259J | CA_n257G CA_n259G/H/I/J CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n78A-n257G-n259K | CA_n257G CA_n259G/H/I/J/K CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n78A-n257G-n259L | CA_n257G CA_n259G/H/I/J/K/L CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K/L |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n78A-n257G-n259M | CA_n257G CA_n259G/H/I/J/K/L/M CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K/L/M |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n78A-n257H-n259A | CA_n257G/H CA_n78A-n257A/G/H CA_n78A-n259A |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n78A-n257H-n259G | CA_n257G/H CA_n259G CA_n78A-n257A/G/H CA_n78A-n259A/G |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n78A-n257H-n259H | CA_n257G/H CA_n259G/H CA_n78A-n257A/G/H CA_n78A-n259A/G/H |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n78A-n257H-n259I | CA_n257G/H CA_n259G/H/I CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n78A-n257H-n259J | CA_n257G/H CA_n259G/H/I/J CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n78A-n257H-n259K | CA_n257G/H CA_n259G/H/I/J/K CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n78A-n257H-n259L | CA_n257G/H CA_n259G/H/I/J/K/L CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K/L |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n78A-n257H-n259M | CA_n257G/H CA_n259G/H/I/J/K/L/M CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K/L/M |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n78A-n257I-n259A | CA_n257G/H/I CA_n78A-n257A/G/H/I CA_n78A-n259A |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n78A-n257I-n259G | CA_n257G/H/I CA_n259G CA_n78A-n257A/G/H/I CA_n78A-n259A/G |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n78A-n257I-n259H | CA_n257G/H/I CA_n259G/H CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n78A-n257I-n259I | CA_n257G/H/I CA_n259G/H/I CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n78A-n257I-n259J | CA_n257G/H/I CA_n259G/H/I/J CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n78A-n257I-n259K | CA_n257G/H/I CA_n259G/H/I/J/K CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n78A-n257I-n259L | CA_n257G/H/I CA_n259G/H/I/J/K/L CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K/L |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n78A-n257I-n259M | CA_n257G/H/I CA_n259G/H/I/J/K/L/M CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K/L/M |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n79A-n257A-n259A | CA_n79A-n257A CA_n79A-n259A |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n79A-n257A-n259G | CA_n259G CA_n79A-n257A CA_n79A-n259A/G |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n79A-n257A-n259H | CA_n259G/H CA_n79A-n257A CA_n79A-n259A/G/H |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n79A-n257A-n259I | CA_n259G/H/I CA_n79A-n257A CA_n79A-n259A/G/H/I |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n79A-n257A-n259J | CA_n259G/H/I/J CA_n79A-n257A CA_n79A-n259A/G/H/I/J |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n79A-n257A-n259K | CA_n259G/H/I/J/K CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n79A-n257A-n259L | CA_n259G/H/I/J/K/L CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n79A-n257A-n259M | CA_n259G/H/I/J/K/L/M CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L/M |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n79A-n257G-n259A | CA_n257G CA_n79A-n257A/G CA_n79A-n259A |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n79A-n257G-n259G | CA_n257G CA_n259G CA_n79A-n257A/G CA_n79A-n259A/G |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n79A-n257G-n259H | CA_n257G CA_n259G/H CA_n79A-n257A/G CA_n79A-n259A/G/H |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n79A-n257G-n259I | CA_n257G CA_n259G/H/I CA_n79A-n257A/G CA_n79A-n259A/G/H/I |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n79A-n257G-n259J | CA_n257G CA_n259G/H/I/J CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n79A-n257G-n259K | CA_n257G CA_n259G/H/I/J/K CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n79A-n257G-n259L | CA_n257G CA_n259G/H/I/J/K/L CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n79A-n257G-n259M | CA_n257G CA_n259G/H/I/J/K/L/M CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L/M |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257G |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n79A-n257H-n259A | CA_n257G/H CA_n79A-n257A/G/H CA_n79A-n259A |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n79A-n257H-n259G | CA_n257G/H CA_n259G CA_n79A-n257A/G/H CA_n79A-n259A/G |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n79A-n257H-n259H | CA_n257G/H CA_n259G/H CA_n79A-n257A/G/H CA_n79A-n259A/G/H |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n79A-n257H-n259I | CA_n257G/H CA_n259G/H/I CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n79A-n257H-n259J | CA_n257G/H CA_n259G/H/I/J CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n79A-n257H-n259K | CA_n257G/H CA_n259G/H/I/J/K CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n79A-n257H-n259L | CA_n257G/H CA_n259G/H/I/J/K/L CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n79A-n257H-n259M | CA_n257G/H CA_n259G/H/I/J/K/L/M CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L/M |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257H |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| CA_n79A-n257I-n259A | CA_n257G/H/I CA_n79A-n257A/G/H/I CA_n79A-n259A |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | 50, 100, 200, 400 |  |  |  |  |
| CA_n79A-n257I-n259G | CA_n257G/H/I CA_n259G CA_n79A-n257A/G/H/I CA_n79A-n259A/G |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259G |  |  |  |  |
| CA_n79A-n257I-n259H | CA_n257G/H/I CA_n259G/H CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259H |  |  |  |  |
| CA_n79A-n257I-n259I | CA_n257G/H/I CA_n259G/H/I CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259I |  |  |  |  |
| CA_n79A-n257I-n259J | CA_n257G/H/I CA_n259G/H/I/J CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259J |  |  |  |  |
| CA_n79A-n257I-n259K | CA_n257G/H/I CA_n259G/H/I/J/K CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259K |  |  |  |  |
| CA_n79A-n257I-n259L | CA_n257G/H/I CA_n259G/H/I/J/K/L CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259L |  |  |  |  |
| CA_n79A-n257I-n259M | CA_n257G/H/I CA_n259G/H/I/J/K/L/M CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L/M |  | n79 |  | 40, 50, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n257 |  | CA_n257I |  |  |  |  |
|  |  |  | n259 |  | CA_n259M |  |  |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively. NOTE 2: The CA configurations are given in Table 5.5A.1-1 of either TS 38.101-1 or TS 38.101-2 where unless otherwise stated BCS0 is referred to. NOTE 3: The delimiter “/” is only used in the uplink configurations for the sake of simplicity. For example, CA_nxA-nyA/B/C denotes CA_nxA-nyA, CA_nxA-nyB and CA_nxA-nyC, where nx and ny are two NR bands, ny is a FR2 band and A, B and C are the corresponding bandwidth classes respectively. |  |  |  |  |  |  |  |  |  |

Table 5.5A.1-3: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (four bands)

| NR CA configuration | Uplink configuration |  | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n8A-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n8A-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n8A-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n8A-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n8A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n3A-n8A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n3A-n8A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n3A-n8A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n3A-n28A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n257A CA_n3A-n28A CA_n3A-n257A CA_n28A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n28A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n257A/G CA_n3A-n28A CA_n3A-n257A/G CA_n28A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n28A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n257A/G/H CA_n3A-n28A CA_n3A-n257A/G/H CA_n28A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n28A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n257A/G/H/I CA_n3A-n28A CA_n3A-n257A/G/H/I CA_n28A-n257A/G/H/I |  | n1 | 5, 10,15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n41A-n257A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n257A CA_n3A-n41A CA_n3A-n257A CA_n41A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n41A-n257G | CA_n1A-n3A CA_n1A-n41A CA_n1A-n257A/G CA_n3A-n41A CA_n3A-n257A/G CA_n41A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n41A-n257H | CA_n1A-n3A CA_n1A-n41A CA_n1A-n257A/G/H CA_n3A-n41A CA_n3A-n257A/G/H CA_n41A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n41A-n257I | CA_n1A-n3A CA_n1A-n41A CA_n1A-n257A/G/H/I CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n3A-n41A CA_n3A-n257A CA_n41A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n77A-n257A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n257A CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n77A-n257G | CA_n1A-n3A CA_n1A-n77A CA_n1A-n257A/G CA_n3A-n77A CA_n3A-n257A/G CA_n77A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n77A-n257H | -CA_n1A-n3A CA_n1A-n77A CA_n1A-n257A/G/H CA_n3A-n77A CA_n3A-n257A/G/H CA_n77A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n77A-n257I | CA_n1A-n3A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G/G/H/I CA_n3A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n77A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n3A-n77A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n3A-n77A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n3A-n77A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n3A-n77(2A)-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n77(2A)-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n77(2A)-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n77(2A)-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n77(2A)-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n3A-n77(2A)-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n3A-n77(2A)-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n3A-n77(2A)-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n3A-n79A-n257A | CA_n1A-n3A CA_n1A-n79A CA_n1A-n257A CA_n3A-n79A CA_n3A-n257A CA_n79A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n79A-n257G | CA_n1A-n3A CA_n1A-n79A CA_n1A-n257A/G CA_n3A-n79A CA_n3A-n257A/G CA_n79A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n79A-n257H | CA_n1A-n3A CA_n1A-n79A CA_n1A-n257A/G/H CA_n3A-n79A CA_n3A-n257A/G/H CA_n79A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25 ,30 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n79A-n257I | CA_n1A-n3A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n8A-n77A-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n8A-n77A-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n8A-n77A-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90,100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n8A-n77A-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90,100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n8A-n77A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n8A-n77A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n8A-n77A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n8A-n77A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n8A-n77(2A)-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n8A-n77(2A)-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n8A-n77(2A)-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n8A-n77(2A)-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n8A-n77(2A)-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n8A-n77(2A)-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n8A-n77(2A)-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n8A-n77(2A)-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n8A-n78A-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n8A-n78A-n257D | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257D |  |
| CA_n1A-n8A-n78A-n257E | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257E |  |
| CA_n1A-n8A-n78A-n257F | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257F |  |
| CA_n1A-n8A-n78A-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n8A-n78A-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n8A-n78A-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n8A-n78A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n8A-n78A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n8A-n78A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n8A-n78A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n28A-n41A-n257A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A CA_n28A-n41A CA_n28A-n257A CA_n41A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n28A-n41A-n257G | CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G CA_n28A-n41A CA_n28A-n257A/G CA_n41A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n28A-n41A-n257H | CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G/H CA_n28A-n41A CA_n28A-n257A/G/H CA_n41A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n28A-n41A-n257I | CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G/H/I CA_n28A-n41A CA_n28A-n257A/G/H/I CA_n41A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n28A-n77A-n257A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n28A-n77A-n257G | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n28A-n77A-n257H | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n28A-n77A-n257I | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n28A-n77(2A)-n257A |  | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n28A-n77(2A)-n257G |  | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n28A-n77(2A)-n257H |  | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n28A-n77(2A)-n257I |  | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n28A-n79A-n257A | CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n28A-n79A-n257G | CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G CA_n28A-n79A CA_n28A-n257A/G CA_n79A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n28A-n79A-n257H | CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G/H CA_n28A-n79A CA_n28A-n257A/G/H CA_n79A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n28A-n79A-n257I | CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n41A-n77A-n257A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n41A-n77A-n257G | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n41A-n77A-n257H | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n41A-n77A-n257I | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n41A-n77(2A)-n257A |  | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n41A-n77(2A)-n257G |  | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n41A-n77(2A)-n257H |  | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n41A-n77(2A)-n257I |  | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n41A-n79A-n257A |  | CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n41A-n79A-n257G |  | CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n41A-n79A-n257H |  | CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n41A-n79A-n257I |  | CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n77A-n79A-n257A | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n77A-n79A-n257G | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G CA_n257G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n77A-n79A-n257H | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H CA_n257G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n77A-n79A-n257I | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I CA_n257G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n77(2A)-n79A-n257A |  | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n77(2A)-n79A-n257G |  | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n77(2A)-n79A-n257H |  | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n77(2A)-n79A-n257I |  | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n78A-n79A-n257A | CA_n1A-n78A CA_n1A-n79A CA_n1A-n257A CA_n78A-n79A CA_n78A-n257A CA_n79A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n78A-n79A-n257G | CA_n1A-n78A CA_n1A-n79A CA_n1A-n257A/G CA_n78A-n79A CA_n78A-n257A/G CA_n79A-n257A/G CA_n257G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n78A-n79A-n257H | CA_n1A-n78A CA_n1A-n79A CA_n1A-n257A/G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n79A-n257A/G/H CA_n257G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n78A-n79A-n257I | CA_n1A-n78A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n79A-n257A/G/H/I CA_n257G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n2A-n5A-n48A-n260A | CA_n2A-n260A CA_n5A-n260A CA_n48A-n260A |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n48A-n260G | CA_n2A-n260A CA_n5A-n260A CA_n48A-n260A CA_n2A-n260G CA_n5A-n260G CA_n48A-n260G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n2A-n5A-n48A-n260H | CA_n2A-n260A CA_n5A-n260A CA_n48A-n260A CA_n2A-n260G CA_n5A-n260G CA_n48A-n260G CA_n2A-n260H CA_n5A-n260H CA_n48A-n260H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n2A-n5A-n48A-n260I | CA_n2A_n260A/G/H/I CA_n5A_n260A/G/H/I CA_n48A_n260A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n5A-n48A-n260J | CA_n2A_n260A/G/H/I CA_n5A_n260A/G/H/I CA_n48A_n260A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n5A-n48A-n260K | CA_n2A_n260A/G/H/I CA_n5A_n260A/G/H/I CA_n48A_n260A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n5A-n48A-n260L | CA_n2A_n260A/G/H/I CA_n5A_n260A/G/H/I CA_n48A_n260A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n5A-n48A-n260M | CA_n2A_n260A/G/H/I CA_n5A_n260A/G/H/I CA_n48A_n260A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n5A-n48A-n261A | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n48A-n261G | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n2A-n5A-n48A-n261H | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n2A-n5A-n48A-n261I | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n2A-n5A-n48A-n261J | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n2A-n5A-n48A-n261K | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n2A-n5A-n48A-n261L | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n2A-n5A-n48A-n261M | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n2A-n5A-n48A-n261(G-H) | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n5A-n48A-n261(2H) | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n5A-n48A-n261(A-G-H) | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n5A-n48A-n261(H-I) | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n5A-n48A-n261(A-G-I) | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n5A-n48A-n261(A-G) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n5A-n48A-n261(A-H) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n5A-n48A-n261(A-I) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H CA_n2A-n261I CA_n5A-n261I CA_n48A-n261I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n5A-n48A-n261(2A-G) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n2A-n5A-n48A-n261(2A-H) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n5A-n48A-n261(2A-I) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H CA_n2A-n261I CA_n5A-n261I CA_n48A-n261I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n5A-n48A-n261(A-2G) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n2A-n5A-n48A-n261(G-H) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n5A-n48A-n261(G-I) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H CA_n2A-n261I CA_n5A-n261I CA_n48A-n261I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n5A-n48A-n261(2A) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n2A-n5A-n48A-n261(3A) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n2A-n5A-n48A-n261(2G) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n5A-n48A-n261(2H) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H CA_n2A-n261I CA_n5A-n261I CA_n48A-n261I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n5A-n48A-n261(A-G-H) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H CA_n2A-n261I CA_n5A-n261I CA_n48A-n261I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n5A-n48A-n261(H-I) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H CA_n2A-n261I CA_n5A-n261I CA_n48A-n261I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n5A-n48A-n261(A-G-I) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A CA_n2A-n261G CA_n5A-n261G CA_n48A-n261G CA_n2A-n261H CA_n5A-n261H CA_n48A-n261H CA_n2A-n261I CA_n5A-n261I CA_n48A-n261I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n5A-n66A-n260A |  | CA_n2A-n260A CA_n5A-n260A CA_n66A-n260A | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n66A-n260G |  | CA_n2A-n260A CA_n2A-n260G CA_n5A-n260A CA_n5A-n260G CA_n66A-n260A CA_n66A-n260G | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n2A-n5A-n66A-n260H |  | CA_n2A-n260A CA_n2A-n260G CA_n2A-n260H CA_n5A-n260A CA_n5A-n260G CA_n5A-n260H CA_n66A-n260A CA_n66A-n260G CA_n66A-n260H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n2A-n5A-n66A-n260I |  | CA_n2A-n260A CA_n2A-n260G CA_n2A-n260H CA_n2A-n260I CA_n5A-n260A CA_n5A-n260G CA_n5A-n260H CA_n5A-n260I CA_n66A-n260A CA_n66A-n260G CA_n66A-n260H CA_n66A-n260I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n5A-n66A-n260J |  | CA_n2A-n260A CA_n2A-n260G CA_n2A-n260H CA_n2A-n260I CA_n5A-n260A CA_n5A-n260G CA_n5A-n260H CA_n5A-n260I CA_n66A-n260A CA_n66A-n260G CA_n66A-n260H CA_n66A-n260I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n5A-n66A-n260K |  | CA_n2A-n260A CA_n2A-n260G CA_n2A-n260H CA_n2A-n260I CA_n5A-n260A CA_n5A-n260G CA_n5A-n260H CA_n5A-n260I CA_n66A-n260A CA_n66A-n260G CA_n66A-n260H CA_n66A-n260I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n5A-n66A-n260L |  | CA_n2A-n260A CA_n2A-n260G CA_n2A-n260H CA_n2A-n260I CA_n5A-n260A CA_n5A-n260G CA_n5A-n260H CA_n5A-n260I CA_n66A-n260A CA_n66A-n260G CA_n66A-n260H CA_n66A-n260I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n5A-n66A-n260M |  | CA_n2A-n260A CA_n2A-n260G CA_n2A-n260H CA_n2A-n260I CA_n5A-n260A CA_n5A-n260G CA_n5A-n260H CA_n5A-n260I CA_n66A-n260A CA_n66A-n260G CA_n66A-n260H CA_n66A-n260I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n5A-n66A-n261A |  | CA_n2A-n261A CA_n5A-n261A CA_n66A-n261A | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n66A-n261G |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n2A-n5A-n66A-n261H |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n2A-n5A-n66A-n261I |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n2A-n5A-n66A-n261J |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n2A-n5A-n66A-n261K |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n2A-n5A-n66A-n261L |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n2A-n5A-n66A-n261M |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n2A-n5A-n66A-n261(A-G) |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n5A-n66A-n261(2G) |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n5A-n66A-n261(A-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n5A-n66A-n261(G-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n5A-n66A-n261(2A-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n5A-n66A-n261(A-2G) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n2A-n5A-n66A-n261(A-G-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n5A-n66A-n261(A-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n5A-n66A-n261(G-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n5A-n66A-n261(2H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n5A-n66A-n261(2A-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n5A-n66A-n261(A-G-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n5A-n66A-n261(H-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n5A-n77A-n260A |  | CA_n2A-n260A CA_n5A-n260A CA_n77A-n260A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n77A-n260I |  | CA_n2A_n260A/G/H/I CA_n5A_n260A/G/H/I CA_n77A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n5A-n77A-n260J |  | CA_n2A_n260A/G/H/I CA_n5A_n260A/G/H/I CA_n77A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n5A-n77A-n260K |  | CA_n2A_n260A/G/H/I CA_n5A_n260A/G/H/I CA_n77A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n5A-n77A-n260L |  | CA_n2A_n260A/G/H/I CA_n5A_n260A/G/H/I CA_n77A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n5A-n77A-n260M |  | CA_n2A_n260A/G/H/I CA_n5A_n260A/G/H/I CA_n77A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n5A-n77A-n260G |  | CA_n2A-n260A CA_n5A-n260A CA_n77A-n260A CA_n2A-n260G CA_n5A-n260G CA_n77A-n260G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n2A-n5A-n77A-n260H |  | CA_n2A-n260A CA_n5A-n260A CA_n77A-n260A CA_n2A-n260G CA_n5A-n260G CA_n77A-n260G CA_n2A-n260H CA_n5A-n260H CA_n77A-n260H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n2A-n5A-n77A-n260I |  | CA_n2A-n260A CA_n5A-n260A CA_n77A-n260A CA_n2A-n260G CA_n5A-n260G CA_n77A-n260G CA_n2A-n260H CA_n5A-n260H CA_n77A-n260H CA_n2A-n260I CA_n5A-n260I CA_n77A-n260I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n5A-n77A-n260J |  | CA_n2A-n260A CA_n5A-n260A CA_n77A-n260A CA_n2A-n260G CA_n5A-n260G CA_n77A-n260G CA_n2A-n260H CA_n5A-n260H CA_n77A-n260H CA_n2A-n260I CA_n5A-n260I CA_n77A-n260I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n5A-n77A-n260K |  | CA_n2A-n260A CA_n5A-n260A CA_n77A-n260A CA_n2A-n260G CA_n5A-n260G CA_n77A-n260G CA_n2A-n260H CA_n5A-n260H CA_n77A-n260H CA_n2A-n260I CA_n5A-n260I CA_n77A-n260I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n5A-n77A-n260L |  | CA_n2A-n260A CA_n5A-n260A CA_n77A-n260A CA_n2A-n260G CA_n5A-n260G CA_n77A-n260G CA_n2A-n260H CA_n5A-n260H CA_n77A-n260H CA_n2A-n260I CA_n5A-n260I CA_n77A-n260I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n5A-n77A-n260M |  | CA_n2A-n260A CA_n5A-n260A CA_n77A-n260A CA_n2A-n260G CA_n5A-n260G CA_n77A-n260G CA_n2A-n260H CA_n5A-n260H CA_n77A-n260H CA_n2A-n260I CA_n5A-n260I CA_n77A-n260I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n5A-n77A-n261A |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n77A-n261G |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n2A-n5A-n77A-n261H |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G CA_n2A-n261H CA_n5A-n261H CA_n77A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n2A-n5A-n77A-n261I |  | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n2A-n5A-n77A-n261J |  | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n2A-n5A-n77A-n261K |  | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n2A-n5A-n77A-n261L |  | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n2A-n5A-n77A-n261M |  | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n2A-n5A-n77A-n261(G-I) |  | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n5A-n77A-n261(2H) |  | CA_n2A_n261A/G/H CA_n5A_n261A/G/H CA_n77A_n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n5A-n77A-n261(A-G-H) |  | CA_n2A_n261A/G/H CA_n5A_n261A/G/H CA_n77A_n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n5A-n77A-n261(H-I) |  | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n5A-n77A-n261(A-G-I) |  | CA_n2A_n261A/G/H/I CA_n5A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n48A-n66A-n260A |  | CA_n2A_n260A CA_n48A_n260 CA_n66A_n260A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n48A-n66A-n260I |  | CA_n2A_n260A/G/H/I CA_n48A_n260A/G/H/I CA_n66A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n48A-n66A-n260J |  | CA_n2A_n260A/G/H/I CA_n48A_n260A/G/H/I CA_n66A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n48A-n66A-n260K |  | CA_n2A_n260A/G/H/I CA_n48A_n260A/G/H/I CA_n66A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n48A-n66A-n260L |  | CA_n2A_n260A/G/H/I CA_n48A_n260A/G/H/I CA_n66A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n48A-n66A-n260M |  | CA_n2A_n260A/G/H/I CA_n48A_n260A/G/H/I CA_n66A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n48A-n66A-n261A |  | CA_n2A_n261A CA_n66A_n261A CA_n48A_n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n48A-n66A-n261I |  | CA_n2A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n2A-n48A-n66A-n261J |  | CA_n2A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n2A-n48A-n66A-n261K |  | CA_n2A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n2A-n48A-n66A-n261L |  | CA_n2A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n2A-n48A-n66A-n261M |  | CA_n2A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n2A-n48A-n66A-n261(G-H) |  | CA_n2A_n261A/G/H CA_n48A_n261A/G/H CA_n66A_n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n48A-n66A-n261(2H) |  | CA_n2A_n261A/G/H CA_n48A_n261A/G/H CA_n66A_n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n48A-n66A-n261(A-G-H) |  | CA_n2A_n261A/G/H CA_n48A_n261A/G/H CA_n66A_n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n48A-n66A-n261(H-I) |  | CA_n2A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n48A-n66A-n261(A-G-I) |  | CA_n2A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n66A-n77A-n260A |  | CA_n2A_n260A CA_n66A_n260A CA_n77A_n260A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n66A-n77A-n260I |  | CA_n2A_n260A/G/H/I CA_n66A_n260A/G/H/I CA_n77A_n260A/G/H/ I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n66A-n77A-n260J |  | CA_n2A_n260A/G/H/I CA_n66A_n260A/G/H/I CA_n77A_n260A/G/H/ I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n66A-n77A-n260K |  | CA_n2A_n260A/G/H/I CA_n66A_n260A/G/H/I CA_n77A_n260A/G/H/ | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n66A-n77A-n260L |  | CA_n2A_n260A/G/H/I CA_n66A_n260A/G/H/I CA_n77A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n66A-n77A-n260M |  | CA_n2A_n260A/G/H/I CA_n66A_n260A/G/H/I CA_n77A_n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n66A-n77A-n261A |  | CA_n2A_n261A CA_n66A_n261A CA_n77A_n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n66A-n77A-n261I |  | CA_n2A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n2A-n66A-n77A-n261J |  | CA_n2A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n2A-n66A-n77A-n261K |  | CA_n2A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n2A-n66A-n77A-n261L |  | CA_n2A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n2A-n66A-n77A-n261M |  | CA_n2A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n2A-n66A-n77A-n261(G-I) |  | CA_n2A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n66A-n77A-n261(2H) |  | CA_n2A_n261A/G/H CA_n66A_n261A/G/H CA_n77A_n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n66A-n77A-n261(A-G-H) |  | CA_n2A_n261A/G/H CA_n66A_n261A/G/H CA_n77A_n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n66A-n77A-n261(H-I) |  | CA_n2A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n66A-n77A-n261(A-G-I) |  | CA_n2A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n5A-n77A-n261(A-G) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n5A-n77A-n261(A-H) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G CA_n2A-n261H CA_n5A-n261H CA_n77A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n5A-n77A-n261(A-I) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G CA_n2A-n261H CA_n5A-n261H CA_n77A-n261H CA_n2A-n261I CA_n5A-n261I CA_n77A-n261I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n5A-n77A-n261(2A-H) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G CA_n2A-n261H CA_n5A-n261H CA_n77A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n5A-n77A-n261(2A-I) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G CA_n2A-n261H CA_n5A-n261H CA_n77A-n261H CA_n2A-n261I CA_n5A-n261I CA_n77A-n261I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n5A-n77A-n261(G-H) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G CA_n2A-n261H CA_n5A-n261H CA_n77A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n5A-n77A-n261(2A) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n2A-n5A-n77A-n261(3A) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n2A-n5A-n77A-n261(2G) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n5A-n77A-n261(A-2G) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n2A-n5A-n77A-n261(2A-G) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A CA_n2A-n261G CA_n5A-n261G CA_n77A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n2A-n48A-n66A-n260G |  | CA_n2A-n260A CA_n48A-n260A CA_n66A-n260A CA_n2A-n260G CA_n48A-n260G CA_n66A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n2A-n48A-n66A-n260H |  | CA_n2A-n260A CA_n48A-n260A CA_n66A-n260A CA_n2A-n260G CA_n48A-n260G CA_n66A-n261G CA_n2A-n260H CA_n48A-n260H CA_n66A-n260H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n2A-n48A-n66A-n261G |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n2A-n48A-n66A-n261H |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G CA_n2A-n261H CA_n48A-n261H CA_n66A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n2A-n48A-n66A-n261(A-G) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n48A-n66A-n261(A-H) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G CA_n2A-n261H CA_n48A-n261H CA_n66A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n48A-n66A-n261(A-I) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G CA_n2A-n261H CA_n48A-n261H CA_n66A-n261H CA_n2A-n261I CA_n48A-n261I CA_n66A-n261I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n48A-n66A-n261(2A-G) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n2A-n48A-n66A-n261(2A-H) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G CA_n2A-n261H CA_n48A-n261H CA_n66A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n48A-n66A-n261(2A-I) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G CA_n2A-n261H CA_n48A-n261H CA_n66A-n261H CA_n2A-n261I CA_n48A-n261I CA_n66A-n261I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n48A-n66A-n261(G-I) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G CA_n2A-n261H CA_n48A-n261H CA_n66A-n261H CA_n2A-n261I CA_n48A-n261I CA_n66A-n261I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n48A-n66A-n261(2A) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n2A-n48A-n66A-n261(3A) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n2A-n48A-n66A-n261(2G) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n48A-n66A-n261(A-2G) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A CA_n2A-n261G CA_n48A-n261G CA_n66A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n66A-n77A-n260G |  | CA_n2A-n260A CA_n66A-n260A CA_n77A-n260A CA_n2A-n260G CA_n66A-n260G CA_n77A-n260G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n2A-n66A-n77A-n260H |  | CA_n2A-n260A CA_n66A-n260A CA_n77A-n260A CA_n2A-n260G CA_n66A-n260G CA_n77A-n260G CA_n2A-n260H CA_n66A-n260H CA_n77A-n260H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n2A-n66A-n77A-n261G |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n2A-n66A-n77A-n261H |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G CA_n2A-n261H CA_n66A-n261H CA_n77A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n2A-n66A-n77A-n261(A-G) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n66A-n77A-n261(A-H) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G CA_n2A-n261H CA_n66A-n261H CA_n77A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n66A-n77A-n261(A-I) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G CA_n2A-n261H CA_n66A-n261H CA_n77A-n261H CA_n2A-n261I CA_n66A-n261I CA_n77A-n261I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n66A-n77A-n261(2A-G) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n2A-n66A-n77A-n261(2A-H) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G CA_n2A-n261H CA_n66A-n261H CA_n77A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n66A-n77A-n261(2A-I) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G CA_n2A-n261H CA_n66A-n261H CA_n77A-n261H CA_n2A-n261I CA_n66A-n261I CA_n77A-n261I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n66A-n77A-n261(G-H) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G CA_n2A-n261H CA_n66A-n261H CA_n77A-n261H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n66A-n77A-n261(2A) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n2A-n66A-n77A-n261(3A) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n2A-n66A-n77A-n261(2G) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n66A-n77A-n261(A-2G) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A CA_n2A-n261G CA_n66A-n261G CA_n77A-n261G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n3A-n7A-n78A-n258A | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3A-n7A-n78A-n258B | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258B |  |
| CA_n3A-n7A-n78A-n258C | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258C |  |
| CA_n3A-n7A-n78A-n258D | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258D |  |
| CA_n3A-n7A-n78A-n258E | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258E |  |
| CA_n3A-n7A-n78A-n258F | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258F |  |
| CA_n3A-n7A-n78A-n258G | CA_n3A-n258A/G CA_n7A-n258A/G CA_n78A-n258A/G CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258G |  |
| CA_n3A-n7A-n78A-n258H | CA_n3A-n258A/G/H CA_n7A-n258A/G/H CA_n78A-n258A/G/H CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258H |  |
| CA_n3A-n7A-n78A-n258I | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258I |  |
| CA_n3A-n7A-n78A-n258J | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258J |  |
| CA_n3A-n7A-n78A-n258K | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258K |  |
| CA_n3A-n7A-n78A-n258L | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258L |  |
| CA_n3A-n7A-n78A-n258M | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258M |  |
| CA_n3A-n7B-n78A-n258A | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3A-n7B-n78A-n258B | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258B |  |
| CA_n3A-n7B-n78A-n258C | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258C |  |
| CA_n3A-n7B-n78A-n258D | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258D |  |
| CA_n3A-n7B-n78A-n258E | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258E |  |
| CA_n3A-n7B-n78A-n258F | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258F |  |
| CA_n3A-n7B-n78A-n258G | CA_n3A-n258A/G CA_n7A-n258A/G CA_n78A-n258A/G CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258G |  |
| CA_n3A-n7B-n78A-n258H | CA_n3A-n258A/G/H CA_n7A-n258A/G/H CA_n78A-n258A/G/H CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258H |  |
| CA_n3A-n7B-n78A-n258I | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258I |  |
| CA_n3A-n7B-n78A-n258J | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258J |  |
| CA_n3A-n7B-n78A-n258K | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258K |  |
| CA_n3A-n7B-n78A-n258L | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258L |  |
| CA_n3A-n7B-n78A-n258M | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258M |  |
| CA_n3A-n8A-n77A-n257A | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n8A-n77A-n257G | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n8A-n77A-n257H | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n8A-n77A-n257I | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n8A-n77A-n257J | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n3A-n8A-n77A-n257K | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n3A-n8A-n77A-n257L | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n3A-n8A-n77A-n257M | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n3A-n8A-n77(2A)-n257A | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n8A-n77(2A)-n257G | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n8A-n77(2A)-n257H | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n8A-n77(2A)-n257I | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n8A-n77(2A)-n257J | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n3A-n8A-n77(2A)-n257K | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n3A-n8A-n77(2A)-n257L | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n3A-n8A-n77(2A)-n257M | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n3A-n28A-n41A-n257A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A CA_n28A-n41A CA_n28A-n257A CA_n41A-n257A |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n5A-n48A-n66A-n260A | CA_n5A-n260A CA_n48A-n260A CA_n66A-n260A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n5A-n48A-n66A-n260G | CA_n5A-n260A CA_n48A-n260A CA_n66A-n260A CA_n5A-n260G CA_n48A-n260G CA_n66A-n260G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n5A-n48A-n66A-n260H | CA_n5A-n260A CA_n48A-n260A CA_n66A-n260A CA_n5A-n260G CA_n48A-n260G CA_n66A-n260G CA_n5A-n260H CA_n48A-n260H CA_n66A-n260H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n5A-n48A-n66A-n260I | CA_n5A-n260A CA_n48A-n260A CA_n66A-n260A CA_n5A-n260G CA_n48A-n260G CA_n66A-n260G CA_n5A-n260H CA_n48A-n260H CA_n66A-n260H CA_n5A-n260I CA_n48A-n260I CA_n66A-n260I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n5A-n48A-n66A-n260J | CA_n5A-n260A CA_n48A-n260A CA_n66A-n260A CA_n5A-n260G CA_n48A-n260G CA_n66A-n260G CA_n5A-n260H CA_n48A-n260H CA_n66A-n260H CA_n5A-n260I CA_n48A-n260I CA_n66A-n260I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n5A-n48A-n66A-n260K | CA_n5A-n260A CA_n48A-n260A CA_n66A-n260A CA_n5A-n260G CA_n48A-n260G CA_n66A-n260G CA_n5A-n260H CA_n48A-n260H CA_n66A-n260H CA_n5A-n260I CA_n48A-n260I CA_n66A-n260I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n5A-n48A-n66A-n260L | CA_n5A-n260A CA_n48A-n260A CA_n66A-n260A CA_n5A-n260G CA_n48A-n260G CA_n66A-n260G CA_n5A-n260H CA_n48A-n260H CA_n66A-n260H CA_n5A-n260I CA_n48A-n260I CA_n66A-n260I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n5A-n48A-n66A-n260M | CA_n5A-n260A CA_n48A-n260A CA_n66A-n260A CA_n5A-n260G CA_n48A-n260G CA_n66A-n260G CA_n5A-n260H CA_n48A-n260H CA_n66A-n260H CA_n5A-n260I CA_n48A-n260I CA_n66A-n260I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n5A-n48A-n66A-n261A | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n5A-n48A-n66A-n261G | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n5A-n48A-n66A-n261H | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n5A-n48A-n66A-n261I | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H CA_n5A-n261I CA_n48A-n261I CA_n66A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n5A-n48A-n66A-n261J | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H CA_n5A-n261I CA_n48A-n261I CA_n66A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n5A-n48A-n66A-n261K | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H CA_n5A-n261I CA_n48A-n261I CA_n66A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n5A-n48A-n66A-n261L | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H CA_n5A-n261I CA_n48A-n261I CA_n66A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n5A-n48A-n66A-n261M | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H CA_n5A-n261I CA_n48A-n261I CA_n66A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n5A-n48A-n66A-n261(A-G) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n5A-n48A-n66A-n261(A-H) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n5A-n48A-n66A-n261(A-I) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H CA_n5A-n261I CA_n48A-n261I CA_n66A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n5A-n48A-n66A-n261(2A-G) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n5A-n48A-n66A-n261(2A-H) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n5A-n48A-n66A-n261(2A-I) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H CA_n5A-n261I CA_n48A-n261I CA_n66A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n5A-n48A-n66A-n261(G-I) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G CA_n5A-n261H CA_n48A-n261H CA_n66A-n261H CA_n5A-n261I CA_n48A-n261I CA_n66A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n5A-n48A-n66A-n261(2A) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n5A-n48A-n66A-n261(3A) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n5A-n48A-n66A-n261(2G) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n5A-n48A-n66A-n261(A-2G) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A CA_n5A-n261G CA_n48A-n261G CA_n66A-n261G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n5A-n66A-n77A-n260G | CA_n2A-n260A CA_n66A-n260A CA_n77A-n260A CA_n2A-n260G CA_n66A-n260G CA_n77A-n260G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n5A-n66A-n77A-n260H | CA_n2A-n260A CA_n66A-n260A CA_n77A-n260A CA_n2A-n260G CA_n66A-n260G CA_n77A-n260G CA_n2A-n260H CA_n66A-n260H CA_n77A-n260H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n5A-n66A-n77A-n261G | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n5A-n66A-n77A-n261H | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G CA_n5A-n261H CA_n66A-n261H CA_n77A-n261H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n5A-n66A-n77A-n261(A-G) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n5A-n66A-n77A-n261(A-H) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G CA_n5A-n261H CA_n66A-n261H CA_n77A-n261H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n5A-n66A-n77A-n261(A-I) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G CA_n5A-n261H CA_n66A-n261H CA_n77A-n261H CA_n5A-n261I CA_n66A-n261I CA_n77A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n5A-n66A-n77A-n261(A-2G) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n5A-n66A-n77A-n261(2A-G) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n5A-n66A-n77A-n261(2A-H) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G CA_n5A-n261H CA_n66A-n261H CA_n77A-n261H CA_n5A-n261I CA_n66A-n261I CA_n77A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n5A-n66A-n77A-n261(2A-I) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G CA_n5A-n261H CA_n66A-n261H CA_n77A-n261H CA_n5A-n261I CA_n66A-n261I CA_n77A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n5A-n66A-n77A-n261(G-I) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G CA_n5A-n261H CA_n66A-n261H CA_n77A-n261H CA_n5A-n261I CA_n66A-n261I CA_n77A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n5A-n66A-n77A-n261(2A) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G CA_n5A-n261H CA_n66A-n261H CA_n77A-n261H CA_n5A-n261I CA_n66A-n261I CA_n77A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n5A-n66A-n77A-n261(3A) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G CA_n5A-n261H CA_n66A-n261H CA_n77A-n261H CA_n5A-n261I CA_n66A-n261I CA_n77A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n5A-n66A-n77A-n261(2G) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A CA_n5A-n261G CA_n66A-n261G CA_n77A-n261G CA_n5A-n261H CA_n66A-n261H CA_n77A-n261H CA_n5A-n261I CA_n66A-n261I CA_n77A-n261I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n3A-n28A-n41A-n257G | CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G CA_n28A-n41A CA_n28A-n257A/G CA_n41A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n41A-n257H | CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G/H CA_n28A-n41A CA_n28A-n257A/G/H CA_n41A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n41A-n257I | CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G/H/I CA_n28A-n41A CA_n28A-n257A/G/H/I CA_n41A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n77A-n257A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n257A CA_n28A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n77A-n257D | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257D |  |
| CA_n3A-n28A-n77A-n257G | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n257A/G CA_n28A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n77A-n257H | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n257A/G/H CA_n28A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n77A-n257I | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n257A/G/H/I CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n77(2A)-n257A | CA_n3A-n257A CA_n28A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n77(2A)-n257D | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257D |  |
| CA_n3A-n28A-n77(2A)-n257G | CA_n3A-n257A/G CA_n28A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n77(2A)-n257H | CA_n3A-n257A/G/H CA_n28A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n77(2A)-n257I | CA_n3A-n257A/G/H/I CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n77(3A)-n257A | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(3A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n77(3A)-n257D | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(3A) |  |
|  |  |  | n257 | CA_n257D |  |
| CA_n3A-n28A-n77(3A)-n257G | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(3A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n77(3A)-n257H | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(3A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n77(3A)-n257I | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(3A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n78A-n257A | CA_n28A-n257A CA_n78A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n78A-n257D | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257D |  |
| CA_n3A-n28A-n78A-n257G | CA_n3A-n257A/G CA_n28A-n257A/G CA_n78A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n78A-n257H | CA_n3A-n257A/G/H CA_n28A-n257A/G/H CA_n78A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n78A-n257I | CA_n3A-n257A/G/H/I CA_n28A-n257A/G/H/I CA_n78A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n79A-n257A | CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n79A-n257G | CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G CA_n28A-n79A CA_n28A-n257A/G CA_n79A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n79A-n257H | CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G/H CA_n28A-n79A CA_n28A-n257A/G/H CA_n79A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n79A-n257I | CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n79A-n257A/G/H /I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n41A-n77A-n257A | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n41A-n77A-n257G | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n41A-n77A-n257H | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n41A-n77A-n257I | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n41A-n77(2A)-n257A |  | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n41A-n77(2A)-n257G |  | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n41A-n77(2A)-n257H |  | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n41A-n77(2A)-n257I |  | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n41A-n79A-n257A |  | CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n3 | 10, 15, 20, 25, 30 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n41A-n79A-n257G |  | A_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n3 | 10, 15, 20, 25, 30 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n41A-n79A-n257H |  | CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n3 | 10, 15, 20, 25, 30 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n41A-n79A-n257I |  | CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n3 | 10, 15, 20, 25, 30 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n77A-n79A-n257A | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n77A-n79A-n257G | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n77A-n79A-n257H | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n77A-n79A-n257I | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n77(2A)-n79A-n257A | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n77(2A)-n79A-n257G | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n77(2A)-n79A-n257H | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n77(2A)-n79A-n257I | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n5A-n48A-n66A-n260A | CA_n5A_n260A CA_n48A_n260A CA_n66A_n260A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n5A-n48A-n66A-n260I | CA_n5A_n260A/G/H/I CA_n48A_n260A/G/H/I CA_n66A_n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n5A-n48A-n66A-n260J | CA_n5A_n260A/G/H/I CA_n48A_n260A/G/H/I CA_n66A_n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n5A-n48A-n66A-n260K | CA_n5A_n260A/G/H/I CA_n48A_n260A/G/H/I CA_n66A_n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n5A-n48A-n66A-n260L | CA_n5A_n260A/G/H/I CA_n48A_n260A/G/H/I CA_n66A_n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n5A-n48A-n66A-n260M | CA_n5A_n260A/G/H/I CA_n48A_n260A/G/H/I CA_n66A_n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n5A-n48A-n66A-n261A | CA_n5A_n261A CA_n48A_n261A CA_n66A_n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n5A-n48A-n66A-n261I | CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n5A-n48A-n66A-n261J | CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n5A-n48A-n66A-n261K | CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n5A-n48A-n66A-n261L | CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n5A-n48A-n66A-n261M | CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n5A-n48A-n66A-n261(G-H) | CA_n5A_n261A/G/H CA_n48A_n261A/G/H CA_n66A_n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n5A-n48A-n66A-n261(2H) | CA_n5A_n261A/G/H CA_n48A_n261A/G/H CA_n66A_n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n5A-n48A-n66A-n261(A-G-H) | CA_n5A_n261A/G/H CA_n48A_n261A/G/H CA_n66A_n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n5A-n48A-n66A-n261(H-I) | CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H /I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n5A-n48A-n66A-n261(A-G-I) | CA_n5A_n261A/G/H/I CA_n48A_n261A/G/H/I CA_n66A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n5A-n66A-n77A-n260A | CA_n5A_n260A CA_n66A_n260A CA_n77A_n260A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n5A-n66A-n77A-n260I | CA_n2A_n260A/G/H/I CA_n66A_n260A/G/H/I CA_n77A_n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n5A-n66A-n77A-n260J | CA_n2A_n260A/G/H/I CA_n66A_n260A/G/H/I CA_n77A_n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n5A-n66A-n77A-n260K | CA_n2A_n260A/G/H/I CA_n66A_n260A/G/H/I CA_n77A_n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n5A-n66A-n77A-n260L | CA_n2A_n260A/G/H/I CA_n66A_n260A/G/H/I CA_n77A_n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n5A-n66A-n77A-n260M | CA_n2A_n260A/G/H/I CA_n66A_n260A/G/H/I CA_n77A_n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n5A-n66A-n77A-n261A | CA_n5A_n261A CA_n66A_n261A CA_n77A_n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n5A-n66A-n77A-n261I | CA_n5A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n5A-n66A-n77A-n261J | CA_n5A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n5A-n66A-n77A-n261K | CA_n5A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n5A-n66A-n77A-n261L | CA_n5A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n5A-n66A-n77A-n261M | CA_n5A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n5A-n66A-n77A-n261(G-H) | CA_n5A_n261A/G/H CA_n66A_n261A/G/H CA_n77A_n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n5A-n66A-n77A-n261(2H) | CA_n5A_n261A/G/H CA_n66A_n261A/G/H CA_n77A_n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n5A-n66A-n77A-n261(A-G-H) | CA_n5A_n261A/G/H CA_n66A_n261A/G/H CA_n77A_n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n5A-n66A-n77A-n261(H-I) | CA_n5A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n5A-n66A-n77A-n261(A-G-I) | CA_n5A_n261A/G/H/I CA_n66A_n261A/G/H/I CA_n77A_n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n28A-n41A-n77A-n257A | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n41A-n77A-n257G | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n41A-n77A-n257H | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n41A-n77A-n257I | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n28A-n41A-n77(2A)-n257A | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n41A-n77(2A)-n257G | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n41A-n77(2A)-n257H | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n41A-n77(2A)-n257I | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n28A-n41A-n79A-n257A |  | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n79A-n257A | n28 | 5, 10 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n41A-n79A-n257G |  | CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n28 | 5, 10 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n41A-n79A-n257H |  | CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n28 | 5, 10 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n41A-n79A-n257I |  | CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n28 | 5, 10 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n28A-n77A-n79A-n257A | CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n77A-n79A-n257G | CA_n257G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n77A-n79A-n257H | CA_n257G/H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/HI CA_n79A-n257A/G/H |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n77A-n79A-n257I | CA_n257G/H/I CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n28A-n77(2A)-n79A-n257A | CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n77(2A)-n79A-n257G | CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n77(2A)-n79A-n257H | CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n77(2A)-n79A-n257I | CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n28A-n78A-n79A-n257A | CA_n28A-n78A CA_n28A-n79A CA_n28A-n257A CA_n78A-n79A CA_n78A-n257A CA_n79A-n257A |  | n28 | 5, 10, 15 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 100, 200, 400 |  |
| CA_n28A-n78A-n79A-n257G | CA_n28A-n78A CA_n28A-n79A CA_n28A-n257A/G CA_n78A-n79A CA_n78A-n257A/G CA_n79A-n257A/G |  | n28 | 5, 10, 15 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n78A-n79A-n257H | CA_n28A-n78A CA_n28A-n79A CA_n28A-n257A/G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n79A-n257A/G/H |  | n28 | 5, 10, 15 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n78A-n79A-n257I | CA_n28A-n78A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n28 | 5, 10, 15 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n41A-n77A-n79A-n257A |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n41A-n77A-n79A-n257G |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n41A-n77A-n79A-n257H |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n41A-n77A-n79A-n257I |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n41A-n77(2A)-n79A-n257A |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n41A-n77(2A)-n79A-n257G |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n41A-n77(2A)-n79A-n257H |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n41A-n77(2A)-n79A-n257I |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n77A-n79A-n257A-n259A |  | CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A CA_n79A-n257A CA_n79A-n259A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n77A-n79A-n257A-n259G |  | CA_n259G CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G CA_n79A-n257A CA_n79A-n259A/G | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n77A-n79A-n257A-n259H |  | CA_n259G/H CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H CA_n79A-n257A CA_n79A-n259A/G/H | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n77A-n79A-n257A-n259I |  | CA_n259G/H/I CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H/I CA_n79A-n257A CA_n79A-n259A/G/H/I | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n77A-n79A-n257A-n259J |  | CA_n259G/H/I/J CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H/I/J CA_n79A-n257A CA_n79A-n259A/G/H/I/J | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n77A-n79A-n257A-n259K |  | CA_n259G/H/I/J/K CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n77A-n79A-n257A-n259L |  | CA_n259G/H/I/J/K/L CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K/L CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n77A-n79A-n257A-n259M |  | CA_n259G/H/I/J/K/L/M CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K/L/M CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L/M | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n77A-n79A-n257G-n259A |  | CA_n257G CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A CA_n79A-n257A/G CA_n79A-n259A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n77A-n79A-n257G-n259G |  | CA_n257G CA_n259G CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G CA_n79A-n257A/G CA_n79A-n259A/G | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n77A-n79A-n257G-n259H |  | CA_n257G CA_n259G/H CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H CA_n79A-n257A/G CA_n79A-n259A/G/H | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n77A-n79A-n257G-n259I |  | CA_n257G CA_n259G/H/I CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H/I CA_n79A-n257A/G CA_n79A-n259A/G/H/I I | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n77A-n79A-n257G-n259J |  | CA_n257G CA_n259G/H/I/J CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n77A-n79A-n257G-n259K |  | CA_n257G CA_n259G/H/I/J/K CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n77A-n79A-n257G-n259L |  | CA_n257G CA_n259G/H/I/J/K/L CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K/L CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n77A-n79A-n257G-n259M |  | CA_n257G CA_n259G/H/I/J/K/L/M CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L/M | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n77A-n79A-n257H-n259A |  | CA_n257G/H CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G/H CA_n77A-n259A CA_n79A-n257A CA_n79A-n257G/H CA_n79A-n259A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n77A-n79A-n257H-n259G |  | CA_n257G/H CA_n259G CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G CA_n79A-n257A/G/H CA_n79A-n259A/G | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n77A-n79A-n257H-n259H |  | CA_n257G/H CA_n259G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H CA_n79A-n257A/G/H CA_n79A-n259A/G/H | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n77A-n79A-n257H-n259I |  | CA_n257G/H CA_n259G/H/I CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n77A-n79A-n257H-n259J |  | CA_n257G/H CA_n259G/H/I/J CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n77A-n79A-n257H-n259K |  | CA_n257G/H CA_n259G/H/I/J/K CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n77A-n79A-n257H-n259L |  | CA_n257G/H CA_n259G/H/I/J/K/L CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K/L CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n77A-n79A-n257H-n259M |  | CA_n257G/H CA_n259G/H/I/J/K/L/M CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L/M | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n77A-n79A-n257I-n259A |  | CA_n257G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A CA_n79A-n257A/G/H/I CA_n79A-n259A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n77A-n79A-n257I-n259G |  | CA_n257G/H/I CA_n259G CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G CA_n79A-n257A/G/H/I CA_n79A-n259A/G | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n77A-n79A-n257I-n259H |  | CA_n257G/H/I CA_n259G/H CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n77A-n79A-n257I-n259I |  | CA_n257G/H/I CA_n259G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n77A-n79A-n257I-n259J |  | CA_n257G/H/I CA_n259G/H/I/J CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n77A-n79A-n257I-n259K |  | CA_n257G/H/I CA_n259G/H/I/J/K CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n77A-n79A-n257I-n259L |  | CA_n257G/H/I CA_n259G/H/I/J/K/L CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K/L CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n77A-n79A-n257I-n259M |  | CA_n257G/H/I CA_n259G/H/I/J/K/L/M CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L/M | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n78A-n79A-n257A-n259A |  | CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A CA_n79A-n257A CA_n79A-n259A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n78A-n79A-n257A-n259G |  | CA_n259G CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G CA_n79A-n257A CA_n79A-n29A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n78A-n79A-n257A-n259H |  | CA_n259G/H CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H CA_n79A-n257A CA_n79A-n259A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n78A-n79A-n257A-n259I |  | CA_n259G/H/I CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H/I CA_n79A-n257A CA_n79A-n259A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n78A-n79A-n257A-n259J |  | CA_n259G/H/I/J CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H/I/J CA_n79A-n257A CA_n79A-n259A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n78A-n79A-n257A-n259K |  | CA_n259G/H/I/J/K CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n78A-n79A-n257A-n259L |  | CA_n259G/H/I/J/K/L CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K/L CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n78A-n79A-n257A-n259M |  | CA_n259G/H/I/J/K/L/M CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K/L/M CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n78A-n79A-n257G-n259A |  | CA_n257G CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A CA_n79A-n257A/G CA_n79A-n259A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n78A-n79A-n257G-n259G |  | CA_n257G CA_n259G CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G CA_n79A-n257A/G CA_n79A-n259A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n78A-n79A-n257G-n259H |  | CA_n257G CA_n259G/H CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H CA_n79A-n257A/G CA_n79A-n259A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n78A-n79A-n257G-n259I |  | CA_n257G CA_n259G/H/I CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H/I CA_n79A-n257A/G CA_n79A-n259A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n78A-n79A-n257G-n259J |  | CA_n257G CA_n259G/H/I/J CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n78A-n79A-n257G-n259K |  | CA_n257G CA_n259G/H/I/J/K CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n78A-n79A-n257G-n259L |  | CA_n257G CA_n259G/H/I/J/K/L CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K/L CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n78A-n79A-n257G-n259M |  | CA_n257G CA_n259G/H/I/J/K/L/M CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n78A-n79A-n257H-n259A |  | CA_n257G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A CA_n79A-n257A/G/H CA_n79A-n259A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n78A-n79A-n257H-n259G |  | CA_n257G/H CA_n259G CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G CA_n79A-n257A/G/H CA_n79A-n259A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n78A-n79A-n257H-n259H |  | CA_n257G/H CA_n259G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H CA_n79A-n257A/G/H CA_n79A-n259A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n78A-n79A-n257H-n259I |  | CA_n257G/H CA_n259G/H/I CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n78A-n79A-n257H-n259J |  | CA_n257G/H CA_n259G/H/I/J CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n78A-n79A-n257H-n259K |  | CA_n257G/H CA_n259G/H/I/J/K CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n78A-n79A-n257H-n259L |  | CA_n257G/H CA_n259G/H/I/J/K/L CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K/L CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n78A-n79A-n257H-n259M |  | CA_n257G/H CA_n259G/H/I/J/K/L/M CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n78A-n79A-n257I-n259A |  | CA_n257G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A CA_n79A-n257A/G/H/I CA_n79A-n259A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n78A-n79A-n257I-n259G |  | CA_n257G/H/I CA_n259G CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G CA_n79A-n257A/G/H/I CA_n79A-n259A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n78A-n79A-n257I-n259H |  | CA_n257G/H/I CA_n259G/H CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n78A-n79A-n257I-n259I |  | CA_n257G/H/I CA_n259G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n78A-n79A-n257I-n259J |  | CA_n257G/H/I CA_n259G/H/I/J CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n78A-n79A-n257I-n259K |  | CA_n257G/H/I CA_n259G/H/I/J/K CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n78A-n79A-n257I-n259L |  | CA_n257G/H/I CA_n259G/H/I/J/K/L CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K/L CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n78A-n79A-n257I-n259M |  | CA_n257G/H/I CA_n259G/H/I/J/K/L/M CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259M |  |
| NOTE 1: The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively. NOTE 2: The CA configurations are given in Table 5.5A.1-1 of either TS 38.101-1 or TS 38.101-2 where unless otherwise stated BCS0 is referred to. NOTE 3: The delimiter “/” is only used in the uplink configurations for the sake of simplicity. For example, CA_nxA-nyA/B/C denotes CA_nxA-nyA, CA_nxA-nyB and CA_nxA-nyC, where nx and ny are two NR bands, ny is a FR2 band and A, B and C are the corresponding bandwidth classes respectively. |  |  |  |  |  |

Table 5.5A.1-4: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (five bands)

| NR CA configuration | Uplink configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n8A-n77A-n257A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n8A-n77A-n257G | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n8A-n77A-n257H | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n8A-n77A-n257I | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n8A-n77A-n257J | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257J |  |
| CA_n1A-n3A-n8A-n77A-n257K | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257K |  |
| CA_n1A-n3A-n8A-n77A-n257L | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257L |  |
| CA_n1A-n3A-n8A-n77A-n257M | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257M |  |
| CA_n1A-n3A-n8A-n77(2A)-n257A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n8A-n77(2A)-n257G | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n8A-n77(2A)-n257H | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n8A-n77(2A)-n257I | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n8A-n77(2A)-n257J | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n257 | CA_n257J |  |
| CA_n1A-n3A-n8A-n77(2A)-n257K | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n257 | CA_n257K |  |
| CA_n1A-n3A-n8A-n77(2A)-n257L | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n257 | CA_n257L |  |
| CA_n1A-n3A-n8A-n77(2A)-n257M | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n257 | CA_n257M |  |
| CA_n1A-n3A-n28A-n41A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A CA_n28A-n41A CA_n28A-n257A CA_n41A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n28A-n41A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G CA_n28A-n41A CA_n28A-n257A/G CA_n41A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n28A-n41A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G/H CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G/H CA_n28A-n41A CA_n28A-n257A/G/H CA_n41A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n28A-n41A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G/H/I CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G/H/I CA_n28A-n41A CA_n28A-n257A/G/H/I CA_n41A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n28A-n77A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n28A-n77A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A/G CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n28A-n77A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A/G/H CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n28A-n77A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n28A-n79A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n28A-n79A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G CA_n28A-n79A CA_n28A-n257A/G CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n28A-n79A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G/H CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G/H CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n28A-n79A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n41A-n77A-n257A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n41A-n77A-n257G | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n41A-n77A-n257H | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n41A-n77A-n257I | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n41A-n79A-n257A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n41A-n79A-n257G | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n41A-n79A-n257H | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n41A-n79A-n257I | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n77A-n79A-n257A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n77A-n79A-n257G | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n77A-n79A-n257H | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n77A-n79A-n257I | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n28A-n41A-n77A-n257A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n28A-n41A-n77A-n257G | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n28A-n41A-n77A-n257H | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n28A-n41A-n77A-n257I | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n28A-n41A-n79A-n257A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n28A-n41A-n79A-n257G | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n28A-n41A-n79A-n257H | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n28A-n41A-n79A-n257I | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n28A-n77A-n79A-n257A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n28A-n77A-n79A-n257G | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n28A-n77A-n79A-n257H | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n28A-n77A-n79A-n257I | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n41A-n77A-n79A-n257A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n41A-n77A-n79A-n257G | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n41A-n77A-n79A-n257H | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n41A-n77A-n79A-n257I | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n3A-n28A-n41A-n77A-n257A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n28A-n41A-n77A-n257G | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n3A-n28A-n41A-n77A-n257H | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n3A-n28A-n41A-n77A-n257I | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n3A-n28A-n41A-n79A-n257A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n41A-n79A-n257G | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n41A-n79A-n257H | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n41A-n79A-n257I | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |
|  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n77A-n79A-n257A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n77A-n79A-n257G | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n77A-n79A-n257H | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n77A-n79A-n257I | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n3A-n257G CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n77(2A)-n79A-n257A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n77(2A)-n79A-n257G | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n77(2A)-n79A-n257H | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n77(2A)-n79A-n257I | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257I |  |
| CA_n3A-n41A-n77A-n79A-n257A | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n41A-n77A-n79A-n257G | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n3A-n41A-n77A-n79A-n257H | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n3A-n41A-n77A-n79A-n257I | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257I |  |
| CA_n28A-n41A-n77A-n79A-n257A | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n41A-n77A-n79A-n257G | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n28A-n41A-n77A-n79A-n257H | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n28A-n41A-n77A-n79A-n257I | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257I |  |
| NOTE 1: The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively. NOTE 3: The delimiter “/” is only used in the uplink configurations for the sake of simplicity. For example, CA_nxA-nyA/B/C denotes CA_nxA-nyA, CA_nxA-nyB and CA_nxA-nyC, where nx and ny are two NR bands, ny is a FR2 band and A, B and C are the corresponding bandwidth classes respectively. |  |  |  |  |

## 5.5B	Configuration for DC

### 5.5B.1	General

The operating bands and bandwidth classes are specified for operation with EN-DC, NGEN-DC, NE-DC or NR-DC configured. The EN-DC, NGEN-DC or NE-DC band combinations include at least one E-UTRA operating band.

For EN-DC or NE-DC configurations indicated by column "Single Uplink allowed" (e.g., problematic band combinations as defined in TS 38.306 [11]) in tables in this clause the UE may indicate capability of not supporting simultaneous dual and triple uplink operation due to possible intermodulation interference to its own primary downlink channel bandwidth of PCell or PSCell if the intermodulation order is 2 or if the intermodulation order is 3 for the combinations when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz. When LTE and NR transmissions collide, simultaneous dual transmissions may not be supported by UE for these EN-DC band combinations for which only single switched UL is supported.

In the case for EN-DC or NE-DC configurations listed in tables in this clause for which the intermodulation products caused by the dual and triple uplink operation fall into the receive band but do not interfere with its own primary downlink channel bandwidth of PCell or PSCell as defined in Annex I the UE is mandated to operate in dual and triple uplink mode. Single Uplink is also allowed for certain band combinations where intermodulation or reverse intermodulation products could create difficulty for meeting emission requirementsFor EN-DC combinations of order 3 or higher, "Single Uplink allowed" UL configurations captured in Table 5.5B.2-1, Table 5.5B.3-1, and Table 5.5B.4-1 apply.

If multiple UL DC configurations are listed for multiple DL DC configurations, valid uplink configurations are such that uplink does not have more carriers than downlink.

The configurations for operating bands for DC including Band n41 also apply for the corresponding operating bands for DC with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for operating bands for DC with Band n90 are not listed in the tables below but are covered by this specification.

Non contiguous resource allocation and almost contiguous allocation are not applicable for E UTRA or NR carrier part of intra band EN DC configuration.

If the mandatory simultaneous Rx/Tx capability applies for a lower order DC configuration, when the applicable lower order DC configuration is a band pair in a higher order DC configuration, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order DC configuration.

For a higher order EN-DC band combination of which DC_20_n28/ DC_28_n20/ CA_20-28/ CA_n20-n28 is a subset, the frequency range in band n28/28 is restricted for the higher order band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.

For NR inter-band dual connectivity specified in 5.5B.7, the corresponding NR CA configurations in 5.5A.1, i.e., dual uplink inter-band carrier aggregation between FR1 and FR2 with uplink assigned to two NR bands, are applicable to Dual Connectivity.

NOTE 1:	Requirements for the dual connectivity configurations are defined in the clause corresponding NR uplink CA between FR1 and FR2 configurations, unless otherwise specified.

### 5.5B.2	Intra-band contiguous EN-DC

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)3AA | DC_(n)3AA6 | Yes6 |
| DC_(n)5AA | DC_(n)5AA6 | Yes6 |
| DC_(n)7AA | DC_(n)7AA6 | Yes6 |
| DC_(n)12AA | DC_(n)12AA6 | Yes6 |
| DC_(n)38AA | DC_(n)38AA6 | Yes6 |
| DC_(n)41AA DC_(n)41AB DC_(n)41CA DC_(n)41DA | DC_(n)41AA | Yes3 |
| DC_(n)48AA | DC_(n)48AA6 | Yes6 |
| DC_(n)48CA | DC_(n)48AA6 | Yes6 |
| DC_(n)48DA | DC_(n)48AA6 | Yes6 |
| DC_(n)66AA | DC_(n)66AA6 | Yes |
| DC_(n)71AA2 | DC_(n)71AA | No4 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 3: Single UL allowed due to potential emission issues, not self-interference. NOTE 4: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. Uplink DC_(n)71AA is not applicable to BCS2. NOTE 5: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 6: Only single switched UL is supported NOTE XX: The PC2 Uplink EN-DC configuration supported in Table 6.2B.1.1-1 is applicable to the same EN-DC configuration. NOTE YY: The PC1.5 Uplink EN-DC configuration supported in Table 6.2B.1.1-1 is applicable to the same EN-DC configuration. |  |  |

### 5.5B.2a	Intra-band contiguous NE-DC

Table 5.5B.2a-1: Intra-band contiguous NE-DC configurations

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_3(n)AA | DC_3(n)AA2 | Yes2 |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported |  |  |

### 5.5B.3	Intra-band non-contiguous EN-DC

Table 5.5B.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_1A_n1A | DC_1A_n1A5 | Yes5 |
| DC_2A_n2A | DC_2A_n2A5 | Yes5 |
| DC_3A_n3A | DC_3A_n3A | Yes7 |
| DC_5A_n5A | DC_5A_n5A5 | Yes5 |
| DC_7A_n7A6 | DC_7A_n7A5 | Yes5 |
| DC_40A_n40A | DC_40A_n40A5 | Yes5 |
| DC_41A_n41A DC_41C_n41A DC_41D_n41A | DC_41A_n41A | Yes4 |
| DC_48A_n48A | DC_48A_n48A5 | Yes5 |
| DC_48A-48A_n48A | DC_48A_n48A5 | Yes5 |
| DC_48C_n48A | DC_48A_n48A5 | Yes5 |
| DC_48D_n48A | DC_48A_n48A5 | Yes5 |
| DC_66A_n66A | DC_66A_n66A5 | Yes5 |
| DC_66B_n66A | DC_66A_n66A5 | Yes5 |
| DC_66A-66A_n66A | DC_66A_n66A5 | Yes5 |
| DC_71A_n71A3 | DC_71A_n71A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void. NOTE 3: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 7: Single UL allowed due to potential emission issues and self-interference. NOTE 8: The PC2 Uplink EN-DC configuration supported in Table 6.2B.1.2-1 is applicable to the same EN-DC configuration. NOTE 9: The PC1.5 Uplink EN-DC configuration supported in Table 6.2B.1.2-1 is applicable to the same EN-DC configuration. |  |  |

Table 5.5B.3-2: Intra-band EN-DC configurations for mixed intra-band contiguous and non-contiguous EN-DC

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)48CA6 | DC_48A_n48A5 | Yes5 |
| DC_(n)48DA6 | DC_48A_n48A5 | Yes5 |
| DC_48A-(n)48AA6 | DC_(n)48AA5 DC_48A_n48A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void NOTE 3: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: The UE supporting these configurations indicates ‘both’ by IE intraBandENDC-Support. |  |  |

### 5.5B.4	Inter-band EN-DC within FR1

#### 5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |  |
| DC_1A_n5A | DC_1A_n5A | No |  |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A DC_1A_n7B | No |  |
| DC_1A-1A_n7A DC_1A-1A_n7B | DC_1A_n7A | No |  |
| DC_1A_n8A | DC_1A_n8A | No |  |
| DC_1A_n20A | DC_1A_n20A | No |  |
| DC_1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n26A | DC_1A_n26A | No |  |
| DC_1A-1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A | No |  |
| DC_1A_n40A DC_1A_n40B | DC_1A_n40A | No |  |
| DC_1A_n41A7 | DC_1A_n41A | No |  |
| DC_1A_n50A | DC_1A_n50A | No |  |
| DC_1A_n51A | DC_1A_n51A | No |  |
| DC_1A_n71A DC_1A_n71B | DC_1A_n71A | No |  |
| DC_1A_n77A7 DC_1A_n77C7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n77(2A)7,21 DC_1A_n77(3A)7 | DC_1A_n77A21 | DC_1_n77 | No |
| DC_1A_n78A7 DC_1A_n78C7, 21 | DC_1A_n78A 21 | No | No |
| DC_1A_n78(2A)7,21 DC_1A_n78(A-C)7 | DC_1A_n78A21 | No | No |
| DC_1A-1A_n78A | DC_1A_n78A | No | No |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No | No |
| DC_1A_n105A | DC_1A_n105A | No |  |
| DC_2A_n5A | DC_2A_n5A | No |  |
| DC_2A-2A_n5A | DC_2A_n5A | No |  |
| DC_2A_n7A | DC_2A_n7A | No |  |
| DC_2A_n7(2A) | DC_2A_n7A | No |  |
| DC_2A-2A_n7A | DC_2A_n7A | No |  |
| DC_2A_n12A | DC_2A_n12A | No |  |
| DC_2A_n25A11, 13, 20 | N/A | N/A |  |
| DC_2A_n28A | DC_2A_n28A | No |  |
| DC_2A_n30A | DC_2A_n30A | No |  |
| DC_2A-2A_n30A | DC_2A_n30A | No |  |
| DC_2A_n38A | DC_2A_n38A | No |  |
| DC_2A-2A_n38A | DC_2A_n38A | No |  |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |  |
| DC_2A_n41(2A) | DC_2A_n41A | No |  |
| DC_2A-2A_n41A | DC_2A_n41A | No |  |
| DC_2A_n46A | DC_2A_n46A | No |  |
| DC_2A_n48A DC_2A_n48B | DC_2A_n48A | No |  |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n66(2A) | DC_2A_n66A | DC_2_n66 |  |
| DC_2A-2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A | DC_2A_n71A | No |  |
| DC_2A-2A_n71A | DC_2A_n71A | No |  |
| DC_2A_n77A DC_2A_n77C21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A_n77(2A) 21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A-2A_n77A21 DC_2A-2A_n77C21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A-2A_n77(2A) 21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_2A-2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |
| DC_2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |
| DC_2A-2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_3A_n1A DC_3C_n1A | DC_3A_n1A DC_3C_n1A | DC_3_n1 |  |
| DC_3A-3A_n1A | DC_3A_n1A | DC_3_n1 |  |
| DC_3A_n5A DC_3C_n5A | DC_3A_n5A | DC_3_n5 |  |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | DC_3A_n7A DC_3A_n7B DC_3C_n7A | No |  |
| DC_3A-3A_n7A DC_3A-3A_n7B | DC_3A_n7A | No |  |
| DC_3A_n8A | DC_3A_n8A | No |  |
| DC_3A-3A_n8A | DC_3A_n8A | No |  |
| DC_3A_n20A DC_3C_n20A | DC_3A_n20A | No |  |
| DC_3A_n26A DC_3C_n26A | DC_3A_n26A DC_3C_n26A | Yes |  |
| DC_3A_n28A DC_3C_n28A | DC_3A_n28A DC_3C_n28A | No |  |
| DC_3A_n34A | DC_3A_n34A | No |  |
| DC_3A_n38A DC_3C_n38A | DC_3A_n38A | No |  |
| DC_3A_n40A DC_3A_n40B | DC_3A_n40A | No |  |
| DC_3A_n41A7 DC_3A_n41C DC_3C_n41A7 | DC_3A_n41A DC_3C_n41A | DC_3_n41 | No |
| DC_3A_n50A | DC_3A_n50A | No |  |
| DC_3A_n51A | DC_3A_n51A | No |  |
| DC_3A_n71A DC_3A_n71B | DC_3A_n71A | No |  |
| DC_3A_n77A7 DC_3A_n77C7 DC_3C_n77A7,21 | DC_3A_n77A21 DC_3C_n77A | DC_3_n77 | No |
| DC_3A_n77(2A)7,21 DC_3A_n77(3A)7 DC_3C_n77(2A)7,21 | DC_3A_n77A,21 DC_3C_n77A | DC_3_n77 | No |
| DC_3A-3A_n77A7 | DC_3A_n77A | DC_3_n77 | No |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 | DC_3A_n78A DC_3C_n78A | DC_3_n78 | No |
| DC_3A_n78(2A)7,21 DC_3A_n78(A-C)7 DC_3C_n78(2A)7 | DC_3A_n78A,21 DC_3C_n78A | DC_3_n78 | No |
| DC_3A-3A_n78A7, 21 | DC_3A_n78A21 | DC_3_n78 | No |
| DC_3A_n79A7 DC_3A_n79C7 DC_3C_n79A7 | DC_3A_n79A DC_3C_n79A | No | No |
| DC_3A_n105A | DC_3A_n105A | No |  |
| DC_4A_n2A | DC_4A_n2A | No |  |
| DC_4A_n5A | DC_4A_n5A | DC_4_n5 |  |
| DC_4A_n7A | DC_4A_n7A | No |  |
| DC_4A_n28A | DC_4A_n28A | No |  |
| DC_4A_n38A | DC_4A_n38A | No |  |
| DC_4A_n41A | DC_4A_n41A | No |  |
| DC_4A_n78A | DC_4A_n78A | No |  |
| DC_4A_n78(2A) | DC_4A_n78A | No |  |
| DC_5A_n1A | DC_5A_n1A | No |  |
| DC_5A_n2A DC_5B_n2A | DC_5A_n2A | No |  |
| DC_5A_n2(2A) | DC_5A_n2A | No |  |
| DC_5A-5A_n2A | DC_5A_n2A | No |  |
| DC_5A_n3A | DC_5A_n3A | DC_5_n3 |  |
| DC_5A_n7A | DC_5A_n7A | DC_5_n7 |  |
| DC_5A_n7(2A) | DC_5A_n7A | DC_5_n7 |  |
| DC_5A_n12A | DC_5A_n12A | No |  |
| DC_5A_n25A | DC_5A_n25A | No |  |
| DC_5A_n30A | DC_5A_n30A | No |  |
| DC_5A_n38A | DC_5A_n38A | DC_5_n38 |  |
| DC_5A_n40A | DC_5A_n40A | No |  |
| DC_5A_n41A | DC_5A_n41A | No |  |
| DC_5A_n48A DC_5A_n48B | DC_5A_n48A | No |  |
| DC_5A_n66A DC_5B_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A-5A_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A_n77A DC_5A_n77C21 | DC_5A_n77A21 | No |  |
| DC_5A_n77(2A) 21 DC_5A_n77(3A) | DC_5A_n77A21 | No |  |
| DC_5A_n71A | DC_5A_n71A | No |  |
| DC_5A_n78A7 DC_5A_n78C7 | DC_5A_n78A | No | No |
| DC_5A_n78(2A)7 DC_5A_n78(A-C)7 | DC_5A_n78A | No | No |
| DC_5A_n79A | DC_5A_n79A | No | No |
| DC_7A_n1A DC_7C_n1A | DC_7A_n1A DC_7C_n1A | No |  |
| DC_7A-7A_n1A | DC_7A_n1A | No |  |
| DC_7A_n2A DC_7C_n2A | DC_7A_n2A | No |  |
| DC_7A_n2(2A) | DC_7A_n2A | No |  |
| DC_7A_n3A DC_7C_n3A | DC_7A_n3A DC_7C_n3A | No |  |
| DC_7A_n5A DC_7C_n5A | DC_7A_n5A DC_7C_n5A | DC_7_n5 |  |
| DC_7A-7A_n5A | DC_7A_n5A | DC_7_n5 |  |
| DC_7A_n8A | DC_7A_n8A | No |  |
| DC_7A-7A_n8A | DC_7A_n8A | No |  |
| DC_7A_n12A | DC_7A_n12A | No |  |
| DC_7A-7A_n78(2A)7 | DC_7A_n78A | No |  |
| DC_7A_n20A | DC_7A_n20A | No |  |
| DC_7A_n25A DC_7C_n25A | DC_7A_n25A | No |  |
| DC_7A_n26A DC_7C_n26A | DC_7A_n26A DC_7C_n26A | Yes |  |
| DC_7A-7A_n25A | DC_7A_n25A | No |  |
| DC_7A_n28A DC_7C_n28A | DC_7A_n28A DC_7C_n28A | No |  |
| DC_7A_n40A | DC_7A_n40A | Yes |  |
| DC_7A-7A_n40A | DC_7A_n40A | Yes |  |
| DC_7A-7A_n28A | DC_7A_n28A | No |  |
| DC_7A_n51A | DC_7A_n51A | No |  |
| DC_7A_n66A DC_7C_n66A | DC_7A_n66A | No |  |
| DC_7A-7A_n66A | DC_7A_n66A | No |  |
| DC_7A_n71A | DC_7A_n71A | No |  |
| DC_7A_n77A7 DC_7C_n77A | DC_7A_n77A | No |  |
| DC_7A_n77(2A) DC_7A_n77(3A) DC_7C_n77(2A) | DC_7A_n77A | No |  |
| DC_7A-7A_n77A7 | DC_7A_n77A | No |  |
| DC_7A-7A_n77(2A) DC_7A-7A_n77(3A) | DC_7A_n77A | No |  |
| DC_7A_n78A7 DC_7C_n78A7 DC_7A_n78C7 | DC_7A_n78A DC_7C_n78A | No |  |
| DC_7A_n78(2A)7 DC_7A_n78(A-C)7 DC_7C_n78(2A)7 | DC_7A_n78A DC_7C_n78A | No |  |
| DC_7A-7A_n78A7, 21 DC_7A-7A_n78C7 | DC_7A_n78A21 | No |  |
| DC_7A-7A_n78(A-C)7 | DC_7A_n78A | No |  |
| DC_7A_n79A DC_7A_n79C | DC_7A_n79A | No |  |
| DC_7A_n105A | DC_7A_n105A | No |  |
| DC_8A_n1A | DC_8A_n1A | No |  |
| DC_8A_n2A | DC_8A_n2A | DC_8_n2 |  |
| DC_8A_n3A | DC_8A_n3A | No |  |
| DC_8A_n7A | DC_8A_n7A | No |  |
| DC_8A_n20A | DC_8A_n20A | Yes |  |
| DC_8A_n28A | DC_8A_n28A | No |  |
| DC_8A_n34A | DC_8A_n34A | No |  |
| DC_8A_n38A | DC_8A_n38A | No |  |
| DC_8A_n39A | DC_8A_n39A | No |  |
| DC_8A_n40A7 | DC_8A_n40A | No |  |
| DC_8A_n41A7 DC_8A_n41C | DC_8A_n41A | No | No |
| DC_8A_n41(2A) | DC_8A_n41A | No | No |
| DC_8A_n77A7 | DC_8A_n77A | No | No |
| DC_8A_n77(2A)7 DC_8A_n77(3A)7 | DC_8A_n77A | No | No |
| DC_8A_n78A7 DC_8B_n78A7 | DC_8A_n78A DC_8B_n78A | No | No |
| DC_8A_n78(2A)7 | DC_8A_n78A | No | No |
| DC_8A_n79A7 DC_8A_n79C | DC_8A_n79A DC_8A_n79C | No | No |
| DC_8A_n93A | DC_8A_n93A_ULSUP-TDM | N/A |  |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |  |
| DC_11A_n1A | DC_11A_n1A | No |  |
| DC_11A_n3A | DC_11A_n3A | No |  |
| DC_11A_n28A | DC_11A_n28A | No |  |
| DC_11A_n41A7 | DC_11A_n41A | No |  |
| DC_11A_n77A7 | DC_11A_n77A | No | No |
| DC_11A_n77(2A)7 DC_11A_n77(3A)7 | DC_11A_n77A | No | No |
| DC_11A_n78A7 | DC_11A_n78A | No | No |
| DC_11A_n78(2A) | DC_11A_n78A | No | No |
| DC_11A_n79A7 | DC_11A_n79A | No |  |
| DC_12A_n2A | DC_12A_n2A | No |  |
| DC_12A_n2(2A) | DC_12A_n2A | No |  |
| DC_12A_n5A | DC_12A_n5A | No |  |
| DC_12A_n7A | DC_12A_n7A | No |  |
| DC_12A_n7(2A) | DC_12A_n7A | No |  |
| DC_12A_n25A | DC_12A_n25A | No |  |
| DC_12A_n30A | DC_12A_n30A | No |  |
| DC_12A_n38A | DC_12A_n38A | No |  |
| DC_12A_n41A | DC_12A_n41A | No |  |
| DC_12A_n66A | DC_12A_n66A | No |  |
| DC_12A_n66(2A) | DC_12A_n66A | No |  |
| DC_12A_n71A | DC_12A_n71A18,19 | DC_12_n71 |  |
| DC_12A_n77A | DC_12A_n77A | DC_12_n77 |  |
| DC_12A_n77(2A) 21 | DC_12A_n77A21 | DC_12_n77 |  |
| DC_12A_n78A | DC_12A_n78A | DC_12_n78 |  |
| DC_12A_n78(2A) | DC_12A_n78A | DC_12_n78 |  |
| DC_13A_n2A | DC_13A_n2A | No |  |
| DC_13A_n5A | DC_13A_n5A | DC_13_n5 |  |
| DC_13A_n7A | DC_13A_n7A | No |  |
| DC_13A_n7(2A) | DC_13A_n7A | No |  |
| DC_13A_n25A | DC_13A_n25A | No |  |
| DC_13A_n48A DC_13A_n48B | DC_13A_n48A | No |  |
| DC_13A_n66A | DC_13A_n66A | No |  |
| DC_13A_n71A | DC_13A_n71A | No |  |
| DC_13A_n77A DC_13A_n77C21 | DC_13A_n77A21 | No |  |
| DC_13A_n78A | DC_13A_n78A | No |  |
| DC_13A_n78(2A) | DC_13A_n78A | No |  |
| DC_14A_n2A | DC_14A_n2A | No |  |
| DC_14A_n5A | DC_14A_n5A | DC_14_n5 |  |
| DC_14A_n30A | DC_14A_n30A | No |  |
| DC_14A_n66A | DC_14A_n66A | No |  |
| DC_14A_n77A | DC_14A_n77A | No |  |
| DC_14A_n77(2A) 21 | DC_14A_n77A21 | No |  |
| DC_18A_n3A | DC_18A_n3A | No |  |
| DC_18A_n28A | DC_18A_n28A | No |  |
| DC_18A_n41A16 | DC_18A_n41A | No |  |
| DC_18A_n77A7 DC_18A_n77(2A)7 | DC_18A_n77A | No | No |
| DC_18A_n78A7 | DC_18A_n78A | No | No |
| DC_18A_n78(2A)7 | DC_18A_n78A | No | No |
| DC_20A_n91A | DC_20A_n91A_ULSUP-TDM | N/A |  |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |  |
| DC_18A_n79A7 | DC_18A_n79A | No |  |
| DC_19A_n1A | DC_19A_n1A | No |  |
| DC_19A_n77A7 DC_19A_n77C7 | DC_19A_n77A | No |  |
| DC_19A_n77(2A)7, 21 | DC_19A_n77A, 21 | No |  |
| DC_19A_n78A7 DC_19A_n78C7 | DC_19A_n78A | No | No |
| DC_19A_n78(2A)7, 21 | DC_19A_n78A, 21 | No | No |
| DC_19A_n79A7 DC_19A_n79C7 | DC_19A_n79A | No | No |
| DC_20A_n1A | DC_20A_n1A | No |  |
| DC_20A_n3A | DC_20A_n3A | No |  |
| DC_20A_n7A | DC_20A_n7A | DC_20_n7 |  |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |  |
| DC_20A_n28A8,11,13 | DC_20A_n28A | No |  |
| DC_20A_n38A | DC_20A_n38A | No |  |
| DC_20A_n41A | DC_20A_n41A | DC_20_n41 |  |
| DC_20A_n50A | DC_20A_n50A | No |  |
| DC_20A_n51A | DC_20A_n51A | No |  |
| DC_20A_n77A7 | DC_20A_n77A | No |  |
| DC_20A_n78A7 DC_20A_n78C7 | DC_20A_n78A | No |  |
| DC_20A_n78(2A)7 | DC_20A_n78A | No |  |
| DC_21A_n1A | DC_21A_n1A | No |  |
| DC_21A_n28A17 | DC_21A_n28A | DC_21_n28 |  |
| DC_21A_n77A7 DC_21A_n77C7 | DC_21A_n77A | No |  |
| DC_21A_n77(2A)7,21 | DC_21A_n77A,21 | No |  |
| DC_21A_n78A7 DC_21A_n78C7 | DC_21A_n78A | No | No |
| DC_21A_n78(2A)7,21 | DC_21A_n78A,21 | No | No |
| DC_21A_n79A7 DC_21A_n79C7 | DC_21A_n79A | No | No |
| DC_25A_n41A | DC_25A_n41A | No |  |
| DC_25A-25A_n41A | DC_25A_n41A | No |  |
| DC_25A_n77A | DC_25A_n77A | DC_25_n77 |  |
| DC_25A-25A_n77A | DC_25A_n77A | DC_25_n77 |  |
| DC_25A_n78A | DC_25A_n78A | DC_25_n78 |  |
| DC_25A-25A_n78A | DC_25A_n78A | DC_25_n78 |  |
| DC_26A_n25A | DC_26A_n25A | No |  |
| DC_26A_n41A | DC_26A_n41A | No |  |
| DC_26A_n77A7 | DC_26A_n77A | No |  |
| DC_26A_n78A7 DC_26A_n78A7 | DC_26A_n78A | No |  |
| DC_26A_n78(2A) | DC_26A_n78A | No |  |
| DC_26A_n79A7 | DC_26A_n79A | No |  |
| DC_28A_n1A | DC_28A_n1A | No |  |
| DC_28A_n2A | DC_28A_n2A | No |  |
| DC_28A_n3A | DC_28A_n3A | No |  |
| DC_28A_n5A | DC_28A_n5A | No |  |
| DC_28A_n7A DC_28A_n7B | DC_28A_n7A DC_28A_n7B | No |  |
| DC_28A_n51A | DC_28A_n51A | No |  |
| DC_28A_n8A | DC_28A_n8A | No |  |
| DC_28A_n20A8,11,13 | DC_28A_n20A | No |  |
| DC_28A_n38A | DC_28A_n38A | No |  |
| DC_28A_n40A | DC_28A_n40A | No |  |
| DC_28A_n41A7 | DC_28A_n41A | No |  |
| DC_28A_n50A | DC_28A_n50A | No |  |
| DC_28A_n66A | DC_28A_n66A | No |  |
| DC_28A_n77A7 DC_28A_n77C7 | DC_28A_n77A | No | No |
| DC_28A_n77(2A)7 | DC_28A_n77A | No | No |
| DC_28A_n78A7 DC_28A_n78C7 | DC_28A_n78A | No | No |
| DC_28A_n78(2A)7 | DC_28A_n78A | No | No |
| DC_28A_n79A7 DC_28A_n79C7 | DC_28A_n79A | No |  |
| DC_30A_n2A | DC_30A_n2A | No |  |
| DC_30A_n5A | DC_30A_n5A | No |  |
| DC_30A_n66A | DC_30A_n66A | No |  |
| DC_30A_n77A | DC_30A_n77A | No |  |
| DC_30A_n77(2A) 21 | DC_30A_n77A21 | No |  |
| DC_38A_n1A | DC_38A_n1A | No |  |
| DC_38A_n3A | DC_38A_n3A | No |  |
| DC_38A_n8A | DC_38A_n8A | No |  |
| DC_38A_n28A | DC_38A_n28A | No |  |
| DC_38A_n78A7 | DC_38A_n78A | No |  |
| DC_38A_n79A DC_38A_n79C | DC_38A_n79A | No |  |
| DC_39A_n40A3 | DC_39A_n40A | No |  |
| DC_39A_n41A3 DC_39C_n41A3 DC_39A_n41C3 | DC_39A_n41A DC_39C_n41A | No | No |
| DC_39A_n78A5,7 | DC_39A_n78A | No |  |
| DC_39A_n79A7 DC_39A_n79C7 | DC_39A_n79A | No | No |
| DC_40A_n1A DC_40C_n1A | DC_40A_n1A | No |  |
| DC_40A_n41A3 DC_40A_n41C3 DC_40C_n41A3 | DC_40A_n41A | No |  |
| DC_40A_n41(2A)3 | DC_40A_n41A | No |  |
| DC_40A_n77A DC_40A_n77C | DC_40A_n77A | No |  |
| DC_40A_n78A DC_40C_n78A | DC_40A_n78A DC_40C_n78A | No |  |
| DC_40A_n78(2A) DC_40C_n78(2A) | DC_40A_n78A DC_40C_n78A | No |  |
| DC_40A_n79A7,12 DC_40A_n79C7,12 DC_40C_n79A7,12 | DC_40A_n79A | No | No |
| DC_41A_n1A DC_41C_n1A | DC_41A_n1A DC_41C_n1A | No | DC_41A_n1A DC_41C_n1A |
| DC_41A_n3A7 DC_41C_n3A7 | DC_41A_n3A DC_41C_n3A | No |  |
| DC_41A_n28A7 DC_41C_n28A7 | DC_41A_n28A DC_41C_n28A | No |  |
| DC_41A_n77A DC_41C_n77A | DC_41A_n77A DC_41C_n77A | No |  |
| DC_41A_n77(2A) DC_41C_n77(2A) | DC_41A_n77A DC_41C_n77A | No |  |
| DC_41A_n78A DC_41C_n78A DC_41D_n78A | DC_41A_n78A DC_41C_n78A | No |  |
| DC_41A_n78(2A) DC_41C_n78(2A) | DC_41A_n78A DC_41C_n78A | No |  |
| DC_41A_n79A6,7 DC_41A_n79C6,7 DC_41C_n79A6,7 | DC_41A_n79A DC_41C_n79A | No | No |
| DC_42A_n1A7 DC_42C_n1A7 | DC_42A_n1A DC_42C_n1A | No |  |
| DC_42A_n3A7 DC_42C_n3A7 | DC_42A_n3A DC_42C_n3A | DC_42_n3 |  |
| DC_42A_n28A7 DC_42C_n28A7 | DC_42A_n28A DC_42C_n28A | No |  |
| DC_42A_n51A | DC_42A_n51A | No |  |
| DC_42A_n77A3,4,9,11 DC_42A_n77C3,4,9,11 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9,11 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C | N/A | N/A |  |
| DC_42A_n77(2A)3,4,9,11 DC_42C_n77(2A)3,4,9,11 | N/A | N/A |  |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42D_n78C3,4,9,11 DC_42E_n78A3,4,9,11 DC_42E_n78C3,4,9,11 | N/A | N/A |  |
| DC_42A_n79A9,15 DC_42A_n79C9,15 DC_42C_n79A9,15 DC_42C_n79C9,15 DC_42D_n79A9,15 DC_42D_n79C9,15 DC_42E_n79A9,15 DC_42E_n79C9,15 | N/A | N/A |  |
| DC_46A_n77A2 | N/A | N/A |  |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 | N/A | N/A |  |
| DC_48A_n2A DC_48C_n2A DC_48D_n2A DC_48E_n2A | DC_48A_n2A | No |  |
| DC_48A_n5A DC_48C_n5A DC_48D_n5A DC_48E_n5A | DC_48A_n5A | No |  |
| DC_48A_n12A | DC_48A_n12A | No |  |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | DC_48A_n25A | No |  |
| DC_48A_n46A DC_48B_n46A DC_48C_n46A DC_48D_n46A DC_48E_n46A DC_48A_n46B DC_48B_n46B DC_48C_n46B DC_48D_n46B DC_48E_n46B DC_48A_n46C DC_48B_n46C DC_48C_n46C DC_48D_n46C DC_48E_n46C DC_48A_n46D DC_48B_n46D DC_48C_n46D DC_48D_n46D DC_48E_n46D | DC_48A_n46A DC_48B_n46A | No |  |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A DC_48E_n66A | DC_48A_n66A | No |  |
| DC_48A_n71A DC_48B_n71A DC_48C_n71A DC_48D_n71A | DC_48A_n71A | No |  |
| DC_48A-48A_n71A DC_48A-48A-48A_n71A | DC_48A_n71A | No |  |
| DC_48A_n77A3. 4. 9, 11 DC_48C_n77A3. 4. 9, 11 DC_48A_n77C3. 4. 9, 11 DC_48C_n77C3. 4. 9, 11 DC_48D_n77A3. 4. 9, 11 DC_48D_n77C3. 4. 9, 11 DC_48E_n77A3. 4. 9, 11 | N/A | N/A |  |
| DC_48A-48A_n77A | N/A | N/A |  |
| DC_48A-48A-48A_n77A | N/A | N/A |  |
| DC_66A_n2A DC_66B_n2A DC_66C_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n2(2A) | DC_66A_n2A | DC_66_n2 |  |
| DC_66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A-66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n5A DC_66B_n5A DC_66C_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A_n7A | DC_66A_n7A | No |  |
| DC_66A_n7(2A) | DC_66A_n7A | No |  |
| DC_66A-66A_n7A | DC_66A_n7A | No |  |
| DC_66A-66A_n7(2A) | DC_66A_n7A | No |  |
| DC_66A_n12A | DC_66A_n12A | No |  |
| DC_66A_n25A | DC_66A_n25A | DC_66_n25 |  |
| DC_66A_n28A | DC_66A_n28A | No |  |
| DC_66A_n30A | DC_66A_n30A | No |  |
| DC_66A-66A_n30A | DC_66A_n30A | No |  |
| DC_66A_n38A | DC_66A_n38A | No |  |
| DC_66A-66A_n38A | DC_66A_n38A | No |  |
| DC_66A_n41A DC_66A_n41C | DC_66A_n41A | No |  |
| DC_66A_n41(2A) | DC_66A_n41A | No |  |
| DC_66A_n46A | DC_66A_n46A | No |  |
| DC_66A_n48A DC_66A_n48B | DC_66A_n48A | No |  |
| DC_66A-66A_n48A DC_66A-66A_n48B | DC_66A_n48A | No |  |
| DC_66A_n71A DC_66C_n71A DC_66A_n71B | DC_66A_n71A | No |  |
| DC_66A-66A_n71A | DC_66A_n71A | No |  |
| DC_66A_n77A DC_66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A_n77(2A) 21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A_n77A21 DC_66A-66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A_n77(2A) 21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A-66A_n77A21 DC_66A-66A-66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A-66A_n77(2A)21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A_n78A | DC_66A_n78A | No |  |
| DC_66A_n78(2A) | DC_66A_n78A | No |  |
| DC_66A-66A_n78A | DC_66A_n78A | No |  |
| DC_66A-66A_n78(2A) | DC_66A_n78A | No |  |
| DC_71A_n2A | DC_71A_n2A | No |  |
| DC_71A_n2(2A) | DC_71A_n2A | No |  |
| DC_71A_n5A | DC_71A_n5A | No |  |
| DC_71A_n12A | DC_71A_n12A18,19 | Yes |  |
| DC_71A_n38A | DC_71A_n38A | No |  |
| DC_71A_n7A | DC_71A_n7A | No |  |
| DC_71A_n25A | DC_71A_n7A | No |  |
| DC_71A_n41A | DC_71A_n41A | No |  |
| DC_71A_n48A | DC_71A_n48A | No |  |
| DC_71A_n66A | DC_71A_n66A | No |  |
| DC_71A_n77A DC_71A_n77C | DC_71A_n77A | No |  |
| DC_71A_n77(2A) | DC_71A_n77A | No |  |
| DC_71A_n78A | DC_71A_n78A | No |  |
| DC_71A_n78(2A) | DC_71A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 / 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. This restriction also apply for any band combinations when DC_20_n28/ DC_28_n20/ CA_20-28/ CA_n20-n28 is a subset of a higher order band combination. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 or Band 48 is not used. NOTE 10: Void. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For UEs indicating interBandMRDC-WithOverlapDL-Bands-r16, the power imbalance requirement defined in clause 7.6B.2.6 apply. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. Same restrictions are applied to related higher order configurations. NOTE 16: The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 18: Only single switched UL is supported. NOTE 19: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 20: The combination is not used alone as fallback mode of other band combinations in which UL in Band 2 is not used. NOTE 21: For this DC configuration, reference sensitivity exceptions for Power Class 2, if allowed, are specified in Clause 7.3B.2.3. If the uplink EN-DC configuration supported in Table 6.2B.1.3-1 is applicable to the same EN-DC configuration, the note is not shown as the reference sensitivity exceptions, if any, have been confirmed. NOTE 22: The PC2 Uplink EN-DC configuration supported in Table 6.2B.1.3-1 is applicable to the same EN-DC configuration without additional indication of NOTE 21. |  |  |  |

#### 5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n1A | DC_1A_n1A2 DC_3A_n1A |
| DC_1A-3A_n3A | DC_1A_n3A DC_3A_n3A2 |
| DC_1A-(n)3AA | DC_1A_n3A |
| DC_1A-3A_n5A DC_1A-3C_n5A | DC_1A_n5A DC_3A_n5A |
| DC_1A-3A_n7A DC_1A-3A_n7B DC_1A-3C_n7A DC_1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-1A-3A_n7A DC_1A-1A-3A_n7B DC_1A-1A-3C_n7A DC_1A-1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-3A-3A_n7A DC_1A-3A-3A_n7B | DC_1A_n7A DC_3A_n7A |
| DC_1A-1A-3A-3A_n7A DC_1A-1A-3A-3A_n7B | DC_1A_n7A DC_3A_n7A |
| DC_1A-3A_n8A | DC_1A_n8A DC_3A_n8A |
| DC_1A-3A_n26A DC_1A-3C_n26A | DC_1A_n26A DC_3A_n26A |
| DC_1A-3A_n28A DC_1A-3C_n28A | DC_1A_n28A DC_3A_n28A |
| DC_1A-1A-3A_n28A DC_1A-1A-3C_n28A | DC_1A_n28A DC_3A_n28A |
| DC_1A_n3A-n28A | DC_1A_n3A DC_1A_n28A |
| DC_1A-3A_n38A | DC_1A_n38A DC_3A_n38A |
| DC_1A_n3A-n38A | DC_1A_n3A DC_1A_n38A |
| DC_1A-3A_n40A | DC_1A_n40A DC_3A_n40A |
| DC_1A-3A_n41A5 DC_1A-3C_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A |
| DC_1A_n3A-n41A5 | DC_1A_n3A DC_1A_n41A |
| DC_1A-3A_n71A DC_1A-3A_n71B | DC_1A_n71A DC_3A_n71A |
| DC_1A-3A_n77A5, 14 DC_1A-3A_n77C5 DC_1A-3C_n77A5 | DC_1A_n77A14 DC_3A_n77A14 DC_3C_n77A |
| DC_1A-3A_n77(2A)5,14 DC_1A-3C_n77(2A)5 | DC_1A_n77A14 DC_3A_n77A14 DC_3C_n77A |
| DC_1A-3A_n77(3A)5 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n78A5,14 DC_1A-3A_n78C5 DC_1A-3C_n78A5 | DC_1A_n78A14 DC_3A_n78A14 DC_3C_n78A |
| DC_1A-3A_n78(2A)514 DC_1A-3C_n78(2A)5 | DC_1A_n78A14 DC_3A_n78A14 DC_3C_n78A |
| DC_1A-3A_n78(A-C)5 | DC_1A_n78A DC_3A_n78A |
| DC_1A-1A-3A_n78A DC_1A-1A-3C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A_n3A-n8A | DC_1A_n3A DC_1A_n8A |
| DC_1A_n3A-n75A | DC_1A_n3A |
| DC_1A_n3A-n77A5 | DC_1A_n3A DC_1A_n77A |
| DC_1A_n3A-n77(2A) 5 | DC_1A_n3A DC_1A_n77A |
| DC_1A_n3A-n78A5 | DC_1A_n3A DC_1A_n78A |
| DC_1A_n3A-n78(2A)5 | DC_1A_n3A DC_1A_n78A |
| DC_1A_n3A-n79A | DC_1A_n3A DC_1A_n79A |
| DC_1A-3A_n79A5,14 DC_1A-3A_n79C5 | DC_1A_n79A14 DC_3A_n79A14 |
| DC_1A-3A_n105A | DC_1A_n105A DC_3A_n105A |
| DC_1A-5A_n40A | DC_1A_n40A DC_5A_n40A |
| DC_1A_n5A-n40A | DC_1A_n5A DC_1A_n40A |
| DC_1A-5A_n77A | DC_1A_n77A DC_5A_n77A |
| DC_1A-5A_n77(2A) DC_1A-5A_n77(3A) | DC_1A_n77A DC_5A_n77A |
| DC_1A-5A_n78A5 DC_1A-5A_n78C5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n78(2A)5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n78(A-C)5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-1A-5A_n78A | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n79A | DC_1A_n79A DC_5A_n79A |
| DC_1A_n5A-n78A5 | DC_1A_n5A DC_1A_n78A |
| DC_1A-7A_n1A | DC_1A_n1A DC_7A_n1A |
| DC_1A-7A_n3A DC_1A-7C_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A |
| DC_1A-7A_n5A DC_1A-7C_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-(n)7AA | DC_1A_n7A |
| DC_1A-7A_n8A | DC_1A_n8A DC_7A_n8A |
| DC_1A-7A_n20A | DC_1A_n20A DC_7A_n20A |
| DC_1A-7A_n26A | DC_1A_n26A DC_7A_n26A |
| DC_1A-7C_n26A | DC_1A_n26A DC_7A_n26A DC_7C_n26A |
| DC_1A-7A_n28A5 DC_1A-7C_n28A5 | DC_1A_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-1A-7A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A-7A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A_n40A | DC_1A_n40A DC_7A_n40A |
| DC_1A-7A-7A_n40A | DC_1A_n40A DC_7A_n40A |
| DC_1A-7A_n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A_n77(2A) DC_1A-7A_n77(3A) | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A_n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A_n77(2A) DC_1A-7A-7A_n77(3A) | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A_n78A5 DC_1A-7C_n78A5 DC_1A-7A_n78C5 | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n78(2A)5 DC_1A-7C_n78(2A)5 | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n78(A-C)5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-1A-7A_n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78A5 DC_1A-7A-7A_n78C5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78(2A)5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78(A-C)5 | DC_1A_n78A DC_7A_n78A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A | DC_1A_n7A DC_1A_n78A |
| DC_1A_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A |
| DC_1A-7A_n105A | DC_1A_n105A DC_7A_n105A |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A_n7A | DC_8A_n7A DC_1A_n7A |
| DC_1A-8A_n20A | DC_1A_n20A DC_8A_n20A |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A |
| DC_1A-8A_n40A | DC_1A_n40A DC_8A_n40A |
| DC_1A_n8A-n40A | DC_1A_n8A DC_1A_n40A |
| DC_1A-8A_n77A5 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n77(2A)5 | DC_1A_n77A DC_8A_n77A |
| DC_1A_n8A-n77A | DC_1A_n8A DC_1A_n77A |
| DC_1A_n8A-n77(2A) | DC_1A_n8A DC_1A_n77A |
| DC_1A-8A_n77(3A)5 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n78A5 | DC_1A_n78A DC_8A_n78A |
| DC_1A-8A_n78(2A)5 | DC_1A_n78A DC_8A_n78A |
| DC_1A_n8A-n78A5 | DC_1A_n8A DC_1A_n78A |
| DC_1A-8A_n79A5 | DC_1A_n79A DC_8A_n79A |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A |
| DC_1A-11A_n28A | DC_1A_n28A DC_11A_n28A |
| DC_1A-11A_n41A5 | DC_1A_n41A DC_11A_n41A |
| DC_1A-11A_n77A5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n77(2A)5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n77(3A)5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n78A5 | DC_1A_n78A DC_11A_n78A |
| DC_1A-11A_n78(2A)5 | DC_1A_n78A DC_11A_n78A |
| DC_1A-11A_n79A5 | DC_1A_n79A DC_11A_n79A |
| DC_1A-18A_n3A | DC_1A_n3A DC_18A_n3A |
| DC_1A-18A_n28A | DC_1A_n28A DC_18A_n28A |
| DC_1A-18A_n41A | DC_1A_n41A DC_18A_n41A |
| DC_1A-18A_n77A5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A_n77(2A)5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A_n78A5 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n78(2A)5 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A_n77A5,14 DC_1A-19A_n77C5 | DC_1A_n77A14 DC_19A_n77A14 |
| DC_1A-19A_n77(2A)5,14 | DC_1A_n77A14 DC_19A_n77A14 |
| DC_1A-19A_n78A5 DC_1A-19A_n78C5 | DC_1A_n78A14 DC_19A_n78A14 |
| DC_1A-19A_n78(2A)5 | DC_1A_n78A14 DC_19A_n78A14 |
| DC_1A-19A_n79A514 DC_1A-19A_n79C5 | DC_1A_n79A14 DC_19A_n79A14 |
| DC_1A-20A_n1A | DC_1A_n1A2 DC_20A_n1A |
| DC_1A-20A_n3A DC_1C-20A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A_n7A | DC_1A_n7A DC_20A_n7A |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A_n28A | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A_n38A | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n41A | DC_1A_n41A DC_20A_n41A |
| DC_1A-20A_n78A5 DC_1A-20A_n78C5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n78(2A)5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-21A_n28A13 | DC_1A_n28A DC_21A_n28A |
| DC_1A-21A_n77A5, 14 DC_1A-21A_n77C5, 14 | DC_1A_n77A14 DC_21A_n77A14 |
| DC_1A-21A_n77(2A)5,14 | DC_1A_n77A14 DC_21A_n77A14 |
| DC_1A-21A_n78A5,14 DC_1A-21A_n78C5 | DC_1A_n78A14 DC_21A_n78A14 |
| DC_1A-21A_n78(2A)5,14 | DC_1A_n78A14 DC_21A_n78A14 |
| DC_1A-21A_n79A5,14 DC_1A-21A_n79C5 | DC_1A_n79A14 DC_21A_n79A14 |
| DC_1A-26A_n78A DC_1A-26A_n78(2A) | DC_1A_n78A DC_26A_n78A |
| DC_1A-26A_n78(2A) | DC_1A_n78A DC_26A_n78A |
| DC_1A_n26A-n78A | DC_1A_n26A DC_1A_n78A |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |
| DC_1A-28A_n7A DC_1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A-1A-28A_n7A DC_1A-1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A-28A_n20A22 | DC_1A_n20A DC_28A_n20A22 |
| DC_1A-28A_n38A | DC_1A_n38A DC_28A_n38A |
| DC_1A_n28A-n40A | DC_1A_n28A DC_1A_n40A |
| DC_1A-28A_n40A | DC_1A_n40A DC_28A_n40A |
| DC_1A_n28A-n41A5 | DC_1A_n28A DC_1A_n41A |
| DC_1A_n28A-n75A | DC_1A_n28A |
| DC_1A-28A_n77A5 DC_1A-28A_n77C5 | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A_n78A5 DC_1A-28A_n78C5 DC_1A-28A_n78(2A)5 | DC_1A_n78A DC_28A_n78A |
| DC_1A-1A-28A_n78A | DC_1A_n78A DC_28A_n78A |
| DC_1A-28A_n78(2A) | DC_1A_n78A DC_28A_n78A |
| DC_1A_n28A-n77A5 | DC_1A_n28A DC_1A_n77A |
| DC_1A_n28A-n77(2A)5 | DC_1A_n28A DC_1A_n77A |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |
| DC_1A_n28A-n78(2A)5 | DC_1A_n28A DC_1A_n78A |
| DC_1A-28A_n79A5 DC_1A-28A_n79C5 | DC_1A_n79A DC_28A_n79A |
| DC_1A_n28A-n79A5 | DC_1A_n28A DC_1A_n79A |
| DC_1A-32A_n3A | DC_1A_n3A |
| DC_1A-32A_n8A | DC_1A_n8A |
| DC_1A-32A_n28A | DC_1A_n28A |
| DC_1A-32A_n78A DC_1A-32A_n78C | DC_1A_n78A |
| DC_1A-32A_n78(2A) | DC_1A_n78A |
| DC_1A-38A_n3A | DC_1A_n3A |
| DC_1A-38A_n8A | DC_1A_n8A DC_38A_n8A |
| DC_1A-38A_n28A | DC_1A_n28A DC_38A_n28A |
| DC_1A-(n)38AA | DC_1A_n38A |
| DC_1A_n38A-n78A | DC_1A_n38A DC_1A_n78A |
| DC_1A-38A_n78A | DC_1A_n78A |
| DC_1A-38A_n78(2A) | DC_1A_n78A |
| DC_1A_n40A-n77A | DC_1A_n40A DC_1A_n77A |
| DC_1A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A |
| DC_1A-40A_n78A DC_1A-40C_n78A | DC_1A_n78A DC_40A_n78A |
| DC_1A-40A_n78(2A) DC_1A-40C_n78(2A) | DC_1A_n78A DC_40A_n78A |
| DC_1A_n40A-n78A DC_1A_n40A-n78C | DC_1A_n40A DC_1A_n78A |
| DC_1A_n40A-n78(2A) | DC_1A_n40A DC_1A_n78A |
| DC_1A-41A_n3A5 DC_1A-41C_n3A5 | DC_1A_n3A DC_41A_n3A DC_41C_n3A |
| DC_1A-41A_n28A5 DC_1A-41C_n28A5 | DC_1A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-(n)41AA DC_1A-(n)41CA DC_1A-(n)41DA | DC_1A_n41A |
| DC_1A-41A_n41A DC_1A-41C_n41A | DC_1A_n41A |
| DC_1A-41A_n77A DC_1A-41C_n77A | DC_1A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | DC_1A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A_n41A-n77A | DC_1A_n41A DC_1A_n77A |
| DC_1A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A_n41A-n78A | DC_1A_n41A DC_1A_n78A |
| DC_1A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A |
| DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | DC_1A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-41A_n79A5 DC_1A-41C_n79A5 | DC_1A_n79A |
| DC_1A-42A_n3A5 | DC_1A_n3A DC_42A_n3A |
| DC_1A-42C_n3A5 | DC_1A_n3A DC_42A_n3A DC_42C_n3A |
| DC_1A-42A_n28A5 | DC_1A_n28A DC_42A_n28A |
| DC_1A-42C_n28A5 | DC_1A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-42A_n77A14, 15,16 DC_1A-42A_n77C15,16 DC_1A-42C_n77A14, 15,16 DC_1A-42C_n77C15,16 DC_1A-42D_n77A14, 15,16 DC_1A-42D_n77C15,16 DC_1A-42E_n77A14, 15,16 DC_1A-42E_n77C15,16 | DC_1A_n77A14, |
| DC_1A-42A_n77(2A)15,16 DC_1A-42C_n77(2A)15,16 | DC_1A_n77A |
| DC_1A-42A_n78A14,15,16 DC_1A-42A_n78C15,16 DC_1A-42C_n78A14,15,16 DC_1A-42C_n78C15,16 DC_1A-42D_n78A14,15,16 DC_1A-42D_n78C15,16 DC_1A-42E_n78A14,15,16 DC_1A-42E_n78C15,16 | DC_1A_n78A14 |
| DC_1A-42A_n79A14 DC_1A-42A_n79C DC_1A-42C_n79A14 DC_1A-42C_n79C DC_1A-42D_n79A14 DC_1A-42D_n79C DC_1A-42E_n79A14 DC_1A-42E_n79C | DC_1A_n79A14 |
| DC_1A_n75A-n78A | DC_1A_n78A |
| DC_1A_n75A-n78(2A) | DC_1A_n78A |
| DC_1A_n77A-n79A14, 23 | DC_1A_n77A14 DC_1A_n79A14 |
| DC_1A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A_SUL_n77A-n80A | DC_1A_n77A DC_1A_n80A |
| DC_1A_SUL_n77A-n84A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |
| DC_1A_n78A-n79A14, 24 | DC_1A_n78A14 DC_1A_n79A14 |
| DC_1A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A |
| DC_1A_SUL_n78A-n84A5 | DC_1A_n78A, DC_1A_n84A_ULSUP-TDM_n78A |
| DC_1A_SUL_n79A-n84A | DC_1A_n79A, DC_1A_n84A_ULSUP-TDM_n79A |
| DC_1A_n78A-n105A | DC_1A_n78A DC_1A_n105A |
| DC_2A_n2A-n38A | DC_2A_n38A |
| DC_2A_n2A-n41A | DC_2A_n41A |
| DC_2A_n2A-n66A | DC_2A_n66A |
| DC_2A_n2A-n71A | DC_2A_n71A |
| DC_2A_n2A-n77A14 DC_2A_n2A-n77C14 | DC_2A_n77A14 |
| DC_2A_n2A-n78A | DC_2A_n78A |
| DC_2A-4A_n28A | DC_2A_n28A DC_4A_n28A |
| DC_2A-4A_n38A | DC_2A_n38A DC_4A_n38A |
| DC_2A-4A_n41A | DC_2A_n41A DC_4A_n41A |
| DC_2A-4A_n78A | DC_2A_n78A DC_4A_n78A |
| DC_2A-5A_n2A | DC_5A_n2A DC_2A_n2A2 |
| DC_2A-5B_n2A | DC_5A_n2A |
| DC_2A-5A-5A_n2A | DC_5A_n2A |
| DC_2A-5A_n5A | DC_2A_n5A |
| DC_2A-2A-5A_n5A | DC_2A_n5A |
| DC_2A-(n)5AA | DC_2A_n5A DC_(n)5AA2 |
| DC_2A-2A-(n)5AA | DC_2A_n5A DC_(n)5AA2 |
| DC_2A-5A_n7A | DC_2A_n7A DC_5A_n7A |
| DC_2A-5A_n12A | DC_2A_n12A DC_5A_n12A |
| DC_2A-5A_n30A | DC_2A_n30A DC_5A_n30A |
| DC_2A-2A-5A_n30A | DC_2A_n30A DC_5A_n30A |
| DC_2A-5A_n48A DC_2A-5A_n48B | DC_2A_n48A DC_5A_n48A |
| DC_2A-5A_n66A DC_2A-5B_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-5A_n66A DC_2A-2A-5A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A_n71A | DC_2A_n71A DC_5A_n71A |
| DC_2A-5A_n77A14 DC_2A-5A_n77C14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-5A_n77(2A)14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-2A-5A_n77A14 DC_2A-2A-5A_n77C14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-2A-5A_n77(2A)14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-5A_n78A | DC_2A_n78A DC_5A_n78A |
| DC_2A-5A_n78(2A) | DC_2A_n78A DC_5A_n78A |
| DC_2A-7A_n2A | DC_7A_n2A |
| DC_2A-7A_n5A DC_2A-7C_n5A | DC_2A_n5A DC_7A_n5A |
| DC_2A-7A-7A_n5A | DC_2A_n5A DC_7A_n5A |
| DC_2A-7A_n7A | DC_2A_n7A DC_7A_n7A2 |
| DC_2A-7A_n25A15, 16 DC_2A-7A-7A_n25A15, 16 DC_2A-7C_n25A15, 16 | DC_7A_n25A |
| DC_2A-7A_n28A DC_2A-7C_n28A | DC_2A_n28A DC_7A_n28A |
| DC_2A_n5A-n77A14 DC_2A-2A_n5A-n77A14 DC_2A_n5A-n77C14 DC_2A-2A_n5A-n77C14 | DC_2A_n5A DC_2A_n77A14 |
| DC_2A-7A_n66A DC_2A-7C_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7C_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A_n7A-n66A | DC_2A_n7A DC_7A_n66A |
| DC_2A_n7(2A)-n66A | DC_7A_n66A |
| DC_2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-7A_n77A DC_2A-7C_n77A | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-7A_n77A | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-7A_n77(2A) | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A_n78A DC_2A-7C_n78A | DC_2A_n78A DC_7A_n78A DC_7C_n78A |
| DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) | DC_2A_n78A DC_7A_n78A DC_7C_n78A |
| DC_2A-2A-7A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A_n7A-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7(2A)-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7A-n78(2A) | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7(2A)-n78(2A) | DC_2A_n7A DC_2A_n78A |
| DC_2A-7A-7A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-7A_n78(2A) | DC_2A_n78A DC_7A_n78A |
| DC_2A-8A_n2A | DC_2A_n2A2 DC_8A_n2A |
| DC_2A-12A_n2A | DC_12A_n2A |
| DC_2A-12A_n5A | DC_2A_n5A DC_12A_n5A |
| DC_2A-2A-12A_n5A | DC_2A_n5A DC_12A_n5A |
| DC_2A-12A_n7A | DC_2A_n7A DC_12A_n7A |
| DC_2A-12A_n7(2A) | DC_2A_n7A DC_12A_n7A |
| DC_2A-(n)12AA | DC_2A_n12A DC_(n)12AA2 |
| DC_2A-12A_n30A | DC_2A_n30A DC_12A_n30A |
| DC_2A-2A-12A_n30A | DC_2A_n30A DC_12A_n30A |
| DC_2A-12A_n41A | DC_2A_n41A DC_12A_n41A |
| DC_2A-2A-12A_n41A | DC_2A_n41A DC_12A_n41A |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-12A_n77A14 DC_2A-2A-12A_n77A14 | DC_2A_n77A14 DC_12A_n77A14 |
| DC_2A-12A_n77(2A) 14 DC_2A-2A-12A_n77(2A) 14 | DC_2A_n77A14 DC_12A_n77A14 |
| DC_2A_n12A-n77A | DC_2A_n77A DC_2A_n12A |
| DC_2A-13A_n2A | DC_13A_n2A |
| DC_2A-12A_n78A | DC_2A_n78A DC_12A_n78A |
| DC_2A-12A_n78(2A) | DC_2A_n78A DC_12A_n78A |
| DC_2A-2A-12A_n78A | DC_2A_n78A DC_12A_n78A |
| DC_2A-13A_n5A | DC_2A_n5A |
| DC_2A-2A-13A_n5A | DC_2A_n5A |
| DC_2A-13A_n25A16,20 | DC_13A_n25A |
| DC_2A-13A_n48A DC_2A-13A_n48B | DC_2A_n48A DC_13A_n48A |
| DC_2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-13A_n77A14 DC_2A-13A_n77C14 DC_2A-2A-13A_n77C14 | DC_2A_n77A14 DC_13A_n77A14 |
| DC_2A-2A-13A_n77A | DC_2A_n77A14 DC_13A_n77A14 |
| DC_2A-14A_n2A | DC_2A_n2A2 DC_14A_n2A |
| DC_2A-14A_n5A | DC_2A_n5A DC_14A_n5A |
| DC_2A-2A-14A_n5A | DC_2A_n5A DC_14A_n5A |
| DC_2A-14A_n30A | DC_2A_n30A DC_14A_n30A |
| DC_2A-2A-14A_n30A | DC_2A_n30A DC_14A_n30A |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-14A_n77A14 DC_2A-2A-14A_n77A14 | DC_2A_n77A14 DC_14A_n77A14 |
| DC_2A-14A_n77(2A) 14 DC_2A-2A-14A_n77(2A) 14 | DC_2A_n77A14 DC_14A_n77A14 |
| DC_2A_n25A-n66A | DC_2A_n66A |
| DC_2A-28A_n7A | DC_2A_n7A DC_28A_n7A |
| DC_2A-28A_n66A | DC_2A_n66A DC_28A_n66A |
| DC_2A-28A_n78A | DC_2A_n78A DC_28A_n78A |
| DC_2A-28A_n78(2A) | DC_2A_n78A DC_28A_n78A |
| DC_2A-29A_n30A | DC_2A_n30A |
| DC_2A-2A-29A_n30A | DC_2A_n30A |
| DC_2A-29A_n66A | DC_2A_n66A |
| DC_2A-2A-29A_n66A | DC_2A_n66A |
| DC_2A-29A_n77A14 DC_2A-2A-29A_n77A14 | DC_2A_n77A14 |
| DC_2A-29A_n78A | DC_2A_n78A |
| DC_2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-30A_n2A | DC_2A_n2A2 DC_30A_n2A |
| DC_2A-2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-30A_n77A14 DC_2A-2A-30A_n77A14 | DC_2A_n77A14 DC_30A_n77A14 |
| DC_2A-30A_n77(2A) 14 DC_2A-2A-30A_n77(2A) 14 | DC_2A_n77A14 DC_30A_n77A14 |
| DC_2A_n38A-n66A | DC_2A_n38A DC_2A_n66A |
| DC_2A_n38A-n71A | DC_2A_n38A DC_2A_n71A |
| DC_2A-38A_n78A | DC_2A_n78A DC_38A_n78A |
| DC_2A_n38A-n78A | DC_2A_n38A DC_2A_n78A |
| DC_2A_n41A-n66A DC_2A_n41C-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41(2A)-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41A-n71A DC_2A_n41C-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-2A_n41A-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n2A3 DC_2A-46C_n2A3 DC_2A-46D_n2A3 DC_2A-46E_n2A3 | DC_2A_n2A2 |
| DC_2A-46A_n5A3 DC_2A-46C_n5A3 DC_2A-46D_n5A3 DC_2A-46E_n5A3 DC_2A-2A-46A_n5A3 DC_2A-2A-46C_n5A3 DC_2A-2A-46D_n5A3 | DC_2A_n5A |
| DC_2A-46A_n41A DC_2A-46C_n41A DC_2A-46D_n41A | DC_2A_n41A |
| DC_2A-46A_n41(2A) DC_2A-46C_n41(2A) DC_2A-46D_n41(2A) | DC_2A_n41A |
| DC_2A-46A_n66A DC_2A-46C_n66A DC_2A-46D_n66A DC_2A-46E_n66A | DC_2A_n66A |
| DC_2A-46A_n71A DC_2A-46C_n71A DC_2A-46D_n71A | DC_2A_n71A |
| DC_2A-46A_n77A | DC_2A_n77A |
| DC_2A-46A-46A_n77A | DC_2A_n77A |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | DC_2A_n2A2 DC_48A_n2A21 |
| DC_2A-48A_n5A | DC_2A_n5A DC_48A_n5A |
| DC_2A-48C_n5A DC_2A-48D_n5A DC_2A-48E_n5A | DC_2A_n5A |
| DC_2A_n48A-n66A | DC_2A_n48A DC_2A_n66A |
| DC_2A-48A_n71A | DC_2A_n71A DC_48A_n71A |
| DC_2A-48A_n12A | DC_2A_n12A DC_48A_n12A |
| DC_2A-48A_n48A | DC_2A_n48A |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A DC_2A-48E_n66A | DC_2A_n66A DC_48A_n66A |
| DC_2A-48A_n77A14,15,16 | DC_2A_n77A14 |
| DC_2A-48A-48A_n77A14,15,16 | DC_2A_n77A DC_48A_n77A |
| DC_2A-48A-48A-48A_n77A14,15,16 | DC_2A_n77A DC_48A_n77A |
| DC_2A-48C_n77A14,15,16 DC_2A-48D_n77A14,15,16 DC_2A-48E_n77A14,15,16 DC_2A-48A_n77C14,15,16 DC_2A-48C_n77C14,15,16 DC_2A-48D_n77C14,15,16 | DC_2A_n77A14 |
| DC_2A-66A_n2A | DC_2A_n2A2 DC_66A_n2A |
| DC_2A-66A-66A_n2A | DC_66A_n2A |
| DC_2A-66A_n5A DC_2A-66B_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A_n7A | DC_2A_n7A DC_66A_n7A |
| DC_2A-66A-66A_n7A | DC_2A_n7A DC_66A_n7A |
| DC_2A-66A_n12A | DC_2A_n12A DC_66A_n12A |
| DC_2A-66A_n25A16,20 | DC_66A_n25A |
| DC_2A-66A_n28A | DC_2A_n28A DC_66A_n28A |
| DC_2A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-2A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-66A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-66A_n41A14 DC_2A-66A_n41C DC_2C-66A_n41A | DC_2A_n41A DC_66A_n41A14 |
| DC_2A-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-2A-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-66A-66A_n66A DC_2A-66B_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-(n)66AA | DC_2A_n66A |
| DC_2A-2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-2A-66A-66A_n66A | DC_2A_n66A |
| DC_2A-66A_n71A DC_2A-66A_n71B DC_2A-66C_n71A DC_2C-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-2A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-2A_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-66A_n77A14 DC_2A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A_n77(2A)14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-2A-66A_n77A14 DC_2A-2A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-2A-66A_n77(2A) 14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A-66A_n77A14 DC_2A-66A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A-66A_n77(2A) 14 | DC_2A_ n77A14 DC_66A_ n77A14 |
| DC_2A-2A-66A-66A_n77A14 DC_2A-2A-66A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A_n66A-n77A14 DC_2A_n66A-n77C14 DC_2A-2A_n66A-n77A14 DC_2A-2A_n66A-n77C14 | DC_2A_n77A14 DC_2A_n66A |
| DC_2A-66A_n78A DC_2A-2A-66A_n78A | DC_2A_n78A DC_66A_n78A |
| DC_2A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A |
| DC_2A_n66A-n78A DC_2A-2A_n66A-n78A | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66A-n78(2A) | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66(2A)-n78A | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66(2A)-n78(2A) | DC_2A_n66A DC_2A_n78A |
| DC_2A-66A-66A_n78A | DC_2A_n78A DC_66A_n78A |
| DC_2A-66A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A |
| DC_2A-71A_n2A | DC_71A_n2A |
| DC_2A-71A_n7A | DC_2A_n7A DC_71A_n7A |
| DC_2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-71A_n41A | DC_2A_n41A DC_71A_n41A |
| DC_2A-2A-71A_n41A | DC_2A_n41A DC_71A_n41A |
| DC_2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-71A_n71A | DC_2A_n71A |
| DC_2A-71A_n77A | DC_2A_n77A DC_71A_n77A |
| DC_2A-71A_n77(2A) | DC_2A_n77A DC_71A_n77A |
| DC_2A_n71A-n77A | DC_2A_n71A DC_2A_n77A |
| DC_2A-2A_n71A-n77A | DC_2A_n71A DC_2A_n77A |
| DC_2A-71A_n77(2A) | DC_2A_n71A |
|  | DC_2A_n77A |
| DC_2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A-71A_n78(2A) | DC_71A_n78A DC_2A_n78A |
| DC_2A-2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A_n71A-n78A | DC_2A_n71A DC_2A_n78A |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |
| DC_3A_n1A-n7A | DC_3A_n1A DC_3A_n7A |
| DC_3C_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_3C_n1A DC_3C_n7A |
| DC_3A_n1A-n8A | DC_3A_n1A DC_3A_n8A |
| DC_3A-3A_n1A-n8A | DC_3A_n1A DC_3A_n8A |
| DC_3A_n1A-n28A | DC_3A_n1A DC_3A_n28A |
| DC_3C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A |
| DC_3A_n1A-n38A | DC_3A_n1A DC_3A_n38A |
| DC_3A_n1A-n40A | DC_3A_n1A DC_3A_n40A |
| DC_3A_n1A-n41A | DC_3A_n1A DC_3A_n41A |
| DC_3A_n1A-n75A | DC_3A_n1A |
| DC_3C_n1A-n75A | DC_3C_n1A |
| DC_3A_n1A-n77A5 | DC_3A_n1A DC_3A_n77A |
| DC_3A_n1A-n78A5 14 DC_3C_n1A-n78A5 | DC_3A_n1A DC_3C_n1A DC_3A_n78A14 DC_3C_n78A |
| DC_3A_n1A-n78(2A)5 DC_3C_n1A-n78(2A)5 | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A |
| DC_3A-3A_n1A-n78A5 | DC_3A_n1A DC_3A_n78A |
| DC_3A_n1A-n79A5 | DC_3A_n1A DC_3A_n79A |
| DC_(n)3AA-n7A | DC_(n)3AA2 DC_3A_n7A |
| DC_3A_n3A-n7A | DC_3A_n3A2 DC_3A_n7A |
| DC_(n)3AA-n8A | DC_(n)3AA2 DC_3A_n8A |
| DC_(n)3AA-n28A | DC_(n)3AA2 DC_3A_n28A |
| DC_3A_n3A-n28A | DC_3A_n3A2 DC_3A_n28A |
| DC_3A_n3A-n41A | DC_3A_n41A DC_3A_n3A2 |
| DC_(n)3AA-n67A | DC_(n)3AA2 |
| DC_3A_n3A-n67A | DC_3A_n3A2 |
| DC_3A_n3A-n77A5 | DC_3A_n77A DC_3A_n3A2 |
| DC_(n)3AA-n77A | DC_(n)3AA2 DC_3A_n77A |
| DC_(n)3AA-n77(2A) | DC_(n)3AA2 DC_3A_n77A |
| DC_(n)3AA-n78A | DC_(n)3AA1 DC_3A_n78A |
| DC_(n)3AA-n78(2A) | DC_(n)3AA1 DC_3A_n78A |
| DC_3A_n3A-n78A5 | DC_3A_n78A DC_3A_n3A2 |
| DC_3A-5A_n40A | DC_3A_n40A DC_5A_n40A |
| DC_3A_n5A-n40A | DC_3A_n5A DC_3A_n40A |
| DC_3A-5A_n77A | DC_3A_n77A DC_5A_n77A |
| DC_3A-5A_n77(2A) DC_3A-5A_n77(3A) | DC_3A_n77A DC_5A_n77A |
| DC_3A-5A_n78A5 DC_3C-5A_n78A DC_3A-5A_n78C5 | DC_3A_n78A DC_5A_n78A |
| DC_3A-5A_n78(2A)5 | DC_3A_n78A DC_5A_n78A |
| DC_3A-5A_n78(A-C)5 | DC_3A_n78A DC_5A_n78A |
| DC_3A_n5A-n78A5, 14 DC_3C_n5A-n78A5, 14 | DC_3A_n5A DC_3A_n78A14 DC_3C_n78A14 |
| DC_3A-5A_n79A5 | DC_3A_n79A DC_5A_n79A |
| DC_3A-7A_n1A DC_3A-7C_n1A DC_3C-7A_n1A DC_3C-7C_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A |
| DC_3A-3A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A_n3A DC_3A-7C_n3A | DC_3A_n3A2 DC_7A_n3A |
| DC_3A-7A_n5A DC_3C-7A_n5A DC_3A-7C_n5A DC_3C-7C_n5A | DC_3A_n5A DC_7A_n5A DC_7C_n5A |
| DC_3A-7A_n7A DC_3C-7A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A2 |
| DC_3A-3A-7A_n7A | DC_3A_n7A DC_7A_n7A2 |
| DC_3A-(n)7AA DC_3C-(n)7AA | DC_3A_n7A |
| DC_3A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-3A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-7A_n26A DC_3A-7C_n26A DC_3C-7A_n26A DC_3C-7C_n26A | DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |
| DC_3A-7A-7A_n28A | DC_3A_n28A DC_7A_n28A |
| DC_3A-7A_n40A | DC_3A_n40A DC_7A_n40A |
| DC_3A-7A-7A_n40A | DC_3A_n40A DC_7A_n40A |
| DC_3A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-3A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-3A-7A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A_n77(2A) DC_3A-7A_n77(3A) | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A-7A_n77(2A) DC_3A-7A-7A_n77(3A) | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A_n78A5,14 DC_3C-7A_n78A5,14 DC_3A-7C_n78A5,14 DC_3C-7C_n78A5,14 DC_3A-7A_n78C5 | DC_3A_n78A14 DC_3C_n78A14 DC_7A_n78A14 DC_7C_n78A14 |
| DC_3A_n7A-n28A DC_3C_n7A-n28A | DC_3A_n7A DC_3A_n28A DC_3C_n7A |
| DC_3A-7A_n78(2A)5 DC_3C-7A_n78(2A)5 DC_3A-7C_n78(2A)5 DC_3C-7C_n78(2A)5 | DC_3A_n78A DC_7A_n78A DC_3C_n78A DC_7C_n78A |
| DC_3A-7A_n78(A-C)5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A_n78A5, 14 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A-7A-7A_n78A5, 14 DC_3A-7A-7A_n78C5 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A-7A-7A_n78(2A)5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-7A-7A_n78(A-C)5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A-7A_n78A5, 14 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A_n7A-n78A5 DC_3A_n7B-n78A5 DC_3C_n7A-n78A5 DC_3C_n7B-n78A5 | DC_3A_n7A DC_3C_n7A DC_3A_n78A DC_3C_n78A |
| DC_3A-3A_n7A-n78A5 DC_3A-3A_n7B-n78A5 | DC_3A_n7A DC_3A_n7B DC_3A_n78A |
| DC_3A_n7A-n78(2A)5 DC_3C_n7A-n78(2A)5 | DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_3A-7A_n105A | DC_3A_n105A DC_7A_n105A |
| DC_3A-8A_n1A DC_3C-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-3A-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A_n7A | DC_3A_n7A DC_8A_n7A |
| DC_3A-3A_n8A-n78A5 | DC_3A_n8A DC_3A_n78A |
| DC_3A_n8A-n40A | DC_3A_n8A DC_3A_n40A |
| DC_3A-8A_n41A | DC_3A_n41A DC_8A_n41A |
| DC_3A_n8A-n41A | DC_3A_n41A DC_3A_n8A |
| DC_3A-8A_n28A DC_3C-8A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n40A | DC_3A_n40A DC_8A_n40A |
| DC_3A-8A_n77A5 DC_3C-8A_n77A | DC_3A_n77A DC_3C_n77A DC_8A_n77A |
| DC_3A-8A_n77(2A) 5 DC_3C-8A_n77(2A) | DC_3A_n77A DC_3C_n77A DC_8A_n77A |
| DC_3A-8A_n77(3A) 5 | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n78A5, 14 DC_3C-8A_n78A5 | DC_3A_n78A14 DC_8A_n78A14 |
| DC_3A-8A_n78(2A) | DC_3A_n78A DC_8A_n78A |
| DC_3A-3A-8A_n78A5, 14 | DC_3A_n78A14 DC_8A_n78A14 |
| DC_3A-8B_n78A5 | DC_3A_n78A DC_8A_n78A DC_8B_n78A |
| DC_3A-3A-8B_n78A5 | DC_3A_n78A DC_8A_n78A DC_8B_n78A |
| DC_3A-8A_n79A5 DC_3A-8A_n79C5 | DC_3A_n79A DC_8A_n79A |
| DC_3A_n8A-n77A5 | DC_3A_n8A DC_3A_n77A |
| DC_3A_n8A-n77(2A)5 | DC_3A_n8A DC_3A_n77A |
| DC_3A_n8A-n78A5 | DC_3A_n8A DC_3A_n78A |
| DC_3A-11A_n28A | DC_3A_n28A DC_11A_n28A |
| DC_3A-11A_n77A5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-11A_n77(2A) 5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-11A_n77(3A) 5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-18A_n3A | DC_3A_n3A2 DC_18A_n3A |
| DC_3A-18A_n28A | DC_3A_n28A DC_18A_n28A |
| DC_3A-18A_n41A | DC_3A_n41A DC_18A_n41A |
| DC_3A-18A_n77A | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A_n77(2A) | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A_n78(2A) | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A_n1A | DC_3A_n1A DC_19A_n1A |
| DC_3A-19A_n77A5,14 DC_3A-19A_n77C5 | DC_3A_n77A14 DC_19A_n77A14 |
| DC_3A-19A_n77(2A)5,14 | DC_3A_n77A14 DC_19A_n77A14 |
| DC_3A-19A_n78A5,14 DC_3A-19A_n78C5 | DC_3A_n78A14 DC_19A_n78A14 |
| DC_3A-19A_n78(2A)5,14 | DC_3A_n78A14 DC_19A_n78A14 |
| DC_3A-19A_n79A5,14 DC_3A-19A_n79C5 | DC_3A_n79A14 DC_19A_n79A14 |
| DC_3A-20A_n1A DC_3C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-3A-20A_n1A | DC_3A_n1A DC_20A_n1A |
| DC_3A-20A_n3A | DC_3A_n3A2 DC_20A_n3A |
| DC_3A-20A_n7A DC_3C-20A_n7A | DC_3A_n7A DC_3C_n7A DC_20A_n7A |
| DC_3A-20A_n8A | DC_3A_n8A DC_20A_n8A |
| DC_3A-20A_n28A5,6,16,20 DC_3C-20A_n28A5,6,16,20 | DC_3A_n28A DC_20A_n28A |
| DC_3A-20A_n41A | DC_3A_n41A DC_20A_n41A |
| DC_3C-20A_n41A | DC_3C_n41A DC_20A_n41A |
| DC_3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_3A_n20A-n67A DC_3C_n20A-n67A | DC_3A_n20A |
| DC_3A-20A_n78A5 DC_3C-20A_n78A5 DC_3A-20A_n78C5 | DC_3A_n78A DC_3C_n78A DC_20A_n78A |
| DC_3A-3A-20A_n78A | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A_n78(2A)5 | DC_3A_n78A DC_20A_n78A |
| DC_3A_n20A-n78A | DC_3A_n20A DC_3A_n78A |
| DC_3A-21A_n1A10,11 | DC_3A_n1A DC_21A_n1A |
| DC_3A-21A_n28A13 | DC_3A_n28A DC_21A_n28A |
| DC_3A-21A_n77A5, 14 DC_3A-21A_n77C5, 14 | DC_3A_n77A14 DC_21A_n77A14 |
| DC_3A-21A_n77(2A)5,14 | DC_3A_n77A14 DC_21A_n77A14 |
| DC_3A-21A_n78A5,14 DC_3A-21A_n78C5 | DC_3A_n78A14 DC_21A_n78A14 |
| DC_3A-21A_n78(2A)5,14 | DC_3A_n78A14 DC_21A_n78A14 |
| DC_3A-21A_n79A5,14 DC_3A-21A_n79C5 | DC_3A_n79A14 DC_21A_n79A14 |
| DC_3A-26A_n78A DC_3A-26A_n78(2A) DC_3C-26A_n78A | DC_3A_n78A DC_26A_n78A |
| DC_3A-26A_n78(2A) DC_3C-26A_n78(2A) | DC_3A_n78A DC_26A_n78A |
| DC_3A_n26A-n78A | DC_3A_n26A DC_3A_n78A |
| DC_3C_n26A-n78A | DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |
| DC_3A-28A_n1A DC_3C-28A_n1A | DC_28A_n1A DC_3A_n1A |
| DC_3A-28A_n3A | DC_3A_n3A2 DC_28A_n3A |
| DC_3A-28A_n5A DC_3C-28A_n5A | DC_3A_n5A DC_28A_n5A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |
| DC_3A-28A_n40A | DC_3A_n40A DC_28A_n40A |
| DC_3A-3A-28A_n7A DC_3A-3A-28A_n7B | DC_3A_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |
| DC_3A-28A_n38A | DC_3A_n38A DC_28A_n38A |
| DC_3A_n28A-n40A | DC_3A_n28A DC_3A_n40A |
| DC_3A_n28A-n41A5 | DC_3A_n28A DC_3A_n41A |
| DC_3A-28A_n41A5 | DC_3A_n41A DC_28A_n41A |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | DC_3A_n28A |
| DC_3A-28A_n77A5 DC_3A-28A_n77C5 | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A_n77(2A)5 | DC_3A_n77A DC_28A_n77A |
| DC_3A_n28A-n77A5 | DC_3A_n28A DC_3A_n77A |
| DC_3A_n28A-n77(2A)5 | DC_3A_n28A DC_3A_n77A |
| DC_3A-28A_n78A5,14 DC_3C-28A_n78A5,14 DC_3A-28A_n78(2A)5,14 DC_3A-28A_n78C5 | DC_3A_n78A14 DC_3C_n78A14 DC_28A_n78A14 |
| DC_3A-3A-28A_n78A | DC_3A_n78A DC_28A_n78A |
| DC_3C-28A_n78(2A)5 | DC_3A_n78A DC_28A_n78A |
| DC_3A_n28A-n78A5, 14 DC_3C_n28A-n78A5, 14 | DC_3A_n28A DC_3A_n78A14 DC_3C_n78A14 |
| DC_3A_n28A-n78(2A)5 DC_3C_n28A-n78(2A)5 | DC_3A_n28A DC_3A_n78A DC_3C_n78A |
| DC_3A-28A_n79A5 DC_3A-28A_n79C5 | DC_3A_n79A DC_28A_n79A |
| DC_3A_n28A-n79A5 | DC_3A_n28A DC_3A_n79A |
| DC_3A-32A_n1A DC_3C-32A_n1A | DC_3A_n1A DC_3C_n1A |
| DC_3A-32A_n7A | DC_3A_n7A |
| DC_3A-32A_n28A DC_3C-32A_n28A | DC_3A_n28A |
| DC_3A-32A_n78A DC_3C-32A_n78A DC_3A-32A_n78C | DC_3A_n78A DC_3C_n78A |
| DC_3A-32A_n78(2A) | DC_3A_n78A DC_3C_n78A |
| DC_3A-38A_n28A DC_3C-38A_n28A | DC_3A_n28A DC_38A_n28A |
| DC_3A_n38A-n40A | DC_3A_n38A DC_3A_n40A |
| DC_3A-38A_n78A | DC_3A_n78A |
| DC_3A-38A_n78(2A) | DC_3A_n78A |
| DC_3A_n38A-n78A | DC_3A_n38A DC_3A_n78A |
| DC_3C-38A_n78A | DC_3A_n78A DC_3C_n78A DC_38A_n78A |
| DC_3A-40A_n1A DC_3A-40C_n1A | DC_3A_n1A DC_40A_n1A |
| DC_3A_n40A-n41A DC_3A_n40A-n41C | DC_3A_n40A DC_3A_n41A |
| DC_3A_n40A-n77A | DC_3A_n40A DC_3A_n77A |
| DC_3A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A |
| DC_3A-40A_n78A DC_3A-40C_n78A | DC_3A_n78A DC_40A_n78A |
| DC_3A-40A_n78(2A) DC_3A-40C_n78(2A) | DC_3A_n78A DC_40A_n78A |
| DC_3A_n40A-n78A DC_3A_n40A-n78C | DC_3A_n40A DC_3A_n78A |
| DC_3A_n40A-n79A DC_3A_n40A-n79C | DC_3A_n40A DC_3A_n79A |
| DC_3A-41A_n1A | DC_3A_n1A DC_41A_n1A |
| DC_3A-41C_n1A | DC_3A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-3A-41A_n1A | DC_3A_n1A DC_41A_n1A |
| DC_3A-3A-41C_n1A | DC_3A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-41A_n3A DC_3A-41C_n3A | DC_3A_n3A2 DC_41A_n3A DC_41C_n3A |
| DC_3A-41A_n28A5 | DC_3A_n28A DC_41A_n28A |
| DC_3A-41C_n28A5 | DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_3A-41A_n41A DC_3A-41C_n41A DC_3A-41D_n41A | DC_3A_n41A |
| DC_3A-(n)41AA DC_3A-(n)41CA DC_3A-(n)41DA | DC_3A_n41A DC_(n)41AA |
| DC_3A-41A_n77A DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_3A-41A_n78A DC_3A-41C_n78A | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-3A-41A_n78A DC_3A-3A-41C_n78A | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A_n41A-n78A | DC_3A_n41A DC_3A_n78A |
| DC_3A_n41A-n78(2A) | DC_3A_n41A DC_3A_n78A |
| DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-42A_n1A5 DC_3A-42C_n1A5 | DC_3A_n1A DC_42A_n1A |
| DC_3A-42A_n28A5 | DC_3A_n28A DC_42A_n28A |
| DC_3A-42C_n28A5 | DC_3A_n28A DC_42A_n28A DC_42C_n28A |
| DC_3A-41A_n79A5 DC_3A-41C_n79A5 | DC_3A_n79A DC_41A_n79A |
| DC_3A_n41A-n77A | DC_3A_n41A DC_3A_n77A |
| DC_3A_n41A-n77(2A) | DC_3A_n41A DC_3A_n77A |
| DC_3A_n41A-n79A5 DC_3A_n41C-n79A5 DC_3A_n41A-n79C5 DC_3A_n41C-n79C5 | DC_3A_n41A DC_3A_n79A |
| DC_3A_SUL_n41A-n80A DC_3C_SUL_n41A-n80A | DC_3A_n41A DC_3C_n41A DC_3A_n80A_ULSUP-TDM_n41A DC_3C_n80A_ULSUP-TDM_n41A |
| DC_3A-42A_n77A14, 15,16 DC_3A-42A_n77C15,16 DC_3A-42C_n77A14, 15,16 DC_3A-42C_n77C15,16 DC_3A-42D_n77A14, 15,16 DC_3A-42D_n77C15,16 DC_3A-42E_n77A14, 15,16 DC_3A-42E_n77C15,16 | DC_3A_n77A14, |
| DC_3A-42A_n77(2A)15,16 DC_3A-42C_n77(2A)15,16 | DC_3A_n77A |
| DC_3A-42A_n78A14,15,16 DC_3A-42A_n78C15,16 DC_3A-42C_n78A14,15,16 DC_3A-42C_n78C15,16 DC_3A-42D_n78A14,15,16 DC_3A-42D_n78C15,16 DC_3A-42E_n78A14,15,16 DC_3A-42E_n78C15,16 | DC_3A_n78A14 |
| DC_3A-42A_n79A14 DC_3A-42A_n79C DC_3A-42C_n79A14 DC_3A-42C_n79C DC_3A-42D_n79A14 DC_3A-42D_n79C DC_3A-42E_n79A14 DC_3A-42E_n79C | DC_3A_n79A14 |
| DC_3A-67A_n3A | DC_3A_n3A2 |
| DC_3A_n75A-n78A | DC_3A_n78A |
| DC_3A_n75A-n78(2A) | DC_3A_n78A |
| DC_3A_n77A-n79A14, 23 | DC_3A_n77A14 DC_3A_n79A14 |
| DC_3A_n78A-n79A14, 24 DC_3A_n78A-n79C | DC_3A_n78A14 DC_3A_n79A14 |
| DC_3A_SUL_n77A-n80A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A |
| DC_3A_SUL_n77A-n84A | DC_3A_n77A DC_3A_n84A |
| DC_3A_SUL_n78A-n80A5 DC_3C_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_3A_SUL_n78A-n82A5 | DC_3A_n78A DC_3A_n82A |
| DC_3A_SUL_n78A-n84A | DC_3A_n78A DC_3A_n84A |
| DC_3A_n78A-n105A | DC_3A_n78A DC_3A_n105A |
| DC_3A_SUL_n79A-n80A5 | DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A |
| DC_4A-7A_n28A | DC_4A_n28A DC_7A_n28A |
| DC_4A-7A_n78A DC_4A-7C_n78A | DC_4A_n78A DC_7A_n78A DC_7C_n78A |
| DC_5A_n1A-n78A | DC_5A_n1A DC_5A_n78A |
| DC_5A_n2A-n77A14 DC_5A_n2A-n77C14 | DC_5A_n77A14 DC_5A_n2A |
| DC_5A_n2A-n78A | DC_5A_n2A DC_5A_n78A |
| DC_5A_n3A-n78A | DC_5A_n3A DC_5A_n78A |
| DC_5A_n5A-n77A14 DC_5A_n5A-n77C14 | DC_5A_n77A14 |
| DC_5A-7A_n2A | DC_7A_n2A |
| DC_5A-7A_n7A | DC_5A_n7A DC_7A_n7A2 |
| DC_5A-7A_n40A | DC_5A_n40A DC_7A_n40A |
| DC_5A-7A-7A_n40A | DC_5A_n40A DC_7A_n40A |
| DC_5A-7A_n66A DC_5A-7C_n66A | DC_5A_n66A DC_7A_n66A |
| DC_5A-7A-7A_n66A | DC_5A_n66A DC_7A_n66A |
| DC_5A-7A_n71A | DC_5A_n71A DC_7A_n71A |
| DC_5A-7A_n77A | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A_n77A | DC_5A_n71A DC_7A_n71A |
| DC_5A-7A_n77(2A) DC_5A-7A_n77(3A) | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A_n77(2A) DC_5A-7A-7A_n77(3A) | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A_n78A DC_5A-7A_n78C DC_5A-7C_n78A | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A_n78(2A) | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A_n78(A-C) | DC_5A_n78A DC_7A_n78A |
| DC_5A_n7A-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7(2A)-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7A-n78(2A) | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7(2A)-n78(2A) | DC_5A_n7A DC_5A_n78A |
| DC_5A-7A-7A_n78A DC_5A-7A-7A_n78C | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A-7A_n78(2A) | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A-7A_n78(A-C) | DC_5A_n78A DC_7A_n78A |
| DC_5A-(n)12AA | DC_5A_n12A DC_(n)12AA2 |
| DC_5A-13A_n2A | DC_5A_n2A DC_13A_n2A |
| DC_5A-13A_n66A | DC_5A_n66A DC_13A_n66A |
| DC_5A-13A_n77A DC_5A-13A_n77C | DC_5A_n77A DC_13A_n77A |
| DC_5A-30A_n2A | DC_5A_n2A DC_30A_n2A |
| DC_5A-30A_n5A | DC_30A_n5A |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |
| DC_5A-30A_n77A14 | DC_5A_n77A14 DC_30A_n77A14 |
| DC_5A-30A_n77(2A) 14 | DC_5A_n77A14 DC_30A_n77A14 |
| DC_5A_n38A-n66A | DC_5A_n38A DC_5A_n66A |
| DC_5A_n40A-n77A | DC_5A_n40A DC_5A_n77A |
| DC_5A_n40A-n77(2A) | DC_5A_n40A DC_5A_n77A |
| DC_5A_n40A-n78A DC_5A_n40A-n78C | DC_5A_n40A DC_5A_n78A |
| DC_5A-41A_n79A | DC_5A_n79A DC_41A_n79A |
| DC_5A-46A_n66A | DC_5A_n66A DC_46A_n66A |
| DC_5A-48A_n5A | DC_48A_n5A |
| DC_5A-48A_n12A | DC_5A_n12A DC_48A_n12A |
| DC_5A-48A_n71A | DC_5A_n71A DC_48A_n71A |
| DC_5A-48A_n77A14,15,16 DC_5A-48C_n77A14,15,16 DC_5A-48D_n77A14,15,16 DC_5A-48A_n77C14,15,16 DC_5A-48C_n77C14,15,16 DC_5A-48D_n77C14,15,16 | DC_5A_n77A14 |
| DC_5A-66A_n2A DC_5B-66A_n2A DC_5A-66B_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-5A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-5A-66A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-66A_n5A | DC_66A_n5A |
| DC_5A-66A-66A_n5A | DC_66A_n5A |
| DC_5A-66A_n7A | DC_5A_n7A DC_66A_n7A |
| DC_5A-66A-66A_n7A | DC_5A_n7A DC_66A_n7A |
| DC_5A-66A_n12A | DC_5A_n12A DC_66A_n12A |
| DC_5A-66A_n30A | DC_5A_n30A DC_66A_n30A |
| DC_5A-66A-66A_n30A | DC_5A_n30A DC_66A_n30A |
| DC_5A-66A_n48A DC_5A-66A_n48B | DC_5A_n48A DC_66A_n48A |
| DC_5A-66A-66A_n48A DC_5A-66A-66A_n48B | DC_5A_n48A DC_66A_n48A |
| DC_5A-66A_n66A DC_5B-66A_n66A | DC_5A_n66A |
| DC_5A-5A-66A_n66A | DC_5A_n66A |
| DC_5A-66A-66A_n66A DC_5B-66A-66A_n66A | DC_5A_n66A |
| DC_5A-5A-66A-66A_n66A | DC_5A_n66A |
| DC_5A-66A_n71A | DC_5A_n71A DC_66A_n71A |
| DC_5A-66A_n77A14 DC_5A-66A_n77C14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A-66A_n77(2A) 14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A-66A-66A_n77A14 DC_5A-66A-66A_n77C14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A-66A-66A_n77(2A) 14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A_n66A-n77A14 DC_5A_n66A-n77C14 | DC_5A_n66A DC_5A_n77A14 |
| DC_5A-66A_n78A | DC_5A_n78A DC_66A_n78A |
| DC_5A-66A_n78(2A) | DC_5A_n78A DC_66A_n78A |
| DC_5A_n66A-n78A | DC_5A_n66A DC_5A_n78A |
| DC_5A-66A-66A_n78A | DC_5A_n78A DC_66A_n78A |
| DC_7A_n1A-n8A | DC_7A_n1A DC_7A_n8A |
| DC_7A-7A_n1A-n8A | DC_7A_n1A DC_7A_n8A |
| DC_7A_n1A-n28A | DC_7A_n1A DC_7A_n28A |
| DC_7C_n1A-n28A | DC_7A_n1A DC_7A_n28A DC_7C_n1A DC_7C_n28A |
| DC_7A_n1A-n40A | DC_7A_n1A DC_7A_n40A |
| DC_7A_n1A-n75A | DC_7A_n1A |
| DC_7A_n1A-n78A5, 14 DC_7C_n1A-n78A5 | DC_7A_n1A DC_7A_n78A14 DC_7C_n1A DC_7C_n78A |
| DC_7A_n1A-n78(2A)5 DC_7C_n1A-n78(2A)5 | DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_7A-7A_n1A-n78A5 14 | DC_7A_n1A DC_7A_n78A14 |
| DC_7A_n2A-n66A | DC_7A_n2A DC_7A_n66A |
| DC_7A_n2A-n71A | DC_7A_n2A DC_7A_n71A |
| DC_7A_n2A-n78A | DC_7A_n2A DC_7A_n78A |
| DC_7A_n3A-n78A DC_7C_n3A-n78A | DC_7A_n3A DC_7A_n78A DC_7C_n3A DC_7C_n78A |
| DC_7A_n3A-n78(2A) DC_7C_n3A-n78(2A) | DC_7A_n3A DC_7A_n78A DC_7C_n3A DC_7C_n78A |
| DC_7A_n5A-n40A | DC_7A_n5A DC_7A_n40A |
| DC_7A_n5A-n78A14 DC_7C_n5A-n78A14 | DC_7A_n5A DC_7C_n5A DC_7A_n78A14 DC_7C_n78A14 |
| DC_7A_n7A-n78A5 | DC_7A_n78A DC_7A_n7A2 |
| DC_7A_n7A-n78(2A) | DC_7A_n78A DC_7A_n7A2 |
| DC_7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A |
| DC_7A-8A_n7A | DC_7A_n7A DC_8A_n7A |
| DC_7A-8A_n20A | DC_7A_n20A DC_8A_n20A |
| DC_7A-8A_n28A | DC_7A_n28A DC_8A_n28A |
| DC_7A-7A-8A_n28A | DC_7A_n28A DC_8A_n28A |
| DC_7A-8A_n40A | DC_7A_n40A DC_8A_n40A |
| DC_7A_n8A-n40A | DC_7A_n8A DC_7A_n40A |
| DC_7A-8A_n77A5 | DC_7A_n77A DC_8A_n77A |
| DC_7A-8A_n78A5, 14 | DC_7A_n78A14 DC_8A_n78A14 |
| DC_7A-8A_n78(2A) | DC_7A_n78A DC_8A_n78A |
| DC_7A-7A-8A_n78A5, 14 | DC_7A_n78A14 DC_8A_n78A14 |
| DC_7A-7A_n8A-n78A5 | DC_7A_n8A DC_7A_n78A |
| DC_7A-8B_n78A5 DC_7A-7A-8B_n78A5 | DC_7A_n78A DC_8A_n78A DC_8B_n78A |
| DC_7A_n8A-n78A5 | DC_7A_n8A DC_7A_n78A |
| DC_7A-12A_n2A | DC_7A_n2A DC_12A_n2A |
| DC_7A-12A_n2(2A) | DC_7A_n2A DC_12A_n2A |
| DC_7A-12A_n25A | DC_7A_n25A DC_12A_n25A |
| DC_7A-12A_n66A | DC_7A_n66A DC_12A_n66A |
| DC_7A-12A_n77A | DC_7A_n77A DC_12A_n77A |
| DC_7A-12A_n77A(2A) | DC_7A_n77A DC_12A_n77A |
| DC_7A_n12A-n77A | DC_7A_n12A DC_7A_n77A |
| DC_7A-12A_n78A | DC_7A_n78A DC_12A_n78A |
| DC_7A-12A_n78(2A) | DC_7A_n78A DC_12A_n78A |
| DC_7A-13A_n25A DC_7C-13A_n25A | DC_7A_n25A DC_13A_n25A |
| DC_7A-7A-13A_n25A | DC_7A_n25A DC_13A_n25A |
| DC_7A-13A_n66A DC_7C-13A_n66A | DC_7A_n66A DC_13A_n66A |
| DC_7A-7A-13A_n66A | DC_7A_n66A DC_13A_n66A |
| DC_7A-20A_n1A DC_7C-20A_n1A | DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_7A-20A_n3A DC_7C-20A_n3A | DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A_n28A6,16,20 | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A_n78A5 DC_7A-20A_n78C5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-20A_n78(2A)5 | DC_7A_n78A DC_20A_n78A |
| DC_7A_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7A-7A_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7C_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7A-25A_n77A DC_7C-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-7A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-25A-25A_n77A DC_7C-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-7A-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-25A_n78A DC_7C-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-7A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-25A-25A_n78A DC_7C-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-7A-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-26A_n78A DC_7C-26A_n78A | DC_7A_n78A DC_26A_n78A |
| DC_7A-26A_n78(2A) DC_7C-26A_n78(2A) | DC_7A_n78A DC_26A_n78A |
| DC_7A_n26A-n78A DC_7A_n26A-n78(2A) | DC_7A_n26A DC_7A_n78A |
| DC_7C_n26A-n78A DC_7C_n26A-n78(2A) | DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_7A-28A_n1A | DC_28A_n1A DC_7A_n1A |
| DC_7A-7A-28A_n1A | DC_28A_n1A DC_7A_n1A |
| DC_7A-28A_n2A | DC_7A_n2A DC_28A_n2A |
| DC_7A-28A_n3A DC_7C-28A_n3A | DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_7A-28A_n5A6 DC_7C-28A_n5A6 | DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_7A-28A_n7A | DC_7A_n7A2 DC_28A_n7A |
| DC_7A-28A_n20A | DC_7A_n20A DC_28A_n20A |
| DC_7A_n28A-n40A | DC_7A_n28A DC_7A_n40A |
| DC_7A-28A_n40A | DC_7A_n40A DC_28A_n40A |
| DC_7A-28A_n66A DC_7C-28A_n66A | DC_7A_n66A DC_28A_n66A |
| DC_7A-28A_n78A5,14 DC_7C-28A_n78A5,14 DC_7A-28A_n78(2A)5,14 DC_7C-28A_n78(2A)5,14 | DC_7A_n78A14 DC_7C_n78A14 DC_28A_n78A14 |
| DC_7A-28A_n78(2A) DC_7C-28A_n78(2A) | DC_7A_n78A DC_28A_n78A |
| DC_7A_n28A-n78A5,14 DC_7C_n28A-n78A14 | DC_7A_n28A DC_7A_n78A14 DC_7C_n28A DC_7C_n78A14 |
| DC_7A-29A_n78A DC_7C-29A_n78A | DC_7A_n78A |
| DC_7A-7A-29A_n78A | DC_7A_n78A |
| DC_7A-32A_n1A | DC_7A_n1A |
| DC_7A-32A_n3A DC_7C-32A_n3A | DC_7A_n3A |
| DC_7A-32A_n8A | DC_7A_n8A |
| DC_7A-32A_n28A | DC_7A_n28A |
| DC_7A-32A_n78A | DC_7A_n78A |
| DC_7A-40A_n1A DC_7A-40C_n1A | DC_7A_n1A DC_40A_n1A |
| DC_7A_n40A-n77A | DC_7A_n40A DC_7A_n77A |
| DC_7A_n40A-n77(2A) | DC_7A_n40A DC_7A_n77A |
| DC_7A-40A_n78A DC_7A-40C_n78A | DC_7A_n78A DC_40A_n78A |
| DC_7A-40A_n78(2A) DC_7A-40C_n78(2A) | DC_7A_n78A DC_40A_n78A |
| DC_7A_n40A-n78A DC_7A_n40A-n78C | DC_7A_n40A DC_7A_n78A |
| DC_7A-46A_n78A3 DC_7A-46C_n78A3 DC_7A-46D_n78A3 DC_7A-46E_n78A3 | DC_7A_n78A |
| DC_7A-66A_n2A | DC_7A_n2A DC_66A_n2A |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | DC_7A_n5A DC_66A_n5A |
| DC_7A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |
| DC_7A-66A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |
| DC_7A-66A_n12A | DC_7A_n12A DC_66A_n12A |
| DC_7A-66A_n25A DC_7C-66A_n25A | DC_7A_n25A DC_66A_n25A |
| DC_7A-7A-66A_n25A | DC_7A_n25A DC_66A_n25A |
| DC_7A-66A_n28A | DC_7A_n28A DC_66A_n28A |
| DC_7A-66A_n66A DC_7C-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-7A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-7A-66A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A-66A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A_n66A-n71A | DC_7A_n66A DC_7A_n71A |
| DC_7A-66A_n77A DC_7C-66A_n77A | DC_7A_n77A DC_66A_n77A |
| DC_7A-7A-66A_n77A | DC_7A_n66A DC_66A_n77A |
| DC_7A-7A-66A_n77(2A) | DC_7A_n66A DC_66A_n77A |
| DC_7A-66A_n77(2A) DC_7C-66A_n77(2A) | DC_7A_n66A DC_66A_n77A |
| DC_7A_n66A-n77A DC_7C_n66A-n77A | DC_7A_n66A DC_7A_n77A |
| DC_7A-7A_n66A-n77A | DC_7A_n66A DC_7A_n77A |
| DC_7A_n66A-n78A DC_7C_n66A-n78A | DC_7A_n66A DC_7A_n78A |
| DC_7A-7A_n66A-n78A | DC_7A_n66A DC_7A_n78A |
| DC_7A-66A_n78A DC_7C-66A_n78A | DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) | DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_7A-7A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-66A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-66A-66A_n78A DC_7C-66A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-71A_n2A | DC_7A_n2A DC_71A_n2A |
| DC_7A-71A_n2(2A) | DC_7A_n2A DC_71A_n2A |
| DC_7A-71A_n25A | DC_7A_n25A DC_71A_n25A |
| DC_7A-71A_n66A | DC_7A_n66A DC_71A_n66A |
| DC_7A-71A_n77A | DC_7A_n77A DC_71A_n77A |
| DC_7A-71A_n77(2A) | DC_7A_n77A DC_71A_n77A |
| DC_7A_n71A-n77A | DC_7A_n71A DC_7A_n77A |
| DC_7A-71A_n78A | DC_7A_n78A DC_71A_n78A |
| DC_7A-71A_n78(2A) | DC_7A_n78A DC_71A_n78A |
| DC_7A_n71A-n78A | DC_7A_n71A DC_7A_n78A |
| DC_7A_n78A-n79A DC_7A_n78A-n79C | DC_7A_n78A DC_7A_n79A |
| DC_7A_SUL_n78A-n80A | DC_7A_n78A DC_7A_n80A |
| DC_7A_n78A-n105A | DC_7A_n78A DC_7A_n105A |
| DC_8A_n1A-n3A | DC_8A_n1A DC_8A_n3A |
| DC_8A_n1A-n28A | DC_8A_n1A DC_8A_n28A |
| DC_8A_n1A-n40A | DC_8A_n1A DC_8A_n40A |
| DC_8A_n1A-n77A5 | DC_8A_n1A DC_8A_n77A |
| DC_8A_n1A-n77(2A)5 | DC_8A_n1A DC_8A_n77A |
| DC_8A_n1A-n78A5 | DC_8A_n1A DC_8A_n78A |
| DC_8A-(n)3AA | DC_(n)3AA DC_8A_n3A |
| DC_8A_n3A-n28A | DC_8A_n3A DC_8A_n28A |
| DC_8A_n3A-n77A5 | DC_8A_n3A DC_8A_n77A |
| DC_8A_n3A-n77(2A) 5 | DC_8A_n3A DC_8A_n77A |
| DC_8A_n3A-n78A | DC_8A_n3A DC_8A_n78A |
| DC_8A_n3A-n79A | DC_8A_n3A DC_8A_n79A |
| DC_8A-11A_n1A | DC_8A_n1A DC_11A_n1A |
| DC_8A-11A_n3A | DC_8A_n3A DC_11A_n3A |
| DC_8A-11A_n28A | DC_8A_n28A DC_11A_n28A |
| DC_8A-11A_n77A5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n77(2A)5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n77(3A)5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n78A5 | DC_8A_n78A DC_11A_n78A |
| DC_8A-11A_n79A5 | DC_8A_n79A DC_11A_n79A |
| DC_8A-20A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_8A-20A_n3A | DC_8A_n3A DC_20A_n3A |
| DC_8A-20A_n28A6,16,19,20 | DC_8A_n28A DC_20A_n28A |
| DC_8A-20A_n78A | DC_8A_n78A DC_20A_n78A |
| DC_8A-28A_n3A | DC_8A_n3A DC_28A_n3A |
| DC_8A-28A_n78A | DC_8A_n78A DC_28A_n78A |
| DC_8A_n28A-n77A5 | DC_8A_n28A DC_8A_n77A |
| DC_8A_n28A-n77(2A)5 | DC_8A_n28A DC_8A_n77A |
| DC_8A_n28A-n78A5 | DC_8A_n28A DC_8A_n78A |
| DC_8A_n28A-n79A5 | DC_8A_n28A DC_8A_n79A |
| DC_8A-32A_n1A | DC_8A_n1A |
| DC_8A-32A_n3A | DC_8A_n3A |
| DC_8A-32A_n28A | DC_8A_n28A |
| DC_8A-32A_n78A | DC_8A_n78A |
| DC_8A-38A_n1A | DC_8A_n1A DC_38A_n1A |
| DC_8A_n38A-n40A | DC_8A_n38A DC_8A_n40A |
| DC_8A_n39A-n40A | DC_8A_n39A DC_8A_n40A |
| DC_8A_n39A-n41A | DC_8A_n39A DC_8A_n41A |
| DC_8A_n39A-n79A | DC_8A_n39A DC_8A_n79A |
| DC_8A-40A_n1A DC_8A-40C_n1A | DC_8A_n1A DC_40A_n1A |
| DC_8A_n40A-n41A | DC_8A_n40A DC_8A_n41A |
| DC_8A-40A_n78A DC_8A-40C_n78A | DC_8A_n78A DC_40A_n78A |
| DC_8A-40A_n78(2A) DC_8A-40C_n78(2A) | DC_8A_n78A DC_40A_n78A |
| DC_8A_n40A-n78A | DC_8A_n40A DC_8A_n78A |
| DC_8A_n40A-n79A DC_8A_n40A-n79C | DC_8A_n40A DC_8A_n79A |
| DC_8A-41A_n1A DC_8A-41C_n1A | DC_8A_n1A DC_41A_n1A |
| DC_8A-41A_n3A5 DC_8A-41C_n3A5 | DC_8A_n3A DC_41A_n3A DC_41C_n3A |
| DC_8A-41A_n77A DC_8A-41C_n77A | DC_8A_n77A DC_41A_n77A DC_41C_n77A |
| DC_8A-41A_n78A | DC_8A_n78A DC_41A_n78A |
| DC_8A-41C_n78A | DC_8A_n78A DC_41A_n78A DC_41C_n78A |
| DC_8A_n41A-n79A5 | DC_8A_n41A DC_8A_n79A |
| DC_8A-42A_n1A5 DC_8A-42C_n1A5 | DC_8A_n1A DC_42A_n1A DC_42C_n1A |
| DC_8A-42A_n3A5 | DC_8A_n3A DC_42A_n3A |
| DC_8A-42C_n3A5 | DC_8A_n3A DC_42A_n3A DC_42C_n3A |
| DC_8A-42A_n28A5 | DC_8A_n28A DC_42A_n28A |
| DC_8A-42C_n28A5 | DC_8A_n28A DC_42A_n28A DC_42C_n28A |
| DC_8A-42A_n77A15,16 DC_8A-42C_n77A15,16 | DC_8A_n77A |
| DC_8A-42A_n77(2A) 15,16 DC_8A-42C_n77(2A) 15,16 | DC_8A_n77A |
| DC_8A_SUL_n41A-n81A | DC_8A_n41A, DC_8A_n81A_ULSUP-TDM_n41A |
| DC_8A_n77A-n79A | DC_8A_n77A DC_8A_n79A |
| DC_8A_n77(2A)-n79A | DC_8A_n77A DC_8A_n79A |
| DC_8A_SUL_n78A-n80A | DC_8A_n78A DC_8A_n80A |
| DC_8A_SUL_n78A-n81A5 | DC_8A_n78A, DC_8A_n81A_ULSUP-TDM_n78A |
| DC_8A_SUL_n79A-n81A5 | DC_8A_n79A, DC_8A_n81A_ULSUP-TDM_n79A |
| DC_11A_n1A-n77A5 | DC_11A_n1A DC_11A_n77A |
| DC_11A_n1A-n77(2A)5 | DC_11A_n1A DC_11A_n77A |
| DC_11A_n3A-n28A | DC_11A_n3A DC_11A_n28A |
| DC_11A_n3A-n77A | DC_11A_n3A DC_11A_n77A |
| DC_11A_n3A-n77(2A) | DC_11A_n3A DC_11A_n77A |
| DC_11A_n3A-n79A5 | DC_11A_n3A DC_11A_n79A |
| DC_11A-18A_n3A | DC_11A_n3A DC_18A_n3A |
| DC_11A-18A_n28A | DC_11A_n28A |
| DC_11A-18A_n41A | DC_11A_n41A DC_18A_n41A |
| DC_11A-18A_n77A | DC_11A_n77A DC_18A_n77A |
| DC_11A-18A_n77(2A) | DC_11A_n77A DC_18A_n77A |
| DC_11A-18A_n78A | DC_11A_n78A DC_18A_n78A |
| DC_11A-18A_n78(2A) | DC_11A_n78A DC_18A_n78A |
| DC_11A_n28A-n77A5 | DC_11A_n28A DC_11A_n77A |
| DC_11A_n28A-n77(2A) 5 | DC_11A_n28A DC_11A_n77A |
| DC_11A_n77A-n79A | DC_11A_n77A DC_11A_n79A |
| DC_11A_n77(2A)-n79A | DC_11A_n77A DC_11A_n79A |
| DC_12A_n2A-n38A | DC_12A_n2A DC_12A_n38A |
| DC_12A_n2A-n41A | DC_12A_n2A DC_12A_n41A |
| DC_12A_n2A-n78A | DC_12A_n2A DC_12A_n78A |
| DC_12A-(n)5AA | DC_12A_n5A DC_(n)5AA2 |
| DC_12A_n7A-n66A | DC_12A_n7A DC_12A_n66A |
| DC_12A_n7(2A)-n66A | DC_12A_n7A DC_12A_n66A |
| DC_12A_n7A-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7(2A)-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7A-n78(2A) | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7(2A)-n78(2A) | DC_12A_n7A DC_12A_n78A |
| DC_12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_12A-30A_n5A | DC_12A_n5A DC_30A_n5A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |
| DC_12A-30A_n77A14 | DC_12A_n77A14 DC_30A_n77A14 |
| DC_12A-30A_n77(2A) 14 | DC_12A_n77A14 DC_30A_n77A14 |
| DC_12A-48A_n5A | DC_12A_n5A DC_48A_n5A |
| DC_12A-48A_n12A | DC_48A_n12A |
| DC_12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A_n2(2A) | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A_n5A | DC_12A_n5A DC_66A_n5A |
| DC_12A-66A_n7A | DC_12A_n7A DC_66A_n7A |
| DC_12A-66A-66A_n5A | DC_12A_n5A DC_66A_n5A |
| DC_12A-66A_n12A | DC_66A_n12A |
| DC_12A-66A_n25A | DC_12A_n25A DC_66A_n25A |
| DC_12A-66A_n30A | DC_12A_n30A DC_66A_n30A |
| DC_12A-66A-66A_n30A | DC_12A_n30A DC_66A_n30A |
| DC_12A-66A_n41A | DC_12A_n41A DC_66A_n41A |
| DC_12A-66A_n66A | DC_12A_n66A DC_66A_n66A2 |
| DC_12A-66A_n77A14 DC_12A-66A-66A_n77A14 | DC_12A_n77A14 DC_66A_n77A14 |
| DC_12A-66A_n77(2A) 14 DC_12A-66A-66A_n77(2A) 14 | DC_12A_n77A14 DC_66A_n77A14 |
| DC_12A-66A_n78A | DC_12A_n78A DC_66A_n78A |
| DC_12A-66A_n78(2A) | DC_12A_n78A DC_66A_n78A |
| DC_12A_n66A-n78A | DC_12A_n66A DC_12A_n78A |
| DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | DC_12A_n66A DC_12A_n78A |
| DC_13A_n2A-n77A14 DC_13A_n2A-n77C14 | DC_13A_n2A DC_13A_n77A14 |
| DC_13A_n5A-n48A | DC_13A_n48A |
| DC_13A_n5A-n77A14 DC_13A_n5A-n77C14 | DC_13A_n77A14 |
| DC_13A_n7A-n78A | DC_13A_n7A DC_13A_n78A |
| DC_13A_n25A-n66A | DC_13A_n25A DC_13A_n66A |
| DC_13A-46A_n2A3 | DC_13A_n2A |
| DC_13A-46A_n5A | DC_13A_n5A |
| DC_13A-46A_n66A3 | DC_13A_n66A |
| DC_13A-46A_n77A DC_13A-46A-46A_n77A | DC_13A_n77A |
| DC_13A_n48A-n66A | DC_13A_n48A DC_13A_n66A |
| DC_13A-66A_n2A DC_13A-66B_n2A DC_13A-66C_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A_n5A DC_13A-66A-66A_n5A | DC_13A_n5A DC_66A_n5A |
| DC_13A-66A_n48A DC_13A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A_n66A DC_13A-66B_n66A | DC_13A_n66A |
| DC_13A-66A-66A_n66A | DC_13A_n66A |
| DC_13A-66A_n77A14 DC_13A-66A_n77C14 DC_13A-66A-66A_n77C14 | DC_13A_n77A14 DC_66A_n77A14 |
| DC_13A-66A-66A_n77A | DC_13A_n77A14 DC_66A_n77A14 |
| DC_13A_n66A-n77A14 DC_13A_n66A-n77C14 | DC_13A_n66A DC_13A_n77A14 |
| DC_13A-48A_n2A DC_13A-48B_n2A DC_13A-48C_n2A DC_13A-48D_n2A DC_13A-48E_n2A | DC_13A_n2A |
| DC_13A-48A_n66A DC_13A-48B_n66A DC_13A-48C_n66A DC_13A-48D_n66A DC_13A-48E_n66A | DC_13A_n66A |
| DC_13A-48A_n77A14,15,16 DC_13A-48A_n77C14,15,16 DC_13A-48C_n77A14,15,16 DC_13A-48C_n77C14,15,16 DC_13A-48D_n77A14,15,16 DC_13A-48D_n77C14,15,16 DC_13A-48A-48A_n77A14,15,16 | DC_13A_n77A14 |
| DC_14A-30A_n2A | DC_14A_n2A DC_30A_n2A |
| DC_14A-30A_n5A | DC_14A_n5A DC_30A_n5A |
| DC_14A-30A_n66A | DC_14A_n66A DC_30A_n66A |
| DC_14A-30A_n77A14 | DC_14A_n77A14 DC_30A_n77A14 |
| DC_14A-30A_n77(2A) 14 | DC_14A_n77A14 DC_30A_n77A14 |
| DC_14A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A_n5A | DC_14A_n5A DC_66A_n5A |
| DC_14A-66A-66A_n5A | DC_14A_n5A DC_66A_n5A |
| DC_14A-66A_n30A DC_14A-66A-66A_n30A | DC_14A_n30A DC_66A_n30A |
| DC_14A-66A_n66A | DC_14A_n66A DC_66A_n66A2 |
| DC_14A-66A_n77A14 DC_14A-66A-66A_n77A14 | DC_14A_n77A14 DC_66A_n77A14 |
| DC_14A-66A_n77(2A) 14 DC_14A-66A-66A_n77(2A) 14 | DC_14A_n77A14 DC_66A_n77A14 |
| DC_18A_n3A-n41A | DC_18A_n3A DC_18A_n41A |
| DC_18A_n3A-n77A | DC_18A_n3A DC_18A_n77A |
| DC_18A_n3A-n78A | DC_18A_n3A DC_18A_n78A |
| DC_18A_n28A-n41A | DC_18A_n28A DC_18A_n41A |
| DC_18A-28A_n77A5 | DC_18A_n77A DC_28A_n77A |
| DC_18A_n28A-n77A5 | DC_18A_n28A DC_18A_n77A |
| DC_18A_n28A-n77(2A)5 | DC_18A_n28A DC_18A_n77A |
| DC_18A-28A_n78A5 | DC_18A_n78A DC_28A_n78A |
| DC_18A_n28A-n78A5 | DC_18A_n28A DC_18A_n78A |
| DC_18A_n28A-n78(2A)5 | DC_18A_n28A DC_18A_n78A |
| DC_18A-28A_n79A5 | DC_18A_n79A DC_28A_n79A |
| DC_18A-41A_n3A DC_18A-41C_n3A | DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_18A-41A_n77A DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_18A-41A_n78A DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_18A_n41A-n77A | DC_18A_n41A DC_18A_n77A |
| DC_18A_n41A-n77(2A) | DC_18A_n41A DC_18A_n77A |
| DC_18A-42A_n77A15,16 DC_18A-42C_n77A15,16 | DC_18A_n77A |
| DC_18A_n41A-n78A | DC_18A_n41A DC_18A_n78A |
| DC_18A_n41A-n78(2A) | DC_18A_n41A DC_18A_n78A |
| DC_18A-42A_n78A15,16 DC_18A-42C_n78A15,16 | DC_18A_n78A |
| DC_18A-42A_n79A DC_18A-42C_n79A | DC_18A_n79A |
| DC_19A-21A_n1A | DC_19A_n1A DC_21A_n1A |
| DC_19A_n1A-n77A5 | DC_19A_n1A DC_19A_n77A |
| DC_19A_n1A-n78A5 | DC_19A_n1A DC_19A_n78A |
| DC_19A_n1A-n79A5 | DC_19A_n1A DC_19A_n79A |
| DC_19A-21A_n77A5,14 DC_19A-21A_n77C5 | DC_19A_n77A14 DC_21A_n77A14 |
| DC_19A-21A_n77(2A)5,14 | DC_19A_n77A14 DC_21A_n77A14 |
| DC_19A-21A_n78A514 DC_19A-21A_n78C5 | DC_19A_n78A14 DC_21A_n78A14 |
| DC_19A-21A_n78(2A)514 | DC_19A_n78A14 DC_21A_n78A14 |
| DC_19A-21A_n79A5,14 DC_19A-21A_n79C5 | DC_19A_n79A14 DC_21A_n79A14 |
| DC_19A-42A_n1A5,10,12 DC_19A-42C_n1A5,10,12 | DC_19A_n1A DC_42A_n1A |
| DC_19A-42A_n77A14,15,16 DC_19A-42A_n77C15,16 DC_19A-42C_n77A14,15,16 DC_19A-42C_n77C15,16 DC_19A-42D_n77A15,16 DC_19A-42D_n77C15,16 | DC_19A_n77A14 |
| DC_19A-42A_n78A14,15,16 DC_19A-42A_n78C15,16 DC_19A-42C_n78A14,15,16 DC_19A-42C_n78C15,16 DC_19A-42D_n78A15,16 DC_19A-42D_n78C15,16 | DC_19A_n78A14 |
| DC_19A-42A_n79A14 DC_19A-42A_n79C DC_19A-42C_n79A14 DC_19A-42C_n79C DC_19A-42D_n79A DC_19A-42D_n79C | DC_19A_n79A14 |
| DC_19A_n77A-n79A14 | DC_19A_n77A14 DC_19A_n79A14 |
| DC_19A_n78A-n79A14 | DC_19A_n78A14 DC_19A_n79A14 |
| DC_20A_n1A-n7A | DC_20A_n1A DC_20A_n7A |
| DC_20A_n1A-n28A16,20 | DC_20A_n1A DC_20A_n28A |
| DC_20A_n1A-n67A | DC_20A_n1A |
| DC_20A_n1A-n75A | DC_20A_n1A |
| DC_20A_n1A-n78A | DC_20A_n1A DC_20A_n78A |
| DC_20A-(n)3AA | DC_(n)3AA2 DC_20A_n3A |
| DC_20A_n3A-n38A | DC_20A_n3A DC_20A_n38A |
| DC_20A_n3A-n67A | DC_20A_n3A |
| DC_20A_n3A-n78A | DC_20A_n3A DC_20A_n78A |
| DC_20A_n7A-n28A, 16, 20 | DC_20A_n7A DC_20A_n28A |
| DC_20A_n7A-n78A | DC_20A_n7A DC_20A_n78A |
| DC_20A_n8A-n75A6 | DC_20A_n8A |
| DC_20A_n8A-n78A | DC_20A_n78A DC_20A_n8A |
| DC_20A-28A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_20A-28A_n3A | DC_20A_n3A DC_28A_n3A |
| DC_20A-28A_n78A | DC_20A_n78A DC_28A_n78A |
| DC_20A_n28A-n75A6,16,20 | DC_20A_n28A |
| DC_20A_n28A-n78A5,6,16,20 | DC_20A_n28A DC_20A_n78A |
| DC_20A-32A_n1A | DC_20A_n1A |
| DC_20A-32A_n3A | DC_20A_n3A |
| DC_20A-32A_n8A | DC_20A_n8A |
| DC_20A-32A_n28A16,20 | DC_20A_n28A |
| DC_20A-32A_n7A | DC_20A_n7A |
| DC_20A-32A_n78A DC_20A-32A_n78C | DC_20A_n78A |
| DC_20A-32A_n78(2A) | DC_20A_n78A |
| DC_20A-38A_n1A | DC_20A_n1A DC_38A_n1A |
| DC_20A-38A_n3A | DC_20A_n3A |
| DC_20A-38A_n8A | DC_38A_n8A |
| DC_20A-(n)38AA | DC_20A_n38A |
| DC_20A-38A_n78A | DC_20A_n78A DC_38A_n78A |
| DC_20A-38A_n78(2A) | DC_20A_n78A |
| DC_20A_n38A-n78A | DC_20A_n38A DC_20A_n78A |
| DC_20A-40A_n1A DC_20A-40C_n1A | DC_20A_n1A DC_40A_n1A |
| DC_20A-40A_n78A DC_20A-40C_n78A | DC_20A_n78A DC_40A_n78A |
| DC_20A-40A_n78(2A) DC_20A-40C_n78(2A) | DC_20A_n78A DC_40A_n78A |
| DC_20A-41A_n1A | DC_20A_n1A DC_41A_n1A |
| DC_20A-41C_n1A | DC_20A_n1A DC_41A_n1A DC_41C_n1A |
| DC_20A-41A_n41A DC_20A-41C_n41A | DC_41A_n41A |
| DC_20A-41A_n78A | DC_20A_n78A DC_41A_n78A |
| DC_20A-41C_n78A | DC_20A_n78A DC_41A_n78A DC_41C_n78A |
| DC_20A_n41A-n78A | DC_20A_n41A DC_20A_n78A |
| DC_20A-(n)41AA DC_20A-(n)41CA DC_20A-(n)41DA | DC_20A_n41A |
| DC_20A-67A_n3A | DC_20A_n3A |
| DC_20A_n75A-n78A5 | DC_20A_n78A |
| DC_20A_n76A-n78A5 | DC_20A_n78A |
| DC_20A_SUL_n78A-n80A | DC_20A_n78A DC_20A_n80A |
| DC_20A_SUL_n78A-n82A5 | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A |
| DC_20A_SUL_n78A-n83A5 | DC_20A_n78A DC_20A_n83A |
| DC_20A_n78A-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |
| DC_20A_n78(2A)-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |
| DC_21A_n1A-n77A5 | DC_21A_n1A DC_21A_n77A |
| DC_21A_n1A-n78A5 | DC_21A_n1A DC_21A_n78A |
| DC_21A_n1A-n79A5 | DC_21A_n1A DC_21A_n79A |
| DC_21A-28A_n77A5 DC_21A-28A_n77C | DC_21A_n77A DC_28A_n77A |
| DC_21A_n28A-n77A5,13 | DC_21A_n28A DC_21A_n77A |
| DC_21A-28A_n78A5 DC_21A-28A_n78C | DC_21A_n78A DC_28A_n78A |
| DC_21A_n28A-n78A5,13 | DC_21A_n28A DC_21A_n78A |
| DC_21A-28A_n79A5 DC_21A-28A_n79C | DC_21A_n79A DC_28A_n79A |
| DC_21A_n28A-n79A5,13 | DC_21A_n28A DC_21A_n79A |
| DC_21A-42A_n1A510,12 DC_21A-42C_n1A510,12 | DC_21A_n1A DC_42A_n1A |
| DC_21A-42A_n77A14, 15,16 DC_21A-42A_n77C15,16 DC_21A-42C_n77A14, 15,16 DC_21A-42C_n77C15,16 DC_21A-42D_n77A15,16 DC_21A-42D_n77C15,16 DC_21A-42E_n77A15,16 DC_21A-42E_n77C15,16 | DC_21A_n77A14, |
| DC_21A-42A_n78A14,15,16 DC_21A-42A_n78C15,16 DC_21A-42C_n78A14,15,16 DC_21A-42C_n78C15,16 DC_21A-42D_n78A14,15,16 DC_21A-42D_n78C15,16 DC_21A-42E_n78A14,15,16 DC_21A-42E_n78C15,16 | DC_21A_n78A14 |
| DC_21A-42A_n79A14 DC_21A-42A_n79C DC_21A-42C_n79A14 DC_21A-42C_n79C DC_21A-42D_n79A DC_21A-42D_n79C DC_21A-42E_n79A DC_21A-42E_n79C | DC_21A_n79A14 |
| DC_28A-(n)7AA | DC_28A_n7A |
| DC_28A-32A_n1A | DC_28A_n1A |
| DC_28A-32A_n3A | DC_28A_n3A |
| DC_28A-38A_n1A | DC_28A_n1A DC_38A_n1A |
| DC_28A-38A_n78A | DC_28A_n78A DC_38A_n78A |
| DC_28A-66A_n7A | DC_28A_n7A DC_66A_n7A |
| DC_28A-66A_n66A | DC_28A_n66A DC_66A_n66A2 |
| DC_21A_n77A-n79A14, 23 | DC_21A_n77A14 DC_21A_n79A14 |
| DC_21A_n78A-n79A14, 23 | DC_21A_n78A14 DC_21A_n79A14 |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-(n)41CA DC_25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |
| DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |
| DC_25A-66A_n77A | DC_25A_n77A DC_66A_n77A |
| DC_25A-25A-66A_n77A | DC_25A_n77A DC_66A_n77A |
| DC_25A-66A_n78A | DC_25A_n78A DC_66A_n78A |
| DC_25A-25A-66A_n78A | DC_25A_n78A DC_66A_n78A |
| DC_28A_n5A-n40A | DC_28A_n5A DC_28A_n40A |
| DC_28A-40A_n78A DC_28A-40C_n78A | DC_28A_n78A DC_40A_n78A |
| DC_28A-41A_n77A DC_28A-41C_n77A | DC_28A_n77A DC_41A_n77A |
| DC_28A-41A_n78A DC_28A-41C_n78A | DC_28A_n78A DC_41A_n78A |
| DC_28A-41A_n79A5 DC_28A-41C_n79A5 | DC_28A_n79A DC_41A_n79A |
| DC_28A_n1A-n40A | DC_28A_n1A DC_28A_n40A |
| DC_28A_n1A-n78A5 | DC_28A_n1A DC_28A_n78A |
| DC_28A_n3A-n77A5 | DC_28A_n3A DC_28A_n77A |
| DC_28A_n3A-n78A5 | DC_28A_n3A DC_28A_n78A |
| DC_28A_n5A-n78A5 | DC_28A_n5A DC_28A_n78A |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A |
| DC_28A_n7B-n78A | DC_28A_n7A DC_28A_n7B DC_28A_n78A |
| DC_28A_n8A-n78A5 | DC_28A_n8A DC_28A_n78A |
| DC_28A_n40A-n78A | DC_28A_n40A DC_28A_n78A |
| DC_28A_SUL_n41A-n83A5 | DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41A |
| DC_28A-42A_n77A15,16 DC_28A-42A_n77C15,16 DC_28A-42C_n77A15,16 DC_28A-42C_n77C15,16 | DC_28A_n77A |
| DC_28A-42A_n78A15,16 DC_28A-42A_n78C15,16 DC_28A-42C_n78A15,16 DC_28A-42C_n78C15,16 | DC_28A_n78A |
| DC_28A-42A_n79A DC_28A-42A_n79C DC_28A-42C_n79A DC_28A-42C_n79C | DC_28A_n79A |
| DC_28A_SUL_n78A-n83A5 | DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A |
| DC_29A-30A_n2A | DC_30A_n2A |
| DC_29A-30A_n66A | DC_30A_n66A |
| DC_29A-30A_n77A14 | DC_30A_n77A14 |
| DC_29A-66A_n2A | DC_66A_n2A |
| DC_29A-66A-66A_n2A | DC_66A_n2A |
| DC_29A-66A_n30A | DC_66A_n30A |
| DC_29A-66A-66A_n30A | DC_66A_n30A |
| DC_29A-66A_n77A14 DC_29A-66A-66A_n77A14 | DC_66A_n77A14 |
| DC_29A-66A_n78A | DC_66A_n78A |
| DC_30A-(n)5AA | DC_30A_n5A DC_(n)5AA2 |
| DC_30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A_n66A | DC_30A_n66A DC_66A_n66A2 |
| DC_30A-66A_n77A14 DC_30A-66A-66A_n77A14 | DC_30A_n77A14 DC_66A_n77A14 |
| DC_30A-66A_n77(2A) 14 DC_30A-66A-66A_n77(2A) 14 | DC_30A_n77A14 DC_66A_n77A14 |
| DC_32A-38A_n1A | DC_38A_n1A |
| DC_32A-38A_n28A | DC_38A_n28A |
| DC_38A_n3A-n78A | DC_38A_n3A DC_38A_n78A |
| DC_38A_n28A-n78A | DC_38A_n28A DC_38A_n78A |
| DC_39A_n40A-n41A DC_39A_n40A-n41C | DC_39A_n40A DC_39A_n41A |
| DC_39A_n40A-n79A DC_39A_n40A-n79C | DC_39A_n40A DC_39A_n79A |
| DC_39A_n41A-n79A | DC_39A_n41A DC_39A_n79A |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | DC_40A_n1A DC_40A_n78A |
| DC_40A_n41A-n79A | DC_40A_n41A DC_40A_n79A |
| DC_40A-42A_n77A DC_40A-42A_n77C | DC_40A_n77A |
| DC_40A-42A_n78A | DC_40A_n78A |
| DC_41A_n1A-n3A | DC_41A_n1A DC_41A_n3A |
| DC_41C_n1A-n3A | DC_41A_n1A DC_41A_n3A |
| DC_41A_n1A-n77A | DC_41A_n1A DC_41A_n77A |
| DC_41C_n1A-n77A | DC_41A_n1A DC_41A_n77A DC_41C_n77A |
| DC_41A_n1A-n78A | DC_41A_n1A DC_41A_n78A |
| DC_41C_n1A-n78A | DC_41A_n1A DC_41A_n78A |
| DC_41A_n3A-n41A | DC_41A_n3A DC_41A_n41A |
| DC_41A_n3A-n77A | DC_41A_n3A DC_41A_n77A |
| DC_41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_41A_n3A-n78A | DC_41A_n3A DC_41A_n78A |
| DC_41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_41A_n28A-n41A | DC_41A_n28A |
| DC_41A_n28A-n77A | DC_41A_n28A DC_41A_n77A |
| DC_41C_n28A-n77A | DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_41A_n28A-n78A | DC_41A_n28A DC_41A_n78A |
| DC_41C_n28A-n78A | DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_(n)41AA-n78A DC_(n)41CA-n78A DC_(n)41DA-n78A | DC_41A_n78A |
| DC_41A_n41A-n77A | DC_41A_n77A |
| DC_41A_n41A-n78A DC_41C_n41A-n78A | DC_41A_n78A |
| DC_41A-42A_n77A15,16 DC_41A-42C_n77A15,16 DC_41C-42A_n77A15,16 DC_41C-42C_n77A15,16 | DC_41A_n77A |
| DC_41A-42A_n77(2A)15,16 DC_41A-42C_n77(2A)15,16 | DC_41A_n77A |
| DC_41A-42A_n78A15,16 DC_41A-42C_n78A15,16 DC_41C-42A_n78A15,16 DC_41C-42C_n78A15,16 | DC_41A_n78A |
| DC_41A-42A_n79A DC_41A-42C_n79A DC_41C-42A_n79A DC_41C-42C_n79A | DC_41A_n79A |
| DC_42A_n1A-n3A5 | DC_42A_n1A DC_42A_n3A |
| DC_42C_n1A-n3A5, | DC_42A_n1A DC_42A_n3A DC_42C_n1A DC_42C_n3A |
| DC_42A_n1A-n77A15,16 | DC_42A_n1A |
| DC_42C_n1A-n77A15,16 | DC_42A_n1A DC_42C_n1A |
| DC_42A_n1A-n78A15,16 DC_42C_n1A-n78A15,16 | N/A |
| DC_42A_n1A-n79A DC_42C_n1A-n79A | N/A |
| DC_42A_n3A-n28A | DC_42A_n3A DC_42A_n28A |
| DC_42C_n3A-n28A | DC_42A_n3A DC_42A_n28A DC_42C_n28A |
| DC_42A_n3A-n77A15,16 | DC_42A_n3A |
| DC_42A_n3A-n77(2A)15,16 | DC_42A_n3A |
| DC_42C_n3A-n77A15,16 | DC_42A_n3A DC_42C_n3A |
| DC_42C_n3A-n77(2A)15,16 | DC_42A_n3A DC_42C_n3A |
| DC_42A_n28A-n77A15,16 | DC_42A_n28A |
| DC_42A_n28A-n77(2A)15,16 | DC_42A_n28A |
| DC_42C_n28A-n77A15,16 | DC_42A_n28A DC_42C_n28A |
| DC_42C_n28A-n77(2A)15,16 | DC_42A_n28A DC_42C_n28A |
| DC_46A-48A_n2A3 DC_46C-48A_n2A3 DC_46D-48A_n2A3 DC_46E-48A_n2A3 | DC_48A_n2A |
| DC_46A-48A_n5A3 DC_46C-48A_n5A3 DC_46D-48A_n5A3 DC_46E-48A_n5A3 | DC_48A_n5A |
| DC_46A-48A_n66A3 DC_46C-48A_n66A3 DC_46D-48A_n66A3 DC_46E-48A_n66A3 | DC_48A_n66A |
| DC_46A-66A_n5A DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A DC_46A-66A-66A_n5A DC_46C-66A-66A_n5A DC_46D-66A-66A_n5A | DC_66A_n5A |
| DC_46A-66A_n25A DC_46C-66A_n25A DC_46D-66A_n25A | DC_66A_n25A |
| DC_46A-66A_n41A DC_46C-66A_n41A DC_46D-66A_n41A | DC_66A_n41A |
| DC_46A-66A_n41(2A) DC_46C-66A_n41(2A) DC_46D-66A_n41(2A) | DC_66A_n41A |
| DC_46A-66A_n71A DC_46C-66A_n71A DC_46D-66A_n71A | DC_66A_n71A |
| DC_46A-66A_n77A DC_46A-46A-66A_n77A | DC_66A_n77A |
| DC_48A-(n)5AA | DC_48A_n5A DC_(n)5AA2 |
| DC_48A-(n)12AA | DC_48A_n12A DC_(n)12AA2 |
| DC_48A_n25A-n48A | DC_48A_n25A |
| DC_48A_n48A-n66A | DC_48A_n66A |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | DC_66A_n2A DC_48A_n2A |
| DC_48A-66A_n5A DC_48B-66A_n5A DC_48C-66A_n5A DC_48D-66A_n5A DC_48E-66A_n5A | DC_66A_n5A |
| DC_48A-66A_n12A | DC_48A_n12A DC_66A_n12A |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | DC_48A_n25A DC_66A_n25A |
| DC_48A-66A_n48A | DC_66A_n48A |
| DC_48A-66A_n66A DC_48C-66A_n66A DC_48D-66A_n66A DC_48E-66A_n66A | DC_66A_n66A2 DC_48A_n66A |
| DC_48A-66A_n71A | DC_48A_n71A DC_66A_n71A |
| DC_48A-66A_n77A14,15,16 DC_48A-66A_n77C14,15,16 DC_48C-66A_n77A14,15,16 DC_48C-66A_n77C14,15,16 DC_48D-66A_n77A14,15,16 DC_48D-66A_n77C14,15,16 DC_48E-66A_n77A14,15,16 | DC_66A_n77A14 |
| DC_48A-48A-66A_n77A14,15,16 | DC_66A_n77A |
| DC_67A-(n)3AA | DC_(n)3AA2 |
| DC_66A-(n)5AA | DC_66A_n5A DC_(n)5AA2 |
| DC_66A-66A-(n)5AA | DC_66A_n5A DC_(n)5AA2 |
| DC_66A_n2A-n38A | DC_66A_n2A DC_66A_n38A |
| DC_66A_n2A-n66A | DC_66A_n2A |
| DC_66A_n2A-n71A | DC_66A_n2A DC_66A_n71A |
| DC_66A_n2A-n77A14 DC_66A_n2A-n77C14 DC_66A-66A_n2A-n77C14 | DC_66A_n2A DC_66A_n77A14 |
| DC_66A-66A_n2A-n77A14 | DC_66A_n2A DC_66A_n77A14 |
| DC_66A_n2A-n78A | DC_66A_n2A DC_66A_n78A |
| DC_66A_n5A-n48A | DC_66A_n5A DC_66A_n48A |
| DC_66A_n5A-n77A14 DC_66A_n5A-n77C14 DC_66A-66A_n5A-n77C14 | DC_66A_n5A DC_66A_n77A14 |
| DC_66A-66A_n5A-n77A14 | DC_66A_n5A DC_66A_n77A14 |
| DC_66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A_n12A-n77A | DC_66A_n77A DC_66A_n12A |
| DC_66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_66A_n38A-n66A | DC_66A_n38A DC_66A_n66A2 |
| DC_66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_66A_n66A-n77A14 DC_66A_n66A-n77C14 | DC_66A_n77A14 |
| DC_66A_n66A-n78A | DC_66A_n66A2 DC_66A_n78A |
| DC_66A-(n)12AA | DC_66A_n12A DC_(n)12AA2 |
| DC_66A-(n)71AA DC_66C-(n)71AA | DC_66A_n71A DC_(n)71AA |
| DC_66A_n25A-n41A DC_66A_n25A-n41C | DC_66A_n25A DC_66A_n41A |
| DC_66A_n25A-n41(2A) | DC_66A_n25A DC_66A_n41A |
| DC_66A_n25A-n48A | DC_66A_n25A DC_66A_n48A |
| DC_66A_n25A-n66A | DC_66A_n25A DC_66A_n66A2 |
| DC_66A_n38A-n71A | DC_66A_n38A DC_66A_n71A |
| DC_66A_n41A-n71A DC_66A_n41C-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A_n66A-n71A | DC_66A_n66A DC_66A_n71A |
| DC_66A-71A_n2A | DC_71A_n2A DC_66A_n2A |
| DC_66A-71A_n7A | DC_66A_n7A DC_71A_n7A |
| DC_66A-71A_n25A | DC_66A_n25A DC_71A_n25A |
| DC_66A-71A_n38A | DC_71A_n38A DC_66A_n38A |
| DC_66A-71A_n41A | DC_66A_n41A DC_71A_n41A |
| DC_66A-71A_n66A | DC_71A_n66A DC_66A_n66A2 |
| DC_66A-71A_n71A | DC_66A_n71A |
| DC_66A-71A_n77A | DC_66A_n77A DC_71A_n77A |
| DC_66A-71A_n77(2A) | DC_66A_n77A DC_71A_n77A |
| DC_66A_n71A-n77A | DC_66A_n71A DC_66A_n77A |
| DC_66A-71A_n78A | DC_71A_n78A DC_66A_n78A |
| DC_66A-71A_n78(2A) | DC_71A_n78A DC_66A_n78A |
| DC_66A_n71A-n78A | DC_66A_n71A DC_66A_n78A |
| DC_66A_SUL_n78A-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |
| DC_66A_SUL_n78(2A)-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |
| DC_71A_n2A-n41A | DC_71A_n2A DC_71A_n41A |
| DC_71A_n2A-n66A | DC_71A_n2A DC_71A_n66A |
| DC_71A_n2A-n77A | DC_71A_n77A DC_71A_n2A |
| DC_71A_n2A-n78A | DC_71A_n2A DC_71A_n78A |
| DC_71A_n38A-n66A | DC_71A_n38A DC_71A_n66A |
| DC_71A_n38A-n78A | DC_71A_n38A DC_71A_n78A |
| DC_71A_n66A-n77A | DC_71A_n66A DC_71A_n77A |
| DC_71A_n66A-n78A | DC_71A_n66A DC_71A_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: N/A NOTE 7: Void. NOTE 8: Void NOTE 9: Void NOTE 10: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE 11: The frequency range in band 3 is restricted for this band combination to 1765 - 1785 MHz for the UL and 1860-1880 MHz for the DL. NOTE 12: The frequency range in band 42 is restricted for this band combination to 3440 - 3520 MHz. NOTE 13: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 17: Void. NOTE 18: Void. NOTE 19: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 20: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 21: The downlink DC_2_n2 RESSENS requirements only apply when the band n2 downlink carrier is configured closer to the uplink operating band than the E-UTRA Band 2 downlink carrier. NOTE 22: The frequency range in band 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758 - 788 MHz for the DL. NOTE 23: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. NOTE 24: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. |  |

#### 5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A |
| DC_1A-3A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A |
| DC_1A-3A_n3A-n78A2 | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A |
| DC_1A-3A-5A_n40A | DC_1A_n40A DC_3A_n40A DC_5A_n40A |
| DC_1A-3A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_3A_n5A DC_3A_n40A |
| DC_1A-3A-5A_n77A DC_1A-3A-5A_n77(3A) DC_1A-3A-5A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A |
| DC_1A-3A-5A_n78A2 DC_1A-3A-5A_n78C2 DC_1A-3C-5A_n78A DC_1A-1A-3A-5A_n78A DC_1A-1A-3C-5A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A-5A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A-5A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A_n5A-n78A2 DC_1A-3C_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-5A_n79A2 | DC_1A_n79A DC_3A_n79A DC_5A_n79A |
| DC_1A-3A-7A_n1A | DC_1A_n1A DC_3A_n1A DC_7A_n1A |
| DC_1A-3A-7A_n3A DC_1A-3A-7C_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A |
| DC_1A-3A-7A_n5A DC_1A-3A-7C_n5A DC_1A-3C-7A_n5A DC_1A-3C-7C_n5A | DC_1A_n5A DC_3A_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-3A-7A_n7A DC_1A-3C-7A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 |
| DC_1A-1A-3A-7A_n7A DC_1A-1A-3C-7A_n7A DC_1A-3A-3A-7A_n7A DC_1A-1A-3A-3A-7A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 |
| DC_1A-3A-(n)7AA DC_1A-3C-(n)7AA | DC_1A_n7A DC_3A_n7A |
| DC_1A-3A-7A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A |
| DC_1A-3A-7A_n26A DC_1A-3A-7C_n26A DC_1A-3C-7A_n26A DC_1A-3C-7C_n26A | DC_1A_n26A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |
| DC_1A-3A-7A_n28A DC_1A-3A-7C_n28A DC_1A-3C-7A_n28A DC_1A-3C-7C_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-1A-3A-7A_n28A DC_1A-1A-3C-7A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A |
| DC_1A-3A-7A_n38A12,13 | CA_1A-3A |
| DC_1A-3A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A |
| DC_1A-3A-7A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A |
| DC_1A-3A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A_n77(2A) DC_1A-3A-7A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A-7A_n77(2A) DC_1A-3A-7A-7A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A_n78A2 DC_1A-3A-7C_n78A DC_1A-3C-7A_n78A2 DC_1A-3C-7C_n78A DC_1A-3A-7A_n78C2 | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n78(2A) DC_1A-3C-7A_n78(2A) DC_1A-3A-7C_n78(2A) DC_1A-3C-7C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-1A-3A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A_n7A-n78A DC_1A-3A_n7B-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A |
| DC_1A-3A_n7A-n78(2A) DC_1A-3C_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_1A-3C_n7A-n78A DC_1A-3C_n7B-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_1A-3A-7A-7A_n78A2 DC_1A-1A-3C-7A_n78A DC_1A-3A-7A-7A_n78C2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A-7A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A_n105A | DC_1A_n105A DC_3A_n105A DC_7A_n105A |
| DC_1A-3A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A |
| DC_1A-3A-8A_n77A2 DC_1A-3C-8A_n77A | DC_1A_n77A DC_3A_n77A DC_3C_n77A DC_8A_n77A |
| DC_1A-3A-8A_n77(2A)2 DC_1A-3C-8A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_3C_n77A DC_8A_n77A |
| DC_1A_n3A-n8A-n77A | DC_1A_n3A DC_1A_n8A DC_1A_n77A |
| DC_1A_n3A-n8A-n77(2A) | DC_1A_n3A DC_1A_n8A DC_1A_n77A |
| DC_1A-3A-8A_n77(3A)2 | DC_1A_n77A DC_3A_n77A DC_8A_n77A |
| DC_1A-3A-8A_n78A2 DC_1A-3C-8A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A-8A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A |
| DC_1A-3A-8A_n79A2 | DC_1A_n79A DC_3A_n79A DC_8A_n79A |
| DC_1A-3A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_11A_n28A |
| DC_1A-3A-11A_n77A2 | DC_1A_n77A DC_3A_n77A DC_11A_n77A |
| DC_1A-3A-11A_n77(2A) 2 DC_1A-3A-11A_n77(3A)2 | DC_1A_n77A DC_3A_n77A DC_11A_n77A |
| DC_1A-3A-18A_n3A | DC_1A_n3A DC_3A_n3A4 DC_18A_n3A |
| DC_1A-3A-18A_n28A | DC_1A_n28A DC_3A_n28A DC_18A_n28A |
| DC_1A-3A-18A_n41A | DC_1A_n41A DC_3A_n41A DC_18A_n41A |
| DC_1A-3A-18A_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-19A_n77A2 DC_1A-3A-19A_n77C2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A_n77(2A)2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A_n78A2 DC_1A-3A-19A_n78C2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A_n79A2 DC_1A-3A-19A_n79C2 | DC_1A_n79A DC_3A_n79A DC_19A_n79A |
| DC_1A-3A-20A_n1A | DC_1A_n1A DC_3A_n1A DC_20A_n1A |
| DC_1A-3A-20A_n3A | DC_1A_n3A DC_3A_n3A DC_20A_n3A |
| DC_1A-3A-20A_n7A | DC_1A_n7A DC_3A_n7A DC_20A_n7A |
| DC_1A-3A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_20A_n8A |
| DC_1A-3A-20A_n28A3,8,14 DC_1A-3C-20A_n28A3,8,14 | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_1A-3A-20A_n41A DC_1A-3C-20A_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A DC_20A_n41A |
| DC_1A-3A-20A_n78A2 DC_1A-3A-20A_n78C2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-21A_n77A2 DC_1A-3A-21A_n77C2 | DC_1A_n77A DC_3A_n77A DC_21A_n77A |
| DC_1A-3A-21A_n77(2A)2 | DC_1A_n77A DC_3A_n77A DC_21A_n77A |
| DC_1A-3A-21A_n78A2 DC_1A-3A-21A_n78C2 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A_n79A2 DC_1A-3A-21A_n79C2 | DC_1A_n79A DC_3A_n79A DC_21A_n79A |
| DC_1A-3A-26A_n78A DC_1A-3C-26A_n78A DC_1A-3A-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_26A_n78A |
| DC_1A-3A-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_26A_n78A |
| DC_1A-3C-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_26A_n78A |
| DC_1A-3A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A |
| DC_1A-3C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A |
| DC_1A-3A-28A_n5A DC_1A-3C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_28A_n5A |
| DC_1A-3A-28A_n7A DC_1A-3C-28A_n7A DC_1A-3A-28A_n7B DC_1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-3A-3A-28A_n7A DC_1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_28A_n7A |
| DC_1A-1A-3A-28A_n7A DC_1A-1A-3C-28A_n7A DC_1A-1A-3A-28A_n7B DC_1A-1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-1A-3A-3A-28A_n7A DC_1A-1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-3A-28A_n38A | DC_1A_n38A DC_3A_n38A DC_28A_n38A |
| DC_1A-3A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_28A_n40A |
| DC_1A-3A_n28A-n41A2 | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A |
| DC_1A-3A_n28A-n75A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3C_n28A-n75A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3A-28A_n77A2 DC_1A-3A-28A_n77C2 | DC_1A_n77A DC_3A_n77A DC_28A_n77A |
| DC_1A-3A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |
| DC_1A-3A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |
| DC_1A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A |
| DC_1A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A |
| DC_1A-3A-28A_n78A2 DC_1A-3C-28A_n78A2 DC_1A-3A-28A_n78C2 DC_1A-1A-3A-28A_n78A DC_1A-1A-3C-28A_n78A DC_1A-3A-28A_n78(2A) 2 DC_1A-3C-28A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n79A2 DC_1A-3A-28A_n79C2 | DC_1A_n79A DC_3A_n79A DC_28A_n79A |
| DC_1A-3A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_3A_n28A DC_3A_n79A |
| DC_1A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A |
| DC_1A-3A_n28A-n78A2 DC_1A-3C_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n28A-n78(2A)2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-32A_n28A DC_1A-3C-32A_n28A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3A-32A_n78A DC_1A-3A-32A_n78C | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-32A_n78(2A) | DC_1A_n78A DC_3A_n78A |
| DC_1A-3C-32A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-38A_n28A DC_1A-3C-38A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_38A_n28A |
| DC_1A-3A-38A_n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-38A_n78(2A) DC_1A-3C-38A_n78(2A) | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A_n38A-n78A | DC_1A_n38A DC_1A_n78A DC_3A_n38A DC_3A_n78A |
| DC_1A-3C-38A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_38A_n78A |
| DC_1A-3A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A |
| DC_1A-3A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A |
| DC_1A-3A_n40A-n78A DC_1A-3A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A |
| DC_1A-3A-40A_n78A DC_1A-3A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_40A_n78A |
| DC_1A-3A-40A_n78(2A) DC_1A-3A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_40A_n78A |
| DC_1A-3A-41A_n3A DC_1A-3A-41C_n3A | DC_1A_n3A DC_3A_n3A4 DC_41A_n3A DC_41C_n3A |
| DC_1A-3A-41A_n28A2 DC_1A-3A-41C_n28A2 | DC_1A_n28A DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-3A-41A_n41A | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A-(n)41AA | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A-41A_n77A DC_1A-3A-41C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-3A-41A_n77(2A) DC_1A-3A-41C_n77(2A) | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-3A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A |
| DC_1A-3A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A |
| DC_1A-3A-41A_n78A DC_1A-3A-41C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-3A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A |
| DC_1A-3A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A |
| DC_1A-3A-41A_n78(2A) DC_1A-3A-41C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-3A-41A_n79A2 DC_1A-3A-41C_n79A2 | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-42A_n28A2 DC_1A-3A-42C_n28A2 | DC_1A_n28A DC_3A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-3A-42A_n77A7,8 DC_1A-3A-42A_n77C7,8 DC_1A-3A-42C_n77A7,8 DC_1A-3A-42C_n77C7,8 DC_1A-3A-42D_n77A7,8 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A-42A_n77(2A) 7,8 DC_1A-3A-42C_n77(2A) 7,8 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A-42A_n78A7,8 DC_1A-3A-42A_n78C7,8 DC_1A-3A-42C_n78A7,8 DC_1A-3A-42C_n78C7,8 DC_1A-3A-42D_n78A7,8 | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-42A_n79A DC_1A-3A-42A_n79C DC_1A-3A-42C_n79A DC_1A-3A-42C_n79C DC_1A-3A-42D_n79A | DC_1A_n79A DC_3A_n79A |
| DC_1A-3A_n75A-n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_3A_n77A DC_3A_n79A |
| DC_1A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |
| DC_1A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |
| DC_1A-3A_n78A-n79A | DC_1A_n78A DC_1A_n79A DC_3A_n78A DC_3A_n79A |
| DC_1A-3A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_1A-5A-7A_n40A | DC_1A_n40A DC_5A_n40A DC_7A_n40A |
| DC_1A-5A-7A-7A_n40A | DC_1A_n40A DC_5A_n40A DC_7A_n40A |
| DC_1A-5A-7A_n77A | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A_n77(2A) DC_1A-5A-7A_n77(3A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A-7A_n77A | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A-7A_n77(2A) DC_1A-5A-7A-7A_n77(3A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A_n78A DC_1A-5A-7A_n78C DC_1A-1A-5A-7A_n78A | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A_n78(2A) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A_n78(A-C) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78A DC_1A-5A-7A-7A_n78C | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78(2A) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78(A-C) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-5A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-5A_n40A-n78A DC_1A-5A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_5A_n40A DC_5A_n78A |
| DC_1A-5A-41A_n79A | DC_1A_n79A DC_5A_n79A DC_41A_n79A |
| DC_1A-7A_n3A-n38A | DC_1A_n3A |
| DC_1A-7A_n3A-n78A DC_1A-7C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_7A_n3A DC_7C_n3A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n3A-n78(2A) DC_1A-7C_n3A-n78(2A) | DC_1A_n3A DC_1A_n78A DC_7A_n3A DC_7C_n3A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_7A_n5A DC_7A_n40A |
| DC_1A-7A_n5A-n78A DC_1A-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A |
| DC_1A-7A_n38A-n78A | DC_1A_n78A |
| DC_1A-7A-8A_n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A |
| DC_1A-7A-8A_n7A | DC_1A_n7A DC_7A_n7A DC_8A_n7A |
| DC_1A-7A-8A_n20A | DC_1A_n20A DC_7A_n20A DC_8A_n20A |
| DC_1A-7A-8A_n28A | DC_1A_n28A DC_7A_n28A DC_8A_n28A |
| DC_1A-7A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-8A_n78A DC_1A-7A-7A-8A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A-8A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_7A_n8A DC_7A_n78A |
| DC_1A-7A-20A_n3A DC_1A-7C-20A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_1A-7A-20A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-7A-20A_n28A3,8,14 | DC_1A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-7A-20A_n38A12,13 | CA_1A-20A |
| DC_1A-7A-20A_n78A2 DC_1A-7A-20A_n78C2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-20A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-26A_n78A DC_1A-7C-26A_n78A | DC_1A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-7A-26A_n78(2A) DC_1A-7C-26A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A-28A_n3A DC_1A-7C-28A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_1A-7A-28A_n5A DC_1A-7C-28A_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-7A-28A_n20A | DC_1A_n20A DC_7A_n20A DC_28A_n20A |
| DC_1A-7A-28A_n38A | 1A16 28A16 |
| DC_1A-7A-28A_n40A | DC_1A_n40A DC_7A_n40A DC_28A_n40A |
| DC_1A-7A-28A_n78A DC_1A-7C-28A_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-7A-28A_n78(2A) DC_1A-7C-28A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-1A-7A-28A_n78A | DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A_n28A-n78A2 DC_1A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_1A-7A-32A_n3A DC_1A-7C-32A_n3A | DC_1A_n3A DC_7A_n3A |
| DC_1A-7A-32A_n8A | DC_1A_n8A DC_7A_n8A |
| DC_1A-7A-32A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A-32A_n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-38A_n3A | DC_1A_n3A |
| DC_1A-7A-38A_n8A | DC_1A_n8A |
| DC_1A-7A-38A_n28A10 | DC_1A_n28A |
| DC_1A-7A-38A_n78A10 | DC_1A_n78A |
| DC_1A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A-40A_n78A DC_1A-7A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-7A-40A_n78(2A) DC_1A-7A-40C_n78(2A) | DC_1A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-7A_n40A-n78A DC_1A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-7A_n75A-n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-8A-(n)3AA | DC_1A_n3A DC_(n)3AA4 DC_8A_n3A |
| DC_1A-8A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A |
| DC_1A-8A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A |
| DC_1A-8A_n3A-n77(2A)2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A |
| DC_1A-8A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A |
| DC_1A-8A-11A_n3A | DC_1A_n3A DC_8A_n3A DC_11A_n3A |
| DC_1A-8A-11A_n28A | DC_1A_n28A DC_8A_n28A DC_11A_n28A |
| DC_1A-8A-11A_n77A2 | DC_1A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-8A-11A_n77(2A)2 DC_1A-8A-11A_n77(3A)2 | DC_1A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-8A-11A_n78A2 | DC_1A_n78A DC_8A_n78A DC_11A_n78A |
| DC_1A-8A-11A_n79A2 | DC_1A_n79A DC_8A_n79A DC_11A_n79A |
| DC_1A-8A-20A_n3A | DC_1A_n3A DC_8A_n3A DC_20A_n3A |
| DC_1A-8A-20A_n28A3,8,11,14 | DC_1A_n28A DC_8A_n28A DC_20A_n28A |
| DC_1A-8A-20A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-8A-28A_n3A | DC_1A_n3A DC_8A_n3A DC_28A_n3A |
| DC_1A-8A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n28A-n77(2A)2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A-28A_n78A | DC_1A_n78A DC_8A_n78A DC_28A_n78A |
| DC_1A-8A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-8A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_8A_n28A DC_8A_n79A |
| DC_1A-8A-32A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A-32A_n78A | DC_1A_n78A DC_8A_n78A |
| DC_1A-8A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_8A_n40A DC_8A_n78A |
| DC_1A-8A-40A_n78A DC_1A-8A-40C_n78A | DC_1A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-8A-40A_n78(2A) DC_1A-8A-40C_n78(2A) | DC_1A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-8A-42A_n3A2 DC_1A-8A-42C_n3A2 | DC_1A_n3A DC_8A_n3A DC_42A_n3A DC_42C_n3A |
| DC_1A-8A-42A_n28A2 DC_1A-8A-42C_n28A2 | DC_1A_n28A DC_8A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42A_n77A7,8 DC_1A-8A-42C_n77A7,8 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A-42A_n77(2A) 7,8 DC_1A-8A-42C_n77(2A) 7,8 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n77A-n79A DC_1A-8A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A |
| DC_1A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_11A_n3A DC_11A_n28A |
| DC_1A-11A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-11A_n3A-n77(2A) 2 | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-11A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_11A_n3A DC_11A_n79A |
| DC_1A-11A-18A_n3A | DC_1A_n3A DC_11A_n3A DC_18A_n3A |
| DC_1A-11A-18A_n28A | DC_1A_n28A DC_11A_n28A DC_18A_n28A |
| DC_1A-11A-18A_n41A | DC_1A_n41A DC_11A_n41A DC_18A_n41A |
| DC_1A-11A-18A_n77A | DC_1A_n77A DC_11A_n77A DC_18A_n77A |
| DC_1A-11A-18A_n77(2A) | DC_1A_n77A DC_11A_n77A DC_18A_n77A |
| DC_1A-11A-18A_n78A | DC_1A_n78A DC_11A_n78A DC_18A_n78A |
| DC_1A-11A-18A_n78(2A) | DC_1A_n78A DC_11A_n78A DC_18A_n78A |
| DC_1A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-11A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_18A_n3A DC_18A_n41A |
| DC_1A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_18A_n3A DC_18A_n77A |
| DC_1A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_18A_n3A DC_18A_n78A |
| DC_1A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_18A_n28A DC_18A_n41A |
| DC_1A-18A-28A_n77A | DC_1A_n77A DC_18A_n77A DC_28A_n77A |
| DC_1A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-18A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-18A-28A_n78A | DC_1A_n78A DC_18A_n78A DC_28A_n78A |
| DC_1A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-18A_n28A-n78(2A) | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-18A-28A_n79A2 | DC_1A_n79A DC_18A_n79A DC_28A_n79A |
| DC_1A-18A-41A_n3A DC_1A-18A-41C_n3A | DC_1A_n3A DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_1A-18A-41A_n77A DC_1A-18A-41C_n77A | DC_1A_n77A DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-18A-41A_n78A DC_1A-18A-41C_n78A | DC_1A_n78A DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-18A_n41A-n78A | DC_1A_n41A DC_18A_n41A DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n41A-n78(2A) | DC_1A_n41A DC_18A_n41A DC_1A_n78A DC_18A_n78A |
| DC_1A-18A-42A_n77A7,8 DC_1A-18A-42C_n77A7,8 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A-42A_n78A7,8 DC_1A-18A-42C_n78A7,8 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A-42A_n79A DC_1A-18A-42C_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A-21A_n77A2 DC_1A-19A-21A_n77C2 | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A_n77(2A) 2 | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A_n78A2 DC_1A-19A-21A_n78C2 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A_n78(2A) 2 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A_n79A2 DC_1A-19A-21A_n79C2 | DC_1A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-19A-42A_n77A7,8 DC_1A-19A-42A_n77C7,8 DC_1A-19A-42C_n77A7,8 DC_1A-19A-42C_n77C7,8 | DC_1A_n77A DC_19A_n77A |
| DC_1A-19A-42A_n78A7,8 DC_1A-19A-42A_n78C7,8 DC_1A-19A-42C_n78A7,8 DC_1A-19A-42C_n78C7,8 | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A-42A_n79A DC_1A-19A-42A_n79C DC_1A-19A-42C_n79A DC_1A-19A-42C_n79C | DC_1A_n79A DC_19A_n79A |
| DC_1A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_1A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_1A-20A_n3A-n38A | DC_1A_n3A DC_20A_n3A DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n7A-n78A | DC_1A_n7A DC_20A_n7A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-20A-28A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-20A_n28A-n75A | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A-28A_n78A | DC_1A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-20A_n28A-n78A2,3,8,14 | DC_1A_n28A DC_1A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-20A-32A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A-32A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A-32A_n28A8,14 | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A-32A_n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A-(n)38AA | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A-38A_n8A | DC_1A_n8A DC_20A_n8A DC_38A_n8A |
| DC_1A-20A-38A_n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-38A_n78(2A) | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n38A-n78A | DC_1A_n38A DC_20A_n38A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-40A_n78A DC_1A-20A-40C_n78A | DC_1A_n78A DC_20A_n78A DC_40A_n78A |
| DC_1A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_20A_n41A DC_20A_n78A |
| DC_1A-21A-28A_n77A2 | DC_1A_n77A DC_21A_n77A DC_28A_n77A |
| DC_1A-21A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_21A_n28A DC_21A_n77A |
| DC_1A-21A-28A_n78A2 | DC_1A_n78A DC_21A_n78A DC_28A_n78A |
| DC_1A-21A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_21A_n28A DC_21A_n78A |
| DC_1A-21A-28A_n79A2 | DC_1A_n79A DC_21A_n79A DC_28A_n79A |
| DC_1A-21A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_21A_n28A DC_21A_n79A |
| DC_1A-21A-42A_n77A7,8 DC_1A-21A-42A_n77C7,8 DC_1A-21A-42C_n77A7,8 DC_1A-21A-42C_n77C7,8 DC_1A-21A-42D_n77A7,8 DC_1A-21A-42D_n77C7,8 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A-42A_n78A7,8 DC_1A-21A-42A_n78C7,8 DC_1A-21A-42C_n78A7,8 DC_1A-21A-42C_n78C7,8 DC_1A-21A-42D_n78A7,8 DC_1A-21A-42D_n78C7,8 | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A-42A_n79A DC_1A-21A-42A_n79C DC_1A-21A-42C_n79A DC_1A-21A-42C_n79C DC_1A-21A-42D_n79A DC_1A-21A-42D_n79C | DC_1A_n79A DC_21A_n79A |
| DC_1A-21A_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A-21A_n78A-n79A | DC_1A_n78A DC_1A_n79A |
| DC_1A-28A_n3A-n77A2 | DC_28A_n3A DC_28A_n77A |
| DC_1A-28A_n3A-n78A2 | DC_1A_n3A DC_1A_n78A DC_28A_n3A DC_28A_n78A |
| DC_1A-28A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_28A_n5A DC_28A_n40A |
| DC_1A-28A_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-28A-(n)7AA | DC_1A_n7A DC_28A_n7A |
| DC_1A-28A_n7A-n78A | DC_1A_n7A DC_28A_n7A DC_1A_n78A DC_28A_n78A |
| DC_1A-28A_n7B-n78A | DC_1A_n7A DC_1A_n7B DC_28A_n7A DC_28A_n7B DC_1A_n78A DC_28A_n78A |
| DC_1A-28A-32A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A-40A_n78A DC_1A-28A-40C_n78A | DC_1A_n78A DC_28A_n78A DC_40A_n78A |
| DC_1A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-28A-42A_n77A7,8 DC_1A-28A-42A_n77C7,8 DC_1A-28A-42C_n77A7,8 DC_1A-28A-42C_n77C7,8 | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A-42A_n78A7,8 DC_1A-28A-42A_n78C7,8 DC_1A-28A-42C_n78A7,8 DC_1A-28A-42C_n78C7,8 | DC_1A_n78A DC_28A_n78A |
| DC_1A-28A-42A_n79A DC_1A-28A-42A_n79C DC_1A-28A-42C_n79A DC_1A-28A-42C_n79C | DC_1A_n79A DC_28A_n79A |
| DC_1A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_41A_n3A |
| DC_1A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A |
| DC_1A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A |
| DC_1A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_38A_n3A DC_38A_n78A |
| DC_1A-38A_n7A-n78A | DC_1A_n78A |
| DC_1A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_41A_n3A DC_41A_n77A |
| DC_1A-41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_1A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_41A_n3A DC_41A_n78A |
| DC_1A-41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_1A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_41A_n28A |
| DC_1A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A |
| DC_1A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_1A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A |
| DC_1A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_1A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_41A_n77A |
| DC_1A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_41A_n78A |
| DC_1A-42A_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A |
| DC_1A-42C_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A DC_42C_n3A DC_42C_n28A |
| DC_1A-42A_n3A-n77A7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A |
| DC_1A-42A_n3A-n77(2A) 7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A |
| DC_1A-42C_n3A-n77A7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-42C_n3A-n77(2A)7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-42A_n28A-n77A7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A |
| DC_1A-42A_n28A-n77(2A)7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A |
| DC_1A-42C_n28A-n77A7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-42C_n28A-n77(2A)7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-41A-42A_n77A7,8 DC_1A-41A-42C_n77A7,8 DC_1A-41C-42A_n77A7,8 DC_1A-41C-42C_n77A7,8 | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A-42A_n77(2A)7,8 DC_1A-41A-42C_n77(2A)7,8 | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A-42A_n78A7,8 DC_1A-41A-42C_n78A7,8 DC_1A-41C-42A_n78A7,8 DC_1A-41C-42C_n78A7,8 | DC_1A_n78A DC_41A_n78A |
| DC_1A-41A-42A_n79A DC_1A-41A-42C_n79A DC_1A-41C-42A_n79A DC_1A-41C-42C_n79A | DC_1A_n79A DC_41A_n79A |
| DC_1A-42A_n77A-n79A7,8 DC_1A-42C_n77A-n79A7,8 | DC_1A_n77A DC_1A_n79A |
| DC_1A-42A_n78A-n79A7,8 DC_1A-42C_n78A-n79A7,8 | DC_1A_n78A DC_1A_n79A |
| DC_2A-4A-7A_n28A | DC_2A_n28A DC_4A_n28A DC_7A_n28A |
| DC_2A-4A-7A_n78A DC_2A-4A-7C_n78A | DC_2A_n78A DC_4A_n78A |
| DC_2A-5A_n2A-n77A DC_2A-5A_n2A-n77C | DC_2A_n77A DC_5A_n2A DC_5A_n77A |
| DC_2A-5A_n2A-n78A | DC_5A_n2A DC_2A_n78A DC_5A_n78A |
| DC_2A-5A_n5A-n77A DC_2A-5A_n5A-n77C | DC_2A_n5A DC_2A_n77A DC_5A_n77A |
| DC_2A-5A-7A_n2A | DC_5A_n2A DC_7A_n2A |
| DC_2A-5A-7A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 |
| DC_2A-5A-7A_n66A DC_2A-5A-7C_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |
| DC_2A-5A-7A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A |
| DC_2A-2A-5A-7A_n66A DC_2A-5A-7A-7A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |
| DC_2A-5A-(n)12AA | DC_5A_n12A DC_2A_n12A DC_(n)12AA4 |
| DC_2A-12A-(n)5AA | DC_2A_n5A DC_12A_n5A DC_(n)5AA4 |
| DC_2A-5A-30A_n2A | DC_2A_n2A4 DC_5A_n2A DC_30A_n2A |
| DC_2A-5A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |
| DC_2A-2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |
| DC_2A-5A-30A_n77A9 DC_2A-2A-5A-30A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_30A_n77A9 |
| DC_2A-5A-30A_n77(2A) 9 | DC_2A_n77A9 DC_5A_n77A9 DC_30A_n77A9 |
| DC_2A-5A-48A_n12A | DC_2A_n12A DC_5A_n12A DC_48A_n12A |
| DC_2A-5A-48A_n77A7,8,9 DC_2A-5A-48A_n77C7,8,9 DC_2A-5A-48C_n77A7,8,9 DC_2A-5A-48C_n77C7,8,9 | DC_2A_n77A DC_5A_n77A |
| DC_2A-5A-66A_n2A DC_2A-5B-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-5A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-66A-66A_n2A DC_2A-5B-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-5A-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-5A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |
| DC_2A-5A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |
| DC_2A-5A-66A_n12A | DC_2A_n12A DC_5A_n12A DC_66A_n12A |
| DC_2A-5A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-2A-5A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-5A-66A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-5A-66A_n48A DC_2A-5A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |
| DC_2A-5A-66A-66A_n48A DC_2A-5A-66A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |
| DC_2A-5A-66A_n66A DC_2A-5B-66A_n66A | DC_2A_n66A DC_5A_n66A DC_66A_n66A4 |
| DC_2A-5A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-2A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-66A-66A_n66A DC_2A-5B-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-2A-5A-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-5A-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-66A_n71A | DC_2A_n71A DC_5A_n71A DC_66A_n71A |
| DC_2A-5A-66A_n77A9 DC_2A-5A-66A_n77C9 DC_2A-2A-5A-66A_n77C9 DC_2A-5A-66A-66A_n77C9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A_n77(2A) 9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-2A-5A-66A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A-66A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A_n78A | DC_2A_n78A DC_5A_n78A DC_66A_n78A |
| DC_2A-5A_n66A-n77A DC_2A-5A_n66A-n77C | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A |
| DC_2A-5A_n66A-n78A | DC_2A_n66A DC_5A_n66A DC_2A_n78A DC_5A_n78A |
| DC_2A-7A_n2A-n71A | DC_2A_n71A DC_7A_n2A DC_7A_n71A |
| DC_2A-7A_n2A-n78A | DC_7A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-12A_n2A | DC_7A_n2A DC_12A_n2A |
| DC_2A-7A-12A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A |
| DC_2A-2A-7A-12A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A |
| DC_2A-7A-12A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A |
| DC_2A-2A-7A-12A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A |
| DC_2A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7C-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7A-13A_n66A DC_2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-2A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7A-7A_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7C_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7A-28A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_2A-7A-28A_n66A DC_2A-7C-28A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A |
| DC_2A-7A-28A_n78A DC_2A-7C-28A_n78A | DC_2A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_2A-7A-28A_n78(2A) DC_2A-7C-28A_n78(2A) | DC_2A_n78A DC_28A_n78A |
| DC_2A-7A_n38A-n66A DC_2A-7C_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-7A_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-29A_n78A DC_2A-7C-29A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-7A-29A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-38A_n78A DC_2A-7C-38A_n78A | DC_2A_n78A |
| DC_2A-7A_n38A-n78A DC_2A-7C_n38A-n78A | DC_2A_n78A |
| DC_2A-7A-7A_n38A-n78A | DC_2A_n78A |
| DC_2A-7A-66A_n2A | DC_7A_n2A DC_66A_n2A |
| DC_2A-7A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-7A-66A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7C-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7A-66A_n28A | DC_2A_n28A DC_7A_n28A DC_66A_n28A |
| DC_2A-7A-66A_n38A | 2A5 66A5 |
| DC_2A-2A-7A-66A_n38A | 2A5 66A5 |
| DC_2A-7A-66A_n66A DC_2A-7C-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-7A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-7A-66A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |
| DC_2A-2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |
| DC_2A-7A_n66A-n71A | DC_2A_n66A DC_2A_n71A DC_7A_n66A DC_7A_n71A |
| DC_2A-7A-66A_n77A DC_2A-7C-66A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-66A_n77(2A) DC_2A-7C-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-7A-66A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-7A-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A_n66A-n77A DC_2A-7C_n66A-n77A DC_2A-7A-7A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-66A_n78A DC_2A-7C-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-2A-7A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A_n66A-n78A DC_2A-7C_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-7A-66A_n78(2A) DC_2A-7C-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-7A-7A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-66A-66A_n78A DC_2A-7C-66A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-66A-66A_n78(2A) DC_2A-7C-66A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-66A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-66A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-71A_n2A | DC_7A_n2A DC_71A_n2A |
| DC_2A-7A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_71A_n66A |
| DC_2A-2A-7A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_71A_n66A |
| DC_2A-7A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_71A_n78A |
| DC_2A-2A-7A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_71A_n78A |
| DC_2A-7A_n71A-n78A | DC_2A_n71A DC_7A_n71A DC_2A_n78A DC_7A_n78A |
| DC_2A-12A_n2A-n41A | DC_2A_n41A DC_12A_n2A DC_12A_n41A |
| DC_2A-12A_n2A-n78A | DC_12A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_2A-12A-48A_n5A | DC_2A_n5A DC_12A_n5A DC_48A_n5A |
| DC_2A-12A-66A_n5A | DC_2A_n5A DC_12A_n5A DC_66A_n5A |
| DC_2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |
| DC_2A-2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |
| DC_2A-12A-30A_n77A9 DC_2A-2A-12A-30A_n77A9 | DC_2A_n77A9 DC_12A_n77A9 DC_30A_n77A9 |
| DC_2A-12A-30A_n77(2A)9 | DC_2A_n77A9 DC_12A_n77A9 DC_30A_n77A9 |
| DC_2A-12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_2A-12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_2A-12A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-2A-12A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-12A-66A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-12A-66A_n41A | DC_2A_n41A DC_12A_n41A DC_66A_n41A |
| DC_2A-2A-12A-66A_n41A | DC_2A_n41A DC_12A_n41A DC_66A_n41A |
| DC_2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-12A-66A_n77A9 DC_2A-2A-12A-66A_n77A9 DC_2A-12A-66A-66A_n77A9 | DC_2A_n77A9 DC_12A_n77A9 DC_66A_n77A9 |
| DC_2A-12A-66A_n77(2A) 9 | DC_2A_n77A9 DC_12A_n77A9 DC_66A_n77A9 |
| DC_2A-12A-66A_n78A | DC_2A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-2A-12A-66A_n78A | DC_2A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-12A_n66A-n78A | DC_2A_n66A DC_12A_n66A DC_2A_n78A DC_12A_n78A |
| DC_2A-13A_n2A-n77A DC_2A-13A_n2A-n77C | DC_2A_n77A DC_13A_n2A DC_13A_n77A |
| DC_2A-13A_n5A-n77A9 DC_2A-2A-13A_n5A-n77A9 DC_2A-13A_n5A-n77C9 | DC_2A_n5A DC_2A_n77A DC_13A_n77A |
| DC_2A-13A_n25A-n66A8,14 | DC_2A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-13A-48A_n77A7,8,9 DC_2A-13A-48A_n77C7,8,9 DC_2A-13A-48C_n77A7,8,9 DC_2A-13A-48C_n77C7,8,9 | DC_2A_n77A9 DC_13A_n77A |
| DC_2A-13A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_2A-13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_2A-13A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-13A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-13A-66A_n48A DC_2A-13A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |
| DC_2A-13A-66A-66A_n48A DC_2A-13A-66A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |
| DC_2A-13A-66A_n66A DC_2A-2A-13A-66A_n66A DC_2A-13A-66A-66A_n66A DC_2A-2A-13A-66A-66A_n66A | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-13A-66B_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-13A-66A_n77A9 DC_2A-13A-66A_n77C9 DC_2A-2A-13A-66A_n77C9 DC_2A-2A-13A-66A-66A_n77A DC_2A-13A-66A-66A_n77C9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-2A-13A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-13A-66A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-13A_n66A-n77A9 DC_2A-13A_n66A-n77C9 DC_2A-2A-13A_n66A-n77A9 | DC_2A_n66A DC_2A_n77A9 DC_13A_n66A DC_13A_n77A9 |
| DC_2A-14A-30A_n2A | DC_2A_n2A4 DC_14A_n2A DC_30A_n2A |
| DC_2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-14A-30A_n77A9 DC_2A-2A-14A-30A_n77A9 | DC_2A_n77A9 DC_14A_n77A9 DC_30A_n77A9 |
| DC_2A-14A-30A_n77(2A) 9 | DC_2A_n77A9 DC_14A_n77A9 DC_30A_n77A9 |
| DC_2A-14A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-2A-14A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-14A-66A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-14A-66A_n77A9 DC_2A-2A-14A-66A_n77A9 DC_2A-14A-66A-66A_n77A9 | DC_2A_n77A9 DC_14A_n77A9 DC_66A_n77A9 |
| DC_2A-14A-66A_n77(2A) 9 | DC_2A_n77A9 DC_14A_n77A9 DC_66A_n77A9 |
| DC_2A-28A-66A_n7A | DC_2A_n7A DC_28A_n7A DC_66A_n7A |
| DC_2A-28A-66A_n66A | DC_2A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_2A-29A-30A_n2A | DC_2A_n2A4 DC_30A_n2A |
| DC_2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-29A-30A_n77A9 DC_2A-2A-29A-30A_n77A9 | DC_2A_n77A9 DC_30A_n77A9 |
| DC_2A-29A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |
| DC_2A-29A-66A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |
| DC_2A-29A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-29A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-29A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |
| DC_2A-2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |
| DC_2A-29A-66A_n77A9 | DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-29A-66A_n78A | DC_2A_n78A DC_66A_n78A |
| DC_2A-30A-(n)5AA DC_2A-2A-30A-(n)5AA | DC_2A_n5A DC_30A_n5A DC_(n)5AA4 |
| DC_2A-30A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-30A-66A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-30A-66A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-30A-66A_n77A9 DC_2A-2A-30A-66A_n77A9 DC_2A-30A-66A-66A_n77A9 | DC_2A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_2A-30A-66A_n77(2A) 9 | DC_2A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_2A-46A_n41A-n66A DC_2A-46C_n41A-n66A DC_2A-46D_n41A-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A-46A_n41A-n71A DC_2A-46C_n41A-n71A DC_2A-46D_n41A-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n41(2A)-n71A DC_2A-46C_n41(2A)-n71A DC_2A-46D_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A-48A_n2A DC_2A-46C-48A_n2A DC_2A-46D-48A_n2A DC_2A-46E-48A_n2A | DC_2A_n2A4 DC_48A_n2A |
| DC_2A-46A-48A_n5A DC_2A-46C-48A_n5A DC_2A-46D-48A_n5A DC_2A-46E-48A_n5A | DC_2A_n5A DC_48A_n5A |
| DC_2A-46A-48A_n66A DC_2A-46C-48A_n66A DC_2A-46D-48A_n66A DC_2A-46E-48A_n66A | DC_2A_n66A DC_48A_n66A |
| DC_2A-46A-66A_n5A DC_2A-46C-66A_n5A DC_2A-46D-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-46A-66A_n41A DC_2A-46C-66A_n41A DC_2A-46D-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-46A-66A_n41(2A) DC_2A-46C-66A_n41(2A) DC_2A-46D-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-46A-66A_n71A DC_2A-46C-66A_n71A DC_2A-46D-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-48A-(n)5AA | DC_2A_n5A DC_48A_n5A DC_(n)5AA4 |
| DC_2A-46A_n66A-n71A DC_2A-46C_n66A-n71A DC_2A-46D_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-48A_n48A-n66A | DC_2A_n48A DC_2A_n66A DC_48A_n66A |
| DC_2A-48A-66A_n2A DC_2A-48C-66A_n2A DC_2A-48D-66A_n2A DC_2A-48E-66A_n2A | DC_66A_n2A DC_48A_n2A DC_2A_n2A4 |
| DC_2A-48A-66A_n5A | DC_2A_n5A DC_48A_n5A DC_66A_n5A |
| DC_2A-48C-66A_n5A DC_2A-48D-66A_n5A DC_2A-48E-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-48A-66A_n12A | DC_2A_n12A DC_48A_n12A DC_66A_n12A |
| DC_2A-48A-66A_n66A DC_2A-48C-66A_n66A DC_2A-48D-66A_n66A DC_2A-48E-66A_n66A | DC_66A_n66A4 DC_48A_n66A DC_2A_n66A |
| DC_2A-48A-66A_n71A | DC_2A_n71A DC_48A_n71A DC_66A_n71A |
| DC_2A-48A-66A_n77A7,8,9 DC_2A-48C-66A_n77A7,8,9 DC_2A-48A-66A_n77C7,8,9 DC_2A-48C-66A_n77C7,8,9 DC_2A-48D-66A_n77A7,8,9 DC_2A-48E-66A_n77A7,8,9 | DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-66A_n2A-n71A | DC_2A_n2A4 DC_2A_n71A DC_66A_n2A DC_66A_n71A |
| DC_2A-66A_n2A-n77A DC_2A-66A_n2A-n77C | DC_2A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-66A-66A_n2A-n77A | DC_2A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-66A-(n)5AA DC_2A-2A-66A-(n)5AA DC_2A-66A-66A-(n)5AA | DC_2A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_2A-66A_n2A-n78A | DC_66A_n2A DC_2A_n78A DC_66A_n78A |
| DC_2A-66A_n5A-n77A9 DC_2A-2A-66A_n5A-n77A9 DC_2A-66A-66A_n5A-n77A9 DC_2A-66A_n5A-n77C9 | DC_2A_n5A DC_2A_n77A9 DC_5A_n77A DC_66A_n5A DC_66A_n77A9 |
| DC_2A-66A_n25A-n66A7,8 | DC_2A_n66A DC_66A_n25A |
| DC_2A-66A_n38A-n78A | DC_2A_n38A DC_2A_n78A DC_66A_n38A DC_66A_n78A |
| DC_2A-66A_n66A-n71A | DC_2A_n66A DC_2A_n71A DC_66A_n66A4 DC_66A_n71A |
| DC_2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |
| DC_2A-2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |
| DC_2A-66A-71A_n41A | DC_2A_n41A DC_66A_n41A DC_71A_n41A |
| DC_2A-2A-66A-71A_n41A | DC_2A_n41A DC_66A_n41A DC_71A_n41A |
| DC_2A-66A-71A_n66A | DC_2A_n66A DC_66A_n66A4 DC_71A_n66A |
| DC_2A-66A-71A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A_n71A-n77A | DC_2A_n71A DC_2A_n77A DC_66A_n71A DC_66A_n77A |
| DC_2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-66A-(n)71AA DC_2A-66C-(n)71AA | DC_2A_n71A DC_66A_n71A DC_(n)71AA |
| DC_2A-66A_n41A-n71A DC_2A-66A_n41C-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A_n66A-n77A9 DC_2A-2A-66A_n66A-n77A9 DC_2A-66A_n66A-n77C9 | DC_2A_n66A DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_66A_n66A4 |
| DC_2A-66A-71A_n2A | DC_66A_n2A DC_71A_n2A |
| DC_2A-66A_n71A-n78A | DC_2A_n71A DC_66A_n71A DC_2A_n78A DC_66A_n78A |
| DC_2A-71A_n2A-n41A | DC_2A_n41A DC_71A_n2A DC_71A_n41A |
| DC_2A-71A_n2A-n78A | DC_71A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-71A_n66A-n78A | DC_2A_n66A DC_71A_n66A DC_2A_n78A DC_71A_n78A |
| DC_3A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A |
| DC_3A-3A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A |
| DC_3A_n1A-n28A-n75A | DC_3A_n1A DC_3A_n28A |
| DC_3A_n1A-n40A-n78A | DC_3A_n1A DC_3A_n40A DC_3A_n78A |
| DC_3A_n1A-n75A-n78A | DC_3A_n1A DC_3A_n78A |
| DC_3A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A |
| DC_3A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A |
| DC_3A-5A-7A_n40A | DC_3A_n40A DC_5A_n40A DC_7A_n40A |
| DC_3A-5A-7A-7A_n40A | DC_3A_n40A DC_5A_n40A DC_7A_n40A |
| DC_3A-5A-7A_n77A | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A_n77(2A) DC_3A-5A-7A_n77(3A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A-7A_n77A | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A-7A_n77(2A) DC_3A-5A-7A-7A_n77(3A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A_n78A DC_3C-5A-7A_n78A DC_3A-5A-7A_n78C | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A_n78(2A) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A_n78(A-C) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78A DC_3A-5A-7A-7A_n78C | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78(2A) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78(A-C) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_3A-5A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_3A-5A_n40A-n78A DC_3A-5A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A |
| DC_3A_n5A-n40A-n78A | DC_3A_n5A DC_3A_n40A DC_3A_n78A |
| DC_3A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-3A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-7A_n1A-n28A DC_3C-7A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_7A_n1A DC_7A_n28A |
| DC_3A-7C_n1A-n28A DC_3C-7C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_7A_n1A DC_7A_n28A DC_7C_n1A DC_7C_n28A |
| DC_3A-7A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A |
| DC_3A-7A_n1A-n78A2, 9 DC_3C-7A_n1A-n78A2 | DC_3A_n1A DC_3C_n1A DC_3A_n78A9 DC_3C_n78A DC_7A_n1A DC_7A_n78A9 |
| DC_3A-7A_n1A-n78(2A)2 DC_3C-7A_n1A-n78(2A)2 | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A DC_7A_n1A DC_7A_n78A |
| DC_3A-3A-7A_n1A-n78A2, 9 | DC_3A_n1A DC_3A_n78A9 DC_7A_n1A DC_7A_n78A9 |
| DC_3A-7A-7A_n1A-n78A2, 9 | DC_3A_n1A DC_3A_n78A9 DC_7A_n1A DC_7A_n78A9 |
| DC_3A-3A-7A-7A_n1A-n78A2, 9 | DC_3A_n1A DC_3A_n78A9 DC_7A_n1A DC_7A_n78A9 |
| DC_3A-7C_n1A-n78A DC_3C-7C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_3A-7C_n1A-n78(2A)2 DC_3C-7C_n1A-n78(2A)2 | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_3A-5A-41A_n79A | DC_3A_n79A DC_5A_n79A DC_41A_n79A |
| DC_3A-7A_n1A-n75A | DC_3A_n1A DC_7A_n1A |
| DC_3C-7A_n1A-n75A | DC_3C_n1A DC_3A_n1A DC_7A_n1A |
| DC_3A-7A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_3A_n78A DC_7A_n78A |
| DC_3A-7C_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_3A_n78A DC_7C_n78A DC_7A_n78A |
| DC_3A-7A_n5A-n40A | DC_3A_n5A DC_3A_n40A DC_7A_n5A DC_7A_n40A |
| DC_3A-7A_n5A-n78A9 DC_3A-7C_n5A-n78A9 DC_3C-7A_n5A-n78A9 DC_3C-7C_n5A-n78A9 | DC_3A_n5A DC_3A_n78A9 DC_3C_n78A9 DC_7A_n5A DC_7C_n5A DC_7A_n78A9 DC_7C_n78A9 |
| DC_3A-7A_n7A-n78A2 | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A_n7A-n78A2 | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A |
| DC_3C-7A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-8A_n1A DC_3C-7A-8A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-3A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-3A-7A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-8A_n28A | DC_3A_n28A DC_7A_n28A DC_8A_n28A |
| DC_3A-7A-8A_n40A | DC_3A_n40A DC_7A_n40A DC_8A_n40A |
| DC_3A-7A-8A_n77A2 | DC_3A_n77A DC_7A_n77A DC_8A_n77A |
| DC_3A-7A-8A_n78A2, 9 DC_3A-7A-8B_n78A2 DC_3C-7A-8A_n78A | DC_3A_n78A9 DC_3C_n78A DC_7A_n78A 9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-7A-8A_n78(2A) | DC_3A_n78A, DC_7A_n78A, DC_8A_n78A |
| DC_3A-3A-7A-8A_n78A2, 9 DC_3A-3A-7A-8B_n78A2 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-7A-7A-8A_n78A2, 9 DC_3A-7A-7A-8B_n78A2 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-3A-7A-7A-8A_n78A2, 9 DC_3A-3A-7A-7A-8B_n78A2 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-3A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-7A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-3A-7A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-7A-20A_n1A DC_3C-7A-20A_n1A DC_3A-7C-20A_n1A DC_3C-7C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_3A-7A-20A_n3A | DC_3A_n3A DC_7A_n3A DC_20A_n3A |
| DC_3A-7A-20A_n8A | DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_3A-7A-20A_n28A3,8,14 DC_3C-7A-20A_n28A3 | DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_3A-7A-20A_n38A12,13 | CA_3A-20A |
| DC_3A-7A-20A_n78A2 DC_3C-7A-20A_n78A2 DC_3A-7A-20A_n78C2 | DC_3A_n78A DC_3C_n78A DC_20A_n78A DC_7A_n78A |
| DC_3A-7A-20A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-26A_n78A DC_3C-7A-26A_n78A DC_3A-7C-26A_n78A DC_3C-7C-26A_n78A | DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_3A-7A-26A_n78(2A) DC_3A-7C-26A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_3A-7A_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_3A_n26A DC_7A_n26A |
| DC_3C-7A-26A_n78(2A) DC_3C-7C-26A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_3A-7C_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_7A_n26A DC_7C_n26A |
| DC_3C-7A_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A |
| DC_3C-7C_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |
| DC_3A-7A-28A_n1A DC_3C-7A-28A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_28A_n1A |
| DC_3A-7A-7A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_28A_n1A |
| DC_3A-7A-28A_n3A DC_3A-7C-28A_n3A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_3A-7A-28A_n5A DC_3A-7C-28A_n5A DC_3C-7A-28A_n5A DC_3C-7C-28A_n5A | DC_3A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_3A-7A-28A_n7A DC_3C-7A-28A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_3A-3A-7A-28A_n7A | DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_3A-7A-28A_n38A | 3A17 28A17 |
| DC_3A-7A-28A_n40A | DC_3A_n40A DC_7A_n40A DC_28A_n40A |
| DC_3A-7A-28A_n78A2, 9 DC_3A-7C-28A_n78A2, 9 DC_3C-7A-28A_n78A9 DC_3C-7C-28A_n78A9 | DC_3A_n78A9 DC_3C_n78A9 DC_7A_n78A9 DC_7C_n78A9 DC_28A_n78A9 |
| DC_3A-7A-28A_n78(2A) DC_3A-7C-28A_n78(2A) DC_3C-7A-28A_n78(2A)2 DC_3C-7C-28A_n78(2A)2 | DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A_n28A-n78A2, 9 DC_3A-7C_n28A-n78A9 DC_3C-7A_n28A-n78A9 DC_3C-7C_n28A-n78A9 | DC_3A_n28A DC_3A_n78A9 DC_3C_n78A9 DC_7A_n28A DC_7A_n78A9 DC_7C_n28A DC_7C_n78A9 |
| DC_3A-7A-32A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A-32A_n28A DC_3C-7A-32A_n28A | DC_3A_n28A DC_7A_n28A |
| DC_3A-7A-32A_n78A DC_3C-7A-32A_n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-38A_n28A10 DC_3C-7A-38A_n28A10 | DC_3A_n28A |
| DC_3A-7A-38A_n78A15 DC_3C-7A-38A_n78A15 | DC_3A_n78A DC_3C_n78A |
| DC_3A-7A_n38A-n78A | DC_3A_n78A |
| DC_3A-7A-40A_n1A DC_3A-7A-40C_n1A | DC_3A_n1A DC_7A_n1A DC_40A_n1A |
| DC_3A-7A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A-40A_n78A DC_3A-7A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_3A-7A-40A_n78(2A) DC_3A-7A-40C_n78(2A) | DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_3A-7A_n40A-n78A DC_3A-7A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |
| DC_3A-7A_n75A-n78A | DC_3A_n78A DC_7A_n78A |
| DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_7A_n78A DC_7A_n80A |
| DC_3A-8A_n1A-n28A | DC_3A_n1A DC_8A_n1A DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n1A-n40A | DC_3A_n1A DC_8A_n1A DC_3A_n40A DC_8A_n40A |
| DC_3A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-3A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A |
| DC_(n)3AA-n8A-n77A DC_(n)3AA-n8A-n77(2A) | DC_(n)3AA4 DC_3A_n8A DC_3A_n77A |
| DC_3A-8A-11A_n28A | DC_3A_n28A DC_8A_n28A DC_11A_n28A |
| DC_3A-8A-11A_n77A2 | DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_3A-8A-11A_n77(2A) 2 DC_3A-8A-11A_n77(3A)2 | DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_3A-8A-20A_n1A | DC_3A_n1A DC_8A_n1A DC_20A_n1A |
| DC_3A-8A-20A_n28A DC_3C-8A-20A_n28A | DC_3A_n28A DC_3C_n28A DC_8A_n28A DC_20A_n28A |
| DC_3A-8A-20A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A |
| DC_3A-8A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_3A-8A_n28A-n77(2A)2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_3A-8A-28A_n78A | DC_3A_n78A DC_8A_n78A DC_28A_n78A |
| DC_3A-8A_n28A-n78A2 | DC_3A_n28A DC_3A_n78A DC_8A_n28A DC_8A_n78A |
| DC_3A-8A-32A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A-32A_n28A DC_3C-8A-32A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A-32A_n78A | DC_3A_n78A DC_8A_n78A |
| DC_3A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_8A_n40A DC_8A_n78A |
| DC_3A-8A-40A_n1A DC_3A-8A-40C_n1A | DC_3A_n1A DC_8A_n1A DC_40A_n1A |
| DC_3A-8A-40A_n78A DC_3A-8A-40C_n78A | DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-8A-40A_n78(2A) DC_3A-8A-40C_n78(2A) | DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-8A_n40A-n79A DC_3A-8A_n40A-n79C | DC_3A_n40A DC_3A_n79A DC_8A_n40A DC_8A_n79A |
| DC_3A-8A-41A_n1A DC_3A-8A-41C_n1A | DC_3A_n1A DC_8A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-3A-8A-41A_n1A DC_3A-3A-8A-41C_n1A | DC_3A_n1A DC_8A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-8A-41A_n78A DC_3A-8A-41C_n78A | DC_3A_n78A DC_8A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-3A-8A-41A_n78A DC_3A-3A-8A-41C_n78A | DC_3A_n78A DC_8A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-8A-42A_n77A7,8 DC_3A-8A-42C_n77A7,8 | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n77A-n79A | DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A |
| DC_3A-8A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_8A_n78A DC_8A_n80A |
| DC_3A-11A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-11A_n28A-n77(2A) 2 | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-18A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_18A_n3A DC_18A_n41A |
| DC_3A-18A_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_18A_n3A DC_18A_n77A |
| DC_3A-18A_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_18A_n3A DC_18A_n78A |
| DC_3A-18A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |
| DC_3A-18A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_3A-18A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_3A-18A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_3A-18A_n28A-n78(2A) | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_3A-18A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_3A-18A_n41A-n77(2A) | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_3A-18A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_3A-18A_n41A-n78(2A) | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_3A-18A-42A_n77A7,8 DC_3A-18A-42C_n77A7,8 | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A-42A_n78A7,8 DC_3A-18A-42C_n78A7,8 | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A-42A_n79A DC_3A-18A-42C_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A_n1A-n77A2 | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A |
| DC_3A-19A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A |
| DC_3A-19A_n1A-n79A2 | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A |
| DC_3A-19A-21A_n77A2 DC_3A-19A-21A_n77C2 | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A_n78A2 DC_3A-19A-21A_n78C2 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A_n79A2 DC_3A-19A-21A_n79C2 | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-42A_n1A2 DC_3A-19A-42C_n1A2 | DC_3A_n1A DC_19A_n1A DC_42A_n1A |
| DC_3A-19A-42A_n77A7,8 DC_3A-19A-42A_n77C7,8 DC_3A-19A-42C_n77A7,8 DC_3A-19A-42C_n77C7,8 DC_3A-19A-42D_n77A7,8 DC_3A-19A-42D_n77C7,8 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A-42A_n78A7,8 DC_3A-19A-42A_n78C7,8 DC_3A-19A-42C_n78A7,8 DC_3A-19A-42C_n78C7,8 DC_3A-19A-42D_n78A7,8 DC_3A-19A-42D_n78C7,8 | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A-42A_n79A2 DC_3A-19A-42A_n79C2 DC_3A-19A-42C_n79A2 DC_3A-19A-42C_n79C2 DC_3A-19A-42D_n79A DC_3A-19A-42D_n79C | DC_3A_n79A DC_19A_n79A |
| DC_3A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_3A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_3A-20A_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_20A_n1A DC_20A_n7A |
| DC_3C-20A_n1A-n7A | DC_3A_n1A DC_3C_n1A DC_3A_n7A DC_3C_n7A DC_20A_n1A DC_20A_n7A |
| DC_3A-20A_n1A-n28A8,14 | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_20A_n28A |
| DC_3C-20A_n1A-n28A8,14 | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_3C_n1A DC_20A_n28A |
| DC_3A-20A_n1A-n75A | DC_3A_n1A DC_20A_n1A |
| DC_3C-20A_n1A-n75A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A_n1A-n78A DC_3A-3A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A |
| DC_3C-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_3C_n1A DC_3C_n78A |
| DC_3A-20A_n3A-n67A | DC_3A_n3A4 DC_20A_n3A |
| DC_3A-20A_n7A-n28A8,14 | DC_3A_n7A DC_3A_n28A DC_20A_n7A DC_20A_n28A |
| DC_3C-20A_n7A-n28A8,14 | DC_3A_n7A DC_3A_n28A DC_3C_n7A DC_20A_n7A DC_20A_n28A |
| DC_3A-20A_n7A-n78A | DC_3A_n7A DC_3A_n78A DC_20A_n7A DC_20A_n78A |
| DC_3A-20A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_20A_n8A DC_20A_n78A |
| DC_3A-20A-28A_n1A | DC_3A_n1A DC_20A_n1A DC_28A_n1A |
| DC_3A-20A_n28A-n75A | DC_3A_n28A DC_20A_n28A |
| DC_3C-20A_n28A-n75A | DC_20A_n28A DC_3A_n28A |
| DC_3A-20A-28A_n78A | DC_3A_n78A DC_20A_n78A DC_28A_n78A |
| DC_3A-20A_n28A-n78A2,3,8,14 DC_3C-20A_n28A-n78A2,3,8,14 | DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-20A-32A_n1A DC_3C-20A-32A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A-32A_n7A | DC_3A_n7A DC_20A_n7A |
| DC_3A-20A-32A_n28A8,14 DC_3C-20A-32A_n28A8,14 | DC_3A_n28A DC_20A_n28A |
| DC_3A-20A-32A_n78A | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A-38A_n78A DC_3C-20A-38A_n78A | DC_3A_n78A DC_3C_n78A DC_20A_n78A DC_38A_n78A |
| DC_3A-20A-38A_n78(2A) | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A_n38A-n78A | DC_3A_n78A DC_20A_n78A DC_3A_n38A DC_20A_n38A |
| DC_3A-20A-40A_n78A DC_3A-20A-40C_n78A | DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_3A-20A-40A_n78(2A) DC_3A-20A-40C_n78(2A) | DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_3A-20A-41A_n1A DC_3A-20A-41C_n1A | DC_3A_n1A DC_20A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-3A-20A-41A_n1A DC_3A-3A-20A-41C_n1A | DC_3A_n1A DC_20A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-20A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |
| DC_3A-20A-41A_n78A DC_3A-20A-41C_n78A | DC_3A_n78A DC_20A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-3A-20A-41A_n78A DC_3A-3A-20A-41C_n78A | DC_3A_n78A DC_20A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-20A-67A_n3A | DC_3A_n3A4 DC_20A_n3A |
| DC_3A-20A_SUL_n78A-n80A DC_3C-20A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_20A_n78A DC_20A_n80A |
| DC_3A-21A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_21A_n28A DC_21A_n77A |
| DC_3A-21A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_21A_n28A DC_21A_n78A |
| DC_3A-21A_n28A-n79A2 | DC_3A_n28A DC_3A_n79A DC_21A_n28A DC_21A_n79A |
| DC_3A-21A-42A_n1A2 DC_3A-21A-42C_n1A2 | DC_3A_n1A DC_21A_n1A DC_42A_n1A |
| DC_3A-21A_n1A-n77A2 | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A |
| DC_3A-21A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A |
| DC_3A-21A_n1A-n79A2 | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A |
| DC_3A-21A-42A_n77A7,8 DC_3A-21A-42A_n77C7,8 DC_3A-21A-42C_n77A7,8 DC_3A-21A-42C_n77C7,8 DC_3A-21A-42D_n77A7,8 DC_3A-21A-42D_n77C7,8 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A-42A_n78A7,8 DC_3A-21A-42A_n78C7,8 DC_3A-21A-42C_n78A7,8 DC_3A-21A-42C_n78C7,8 DC_3A-21A-42D_n78A7,8 DC_3A-21A-42D_n78C7,8 | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A-42A_n79A DC_3A-21A-42A_n79C DC_3A-21A-42C_n79A DC_3A-21A-42C_n79C DC_3A-21A-42D_n79A DC_3A-21A-42D_n79C | DC_3A_n79A DC_21A_n79A |
| DC_3A-21A_n77A-n79A | DC_3A_n77A DC_3A_n79A DC_21A_n77A DC_21A_n79A |
| DC_3A-21A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_21A_n78A DC_21A_n79A |
| DC_3A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_28A_n1A DC_28A_n40A |
| DC_3A-28A_n1A-n78A2 | DC_3A_n1A DC_28A_n1A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n3A-n78A2 | DC_3A_n3A4 DC_28A_n3A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n5A-n40A | DC_3A_n5A DC_3A_n40A DC_28A_n5A DC_28A_n40A |
| DC_3A-28A_n5A-n78A2 DC_3C-28A_n5A-n78A2 | DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A |
| DC_3A-28A-(n)7AA DC_3C-28A-(n)7AA | DC_3A_n7A DC_28A_n7A |
| DC_3A-28A_n7A-n78A | DC_3A_n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |
| DC_3A-3A-28A_n7A-n78A | DC_3A_n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |
| DC_3A-3A-28A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |
| DC_3C-28A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_3C-28A_n7B-n78A | DC_3A_n7A DC_3C_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_3A-28A-32A_n1A | DC_3A_n1A DC_28A_n1A |
| DC_3A-28A-40A_n78A DC_3A-28A-40C_n78A | DC_3A_n78A DC_28A_n78A DC_40A_n78A |
| DC_3A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_28A_n40A DC_28A_n78A |
| DC_3A-28A-41A_n78A DC_3A-28A-41C_n78A | DC_3A_n78A DC_28A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-28A-42A_n77A7,8 DC_3A-28A-42A_n77C7,8 DC_3A-28A-42C_n77A7,8 DC_3A-28A-42C_n77C7,8 | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A-42A_n78A7,8 DC_3A-28A-42A_n78C7,8 DC_3A-28A-42C_n78A7,8 DC_3A-28A-42C_n78C7,8 | DC_3A_n78A DC_28A_n78A |
| DC_3A-28A-42A_n79A DC_3A-28A-42A_n79C DC_3A-28A-42C_n79A DC_3A-28A-42C_n79C | DC_3A_n79A DC_28A_n79A |
| DC_3A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A |
| DC_3A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A |
| DC_3A-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A |
| DC_3C-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A |
| DC_3A-32A_n1A-n78A DC_3C-32A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A |
| DC_3A-38A_n7A-n78A | DC_3A_n78A |
| DC_3A-32A-38A_n28A DC_3C-32A-38A_n28A | DC_3A_n28A DC_38A_n28A |
| DC_3A-38A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_3C-38A_n28A-n78A | DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_3A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A_n40A-n41A-n79A | DC_3A_n40A DC_3A_n41A DC_3A_n79A |
| DC_3A-41A_n1A-n78A DC_3A-3A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-41C_n1A-n78A DC_3A-3A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_3A-41A_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A |
| DC_3A-41C_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_3A-41A_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A |
| DC_3A-41C_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_3A-41A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_41A_n28A |
| DC_3A-41A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A |
| DC_3A-41C_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_3A-41A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |
| DC_3A-41C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_3A-41A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_41A_n77A |
| DC_3A-41A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42A_n77A7,8 DC_3A-41A-42C_n77A7,8 DC_3A-41C-42A_n77A7,8 DC_3A-41C-42C_n77A7,8 | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42A_n77(2A)7,8 DC_3A-41A-42C_n77(2A)7,8 | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42A_n78A7,8 DC_3A-41A-42C_n78A7,8 DC_3A-41C-42A_n78A7,8 DC_3A-41C-42C_n78A7,8 | DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42A_n79A DC_3A-41A-42C_n79A DC_3A-41C-42A_n79A DC_3A-41C-42C_n79A | DC_3A_n79A DC_41A_n79A |
| DC_3A-42A_n1A-n77A7,8 DC_3A-42C_n1A-n77A7,8 | DC_3A_n1A DC_3A_n77A |
| DC_3A-42A_n1A-n78A7,8 DC_3A-42C_n1A-n78A7,8 | DC_3A_n1A DC_3A_n78A |
| DC_3A-42A_n1A-n79A DC_3A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A |
| DC_3A-42A_n28A-n77A7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_3A-42A_n28A-n77(2A)7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_3A-42C_n28A-n77A7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_3A-42C_n28A-n77(2A)7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_3A-42A_n77A-n79A7,8 DC_3A-42C_n77A-n79A7,8 | DC_3A_n77A DC_3A_n79A |
| DC_3A-42A_n78A-n79A7,8 DC_3A-42C_n78A-n79A7,8 | DC_3A_n78A DC_3A_n79A |
| DC_5A-7A_n2A-n78A | DC_5A_n2A DC_7A_n2A DC_5A_n78A DC_7A_n78A |
| DC_5A-7A_n40A-n77A | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A_n40A-n77(2A) | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A_n40A-n78A | DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_5A-7A_n40A-n78C | DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_5A-7A-66A_n2A | DC_5A_n2A DC_7A_n2A DC_66A_n2A |
| DC_5A-7A-66A_n7A | DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_5A-7A-66A-66A_n7A | DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_5A-7A-66A_n66A DC_5A-7C-66A_n66A DC_5A-7A-7A-66A_n66A | DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_5A-7A-66A_n78A DC_5A-7C-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_5A-7A-66A-66A_n78A DC_5A-7C-66A-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_5A-7A_n66A-n78A | DC_5A_n66A DC_7A_n66A DC_5A_n78A DC_7A_n78A |
| DC_5A-30A-66A_n2A DC_5A-30A-66A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A |
| DC_5A-30A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_5A-30A-66A_n66A | DC_5A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_5A-30A-66A_n77A9 DC_5A-30A-66A-66A_n77A9 | DC_5A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_5A-30A-66A_n77(2A) 9 | DC_5A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_5A-48A-(n)12AA | DC_5A_n12A DC_48A_n12A DC_(n)12AA4 |
| DC_5A-48A-66A_n12A | DC_5A_n12A DC_48A_n12A DC_66A_n12A |
| DC_5A-48A-66A_n71A | DC_5A_n71A DC_48A_n71A DC_66A_n71A |
| DC_5A-66A_n2A-n77A DC_5A-66A_n2A-n77C | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A |
| DC_5A-66A-66A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A |
| DC_5A-66A_n5A-n77A DC_5A-66A_n5A-n77C | DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_5A-66A-66A_n5A-n77A | DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_5A-48A-66A_n77A7,8,9 DC_5A-48A-66A_n77C7,8,9 DC_5A-48C-66A_n77A7,8,9 DC_5A-48C-66A_n77C7,8,9 | DC_5A_n77A DC_66A_n77A |
| DC_5A-66A_n2A-n78A | DC_5A_n2A DC_66A_n2A DC_5A_n78A DC_66A_n78A |
| DC_5A-66A-(n)12AA | DC_5A_n12A DC_66A_n12A DC_(n)12AA4 |
| DC_5A-66A_n66A-n77A9 DC_5A-66A_n66A-n77C9 | DC_5A_n66A DC_5A_n77A9 DC_66A_n77A9 |
| DC_7A_n1A-n8A-n78A2 | DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_7A-7A_n1A-n8A-n78A2 | DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_7A-8A_n1A-n40A | DC_7A_n1A DC_8A_n1A DC_7A_n40A DC_8A_n40A |
| DC_7A-8A_n1A-n78A2 | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_7A-7A-8A_n1A-n78A2 | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_7A-8A-20A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |
| DC_7A-8A-20A_n3A | DC_7A_n3A DC_8A_n3A DC_20A_n3A |
| DC_7A-8A-20A_n28A9 | DC_7A_n28A |
| DC_7A-8A-20A_n78A | DC_7A_n78A DC_8A_n78A DC_20A_n78A |
| DC_7A-8A-32A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A-32A_n78A | DC_7A_n78A DC_8A_n78A |
| DC_7A-8A-38A_n1A | DC_8A_n1A |
| DC_7A-8A_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_7A-8A-40A_n1A DC_7A-8A-40C_n1A | DC_7A_n1A DC_8A_n1A DC_40A_n1A |
| DC_7A-8A-40A_n78A DC_7A-8A-40C_n78A | DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_7A-8A-40A_n78(2A) DC_7A-8A-40C_n78(2A) | DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_7A-8A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A |
| DC_7A-12A_n2A-n78A | DC_7A_n2A DC_12A_n2A DC_7A_n78A DC_12A_n78A |
| DC_7A-12A-66A_n2A | DC_7A_n2A DC_12A_n2A DC_66A_n2A |
| DC_7A-12A-66A_n78A | DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_7A-12A_n66A-n78A | DC_7A_n66A DC_12A_n66A DC_7A_n78A DC_12A_n78A |
| DC_7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7A-7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7C-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7A-13A-66A_n66A DC_7C-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_7A-7A-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_7A-20A_n1A-n75A | DC_3A_n1A DC_7A_n1A |
| DC_7A-20A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |
| DC_7A-20A_n3A-n38A | DC_20A_n3A |
| DC_7A-20A_n3A-n78A | DC_7A_n3A DC_20A_n3A DC_7A_n78A DC_20A_n78A |
| DC_7A-20A_n8A-n78A | DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_7A-20A-28A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |
| DC_7A-20A-28A_n3A DC_7C-20A-28A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_7A-20A_n28A-n78A2,3 | DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_7A-20A-32A_n1A | DC_7A_n1A DC_20A_n1A |
| DC_7A-20A-32A_n3A DC_7C-20A-32A_n3A | DC_7A_n3A DC_20A_n3A |
| DC_7A-20A-32A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A-32A_n28A | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A-32A_n78A | DC_7A_n78A DC_20A_n78A |
| DC_7A-20A-38A_n1A | DC_20A_n1A |
| DC_7A-20A-38A_n3A | DC_20A_n3A |
| DC_7A-20A-38A_n8A | DC_20A_n8A |
| DC_7A-20A-38A_n78A10 | DC_20A_n78A |
| DC_7A-20A_n38A-n78A10 | DC_20A_n78A |
| DC_7A-25A-66A_n77A DC_7C-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-7A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-25A-25A-66A_n77A DC_7C-25A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-7A-25A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-25A-66A_n78A DC_7C-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-7A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-25A-25A-66A_n78A DC_7C-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-7A-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-28A_n1A-n40A | DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A |
| DC_7A-28A_n1A-n78A | DC_7A_n1A DC_28A_n1A DC_7A_n78A DC_28A_n78A |
| DC_7A-28A_n3A-n78A | DC_7A_n3A DC_28A_n3A DC_7A_n78A DC_28A_n78A |
| DC_7C-28A_n3A-n78A | DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_7A-28A_n5A-n40A | DC_7A_n5A DC_7A_n40A DC_28A_n5A DC_28A_n40A |
| DC_7A-28A_n5A-n78A DC_7C-28A_n5A-n78A | DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_7A-28A_n7A-n78A | DC_7A_n7A4 DC_28A_n7A DC_7A_n78A DC_28A_n78A |
| DC_7A-28A-32A_n1A | DC_7A_n1A DC_28A_n1A |
| DC_7A-28A-32A_n3A DC_7C-28A-32A_n3A | DC_7A_n3A DC_28A_n3A |
| DC_7A-28A-38A_n1A | DC_28A_n1A |
| DC_7A-28A-38A_n78A DC_7C-28A-38A_n78A | DC_28A_n78A |
| DC_7A-28A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_7A-66A_n38A-n78A DC_7C-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_7A-7A-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_7A-28A-66A_n7A | DC_7A_n7A4 DC_28A_n7A DC_66A_n7A |
| DC_7A-28A-66A_n66A DC_7C-28A-66A_n66A | DC_7A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_7A-29A-66A_n78A DC_7C-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-32A_n1A-n78A | DC_7A_n1A DC_7A_n78A |
| DC_7A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-66A_n2A-n71A | DC_7A_n2A DC_7A_n71A DC_66A_n2A DC_66A_n71A |
| DC_7A-66A_n2A-n78A | DC_7A_n2A DC_66A_n2A DC_7A_n78A DC_66A_n78A |
| DC_7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7A-7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7C-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7A-66A_n66A-n71A | DC_7A_n66A DC_7A_n71A DC_66A_n66A4 DC_66A_n71A |
| DC_7A-66A_n66A-n77A DC_7C-66A_n66A-n77A DC_7A-7A-66A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_66A_n77A |
| DC_7A-66A_n66A-n78A DC_7C-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_7A-7A-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |
| DC_7A-66A-71A_n78A | DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_7A-66A_n71A-n78A | DC_7A_n71A DC_66A_n71A DC_7A_n78A DC_66A_n78A |
| DC_7A-71A_n2A-n66A | DC_7A_n2A DC_7A_n66A DC_71A_n2A DC_71A_n66A |
| DC_7A-71A_n2A-n78A | DC_7A_n2A DC_71A_n2A DC_7A_n78A DC_71A_n78A |
| DC_7A-71A_n66A-n78A | DC_7A_n66A DC_71A_n66A DC_7A_n78A DC_71A_n78A |
| DC_8A_n1A-n3A-n77A | DC_8A_n1A DC_8A_n3A DC_8A_n77A |
| DC_8A_n3A-n28A-n77A2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_8A_n3A-n28A-n77(2A) 2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_8A_n3A-n28A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n79A |
| DC_8A_n3A-n77A-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_8A_n3A-n77(2A)-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_8A-11A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A |
| DC_8A-11A_n1A-n77(2A) | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A |
| DC_8A-11A_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A |
| DC_8A-11A_n3A-n77A2 | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_8A-11A_n3A-n77(2A) 2 | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_8A-11A_n3A-n79A | DC_8A_n3A DC_8A_n79A DC_11A_n3A DC_11A_n79A |
| DC_8A-11A_n28A-n77A2 | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n28A-n77(2A) 2 | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n77A-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_8A-11A_n77(2A)-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_8A-20A-28A_n3A | DC_8A_n3A DC_20A_n3A DC_28A_n3A |
| DC_8A-20A-28A_n78A | DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_8A-20A-32A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_8A-20A-32A_n3A | DC_8A_n3A DC_20A_n3A |
| DC_8A_n28A-n77A-n79A | DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_8A-20A-38A_n1A | DC_8A_n1A DC_20A_n1A DC_38A_n1A |
| DC_8A-32A-38A_n1A | DC_8A_n1A DC_38A_n1A |
| DC_8A_n39A-n40A-n41A | DC_8A_n39A DC_8A_n40A DC_8A_n41A |
| DC_8A_n39A-n40A-n79A | DC_8A_n39A DC_8A_n40A DC_8A_n79A |
| DC_8A_n39A-n41A-n79A | DC_8A_n39A DC_8A_n41A DC_8A_n79A |
| DC_8A_n40A-n41A-n79A | DC_8A_n40A DC_8A_n41A DC_8A_n79A |
| DC_8A-41A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_41A_n1A DC_41A_n3A |
| DC_8A-41C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_41A_n1A DC_41A_n3A |
| DC_8A-41A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A |
| DC_8A-41C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A |
| DC_8A-40A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_8A-40C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_8A-41A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_8A-41C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_8A-41A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41A_n77A |
| DC_8A-41C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41C_n3A DC_41A_n77A DC_41C_n77A |
| DC_8A-42A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42A_n3A |
| DC_8A-42C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42C_n1A DC_42A_n3A DC_42C_n3A |
| DC_8A-42A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A |
| DC_8A-42C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A DC_42C_n1A |
| DC_8A-42A_n3A-n28A2 | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A |
| DC_8A-42C_n3A-n28A2 | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_8A-42A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A |
| DC_8A-42A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A |
| DC_8A-42C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A |
| DC_8A-42C_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A |
| DC_8A-42A_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_8A-42A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_8A-42C_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_8A-42C_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_12A-30A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_12A-30A-66A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_11A_n3A-n28A-n77A2 | DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_11A_n3A-n28A-n77(2A) 2 | DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_11A_n3A-n77A-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_11A_n3A-n77(2A)-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_12A-30A-66A_n66A | DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_12A-30A-66A_n77A9 DC_12A-30A-66A-66A_n77A9 | DC_12A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_12A-30A-66A_n77(2A) 9 | DC_12A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_12A-48A-(n)5AA | DC_12A_n5A DC_48A_n5A DC_(n)5AA4 |
| DC_12A-48A-66A_n5A | DC_12A_n5A DC_48A_n5A DC_66A_n5A |
| DC_12A-66A-(n)5AA | DC_12A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_12A-66A_n2A-n78A | DC_12A_n2A DC_66A_n2A DC_12A_n78A DC_66A_n78A |
| DC_13A-48A-66A_n77A9 DC_13A-48C-66A_n77A9 DC_13A-48A-66A_n77C9 DC_13A-48C-66A_n77C9 | DC_13A_n77A9 DC_66A_n77A9 |
| DC_13A-66A_n2A-n77A9 DC_13A-66A-66A_n2A-n77A9 DC_13A-66A_n2A-n77C9 | DC_13A_n2A DC_13A_n77A9 DC_66A_n2A DC_66A_n77A9 |
| DC_13A-66A_n5A-n48A | DC_13A_n48A DC_66A_n5A DC_66A_n48A |
| DC_13A-66A_n5A-n77A9 DC_13A-66A-66A_n5A-n77A9 DC_13A-66A_n5A-n77C9 DC_13A-66A-66A_n5A-n77C9 | DC_66A_n5A DC_13A_n77A DC_66A_n77A |
| DC_13A-66A_n66A-n77A9 DC_13A-66A_n66A-n77C | DC_13A_n66A DC_13A_n77A9 DC_66A_n77A9 |
| DC_14A-30A-66A_n2A | DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_14A-30A-66A-66A_n2A | DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_14A-30A-66A_n66A | DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_14A-30A-66A_n77A9 DC_14A-30A-66A-66A_n77A9 | DC_14A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_14A-30A-66A_n77(2A) 9 | DC_14A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_18A-41A_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A |
| DC_18A-41C_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_18A-41A_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A |
| DC_18A-41C_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_19A_n1A-n77A-n79A | DC_19A_n1A DC_19A_n77A DC_19A_n79A |
| DC_19A_n1A-n78A-n79A | DC_19A_n1A DC_19A_n78A DC_19A_n79A |
| DC_19A-21A_n1A-n77A2 | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A |
| DC_19A-21A_n1A-n78A2 | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A |
| DC_19A-21A_n1A-n79A2 | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A |
| DC_19A-21A-42A_n1A2 DC_19A-21A-42C_n1A2 | DC_19A_n1A DC_21A_n1A DC_42A_n1A |
| DC_19A-21A-42A_n77A DC_19A-21A-42A_n77C DC_19A-21A-42C_n77A DC_19A-21A-42C_n77C | DC_19A_n77A DC_21A_n77A |
| DC_19A-21A-42A_n78A DC_19A-21A-42A_n78C DC_19A-21A-42C_n78A DC_19A-21A-42C_n78C | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A-42A_n79A DC_19A-21A-42A_n79C DC_19A-21A-42C_n79A DC_19A-21A-42C_n79C | DC_19A_n79A DC_21A_n79A |
| DC_19A-21A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A-21A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_19A-42A_n1A-n77A DC_19A-42C_n1A-n77A | DC_19A_n1A DC_19A_n77A |
| DC_19A-42A_n1A-n78A DC_19A-42C_n1A-n78A | DC_19A_n1A DC_19A_n78A |
| DC_19A-42A_n1A-n79A DC_19A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A |
| DC_19A-42A_n77A-n79A DC_19A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A-42A_n78A-n79A DC_19A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_20A-(n)3AA-n67A | DC_(n)3AA4 DC_20A_n3A |
| DC_20A-28A-32A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_20A-28A-32A_n3A | DC_20A_n3A DC_28A_n3A |
| DC_20A-28A-38A_n1A | DC_20A_n1A DC_28A_n1A DC_38A_n1A |
| DC_20A-32A_n1A-n28A | DC_20A_n1A DC_20A_n28A |
| DC_20A-32A-38A_n1A | DC_20A_n1A DC_38A_n1A |
| DC_20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A DC_38A_n3A DC_38A_n78A |
| DC_20A-41A_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_20A-41C_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_20A-67A-(n)3AA | DC_(n)3AA4 DC_20A_n3A |
| DC_21A_n1A-n77A-n79A | DC_21A_n1A DC_21A_n77A DC_21A_n79A |
| DC_21A_n1A-n78A-n79A | DC_21A_n1A DC_21A_n78A DC_21A_n79A |
| DC_21A-28A-42A_n77A DC_21A-28A-42C_n77A | DC_21A_n77A DC_28A_n77A |
| DC_21A-28A-42A_n78A DC_21A-28A-42C_n78A | DC_21A_n78A DC_28A_n78A |
| DC_21A-28A-42A_n79A DC_21A-28A-42C_n79A | DC_21A_n79A DC_28A_n79A |
| DC_21A_n28A-n77A-n79A | DC_21A_n28A DC_21A_n77A DC_21A_n79A |
| DC_21A_n28A-n78A-n79A | DC_21A_n28A DC_21A_n78A DC_21A_n79A |
| DC_21A-42A_n1A-n77A DC_21A-42C_n1A-n77A | DC_21A_n1A DC_21A_n77A |
| DC_21A-42A_n1A-n78A DC_21A-42C_n1A-n78A | DC_21A_n1A DC_21A_n78A |
| DC_21A-42A_n1A-n79A DC_21A-42C_n1A-n79A | DC_21A_n1A DC_21A_n79A |
| DC_21A-42A_n77A-n79A DC_21A-42C_n77A-n79A | DC_21A_n77A DC_21A_n79A |
| DC_21A-42A_n78A-n79A DC_21A-42C_n78A-n79A | DC_21A_n78A DC_21A_n79A |
| DC_28A_n5A-n40A-n78A | DC_28A_n5A DC_28A_n40A DC_28A_n78A |
| DC_28A-32A-38A_n1A | DC_28A_n1A DC_38A_n1A |
| DC_28A-41A-42A_n78A DC_28A-41C-42A_n78A DC_28A-41A-42C_n78A DC_28A-41C-42C_n78A | DC_28A_n78A DC_41A_n78A DC_41C_n78A |
| DC_29A-30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_29A-30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_29A-30A-66A_n66A | DC_30A_n66A DC_66A_n66A4 |
| DC_29A-30A-66A_n77A9 | DC_30A_n77A9 DC_66A_n77A9 |
| DC_30A-66A-(n)5AA | DC_30A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_42A_n1A-n77A-n79A7,8 | N/A |
| DC_42A_n1A-n78A-n79A7,8 | N/A |
| DC_42A_n3A-n28A-n77A7,8 | DC_42A_n3A DC_42A_n28A |
| DC_42A_n3A-n28A-n77(2A)7,8 | DC_42A_n3A DC_42A_n28A |
| DC_42C_n3A-n28A-n77A7,8 | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_42C_n3A-n28A-n77(2A)7,8 | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_46A-66A_n25A-n41A DC_46C-66A_n25A-n41A DC_46D-66A_n25A-n41A | DC_66A_n25A DC_66A_n41A |
| DC_46A-66A_n25A-n71A DC_46C-66A_n25A-n71A DC_46D-66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_46A-66A_n41A-n71A DC_46C-66A_n41A-n71A DC_46D-66A_n41A-n71A | DC_66A_n41A DC_66A_n71A |
| DC_46A-66A_n41(2A)-n71A DC_46C-66A_n41(2A)-n71A DC_46D-66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| DC_48A-66A_n25A-n48A | DC_48A_n25A DC_66A_n25A DC_66A_n48A |
| DC_66A-71A_n2A-n78A | DC_66A_n2A DC_71A_n2A DC_66A_n78A DC_71A_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: UL carrier shall be supported in Band 2 or band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 6: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 8: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 9: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 10: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 11: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 12: Void. NOTE 13: Power imbalance between downlink carriers on Band 7 and band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 15: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 16: UL carrier shall be supported in Band 1 or band 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 17: UL carrier shall be supported in Band 3 or band 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

#### 5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)

Table 5.5B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_5A_n40A DC_7A_n40A |
| DC_1A-3A-5A-7A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_5A_n40A DC_7A_n40A |
| DC_1A-3A-5A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A_n77(2A) DC_1A-3A-5A-7A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A-7A_n77(2A) DC_1A-3A-5A-7A-7A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A_n78A DC_1A-3C-5A-7A_n78A DC_1A-3A-5A-7A_n78C | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A-7A_n78A DC_1A-3A-5A-7A-7A_n78C | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A-7A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-1A-3A-5A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-3A-5A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-3A-5A_n40A-n78A DC_1A-3A-5A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A |
| DC_1A-3A-5A-41A_n79A | DC_1A_n79A DC_3A_n79A DC_5A_n79A DC_41A_n79A |
| DC_1A-3A-7A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7C_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_3A_n5A DC_3A_n40A DC_7A_n5A DC_7A_n40A |
| DC_1A-3A-7A_n5A-n78A DC_1A-3C-7A_n5A-n78A DC_1A-3A-7C_n5A-n78A DC_1A-3C-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3C-7A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_1A-3A-7A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_8A_n28A |
| DC_1A-3A-7A-8A_n78A DC_1A-3C-7A-8A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A-8A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_1A-3A-7A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-3A-7A-20A_n28A3 | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-3A-7A-20A_n78A2 DC_1A-3A-7A-20A_n78C2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A-20A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A-26A_n78A DC_1A-3C-7A-26A_n78A DC_1A-3A-7C-26A_n78A DC_1A-3C-7C-26A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-3A-7A-26A_n78(2A) DC_1A-3A-7C-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-3C-7A-26A_n78(2A) DC_1A-3C-7C-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-3A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-3C-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-3A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-3C-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A-28A_n3A DC_1A-3A-7C-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_1A-3A-7A-28A_n5A DC_1A-3C-7A-28A_n5A DC_1A-3A-7C-28A_n5A DC_1A-3C-7C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_1A-3A-7A-28A_n7A DC_1A-3C-7A-28A_n7A DC_1A-3A-3A-7A-28A_n7A DC_1A-1A-3C-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-3A-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-3A-3A-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-3A-7A-28A_n38A | 1A7 3A7 28A7 |
| DC_1A-3A-7A_n28A-n38A | DC_1A_n28A7 DC_3A_n28A7 |
| DC_1A-3A-7A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A DC_28A_n40A |
| DC_1A-3A-7A-28A_n78A DC_1A-3A-7C-28A_n78A DC_1A-3C-7A-28A_n78A DC_1A-3C-7C-28A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-3A-7A-28A_n78(2A) DC_1A-3A-7C-28A_n78(2A) DC_1A-3C-7A-28A_n78(2A) DC_1A-3C-7C-28A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-1A-3A-7A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-7A_n28A-n78A2 DC_1A-3A-7C_n28A-n78A DC_1A-3C-7A_n28A-n78A DC_1A-3C-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_1A-3A-7A-32A_n28A DC_1A-3C-7A-32A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A |
| DC_1A-3A-7A-32A_n78A DC_1A-3C-7A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_1A-3A-7A-38A_n28A7 DC_1A-3C-7A-38A_n28A7 | DC_1A_n28A DC_3A_n28A |
| DC_1A-3A-7A-38A_n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-7A_n38A-n78A7 | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-7A-40A_n78A DC_1A-3A-7A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-3A-7A-40A_n78(2A) DC_1A-3A-7A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-3A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-7A_n75A-n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-8A-40A_n78A DC_1A-3A-8A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-8A-40A_n78(2A) DC_1A-3A-8A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-7A_n40A-n78A DC_1A-3A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-3A-8A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A DC_11A_n28A |
| DC_1A-3A-8A-11A_n77A2 DC_1A-3A-8A-11A_n77(2A) 2 DC_1A-3A-8A-11A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-3A-8A-20A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-3A-8A_n28A-n77A2 DC_1A-3A-8A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-3A-8A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_28A_n78A |
| DC_1A-3A-8A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-3A-8A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A-8A-42A_n77A DC_1A-3A-8A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_8A_n77A |
| DC_1A-3A-8A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A |
| DC_1A-3A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_18A_n3A DC_18A_n41A |
| DC_1A-3A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A DC_18A_n3A DC_18A_n77A |
| DC_1A-3A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A DC_18A_n3A DC_18A_n78A |
| DC_1A-3A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |
| DC_1A-3A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A1 DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-3A-18A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A1 DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-3A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A1 DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-3A-18A_n28A-n78(2A) | DC_1A_n28A DC_1A_n78A DC_3A_n28A1 DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-3A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-3A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-3A-18A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_1A-3A-18A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_1A-3A-18A-42A_n77A DC_1A-3A-18A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A-42A_n78A DC_1A-3A-18A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A-42A_n79A DC_1A-3A-18A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-19A-21A_n77A2 DC_1A-3A-19A-21A_n77C2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-3A-19A-21A_n78A2 DC_1A-3A-19A-21A_n78C2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-3A-19A-21A_n79A2 DC_1A-3A-19A-21A_n79C2 | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-3A-19A-42A_n77A5,6 DC_1A-3A-19A-42A_n77C5,6 DC_1A-3A-19A-42C_n77A5,6 DC_1A-3A-19A-42C_n77C5,6 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A-42A_n78A5,6 DC_1A-3A-19A-42A_n78C5,6 DC_1A-3A-19A-42C_n78A5,6 DC_1A-3A-19A-42C_n78C5,6 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A-42A_n79A DC_1A-3A-19A-42A_n79C DC_1A-3A-19A-42C_n79A DC_1A-3A-19A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A |
| DC_1A-3A-20A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_20A_n7A DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-3A-20A_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3C-20A_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3A-20A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-3A-20A_n28A-n78A2,3,6,11 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-3A-20A-32A_n28A6,11 DC_1A-3C-20A-32A_n28A,6,11 | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3A-20A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A-38A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_1A_n38A DC_3A_n38A DC_20A_n38A |
| DC_1A-3A-20A-38A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A-40A_n78A DC_1A-3A-20A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_1A-3A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |
| DC_1A-3A-21A-42A_n77A5,6 DC_1A-3A-21A-42A_n77C5,6 DC_1A-3A-21A-42C_n77A5,6 DC_1A-3A-21A-42C_n77C5,6 | DC_1A_n77A DC_3A_n77A DC_21A_n77A |
| DC_1A-3A-21A-42A_n78A5,6 DC_1A-3A-21A-42A_n78C5,6 DC_1A-3A-21A-42C_n78A5,6 DC_1A-3A-21A-42C_n78C5,6 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A-42A_n79A DC_1A-3A-21A-42A_n79C DC_1A-3A-21A-42C_n79A DC_1A-3A-21A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_21A_n79A |
| DC_1A-3A-21A_n77A-n79A | DC_3A_n77A DC_3A_n79A |
| DC_1A-3A-21A_n78A-n79A | DC_3A_n78A DC_3A_n79A |
| DC_1A-3A-28A_n3A-n78A2 | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n5A-n78A2 DC_1A-3C-28A_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-3A-28A-(n)7AA DC_1A-3C-28A-(n)7AA | DC_1A_n7A DC_3A_n7A DC_28A_n7A |
| DC_1A-3A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n7B-n78A | DC_1A_n7A DC_3A_n7A DC_28A_n7A DC_1A_n7B DC_3A_n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3C-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_1A-3C-28A_n7B-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_1A_n7B DC_3A_n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_1A-3A-28A-40A_n78A DC_1A-3A-28A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A DC_40A_n78A |
| DC_1A-3A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-3A-28A-42A_n77A DC_1A-3A-28A-42A_n77C DC_1A-3A-28A-42C_n77A DC_1A-3A-28A-42C_n77C | DC_1A_n77A DC_3A_n77A DC_28A_n77A |
| DC_1A-3A-28A-42A_n78A DC_1A-3A-28A-42A_n78C DC_1A-3A-28A-42C_n78A DC_1A-3A-28A-42C_n78C | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A-42A_n79A DC_1A-3A-28A-42A_n79C DC_1A-3A-28A-42C_n79A DC_1A-3A-28A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_28A_n79A |
| DC_1A-3A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_3A_n28A DC_3A_n77A DC_3A_n79A |
| DC_1A_n3A-n28A-n77A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_1A_n79A |
| DC_1A-3A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A DC_3A_n28A DC_3A_n78A DC_3A_n79A |
| DC_1A-3A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A-3C-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_1A-3A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A |
| DC_1A-3A-41C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_1A-3A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A |
| DC_1A-3A-41C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_1A-3A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_41A_n28A |
| DC_1A-3A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A |
| DC_1A-3A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_1A-3A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |
| DC_1A-3A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_1A-3A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41A-42A_n77A5,6 DC_1A-3A-41A-42C_n77A5,6 DC_1A-3A-41C-42A_n77A5,6 DC_1A-3A-41C-42C_n77A5,6 | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A-42A_n77(2A)5,6 DC_1A-3A-41A-42C_n77(2A)5,6 | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A-42A_n78A5,6 DC_1A-3A-41A-42C_n78A5,6 DC_1A-3A-41C-42A_n78A5,6 DC_1A-3A-41C-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41A-42A_n79A DC_1A-3A-41A-42C_n79A DC_1A-3A-41C-42A_n79A DC_1A-3A-41C-42C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-42A_n28A-n77A5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_1A-3A-42A_n28A-n77(2A)5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_1A-3A-42C_n28A-n77A5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-3A-42C_n28A-n77(2A)5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-5A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-5A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-5A-7A_n40A-n78A DC_1A-5A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-7A-8A-20A_n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A DC_20A_n3A |
| DC_1A-7A-8A-20A_n28A3,9 | DC_1A_n28A DC_7A_n28A DC_8A_n28A DC_20A_n28A |
| DC_1A-7A-8A-20A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-7A-8A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-7A-8A-32A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A-8A-40A_n78A DC_1A-7A-8A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-7A-8A-40A_n78(2A) DC_1A-7A-8A-40C_n78(2A) | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-7A-20A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_20A_n3A DC_20A_n78A |
| DC_1A-7A-20A_n3A-n38A | DC_1A_n3A DC_20A_n3A |
| DC_1A-7A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-7A-20A-28A_n3A DC_1A-7C-20A-28A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-7A-20A_n28A-n78A2,3 | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-7A-20A-32A_n3A DC_1A-7C-20A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A |
| DC_1A-7A-20A-32A_n28A | DC_1A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-7A-20A-32A_n78A | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-20A-32A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-7A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-7A-20A-38A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-7A-20A-38A_n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-7A-20A_n38A-n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-7A-28A_n3A-n78A DC_1A-7C-28A_n3A-n78A | DC_1A_n3A DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A-28A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_7A_n5A DC_7A_n40A DC_28A_n5A DC_28A_n40A |
| DC_1A-7A-28A_n5A-n78A DC_1A-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-7A-28A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A-28A-32A_n3A DC_1A-7C-28A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_28A_n3A |
| DC_1A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-7A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A |
| DC_1A-8A-(n)3AA-n77A DC_1A-8A-(n)3AA-n77(2A) | DC_1A_n3A DC_1A_n77A DC_(n)3AA4 DC_3A_n77A DC_8A_n3A DC_8A_n77A |
| DC_1A-8A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A DC_8A_n3A DC_8A_n28A DC_8A_n79A |
| DC_1A-8A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A |
| DC_1A-8A-11A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n77(2A) 2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A DC_11A_n3A DC_11A_n79A |
| DC_1A-8A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-8A-11A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-8A-20A-28A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-8A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A-42A_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42C_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A |
| DC_1A-8A-42A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A |
| DC_1A-8A-42C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-8A-42C_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-8A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_1A-8A-42A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_1A-8A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-11A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_1A-11A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_1A-19A-21A-42A_n77A5,6 DC_1A-19A-21A-42A_n77C5,6 DC_1A-19A-21A-42C_n77A5,6 DC_1A-19A-21A-42C_n77C5,6 | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A-42A_n78A5,6 DC_1A-19A-21A-42A_n78C5,6 DC_1A-19A-21A-42C_n78A5,6 DC_1A-19A-21A-42C_n78C5,6 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A-42A_n79A DC_1A-19A-21A-42A_n79C DC_1A-19A-21A-42C_n79A DC_1A-19A-21A-42C_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-19A-42A_n77A-n79A5,6 DC_1A-19A-42C_n77A-n79A5,6 | DC_19A_n77A DC_19A_n79A |
| DC_1A-19A-42A_n78A-n79A5,6 DC_1A-19A-42C_n78A-n79A5,6 | DC_1A_n78A DC_1A_n79A DC_19A_n78A DC_19A_n79A |
| DC_1A-20A-28A-32A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_38A_n3A DC_1A_n78A DC_20A_n78A DC_38A_n78A |
| DC_1A-21A-28A-42A_n77A5,6 DC_1A-21A-28A-42C_n77A5,6 | DC_1A_n77A DC_21A_n77A DC_28A_n77A |
| DC_1A-21A-28A-42A_n78A5,6 DC_1A-21A-28A-42C_n78A5,6 | DC_1A_n78A DC_21A_n78A DC_28A_n78A |
| DC_1A-21A-28A-42A_n79A DC_1A-21A-28A-42C_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A |
| DC_1A-21A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_21A_n28A DC_21A_n77A DC_21A_n79A |
| DC_1A-21A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A DC_21A_n28A DC_21A_n78A DC_21A_n79A |
| DC_1A-21A-42A_n77A-n79A5,6 DC_1A-21A-42C_n77A-n79A5,6 | DC_1A_n77A DC_1A_n79A |
| DC_1A-21A-42A_n78A-n79A5,6 DC_1A-21A-42C_n78A-n79A5,6 | DC_1A_n78A DC_1A_n79A DC_21A_n78A DC_21A_n79A |
| DC_1A-42A_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-42A_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-42C_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-42C_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_2A-5A-7A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_5A_n2A DC_5A_n78A DC_7A_n2A DC_7A_n78A |
| DC_2A-5A-7A-66A_n2A | DC_5A_n2A DC_7A_n2A DC_66A_n2A |
| DC_2A-5A-7A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-5A-7A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-5A-7A-66A_n66A DC_2A-5A-7C-66A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-5A-7A-7A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-5A-7A-66A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-5A-7A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_5A_n66A DC_5A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-5A-30A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-5A-30A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-5A-30A-66A_n77A8 | DC_2A_n77A8 DC_5A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-5A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-5A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-5A-66A_n2A-n77A8 DC_2A-5A-66A-66A_n2A-n77A8 | DC_5A_n2A DC_2A_n77A8 DC_5A_n77A8 DC_66A_n2A DC_66A_n77A8 |
| DC_2A-5A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_5A_n2A DC_5A_n78A DC_66A_n2A DC_66A_n78A |
| DC_2A-5A-66A_n66A-n77A8 | DC_2A_n66A DC_2A_n77A8 DC_5A_n66A DC_5A_n77A8 DC_66A_n77A8 |
| DC_2A-7A-12A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_7A_n2A DC_7A_n78A DC_12A_n2A DC_12A_n78A |
| DC_2A-7A-12A-66A_n2A | DC_7A_n2A DC_12A_n2A DC_66A_n2A |
| DC_2A-7A-12A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-2A-7A-12A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-7A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-7C-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-7A-13A-66A_n66A DC_2A-7C-13A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-7A-7A-13A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-7A-28A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A DC_66A_n7A |
| DC_2A-7A-28A-66A_n66A DC_2A-7C-28A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_2A-7A-29A-66A_n78A DC_2A-7C-29A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-29A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_7A_n2A DC_7A_n78A DC_66A_n2A DC_66A_n78A |
| DC_2A-7A-66A_n25A-n66A6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_2A-7A-7A-66A_n25A-n66A6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_2A-7C-66A_n25A-n66A,6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_2A-7A-66A_n66A-n77A DC_2A-7A-7A-66A_n66A-n77A DC_2A-7C-66A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-66A_n66A-n78A DC_2A-7C-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_2A-7A-7A-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_2A-7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |
| DC_2A-7A-66A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-2A-7A-66A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-7A-66A_n71A-n78A | DC_2A_n71A DC_2A_n78A DC_7A_n71A DC_7A_n78A DC_66A_n71A DC_66A_n78A |
| DC_2A-7A-71A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_7A_n2A DC_7A_n78A DC_71A_n2A DC_71A_n78A |
| DC_2A-7A-71A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_71A_n66A DC_71A_n78A |
| DC_2A-12A-30A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-12A-30A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-12A-30A-66A_n77A8 | DC_2A_n77A8 DC_12A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-13A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-12A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_12A_n2A DC_12A_n78A DC_66A_n2A DC_66A_n78A |
| DC_2A-13A-66A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-13A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-13A-66A_n66A-n77A8 DC_2A-2A-13A-66A_n66A-n77A8 | DC_2A_n66A DC_2A_n77A8 DC_13A_n66A DC_13A_n77A8 DC_66A_n77A8 |
| DC_2A-14A-30A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-14A-30A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-14A-30A-66A_n77A8 | DC_2A_n77A8 DC_14A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-29A-30A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-29A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-29A-30A-66A_n77A8 | DC_2A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-30A-66A-(n)5AA | DC_2A_n5A DC_30A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_2A-46A-66A_n41A-n71A DC_2A-46C-66A_n41A-n71A DC_2A-46D-66A_n41A-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A-71A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_66A_n2A DC_66A_n78A DC_71A_n2A DC_71A_n78A |
| DC_3A-5A-7A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-5A-7A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-5A-7A_n40A-n78A DC_3A-5A-7A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_3A-7A-8A_n1A-n40A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_3A_n40A DC_7A_n40A DC_8A_n40A |
| DC_3A-7A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-3A-7A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-7A-7A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-3A-7A-7A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-7A-8A-20A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_20A_n1A |
| DC_3A-7A-8A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_3A-7A-8A-32A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-8A-32A_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_3A-7A-8A-40A_n1A DC_3A-7A-8A-40C_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_40A_n1A |
| DC_3A-7A-8A-40A_n78A DC_3A-7A-8A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-7A-8A-40A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-7A-8A-40C_n78(2A) | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-7A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A |
| DC_3A-7A-20A_n1A-n75A | DC_3A_n1A DC_7A_n1A DC_20A_n1A |
| DC_3A-7A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |
| DC_3C-7A-20A_n1A-n78A | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |
| DC_3A-7A-20A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_3A-7A-20A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_20A_n1A DC_28A_n1A |
| DC_3A-7A-20A_n28A-n78A2,3,6,11 DC_3C-7A-20A_n28A-n78A2,3,6,11 | DC_3A_n28A DC_3A_n78A DC_3C_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-7A-20A-32A_n1A | DC_3A_n1A DC_7A_n1A DC_20A_n1A |
| DC_3A-7A-20A-32A_n78A | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-20A_n38A-n78A DC_3A-7A-20A-38A_n78A DC_3C-7A-20A-38A_n78A | DC_3A_n78A DC_3C_n78A DC_20A_n78A |
| DC_3A-7A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A |
| DC_3A-7A-28A_n1A-n78A | DC_3A_n1A DC_7A_n1A DC_28A_n1A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7C-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_3A-7A-28A_n5A-n40A | DC_3A_n5A DC_3A_n40A DC_7A_n5A DC_7A_n40A DC_28A_n5A DC_28A_n40A |
| DC_3A-7A-28A_n5A-n78A DC_3C-7A-28A_n5A-n78A DC_3A-7C-28A_n5A-n78A DC_3C-7C-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_3A-7A-28A_n7A-n78A | DC_3A_n7A DC_7A_n7A4 DC_28A_n7A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3C-7A-28A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_3A-7A-32A_n1A-n78A | DC_3A_n1A DC_7A_n1A DC_3A_n78A DC_7A_n78A |
| DC_3C-7A-32A_n1A-n78A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-7A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-8A-11A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-8A-11A_n28A-n77(2A) 2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-8A-20A-28A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_3A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-8A-41A_n1A-n78A DC_3A-3A-8A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-8A-41C_n1A-n78A DC_3A-3A-8A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-19A-21A-42A_n77A5,6 DC_3A-19A-21A-42A_n77C5,6 DC_3A-19A-21A-42C_n77A5,6 DC_3A-19A-21A-42C_n77C5,6 | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A-42A_n78A5,6 DC_3A-19A-21A-42A_n78C5,6 DC_3A-19A-21A-42C_n78A5,6 DC_3A-19A-21A-42C_n78C5,6 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A-42A_n79A DC_3A-19A-21A-42A_n79C DC_3A-19A-21A-42C_n79A DC_3A-19A-21A-42C_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-42A_n1A-n77A5,6 DC_3A-19A-42C_n1A-n77A5,6 | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A |
| DC_3A-19A-42A_n1A-n78A5,6 DC_3A-19A-42C_n1A-n78A5,6 | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A |
| DC_3A-19A-42A_n1A-n79A DC_3A-19A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A |
| DC_3A-20A_n1A-n28A-n75A | DC_3A_n1A DC_20A_n1A DC_3A_n28A DC_20A_n28A |
| DC_3A-20A-32A_n1A-n28A | DC_3A_n1A DC_20A_n1A DC_3A_n28A DC_20A_n28A |
| DC_3C-20A-32A_n1A-n28A | DC_3A_n1A DC_3C_n1A DC_20A_n1A DC_3A_n28A DC_20A_n28A |
| DC_3A-21A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A DC_21A_n1A DC_21A_n77A DC_21A_n79A |
| DC_3A-21A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A DC_21A_n1A DC_21A_n78A DC_21A_n79A |
| DC_3A-21A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A DC_21A_n28A DC_21A_n77A DC_21A_n79A |
| DC_3A-7A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_3A-3A-7A_n1A-n8A-n78A2 DC_3A-7A-7A_n1A-n8A-n78A2 DC_3A-3A-7A-7A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_3A-20A-41A_n1A-n78A DC_3A-3A-20A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-20A-41C_n1A-n78A DC_3A-3A-20A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-21A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A DC_21A_n28A DC_21A_n78A DC_21A_n79A |
| DC_3A-21A-42A_n1A-n77A5,6 DC_3A-21A-42C_n1A-n77A5,6 | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A |
| DC_3A-21A-42A_n1A-n78A5,6 DC_3A-21A-42C_n1A-n78A5,6 | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A |
| DC_3A-21A-42A_n1A-n79A DC_3A-21A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A |
| DC_3A-28A-41A-42A_n78A5,6 DC_3A-28A-41A-42C_n78A5,6 DC_3A-28A-41C-42A_n78A5,6 DC_3A-28A-41C-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_5A-7A-66A_n2A-n78A | DC_5A_n2A DC_5A_n78A DC_7A_n2A DC_7A_n78A DC_66A_n2A DC_66A_n78A |
| DC_7A-8A-20A-32A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |
| DC_7A-8A-20A-38A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_7A-8A-32A-38A_n1A | DC_8A_n1A |
| DC_7A-8A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-8A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-12A-66A_n2A-n78A | DC_7A_n2A DC_7A_n78A DC_12A_n2A DC_12A_n78A DC_66A_n2A DC_66A_n78A |
| DC_7A-20A-28A-32A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |
| DC_7A-20A-28A-32A_n3A DC_7C-20A-28A-32A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_7A-20A-28A-38A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_7A-20A-32A-38A_n1A | DC_20A_n1A |
| DC_7A-20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A |
| DC_7A-28A-32A-38A_n1A | DC_28A_n1A |
| DC_7A-66A-71A_n2A-n78A | DC_7A_n2A DC_7A_n78A DC_66A_n2A DC_66A_n78A DC_71A_n2A DC_71A_n78A |
| DC_8A_n3A-n28A-n77A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_8A-11A_n3A-n28A-n77A2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n3A-n28A-n77(2A) 2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n3A-n77A-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_8A-20A-32A-38A_n1A | DC_8A_n1A DC_20A_n1A DC_38A_n1A |
| DC_8A-42A_n3A-n28A-n77A5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_8A-42A_n3A-n28A-n77(2A)5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_8A-42C_n3A-n28A-n77A5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_8A-42C_n3A-n28A-n77(2A)5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_19A-21A-42A_n1A-n77A5,6 DC_19A-21A-42C_n1A-n77A5,6 | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A |
| DC_19A-21A-42A_n1A-n78A5,6 DC_19A-21A-42C_n1A-n78A5,6 | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A |
| DC_19A-21A-42A_n1A-n79A DC_19A-21A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A |
| DC_19A-21A-42A_n77A-n79A5,6 DC_19A-21A-42C_n77A-n79A5,6 | DC_19A_n77A DC_19A_n79A |
| DC_19A-21A-42A_n78A-n79A5,6 DC_19A-21A-42C_n78A-n79A5,6 | DC_19A_n78A DC_19A_n79A |
| DC_19A-42A_n1A-n77A-n79A5,6 | DC_19A_n1A DC_19A_n77A DC_19A_n79A |
| DC_19A-42A_n1A-n78A-n79A5,6 | DC_19A_n1A DC_19A_n78A DC_19A_n79A |
| DC_20A-28A-32A-38A_n1A | DC_20A_n1A DC_28A_n1A DC_38A_n1A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 4: Only single switched UL is supported NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 7: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 8: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 9: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 10: Void. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |

#### 5.5B.4.5	Inter-band EN-DC configurations within FR1 (six bands)

Table 5.5B.4.5-1: Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-5A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-5A-7A_n40A-n78A DC_1A-3A-5A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-3A-7A-8A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-3A-7A-8A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A-8A-40A_n78A DC_1A-3A-7A-8A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-7A-8A-40A_n78(2A) DC_1A-3A-7A-8A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-7A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-3A-7A-20A_n28A-n78A2,3,6,7 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-3A-7A-20A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A-20A-38A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-7A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-7A-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-7C-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-3A-7A-28A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_3A_n5A DC_3A_n40A DC_7A_n5A DC_7A_n40A DC_28A_n5A DC_28A_n40A |
| DC_1A-3A-7A-28A_n5A-n78A DC_1A-3A-7C-28A_n5A-n78A DC_1A-3C-7A-28A_n5A-n78A DC_1A-3C-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-3A-7A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3C-7A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-7A-28A_n38A-n78A | DC_1A_n78A8 DC_3A_n78A8 DC_28A_n78A8 |
| DC_1A-3A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-3A-8A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-8A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-8A-20A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-7A-20A-28A-32A_n3A DC_1A-7C-20A-28A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-7A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A |
| DC_1A-8A_n3A-n28A-n77A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A-11A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-42A_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42A_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42C_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42C_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_2A-5A-7A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_5A_n2A DC_5A_n78A DC_7A_n2A DC_7A_n78A DC_66A_n2A DC_66A_n78A |
| DC_2A-7A-12A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_7A_n2A DC_7A_n78A DC_12A_n2A DC_12A_n78A DC_66A_n2A DC_66A_n78A |
| DC_2A-7A-66A-71A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_7A_n2A DC_7A_n78A DC_66A_n2A DC_66A_n78A DC_71A_n2A DC_71A_n78A |
| DC_3A-7A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-7A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-8A-20A-32A-38A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_7A-20A-28A-32A-38A_n1A | DC_20A_n1A DC_28A_n1A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 8: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

### 5.5B.4a	Inter-band NE-DC within FR1

#### 5.5B.4a.1	Inter-band NE-DC configurations within FR1 (two bands)

Table 5.5B.4a.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_n1A_28A | DC_n1A_28A | No |
| DC_n3A_1A | DC_n3A_1A | DC_n3_1 |
| DC_n3A_8A | DC_n3A_8A | No |
| DC_n8A_1A | DC_n8A_1A | No |
| DC_n8A_3A | DC_n8A_3A | No |
| DC_n28A_3A DC_n28A_3C | DC_n28A_3A | No |
| DC_n28A_8A DC_n28A_8B | DC_n28A_8A | No |
| DC_n28A_20A3 | DC_n28A_20A | No |
| DC_n28A_34A | DC_n28A_34A | No |
| DC_n28A_39A DC_n28A_39C | DC_n28A_39A | No |
| DC_n28A_40A DC_n28A_40C | DC_n28A_40A | No |
| DC_n41A_3A DC_n41A_3C DC_n41C_3A | DC_n41A_3A | No |
| DC_n41A_8A DC_n41A_8B DC_n41C_8A | DC_n41A_8A | No |
| DC_n41A_34A2 | DC_n41A_34A | No |
| DC_n41A_39A2 DC_n41A_39C2 DC_n41C_39A2 | DC_n41A_39A | No |
| DC_n41A_40A2 DC_n41A_40C2 DC_n41C_40A2 | DC_n41A_40A | No |
| DC_n77A_1A DC_n77(2A)_1A | DC_n77A_1A | DC_n77_1 |
| DC_n77A_3A DC_n77(2A)_3A | DC_n77A_3A | DC_n77_3 |
| DC_n77A_8A DC_n77(2A)_8A | DC_n77A_8A | No |
| DC_n78A_1A | DC_n78A_1A | No |
| DC_n78A_3A | DC_n78A_3A | No |
| DC_n78A_3C | DC_n78A_3A | No |
| DC_n78A_5A | DC_n78A_5A | No |
| DC_n78A_7A | DC_n78A_7A | No |
| DC_n78A_7A-7A | DC_n78A_7A | No |
| DC_n78A_8A | DC_n78A_8A | No |
| DC_n78A_26A | DC_n78A_26A | No |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 3: The frequency range in band n28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. |  |  |

#### 5.5B.4a.2	Inter-band NE-DC configurations within FR1 (three bands)

Table 5.5B.4a.2-1: Inter-band NE-DC configurations within FR1 (three bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n3A_1A-8A | DC_n3A_1A DC_n3A-8A |
| DC_n77A_1A-3A | DC_n77A_1A DC_n77A_3A |
| DC_n77(2A)_1A-3A | DC_n77A_1A DC_n77A_3A |
| DC_n77A_1A-8A | DC_n77A_1A DC_n77A_8A |
| DC_n77(2A)_1A-8A | DC_n77A_1A DC_n77A_8A |
| DC_n77A_3A-8A | DC_n77A_3A DC_n77A_8A |
| DC_n77(2A)_3A-8A | DC_n77A_3A DC_n77A_8A |
| DC_n78A_1A-3A DC_n78A_1A-3C | DC_n78A_1A DC_n78A_3A |
| DC_n78A_1A-5A | DC_n78A_1A DC_n78A_5A |
| DC_n78A_1A-7A | DC_n78A_1A DC_n78A_7A |
| DC_n78A_1A-7A-7A | DC_n78A_1A DC_n78A_7A |
| DC_n78A_1A-8A | DC_n78A_1A DC_n78A_8A |
| DC_n78A_3A-5A | DC_n78A_3A DC_n78A_5A |
| DC_n78A_3A-7A | DC_n78A_3A DC_n78A_7A |
| DC_n78A_3A-7A-7A | DC_n78A_3A DC_n78A_7A |
| DC_n78A_3A-8A DC_n78A_3C-8A | DC_n78A_3A DC_n78A_8A |
| DC_n78A_5A-7A | DC_n78A_5A DC_n78A_7A |
| DC_n78A_5A-7A-7A | DC_n78A_5A DC_n78A_7A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. |  |

#### 5.5B.4a.3	Inter-band NE-DC configurations within FR1 (four bands)

Table 5.5B.4a.3-1: Inter-band NE-DC configurations within FR1 (four bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n77A_1A-3A-8A DC_n77(2A)_1A-3A-8A | DC_n77A_1A DC_n77A_3A DC_n77A_8A |
| DC_n78A_1A-3A-5A | DC_n78A_1A DC_n78A_3A DC_n78A_5A |
| DC_n78A_1A-3A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_7A |
| DC_n78A_1A-3A-7A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_7A |
| DC_n78A_1A-3A-8A DC_n78A_1A-3C-8A | DC_n78A_1A DC_n78A_3A DC_n78A_8A |
| DC_n78A_1A-5A-7A | DC_n78A_1A DC_n78A_5A DC_n78A_7A |
| DC_n78A_1A-5A-7A-7A | DC_n78A_1A DC_n78A_5A DC_n78A_7A |
| DC_n78A_3A-5A-7A | DC_n78A_3A DC_n78A_5A DC_n78A_7A |
| DC_n78A_3A-5A-7A-7A | DC_n78A_3A DC_n78A_5A DC_n78A_7A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. |  |

#### 5.5B.4a.4	Inter-band NE-DC configurations within FR1 (five bands)

Table 5.5B.4a.4-1: Inter-band NE-DC configurations within FR1 (five bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n78A_1A-3A-5A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_5A DC_n78A_7A |
| DC_n78A_1A-3A-5A-7A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_5A DC_n78A_7A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. |  |

### 5.5B.5	Inter-band EN-DC including FR2

#### 5.5B.5.1	Inter-band EN-DC configurations including FR2 (two bands)

Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n257A DC_1A_n257D DC_1A_n257E DC_1A_n257F DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M |
| DC_1A_n257(2A) DC_1A_n257(A-G) DC_1A_n257(2G) | DC_1A_n257A DC_1A_n257G DC_1A_n257(2A) DC_1A_n257(2G) |
| DC_1A_n258A DC_1A_n258B DC_1A_n258C DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_1A_n258J DC_1A_n258K DC_1A_n258L DC_1A_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258G DC_1A_n258H DC_1A_n258I |
| DC_1A_n258(2A) | DC_1A_n258A DC_1A_n258(2A) |
| DC_2A_n257A DC_2C_n257A DC_2A_n257G DC_2A_n257H DC_2A_n257I DC_2A_n257J DC_2A_n257K DC_2A_n257L DC_2A_n257M | DC_2A_n257A DC_2A_n257G DC_2A_n257H DC_2A_n257I DC_2A_n257J DC_2A_n257K DC_2A_n257L DC_2A_n257M |
| DC_2A_n257(2A) | DC_2A_n257A |
| DC_2A-2A_n257A | DC_2A_n257A |
| DC_2A_n258A DC_2A_n258D DC_2A_n258G DC_2A_n258H DC_2A_n258I DC_2A_n258J DC_2A_n258K DC_2A_n258L DC_2A_n258M DC_2A_n258O DC_2A_n258P DC_2A_n258Q | DC_2A_n258A DC_2A_n258D DC_2A_n258G DC_2A_n258H DC_2A_n258I DC_2A_n258J DC_2A_n258K DC_2A_n258L DC_2A_n258M DC_2A_n258O DC_2A_n258P DC_2A_n258Q |
| DC_2A_n258(2A) DC_2A_n258(3A) DC_2A_n258(4A) DC_2A_n258(5A) | DC_2A_n258A |
| DC_2A_n260A DC_2A_n260D DC_2A_n260E DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2A_n260O DC_2A_n260P DC_2A_n260Q DC_2C_n260A | DC_2A_n260A DC_2A_n260D DC_2A_n260E DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A_n260(2A) DC_2A_n260(3A) DC_2A_n260(4A) DC_2A_n260(5A) DC_2A_n260(6A) DC_2A_n260(7A) DC_2A_n260(8A) DC_2A_n260(2D) DC_2A_n260(2G) DC_2A_n260(3G) DC_2A_n260(4G) DC_2A_n260(2H) DC_2A_n260(2O) DC_2A_n260(3O) DC_2A_n260(4O) DC_2A_n260(2P) DC_2A_n260(A-D) DC_2A_n260(A-G) DC_2A_n260(A-H) DC_2A_n260(A-O) DC_2A_n260(A-P) DC_2A_n260(A-Q) DC_2A_n260(A-2G) DC_2A_n260(A-2H) DC_2A_n260(A-2O) DC_2A_n260(A-2P) DC_2A_n260(A-3G) DC_2A_n260(A-3O) DC_2A_n260(3A-G) DC_2A_n260(3A-H) DC_2A_n260(3A-O) DC_2A_n260(2A-G) DC_2A_n260(2A-H) DC_2A_n260(2A-O) DC_2A_n260(2A-P) DC_2A_n260(2A-Q) DC_2A_n260(2A-2G) DC_2A_n260(2A-2H) DC_2A_n260(2A-2O) DC_2A_n260(3A-2O) DC_2A_n260(3A-P) DC_2A_n260(4A-O) DC_2A_n260(4A-P) DC_2A_n260(4A-2O) DC_2A_n260(5A-O) DC_2A_n260(G-H) DC_2A_n260(O-P) DC_2A_n260(O-Q) DC_2A_n260(P-Q) DC_2A_n260(A-G-H) DC_2A_n260(A-O-P) DC_2A_n260(A-O-Q) DC_2A_n260(A-P-Q) DC_2A_n260(2A-O-P) DC_2A_n260(2A-G-H) DC_2A_n260(3A-O-P) | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A-2A_n260A DC_2A-2A_n260G DC_2A-2A_n260H DC_2A-2A_n260I DC_2A-2A_n260J DC_2A-2A_n260K DC_2A-2A_n260L DC_2A-2A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A_n261A DC_2A_n261B DC_2A_n261C DC_2A_n261D DC_2A_n261E DC_2A_n261F DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M DC_2A_n261O DC_2A_n261P DC_2A_n261Q | DC_2A_n261A DC_2A_n261B DC_2A_n261C DC_2A_n261D DC_2A_n261E DC_2A_n261F DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M DC_2A_n261O DC_2A_n261P DC_2A_n261Q |
| DC_2A_n261(2A) DC_2A_n261(3A) DC_2A_n261(4A) DC_2A_n261(2I) DC_2A_n261(2H) DC_2A_n261(A-G) DC_2A_n261(A-J) DC_2A_n261(A-K) DC_2A_n261(A-L) DC_2A_n261(A-2G) DC_2A_n261(A-H) DC_2A_n261(A-I) DC_2A_n261(2A-G) DC_2A_n261(2A-I) DC_2A_n261(2A-H) DC_2A_n261(3A-G) DC_2A_n261(G-H) DC_2A_n261(G-I) DC_2A_n261(G-J) DC_2A_n261(2G) DC_2A_n261(H-I) DC_2A_n261(A-G-H) DC_2A_n261(A-G-I) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A-2A_n261A DC_2A-2A_n261G DC_2A-2A_n261H DC_2A-2A_n261I DC_2A-2A_n261J DC_2A-2A_n261K DC_2A-2A_n261L DC_2A-2A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M |
| DC_3A_n257A DC_3A_n257B DC_3A_n257C DC_3A_n257D DC_3A_n257E DC_3A_n257F DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_3C_n257A DC_3C_n257D DC_3C_n257E DC_3C_n257F DC_3C_n257G DC_3C_n257H DC_3C_n257I DC_3C_n257J DC_3C_n257K DC_3C_n257L DC_3C_n257M | DC_3A_n257A DC_3A_n257B DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_3C_n257A |
| DC_3A-3A_n257A DC_3A-3A_n257D DC_3A-3A_n257E DC_3A-3A_n257F DC_3A-3A_n257G DC_3A-3A_n257H DC_3A-3A_n257I DC_3A-3A_n257J DC_3A-3A_n257K DC_3A-3A_n257L DC_3A-3A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_3A_n258A DC_3A_n258B DC_3A_n258C DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K DC_3A_n258L DC_3A_n258M DC_3C_n258A DC_3C_n258B DC_3C_n258C DC_3C_n258D DC_3C_n258E DC_3C_n258F DC_3C_n258G DC_3C_n258H DC_3C_n258I DC_3C_n258J DC_3C_n258K DC_3C_n258L DC_3C_n258M | DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3C_n258A DC_3C_n258G DC_3C_n258H DC_3C_n258I |
| DC_4A_n258A DC_4A_n258G DC_4A_n258H DC_4A_n258I | DC_4A_n258A DC_4A_n258G DC_4A_n258H DC_4A_n258I |
| DC_4A_n260A DC_4A_n260D DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q | DC_4A_n260A DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q |
| DC_4A_n260(2A) DC_4A_n260(3A) DC_4A_n260(4A) DC_4A_n260(5A) DC_4A_n260(6A) DC_4A_n260(7A) DC_4A_n260(8A) DC_4A_n260(2D) DC_4A_n260(2G) DC_4A_n260(3G) DC_4A_n260(4G) DC_4A_n260(2H) DC_4A_n260(2O) DC_4A_n260(2P) DC_4A_n260(3O) DC_4A_n260(4O) DC_4A_n260(A-D) DC_4A_n260(2A-D) DC_4A_n260(3A-D) DC_4A_n260(4A-D) DC_4A_n260(A-O) DC_4A_n260(2A-O) DC_4A_n260(A-D-O) DC_4A_n260(2A-D-O) DC_4A_n260(A-2O) DC_4A_n260(D-2O) DC_4A_n260(A-D-2O) DC_4A_n260(2A-D-2O) DC_4A_n260(A-2D) DC_4A_n260(2A-2D) DC_4A_n260(A-P) DC_4A_n260(2A-P) DC_4A_n260(A-2P) DC_4A_n260(2A-2P) DC_4A_n260(A-G) DC_4A_n260(2A-G) DC_4A_n260(A-2G) DC_4A_n260(2A-2G) DC_4A_n260(G-O) DC_4A_n260(2G-O) DC_4A_n260(A-G-O) DC_4A_n260(2A-G-O) DC_4A_n260(2A-G-2O) DC_4A_n260(2A-G-3O) DC_4A_n260(A-2G-O) DC_4A_n260(2A-2G-O) DC_4A_n260(2A-2G-2O) DC_4A_n260(A-H) DC_4A_n260(A-2H) DC_4A_n260(2A-H) DC_4A_n260(2A-2H) DC_4A_n260(2A-2O) DC_4A_n260(A-3O) DC_4A_n260(2A-3O) DC_4A_n260(A-4O) DC_4A_n260(2A-4O) DC_4A_n260(3A-O) DC_4A_n260(3A-2O) DC_4A_n260(3A-3O) DC_4A_n260(3A-G) DC_4A_n260(3A-2G) DC_4A_n260(4A-G) DC_4A_n260(4A-2G) DC_4A_n260(4A-O) DC_4A_n260(4A-2O) DC_4A_n260(D-2G) DC_4A_n260(2D-O) DC_4A_n260(G-2O) DC_4A_n260(2G-2O) DC_4A_n260(G-3O) DC_4A_n260(2G-3O) DC_4A_n260(G-4O) DC_4A_n260(2G-4O) DC_4A_n260(3G-O) DC_4A_n260(4G-O) DC_4A_n260(H-O) DC_4A_n260(2H-O) DC_4A_n260(A-P-Q) DC_4A_n260(3A-O-P) DC_4A_n260(A-Q) DC_4A_n260(P-Q) DC_4A_n260(2A-O-P) DC_4A_n260(3A-P) DC_4A_n260(A-O-P) DC_4A_n260(2A-3G) DC_4A_n260(2A-G-H) DC_4A_n260(2A-Q) DC_4A_n260(3A-2G-O) DC_4A_n260(3A-D-O) DC_4A_n260(3A-G-O) DC_4A_n260(3A-G-2O) DC_4A_n260(3A-H) DC_4A_n260(4A-G-O) DC_4A_n260(4A-P) DC_4A_n260(5A-G) DC_4A_n260(5A-O) DC_4A_n260(A-G-2O) DC_4A_n260(A-G-3O) DC_4A_n260(A-G-4O) DC_4A_n260(A-G-H) DC_4A_n260(A-H-O) DC_4A_n260(A-2G-2O) DC_4A_n260(A-2G-3O) DC_4A_n260(A-3G-O) DC_4A_n260(A-3G) DC_4A_n260(A-4G) DC_4A_n260(A-D-G) DC_4A_n260(A-O-Q) DC_4A_n260(D-G) DC_4A_n260(D-O) DC_4A_n260(G-H) DC_4A_n260(G-H-O) DC_4A_n260(O-P) DC_4A_n260(O-Q) | DC_4A_n260A DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q |
| DC_4A_n261A DC_4A_n261D DC_4A_n261G DC_4A_n261H DC_4A_n261I DC_4A_n261J DC_4A_n261K DC_4A_n261L DC_4A_n261M | DC_4A_n261A DC_4A_n261D DC_4A_n261G DC_4A_n261H DC_4A_n261I |
| DC_4A_n261(2A) DC_4A_n261(3A) DC_4A_n261(4A) DC_4A_n261(2G) DC_4A_n261(2H) DC_4A_n261(2I) DC_4A_n261(A-D) DC_4A_n261(A-H) DC_4A_n261(A-2H) DC_4A_n261(A-D-H) DC_4A_n261(A-G) DC_4A_n261(A-G-H) DC_4A_n261(A-I) DC_4A_n261(A-2I) DC_4A_n261(G-I) DC_4A_n261(A-G-I) DC_4A_n261(A-H-I) DC_4A_n261(G-H) DC_4A_n261(H-I) DC_4A_n261(D-H) DC_4A_n261(2A-D) DC_4A_n261(2A-G) DC_4A_n261(2A-H) DC_4A_n261(2A-I) DC_4A_n261(A-2G) DC_4A_n261(A-D-G) DC_4A_n261(D-G) | DC_4A_n261A DC_4A_n261H DC_4A_n261I DC_4A_n261G |
| DC_5A_n257A DC_5A_n257D DC_5A_n257E DC_5A_n257F DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_5A_n257J DC_5A_n257K DC_5A_n257L DC_5A_n257M DC_5B_n257A | DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_5B_n257A |
| DC_5A-5A_n257A | DC_5A_n257A |
| DC_5A_n258A DC_5A_n258D DC_5A_n258G DC_5A_n258H DC_5A_n258I DC_5A_n258J DC_5A_n258K DC_5A_n258L DC_5A_n258M DC_5A_n258O DC_5A_n258P DC_5A_n258Q | DC_5A_n258A DC_5A_n258D DC_5A_n258G DC_5A_n258H DC_5A_n258I DC_5A_n258J DC_5A_n258K DC_5A_n258L DC_5A_n258M DC_5A_n258O DC_5A_n258P DC_5A_n258Q |
| DC_5A_n260A DC_5A_n260B DC_5A_n260C DC_5A_n260D DC_5A_n260E DC_5A_n260F DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5B_n260A | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5B_n260A |
| DC_5A_n260(2A) DC_5A_n260(3A) DC_5A_n260(4A) DC_5A_n260(5A) DC_5A_n260(6A) DC_5A_n260(7A) DC_5A_n260(8A) DC_5A_n260(9A) DC_5A_n260(10A) DC_5A_n260(A-I) DC_5A_n260(A-P-Q) DC_5A_n260(3A-O-P) DC_5A_n260(D-G) DC_5A_n260(D-H) DC_5A_n260(D-I) DC_5A_n260(D-O) DC_5A_n260(D-P) DC_5A_n260(D-Q) DC_5A_n260(E-O) DC_5A_n260(E-P) DC_5A_n260(E-Q) DC_5A_n260(G-I) DC_5A_n260(2G) DC_5A_n260(2H) DC_5A_n260(2O) DC_5A_n260(3O) DC_5A_n260(4O) DC_5A_n260(2P) DC_5A_n260(3P) DC_5A_n260(4P) DC_5A_n260(2A-O) DC_5A_n260(A-2O) DC_5A_n260(2A-G) DC_5A_n260(A-2G) DC_5A_n260(2A-2G) DC_5A_n260(2G-O) DC_5A_n260(2A-2G-O) DC_5A_n260(A-2H) DC_5A_n260(2A-H) DC_5A_n260(2A-2H) DC_5A_n260(2A-2O) DC_5A_n260(2A-3O) DC_5A_n260(A-4O) DC_5A_n260(2A-4O) DC_5A_n260(3A-2O) DC_5A_n260(3A-2G) DC_5A_n260(4A-G) DC_5A_n260(4A-2G) DC_5A_n260(4A-O) DC_5A_n260(4A-2O) DC_5A_n260(A-O) DC_5A_n260(A-G) DC_5A_n260(G-O) DC_5A_n260(A-G-O) DC_5A_n260(2A-G-O) DC_5A_n260(A-2G-O) DC_5A_n260(A-H) DC_5A_n260(A-3O) DC_5A_n260(3A-O) DC_5A_n260(3A-G) DC_5A_n260(2D) DC_5A_n260(3G) DC_5A_n260(4G) DC_5A_n260(A-D) DC_5A_n260(2A-D) DC_5A_n260(A-D-O) DC_5A_n260(2A-D-O) DC_5A_n260(D-2O) DC_5A_n260(A-D-2O) DC_5A_n260(2A-D-2O) DC_5A_n260(A-2D) DC_5A_n260(2A-2D) DC_5A_n260(A-P) DC_5A_n260(2A-P) DC_5A_n260(A-2P) DC_5A_n260(2A-2P) DC_5A_n260(3A-3O) DC_5A_n260(D-2G) DC_5A_n260(2D-O) DC_5A_n260(G-2O) DC_5A_n260(2G-2O) DC_5A_n260(G-3O) DC_5A_n260(2G-3O) DC_5A_n260(G-4O) DC_5A_n260(2G-4O) DC_5A_n260(3G-O) DC_5A_n260(4G-O) DC_5A_n260(H-O) DC_5A_n260(2H-O) DC_5A_n260(A-Q) DC_5A_n260(P-Q) DC_5A_n260(2A-4P) DC_5A_n260(2O-2P) DC_5A_n260(3A-P) DC_5A_n260(4A-4O) DC_5A_n260(4A-2Q) DC_5A_n260(6A-2O) DC_5A_n260(6A-2P) DC_5A_n260(6A-3O) DC_5A_n260(8A-2O) DC_5A_n260(2A-O-P) DC_5A_n260(2A-2G-2O) DC_5A_n260(2A-2O-2P) DC_5A_n260(2A-2O-2Q) DC_5A_n260(O-P) DC_5A_n260(A-O-P) DC_5A_n260(2A-2O-P-Q) DC_5A_n260(2A-2O-P) DC_5A_n260(2A-2O-Q) DC_5A_n260(2A-2Q) DC_5A_n260(2A-3G) DC_5A_n260(2A-3O-P) DC_5A_n260(2A-3O-Q) DC_5A_n260(2A-3P) DC_5A_n260(2A-G-2O) DC_5A_n260(2A-G-3O) DC_5A_n260(2A-G-H) DC_5A_n260(2A-O-2P) DC_5A_n260(2A-O-2Q) DC_5A_n260(2A-O-3P) DC_5A_n260(2A-O-P-Q) DC_5A_n260(2A-O-Q) DC_5A_n260(2A-P-Q) DC_5A_n260(2A-Q) DC_5A_n260(2O-2Q) DC_5A_n260(2O-P-Q) DC_5A_n260(2O-P) DC_5A_n260(2O-Q) DC_5A_n260(2Q) DC_5A_n260(3A-2G-O) DC_5A_n260(3A-2O-P) DC_5A_n260(3A-2O-Q) DC_5A_n260(3A-2P) DC_5A_n260(3A-2Q) DC_5A_n260(3A-3P) DC_5A_n260(3A-4O) DC_5A_n260(3A-D-O) DC_5A_n260(3A-D) DC_5A_n260(3A-G-2O) DC_5A_n260(3A-G-O) DC_5A_n260(3A-H) DC_5A_n260(3A-O-2P) DC_5A_n260(3A-O-2Q) DC_5A_n260(3A-O-P-Q) DC_5A_n260(3A-O-Q) DC_5A_n260(3A-P-Q) DC_5A_n260(3A-Q) DC_5A_n260(3O-P) DC_5A_n260(3O-Q) DC_5A_n260(4A-2P) DC_5A_n260(4A-3O) DC_5A_n260(4A-D) DC_5A_n260(4A-G-O) DC_5A_n260(4A-O-P) DC_5A_n260(4A-O-Q) DC_5A_n260(4A-P-Q) DC_5A_n260(4A-P) DC_5A_n260(4A-Q) DC_5A_n260(5A-2O) DC_5A_n260(5A-2P) DC_5A_n260(5A-3O) DC_5A_n260(5A-G) DC_5A_n260(5A-O-P) DC_5A_n260(5A-O) DC_5A_n260(5A-P) DC_5A_n260(5A-Q) DC_5A_n260(6A-O-P) DC_5A_n260(6A-O) DC_5A_n260(6A-P) DC_5A_n260(7A-2O) DC_5A_n260(7A-O) DC_5A_n260(7A-P) DC_5A_n260(8A-O) DC_5A_n260(9A-O) DC_5A_n260(A-2G-2O) DC_5A_n260(A-2G-3O) DC_5A_n260(A-2O-2P) DC_5A_n260(A-2O-2Q) DC_5A_n260(A-2O-P-Q) DC_5A_n260(A-2O-P) DC_5A_n260(A-2O-Q) DC_5A_n260(A-2Q) DC_5A_n260(A-3G-O) DC_5A_n260(A-3G) DC_5A_n260(A-3O-P) DC_5A_n260(A-3O-Q) DC_5A_n260(A-3P) DC_5A_n260(A-4G) DC_5A_n260(A-4P) DC_5A_n260(A-D-G) DC_5A_n260(A-E) DC_5A_n260(A-G-2O) DC_5A_n260(A-G-3O) DC_5A_n260(A-G-4O) DC_5A_n260(A-G-H) DC_5A_n260(A-H-O) DC_5A_n260(A-O-2P) DC_5A_n260(A-O-2Q) DC_5A_n260(A-O-3P) DC_5A_n260(A-O-P-Q) DC_5A_n260(A-O-Q) DC_5A_n260(G-H-O) DC_5A_n260(G-H) DC_5A_n260(O-2P) DC_5A_n260(O-2Q) DC_5A_n260(O-3P) DC_5A_n260(O-P-Q) DC_5A_n260(O-Q) | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260O DC_5A_n260P DC_5A_n260Q |
| DC_5A-5A_n260A | DC_5A_n260A |
| DC_5A_n261A DC_5A_n261B DC_5A_n261C DC_5A_n261D DC_5A_n261E DC_5A_n261F DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261K DC_5A_n261L DC_5A_n261M DC_5A_n261O DC_5A_n261P DC_5A_n261Q | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261K DC_5A_n261L DC_5A_n261M DC_5A_n261O |
| DC_5A_n261(2A) DC_5A_n261(2G) DC_5A_n261(3A) DC_5A_n261(4A) DC_5A_n261(5A) DC_5A_n261(6A) DC_5A_n261(7A) DC_5A_n261(8A) DC_5A_n261(D-G) DC_5A_n261(D-H) DC_5A_n261(D-I) DC_5A_n261(D-O) DC_5A_n261(D-P) DC_5A_n261(D-Q) DC_5A_n261(E-O) DC_5A_n261(E-P) DC_5A_n261(E-Q) DC_5A_n261(2H) DC_5A_n261(2I) DC_5A_n261(A-H) DC_5A_n261(A-I) DC_5A_n261(A-J) DC_5A_n261(A-K) DC_5A_n261(A-L) DC_5A_n261(A-D) DC_5A_n261(A-D-H) DC_5A_n261(A-D-2O) DC_5A_n261(A-G) DC_5A_n261(A-G-H) DC_5A_n261(G-I) DC_5A_n261(A-G-I) DC_5A_n261(A-H-I) DC_5A_n261(G-H) DC_5A_n261(G-J) DC_5A_n261(H-I) DC_5A_n261(A-2D) DC_5A_n261(A-2H) DC_5A_n261(A-2P) DC_5A_n261(A-2Q) DC_5A_n261(A-2I) DC_5A_n261(A-4G) DC_5A_n261(A-4O) DC_5A_n261(A-7O) DC_5A_n261(A-2G-2O) DC_5A_n261(A-3G-O) DC_5A_n261(2A-G) DC_5A_n261(2A-H) DC_5A_n261(2A-I) DC_5A_n261(3A-G) DC_5A_n261(2A-2G-O) DC_5A_n261(2A-2G) DC_5A_n261(2A-2O) DC_5A_n261(2A-3G) DC_5A_n261(2A-3O) DC_5A_n261(2A-4O) DC_5A_n261(2A-5O) DC_5A_n261(2A-6O) DC_5A_n261(2A-D-O) DC_5A_n261(2A-D) DC_5A_n261(2A-G-2O) DC_5A_n261(2A-G-O) DC_5A_n261(2A-O) DC_5A_n261(2A-P) DC_5A_n261(2A-Q) DC_5A_n261(2D) DC_5A_n261(2G-2O) DC_5A_n261(2G-O) DC_5A_n261(2O) DC_5A_n261(2P) DC_5A_n261(2Q) DC_5A_n261(3A-2G) DC_5A_n261(3A-2O) DC_5A_n261(3A-3O) DC_5A_n261(3A-4O) DC_5A_n261(3A-5O) DC_5A_n261(3A-D) DC_5A_n261(3A-G-O) DC_5A_n261(3A-O) DC_5A_n261(3G-O) DC_5A_n261(3G) DC_5A_n261(3O) DC_5A_n261(4A-2O) DC_5A_n261(4A-3O) DC_5A_n261(4A-4O) DC_5A_n261(4A-G) DC_5A_n261(4A-O) DC_5A_n261(4G) DC_5A_n261(4O) DC_5A_n261(5A-2O) DC_5A_n261(5A-3O) DC_5A_n261(5A-O) DC_5A_n261(5O) DC_5A_n261(6A-2O) DC_5A_n261(6A-O) DC_5A_n261(6O) DC_5A_n261(7A-O) DC_5A_n261(7O) DC_5A_n261(A-2G-O) DC_5A_n261(A-2G) DC_5A_n261(A-2O) DC_5A_n261(A-3G) DC_5A_n261(A-3O) DC_5A_n261(A-5O) DC_5A_n261(A-6O) DC_5A_n261(A-D-G) DC_5A_n261(A-D-O) DC_5A_n261(A-E) DC_5A_n261(A-G-2O) DC_5A_n261(A-G-O) DC_5A_n261(A-O-P) DC_5A_n261(A-O-Q) DC_5A_n261(A-O) DC_5A_n261(A-P-Q) DC_5A_n261(A-P) DC_5A_n261(A-Q) DC_5A_n261(D-2O) DC_5A_n261(G-2O) DC_5A_n261(G-O) DC_5A_n261(O-P) DC_5A_n261(O-Q) DC_5A_n261(P-Q) | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |
| DC_7A_n257A DC_7A_n257D DC_7A_n257E DC_7A_n257F DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_7A_n257L DC_7A_n257M | DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A_n257(2A) DC_7A_n257(A-G) DC_7A_n257(2G) | DC_7A_n257A DC_7A_n257G DC_7A_n257(2A) DC_7A_n257(2G) |
| DC_7A-7A_n257A DC_7A-7A_n257D DC_7A-7A_n257E DC_7A-7A_n257F DC_7A-7A_n257G DC_7A-7A_n257H DC_7A-7A_n257I DC_7A-7A_n257J DC_7A-7A_n257K DC_7A-7A_n257L DC_7A-7A_n257M | DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A_n258A DC_7A_n258B DC_7A_n258C DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M DC_7C_n258A DC_7C_n258B DC_7C_n258C DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_7C_n258J DC_7C_n258K DC_7C_n258L DC_7C_n258M | DC_7A_n258A DC_7A_n258B DC_7A_n258C DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M DC_7C_n258A DC_7C_n258B DC_7C_n258C DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_7C_n258J DC_7C_n258K DC_7C_n258L DC_7C_n258M |
| DC_7A_n258(2A) | DC_7A_n258A DC_7A_n258(2A) |
| DC_7A_n260A DC_7A_n260G DC_7A_n260H DC_7A_n260I DC_7A_n260J DC_7A_n260K DC_7A_n260L DC_7A_n260M DC_7A_n260O DC_7A_n260P DC_7A_n260Q | DC_7A_n260A DC_7A_n260G DC_7A_n260H DC_7A_n260I DC_7A_n260J DC_7A_n260K DC_7A_n260L DC_7A_n260M DC_7A_n260O DC_7A_n260P DC_7A_n260Q |
| DC_7A_n261A DC_7A_n261G DC_7A_n261H DC_7A_n261I DC_7A_n261J DC_7A_n261K DC_7A_n261L DC_7A_n261M DC_7A_n261O DC_7A_n261P DC_7A_n261Q | DC_7A_n261A DC_7A_n261G DC_7A_n261H DC_7A_n261I DC_7A_n261J DC_7A_n261K DC_7A_n261L DC_7A_n261M DC_7A_n261O DC_7A_n261P DC_7A_n261Q |
| DC_8A_n257A DC_8A_n257D DC_8A_n257E DC_8A_n257F DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K DC_8A_n257L DC_8A_n257M | DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_8A_n258A DC_8A_n258B DC_8A_n258C DC_8A_n258D DC_8A_n258E DC_8A_n258F DC_8A_n258G DC_8A_n258H DC_8A_n258I DC_8A_n258J DC_8A_n258K DC_8A_n258L DC_8A_n258M | DC_8A_n258A |
| DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I | DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_12A_n257A DC_12A_n257G DC_12A_n257H DC_12A_n257I DC_12A_n257J DC_12A_n257K DC_12A_n257L DC_12A_n257M | DC_12A_n257A DC_12A_n257G DC_12A_n257H DC_12A_n257I DC_12A_n257J DC_12A_n257K DC_12A_n257L DC_12A_n257M |
| DC_12A_n258A DC_12A_n258D DC_12A_n258G DC_12A_n258H DC_12A_n258I DC_12A_n258J DC_12A_n258K DC_12A_n258L DC_12A_n258M DC_12A_n258O DC_12A_n258P DC_12A_n258Q | DC_12A_n258A DC_12A_n258D DC_12A_n258G DC_12A_n258H DC_12A_n258I DC_12A_n258J DC_12A_n258K DC_12A_n258L DC_12A_n258M DC_12A_n258O DC_12A_n258P DC_12A_n258Q |
| DC_12A_n260A DC_12A_n260D DC_12A_n260E DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_12A_n260O DC_12A_n260P DC_12A_n260Q | DC_12A_n260A DC_12A_n260D DC_12A_n260E DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_12A_n260O DC_12A_n260P DC_12A_n260Q |
| DC_12A_n260(2A) DC_12A_n260(2G) DC_12A_n260(A-G) DC_12A_n260(A-H) DC_12A_n260(G-H) DC_12A_n260(A-I) DC_12A_n260(G-I) | DC_12A_n260A |
| DC_12A_n261A DC_12A_n261B DC_12A_n261C DC_12A_n261D DC_12A_n261E DC_12A_n261F DC_12A_n261G DC_12A_n261H DC_12A_n261I DC_12A_n261J DC_12A_n261K DC_12A_n261L DC_12A_n261M DC_12A_n261O DC_12A_n261P DC_12A_n261Q | DC_12A_n261A DC_12A_n261B DC_12A_n261C DC_12A_n261D DC_12A_n261E DC_12A_n261F DC_12A_n261G DC_12A_n261H DC_12A_n261I DC_12A_n261J DC_12A_n261K DC_12A_n261L DC_12A_n261M DC_12A_n261O DC_12A_n261P DC_12A_n261Q |
| DC_13A_n257A | DC_13A_n257A |
| DC_13A_n260A DC_13A_n260D DC_13A_n260G DC_13A_n260H DC_13A_n260I DC_13A_n260J DC_13A_n260K DC_13A_n260L DC_13A_n260M DC_13A_n260O DC_13A_n260P DC_13A_n260Q | DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260I DC_13A_n260J DC_13A_n260K DC_13A_n260L DC_13A_n260M DC_13A_n260O DC_13A_n260P DC_13A_n260Q |
| DC_13A_n260(2A) DC_13A_n260(3A) DC_13A_n260(4A) DC_13A_n260(5A) DC_13A_n260(6A) DC_13A_n260(7A) DC_13A_n260(8A) DC_13A_n260(2D) DC_13A_n260(2G) DC_13A_n260(3G) DC_13A_n260(4G) DC_13A_n260(2H) DC_13A_n260(2O) DC_13A_n260(3O) DC_13A_n260(4O) DC_13A_n260(A-G) DC_13A_n260(A-2G) DC_13A_n260(A-P) DC_13A_n260(A-Q) DC_13A_n260(2A-G) DC_13A_n260(2A-H) DC_13A_n260(2A-2G) DC_13A_n260(2A-2H) DC_13A_n260(3A-G) DC_13A_n260(3A-O) DC_13A_n260(3A-P) DC_13A_n260(3A-2O) DC_13A_n260(4A-O) DC_13A_n260(4A-2O) DC_13A_n260(P-Q) DC_13A_n260(A-P-Q) DC_13A_n260(2A-O-P) DC_13A_n260(3A-O-P) DC_13A_n260(A-H) DC_13A_n260(A-2H) DC_13A_n260(2A-O) DC_13A_n260(A-O) DC_13A_n260(2A-P) DC_13A_n260(A-O-P) DC_13A_n260(O-P) DC_13A_n260(2A-2O) DC_13A_n260(A-2O) DC_13A_n260(G-H) DC_13A_n260(2A-G-H) DC_13A_n260(2A-Q) DC_13A_n260(2P) DC_13A_n260(3A-H) DC_13A_n260(4A-P) DC_13A_n260(5A-O) DC_13A_n260(A-2P) DC_13A_n260(A-3G) DC_13A_n260(A-3O) DC_13A_n260(A-D) DC_13A_n260(A-G-H) DC_13A_n260(A-O-Q) DC_13A_n260(O-Q) | DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260O DC_13A_n260P DC_13A_n260Q |
| DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261J DC_13A_n261K DC_13A_n261I DC_13A_n261L DC_13A_n261M | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_13A_n261L DC_13A_n261M |
| DC_13A_n261(2A) DC_13A_n261(2G) DC_13A_n261(3A) DC_13A_n261(4A) DC_13A_n261(2H) DC_13A_n261(2I) DC_13A_n261(A-G) DC_13A_n261(A-K) DC_13A_n261(A-L) DC_13A_n261(A-2G) DC_13A_n261(A-H) DC_13A_n261(A-I) DC_13A_n261(A-J) DC_13A_n261(2A-G) DC_13A_n261(2A-H) DC_13A_n261(2A-I) DC_13A_n261(3A-G) DC_13A_n261(G-H) DC_13A_n261(G-I) DC_13A_n261(G-J) DC_13A_n261(H-I) DC_13A_n261(A-G-H) DC_13A_n261(A-G-I) | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |
| DC_14A_n258A | DC_14A_n258A |
| DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M |
| DC_18A_n257A DC_18A_n257D DC_18A_n257E DC_18A_n257F DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_18A_n257J DC_18A_n257K DC_18A_n257L DC_18A_n257M | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_19A_n257A DC_19A_n257D DC_19A_n257E DC_19A_n257F DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K DC_19A_n257L DC_19A_n257M | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K DC_19A_n257L DC_19A_n257M |
| DC_20A_n257A DC_20A_n257B DC_20A_n257C DC_20A_n257D DC_20A_n257E DC_20A_n257F DC_20A_n257G DC_20A_n257H DC_20A_n257I DC_20A_n257J DC_20A_n257K DC_20A_n257L DC_20A_n257M | DC_20A_n257A |
| DC_20A_n258A DC_20A_n258B DC_20A_n258C DC_20A_n258D DC_20A_n258E DC_20A_n258F DC_20A_n258G DC_20A_n258H DC_20A_n258I DC_20A_n258J DC_20A_n258K DC_20A_n258L DC_20A_n258M | DC_20A_n258A |
| DC_21A_n257A DC_21A_n257D DC_21A_n257E DC_21A_n257F DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_26A_n257A | DC_26A_n257A |
| DC_26A_n258A DC_26A_n258B DC_26A_n258C DC_26A_n258D DC_26A_n258E DC_26A_n258F DC_26A_n258G DC_26A_n258H DC_26A_n258I DC_26A_n258J DC_26A_n258K DC_26A_n258L DC_26A_n258M | DC_26A_n258A DC_26A_n258G DC_26A_n258H DC_26A_n258I |
| DC_28A_n257A DC_28A_n257D DC_28A_n257E DC_28A_n257F DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_28A_n257J DC_28A_n257K DC_28A_n257L DC_28A_n257M | DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_28A_n257J DC_28A_n257K DC_28A_n257L DC_28A_n257M |
| DC_28A_n258A DC_28A_n258B DC_28A_n258C DC_28A_n258D DC_28A_n258E DC_28A_n258F DC_28A_n258G DC_28A_n258H DC_28A_n258I DC_28A_n258J DC_28A_n258K DC_28A_n258L DC_28A_n258M | DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_30A_n258A | DC_30A_n258A |
| DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M | DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_30A_n260(2A) DC_30A_n260(2G) DC_30A_n260(A-G) DC_30A_n260(A-H) DC_30A_n260(G-H) DC_30A_n260(A-I) DC_30A_n260(G-I) | DC_30A_n260A |
| DC_38A_n257A DC_38A_n257G DC_38A_n257H DC_38A_n257I DC_38A_n257J DC_38A_n257K DC_38A_n257L DC_38A_n257M | DC_38A_n257A DC_38A_n257G DC_38A_n257H DC_38A_n257I DC_38A_n257J DC_38A_n257K DC_38A_n257L DC_38A_n257M |
| DC_38A_n258A DC_38A_n258G DC_38A_n258H DC_38A_n258I DC_38A_n258J DC_38A_n258K DC_38A_n258L DC_38A_n258M | DC_38A_n258A DC_38A_n258G DC_38A_n258H DC_38A_n258I DC_38A_n258J DC_38A_n258K DC_38A_n258L DC_38A_n258M |
| DC_39A_n257A DC_39A_n257D DC_39A_n257E DC_39A_n257F DC_39A_n257G DC_39A_n257H DC_39A_n257I DC_39A_n257J DC_39A_n257K DC_39A_n257L DC_39A_n257M | DC_39A_n257A |
| DC_39A_n258A DC_39A_n258B DC_39A_n258C DC_39A_n258D DC_39A_n258E DC_39A_n258F DC_39A_n258G DC_39A_n258H DC_39A_n258I DC_39A_n258J DC_39A_n258K DC_39A_n258L DC_39A_n258M | DC_39A_n258A DC_39A_n258B DC_39A_n258C DC_39A_n258D DC_39A_n258E DC_39A_n258F DC_39A_n258G DC_39A_n258H DC_39A_n258I DC_39A_n258J DC_39A_n258K DC_39A_n258L DC_39A_n258M |
| DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M DC_40C_n257A DC_40C_n257D DC_40C_n257E DC_40C_n257F DC_40C_n257G DC_40C_n257H DC_40C_n257I DC_40C_n257J DC_40C_n257K DC_40C_n257L DC_40C_n257M | DC_40A_n257A DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M |
| DC_40A-40A_n257A DC_40A-40A_n257D DC_40A-40A_n257E DC_40A-40A_n257F DC_40A-40A_n257G DC_40A-40A_n257H DC_40A-40A_n257I DC_40A-40A_n257J DC_40A-40A_n257K DC_40A-40A_n257L DC_40A-40A_n257M | DC_40A_n257A DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M |
| DC_40A_n258A DC_40A_n258B DC_40A_n258C DC_40A_n258D DC_40A_n258E DC_40A_n258F DC_40A_n258G DC_40A_n258H DC_40A_n258I DC_40A_n258J DC_40A_n258K DC_40A_n258L DC_40A_n258M | DC_40A_n258A DC_40A_n258B DC_40A_n258C DC_40A_n258D DC_40A_n258E DC_40A_n258F DC_40A_n258G DC_40A_n258H DC_40A_n258I DC_40A_n258J DC_40A_n258K DC_40A_n258L DC_40A_n258M |
| DC_41A_n257A DC_41A_n257D DC_41A_n257E DC_41A_n257F DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41A_n257J DC_41A_n257K DC_41A_n257L DC_41A_n257M DC_41C_n257A DC_41C_n257D DC_41C_n257E DC_41C_n257F DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_41C_n257J DC_41C_n257K DC_41C_n257L DC_41C_n257M | DC_41A_n257A DC_41A_n257D DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257D DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n258A DC_41A_n258B DC_41A_n258C DC_41A_n258D DC_41A_n258E DC_41A_n258F DC_41A_n258G DC_41A_n258H DC_41A_n258I DC_41A_n258J DC_41A_n258K DC_41A_n258L DC_41A_n258M | DC_41A_n258A DC_41A_n258B DC_41A_n258C DC_41A_n258D DC_41A_n258E DC_41A_n258F DC_41A_n258G DC_41A_n258H DC_41A_n258I DC_41A_n258J DC_41A_n258K DC_41A_n258L DC_41A_n258M |
| DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F DC_42C_n257G DC_42C_n257H DC_42C_n257I DC_42C_n257J DC_42C_n257K DC_42C_n257L DC_42C_n257M DC_42D_n257A DC_42D_n257D DC_42D_n257E DC_42D_n257F DC_42D_n257G DC_42D_n257H DC_42D_n257I DC_42D_n257J DC_42D_n257K DC_42D_n257L DC_42D_n257M DC_42E_n257A DC_42E_n257D DC_42E_n257E DC_42E_n257F DC_42E_n257G DC_42E_n257H DC_42E_n257I DC_42E_n257J DC_42E_n257K DC_42E_n257L DC_42E_n257M | DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F DC_42D_n257A DC_42D_n257D DC_42D_n257E DC_42D_n257F DC_42E_n257A DC_42E_n257D DC_42E_n257E DC_42E_n257F |
| DC_42A-42A_n257A DC_42A-42A_n257D DC_42A-42A_n257E DC_42A-42A_n257F DC_42A-42A_n257G DC_42A-42A_n257H DC_42A-42A_n257I DC_42A-42A_n257J DC_42A-42A_n257K DC_42A-42A_n257L DC_42A-42A_n257M | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M |
| DC_46A_n260A3 | N/A |
| DC_46A_n261A3 | N/A |
| DC_48A_n257A DC_48C_n257A | DC_48A_n257A DC_48C_n257A |
| DC_48A-48A_n257A | DC_48A_n257A |
| DC_48A_n260A DC_48A_n260G DC_48A_n260H DC_48A_n260I DC_48A_n260J DC_48A_n260K DC_48A_n260L DC_48A_n260M DC_48C_n260A DC_48D_n260A | DC_48A_n260A DC_48A_n260G DC_48A_n260H DC_48A_n260I DC_48C_n260A |
| DC_48A_n260(2A) DC_48C_n260(2A) DC_48D_n260(2A) DC_48A_n260(3A) DC_48C_n260(3A) DC_48D_n260(3A) DC_48A_n260(4A) DC_48C_n260(4A) DC_48D_n260(4A) | DC_48A_n260A DC_48A_n260G DC_48A_n260H DC_48A_n260I DC_48C_n260A |
| DC_48A-48A_n260A | DC_48A_n260A |
| DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48A_n261J DC_48A_n261K DC_48A_n261L DC_48A_n261M DC_48C_n261A DC_48D_n261A | DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48C_n261A |
| DC_48A_n261(A-G) DC_48A_n261(A-G-H) DC_48A_n261(A-G-I) DC_48A_n261(A-H) DC_48A_n261(A-I) DC_48A_n261(A-J) DC_48A_n261(A-K) DC_48A_n261(A-L) DC_48A_n261(A-2G) DC_48A_n261(G-H) DC_48A_n261(G-I) DC_48A_n261(G-J) DC_48A_n261(H-I) DC_48A_n261(2A) DC_48C_n261(2A) DC_48D_n261(2A) DC_48A_n261(3A) DC_48A_n261(2A-G) DC_48A_n261(2A-H) DC_48A_n261(2A-I) DC_48A_n261(2G) DC_48A_n261(2H) DC_48A_n261(4A) DC_48A_n261(3A-G) | DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48C_n261A |
| DC_66A_n257A DC_66A_n257G DC_66A_n257H DC_66A_n257I DC_66A_n257J DC_66A_n257K DC_66A_n257L DC_66A_n257M DC_66C_n257A | DC_66A_n257A DC_66A_n257G |
| DC_66A_n257(2A) DC_66A-66A_n257A | DC_66A_n257A |
| DC_66A_n258A DC_66A_n258D DC_66A_n258G DC_66A_n258H DC_66A_n258I DC_66A_n258J DC_66A_n258K DC_66A_n258L DC_66A_n258M DC_66A_n258O DC_66A_n258P DC_66A_n258Q | DC_66A_n258A DC_66A_n258D DC_66A_n258G DC_66A_n258H DC_66A_n258I DC_66A_n258J DC_66A_n258K DC_66A_n258L DC_66A_n258M DC_66A_n258O DC_66A_n258P DC_66A_n258Q |
| DC_66A_n258(2A) DC_66A_n258(3A) DC_66A_n258(4A) DC_66A_n258(5A) | DC_66A_n258A |
| DC_66A_n260A DC_66A_n260D DC_66A_n260E DC_66A_n260F DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O DC_66A_n260P DC_66A_n260Q | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O DC_66A_n260P DC_66A_n260Q |
| DC_66A_n260(2A) DC_66A_n260(3A) DC_66A_n260(4A) DC_66A_n260(5A) DC_66A_n260(6A) DC_66A_n260(7A) DC_66A_n260(8A) DC_66A_n260(9A) DC_66A_n260(10A) DC_66A_n260(A-I) DC_66A_n260(D-G) DC_66A_n260(D-H) DC_66A_n260(D-I) DC_66A_n260(D-O) DC_66A_n260(D-P) DC_66A_n260(D-Q) DC_66A_n260(E-O) DC_66A_n260(E-P) DC_66A_n260(E-Q) DC_66A_n260(G-I) DC_66A_n260(2G) DC_66A_n260(2H) DC_66A_n260(2O) DC_66A_n260(3O) DC_66A_n260(4O) DC_66A_n260(2P) DC_66A_n260(3P) DC_66A_n260(4P) DC_66A_n260(2A-O) DC_66A_n260(A-2O) DC_66A_n260(2A-G) DC_66A_n260(A-2G) DC_66A_n260(2A-2G) DC_66A_n260(2G-O) DC_66A_n260(2A-2G-O) DC_66A_n260(A-2H) DC_66A_n260(2A-H) DC_66A_n260(2A-2H) DC_66A_n260(2A-2O) DC_66A_n260(2A-3O) DC_66A_n260(A-4O) DC_66A_n260(2A-4O) DC_66A_n260(3A-2O) DC_66A_n260(3A-2G) DC_66A_n260(4A-G) DC_66A_n260(4A-2G) DC_66A_n260(4A-O) DC_66A_n260(4A-2O) DC_66A_n260(A-O) DC_66A_n260(A-G) DC_66A_n260(G-O) DC_66A_n260(A-G-O) DC_66A_n260(2A-G-O) DC_66A_n260(A-2G-O) DC_66A_n260(A-H) DC_66A_n260(A-3O) DC_66A_n260(3A-O) DC_66A_n260(3A-O-P) DC_66A_n260(3A-P) DC_66A_n260(3A-G) DC_66A_n260(2D) DC_66A_n260(3G) DC_66A_n260(4G) DC_66A_n260(A-D) DC_66A_n260(2A-D) DC_66A_n260(A-D-O) DC_66A_n260(2A-D-O) DC_66A_n260(D-2O) DC_66A_n260(A-D-2O) DC_66A_n260(2A-D-2O) DC_66A_n260(2A-O-P) DC_66A_n260(A-2D) DC_66A_n260(2A-2D) DC_66A_n260(A-P) DC_66A_n260(A-P-Q) DC_66A_n260(2A-P) DC_66A_n260(A-2P) DC_66A_n260(2A-2P) DC_66A_n260(3A-3O) DC_66A_n260(D-2G) DC_66A_n260(2D-O) DC_66A_n260(G-H) DC_66A_n260(G-2O) DC_66A_n260(2G-2O) DC_66A_n260(G-3O) DC_66A_n260(2G-3O) DC_66A_n260(G-4O) DC_66A_n260(2G-4O) DC_66A_n260(3G-O) DC_66A_n260(4G-O) DC_66A_n260(H-O) DC_66A_n260(2H-O) DC_66A_n260(2A-2G-2O) DC_66A_n260(6A-2O) DC_66A_n260(8A-2O) DC_66A_n260(2A-2O-2P) DC_66A_n260(6A-3O) DC_66A_n260(4A-4O) DC_66A_n260(6A-2P) DC_66A_n260(2O-2P) DC_66A_n260(2A-4P) DC_66A_n260(4A-2Q) DC_66A_n260(2A-2O-2Q) DC_66A_n260(A-Q) DC_66A_n260(P-Q) DC_66A_n260(A-O-P) DC_66A_n260(O-P) DC_66A_n260(2A-2O-P-Q) DC_66A_n260(2A-2O-P) DC_66A_n260(2A-2O-Q) DC_66A_n260(2A-2Q) DC_66A_n260(2A-3G) DC_66A_n260(2A-3O-P) DC_66A_n260(2A-3O-Q) DC_66A_n260(2A-3P) DC_66A_n260(2A-G-2O) DC_66A_n260(2A-G-3O) DC_66A_n260(2A-G-H) DC_66A_n260(2A-O-2P) DC_66A_n260(2A-O-2Q) DC_66A_n260(2A-O-3P) DC_66A_n260(2A-O-P-Q) DC_66A_n260(2A-O-Q) DC_66A_n260(2A-P-Q) DC_66A_n260(2A-Q) DC_66A_n260(2O-2Q) DC_66A_n260(2O-P-Q) DC_66A_n260(2O-P) DC_66A_n260(2O-Q) DC_66A_n260(2Q) DC_66A_n260(3A-2G-O) DC_66A_n260(3A-2O-P) DC_66A_n260(3A-2O-Q) DC_66A_n260(3A-2P) DC_66A_n260(3A-2Q) DC_66A_n260(3A-3P) DC_66A_n260(3A-4O) DC_66A_n260(3A-D-O) DC_66A_n260(3A-D) DC_66A_n260(3A-G-2O) DC_66A_n260(3A-G-O) DC_66A_n260(3A-H) DC_66A_n260(3A-O-2P) DC_66A_n260(3A-O-2Q) DC_66A_n260(3A-O-P-Q) DC_66A_n260(3A-O-Q) DC_66A_n260(3A-P-Q) DC_66A_n260(3A-Q) DC_66A_n260(3O-P) DC_66A_n260(3O-Q) DC_66A_n260(4A-2P) DC_66A_n260(4A-3O) DC_66A_n260(4A-D) DC_66A_n260(4A-G-O) DC_66A_n260(4A-O-P) DC_66A_n260(4A-O-Q) DC_66A_n260(4A-P-Q) DC_66A_n260(4A-P) DC_66A_n260(4A-Q) DC_66A_n260(5A-2O) DC_66A_n260(5A-2P) DC_66A_n260(5A-3O) DC_66A_n260(5A-G) DC_66A_n260(5A-O-P) DC_66A_n260(5A-O) DC_66A_n260(5A-P) DC_66A_n260(5A-Q) DC_66A_n260(6A-O-P) DC_66A_n260(6A-O) DC_66A_n260(6A-P) DC_66A_n260(7A-2O) DC_66A_n260(7A-O) DC_66A_n260(7A-P) DC_66A_n260(8A-O) DC_66A_n260(9A-O) DC_66A_n260(A-2G-2O) DC_66A_n260(A-2G-3O) DC_66A_n260(A-2O-2P) DC_66A_n260(A-2O-2Q) DC_66A_n260(A-2O-P-Q) DC_66A_n260(A-2O-P) DC_66A_n260(A-2O-Q) DC_66A_n260(A-2Q) DC_66A_n260(A-3G-O) DC_66A_n260(A-3G) DC_66A_n260(A-3O-P) DC_66A_n260(A-3O-Q) DC_66A_n260(A-3P) DC_66A_n260(A-4G) DC_66A_n260(A-4P) DC_66A_n260(A-D-G) DC_66A_n260(A-E) DC_66A_n260(A-G-2O) DC_66A_n260(A-G-3O) DC_66A_n260(A-G-4O) DC_66A_n260(A-G-H) DC_66A_n260(A-H-O) DC_66A_n260(A-O-2P) DC_66A_n260(A-O-2Q) DC_66A_n260(A-O-3P) DC_66A_n260(A-O-P-Q) DC_66A_n260(A-O-Q) DC_66A_n260(G-H-O) DC_66A_n260(O-2P) DC_66A_n260(O-2Q) DC_66A_n260(O-3P) DC_66A_n260(O-P-Q) DC_66A_n260(O-Q) | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260O DC_66A_n260P DC_66A_n260Q |
| DC_66A-66A_n260A DC_66A-66A_n260G DC_66A-66A_n260H DC_66A-66A_n260I DC_66A-66A_n260J DC_66A-66A_n260K DC_66A-66A_n260L DC_66A-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_66A-66A_n260(2A) DC_66A-66A_n260(2G) DC_66A-66A_n260(2H) DC_66A-66A_n260(3A) DC_66A-66A_n260(4A) DC_66A-66A_n260(5A) DC_66A-66A_n260(6A) DC_66A-66A_n260(A-G) DC_66A-66A_n260(A-H) DC_66A-66A_n260(A-2G) DC_66A-66A_n260(G-H) DC_66A-66A_n260(2A-G) DC_66A-66A_n260(2A-2G) DC_66A-66A_n260(3A-G) | DC_66A_n260A DC_66A_n260G DC_66A_n260H |
| DC_66A_n261A DC_66A_n261B DC_66A_n261C DC_66A_n261D DC_66A_n261E DC_66A_n261F DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M DC_66A_n261O DC_66A_n261P DC_66A_n261Q | DC_66A_n261A DC_66A_n261B DC_66A_n261C DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M DC_66A_n261O |
| DC_66A_n261(2A) DC_66A_n261(3A) DC_66A_n261(4A) DC_66A_n261(2G) DC_66A_n261(D-G) DC_66A_n261(D-H) DC_66A_n261(D-I) DC_66A_n261(D-O) DC_66A_n261(D-P) DC_66A_n261(D-Q) DC_66A_n261(E-O) DC_66A_n261(E-P) DC_66A_n261(E-Q) DC_66A_n261(2H) DC_66A_n261(2I) DC_66A_n261(A-H) DC_66A_n261(A-I) DC_66A_n261(A-J) DC_66A_n261(A-K) DC_66A_n261(A-L) DC_66A_n261(A-D) DC_66A_n261(A-D-H) DC_66A_n261(A-G) DC_66A_n261(A-G-H) DC_66A_n261(G-I) DC_66A_n261(G-J) DC_66A_n261(A-G-I) DC_66A_n261(A-H-I) DC_66A_n261(G-H) DC_66A_n261(H-I) DC_66A_n261(A-D-2O) DC_66A_n261(A-2D) DC_66A_n261(A-2G) DC_66A_n261(A-2G-2O) DC_66A_n261(A-3G-O) DC_66A_n261(A-4G) DC_66A_n261(A-2H) DC_66A_n261(A-2I) DC_66A_n261(A-4O) DC_66A_n261(A-7O) DC_66A_n261(A-2P) DC_66A_n261(A-2Q) DC_66A_n261(2A-G) DC_66A_n261(2A-H) DC_66A_n261(2A-I) DC_66A_n261(3A-G) DC_66A_n261(2A-2G-O) DC_66A_n261(2A-2G) DC_66A_n261(2A-2O) DC_66A_n261(2A-3G) DC_66A_n261(2A-3O) DC_66A_n261(2A-4O) DC_66A_n261(2A-5O) DC_66A_n261(2A-6O) DC_66A_n261(2A-D-O) DC_66A_n261(2A-D) DC_66A_n261(2A-G-2O) DC_66A_n261(2A-G-O) DC_66A_n261(2A-O) DC_66A_n261(2A-P) DC_66A_n261(2A-Q) DC_66A_n261(2D) DC_66A_n261(2G-2O) DC_66A_n261(2G-O) DC_66A_n261(2O) DC_66A_n261(2P) DC_66A_n261(2Q) DC_66A_n261(3A-2G) DC_66A_n261(3A-2O) DC_66A_n261(3A-3O) DC_66A_n261(3A-4O) DC_66A_n261(3A-5O) DC_66A_n261(3A-D) DC_66A_n261(3A-G-O) DC_66A_n261(3A-O) DC_66A_n261(3G-O) DC_66A_n261(3G) DC_66A_n261(3O) DC_66A_n261(4A-2O) DC_66A_n261(4A-3O) DC_66A_n261(4A-4O) DC_66A_n261(4A-G) DC_66A_n261(4A-O) DC_66A_n261(4G) DC_66A_n261(4O) DC_66A_n261(5A-2O) DC_66A_n261(5A-3O) DC_66A_n261(5A-O) DC_66A_n261(5A) DC_66A_n261(5O) DC_66A_n261(6A-2O) DC_66A_n261(6A-O) DC_66A_n261(6A) DC_66A_n261(6O) DC_66A_n261(7A-O) DC_66A_n261(7A) DC_66A_n261(7O) DC_66A_n261(8A) DC_66A_n261(A-2G-O) DC_66A_n261(A-2O) DC_66A_n261(A-3G) DC_66A_n261(A-3O) DC_66A_n261(A-5O) DC_66A_n261(A-6O) DC_66A_n261(A-D-G) DC_66A_n261(A-D-O) DC_66A_n261(A-E) DC_66A_n261(A-G-2O) DC_66A_n261(A-G-O) DC_66A_n261(A-O-P) DC_66A_n261(A-O-Q) DC_66A_n261(A-O) DC_66A_n261(A-P-Q) DC_66A_n261(A-P) DC_66A_n261(A-Q) DC_66A_n261(D-2O) DC_66A_n261(G-2O) DC_66A_n261(G-O) DC_66A_n261(O-P) DC_66A_n261(O-Q) DC_66A_n261(P-Q) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_66A-66A_n261A DC_66A-66A_n261G DC_66A-66A_n261H DC_66A-66A_n261I DC_66A-66A_n261J DC_66A-66A_n261K DC_66A-66A_n261L DC_66A-66A_n261M | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |
| DC_66A-66A_n261(2A) DC_66A-66A_n261(2G) DC_66A-66A_n261(3A) DC_66A-66A_n261(4A) DC_66A-66A_n261(A-G) DC_66A-66A_n261(A-G-H) DC_66A-66A_n261(A-G-I) DC_66A-66A_n261(A-2G) DC_66A-66A_n261(A-H) DC_66A-66A_n261(A-I) DC_66A-66A_n261(A-J) DC_66A-66A_n261(A-K) DC_66A-66A_n261(A-L) DC_66A-66A_n261(G-H) DC_66A-66A_n261(G-I) DC_66A-66A_n261(G-J) DC_66A-66A_n261(H-I) DC_66A-66A_n261(2H) DC_66A-66A_n261(2A-G) DC_66A-66A_n261(2A-H) DC_66A-66A_n261(2A-I) DC_66A-66A_n261(3A-G) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_71A_n257A DC_71A_n257G DC_71A_n257H DC_71A_n257I DC_71A_n257J DC_71A_n257K DC_71A_n257L DC_71A_n257M | DC_71A_n257A DC_71A_n257G DC_71A_n257H DC_71A_n257I DC_71A_n257J DC_71A_n257K DC_71A_n257L DC_71A_n257M |
| DC_71A_n258A DC_71A_n258G DC_71A_n258H DC_71A_n258I DC_71A_n258J DC_71A_n258K DC_71A_n258L DC_71A_n258M | DC_71A_n258A DC_71A_n258G DC_71A_n258H DC_71A_n258I DC_71A_n258J DC_71A_n258K DC_71A_n258L DC_71A_n258M |
| DC_71A_n260A DC_71A_n260G DC_71A_n260H DC_71A_n260I DC_71A_n260J DC_71A_n260K DC_71A_n260L DC_71A_n260M DC_71A_n260O DC_71A_n260P DC_71A_n260Q | DC_71A_n260A DC_71A_n260G DC_71A_n260H DC_71A_n260I DC_71A_n260J DC_71A_n260K DC_71A_n260L DC_71A_n260M DC_71A_n260O DC_71A_n260P DC_71A_n260Q |
| DC_71A_n261A DC_71A_n261G DC_71A_n261H DC_71A_n261I DC_71A_n261J DC_71A_n261K DC_71A_n261L DC_71A_n261M DC_71A_n261O DC_71A_n261P DC_71A_n261Q | DC_71A_n261A DC_71A_n261G DC_71A_n261H DC_71A_n261I DC_71A_n261J DC_71A_n261K DC_71A_n261L DC_71A_n261M DC_71A_n261O DC_71A_n261P DC_71A_n261Q |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations. NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

#### 5.5B.5.2	Inter-band EN-DC configurations including FR2 (three bands)

Table 5.5B.5.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |  |
| --- | --- | --- |
| DC_1A-3A_n257A2 DC_1A-3A_n257D2 DC_1A-3A_n257E2 DC_1A-3A_n257F2 DC_1A-3A_n257G DC_1A-3A_n257H DC_1A-3A_n257I DC_1A-3A_n257J DC_1A-3A_n257K DC_1A-3A_n257L DC_1A-3A_n257M DC_1A-3C_n257A DC_1A-3C_n257D DC_1A-3C_n257E DC_1A-3C_n257F DC_1A-3C_n257G DC_1A-3C_n257H DC_1A-3C_n257I DC_1A-3C_n257J DC_1A-3C_n257K DC_1A-3C_n257L DC_1A-3C_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M |  |
| DC_1A-3A_n258A DC_1A-3A_n258B DC_1A-3A_n258C DC_1A-3A_n258D DC_1A-3A_n258E DC_1A-3A_n258F DC_1A-3A_n258G DC_1A-3A_n258H DC_1A-3A_n258I DC_1A-3A_n258J DC_1A-3A_n258K DC_1A-3A_n258L DC_1A-3A_n258M DC_1A-3C_n258A DC_1A-3C_n258B DC_1A-3C_n258C DC_1A-3C_n258D DC_1A-3C_n258E DC_1A-3C_n258F DC_1A-3C_n258G DC_1A-3C_n258H DC_1A-3C_n258I DC_1A-3C_n258J DC_1A-3C_n258K DC_1A-3C_n258L DC_1A-3C_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_3A_n258A DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3C_n258A DC_3C_n258D DC_3C_n258E DC_3C_n258F DC_3C_n258G DC_3C_n258H DC_3C_n258I |  |
| DC_1A-5A_n257A2 DC_1A-5A_n257D DC_1A-5A_n257E DC_1A-5A_n257F DC_1A-5A_n257G DC_1A-5A_n257H DC_1A-5A_n257I DC_1A-5A_n257J DC_1A-5A_n257K DC_1A-5A_n257L DC_1A-5A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I |  |
| DC_1A-7A_n257A2 DC_1A-7A_n257D DC_1A-7A_n257E DC_1A-7A_n257F DC_1A-7A_n257G DC_1A-7A_n257H DC_1A-7A_n257I DC_1A-7A_n257J DC_1A-7A_n257K DC_1A-7A_n257L DC_1A-7A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |  |
| DC_1A-7A-7A_n257A2 DC_1A-7A-7A_n257D DC_1A-7A-7A_n257E DC_1A-7A-7A_n257F DC_1A-7A-7A_n257G DC_1A-7A-7A_n257H DC_1A-7A-7A_n257I DC_1A-7A-7A_n257J DC_1A-7A-7A_n257K DC_1A-7A-7A_n257L DC_1A-7A-7A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |  |
| DC_1A-7A_n258A DC_1A-7A_n258B DC_1A-7A_n258C DC_1A-7A_n258D DC_1A-7A_n258E DC_1A-7A_n258F DC_1A-7A_n258G DC_1A-7A_n258H DC_1A-7A_n258I DC_1A-7A_n258J DC_1A-7A_n258K DC_1A-7A_n258L DC_1A-7A_n258M DC_1A-7C_n258A DC_1A-7C_n258B DC_1A-7C_n258C DC_1A-7C_n258D DC_1A-7C_n258E DC_1A-7C_n258F DC_1A-7C_n258G DC_1A-7C_n258H DC_1A-7C_n258I DC_1A-7C_n258J DC_1A-7C_n258K DC_1A-7C_n258L DC_1A-7C_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_7A_n258A DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n258A DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I |  |
| DC_1A-8A_n257A2 DC_1A-8A_n257D DC_1A-8A_n257E DC_1A-8A_n257F DC_1A-8A_n257G DC_1A-8A_n257H DC_1A-8A_n257I DC_1A-8A_n257J DC_1A-8A_n257K DC_1A-8A_n257L DC_1A-8A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |  |
| DC_1A-8A_n258A DC_1A-8A_n258D DC_1A-8A_n258E DC_1A-8A_n258F DC_1A-8A_n258G DC_1A-8A_n258H DC_1A-8A_n258I DC_1A-8A_n258J DC_1A-8A_n258K DC_1A-8A_n258L DC_1A-8A_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_8A_n258A DC_8A_n258D DC_8A_n258G DC_8A_n258H DC_8A_n258I |  |
| DC_1A-11A_n257A DC_1A-11A_n257D DC_1A-11A_n257G DC_1A-11A_n257H DC_1A-11A_n257I | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |  |
| DC_1A-18A_n257A2 DC_1A-18A_n257D DC_1A-18A_n257E DC_1A-18A_n257F DC_1A-18A_n257G DC_1A-18A_n257H DC_1A-18A_n257I DC_1A-18A_n257J DC_1A-18A_n257K DC_1A-18A_n257L DC_1A-18A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |  |
| DC_1A-19A_n257A2 DC_1A-19A_n257D2 DC_1A-19A_n257E2 DC_1A-19A_n257F2 DC_1A-19A_n257G DC_1A-19A_n257H DC_1A-19A_n257I DC_1A-19A_n257J DC_1A-19A_n257K DC_1A-19A_n257L DC_1A-19A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I |  |
| DC_1A-21A_n257A2 DC_1A-21A_n257D2 DC_1A-21A_n257E2 DC_1A-21A_n257F2 DC_1A-21A_n257G DC_1A-21A_n257H DC_1A-21A_n257I DC_1A-21A_n257J DC_1A-21A_n257K DC_1A-21A_n257L DC_1A-21A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |  |
| DC_1A-26A_n258A DC_1A-26A_n258B DC_1A-26A_n258C DC_1A-26A_n258D DC_1A-26A_n258E DC_1A-26A_n258F DC_1A-26A_n258G DC_1A-26A_n258H DC_1A-26A_n258I DC_1A-26A_n258J DC_1A-26A_n258K DC_1A-26A_n258L DC_1A-26A_n258M | DC_1A_n258A DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_26A_n258A DC_26A_n258G DC_26A_n258H DC_26A_n258I |  |
| DC_1A-28A_n257A2 DC_1A-28A_n257D2 DC_1A-28A_n257E2 DC_1A-28A_n257F2 DC_1A-28A_n257G2 DC_1A-28A_n257H2 DC_1A-28A_n257I2 DC_1A-28A_n257J DC_1A-28A_n257K DC_1A-28A_n257L DC_1A-28A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |  |
| DC_1A-28A_n258A DC_1A-28A_n258B DC_1A-28A_n258C DC_1A-28A_n258D DC_1A-28A_n258E DC_1A-28A_n258F DC_1A-28A_n258G DC_1A-28A_n258H DC_1A-28A_n258I DC_1A-28A_n258J DC_1A-28A_n258K DC_1A-28A_n258L DC_1A-28A_n258M | DC_1A_n258A DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |  |
| DC_1A-41A_n257A DC_1A-41A_n257D DC_1A-41A_n257E DC_1A-41A_n257F DC_1A-41A_n257G DC_1A-41A_n257H DC_1A-41A_n257I DC_1A-41A_n257J DC_1A-41A_n257K DC_1A-41A_n257L DC_1A-41A_n257M DC_1A-41C_n257A DC_1A-41C_n257D DC_1A-41C_n257E DC_1A-41C_n257F DC_1A-41C_n257G DC_1A-41C_n257H DC_1A-41C_n257I DC_1A-41C_n257J DC_1A-41C_n257K DC_1A-41C_n257L DC_1A-41C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |  |
| DC_1A-42A_n257A DC_1A-42A_n257D DC_1A-42A_n257E DC_1A-42A_n257F DC_1A-42A_n257G DC_1A-42A_n257H DC_1A-42A_n257I DC_1A-42A_n257J DC_1A-42A_n257K DC_1A-42A_n257L DC_1A-42A_n257M DC_1A-42C_n257A DC_1A-42C_n257D DC_1A-42C_n257E DC_1A-42C_n257F DC_1A-42C_n257G DC_1A-42C_n257H DC_1A-42C_n257I DC_1A-42C_n257J DC_1A-42C_n257K DC_1A-42C_n257L DC_1A-42C_n257M DC_1A-42D_n257A DC_1A-42D_n257D DC_1A-42D_n257E DC_1A-42D_n257F DC_1A-42D_n257G DC_1A-42D_n257H DC_1A-42D_n257I DC_1A-42D_n257J DC_1A-42D_n257K DC_1A-42D_n257L DC_1A-42D_n257M DC_1A-42E_n257A DC_1A-42E_n257D DC_1A-42E_n257E DC_1A-42E_n257F DC_1A-42E_n257G DC_1A-42E_n257H DC_1A-42E_n257I DC_1A-42E_n257J DC_1A-42E_n257K DC_1A-42E_n257L DC_1A-42E_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |  |
| DC_2A-5A_n257A2 DC_2A-5A_n257G DC_2A-5A_n257H DC_2A-5A_n257I DC_2A-5A_n257J DC_2A-5A_n257K DC_2A-5A_n257L DC_2A-5A_n257M | DC_2A_n257A DC_2A_n257G DC_5A_n257A DC_5A_n257G |  |
| DC_2A-5A_n260A DC_2A-5A_n260G DC_2A-5A_n260H DC_2A-5A_n260I DC_2A-5A_n260J DC_2A-5A_n260K DC_2A-5A_n260L DC_2A-5A_n260M DC_2A-5A_n260O DC_2A-5A_n260P DC_2A-5A_n260Q | DC_2A_n260A DC_5A_n260A DC_2A_n260G DC_5A_n260G DC_2A_n260H DC_5A_n260H DC_2A_n260I DC_5A_n260I DC_2A_n260J DC_5A_n260J DC_2A_n260K DC_5A_n260K DC_2A_n260L DC_5A_n260L DC_2A_n260M DC_5A_n260M DC_2A_n260O DC_5A_n260O |  |
| DC_2A-2A-5A_n260A DC_2A-2A-5A_n260G DC_2A-2A-5A_n260H DC_2A-2A-5A_n260I DC_2A-2A-5A_n260J DC_2A-2A-5A_n260K DC_2A-2A-5A_n260L DC_2A-2A-5A_n260M | DC_2A_n260A DC_5A_n260A DC_2A_n260G DC_5A_n260G DC_2A_n260H DC_5A_n260H DC_2A_n260I DC_5A_n260I DC_2A_n260J DC_5A_n260J DC_2A_n260K DC_5A_n260K DC_2A_n260L DC_5A_n260L DC_2A_n260M DC_5A_n260M |  |
| DC_2A-5A_n261A DC_2A-5A_n261G DC_2A-5A_n261H DC_2A-5A_n261I DC_2A-5A_n261J DC_2A-5A_n261K DC_2A-5A_n261L DC_2A-5A_n261M DC_2A-5A_n261O DC_2A-5A_n261P DC_2A-5A_n261Q | DC_2A_n261A DC_5A_n261A DC_2A_n261G DC_5A_n261G DC_2A_n261H DC_5A_n261H DC_2A_n261I DC_5A_n261I DC_2A_n261O DC_5A_n261O |  |
| DC_2A-5A_n261(2A) DC_2A-5A_n261(3A) DC_2A-5A_n261(4A) DC_2A-5A_n261(A-G) DC_2A-5A_n261(A-H) DC_2A-5A_n261(A-I) DC_2A-5A_n261(A-J) DC_2A-5A_n261(A-K) DC_2A-5A_n261(2A-G) DC_2A-5A_n261(A-L) DC_2A-5A_n261(A-2G) DC_2A-5A_n261(2A-H) DC_2A-5A_n261(2A-I) DC_2A-5A_n261(A-G-H) DC_2A-5A_n261(A-G-I) DC_2A-5A_n261(3A-G) DC_2A-5A_n261(G-H) DC_2A-5A_n261(G-I) DC_2A-5A_n261(G-J) DC_2A-5A_n261(2G) DC_2A-5A_n261(2H) DC_2A-5A_n261(H-I) | DC_2A_n261A DC_5A_n261A DC_2A_n261G DC_5A_n261G DC_2A_n261H DC_5A_n261H DC_2A_n261I DC_5A_n261I DC_2A_n261J DC_5A_n261J DC_2A_n261K DC_5A_n261K DC_2A_n261L DC_5A_n261L DC_2A_n261M DC_5A_n261M |  |
| DC_2A-12A_n260A DC_2A-12A_n260G DC_2A-12A_n260H DC_2A-12A_n260I DC_2A-12A_n260J DC_2A-12A_n260K DC_2A-12A_n260L DC_2A-12A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M |  |
| DC_2A-7A_n257A DC_2A-7A_n257G DC_2A-7A_n257H DC_2A-7A_n257I DC_2A-7A_n257J DC_2A-7A_n257K DC_2A-7A_n257L DC_2A-7A_n257M | DC_2A_n257A DC_2A_n257G DC_7A_n257A DC_7A_n257G |  |
| DC_2A-7A_n258A DC_2A-7A_n258G DC_2A-7A_n258H DC_2A-7A_n258I DC_2A-7A_n258J DC_2A-7A_n258K DC_2A-7A_n258L DC_2A-7A_n258M | DC_2A_n258A DC_7A_n258A DC_2A_n258G DC_7A_n258G DC_2A_n258H DC_7A_n258H DC_2A_n258I DC_7A_n258I |  |
| DC_2A-7A_n260A DC_2A-7A_n260G DC_2A-7A_n260H DC_2A-7A_n260I DC_2A-7A_n260J DC_2A-7A_n260K DC_2A-7A_n260L DC_2A-7A_n260M DC_2A-7A_n260O DC_2A-7A_n260P DC_2A-7A_n260Q | DC_2A_n260A DC_2A_n260G DC_2A_n260O DC_7A_n260A DC_7A_n260G DC_7A_n260O |  |
| DC_2A-7A_n261A DC_2A-7A_n261G DC_2A-7A_n261H DC_2A-7A_n261I DC_2A-7A_n261J DC_2A-7A_n261K DC_2A-7A_n261L DC_2A-7A_n261M DC_2A-7A_n261O DC_2A-7A_n261P DC_2A-7A_n261Q | DC_2A_n261A DC_2A_n261G DC_2A_n261O DC_7A_n261A DC_7A_n261G DC_7A_n261O |  |
| DC_2A-12A_n257A DC_2A-12A_n257G DC_2A-12A_n257H DC_2A-12A_n257I DC_2A-12A_n257J DC_2A-12A_n257K DC_2A-12A_n257L DC_2A-12A_n257M | DC_2A_n257A DC_2A_n257G DC_12A_n257A DC_12A_n257G |  |
| DC_2A-12A_n258A DC_2A-12A_n258G DC_2A-12A_n258H DC_2A-12A_n258I DC_2A-12A_n258J DC_2A-12A_n258K DC_2A-12A_n258L DC_2A-12A_n258M | DC_2A_n258A DC_2A_n258G DC_12A_n258A DC_12A_n258G |  |
| DC_2A-2A-12A_n260A DC_2A-2A-12A_n260G DC_2A-2A-12A_n260H DC_2A-2A-12A_n260I DC_2A-2A-12A_n260J DC_2A-2A-12A_n260K DC_2A-2A-12A_n260L DC_2A-2A-12A_n260M DC_2A-12A_n260O DC_2A-12A_n260P DC_2A-12A_n260Q | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2A_n260O DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_12A_n260O |  |
| DC_2A-13A_n257A2 | DC_2A_n257A DC_13A_n257A |  |
| DC_2A-12A_n261A DC_2A-12A_n261G DC_2A-12A_n261H DC_2A-12A_n261I DC_2A-12A_n261J DC_2A-12A_n261K DC_2A-12A_n261L DC_2A-12A_n261M DC_2A-12A_n261O DC_2A-12A_n261P DC_2A-12A_n261Q | DC_2A_n261A DC_2A_n261G DC_2A_n261O DC_12A_n261A DC_12A_n261G DC_12A_n261O |  |
| DC_2A-13A_n260A2 DC_2A-13A_n260G DC_2A-13A_n260H DC_2A-13A_n260I DC_2A-13A_n260J DC_2A-13A_n260K DC_2A-13A_n260L DC_2A-13A_n260M | DC_2A_n260A DC_13A_n260A DC_2A_n260G DC_13A_n260G DC_2A_n260H DC_13A_n260H DC_2A_n260I DC_13A_n260I DC_2A_n260J DC_13A_n260J DC_2A_n260K DC_13A_n260K DC_2A_n260L DC_13A_n260L DC_2A_n260M DC_13A_n260M |  |
| DC_2A-13A_n260(2A) DC_2A-13A_n260(3A) DC_2A-13A_n260(4A) DC_2A-13A_n260(5A) DC_2A-13A_n260(6A) DC_2A-13A_n260(2G) DC_2A-13A_n260(2H) DC_2A-13A_n260(A-G) DC_2A-13A_n260(A-H) DC_2A-13A_n260(A-2G) DC_2A-13A_n260(2A-G) DC_2A-13A_n260(2A-2G) DC_2A-13A_n260(3A-G) DC_2A-13A_n260(G-H) | DC_2A_n260A DC_13A_n260A DC_2A_n260G DC_13A_n260G DC_2A_n260H DC_13A_n260H |  |
| DC_2A-13A_n261A DC_2A-13A_n261G DC_2A-13A_n261H DC_2A-13A_n261I DC_2A-13A_n261J DC_2A-13A_n261K DC_2A-13A_n261L DC_2A-13A_n261M | DC_2A_n261A DC_13A_n261A DC_2A_n261G DC_13A_n261G DC_2A_n261H DC_13A_n261H DC_2A_n261I DC_13A_n261I DC_2A_n261J DC_13A_n261J DC_2A_n261K DC_13A_n261K DC_2A_n261L DC_13A_n261L DC_2A_n261M DC_13A_n261M |  |
| DC_2A-2A-13A_n261A DC_2A-2A-13A_n261G DC_2A-2A-13A_n261H DC_2A-2A-13A_n261I DC_2A-2A-13A_n261J DC_2A-2A-13A_n261K DC_2A-2A-13A_n261L DC_2A-2A-13A_n261M | DC_2A_n261A DC_13A_n261A DC_2A_n261G DC_13A_n261G DC_2A_n261H DC_13A_n261H DC_2A_n261I DC_13A_n261I DC_2A_n261J DC_13A_n261J DC_2A_n261K DC_13A_n261K DC_2A_n261L DC_13A_n261L DC_2A_n261M DC_13A_n261M |  |
| DC_2A-13A_n261(2A) DC_2A-13A_n261(3A) DC_2A-13A_n261(4A) DC_2A-13A_n261(2G) DC_2A-13A_n261(2H) DC_2A-13A_n261(A-G) DC_2A-13A_n261(A-H) DC_2A-13A_n261(A-I) DC_2A-13A_n261(A-J) DC_2A-13A_n261(A-K) DC_2A-13A_n261(A-L) DC_2A-13A_n261(A-2G) DC_2A-13A_n261(A-G-H) DC_2A-13A_n261(A-G-I) DC_2A-13A_n261(2A-G) DC_2A-13A_n261(2A-H) DC_2A-13A_n261(2A-I) DC_2A-13A_n261(3A-G) DC_2A-13A_n261(G-H) DC_2A-13A_n261(G-I) DC_2A-13A_n261(G-J) DC_2A-13A_n261(H-I) | DC_2A_n261A DC_13A_n261A DC_2A_n261G DC_13A_n261G DC_2A_n261H DC_13A_n261H DC_2A_n261I DC_13A_n261I |  |
| DC_2A-14A_n260A DC_2A-14A_n260G DC_2A-14A_n260H DC_2A-14A_n260I DC_2A-14A_n260J DC_2A-14A_n260K DC_2A-14A_n260L DC_2A-14A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M |  |
| DC_2A-2A-14A_n260A DC_2A-2A-14A_n260G DC_2A-2A-14A_n260H DC_2A-2A-14A_n260I DC_2A-2A-14A_n260J DC_2A-2A-14A_n260K DC_2A-2A-14A_n260L DC_2A-2A-14A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M |  |
| DC_2A-28A_n258A DC_2A-28A_n258G DC_2A-28A_n258H DC_2A-28A_n258I | DC_2A_n258A DC_28A_n258A DC_2A_n258G DC_28A_n258G DC_2A_n258H DC_28A_n258H DC_2A_n258I DC_28A_n258I |  |
| DC_2A-29A_n260A DC_2A-29A_n260G DC_2A-29A_n260H DC_2A-29A_n260I DC_2A-29A_n260J DC_2A-29A_n260K DC_2A-29A_n260L DC_2A-29A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |  |
| DC_2A-2A-29A_n260A DC_2A-2A-29A_n260G DC_2A-2A-29A_n260H DC_2A-2A-29A_n260I DC_2A-2A-29A_n260J DC_2A-2A-29A_n260K DC_2A-2A-29A_n260L DC_2A-2A-29A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |  |
| DC_2A-30A_n260A DC_2A-30A_n260G DC_2A-30A_n260H DC_2A-30A_n260I DC_2A-30A_n260J DC_2A-30A_n260K DC_2A-30A_n260L DC_2A-30A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |  |
| DC_2A-2A-30A_n260A DC_2A-2A-30A_n260G DC_2A-2A-30A_n260H DC_2A-2A-30A_n260I DC_2A-2A-30A_n260J DC_2A-2A-30A_n260K DC_2A-2A-30A_n260L DC_2A-2A-30A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |  |
| DC_2A-46A_n258A DC_2A-46C_n258A DC_2A-46D_n258A | DC_2A_n258A |  |
| DC_2A-46A_n258(2A) DC_2A-46A_n258(3A) DC_2A-46A_n258(4A) DC_2A-46A_n258(5A) DC_2A-46C_n258(2A) DC_2A-46C_n258(3A) DC_2A-46C_n258(4A) DC_2A-46C_n258(5A) DC_2A-46D_n258(2A) DC_2A-46D_n258(3A) DC_2A-46D_n258(4A) DC_2A-46D_n258(5A) | DC_2A_n258A |  |
| DC_2A-46A_n260A DC_2A-46C_n260A DC_2A-46D_n260A DC_2A-46E_n260A DC_2A-46A_n260G DC_2A-46C_n260G DC_2A-46D_n260G DC_2A-46E_n260G DC_2A-46A_n260H DC_2A-46C_n260H DC_2A-46D_n260H DC_2A-46E_n260H DC_2A-46A_n260I DC_2A-46C_n260I DC_2A-46D_n260I DC_2A-46E_n260I DC_2A-46A_n260J DC_2A-46C_n260J DC_2A-46D_n260J DC_2A-46E_n260J DC_2A-46A_n260K DC_2A-46C_n260K DC_2A-46D_n260K DC_2A-46E_n260K DC_2A-46A_n260L DC_2A-46C_n260L DC_2A-46D_n260L DC_2A-46E_n260L DC_2A-46A_n260M DC_2A-46C_n260M DC_2A-46D_n260M DC_2A-46E_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |  |
| DC_2A-2A-46A_n260A DC_2A-2A-46C_n260A DC_2A-2A-46D_n260A DC_2A-2A-46E_n260A DC_2A-2A-46A_n260G DC_2A-2A-46C_n260G DC_2A-2A-46D_n260G DC_2A-2A-46E_n260G DC_2A-2A-46A_n260H DC_2A-2A-46C_n260H DC_2A-2A-46D_n260H DC_2A-2A-46E_n260H DC_2A-2A-46A_n260I DC_2A-2A-46C_n260I DC_2A-2A-46D_n260I DC_2A-2A-46E_n260I DC_2A-2A-46A_n260J DC_2A-2A-46C_n260J DC_2A-2A-46D_n260J DC_2A-2A-46E_n260J DC_2A-2A-46A_n260K DC_2A-2A-46C_n260K DC_2A-2A-46D_n260K DC_2A-2A-46E_n260K DC_2A-2A-46A_n260L DC_2A-2A-46C_n260L DC_2A-2A-46D_n260L DC_2A-2A-46E_n260L DC_2A-2A-46A_n260M DC_2A-2A-46C_n260M DC_2A-2A-46D_n260M DC_2A-2A-46E_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |  |
| DC_2A-46A_n261(A-G) DC_2A-46A_n261(A-H) DC_2A-46A_n261(A-I) DC_2A-46A_n261(A-J) DC_2A-46A_n261(A-K) DC_2A-46A_n261(A-L) DC_2A-46A_n261(G-H) DC_2A-46A_n261(2H) DC_2A-46A_n261(2G) DC_2A-46A_n261(2A) DC_2A-46C_n261(2A) DC_2A-46D_n261(2A) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |  |
| DC_2A-46A-46A_n261(A-G) DC_2A-46A-46A_n261(A-H) DC_2A-46A-46A_n261(A-I) DC_2A-46A-46A_n261(A-J) DC_2A-46A-46A_n261(A-K) DC_2A-46A-46A_n261(A-L) DC_2A-46A-46A_n261(G-H) DC_2A-46A-46A_n261(2A) DC_2A-46A-46A_n261(2G) DC_2A-46A-46A_n261(2H) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |  |
| DC_2A-46A-46A-46A_n261(A-G) DC_2A-46A-46A-46A_n261(A-H) DC_2A-46A-46A-46A_n261(A-I) DC_2A-46A-46A-46A_n261(A-J) DC_2A-46A-46A-46A_n261(A-K) DC_2A-46A-46A-46A_n261(A-L) DC_2A-46A-46A-46A_n261(G-H) DC_2A-46A-46A-46A_n261(2A) DC_2A-46A-46A-46A_n261(2G) DC_2A-46A-46A-46A_n261(2H) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |  |
| DC_2A-46A_n261A DC_2A-46A_n261G DC_2A-46A_n261H DC_2A-46A_n261I DC_2A-46A_n261J DC_2A-46A_n261K DC_2A-46A_n261L DC_2A-46A_n261M DC_2A-46C_n261A DC_2A-46D_n261A | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M |  |
| DC_2A-46A-46A_n261A DC_2A-46A-46A_n261G DC_2A-46A-46A_n261H DC_2A-46A-46A_n261I DC_2A-46A-46A_n261J DC_2A-46A-46A_n261K DC_2A-46A-46A_n261L DC_2A-46A-46A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M |  |
| DC_2A-46A-46A-46A_n261A DC_2A-46A-46A-46A_n261G DC_2A-46A-46A-46A_n261H DC_2A-46A-46A-46A_n261I DC_2A-46A-46A-46A_n261J DC_2A-46A-46A-46A_n261K DC_2A-46A-46A-46A_n261L DC_2A-46A-46A-46A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M |  |
| DC_2A-66A_n257A2 DC_2A-66A_n257G DC_2A-66A_n257H DC_2A-66A_n257I DC_2A-66A_n257J DC_2A-66A_n257K DC_2A-66A_n257L DC_2A-66A_n257M DC_2A-66A_n257(2A) | DC_2A_n257A DC_2A_n257G DC_66A_n257A DC_66A_n257G |  |
| DC_2A-66A_n258A DC_2A-66A_n258G DC_2A-66A_n258H DC_2A-66A_n258I DC_2A-66A_n258J DC_2A-66A_n258K DC_2A-66A_n258L DC_2A-66A_n258M | DC_2A_n258A DC_2A_n258G DC_66A_n258A DC_66A_n258G |  |
| DC_2A-66A_n260A DC_2A-66A_n260G DC_2A-66A_n260H DC_2A-66A_n260I DC_2A-66A_n260J DC_2A-66A_n260K DC_2A-66A_n260L DC_2A-66A_n260M DC_2A-66A_n260O DC_2A-66A_n260P DC_2A-66A_n260Q | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M DC_2A_n260O DC_66A_n260O |  |
| DC_2A-66A_n260(2A) DC_2A-66A_n260(3A) DC_2A-66A_n260(4A) DC_2A-66A_n260(5A) DC_2A-66A_n260(6A) DC_2A-66A_n260(2G) DC_2A-66A_n260(2H) DC_2A-66A_n260(A-G) DC_2A-66A_n260(A-H) DC_2A-66A_n260(A-2G) DC_2A-66A_n260(2A-G) DC_2A-66A_n260(2A-2G) DC_2A-66A_n260(3A-G) DC_2A-66A_n260(G-H) | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H |  |
| DC_2A-2A-66A_n260A DC_2A-2A-66A_n260G DC_2A-2A-66A_n260H DC_2A-2A-66A_n260I DC_2A-2A-66A_n260J DC_2A-2A-66A_n260K DC_2A-2A-66A_n260L DC_2A-2A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M |  |
| DC_2A-66A-66A_n260A DC_2A-66A-66A_n260G DC_2A-66A-66A_n260H DC_2A-66A-66A_n260I DC_2A-66A-66A_n260J DC_2A-66A-66A_n260K DC_2A-66A-66A_n260L DC_2A-66A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M |  |
| DC_2A-2A-66A-66A_n260A DC_2A-2A-66A-66A_n260G DC_2A-2A-66A-66A_n260H DC_2A-2A-66A-66A_n260I DC_2A-2A-66A-66A_n260J DC_2A-2A-66A-66A_n260K DC_2A-2A-66A-66A_n260L DC_2A-2A-66A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M |  |
| DC_2A-66A_n261A DC_2A-66A_n261G DC_2A-66A_n261H DC_2A-66A_n261I DC_2A-66A_n261J DC_2A-66A_n261K DC_2A-66A_n261L DC_2A-66A_n261M DC_2A-66A_n261O DC_2A-66A_n261P DC_2A-66A_n261Q | DC_2A_n261A DC_66A_n261A DC_2A_n261G DC_66A_n261G DC_2A_n261H DC_66A_n261H DC_2A_n261I DC_66A_n261I DC_2A_n261J DC_66A_n261J DC_2A_n261K DC_66A_n261K DC_2A_n261L DC_66A_n261L DC_2A_n261M DC_66A_n261M DC_2A_n261O DC_66A_n261O |  |
| DC_2A-2A-66A_n261A DC_2A-2A-66A_n261G DC_2A-2A-66A_n261H DC_2A-2A-66A_n261I DC_2A-2A-66A_n261J DC_2A-2A-66A_n261K DC_2A-2A-66A_n261L DC_2A-2A-66A_n261M | DC_2A_n261A DC_66A_n261A DC_2A_n261G DC_66A_n261G DC_2A_n261H DC_66A_n261H DC_2A_n261I DC_66A_n261I DC_2A_n261J DC_66A_n261J DC_2A_n261K DC_66A_n261K DC_2A_n261L DC_66A_n261L DC_2A_n261M DC_66A_n261M |  |
| DC_2A-66A-66A_n261A DC_2A-66A-66A_n261G DC_2A-66A-66A_n261H DC_2A-66A-66A_n261I DC_2A-66A-66A_n261J DC_2A-66A-66A_n261K DC_2A-66A-66A_n261L DC_2A-66A-66A_n261M | DC_2A_n261A DC_66A_n261A DC_2A_n261G DC_66A_n261G DC_2A_n261H DC_66A_n261H DC_2A_n261I DC_66A_n261I DC_2A_n261J DC_66A_n261J DC_2A_n261K DC_66A_n261K DC_2A_n261L DC_66A_n261L DC_2A_n261M DC_66A_n261M |  |
| DC_2A-66A_n261(2A) DC_2A-66A_n261(3A) DC_2A-66A_n261(4A) DC_2A-66A_n261(2G) DC_2A-66A_n261(2H) DC_2A-66A_n261(A-G) DC_2A-66A_n261(A-H) DC_2A-66A_n261(A-I) DC_2A-66A_n261(A-J) DC_2A-66A_n261(A-K) DC_2A-66A_n261(A-L) DC_2A-66A_n261(A-2G) DC_2A-66A_n261(A-G-H) DC_2A-66A_n261(A-G-I) DC_2A-66A_n261(2A-G) DC_2A-66A_n261(2A-H) DC_2A-66A_n261(2A-I) DC_2A-66A_n261(3A-G) DC_2A-66A_n261(G-H) DC_2A-66A_n261(G-I) DC_2A-66A_n261(G-J) DC_2A-66A_n261(H-I) | DC_2A_n261A DC_66A_n261A DC_2A_n261G DC_66A_n261G DC_2A_n261H DC_66A_n261H DC_2A_n261I DC_66A_n261I |  |
| DC_2A-66A-66A_n261(2A) DC_2A-66A-66A_n261(3A) DC_2A-66A-66A_n261(4A) DC_2A-66A-66A_n261(A-G) DC_2A-66A-66A_n261(A-H) DC_2A-66A-66A_n261(A-I) DC_2A-66A-66A_n261(A-J) DC_2A-66A-66A_n261(A-K) DC_2A-66A-66A_n261(A-L) DC_2A-66A-66A_n261(A-2G) DC_2A-66A-66A_n261(2A-G) DC_2A-66A-66A_n261(2A-H) DC_2A-66A-66A_n261(2A-I) DC_2A-66A-66A_n261(A-G-H) DC_2A-66A-66A_n261(A-G-I) DC_2A-66A-66A_n261(3A-G) DC_2A-66A-66A_n261(2G) DC_2A-66A-66A_n261(G-H) DC_2A-66A-66A_n261(G-I) DC_2A-66A-66A_n261(G-J) DC_2A-66A-66A_n261(2H) DC_2A-66A-66A_n261(H-I) | DC_2A_n261A DC_66A_n261A DC_2A_n261G DC_66A_n261G DC_2A_n261H DC_66A_n261H DC_2A_n261I DC_66A_n261I |  |
| DC_3A-3A-7A_n257A DC_3A-3A-7A_n257D DC_3A-3A-7A_n257E DC_3A-3A-7A_n257F DC_3A-3A-7A_n257G DC_3A-3A-7A_n257H DC_3A-3A-7A_n257I DC_3A-3A-7A_n257J DC_3A-3A-7A_n257K DC_3A-3A-7A_n257L DC_3A-3A-7A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |  |
| DC_2A-71A_n257A DC_2A-71A_n257G DC_2A-71A_n257H DC_2A-71A_n257I DC_2A-71A_n257J DC_2A-71A_n257K DC_2A-71A_n257L DC_2A-71A_n257M | DC_2A_n257A DC_2A_n257G DC_71A_n257A DC_71A_n257G |  |
| DC_2A-71A_n258A DC_2A-71A_n258G DC_2A-71A_n258H DC_2A-71A_n258I DC_2A-71A_n258J DC_2A-71A_n258K DC_2A-71A_n258L DC_2A-71A_n258M | DC_2A_n258A DC_2A_n258G DC_71A_n258A DC_71A_n258G |  |
| DC_2A-71A_n260A DC_2A-71A_n260G DC_2A-71A_n260H DC_2A-71A_n260I DC_2A-71A_n260J DC_2A-71A_n260K DC_2A-71A_n260L DC_2A-71A_n260M DC_2A-71A_n260O DC_2A-71A_n260P DC_2A-71A_n260Q | DC_2A_n260A DC_2A_n260G DC_2A_n260O DC_71A_n260A DC_71A_n260G DC_71A_n260O |  |
| DC_2A-71A_n261A DC_2A-71A_n261G DC_2A-71A_n261H DC_2A-71A_n261I DC_2A-71A_n261J DC_2A-71A_n261K DC_2A-71A_n261L DC_2A-71A_n261M DC_2A-71A_n261O DC_2A-71A_n261P DC_2A-71A_n261Q | DC_2A_n261A DC_2A_n261G DC_2A_n261O DC_71A_n261A DC_71A_n261G DC_71A_n261O |  |
| DC_3A-3A-7A-7A_n257A DC_3A-3A-7A-7A_n257D DC_3A-3A-7A-7A_n257E DC_3A-3A-7A-7A_n257F DC_3A-3A-7A-7A_n257G DC_3A-3A-7A-7A_n257H DC_3A-3A-7A-7A_n257I DC_3A-3A-7A-7A_n257J DC_3A-3A-7A-7A_n257K DC_3A-3A-7A-7A_n257L DC_3A-3A-7A-7A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |  |
| DC_(n)3AA-n257A DC_(n)3AA-n257G DC_(n)3AA-n257H DC_(n)3AA-n257I DC_(n)3AA-n257J DC_(n)3AA-n257K DC_(n)3AA-n257L DC_(n)3AA-n257M | DC_(n)3AA3 DC_3A_n257A |  |
| DC_3A-5A_n257A2 DC_3A-5A_n257D DC_3A-5A_n257E DC_3A-5A_n257F DC_3A-5A_n257G DC_3A-5A_n257H DC_3A-5A_n257I DC_3A-5A_n257J DC_3A-5A_n257K DC_3A-5A_n257L DC_3A-5A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I |  |
| DC_3A-7A_n257A2 DC_3A-7A_n257D DC_3A-7A_n257E DC_3A-7A_n257F DC_3A-7A_n257G DC_3A-7A_n257H DC_3A-7A_n257I DC_3A-7A_n257J DC_3A-7A_n257K DC_3A-7A_n257L DC_3A-7A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |  |
| DC_3A-7A_n258A DC_3A-7A_n258B DC_3A-7A_n258C DC_3A-7A_n258D DC_3A-7A_n258E DC_3A-7A_n258F DC_3A-7A_n258G DC_3A-7A_n258H DC_3A-7A_n258I DC_3A-7A_n258J DC_3A-7A_n258K DC_3A-7A_n258L DC_3A-7A_n258M DC_3C-7A_n258A DC_3C-7A_n258B DC_3C-7A_n258C DC_3C-7A_n258D DC_3C-7A_n258E DC_3C-7A_n258F DC_3C-7A_n258G DC_3C-7A_n258H DC_3C-7A_n258I DC_3C-7A_n258J DC_3C-7A_n258K DC_3C-7A_n258L DC_3C-7A_n258M DC_3A-7C_n258A DC_3A-7C_n258B DC_3A-7C_n258C DC_3A-7C_n258D DC_3A-7C_n258E DC_3A-7C_n258F DC_3A-7C_n258G DC_3A-7C_n258H DC_3A-7C_n258I DC_3A-7C_n258J DC_3A-7C_n258K DC_3A-7C_n258L DC_3A-7C_n258M DC_3C-7C_n258A DC_3C-7C_n258B DC_3C-7C_n258C DC_3C-7C_n258D DC_3C-7C_n258E DC_3C-7C_n258F DC_3C-7C_n258G DC_3C-7C_n258H DC_3C-7C_n258I DC_3C-7C_n258J DC_3C-7C_n258K DC_3C-7C_n258L DC_3C-7C_n258M | DC_3A_n258A DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3C_n258A DC_3C_n258D DC_3C_n258E DC_3C_n258F DC_3C_n258G DC_3C_n258H DC_3C_n258I DC_7A_n258A DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n258A DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I |  |
| DC_3A-7A-7A_n257A2 DC_3A-7A-7A_n257D DC_3A-7A-7A_n257E DC_3A-7A-7A_n257F DC_3A-7A-7A_n257G DC_3A-7A-7A_n257H DC_3A-7A-7A_n257I DC_3A-7A-7A_n257J DC_3A-7A-7A_n257K DC_3A-7A-7A_n257L DC_3A-7A-7A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |  |
| DC_3A-8A_n257A DC_3A-8A_n257D DC_3A-8A_n257E DC_3A-8A_n257F DC_3A-8A_n257G DC_3A-8A_n257H DC_3A-8A_n257I DC_3A-8A_n257J DC_3A-8A_n257K DC_3A-8A_n257L DC_3A-8A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |  |
| DC_3C-8A_n257A DC_3C-8A_n257D DC_3C-8A_n257E DC_3C-8A_n257F DC_3C-8A_n257G DC_3C-8A_n257H DC_3C-8A_n257I DC_3C-8A_n257J DC_3C-8A_n257K DC_3C-8A_n257L DC_3C-8A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |  |
| DC_3A-3A-8A_n257A DC_3A-3A-8A_n257D DC_3A-3A-8A_n257E DC_3A-3A-8A_n257F DC_3A-3A-8A_n257G DC_3A-3A-8A_n257H DC_3A-3A-8A_n257I DC_3A-3A-8A_n257J DC_3A-3A-8A_n257K DC_3A-3A-8A_n257L DC_3A-3A-8A_n257M | DC_3A_n257A DC_8A_n257A |  |
| DC_3A-8A_n258A DC_3A-8A_n258D DC_3A-8A_n258E DC_3A-8A_n258F DC_3A-8A_n258G DC_3A-8A_n258H DC_3A-8A_n258I DC_3A-8A_n258J DC_3A-8A_n258K DC_3A-8A_n258L DC_3A-8A_n258M | DC_3A_n258A DC_8A_n258A |  |
| DC_3A-11A_n257A DC_3A-11A_n257G DC_3A-11A_n257H DC_3A-11A_n257I | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I |  |
| DC_3A-18A_n257A DC_3A-18A_n257D DC_3A-18A_n257E DC_3A-18A_n257F DC_3A-18A_n257G DC_3A-18A_n257H DC_3A-18A_n257I DC_3A-18A_n257J DC_3A-18A_n257K DC_3A-18A_n257L DC_3A-18A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |  |
| DC_3A-19A_n257A2 DC_3A-19A_n257D2 DC_3A-19A_n257E2 DC_3A-19A_n257F2 DC_3A-19A_n257G DC_3A-19A_n257H DC_3A-19A_n257I DC_3A-19A_n257J DC_3A-19A_n257K DC_3A-19A_n257L DC_3A-19A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I |  |
| DC_3A-21A_n257A2 DC_3A-21A_n257D2 DC_3A-21A_n257E2 DC_3A-21A_n257F2 DC_3A-21A_n257G DC_3A-21A_n257H DC_3A-21A_n257I DC_3A-21A_n257J DC_3A-21A_n257K DC_3A-21A_n257L DC_3A-21A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I |  |
| DC_3A-26A_n258A DC_3A-26A_n258B DC_3A-26A_n258C DC_3A-26A_n258D DC_3A-26A_n258E DC_3A-26A_n258F DC_3A-26A_n258G DC_3A-26A_n258H DC_3A-26A_n258I DC_3A-26A_n258J DC_3A-26A_n258K DC_3A-26A_n258L DC_3A-26A_n258M | DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_26A_n258A DC_26A_n258G DC_26A_n258H DC_26A_n258I |  |
| DC_3A-28A_n257A2 DC_3A-28A_n257D2 DC_3A-28A_n257E2 DC_3A-28A_n257F2 DC_3A-28A_n257G DC_3A-28A_n257H DC_3A-28A_n257I DC_3A-28A_n257J DC_3A-28A_n257K DC_3A-28A_n257L DC_3A-28A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |  |
| DC_3A-28A_n258A DC_3A-28A_n258B DC_3A-28A_n258C DC_3A-28A_n258D DC_3A-28A_n258E DC_3A-28A_n258F DC_3A-28A_n258G DC_3A-28A_n258H DC_3A-28A_n258I DC_3A-28A_n258J DC_3A-28A_n258K DC_3A-28A_n258L DC_3A-28A_n258M | DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K DC_3A_n258L DC_3A_n258M DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I DC_28A_n258J DC_28A_n258K DC_28A_n258L DC_28A_n258M |  |
| DC_3C-28A_n258A DC_3C-28A_n258B DC_3C-28A_n258C DC_3C-28A_n258D DC_3C-28A_n258E DC_3C-28A_n258F DC_3C-28A_n258G DC_3C-28A_n258H DC_3C-28A_n258I DC_3C-28A_n258J DC_3C-28A_n258K DC_3C-28A_n258L DC_3C-28A_n258M | DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K DC_3A_n258L DC_3A_n258M DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I DC_28A_n258J DC_28A_n258K DC_28A_n258L DC_28A_n258M |  |
| DC_3A-41A_n257A DC_3A-41A_n257D DC_3A-41A_n257E DC_3A-41A_n257F DC_3A-41A_n257G DC_3A-41A_n257H DC_3A-41A_n257I DC_3A-41A_n257J DC_3A-41A_n257K DC_3A-41A_n257L DC_3A-41A_n257M DC_3A-41C_n257A DC_3A-41C_n257D DC_3A-41C_n257E DC_3A-41C_n257F DC_3A-41C_n257G DC_3A-41C_n257H DC_3A-41C_n257I DC_3A-41C_n257J DC_3A-41C_n257K DC_3A-41C_n257L DC_3A-41C_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |  |
| DC_3A-42A_n257A2 DC_3A-42A_n257D2 DC_3A-42A_n257E2 DC_3A-42A_n257F2 DC_3A-42A_n257G DC_3A-42A_n257H DC_3A-42A_n257I DC_3A-42A_n257J DC_3A-42A_n257K DC_3A-42A_n257L DC_3A-42A_n257M DC_3A-42C_n257A2 DC_3A-42C_n257D2 DC_3A-42C_n257E2 DC_3A-42C_n257F2 DC_3A-42C_n257G DC_3A-42C_n257H DC_3A-42C_n257I DC_3A-42C_n257J DC_3A-42C_n257K DC_3A-42C_n257L DC_3A-42C_n257M DC_3A-42D_n257A2 DC_3A-42D_n257D DC_3A-42D_n257E DC_3A-42D_n257F DC_3A-42D_n257G DC_3A-42D_n257H DC_3A-42D_n257I DC_3A-42D_n257J DC_3A-42D_n257K DC_3A-42D_n257L DC_3A-42D_n257M DC_3A-42E_n257A2 DC_3A-42E_n257D DC_3A-42E_n257E DC_3A-42E_n257F DC_3A-42E_n257G DC_3A-42E_n257H DC_3A-42E_n257I DC_3A-42E_n257J DC_3A-42E_n257K DC_3A-42E_n257L DC_3A-42E_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |  |
| DC_5A-7A_n257A2 DC_5A-7A_n257D DC_5A-7A_n257E DC_5A-7A_n257F DC_5A-7A_n257G DC_5A-7A_n257H DC_5A-7A_n257I DC_5A-7A_n257J DC_5A-7A_n257K DC_5A-7A_n257L DC_5A-7A_n257M | DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |  |
| DC_5A-7A-7A_n257A DC_5A-7A-7A_n257D DC_5A-7A-7A_n257E DC_5A-7A-7A_n257F DC_5A-7A-7A_n257G DC_5A-7A-7A_n257H DC_5A-7A-7A_n257I DC_5A-7A-7A_n257J DC_5A-7A-7A_n257K DC_5A-7A-7A_n257L DC_5A-7A-7A_n257M | DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |  |
| DC_5A-7A_n258A DC_5A-7A_n258G DC_5A-7A_n258H DC_5A-7A_n258I DC_5A-7A_n258J DC_5A-7A_n258K DC_5A-7A_n258L DC_5A-7A_n258M | DC_5A_n258A DC_5A_n258G DC_7A_n258A DC_7A_n258G |
| DC_5A-7A_n260A DC_5A-7A_n260G DC_5A-7A_n260H DC_5A-7A_n260I DC_5A-7A_n260J DC_5A-7A_n260K DC_5A-7A_n260L DC_5A-7A_n260M DC_5A-7A_n260O DC_5A-7A_n260P DC_5A-7A_n260Q | DC_5A_n260A DC_5A_n260G DC_5A_n260O DC_7A_n260A DC_7A_n260G DC_7A_n260O |
| DC_5A-7A_n261A DC_5A-7A_n261G DC_5A-7A_n261H DC_5A-7A_n261I DC_5A-7A_n261J DC_5A-7A_n261K DC_5A-7A_n261L DC_5A-7A_n261M DC_5A-7A_n261O DC_5A-7A_n261P DC_5A-7A_n261Q | DC_5A_n261A DC_5A_n261G DC_5A_n261O DC_7A_n261A DC_7A_n261G DC_7A_n261O |
| DC_5A-30A_n260A DC_5A-30A_n260G DC_5A-30A_n260H DC_5A-30A_n260I DC_5A-30A_n260J DC_5A-30A_n260K DC_5A-30A_n260L DC_5A-30A_n260M | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |  |
| DC_5A-66A_n257A DC_5A-66A_n257G DC_5A-66A_n257H DC_5A-66A_n257I DC_5A-66A_n257J DC_5A-66A_n257K DC_5A-66A_n257L DC_5A-66A_n257M | DC_5A_n257A DC_5A_n257G DC_66A_n257A DC_66A_n257G |  |
| DC_5A-66A_n258A DC_5A-66A_n258G DC_5A-66A_n258H DC_5A-66A_n258I DC_5A-66A_n258J DC_5A-66A_n258K DC_5A-66A_n258L DC_5A-66A_n258M | DC_5A_n258A DC_5A_n258G DC_66A_n258A DC_66A_n258G |  |
| DC_5A-66A_n260A DC_5A-66A_n260G DC_5A-66A_n260H DC_5A-66A_n260I DC_5A-66A_n260J DC_5A-66A_n260K DC_5A-66A_n260L DC_5A-66A_n260M DC_5A-66A_n260O DC_5A-66A_n260P DC_5A-66A_n260Q | DC_5A_n260A DC_66A_n260A DC_5A_n260G DC_66A_n260G DC_5A_n260H DC_66A_n260H DC_5A_n260I DC_66A_n260I DC_5A_n260J DC_66A_n260J DC_5A_n260K DC_66A_n260K DC_5A_n260L DC_66A_n260L DC_5A_n260M DC_66A_n260M DC_5A_n260O DC_66A_n260O |  |
| DC_5A-66A-66A_n260A DC_5A-66A-66A_n260G DC_5A-66A-66A_n260H DC_5A-66A-66A_n260I DC_5A-66A-66A_n260J DC_5A-66A-66A_n260K DC_5A-66A-66A_n260L DC_5A-66A-66A_n260M | DC_5A_n260A DC_66A_n260A DC_5A_n260G DC_66A_n260G DC_5A_n260H DC_66A_n260H DC_5A_n260I DC_66A_n260I DC_5A_n260J DC_66A_n260J DC_5A_n260K DC_66A_n260K DC_5A_n260L DC_66A_n260L DC_5A_n260M DC_66A_n260M |  |
| DC_5A-66A_n261A DC_5A-66A_n261G DC_5A-66A_n261H DC_5A-66A_n261I DC_5A-66A_n261J DC_5A-66A_n261K DC_5A-66A_n261L DC_5A-66A_n261M DC_5A-66A_n261O DC_5A-66A_n261P DC_5A-66A_n261Q | DC_5A_n261A DC_66A_n261A DC_5A_n261G DC_66A_n261G DC_5A_n261H DC_66A_n261H DC_5A_n261I DC_66A_n261I DC_5A_n261J DC_66A_n26J DC_5A_n261K DC_66A_n261K DC_5A_n261L DC_66A_n261L DC_5A_n261M DC_66A_n261M DC_5A_n261O DC_66A_n261O |  |
| DC_5A-66A-66A_n261A DC_5A-66A-66A_n261G DC_5A-66A-66A_n261H DC_5A-66A-66A_n261I DC_5A-66A-66A_n261J DC_5A-66A-66A_n261K DC_5A-66A-66A_n261L DC_5A-66A-66A_n261M | DC_5A_n261A DC_66A_n261A DC_5A_n261G DC_66A_n261G DC_5A_n261H DC_66A_n261H DC_5A_n261I DC_66A_n261I DC_5A_n261J DC_66A_n26J DC_5A_n261K DC_66A_n261K DC_5A_n261L DC_66A_n261L DC_5A_n261M DC_66A_n261M |  |
| DC_5A-66A_n261(2A) DC_5A-66A_n261(3A) DC_5A-66A_n261(4A) DC_5A-66A_n261(2G) DC_5A-66A_n261(2H) DC_5A-66A_n261(A-G) DC_5A-66A_n261(A-H) DC_5A-66A_n261(A-I) DC_5A-66A_n261(A-J) DC_5A-66A_n261(A-K) DC_5A-66A_n261(A-L) DC_5A-66A_n261(A-2G) DC_5A-66A_n261(2A-G) DC_5A-66A_n261(2A-H) DC_5A-66A_n261(2A-I) DC_5A-66A_n261(A-G-H) DC_5A-66A_n261(A-G-I) DC_5A-66A_n261(3A-G) DC_5A-66A_n261(G-H) DC_5A-66A_n261(G-I) DC_5A-66A_n261(G-J) DC_5A-66A_n261(H-I) | DC_5A_n261A DC_66A_n261A DC_5A_n261G DC_66A_n261G DC_5A_n261H DC_66A_n261H DC_5A_n261I DC_66A_n261I |  |
| DC_5A-66A-66A_n261(2A) DC_5A-66A-66A_n261(3A) DC_5A-66A-66A_n261(4A) DC_5A-66A-66A_n261(A-G) DC_5A-66A-66A_n261(A-H) DC_5A-66A-66A_n261(A-I) DC_5A-66A-66A_n261(A-J) DC_5A-66A-66A_n261(A-K) DC_5A-66A-66A_n261(A-L) DC_5A-66A-66A_n261(A-2G) DC_5A-66A-66A_n261(2A-G) DC_5A-66A-66A_n261(2A-H) DC_5A-66A-66A_n261(A-G-H) DC_5A-66A-66A_n261(A-G-I) DC_5A-66A-66A_n261(2A-I) DC_5A-66A-66A_n261(3A-G) DC_5A-66A-66A_n261(2G) DC_5A-66A-66A_n261(G-H) DC_5A-66A-66A_n261(G-I) DC_5A-66A-66A_n261(G-J) DC_5A-66A-66A_n261(2H) DC_5A-66A-66A_n261(H-I) | DC_5A_n261A DC_66A_n261A DC_5A_n261G DC_66A_n261G DC_5A_n261H DC_66A_n261H DC_5A_n261I DC_66A_n261I |  |
| DC_7A-8A_n257A DC_7A-8A_n257D DC_7A-8A_n257E DC_7A-8A_n257F DC_7A-8A_n257G DC_7A-8A_n257H DC_7A-8A_n257I DC_7A-8A_n257J DC_7A-8A_n257K DC_7A-8A_n257L DC_7A-8A_n257M | DC_7A_n257A DC_8A_n257A |  |
| DC_7A-7A-8A_n257A DC_7A-7A-8A_n257D DC_7A-7A-8A_n257E DC_7A-7A-8A_n257F DC_7A-7A-8A_n257G DC_7A-7A-8A_n257H DC_7A-7A-8A_n257I DC_7A-7A-8A_n257J DC_7A-7A-8A_n257K DC_7A-7A-8A_n257L DC_7A-7A-8A_n257M | DC_7A_n257A DC_8A_n257A |  |
| DC_7A-8A_n258A DC_7A-8A_n258D DC_7A-8A_n258E DC_7A-8A_n258F DC_7A-8A_n258G DC_7A-8A_n258H DC_7A-8A_n258I DC_7A-8A_n258J DC_7A-8A_n258K DC_7A-8A_n258L DC_7A-8A_n258M | DC_7A_n258A DC_8A_n258A |  |
| DC_7A-12A_n257A DC_7A-12A_n257G DC_7A-12A_n257H DC_7A-12A_n257I DC_7A-12A_n257J DC_7A-12A_n257K DC_7A-12A_n257L DC_7A-12A_n257M | DC_7A_n257A DC_7A_n257G DC_12A_n257A DC_12A_n257G |  |
| DC_7A-12A_n258A DC_7A-12A_n258G DC_7A-12A_n258H DC_7A-12A_n258I DC_7A-12A_n258J DC_7A-12A_n258K DC_7A-12A_n258L DC_7A-12A_n258M | DC_7A_n258A DC_7A_n258G DC_12A_n258A DC_12A_n258G |  |
| DC_7A-12A_n260A DC_7A-12A_n260G DC_7A-12A_n260H DC_7A-12A_n260I DC_7A-12A_n260J DC_7A-12A_n260K DC_7A-12A_n260L DC_7A-12A_n260M DC_7A-12A_n260O DC_7A-12A_n260P DC_7A-12A_n260Q | DC_7A_n260A DC_7A_n260G DC_7A_n260O DC_12A_n260A DC_12A_n260G DC_12A_n260O |  |
| DC_7A-12A_n261A DC_7A-12A_n261G DC_7A-12A_n261H DC_7A-12A_n261I DC_7A-12A_n261J DC_7A-12A_n261K DC_7A-12A_n261L DC_7A-12A_n261M DC_7A-12A_n261O DC_7A-12A_n261P DC_7A-12A_n261Q | DC_7A_n261A DC_7A_n261G DC_7A_n261O DC_12A_n261A DC_12A_n261G DC_12A_n261O |  |
| DC_7A-26A_n258A DC_7A-26A_n258B DC_7A-26A_n258C DC_7A-26A_n258D DC_7A-26A_n258E DC_7A-26A_n258F DC_7A-26A_n258G DC_7A-26A_n258H DC_7A-26A_n258I DC_7A-26A_n258J DC_7A-26A_n258K DC_7A-26A_n258L DC_7A-26A_n258M | DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_26A_n258A DC_26A_n258G DC_26A_n258H DC_26A_n258I |  |
| DC_7A-28A_n257A DC_7A-28A_n257G DC_7A-28A_n257H DC_7A-28A_n257I | DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |  |
| DC_7A-28A_n258A DC_7A-28A_n258B DC_7A-28A_n258C DC_7A-28A_n258D DC_7A-28A_n258E DC_7A-28A_n258F DC_7A-28A_n258G DC_7A-28A_n258H DC_7A-28A_n258I DC_7A-28A_n258J DC_7A-28A_n258K DC_7A-28A_n258L DC_7A-28A_n258M DC_7C-28A_n258A DC_7C-28A_n258B DC_7C-28A_n258C DC_7C-28A_n258D DC_7C-28A_n258E DC_7C-28A_n258F DC_7C-28A_n258G DC_7C-28A_n258H DC_7C-28A_n258I DC_7C-28A_n258J DC_7C-28A_n258K DC_7C-28A_n258L DC_7C-28A_n258M | DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n258A DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |  |
| DC_7A-66A_n257A DC_7A-66A_n257G DC_7A-66A_n257H DC_7A-66A_n257I DC_7A-66A_n257J DC_7A-66A_n257K DC_7A-66A_n257L DC_7A-66A_n257M | DC_7A_n257A DC_7A_n257G DC_66A_n257A DC_66A_n257G |  |
| DC_7A-66A_n258A DC_7A-66A_n258G DC_7A-66A_n258H DC_7A-66A_n258I DC_7A-66A_n258J DC_7A-66A_n258K DC_7A-66A_n258L DC_7A-66A_n258M | DC_7A_n258A DC_7A_n258G DC_66A_n258A DC_66A_n258G |  |
| DC_7A-66A_n260A DC_7A-66A_n260G DC_7A-66A_n260H DC_7A-66A_n260I DC_7A-66A_n260J DC_7A-66A_n260K DC_7A-66A_n260L DC_7A-66A_n260M DC_7A-66A_n260O DC_7A-66A_n260P DC_7A-66A_n260Q | DC_7A_n260A DC_7A_n260G DC_7A_n260O DC_66A_n260A DC_66A_n260G DC_66A_n260O |  |
| DC_7A-66A_n261A DC_7A-66A_n261G DC_7A-66A_n261H DC_7A-66A_n261I DC_7A-66A_n261J DC_7A-66A_n261K DC_7A-66A_n261L DC_7A-66A_n261M DC_7A-66A_n261O DC_7A-66A_n261P DC_7A-66A_n261Q | DC_7A_n261A DC_7A_n261G DC_7A_n261O DC_66A_n261A DC_66A_n261G DC_66A_n261O |  |
| DC_7A-71A_n257A DC_7A-71A_n257G DC_7A-71A_n257H DC_7A-71A_n257I DC_7A-71A_n257J DC_7A-71A_n257K DC_7A-71A_n257L DC_7A-71A_n257M | DC_7A_n257A DC_7A_n257G DC_71A_n257A DC_71A_n257G |  |
| DC_7A-71A_n258A DC_7A-71A_n258G DC_7A-71A_n258H DC_7A-71A_n258I DC_7A-71A_n258J DC_7A-71A_n258K DC_7A-71A_n258L DC_7A-71A_n258M | DC_7A_n258A DC_7A_n258G DC_71A_n258A DC_71A_n258G |  |
| DC_7A-71A_n260A DC_7A-71A_n260G DC_7A-71A_n260H DC_7A-71A_n260I DC_7A-71A_n260J DC_7A-71A_n260K DC_7A-71A_n260L DC_7A-71A_n260M DC_7A-71A_n260O DC_7A-71A_n260P DC_7A-71A_n260Q | DC_7A_n260A DC_7A_n260G DC_7A_n260O DC_71A_n260A DC_71A_n260G DC_71A_n260O |  |
| DC_7A-71A_n261A DC_7A-71A_n261G DC_7A-71A_n261H DC_7A-71A_n261I DC_7A-71A_n261J DC_7A-71A_n261K DC_7A-71A_n261L DC_7A-71A_n261M DC_7A-71A_n261O DC_7A-71A_n261P DC_7A-71A_n261Q | DC_7A_n261A DC_7A_n261G DC_7A_n261O DC_71A_n261A DC_71A_n261G DC_71A_n261O |  |
| DC_8A-11A_n257A DC_8A-11A_n257D DC_8A-11A_n257G DC_8A-11A_n257H DC_8A-11A_n257I | DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |  |
| DC_11A-18A_n257A DC_11A-18A_n257G DC_11A-18A_n257H DC_11A-18A_n257I | DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |  |
| DC_12A-30A_n260A DC_12A-30A_n260G DC_12A-30A_n260H DC_12A-30A_n260I DC_12A-30A_n260J DC_12A-30A_n260K DC_12A-30A_n260L DC_12A-30A_n260M | DC_12A_n260A DC_30A_n260A DC_12A_n260G DC_30A_n260G DC_12A_n260H DC_30A_n260H DC_12A_n260I DC_30A_n260I DC_12A_n260J DC_30A_n260J DC_12A_n260K DC_30A_n260K DC_12A_n260L DC_30A_n260L DC_12A_n260M DC_30A_n260M |  |
| DC_12A-66A_n257A DC_12A-66A_n257G DC_12A-66A_n257H DC_12A-66A_n257I DC_12A-66A_n257J DC_12A-66A_n257K DC_12A-66A_n257L DC_12A-66A_n257M | DC_12A_n257A DC_12A_n257G DC_66A_n257A DC_66A_n257G |  |
| DC_12A-66A_n258A DC_12A-66A_n258G DC_12A-66A_n258H DC_12A-66A_n258I DC_12A-66A_n258J DC_12A-66A_n258K DC_12A-66A_n258L DC_12A-66A_n258M | DC_12A_n258A DC_12A_n258G DC_66A_n258A DC_66A_n258G |  |
| DC_12A-66A_n260A DC_12A-66A_n260G DC_12A-66A_n260H DC_12A-66A_n260I DC_12A-66A_n260J DC_12A-66A_n260K DC_12A-66A_n260L DC_12A-66A_n260M DC_12A-66A_n260O DC_12A-66A_n260P DC_12A-66A_n260Q | DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_12A_n260O DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O |  |
| DC_12A-66A_n261A DC_12A-66A_n261G DC_12A-66A_n261H DC_12A-66A_n261I DC_12A-66A_n261J DC_12A-66A_n261K DC_12A-66A_n261L DC_12A-66A_n261M DC_12A-66A_n261O DC_12A-66A_n261P DC_12A-66A_n261Q | DC_12A_n261A DC_12A_n261G DC_12A_n261O DC_66A_n261A DC_66A_n261G DC_66A_n261O |  |
| DC_12A-66A-66A_n260A DC_12A-66A-66A_n260G DC_12A-66A-66A_n260H DC_12A-66A-66A_n260I DC_12A-66A-66A_n260J DC_12A-66A-66A_n260K DC_12A-66A-66A_n260L DC_12A-66A-66A_n260M | DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |  |
| DC_13A-66A_n257A2 | DC_13A_n257A DC_66A_n257A |  |
| DC_13A-66A_n260A DC_13A-66A_n260G DC_13A-66A_n260H DC_13A-66A_n260I DC_13A-66A_n260J DC_13A-66A_n260K DC_13A-66A_n260L DC_13A-66A_n260M | DC_13A_n260A DC_66A_n260A DC_13A_n260G DC_66A_n260G DC_13A_n260H DC_66A_n260H DC_13A_n260I DC_66A_n260I DC_13A_n260J DC_66A_n260J DC_13A_n260K DC_66A_n260K DC_13A_n260L DC_66A_n260L DC_13A_n260M DC_66A_n260M |  |
| DC_13A-66A-66A_n260A DC_13A-66A-66A_n260G DC_13A-66A-66A_n260H DC_13A-66A-66A_n260I DC_13A-66A-66A_n260J DC_13A-66A-66A_n260K DC_13A-66A-66A_n260L DC_13A-66A-66A_n260M | DC_13A_n260A DC_66A_n260A DC_13A_n260G DC_66A_n260G DC_13A_n260H DC_66A_n260H DC_13A_n260I DC_66A_n260I DC_13A_n260J DC_66A_n260J DC_13A_n260K DC_66A_n260K DC_13A_n260L DC_66A_n260L DC_13A_n260M DC_66A_n260M |  |
| DC_13A-66A_n260(2A) DC_13A-66A_n260(3A) DC_13A-66A_n260(4A) DC_13A-66A_n260(5A) DC_13A-66A_n260(6A) DC_13A-66A_n260(2G) DC_13A-66A_n260(2H) DC_13A-66A_n260(A-G) DC_13A-66A_n260(A-H) DC_13A-66A_n260(A-2G) DC_13A-66A_n260(2A-G) DC_13A-66A_n260(2A-2G) DC_13A-66A_n260(3A-G) DC_13A-66A_n260(G-H) | DC_13A_n260A DC_66A_n260A DC_13A_n260G DC_66A_n260G DC_13A_n260H DC_66A_n260H |  |
| DC_13A-66A-66A_n260(2A) DC_13A-66A-66A_n260(3A) DC_13A-66A-66A_n260(4A) DC_13A-66A-66A_n260(5A) DC_13A-66A-66A_n260(6A) DC_13A-66A-66A_n260(2G) DC_13A-66A-66A_n260(2H) DC_13A-66A-66A_n260(A-G) DC_13A-66A-66A_n260(A-H) DC_13A-66A-66A_n260(A-2G) DC_13A-66A-66A_n260(2A-G) DC_13A-66A-66A_n260(2A-2G) DC_13A-66A-66A_n260(3A-G) DC_13A-66A-66A_n260(G-H) | DC_13A_n260A DC_66A_n260A DC_13A_n260G DC_66A_n260G DC_13A_n260H DC_66A_n260H |  |
| DC_13A-66A_n261A DC_13A-66A_n261G DC_13A-66A_n261H DC_13A-66A_n261I DC_13A-66A_n261J DC_13A-66A_n261K DC_13A-66A_n261L DC_13A-66A_n261M | DC_13A_n261A DC_66A_n261A DC_13A_n261G DC_66A_n261G DC_13A_n261H DC_66A_n261H DC_13A_n261I DC_66A_n261I DC_13A_n261J DC_66A_n261J DC_13A_n261K DC_66A_n261K DC_13A_n261L DC_66A_n261L DC_13A_n261M DC_66A_n261M |  |
| DC_13A-66A-66A_n261A DC_13A-66A-66A_n261G DC_13A-66A-66A_n261H DC_13A-66A-66A_n261I DC_13A-66A-66A_n261J DC_13A-66A-66A_n261K DC_13A-66A-66A_n261L DC_13A-66A-66A_n261M | DC_13A_n261A DC_66A_n261A DC_13A_n261G DC_66A_n261G DC_13A_n261H DC_66A_n261H DC_13A_n261I DC_66A_n261I DC_13A_n261J DC_66A_n261J DC_13A_n261K DC_66A_n261K DC_13A_n261L DC_66A_n261L DC_13A_n261M DC_66A_n261M |  |
| DC_13A-66A_n261(2A) DC_13A-66A_n261(3A) DC_13A-66A_n261(4A) DC_13A-66A_n261(2G) DC_13A-66A_n261(2H) DC_13A-66A_n261(A-G) DC_13A-66A_n261(A-H) DC_13A-66A_n261(A-I) DC_13A-66A_n261(A-J) DC_13A-66A_n261(A-K) DC_13A-66A_n261(A-L) DC_13A-66A_n261(A-2G) DC_13A-66A_n261(A-G-H) DC_13A-66A_n261(A-G-I) DC_13A-66A_n261(2A-G) DC_13A-66A_n261(2A-H) DC_13A-66A_n261(2A-I) DC_13A-66A_n261(3A-G) DC_13A-66A_n261(G-H) DC_13A-66A_n261(G-I) DC_13A-66A_n261(G-J) DC_13A-66A_n261(H-I) | DC_13A_n261A DC_66A_n261A DC_13A_n261G DC_66A_n261G DC_13A_n261H DC_66A_n261H DC_13A_n261I DC_66A_n261I |  |
| DC_13A-66A-66A_n261(2A) DC_13A-66A-66A_n261(3A) DC_13A-66A-66A_n261(4A) DC_13A-66A-66A_n261(2G) DC_13A-66A-66A_n261(2H) DC_13A-66A-66A_n261(A-G) DC_13A-66A-66A_n261(A-H) DC_13A-66A-66A_n261(A-I) DC_13A-66A-66A_n261(A-J) DC_13A-66A-66A_n261(A-K) DC_13A-66A-66A_n261(A-L) DC_13A-66A-66A_n261(A-2G) DC_13A-66A-66A_n261(A-G-H) DC_13A-66A-66A_n261(A-G-I) DC_13A-66A-66A_n261(2A-G) DC_13A-66A-66A_n261(2A-H) DC_13A-66A-66A_n261(2A-I) DC_13A-66A-66A_n261(3A-G) DC_13A-66A-66A_n261(G-H) DC_13A-66A-66A_n261(G-I) DC_13A-66A-66A_n261(G-J) DC_13A-66A-66A_n261(H-I) | DC_13A_n261A DC_66A_n261A DC_13A_n261G DC_66A_n261G DC_13A_n261H DC_66A_n261H DC_13A_n261I DC_66A_n261I |  |
| DC_13A-46A_n261A DC_13A-46A_n261G DC_13A-46A_n261H DC_13A-46A_n261I DC_13A-46A_n261J DC_13A-46A_n261K DC_13A-46A_n261L DC_13A-46A_n261M | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_13A_n261L DC_13A_n261M |  |
| DC_13A-46A-46A_n261A DC_13A-46A-46A_n261G DC_13A-46A-46A_n261H DC_13A-46A-46A_n261I DC_13A-46A-46A_n261J DC_13A-46A-46A_n261K DC_13A-46A-46A_n261L DC_13A-46A-46A_n261M | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_13A_n261L DC_13A_n261M |  |
| DC_13A-46A_n261(A-G) DC_13A-46A_n261(A-H) DC_13A-46A_n261(G-H) DC_13A-46A_n261(2A) DC_13A-46A_n261(2G) DC_13A-46A_n261(2H) | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |  |
| DC_13A-46A-46A_n261(A-G) DC_13A-46A-46A_n261(A-H) DC_13A-46A-46A_n261(G-H) DC_13A-46A-46A_n261(2A) DC_13A-46A-46A_n261(2G) DC_13A-46A-46A_n261(2H) | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |  |
| DC_14A-30A_n260A DC_14A-30A_n260G DC_14A-30A_n260H DC_14A-30A_n260I DC_14A-30A_n260J DC_14A-30A_n260K DC_14A-30A_n260L DC_14A-30A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |  |
| DC_14A-66A_n260A DC_14A-66A_n260G DC_14A-66A_n260H DC_14A-66A_n260I DC_14A-66A_n260J DC_14A-66A_n260K DC_14A-66A_n260L DC_14A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |  |
| DC_14A-66A-66A_n260A DC_14A-66A-66A_n260G DC_14A-66A-66A_n260H DC_14A-66A-66A_n260I DC_14A-66A-66A_n260J DC_14A-66A-66A_n260K DC_14A-66A-66A_n260L DC_14A-66A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |  |
| DC_18A-28A_n257A2 | DC_18A_n257A DC_28A_n257A |  |
| DC_18A-42A_n257A DC_18A-42A_n257D DC_18A-42A_n257E DC_18A-42A_n257F DC_18A-42A_n257G DC_18A-42A_n257H DC_18A-42A_n257I DC_18A-42A_n257J DC_18A-42A_n257K DC_18A-42A_n257L DC_18A-42A_n257M DC_18A-42C_n257A DC_18A-42C_n257D DC_18A-42C_n257E DC_18A-42C_n257F DC_18A-42C_n257G DC_18A-42C_n257H DC_18A-42C_n257I DC_18A-42C_n257J DC_18A-42C_n257K DC_18A-42C_n257L DC_18A-42C_n257M | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |  |
| DC_18A-41A_n257A DC_18A-41A_n257G DC_18A-41A_n257H DC_18A-41A_n257I DC_18A-41C_n257A DC_18A-41C_n257G DC_18A-41C_n257H DC_18A-41C_n257I | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |  |
| DC_19A-21A_n257A2 DC_19A-21A_n257D2 DC_19A-21A_n257E2 DC_19A-21A_n257F2 DC_19A-21A_n257G DC_19A-21A_n257H DC_19A-21A_n257I DC_19A-21A_n257J DC_19A-21A_n257K DC_19A-21A_n257L DC_19A-21A_n257M | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |  |
| DC_19A-28A_n257A | DC_19A_n257A DC_28A_n257A |  |
| DC_19A-42A_n257A2 DC_19A-42A_n257D2 DC_19A-42A_n257E2 DC_19A-42A_n257F2 DC_19A-42A_n257G2 DC_19A-42A_n257H2 DC_19A-42A_n257I2 DC_19A-42A_n257J DC_19A-42A_n257K DC_19A-42A_n257L DC_19A-42A_n257M DC_19A-42C_n257A2 DC_19A-42C_n257D DC_19A-42C_n257E DC_19A-42C_n257F DC_19A-42C_n257G2 DC_19A-42C_n257H2 DC_19A-42C_n257I2 DC_19A-42C_n257J DC_19A-42C_n257K DC_19A-42C_n257L DC_19A-42C_n257M DC_19A-42D_n257A DC_19A-42D_n257D2 DC_19A-42D_n257E2 DC_19A-42D_n257F2 | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |  |
| DC_21A-28A_n257A2 DC_21A-28A_n257D2 DC_21A-28A_n257E2 DC_21A-28A_n257F2 | DC_21A_n257A DC_21A_n257D DC_28A_n257A DC_28A_n257D |  |
| DC_21A-42A_n257A2 DC_21A-42A_n257D2 DC_21A-42A_n257E2 DC_21A-42A_n257F2 DC_21A-42A_n257G DC_21A-42A_n257H DC_21A-42A_n257I DC_21A-42A_n257J DC_21A-42A_n257K DC_21A-42A_n257L DC_21A-42A_n257M DC_21A-42C_n257A2 DC_21A-42C_n257D DC_21A-42C_n257E DC_21A-42C_n257F DC_21A-42C_n257G DC_21A-42C_n257H DC_21A-42C_n257I DC_21A-42C_n257J DC_21A-42C_n257K DC_21A-42C_n257L DC_21A-42C_n257M DC_21A-42D_n257A DC_21A-42D_n257D DC_21A-42D_n257E DC_21A-42D_n257F DC_21A-42D_n257G DC_21A-42D_n257H DC_21A-42D_n257I DC_21A-42D_n257J DC_21A-42D_n257K DC_21A-42D_n257L DC_21A-42D_n257M DC_21A-42E_n257A DC_21A-42E_n257D DC_21A-42E_n257E DC_21A-42E_n257F DC_21A-42E_n257G DC_21A-42E_n257H DC_21A-42E_n257I DC_21A-42E_n257J DC_21A-42E_n257K DC_21A-42E_n257L DC_21A-42E_n257M | DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |  |
| DC_28A-41A_n257A DC_28A-41A_n257G DC_28A-41A_n257H DC_28A-41A_n257I DC_28A-41C_n257A DC_28A-41C_n257G DC_28A-41C_n257H DC_28A-41C_n257I | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |  |
| DC_28A-42A_n257A2 DC_28A-42A_n257D2 DC_28A-42A_n257G2 DC_28A-42A_n257H2 DC_28A-42A_n257I2 DC_28A-42A_n257J DC_28A-42A_n257K DC_28A-42A_n257L DC_28A-42A_n257M DC_28A-42C_n257A2 DC_28A-42C_n257D2 DC_28A-42C_n257G2 DC_28A-42C_n257H2 DC_28A-42C_n257I2 DC_28A-42C_n257J DC_28A-42C_n257K DC_28A-42C_n257L DC_28A-42C_n257M | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |  |
| DC_29A-30A_n260A DC_29A-30A_n260G DC_29A-30A_n260H DC_29A-30A_n260I DC_29A-30A_n260J DC_29A-30A_n260K DC_29A-30A_n260L DC_29A-30A_n260M | DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |  |
| DC_29A-66A_n260A DC_29A-66A_n260G DC_29A-66A_n260H DC_29A-66A_n260I DC_29A-66A_n260J DC_29A-66A_n260K DC_29A-66A_n260L DC_29A-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |  |
| DC_30A-66A_n260A DC_30A-66A_n260G DC_30A-66A_n260H DC_30A-66A_n260I DC_30A-66A_n260J DC_30A-66A_n260K DC_30A-66A_n260L DC_30A-66A_n260M | DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |  |
| DC_30A-66A-66A_n260A DC_30A-66A-66A_n260G DC_30A-66A-66A_n260H DC_30A-66A-66A_n260I DC_30A-66A-66A_n260J DC_30A-66A-66A_n260K DC_30A-66A-66A_n260L DC_30A-66A-66A_n260M | DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |  |
| DC_40A-42A_n257A DC_40A-42A_n257D DC_40A-42A_n257E DC_40A-42A_n257F DC_40A-42A_n257G DC_40A-42A_n257H DC_40A-42A_n257I DC_40A-42A_n257J DC_40A-42A_n257K DC_40A-42A_n257L DC_40A-42A_n257M DC_40C-42A_n257A DC_40C-42A_n257D DC_40C-42A_n257E DC_40C-42A_n257F DC_40C-42A_n257G DC_40C-42A_n257H DC_40C-42A_n257I DC_40C-42A_n257J DC_40C-42A_n257K DC_40C-42A_n257L DC_40C-42A_n257M DC_40A-42C_n257A DC_40A-42C_n257D DC_40A-42C_n257E DC_40A-42C_n257F DC_40A-42C_n257G DC_40A-42C_n257H DC_40A-42C_n257I DC_40A-42C_n257J DC_40A-42C_n257K DC_40A-42C_n257L DC_40A-42C_n257M DC_40A-40A-42A_n257A DC_40A-40A-42A_n257D DC_40A-40A-42A_n257E DC_40A-40A-42A_n257F DC_40A-40A-42A_n257G DC_40A-40A-42A_n257H DC_40A-40A-42A_n257I DC_40A-40A-42A_n257J DC_40A-40A-42A_n257K DC_40A-40A-42A_n257L DC_40A-40A-42A_n257M DC_40A-40A-42C_n257A DC_40A-40A-42C_n257D DC_40A-40A-42C_n257E DC_40A-40A-42C_n257F DC_40A-40A-42C_n257G DC_40A-40A-42C_n257H DC_40A-40A-42C_n257I DC_40A-40A-42C_n257J DC_40A-40A-42C_n257K DC_40A-40A-42C_n257L DC_40A-40A-42C_n257M DC_40A-42A-42A_n257A DC_40A-42A-42A_n257D DC_40A-42A-42A_n257E DC_40A-42A-42A_n257F DC_40A-42A-42A_n257G DC_40A-42A-42A_n257H DC_40A-42A-42A_n257I DC_40A-42A-42A_n257J DC_40A-42A-42A_n257K DC_40A-42A-42A_n257L DC_40A-42A-42A_n257M DC_40C-42A-42A_n257A DC_40C-42A-42A_n257D DC_40C-42A-42A_n257E DC_40C-42A-42A_n257F DC_40C-42A-42A_n257G DC_40C-42A-42A_n257H DC_40C-42A-42A_n257I DC_40C-42A-42A_n257J DC_40C-42A-42A_n257K DC_40C-42A-42A_n257L DC_40C-42A-42A_n257M DC_40A-40A-42A-42A_n257A DC_40A-40A-42A-42A_n257D DC_40A-40A-42A-42A_n257E DC_40A-40A-42A-42A_n257F DC_40A-40A-42A-42A_n257G DC_40A-40A-42A-42A_n257H DC_40A-40A-42A-42A_n257I DC_40A-40A-42A-42A_n257J DC_40A-40A-42A-42A_n257K DC_40A-40A-42A-42A_n257L DC_40A-40A-42A-42A_n257M | DC_40A_n257A DC_42A_n257A |  |
| DC_41A-42A_n257A DC_41A-42A_n257D DC_41A-42A_n257E DC_41A-42A_n257F DC_41A-42A_n257G DC_41A-42A_n257H DC_41A-42A_n257I DC_41A-42A_n257J DC_41A-42A_n257K DC_41A-42A_n257L DC_41A-42A_n257M DC_41A-42C_n257A DC_41A-42C_n257D DC_41A-42C_n257E DC_41A-42C_n257F DC_41A-42C_n257G DC_41A-42C_n257H DC_41A-42C_n257I DC_41A-42C_n257J DC_41A-42C_n257K DC_41A-42C_n257L DC_41A-42C_n257M DC_41C-42A_n257A DC_41C-42A_n257D DC_41C-42A_n257E DC_41C-42A_n257F DC_41C-42A_n257G DC_41C-42A_n257H DC_41C-42A_n257I DC_41C-42A_n257J DC_41C-42A_n257K DC_41C-42A_n257L DC_41C-42A_n257M DC_41C-42C_n257A DC_41C-42C_n257D DC_41C-42C_n257E DC_41C-42C_n257F DC_41C-42C_n257G DC_41C-42C_n257H DC_41C-42C_n257I DC_41C-42C_n257J DC_41C-42C_n257K DC_41C-42C_n257L DC_41C-42C_n257M | DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |  |
| DC_46A-48A_n260A DC_46C-48A_n260A DC_46D-48A_n260A DC_46A-48C_n260A DC_46A-48D_n260A DC_46C-48C_n260A DC_46C-48D_n260A DC_46D-48C_n260A DC_46D-48D_n260A | DC_48A_n260A DC_48C_n260A |  |
| DC_46A-48A_n260(2A) DC_46C-48A_n260(2A) DC_46D-48A_n260(2A) DC_46A-48C_n260(2A) DC_46A-48D_n260(2A) DC_46C-48C_n260(2A) DC_46C-48D_n260(2A) DC_46D-48C_n260(2A) DC_46D-48D_n260(2A) DC_46A-48A_n260(3A) DC_46C-48A_n260(3A) DC_46D-48A_n260(3A) DC_46A-48C_n260(3A) DC_46A-48D_n260(3A) DC_46C-48C_n260(3A) DC_46C-48D_n260(3A) DC_46D-48C_n260(3A) DC_46D-48D_n260(3A) DC_46A-48A_n260(4A) DC_46C-48A_n260(4A) DC_46D-48A_n260(4A) DC_46A-48C_n260(4A) DC_46A-48D_n260(4A) DC_46C-48C_n260(4A) DC_46C-48D_n260(4A) DC_46D-48C_n260(4A) DC_46D-48D_n260(4A) | DC_48A_n260A DC_48C_n260A |  |
| DC_46A-48A_n261A DC_46C-48A_n261A DC_46D-48A_n261A DC_46A-48C_n261A DC_46A-48D_n261A DC_46C-48C_n261A DC_46C-48D_n261A DC_46D-48C_n261A DC_46D-48D_n261A | DC_48A_n261A DC_48C_n261A |  |
| DC_46A-48A_n261(2A) DC_46C-48A_n261(2A) DC_46D-48A_n261(2A) DC_46A-48C_n261(2A) DC_46A-48D_n261(2A) DC_46C-48C_n261(2A) DC_46C-48D_n261(2A) DC_46D-48C_n261(2A) DC_46D-48D_n261(2A) | DC_48A_n261A DC_48C_n261A |  |
| DC_46A-66A_n258A DC_46C-66A_n258A DC_46D-66A_n258A | DC_66A_n258A |  |
| DC_46A-66A_n258(2A) DC_46A-66A_n258(3A) DC_46A-66A_n258(4A) DC_46A-66A_n258(5A) DC_46C-66A_n258(2A) DC_46C-66A_n258(3A) DC_46C-66A_n258(4A) DC_46C-66A_n258(5A) DC_46D-66A_n258(2A) DC_46D-66A_n258(3A) DC_46D-66A_n258(4A) DC_46D-66A_n258(5A) | DC_66A_n258A |  |
| DC_46A-66A_n260A DC_46C-66A_n260A DC_46D-66A_n260A DC_46E-66A_n260A DC_46A-66A_n260G DC_46C-66A_n260G DC_46D-66A_n260G DC_46E-66A_n260G DC_46A-66A_n260H DC_46C-66A_n260H DC_46D-66A_n260H DC_46E-66A_n260H DC_46A-66A_n260I DC_46C-66A_n260I DC_46D-66A_n260I DC_46E-66A_n260I DC_46A-66A_n260J DC_46C-66A_n260J DC_46D-66A_n260J DC_46E-66A_n260J DC_46A-66A_n260K DC_46C-66A_n260K DC_46D-66A_n260K DC_46E-66A_n260K DC_46A-66A_n260L DC_46C-66A_n260L DC_46D-66A_n260L DC_46E-66A_n260L DC_46A-66A_n260M DC_46C-66A_n260M DC_46D-66A_n260M DC_46E-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |  |
| DC_46A-66A-66A_n260A DC_46C-66A-66A_n260A DC_46D-66A-66A_n260A DC_46E-66A-66A_n260A DC_46A-66A-66A_n260G DC_46C-66A-66A_n260G DC_46D-66A-66A_n260G DC_46E-66A-66A_n260G DC_46A-66A-66A_n260H DC_46C-66A-66A_n260H DC_46D-66A-66A_n260H DC_46E-66A-66A_n260H DC_46A-66A-66A_n260I DC_46C-66A-66A_n260I DC_46D-66A-66A_n260I DC_46E-66A-66A_n260I DC_46A-66A-66A_n260J DC_46C-66A-66A_n260J DC_46D-66A-66A_n260J DC_46E-66A-66A_n260J DC_46A-66A-66A_n260K DC_46C-66A-66A_n260K DC_46D-66A-66A_n260K DC_46E-66A-66A_n260K DC_46A-66A-66A_n260L DC_46C-66A-66A_n260L DC_46D-66A-66A_n260L DC_46E-66A-66A_n260L DC_46A-66A-66A_n260M DC_46C-66A-66A_n260M DC_46D-66A-66A_n260M DC_46E-66A-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |  |
| DC_46A-66A_n260(2A) DC_46C-66A_n260(2A) DC_46D-66A_n260(2A) | DC_66A_n260A |  |
| DC_46A-66A_n261A DC_46A-66A_n261G DC_46A-66A_n261H DC_46A-66A_n261I DC_46A-66A_n261J DC_46A-66A_n261K DC_46A-66A_n261L DC_46A-66A_n261M DC_46C-66A_n261A DC_46D-66A_n261A | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |  |
| DC_46A-46A-66A_n261A DC_46A-46A-66A_n261G DC_46A-46A-66A_n261H DC_46A-46A-66A_n261I DC_46A-46A-66A_n261J DC_46A-46A-66A_n261K DC_46A-46A-66A_n261L DC_46A-46A-66A_n261M | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |  |
| DC_46A-46A-46A-66A_n261A DC_46A-46A-46A-66A_n261G DC_46A-46A-46A-66A_n261H DC_46A-46A-46A-66A_n261I DC_46A-46A-46A-66A_n261J DC_46A-46A-46A-66A_n261K DC_46A-46A-46A-66A_n261L DC_46A-46A-46A-66A_n261M | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |  |
| DC_46A-66A_n261(2A) DC_46C-66A_n261(2A) DC_46D-66A_n261(2A) DC_46A-66A_n261(2G) DC_46A-66A_n261(3A) DC_46A-66A_n261(A-G) DC_46A-66A_n261(A-H) DC_46A-66A_n261(A-I) DC_46A-66A_n261(A-J) DC_46A-66A_n261(A-K) DC_46A-66A_n261(A-L) DC_46A-66A_n261(G-H) DC_46A-66A_n261(2H) DC_46A-66A_n261(2A-G) DC_46A-66A_n261(2A-H) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |  |
| DC_46A-46A-66A_n261(A-G) DC_46A-46A-66A_n261(A-H) DC_46A-46A-66A_n261(A-I) DC_46A-46A-66A_n261(A-J) DC_46A-46A-66A_n261(A-K) DC_46A-46A-66A_n261(A-L) DC_46A-46A-66A_n261(G-H) DC_46A-46A-66A_n261(2A) DC_46A-46A-66A_n261(2G) DC_46A-46A-66A_n261(3A) DC_46A-46A-66A_n261(2H) DC_46A-46A-66A_n261(2A-G) DC_46A-46A-66A_n261(2A-H) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |  |
| DC_46A-46A-46A-66A_n261(2A) DC_46A-46A-46A-66A_n261(2G) DC_46A-46A-46A-66A_n261(3A) DC_46A-46A-46A-66A_n261(A-G) DC_46A-46A-46A-66A_n261(A-H) DC_46A-46A-46A-66A_n261(A-I) DC_46A-46A-46A-66A_n261(A-J) DC_46A-46A-46A-66A_n261(A-K) DC_46A-46A-46A-66A_n261(A-L) DC_46A-46A-46A-66A_n261(G-H) DC_46A-46A-46A-66A_n261(2H) DC_46A-46A-46A-66A_n261(2A-G) DC_46A-46A-46A-66A_n261(2A-H) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |  |
| DC_66A-71A_n257A DC_66A-71A_n257G DC_66A-71A_n257H DC_66A-71A_n257I DC_66A-71A_n257J DC_66A-71A_n257K DC_66A-71A_n257L DC_66A-71A_n257M | DC_66A_n257A DC_66A_n257G DC_71A_n257A DC_71A_n257G |  |
| DC_66A-71A_n258A DC_66A-71A_n258G DC_66A-71A_n258H DC_66A-71A_n258I DC_66A-71A_n258J DC_66A-71A_n258K DC_66A-71A_n258L DC_66A-71A_n258M | DC_66A_n258A DC_66A_n258G DC_71A_n258A DC_71A_n258G |  |
| DC_66A-71A_n260A DC_66A-71A_n260G DC_66A-71A_n260H DC_66A-71A_n260I DC_66A-71A_n260J DC_66A-71A_n260K DC_66A-71A_n260L DC_66A-71A_n260M DC_66A-71A_n260O DC_66A-71A_n260P DC_66A-71A_n260Q | DC_66A_n260A DC_66A_n260G DC_66A_n260O DC_71A_n260A DC_71A_n260G DC_71A_n260O |  |
| DC_66A-71A_n261A DC_66A-71A_n261G DC_66A-71A_n261H DC_66A-71A_n261I DC_66A-71A_n261J DC_66A-71A_n261K DC_66A-71A_n261L DC_66A-71A_n261M DC_66A-71A_n261O DC_66A-71A_n261P DC_66A-71A_n261Q | DC_66A_n261A DC_66A_n261G DC_66A_n261O DC_71A_n261A DC_71A_n261G DC_71A_n261O |  |
| DC_66A-71A_n257A DC_66A-71A_n257G DC_66A-71A_n257H DC_66A-71A_n257I DC_66A-71A_n257J DC_66A-71A_n257K DC_66A-71A_n257L DC_66A-71A_n257M | DC_66A_n257A DC_66A_n257G DC_71A_n257A DC_71A_n257G |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations. NOTE 3: Only single switched UL is supported. |  |  |

#### 5.5B.5.3	Inter-band EN-DC configurations including FR2 (four bands)

Table 5.5B.5.3-1: Inter-band EN-DC configurations including FR2 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A_n257A2 DC_1A-3A-5A_n257D DC_1A-3A-5A_n257E DC_1A-3A-5A_n257F DC_1A-3A-5A_n257G DC_1A-3A-5A_n257H DC_1A-3A-5A_n257I DC_1A-3A-5A_n257J DC_1A-3A-5A_n257K DC_1A-3A-5A_n257L DC_1A-3A-5A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I |
| DC_1A-3A-7A_n257A2 DC_1A-3A-7A_n257D DC_1A-3A-7A_n257E DC_1A-3A-7A_n257F DC_1A-3A-7A_n257G DC_1A-3A-7A_n257H DC_1A-3A-7A_n257I DC_1A-3A-7A_n257J DC_1A-3A-7A_n257K DC_1A-3A-7A_n257L DC_1A-3A-7A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_1A-3A-7A-7A_n257A DC_1A-3A-7A-7A_n257D DC_1A-3A-7A-7A_n257E DC_1A-3A-7A-7A_n257F DC_1A-3A-7A-7A_n257G DC_1A-3A-7A-7A_n257H DC_1A-3A-7A-7A_n257I DC_1A-3A-7A-7A_n257J DC_1A-3A-7A-7A_n257K DC_1A-3A-7A-7A_n257L DC_1A-3A-7A-7A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_1A-3A-7A_n258A DC_1A-3A-7A_n258B DC_1A-3A-7A_n258C DC_1A-3A-7A_n258D DC_1A-3A-7A_n258E DC_1A-3A-7A_n258F DC_1A-3A-7A_n258G DC_1A-3A-7A_n258H DC_1A-3A-7A_n258I DC_1A-3A-7A_n258J DC_1A-3A-7A_n258K DC_1A-3A-7A_n258L DC_1A-3A-7A_n258M DC_1A-3C-7A_n258A DC_1A-3C-7A_n258B DC_1A-3C-7A_n258C DC_1A-3C-7A_n258D DC_1A-3C-7A_n258E DC_1A-3C-7A_n258F DC_1A-3C-7A_n258G DC_1A-3C-7A_n258H DC_1A-3C-7A_n258I DC_1A-3C-7A_n258J DC_1A-3C-7A_n258K DC_1A-3C-7A_n258L DC_1A-3C-7A_n258M DC_1A-3A-7C_n258A DC_1A-3A-7C_n258B DC_1A-3A-7C_n258C DC_1A-3A-7C_n258D DC_1A-3A-7C_n258E DC_1A-3A-7C_n258F DC_1A-3A-7C_n258G DC_1A-3A-7C_n258H DC_1A-3A-7C_n258I DC_1A-3A-7C_n258J DC_1A-3A-7C_n258K DC_1A-3A-7C_n258L DC_1A-3A-7C_n258M DC_1A-3C-7C_n258A DC_1A-3C-7C_n258B DC_1A-3C-7C_n258C DC_1A-3C-7C_n258D DC_1A-3C-7C_n258E DC_1A-3C-7C_n258F DC_1A-3C-7C_n258G DC_1A-3C-7C_n258H DC_1A-3C-7C_n258I DC_1A-3C-7C_n258J DC_1A-3C-7C_n258K DC_1A-3C-7C_n258L DC_1A-3C-7C_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_3C_n258A DC_3C_n258D DC_3C_n258G DC_3C_n258H DC_3C_n258I DC_3A_n258A DC_3A_n258D DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_7A_n258A DC_7A_n258D DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n258A DC_7C_n258D DC_7C_n258G DC_7C_n258H DC_7C_n258I |
| DC_1A-3A-8A_n257A DC_1A-3A-8A_n257D DC_1A-3A-8A_n257E DC_1A-3A-8A_n257F DC_1A-3A-8A_n257G DC_1A-3A-8A_n257H DC_1A-3A-8A_n257I DC_1A-3A-8A_n257J DC_1A-3A-8A_n257K DC_1A-3A-8A_n257L DC_1A-3A-8A_n257M DC_1A-3C-8A_n257A DC_1A-3C-8A_n257D DC_1A-3C-8A_n257E DC_1A-3C-8A_n257F DC_1A-3C-8A_n257G DC_1A-3C-8A_n257H DC_1A-3C-8A_n257I DC_1A-3C-8A_n257J DC_1A-3C-8A_n257K DC_1A-3C-8A_n257L DC_1A-3C-8A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_1A-3A-11A_n257A DC_1A-3A-11A_n257G DC_1A-3A-11A_n257H DC_1A-3A-11A_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-3A-18A_n257A DC_1A-3A-18A_n257D DC_1A-3A-18A_n257E DC_1A-3A-18A_n257F DC_1A-3A-18A_n257G DC_1A-3A-18A_n257H DC_1A-3A-18A_n257I DC_1A-3A-18A_n257J DC_1A-3A-18A_n257K DC_1A-3A-18A_n257L DC_1A-3A-18A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-3A-19A_n257A2 DC_1A-3A-19A_n257D DC_1A-3A-19A_n257E DC_1A-3A-19A_n257F DC_1A-3A-19A_n257G DC_1A-3A-19A_n257H DC_1A-3A-19A_n257I DC_1A-3A-19A_n257J DC_1A-3A-19A_n257K DC_1A-3A-19A_n257L DC_1A-3A-19A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-3A-21A_n257A2 DC_1A-3A-21A_n257D DC_1A-3A-21A_n257E DC_1A-3A-21A_n257F DC_1A-3A-21A_n257G DC_1A-3A-21A_n257H DC_1A-3A-21A_n257I DC_1A-3A-21A_n257J DC_1A-3A-21A_n257K DC_1A-3A-21A_n257L DC_1A-3A-21A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_1A-3A-28A_n257A2 DC_1A-3A-28A_n257G DC_1A-3A-28A_n257H DC_1A-3A-28A_n257I DC_1A-3A-28A_n257J DC_1A-3A-28A_n257K DC_1A-3A-28A_n257L DC_1A-3A-28A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-3A-28A_n258A DC_1A-3A-28A_n258B DC_1A-3A-28A_n258C DC_1A-3A-28A_n258D DC_1A-3A-28A_n258E DC_1A-3A-28A_n258F DC_1A-3A-28A_n258G DC_1A-3A-28A_n258H DC_1A-3A-28A_n258I DC_1A-3A-28A_n258J DC_1A-3A-28A_n258K DC_1A-3A-28A_n258L DC_1A-3A-28A_n258M DC_1A-3C-28A_n258A DC_1A-3C-28A_n258B DC_1A-3C-28A_n258C DC_1A-3C-28A_n258D DC_1A-3C-28A_n258E DC_1A-3C-28A_n258F DC_1A-3C-28A_n258G DC_1A-3C-28A_n258H DC_1A-3C-28A_n258I DC_1A-3C-28A_n258J DC_1A-3C-28A_n258K DC_1A-3C-28A_n258L DC_1A-3C-28A_n258M | DC_1A_n258A DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3C_n258A DC_3C_n258G DC_3C_n258H DC_3C_n258I DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_1A-3A-41A_n257A DC_1A-3A-41A_n257D DC_1A-3A-41A_n257E DC_1A-3A-41A_n257F DC_1A-3A-41A_n257G DC_1A-3A-41A_n257H DC_1A-3A-41A_n257I DC_1A-3A-41A_n257J DC_1A-3A-41A_n257K DC_1A-3A-41A_n257L DC_1A-3A-41A_n257M DC_1A-3A-41C_n257A DC_1A-3A-41C_n257D DC_1A-3A-41C_n257E DC_1A-3A-41C_n257F DC_1A-3A-41C_n257G DC_1A-3A-41C_n257H DC_1A-3A-41C_n257I DC_1A-3A-41C_n257J DC_1A-3A-41C_n257K DC_1A-3A-41C_n257L DC_1A-3A-41C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-3A-42A_n257A DC_1A-3A-42A_n257D DC_1A-3A-42A_n257E DC_1A-3A-42A_n257F DC_1A-3A-42A_n257G DC_1A-3A-42A_n257H DC_1A-3A-42A_n257I DC_1A-3A-42A_n257J DC_1A-3A-42A_n257K DC_1A-3A-42A_n257L DC_1A-3A-42A_n257M DC_1A-3A-42C_n257A DC_1A-3A-42C_n257D DC_1A-3A-42C_n257E DC_1A-3A-42C_n257F DC_1A-3A-42C_n257G DC_1A-3A-42C_n257H DC_1A-3A-42C_n257I DC_1A-3A-42C_n257J DC_1A-3A-42C_n257K DC_1A-3A-42C_n257L DC_1A-3A-42C_n257M DC_1A-3A-42D_n257A DC_1A-3A-42D_n257G DC_1A-3A-42D_n257H DC_1A-3A-42D_n257I DC_1A-3A-42D_n257J DC_1A-3A-42D_n257K DC_1A-3A-42D_n257L DC_1A-3A-42D_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-5A-7A_n257A2 DC_1A-5A-7A_n257D DC_1A-5A-7A_n257E DC_1A-5A-7A_n257F DC_1A-5A-7A_n257G DC_1A-5A-7A_n257H DC_1A-5A-7A_n257I DC_1A-5A-7A_n257J DC_1A-5A-7A_n257K DC_1A-5A-7A_n257L DC_1A-5A-7A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_1A-5A-7A-7A_n257A DC_1A-5A-7A-7A_n257D DC_1A-5A-7A-7A_n257E DC_1A-5A-7A-7A_n257F DC_1A-5A-7A-7A_n257G DC_1A-5A-7A-7A_n257H DC_1A-5A-7A-7A_n257I DC_1A-5A-7A-7A_n257J DC_1A-5A-7A-7A_n257K DC_1A-5A-7A-7A_n257L DC_1A-5A-7A-7A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_1A-7A-28A_n257A DC_1A-7A-28A_n257G DC_1A-7A-28A_n257H DC_1A-7A-28A_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-7A-28A_n258A DC_1A-7A-28A_n258B DC_1A-7A-28A_n258C DC_1A-7A-28A_n258D DC_1A-7A-28A_n258E DC_1A-7A-28A_n258F DC_1A-7A-28A_n258G DC_1A-7A-28A_n258H DC_1A-7A-28A_n258I DC_1A-7A-28A_n258J DC_1A-7A-28A_n258K DC_1A-7A-28A_n258L DC_1A-7A-28A_n258M DC_1A-7C-28A_n258A DC_1A-7C-28A_n258B DC_1A-7C-28A_n258C DC_1A-7C-28A_n258D DC_1A-7C-28A_n258E DC_1A-7C-28A_n258F DC_1A-7C-28A_n258G DC_1A-7C-28A_n258H DC_1A-7C-28A_n258I DC_1A-7C-28A_n258J DC_1A-7C-28A_n258K DC_1A-7C-28A_n258L DC_1A-7C-28A_n258M | DC_1A_n258A DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n258A DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_1A-8A-11A_n257A DC_1A-8A-11A_n257D DC_1A-8A-11A_n257G DC_1A-8A-11A_n257H DC_1A-8A-11A_n257I | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-11A-18A_n257A DC_1A-11A-18A_n257G DC_1A-11A-18A_n257H DC_1A-11A-18A_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-18A-28A_n257A2 | DC_1A_n257A DC_18A_n257A DC_28A_n257A |
| DC_1A-18A-41A_n257A DC_1A-18A-41A_n257G DC_1A-18A-41A_n257H DC_1A-18A-41A_n257I DC_1A-18A-41C_n257A DC_1A-18A-41C_n257G DC_1A-18A-41C_n257H DC_1A-18A-41C_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-18A-42A_n257A DC_1A-18A-42A_n257D DC_1A-18A-42A_n257E DC_1A-18A-42A_n257F DC_1A-18A-42A_n257G DC_1A-18A-42A_n257H DC_1A-18A-42A_n257I DC_1A-18A-42A_n257J DC_1A-18A-42A_n257K DC_1A-18A-42A_n257L DC_1A-18A-42A_n257M DC_1A-18A-42C_n257A DC_1A-18A-42C_n257D DC_1A-18A-42C_n257E DC_1A-18A-42C_n257F DC_1A-18A-42C_n257G DC_1A-18A-42C_n257H DC_1A-18A-42C_n257I DC_1A-18A-42C_n257J DC_1A-18A-42C_n257K DC_1A-18A-42C_n257L DC_1A-18A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-19A-21A_n257A DC_1A-19A-21A_n257D DC_1A-19A-21A_n257E DC_1A-19A-21A_n257F DC_1A-19A-21A_n257G DC_1A-19A-21A_n257H DC_1A-19A-21A_n257I DC_1A-19A-21A_n257J DC_1A-19A-21A_n257K DC_1A-19A-21A_n257L DC_1A-19A-21A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_1A-19A-28A_n257A | DC_1A_n257A DC_19A_n257A DC_28A_n257A |
| DC_1A-19A-42A_n257A DC_1A-19A-42A_n257D DC_1A-19A-42A_n257E DC_1A-19A-42A_n257F DC_1A-19A-42C_n257A DC_1A-19A-42C_n257D DC_1A-19A-42C_n257E DC_1A-19A-42C_n257F DC_1A-19A-42A_n257G DC_1A-19A-42A_n257H DC_1A-19A-42A_n257I DC_1A-19A-42A_n257J DC_1A-19A-42A_n257K DC_1A-19A-42A_n257L DC_1A-19A-42A_n257M DC_1A-19A-42C_n257G DC_1A-19A-42C_n257H DC_1A-19A-42C_n257I DC_1A-19A-42C_n257J DC_1A-19A-42C_n257K DC_1A-19A-42C_n257L DC_1A-19A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_19A_n257A DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-21A-28A_n257A2 | DC_1A_n257A DC_21A_n257A DC_28A_n257A |
| DC_1A-21A-42A_n257A DC_1A-21A-42A_n257D DC_1A-21A-42A_n257E DC_1A-21A-42A_n257F DC_1A-21A-42A_n257G DC_1A-21A-42A_n257H DC_1A-21A-42A_n257I DC_1A-21A-42A_n257J DC_1A-21A-42A_n257K DC_1A-21A-42A_n257L DC_1A-21A-42A_n257M DC_1A-21A-42C_n257A DC_1A-21A-42C_n257D DC_1A-21A-42C_n257E DC_1A-21A-42C_n257F DC_1A-21A-42C_n257G DC_1A-21A-42C_n257H DC_1A-21A-42C_n257I DC_1A-21A-42C_n257J DC_1A-21A-42C_n257K DC_1A-21A-42C_n257L DC_1A-21A-42C_n257M DC_1A-21A-42D_n257A DC_1A-21A-42D_n257D DC_1A-21A-42D_n257E DC_1A-21A-42D_n257F | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-28A-42A_n257A DC_1A-28A-42A_n257D DC_1A-28A-42A_n257G DC_1A-28A-42A_n257H DC_1A-28A-42A_n257I DC_1A-28A-42A_n257J DC_1A-28A-42A_n257K DC_1A-28A-42A_n257L DC_1A-28A-42A_n257M DC_1A-28A-42C_n257A DC_1A-28A-42C_n257D DC_1A-28A-42C_n257G DC_1A-28A-42C_n257H DC_1A-28A-42C_n257I DC_1A-28A-42C_n257J DC_1A-28A-42C_n257K DC_1A-28A-42C_n257L DC_1A-28A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-41A-42A_n257A DC_1A-41A-42A_n257D DC_1A-41A-42A_n257E DC_1A-41A-42A_n257F DC_1A-41A-42A_n257G DC_1A-41A-42A_n257H DC_1A-41A-42A_n257I DC_1A-41A-42A_n257J DC_1A-41A-42A_n257K DC_1A-41A-42A_n257L DC_1A-41A-42A_n257M DC_1A-41A-42C_n257A DC_1A-41A-42C_n257D DC_1A-41A-42C_n257E DC_1A-41A-42C_n257F DC_1A-41A-42C_n257G DC_1A-41A-42C_n257H DC_1A-41A-42C_n257I DC_1A-41A-42C_n257J DC_1A-41A-42C_n257K DC_1A-41A-42C_n257L DC_1A-41A-42C_n257M DC_1A-41C-42A_n257A DC_1A-41C-42A_n257D DC_1A-41C-42A_n257E DC_1A-41C-42A_n257F DC_1A-41C-42A_n257G DC_1A-41C-42A_n257H DC_1A-41C-42A_n257I DC_1A-41C-42A_n257J DC_1A-41C-42A_n257K DC_1A-41C-42A_n257L DC_1A-41C-42A_n257M DC_1A-41C-42C_n257A DC_1A-41C-42C_n257D DC_1A-41C-42C_n257E DC_1A-41C-42C_n257F DC_1A-41C-42C_n257G DC_1A-41C-42C_n257H DC_1A-41C-42C_n257I DC_1A-41C-42C_n257J DC_1A-41C-42C_n257K DC_1A-41C-42C_n257L DC_1A-41C-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_2A-5A-30A_n260A DC_2A-5A-30A_n260G DC_2A-5A-30A_n260H DC_2A-5A-30A_n260I DC_2A-5A-30A_n260J DC_2A-5A-30A_n260K DC_2A-5A-30A_n260L DC_2A-5A-30A_n260M | DC_2A_n260A DC_5A_n260A DC_30A_n260A DC_2A_n260G DC_5A_n260G DC_30A_n260G DC_2A_n260H DC_5A_n260H DC_30A_n260H DC_2A_n260I DC_5A_n260I DC_30A_n260I DC_2A_n260J DC_5A_n260J DC_30A_n260J DC_2A_n260K DC_5A_n260K DC_30A_n260K DC_2A_n260L DC_5A_n260L DC_30A_n260L DC_2A_n260M DC_5A_n260M DC_30A_n260M |
| DC_2A-2A-5A-30A_n260A DC_2A-2A-5A-30A_n260G DC_2A-2A-5A-30A_n260H DC_2A-2A-5A-30A_n260I DC_2A-2A-5A-30A_n260J DC_2A-2A-5A-30A_n260K DC_2A-2A-5A-30A_n260L DC_2A-2A-5A-30A_n260M | DC_2A_n260A DC_5A_n260A DC_30A_n260A DC_2A_n260G DC_5A_n260G DC_30A_n260G DC_2A_n260H DC_5A_n260H DC_30A_n260H DC_2A_n260I DC_5A_n260I DC_30A_n260I DC_2A_n260J DC_5A_n260J DC_30A_n260J DC_2A_n260K DC_5A_n260K DC_30A_n260K DC_2A_n260L DC_5A_n260L DC_30A_n260L DC_2A_n260M DC_5A_n260M DC_30A_n260M |
| DC_2A-5A-66A_n260A DC_2A-5A-66A_n260G DC_2A-5A-66A_n260H DC_2A-5A-66A_n260I DC_2A-5A-66A_n260J DC_2A-5A-66A_n260K DC_2A-5A-66A_n260L DC_2A-5A-66A_n260M | DC_2A_n260A DC_5A_n260A DC_66A_n260A DC_2A_n260G DC_5A_n260G DC_66A_n260G DC_2A_n260H DC_5A_n260H DC_66A_n260H DC_2A_n260I DC_5A_n260I DC_66A_n260I DC_2A_n260J DC_5A_n260J DC_66A_n260J DC_2A_n260K DC_5A_n260K DC_66A_n260K DC_2A_n260L DC_5A_n260L DC_66A_n260L DC_2A_n260M DC_5A_n260M DC_66A_n260M |
| DC_2A-2A-5A-66A_n260A DC_2A-2A-5A-66A_n260G DC_2A-2A-5A-66A_n260H DC_2A-2A-5A-66A_n260I DC_2A-2A-5A-66A_n260J DC_2A-2A-5A-66A_n260K DC_2A-2A-5A-66A_n260L DC_2A-2A-5A-66A_n260M | DC_2A_n260A DC_5A_n260A DC_66A_n260A DC_2A_n260G DC_5A_n260G DC_66A_n260G DC_2A_n260H DC_5A_n260H DC_66A_n260H DC_2A_n260I DC_5A_n260I DC_66A_n260I DC_2A_n260J DC_5A_n260J DC_66A_n260J DC_2A_n260K DC_5A_n260K DC_66A_n260K DC_2A_n260L DC_5A_n260L DC_66A_n260L DC_2A_n260M DC_5A_n260M DC_66A_n260M |
| DC_2A-5A-66A-66A_n260A DC_2A-5A-66A-66A_n260G DC_2A-5A-66A-66A_n260H DC_2A-5A-66A-66A_n260I DC_2A-5A-66A-66A_n260J DC_2A-5A-66A-66A_n260K DC_2A-5A-66A-66A_n260L DC_2A-5A-66A-66A_n260M | DC_2A_n260A DC_5A_n260A DC_66A_n260A DC_2A_n260G DC_5A_n260G DC_66A_n260G DC_2A_n260H DC_5A_n260H DC_66A_n260H DC_2A_n260I DC_5A_n260I DC_66A_n260I DC_2A_n260J DC_5A_n260J DC_66A_n260J DC_2A_n260K DC_5A_n260K DC_66A_n260K DC_2A_n260L DC_5A_n260L DC_66A_n260L DC_2A_n260M DC_5A_n260M DC_66A_n260M |
| DC_2A-2A-5A-66A-66A_n260A DC_2A-2A-5A-66A-66A_n260G DC_2A-2A-5A-66A-66A_n260H DC_2A-2A-5A-66A-66A_n260I DC_2A-2A-5A-66A-66A_n260J DC_2A-2A-5A-66A-66A_n260K DC_2A-2A-5A-66A-66A_n260L DC_2A-2A-5A-66A-66A_n260M | DC_2A_n260A DC_5A_n260A DC_66A_n260A DC_2A_n260G DC_5A_n260G DC_66A_n260G DC_2A_n260H DC_5A_n260H DC_66A_n260H DC_2A_n260I DC_5A_n260I DC_66A_n260I DC_2A_n260J DC_5A_n260J DC_66A_n260J DC_2A_n260K DC_5A_n260K DC_66A_n260K DC_2A_n260L DC_5A_n260L DC_66A_n260L DC_2A_n260M DC_5A_n260M DC_66A_n260M |
| DC_2A-7A-28A_n258A DC_2A-7A-28A_n258G DC_2A-7A-28A_n258H DC_2A-7A-28A_n258I | DC_2A_n258A DC_2A_n258G DC_2A_n258H DC_2A_n258I DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_2A-12A-30A_n260A DC_2A-12A-30A_n260G DC_2A-12A-30A_n260H DC_2A-12A-30A_n260I DC_2A-12A-30A_n260J DC_2A-12A-30A_n260K DC_2A-12A-30A_n260L DC_2A-12A-30A_n260M | DC_2A_n260A DC_12A_n260A DC_30A_n260A DC_2A_n260G DC_12A_n260G DC_30A_n260G DC_2A_n260H DC_12A_n260H DC_30A_n260H DC_2A_n260I DC_12A_n260I DC_30A_n260I DC_2A_n260J DC_12A_n260J DC_30A_n260J DC_2A_n260K DC_12A_n260K DC_30A_n260K DC_2A_n260L DC_12A_n260L DC_30A_n260L DC_2A_n260M DC_12A_n260M DC_30A_n260M |
| DC_2A-2A-12A-30A_n260A DC_2A-2A-12A-30A_n260G DC_2A-2A-12A-30A_n260H DC_2A-2A-12A-30A_n260I DC_2A-2A-12A-30A_n260J DC_2A-2A-12A-30A_n260K DC_2A-2A-12A-30A_n260L DC_2A-2A-12A-30A_n260M | DC_2A_n260A DC_12A_n260A DC_30A_n260A DC_2A_n260G DC_12A_n260G DC_30A_n260G DC_2A_n260H DC_12A_n260H DC_30A_n260H DC_2A_n260I DC_12A_n260I DC_30A_n260I DC_2A_n260J DC_12A_n260J DC_30A_n260J DC_2A_n260K DC_12A_n260K DC_30A_n260K DC_2A_n260L DC_12A_n260L DC_30A_n260L DC_2A_n260M DC_12A_n260M DC_30A_n260M |
| DC_2A-12A-66A_n260A DC_2A-12A-66A_n260G DC_2A-12A-66A_n260H DC_2A-12A-66A_n260I DC_2A-12A-66A_n260J DC_2A-12A-66A_n260K DC_2A-12A-66A_n260L DC_2A-12A-66A_n260M | DC_2A_n260A DC_12A_n260A DC_66A_n260A DC_2A_n260G DC_12A_n260G DC_66A_n260G DC_2A_n260H DC_12A_n260H DC_66A_n260H DC_2A_n260I DC_12A_n260I DC_66A_n260I DC_2A_n260J DC_12A_n260J DC_66A_n260J DC_2A_n260K DC_12A_n260K DC_66A_n260K DC_2A_n260L DC_12A_n260L DC_66A_n260L DC_2A_n260M DC_12A_n260M DC_66A_n260M |
| DC_2A-2A-12A-66A_n260A DC_2A-2A-12A-66A_n260G DC_2A-2A-12A-66A_n260H DC_2A-2A-12A-66A_n260I DC_2A-2A-12A-66A_n260J DC_2A-2A-12A-66A_n260K DC_2A-2A-12A-66A_n260L DC_2A-2A-12A-66A_n260M | DC_2A_n260A DC_12A_n260A DC_66A_n260A DC_2A_n260G DC_12A_n260G DC_66A_n260G DC_2A_n260H DC_12A_n260H DC_66A_n260H DC_2A_n260I DC_12A_n260I DC_66A_n260I DC_2A_n260J DC_12A_n260J DC_66A_n260J DC_2A_n260K DC_12A_n260K DC_66A_n260K DC_2A_n260L DC_12A_n260L DC_66A_n260L DC_2A_n260M DC_12A_n260M DC_66A_n260M |
| DC_2A-12A-66A-66A_n260A DC_2A-12A-66A-66A_n260G DC_2A-12A-66A-66A_n260H DC_2A-12A-66A-66A_n260I DC_2A-12A-66A-66A_n260J DC_2A-12A-66A-66A_n260K DC_2A-12A-66A-66A_n260L DC_2A-12A-66A-66A_n260M | DC_2A_n260A DC_12A_n260A DC_66A_n260A DC_2A_n260G DC_12A_n260G DC_66A_n260G DC_2A_n260H DC_12A_n260H DC_66A_n260H DC_2A_n260I DC_12A_n260I DC_66A_n260I DC_2A_n260J DC_12A_n260J DC_66A_n260J DC_2A_n260K DC_12A_n260K DC_66A_n260K DC_2A_n260L DC_12A_n260L DC_66A_n260L DC_2A_n260M DC_12A_n260M DC_66A_n260M |
| DC_2A-2A-12A-66A-66A_n260A DC_2A-2A-12A-66A-66A_n260G DC_2A-2A-12A-66A-66A_n260H DC_2A-2A-12A-66A-66A_n260I DC_2A-2A-12A-66A-66A_n260J DC_2A-2A-12A-66A-66A_n260K DC_2A-2A-12A-66A-66A_n260L DC_2A-2A-12A-66A-66A_n260M | DC_2A_n260A DC_12A_n260A DC_66A_n260A DC_2A_n260G DC_12A_n260G DC_66A_n260G DC_2A_n260H DC_12A_n260H DC_66A_n260H DC_2A_n260I DC_12A_n260I DC_66A_n260I DC_2A_n260J DC_12A_n260J DC_66A_n260J DC_2A_n260K DC_12A_n260K DC_66A_n260K DC_2A_n260L DC_12A_n260L DC_66A_n260L DC_2A_n260M DC_12A_n260M DC_66A_n260M |
| DC_2A-13A-66A_n260A DC_2A-13A-66A_n260G DC_2A-13A-66A_n260H DC_2A-13A-66A_n260I DC_2A-13A-66A_n260J DC_2A-13A-66A_n260K DC_2A-13A-66A_n260L DC_2A-13A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260I DC_13A_n260J DC_13A_n260K DC_13A_n260L DC_13A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-13A-66A_n260(A-G) DC_2A-13A-66A_n260(A-H) DC_2A-13A-66A_n260(A-2G) DC_2A-13A-66A_n260(2A) DC_2A-13A-66A_n260(2A-G) DC_2A-13A-66A_n260(2A-2G) DC_2A-13A-66A_n260(3A) DC_2A-13A-66A_n260(3A-G) DC_2A-13A-66A_n260(4A) DC_2A-13A-66A_n260(5A) DC_2A-13A-66A_n260(6A) DC_2A-13A-66A_n260(G-H) DC_2A-13A-66A_n260(2G) DC_2A-13A-66A_n260(2H) | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_66A_n260A DC_66A_n260G DC_66A_n260H |
| DC_2A-13A-66A_n261A DC_2A-13A-66A_n261G DC_2A-13A-66A_n261H DC_2A-13A-66A_n261I DC_2A-13A-66A_n261J DC_2A-13A-66A_n261K DC_2A-13A-66A_n261L DC_2A-13A-66A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_13A_n261L DC_13A_n261M DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |
| DC_2A-13A-66A_n261(A-G) DC_2A-13A-66A_n261(A-G-H) DC_2A-13A-66A_n261(A-G-I) DC_2A-13A-66A_n261(A-2G) DC_2A-13A-66A_n261(A-H) DC_2A-13A-66A_n261(A-I) DC_2A-13A-66A_n261(A-J) DC_2A-13A-66A_n261(A-K) DC_2A-13A-66A_n261(2A) DC_2A-13A-66A_n261(2A-G) DC_2A-13A-66A_n261(2A-H) DC_2A-13A-66A_n261(2A-I) DC_2A-13A-66A_n261(3A) DC_2A-13A-66A_n261(3A-G) DC_2A-13A-66A_n261(4A) DC_2A-13A-66A_n261(G-H) DC_2A-13A-66A_n261(G-I) DC_2A-13A-66A_n261(G-J) DC_2A-13A-66A_n261(2G) DC_2A-13A-66A_n261(H-I) DC_2A-13A-66A_n261(2H) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K |
| DC_2A-14A-30A_n260A DC_2A-14A-30A_n260G DC_2A-14A-30A_n260H DC_2A-14A-30A_n260I DC_2A-14A-30A_n260J DC_2A-14A-30A_n260K DC_2A-14A-30A_n260L DC_2A-14A-30A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_2A-2A-14A-30A_n260A DC_2A-2A-14A-30A_n260G DC_2A-2A-14A-30A_n260H DC_2A-2A-14A-30A_n260I DC_2A-2A-14A-30A_n260J DC_2A-2A-14A-30A_n260K DC_2A-2A-14A-30A_n260L DC_2A-2A-14A-30A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_2A-14A-66A_n260A DC_2A-14A-66A_n260G DC_2A-14A-66A_n260H DC_2A-14A-66A_n260I DC_2A-14A-66A_n260J DC_2A-14A-66A_n260K DC_2A-14A-66A_n260L DC_2A-14A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-2A-14A-66A_n260A DC_2A-2A-14A-66A_n260G DC_2A-2A-14A-66A_n260H DC_2A-2A-14A-66A_n260I DC_2A-2A-14A-66A_n260J DC_2A-2A-14A-66A_n260K DC_2A-2A-14A-66A_n260L DC_2A-2A-14A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-14A-66A-66A_n260A DC_2A-14A-66A-66A_n260G DC_2A-14A-66A-66A_n260H DC_2A-14A-66A-66A_n260I DC_2A-14A-66A-66A_n260J DC_2A-14A-66A-66A_n260K DC_2A-14A-66A-66A_n260L DC_2A-14A-66A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-2A-14A-66A-66A_n260A DC_2A-2A-14A-66A-66A_n260G DC_2A-2A-14A-66A-66A_n260H DC_2A-2A-14A-66A-66A_n260I DC_2A-2A-14A-66A-66A_n260J DC_2A-2A-14A-66A-66A_n260K DC_2A-2A-14A-66A-66A_n260L DC_2A-2A-14A-66A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-29A-30A_n260A DC_2A-29A-30A_n260G DC_2A-29A-30A_n260H DC_2A-29A-30A_n260I DC_2A-29A-30A_n260J DC_2A-29A-30A_n260K DC_2A-29A-30A_n260L DC_2A-29A-30A_n260M | DC_2A_n260A DC_30A_n260A DC_2A_n260G DC_30A_n260G DC_2A_n260H DC_30A_n260H DC_2A_n260I DC_30A_n260I DC_2A_n260J DC_30A_n260J DC_2A_n260K DC_30A_n260K DC_2A_n260L DC_30A_n260L DC_2A_n260M DC_30A_n260M |
| DC_2A-2A-29A-30A_n260A DC_2A-2A-29A-30A_n260G DC_2A-2A-29A-30A_n260H DC_2A-2A-29A-30A_n260I DC_2A-2A-29A-30A_n260J DC_2A-2A-29A-30A_n260K DC_2A-2A-29A-30A_n260L DC_2A-2A-29A-30A_n260M | DC_2A_n260A DC_30A_n260A DC_2A_n260G DC_30A_n260G DC_2A_n260H DC_30A_n260H DC_2A_n260I DC_30A_n260I DC_2A_n260J DC_30A_n260J DC_2A_n260K DC_30A_n260K DC_2A_n260L DC_30A_n260L DC_2A_n260M DC_30A_n260M |
| DC_2A-29A-66A_n260A DC_2A-29A-66A_n260G DC_2A-29A-66A_n260H DC_2A-29A-66A_n260I DC_2A-29A-66A_n260J DC_2A-29A-66A_n260K DC_2A-29A-66A_n260L DC_2A-29A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M |
| DC_2A-30A-66A_n260A DC_2A-30A-66A_n260G DC_2A-30A-66A_n260H DC_2A-30A-66A_n260I DC_2A-30A-66A_n260J DC_2A-30A-66A_n260K DC_2A-30A-66A_n260L DC_2A-30A-66A_n260M | DC_2A_n260A DC_30A_n260A DC_66A_n260A DC_2A_n260G DC_30A_n260G DC_66A_n260G DC_2A_n260H DC_30A_n260H DC_66A_n260H DC_2A_n260I DC_30A_n260I DC_66A_n260I DC_2A_n260J DC_30A_n260J DC_66A_n260J DC_2A_n260K DC_30A_n260K DC_66A_n260K DC_2A_n260L DC_30A_n260L DC_66A_n260L DC_2A_n260M DC_30A_n260M DC_66A_n260M |
| DC_2A-30A-66A-66A_n260A DC_2A-30A-66A-66A_n260G DC_2A-30A-66A-66A_n260H DC_2A-30A-66A-66A_n260I DC_2A-30A-66A-66A_n260J DC_2A-30A-66A-66A_n260K DC_2A-30A-66A-66A_n260L DC_2A-30A-66A-66A_n260M | DC_2A_n260A DC_30A_n260A DC_66A_n260A DC_2A_n260G DC_30A_n260G DC_66A_n260G DC_2A_n260H DC_30A_n260H DC_66A_n260H DC_2A_n260I DC_30A_n260I DC_66A_n260I DC_2A_n260J DC_30A_n260J DC_66A_n260J DC_2A_n260K DC_30A_n260K DC_66A_n260K DC_2A_n260L DC_30A_n260L DC_66A_n260L DC_2A_n260M DC_30A_n260M DC_66A_n260M |
| DC_2A-2A-30A-66A_n260A DC_2A-2A-30A-66A_n260G DC_2A-2A-30A-66A_n260H DC_2A-2A-30A-66A_n260I DC_2A-2A-30A-66A_n260J DC_2A-2A-30A-66A_n260K DC_2A-2A-30A-66A_n260L DC_2A-2A-30A-66A_n260M | DC_2A_n260A DC_30A_n260A DC_66A_n260A DC_2A_n260G DC_30A_n260G DC_66A_n260G DC_2A_n260H DC_30A_n260H DC_66A_n260H DC_2A_n260I DC_30A_n260I DC_66A_n260I DC_2A_n260J DC_30A_n260J DC_66A_n260J DC_2A_n260K DC_30A_n260K DC_66A_n260K DC_2A_n260L DC_30A_n260L DC_66A_n260L DC_2A_n260M DC_30A_n260M DC_66A_n260M |
| DC_2A-46A-66A_n260A DC_2A-46A-66A_n260G DC_2A-46A-66A_n260H DC_2A-46A-66A_n260I DC_2A-46A-66A_n260J DC_2A-46A-66A_n260K DC_2A-46A-66A_n260L DC_2A-46A-66A_n260M DC_2A-46C-66A_n260A DC_2A-46C-66A_n260G DC_2A-46C-66A_n260H DC_2A-46C-66A_n260I DC_2A-46C-66A_n260J DC_2A-46C-66A_n260K DC_2A-46C-66A_n260L DC_2A-46C-66A_n260M DC_2A-46D-66A_n260A DC_2A-46D-66A_n260G DC_2A-46D-66A_n260H DC_2A-46D-66A_n260I DC_2A-46D-66A_n260J DC_2A-46D-66A_n260K DC_2A-46D-66A_n260L DC_2A-46D-66A_n260M DC_2A-46E-66A_n260A DC_2A-46E-66A_n260G DC_2A-46E-66A_n260H DC_2A-46E-66A_n260I DC_2A-46E-66A_n260J DC_2A-46E-66A_n260K DC_2A-46E-66A_n260L DC_2A-46E-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M |
| DC_2A-46A-66A_n261A DC_2A-46C-66A_n261A DC_2A-46D-66A_n261A | DC_2A_n261A DC_66A_n261A |
| DC_2A-46A-66A_n261(2A) DC_2A-46C-66A_n261(2A) DC_2A-46D-66A_n261(2A) | DC_2A_n261A DC_66A_n261A |
| DC_3A-5A-7A_n257A2 DC_3A-5A-7A_n257D DC_3A-5A-7A_n257E DC_3A-5A-7A_n257F DC_3A-5A-7A_n257G DC_3A-5A-7A_n257H DC_3A-5A-7A_n257I DC_3A-5A-7A_n257J DC_3A-5A-7A_n257K DC_3A-5A-7A_n257L DC_3A-5A-7A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_3A-5A-7A-7A_n257A2 DC_3A-5A-7A-7A_n257D DC_3A-5A-7A-7A_n257E DC_3A-5A-7A-7A_n257F DC_3A-5A-7A-7A_n257G DC_3A-5A-7A-7A_n257H DC_3A-5A-7A-7A_n257I DC_3A-5A-7A-7A_n257J DC_3A-5A-7A-7A_n257K DC_3A-5A-7A-7A_n257L DC_3A-5A-7A-7A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_3A-7A-8A_n257A DC_3A-7A-8A_n257D DC_3A-7A-8A_n257E DC_3A-7A-8A_n257F DC_3A-7A-8A_n257G DC_3A-7A-8A_n257H DC_3A-7A-8A_n257I DC_3A-7A-8A_n257J DC_3A-7A-8A_n257K DC_3A-7A-8A_n257L DC_3A-7A-8A_n257M | DC_3A_n257A DC_7A_n257A DC_8A_n257A |
| DC_3A-3A-7A-8A_n257A DC_3A-3A-7A-8A_n257D DC_3A-3A-7A-8A_n257E DC_3A-3A-7A-8A_n257F DC_3A-3A-7A-8A_n257G DC_3A-3A-7A-8A_n257H DC_3A-3A-7A-8A_n257I DC_3A-3A-7A-8A_n257J DC_3A-3A-7A-8A_n257K DC_3A-3A-7A-8A_n257L DC_3A-3A-7A-8A_n257M | DC_3A_n257A DC_7A_n257A DC_8A_n257A |
| DC_3A-7A-7A-8A_n257A DC_3A-7A-7A-8A_n257D DC_3A-7A-7A-8A_n257E DC_3A-7A-7A-8A_n257F DC_3A-7A-7A-8A_n257G DC_3A-7A-7A-8A_n257H DC_3A-7A-7A-8A_n257I DC_3A-7A-7A-8A_n257J DC_3A-7A-7A-8A_n257K DC_3A-7A-7A-8A_n257L DC_3A-7A-7A-8A_n257M | DC_3A_n257A DC_7A_n257A DC_8A_n257A |
| DC_3A-3A-7A-7A-8A_n257A DC_3A-3A-7A-7A-8A_n257D DC_3A-3A-7A-7A-8A_n257E DC_3A-3A-7A-7A-8A_n257F DC_3A-3A-7A-7A-8A_n257G DC_3A-3A-7A-7A-8A_n257H DC_3A-3A-7A-7A-8A_n257I DC_3A-3A-7A-7A-8A_n257J DC_3A-3A-7A-7A-8A_n257K DC_3A-3A-7A-7A-8A_n257L DC_3A-3A-7A-7A-8A_n257M | DC_3A_n257A DC_7A_n257A DC_8A_n257A |
| DC_3A-7A-8A_n258A DC_3A-7A-8A_n258D DC_3A-7A-8A_n258E DC_3A-7A-8A_n258F DC_3A-7A-8A_n258G DC_3A-7A-8A_n258H DC_3A-7A-8A_n258I DC_3A-7A-8A_n258J DC_3A-7A-8A_n258K DC_3A-7A-8A_n258L DC_3A-7A-8A_n258M | DC_3A_n258A DC_7A_n258A DC_8A_n258A |
| DC_3A-7A-28A_n258A DC_3A-7A-28A_n258B DC_3A-7A-28A_n258C DC_3A-7A-28A_n258D DC_3A-7A-28A_n258E DC_3A-7A-28A_n258F DC_3A-7A-28A_n258G DC_3A-7A-28A_n258H DC_3A-7A-28A_n258I DC_3A-7A-28A_n258J DC_3A-7A-28A_n258K DC_3A-7A-28A_n258L DC_3A-7A-28A_n258M DC_3C-7A-28A_n258A DC_3C-7A-28A_n258B DC_3C-7A-28A_n258C DC_3C-7A-28A_n258D DC_3C-7A-28A_n258E DC_3C-7A-28A_n258F DC_3C-7A-28A_n258G DC_3C-7A-28A_n258H DC_3C-7A-28A_n258I DC_3C-7A-28A_n258J DC_3C-7A-28A_n258K DC_3C-7A-28A_n258L DC_3C-7A-28A_n258M DC_3A-7C-28A_n258A DC_3A-7C-28A_n258B DC_3A-7C-28A_n258C DC_3A-7C-28A_n258D DC_3A-7C-28A_n258E DC_3A-7C-28A_n258F DC_3A-7C-28A_n258G DC_3A-7C-28A_n258H DC_3A-7C-28A_n258I DC_3A-7C-28A_n258J DC_3A-7C-28A_n258K DC_3A-7C-28A_n258L DC_3A-7C-28A_n258M DC_3C-7C-28A_n258A DC_3C-7C-28A_n258B DC_3C-7C-28A_n258C DC_3C-7C-28A_n258D DC_3C-7C-28A_n258E DC_3C-7C-28A_n258F DC_3C-7C-28A_n258G DC_3C-7C-28A_n258H DC_3C-7C-28A_n258I DC_3C-7C-28A_n258J DC_3C-7C-28A_n258K DC_3C-7C-28A_n258L DC_3C-7C-28A_n258M | DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3C_n258A DC_3C_n258G DC_3C_n258H DC_3C_n258I DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n258A DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_3A-7A-28A_n257A DC_3A-7A-28A_n257G DC_3A-7A-28A_n257H DC_3A-7A-28A_n257I | DC_3A_n257A DC_7A_n257A DC_28A_n257A |
| DC_3A-8A-11A_n257A DC_3A-8A-11A_n257G DC_3A-8A-11A_n257H DC_3A-8A-11A_n257I | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_3A-18A-42A_n257A DC_3A-18A-42A_n257D DC_3A-18A-42A_n257E DC_3A-18A-42A_n257F DC_3A-18A-42A_n257G DC_3A-18A-42A_n257H DC_3A-18A-42A_n257I DC_3A-18A-42A_n257J DC_3A-18A-42A_n257K DC_3A-18A-42A_n257L DC_3A-18A-42A_n257M DC_3A-18A-42C_n257A DC_3A-18A-42C_n257D DC_3A-18A-42C_n257E DC_3A-18A-42C_n257F DC_3A-18A-42C_n257G DC_3A-18A-42C_n257H DC_3A-18A-42C_n257I DC_3A-18A-42C_n257J DC_3A-18A-42C_n257K DC_3A-18A-42C_n257L DC_3A-18A-42C_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-19A-21A_n257A2 DC_3A-19A-21A_n257D DC_3A-19A-21A_n257E DC_3A-19A-21A_n257F | DC_3A_n257A DC_19A_n257A DC_21A_n257A |
| DC_3A-19A-42A_n257A DC_3A-19A-42A_n257D DC_3A-19A-42A_n257E DC_3A-19A-42A_n257F DC_3A-19A-42A_n257G DC_3A-19A-42A_n257H DC_3A-19A-42A_n257I DC_3A-19A-42A_n257J DC_3A-19A-42A_n257K DC_3A-19A-42A_n257L DC_3A-19A-42A_n257M DC_3A-19A-42C_n257A DC_3A-19A-42C_n257D DC_3A-19A-42C_n257E DC_3A-19A-42C_n257F DC_3A-19A-42C_n257G DC_3A-19A-42C_n257H DC_3A-19A-42C_n257I DC_3A-19A-42C_n257J DC_3A-19A-42C_n257K DC_3A-19A-42C_n257L DC_3A-19A-42C_n257M DC_3A-19A-42D_n257A DC_3A-19A-42D_n257D DC_3A-19A-42D_n257E DC_3A-19A-42D_n257F | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_3A-21A-42A_n257A DC_3A-21A-42A_n257D DC_3A-21A-42A_n257E DC_3A-21A-42A_n257F DC_3A-21A-42A_n257G DC_3A-21A-42A_n257H DC_3A-21A-42A_n257I DC_3A-21A-42A_n257J DC_3A-21A-42A_n257K DC_3A-21A-42A_n257L DC_3A-21A-42A_n257M DC_3A-21A-42C_n257A DC_3A-21A-42C_n257D DC_3A-21A-42C_n257E DC_3A-21A-42C_n257F DC_3A-21A-42C_n257G DC_3A-21A-42C_n257H DC_3A-21A-42C_n257I DC_3A-21A-42C_n257J DC_3A-21A-42C_n257K DC_3A-21A-42C_n257L DC_3A-21A-42C_n257M DC_3A-21A-42D_n257A DC_3A-21A-42D_n257D DC_3A-21A-42D_n257E DC_3A-21A-42D_n257F | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_3A-28A-41A_n257A DC_3A-28A-41A_n257G DC_3A-28A-41A_n257H DC_3A-28A-41A_n257I DC_3A-28A-42A_n257J DC_3A-28A-42A_n257K DC_3A-28A-42A_n257L DC_3A-28A-42A_n257M DC_3A-28A-41C_n257A DC_3A-28A-41C_n257G DC_3A-28A-41C_n257H DC_3A-28A-41C_n257I DC_3A-28A-42C_n257J DC_3A-28A-42C_n257K DC_3A-28A-42C_n257L DC_3A-28A-42C_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-28A-42A_n257A DC_3A-28A-42A_n257D DC_3A-28A-42A_n257G DC_3A-28A-42A_n257H DC_3A-28A-42A_n257I DC_3A-28A-42C_n257A DC_3A-28A-42C_n257D DC_3A-28A-42C_n257G DC_3A-28A-42C_n257H DC_3A-28A-42C_n257I | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-41A-42A_n257A DC_3A-41A-42A_n257D DC_3A-41A-42A_n257E DC_3A-41A-42A_n257F DC_3A-41A-42A_n257G DC_3A-41A-42A_n257H DC_3A-41A-42A_n257I DC_3A-41A-42A_n257J DC_3A-41A-42A_n257K DC_3A-41A-42A_n257L DC_3A-41A-42A_n257M DC_3A-41A-42C_n257A DC_3A-41A-42C_n257D DC_3A-41A-42C_n257E DC_3A-41A-42C_n257F DC_3A-41A-42C_n257G DC_3A-41A-42C_n257H DC_3A-41A-42C_n257I DC_3A-41A-42C_n257J DC_3A-41A-42C_n257K DC_3A-41A-42C_n257L DC_3A-41A-42C_n257M DC_3A-41C-42A_n257A DC_3A-41C-42A_n257D DC_3A-41C-42A_n257E DC_3A-41C-42A_n257F DC_3A-41C-42A_n257G DC_3A-41C-42A_n257H DC_3A-41C-42A_n257I DC_3A-41C-42A_n257J DC_3A-41C-42A_n257K DC_3A-41C-42A_n257L DC_3A-41C-42A_n257M DC_3A-41C-42C_n257A DC_3A-41C-42C_n257D DC_3A-41C-42C_n257E DC_3A-41C-42C_n257F DC_3A-41C-42C_n257G DC_3A-41C-42C_n257H DC_3A-41C-42C_n257I DC_3A-41C-42C_n257J DC_3A-41C-42C_n257K DC_3A-41C-42C_n257L DC_3A-41C-42C_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_(n)3AA-n8A-n257A DC_(n)3AA-n8A-n257G DC_(n)3AA-n8A-n257H DC_(n)3AA-n8A-n257I DC_(n)3AA-n8A-n257J DC_(n)3AA-n8A-n257K DC_(n)3AA-n8A-n257L DC_(n)3AA-n8A-n257M | DC_(n)3AA3 DC_3A_n8A DC_3A_n257A |
| DC_(n)3AA-n77A-n257A DC_(n)3AA-n77A-n257G DC_(n)3AA-n77A-n257H DC_(n)3AA-n77A-n257I DC_(n)3AA-n77A-n257J DC_(n)3AA-n77A-n257K DC_(n)3AA-n77A-n257L DC_(n)3AA-n77A-n257M DC_(n)3AA-n77(2A)-n257A | DC_(n)3AA3 DC_3A_n77A DC_3A_n257A |
| DC_5A-30A-66A_n260A DC_5A-30A-66A_n260G DC_5A-30A-66A_n260H DC_5A-30A-66A_n260I DC_5A-30A-66A_n260J DC_5A-30A-66A_n260K DC_5A-30A-66A_n260L DC_5A-30A-66A_n260M | DC_5A_n260A DC_30A_n260A DC_66A_n260A DC_5A_n260G DC_30A_n260G DC_66A_n260G DC_5A_n260H DC_30A_n260H DC_66A_n260H DC_5A_n260I DC_30A_n260I DC_66A_n260I DC_5A_n260J DC_30A_n260J DC_66A_n260J DC_5A_n260K DC_30A_n260K DC_66A_n260K DC_5A_n260L DC_30A_n260L DC_66A_n260L DC_5A_n260M DC_30A_n260M DC_66A_n260M |
| DC_5A-30A-66A-66A_n260A DC_5A-30A-66A-66A_n260G DC_5A-30A-66A-66A_n260H DC_5A-30A-66A-66A_n260I DC_5A-30A-66A-66A_n260J DC_5A-30A-66A-66A_n260K DC_5A-30A-66A-66A_n260L DC_5A-30A-66A-66A_n260M | DC_5A_n260A DC_30A_n260A DC_66A_n260A DC_5A_n260G DC_30A_n260G DC_66A_n260G DC_5A_n260H DC_30A_n260H DC_66A_n260H DC_5A_n260I DC_30A_n260I DC_66A_n260I DC_5A_n260J DC_30A_n260J DC_66A_n260J DC_5A_n260K DC_30A_n260K DC_66A_n260K DC_5A_n260L DC_30A_n260L DC_66A_n260L DC_5A_n260M DC_30A_n260M DC_66A_n260M |
| DC_8A_n40A-n41A-n258A | DC_8A_n40A DC_8A_n41A DC_8A_n258A |
| DC_12A-30A-66A_n260A DC_12A-30A-66A_n260G DC_12A-30A-66A_n260H DC_12A-30A-66A_n260I DC_12A-30A-66A_n260J DC_12A-30A-66A_n260K DC_12A-30A-66A_n260L DC_12A-30A-66A_n260M | DC_12A_n260A DC_30A_n260A DC_66A_n260A DC_12A_n260G DC_30A_n260G DC_66A_n260G DC_12A_n260H DC_30A_n260H DC_66A_n260H DC_12A_n260I DC_30A_n260I DC_66A_n260I DC_12A_n260J DC_30A_n260J DC_66A_n260J DC_12A_n260K DC_30A_n260K DC_66A_n260K DC_12A_n260L DC_30A_n260L DC_66A_n260L DC_12A_n260M DC_30A_n260M DC_66A_n260M |
| DC_12A-30A-66A-66A_n260A DC_12A-30A-66A-66A_n260G DC_12A-30A-66A-66A_n260H DC_12A-30A-66A-66A_n260I DC_12A-30A-66A-66A_n260J DC_12A-30A-66A-66A_n260K DC_12A-30A-66A-66A_n260L DC_12A-30A-66A-66A_n260M | DC_12A_n260A DC_30A_n260A DC_66A_n260A DC_12A_n260G DC_30A_n260G DC_66A_n260G DC_12A_n260H DC_30A_n260H DC_66A_n260H DC_12A_n260I DC_30A_n260I DC_66A_n260I DC_12A_n260J DC_30A_n260J DC_66A_n260J DC_12A_n260K DC_30A_n260K DC_66A_n260K DC_12A_n260L DC_30A_n260L DC_66A_n260L DC_12A_n260M DC_30A_n260M DC_66A_n260M |
| DC_14A-30A-66A_n260A DC_14A-30A-66A_n260G DC_14A-30A-66A_n260H DC_14A-30A-66A_n260I DC_14A-30A-66A_n260J DC_14A-30A-66A_n260K DC_14A-30A-66A_n260L DC_14A-30A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_14A-30A-66A-66A_n260A DC_14A-30A-66A-66A_n260G DC_14A-30A-66A-66A_n260H DC_14A-30A-66A-66A_n260I DC_14A-30A-66A-66A_n260J DC_14A-30A-66A-66A_n260K DC_14A-30A-66A-66A_n260L DC_14A-30A-66A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_19A-21A-42A_n257A2 DC_19A-21A-42A_n257D2 DC_19A-21A-42A_n257E2 DC_19A-21A-42A_n257F2 DC_19A-21A-42A_n257G2 DC_19A-21A-42A_n257H2 DC_19A-21A-42A_n257I2 DC_19A-21A-42A_n257J DC_19A-21A-42A_n257K DC_19A-21A-42A_n257L DC_19A-21A-42A_n257M DC_19A-21A-42C_n257A2 DC_19A-21A-42C_n257D2 DC_19A-21A-42C_n257E2 DC_19A-21A-42C_n257F2 DC_19A-21A-42C_n257G2 DC_19A-21A-42C_n257H2 DC_19A-21A-42C_n257I2 DC_19A-21A-42C_n257J DC_19A-21A-42C_n257K DC_19A-21A-42C_n257L DC_19A-21A-42C_n257M | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-28A-42A_n257A DC_19A-28A-42C_n257A | DC_19A_n257A DC_28A_n257A DC_42A_n257A |
| DC_21A-28A-42A_n257A2 DC_21A-28A-42C_n257A2 | DC_21A_n257A DC_28A_n257A DC_42A_n257A |
| DC_28A-41A-42A_n257A DC_28A-41A-42A_n257G DC_28A-41A-42A_n257H DC_28A-41A-42A_n257I DC_28A-41C-42A_n257A DC_28A-41C-42A_n257G DC_28A-41C-42A_n257H DC_28A-41C-42A_n257I DC_28A-41A-42C_n257A DC_28A-41A-42C_n257G DC_28A-41A-42C_n257H DC_28A-41A-42C_n257I DC_28A-41C-42C_n257A DC_28A-41C-42C_n257G DC_28A-41C-42C_n257H DC_28A-41C-42C_n257I | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_29A-30A-66A_n260A DC_29A-30A-66A_n260G DC_29A-30A-66A_n260H DC_29A-30A-66A_n260I DC_29A-30A-66A_n260J DC_29A-30A-66A_n260K DC_29A-30A-66A_n260L DC_29A-30A-66A_n260M | DC_30A_n260A DC_66A_n260A DC_30A_n260G DC_66A_n260G DC_30A_n260H DC_66A_n260H DC_30A_n260I DC_66A_n260I DC_30A_n260J DC_66A_n260J DC_30A_n260K DC_66A_n260K DC_30A_n260L DC_66A_n260L DC_30A_n260M DC_66A_n260M |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations. NOTE 3: Only single switched UL is supported. |  |

#### 5.5B.5.4	Inter-band EN-DC configurations including FR2 (five bands)

Table 5.5B.5.4-1: Inter-band EN-DC configurations including FR2 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A_n257A DC_1A-3A-5A-7A_n257D DC_1A-3A-5A-7A_n257E DC_1A-3A-5A-7A_n257F DC_1A-3A-5A-7A_n257G DC_1A-3A-5A-7A_n257H DC_1A-3A-5A-7A_n257I DC_1A-3A-5A-7A_n257J DC_1A-3A-5A-7A_n257K DC_1A-3A-5A-7A_n257L DC_1A-3A-5A-7A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_1A-3A-5A-7A-7A_n257A2 DC_1A-3A-5A-7A-7A_n257D DC_1A-3A-5A-7A-7A_n257E DC_1A-3A-5A-7A-7A_n257F DC_1A-3A-5A-7A-7A_n257G DC_1A-3A-5A-7A-7A_n257H DC_1A-3A-5A-7A-7A_n257I DC_1A-3A-5A-7A-7A_n257J DC_1A-3A-5A-7A-7A_n257K DC_1A-3A-5A-7A-7A_n257L DC_1A-3A-5A-7A-7A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_1A-3A-7A-28A_n257A DC_1A-3A-7A-28A_n257G DC_1A-3A-7A-28A_n257H DC_1A-3A-7A-28A_n257I | DC_1A_n257A DC_3A_n257A DC_7A_n257A DC_28A_n257A |
| DC_1A-3A-7A-28A_n258A DC_1A-3A-7A-28A_n258B DC_1A-3A-7A-28A_n258C DC_1A-3A-7A-28A_n258D DC_1A-3A-7A-28A_n258E DC_1A-3A-7A-28A_n258F DC_1A-3A-7A-28A_n258G DC_1A-3A-7A-28A_n258H DC_1A-3A-7A-28A_n258I DC_1A-3A-7A-28A_n258J DC_1A-3A-7A-28A_n258K DC_1A-3A-7A-28A_n258L DC_1A-3A-7A-28A_n258M DC_1A-3C-7A-28A_n258A DC_1A-3C-7A-28A_n258B DC_1A-3C-7A-28A_n258C DC_1A-3C-7A-28A_n258D DC_1A-3C-7A-28A_n258E DC_1A-3C-7A-28A_n258F DC_1A-3C-7A-28A_n258G DC_1A-3C-7A-28A_n258H DC_1A-3C-7A-28A_n258I DC_1A-3C-7A-28A_n258J DC_1A-3C-7A-28A_n258K DC_1A-3C-7A-28A_n258L DC_1A-3C-7A-28A_n258M DC_1A-3A-7C-28A_n258A DC_1A-3A-7C-28A_n258B DC_1A-3A-7C-28A_n258C DC_1A-3A-7C-28A_n258D DC_1A-3A-7C-28A_n258E DC_1A-3A-7C-28A_n258F DC_1A-3A-7C-28A_n258G DC_1A-3A-7C-28A_n258H DC_1A-3A-7C-28A_n258I DC_1A-3A-7C-28A_n258J DC_1A-3A-7C-28A_n258K DC_1A-3A-7C-28A_n258L DC_1A-3A-7C-28A_n258M DC_1A-3C-7C-28A_n258A DC_1A-3C-7C-28A_n258B DC_1A-3C-7C-28A_n258C DC_1A-3C-7C-28A_n258D DC_1A-3C-7C-28A_n258E DC_1A-3C-7C-28A_n258F DC_1A-3C-7C-28A_n258G DC_1A-3C-7C-28A_n258H DC_1A-3C-7C-28A_n258I DC_1A-3C-7C-28A_n258J DC_1A-3C-7C-28A_n258K DC_1A-3C-7C-28A_n258L DC_1A-3C-7C-28A_n258M | DC_1A_n258A DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3C_n258A DC_3C_n258G DC_3C_n258H DC_3C_n258I DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n258A DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_1A-3A-8A-11A_n257A DC_1A-3A-8A-11A_n257G DC_1A-3A-8A-11A_n257H DC_1A-3A-8A-11A_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-3A-18A-42A_n257A DC_1A-3A-18A-42A_n257D DC_1A-3A-18A-42A_n257E DC_1A-3A-18A-42A_n257F DC_1A-3A-18A-42A_n257G DC_1A-3A-18A-42A_n257H DC_1A-3A-18A-42A_n257I DC_1A-3A-18A-42A_n257J DC_1A-3A-18A-42A_n257K DC_1A-3A-18A-42A_n257L DC_1A-3A-18A-42A_n257M DC_1A-3A-18A-42C_n257A DC_1A-3A-18A-42C_n257D DC_1A-3A-18A-42C_n257E DC_1A-3A-18A-42C_n257F DC_1A-3A-18A-42C_n257G DC_1A-3A-18A-42C_n257H DC_1A-3A-18A-42C_n257I DC_1A-3A-18A-42C_n257J DC_1A-3A-18A-42C_n257K DC_1A-3A-18A-42C_n257L DC_1A-3A-18A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-19A-21A_n257A2 DC_1A-3A-19A-21A_n257D2 DC_1A-3A-19A-21A_n257E2 DC_1A-3A-19A-21A_n257F2 | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_21A_n257A |
| DC_1A-3A-19A-42A_n257A DC_1A-3A-19A-42A_n257D DC_1A-3A-19A-42A_n257E DC_1A-3A-19A-42A_n257F DC_1A-3A-19A-42A_n257G DC_1A-3A-19A-42A_n257H DC_1A-3A-19A-42A_n257I DC_1A-3A-19A-42A_n257J DC_1A-3A-19A-42A_n257K DC_1A-3A-19A-42A_n257L DC_1A-3A-19A-42A_n257M DC_1A-3A-19A-42C_n257A DC_1A-3A-19A-42C_n257D DC_1A-3A-19A-42C_n257E DC_1A-3A-19A-42C_n257F DC_1A-3A-19A-42C_n257G DC_1A-3A-19A-42C_n257H DC_1A-3A-19A-42C_n257I DC_1A-3A-19A-42C_n257J DC_1A-3A-19A-42C_n257K DC_1A-3A-19A-42C_n257L DC_1A-3A-19A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-3A-21A-42A_n257A DC_1A-3A-21A-42A_n257D DC_1A-3A-21A-42A_n257E DC_1A-3A-21A-42A_n257F DC_1A-3A-21A-42A_n257G DC_1A-3A-21A-42A_n257H DC_1A-3A-21A-42A_n257I DC_1A-3A-21A-42A_n257J DC_1A-3A-21A-42A_n257K DC_1A-3A-21A-42A_n257L DC_1A-3A-21A-42A_n257M DC_1A-3A-21A-42C_n257A DC_1A-3A-21A-42C_n257D DC_1A-3A-21A-42C_n257E DC_1A-3A-21A-42C_n257F DC_1A-3A-21A-42C_n257G DC_1A-3A-21A-42C_n257H DC_1A-3A-21A-42C_n257I DC_1A-3A-21A-42C_n257J DC_1A-3A-21A-42C_n257K DC_1A-3A-21A-42C_n257L DC_1A-3A-21A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-3A-28A-42A_n257A DC_1A-3A-28A-42A_n257G DC_1A-3A-28A-42A_n257H DC_1A-3A-28A-42A_n257I DC_1A-3A-28A-42A_n257J DC_1A-3A-28A-42A_n257K DC_1A-3A-28A-42A_n257L DC_1A-3A-28A-42A_n257M DC_1A-3A-28A-42C_n257A DC_1A-3A-28A-42C_n257G DC_1A-3A-28A-42C_n257H DC_1A-3A-28A-42C_n257I DC_1A-3A-28A-42C_n257J DC_1A-3A-28A-42C_n257K DC_1A-3A-28A-42C_n257L DC_1A-3A-28A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-41A-42A_n257A DC_1A-3A-41A-42A_n257D DC_1A-3A-41A-42A_n257E DC_1A-3A-41A-42A_n257F DC_1A-3A-41A-42A_n257G DC_1A-3A-41A-42A_n257H DC_1A-3A-41A-42A_n257I DC_1A-3A-41A-42A_n257J DC_1A-3A-41A-42A_n257K DC_1A-3A-41A-42A_n257L DC_1A-3A-41A-42A_n257M DC_1A-3A-41A-42C_n257A DC_1A-3A-41A-42C_n257D DC_1A-3A-41A-42C_n257E DC_1A-3A-41A-42C_n257F DC_1A-3A-41A-42C_n257G DC_1A-3A-41A-42C_n257H DC_1A-3A-41A-42C_n257I DC_1A-3A-41A-42C_n257J DC_1A-3A-41A-42C_n257K DC_1A-3A-41A-42C_n257L DC_1A-3A-41A-42C_n257M DC_1A-3A-41C-42A_n257A DC_1A-3A-41C-42A_n257D DC_1A-3A-41C-42A_n257E DC_1A-3A-41C-42A_n257F DC_1A-3A-41C-42A_n257G DC_1A-3A-41C-42A_n257H DC_1A-3A-41C-42A_n257I DC_1A-3A-41C-42A_n257J DC_1A-3A-41C-42A_n257K DC_1A-3A-41C-42A_n257L DC_1A-3A-41C-42A_n257M DC_1A-3A-41C-42C_n257A DC_1A-3A-41C-42C_n257D DC_1A-3A-41C-42C_n257E DC_1A-3A-41C-42C_n257F DC_1A-3A-41C-42C_n257G DC_1A-3A-41C-42C_n257H DC_1A-3A-41C-42C_n257I DC_1A-3A-41C-42C_n257J DC_1A-3A-41C-42C_n257K DC_1A-3A-41C-42C_n257L DC_1A-3A-41C-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-19A-21A-42A_n257A DC_1A-19A-21A-42A_n257D DC_1A-19A-21A-42A_n257E DC_1A-19A-21A-42A_n257F DC_1A-19A-21A-42A_n257G DC_1A-19A-21A-42A_n257H DC_1A-19A-21A-42A_n257I DC_1A-19A-21A-42A_n257J DC_1A-19A-21A-42A_n257K DC_1A-19A-21A-42A_n257L DC_1A-19A-21A-42A_n257M DC_1A-19A-21A-42C_n257A DC_1A-19A-21A-42C_n257D DC_1A-19A-21A-42C_n257E DC_1A-19A-21A-42C_n257F DC_1A-19A-21A-42C_n257G DC_1A-19A-21A-42C_n257H DC_1A-19A-21A-42C_n257I DC_1A-19A-21A-42C_n257J DC_1A-19A-21A-42C_n257K DC_1A-19A-21A-42C_n257L DC_1A-19A-21A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-19A-28A-42A_n257A DC_1A-19A-28A-42C_n257A | DC_1A_n257A DC_19A_n257A DC_28A_n257A DC_42A_n257A |
| DC_1A-21A-28A-42A_n257A | DC_1A_n257A DC_21A_n257A DC_28A_n257A DC_42A_n257A |
| DC_2A-5A-30A-66A_n260A DC_2A-5A-30A-66A_n260G DC_2A-5A-30A-66A_n260H DC_2A-5A-30A-66A_n260I DC_2A-5A-30A-66A_n260J DC_2A-5A-30A-66A_n260K DC_2A-5A-30A-66A_n260L DC_2A-5A-30A-66A_n260M | DC_2A_n260A DC_5A_n260A DC_30A_n260A DC_66A_n260A DC_2A_n260G DC_5A_n260G DC_30A_n260G DC_66A_n260G DC_2A_n260H DC_5A_n260H DC_30A_n260H DC_66A_n260H DC_2A_n260I DC_5A_n260I DC_30A_n260I DC_66A_n260I DC_2A_n260J DC_5A_n260J DC_30A_n260J DC_66A_n260J DC_2A_n260K DC_5A_n260K DC_30A_n260K DC_66A_n260K DC_2A_n260L DC_5A_n260L DC_30A_n260L DC_66A_n260L DC_2A_n260M DC_5A_n260M DC_30A_n260M DC_66A_n260M |
| DC_2A-2A-5A-30A-66A_n260A DC_2A-2A-5A-30A-66A_n260G DC_2A-2A-5A-30A-66A_n260H DC_2A-2A-5A-30A-66A_n260I DC_2A-2A-5A-30A-66A_n260J DC_2A-2A-5A-30A-66A_n260K DC_2A-2A-5A-30A-66A_n260L DC_2A-2A-5A-30A-66A_n260M | DC_2A_n260A DC_5A_n260A DC_30A_n260A DC_66A_n260A DC_2A_n260G DC_5A_n260G DC_30A_n260G DC_66A_n260G DC_2A_n260H DC_5A_n260H DC_30A_n260H DC_66A_n260H DC_2A_n260I DC_5A_n260I DC_30A_n260I DC_66A_n260I DC_2A_n260J DC_5A_n260J DC_30A_n260J DC_66A_n260J DC_2A_n260K DC_5A_n260K DC_30A_n260K DC_66A_n260K DC_2A_n260L DC_5A_n260L DC_30A_n260L DC_66A_n260L DC_2A_n260M DC_5A_n260M DC_30A_n260M DC_66A_n260M |
| DC_2A-5A-30A-66A-66A_n260A DC_2A-5A-30A-66A-66A_n260G DC_2A-5A-30A-66A-66A_n260H DC_2A-5A-30A-66A-66A_n260I DC_2A-5A-30A-66A-66A_n260J DC_2A-5A-30A-66A-66A_n260K DC_2A-5A-30A-66A-66A_n260L DC_2A-5A-30A-66A-66A_n260M | DC_2A_n260A DC_5A_n260A DC_30A_n260A DC_66A_n260A DC_2A_n260G DC_5A_n260G DC_30A_n260G DC_66A_n260G DC_2A_n260H DC_5A_n260H DC_30A_n260H DC_66A_n260H DC_2A_n260I DC_5A_n260I DC_30A_n260I DC_66A_n260I DC_2A_n260J DC_5A_n260J DC_30A_n260J DC_66A_n260J DC_2A_n260K DC_5A_n260K DC_30A_n260K DC_66A_n260K DC_2A_n260L DC_5A_n260L DC_30A_n260L DC_66A_n260L DC_2A_n260M DC_5A_n260M DC_30A_n260M DC_66A_n260M |
| DC_2A-12A-30A-66A_n260A DC_2A-12A-30A-66A_n260G DC_2A-12A-30A-66A_n260H DC_2A-12A-30A-66A_n260I DC_2A-12A-30A-66A_n260J DC_2A-12A-30A-66A_n260K DC_2A-12A-30A-66A_n260L DC_2A-12A-30A-66A_n260M | DC_2A_n260A DC_12A_n260A DC_30A_n260A DC_66A_n260A DC_2A_n260G DC_12A_n260G DC_30A_n260G DC_66A_n260G DC_2A_n260H DC_12A_n260H DC_30A_n260H DC_66A_n260H DC_2A_n260I DC_12A_n260I DC_30A_n260I DC_66A_n260I DC_2A_n260J DC_12A_n260J DC_30A_n260J DC_66A_n260J DC_2A_n260K DC_12A_n260K DC_30A_n260K DC_66A_n260K DC_2A_n260L DC_12A_n260L DC_30A_n260L DC_66A_n260L DC_2A_n260M DC_12A_n260M DC_30A_n260M DC_66A_n260M |
| DC_2A-2A-12A-30A-66A_n260A DC_2A-2A-12A-30A-66A_n260G DC_2A-2A-12A-30A-66A_n260H DC_2A-2A-12A-30A-66A_n260I DC_2A-2A-12A-30A-66A_n260J DC_2A-2A-12A-30A-66A_n260K DC_2A-2A-12A-30A-66A_n260L DC_2A-2A-12A-30A-66A_n260M | DC_2A_n260A DC_12A_n260A DC_30A_n260A DC_66A_n260A DC_2A_n260G DC_12A_n260G DC_30A_n260G DC_66A_n260G DC_2A_n260H DC_12A_n260H DC_30A_n260H DC_66A_n260H DC_2A_n260I DC_12A_n260I DC_30A_n260I DC_66A_n260I DC_2A_n260J DC_12A_n260J DC_30A_n260J DC_66A_n260J DC_2A_n260K DC_12A_n260K DC_30A_n260K DC_66A_n260K DC_2A_n260L DC_12A_n260L DC_30A_n260L DC_66A_n260L DC_2A_n260M DC_12A_n260M DC_30A_n260M DC_66A_n260M |
| DC_2A-12A-30A-66A-66A_n260A DC_2A-12A-30A-66A-66A_n260G DC_2A-12A-30A-66A-66A_n260H DC_2A-12A-30A-66A-66A_n260I DC_2A-12A-30A-66A-66A_n260J DC_2A-12A-30A-66A-66A_n260K DC_2A-12A-30A-66A-66A_n260L DC_2A-12A-30A-66A-66A_n260M | DC_2A_n260A DC_12A_n260A DC_30A_n260A DC_66A_n260A DC_2A_n260G DC_12A_n260G DC_30A_n260G DC_66A_n260G DC_2A_n260H DC_12A_n260H DC_30A_n260H DC_66A_n260H DC_2A_n260I DC_12A_n260I DC_30A_n260I DC_66A_n260I DC_2A_n260J DC_12A_n260J DC_30A_n260J DC_66A_n260J DC_2A_n260K DC_12A_n260K DC_30A_n260K DC_66A_n260K DC_2A_n260L DC_12A_n260L DC_30A_n260L DC_66A_n260L DC_2A_n260M DC_12A_n260M DC_30A_n260M DC_66A_n260M |
| DC_2A-14A-30A-66A_n260A DC_2A-14A-30A-66A_n260G DC_2A-14A-30A-66A_n260H DC_2A-14A-30A-66A_n260I DC_2A-14A-30A-66A_n260J DC_2A-14A-30A-66A_n260K DC_2A-14A-30A-66A_n260L DC_2A-14A-30A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-2A-14A-30A-66A_n260A DC_2A-2A-14A-30A-66A_n260G DC_2A-2A-14A-30A-66A_n260H DC_2A-2A-14A-30A-66A_n260I DC_2A-2A-14A-30A-66A_n260J DC_2A-2A-14A-30A-66A_n260K DC_2A-2A-14A-30A-66A_n260L DC_2A-2A-14A-30A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-14A-30A-66A-66A_n260A DC_2A-14A-30A-66A-66A_n260G DC_2A-14A-30A-66A-66A_n260H DC_2A-14A-30A-66A-66A_n260I DC_2A-14A-30A-66A-66A_n260J DC_2A-14A-30A-66A-66A_n260K DC_2A-14A-30A-66A-66A_n260L DC_2A-14A-30A-66A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-29A-30A-66A_n260A DC_2A-29A-30A-66A_n260G DC_2A-29A-30A-66A_n260H DC_2A-29A-30A-66A_n260I DC_2A-29A-30A-66A_n260J DC_2A-29A-30A-66A_n260K DC_2A-29A-30A-66A_n260L DC_2A-29A-30A-66A_n260M | DC_2A_n260A DC_30A_n260A DC_66A_n260A DC_2A_n260G DC_30A_n260G DC_66A_n260G DC_2A_n260H DC_30A_n260H DC_66A_n260H DC_2A_n260I DC_30A_n260I DC_66A_n260I DC_2A_n260J DC_30A_n260J DC_66A_n260J DC_2A_n260K DC_30A_n260K DC_66A_n260K DC_2A_n260L DC_30A_n260L DC_66A_n260L DC_2A_n260M DC_30A_n260M DC_66A_n260M |
| DC_(n)3AA-n8A-n77A-n257A DC_(n)3AA-n8A-n77A-n257G DC_(n)3AA-n8A-n77A-n257H DC_(n)3AA-n8A-n77A-n257I DC_(n)3AA-n8A-n77A-n257J DC_(n)3AA-n8A-n77A-n257K DC_(n)3AA-n8A-n77A-n257L DC_(n)3AA-n8A-n77A-n257M DC_(n)3AA-n8A-n77(2A)-n257A DC_(n)3AA-n8A-n77(2A)-n257G DC_(n)3AA-n8A-n77(2A)-n257H DC_(n)3AA-n8A-n77(2A)-n257I DC_(n)3AA-n8A-n77(2A)-n257J DC_(n)3AA-n8A-n77(2A)-n257K DC_(n)3AA-n8A-n77(2A)-n257L DC_(n)3AA-n8A-n77(2A)-n257M | DC_(n)3AA3 DC_3A_n8A DC_3A_n77A DC_3A_n257A |
| DC_3A-19A-21A-42A_n257A DC_3A-19A-21A-42A_n257D DC_3A-19A-21A-42A_n257E DC_3A-19A-21A-42A_n257F DC_3A-19A-21A-42C_n257A DC_3A-19A-21A-42C_n257D DC_3A-19A-21A-42C_n257E DC_3A-19A-21A-42C_n257F | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D |
| DC_3A-28A-41A-42A_n257A DC_3A-28A-41A-42A_n257G DC_3A-28A-41A-42A_n257H DC_3A-28A-41A-42A_n257I DC_3A-28A-41A-42C_n257A DC_3A-28A-41A-42C_n257G DC_3A-28A-41A-42C_n257H DC_3A-28A-41A-42C_n257I DC_3A-28A-41C-42A_n257A DC_3A-28A-41C-42A_n257G DC_3A-28A-41C-42A_n257H DC_3A-28A-41C-42A_n257I DC_3A-28A-41C-42C_n257A DC_3A-28A-41C-42C_n257G DC_3A-28A-41C-42C_n257H DC_3A-28A-41C-42C_n257I | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations. NOTE 3: Only single switched UL is supported. |  |

#### 5.5B.5.5	Void

### 5.5B.5a	Inter-band NE-DC including FR2

#### 5.5B.5a.1	Inter-band NE-DC configurations including FR2 (two bands)

Table 5.5B.5a.1-1: Inter-band NE-DC configurations including FR2 (two bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n257A_1A DC_n257G_1A DC_n257H_1A DC_n257I_1A DC_n257J_1A DC_n257K_1A DC_n257L_1A DC_n257M_1A | DC_n257A_1A DC_n257G_1A DC_n257H_1A DC_n257I_1A DC_n257J_1A DC_n257K_1A DC_n257L_1A DC_n257M_1A |
| DC_n257A_3A DC_n257G_3A DC_n257H_3A DC_n257I_3A DC_n257J_3A DC_n257K_3A DC_n257L_3A DC_n257M_3A DC_n257A_3C DC_n257G_3C DC_n257H_3C DC_n257I_3C DC_n257J_3C DC_n257K_3C DC_n257L_3C DC_n257M_3C | DC_n257A_3A DC_n257G_3A DC_n257H_3A DC_n257I_3A DC_n257J_3A DC_n257K_3A DC_n257L_3A DC_n257M_3A DC_n257A_3C DC_n257G_3C DC_n257H_3C DC_n257I_3C DC_n257J_3C DC_n257K_3C DC_n257L_3C DC_n257M_3C |
| DC_n257A_5A DC_n257G_5A DC_n257H_5A DC_n257I_5A DC_n257J_5A DC_n257K_5A DC_n257L_5A DC_n257M_5A | DC_n257A_5A |
| DC_n257A_7A DC_n257G_7A DC_n257H_7A DC_n257I_7A DC_n257J_7A DC_n257K_7A DC_n257L_7A DC_n257M_7A | DC_n257A_7A |
| DC_n257A_7A-7A DC_n257G_7A-7A DC_n257H_7A-7A DC_n257I_7A-7A DC_n257J_7A-7A DC_n257K_7A-7A DC_n257L_7A-7A DC_n257M_7A-7A | DC_n257A_7A |
| DC_n257A_8A DC_n257G_8A DC_n257H_8A DC_n257I_8A DC_n257J_8A DC_n257K_8A DC_n257L_8A DC_n257M_8A | DC_n257A_8A |
| DC_n257A_26A | DC_n257A_26A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band NE-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations |  |

#### 5.5B.5a.2	Inter-band NE-DC configurations including FR2 (three bands)

Table 5.5B.5a.2-1: Inter-band NE-DC configurations including FR2 (three bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n257A_1A-3A DC_n257G_1A-3A DC_n257H_1A-3A DC_n257I_1A-3A DC_n257J_1A-3A DC_n257K_1A-3A DC_n257L_1A-3A DC_n257M_1A-3A DC_n257A_1A-3C DC_n257G_1A-3C DC_n257H_1A-3C DC_n257I_1A-3C DC_n257J_1A-3C DC_n257K_1A-3C DC_n257L_1A-3C DC_n257M_1A-3C | DC_n257A_1A DC_n257A_3A DC_n257G_3A DC_n257H_3A DC_n257I_3A DC_n257J_3A DC_n257K_3A DC_n257L_3A DC_n257M_3A |
| DC_n257A_1A-5A DC_n257G_1A-5A DC_n257H_1A-5A DC_n257I_1A-5A DC_n257J_1A-5A DC_n257K_1A-5A DC_n257L_1A-5A DC_n257M_1A-5A | DC_n257A_1A DC_n257A_5A |
| DC_n257A_1A-7A DC_n257G_1A-7A DC_n257H_1A-7A DC_n257I_1A-7A DC_n257J_1A-7A DC_n257K_1A-7A DC_n257L_1A-7A DC_n257M_1A-7A | DC_n257A_1A DC_n257A_7A |
| DC_n257A_1A-7A-7A DC_n257G_1A-7A-7A DC_n257H_1A-7A-7A DC_n257I_1A-7A-7A DC_n257J_1A-7A-7A DC_n257K_1A-7A-7A DC_n257L_1A-7A-7A DC_n257M_1A-7A-7A | DC_n257A_1A DC_n257A_7A |
| DC_n257A_1A-8A DC_n257G_1A-8A DC_n257H_1A-8A DC_n257I_1A-8A DC_n257J_1A-8A DC_n257K_1A-8A DC_n257L_1A-8A DC_n257M_1A-8A | DC_n257A_1A DC_n257A_8A |
| DC_n257A_3A-5A DC_n257G_3A-5A DC_n257H_3A-5A DC_n257I_3A-5A DC_n257J_3A-5A DC_n257K_3A-5A DC_n257L_3A-5A DC_n257M_3A-5A | DC_n257A_3A DC_n257A_5A |
| DC_n257A_3A-7A DC_n257G_3A-7A DC_n257H_3A-7A DC_n257I_3A-7A DC_n257J_3A-7A DC_n257K_3A-7A DC_n257L_3A-7A DC_n257M_3A-7A | DC_n257A_3A DC_n257A_7A |
| DC_n257A_3A-7A-7A DC_n257G_3A-7A-7A DC_n257H_3A-7A-7A DC_n257I_3A-7A-7A DC_n257J_3A-7A-7A DC_n257K_3A-7A-7A DC_n257L_3A-7A-7A DC_n257M_3A-7A-7A | DC_n257A_3A DC_n257A_7A |
| DC_n257A_3A-8A DC_n257G_3A-8A DC_n257H_3A-8A DC_n257I_3A-8A DC_n257J_3A-8A DC_n257K_3A-8A DC_n257L_3A-8A DC_n257M_3A-8A DC_n257A_3C-8A DC_n257G_3C-8A DC_n257H_3C-8A DC_n257I_3C-8A DC_n257J_3C-8A DC_n257K_3C-8A DC_n257L_3C-8A DC_n257M_3C-8A | DC_n257A_3A DC_n257A_8A |
| DC_n257A_5A-7A DC_n257G_5A-7A DC_n257H_5A-7A DC_n257I_5A-7A DC_n257J_5A-7A DC_n257K_5A-7A DC_n257L_5A-7A DC_n257M_5A-7A | DC_n257A_5A DC_n257A_7A |
| DC_n257A_5A-7A-7A DC_n257G_5A-7A-7A DC_n257H_5A-7A-7A DC_n257I_5A-7A-7A DC_n257J_5A-7A-7A DC_n257K_5A-7A-7A DC_n257L_5A-7A-7A DC_n257M_5A-7A-7A | DC_n257A_5A DC_n257A_7A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the presNEt release of specifications. NOTE 2: Applicable for UE supporting inter-band NE-DC with mandatory simultaneous Rx/Tx capability |  |

#### 5.5B.5a.3	Inter-band NE-DC configurations including FR2 (four bands)

Table 5.5B.5a.3-1: Inter-band NE-DC configurations including FR2 (four bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n257A_1A-3A-5A DC_n257G_1A-3A-5A DC_n257H_1A-3A-5A DC_n257I_1A-3A-5A DC_n257J_1A-3A-5A DC_n257K_1A-3A-5A DC_n257L_1A-3A-5A DC_n257M_1A-3A-5A | DC_n257A_1A DC_n257A_3A DC_n257A_5A |
| DC_n257A_1A-3A-7A DC_n257G_1A-3A-7A DC_n257H_1A-3A-7A DC_n257I_1A-3A-7A DC_n257J_1A-3A-7A DC_n257K_1A-3A-7A DC_n257L_1A-3A-7A DC_n257M_1A-3A-7A | DC_n257A_1A DC_n257A_3A DC_n257A_7A |
| DC_n257A_1A-3A-7A-7A DC_n257G_1A-3A-7A-7A DC_n257H_1A-3A-7A-7A DC_n257I_1A-3A-7A-7A DC_n257J_1A-3A-7A-7A DC_n257K_1A-3A-7A-7A DC_n257L_1A-3A-7A-7A DC_n257M_1A-3A-7A-7A | DC_n257A_1A DC_n257A_3A DC_n257A_7A |
| DC_n257A_1A-3A-8A DC_n257G_1A-3A-8A DC_n257H_1A-3A-8A DC_n257I_1A-3A-8A DC_n257J_1A-3A-8A DC_n257K_1A-3A-8A DC_n257L_1A-3A-8A DC_n257M_1A-3A-8A DC_n257A_1A-3C-8A DC_n257G_1A-3C-8A DC_n257H_1A-3C-8A DC_n257I_1A-3C-8A DC_n257J_1A-3C-8A DC_n257K_1A-3C-8A DC_n257L_1A-3C-8A DC_n257M_1A-3C-8A | DC_n257A_1A DC_n257A_3A DC_n257A_8A |
| DC_n257A_1A-5A-7A DC_n257G_1A-5A-7A DC_n257H_1A-5A-7A DC_n257I_1A-5A-7A DC_n257J_1A-5A-7A DC_n257K_1A-5A-7A DC_n257L_1A-5A-7A DC_n257M_1A-5A-7A | DC_n257A_1A DC_n257A_5A DC_n257A_7A |
| DC_n257A_1A-5A-7A-7A DC_n257G_1A-5A-7A-7A DC_n257H_1A-5A-7A-7A DC_n257I_1A-5A-7A-7A DC_n257J_1A-5A-7A-7A DC_n257K_1A-5A-7A-7A DC_n257L_1A-5A-7A-7A DC_n257M_1A-5A-7A-7A | DC_n257A_1A DC_n257A_5A DC_n257A_7A |
| DC_n257A_3A-5A-7A DC_n257G_3A-5A-7A DC_n257H_3A-5A-7A DC_n257I_3A-5A-7A DC_n257J_3A-5A-7A DC_n257K_3A-5A-7A DC_n257L_3A-5A-7A DC_n257M_3A-5A-7A | DC_n257A_3A DC_n257A_5A DC_n257A_7A |
| DC_n257A_3A-5A-7A-7A DC_n257G_3A-5A-7A-7A DC_n257H_3A-5A-7A-7A DC_n257I_3A-5A-7A-7A DC_n257J_3A-5A-7A-7A DC_n257K_3A-5A-7A-7A DC_n257L_3A-5A-7A-7A DC_n257M_3A-5A-7A-7A | DC_n257A_3A DC_n257A_5A DC_n257A_7A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the presNEt release of specifications. NOTE 2: Applicable for UE supporting inter-band NE-DC with mandatory simultaneous Rx/Tx capability |  |

#### 5.5B.5a.4	Inter-band NE-DC configurations including FR2 (five bands)

Table 5.5B.5a.4-1: Inter-band NE-DC configurations including FR2 (five bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n257A_1A-3A-5A-7A DC_n257G_1A-3A-5A-7A DC_n257H_1A-3A-5A-7A DC_n257I_1A-3A-5A-7A DC_n257J_1A-3A-5A-7A DC_n257K_1A-3A-5A-7A DC_n257L_1A-3A-5A-7A DC_n257M_1A-3A-5A-7A | DC_n257A_1A DC_n257A_3A DC_n257A_5A DC_n257A_7A |
| DC_n257A_1A-3A-5A-7A-7A DC_n257G_1A-3A-5A-7A-7A DC_n257H_1A-3A-5A-7A-7A DC_n257I_1A-3A-5A-7A-7A DC_n257J_1A-3A-5A-7A-7A DC_n257K_1A-3A-5A-7A-7A DC_n257L_1A-3A-5A-7A-7A DC_n257M_1A-3A-5A-7A-7A | DC_n257A_1A DC_n257A_3A DC_n257A_5A DC_n257A_7A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band NE-DC with mandatory simultaneous Rx/Tx capability |  |

### 5.5B.6	Inter-band EN-DC including FR1 and FR2

#### 5.5B.6.1	Void

#### 5.5B.6.2	Inter-band EN-DC configurations including FR1 and FR2 (three bands)

Table 5.5B.6.2-1: Inter-band EN-DC configurations including FR1 and FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n3A-n257A2 DC_1A_n3A-n257G2 DC_1A_n3A-n257H2 DC_1A_n3A-n257I2 | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n8A-n257A DC_1A_n8A-n257G DC_1A_n8A-n257H DC_1A_n8A-n257I DC_1A_n8A-n257J DC_1A_n8A-n257K DC_1A_n8A-n257L DC_1A_n8A-n257M | DC_1A_n8A DC_1A_n257A |
| DC_1A_n28A-n257A2 DC_1A_n28A-n257G2 DC_1A_n28A-n257H2 DC_1A_n28A-n257I2 | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n38A-n257A DC_1A_n38A-n257G DC_1A_n38A-n257H DC_1A_n38A-n257I | DC_1A_n38A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n40A-n258A DC_1A_n40A-n258D DC_1A_n40A-n258G DC_1A_n40A-n258E DC_1A_n40A-n258F DC_1A_n40A-n258H DC_1A_n40A-n258I DC_1A_n40A-n258J DC_1A_n40A-n258K DC_1A_n40A-n258L DC_1A_n40A-n258M | DC_1A_n40A DC_1A_n258A |
| DC_1A_n77A-n257J DC_1A_n77A-n257K DC_1A_n77A-n257L DC_1A_n77A-n257M DC_1A_n77A-n257A | DC_1A_n77A DC_1A_n257A |
| DC_1A_n77A-n257A2 DC_1A_n77A-n257D2 DC_1A_n77A-n257E2 DC_1A_n77A-n257F2 DC_1A_n77A-n257G2 DC_1A_n77A-n257H2 DC_1A_n77A-n257I2 DC_1A_n77C-n257A2 DC_1A_n77C-n257D2 DC_1A_n77C-n257E2 DC_1A_n77C-n257F2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I |
| DC_1A_n77(2A)-n257A2 DC_1A_n77(2A)-n257D2 DC_1A_n77(2A)-n257G2 DC_1A_n77(2A)-n257H2 DC_1A_n77(2A)-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n77A-n258A | DC_1A_n77A DC_1A_n258A |
| DC_1A_n78A-n257A2 DC_1A_n78A-n257D2 DC_1A_n78A-n257E2 DC_1A_n78A-n257F2 DC_1A_n78C-n257A2 DC_1A_n78C-n257D2 DC_1A_n78C-n257E2 DC_1A_n78C-n257F2 | DC_1A_n78A DC_1A_n257A DC_1A_n257D DC_1A_n78A-n257A |
| DC_1A_n78A-n257G2 DC_1A_n78A-n257H2 DC_1A_n78A-n257I2 DC_1A_n78A-n257J2 DC_1A_n78A-n257K2 DC_1A_n78A-n257L2 DC_1A_n78A-n257M2 DC_1A_n78C-n257G DC_1A_n78C-n257H DC_1A_n78C-n257I DC_1A_n78C-n257J DC_1A_n78C-n257K DC_1A_n78C-n257L DC_1A_n78C-n257M | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I |
| DC_1A_n78A-n258A DC_1A_n78A-n258D DC_1A_n78A-n258E DC_1A_n78A-n258F DC_1A_n78A-n258G DC_1A_n78A-n258H DC_1A_n78A-n258I DC_1A_n78A-n258J DC_1A_n78A-n258K DC_1A_n78A-n258L DC_1A_n78A-n258M | DC_1A_n78A DC_1A_n258A |
| DC_1A_n79A-n257A2 DC_1A_n79A-n257D2 DC_1A_n79A-n257E2 DC_1A_n79A-n257F2 DC_1A_n79A-n257G2 DC_1A_n79A-n257H2 DC_1A_n79A-n257I2 DC_1A_n79C-n257A2 DC_1A_n79C-n257D2 DC_1A_n79C-n257E2 DC_1A_n79C-n257F2 | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I |
| DC_1A_n79A-n258A | DC_1A_n79A DC_1A_n258A |
| DC_1A_n105A-n257A | DC_1A_n105A DC_1A_n257A |
| DC_1A_n105A-n258A | DC_1A_n105A DC_1A_n258A |
| DC_2A_n7A-n258A DC_2A_n7A-n258G DC_2A_n7A-n258H DC_2A_n7A-n258I | DC_2A_n7A DC_2A_n258A DC_2A_n258G DC_2A_n258H DC_2A_n258I |
| DC_2A_n12A-n258A | DC_2A_n258A DC_2A_n12A |
| DC_2A_n12A-n260A | DC_2A_n260A DC_2A_n12A |
| DC_2A_n12A-n261A | DC_2A_n261A DC_2A_n12A |
| DC_2A_n41A-n260A | DC_2A_n41A |
| DC_2A_n41A-n260(2A) DC_2A_n41A-n260(3A) DC_2A_n41A-n260(4A) | DC_2A_n41A |
| DC_2A_n41A-n261A | DC_2A_n41A |
| DC_2A_n41A-n261(2A) | DC_2A_n41A |
| DC_2A_n71A-n261A | DC_2A_n261A DC_2A_n71A |
| DC_2A_n71A-n261(2A) | DC_2A_n261A DC_2A_n71A |
| DC_3A_n1A-n257A2 DC_3A_n1A-n257D2 DC_3A_n1A-n257E2 DC_3A_n1A-n257F2 DC_3A_n1A-n257G2 DC_3A_n1A-n257H2 DC_3A_n1A-n257I2 DC_3A_n1A-n257J2 DC_3A_n1A-n257K2 DC_3A_n1A-n257L2 DC_3A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_3A-3A_n1A-n257A2 DC_3A-3A_n1A-n257D2 DC_3A-3A_n1A-n257E2 DC_3A-3A_n1A-n257F2 DC_3A-3A_n1A-n257G2 DC_3A-3A_n1A-n257H2 DC_3A-3A_n1A-n257I2 DC_3A-3A_n1A-n257J2 DC_3A-3A_n1A-n257K2 DC_3A-3A_n1A-n257L2 DC_3A-3A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_3A_n8A-n257A DC_3A_n8A-n257G DC_3A_n8A-n257H DC_3A_n8A-n257I DC_3A_n8A-n257J DC_3A_n8A-n257K DC_3A_n8A-n257L DC_3A_n8A-n257M | DC_3A_n8A DC_3A_n257A |
| DC_3A_n28A-n257A2 DC_3A_n28A-n257G2 DC_3A_n28A-n257H2 DC_3A_n28A-n257I2 | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_3A_n34A-n258A | DC_3A_n34A DC_3A_n258A DC_3A_n34A-n258A |
| DC_3A_n38A-n257A DC_3A_n38A-n257G DC_3A_n38A-n257H DC_3A_n38A-n257I | DC_3A_n38A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_3A_n40A-n258A DC_3A_n40A-n258D DC_3A_n40A-n258E DC_3A_n40A-n258F DC_3A_n40A-n258G DC_3A_n40A-n258H DC_3A_n40A-n258I DC_3A_n40A-n258J DC_3A_n40A-n258K DC_3A_n40A-n258L DC_3A_n40A-n258M | DC_3A_n40A DC_3A_n258A DC_3A_n40A-n258A |
| DC_3A_n41A-n258A | DC_3A_n41A-n258A |
| DC_3A_n77A-n257A2 DC_3A_n77A-n257D2 DC_3A_n77A-n257E2 DC_3A_n77A-n257F2 DC_3A_n77A-n257G2 DC_3A_n77A-n257H2 DC_3A_n77A-n257I2 DC_3A_n77C-n257A2 DC_3A_n77C-n257D2 DC_3A_n77C-n257E2 DC_3A_n77C-n257F2 | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I |
| DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_3A_n77A-n257J DC_3A_n77A-n257K DC_3A_n77A-n257L DC_3A_n77A-n257M DC_3A_n77(2A)-n257A DC_3A_n77(2A)-n257G DC_3A_n77(2A)-n257H DC_3A_n77(2A)-n257I DC_3A_n77(2A)-n257J DC_3A_n77(2A)-n257K DC_3A_n77(2A)-n257L DC_3A_n77(2A)-n257M | DC_3A_n77A DC_3A_n257A |
| DC_3A_n77(2A)-n257A2 DC_3A_n77(2A)-n257D2 DC_3A_n77(2A)-n257G2 DC_3A_n77(2A)-n257H2 DC_3A_n77(2A)-n257I2 | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_3A_n77A-n258A | DC_3A_n77A DC_3A_n258A DC_3A_n77A-n258A |
| DC_3A_n78A-n257A2 DC_3A_n78A-n257D2 DC_3A_n78A-n257E2 DC_3A_n78A-n257F2 DC_3A_n78A-n257G2 DC_3A_n78A-n257H2 DC_3A_n78A-n257I2 DC_3A_n78A-n257J2 DC_3A_n78A-n257K2 DC_3A_n78A-n257L2 DC_3A_n78A-n257M2 DC_3A_n78C-n257A2 DC_3A_n78C-n257D2 DC_3A_n78C-n257E2 DC_3A_n78C-n257F2 DC_3A_n78C-n257G DC_3A_n78C-n257H DC_3A_n78C-n257I DC_3A_n78C-n257J DC_3A_n78C-n257K DC_3A_n78C-n257L DC_3A_n78C-n257M | DC_3A_n78A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_3C_n78A-n257A2 DC_3C_n78A-n257D2 DC_3C_n78A-n257E2 DC_3C_n78A-n257F2 DC_3C_n78A-n257G2 DC_3C_n78A-n257H2 DC_3C_n78A-n257I2 DC_3C_n78A-n257J2 DC_3C_n78A-n257K2 DC_3C_n78A-n257L2 DC_3C_n78A-n257M2 | DC_3A_n78A DC_3A_n257A |
| DC_3A_n78A-n258A DC_3A_n78A-n258D DC_3A_n78A-n258E DC_3A_n78A-n258F DC_3A_n78A-n258G DC_3A_n78A-n258H DC_3A_n78A-n258I DC_3A_n78A-n258J DC_3A_n78A-n258K DC_3A_n78A-n258L DC_3A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K |
| DC_3A-3A_n78A-n257A2 DC_3A-3A_n78A-n257D2 DC_3A-3A_n78A-n257E2 DC_3A-3A_n78A-n257F2 DC_3A-3A_n78A-n257G2 DC_3A-3A_n78A-n257H2 DC_3A-3A_n78A-n257I2 DC_3A-3A_n78A-n257J2 DC_3A-3A_n78A-n257K2 DC_3A-3A_n78A-n257L2 DC_3A-3A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_3A_n79A-n257A2 DC_3A_n79A-n257D2 DC_3A_n79A-n257E2 DC_3A_n79A-n257F2 DC_3A_n79A-n257G2 DC_3A_n79A-n257H2 DC_3A_n79A-n257I2 DC_3A_n79C-n257A2 DC_3A_n79C-n257D2 DC_3A_n79C-n257E2 DC_3A_n79C-n257F2 | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I |
| DC_3A_n79A-n258A DC_3A_n79A-n258D DC_3A_n79A-n258E DC_3A_n79A-n258F DC_3A_n79A-n258G DC_3A_n79A-n258H DC_3A_n79A-n258I DC_3A_n79A-n258J DC_3A_n79A-n258K DC_3A_n79A-n258L | DC_3A_n79A DC_3A_n258A DC_3A_n79A-n258A |
| DC_3A_n105A-n257A | DC_3A_n105A DC_3A_n257A |
| DC_3A_n105A-n258A | DC_3A_n105A DC_3A_n258A |
| DC_5A_n78A-n257A2 DC_5A_n78A-n257D DC_5A_n78A-n257E DC_5A_n78A-n257F DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_5A_n78A-n257J DC_5A_n78A-n257K DC_5A_n78A-n257L DC_5A_n78A-n257M DC_5A_n78C-n257A DC_5A_n78C-n257D DC_5A_n78C-n257E DC_5A_n78C-n257F DC_5A_n78C-n257G DC_5A_n78C-n257H DC_5A_n78C-n257I DC_5A_n78C-n257J DC_5A_n78C-n257K DC_5A_n78C-n257L DC_5A_n78C-n257M | DC_5A_n78A DC_5A_n257A |
| DC_7A_n1A-n257A2 DC_7A_n1A-n257D2 DC_7A_n1A-n257E2 DC_7A_n1A-n257F2 DC_7A_n1A-n257G2 DC_7A_n1A-n257H2 DC_7A_n1A-n257I2 DC_7A_n1A-n257J2 DC_7A_n1A-n257K2 DC_7A_n1A-n257L2 DC_7A_n1A-n257M2 | DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A-7A_n1A-n257A2 DC_7A-7A_n1A-n257D2 DC_7A-7A_n1A-n257E2 DC_7A-7A_n1A-n257F2 DC_7A-7A_n1A-n257G2 DC_7A-7A_n1A-n257H2 DC_7A-7A_n1A-n257I2 DC_7A-7A_n1A-n257J2 DC_7A-7A_n1A-n257K2 DC_7A-7A_n1A-n257L2 DC_7A-7A_n1A-n257M2 | DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A_n40A-n258A DC_7A_n40A-n258D DC_7A_n40A-n258G DC_7A_n40A-n258E DC_7A_n40A-n258F DC_7A_n40A-n258H DC_7A_n40A-n258I DC_7A_n40A-n258J DC_7A_n40A-n258K DC_7A_n40A-n258L DC_7A_n40A-n258M | DC_7A_n40A DC_7A_n258A |
| DC_7A_n78A-n257A2 DC_7A_n78A-n257D2 DC_7A_n78A-n257E2 DC_7A_n78A-n257F2 DC_7A_n78A-n257G2 DC_7A_n78A-n257H2 DC_7A_n78A-n257I2 DC_7A_n78A-n257J2 DC_7A_n78A-n257K2 DC_7A_n78A-n257L2 DC_7A_n78A-n257M2 DC_7A_n78C-n257A DC_7A_n78C-n257D DC_7A_n78C-n257E DC_7A_n78C-n257F DC_7A_n78C-n257G DC_7A_n78C-n257H DC_7A_n78C-n257I DC_7A_n78C-n257J DC_7A_n78C-n257K DC_7A_n78C-n257L DC_7A_n78C-n257M | DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A-7A_n78A-n257A2 DC_7A-7A_n78A-n257D2 DC_7A-7A_n78A-n257E2 DC_7A-7A_n78A-n257F2 DC_7A-7A_n78A-n257G2 DC_7A-7A_n78A-n257H2 DC_7A-7A_n78A-n257I2 DC_7A-7A_n78A-n257J2 DC_7A-7A_n78A-n257K2 DC_7A-7A_n78A-n257L2 DC_7A-7A_n78A-n257M2 DC_7A-7A_n78C-n257A DC_7A-7A_n78C-n257D DC_7A-7A_n78C-n257E DC_7A-7A_n78C-n257F DC_7A-7A_n78C-n257G DC_7A-7A_n78C-n257H DC_7A-7A_n78C-n257I DC_7A-7A_n78C-n257J DC_7A-7A_n78C-n257K DC_7A-7A_n78C-n257L DC_7A-7A_n78C-n257M | DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_7A_n78A-n257A |
| DC_7A_n78A-n258A DC_7A_n78A-n258D DC_7A_n78A-n258E DC_7A_n78A-n258F DC_7A_n78A-n258G DC_7A_n78A-n258H DC_7A_n78A-n258I DC_7A_n78A-n258J DC_7A_n78A-n258K DC_7A_n78A-n258L DC_7A_n78A-n258M DC_7C_n78A-n258A DC_7C_n78A-n258G DC_7C_n78A-n258H DC_7C_n78A-n258I DC_7C_n78A-n258J DC_7C_n78A-n258K DC_7C_n78A-n258L DC_7C_n78A-n258M | DC_7A_n78A DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n78A DC_7C_n258A DC_7C_n258G DC_7C_n258H DC_7C_n258I |
| DC_7A_n79A-n257A DC_7A_n79A-n257G DC_7A_n79A-n257H DC_7A_n79A-n257I DC_7A_n79A-n257J DC_7A_n79A-n257K DC_7A_n79A-n257L DC_7A_n79A-n257M | DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_7A_n257L DC_7A_n257M DC_7A_n79A |
| DC_7A_n79A-n258A DC_7A_n79A-n258G DC_7A_n79A-n258H DC_7A_n79A-n258I DC_7A_n79A-n258J DC_7A_n79A-n258K DC_7A_n79A-n258L DC_7A_n79A-n258M | DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M DC_7A_n79A |
| DC_8A_n1A-n257A2 DC_8A_n1A-n257D2 DC_8A_n1A-n257E2 DC_8A_n1A-n257F2 DC_8A_n1A-n257G2 DC_8A_n1A-n257H2 DC_8A_n1A-n257I2 DC_8A_n1A-n257J2 DC_8A_n1A-n257K2 DC_8A_n1A-n257L2 DC_8A_n1A-n257M2 | DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_8A_n3A-n257A DC_8A_n3A-n257G DC_8A_n3A-n257H DC_8A_n3A-n257I DC_8A_n3A-n257J DC_8A_n3A-n257K DC_8A_n3A-n257L DC_8A_n3A-n257M | DC_8A_n3A DC_8A_n257A |
| DC_8A_n34A-n258A | DC_8A_n34A DC_8A_n258A |
| DC_8A_n39A-n258A | DC_8A_n39A DC_8A_n258A |
| DC_8A_n40A-n258A DC_8A_n40A-n258D DC_8A_n40A-n258E DC_8A_n40A-n258F DC_8A_n40A-n258G DC_8A_n40A-n258H DC_8A_n40A-n258I DC_8A_n40A-n258J DC_8A_n40A-n258K DC_8A_n40A-n258L DC_8A_n40A-n258M | DC_8A_n40A DC_8A_n258A DC_8A_n40A-n258A |
| DC_8A_n41A-n258A | DC_8A_n41A DC_8A_n258A |
| DC_8A_n77A-n257A2 DC_8A_n77A-n257D2 DC_8A_n77A-n257G2 DC_8A_n77A-n257H2 DC_8A_n77A-n257I2 | DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_8A_n77(2A)-n257A2 DC_8A_n77(2A)-n257D2 DC_8A_n77(2A)-n257G2 DC_8A_n77(2A)-n257H2 DC_8A_n77(2A)-n257I2 | DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_8A_n78A-n257A2 DC_8A_n78A-n257D2 DC_8A_n78A-n257E2 DC_8A_n78A-n257F2 DC_8A_n78A-n257G2 DC_8A_n78A-n257H2 DC_8A_n78A-n257I2 DC_8A_n78A-n257J2 DC_8A_n78A-n257K2 DC_8A_n78A-n257L2 DC_8A_n78A-n257M2 | DC_8A_n78A DC_8A_n257A |
| DC_8A_n78A-n258A DC_8A_n78A-n258D DC_8A_n78A-n258E DC_8A_n78A-n258F DC_8A_n78A-n258G DC_8A_n78A-n258H DC_8A_n78A-n258I DC_8A_n78A-n258J DC_8A_n78A-n258K DC_8A_n78A-n258L DC_8A_n78A-n258M | DC_8A_n78A DC_8A_n258A |
| DC_8A_n79A-n258A | DC_8A_n79A DC_8A_n258A DC_8A_n79A-n258A |
| DC_11A_n77A-n257A2 DC_11A_n77A-n257D2 DC_11A_n77A-n257G2 DC_11A_n77A-n257H2 DC_11A_n77A-n257I2 | DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_11A_n77(2A)-n257A2 DC_11A_n77(2A)-n257D2 DC_11A_n77(2A)-n257G2 DC_11A_n77(2A)-n257H2 DC_11A_n77(2A)-n257I2 | DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_18A_n3A-n257A DC_18A_n3A-n257G DC_18A_n3A-n257H DC_18A_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_18A_n78A-n257A DC_18A_n78A-n257G DC_18A_n78A-n257H DC_18A_n78A-n257I | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_19A_n77A-n257A2 DC_19A_n77A-n257D2 DC_19A_n77A-n257E2 DC_19A_n77A-n257F2 DC_19A_n77A-n257G2 DC_19A_n77A-n257H2 DC_19A_n77A-n257I2 DC_19A_n77C-n257A2 DC_19A_n77C-n257D2 DC_19A_n77C-n257E2 DC_19A_n77C-n257F2 | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n77A-n257A DC_19A_n77A-n257G DC_19A_n77A-n257H DC_19A_n77A-n257I |
| DC_19A_n78A-n257A2 DC_19A_n78A-n257D2 DC_19A_n78A-n257E2 DC_19A_n78A-n257F2 DC_19A_n78A-n257G2 DC_19A_n78A-n257H2 DC_19A_n78A-n257I2 DC_19A_n78C-n257A2 DC_19A_n78C-n257D2 DC_19A_n78C-n257E2 DC_19A_n78C-n257F2 | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n78A-n257A DC_19A_n78A-n257G DC_19A_n78A-n257H DC_19A_n78A-n257I |
| DC_19A_n79A-n257A2 DC_19A_n79A-n257D2 DC_19A_n79A-n257E2 DC_19A_n79A-n257F2 DC_19A_n79A-n257G2 DC_19A_n79A-n257H2 DC_19A_n79A-n257I2 DC_19A_n79C-n257A2 DC_19A_n79C-n257D2 DC_19A_n79C-n257E2 DC_19A_n79C-n257F2 | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_21A_n77A-n257A2 DC_21A_n77A-n257G2 DC_21A_n77A-n257H2 DC_21A_n77A-n257I2 | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_21A_n78A-n257A2 DC_21A_n78A-n257G2 DC_21A_n78A-n257H2 DC_21A_n78A-n257I2 | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_21A_n79A-n257A2 DC_21A_n79A-n257G2 DC_21A_n79A-n257H2 DC_21A_n79A-n257I2 | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_28A_n3A-n257A2 DC_28A_n3A-n257G2 DC_28A_n3A-n257H2 DC_28A_n3A-n257I2 | DC_28A_n3A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n7A-n258A DC_28A_n7A-n258B DC_28A_n7A-n258C DC_28A_n7A-n258D DC_28A_n7A-n258E DC_28A_n7A-n258F DC_28A_n7A-n258G DC_28A_n7A-n258H DC_28A_n7A-n258I DC_28A_n7A-n258J DC_28A_n7A-n258K DC_28A_n7A-n258L DC_28A_n7A-n258M DC_28A_n7B-n258A DC_28A_n7B-n258B DC_28A_n7B-n258C DC_28A_n7B-n258D DC_28A_n7B-n258E DC_28A_n7B-n258F DC_28A_n7B-n258G DC_28A_n7B-n258H DC_28A_n7B-n258I DC_28A_n7B-n258J DC_28A_n7B-n258K DC_28A_n7B-n258L DC_28A_n7B-n258M | DC_28A_n7A DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_28A_n38A-n257A DC_28A_n38A-n257G DC_28A_n38A-n257H DC_28A_n38A-n257I | DC_28A_n38A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n77A-n257A2 DC_28A_n77A-n257D2 DC_28A_n77A-n257G2 DC_28A_n77A-n257H2 DC_28A_n77A-n257I2 | DC_28A_n77A DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n77(2A)-n257A2 DC_28A_n77(2A)-n257D2 DC_28A_n77(2A)-n257G2 DC_28A_n77(2A)-n257H2 DC_28A_n77(2A)-n257I2 | DC_28A_n77A DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n78A-n257A2 DC_28A_n78A-n257G2 DC_28A_n78A-n257H2 DC_28A_n78A-n257I2 | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n8A-n258A | DC_28A_n8A DC_28A_n258A |
| DC_28A_n78A-n258A DC_28A_n78A-n258G DC_28A_n78A-n258H DC_28A_n78A-n258I DC_28A_n78A-n258J DC_28A_n78A-n258K DC_28A_n78A-n258L DC_28A_n78A-n258M | DC_28A_n78A DC_28A_n258A |
| DC_39A_n40A-n258A | DC_39A_n40A DC_39A_n258A DC_39A_n40A-n258A |
| DC_39A_n41A-n258A DC_39A_n41C-n258A | DC_39A_n41A DC_39A_n258A DC_39A_n41A-n258A |
| DC_39A_n79A-n258A DC_39A_n79C-n258A | DC_39A_n79A DC_39A_n258A DC_39A_n79A-n258A |
| DC_40A_n41A-n258A DC_40A_n41C-n258A | DC_40A_n41A DC_40A_n258A DC_40A_n41A-n258A |
| DC_40A_n77A-n257A DC_40A_n77A-n257D DC_40A_n77A-n257E DC_40A_n77A-n257F DC_40A_n77A-n257G DC_40A_n77A-n257H DC_40A_n77A-n257I DC_40A_n77A-n257J DC_40A_n77A-n257K DC_40A_n77A-n257L DC_40A_n77A-n257M DC_40A_n77C-n257A DC_40A_n77C-n257D DC_40A_n77C-n257E DC_40A_n77C-n257F DC_40A_n77C-n257G DC_40A_n77C-n257H DC_40A_n77C-n257I DC_40A_n77C-n257J DC_40A_n77C-n257K DC_40A_n77C-n257L DC_40A_n77C-n257M | DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M |
| DC_40A_n78A-n257A DC_40A_n78A-n257D DC_40A_n78A-n257E DC_40A_n78A-n257F DC_40A_n78A-n257G DC_40A_n78A-n257H DC_40A_n78A-n257I DC_40A_n78A-n257J DC_40A_n78A-n257K DC_40A_n78A-n257L DC_40A_n78A-n257M | DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M |
| DC_40A_n79A-n258A DC_40A_n79C-n258A | DC_40A_n79A DC_40A_n258A DC_40A_n79A-n258A |
| DC_41A_n3A-n257A2 DC_41A_n3A-n257G2 DC_41A_n3A-n257H2 DC_41A_n3A-n257I2 DC_41C_n3A-n257A2 DC_41C_n3A-n257G2 DC_41C_n3A-n257H2 DC_41C_n3A-n257I2 | DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n28A-n257A2 DC_41A_n28A-n257G2 DC_41A_n28A-n257H2 DC_41A_n28A-n257I2 DC_41C_n28A-n257A2 DC_41C_n28A-n257G2 DC_41C_n28A-n257H2 DC_41C_n28A-n257I2 | DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n77A-n257A DC_41A_n77A-n257G DC_41A_n77A-n257H DC_41A_n77A-n257I DC_41C_n77A-n257A DC_41C_n77A-n257G DC_41C_n77A-n257H DC_41C_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n78A-n257A DC_41A_n78A-n257G DC_41A_n78A-n257H DC_41A_n78A-n257I DC_41C_n78A-n257A DC_41C_n78A-n257G DC_41C_n78A-n257H DC_41C_n78A-n257I | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n79A-n258A DC_41A_n79C-n258A | DC_41A_n79A DC_41A_n258A DC_41A_n79A-n258A |
| DC_42A_n77A-n257A DC_42A_n77A-n257D DC_42A_n77A-n257E DC_42A_n77A-n257F DC_42A_n77A-n257G DC_42A_n77A-n257H DC_42A_n77A-n257I DC_42A_n77A-n257J DC_42A_n77A-n257K DC_42A_n77A-n257L DC_42A_n77A-n257M DC_42A_n77C-n257A DC_42A_n77C-n257D DC_42A_n77C-n257E DC_42A_n77C-n257F DC_42A_n77C-n257G DC_42A_n77C-n257H DC_42A_n77C-n257I DC_42A_n77C-n257J DC_42A_n77C-n257K DC_42A_n77C-n257L DC_42A_n77C-n257M DC_42C_n77A-n257A DC_42C_n77A-n257G DC_42C_n77A-n257H DC_42C_n77A-n257I | DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257K DC_42A_n257L DC_42A_n257M |
| DC_42A_n78A-n257A DC_42A_n78A-n257D DC_42A_n78A-n257E DC_42A_n78A-n257F DC_42A_n78A-n257G DC_42A_n78A-n257H DC_42A_n78A-n257I DC_42A_n78A-n257J DC_42A_n78A-n257K DC_42A_n78A-n257L DC_42A_n78A-n257M DC_42C_n78A-n257A DC_42C_n78A-n257G DC_42C_n78A-n257H DC_42C_n78A-n257I | DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257L DC_42A_n257M DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_42A_n79A-n257A DC_42A_n79A-n257G DC_42A_n79A-n257H DC_42A_n79A-n257I DC_42C_n79A-n257A DC_42C_n79A-n257G DC_42C_n79A-n257H DC_42C_n79A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_66A_n5A-n260A DC_66A_n5A-n260G DC_66A_n5A-n260H DC_66A_n5A-n260I DC_66A_n5A-n260J DC_66A_n5A-n260K DC_66A_n5A-n260L DC_66A_n5A-n260M | DC_66A_n5A DC_66A_n260A DC_66A_n5A-n260A |
| DC_66A_n5A-n260(2A) DC_66A_n5A-n260(3A) DC_66A_n5A-n260(4A) DC_66A_n5A-n260(5A) DC_66A_n5A-n260(6A) DC_66A_n5A-n260(2H) DC_66A_n5A-n260(2G) DC_66A_n5A-n260(A-2G) DC_66A_n5A-n260(A-H) DC_66A_n5A-n260(A-G) DC_66A_n5A-n260(G-H) DC_66A_n5A-n260(2A-G) DC_66A_n5A-n260(2A-2G) DC_66A_n5A-n260(3A-G) | DC_66A_n5A-n260A |
| DC_66A_n5A-n261A DC_66A_n5A-n261G DC_66A_n5A-n261H DC_66A_n5A-n261I DC_66A_n5A-n261J DC_66A_n5A-n261K DC_66A_n5A-n261L DC_66A_n5A-n261M | DC_66A_n5A-n261A |
| DC_66A_n5A-n261(2A) DC_66A_n5A-n261(3A) DC_66A_n5A-n261(4A) DC_66A_n5A-n261(2G) DC_66A_n5A-n261(2H) DC_66A_n5A-n261(A-G) DC_66A_n5A-n261(A-H) DC_66A_n5A-n261(A-I) DC_66A_n5A-n261(A-J) DC_66A_n5A-n261(A-K) DC_66A_n5A-n261(G-H) DC_66A_n5A-n261(G-I) DC_66A_n5A-n261(G-J) DC_66A_n5A-n261(H-I) DC_66A_n5A-n261(A-2G) DC_66A_n5A-n261(A-G-H) DC_66A_n5A-n261(A-G-I) DC_66A_n5A-n261(2A-G) DC_66A_n5A-n261(2A-H) DC_66A_n5A-n261(2A-I) DC_66A_n5A-n261(3A-G) | DC_66A_n5A-n261A |
| DC_66A_n12A-n258A | DC_66A_n12A DC_66A_n258A |
| DC_66A_n12A-n260A | DC_66A_n12A DC_66A_n260A |
| DC_66A_n12A-n261A | DC_66A_n12A DC_66A_n261A |
| DC_66A_n41A-n260A | DC_66A_n41A |
| DC_66A_n41A-n260(2A) DC_66A_n41A-n260(3A) DC_66A_n41A-n260(4A) | DC_66A_n41A |
| DC_66A_n41A-n261A | DC_66A_n41A |
| DC_66A_n41A-n261(2A) | DC_66A_n41A |
| DC_66A_n71A-n260A | DC_66A_n71A DC_66A_n260A |
| DC_66A_n71A-n260(2A) | DC_66A_n71A DC_66A_n260A |
| DC_66A_n71A-n261A | DC_66A_n71A DC_66A_n261A |
| DC_66A_n71A-n261(2A) | DC_66A_n71A DC_66A_n261A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. |  |

#### 5.5B.6.3	Inter-band EN-DC configurations including FR1 and FR2 (four bands)

Table 5.5B.6.3-1: Inter-band EN-DC configurations including FR1 and FR2 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-(n)3AA-n257A DC_1A-(n)3AA-n257G DC_1A-(n)3AA-n257H DC_1A-(n)3AA-n257I DC_1A-(n)3AA-n257J DC_1A-(n)3AA-n257K DC_1A-(n)3AA-n257L DC_1A-(n)3AA-n257M | DC_1A_n3A DC_1A_n257A DC_(n)3AA3 DC_3A_n257A |
| DC_1A_n3A-n8A-n257A DC_1A_n3A-n8A-n257G DC_1A_n3A-n8A-n257H DC_1A_n3A-n8A-n257I DC_1A_n3A-n8A-n257J DC_1A_n3A-n8A-n257K DC_1A_n3A-n8A-n257L DC_1A_n3A-n8A-n257M | DC_1A_n3A DC_1A_n8A DC_1A_n257A |
| DC_1A-3A_n8A-n257G DC_1A-3A_n8A-n257H DC_1A-3A_n8A-n257I DC_1A-3A_n8A-n257J DC_1A-3A_n8A-n257K DC_1A-3A_n8A-n257L DC_1A-3A_n8A-n257M | DC_1A_n8A DC_1A_n257A DC_3A_n8A DC_3A_n257A |
| DC_1A-3A_n28A-n257A2 DC_1A-3A_n28A-n257G2 DC_1A-3A_n28A-n257H2 DC_1A-3A_n28A-n257I2 | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_1A-3A_n38A-n257A DC_1A-3A_n38A-n257G DC_1A-3A_n38A-n257H DC_1A-3A_n38A-n257I | DC_1A_n38A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n38A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_1A-3A_n77A-n257A2 | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_3A_n257A |
| DC_1A-3A_n77A-n257D2 | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257D DC_3A_n257A DC_3A_n257D |
| DC_1A-3A_n77A-n257G2 | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n257A DC_3A_n257G DC_1A_n77A-n257A DC_1A_n77A-n257G DC_3A_n77A-n257A DC_3A_n77A-n257G |
| DC_1A-3A_n77A-n257H2 | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H |
| DC_1A-3A_n77A-n257I2 | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I |
| DC_1A-3A_n78A-n257A2 DC_1A-3A_n78A-n257D2 DC_1A-3A_n78A-n257E2 DC_1A-3A_n78A-n257F2 DC_1A-3A_n78A-n257G2 DC_1A-3A_n78A-n257H2 DC_1A-3A_n78A-n257I2 DC_1A-3A_n78A-n257J2 DC_1A-3A_n78A-n257K2 DC_1A-3A_n78A-n257L2 DC_1A-3A_n78A-n257M2 DC_1A-3C_n78A-n257A DC_1A-3C_n78A-n257D DC_1A-3C_n78A-n257E DC_1A-3C_n78A-n257F DC_1A-3C_n78A-n257G DC_1A-3C_n78A-n257H DC_1A-3C_n78A-n257I DC_1A-3C_n78A-n257J DC_1A-3C_n78A-n257K DC_1A-3C_n78A-n257L DC_1A-3C_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_1A-3A_n78C-n257A DC_1A-3A_n78C-n257D DC_1A-3A_n78C-n257E DC_1A-3A_n78C-n257F DC_1A-3A_n78C-n257G DC_1A-3A_n78C-n257H DC_1A-3A_n78C-n257I DC_1A-3A_n78C-n257J DC_1A-3A_n78C-n257K DC_1A-3A_n78C-n257L DC_1A-3A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_1A-3A_n78A-n258A DC_1A-3A_n78A-n258D DC_1A-3A_n78A-n258E DC_1A-3A_n78A-n258F DC_1A-3A_n78A-n258G DC_1A-3A_n78A-n258H DC_1A-3A_n78A-n258I DC_1A-3A_n78A-n258J DC_1A-3A_n78A-n258K DC_1A-3A_n78A-n258L DC_1A-3A_n78A-n258M | DC_1A_n78A DC_1A_n258A DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_3A_n78A DC_3A_n258A DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I |
| DC_1A-3A_n79A-n257A2 DC_1A-3A_n79A-n257G2 DC_1A-3A_n79A-n257H2 DC_1A-3A_n79A-n257I2 | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I |
| DC_1A-3A_n105A-n257A | DC_1A_n105A DC_1A_n257A DC_3A_n105A DC_3A_n257A |
| DC_1A-3A_n105A-n258A | DC_1A_n105A DC_1A_n258A DC_3A_n105A DC_3A_n258A |
| DC_1A-5A_n78A-n257A DC_1A-5A_n78A-n257D DC_1A-5A_n78A-n257E DC_1A-5A_n78A-n257F DC_1A-5A_n78A-n257G DC_1A-5A_n78A-n257H DC_1A-5A_n78A-n257I DC_1A-5A_n78A-n257J DC_1A-5A_n78A-n257K DC_1A-5A_n78A-n257L DC_1A-5A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_1A-5A_n78C-n257A DC_1A-5A_n78C-n257D DC_1A-5A_n78C-n257E DC_1A-5A_n78C-n257F DC_1A-5A_n78C-n257G DC_1A-5A_n78C-n257H DC_1A-5A_n78C-n257I DC_1A-5A_n78C-n257J DC_1A-5A_n78C-n257K DC_1A-5A_n78C-n257L DC_1A-5A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_1A-7A_n38A-n257A DC_1A-7A_n38A-n257G DC_1A-7A_n38A-n257H DC_1A-7A_n38A-n257I | DC_1A_n38A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_7A_n38A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_1A-7A_n78A-n257A DC_1A-7A_n78A-n257D DC_1A-7A_n78A-n257E DC_1A-7A_n78A-n257F DC_1A-7A_n78A-n257G DC_1A-7A_n78A-n257H DC_1A-7A_n78A-n257I DC_1A-7A_n78A-n257J DC_1A-7A_n78A-n257K DC_1A-7A_n78A-n257L DC_1A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-7A_n78C-n257A DC_1A-7A_n78C-n257D DC_1A-7A_n78C-n257E DC_1A-7A_n78C-n257F DC_1A-7A_n78C-n257G DC_1A-7A_n78C-n257H DC_1A-7A_n78C-n257I DC_1A-7A_n78C-n257J DC_1A-7A_n78C-n257K DC_1A-7A_n78C-n257L DC_1A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-7A-7A_n78A-n257A DC_1A-7A-7A_n78A-n257D DC_1A-7A-7A_n78A-n257E DC_1A-7A-7A_n78A-n257F DC_1A-7A-7A_n78A-n257G DC_1A-7A-7A_n78A-n257H DC_1A-7A-7A_n78A-n257I DC_1A-7A-7A_n78A-n257J DC_1A-7A-7A_n78A-n257K DC_1A-7A-7A_n78A-n257L DC_1A-7A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-7A-7A_n78C-n257A DC_1A-7A-7A_n78C-n257D DC_1A-7A-7A_n78C-n257E DC_1A-7A-7A_n78C-n257F DC_1A-7A-7A_n78C-n257G DC_1A-7A-7A_n78C-n257H DC_1A-7A-7A_n78C-n257I DC_1A-7A-7A_n78C-n257J DC_1A-7A-7A_n78C-n257K DC_1A-7A-7A_n78C-n257L DC_1A-7A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-7A_n78A-n258A DC_1A-7A_n78A-n258D DC_1A-7A_n78A-n258E DC_1A-7A_n78A-n258F DC_1A-7A_n78A-n258G DC_1A-7A_n78A-n258H DC_1A-7A_n78A-n258I DC_1A-7A_n78A-n258J DC_1A-7A_n78A-n258K DC_1A-7A_n78A-n258L DC_1A-7A_n78A-n258M | DC_1A_n78A DC_1A_n258A DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_7A_n78A DC_7A_n258A DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I |
| DC_1A-8A_n40A-n258A DC_1A-8A_n40A-n258D DC_1A-8A_n40A-n258E DC_1A-8A_n40A-n258F DC_1A-8A_n40A-n258G DC_1A-8A_n40A-n258H DC_1A-8A_n40A-n258I DC_1A-8A_n40A-n258J DC_1A-8A_n40A-n258K DC_1A-8A_n40A-n258L DC_1A-8A_n40A-n258M | DC_1A_n40A DC_1A_n258A DC_8A_n40A DC_8A_n258A |
| DC_1A-8A_n77A-n257A2 DC_1A-8A_n77A-n257D2 DC_1A-8A_n77A-n257G2 DC_1A-8A_n77A-n257H2 DC_1A-8A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_1A-8A_n77(2A)-n257A2 DC_1A-8A_n77(2A)-n257D2 DC_1A-8A_n77(2A)-n257G2 DC_1A-8A_n77(2A)-n257H2 DC_1A-8A_n77(2A)-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_1A-8A_n78A-n257A2 DC_1A-8A_n78A-n257D2 DC_1A-8A_n78A-n257E2 DC_1A-8A_n78A-n257F2 DC_1A-8A_n78A-n257G2 DC_1A-8A_n78A-n257H2 DC_1A-8A_n78A-n257I2 DC_1A-8A_n78A-n257J2 DC_1A-8A_n78A-n257K2 DC_1A-8A_n78A-n257L2 DC_1A-8A_n78A-n257M2 | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A |
| DC_1A-8A_n78A-n258A DC_1A-8A_n78A-n258D DC_1A-8A_n78A-n258E DC_1A-8A_n78A-n258F DC_1A-8A_n78A-n258G DC_1A-8A_n78A-n258H DC_1A-8A_n78A-n258I DC_1A-8A_n78A-n258J DC_1A-8A_n78A-n258K DC_1A-8A_n78A-n258L DC_1A-8A_n78A-n258M | DC_1A_n78A DC_1A_n258A DC_8A_n78A DC_8A_n258A |
| DC_1A-11A_n77A-n257A2 DC_1A-11A_n77A-n257D2 DC_1A-11A_n77A-n257G2 DC_1A-11A_n77A-n257H2 DC_1A-11A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-11A_n77(2A)-n257A2 DC_1A-11A_n77(2A)-n257D2 DC_1A-11A_n77(2A)-n257G2 DC_1A-11A_n77(2A)-n257H2 DC_1A-11A_n77(2A)-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-18A_n3A-n257A DC_1A-18A_n3A-n257G DC_1A-18A_n3A-n257H DC_1A-18A_n3A-n257I | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-18A_n78A-n257A DC_1A-18A_n78A-n257G DC_1A-18A_n78A-n257H DC_1A-18A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-19A_n77A-n257A2 DC_1A-19A_n77A-n257G2 DC_1A-19A_n77A-n257H2 DC_1A-19A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-19A_n78A-n257A2 DC_1A-19A_n78A-n257G2 DC_1A-19A_n78A-n257H2 DC_1A-19A_n78A-n257I2 | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-19A_n79A-n257A2 DC_1A-19A_n79A-n257G2 DC_1A-19A_n79A-n257H2 DC_1A-19A_n79A-n257I2 | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_1A-21A_n77A-n257A2 DC_1A-21A_n77A-n257G2 DC_1A-21A_n77A-n257H2 DC_1A-21A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_1A-21A_n78A-n257A2 DC_1A-21A_n78A-n257G2 DC_1A-21A_n78A-n257H2 DC_1A-21A_n78A-n257I2 | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_1A-21A_n79A-n257A2 DC_1A-21A_n79A-n257G2 DC_1A-21A_n79A-n257H2 DC_1A-21A_n79A-n257I2 | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_1A-28A_n3A-n257A2 DC_1A-28A_n3A-n257G2 DC_1A-28A_n3A-n257H2 DC_1A-28A_n3A-n257I2 | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n3A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-28A_n38A-n257A DC_1A-28A_n38A-n257G DC_1A-28A_n38A-n257H DC_1A-28A_n38A-n257I | DC_1A_n38A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n38A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-28A_n78A-n257A2 DC_1A-28A_n78A-n257G2 DC_1A-28A_n78A-n257H2 DC_1A-28A_n78A-n257I2 | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-41A_n3A-n257A2 DC_1A-41A_n3A-n257G DC_1A-41A_n3A-n257H DC_1A-41A_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257I |
| DC_1A-41C_n3A-n257A2 DC_1A-41C_n3A-n257G DC_1A-41C_n3A-n257H DC_1A-41C_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257I |
| DC_1A-41A_n28A-n257A2 DC_1A-41A_n28A-n257G2 DC_1A-41A_n28A-n257H2 DC_1A-41A_n28A-n257I2 DC_1A-41C_n28A-n257A2 DC_1A-41C_n28A-n257G2 DC_1A-41C_n28A-n257H2 DC_1A-41C_n28A-n257I2 | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-41A_n77A-n257A DC_1A-41A_n77A-n257G DC_1A-41A_n77A-n257H DC_1A-41A_n77A-n257I DC_1A-41C_n77A-n257A DC_1A-41C_n77A-n257G DC_1A-41C_n77A-n257H DC_1A-41C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-41A_n78A-n257A DC_1A-41A_n78A-n257G DC_1A-41A_n78A-n257H DC_1A-41A_n78A-n257I DC_1A-41C_n78A-n257A DC_1A-41C_n78A-n257G DC_1A-41C_n78A-n257H DC_1A-41C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-42A_n77A-n257A DC_1A-42A_n77A-n257G DC_1A-42A_n77A-n257H DC_1A-42A_n77A-n257I DC_1A-42C_n77A-n257A DC_1A-42C_n77A-n257G DC_1A-42C_n77A-n257H DC_1A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I |
| DC_1A-42A_n78A-n257A DC_1A-42A_n78A-n257G DC_1A-42A_n78A-n257H DC_1A-42A_n78A-n257I DC_1A-42C_n78A-n257A DC_1A-42C_n78A-n257G DC_1A-42C_n78A-n257H DC_1A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I |
| DC_1A-42A_n79A-n257A DC_1A-42A_n79A-n257G DC_1A-42A_n79A-n257H DC_1A-42A_n79A-n257I DC_1A-42C_n79A-n257A DC_1A-42C_n79A-n257G DC_1A-42C_n79A-n257H DC_1A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I |
| DC_2A-28A_n7A-n258A DC_2A-28A_n7A-n258G DC_2A-28A_n7A-n258H DC_2A-28A_n7A-n258I | DC_2A_n7A DC_2A_n258A DC_2A_n258G DC_2A_n258H DC_2A_n258I DC_28A_n7A DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_2A-66A_n41A-n260A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n41A-n260(2A) DC_2A-66A_n41A-n260(3A) DC_2A-66A_n41A-n260(4A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n41A-n261A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n41A-n261(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n71A-n261A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A_n71A-n261(2A) | DC_2A_n71A DC_66A_n71A |
| DC_3A_n1A-n78A-n257A2 DC_3A_n1A-n78A-n257D2 DC_3A_n1A-n78A-n257E2 DC_3A_n1A-n78A-n257F2 DC_3A_n1A-n78A-n257G2 DC_3A_n1A-n78A-n257H2 DC_3A_n1A-n78A-n257I2 DC_3A_n1A-n78A-n257J2 DC_3A_n1A-n78A-n257K2 DC_3A_n1A-n78A-n257L2 DC_3A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A |
| DC_3A_n41A-n79A-n258A | DC_3A_n41A DC_3A_n79A DC_3A_n258A |
| DC_3A-3A_n1A-n78A-n257A2 DC_3A-3A_n1A-n78A-n257D2 DC_3A-3A_n1A-n78A-n257E2 DC_3A-3A_n1A-n78A-n257F2 DC_3A-3A_n1A-n78A-n257G2 DC_3A-3A_n1A-n78A-n257H2 DC_3A-3A_n1A-n78A-n257I2 DC_3A-3A_n1A-n78A-n257J2 DC_3A-3A_n1A-n78A-n257K2 DC_3A-3A_n1A-n78A-n257L2 DC_3A-3A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A |
| DC_3A-5A_n78A-n257A DC_3A-5A_n78A-n257D DC_3A-5A_n78A-n257E DC_3A-5A_n78A-n257F DC_3A-5A_n78A-n257G DC_3A-5A_n78A-n257H DC_3A-5A_n78A-n257I DC_3A-5A_n78A-n257J DC_3A-5A_n78A-n257K DC_3A-5A_n78A-n257L DC_3A-5A_n78A-n257M | DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_3A-5A_n78C-n257A DC_3A-5A_n78C-n257D DC_3A-5A_n78C-n257E DC_3A-5A_n78C-n257F DC_3A-5A_n78C-n257G DC_3A-5A_n78C-n257H DC_3A-5A_n78C-n257I DC_3A-5A_n78C-n257J DC_3A-5A_n78C-n257K DC_3A-5A_n78C-n257L DC_3A-5A_n78C-n257M | DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_3A-7A_n1A-n257A2 DC_3A-7A_n1A-n257D2 DC_3A-7A_n1A-n257E2 DC_3A-7A_n1A-n257F2 DC_3A-7A_n1A-n257G2 DC_3A-7A_n1A-n257H2 DC_3A-7A_n1A-n257I2 DC_3A-7A_n1A-n257J2 DC_3A-7A_n1A-n257K2 DC_3A-7A_n1A-n257L2 DC_3A-7A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-3A-7A_n1A-n257A2 DC_3A-3A-7A_n1A-n257D2 DC_3A-3A-7A_n1A-n257E2 DC_3A-3A-7A_n1A-n257F2 DC_3A-3A-7A_n1A-n257G2 DC_3A-3A-7A_n1A-n257H2 DC_3A-3A-7A_n1A-n257I2 DC_3A-3A-7A_n1A-n257J2 DC_3A-3A-7A_n1A-n257K2 DC_3A-3A-7A_n1A-n257L2 DC_3A-3A-7A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-7A-7A_n1A-n257A2 DC_3A-7A-7A_n1A-n257D2 DC_3A-7A-7A_n1A-n257E2 DC_3A-7A-7A_n1A-n257F2 DC_3A-7A-7A_n1A-n257G2 DC_3A-7A-7A_n1A-n257H2 DC_3A-7A-7A_n1A-n257I2 DC_3A-7A-7A_n1A-n257J2 DC_3A-7A-7A_n1A-n257K2 DC_3A-7A-7A_n1A-n257L2 DC_3A-7A-7A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-3A-7A-7A_n1A-n257A2 DC_3A-3A-7A-7A_n1A-n257D2 DC_3A-3A-7A-7A_n1A-n257E2 DC_3A-3A-7A-7A_n1A-n257F2 DC_3A-3A-7A-7A_n1A-n257G2 DC_3A-3A-7A-7A_n1A-n257H2 DC_3A-3A-7A-7A_n1A-n257I2 DC_3A-3A-7A-7A_n1A-n257J2 DC_3A-3A-7A-7A_n1A-n257K2 DC_3A-3A-7A-7A_n1A-n257L2 DC_3A-3A-7A-7A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-7A_n38A-n257A DC_3A-7A_n38A-n257G DC_3A-7A_n38A-n257H DC_3A-7A_n38A-n257I | DC_3A_n38A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_7A_n38A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_3A-7A_n40A-n258A DC_3A-7A_n40A-n258D DC_3A-7A_n40A-n258G DC_3A-7A_n40A-n258E DC_3A-7A_n40A-n258F DC_3A-7A_n40A-n258H DC_3A-7A_n40A-n258I DC_3A-7A_n40A-n258J DC_3A-7A_n40A-n258K DC_3A-7A_n40A-n258L DC_3A-7A_n40A-n258M | DC_3A_n40A DC_7A_n40A DC_3A_n258A DC_7A_n258A |
| DC_3A-3A-7A_n78A-n257A2 DC_3A-3A-7A_n78A-n257D2 DC_3A-3A-7A_n78A-n257E2 DC_3A-3A-7A_n78A-n257F2 DC_3A-3A-7A_n78A-n257G2 DC_3A-3A-7A_n78A-n257H2 DC_3A-3A-7A_n78A-n257I2 DC_3A-3A-7A_n78A-n257J2 DC_3A-3A-7A_n78A-n257K2 DC_3A-3A-7A_n78A-n257L2 DC_3A-3A-7A_n78A-n257M2 DC_3A-3A-7A-7A_n78A-n257A2 DC_3A-3A-7A-7A_n78A-n257D2 DC_3A-3A-7A-7A_n78A-n257E2 DC_3A-3A-7A-7A_n78A-n257F2 DC_3A-3A-7A-7A_n78A-n257G2 DC_3A-3A-7A-7A_n78A-n257H2 DC_3A-3A-7A-7A_n78A-n257I2 DC_3A-3A-7A-7A_n78A-n257J2 DC_3A-3A-7A-7A_n78A-n257K2 DC_3A-3A-7A-7A_n78A-n257L2 DC_3A-3A-7A-7A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-7A_n78A-n257A2 DC_3A-7A_n78A-n257D2 DC_3A-7A_n78A-n257E2 DC_3A-7A_n78A-n257F2 DC_3A-7A_n78A-n257G2 DC_3A-7A_n78A-n257H2 DC_3A-7A_n78A-n257I2 DC_3A-7A_n78A-n257J2 DC_3A-7A_n78A-n257K2 DC_3A-7A_n78A-n257L2 DC_3A-7A_n78A-n257M2 DC_3A-7A_n78C-n257A DC_3A-7A_n78C-n257D DC_3A-7A_n78C-n257E DC_3A-7A_n78C-n257F DC_3A-7A_n78C-n257G DC_3A-7A_n78C-n257H DC_3A-7A_n78C-n257I DC_3A-7A_n78C-n257J DC_3A-7A_n78C-n257K DC_3A-7A_n78C-n257L DC_3A-7A_n78C-n257M | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_3A-7A-7A_n78A-n257A2 DC_3A-7A-7A_n78A-n257D2 DC_3A-7A-7A_n78A-n257E2 DC_3A-7A-7A_n78A-n257F2 DC_3A-7A-7A_n78A-n257G2 DC_3A-7A-7A_n78A-n257H2 DC_3A-7A-7A_n78A-n257I2 DC_3A-7A-7A_n78A-n257J2 DC_3A-7A-7A_n78A-n257K2 DC_3A-7A-7A_n78A-n257L2 DC_3A-7A-7A_n78A-n257M2 DC_3A-7A-7A_n78C-n257A DC_3A-7A-7A_n78C-n257D DC_3A-7A-7A_n78C-n257E DC_3A-7A-7A_n78C-n257F DC_3A-7A-7A_n78C-n257G DC_3A-7A-7A_n78C-n257H DC_3A-7A-7A_n78C-n257I DC_3A-7A-7A_n78C-n257J DC_3A-7A-7A_n78C-n257K DC_3A-7A-7A_n78C-n257L DC_3A-7A-7A_n78C-n257M | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_3A-7A_n78A-n258A DC_3A-7A_n78A-n258D DC_3A-7A_n78A-n258E DC_3A-7A_n78A-n258F DC_3A-7A_n78A-n258G DC_3A-7A_n78A-n258H DC_3A-7A_n78A-n258I DC_3A-7A_n78A-n258J DC_3A-7A_n78A-n258K DC_3A-7A_n78A-n258L DC_3A-7A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_7A_n78A DC_7A_n258A DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I |
| DC_3A-8A_n1A-n257A2 DC_3A-8A_n1A-n257D2 DC_3A-8A_n1A-n257E2 DC_3A-8A_n1A-n257F2 DC_3A-8A_n1A-n257G2 DC_3A-8A_n1A-n257H2 DC_3A-8A_n1A-n257I2 DC_3A-8A_n1A-n257J2 DC_3A-8A_n1A-n257K2 DC_3A-8A_n1A-n257L2 DC_3A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-8A_n1A-n257A2 DC_3A-3A-8A_n1A-n257D2 DC_3A-3A-8A_n1A-n257E2 DC_3A-3A-8A_n1A-n257F2 DC_3A-3A-8A_n1A-n257G2 DC_3A-3A-8A_n1A-n257H2 DC_3A-3A-8A_n1A-n257I2 DC_3A-3A-8A_n1A-n257J2 DC_3A-3A-8A_n1A-n257K2 DC_3A-3A-8A_n1A-n257L2 DC_3A-3A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-8A_n40A-n258A DC_3A-8A_n40A-n258D DC_3A-8A_n40A-n258E DC_3A-8A_n40A-n258F DC_3A-8A_n40A-n258G DC_3A-8A_n40A-n258H DC_3A-8A_n40A-n258I DC_3A-8A_n40A-n258J DC_3A-8A_n40A-n258K DC_3A-8A_n40A-n258L DC_3A-8A_n40A-n258M | DC_3A_n40A DC_3A_n258A DC_8A_n40A DC_8A_n258A |
| DC_3A_n8A-n77A-n257A DC_3A_n8A-n77A-n257G DC_3A_n8A-n77A-n257H DC_3A_n8A-n77A-n257I DC_3A_n8A-n77A-n257J DC_3A_n8A-n77A-n257K DC_3A_n8A-n77A-n257L DC_3A_n8A-n77A-n257M | DC_3A_n8A DC_3A_n77A DC_3A_n257A |
| DC_3A_n8A-n77(2A)-n257A DC_3A_n8A-n77(2A)-n257G DC_3A_n8A-n77(2A)-n257H DC_3A_n8A-n77(2A)-n257I DC_3A_n8A-n77(2A)-n257J DC_3A_n8A-n77(2A)-n257K DC_3A_n8A-n77(2A)-n257L DC_3A_n8A-n77(2A)-n257M | DC_3A_n8A DC_3A_n77A DC_3A_n257A |
| DC_3A-8A_n78A-n257A2 DC_3A-8A_n78A-n257D2 DC_3A-8A_n78A-n257E2 DC_3A-8A_n78A-n257F2 DC_3A-8A_n78A-n257G2 DC_3A-8A_n78A-n257H2 DC_3A-8A_n78A-n257I2 DC_3A-8A_n78A-n257J2 DC_3A-8A_n78A-n257K2 DC_3A-8A_n78A-n257L2 DC_3A-8A_n78A-n257M2 DC_3C-8A_n78A-n257A DC_3C-8A_n78A-n257D DC_3C-8A_n78A-n257G DC_3C-8A_n78A-n257E DC_3C-8A_n78A-n257F DC_3C-8A_n78A-n257H DC_3C-8A_n78A-n257I DC_3C-8A_n78A-n257J DC_3C-8A_n78A-n257K DC_3C-8A_n78A-n257L DC_3C-8A_n78A-n257M | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-8A_n78A-n257A2 DC_3A-3A-8A_n78A-n257D2 DC_3A-3A-8A_n78A-n257E2 DC_3A-3A-8A_n78A-n257F2 DC_3A-3A-8A_n78A-n257G2 DC_3A-3A-8A_n78A-n257H2 DC_3A-3A-8A_n78A-n257I2 DC_3A-3A-8A_n78A-n257J2 DC_3A-3A-8A_n78A-n257K2 DC_3A-3A-8A_n78A-n257L2 DC_3A-3A-8A_n78A-n257M2 | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-8A_n78A-n258A DC_3A-8A_n78A-n258D DC_3A-8A_n78A-n258E DC_3A-8A_n78A-n258F DC_3A-8A_n78A-n258G DC_3A-8A_n78A-n258H DC_3A-8A_n78A-n258I DC_3A-8A_n78A-n258J DC_3A-8A_n78A-n258K DC_3A-8A_n78A-n258L DC_3A-8A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_8A_n78A DC_8A_n258A |
| DC_3A-18A_n78A-n257A DC_3A-18A_n78A-n257G DC_3A-18A_n78A-n257H DC_3A-18A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_3A-19A_n77A-n257A2 DC_3A-19A_n77A-n257G2 DC_3A-19A_n77A-n257H2 DC_3A-19A_n77A-n257I2 | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_19A_n77A-n257A DC_19A_n77A-n257G DC_19A_n77A-n257H DC_19A_n77A-n257I |
| DC_3A-19A_n78A-n257A2 DC_3A-19A_n78A-n257G2 DC_3A-19A_n78A-n257H2 DC_3A-19A_n78A-n257I2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_19A_n78A-n257A DC_19A_n78A-n257G DC_19A_n78A-n257H DC_19A_n78A-n257I |
| DC_3A-19A_n79A-n257A2 DC_3A-19A_n79A-n257G2 DC_3A-19A_n79A-n257H2 DC_3A-19A_n79A-n257I2 | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_3A-21A_n77A-n257A2 DC_3A-21A_n77A-n257G2 DC_3A-21A_n77A-n257H2 DC_3A-21A_n77A-n257I2 | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_3A-21A_n78A-n257A2 DC_3A-21A_n78A-n257G2 DC_3A-21A_n78A-n257H2 DC_3A-21A_n78A-n257I2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_3A-21A_n79A-n257A2 DC_3A-21A_n79A-n257G2 DC_3A-21A_n79A-n257H2 DC_3A-21A_n79A-n257I2 | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_3A-28A_n38A-n257A DC_3A-28A_n38A-n257G DC_3A-28A_n38A-n257H DC_3A-28A_n38A-n257I | DC_3A_n38A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n38A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-28A_n77A-n257A2 DC_3A-28A_n77A-n257D2 DC_3A-28A_n77A-n257G2 DC_3A-28A_n77A-n257H2 DC_3A-28A_n77A-n257I2 | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n77A DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-28A_n77(2A)-n257A2 DC_3A-28A_n77(2A)-n257D2 DC_3A-28A_n77(2A)-n257G2 DC_3A-28A_n77(2A)-n257H2 DC_3A-28A_n77(2A)-n257I2 | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n77A DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-28A_n78A-n257A2 DC_3A-28A_n78A-n257G2 DC_3A-28A_n78A-n257H2 DC_3A-28A_n78A-n257I2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-41A_n28A-n257A2 DC_3A-41A_n28A-n257G2 DC_3A-41A_n28A-n257H2 DC_3A-41A_n28A-n257I2 DC_3A-41C_n28A-n257A2 DC_3A-41C_n28A-n257G2 DC_3A-41C_n28A-n257H2 DC_3A-41C_n28A-n257I2 | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-41A_n77A-n257A DC_3A-41A_n77A-n257G DC_3A-41A_n77A-n257H DC_3A-41A_n77A-n257I DC_3A-41C_n77A-n257A DC_3A-41C_n77A-n257G DC_3A-41C_n77A-n257H DC_3A-41C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-41A_n78A-n257A DC_3A-41A_n78A-n257G DC_3A-41A_n78A-n257H DC_3A-41A_n78A-n257I DC_3A-41C_n78A-n257A DC_3A-41C_n78A-n257G DC_3A-41C_n78A-n257H DC_3A-41C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-42A_n77A-n257A DC_3A-42A_n77A-n257G DC_3A-42A_n77A-n257H DC_3A-42A_n77A-n257I DC_3A-42C_n77A-n257A DC_3A-42C_n77A-n257G DC_3A-42C_n77A-n257H DC_3A-42C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I |
| DC_3A-42A_n78A-n257A DC_3A-42A_n78A-n257G DC_3A-42A_n78A-n257H DC_3A-42A_n78A-n257I DC_3A-42C_n78A-n257A DC_3A-42C_n78A-n257G DC_3A-42C_n78A-n257H DC_3A-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_3A-42A_n79A-n257A DC_3A-42A_n79A-n257G DC_3A-42A_n79A-n257H DC_3A-42A_n79A-n257I DC_3A-42C_n79A-n257A DC_3A-42C_n79A-n257G DC_3A-42C_n79A-n257H DC_3A-42C_n79A-n257I | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I |
| DC_5A-7A_n78A-n257A DC_5A-7A_n78A-n257D DC_5A-7A_n78A-n257E DC_5A-7A_n78A-n257F DC_5A-7A_n78A-n257G DC_5A-7A_n78A-n257H DC_5A-7A_n78A-n257I DC_5A-7A_n78A-n257J DC_5A-7A_n78A-n257K DC_5A-7A_n78A-n257L DC_5A-7A_n78A-n257M | DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_5A-7A_n78C-n257A DC_5A-7A_n78C-n257D DC_5A-7A_n78C-n257E DC_5A-7A_n78C-n257F DC_5A-7A_n78C-n257G DC_5A-7A_n78C-n257H DC_5A-7A_n78C-n257I DC_5A-7A_n78C-n257J DC_5A-7A_n78C-n257K DC_5A-7A_n78C-n257L DC_5A-7A_n78C-n257M | DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_5A-7A-7A_n78A-n257A DC_5A-7A-7A_n78A-n257D DC_5A-7A-7A_n78A-n257E DC_5A-7A-7A_n78A-n257F DC_5A-7A-7A_n78A-n257G DC_5A-7A-7A_n78A-n257H DC_5A-7A-7A_n78A-n257I DC_5A-7A-7A_n78A-n257J DC_5A-7A-7A_n78A-n257K DC_5A-7A-7A_n78A-n257L DC_5A-7A-7A_n78A-n257M | DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_5A-7A-7A_n78C-n257A DC_5A-7A-7A_n78C-n257D DC_5A-7A-7A_n78C-n257E DC_5A-7A-7A_n78C-n257F DC_5A-7A-7A_n78C-n257G DC_5A-7A-7A_n78C-n257H DC_5A-7A-7A_n78C-n257I DC_5A-7A-7A_n78C-n257J DC_5A-7A-7A_n78C-n257K DC_5A-7A-7A_n78C-n257L DC_5A-7A-7A_n78C-n257M | DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_7A_n1A-n78A-n257A2 DC_7A_n1A-n78A-n257D2 DC_7A_n1A-n78A-n257E2 DC_7A_n1A-n78A-n257F2 DC_7A_n1A-n78A-n257G2 DC_7A_n1A-n78A-n257H2 DC_7A_n1A-n78A-n257I2 DC_7A_n1A-n78A-n257J2 DC_7A_n1A-n78A-n257K2 DC_7A_n1A-n78A-n257L2 DC_7A_n1A-n78A-n257M2 | DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_7A-7A_n1A-n78A-n257A2 DC_7A-7A_n1A-n78A-n257D2 DC_7A-7A_n1A-n78A-n257E2 DC_7A-7A_n1A-n78A-n257F2 DC_7A-7A_n1A-n78A-n257G2 DC_7A-7A_n1A-n78A-n257H2 DC_7A-7A_n1A-n78A-n257I2 DC_7A-7A_n1A-n78A-n257J2 DC_7A-7A_n1A-n78A-n257K2 DC_7A-7A_n1A-n78A-n257L2 DC_7A-7A_n1A-n78A-n257M2 | DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_7A-8A_n1A-n257A2 DC_7A-8A_n1A-n257D2 DC_7A-8A_n1A-n257E2 DC_7A-8A_n1A-n257F2 DC_7A-8A_n1A-n257G2 DC_7A-8A_n1A-n257H2 DC_7A-8A_n1A-n257I2 DC_7A-8A_n1A-n257J2 DC_7A-8A_n1A-n257K2 DC_7A-8A_n1A-n257L2 DC_7A-8A_n1A-n257M2 | DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_7A-7A-8A_n1A-n257A2 DC_7A-7A-8A_n1A-n257D2 DC_7A-7A-8A_n1A-n257E2 DC_7A-7A-8A_n1A-n257F2 DC_7A-7A-8A_n1A-n257G2 DC_7A-7A-8A_n1A-n257H2 DC_7A-7A-8A_n1A-n257I2 DC_7A-7A-8A_n1A-n257J2 DC_7A-7A-8A_n1A-n257K2 DC_7A-7A-8A_n1A-n257L2 DC_7A-7A-8A_n1A-n257M2 | DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_7A-8A_n40A-n258A DC_7A-8A_n40A-n258D DC_7A-8A_n40A-n258E DC_7A-8A_n40A-n258F DC_7A-8A_n40A-n258G DC_7A-8A_n40A-n258H DC_7A-8A_n40A-n258I DC_7A-8A_n40A-n258J DC_7A-8A_n40A-n258K DC_7A-8A_n40A-n258L DC_7A-8A_n40A-n258M | DC_7A_n40A DC_7A_n258A DC_8A_n40A DC_8A_n258A |
| DC_7A-8A_n78A-n257A2 DC_7A-8A_n78A-n257D2 DC_7A-8A_n78A-n257E2 DC_7A-8A_n78A-n257F2 DC_7A-8A_n78A-n257G2 DC_7A-8A_n78A-n257H2 DC_7A-8A_n78A-n257I2 DC_7A-8A_n78A-n257J2 DC_7A-8A_n78A-n257K2 DC_7A-8A_n78A-n257L2 DC_7A-8A_n78A-n257M2 | DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_7A-7A-8A_n78A-n257A2 DC_7A-7A-8A_n78A-n257D2 DC_7A-7A-8A_n78A-n257E2 DC_7A-7A-8A_n78A-n257F2 DC_7A-7A-8A_n78A-n257G2 DC_7A-7A-8A_n78A-n257H2 DC_7A-7A-8A_n78A-n257I2 DC_7A-7A-8A_n78A-n257J2 DC_7A-7A-8A_n78A-n257K2 DC_7A-7A-8A_n78A-n257L2 DC_7A-7A-8A_n78A-n257M2 | DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_7A-8A_n78A-n258A DC_7A-8A_n78A-n258D DC_7A-8A_n78A-n258E DC_7A-8A_n78A-n258F DC_7A-8A_n78A-n258G DC_7A-8A_n78A-n258H DC_7A-8A_n78A-n258I DC_7A-8A_n78A-n258J DC_7A-8A_n78A-n258K DC_7A-8A_n78A-n258L DC_7A-8A_n78A-n258M | DC_7A_n78A DC_7A_n258A DC_8A_n78A DC_8A_n258A |
| DC_7A-28A_n38A-n257A DC_7A-28A_n38A-n257G DC_7A-28A_n38A-n257H DC_7A-28A_n38A-n257I | DC_7A_n38A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_28A_n38A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_7A-28A_n78A-n257A DC_7A-28A_n78A-n257G DC_7A-28A_n78A-n257H DC_7A-28A_n78A-n257I | DC_7A_n78A DC_28A_n78A DC_7A_n257A DC_28A_n257A |
| DC_8A_n1A-n78A-n257A2 | DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_8A-11A_n77A-n257A2 DC_8A-11A_n77A-n257D2 DC_8A-11A_n77A-n257G2 DC_8A-11A_n77A-n257H2 DC_8A-11A_n77A-n257I2 | DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_8A-11A_n77(2A)-n257A2 DC_8A-11A_n77(2A)-n257D2 DC_8A-11A_n77(2A)-n257G2 DC_8A-11A_n77(2A)-n257H2 DC_8A-11A_n77(2A)-n257I2 | DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_18A-41A_n3A-n257A DC_18A-41A_n3A-n257G DC_18A-41A_n3A-n257H DC_18A-41A_n3A-n257I DC_18A-41C_n3A-n257A DC_18A-41C_n3A-n257G DC_18A-41C_n3A-n257H DC_18A-41C_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_18A-42A_n78A-n257A DC_18A-42A_n78A-n257G DC_18A-42A_n78A-n257H DC_18A-42A_n78A-n257I DC_18A-42C_n78A-n257A DC_18A-42C_n78A-n257G DC_18A-42C_n78A-n257H DC_18A-42C_n78A-n257I | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_19A-21A_n77A-n257A2 DC_19A-21A_n77A-n257G2 DC_19A-21A_n77A-n257H2 DC_19A-21A_n77A-n257I2 | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_19A-21A_n78A-n257A2 DC_19A-21A_n78A-n257G2 DC_19A-21A_n78A-n257H2 DC_19A-21A_n78A-n257I2 | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_19A-21A_n79A-n257A2 DC_19A-21A_n79A-n257G2 DC_19A-21A_n79A-n257H2 DC_19A-21A_n79A-n257I2 | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_19A-42A_n77A-n257A DC_19A-42A_n77A-n257G DC_19A-42A_n77A-n257H DC_19A-42A_n77A-n257I DC_19A-42C_n77A-n257A DC_19A-42C_n77A-n257G DC_19A-42C_n77A-n257H DC_19A-42C_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-42A_n78A-n257A DC_19A-42A_n78A-n257G DC_19A-42A_n78A-n257H DC_19A-42A_n78A-n257I DC_19A-42C_n78A-n257A DC_19A-42C_n78A-n257G DC_19A-42C_n78A-n257H DC_19A-42C_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-42A_n79A-n257A DC_19A-42A_n79A-n257G DC_19A-42A_n79A-n257H DC_19A-42A_n79A-n257I DC_19A-42C_n79A-n257A DC_19A-42C_n79A-n257G DC_19A-42C_n79A-n257H DC_19A-42C_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_21A-42A_n77A-n257A DC_21A-42A_n77A-n257G DC_21A-42A_n77A-n257H DC_21A-42A_n77A-n257I DC_21A-42C_n77A-n257A DC_21A-42C_n77A-n257G DC_21A-42C_n77A-n257H DC_21A-42C_n77A-n257I | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_21A-42A_n78A-n257A DC_21A-42A_n78A-n257G DC_21A-42A_n78A-n257H DC_21A-42A_n78A-n257I DC_21A-42C_n78A-n257A DC_21A-42C_n78A-n257G DC_21A-42C_n78A-n257H DC_21A-42C_n78A-n257I | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_21A-42A_n79A-n257A DC_21A-42A_n79A-n257G DC_21A-42A_n79A-n257H DC_21A-42A_n79A-n257I DC_21A-42C_n79A-n257A DC_21A-42C_n79A-n257G DC_21A-42C_n79A-n257H DC_21A-42C_n79A-n257I | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_28A-41A_n78A-n257A DC_28A-41A_n78A-n257G DC_28A-41A_n78A-n257H DC_28A-41A_n78A-n257I DC_28A-41C_n78A-n257A DC_28A-41C_n78A-n257G DC_28A-41C_n78A-n257H DC_28A-41C_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_28A-42A_n78A-n257A DC_28A-42A_n78A-n257G DC_28A-42A_n78A-n257H DC_28A-42A_n78A-n257I DC_28A-42C_n78A-n257A DC_28A-42C_n78A-n257G DC_28A-42C_n78A-n257H DC_28A-42C_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_40A-42A_n77A-n257A DC_40A-42A_n77A-n257D DC_40A-42A_n77A-n257E DC_40A-42A_n77A-n257F DC_40A-42A_n77A-n257G DC_40A-42A_n77A-n257H DC_40A-42A_n77A-n257I DC_40A-42A_n77A-n257J DC_40A-42A_n77A-n257K DC_40A-42A_n77A-n257L DC_40A-42A_n77A-n257M DC_40A-42A_n77C-n257A DC_40A-42A_n77C-n257D DC_40A-42A_n77C-n257E DC_40A-42A_n77C-n257F DC_40A-42A_n77C-n257G DC_40A-42A_n77C-n257H DC_40A-42A_n77C-n257I DC_40A-42A_n77C-n257J DC_40A-42A_n77C-n257K DC_40A-42A_n77C-n257L DC_40A-42A_n77C-n257M | DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_40A_n77A |
| DC_40A-42A_n78A-n257A DC_40A-42A_n78A-n257D DC_40A-42A_n78A-n257E DC_40A-42A_n78A-n257F DC_40A-42A_n78A-n257G DC_40A-42A_n78A-n257H DC_40A-42A_n78A-n257I DC_40A-42A_n78A-n257J DC_40A-42A_n78A-n257K DC_40A-42A_n78A-n257L DC_40A-42A_n78A-n257M | DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_40A_n78A |
| DC_41A-42A_n77A-n257A DC_41A-42A_n77A-n257G DC_41A-42A_n77A-n257H DC_41A-42A_n77A-n257I DC_41A-42C_n77A-n257A DC_41A-42C_n77A-n257G DC_41A-42C_n77A-n257H DC_41A-42C_n77A-n257I DC_41C-42A_n77A-n257A DC_41C-42A_n77A-n257G DC_41C-42A_n77A-n257H DC_41C-42A_n77A-n257I DC_41C-42C_n77A-n257A DC_41C-42C_n77A-n257G DC_41C-42C_n77A-n257H DC_41C-42C_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_41A-42A_n78A-n257A DC_41A-42A_n78A-n257G DC_41A-42A_n78A-n257H DC_41A-42A_n78A-n257I DC_41A-42C_n78A-n257A DC_41A-42C_n78A-n257G DC_41A-42C_n78A-n257H DC_41A-42C_n78A-n257I DC_41C-42A_n78A-n257A DC_41C-42A_n78A-n257G DC_41C-42A_n78A-n257H DC_41C-42A_n78A-n257I DC_41C-42C_n78A-n257A DC_41C-42C_n78A-n257G DC_41C-42C_n78A-n257H DC_41C-42C_n78A-n257I | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 3: Only single switched UL is supported. |  |

#### 5.5B.6.4	Inter-band EN-DC configurations including FR1 and FR2 (five bands)

Table 5.5B.6.4-1: Inter-band EN-DC configurations including FR1 and FR2 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A_n78A-n257A DC_1A-3A-5A_n78A-n257D DC_1A-3A-5A_n78A-n257E DC_1A-3A-5A_n78A-n257F DC_1A-3A-5A_n78A-n257G DC_1A-3A-5A_n78A-n257H DC_1A-3A-5A_n78A-n257I DC_1A-3A-5A_n78A-n257J DC_1A-3A-5A_n78A-n257K DC_1A-3A-5A_n78A-n257L DC_1A-3A-5A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_1A-3A-5A_n78C-n257A DC_1A-3A-5A_n78C-n257D DC_1A-3A-5A_n78C-n257E DC_1A-3A-5A_n78C-n257F DC_1A-3A-5A_n78C-n257G DC_1A-3A-5A_n78C-n257H DC_1A-3A-5A_n78C-n257I DC_1A-3A-5A_n78C-n257J DC_1A-3A-5A_n78C-n257K DC_1A-3A-5A_n78C-n257L DC_1A-3A-5A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_1A-3A-7A_n38A-n257A DC_1A-3A-7A_n38A-n257G DC_1A-3A-7A_n38A-n257H DC_1A-3A-7A_n38A-n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_1A-3A-7A_n78A-n257A DC_1A-3A-7A_n78A-n257D DC_1A-3A-7A_n78A-n257E DC_1A-3A-7A_n78A-n257F DC_1A-3A-7A_n78A-n257G DC_1A-3A-7A_n78A-n257H DC_1A-3A-7A_n78A-n257I DC_1A-3A-7A_n78A-n257J DC_1A-3A-7A_n78A-n257K DC_1A-3A-7A_n78A-n257L DC_1A-3A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-7A_n78C-n257A DC_1A-3A-7A_n78C-n257D DC_1A-3A-7A_n78C-n257E DC_1A-3A-7A_n78C-n257F DC_1A-3A-7A_n78C-n257G DC_1A-3A-7A_n78C-n257H DC_1A-3A-7A_n78C-n257I DC_1A-3A-7A_n78C-n257J DC_1A-3A-7A_n78C-n257K DC_1A-3A-7A_n78C-n257L DC_1A-3A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-7A-7A_n78A-n257A DC_1A-3A-7A-7A_n78A-n257D DC_1A-3A-7A-7A_n78A-n257E DC_1A-3A-7A-7A_n78A-n257F DC_1A-3A-7A-7A_n78A-n257G DC_1A-3A-7A-7A_n78A-n257H DC_1A-3A-7A-7A_n78A-n257I DC_1A-3A-7A-7A_n78A-n257J DC_1A-3A-7A-7A_n78A-n257K DC_1A-3A-7A-7A_n78A-n257L DC_1A-3A-7A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-7A-7A_n78C-n257A DC_1A-3A-7A-7A_n78C-n257D DC_1A-3A-7A-7A_n78C-n257E DC_1A-3A-7A-7A_n78C-n257F DC_1A-3A-7A-7A_n78C-n257G DC_1A-3A-7A-7A_n78C-n257H DC_1A-3A-7A-7A_n78C-n257I DC_1A-3A-7A-7A_n78C-n257J DC_1A-3A-7A-7A_n78C-n257K DC_1A-3A-7A-7A_n78C-n257L DC_1A-3A-7A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-7A_n78A-n258A DC_1A-3A-7A_n78A-n258D DC_1A-3A-7A_n78A-n258E DC_1A-3A-7A_n78A-n258F DC_1A-3A-7A_n78A-n258G DC_1A-3A-7A_n78A-n258H DC_1A-3A-7A_n78A-n258I DC_1A-3A-7A_n78A-n258J DC_1A-3A-7A_n78A-n258K DC_1A-3A-7A_n78A-n258L DC_1A-3A-7A_n78A-n258M | DC_1A_n78A DC_1A_n258A DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_3A_n78A DC_3A_n258A DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_7A_n78A DC_7A_n258A DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I |
| DC_1A_n3A-n8A-n77A-n257A DC_1A_n3A-n8A-n77A-n257G DC_1A_n3A-n8A-n77A-n257H DC_1A_n3A-n8A-n77A-n257I DC_1A_n3A-n8A-n77A-n257J DC_1A_n3A-n8A-n77A-n257K DC_1A_n3A-n8A-n77A-n257L DC_1A_n3A-n8A-n77A-n257M | DC_1A_n3A DC_1A_n8A DC_1A_n77A DC_1A_n257A |
| DC_1A_n3A-n8A-n77(2A)-n257A DC_1A_n3A-n8A-n77(2A)-n257G DC_1A_n3A-n8A-n77(2A)-n257H DC_1A_n3A-n8A-n77(2A)-n257I DC_1A_n3A-n8A-n77(2A)-n257J DC_1A_n3A-n8A-n77(2A)-n257K DC_1A_n3A-n8A-n77(2A)-n257L DC_1A_n3A-n8A-n77(2A)-n257M | DC_1A_n3A DC_1A_n8A DC_1A_n77A DC_1A_n257A |
| DC_1A-3A-8A_n78A-n257A2 DC_1A-3A-8A_n78A-n257D2 DC_1A-3A-8A_n78A-n257E2 DC_1A-3A-8A_n78A-n257F2 DC_1A-3A-8A_n78A-n257G2 DC_1A-3A-8A_n78A-n257H2 DC_1A-3A-8A_n78A-n257I2 DC_1A-3A-8A_n78A-n257J2 DC_1A-3A-8A_n78A-n257K2 DC_1A-3A-8A_n78A-n257L2 DC_1A-3A-8A_n78A-n257M2 DC_1A-3C-8A_n78A-n257A DC_1A-3C-8A_n78A-n257D DC_1A-3C-8A_n78A-n257E DC_1A-3C-8A_n78A-n257F DC_1A-3C-8A_n78A-n257G DC_1A-3C-8A_n78A-n257H DC_1A-3C-8A_n78A-n257I DC_1A-3C-8A_n78A-n257J DC_1A-3C-8A_n78A-n257K DC_1A-3C-8A_n78A-n257L DC_1A-3C-8A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_8A_n78A DC_8A_n257A |
| DC_1A-3A_n8A-n77A-n257A DC_1A-3A_n8A-n77A-n257G DC_1A-3A_n8A-n77A-n257H DC_1A-3A_n8A-n77A-n257I DC_1A-3A_n8A-n77A-n257J DC_1A-3A_n8A-n77A-n257K DC_1A-3A_n8A-n77A-n257L DC_1A-3A_n8A-n77A-n257M DC_1A-3A_n8A-n77(2A)-n257A DC_1A-3A_n8A-n77(2A)-n257G DC_1A-3A_n8A-n77(2A)-n257H DC_1A-3A_n8A-n77(2A)-n257I DC_1A-3A_n8A-n77(2A)-n257J DC_1A-3A_n8A-n77(2A)-n257K DC_1A-3A_n8A-n77(2A)-n257L DC_1A-3A_n8A-n77(2A)-n257M | DC_1A_n8A DC_1A_n77A DC_1A_n257A DC_3A_n8A DC_3A_n77A DC_3A_n257A |
| DC_1A-3A-18A_n78A-n257A DC_1A-3A-18A_n78A-n257G DC_1A-3A-18A_n78A-n257H DC_1A-3A-18A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-3A-21A_n77A-n257A2 DC_1A-3A-21A_n77A-n257G2 DC_1A-3A-21A_n77A-n257H2 DC_1A-3A-21A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_1A-3A-21A_n78A-n257A2 DC_1A-3A-21A_n78A-n257G2 DC_1A-3A-21A_n78A-n257H2 DC_1A-3A-21A_n78A-n257I2 | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_1A-3A-21A_n79A-n257A2 DC_1A-3A-21A_n79A-n257G2 DC_1A-3A-21A_n79A-n257H2 DC_1A-3A-21A_n79A-n257I2 | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_1A-3A-28A_n38A-n257A DC_1A-3A-28A_n38A-n257G DC_1A-3A-28A_n38A-n257H DC_1A-3A-28A_n38A-n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-3A-28A_n78A-n257A2 DC_1A-3A-28A_n78A-n257G2 DC_1A-3A-28A_n78A-n257H2 DC_1A-3A-28A_n78A-n257I2 | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-3A-41A_n28A-n257A2 DC_1A-3A-41A_n28A-n257G DC_1A-3A-41A_n28A-n257H DC_1A-3A-41A_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A DC_41A_n257I |
| DC_1A-3A-41C_n28A-n257A2 DC_1A-3A-41C_n28A-n257G DC_1A-3A-41C_n28A-n257H DC_1A-3A-41C_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257I |
| DC_1A-3A-41A_n77A-n257A DC_1A-3A-41A_n77A-n257G DC_1A-3A-41A_n77A-n257H DC_1A-3A-41A_n77A-n257I DC_1A-3A-41C_n77A-n257A DC_1A-3A-41C_n77A-n257G DC_1A-3A-41C_n77A-n257H DC_1A-3A-41C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-3A-41A_n78A-n257A DC_1A-3A-41A_n78A-n257G DC_1A-3A-41A_n78A-n257H DC_1A-3A-41A_n78A-n257I DC_1A-3A-41C_n78A-n257A DC_1A-3A-41C_n78A-n257G DC_1A-3A-41C_n78A-n257H DC_1A-3A-41C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-3A-42A_n77A-n257A DC_1A-3A-42A_n77A-n257G DC_1A-3A-42A_n77A-n257H DC_1A-3A-42A_n77A-n257I DC_1A-3A-42C_n77A-n257A DC_1A-3A-42C_n77A-n257G DC_1A-3A-42C_n77A-n257H DC_1A-3A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-42A_n78A-n257A DC_1A-3A-42A_n78A-n257G DC_1A-3A-42A_n78A-n257H DC_1A-3A-42A_n78A-n257I DC_1A-3A-42C_n78A-n257A DC_1A-3A-42C_n78A-n257G DC_1A-3A-42C_n78A-n257H DC_1A-3A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-5A-7A_n78A-n257A DC_1A-5A-7A_n78A-n257D DC_1A-5A-7A_n78A-n257E DC_1A-5A-7A_n78A-n257F DC_1A-5A-7A_n78A-n257G DC_1A-5A-7A_n78A-n257H DC_1A-5A-7A_n78A-n257I DC_1A-5A-7A_n78A-n257J DC_1A-5A-7A_n78A-n257K DC_1A-5A-7A_n78A-n257L DC_1A-5A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-5A-7A_n78C-n257A DC_1A-5A-7A_n78C-n257D DC_1A-5A-7A_n78C-n257E DC_1A-5A-7A_n78C-n257F DC_1A-5A-7A_n78C-n257G DC_1A-5A-7A_n78C-n257H DC_1A-5A-7A_n78C-n257I DC_1A-5A-7A_n78C-n257J DC_1A-5A-7A_n78C-n257K DC_1A-5A-7A_n78C-n257L DC_1A-5A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-5A-7A-7A_n78A-n257A DC_1A-5A-7A-7A_n78A-n257D DC_1A-5A-7A-7A_n78A-n257E DC_1A-5A-7A-7A_n78A-n257F DC_1A-5A-7A-7A_n78A-n257G DC_1A-5A-7A-7A_n78A-n257H DC_1A-5A-7A-7A_n78A-n257I DC_1A-5A-7A-7A_n78A-n257J DC_1A-5A-7A-7A_n78A-n257K DC_1A-5A-7A-7A_n78A-n257L DC_1A-5A-7A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-5A-7A-7A_n78C-n257A DC_1A-5A-7A-7A_n78C-n257D DC_1A-5A-7A-7A_n78C-n257E DC_1A-5A-7A-7A_n78C-n257F DC_1A-5A-7A-7A_n78C-n257G DC_1A-5A-7A-7A_n78C-n257H DC_1A-5A-7A-7A_n78C-n257I DC_1A-5A-7A-7A_n78C-n257J DC_1A-5A-7A-7A_n78C-n257K DC_1A-5A-7A-7A_n78C-n257L DC_1A-5A-7A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-7A-28A_n38A-n257A DC_1A-7A-28A_n38A-n257G DC_1A-7A-28A_n38A-n257H DC_1A-7A-28A_n38A-n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-7A-28A_n78A-n257A DC_1A-7A-28A_n78A-n257G DC_1A-7A-28A_n78A-n257H DC_1A-7A-28A_n78A-n257I | DC_1A_n78A DC_7A_n78A DC_28A_n78A DC_1A_n257A DC_7A_n257A DC_28A_n257A |
| DC_1A-8A-(n)3AA-n257A DC_1A-8A-(n)3AA-n257G DC_1A-8A-(n)3AA-n257H DC_1A-8A-(n)3AA-n257I DC_1A-8A-(n)3AA-n257J DC_1A-8A-(n)3AA-n257K DC_1A-8A-(n)3AA-n257L DC_1A-8A-(n)3AA-n257M | DC_1A_n3A DC_1A_n257A DC_(n)3AA 3 DC_3A_n257A DC_8A_n3A DC_8A_n257A |
| DC_1A-8A-11A_n77A-n257A2 DC_1A-8A-11A_n77A-n257D2 DC_1A-8A-11A_n77A-n257G2 DC_1A-8A-11A_n77A-n257H2 DC_1A-8A-11A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-8A-11A_n77(2A)-n257A2 DC_1A-8A-11A_n77(2A)-n257D2 DC_1A-8A-11A_n77(2A)-n257G2 DC_1A-8A-11A_n77(2A)-n257H2 DC_1A-8A-11A_n77(2A)-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-18A-41A_n3A-n257A DC_1A-18A-41A_n3A-n257G DC_1A-18A-41A_n3A-n257H DC_1A-18A-41A_n3A-n257I DC_1A-18A-41C_n3A-n257A DC_1A-18A-41C_n3A-n257G DC_1A-18A-41C_n3A-n257H DC_1A-18A-41C_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_41A_n3A DC_41C_n3A DC_41A_n257A DC_41C_n257A DC_18A_n257I DC_41A_n257I DC_41C_n257I |
| DC_1A-18A-42A_n78A-n257A DC_1A-18A-42A_n78A-n257G DC_1A-18A-42A_n78A-n257H DC_1A-18A-42A_n78A-n257I DC_1A-18A-42C_n78A-n257A DC_1A-18A-42C_n78A-n257G DC_1A-18A-42C_n78A-n257H DC_1A-18A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-19A-42A_n77A-n257A DC_1A-19A-42A_n77A-n257G DC_1A-19A-42A_n77A-n257H DC_1A-19A-42A_n77A-n257I DC_1A-19A-42C_n77A-n257A DC_1A-19A-42C_n77A-n257G DC_1A-19A-42C_n77A-n257H DC_1A-19A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-19A-42A_n78A-n257A DC_1A-19A-42A_n78A-n257G DC_1A-19A-42A_n78A-n257H DC_1A-19A-42A_n78A-n257I DC_1A-19A-42C_n78A-n257A DC_1A-19A-42C_n78A-n257G DC_1A-19A-42C_n78A-n257H DC_1A-19A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-19A-42A_n79A-n257A DC_1A-19A-42A_n79A-n257G DC_1A-19A-42A_n79A-n257H DC_1A-19A-42A_n79A-n257I DC_1A-19A-42C_n79A-n257A DC_1A-19A-42C_n79A-n257G DC_1A-19A-42C_n79A-n257H DC_1A-19A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_1A-21A-42A_n77A-n257A DC_1A-21A-42A_n77A-n257G DC_1A-21A-42A_n77A-n257H DC_1A-21A-42A_n77A-n257I DC_1A-21A-42C_n77A-n257A DC_1A-21A-42C_n77A-n257G DC_1A-21A-42C_n77A-n257H DC_1A-21A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_1A-21A-42A_n78A-n257A DC_1A-21A-42A_n78A-n257G DC_1A-21A-42A_n78A-n257H DC_1A-21A-42A_n78A-n257I DC_1A-21A-42C_n78A-n257A DC_1A-21A-42C_n78A-n257G DC_1A-21A-42C_n78A-n257H DC_1A-21A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_1A-21A-42A_n79A-n257A DC_1A-21A-42A_n79A-n257G DC_1A-21A-42A_n79A-n257H DC_1A-21A-42A_n79A-n257I DC_1A-21A-42C_n79A-n257A DC_1A-21A-42C_n79A-n257G DC_1A-21A-42C_n79A-n257H DC_1A-21A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_1A-28A-42A_n78A-n257A DC_1A-28A-42A_n78A-n257G DC_1A-28A-42A_n78A-n257H DC_1A-28A-42A_n78A-n257I DC_1A-28A-42C_n78A-n257A DC_1A-28A-42C_n78A-n257G DC_1A-28A-42C_n78A-n257H DC_1A-28A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-41A-42A_n77A-n257A DC_1A-41A-42A_n77A-n257G DC_1A-41A-42A_n77A-n257H DC_1A-41A-42A_n77A-n257I DC_1A-41A-42C_n77A-n257A DC_1A-41A-42C_n77A-n257G DC_1A-41A-42C_n77A-n257H DC_1A-41A-42C_n77A-n257I DC_1A-41C-42A_n77A-n257A DC_1A-41C-42A_n77A-n257G DC_1A-41C-42A_n77A-n257H DC_1A-41C-42A_n77A-n257I DC_1A-41C-42C_n77A-n257A DC_1A-41C-42C_n77A-n257G DC_1A-41C-42C_n77A-n257H DC_1A-41C-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-41A-42A_n78A-n257A DC_1A-41A-42A_n78A-n257G DC_1A-41A-42A_n78A-n257H DC_1A-41A-42A_n78A-n257I DC_1A-41A-42C_n78A-n257A DC_1A-41A-42C_n78A-n257G DC_1A-41A-42C_n78A-n257H DC_1A-41A-42C_n78A-n257I DC_1A-41C-42A_n78A-n257A DC_1A-41C-42A_n78A-n257G DC_1A-41C-42A_n78A-n257H DC_1A-41C-42A_n78A-n257I DC_1A-41C-42C_n78A-n257A DC_1A-41C-42C_n78A-n257G DC_1A-41C-42C_n78A-n257H DC_1A-41C-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-5A-7A-7A_n78A-n257A DC_3A-5A-7A-7A_n78A-n257D DC_3A-5A-7A-7A_n78A-n257E DC_3A-5A-7A-7A_n78A-n257F DC_3A-5A-7A-7A_n78A-n257G DC_3A-5A-7A-7A_n78A-n257H DC_3A-5A-7A-7A_n78A-n257I DC_3A-5A-7A-7A_n78A-n257J DC_3A-5A-7A-7A_n78A-n257K DC_3A-5A-7A-7A_n78A-n257L DC_3A-5A-7A-7A_n78A-n257M DC_3A-5A-7A-7A_n78C-n257A DC_3A-5A-7A-7A_n78C-n257D DC_3A-5A-7A-7A_n78C-n257E DC_3A-5A-7A-7A_n78C-n257F DC_3A-5A-7A-7A_n78C-n257G DC_3A-5A-7A-7A_n78C-n257H DC_3A-5A-7A-7A_n78C-n257I DC_3A-5A-7A-7A_n78C-n257J DC_3A-5A-7A-7A_n78C-n257K DC_3A-5A-7A-7A_n78C-n257L DC_3A-5A-7A-7A_n78C-n257M DC_3A-5A-7A_n78A-n257A DC_3A-5A-7A_n78A-n257D DC_3A-5A-7A_n78A-n257E DC_3A-5A-7A_n78A-n257F DC_3A-5A-7A_n78A-n257G DC_3A-5A-7A_n78A-n257H DC_3A-5A-7A_n78A-n257I DC_3A-5A-7A_n78A-n257J DC_3A-5A-7A_n78A-n257K DC_3A-5A-7A_n78A-n257L DC_3A-5A-7A_n78A-n257M DC_3A-5A-7A_n78C-n257A DC_3A-5A-7A_n78C-n257D DC_3A-5A-7A_n78C-n257E DC_3A-5A-7A_n78C-n257F DC_3A-5A-7A_n78C-n257G DC_3A-5A-7A_n78C-n257H DC_3A-5A-7A_n78C-n257I DC_3A-5A-7A_n78C-n257J DC_3A-5A-7A_n78C-n257K DC_3A-5A-7A_n78C-n257L DC_3A-5A-7A_n78C-n257M | DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_3A-7A_n1A-n78A-n257A2 DC_3A-7A_n1A-n78A-n257D2 DC_3A-7A_n1A-n78A-n257E2 DC_3A-7A_n1A-n78A-n257F2 DC_3A-7A_n1A-n78A-n257G2 DC_3A-7A_n1A-n78A-n257H2 DC_3A-7A_n1A-n78A-n257I2 DC_3A-7A_n1A-n78A-n257J2 DC_3A-7A_n1A-n78A-n257K2 DC_3A-7A_n1A-n78A-n257L2 DC_3A-7A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-3A-7A_n1A-n78A-n257A2 DC_3A-3A-7A_n1A-n78A-n257D2 DC_3A-3A-7A_n1A-n78A-n257E2 DC_3A-3A-7A_n1A-n78A-n257F2 DC_3A-3A-7A_n1A-n78A-n257G2 DC_3A-3A-7A_n1A-n78A-n257H2 DC_3A-3A-7A_n1A-n78A-n257I2 DC_3A-3A-7A_n1A-n78A-n257J2 DC_3A-3A-7A_n1A-n78A-n257K2 DC_3A-3A-7A_n1A-n78A-n257L2 DC_3A-3A-7A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-7A-7A_n1A-n78A-n257A2 DC_3A-7A-7A_n1A-n78A-n257D2 DC_3A-7A-7A_n1A-n78A-n257E2 DC_3A-7A-7A_n1A-n78A-n257F2 DC_3A-7A-7A_n1A-n78A-n257G2 DC_3A-7A-7A_n1A-n78A-n257H2 DC_3A-7A-7A_n1A-n78A-n257I2 DC_3A-7A-7A_n1A-n78A-n257J2 DC_3A-7A-7A_n1A-n78A-n257K2 DC_3A-7A-7A_n1A-n78A-n257L2 DC_3A-7A-7A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-3A-7A-7A_n1A-n78A-n257A2 DC_3A-3A-7A-7A_n1A-n78A-n257D2 DC_3A-3A-7A-7A_n1A-n78A-n257E2 DC_3A-3A-7A-7A_n1A-n78A-n257F2 DC_3A-3A-7A-7A_n1A-n78A-n257G2 DC_3A-3A-7A-7A_n1A-n78A-n257H2 DC_3A-3A-7A-7A_n1A-n78A-n257I2 DC_3A-3A-7A-7A_n1A-n78A-n257J2 DC_3A-3A-7A-7A_n1A-n78A-n257K2 DC_3A-3A-7A-7A_n1A-n78A-n257L2 DC_3A-3A-7A-7A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-7A-8A_n1A-n257A2 DC_3A-7A-8A_n1A-n257D2 DC_3A-7A-8A_n1A-n257E2 DC_3A-7A-8A_n1A-n257F2 DC_3A-7A-8A_n1A-n257G2 DC_3A-7A-8A_n1A-n257H2 DC_3A-7A-8A_n1A-n257I2 DC_3A-7A-8A_n1A-n257J2 DC_3A-7A-8A_n1A-n257K2 DC_3A-7A-8A_n1A-n257L2 DC_3A-7A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-7A-8A_n1A-n257A2 DC_3A-3A-7A-8A_n1A-n257D2 DC_3A-3A-7A-8A_n1A-n257E2 DC_3A-3A-7A-8A_n1A-n257F2 DC_3A-3A-7A-8A_n1A-n257G2 DC_3A-3A-7A-8A_n1A-n257H2 DC_3A-3A-7A-8A_n1A-n257I2 DC_3A-3A-7A-8A_n1A-n257J2 DC_3A-3A-7A-8A_n1A-n257K2 DC_3A-3A-7A-8A_n1A-n257L2 DC_3A-3A-7A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-7A-7A-8A_n1A-n257A2 DC_3A-7A-7A-8A_n1A-n257D2 DC_3A-7A-7A-8A_n1A-n257E2 DC_3A-7A-7A-8A_n1A-n257F2 DC_3A-7A-7A-8A_n1A-n257G2 DC_3A-7A-7A-8A_n1A-n257H2 DC_3A-7A-7A-8A_n1A-n257I2 DC_3A-7A-7A-8A_n1A-n257J2 DC_3A-7A-7A-8A_n1A-n257K2 DC_3A-7A-7A-8A_n1A-n257L2 DC_3A-7A-7A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-7A-7A-8A_n1A-n257A2 DC_3A-3A-7A-7A-8A_n1A-n257D2 DC_3A-3A-7A-7A-8A_n1A-n257E2 DC_3A-3A-7A-7A-8A_n1A-n257F2 DC_3A-3A-7A-7A-8A_n1A-n257G2 DC_3A-3A-7A-7A-8A_n1A-n257H2 DC_3A-3A-7A-7A-8A_n1A-n257I2 DC_3A-3A-7A-7A-8A_n1A-n257J2 DC_3A-3A-7A-7A-8A_n1A-n257K2 DC_3A-3A-7A-7A-8A_n1A-n257L2 DC_3A-3A-7A-7A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-7A-8A_n40A-n258A DC_3A-7A-8A_n40A-n258D DC_3A-7A-8A_n40A-n258E DC_3A-7A-8A_n40A-n258F DC_3A-7A-8A_n40A-n258G DC_3A-7A-8A_n40A-n258H DC_3A-7A-8A_n40A-n258I DC_3A-7A-8A_n40A-n258J DC_3A-7A-8A_n40A-n258K DC_3A-7A-8A_n40A-n258L DC_3A-7A-8A_n40A-n258M | DC_3A_n40A DC_3A_n258A DC_7A_n40A DC_7A_n258A DC_8A_n40A DC_8A_n258A |
| DC_3A-7A-8A_n78A-n257A2 DC_3A-7A-8A_n78A-n257D2 DC_3A-7A-8A_n78A-n257E2 DC_3A-7A-8A_n78A-n257F2 DC_3A-7A-8A_n78A-n257G2 DC_3A-7A-8A_n78A-n257H2 DC_3A-7A-8A_n78A-n257I2 DC_3A-7A-8A_n78A-n257J2 DC_3A-7A-8A_n78A-n257K2 DC_3A-7A-8A_n78A-n257L2 DC_3A-7A-8A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-7A-8A_n78A-n257A2 DC_3A-3A-7A-8A_n78A-n257D2 DC_3A-3A-7A-8A_n78A-n257E2 DC_3A-3A-7A-8A_n78A-n257F2 DC_3A-3A-7A-8A_n78A-n257G2 DC_3A-3A-7A-8A_n78A-n257H2 DC_3A-3A-7A-8A_n78A-n257I2 DC_3A-3A-7A-8A_n78A-n257J2 DC_3A-3A-7A-8A_n78A-n257K2 DC_3A-3A-7A-8A_n78A-n257L2 DC_3A-3A-7A-8A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-7A-7A-8A_n78A-n257A2 DC_3A-7A-7A-8A_n78A-n257D2 DC_3A-7A-7A-8A_n78A-n257E2 DC_3A-7A-7A-8A_n78A-n257F2 DC_3A-7A-7A-8A_n78A-n257G2 DC_3A-7A-7A-8A_n78A-n257H2 DC_3A-7A-7A-8A_n78A-n257I2 DC_3A-7A-7A-8A_n78A-n257J2 DC_3A-7A-7A-8A_n78A-n257K2 DC_3A-7A-7A-8A_n78A-n257L2 DC_3A-7A-7A-8A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-7A-7A-8A_n78A-n257A2 DC_3A-3A-7A-7A-8A_n78A-n257D2 DC_3A-3A-7A-7A-8A_n78A-n257E2 DC_3A-3A-7A-7A-8A_n78A-n257F2 DC_3A-3A-7A-7A-8A_n78A-n257G2 DC_3A-3A-7A-7A-8A_n78A-n257H2 DC_3A-3A-7A-7A-8A_n78A-n257I2 DC_3A-3A-7A-7A-8A_n78A-n257J2 DC_3A-3A-7A-7A-8A_n78A-n257K2 DC_3A-3A-7A-7A-8A_n78A-n257L2 DC_3A-3A-7A-7A-8A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-7A-8A_n78A-n258A DC_3A-7A-8A_n78A-n258D DC_3A-7A-8A_n78A-n258E DC_3A-7A-8A_n78A-n258F DC_3A-7A-8A_n78A-n258G DC_3A-7A-8A_n78A-n258H DC_3A-7A-8A_n78A-n258I DC_3A-7A-8A_n78A-n258J DC_3A-7A-8A_n78A-n258K DC_3A-7A-8A_n78A-n258L DC_3A-7A-8A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_7A_n78A DC_7A_n258A DC_8A_n78A DC_8A_n258A |
| DC_3A-7A-28A_n38A-n257A DC_3A-7A-28A_n38A-n257G DC_3A-7A-28A_n38A-n257H DC_3A-7A-28A_n38A-n257I | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-7A-28A_n78A-n257A DC_3A-7A-28A_n78A-n257G DC_3A-7A-28A_n78A-n257H DC_3A-7A-28A_n78A-n257I | DC_3A_n78A DC_7A_n78A DC_28A_n78A DC_3A_n257A DC_7A_n257A DC_28A_n257A |
| DC_3A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-3A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-18A-42A_n78A-n257A DC_3A-18A-42A_n78A-n257G DC_3A-18A-42A_n78A-n257H DC_3A-18A-42A_n78A-n257I DC_3A-18A-42C_n78A-n257A DC_3A-18A-42C_n78A-n257G DC_3A-18A-42C_n78A-n257H DC_3A-18A-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-41A-42A_n77A-n257A DC_3A-41A-42A_n77A-n257G DC_3A-41A-42A_n77A-n257H DC_3A-41A-42A_n77A-n257I DC_3A-41A-42C_n77A-n257A DC_3A-41A-42C_n77A-n257G DC_3A-41A-42C_n77A-n257H DC_3A-41A-42C_n77A-n257I DC_3A-41C-42A_n77A-n257A DC_3A-41C-42A_n77A-n257G DC_3A-41C-42A_n77A-n257H DC_3A-41C-42A_n77A-n257I DC_3A-41C-42C_n77A-n257A DC_3A-41C-42C_n77A-n257G DC_3A-41C-42C_n77A-n257H DC_3A-41C-42C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-28A-41A_n78A-n257A DC_3A-28A-41A_n78A-n257G DC_3A-28A-41A_n78A-n257H DC_3A-28A-41A_n78A-n257I DC_3A-28A-41C_n78A-n257A DC_3A-28A-41C_n78A-n257G DC_3A-28A-41C_n78A-n257H DC_3A-28A-41C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-28A-42A_n78A-n257A DC_3A-28A-42A_n78A-n257G DC_3A-28A-42A_n78A-n257H DC_3A-28A-42A_n78A-n257I DC_3A-28A-42C_n78A-n257A DC_3A-28A-42C_n78A-n257G DC_3A-28A-42C_n78A-n257H DC_3A-28A-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-41A-42A_n78A-n257A DC_3A-41A-42A_n78A-n257G DC_3A-41A-42A_n78A-n257H DC_3A-41A-42A_n78A-n257I DC_3A-41A-42C_n78A-n257A DC_3A-41A-42C_n78A-n257G DC_3A-41A-42C_n78A-n257H DC_3A-41A-42C_n78A-n257I DC_3A-41C-42A_n78A-n257A DC_3A-41C-42A_n78A-n257G DC_3A-41C-42A_n78A-n257H DC_3A-41C-42A_n78A-n257I DC_3A-41C-42C_n78A-n257A DC_3A-41C-42C_n78A-n257G DC_3A-41C-42C_n78A-n257H DC_3A-41C-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_7A-8A_n1A-n78A-n257A2 | DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_7A-7A-8A_n1A-n78A-n257A2 | DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_19A-21A-42A_n77A-n257A DC_19A-21A-42A_n77A-n257G DC_19A-21A-42A_n77A-n257H DC_19A-21A-42A_n77A-n257I DC_19A-21A-42C_n77A-n257A DC_19A-21A-42C_n77A-n257G DC_19A-21A-42C_n77A-n257H DC_19A-21A-42C_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-21A-42A_n78A-n257A DC_19A-21A-42A_n78A-n257G DC_19A-21A-42A_n78A-n257H DC_19A-21A-42A_n78A-n257I DC_19A-21A-42C_n78A-n257A DC_19A-21A-42C_n78A-n257G DC_19A-21A-42C_n78A-n257H DC_19A-21A-42C_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-21A-42A_n79A-n257A DC_19A-21A-42A_n79A-n257G DC_19A-21A-42A_n79A-n257H DC_19A-21A-42A_n79A-n257I DC_19A-21A-42C_n79A-n257A DC_19A-21A-42C_n79A-n257G DC_19A-21A-42C_n79A-n257H DC_19A-21A-42C_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_28A-41A-42A_n78A-n257A DC_28A-41A-42A_n78A-n257G DC_28A-41A-42A_n78A-n257H DC_28A-41A-42A_n78A-n257I DC_28A-41A-42C_n78A-n257A DC_28A-41A-42C_n78A-n257G DC_28A-41A-42C_n78A-n257H DC_28A-41A-42C_n78A-n257I DC_28A-41C-42A_n78A-n257A DC_28A-41C-42A_n78A-n257G DC_28A-41C-42A_n78A-n257H DC_28A-41C-42A_n78A-n257I DC_28A-41C-42C_n78A-n257A DC_28A-41C-42C_n78A-n257G DC_28A-41C-42C_n78A-n257H DC_28A-41C-42C_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 3: Only single switched UL is supported. |  |

#### 5.5B.6.5	Inter-band EN-DC configurations including FR1 and FR2 (six bands)

Table 5.5B.6.5-1: Inter-band EN-DC configurations including FR1 and FR2 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A-7A_n78A-n257A DC_1A-3A-5A-7A-7A_n78A-n257D DC_1A-3A-5A-7A-7A_n78A-n257E DC_1A-3A-5A-7A-7A_n78A-n257F DC_1A-3A-5A-7A-7A_n78A-n257G DC_1A-3A-5A-7A-7A_n78A-n257H DC_1A-3A-5A-7A-7A_n78A-n257I DC_1A-3A-5A-7A-7A_n78A-n257J DC_1A-3A-5A-7A-7A_n78A-n257K DC_1A-3A-5A-7A-7A_n78A-n257L DC_1A-3A-5A-7A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-5A-7A-7A_n78C-n257A DC_1A-3A-5A-7A-7A_n78C-n257D DC_1A-3A-5A-7A-7A_n78C-n257E DC_1A-3A-5A-7A-7A_n78C-n257F DC_1A-3A-5A-7A-7A_n78C-n257G DC_1A-3A-5A-7A-7A_n78C-n257H DC_1A-3A-5A-7A-7A_n78C-n257I DC_1A-3A-5A-7A-7A_n78C-n257J DC_1A-3A-5A-7A-7A_n78C-n257K DC_1A-3A-5A-7A-7A_n78C-n257L DC_1A-3A-5A-7A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-5A-7A_n78A-n257A DC_1A-3A-5A-7A_n78A-n257D DC_1A-3A-5A-7A_n78A-n257E DC_1A-3A-5A-7A_n78A-n257F DC_1A-3A-5A-7A_n78A-n257G DC_1A-3A-5A-7A_n78A-n257H DC_1A-3A-5A-7A_n78A-n257I DC_1A-3A-5A-7A_n78A-n257J DC_1A-3A-5A-7A_n78A-n257K DC_1A-3A-5A-7A_n78A-n257L DC_1A-3A-5A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-5A-7A_n78C-n257A DC_1A-3A-5A-7A_n78C-n257D DC_1A-3A-5A-7A_n78C-n257E DC_1A-3A-5A-7A_n78C-n257F DC_1A-3A-5A-7A_n78C-n257G DC_1A-3A-5A-7A_n78C-n257H DC_1A-3A-5A-7A_n78C-n257I DC_1A-3A-5A-7A_n78C-n257J DC_1A-3A-5A-7A_n78C-n257K DC_1A-3A-5A-7A_n78C-n257L DC_1A-3A-5A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-7A-28A_n38A-n257A DC_1A-3A-7A-28A_n38A-n257G DC_1A-3A-7A-28A_n38A-n257H DC_1A-3A-7A-28A_n38A-n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-3A-7A-28A_n78A-n257A DC_1A-3A-7A-28A_n78A-n257G DC_1A-3A-7A-28A_n78A-n257H DC_1A-3A-7A-28A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-3A-18A-42A_n78A-n257A DC_1A-3A-18A-42A_n78A-n257G DC_1A-3A-18A-42A_n78A-n257H DC_1A-3A-18A-42A_n78A-n257I DC_1A-3A-18A-42C_n78A-n257A DC_1A-3A-18A-42C_n78A-n257G DC_1A-3A-18A-42C_n78A-n257H DC_1A-3A-18A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-28A-42A_n78A-n257A DC_1A-3A-28A-42A_n78A-n257G DC_1A-3A-28A-42A_n78A-n257H DC_1A-3A-28A-42A_n78A-n257I DC_1A-3A-28A-42C_n78A-n257A DC_1A-3A-28A-42C_n78A-n257G DC_1A-3A-28A-42C_n78A-n257H DC_1A-3A-28A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-41A-42A_n77A-n257A DC_1A-3A-41A-42A_n77A-n257G DC_1A-3A-41A-42A_n77A-n257H DC_1A-3A-41A-42A_n77A-n257I DC_1A-3A-41C-42A_n77A-n257A DC_1A-3A-41C-42A_n77A-n257G DC_1A-3A-41C-42A_n77A-n257H DC_1A-3A-41C-42A_n77A-n257I DC_1A-3A-41A-42C_n77A-n257A DC_1A-3A-41A-42C_n77A-n257G DC_1A-3A-41A-42C_n77A-n257H DC_1A-3A-41A-42C_n77A-n257I DC_1A-3A-41C-42C_n77A-n257A DC_1A-3A-41C-42C_n77A-n257G DC_1A-3A-41C-42C_n77A-n257H DC_1A-3A-41C-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-41A-42A_n78A-n257A DC_1A-3A-41A-42A_n78A-n257G DC_1A-3A-41A-42A_n78A-n257H DC_1A-3A-41A-42A_n78A-n257I DC_1A-3A-41A-42C_n78A-n257A DC_1A-3A-41A-42C_n78A-n257G DC_1A-3A-41A-42C_n78A-n257H DC_1A-3A-41A-42C_n78A-n257I DC_1A-3A-41C-42A_n78A-n257A DC_1A-3A-41C-42A_n78A-n257G DC_1A-3A-41C-42A_n78A-n257H DC_1A-3A-41C-42A_n78A-n257I DC_1A-3A-41C-42C_n78A-n257A DC_1A-3A-41C-42C_n78A-n257G DC_1A-3A-41C-42C_n78A-n257H DC_1A-3A-41C-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-8A-(n)3AA-n77A-n257A DC_1A-8A-(n)3AA-n77A-n257G DC_1A-8A-(n)3AA-n77A-n257H DC_1A-8A-(n)3AA-n77A-n257I DC_1A-8A-(n)3AA-n77A-n257J DC_1A-8A-(n)3AA-n77A-n257K DC_1A-8A-(n)3AA-n77A-n257L DC_1A-8A-(n)3AA-n77A-n257M DC_1A-8A-(n)3AA-n77(2A)-n257A DC_1A-8A-(n)3AA-n77(2A)-n257G DC_1A-8A-(n)3AA-n77(2A)-n257H DC_1A-8A-(n)3AA-n77(2A)-n257I DC_1A-8A-(n)3AA-n77(2A)-n257J DC_1A-8A-(n)3AA-n77(2A)-n257K DC_1A-8A-(n)3AA-n77(2A)-n257L DC_1A-8A-(n)3AA-n77(2A)-n257M | DC_1A_n3A DC_1A_n77A DC_1A_n257A DC_(n)3AA3 DC_3A_n77A DC_3A_n257A DC_8A_n3A DC_8A_n77A DC_8A_n257A |
| DC_3A-7A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-3A-7A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-7A-7A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-3A-7A-7A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-28A-41A-42A_n78A-n257A DC_3A-28A-41A-42A_n78A-n257G DC_3A-28A-41A-42A_n78A-n257H DC_3A-28A-41A-42A_n78A-n257I DC_3A-28A-41A-42C_n78A-n257A DC_3A-28A-41A-42C_n78A-n257G DC_3A-28A-41A-42C_n78A-n257H DC_3A-28A-41A-42C_n78A-n257I DC_3A-28A-41C-42A_n78A-n257A DC_3A-28A-41C-42A_n78A-n257G DC_3A-28A-41C-42A_n78A-n257H DC_3A-28A-41C-42A_n78A-n257I DC_3A-28A-41C-42C_n78A-n257A DC_3A-28A-41C-42C_n78A-n257G DC_3A-28A-41C-42C_n78A-n257H DC_3A-28A-41C-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 3: Only single switched UL is supported. |  |

### 5.5B.7	Inter-band NR-DC between FR1 and FR2

#### 5.5B.7.0	General

The configurations and bandwidth combination sets for the FR1-FR2 NR-DC combinations in the following sub-sections are defined in the tables for FR1-FR2 carrier aggregation in section 5.5A.1.

#### 5.5B.7.1	Inter-band NR-DC configurations between FR1 and FR2 (two bands)

Table 5.5B.7-1: Inter-band NR-DC configurations between FR1 and FR2 (two bands)

| Downlink NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n1A-n257A DC_n1A-n257D DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n1A-n257J DC_n1A-n257K DC_n1A-n257L DC_n1A-n257M | DC_n1A-n257A DC_n1A-n257D DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n1A-n257J DC_n1A-n257K |
| DC_n1A-n3A-n258A DC_n1A-n3A-n258D DC_n1A-n3A-n258G DC_n1A-n3A-n258H DC_n1A-n3A-n258I DC_n1A-n3A-n258J | DC_n1A-n3A DC_n1A-n258A DC_n1A-n258D DC_n1A-n258G DC_n1A-n258H DC_n1A-n258I DC_n1A-n258J DC_n3A-n258A DC_n3A-n258D DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n3A-n258J |
| DC_n1A-n258A DC_n1A-n258B DC_n1A-n258C DC_n1A-n258D DC_n1A-n258E DC_n1A-n258F DC_n1A-n258G DC_n1A-n258H DC_n1A-n258I DC_n1A-n258J DC_n1A-n258R2 DC_n1A-n258R3 DC_n1A-n258R4 DC_n1A-n258R5 DC_n1A-n258R6 DC_n1A-n258R7 DC_n1A-n258R8 DC_n1A-n258R9 DC_n1A-n258R10 | DC_n1A-n258A DC_n1A-n258G DC_n1A-n258H DC_n1A-n258I DC_n1A-n258R2 DC_n1A-n258R3 DC_n1A-n258R4 |
| DC_n1A-n258K DC_n1A-n258L DC_n1A-n258M | DC_n1A-n258A |
| DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n2A-n260J DC_n2A-n260K DC_n2A-n260L DC_n2A-n260M DC_n2A-n260R2 DC_n2A-n260R3 DC_n2A-n260R4 DC_n2A-n260R5 DC_n2A-n260R6 DC_n2A-n260R7 DC_n2A-n260R8 DC_n2A-n260R9 DC_n2A-n260R10 | DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n2A-n260J DC_n2A-n260K DC_n2A-n260L DC_n2A-n260M DC_n2A-n260R2 DC_n2A-n260R3 DC_n2A-n260R4 |
| DC_n1A-n28A-n258A DC_n1A-n28A-n258D DC_n1A-n28A-n258G DC_n1A-n28A-n258H DC_n1A-n28A-n258I DC_n1A-n28A-n258J | DC_n1A-n28A DC_n1A-n258A DC_n1A-n258D DC_n1A-n258G DC_n1A-n258H DC_n1A-n258I DC_n1A-n258J DC_n28A-n258A DC_n28A-n258D DC_n28A-n258G DC_n28A-n258H DC_n28A-n258I DC_n28A-n258J |
| DC_n2(2A)-n260A DC_n2(2A)-n260G DC_n2(2A)-n260H DC_n2(2A)-n260I DC_n2(2A)-n260J DC_n2(2A)-n260K DC_n2(2A)-n260L DC_n2(2A)-n260M | DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n2A-n260J DC_n2A-n260K DC_n2A-n260L DC_n2A-n260M |
| DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n2A-n261J DC_n2A-n261K DC_n2A-n261L DC_n2A-n261M | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I |
| DC_n2A-n261(2A) DC_n2A-n261(3A) DC_n2A-n261(4A) DC_n2A-n261(2G) DC_n2A-n261(2H) DC_n2A-n261(2I) DC_n2A-n261(A-G) DC_n2A-n261(A-H) DC_n2A-n261(A-I) DC_n2A-n261(A-J) DC_n2A-n261(A-K) DC_n2A-n261(A-L) DC_n2A-n261(G-H) DC_n2A-n261(H-I) DC_n2A-n261(G-I) DC_n2A-n261(A-G-H) DC_n2A-n261(A-G-I) DC_n2A-n261(2A-H) DC_n2A-n261(2A-G) DC_n2A-n261(2A-I) DC_n2A-n261(A-2G) | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I |
| DC_n3A-n257A1 DC_n3A-n257D1 DC_n3A-n257G1 DC_n3A-n257H1 DC_n3A-n257I1 | DC_n3A-n257A DC_n3A-n257D DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I |
| DC_n3A-n257(2A) DC_n3A-n257(A-G) DC_n3A-n257(2G) DC_n3(2A)-n257A DC_n3(2A)-n257G DC_n3(2A)-n257H DC_n3(2A)-n257I | DC_n3A-n257A DC_n3A-n257G DC_n3A-n257I DC_n3A-n257H DC_n3A-n257(2A) DC_n3A-n257(2G) |
| DC_n3A-n258A DC_n3A-n258B DC_n3A-n258C DC_n3A-n258D DC_n3A-n258E DC_n3A-n258F DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n3A-n258J DC_n3A-n258R2 DC_n3A-n258R3 DC_n3A-n258R4 DC_n3A-n258R5 DC_n3A-n258R6 DC_n3A-n258R7 DC_n3A-n258R8 DC_n3A-n258R9 DC_n3A-n258R10 DC_n3B-n258A DC_n3B-n258B DC_n3B-n258C DC_n3B-n258D DC_n3B-n258E DC_n3B-n258F DC_n3B-n258G DC_n3B-n258H DC_n3B-n258I DC_n3B-n258J DC_n3B-n258K DC_n3B-n258L DC_n3B-n258M DC_n3B-n258R2 DC_n3B-n258R3 DC_n3B-n258R4 DC_n3B-n258R5 DC_n3B-n258R6 DC_n3B-n258R7 DC_n3B-n258R8 DC_n3B-n258R9 DC_n3B-n258R10 | DC_n3A-n258A DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n3A-n258R2 DC_n3A-n258R3 DC_n3A-n258R4 DC_n3B-n258A DC_n3B-n258G DC_n3B-n258H DC_n3B-n258I DC_n3B-n258R2 DC_n3B-n258R3 DC_n3B-n258R4 |
| DC_n3A-n258K DC_n3A-n258L DC_n3A-n258M | DC_n3A-n258A |
| DC_n3A-n258(2A) | DC_n3A-n258A DC_n3A-n258(2A) |
| DC_n5A-n258A DC_n5A-n258B DC_n5A-n258C DC_n5A-n258D DC_n5A-n258E DC_n5A-n258F DC_n5A-n258G DC_n5A-n258H DC_n5A-n258I DC_n5A-n258J DC_n5A-n258K DC_n5A-n258L DC_n5A-n258M | DC_n5A-n258A DC_n5A-n258G DC_n5A-n258H DC_n5A-n258I |
| DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n5A-n260J DC_n5A-n260K DC_n5A-n260L DC_n5A-n260M DC_n5A-n260R2 DC_n5A-n260R3 DC_n5A-n260R4 DC_n5A-n260R5 DC_n5A-n260R6 DC_n5A-n260R7 DC_n5A-n260R8 DC_n5A-n260R9 DC_n5A-n260R10 | DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n5A-n260J DC_n5A-n260K DC_n5A-n260L DC_n5A-n260M DC_n5A-n260R2 DC_n5A-n260R3 DC_n5A-n260R4 |
| DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n5A-n261J DC_n5A-n261K DC_n5A-n261L DC_n5A-n261M | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I |
| DC_n5A-n261(2A) DC_n5A-n261(3A) DC_n5A-n261(4A) DC_n5A-n261(2G) DC_n5A-n261(2H) DC_n5A-n261(2I) DC_n5A-n261(A-G) DC_n5A-n261(A-H) DC_n5A-n261(A-I) DC_n5A-n261(A-J) DC_n5A-n261(A-K) DC_n5A-n261(A-L) DC_n5A-n261(G-H) DC_n5A-n261(H-I) DC_n5A-n261(G-I) DC_n5A-n261(A-G-H) DC_n5A-n261(A-G-I) DC_n5A-n261(2A-H) DC_n5A-n261(2A-G) DC_n5A-n261(2A-I) DC_n5A-n261(A-2G) | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I |
| DC_n7A-n257A DC_n7A-n257G DC_n7A-n257H DC_n7A-n257I DC_n7A-n257J DC_n7A-n257K DC_n7A-n257L DC_n7A-n257M | DC_n7A-n257A DC_n7A-n257G DC_n7A-n257H DC_n7A-n257I DC_n7A-n257J DC_n7A-n257K DC_n7A-n257L DC_n7A-n257M |
| DC_n7A-n258A DC_n7A-n258B DC_n7A-n258C DC_n7A-n258D DC_n7A-n258E DC_n7A-n258F DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n7A-n258J DC_n7A-n258K DC_n7A-n258L DC_n7A-n258M DC_n7A-n258R2 DC_n7A-n258R3 DC_n7A-n258R4 DC_n7A-n258R5 DC_n7A-n258R6 DC_n7A-n258R7 DC_n7A-n258R8 DC_n7A-n258R9 DC_n7A-n258R10 DC_n7B-n258A DC_n7B-n258B DC_n7B-n258C DC_n7B-n258D DC_n7B-n258E DC_n7B-n258F DC_n7B-n258G DC_n7B-n258H DC_n7B-n258I DC_n7B-n258J DC_n7B-n258K DC_n7B-n258L DC_n7B-n258M DC_n7B-n258R2 DC_n7B-n258R3 DC_n7B-n258R4 DC_n7B-n258R5 DC_n7B-n258R6 DC_n7B-n258R7 DC_n7B-n258R8 DC_n7B-n258R9 DC_n7B-n258R10 | DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I I DC_n7A-n258R2 DC_n7A-n258R3 DC_n7A-n258R4 DC_n7B-n258A DC_n7B-n258G DC_n7B-n258H DC_n7B-n258I DC_n7B-n258R2 DC_n7B-n258R3 DC_n7B-n258R4 |
| DC_n8A-n257A DC_n8A-n257D DC_n8A-n257E DC_n8A-n257F DC_n8A-n257G DC_n8A-n257H DC_n8A-n257I DC_n8A-n257J DC_n8A-n257K DC_n8A-n257L DC_n8A-n257M | DC_n8A-n257A DC_n8A-n257G DC_n8A-n257H DC_n8A-n257I DC_n8A-n257J DC_n8A-n257K |
| DC_n8A-n258A DC_n8A-n258B DC_n8A-n258C DC_n8A-n258D DC_n8A-n258E DC_n8A-n258F DC_n8A-n258G DC_n8A-n258H DC_n8A-n258I DC_n8A-n258J DC_n8A-n258K DC_n8A-n258L DC_n8A-n258M | DC_n8A-n258A |
| DC_n12A-n260A DC_n12A-n260G DC_n12A-n260H DC_n12A-n260I DC_n12A-n260J DC_n12A-n260K DC_n12A-n260L DC_n12A-n260M | DC_n12A-n260A DC_n12A-n260G DC_n12A-n260H DC_n12A-n260I DC_n12A-n260J DC_n12A-n260K DC_n12A-n260L DC_n12A-n260M |
| DC_n14A-n260A DC_n14A-n260G DC_n14A-n260H DC_n14A-n260I DC_n14A-n260J DC_n14A-n260K DC_n14A-n260L DC_n14A-n260M | DC_n14A-n260A DC_n14A-n260G DC_n14A-n260H DC_n14A-n260I DC_n14A-n260J DC_n14A-n260K DC_n14A-n260L DC_n14A-n260M |
| DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I | DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I |
| DC_n25A-n257A DC_n25A-n257G DC_n25A-n257H DC_n25A-n257I DC_n25A-n257J DC_n25A-n257K DC_n25A-n257L DC_n25A-n257M | DC_n25A-n257A DC_n25A-n257G DC_n25A-n257H DC_n25A-n257I DC_n25A-n257J DC_n25A-n257K DC_n25A-n257L DC_n25A-n257M |
| DC_n25A-n258A DC_n25A-n258G DC_n25A-n258H | DC_n25A-n258A DC_n25A-n258G DC_n25A-n258H |
| DC_n25A-n258(2A) DC_n25A-n258(3A) DC_n25A-n258(4A) DC_n25A-n258(5A) DC_n25A-n258(2G) DC_n25A-n258(A-G) DC_n25A-n258(A-H) DC_n25A-n258(G-H) | DC_n25A-n258A DC_n25A-n258G DC_n25A-n258H |
| DC_n25A-n260A DC_n25A-n260G DC_n25A-n260H DC_n25A-n260I DC_n25A-n260J DC_n25A-n260K DC_n25A-n260L DC_n25A-n260M | DC_n25A-n260A DC_n25A-n260G DC_n25A-n260H DC_n25A-n260I DC_n25A-n260J DC_n25A-n260K DC_n25A-n260L DC_n25A-n260M |
| DC_n25A-n260(2A) DC_n25A-n260(3A) DC_n25A-n260(4A) DC_n25A-n260(5A) DC_n25A-n260(6A) DC_n25A-n260(7A) DC_n25A-n260(8A) | DC_n25A-n260A |
| DC_n25A-n261A | DC_n25A-n261A |
| DC_n25A-n261(2A) | DC_n25A-n261A |
| DC_n26A-n258A DC_n26A-n258B DC_n26A-n258C DC_n26A-n258D DC_n26A-n258E DC_n26A-n258F DC_n26A-n258G DC_n26A-n258H DC_n26A-n258I DC_n26A-n258J DC_n26A-n258K DC_n26A-n258L DC_n26A-n258M DC_n26A-n258R2 DC_n26A-n258R3 DC_n26A-n258R4 DC_n26A-n258R5 DC_n26A-n258R6 DC_n26A-n258R7 DC_n26A-n258R8 DC_n26A-n258R9 DC_n26A-n258R10 | DC_n26A-n258A DC_n26A-n258G DC_n26A-n258H DC_n26A-n258I DC_n26A-n258R2 DC_n26A-n258R3 DC_n26A-n258R4 |
| DC_n26(2A)-n258A DC_n26(2A)-n258B DC_n26(2A)-n258C DC_n26(2A)-n258D DC_n26(2A)-n258E DC_n26(2A)-n258F DC_n26(2A)-n258G DC_n26(2A)-n258H DC_n26(2A)-n258I DC_n26(2A)-n258J DC_n26(2A)-n258K DC_n26(2A)-n258L DC_n26(2A)-n258M | DC_n26A-n258A DC_n26A-n258G DC_n26A-n258H DC_n26A-n258I |
| DC_n28A-n257A DC_n28A-n257D DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I | DC_n28A-n257A DC_n28A-n257D DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I |
| DC_n28A-n258A DC_n28A-n258B DC_n28A-n258C DC_n28A-n258D DC_n28A-n258E DC_n28A-n258F DC_n28A-n258G DC_n28A-n258H DC_n28A-n258I DC_n28A-n258J DC_n28A-n258K DC_n28A-n258L DC_n28A-n258M DC_n28A-n258R2 DC_n28A-n258R3 DC_n28A-n258R4 DC_n28A-n258R5 DC_n28A-n258R6 DC_n28A-n258R7 DC_n28A-n258R8 DC_n28A-n258R9 DC_n28A-n258R10 | DC_n28A-n258A DC_n28A-n258G DC_n28A-n258H DC_n28A-n258I DC_n28A-n258R2 DC_n28A-n258R3 DC_n28A-n258R4 |
| DC_n30A-n260A DC_n30A-n260G DC_n30A-n260H DC_n30A-n260I DC_n30A-n260J DC_n30A-n260K DC_n30A-n260L DC_n30A-n260M | DC_n30A-n260A DC_n30A-n260G DC_n30A-n260H DC_n30A-n260I DC_n30A-n260J DC_n30A-n260K DC_n30A-n260L DC_n30A-n260M |
| DC_n39A-n258A DC_n39A-n258B DC_n39A-n258C DC_n39A-n258D DC_n39A-n258E DC_n39A-n258F DC_n39A-n258G DC_n39A-n258H DC_n39A-n258I DC_n39A-n258J DC_n39A-n258K DC_n39A-n258L DC_n39A-n258M | DC_n39A-n258A |
| DC_n40A-n257A DC_n40A-n257D DC_n40A-n257E DC_n40A-n257F DC_n40A-n257G DC_n40A-n257H DC_n40A-n257I DC_n40A-n257J DC_n40A-n257K DC_n40A-n257L DC_n40A-n257M | DC_n40A-n257A DC_n40A-n257G DC_n40A-n257H DC_n40A-n257I DC_n40A-n257J DC_n40A-n257K DC_n40A-n257L DC_n40A-n257M |
| DC_n40A-n258A DC_n40A-n258G DC_n40A-n258H DC_n40A-n258I DC_n40A-n258J DC_n40A-n258K DC_n40A-n258L DC_n40A-n258M | DC_n40A-n258A |
| DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I | DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |
| DC_n41(2A)-n257A DC_n41(2A)-n257G DC_n41(2A)-n257H DC_n41(2A)-n257I | DC_n41A-n257A DC_n41A-n257G DC_n41A-n257I DC_n41A-n257H |
| DC_n41A-n258A DC_n41A-n258G DC_n41A-n258H DC_n41C-n258A DC_n41C-n258G DC_n41C-n258H | DC_n41A-n258A DC_n41A-n258G DC_n41A-n258H |
| DC_n41A-n258(2A) DC_n41A-n258(3A) DC_n41A-n258(4A) DC_n41A-n258(5A) DC_n41C-n258(2A) DC_n41C-n258(3A) DC_n41C-n258(4A) DC_n41C-n258(5A) DC_n41(2A)-n258A DC_n41(2A)-n258G DC_n41(2A)-n258H DC_n41(2A)-n258(2A) DC_n41(2A)-n258(3A) DC_n41(2A)-n258(4A) DC_n41(2A)-n258(5A) DC_n41A-n258(2G) DC_n41C-n258(2G) DC_n41(2A)-n258(2G) DC_n41A-n258(A-G) DC_n41C-n258(A-G) DC_n41(2A)-n258(A-G) DC_n41A-n258(A-H) DC_n41C-n258(A-H) DC_n41(2A)-n258(A-H) DC_n41A-n258(G-H) DC_n41C-n258(G-H) DC_n41(2A)-n258(G-H) | DC_n41A-n258A DC_n41A-n258G DC_n41A-n258H |
| DC_n41A-n260A DC_n41A-n260G DC_n41A-n260H DC_n41A-n260I DC_n41A-n260J DC_n41A-n260K DC_n41A-n260L DC_n41A-n260M DC_n41C-n260A DC_n41C-n260G DC_n41C-n260H DC_n41C-n260I DC_n41C-n260J DC_n41C-n260K DC_n41C-n260L DC_n41C-n260M | DC_n41A-n260A DC_n41A-n260G DC_n41A-n260H DC_n41A-n260I DC_n41A-n260J DC_n41A-n260K DC_n41A-n260L DC_n41A-n260M |
| DC_n41A-n260(2A) DC_n41A-n260(3A) DC_n41A-n260(4A) DC_n41A-n260(5A) DC_n41A-n260(6A) DC_n41A-n260(7A) DC_n41A-n260(8A) DC_n41(2A)-n260A DC_n41(2A)-n260(2A) DC_n41(2A)-n260(3A) DC_n41(2A)-n260(4A) DC_n41(2A)-n260(5A) DC_n41(2A)-n260(6A) DC_n41(2A)-n260(7A) DC_n41(2A)-n260(8A) DC_n41(2A)-n260G DC_n41(2A)-n260H DC_n41(2A)-n260I DC_n41(2A)-n260J DC_n41(2A)-n260K DC_n41(2A)-n260L DC_n41(2A)-n260M DC_n41C-n260(2A) DC_n41C-n260(3A) DC_n41C-n260(4A) DC_n41C-n260(5A) DC_n41C-n260(6A) DC_n41C-n260(7A) DC_n41C-n260(8A) | DC_n41A-n260A DC_n41A-n260G DC_n41A-n260H DC_n41A-n260I DC_n41A-n260J DC_n41A-n260K DC_n41A-n260L DC_n41A-n260M |
| DC_n41A-n261A DC_n41C-n261A | DC_n41A-n261A |
| DC_n41A-n261(2A) DC_n41C-n261(2A) DC_n41(2A)-n261A DC_n41(2A)-n261(2A) | DC_n41A-n261A |
| DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n48A-n260J DC_n48A-n260K DC_n48A-n260L DC_n48A-n260M DC_n48A-n260R2 DC_n48A-n260R3 DC_n48A-n260R4 DC_n48A-n260R5 DC_n48A-n260R6 DC_n48A-n260R7 DC_n48A-n260R8 DC_n48A-n260R9 DC_n48A-n260R10 DC_n48B-n260A DC_n48B-n260G DC_n48B-n260H DC_n48B-n260I DC_n48B-n260J DC_n48B-n260K DC_n48B-n260L DC_n48B-n260M DC_n48C-n260A DC_n48C-n260G DC_n48C-n260H DC_n48C-n260I DC_n48C-n260J DC_n48C-n260K DC_n48C-n260L DC_n48C-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n48A-n260R2 DC_n48A-n260R3 DC_n48A-n260R4 DC_n48B-n260A DC_n48B-n260G DC_n48B-n260H DC_n48B-n260I |
| DC_n48(2A)-n260A DC_n48(2A)-n260G DC_n48(2A)-n260H DC_n48(2A)-n260I DC_n48(2A)-n260J DC_n48(2A)-n260K DC_n48(2A)-n260L DC_n48(2A)-n260M DC_n48(3A)-n260A DC_n48(3A)-n260G DC_n48(3A)-n260H DC_n48(3A)-n260I DC_n48(3A)-n260J DC_n48(3A)-n260K DC_n48(3A)-n260L DC_n48(3A)-n260M DC_n48(4A)-n260A DC_n48(4A)-n260G DC_n48(4A)-n260H DC_n48(4A)-n260I DC_n48(4A)-n260J DC_n48(4A)-n260K DC_n48(4A)-n260L DC_n48(4A)-n260M DC_n48(A-B)-n260A DC_n48(A-B)-n260G DC_n48(A-B)-n260H DC_n48(A-B)-n260I DC_n48(A-B)-n260J DC_n48(A-B)-n260K DC_n48(A-B)-n260L DC_n48(A-B)-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n48A-n261J DC_n48A-n261K DC_n48A-n261L DC_n48A-n261M DC_n48B-n261A DC_n48B-n261G DC_n48B-n261H DC_n48B-n261I DC_n48B-n261J DC_n48B-n261K DC_n48B-n261L DC_n48B-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n48A-n261(2A) DC_n48A-n261(2G) DC_n48A-n261(2H) DC_n48A-n261(2I) DC_n48A-n261(3A) DC_n48A-n261(4A) DC_n48A-n261(A-G) DC_n48A-n261(A-H) DC_n48A-n261(A-I) DC_n48A-n261(G-H) DC_n48A-n261(H-I) DC_n48A-n261(G-I) DC_n48A-n261(2A-G) DC_n48A-n261(2A-H) DC_n48A-n261(2A-I) DC_n48A-n261(A-2G) DC_n48A-n261(A-G-H) DC_n48A-n261(A-G-I) DC_n48(2A)-n261A DC_n48(2A)-n261G DC_n48(2A)-n261H DC_n48(2A)-n261I DC_n48(2A)-n261J DC_n48(2A)-n261K DC_n48(2A)-n261L DC_n48(2A)-n261M DC_n48(2A)-n261(2A-G) DC_n48(2A)-n261(2A-H) DC_n48(2A)-n261(2A-I) DC_n48(2A)-n261(2A) DC_n48(2A)-n261(2G) DC_n48(2A)-n261(3A) DC_n48(2A)-n261(A-2G) DC_n48(2A)-n261(A-G) DC_n48(2A)-n261(A-H) DC_n48(2A)-n261(A-I) DC_n48(2A)-n261(G-H) DC_n48(2A)-n261(2H) DC_n48(2A)-n261(G-I) DC_n48(2A)-n261(A-G-H) DC_n48(2A)-n261(H-I) DC_n48(2A)-n261(A-G-I) DC_n48B-n261(G-H) DC_n48B-n261(2H) DC_n48B-n261(G-I) DC_n48B-n261(A-G-H) DC_n48B-n261(H-I) DC_n48B-n261(A-G-I) DC_n48B-n261(2A-G) DC_n48B-n261(2A-H) DC_n48B-n261(2A-I) DC_n48B-n261(2A) DC_n48B-n261(2G) DC_n48B-n261(3A) DC_n48B-n261(A-2G) DC_n48B-n261(A-G) DC_n48B-n261(A-H) DC_n48B-n261(A-I) DC_n48(A-B)-n261A DC_n48(A-B)-n261G DC_n48(A-B)-n261H DC_n48(A-B)-n261I DC_n48(A-B)-n261J DC_n48(A-B)-n261K DC_n48(A-B)-n261L DC_n48(A-B)-n261M DC_n48(A-B)-n261(G-H) DC_n48(A-B)-n261(2H) DC_n48(A-B)-n261(2A) DC_n48(A-B)-n261(3A) DC_n48(A-B)-n261(A-G) DC_n48(A-B)-n261(2A-G) DC_n48(A-B)-n261(A-H) DC_n48(A-B)-n261(2G) DC_n48(A-B)-n261(A-I) DC_n48(A-B)-n261(2A-H) DC_n48(A-B)-n261(A-2G) DC_n48(A-B)-n261(2A-I) DC_n48(A-B)-n261(G-I) DC_n48(A-B)-n261(A-G-H) DC_n48(A-B)-n261(H-I) DC_n48(A-B)-n261(A-G-I) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n66A-n257A DC_n66A-n257G DC_n66A-n257H DC_n66A-n257I | DC_n66A-n257A DC_n66A-n257G DC_n66A-n257H DC_n66A-n257I |
| DC_n66A-n258A DC_n66A-n258G DC_n66A-n258H | DC_n66A-n258A DC_n66A-n258G DC_n66A-n258H |
| DC_n66A-n258(2A) DC_n66A-n258(3A) DC_n66A-n258(4A) DC_n66A-n258(5A) DC_n66A-n258(2G) DC_n66A-n258(A-G) DC_n66A-n258(A-H) DC_n66A-n258(G-H) | DC_n66A-n258A DC_n66A-n258G DC_n66A-n258H |
| DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I DC_n66A-n260J DC_n66A-n260K DC_n66A-n260L DC_n66A-n260M | DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I DC_n66A-n260J DC_n66A-n260K DC_n66A-n260L DC_n66A-n260M |
| DC_n66A-n260(2A) DC_n66A-n260(3A) DC_n66A-n260(4A) DC_n66A-n260(5A) DC_n66A-n260(6A) DC_n66A-n260(7A) DC_n66A-n260(8A) DC_n66(2A)-n260A DC_n66(2A)-n260G DC_n66(2A)-n260H DC_n66(2A)-n260I DC_n66(2A)-n260J DC_n66(2A)-n260K DC_n66(2A)-n260L DC_n66(2A)-n260M DC_n66A-n260R2 DC_n66A-n260R3 DC_n66A-n260R4 DC_n66A-n260R5 DC_n66A-n260R6 DC_n66A-n260R7 DC_n66A-n260R8 DC_n66A-n260R9 DC_n66A-n260R10 | DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I DC_n66A-n260J DC_n66A-n260K DC_n66A-n260L DC_n66A-n260M DC_n66A-n260R2 DC_n66A-n260R3 DC_n66A-n260R4 |
| DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I DC_n66A-n261J DC_n66A-n261K DC_n66A-n261L DC_n66A-n261M DC_n66A-n261O DC_n66A-n261P DC_n66A-n261Q | DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I DC_n66A-n261J DC_n66A-n261K DC_n66A-n261L DC_n66A-n261M |
| DC_n66A-n261(2A) DC_n66A-n261(3A) DC_n66A-n261(4A) DC_n66A-n261(2G) DC_n66A-n261(2H) DC_n66A-n261(2I) DC_n66A-n261(A-G) DC_n66A-n261(A-H) DC_n66A-n261(A-I) DC_n66A-n261(A-J) DC_n66A-n261(A-K) DC_n66A-n261(A-L) DC_n66A-n261(G-H) DC_n66A-n261(H-I) DC_n66A-n261(G-I) DC_n66A-n261(A-G-H) DC_n66A-n261(A-G-I) DC_n66A-n261(2A-H) DC_n66A-n261(2A-G) DC_n66A-n261(2A-I) DC_n66A-n261(A-2G) | DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n71A-n257A DC_n71A-n257G DC_n71A-n257H DC_n71A-n257I | DC_n71A-n257A DC_n71A-n257G DC_n71A-n257H DC_n71A-n257I |
| DC_n77A-n257A1 DC_n77A-n257D1 DC_n77A-n257E1 DC_n77A-n257F1 DC_n77A-n257G1 DC_n77A-n257H1 DC_n77A-n257I1 DC_n77A-n257J1 DC_n77A-n257K1 DC_n77A-n257L1 DC_n77A-n257M1 DC_n77C-n257A DC_n77C-n257D DC_n77C-n257E DC_n77C-n257F | DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n77A-n257J DC_n77A-n257K DC_n77A-n257L DC_n77A-n257M |
| DC_n77(2A)-n257A1 DC_n77(2A)-n257D DC_n77(2A)-n257E DC_n77(2A)-n257F DC_n77(2A)-n257G1 DC_n77(2A)-n257H1 DC_n77(2A)-n257I1 DC_n77(2A)-n257J DC_n77(2A)-n257K DC_n77(2A)-n257L DC_n77(2A)-n257M | DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n77A-n257J DC_n77A-n257K DC_n77A-n257L DC_n77A-n257M |
| DC_n77(3A)-n257A DC_n77(3A)-n257G DC_n77(3A)-n257H DC_n77(3A)-n257I | DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n77A-n258A DC_n77A-n258D DC_n77A-n258G DC_n77A-n258H DC_n77A-n258I DC_n77A-n258J | DC_n77A-n258A DC_n77A-n258D DC_n77A-n258G DC_n77A-n258H DC_n77A-n258I DC_n77A-n258J |
| DC_n77(2A)-n258A DC_n77(2A)-n258D DC_n77(2A)-n258G DC_n77(2A)-n258H DC_n77(2A)-n258I DC_n77(2A)-n258J DC_n77(3A)-n258A DC_n77(3A)-n258D DC_n77(3A)-n258G DC_n77(3A)-n258H DC_n77(3A)-n258I DC_n77(3A)-n258J | DC_n77A-n258A DC_n77A-n258D DC_n77A-n258G DC_n77A-n258H DC_n77A-n258I DC_n77A-n258J |
| DC_n77A-n259A1 DC_n77A-n259G1 DC_n77A-n259H1 DC_n77A-n259I1 DC_n77A-n259J1 DC_n77A-n259K1 DC_n77A-n259L1 DC_n77A-n259M1 | DC_n77A-n259A DC_n77A-n259G DC_n77A-n259H DC_n77A-n259I DC_n77A-n259J DC_n77A-n259K DC_n77A-n259L DC_n77A-n259M |
| DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I DC_n77A-n260J DC_n77A-n260K DC_n77A-n260L DC_n77A-n260M DC_n77A-n260R2 DC_n77A-n260R3 DC_n77A-n260R4 DC_n77A-n260R5 DC_n77A-n260R6 DC_n77A-n260R7 DC_n77A-n260R8 DC_n77A-n260R9 DC_n77A-n260R10 DC_n77C-n260A DC_n77C-n260G DC_n77C-n260H DC_n77C-n260I DC_n77C-n260J DC_n77C-n260K DC_n77C-n260L DC_n77C-n260M | DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I DC_n77A-n260J DC_n77A-n260K DC_n77A-n260L DC_n77A-n260M DC_n77A-n260R2 DC_n77A-n260R3 DC_n77A-n260R4 |
| DC_n77(2A)-n260A DC_n77(2A)-n260G DC_n77(2A)-n260H DC_n77(2A)-n260I DC_n77(2A)-n260J DC_n77(2A)-n260K DC_n77(2A)-n260L DC_n77(2A)-n260M | DC_n77(2A) DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I DC_n77A-n260J DC_n77A-n260K DC_n77A-n260L DC_n77A-n260M |
| DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I DC_n77A-n261J DC_n77A-n261K DC_n77A-n261L DC_n77A-n261M DC_n77C-n261A DC_n77C-n261G DC_n77C-n261H DC_n77C-n261I DC_n77C-n261J DC_n77C-n261K DC_n77C-n261L DC_n77C-n261M | DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I DC_n77A-n261J DC_n77A-n261K DC_n77A-n261L DC_n77A-n261M |
| DC_n77A-n261(2A) DC_n77A-n261(2G) DC_n77A-n261(2H) DC_n77A-n261(2I) DC_n77A-n261(3A) DC_n77A-n261(4A) | DC_n77A-n261A |
| DC_n77A-n261(A-G) DC_n77A-n261(A-H) DC_n77A-n261(A-I) DC_n77A-n261(G-H) DC_n77A-n261(G-I) DC_n77A-n261(H-I) DC_n77A-n261(A-J) DC_n77A-n261(A-K) DC_n77A-n261(A-L) DC_n77A-n261(A-G-H) DC_n77A-n261(A-G-I) DC_n77A-n261(2A-H) DC_n77A-n261(2A-G) DC_n77A-n261(2A-I) DC_n77A-n261(A-2G) DC_n77C-n261(G-H) DC_n77C-n261(2H) DC_n77C-n261(G-I) DC_n77C-n261(A-G-H) DC_n77C-n261(H-I) DC_n77C-n261(A-G-I) DC_n77C-n261(2A-G) DC_n77C-n261(2A-H) DC_n77C-n261(2A-I) DC_n77C-n261(2A) DC_n77C-n261(2G) DC_n77C-n261(3A) DC_n77C-n261(A-2G) DC_n77C-n261(A-G) DC_n77C-n261(A-H) DC_n77C-n261(A-I) | DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n78A-n257A DC_n78A-n257D DC_n78A-n257E DC_n78A-n257F DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n78A-n257J DC_n78A-n257K DC_n78A-n257L DC_n78A-n257M DC_n78C-n257A DC_n78C-n257D DC_n78C-n257E DC_n78C-n257F DC_n78C-n257G DC_n78C-n257H DC_n78C-n257I DC_n78C-n257J DC_n78C-n257K DC_n78C-n257L DC_n78C-n257M | DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n78A-n257(2A) DC_n78A-n257(A-G) DC_n78A-n257(2G) DC_n78(2A)-n257A DC_n78(2A)-n257G DC_n78(2A)-n257H DC_n78(2A)-n257I | DC_n78A-n257A DC_n78A-n257G DC_n78A-n257I DC_n78A-n257H DC_n78A-n257(2A) DC_n78A-n257(2G) |
| DC_n78A-n258A DC_n78A-n258B DC_n78A-n258C DC_n78A-n258D DC_n78A-n258E DC_n78A-n258F DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n78A-n258J DC_n78A-n258K DC_n78A-n258L DC_n78A-n258M DC_n78A-n258R2 DC_n78A-n258R3 DC_n78A-n258R4 DC_n78A-n258R5 DC_n78A-n258R6 DC_n78A-n258R7 DC_n78A-n258R8 DC_n78A-n258R9 DC_n78A-n258R10 DC_n78C-n258A DC_n78C-n258B DC_n78C-n258C DC_n78C-n258D DC_n78C-n258E DC_n78C-n258F DC_n78C-n258G DC_n78C-n258H DC_n78C-n258I DC_n78C-n258J DC_n78C-n258K DC_n78C-n258L DC_n78C-n258M | DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n78A-n258R2 DC_n78A-n258R3 DC_n78A-n258R4 |
| DC_n78A-n258(2A) | DC_n78A-n258A DC_n78A-n258(2A) |
| DC_n78A-n259A1 DC_n78A-n259G1 DC_n78A-n259H1 DC_n78A-n259I1 DC_n78A-n259J1 DC_n78A-n259K1 DC_n78A-n259L1 DC_n78A-n259M1 | DC_n78A-n259A DC_n78A-n259G DC_n78A-n259H DC_n78A-n259I DC_n78A-n259J DC_n78A-n259K DC_n78A-n259L DC_n78A-n259M |
| DC_n79A-n257A1 DC_n79A-n257D1 DC_n79A-n257E1 DC_n79A-n257F1 DC_n79A-n257G1 DC_n79A-n257H1 DC_n79A-n257I1 DC_n79A-n257J DC_n79A-n257K DC_n79A-n257L DC_n79A-n257M DC_n79C-n257A DC_n79C-n257D DC_n79C-n257E DC_n79C-n257F | DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n79A-n258A DC_n79A-n258D DC_n79A-n258E DC_n79A-n258F DC_n79A-n258G DC_n79A-n258H DC_n79A-n258I DC_n79A-n258J DC_n79A-n258K DC_n79A-n258L DC_n79A-n258M | DC_n79A-n258A DC_n79A-n258D DC_n79A-n258G DC_n79A-n258H DC_n79A-n258I DC_n79A-n258J |
| DC_n79A-n259A1 DC_n79A-n259G1 DC_n79A-n259H1 DC_n79A-n259I1 DC_n79A-n259J1 DC_n79A-n259K1 DC_n79A-n259L1 DC_n79A-n259M1 | DC_n79A-n259A DC_n79A-n259G DC_n79A-n259H DC_n79A-n259I DC_n79A-n259J DC_n79A-n259K DC_n79A-n259L DC_n79A-n259M |
| NOTE 1: Applicable for UE supporting inter-band NR DC with mandatory simultaneous Rx/Tx capability. |  |

#### 5.5B.7.2	Inter-band NR-DC configurations between FR1 and FR2 (three bands)

Table 5.5B.7-2: Inter-band NR-DC configurations between FR1 and FR2 (three bands)

| Downlink NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n1A-n3A-n257A DC_n1A-n3A-n257G DC_n1A-n3A-n257H DC_n1A-n3A-n257I | DC_n1A-n3A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I |
| DC_n1A-n18A-n257A DC_n1A-n18A-n257G DC_n1A-n18A-n257H DC_n1A-n18A-n257I | DC_n1A-n18A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I |
| DC_n1A-n28A-n257A DC_n1A-n28A-n257G DC_n1A-n28A-n257H DC_n1A-n28A-n257I | DC_n1A-n28A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I |
| DC_n1A-n41A-n257A DC_n1A-n41A-n257G DC_n1A-n41A-n257H DC_n1A-n41A-n257I | DC_n1A-n41A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |
| DC_n1A-n77A-n257A DC_n1A-n77A-n257G DC_n1A-n77A-n257H DC_n1A-n77A-n257I | DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n1A-n77(2A)-n257A DC_n1A-n77(2A)-n257G DC_n1A-n77(2A)-n257H DC_n1A-n77(2A)-n257I | DC_n1A-n77A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n1A-n78A-n257A1 DC_n1A-n78A-n257G1 DC_n1A-n78A-n257H1 DC_n1A-n78A-n257I1 DC_n1A-n78A-n257J1 DC_n1A-n78A-n257K1 DC_n1A-n78A-n257L1 DC_n1A-n78A-n257M1 | DC_n1A-n78A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n1A-n257J DC_n1A-n257K DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n78A-n257J DC_n78A-n257K |
| DC_n1A-n79A-n257A DC_n1A-n79A-n257G DC_n1A-n79A-n257H DC_n1A-n79A-n257I | DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n2A-n5A-n260A DC_n2A-n5A-n260G DC_n2A-n5A-n260H DC_n2A-n5A-n260I DC_n2A-n5A-n260J DC_n2A-n5A-n260K DC_n2A-n5A-n260L DC_n2A-n5A-n260M | DC_n2A-n5A DC_n2A-n260A DC_n5A-n260A DC_n2A-n260G DC_n5A-n260G DC_n2A-n260H DC_n5A-n260H DC_n2A-n260I DC_n5A-n260I DC_n2A-n260J DC_n5A-n260J DC_n2A-n260K DC_n5A-n260K DC_n2A-n260L DC_n5A-n260L DC_n2A-n260M DC_n5A-n260M |
| DC_n2A-n5A-n261A DC_n2A-n5A-n261G DC_n2A-n5A-n261H DC_n2A-n5A-n261I DC_n2A-n5A-n261J DC_n2A-n5A-n261K DC_n2A-n5A-n261L DC_n2A-n5A-n261M | DC_n2A-n5A DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I |
| DC_n2A-n5A-n261(2G) DC_n2A-n5A-n261(G-H) DC_n2A-n5A-n261(A-G-H) DC_n2A-n5A-n261(G-I) DC_n2A-n5A-n261(2H) DC_n2A-n5A-n261(A-G-I) DC_n2A-n5A-n261(H-I) DC_n2A-n5A-n261(A-G) DC_n2A-n5A-n261(A-H) DC_n2A-n5A-n261(2A-H) DC_n2A-n5A-n261(A-2G) DC_n2A-n5A-n261(A-I) DC_n2A-n5A-n261(2A-I) DC_n2A-n5A-n261(2A) DC_n2A-n5A-n261(3A) DC_n2A-n5A-n261(2A-G) | DC_n2A-n5A DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I |
| DC_n2A-n12A-n260A DC_n2A-n12A-n260G DC_n2A-n12A-n260H DC_n2A-n12A-n260I DC_n2A-n12A-n260J DC_n2A-n12A-n260K DC_n2A-n12A-n260L DC_n2A-n12A-n260M | DC_n2A-n12A DC_n2A-n260A DC_n12A-n260A DC_n2A-n260G DC_n12A-n260G DC_n2A-n260H DC_n12A-n260H DC_n2A-n260I DC_n12A-n260I DC_n2A-n260J DC_n12A-n260J DC_n2A-n260K DC_n12A-n260K DC_n2A-n260L DC_n12A-n260L DC_n2A-n260M DC_n12A-n260M |
| DC_n2A-n14A-n260A DC_n2A-n14A-n260G DC_n2A-n14A-n260H DC_n2A-n14A-n260I DC_n2A-n14A-n260J DC_n2A-n14A-n260K DC_n2A-n14A-n260L DC_n2A-n14A-n260M | DC_n2A-n14A DC_n2A-n260A DC_n14A-n260A DC_n2A-n260G DC_n14A-n260G DC_n2A-n260H DC_n14A-n260H DC_n2A-n260I DC_n14A-n260I DC_n2A-n260J DC_n14A-n260J DC_n2A-n260K DC_n14A-n260K DC_n2A-n260L DC_n14A-n260L DC_n2A-n260M DC_n14A-n260M |
| DC_n2A-n30A-n260A DC_n2A-n30A-n260G DC_n2A-n30A-n260H DC_n2A-n30A-n260I DC_n2A-n30A-n260J DC_n2A-n30A-n260K DC_n2A-n30A-n260L DC_n2A-n30A-n260M | DC_n2A-n30A DC_n2A-n260A DC_n30A-n260A DC_n2A-n260G DC_n30A-n260G DC_n2A-n260H DC_n30A-n260H DC_n2A-n260I DC_n30A-n260I DC_n2A-n260J DC_n30A-n260J DC_n2A-n260K DC_n30A-n260K DC_n2A-n260L DC_n30A-n260L DC_n2A-n260M DC_n30A-n260M |
| DC_n2A-n48A-n260A DC_n2A-n48A-n260G DC_n2A-n48A-n260H DC_n2A-n48A-n260I DC_n2A-n48A-n260J DC_n2A-n48A-n260K DC_n2A-n48A-n260L DC_n2A-n48A-n260M | DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n2A-n48(2A)-n260A DC_n2A-n48(2A)-n260G DC_n2A-n48(2A)-n260H DC_n2A-n48(2A)-n260I DC_n2A-n48(2A)-n260J DC_n2A-n48(2A)-n260K DC_n2A-n48(2A)-n260L DC_n2A-n48(2A)-n260M | DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n2A-n48B-n260A DC_n2A-n48B-n260G DC_n2A-n48B-n260H DC_n2A-n48B-n260I DC_n2A-n48B-n260J DC_n2A-n48B-n260K DC_n2A-n48B-n260L DC_n2A-n48B-n260M | DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n2A-n48A-n261A DC_n2A-n48A-n261G DC_n2A-n48A-n261H DC_n2A-n48A-n261I DC_n2A-n48A-n261J DC_n2A-n48A-n261K DC_n2A-n48A-n261L DC_n2A-n48A-n261M | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n48A-n261(G-H) DC_n2A-n48A-n261(A-G-H) DC_n2A-n48A-n261(2H) DC_n2A-n48A-n261(H-I) DC_n2A-n48A-n261(A-G-I) DC_n2A-n48A-n261(A-H) DC_n2A-n48A-n261(2G) DC_n2A-n48A-n261(2A-H) DC_n2A-n48A-n261(A-2G) DC_n2A-n48A-n261(G-I) DC_n2A-n48A-n261(2A-I) DC_n2A-n48A-n261(A-G) DC_n2A-n48A-n261(2A-G) DC_n2A-n48A-n261(A-I) DC_n2A-n48A-n261(2A) DC_n2A-n48A-n261(3A) | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n48(2A)-n261A DC_n2A-n48(2A)-n261G DC_n2A-n48(2A)-n261H DC_n2A-n48(2A)-n261I DC_n2A-n48(2A)-n261J DC_n2A-n48(2A)-n261K DC_n2A-n48(2A)-n261L DC_n2A-n48(2A)-n261M | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n48(2A)-n261(G-H) DC_n2A-n48(2A)-n261(A-G-H) DC_n2A-n48(2A)-n261(2H) DC_n2A-n48(2A)-n261(H-I) DC_n2A-n48(2A)-n261(A-G-I) DC_n2A-n48(2A)-n261(A-H) DC_n2A-n48(2A)-n261(2G) DC_n2A-n48(2A)-n261(2A-H) DC_n2A-n48(2A)-n261(A-2G) DC_n2A-n48(2A)-n261(G-I) DC_n2A-n48(2A)-n261(2A-I) DC_n2A-n48(2A)-n261(A-G) DC_n2A-n48(2A)-n261(2A-G) DC_n2A-n48(2A)-n261(A-I) DC_n2A-n48(2A)-n261(2A) DC_n2A-n48(2A)-n261(3A) | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n48B-n261A DC_n2A-n48B-n261G DC_n2A-n48B-n261H DC_n2A-n48B-n261I DC_n2A-n48B-n261J DC_n2A-n48B-n261K DC_n2A-n48B-n261L DC_n2A-n48B-n261M | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n48B-n261(G-H) DC_n2A-n48B-n261(A-G-H) DC_n2A-n48B-n261(2H) DC_n2A-n48B-n261(H-I) DC_n2A-n48B-n261(A-G-I) DC_n2A-n48B-n261(A-H) DC_n2A-n48B-n261(2G) DC_n2A-n48B-n261(2A-H) DC_n2A-n48B-n261(A-2G) DC_n2A-n48B-n261(G-I) DC_n2A-n48B-n261(2A-I) DC_n2A-n48B-n261(A-G) DC_n2A-n48B-n261(2A-G) DC_n2A-n48B-n261(A-I) DC_n2A-n48B-n261(2A) DC_n2A-n48B-n261(3A) | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n66A-n260A DC_n2A-n66A-n260G DC_n2A-n66A-n260H DC_n2A-n66A-n260I DC_n2A-n66A-n260J DC_n2A-n66A-n260K DC_n2A-n66A-n260L DC_n2A-n66A-n260M | DC_n2A-n66A DC_n2A-n260A DC_n66A-n260A DC_n2A-n260G DC_n66A-n260G DC_n2A-n260H DC_n66A-n260H DC_n2A-n260I DC_n66A-n260I DC_n2A-n260J DC_n66A-n260J DC_n2A-n260K DC_n66A-n260K DC_n2A-n260L DC_n66A-n260L DC_n2A-n260M DC_n66A-n260M |
| DC_n2A-n66A-n261A DC_n2A-n66A-n261G DC_n2A-n66A-n261H DC_n2A-n66A-n261I DC_n2A-n66A-n261J DC_n2A-n66A-n261K DC_n2A-n66A-n261L DC_n2A-n66A-n261M | DC_n2A-n66A DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n2A-n66A-n261(2G) DC_n2A-n66A-n261(G-H) DC_n2A-n66A-n261(A-G-H) DC_n2A-n66A-n261(G-I) DC_n2A-n66A-n261(2H) DC_n2A-n66A-n261(A-G-I) DC_n2A-n66A-n261(H-I) DC_n2A-n66A-n261(A-G) DC_n2A-n66A-n261(A-H) DC_n2A-n66A-n261(2A-H) DC_n2A-n66A-n261(A-2G) DC_n2A-n66A-n261(A-I) DC_n2A-n66A-n261(2A-I) DC_n2A-n66A-n261(2A) DC_n2A-n66A-n261(3A) DC_n2A-n66A-n261(2A-G) | DC_n2A-n66A DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n2A-n77A-n260A DC_n2A-n77A-n260G DC_n2A-n77A-n260H DC_n2A-n77A-n260I DC_n2A-n77A-n260J DC_n2A-n77A-n260K DC_n2A-n77A-n260L DC_n2A-n77A-n260M DC_n2A-n77C-n260A DC_n2A-n77C-n260G DC_n2A-n77C-n260H DC_n2A-n77C-n260I DC_n2A-n77C-n260J DC_n2A-n77C-n260K DC_n2A-n77C-n260L DC_n2A-n77C-n260M | DC_n2A-n77A DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n2A-n260J DC_n2A-n260K DC_n2A-n260L DC_n2A-n260M DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I DC_n77A-n260J DC_n77A-n260K DC_n77A-n260L DC_n77A-n260M |
| DC_n2A-n77A-n261A DC_n2A-n77A-n261G DC_n2A-n77A-n261H DC_n2A-n77A-n261I DC_n2A-n77A-n261J DC_n2A-n77A-n261K DC_n2A-n77A-n261L DC_n2A-n77A-n261M DC_n2A-n77C-n261A DC_n2A-n77C-n261G DC_n2A-n77C-n261H DC_n2A-n77C-n261I DC_n2A-n77C-n261J DC_n2A-n77C-n261K DC_n2A-n77C-n261L DC_n2A-n77C-n261M | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n2A-n77A-n261(G-H) DC_n2A-n77A-n261(A-G-H) DC_n2A-n77A-n261(G-I) DC_n2A-n77A-n261(2H) DC_n2A-n77A-n261(A-G-I) DC_n2A-n77A-n261(H-I) DC_n2A-n77A-n261(A-H) DC_n2A-n77A-n261(2G) DC_n2A-n77A-n261(2A-H) DC_n2A-n77A-n261(A-2G) DC_n2A-n77A-n261(A-I) DC_n2A-n77A-n261(2A-I) DC_n2A-n77A-n261(A-G) DC_n2A-n77A-n261(2A-G) DC_n2A-n77A-n261(2A) DC_n2A-n77A-n261(3A) DC_n2A-n77C-n261(G-H) DC_n2A-n77C-n261(A-G-H) DC_n2A-n77C-n261(G-I) DC_n2A-n77C-n261(2H) DC_n2A-n77C-n261(A-G-I) DC_n2A-n77C-n261(H-I) DC_n2A-n77C-n261(A-H) DC_n2A-n77C-n261(2G) DC_n2A-n77C-n261(2A-H) DC_n2A-n77C-n261(A-2G) DC_n2A-n77C-n261(A-I) DC_n2A-n77C-n261(2A-I) DC_n2A-n77C-n261(A-G) DC_n2A-n77C-n261(2A-G) DC_n2A-n77C-n261(2A) DC_n2A-n77C-n261(3A) | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n3A-n7A-n258A DC_n3A-n7A-n258B DC_n3A-n7A-n258C DC_n3A-n7A-n258D DC_n3A-n7A-n258E DC_n3A-n7A-n258F DC_n3A-n7A-n258G DC_n3A-n7A-n258H DC_n3A-n7A-n258I DC_n3A-n7A-n258J DC_n3A-n7A-n258K DC_n3A-n7A-n258L DC_n3A-n7A-n258M | DC_n3A-n258A DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I |
| DC_n3A-n7B-n258A DC_n3A-n7B-n258B DC_n3A-n7B-n258C DC_n3A-n7B-n258D DC_n3A-n7B-n258E DC_n3A-n7B-n258F DC_n3A-n7B-n258G DC_n3A-n7B-n258H DC_n3A-n7B-n258I DC_n3A-n7B-n258J DC_n3A-n7B-n258K DC_n3A-n7B-n258L DC_n3A-n7B-n258M | DC_n3A-n258A DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I |
| DC_n3A-n18A-n257A DC_n3A-n18A-n257G DC_n3A-n18A-n257H DC_n3A-n18A-n257I | DC_n3A-n18A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I |
| DC_n3A-n28A-n257A1 DC_n3A-n28A-n257G1 DC_n3A-n28A-n257H1 DC_n3A-n28A-n257I1 | DC_n3A-n28A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I |
| DC_n3A-n28A-n258A DC_n3A-n28A-n258D DC_n3A-n28A-n258G DC_n3A-n28A-n258H DC_n3A-n28A-n258I DC_n3A-n28A-n258J | DC_n3A-n28A DC_n3A-n258A DC_n3A-n258D DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n3A-n258J DC_n28A-n258A DC_n28A-n258D DC_n28A-n258G DC_n28A-n258H DC_n28A-n258I DC_n28A-n258J |
| DC_n3A-n41A-n257A DC_n3A-n41A-n257G DC_n3A-n41A-n257H DC_n3A-n41A-n257I | DC_n3A-n41A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |
| DC_n3A-n77A-n257A1 DC_n3A-n77A-n257G1 DC_n3A-n77A-n257H1 DC_n3A-n77A-n257I1 | DC_n3A-n77A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n3A-n77(2A)-n257A1 DC_n3A-n77(2A)-n257G1 DC_n3A-n77(2A)-n257H1 DC_n3A-n77(2A)-n257I1 | DC_n3A-n77A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n3A-n78A-n257A1 DC_n3A-n78A-n257G1 DC_n3A-n78A-n257H1 DC_n3A-n78A-n257I1 | DC_n3A-n78A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n3A-n78A-n258A DC_n3A-n78A-n258B DC_n3A-n78A-n258C DC_n3A-n78A-n258D DC_n3A-n78A-n258E DC_n3A-n78A-n258F DC_n3A-n78A-n258G DC_n3A-n78A-n258H DC_n3A-n78A-n258I DC_n3A-n78A-n258J DC_n3A-n78A-n258K DC_n3A-n78A-n258L DC_n3A-n78A-n258M | DC_n3A-n258A DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n3A-n78A |
| DC_n3A-n79A-n257A DC_n3A-n79A-n257G DC_n3A-n79A-n257H DC_n3A-n79A-n257I | DC_n3A-n79A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n5A-n30A-n260A DC_n5A-n30A-n260G DC_n5A-n30A-n260H DC_n5A-n30A-n260I DC_n5A-n30A-n260J DC_n5A-n30A-n260K DC_n5A-n30A-n260L DC_n5A-n30A-n260M | DC_n5A-n30A DC_n5A-n260A DC_n30A-n260A DC_n5A-n260G DC_n30A-n260G DC_n5A-n260H DC_n30A-n260H DC_n5A-n260I DC_n30A-n260I DC_n5A-n260J DC_n30A-n260J DC_n5A-n260K DC_n30A-n260K DC_n5A-n260L DC_n30A-n260L DC_n5A-n260M DC_n30A-n260M |
| DC_n5A-n48A-n260A DC_n5A-n48A-n260G DC_n5A-n48A-n260H DC_n5A-n48A-n260I DC_n5A-n48A-n260J DC_n5A-n48A-n260K DC_n5A-n48A-n260L DC_n5A-n48A-n260M | DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n5A-n48(2A)-n260A DC_n5A-n48(2A)-n260G DC_n5A-n48(2A)-n260H DC_n5A-n48(2A)-n260I DC_n5A-n48(2A)-n260J DC_n5A-n48(2A)-n260K DC_n5A-n48(2A)-n260L DC_n5A-n48(2A)-n260M | DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n5A-n48B-n260A DC_n5A-n48B-n260G DC_n5A-n48B-n260H DC_n5A-n48B-n260I DC_n5A-n48B-n260J DC_n5A-n48B-n260K DC_n5A-n48B-n260L DC_n5A-n48B-n260M | DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n5A-n48A-n261A DC_n5A-n48A-n261G DC_n5A-n48A-n261H DC_n5A-n48A-n261I DC_n5A-n48A-n261J DC_n5A-n48A-n261K DC_n5A-n48A-n261L DC_n5A-n48A-n261M | DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n5A-n48A-n261(G-H) DC_n5A-n48A-n261(A-G-H) DC_n5A-n48A-n261(2H) DC_n5A-n48A-n261(H-I) DC_n5A-n48A-n261(A-G-I) DC_n5A-n48A-n261(A-H) DC_n5A-n48A-n261(2G) DC_n5A-n48A-n261(2A-H) DC_n5A-n48A-n261(A-2G) DC_n5A-n48A-n261(G-I) DC_n5A-n48A-n261(2A-I) DC_n5A-n48A-n261(A-G) DC_n5A-n48A-n261(2A-G) DC_n5A-n48A-n261(A-I) DC_n5A-n48A-n261(2A) DC_n5A-n48A-n261(3A) | DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n5A-n48(2A)-n261A DC_n5A-n48(2A)-n261G DC_n5A-n48(2A)-n261H DC_n5A-n48(2A)-n261I DC_n5A-n48(2A)-n261J DC_n5A-n48(2A)-n261K DC_n5A-n48(2A)-n261L DC_n5A-n48(2A)-n261M | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n5A-n48(2A)-n261(G-H) DC_n5A-n48(2A)-n261(A-G-H) DC_n5A-n48(2A)-n261(2H) DC_n5A-n48(2A)-n261(H-I) DC_n5A-n48(2A)-n261(A-G-I) DC_n5A-n48(2A)-n261(A-H) DC_n5A-n48(2A)-n261(2G) DC_n5A-n48(2A)-n261(2A-H) DC_n5A-n48(2A)-n261(A-2G) DC_n5A-n48(2A)-n261(G-I) DC_n5A-n48(2A)-n261(2A-I) DC_n5A-n48(2A)-n261(A-G) DC_n5A-n48(2A)-n261(2A-G) DC_n5A-n48(2A)-n261(A-I) DC_n5A-n48(2A)-n261(2A) DC_n5A-n48(2A)-n261(3A) | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n5A-n48B-n261A DC_n5A-n48B-n261G DC_n5A-n48B-n261H DC_n5A-n48B-n261I DC_n5A-n48B-n261J DC_n5A-n48B-n261K DC_n5A-n48B-n261L DC_n5A-n48B-n261M | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n5A-n48B-n261(G-H) DC_n5A-n48B-n261(A-G-H) DC_n5A-n48B-n261(2H) DC_n5A-n48B-n261(H-I) DC_n5A-n48B-n261(A-G-I) DC_n5A-n48B-n261(A-H) DC_n5A-n48B-n261(2G) DC_n5A-n48B-n261(2A-H) DC_n5A-n48B-n261(A-2G) DC_n5A-n48B-n261(G-I) DC_n5A-n48B-n261(2A-I) DC_n5A-n48B-n261(A-G) DC_n5A-n48B-n261(2A-G) DC_n5A-n48B-n261(A-I) DC_n5A-n48B-n261(2A) DC_n5A-n48B-n261(3A) | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n5A-n66A-n260A DC_n5A-n66A-n260G DC_n5A-n66A-n260H DC_n5A-n66A-n260I DC_n5A-n66A-n260J DC_n5A-n66A-n260K DC_n5A-n66A-n260L DC_n5A-n66A-n260M | DC_n5A-n66A DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n5A-n260J DC_n5A-n260K DC_n5A-n260L DC_n5A-n260M DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I DC_n66A-n260J DC_n66A-n260K DC_n66A-n260L DC_n66A-n260M |
| DC_n5A-n66A-n261A DC_n5A-n66A-n261G DC_n5A-n66A-n261H DC_n5A-n66A-n261I DC_n5A-n66A-n261J DC_n5A-n66A-n261K DC_n5A-n66A-n261L DC_n5A-n66A-n261M | DC_n5A-n66A DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n5A-n66A-n261(2G) DC_n5A-n66A-n261(G-H) DC_n5A-n66A-n261(A-G-H) DC_n5A-n66A-n261(G-I) DC_n5A-n66A-n261(2H) DC_n5A-n66A-n261(A-G-I) DC_n5A-n66A-n261(H-I) DC_n5A-n66A-n261(2A-G) DC_n5A-n66A-n261(2A-H) DC_n5A-n66A-n261(2A-I) DC_n5A-n66A-n261(2A) DC_n5A-n66A-n261(3A) DC_n5A-n66A-n261(A-2G) DC_n5A-n66A-n261(A-G) DC_n5A-n66A-n261(A-H) DC_n5A-n66A-n261(A-I) | DC_n5A-n66A DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n5A-n77A-n260A DC_n5A-n77A-n260G DC_n5A-n77A-n260H DC_n5A-n77A-n260I DC_n5A-n77A-n260J DC_n5A-n77A-n260K DC_n5A-n77A-n260L DC_n5A-n77A-n260M DC_n5A-n77C-n260A DC_n5A-n77C-n260G DC_n5A-n77C-n260H DC_n5A-n77C-n260I DC_n5A-n77C-n260J DC_n5A-n77C-n260K DC_n5A-n77C-n260L DC_n5A-n77C-n260M | DC_n5A-n77A DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n5A-n260J DC_n5A-n260K DC_n5A-n260L DC_n5A-n260M DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I DC_n77A-n260J DC_n77A-n260K DC_n77A-n260L DC_n77A-n260M |
| DC_n5A-n77A-n261A DC_n5A-n77A-n261G DC_n5A-n77A-n261H DC_n5A-n77A-n261I DC_n5A-n77A-n261J DC_n5A-n77A-n261K DC_n5A-n77A-n261L DC_n5A-n77A-n261M DC_n5A-n77C-n261A DC_n5A-n77C-n261G DC_n5A-n77C-n261H DC_n5A-n77C-n261I DC_n5A-n77C-n261J DC_n5A-n77C-n261K DC_n5A-n77C-n261L DC_n5A-n77C-n261M | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n5A-n77A-n261(G-H) DC_n5A-n77A-n261(A-G-H) DC_n5A-n77A-n261(G-I) DC_n5A-n77A-n261(2H) DC_n5A-n77A-n261(A-G-I) DC_n5A-n77A-n261(H-I) DC_n5A-n77A-n261(A-H) DC_n5A-n77A-n261(2G) DC_n5A-n77A-n261(2A-H) DC_n5A-n77A-n261(A-2G) DC_n5A-n77A-n261(A-I) DC_n5A-n77A-n261(2A-I) DC_n5A-n77A-n261(A-G) DC_n5A-n77A-n261(2A-G) DC_n5A-n77A-n261(2A) DC_n5A-n77A-n261(3A) DC_n5A-n77C-n261(G-H) DC_n5A-n77C-n261(A-G-H) DC_n5A-n77C-n261(G-I) DC_n5A-n77C-n261(2H) DC_n5A-n77C-n261(A-G-I) DC_n5A-n77C-n261(H-I) DC_n5A-n77C-n261(A-H) DC_n5A-n77C-n261(2G) DC_n5A-n77C-n261(2A-H) DC_n5A-n77C-n261(A-2G) DC_n5A-n77C-n261(A-I) DC_n5A-n77C-n261(2A-I) DC_n5A-n77C-n261(A-G) DC_n5A-n77C-n261(2A-G) DC_n5A-n77C-n261(2A) DC_n5A-n77C-n261(3A) | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n7A-n25A-n257A DC_n7A-n25A-n257G DC_n7A-n25A-n257H DC_n7A-n25A-n257I DC_n7A-n25A-n257J DC_n7A-n25A-n257K DC_n7A-n25A-n257L DC_n7A-n25A-n257M | DC_n7A-n257A DC_n7A-n257G DC_n7A-n257H DC_n7A-n257I DC_n7A-n257J DC_n7A-n257K DC_n7A-n257L DC_n7A-n257M DC_n25A-n257A DC_n25A-n257G DC_n25A-n257H DC_n25A-n257I DC_n25A-n257J DC_n25A-n257K DC_n25A-n257L DC_n25A-n257M |
| DC_n7A-n25A-n260A DC_n7A-n25A-n260G DC_n7A-n25A-n260H DC_n7A-n25A-n260I DC_n7A-n25A-n260J DC_n7A-n25A-n260K DC_n7A-n25A-n260L DC_n7A-n25A-n260M | DC_n7A-n260A DC_n7A-n260G DC_n7A-n260H DC_n7A-n260I DC_n7A-n260J DC_n7A-n260K DC_n7A-n260L DC_n7A-n260M DC_n25A-n260A DC_n25A-n260G DC_n25A-n260H DC_n25A-n260I DC_n25A-n260J DC_n25A-n260K DC_n25A-n260L DC_n25A-n260M |
| DC_n7A-n66A-n257A DC_n7A-n66A-n257G DC_n7A-n66A-n257H DC_n7A-n66A-n257I | DC_n7A-n257A DC_n7A-n257G DC_n7A-n257H DC_n7A-n257I DC_n66A-n257A DC_n66A-n257G DC_n66A-n257H DC_n66A-n257I |
| DC_n7A-n71A-n257A DC_n7A-n71A-n257G DC_n7A-n71A-n257H DC_n7A-n71A-n257I | DC_n7A-n257A DC_n7A-n257G DC_n7A-n257H DC_n7A-n257I DC_n71A-n257A DC_n71A-n257G DC_n71A-n257H DC_n71A-n257I |
| DC_n7A-n78A-n258A DC_n7A-n78A-n258B DC_n7A-n78A-n258C DC_n7A-n78A-n258D DC_n7A-n78A-n258E DC_n7A-n78A-n258F DC_n7A-n78A-n258G DC_n7A-n78A-n258H DC_n7A-n78A-n258I DC_n7A-n78A-n258J DC_n7A-n78A-n258K DC_n7A-n78A-n258L DC_n7A-n78A-n258M | DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n7A-n78A |
| DC_n7B-n78A-n258A DC_n7B-n78A-n258B DC_n7B-n78A-n258C DC_n7B-n78A-n258D DC_n7B-n78A-n258E DC_n7B-n78A-n258F DC_n7B-n78A-n258G DC_n7B-n78A-n258H DC_n7B-n78A-n258I DC_n7B-n78A-n258J DC_n7B-n78A-n258K DC_n7B-n78A-n258L DC_n7B-n78A-n258M | DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n7A-n78A |
| DC_n12A-n30A-n260A DC_n12A-n30A-n260G DC_n12A-n30A-n260H DC_n12A-n30A-n260I DC_n12A-n30A-n260J DC_n12A-n30A-n260K DC_n12A-n30A-n260L DC_n12A-n30A-n260M | DC_n12A-n30A DC_n12A-n260A DC_n30A-n260A DC_n12A-n260G DC_n30A-n260G DC_n12A-n260H DC_n30A-n260H DC_n12A-n260I DC_n30A-n260I DC_n12A-n260J DC_n30A-n260J DC_n12A-n260K DC_n30A-n260K DC_n12A-n260L DC_n30A-n260L DC_n12A-n260M DC_n30A-n260M |
| DC_n12A-n66A-n260A DC_n12A-n66A-n260G DC_n12A-n66A-n260H DC_n12A-n66A-n260I DC_n12A-n66A-n260J DC_n12A-n66A-n260K DC_n12A-n66A-n260L DC_n12A-n66A-n260M | DC_n12A-n66A DC_n12A-n260A DC_n66A-n260A DC_n12A-n260G DC_n66A-n260G DC_n12A-n260H DC_n66A-n260H DC_n12A-n260I DC_n66A-n260I DC_n12A-n260J DC_n66A-n260J DC_n12A-n260K DC_n66A-n260K DC_n12A-n260L DC_n66A-n260L DC_n12A-n260M DC_n66A-n260M |
| DC_n12A-n77A-n260A DC_n12A-n77A-n260G DC_n12A-n77A-n260H DC_n12A-n77A-n260I DC_n12A-n77A-n260J DC_n12A-n77A-n260K DC_n12A-n77A-n260L DC_n12A-n77A-n260M | DC_n12A-n77A DC_n12A-n260A DC_n77A-n260A DC_n12A-n260G DC_n77A-n260G DC_n12A-n260H DC_n77A-n260H DC_n12A-n260I DC_n77A-n260I DC_n12A-n260J DC_n77A-n260J DC_n12A-n260K DC_n77A-n260K DC_n12A-n260L DC_n77A-n260L DC_n12A-n260M DC_n77A-n260M |
| DC_n14A-n30A-n260A DC_n14A-n30A-n260G DC_n14A-n30A-n260H DC_n14A-n30A-n260I DC_n14A-n30A-n260J DC_n14A-n30A-n260K DC_n14A-n30A-n260L DC_n14A-n30A-n260M | DC_n14A-n30A DC_n14A-n260A DC_n30A-n260A DC_n14A-n260G DC_n30A-n260G DC_n14A-n260H DC_n30A-n260H DC_n14A-n260I DC_n30A-n260I DC_n14A-n260J DC_n30A-n260J DC_n14A-n260K DC_n30A-n260K DC_n14A-n260L DC_n30A-n260L DC_n14A-n260M DC_n30A-n260M |
| DC_n14A-n66A-n260A DC_n14A-n66A-n260G DC_n14A-n66A-n260H DC_n14A-n66A-n260I DC_n14A-n66A-n260J DC_n14A-n66A-n260K DC_n14A-n66A-n260L DC_n14A-n66A-n260M | DC_n14A-n66A DC_n14A-n260A DC_n66A-n260A DC_n14A-n260G DC_n66A-n260G DC_n14A-n260H DC_n66A-n260H DC_n14A-n260I DC_n66A-n260I DC_n14A-n260J DC_n66A-n260J DC_n14A-n260K DC_n66A-n260K DC_n14A-n260L DC_n66A-n260L DC_n14A-n260M DC_n66A-n260M |
| DC_n14A-n77A-n260A DC_n14A-n77A-n260G DC_n14A-n77A-n260H DC_n14A-n77A-n260I DC_n14A-n77A-n260J DC_n14A-n77A-n260K DC_n14A-n77A-n260L DC_n14A-n77A-n260M | DC_n14A-n77A DC_n14A-n260A DC_n77A-n260A DC_n14A-n260G DC_n77A-n260G DC_n14A-n260H DC_n77A-n260H DC_n14A-n260I DC_n77A-n260I DC_n14A-n260J DC_n77A-n260J DC_n14A-n260K DC_n77A-n260K DC_n14A-n260L DC_n77A-n260L DC_n14A-n260M DC_n77A-n260M |
| DC_n18A-n28A-n257A DC_n18A-n28A-n257G DC_n18A-n28A-n257H DC_n18A-n28A-n257I | DC_n18A-n28A DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I |
| DC_n18A-n41A-n257A DC_n18A-n41A-n257G DC_n18A-n41A-n257H DC_n18A-n41A-n257I | DC_n18A-n41A DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |
| DC_n18A-n77A-n257A DC_n18A-n77A-n257G DC_n18A-n77A-n257H DC_n18A-n77A-n257I | DC_n18A-n77A DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n18A-n77(2A)-n257A DC_n18A-n77(2A)-n257G DC_n18A-n77(2A)-n257H DC_n18A-n77(2A)-n257I | DC_n18A-n77A DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n18A-n78A-n257A DC_n18A-n78A-n257G DC_n18A-n78A-n257H DC_n18A-n78A-n257I | DC_n18A-n78A DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n25A-n41A-n260A DC_n25A-n41A-n260G DC_n25A-n41A-n260H DC_n25A-n41A-n260I DC_n25A-n41A-n260(2A) | DC_n25A-n260A DC_n41A-n260A |
| DC_n26A-n78A-n258A DC_n26A-n78A-n258B DC_n26A-n78A-n258C DC_n26A-n78A-n258D DC_n26A-n78A-n258E DC_n26A-n78A-n258F DC_n26A-n78A-n258G DC_n26A-n78A-n258H DC_n26A-n78A-n258I DC_n26A-n78A-n258J DC_n26A-n78A-n258K DC_n26A-n78A-n258L DC_n26A-n78A-n258M | DC_n26A-n258A DC_n26A-n258G DC_n26A-n258H DC_n26A-n258I DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I |
| DC_n28A-n41A-n257A DC_n28A-n41A-n257G DC_n28A-n41A-n257H DC_n28A-n41A-n257I | DC_n28A-n41A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |
| DC_n28A-n77A-n257A1 DC_n28A-n77A-n257G1 DC_n28A-n77A-n257H1 DC_n28A-n77A-n257I1 | DC_n28A-n77A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n28A-n77(2A)-n257A DC_n28A-n77(2A)-n257G DC_n28A-n77(2A)-n257H DC_n28A-n77(2A)-n257I | DC_n28A-n77A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n28A-n78A-n257A1 DC_n28A-n78A-n257G1 DC_n28A-n78A-n257H1 DC_n28A-n78A-n257I1 | DC_n28A-n78A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n28A-n78A-n258A DC_n28A-n78A-n258B DC_n28A-n78A-n258C DC_n28A-n78A-n258D DC_n28A-n78A-n258E DC_n28A-n78A-n258F DC_n28A-n78A-n258G DC_n28A-n78A-n258H DC_n28A-n78A-n258I DC_n28A-n78A-n258J DC_n28A-n78A-n258K DC_n28A-n78A-n258L DC_n28A-n78A-n258M | DC_n28A-n258A DC_n28A-n258G DC_n28A-n258H DC_n28A-n258I DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I |
| DC_n28A-n79A-n257A1 DC_n28A-n79A-n257G1 DC_n28A-n79A-n257H1 DC_n28A-n79A-n257I1 | DC_n28A-n79A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n30A-n66A-n260A DC_n30A-n66A-n260G DC_n30A-n66A-n260H DC_n30A-n66A-n260I DC_n30A-n66A-n260J DC_n30A-n66A-n260K DC_n30A-n66A-n260L DC_n30A-n66A-n260M | DC_n30A-n66A DC_n30A-n260A DC_n30A-n260G DC_n30A-n260H DC_n30A-n260I DC_n30A-n260J DC_n30A-n260K DC_n30A-n260L DC_n30A-n260M DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I DC_n66A-n260J DC_n66A-n260K DC_n66A-n260L DC_n66A-n260M |
| DC_n30A-n77A-n260A DC_n30A-n77A-n260G DC_n30A-n77A-n260H DC_n30A-n77A-n260I DC_n30A-n77A-n260J DC_n30A-n77A-n260K DC_n30A-n77A-n260L DC_n30A-n77A-n260M | DC_n30A-n77A DC_n30A-n260A DC_n77A-n260A DC_n30A-n260G DC_n77A-n260G DC_n30A-n260H DC_n77A-n260H DC_n30A-n260I DC_n77A-n260I DC_n30A-n260J DC_n77A-n260J DC_n30A-n260K DC_n77A-n260K DC_n30A-n260L DC_n77A-n260L DC_n30A-n260M DC_n77A-n260M |
| DC_n40A-n77A-n257A DC_n40A-n77A-n257D DC_n40A-n77A-n257E DC_n40A-n77A-n257F DC_n40A-n77A-n257G DC_n40A-n77A-n257H DC_n40A-n77A-n257I DC_n40A-n77A-n257J DC_n40A-n77A-n257K DC_n40A-n77A-n257L DC_n40A-n77A-n257M DC_n40A-n77C-n257A DC_n40A-n77C-n257D DC_n40A-n77C-n257E DC_n40A-n77C-n257F | DC_n40A-n77A DC_n40A-n257A DC_n40A-n257D DC_n40A-n257E DC_n40A-n257F DC_n40A-n257G DC_n40A-n257H DC_n40A-n257I DC_n40A-n257J DC_n40A-n257K DC_n40A-n257L DC_n40A-n257M DC_n77A-n257A DC_n77A-n257E DC_n77A-n257F DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n77A-n257J DC_n77A-n257K DC_n77A-n257L DC_n77A-n257M |
| DC_n40A-n78A-n257A DC_n40A-n78A-n257D DC_n40A-n78A-n257E DC_n40A-n78A-n257F DC_n40A-n78A-n257G DC_n40A-n78A-n257H DC_n40A-n78A-n257I DC_n40A-n78A-n257J DC_n40A-n78A-n257K DC_n40A-n78A-n257L DC_n40A-n78A-n257M DC_n40A-n78C-n257A DC_n40A-n78C-n257D DC_n40A-n78C-n257E DC_n40A-n78C-n257F DC_n40A-n78C-n257G DC_n40A-n78C-n257H DC_n40A-n78C-n257I DC_n40A-n78C-n257J DC_n40A-n78C-n257K DC_n40A-n78C-n257L DC_n40A-n78C-n257M | DC_n40A-n78A DC_n40A-n257A DC_n40A-n257D DC_n40A-n257E DC_n40A-n257F DC_n40A-n257G DC_n40A-n257H DC_n40A-n257I DC_n40A-n257J DC_n40A-n257K DC_n40A-n257L DC_n40A-n257M DC_n78A-n257A DC_n78A-n257E DC_n78A-n257F DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n78A-n257J DC_n78A-n257K DC_n78A-n257L DC_n78A-n257M |
| DC_n41A-n66A-n260A DC_n41A-n66A-n260G DC_n41A-n66A-n260H DC_n41A-n66A-n260I | DC_n41A-n260A DC_n41A-n260G DC_n41A-n260H DC_n41A-n260I DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I |
| DC_n41A-n66A-n260(2A) | DC_n41A-n260A DC_n66A-n260A |
| DC_n41A-n77A-n257A DC_n41A-n77A-n257G DC_n41A-n77A-n257H DC_n41A-n77A-n257I | DC_n41A-n77A DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n41A-n77(2A)-n257A DC_n41A-n77(2A)-n257G DC_n41A-n77(2A)-n257H DC_n41A-n77(2A)-n257I | DC_n41A-n77A DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n41A-n78A-n257A DC_n41A-n78A-n257G DC_n41A-n78A-n257H DC_n41A-n78A-n257I | DC_n41A-n78A DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n48A-n66A-n260A DC_n48A-n66A-n260G DC_n48A-n66A-n260H DC_n48A-n66A-n260I DC_n48A-n66A-n260J DC_n48A-n66A-n260K DC_n48A-n66A-n260L DC_n48A-n66A-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I |
| DC_n48(2A)-n66A-n260A DC_n48(2A)-n66A-n260G DC_n48(2A)-n66A-n260H DC_n48(2A)-n66A-n260I DC_n48(2A)-n66A-n260J DC_n48(2A)-n66A-n260K DC_n48(2A)-n66A-n260L DC_n48(2A)-n66A-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I |
| DC_n48B-n66A-n260A DC_n48B-n66A-n260G DC_n48B-n66A-n260H DC_n48B-n66A-n260I DC_n48B-n66A-n260J DC_n48B-n66A-n260K DC_n48B-n66A-n260L DC_n48B-n66A-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I |
| DC_n48A-n77A-n260A DC_n48A-n77A-n260G DC_n48A-n77A-n260H DC_n48A-n77A-n260I DC_n48A-n77A-n260J DC_n48A-n77A-n260K DC_n48A-n77A-n260L DC_n48A-n77A-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I |
| DC_n48A-n77C-n260A DC_n48A-n77C-n260G DC_n48A-n77C-n260H DC_n48A-n77C-n260I DC_n48A-n77C-n260J DC_n48A-n77C-n260K DC_n48A-n77C-n260L DC_n48A-n77C-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I |
| DC_n48A-n66A-n261A DC_n48A-n66A-n261G DC_n48A-n66A-n261H DC_n48A-n66A-n261I DC_n48A-n66A-n261J DC_n48A-n66A-n261K DC_n48A-n66A-n261L DC_n48A-n66A-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n48A-n66A-n261(G-H) DC_n48A-n66A-n261(A-G-H) DC_n48A-n66A-n261(2H) DC_n48A-n66A-n261(H-I) DC_n48A-n66A-n261(A-G-I) DC_n48A-n66A-n261(A-H) DC_n48A-n66A-n261(2G) DC_n48A-n66A-n261(2A-H) DC_n48A-n66A-n261(A-2G) DC_n48A-n66A-n261(G-I) DC_n48A-n66A-n261(2A-I) DC_n48A-n66A-n261(A-G) DC_n48A-n66A-n261(2A-G) DC_n48A-n66A-n261(A-I) DC_n48A-n66A-n261(2A) DC_n48A-n66A-n261(3A) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n48(2A)-n66A-n261A DC_n48(2A)-n66A-n261G DC_n48(2A)-n66A-n261H DC_n48(2A)-n66A-n261I DC_n48(2A)-n66A-n261J DC_n48(2A)-n66A-n261K DC_n48(2A)-n66A-n261L DC_n48(2A)-n66A-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n48(2A)-n66A-n261(G-H) DC_n48(2A)-n66A-n261(A-G-H) DC_n48(2A)-n66A-n261(2H) DC_n48(2A)-n66A-n261(H-I) DC_n48(2A)-n66A-n261(A-G-I) DC_n48(2A)-n66A-n261(A-H) DC_n48(2A)-n66A-n261(2G) DC_n48(2A)-n66A-n261(2A-H) DC_n48(2A)-n66A-n261(A-2G) DC_n48(2A)-n66A-n261(G-I) DC_n48(2A)-n66A-n261(2A-I) DC_n48(2A)-n66A-n261(A-G) DC_n48(2A)-n66A-n261(2A-G) DC_n48(2A)-n66A-n261(A-I) DC_n48(2A)-n66A-n261(2A) DC_n48(2A)-n66A-n261(3A) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n48A-n77A-n261A DC_n48A-n77A-n261G DC_n48A-n77A-n261H DC_n48A-n77A-n261I DC_n48A-n77A-n261J DC_n48A-n77A-n261K DC_n48A-n77A-n261L DC_n48A-n77A-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n48A-n77A-n261(G-H) DC_n48A-n77A-n261(2H) DC_n48A-n77A-n261(A-G-H) DC_n48A-n77A-n261(H-I) DC_n48A-n77A-n261(A-G-I) DC_n48A-n77A-n261(A-H) DC_n48A-n77A-n261(2G) DC_n48A-n77A-n261(2A-H) DC_n48A-n77A-n261(A-2G) DC_n48A-n77A-n261(G-I) DC_n48A-n77A-n261(2A-I) DC_n48A-n77A-n261(A-G) DC_n48A-n77A-n261(2A-G) DC_n48A-n77A-n261(A-I) DC_n48A-n77A-n261(2A) DC_n48A-n77A-n261(3A) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n48A-n77C-n261A DC_n48A-n77C-n261G DC_n48A-n77C-n261H DC_n48A-n77C-n261I DC_n48A-n77C-n261J DC_n48A-n77C-n261K DC_n48A-n77C-n261L DC_n48A-n77C-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n48A-n77C-n261(G-H) DC_n48A-n77C-n261(2H) DC_n48A-n77C-n261(A-G-H) DC_n48A-n77C-n261(H-I) DC_n48A-n77C-n261(A-G-I) DC_n48A-n77C-n261(A-H) DC_n48A-n77C-n261(2G) DC_n48A-n77C-n261(2A-H) DC_n48A-n77C-n261(A-2G) DC_n48A-n77C-n261(G-I) DC_n48A-n77C-n261(2A-I) DC_n48A-n77C-n261(A-G) DC_n48A-n77C-n261(2A-G) DC_n48A-n77C-n261(A-I) DC_n48A-n77C-n261(2A) DC_n48A-n77C-n261(3A) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n48B-n66A-n261A DC_n48B-n66A-n261G DC_n48B-n66A-n261H DC_n48B-n66A-n261I DC_n48B-n66A-n261J DC_n48B-n66A-n261K DC_n48B-n66A-n261L DC_n48B-n66A-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n48B-n66A-n261(G-H) DC_n48B-n66A-n261(A-G-H) DC_n48B-n66A-n261(2H) DC_n48B-n66A-n261(H-I) DC_n48B-n66A-n261(A-G-I) DC_n48B-n66A-n261(A-H) DC_n48B-n66A-n261(2G) DC_n48B-n66A-n261(2A-H) DC_n48B-n66A-n261(A-2G) DC_n48B-n66A-n261(G-I) DC_n48B-n66A-n261(2A-I) DC_n48B-n66A-n261(A-G) DC_n48B-n66A-n261(2A-G) DC_n48B-n66A-n261(A-I) DC_n48B-n66A-n261(2A) DC_n48B-n66A-n261(3A) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n66A-n77A-n260A DC_n66A-n77A-n260G DC_n66A-n77A-n260H DC_n66A-n77A-n260I DC_n66A-n77A-n260J DC_n66A-n77A-n260K DC_n66A-n77A-n260L DC_n66A-n77A-n260M DC_n66A-n77C-n260A DC_n66A-n77C-n260G DC_n66A-n77C-n260H DC_n66A-n77C-n260I DC_n66A-n77C-n260J DC_n66A-n77C-n260K DC_n66A-n77C-n260L DC_n66A-n77C-n260M | DC_n66A-n77A DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I DC_n66A-n260J DC_n66A-n260K DC_n66A-n260L DC_n66A-n260M DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I DC_n77A-n260J DC_n77A-n260K DC_n77A-n260L DC_n77A-n260M |
| DC_n66A-n77A-n261A DC_n66A-n77A-n261G DC_n66A-n77A-n261H DC_n66A-n77A-n261I DC_n66A-n77A-n261J DC_n66A-n77A-n261K DC_n66A-n77A-n261L DC_n66A-n77A-n261M DC_n66A-n77C-n261A DC_n66A-n77C-n261G DC_n66A-n77C-n261H DC_n66A-n77C-n261I DC_n66A-n77C-n261J DC_n66A-n77C-n261K DC_n66A-n77C-n261L DC_n66A-n77C-n261M | DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n66A-n77A-n261(G-H) DC_n66A-n77A-n261(A-G-H) DC_n66A-n77A-n261(G-I) DC_n66A-n77A-n261(2H) DC_n66A-n77A-n261(A-G-I) DC_n66A-n77A-n261(H-I) DC_n66A-n77A-n261(A-H) DC_n66A-n77A-n261(2G) DC_n66A-n77A-n261(2A-H) DC_n66A-n77A-n261(A-2G) DC_n66A-n77A-n261(A-I) DC_n66A-n77A-n261(2A-I) DC_n66A-n77A-n261(A-G) DC_n66A-n77A-n261(2A-G) DC_n66A-n77A-n261(2A) DC_n66A-n77A-n261(3A) DC_n66A-n77C-n261(G-H) DC_n66A-n77C-n261(A-G-H) DC_n66A-n77C-n261(G-I) DC_n66A-n77C-n261(2H) DC_n66A-n77C-n261(A-G-I) DC_n66A-n77C-n261(H-I) DC_n66A-n77C-n261(A-H) DC_n66A-n77C-n261(2G) DC_n66A-n77C-n261(2A-H) DC_n66A-n77C-n261(A-2G) DC_n66A-n77C-n261(A-I) DC_n66A-n77C-n261(2A-I) DC_n66A-n77C-n261(A-G) DC_n66A-n77C-n261(2A-G) DC_n66A-n77C-n261(2A) DC_n66A-n77C-n261(3A) | DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n77A-n79A-n257A DC_n77A-n79A-n257G DC_n77A-n79A-n257H DC_n77A-n79A-n257I | DC_n77A-n79A DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n77(2A)-n79A-n257A DC_n77(2A)-n79A-n257G DC_n77(2A)-n79A-n257H DC_n77(2A)-n79A-n257I | DC_n77A-n79A DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n77A-n79A-n259A DC_n77A-n79A-n259G DC_n77A-n79A-n259H DC_n77A-n79A-n259I DC_n77A-n79A-n259J DC_n77A-n79A-n259K DC_n77A-n79A-n259L DC_n77A-n79A-n259M | DC_n77A-n79A DC_n77A-n259A DC_n77A-n259G DC_n77A-n259H DC_n77A-n259I DC_n77A-n259J DC_n77A-n259K DC_n77A-n259L DC_n77A-n259M DC_n79A-n259A DC_n79A-n259G DC_n79A-n259H DC_n79A-n259I DC_n79A-n259J DC_n79A-n259K DC_n79A-n259L DC_n79A-n259M |
| DC_n77A-n257A-n259A1 DC_n77A-n257A-n259G1 DC_n77A-n257A-n259H1 DC_n77A-n257A-n259I1 DC_n77A-n257A-n259J1 DC_n77A-n257A-n259K1 DC_n77A-n257A-n259L1 DC_n77A-n257A-n259M1 DC_n77A-n257G-n259A1 DC_n77A-n257G-n259G1 DC_n77A-n257G-n259H1 DC_n77A-n257G-n259I1 DC_n77A-n257G-n259J1 DC_n77A-n257G-n259K1 DC_n77A-n257G-n259L1 DC_n77A-n257G-n259M1 DC_n77A-n257H-n259A1 DC_n77A-n257H-n259G1 DC_n77A-n257H-n259H1 DC_n77A-n257H-n259I1 DC_n77A-n257H-n259J1 DC_n77A-n257H-n259K1 DC_n77A-n257H-n259L1 DC_n77A-n257H-n259M1 DC_n77A-n257I-n259A1 DC_n77A-n257I-n259G1 DC_n77A-n257I-n259H1 DC_n77A-n257I-n259I1 DC_n77A-n257I-n259J1 DC_n77A-n257I-n259K1 DC_n77A-n257I-n259L1 DC_n77A-n257I-n259M1 | DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n77A-n259A DC_n77A-n259G DC_n77A-n259H DC_n77A-n259I DC_n77A-n259J DC_n77A-n259K DC_n77A-n259L DC_n77A-n259M |
| DC_n77A-n79A-n258A DC_n77A-n79A-n258D DC_n77A-n79A-n258G DC_n77A-n79A-n258H DC_n77A-n79A-n258I DC_n77A-n79A-n258J | DC_n77A-n79A DC_n77A-n258A DC_n77A-n258D DC_n77A-n258G DC_n77A-n258H DC_n77A-n258I DC_n77A-n258J DC_n79A-n258A DC_n79A-n258D DC_n79A-n258G DC_n79A-n258H DC_n79A-n258I DC_n79A-n258J |
| DC_n77(2A)-n79A-n258A DC_n77(2A)-n79A-n258D DC_n77(2A)-n79A-n258G DC_n77(2A)-n79A-n258H DC_n77(2A)-n79A-n258I DC_n77(2A)-n79A-n258J | DC_n77A-n79A DC_n77A-n258A DC_n77A-n258D DC_n77A-n258G DC_n77A-n258H DC_n77A-n258I DC_n77A-n258J DC_n79A-n258A DC_n79A-n258D DC_n79A-n258G DC_n79A-n258H DC_n79A-n258I DC_n79A-n258J |
| DC_n78A-n79A-n257A DC_n78A-n79A-n257G DC_n78A-n79A-n257H DC_n78A-n79A-n257I | DC_n78A-n79A DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n78(2A)-n79A-n257A DC_n78(2A)-n79A-n257G DC_n78(2A)-n79A-n257H DC_n78(2A)-n79A-n257I | DC_n78A-n79A DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n78A-n79A-n259A DC_n78A-n79A-n259G DC_n78A-n79A-n259H DC_n78A-n79A-n259I DC_n78A-n79A-n259J DC_n78A-n79A-n259K DC_n78A-n79A-n259L DC_n78A-n79A-n259M | DC_n78A-n79A DC_n78A-n259A DC_n78A-n259G DC_n78A-n259H DC_n78A-n259I DC_n78A-n259J DC_n78A-n259K DC_n78A-n259L DC_n78A-n259M DC_n79A-n259A DC_n79A-n259G DC_n79A-n259H DC_n79A-n259I DC_n79A-n259J DC_n79A-n259K DC_n79A-n259L DC_n79A-n259M |
| NOTE 1: Applicable for UE supporting inter-band NR DC with mandatory simultaneous Rx/Tx capability. |  |

#### 5.5B.7.3	Inter-band NR-DC configurations between FR1 and FR2 (four bands)

Table 5.5B.7-3: Inter-band NR-DC configurations between FR1 and FR2 (four bands)

| Downlink NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n1A-n77A-n79A-n257A DC_n1A-n77A-n79A-n257G DC_n1A-n77A-n79A-n257H DC_n1A-n77A-n79A-n257I | DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n1A-n78A-n79A-n257A DC_n1A-n78A-n79A-n257G DC_n1A-n78A-n79A-n257H DC_n1A-n78A-n79A-n257I | DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n2A-n5A-n48A-n260A DC_n2A-n5A-n48A-n260G DC_n2A-n5A-n48A-n260H DC_n2A-n5A-n48A-n260I DC_n2A-n5A-n48A-n260J DC_n2A-n5A-n48A-n260K DC_n2A-n5A-n48A-n260L DC_n2A-n5A-n48A-n260M | DC_n2A-n260A DC_n5A-n260A DC_n48A-n260A DC_n2A-n260G DC_n5A-n260G DC_n48A-n260G DC_n2A-n260H DC_n5A-n260H DC_n48A-n260H DC_n2A-n260I DC_n5A-n260I DC_n48A-n260I |
| DC_n2A-n5A-n48A-n261A DC_n2A-n5A-n48A-n261G DC_n2A-n5A-n48A-n261H DC_n2A-n5A-n48A-n261I DC_n2A-n5A-n48A-n261J DC_n2A-n5A-n48A-n261K DC_n2A-n5A-n48A-n261L DC_n2A-n5A-n48A-n261M DC_n2A-n5A-n48A-n261(A-G) DC_n2A-n5A-n48A-n261(A-H) DC_n2A-n5A-n48A-n261(A-I) DC_n2A-n5A-n48A-n261(A-2G) DC_n2A-n5A-n48A-n261(2A-G) DC_n2A-n5A-n48A-n261(2A-H) DC_n2A-n5A-n48A-n261(2A-I) DC_n2A-n5A-n48A-n261(G-H) DC_n2A-n5A-n48A-n261(G-I) DC_n2A-n5A-n48A-n261(2A) DC_n2A-n5A-n48A-n261(3A) DC_n2A-n5A-n48A-n261(2G) DC_n2A-n5A-n48A-n261(2H) DC_n2A-n5A-n48A-n261(A-G-H) DC_n2A-n5A-n48A-n261(H-I) DC_n2A-n5A-n48A-n261(A-G-I) | DC_n2A-n261A DC_n5A-n261A DC_n48A-n261A DC_n2A-n261G DC_n5A-n261G DC_n48A-n261G DC_n2A-n261H DC_n5A-n261H DC_n48A-n261H DC_n2A-n261I DC_n5A-n261I DC_n48A-n261I |
| DC_n2A-n5A-n66A-n260A DC_n2A-n5A-n77A-n260G DC_n2A-n5A-n77A-n260H DC_n2A-n5A-n66A-n260G DC_n2A-n5A-n66A-n260H DC_n2A-n5A-n66A-n260I DC_n2A-n5A-n66A-n260J DC_n2A-n5A-n66A-n260K DC_n2A-n5A-n66A-n260L DC_n2A-n5A-n66A-n260M | DC_n2A-n260A DC_n5A-n260A DC_n77A-n260A DC_n2A-n260G DC_n5A-n260G DC_n77A-n260G DC_n2A-n260H DC_n5A-n260H DC_n77A-n260H DC_n2A-n260I DC_n5A-n260I DC_n77A-n260I |
| DC_n2A-n5A-n66A-n261A DC_n2A-n5A-n66A-n261G DC_n2A-n5A-n66A-n261H DC_n2A-n5A-n66A-n261I DC_n2A-n5A-n66A-n261J DC_n2A-n5A-n66A-n261K DC_n2A-n5A-n66A-n261L DC_n2A-n5A-n66A-n261M DC_n2A-n5A-n66A-n261(2G) DC_n2A-n5A-n66A-n261(G-H) DC_n2A-n5A-n66A-n261(A-G-H) DC_n2A-n5A-n66A-n261(G-I) DC_n2A-n5A-n66A-n261(2H) DC_n2A-n5A-n66A-n261(A-G-I) DC_n2A-n5A-n66A-n261(H-I) | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n2A-n5A-n77A-n260A DC_n2A-n5A-n77A-n260G DC_n2A-n5A-n77A-n260H DC_n2A-n5A-n77A-n260I DC_n2A-n5A-n77A-n260J DC_n2A-n5A-n77A-n260K DC_n2A-n5A-n77A-n260L DC_n2A-n5A-n77A-n260M | DC_n2A-n260A DC_n5A-n260A DC_n77A-n260A DC_n2A-n260G DC_n5A-n260G DC_n77A-n260G DC_n2A-n260H DC_n5A-n260H DC_n77A-n260H DC_n2A-n260I DC_n5A-n260I DC_n77A-n260I |
| DC_n2A-n5A-n77A-n261A DC_n2A-n5A-n77A-n261G DC_n2A-n5A-n77A-n261H DC_n2A-n5A-n77A-n261I DC_n2A-n5A-n77A-n261J DC_n2A-n5A-n77A-n261K DC_n2A-n5A-n77A-n261L DC_n2A-n5A-n77A-n261M DC_n2A-n5A-n77A-n261(A-G) DC_n2A-n5A-n77A-n261(A-H) DC_n2A-n5A-n77A-n261(A-I) DC_n2A-n5A-n77A-n261(A-2G) DC_n2A-n5A-n77A-n261(2A-G) DC_n2A-n5A-n77A-n261(2A-H) DC_n2A-n5A-n77A-n261(2A-I) DC_n2A-n5A-n77A-n261(G-H) DC_n2A-n5A-n77A-n261(2A) DC_n2A-n5A-n77A-n261(3A) DC_n2A-n5A-n77A-n261(2G) DC_n2A-n5A-n77A-n261(2H) DC_n2A-n5A-n77A-n261(A-G-H) DC_n2A-n5A-n77A-n261(H-I) DC_n2A-n5A-n77A-n261(A-G-I) | DC_n2A-n261A DC_n5A-n261A DC_n77A-n261A DC_n2A-n261G DC_n5A-n261G DC_n77A-n261G DC_n2A-n261H DC_n5A-n261H DC_n77A-n261H DC_n2A-n261I DC_n5A-n261I DC_n77A-n261I |
| DC_n2A-n48A-n66A-n260A DC_n2A-n48A-n66A-n260G DC_n2A-n48A-n66A-n260H DC_n2A-n48A-n66A-n260I DC_n2A-n48A-n66A-n260J DC_n2A-n48A-n66A-n260K DC_n2A-n48A-n66A-n260L DC_n2A-n48A-n66A-n260M | DC_n2A-n260A DC_n66A-n260A DC_n48A-n260A DC_n2A-n260G DC_n66A-n260G DC_n48A-n260G DC_n2A-n260H DC_n66A-n260H DC_n48A-n260H DC_n2A-n260I DC_n66A-n260I DC_n48A-n260I |
| DC_n2A-n48A-n66A-n261A DC_n2A-n48A-n66A-n261G DC_n2A-n48A-n66A-n261H DC_n2A-n48A-n66A-n261I DC_n2A-n48A-n66A-n261J DC_n2A-n48A-n66A-n261K DC_n2A-n48A-n66A-n261L DC_n2A-n48A-n66A-n261M DC_n2A-n48A-n66A-n261(A-G) DC_n2A-n48A-n66A-n261(A-H) DC_n2A-n48A-n66A-n261(A-I) DC_n2A-n48A-n66A-n261(A-2G) DC_n2A-n48A-n66A-n261(2A-G) DC_n2A-n48A-n66A-n261(2A-H) DC_n2A-n48A-n66A-n261(2A-I) DC_n2A-n48A-n66A-n261(G-H) DC_n2A-n48A-n66A-n261(2A) DC_n2A-n48A-n66A-n261(3A) DC_n2A-n48A-n66A-n261(2G) DC_n2A-n48A-n66A-n261(2H) DC_n2A-n48A-n66A-n261(A-G-H) DC_n2A-n48A-n66A-n261(H-I) DC_n2A-n48A-n66A-n261(A-G-I) | DC_n2A-n261A DC_n66A-n261A DC_n48A-n261A DC_n2A-n261G DC_n66A-n261G DC_n48A-n261G DC_n2A-n261H DC_n66A-n261H DC_n48A-n261H DC_n2A-n261I DC_n66A-n261I DC_n48A-n261I |
| DC_n2A-n66A-n77A-n260A DC_n2A-n66A-n77A-n260G DC_n2A-n66A-n77A-n260H DC_n2A-n66A-n77A-n260I DC_n2A-n66A-n77A-n260J DC_n2A-n66A-n77A-n260K DC_n2A-n66A-n77A-n260L DC_n2A-n66A-n77A-n260M | CA_n2A_n260A CA_n66A-n260A CA_n77A-n260A CA_n2A-n260G CA_n66A-n260G CA_n77A-n260G CA_n2A-n260H CA_n66A-n260H CA_n77A-n260H CA_n2A-n260I CA_n66A-n260I CA_n77A-n260I |
| DC_n2A-n66A-n77A-n261A DC_n2A-n66A-n77A-n261G DC_n2A-n66A-n77A-n261H DC_n2A-n66A-n77A-n261I DC_n2A-n66A-n77A-n261J DC_n2A-n66A-n77A-n261K DC_n2A-n66A-n77A-n261L DC_n2A-n66A-n77A-n261M DC_n2A-n66A-n77A-n261(A-G) DC_n2A-n66A-n77A-n261(A-H) DC_n2A-n66A-n77A-n261(A-I) DC_n2A-n66A-n77A-n261(A-2G) DC_n2A-n66A-n77A-n261(2A-G) DC_n2A-n66A-n77A-n261(2A-H) DC_n2A-n66A-n77A-n261(2A-I) DC_n2A-n66A-n77A-n261(G-H) DC_n2A-n66A-n77A-n261(2A) DC_n2A-n66A-n77A-n261(3A) DC_n2A-n66A-n77A-n261(2G) DC_n2A-n66A-n77A-n261(2H) DC_n2A-n66A-n77A-n261(A-G-H) DC_n2A-n66A-n77A-n261(H-I) DC_n2A-n66A-n77A-n261(A-G-I) | DC_n2A-n261A DC_n66A-n261A DC_n77A-n261A DC_n2A-n261G DC_n66A-n261G DC_n77A-n261G DC_n2A-n261H DC_n66A-n261H DC_n77A-n261H DC_n2A-n261I DC_n66A-n261I DC_n77A-n261I |
| DC_n3A-n7A-n78A-n258A DC_n3A-n7A-n78A-n258B DC_n3A-n7A-n78A-n258C DC_n3A-n7A-n78A-n258D DC_n3A-n7A-n78A-n258E DC_n3A-n7A-n78A-n258F DC_n3A-n7A-n78A-n258G DC_n3A-n7A-n78A-n258H DC_n3A-n7A-n78A-n258I DC_n3A-n7A-n78A-n258J DC_n3A-n7A-n78A-n258K DC_n3A-n7A-n78A-n258L DC_n3A-n7A-n78A-n258M | DC_n3A-n7A DC_n3A-n78A DC_n7A-n78A DC_n3A-n258A DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I |
| DC_n3A-n7B-n78A-n258A DC_n3A-n7B-n78A-n258B DC_n3A-n7B-n78A-n258C DC_n3A-n7B-n78A-n258D DC_n3A-n7B-n78A-n258E DC_n3A-n7B-n78A-n258F DC_n3A-n7B-n78A-n258G DC_n3A-n7B-n78A-n258H DC_n3A-n7B-n78A-n258I DC_n3A-n7B-n78A-n258J DC_n3A-n7B-n78A-n258K DC_n3A-n7B-n78A-n258L DC_n3A-n7B-n78A-n258M | DC_n3A-n7A DC_n3A-n78A DC_n7A-n78A DC_n3A-n258A DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I |
| DC_n3A-n28A-n77A-n257A | DC_n3A-n28A DC_n3A-n77A DC_n3A-n257A DC_n28A-n77A DC_n28A-n257A DC_n77A-n257A |
| DC_n3A-n28A-n77A-n257G | DC_n3A-n28A DC_n3A-n77A DC_n3A-n257A DC_n28A-n77A DC_n28A-n257A DC_n77A-n257A DC_n3A-n257G DC_n28A-n257G DC_n77A-n257G |
| DC_n3A-n28A-n77A-n257H | DC_n3A-n28A DC_n3A-n77A DC_n3A-n257A DC_n28A-n77A DC_n28A-n257A DC_n77A-n257A DC_n3A-n257G DC_n28A-n257G DC_n77A-n257G DC_n3A-n257H DC_n28A-n257H DC_n77A-n257H |
| DC_n3A-n28A-n77A-n257I | DC_n3A-n28A DC_n3A-n77A DC_n3A-n257A DC_n28A-n77A DC_n28A-n257A DC_n77A-n257A DC_n3A-n257G DC_n28A-n257G DC_n77A-n257G DC_n3A-n257H DC_n28A-n257H DC_n77A-n257H DC_n3A-n257I DC_n28A-n257I DC_n77A-n257I |
| DC_n3A-n28A-n77(2A)-n257A DC_n3A-n28A-n77(2A)-n257G DC_n3A-n28A-n77(2A)-n257H DC_n3A-n28A-n77(2A)-n257I | DC_n3A-n28A DC_n3A-n77A DC_n3A-n257A DC_n28A-n77A DC_n28A-n257A DC_n77A-n257A DC_n3A-n257G DC_n28A-n257G DC_n77A-n257G DC_n3A-n257H DC_n28A-n257H DC_n77A-n257H DC_n3A-n257I DC_n28A-n257I DC_n77A-n257I |
| DC_n3A-n28A-n78A-n257A | DC_n3A-n257A DC_n28A-n257A DC_n78A-n257A |
| DC_n3A-n28A-n78A-n257G | DC_n3A-n257A DC_n28A-n257A DC_n78A-n257A DC_n3A-n257G DC_n28A-n257G DC_n78A-n257G |
| DC_n3A-n28A-n78A-n257H | DC_n3A-n257A DC_n28A-n257A DC_n78A-n257A DC_n3A-n257G DC_n28A-n257G DC_n78A-n257G DC_n3A-n257H DC_n28A-n257H DC_n78A-n257H |
| DC_n3A-n28A-n78A-n257I | DC_n3A-n257A DC_n28A-n257A DC_n78A-n257A DC_n3A-n257G DC_n28A-n257G DC_n78A-n257G DC_n3A-n257H DC_n28A-n257H DC_n78A-n257H DC_n3A-n257I DC_n28A-n257I DC_n78A-n257I |
| DC_n3A-n28A-n79A-n257A DC_n3A-n28A-n79A-n257G DC_n3A-n28A-n79A-n257H DC_n3A-n28A-n79A-n257I | DC_n3A-n28A DC_n3A-n79A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n28A-n79A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n3A-n77A-n79A-n257A DC_n3A-n77A-n79A-n257G DC_n3A-n77A-n79A-n257H DC_n3A-n77A-n79A-n257I | DC_n3A-n257A DC_n77A-n257A DC_n79A-n257A DC_n3A-n257G DC_n77A-n257G DC_n79A-n257G DC_n3A-n257H DC_n77A-n257H DC_n79A-n257H DC_n3A-n257I DC_n77A-n257I DC_n79A-n257I |
| DC_n3A-n77(2A)-n79A-n257A DC_n3A-n77(2A)-n79A-n257G DC_n3A-n77(2A)-n79A-n257H DC_n3A-n77(2A)-n79A-n257I | DC_n3A-n257A DC_n77A-n257A DC_n79A-n257A DC_n3A-n257G DC_n77A-n257G DC_n79A-n257G DC_n3A-n257H DC_n77A-n257H DC_n79A-n257H DC_n3A-n257I DC_n77A-n257I DC_n79A-n257I |
| DC_n5A-n48A-n66A-n260A DC_n5A-n48A-n66A-n260G DC_n5A-n48A-n66A-n260H DC_n5A-n48A-n66A-n260I DC_n5A-n48A-n66A-n260J DC_n5A-n48A-n66A-n260K DC_n5A-n48A-n66A-n260L DC_n5A-n48A-n66A-n260M | CA_n5A-n260A CA_n66A-n260A CA_n48A-n260A CA_n5A-n260G CA_n66A-n260G CA_n48A-n260G CA_n5A-n260H CA_n66A-n260H CA_n48A-n260H CA_n5A-n260I CA_n66A-n260I CA_n48A-n260I |
| DC_n5A-n48A-n66A-n261A DC_n5A-n48A-n66A-n261G DC_n5A-n48A-n66A-n261H DC_n5A-n48A-n66A-n261I DC_n5A-n48A-n66A-n261J DC_n5A-n48A-n66A-n261K DC_n5A-n48A-n66A-n261L DC_n5A-n48A-n66A-n261M DC_n5A-n48A-n66A-n261(A-G) DC_n5A-n48A-n66A-n261(A-H) DC_n5A-n48A-n66A-n261(A-I) DC_n5A-n48A-n66A-n261(A-2G) DC_n5A-n48A-n66A-n261(2A-G) DC_n5A-n48A-n66A-n261(2A-H) DC_n5A-n48A-n66A-n261(2A-I) DC_n5A-n48A-n66A-n261(G-H) DC_n5A-n48A-n66A-n261(2A) DC_n5A-n48A-n66A-n261(3A) DC_n5A-n48A-n66A-n261(2G) DC_n5A-n48A-n66A-n261(2H) DC_n5A-n48A-n66A-n261(A-G-H) DC_n5A-n48A-n66A-n261(H-I) DC_n5A-n48A-n66A-n261(A-G-I) | DC_n5A-n261A DC_n66A-n261A DC_n48A-n261A DC_n5A-n261G DC_n66A-n261G DC_n48A-n261G DC_n5A-n261H DC_n66A-n261H DC_n48A-n261H DC_n5A-n261I DC_n66A-n261I DC_n48A-n261I |
| DC_n5A-n66A-n77A-n260A DC_n5A-n66A-n77A-n260G DC_n5A-n66A-n77A-n260H DC_n5A-n66A-n77A-n260I DC_n5A-n66A-n77A-n260J DC_n5A-n66A-n77A-n260K DC_n5A-n66A-n77A-n260L DC_n5A-n66A-n77A-n260M | CA_n5A-n260A CA_n66A-n260A CA_n77A-n260A CA_n5A-n260G CA_n66A-n260G CA_n77A-n260G CA_n5A-n260H CA_n66A-n260H CA_n77A-n260H CA_n5A-n260I CA_n66A-n260I CA_n77A-n260I |
| DC_n5A-n66A-n77A-n261A DC_n5A-n66A-n77A-n261G DC_n5A-n66A-n77A-n261H DC_n5A-n66A-n77A-n261I DC_n5A-n66A-n77A-n261J DC_n5A-n66A-n77A-n261K DC_n5A-n66A-n77A-n261L DC_n5A-n66A-n77A-n261M DC_n5A-n66A-n77A-n261(A-G) DC_n5A-n66A-n77A-n261(A-H) DC_n5A-n66A-n77A-n261(A-I) DC_n5A-n66A-n77A-n261(A-2G) DC_n5A-n66A-n77A-n261(2A-G) DC_n5A-n66A-n77A-n261(2A-H) DC_n5A-n66A-n77A-n261(2A-I) DC_n5A-n66A-n77A-n261(G-H) DC_n5A-n66A-n77A-n261(2A) DC_n5A-n66A-n77A-n261(3A) DC_n5A-n66A-n77A-n261(2G) DC_n5A-n66A-n77A-n261(2H) DC_n5A-n66A-n77A-n261(A-G-H) DC_n5A-n66A-n77A-n261(H-I) DC_n5A-n66A-n77A-n261(A-G-I) | DC_n5A-n261A DC_n66A-n261A DC_n77A-n261A DC_n5A-n261G DC_n66A-n261G DC_n77A-n261G DC_n5A-n261H DC_n66A-n261H DC_n77A-n261H DC_n5A-n261I DC_n66A-n261I DC_n77A-n261I |
| DC_n28A-n77A-n79A-n257A DC_n28A-n77A-n79A-n257G DC_n28A-n77A-n79A-n257H DC_n28A-n77A-n79A-n257I | DC_n28A-n77A DC_n28A-n79A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n77A-n79A DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n28A-n77(2A)-n79A-n257A DC_n28A-n77(2A)-n79A-n257G DC_n28A-n77(2A)-n79A-n257H DC_n28A-n77(2A)-n79A-n257I | DC_n28A-n77A DC_n28A-n79A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n77A-n79A DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n28A-n78A-n79A-n257A DC_n28A-n78A-n79A-n257G DC_n28A-n78A-n79A-n257H DC_n28A-n78A-n79A-n257I | DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n77A-n79A-n257A-n259A DC_n77A-n79A-n257A-n259G DC_n77A-n79A-n257A-n259H DC_n77A-n79A-n257A-n259I DC_n77A-n79A-n257A-n259J DC_n77A-n79A-n257A-n259K DC_n77A-n79A-n257A-n259L DC_n77A-n79A-n257A-n259M DC_n77A-n79A-n257G-n259A DC_n77A-n79A-n257G-n259G DC_n77A-n79A-n257G-n259H DC_n77A-n79A-n257G-n259I DC_n77A-n79A-n257G-n259J DC_n77A-n79A-n257G-n259K DC_n77A-n79A-n257G-n259L DC_n77A-n79A-n257G-n259M DC_n77A-n79A-n257H-n259A DC_n77A-n79A-n257H-n259G DC_n77A-n79A-n257H-n259H DC_n77A-n79A-n257H-n259I DC_n77A-n79A-n257H-n259J DC_n77A-n79A-n257H-n259K DC_n77A-n79A-n257H-n259L DC_n77A-n79A-n257H-n259M DC_n77A-n79A-n257I-n259A DC_n77A-n79A-n257I-n259G DC_n77A-n79A-n257I-n259H DC_n77A-n79A-n257I-n259I DC_n77A-n79A-n257I-n259J DC_n77A-n79A-n257I-n259K DC_n77A-n79A-n257I-n259L DC_n77A-n79A-n257I-n259M | DC_n77A-n79A DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n77A-n259A DC_n77A-n259G DC_n77A-n259H DC_n77A-n259I DC_n77A-n259J DC_n77A-n259K DC_n77A-n259L DC_n77A-n259M DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I DC_n79A-n259A DC_n79A-n259G DC_n79A-n259H DC_n79A-n259I DC_n79A-n259J DC_n79A-n259K DC_n79A-n259L DC_n79A-n259M |

#### 5.5B.7.4	Inter-band NR-DC configurations between FR1 and FR2 (five bands)

Table 5.5B.7-4: Inter-band NR-DC configurations between FR1 and FR2 (five bands)

| Downlink NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n3A-n28A-n77A-n79A-n257A DC_n3A-n28A-n77A-n79A-n257G DC_n3A-n28A-n77A-n79A-n257H DC_n3A-n28A-n77A-n79A-n257I | DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n3A-n28A-n77(2A)-n79A-n257A DC_n3A-n28A-n77(2A)-n79A-n257G DC_n3A-n28A-n77(2A)-n79A-n257H DC_n3A-n28A-n77(2A)-n79A-n257I | DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |

### 5.5B.6a	Inter-band NE-DC including FR1 and FR2

#### 5.5B.6a.1	Void

#### 5.5B.6a.2	Inter-band NE-DC configurations including FR1 and FR2 (three bands)

Table 5.5B.6a.2-1: Inter-band NE-DC configurations including FR1 and FR2 (three bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n77A-n257A_1A DC_n77A-n257G_1A DC_n77A-n257H_1A DC_n77A-n257I_1A DC_n77A-n257J_1A DC_n77A-n257K_1A DC_n77A-n257L_1A DC_n77A-n257M_1A | DC_n77A_1A DC_n257A_1A |
| DC_n77(2A)-n257A_1A DC_n77(2A)-n257G_1A DC_n77(2A)-n257H_1A DC_n77(2A)-n257I_1A DC_n77(2A)-n257J_1A DC_n77(2A)-n257K_1A DC_n77(2A)-n257L_1A DC_n77(2A)-n257M_1A | DC_n77A_1A DC_n257A_1A |
| DC_n77A-n257A_3A DC_n77A-n257G_3A DC_n77A-n257H_3A DC_n77A-n257I_3A DC_n77A-n257J_3A DC_n77A-n257K_3A DC_n77A-n257L_3A DC_n77A-n257M_3A | DC_n77A_3A DC_n257A_3A |
| DC_n77(2A)-n257A_3A DC_n77(2A)-n257G_3A DC_n77(2A)-n257H_3A DC_n77(2A)-n257I_3A DC_n77(2A)-n257J_3A DC_n77 (2A) -n257K_3A DC_n77(2A)-n257L_3A DC_n77(2A)-n257M_3A | DC_n77A_3A DC_n257A_3A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band NE-DC with mandatory simultaneous Rx/Tx capability |  |

#### 5.5B.6a.3	Inter-band NE-DC configurations including FR1 and FR2 (four bands)

Table 5.5B.6a.3-1: Inter-band NE-DC configurations including FR1 and FR2 (four bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n77A-n257A_1A-3A DC_n77A-n257G_1A-3A DC_n77A-n257H_1A-3A DC_n77A-n257I_1A-3A DC_n77A-n257J_1A-3A DC_n77A-n257K_1A-3A DC_n77A-n257L_1A-3A DC_n77A-n257M_1A-3A | DC_n77A_1A DC_n257A_1A DC_n77A_3A DC_n257A-3A |
| DC_n77(2A)-n257A_1A-3A DC_n77(2A)-n257G_1A-3A DC_n77(2A)-n257H_1A-3A DC_n77(2A)-n257I_1A-3A DC_n77(2A)-n257J_1A-3A DC_n77 (2A) -n257K_1A-3A DC_n77(2A)-n257L_1A-3A DC_n77(2A)-n257M_1A-3A | DC_n77A_1A DC_n257A_1A DC_n77A_3A DC_n257A-3A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band NE-DC with mandatory simultaneous Rx/Tx capability |  |

## 5.5C	Void

## 5.5D	Void

## 5.5E	Configuration for V2X operation

### 5.5E.1	General

The operating bands and bandwidth classes are specified for V2X operation.

### 5.5E.2	Intra-band contiguous V2X operation in FR1

Table 5.5E.2-1: Intra-band contiguous V2X configurations

| V2X configuration | SL transmission |
| --- | --- |
| V2X_(n)47AA | E-UTRA Band 47 or NR band n47 |
| NOTE 1: Only single switched SL is supported. |  |

### 5.5E.3	Intra-band non-contiguous V2X operation in FR1

Table 5.5E.3-1: Intra-band non-contiguous V2X configurations

| V2X configuration | SL transmission |
| --- | --- |
| V2X_47A_n47A | E-UTRA Band 47 or NR band n47 |
| NOTE 1: Only single switched SL is supported. |  |

### 5.5E.4	Inter-band V2X operation in FR1

#### 5.5E.4.1	Inter-band V2X configurations within FR1 (two bands)

Table 5.5E.4.1-1: Inter-band V2X configurations

| V2X configuration | V2X transmission configuration |
| --- | --- |
| V2X_1A_n47A | V2X_1A_n47A |
| V2X_n1A_47A | V2X_n1A_47A |
| V2X_5A_n47A | V2X_5A_n47A |
| V2X_n5A_47A | V2X_n5A_47A |
| V2X_8A_n47A | V2X_8A_n47A |
| V2X_n8A_47A | V2X_n8A_47A |
| V2X_3A_n47A | V2X_3A_n47A |
| V2X_20A_n38A | V2X_20A_n38A |
| V2X_n3A_47A | V2X_n3A_47A |
| V2X_n34A_47A | V2X_n34A_47A |
| V2X_34A_n47A | V2X_34A_n47A |
| V2X_n39A_47A | V2X_n39A_47A |
| V2X_39A_n47A | V2X_39A_n47A |
| V2X_n40A_47A | V2X_n40A_47A |
| V2X_40A_n47A | V2X_40A_n47A |
| V2X_n71A_47A | V2X_n71A_47A |
| V2X_n78A_47A | V2X_n78A_47A |
| V2X_n79A_47A | V2X_n79A_47A |
| NOTE 1: V2X transmission configurations are the configurations supported by the present release of specifications. |  |