3GPP TR 37.843 V15.7.0 (2020-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA);

Radio Frequency (RF) requirement background for

Active Antenna System (AAS) Base Station (BS) radiated requirements

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	16

1	Scope	17

2	References	17

3	Definitions, symbols and abbreviations	19

3.1	Definitions	19

3.2	Symbols	21

3.3	Abbreviations	22

4	General	23

4.1	Work Item scope	23

4.2	Work Item objectives	23

4.3	General AAS BS RF architecture	23

4.4	Specification organization	25

4.4.1	TS 37.105: RF and demodulation requirements	25

4.4.2	TS 37.114: EMC requirements	25

4.5	Coordinate system	25

4.6	AAS BS classifications	26

4.6.1	Minimum coupling loss and minimum distance	26

4.6.2	BS classifications	26

4.7	Polarization	26

5	Radiated transmitter characteristics	27

5.1	General	27

5.1.1	Spatial requirements	27

5.1.2	Co-location requirements	31

5.1.2.1	Core requirements	31

5.1.2.2	Conformance requirements	33

5.1.2.2.1	General	33

5.1.2.2.2	Co-location test antenna	34

5.1.2.2.3	Standard test antenna	35

5.1.3	Transmit loss factor assumption	35

5.2	Base station output power	36

5.2.1	Background information on the conducted requirement	36

5.2.2	OTA requirement	36

5.2.3	Output power accuracy	36

5.2.3.1	Core requirement	36

5.2.3.2	Conformance requirement	37

5.2.3.3	Extreme conditions	37

5.2.4	BS class power limits	38

5.2.5	Control channel accuracy	39

5.2.5.1	Background	39

5.2.5.2	Core requirement	40

5.2.5.3	Conformance requirement	41

5.3	Output power dynamics	41

5.3.1	Background information on the conducted requirement	41

5.3.2	Output power dynamics OTA: Core requirement	42

5.3.3	Output power dynamics OTA: Conformance requirement	42

5.4	Transmit ON/OFF power	42

5.4.1	Background	42

5.4.2	OTA AAS BS – Core requirement	43

5.5	Transmitted signal quality	43

5.5.1	General	43

5.5.2	Frequency error	43

5.5.2.1	Background information on the conducted requirement	43

5.5.2.2	Frequency error OTA: Core requirement	43

5.5.2.3	Frequency error OTA: Conformance requirement	44

5.5.3	Time Alignment Error	44

5.5.4	Modulation Quality	45

5.5.4.1	Background information on the conducted requirement	45

5.5.4.2	Modulation Quality OTA: Core requirement	46

5.5.4.3	Modulation Quality OTA: Conformance requirement	47

5.5.5	Transmit pulse shape filter	47

5.5.5.1	Background information on the conducted requirement	47

5.5.5.2	Transmit pulse shape filter OTA: Core requirement	47

5.5.5.3	Transmit pulse shape filter OTA: Conformance requirement	47

5.6	Unwanted emissions	48

5.6.1	General	48

5.6.1.1	Background information on the conducted requirement	48

5.6.1.2	OTA unwanted emissions	48

5.6.2	Occupied bandwidth	49

5.6.2.1	Background information on the conducted requirement	49

5.6.2.2	Occupied bandwidth OTA: Core requirement	49

5.6.2.3	Occupied bandwidth OTA: Conformance requirement	49

5.6.3	Adjacent Channel Leakage Radio	49

5.6.3.1	Background information on the conducted requirement	49

5.6.3.2	ACLR OTA: Core requirement	50

5.6.3.3	ACLR OTA: Conformance requirement	51

5.6.4	Spectrum emission mask	51

5.6.4.1	Background information on the conducted requirement	51

5.6.4.2	Spectrum emission mask OTA: Core requirement	51

5.6.4.3	Spectrum emission mask OTA: Conformance requirement	51

5.6.5	Operating band unwanted emission	51

5.6.5.1	Background information on the conducted requirement	51

5.6.5.2	Operating band unwanted emission OTA: Core requirement	52

5.6.5.3	Operating band unwanted emission OTA: Conformance requirement	52

5.6.6	Transmitter spurious emissions	52

5.6.6.1	Mandatory Spurious emission requirements	52

5.6.6.1.1	Background information on the conducted requirement	52

5.6.6.1.2	Transmitter spurious emissions OTA: Core requirement	53

5.6.6.1.3	Transmitter spurious emissions OTA: Conformance requirement	53

5.6.6.2	Co-location requirements	54

5.6.6.2.1	Background	54

5.6.6.2.2	Core requirements	54

5.6.6.2.3	Conformance requirements	55

5.7	Transmitter intermodulation	56

5.7.1	General	56

5.7.2	Background information on the conducted requirement	56

6	Radiated receiver characteristics	57

6.1	General	57

6.1.1	Spatial requirements	57

6.1.2	Receive loss factor assumption	58

6.1.3	OTA offsets	58

6.2	Reference sensitivity level	58

6.2.1	Background information on the conducted requirement	58

6.2.2	OTA sensitivity requirements	59

6.2.2.1	General	59

6.2.2.2	Core requirement	59

6.2.2.2.1	OTA Sensitivity	59

6.2.2.2.2	OTA reference sensitivity	59

6.2.2.3	Conformance requirement	61

6.2.2.3.1	OTA Sensitivity	61

6.2.2.3.2	OTA reference sensitivity	61

6.2.2.4	Extreme conditions	61

6.3	Dynamic range	62

6.3.1	Background information on the conducted requirement	62

6.3.2	Core requirement: Receiver dynamic range	62

6.3.3	Conformance requirement: Receiver dynamic range	62

6.4	In-band selectivity and blocking	62

6.4.1	Background information on the conducted requirement	62

6.4.2	In-band selectivity and blocking: Core requirement	63

6.4.3	In-band selectivity and blocking: Conformance requirement	67

6.5	Out-of-band blocking	67

6.5.1	Background	67

6.5.2	Out-of-band blocking OTA core requirement	67

6.5.2.1	Interferer level	67

6.5.2.2	Wanted signal level and direction	69

6.5.2.3	Out of band region	69

6.5.2.4	Frequency range	70

6.5.3	Out-of-band blocking OTA conformance requirement	70

6.6	Receiver spurious emissions	70

6.6.1	Background information on the conducted requirement	70

6.6.2	Receiver spurious emissions OTA: Core requirement	70

6.6.3	Receiver spurious emissions OTA: Conformance requirement	71

6.7	Receiver intermodulation	71

6.7.1	Background information on the conducted requirement	71

6.7.2	Receiver intermodulation: Core requirement	71

6.7.3	Receiver intermodulation: Conformance requirement	72

6.8	In-channel selectivity	72

6.8.1	Background information on the conducted requirement	72

6.8.2	In-channel selectivity: Core requirement	72

6.8.3	In-channel selectivity: Conformance requirement	72

6.9	ACS and NB blocking	73

6.9.1	Background information on the conducted requirement	73

6.9.2	ACS and NB blocking: Core requirement	73

6.9.3	ACS and NB blocking: Conformance requirement	74

7	Performance requirements	74

7.1	General	74

7.2	Core requirements for OTA tests	74

7.3	UTRA FDD requirements	74

7.4	E-UTRA requirements	75

7.5	Propagation conditions	78

7.6	Conformance requirements	78

7.7	OTA test setup	78

7.8	BS demodulation requirements feasible OTA	80

8	EMC requirements	81

8.1	General	81

8.2	Regulatory EMC requirements	82

8.3	Field strength in EMC chamber	83

8.3.1	General	83

8.3.2	Conversion between dBm to V/m	84

8.4	Protection of measurement equipment	84

8.5	EMC rated emission requirement	85

8.6	Radiated Immunity	85

8.6.1	General	85

8.6.2	Measurement set-up for testing radiated immunity	86

8.6.3	Spatial exclusions for radiated immunity testing	86

9	Manufacturer declarations	87

9.1	General	87

9.2	Void	88

10	Conformance testing aspects	88

10.1	OTA conformance testing framework	88

10.1.1	General	88

10.1.2	Uncertainty budget calculation principle	88

10.2	Measurement uncertainty for TX directional requirements	88

10.2.1	General	88

10.2.2	EIRP accuracy	89

10.2.2.1	General	89

10.2.2.2	CATR	89

10.2.2.2.1	MU assessment	89

10.2.2.2.1.1	MU budget	89

10.2.2.2.1.2	MU value	89

10.2.2.3	Plane Wave Synthesizer	91

10.2.2.3.1	Description	91

10.2.2.3.2	Procedure	91

10.2.2.3.2A	Test Method limitations and scope	93

10.2.2.3.3	MU assessment	93

10.2.2.3.3.1	MU budget	93

10.2.2.3.3.2	MU value	94

10.2.3	OTA E-UTRA DL RS power	94

10.2.3.1	General	95

10.2.3.2	In-door anechoic chamber	95

10.2.3.2.1	General	95

10.2.3.2.2	Calibration	97

10.2.3.2.3	Procedure	98

10.2.3.2.4	MU assessment	98

10.2.3.2.4.1	MU Budget	98

10.2.3.2.4.2	MU Value	99

10.2.3.3	CATR	100

10.2.3.3.1	General	100

10.2.3.3.2	Calibration	101

10.2.3.3.3	Procedure	102

10.2.3.3.4	MU assessment	103

10.2.3.3.4.1	MU Budget	103

10.2.3.3.4.2	MU Value	104

10.2.3.4	Near Field	104

10.2.3.4.1	General	104

10.2.3.4.2	Calibration	105

10.2.3.4.3	Procedure	106

10.2.3.4.4	MU assessment	106

10.2.3.4.4.1	MU budget	106

10.2.3.4.4.2	MU value	107

10.2.3.5	Summary	109

10.2.4	OTA Output power dynamics	109

10.2.4.1	General	109

10.2.4.2	In-door anechoic chamber	110

10.2.4.2.1	General	110

10.2.4.2.2	Calibration	110

10.2.4.2.3	Procedure	110

10.2.4.2.4	MU assessment	110

10.2.4.2.4.1	MU Budget	110

10.2.4.2.4.2	MU Value	111

10.2.4.3	CATR	112

10.2.4.3.1	General	112

10.2.4.3.2	Calibration	112

10.2.4.3.3	Procedure	112

10.2.4.3.4	MU assessment	113

10.2.4.3.4.1	MU Budget	113

10.2.4.3.4.2	MU Value	113

10.2.4.4	Near Field Test range	114

10.2.4.4.1	Description	114

10.2.4.4.2	Calibration	114

10.2.4.4.3	Procedure	114

10.2.4.4.4	MU assessment	114

10.2.4.4.4.1	MU budget	114

10.2.4.4.4.2	MU value	114

10.2.4.5	Summary	115

10.2.5	OTA Transmitted signal quality –Frequency Error	116

10.2.5.1	General	116

10.2.5.2	In-door anechoic chamber	116

10.2.5.2.1	General	116

10.2.5.2.2	Calibration	117

10.2.5.2.3	Procedure	117

10.2.5.2.4	MU assessment	117

10.2.5.3	CATR	117

10.2.5.3.1	General	117

10.2.5.3.2	Calibration	117

10.2.5.3.3	Procedure	117

10.2.5.3.4	MU assessment	117

10.2.5.4	Near Field	118

10.2.5.4.1	General	118

10.2.5.4.2	Calibration	118

10.2.5.4.3	Procedure	118

10.2.5.4.4	MU assessment	118

10.2.5.5	Summary	118

10.2.6	OTA Transmitted signal quality –EVM	118

10.2.6.1	General	118

10.2.6.1.1	Test equipment MU	119

10.2.6.2	In-door anechoic chamber	119

10.2.6.2.1	General	119

10.2.6.2.2	Calibration	119

10.2.6.2.3	Procedure	119

10.2.6.2.3.1	MU Budget	120

10.2.6.2.3.2	MU Value	120

10.2.6.3	CATR	121

10.2.6.3.1	General	121

10.2.6.3.2	Calibration	121

10.2.6.3.3	Procedure	121

10.2.6.3.4	MU assessment	121

10.2.6.3.4.1	MU Budget	121

10.2.6.3.4.2	MU Value	122

10.2.6.4	Near Field	122

10.2.6.4.1	General	122

10.2.6.4.2	Test method limitations and scope	122

10.2.6.4.3	Calibration	123

10.2.6.4.4	Procedure	123

10.2.6.4.5	MU assessment	123

10.2.6.4.5.1	MU Budget	123

10.2.6.4.5.2	MU Value	123

10.2.6.5	Summary	124

10.2.7	OTA Transmitted signal quality –TAE	124

10.2.7.1	General	124

10.2.7.2	In-door anechoic chamber	125

10.2.7.2.1	General	125

10.2.7.2.2	Calibration	125

10.2.7.2.3	Procedure	125

10.2.7.2.4	MU assessment	125

10.2.7.3	CATR	125

10.2.7.3.1	General	125

10.2.7.3.2	Calibration	125

10.2.7.3.3	Procedure	126

10.2.7.3.4	MU assessment	126

10.2.7.4	Near Field	127

10.2.7.4.1	General	127

10.2.7.4.2	Calibration	127

10.2.7.4.3	Procedure	127

10.2.7.4.4	MU assessment	127

10.2.7.5	Summary	127

10.2.8	OTA Occupied bandwidth	127

10.2.8.1	General	127

10.2.8.2	In-door anechoic chamber	128

10.2.8.2.1	General	128

10.2.8.2.2	Calibration	128

10.2.8.2.3	Procedure	128

10.2.8.2.4	MU assessment	129

10.2.8.2.4.1	MU Budget	129

10.2.8.2.4.2	MU Value	129

10.2.8.3	CATR	129

10.2.8.3.1	General	129

10.2.8.3.2	Calibration	129

10.2.8.3.3	Procedure	129

10.2.8.3.4	MU assessment	129

10.2.8.3.4.1	MU Budget	129

10.2.8.3.4.2	MU Value	129

10.2.8.4	Near field	129

10.2.8.4.1	General	129

10.2.8.4.2	Calibration	129

10.2.8.4.3	Procedure	130

10.2.8.4.4	MU assessment	130

10.2.8.4.4.1	MU Budget	130

10.2.8.4.4.2	MU Value	130

10.2.8.5	Summary	130

10.2.9	EIRP accuracy – extreme conditions	130

10.2.9.1	General	130

10.2.9.2	In-door anechoic chamber	130

10.2.9.2.1	General	130

10.2.9.2.2	Calibration	131

10.2.9.2.3	Procedure	131

10.2.9.2.4	MU assessment	132

10.2.9.2.4.1	MU Budget	132

10.2.9.2.4.2	MU Value	132

10.2.9.3	CATR	134

10.2.9.3.1	General	134

10.2.9.3.2	Calibration	134

10.2.9.3.3	Procedure	134

10.2.9.3.4	MU assessment	135

10.2.9.3.4.1	MU Budget	135

10.2.9.3.4.2	MU Value	135

10.2.9.3	Summary	137

10.3	Measurement uncertainty for Rx Directional requirements	137

10.3.1	General	137

10.3.2	OTA sensitivity	137

10.3.2.1	General	137

10.3.2.2	Plane Wave Synthesizer	138

10.3.2.2.1	Procedure	138

10.3.2.2.2	MU assessment	139

10.3.2.2.2.1	MU budget	139

10.3.2.2.2.2	MU value	140

10.3.3	OTA reference sensitivity	140

10.3.4	OTA Dynamic range	141

10.3.4.1	General	141

10.3.4.2	In-door anechoic chamber	141

10.3.4.2.1	General	141

10.3.4.2.2	Calibration	141

10.3.4.2.3	Procedure	142

10.3.4.2.4	MU assessment	142

10.3.4.3	CATR	142

10.3.4.3.1	General	142

10.3.4.3.2	Calibration	142

10.3.4.3.3	Procedure	143

10.3.4.3.4	MU assessment	143

10.3.4.4	Summary	143

10.3.5	OTA adjacent channel selectivity, general blocking and narrowband blocking	143

10.3.5.1	General	143

10.3.5.2	In-door anechoic chamber	143

10.3.5.2.1	General	143

10.3.5.2.2	Calibration	144

10.3.5.2.3	Procedure	144

10.3.5.2.4	MU assessment	145

10.3.5.2.4.1	MU Budget	145

10.3.5.2.4.2	MU Value	145

10.3.5.3	CATR	146

10.3.5.3.1	General	146

10.3.5.3.2	Calibration	146

10.3.5.3.3	Procedure	146

10.3.5.3.4	MU assessment	147

10.3.5.3.4.1	MU Budget	147

10.3.5.3.4.2	MU Value	147

10.3.5.4	Summary	148

10.3.6	OTA receiver intermodulation	148

10.3.6.1	General	148

10.3.6.2	In-door anechoic chamber	148

10.3.6.2.1	General	148

10.3.6.2.2	Calibration	149

10.3.6.2.3	Procedure	149

10.3.6.2.4	MU assessment	150

10.3.6.2.4.1	MU Budget	150

10.3.6.2.4.2	MU Value	150

10.3.6.3	CATR	151

10.3.6.3.1	General	151

10.3.6.3.2	Calibration	151

10.3.6.3.3	Procedure	152

10.3.6.3.4	MU assessment	152

10.3.6.3.4.1	MU Budget	152

10.3.6.3.4.2	MU Value	152

10.3.6.4	Summary	153

10.3.7	OTA in-channel selectivity	153

10.3.7.1	General	153

10.3.7.2	In-door anechoic chamber	154

10.3.7.2.1	General	154

10.3.7.2.2	Calibration	154

10.3.7.2.3	Procedure	154

10.3.7.2.4	MU assessment	154

10.3.7.2.4.1	MU Budget	154

10.3.7.2.4.2	MU Value	154

10.3.7.3	CATR	154

10.3.7.3.1	General	154

10.3.7.3.2	Calibration	155

10.3.7.3.3	Procedure	155

10.3.7.3.4	MU assessment	156

10.3.7.3.4.1	MU Budget	156

10.3.7.3.4.2	MU Value	156

10.3.7.4	Summary	156

10.4	Measurement uncertainty for In-band TRP requirements	156

10.4.1	OTA Base station output power requirement	156

10.4.1.1	General	156

10.4.1.2	In-door anechoic chamber	156

10.4.1.2.1	General	156

10.4.1.2.2	Calibration	157

10.4.1.2.3	Procedure	157

10.4.1.2.4	MU assessment	158

10.4.1.2.4.1	MU Budget	158

10.4.1.2.4.2	MU Value	158

10.4.1.3	CATR	160

10.4.1.3.1	General	160

10.4.1.3.2	Calibration	161

10.4.1.3.3	Procedure	161

10.4.1.3.4	MU assessment	162

10.4.1.3.4.1	MU Budget	162

10.4.1.3.4.2	MU Value	162

10.4.1.4	Near Field	164

10.4.1.4.1	General	164

10.4.1.4.2	Calibration	164

10.4.1.4.3	Procedure	164

10.4.1.4.4	MU assessment	164

10.4.1.4.4.1	MU Budget	164

10.4.1.4.4.2	MU Value	165

10.4.1.4A	Reverberation chamber	167

10.4.1.4A.1	General	167

10.4.1.4A.2	Calibration	167

10.4.1.4A.3	Procedure	167

10.4.1.4A.4	MU assessment	167

10.4.1.4A.4.1	MU Budget	167

10.4.1.4A.4.2	MU Value	168

10.4.1.4B	Plane Wave Synthesizer	168

10.4.1.4B.1	General	168

10.4.1.4B.2	Calibration	168

10.4.1.4B.3	Procedure	168

10.4.1.4B.4	Test Method limitations and scope	169

10.4.1.4B.5	MU assessment	169

10.4.1.5	Summary	171

10.4.2	OTA Adjacent channel leakage power ratio	171

10.4.2.1	General	171

10.4.2.2	In-door anechoic chamber	171

10.4.2.2.1	General	171

10.4.2.2.2	Calibration	172

10.4.2.2.3	Procedure	172

10.4.2.2.4	MU assessment	172

10.4.2.2.4.1	MU Budget	172

10.4.2.2.4.2	MU Value	172

10.4.2.3.1	General	174

10.4.2.3.2	Calibration	174

10.4.2.4	Near Field	177

10.4.2.4.1	Description	177

10.4.2.4.2	Calibration	177

10.4.2.4.3	Procedure	177

10.4.2.4.4	MU assessment	178

10.4.2.4.4.1	MU Budget	178

10.4.2.4.4.2	MU Value	178

10.4.2.4A	Reverberation chamber	180

10.4.2.4A.1	General	180

10.4.2.4A.2	Calibration	180

10.4.2.4A.3	Procedure	180

10.4.2.4A.4	MU assessment	180

10.4.2.4A.4.1	MU Budget	180

10.4.2.4A.4.2	MU Value	181

10.4.2.4B	Plane Wave Synthesizer	182

10.4.2.4B.1	General	182

10.4.2.4B.2	Calibration	182

10.4.2.4B.3	Procedure	182

10.4.2.4B.4	Test Method limitations and scope	183

10.4.2.4B.5	MU assessment	183

10.4.2.5	Summary	184

10.4.3	OTA Operating band unwanted emission	185

10.4.3.1	General	185

10.4.3.2	In-door anechoic chamber	185

10.4.3.2.1	General	185

10.4.3.2.2	Calibration	185

10.4.3.2.3	Procedure	185

10.4.3.2.4	MU assessment	185

10.4.3.2.4.1	MU Budget	185

10.4.3.2.4.2	MU Value	185

10.4.3.4	Near Field	190

10.4.3.4.1	General	190

10.4.3.4.2	Calibration	190

10.4.3.4.3	Procedure	190

10.4.3.4.4	MU assessment	190

10.4.3.4.4.1	MU Budget	190

10.4.3.4.4.2	MU Value	190

10.4.3.4A	Reverberation chamber	190

10.4.3.4A.1	General	190

10.4.3.4A.2	Calibration	190

10.4.3.4A.3	Procedure	190

10.4.3.4A.4	MU assessment	191

10.4.3.4A.4.1	MU Budget	191

10.4.3.4A.4.2	MU Value	191

10.4.3.4B	Plane Wave Synthesizer	191

10.4.3.4B.1	General	191

10.4.3.4B.2	Calibration	191

10.4.3.4B.3	Procedure	192

10.4.3.4B.4	MU assessment	192

10.4.3.5	Summary	192

10.4.4	OTA Spectrum emission mask	192

10.4.4.1	General	192

10.4.4.2	In-door anechoic chamber	192

10.4.4.2.1	General	192

10.4.4.2.2	Calibration	192

10.4.4.2.3	Procedure	193

10.4.4.2.4	MU assessment	193

10.4.4.2.4.1	MU Budget	193

10.4.4.2.4.2	MU Value	193

10.4.4.3	CATR	195

10.4.4.3.1	General	195

10.4.4.3.2	Calibration	195

10.4.4.3.3	Procedure	195

10.4.4.3.4	MU assessment	195

10.4.4.3.4.1	MU Budget	195

10.4.4.3.4.2	MU Value	195

10.4.4.4	Near Field	197

10.4.4.4.1	General	197

10.4.4.4.2	Calibration	197

10.4.4.4.3	Procedure	197

10.4.4.4.4	MU assessment	197

10.4.4.4.4.1	MU Budget	197

10.4.4.4.4.2	MU Value	197

10.4.4.4A	Reverberation chamber	197

10.4.4.4A.1	General	197

10.4.4.4A.2	Calibration	197

10.4.4.4A.3	Procedure	197

10.4.4.4A.4	MU assessment	198

10.4.4.4A.4.1	MU Budget	198

10.4.4.4A.4.2	MU Value	198

10.4.4.4B	Plane Wave Synthesizer	198

10.4.4.4B.1	General	198

10.4.4.4B.2	Calibration	198

10.4.4.4B.3	Procedure	199

10.4.4.4B.4	MU assessment	199

10.4.4.5	Summary	199

10.5	Measurement uncertainty for Out-of-band TRP requirements	199

10.5.1	General	199

10.5.2	Transmitter mandatory spurious emissions	199

10.5.2.1	General	199

10.5.2.2	General chamber	201

10.5.2.2.1	General	201

10.5.2.2.2	Calibration	201

10.5.2.2.3	Procedure	201

10.5.2.2.4	MU assessment	201

10.5.2.2.4.1	MU Budget	201

10.5.2.2.4.2	MU Value	202

10.5.2.3	Void	203

10.5.2.3A	Reverberation chamber	203

10.5.2.3A.1	General	203

10.5.2.3A.2	Chamber characterization	204

10.5.2.3A.3	Calibration	205

10.5.2.3A.4	Procedure	206

10.5.2.3A.4A	Test Method limitations and scope	207

10.5.2.3A.5	MU assessment	207

10.5.2.4	Test Tolerance	208

10.5.2.5	Summary	208

10.5.3	Receiver spurious emissions	209

10.5.3.1	General	209

10.5.3.2	General chamber	209

10.5.3.2.1	General	209

10.5.3.2.2	Calibration	209

10.5.3.2.3	Procedure	209

10.5.3.2.4	MU assessment	209

10.5.3.2.4.1	MU Budget	209

10.5.3.2.4.2	MU Value	210

10.5.3.3	Void	211

10.5.3.4	Test Tolerance	211

10.5.3.5	Summary	211

10.5.4	Additional (co-existence) spurious emissions	212

10.5.4.1	General	212

10.5.4.2	CATR	212

10.5.4.2.1	General	212

10.5.4.2.2	Calibration	212

10.5.4.2.3	Procedure	212

10.5.4.2.4	MU assessment	213

10.5.4.2.4.1	MU Budget	213

10.5.4.2.4.2	MU Value	213

10.5.4.3	Void	214

10.5.4.4	Test Tolerance	214

10.5.4.4	Summary	214

10.6	Measurement uncertainty for Co-location requirements	215

10.6.1	General	215

10.6.2	OTA transmitter OFF power	215

10.6.2.1	General	215

10.6.2.2	In-door anechoic chamber	215

10.6.2.2.1	General	215

10.6.2.2.2	Calibration	215

10.6.2.2.3	Procedure	215

10.6.2.2.4	MU assessment	216

10.6.2.2.4.1	MU Budget	216

10.6.2.2.4.2	MU Value	216

10.6.2.3	CATR	216

10.6.2.3.1	General	216

10.6.2.3.2	Calibration	216

10.6.2.3.3	Procedure	216

10.6.2.3.4	MU assessment	217

10.6.2.3.4.1	MU Budget	217

10.6.2.3.4.2	MU Value	217

10.6.2.5	Summary	219

10.6.3	OTA co-location spurious emissions	219

10.6.3.1	General	219

10.6.3.2	In-door anechoic chamber	219

10.6.3.2.1	General	219

10.6.3.2.2	Calibration	219

10.6.3.2.3	Procedure	219

10.6.3.2.4	MU assessment	219

10.6.3.2.4.1	MU Budget	219

10.6.3.2.4.2	MU Value	219

10.6.3.3	CATR	219

10.6.3.3.1	General	219

10.6.3.3.2	Calibration	219

10.6.3.3.3	Procedure	219

10.6.3.3.4	MU assessment	220

10.6.3.3.4.1	MU Budget	220

10.6.3.3.4.2	MU Value	220

10.6.3.5	Summary	222

10.6.4	OTA Transmitter intermodulation	222

10.6.4.1	General	222

10.6.4.2	In-door anechoic chamber	222

10.6.4.2.1	General	222

10.6.4.2.2	Calibration	223

10.6.4.2.3	Procedure	223

10.6.4.2.4	MU assessment	223

10.6.4.2.4.1	MU Budget	223

10.6.4.2.4.2	MU Value	224

10.6.4.3	CATR	225

10.6.4.3.1	General	225

10.6.4.3.2	Calibration	225

10.6.4.3.3	Procedure	225

10.6.4.3.4	MU assessment	225

10.6.4.3.4.1	MU Budget	225

10.6.4.3.4.2	MU Value	225

10.6.4.4	Shielded Anechoic Chamber	225

10.6.4.4.1	General	225

10.6.4.4.2	Calibration	226

10.6.4.4.3	Procedure	226

10.6.4.4.4	MU assessment	227

10.6.4.4.4.1	MU Budget	227

10.6.4.4.4.2	MU Value	227

10.6.4.5	Summary	228

10.6.5	OTA co-location blocking	228

10.6.5.1	General	228

10.6.5.2	In-door anechoic chamber	228

10.6.5.2.1	General	228

10.6.5.2.2	Calibration	228

10.6.5.2.3	Procedure	228

10.6.5.2.4	MU assessment	229

10.6.5.2.4.1	MU Budget	229

10.6.5.2.4.2	MU Value	229

10.6.5.3	CATR	229

10.6.5.3.1	General	229

10.6.5.3.2	Calibration	229

10.6.5.3.3	Procedure	229

10.6.5.3.4	MU assessment	230

10.6.5.3.4.1	MU Budget	230

10.6.5.3.4.2	MU Value	230

10.6.5.5	Summary	231

10.7	Measurement uncertainty for Out-of-band Blocking requirements	231

10.7.1	General	231

10.7.2	General Chamber	232

10.7.2.4.2	MU Value	234

10.7.2.4.3	Interferer MU total	235

10.7.3	Summary	236

10.8.2.1	Spherical equal angle grid	237

10.8.2.2	Reference angular step criteria	238

10.8.2.3	Spherical equal area grids	242

10.8.2.4	Spherical Fibonacci grids	243

10.8.2.5	Orthogonal cuts grids	244

10.8.2.6	Wave vector space sampling grid	246

10.8.3	TRP measurement procedures	249

10.8.3.1	Procedures for base station output power	250

10.8.3.1.1	General	250

10.8.3.1.2	Two cuts with pattern multiplication	250

10.8.3.1.3	Full sphere	250

10.8.3.1.4	Beam-based directions	251

10.8.3.2	Procedures for SEM and OBUE	251

10.8.3.2.1	General	251

10.8.3.2.2	Two cuts with pattern multiplication	251

10.8.3.2.3	Two or Three cuts	251

10.8.3.2.4	Full sphere	251

10.8.3.2.5	Beam-based directions	251

10.8.3.3	Procedures for ACLR	252

10.8.3.3.1	General	252

10.8.3.3.2	TRP fraction method	252

10.8.3.4	Procedures for transmitter spurious emissions and EMC emissions	252

10.8.3.4.0	General	252

10.8.3.4.1	Pre-scan	252

10.8.3.4.2	Peak method	253

10.8.3.4.3	Equal Sector with peak average method	253

10.8.3.4.4	Two or three cuts with dense sampling	253

10.8.3.4.5	Full sphere with sparse sampling	254

10.8.3.4.6	Full sphere	254

10.8.4	Angular alignment in TRP measurements	254

10.11	Measurement uncertainty for demodulation performance requirements	256

10.11.1	General	256

10.11.2	In-door anechoic chamber	257

10.11.2.1	General	257

10.11.2.2	Calibration	257

10.11.2.3	Procedure	257

10.11.2.4	MU assessment	257

10.11.3	CATR	257

10.11.3.1	General	257

10.11.3.2	Calibration	258

10.11.3.3	Procedure	258

10.11.3.4	MU assessment	258

Annex A:	Void	259

Annex B: OTA sensitivity measurement error contribution descriptions	260

B.1	Indoor Anechoic Chamber	260

B.2	Compact Antenna Test Range	260

B.3	One Dimensional Compact Range	260

B.4	Near Field Test Range	260

Annex C: Beam sweeping	261

C.1	Simulation results	261

C.2	Measurement results	262

C.3	Design of beam sweeping test signal	264

Annex D: Sparse sampling for spurious emissions	265

Annex E: Radiated transmit measurement error contribution descriptions	270

E.1	Indoor Anechoic Chamber	270

E.2	Compact Antenna Test Range	272

E.3	Near Field Test Range	274

E.4	General Chamber	278

E.5	Reverberation Chamber	281

E.6	Plane Wave Synthesizer	281

Annex F:	Test equipment uncertainty values	284

F.1	Measurement Equipment uncertainty values	284

F.2	MU of TE derived from conducted specification	284

Annex G:	Power density measurements close to EUT	286

Annex H:	Change history	288

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is the Technical Report for the Work Item on Base Station (BS) radiated Radio Frequency (RF) requirements for Active Antenna System (AAS) [2]. The present document captures the background and the decisions on the specification of radiated RF requirements that are applicable to AAS BS.

The AAS BS specification contains requirements for both single RAT UTRA FDD, UTRA TDD and E-UTRA as well as MSR (including single RAT MSR requirements for each of the RATS), it was agreed that it may not be necessary to support so many variants for the OTA AAS BS hence a specification complexity reduction investigation was carried out. It was found the following was not needed for the OTA AAS BS requirements:

1.	Radiated requirements applicable for eAAS not support UTRA TDD.

2.	Radiated requirements applicable for eAAS not include UTRA pilot requirements.

NOTE:	In Rel-15, multiple clauses related to the OTA measurements of the BS were shifted to the OTA BS testing TR 37.941 [36], which includes such aspects as e.g., test tolerance and measurement uncertainty derivations, OTA test chambers descriptions, calibration and test procedure descriptions, etc..

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	RP-170844: "Work Item Description for Enhancements of Base Station (BS) RF and EMC requirements for Active Antenna System (AAS)"

[3]	3GPP TS 37.105: "Active Antenna System (AAS) Base Station (BS) transmission and reception"

[4]	3GPP TR 37.842: "Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)"

[5]	3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios"

[6]	3GPP TR 37.840: "Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station"

[7]	3GPP TS 37.104: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception"

[8]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

[9]	Recommendation ITU-R SM.328-11: "Spectra and bandwidth of emissions", ITU/R

[10]	3GPP TR 25.951: "FDD Base Station (BS) classification"

[11]	3GPP TR 37.809: "Evolved Universal Terrestrial Radio Access (E-UTRA) medium range and Multi-Standard Radio (MSR) medium range / local area Base Station (BS) class requirements"

[12]	3GPP TR 36.931: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) requirements for LTE Pico Node B"

[13]	3GPP TS 37.114: "Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC)"

[14]	3GPP TS 37.113: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)"

[15]	3GPP TS 36.113: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)"

[16]	3GPP TS 25.113: "Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)"

[17]	Conversion of power to field strength in the book "Automotive Antenna Design and Applications", by Victor Rabinovich, Nikolai Alexandrov, Basim Alkhateeb; 2010

[18]	Recommendation ITU-R SM.329-10: "Unwanted emissions in the spurious domain"

[19]	3GPP TS 25.104: "Universal Terrestrial Radio Access (UTRA); Base Station (BS) radio transmission and reception (FDD)"

[20]	3GPP TS 25.105: "Universal Terrestrial Radio Access (UTRA); Base Station (BS) radio transmission and reception (TDD)"

[21]	3GPP TS 25.141: "Base Station (BS) conformance testing (FDD)"

[22]	3GPP TS 36.141: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing"

[23]	3GPP TS 37.145-1: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing"

[24]	3GPP TS 37.145-2: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: Radiated conformance testing"

[25]	IEEE Std 149: "IEEE Standard Test Procedures for Antennas", IEEE

[26]	TSGR4#8(99)631: "Antenna-to-Antenna Isolation Measurements", Allgon

[27]	Report ITU-R M.2244: "Isolation between antennas of IMT base stations in the land mobile service", ITU-R

[28]	IEC 61000-4-3: 2006+AMD1:2007+AMD2:2010: “Electromagnetic compatibility (EMC) - Part 4-3: Testing and measurement techniques - Radiated, radio-frequency, electromagnetic field immunity test”

[29]	Hald, J.; Hansen, J. E.; Jensen, F. & Holm Larsen, F. Hansen, J. (Ed.) Spherical Near-Field Antenna Measurements, Peter Peregrinus Ltd., 1998, vol 26.

[30]	"Sparse Sampling Analysis Tool" Matlab code: https://se.mathworks.com/matlabcentral/fileexchange/67143-sparse-sampling-analysis-tool?s_tid=srchtitle

[31]	J. Fridén, A. Razavi, and A. Stjernman, “Angular sampling, Test Signal, and Near-Field Aspects for Over-the-Air Total Radiated Power Assessment in Anechoic Chambers”, IEEE Access, 2018, https://ieeexplore.ieee.org/stamp/stamp.jsp?arnumber=8470084.

[32]	IEEE Std 145: “IEEE Standard Definitions of Terms for Antennas”, IEEE

[33]	IEC 61000-4-21. “Electromagnetic compatibility (EMC) - Part 4-21: Testing and measurement techniques - Reverberation chamber test methods” Edition 2.0 2011-01, The International Electrotechnical Commision (IEC), 2011

[34]	Hill, D.A., “Boundary fields in reverberation chambers,” IEEE Transactions on Electromagnetic Compatibility, vol. 47, no. 2, pp. 281-290, May 2005

[35]	Krauthäuser, H. G.; Winzerling, T.; J., N.; Eulig, N. & Enders, A. “Statistical interpretation of autocorrelation coefficients for fields in mode-stirred chambers” 2005 International Symposium on Electromagnetic Compatibility, EMC 2005, 2005, 2, 550-555

[36]		3GPP TR 37.941: "Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

NOTE:	Multi-word definitions are treated as linguistic expressions and printed in italic font throughout this requirement specification. Linguistic expressions may not be split and are printed in their entirety.

active antenna system base station: base station system which combines an antenna array with a transceiver unit array and a radio distribution network

array element: subdivision of a passive antenna array, consisting of a single radiating element or a group of radiating elements, with a fixed radiation pattern

antenna array: group of radiating elements characterized by the geometry and the properties of the array elements

antenna gain: ratio of the radiation intensity, in a given direction, to the radiation intensity that would be obtained if the power accepted by the antenna were radiated isotropically

NOTE:	If the direction is not specified, the direction of maximum radiation intensity is implied.

array factor: radiation pattern of an array antenna when each array element is considered to radiate isotropically

NOTE:	When the radiation pattern of individual array elements are identical, and the array elements are congruent under translation, then the product of the array factor and the array element radiation pattern gives the radiation pattern of the entire array.

angle of arrival: is the direction of propagation of electromagnetic wave incident on an AAS BS antenna array

basic limit: emissions limit taken from the non-AAS BS specifications that is converted into a per TAB connector TX min cell group AAS BS emissions limit, or into a per TAB connector RX min cell group AAS BS emissions limit by scaling, depending on the context

beam: main lobe of a radiation pattern from an AAS BS

NOTE:	For certain AAS BS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is supposed to be found

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

co-location reference antenna: reference antenna used for co-location requirements.

directions diagram: two dimensional Cartesian diagram showing φ on the horizontal axis and minus θ on the vertical axis

directivity: ratio of the radiation intensity in a given direction from the antenna to the radiation intensity averaged over all directions

NOTE:	If the direction is not specified, the direction of maximum radiation intensity is implied.

EIRP accuracy directions set: beam peak directions for which the EIRP accuracy requirement is intended to be met

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

EMC antenna port:	RF connector referred to as antenna port in EMC requirements

equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

NOTE:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

equivalent isotropic sensitivity: sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

NOTE 1:	The sensitivity is the minimum received power level at which a RAT specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

hybrid AAS BS:	AAS BS which has both a conducted RF interface and a radiated RF interface in the far field and conforms to a hybrid requirements set

hybrid requirements set: Complete set of requirements applied to a hybrid AAS BS with both conducted and radiated  requirements

minSENS: the lowest declared EIS value for the OSDD’s declared for OTA sensitivity requirement

minSENS RoAoA:	the reference RoAoA associated with the OSDD with the lowest declared EIS value.

multi-band TAB connector: TAB connector supporting operation in multiple operating bands through common active electronic components(s)

NOTE:	For common TX and RX TAB connectors, the definition applies where common active electronic components are in the transmit path and/or in the receive path.

OTA AAS BS: AAS BS which has ≥8 transceiver units for E-UTRA or MSR and ≥4 transceiver units for UTRA per cell and has a radiated RF interface only and conforms to the OTA requirements set.

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

OTA REFSENS RoAoA: is the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction.

NOTE:	This contour will be related to the average element/sub-array radiation pattern 3dB beam width

OTA requirements set:	complete set of OTA requirements applied to an OTA AAS BS

OTA sensitivity directions declaration: set of manufacturer declarations comprising at least one set of declared minimum EIS values (with related RAT and channel bandwidth), and related directions over which the EIS applies

NOTE:	All the directions apply to all the EIS values in an OSDD.

polarization match: condition that exists when a plane wave, incident upon an antenna from a given direction, has a polarization that is the same as the receiving polarization of the antenna in that direction

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

radiating element: basic building block of an array element characterized by its radiation properties

radiation pattern: angular distribution of the radiated electromagnetic field or power level in the far field region

radio distribution network: linear passive network which distributes the RF power generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping.

Reference RoAoA:	The sensitivity RoAoA associated with the receiver target reference direction for each OSDD.

receiver target: AoA in which reception is performed by AAS BS

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to particular OSDD

receiver target reference direction: direction inside the OTA sensitivity directions declaration declared by the manufacturer for conformance testing. For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA

sensitivity RoAoA: RoAoA within the OTA sensitivity directions declaration, within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific AAS BS direction setting

single direction requirement: AAS BS requirement which is applied in a specific direction within the OTA coverage range for the Tx and when the AoA of the incident wave of a received signal is within the OTA REFSENS RoAoA or the minSENS RoAoA as appropriate for the receiver.

TAB connector: transceiver array boundary connector

total radiated power: is the total power radiated by the antenna.

NOTE 1:	The total radiated power is the power radiating in all direction for two orthogonal polarizations.

NOTE 2:	total radiated power is defined in both the near-field region and the far-field region.

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transceiver unit: active unit consisting of transmitter and/or receiver which transmits and/or receives radio signals, and which may include passive RF filters

transceiver unit array: array of transceiver units which generate radio signals in the transmit direction and accept radio signals in the receive direction

transmitter OFF period: time period during which the transmitter is scheduled not to transmit

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

transmitter ON period: time period during which the transmitter is transmitting data and/or reference symbols

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

transmitter transient period: time period during which the transmitter unit belonging to the transceiver unit array is changing from the OFF period to the ON period or vice versa

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

TRP requirement: AAS BS requirements, which requires dual-polarized measurements of the figure of merit over the whole sphere around the DUT

TRP summation error: the relative difference between theoretical TRP and numerically estimated TRP

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BeWθ	The beamwidth in the θ-axis in degrees.

BeWφ	The beamwidth in the φ-axis in degrees.

BeWθ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees.

BeWφ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees.

DRX_OTA_MARGIN	Directivity implementation margin in UL

EVM	Error Vector Magnitude

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular operating band.

PRated,c,TRP	The rated total radiated power when all the transmitter units are operating at their rated output power for a single carrier

The azimuth angle (defined between -180° and 180°)

Elevation angle of the signal direction (defined between -90° and 90°, 0° represents the direction perpendicular to the antenna array)

TRPEstimate	Numerically approximated TRP

TRPReference	Theoretical TRP

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AA	Antenna Array

AAS BS	Active Antenna System Base Station

ACIR	Adjacent Channel Interference Ratio

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

BS	Base Station

CACLR	Cumulative ACLR

CATR	Compact Antenna Test Range

DUT	Device Under Test

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

EMC	Electromagnetic compatibility

EMC RE	EMC Radiated Emissions

ERP	Effective Radiated Power

EUT	Equipment Under Test

FRC	Fixed Reference Channel

FSF	Fixed Scaling Factor

LA	Local Area

MCL	Minimum Coupling Loss

MR	Medium Range

OBUE	Operating Band Unwanted Emissions

OSDD	OTA Sensitivity Direction Declaration

OTA	Over-the-Air

RDN	Radio Distribution Network

RF RSE	RF Radiated Spurious Emissions

RIB	Radiated Interface Boundary

RMC	Reference measurement channel

RoAoA	Range of Angles of Arrival

RXU	Receiver Unit

SEM	Spectrum Emission Mask

SGH	Standard Gain Horn

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TRP	Total Radiated Power

TRXU	Transceiver Unit

TRXUA	Transceiver Unit Array

TXU	Transmitter Unit

WA	Wide Area

# 4	General

## 4.1	Work Item scope

Rel-15 AAS BS Work Item (WI) is to specify radiated Radio Frequency (RF), BS demodulation and EMC requirements for Active Antenna System (AAS) Base Station (BS) covering single Radio Access Technology (RAT) capable BS supporting UTRA or E-UTRA, and multi-RAT capable BS operating in Band Category (BC) 1 and BC3, for Wide Area (WA), Medium Range (MR), and Local Area (LA) coverage to ensure necessary coexistence. Wide Area BS class will be prioritized.

## 4.2	Work Item objectives

The radiated AAS BS specification is based on translating the Rel-13 AAS BS conducted requirements from TS 37.105 [3] to over the air requirements which provide the same level of performance and protection as the Rel-13 AAS BS requirements do.

It is composed of the following work tasks:

a) Core RF requirements

b) Conformance testing requirements, derived from the core RF requirements, and demodulation testing requirements

Due to the necessary study required to arrive at requirement that does not require excessive measurement and will be practical (i.e. easy to measure) and robust, it is decided to include a study phase to identify the OTA core and conformance concepts before writing the specification text.

Detailed WID texts are presented in [2].

## 4.3	General AAS BS RF architecture

An AAS BS may have 2 different types of RF interface at which the RF and performance requirements are defined:

-	Conducted RF interface at the transceiver array boundary (TAB) and a radiated interface boundary (RIB) in the far field for a hybrid AAS BS.

-	Radiated interface boundary (RIB) only for an OTA AAS BS.

For both types of AAS BS the radio architecture is represented by three main functional blocks:

-	the transceiver unit array (TRXUA),

-	the radio distribution network (RDN), and

-	the antenna array (AA).

The transceiver units (TRXU) interface with the base band processing within the AAS BS.

The TRXUA consists of multiple transmitter units (TXU) and receiver units (RXUs). The TXU takes the baseband input from the AAS BS and provides the RF TX outputs. The OTA AAS BS is restricted to architectures which have ≥8 TRXUs for E_UTRA and ≥4 TRXUs for UTRA, restricting the minimum number of TRXUs in the OTA AAS BS architecture allows the emissions scaling factor to be fixed and avoids defining an OTA definition of an active transceiver unit which is difficult without the ability to count TAB connectors. AAS BS with small numbers of TRXUs should retain a conducted interface and are classified as hybrid AAS BS.

The RF TX outputs are distributed to the AA via a RDN. The RXU performs the reverse of the TXU operations. The RDN performs the distribution of the TX outputs into the corresponding antenna paths and antenna elements, and a distribution of RX inputs from antenna paths in the reverse direction. The TXU and RXU can be separated and can have different mapping through the RDN towards the AA.

For the hybrid AAS BS the transceiver array boundary (TAB) is the point(s) at which the TRXUA is connected to the RDN. The point where a TXU or RXU connects with the RDN is equivalent to an "antenna connector" of a non-AAS BS and is called TAB connector. The TAB connector is defined as conducted requirement reference point. The transmitted signal per carrier from one Transmitter Unit appear at one or more TAB connector(s) and the received signals per carrier from one or more TAB connector(s) appear at a single RXU.

Figure 4.3-1 shows a general architecture of a hybrid AAS BS radio architecture, where M is the total number of transceiver units and K is the total number of TAB connectors at the transceiver array boundary.

The OTA AAS BS has no transceiver array boundary or TAB connectors defined as it has no conducted requirements. The OTA AAS BS radiated requirements are defined at the radiated interface boundary (RIB).

Figure 4.3-2 shows a general architecture of an OTA AAS BS radio architecture, where P is the total number of transceiver units and P ≥ 8 (E-UTRA) or P ≥ 4 (UTRA) for OTA AAS BS.

NOTE 1:	The RDN may consist of a simple one to one mapping between the TXU(s)/RXU(s) and the passive antenna array. In this case, the RDN would be a logical entity but not necessarily a physical entity.

NOTE 2:	The antenna array includes various implementations and configurations e.g. polarization, array geometry (including element factor and element separation), etc.

NOTE 3:	The physical location of the TRXUA, the RDN, and the AA may differ from this logical representation and is implementation dependent.

NOTE 4:	No specific mapping in the RDN between TAB connectors and antenna elements is assumed. Further the number of separate receiver and transmitter units as well as the mapping in the RDN between TAB connectors and radiating elements can differ between the transmit and receive directions. Both AAS BS reference architectures allow for full asymmetry between receiver path and transmit path.

NOTE 5:	For AAS BS capable of supporting applications employing beamforming, all or subgroups of TAB connectors can be configured with designated amplitude and phase weights such that one or more beams are radiated from the antenna array.

NOTE 6:	If text of this TR and figures 4.3-1, 4.3-2 contradict each other, then the TR text applies.

NOTE 7:	In the case of the OTA AAS BS, the fixed scaling factor (FSF) equals 8 for E-UTRA and 4 for UTRA and is based on the maximum number of layers/streams specified in Rel-12. The FSF value may be further reconsidered for future releases if the maximum number of layers/streams supported by RAT(s) supported by the AAS BS is changed.

NOTE 8:	If an OTA AAS BS is declared to support more than 1 cell (Ncells>1) the total number of transceiver units is greater than FSF* Ncells , i.e. for 8* Ncells for E-UTRA or 4* Ncells for UTRA.

Figure 4.3-1: General architecture of hybrid AAS BS and the reference points for the conducted and radiated requirements

Figure 4.3-2: General architecture of OTA AAS BS and the reference point for the radiated requirements

## 4.4	Specification organization

### 4.4.1	TS 37.105: RF and demodulation requirements

The existing AAS BS technical specification TS 37.105 [3] will be updated to include both hybrid requirements set(s) and OTA requirements set.

The full OTA set of RF requirements will be added as additional sub-clauses to the existing radiated requirements in the Rel-15 version of the technical specification TS 37.105.

The updated technical specification TS 37.105 will contain all the existing conducted requirements as specified and the radiated requirements clause will be modified to contain a full set of OTA requirements for OTA AAS BS.

The requirements in TS 37.105 can be used to describe either an OTA requirements set or a hybrid requirements set(s). There will be at least one hybrid requirements set which is the same as the existing AAS BS Rel-13 set of conducted and radiated requirements.

The applicability of requirements clause in TS 37.105 can be modified to define which sub-clauses are required to show compliance to either the OTA requirements set or the hybrid requirements set(s).

### 4.4.2	TS 37.114: EMC requirements

The existing EMC requirements defined in TS 37.114 [13] for hybrid AAS BS will be extended to also include EMC requirements for OTA AAS BS.

In the Rel-15 version of the TS 37.114 the set of EMC requirements for OTA AAS BS will be added as additional sub-clauses to the already existing EMC requirements defined for the hybrid AAS BS.

## 4.5	Coordinate system

For the description of the coordinate system for the OTA BS measurements, refer to TR 37.941 [36].

## 4.6	AAS BS classifications

### 4.6.1	Minimum coupling loss and minimum distance

AAS BS is classified according to the deployment scenarios defined in sub-clause 5.2.1 in TR 37.840 [6] and those scenarios are the same as for non-AAS BS. One of the key parameters that characterise each of the deployment scenarios is the MCL between BS and UE, which includes the path loss and the antenna gains of both UE and BS. For OTA AAS BS, without antenna connectors, minimum distance along the ground is used to characterise the deployment scenarios.

### 4.6.2	BS classifications

AAS BS classes for BS without antenna connectors are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

AAS BS classes for BS with antenna connectors are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to minimum coupling loss equal to 45 dB.

The deployment scenarios associated with BS classes and its definitions are exactly the same for BS both with and without connectors. An MCL of 70dB corresponds to a minimum distance of around 35m, 53dB to around 5m and 45dB to around 2m respectively for BS with connectors.

## 4.7	Polarization

Polarization is a vital characteristic for antennas used for radio communication. Both radio communication antennas and electromagnetic waves are said to have a polarization. According to eastablished and accepted standard definitions of terms for antennas [32], the viewpoint taken for polarization is that this term can be used in three related meanings;

-	Polarization can refer to a field vector at some point in space.

-	Polarization can refer to a plane wave.

-	Polarization can refer to an antenna.

Based on the geneal description above, polarization aspects for OTA requirements can be summarized as listed in table 4.7-1 for transmitter requirements and 4.7-2 for receiver requirements.

Table 4.7-1: Description of polarization aspects for transmitter requirements

| Requirement | Description |
| --- | --- |
| Radiated transmit power | The declared EIRP is defined as EIRP = EIRPp1 + EIRPp2, where p1 and p2 represents two orthogonal polarizations. |
| OTA Base Station output power | The declared output power based on TRP captures radiated power for two orthogonal polarizations. |
| OTA Output power dynamics | The EIRP level in this requirement is defined as EIRP = EIRPp1 + EIRPp2, where p1 and p2 represents two orthogonal polarizations. |
| OTA Transmit ON/OFF power | The TDD OFF power emission is measured at the CLTA outputs as a power sum of supported polarizations. |
| OTA Transmitter signal quality | The EIRP level in this requirement is defined as EIRP = EIRPp1 + EIRPp2, where p1 and p2 represents two orthogonal polarizations. |
| OTA Unwanted Emissions | Radiated unwanted emission based on TRP captures radiated power for two orthogonal polarizations. Except for co-location requirements, where the emission is measured at the CLTA outputs as a power sum of supported polarizations. |
| OTA Transmitter intermodulation | Radiated unwanted emission based on TRP captures radiated power for two orthogonal polarizations. The interferer signal is fed to the CLTA, where the power is divided per supported polarizations. |

Table 4.7-2: Description of polarization aspects for receiver requirements

| Requirement | Description |
| --- | --- |
| OTA sensitivity | The declared EIS level applies to each supported polarization, under assumption of polarization match. |
| OTA Reference sensitivity level | The wanted signal applies to all supported polarizations, under the assumption of polarization match. |
| OTA Dynamic range | The wanted and interferer signals apply to all supported polarizations, under the assumption of polarization match. |
| OTA Adjacent channel selectivity, general blocking, and narrowband blocking | The wanted and interferer signals apply to all supported polarizations, under the assumption of polarization match. |
| OTA Blocking | The wanted signal applies to all supported polarizations, under the assumption of polarization match. The interferer shall be polarization matched in band and the polarization maintained for out-of-band frequencies. Except for co-location requirements, where the interferer signal is fed to the CLTA per supported polarization. |
| OTA Receiver spurious emissions | OTA receiver emission based on TRP captures radiated power for two orthogonal polarizations. |
| OTA Receiver intermodulation | The wanted and interferer signals apply to all supported polarizations, under the assumption of polarization match. |
| OTA In-channel selectivity | The wanted and interferer signals apply to all supported polarizations, under the assumption of polarization match. |

# 5	Radiated transmitter characteristics

## 5.1	General

### 5.1.1	Spatial requirements

OTA requirements for the AAS BS are belonging to one of the three groups:

-	Directional requirements

-	TRP requirements

-	Co-location requirements

For the requirements classification and descriptions of the above requirement groups, refer to TR 37.941 [36].

All OTA requirements are met in either the OTA coverage range or OTA peak direction set(s) as shown in the overview table of the radiated Tx requirements in table 5.1-1.

Table 5.1-1: Overview of radiated Tx requirements

| AAS BS requirement |  | OTA requirement type | Coverage range | Notes |
| --- | --- | --- | --- | --- |
| Base station output power | Output power accuracy for EIRP | Directional requirement | OTA peak directions set | Output power accuracy for EIRP requirement is already included as a core requirement in TS 37.105. |
|  | Output power accuracy for TRP | TRP | n/a |  |
| E-UTRA DL RS power |  | Directional Requirement | OTA peak directions set | Conformance testing is carried out in the reference direction |
| Output power dynamics |  | Directional requirement | OTA peak directions set | Conformance testing is carried out in the reference direction. |
| Transmit ON/OFF power |  | co-location requirement | n/a |  |
| Frequency Error |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Time Alignment Error |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Modulation Quality (EVM) |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction and the maximum directions of the OTA coverage range on each axis. |
| Unwanted emissions |  |  |  |  |
| Occupied Bandwidth |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Adjacent Channel Leakage Radio (ACLR) |  | TRP | n/a |  |
| Spectrum emission mask |  | TRP | n/a |  |
| Spurious emissions |  |  |  |  |
| Mandatory Requirements |  | TRP | n/a |  |
| Protection of the BS receiver of own or different BS |  | co-location requirement | n/a |  |
| Additional spurious emissions requirements |  | TRP | n/a | Includes co-existence in same geographical area |
| Co-location with other base stations |  | co-location requirement | n/a |  |
| Transmitter intermodulation |  | co-location requirement | n/a | The interferer is applied as a co-location requirements, the radiated emissions requirements are specified in the appropriated referenced sub-clause. Generally TRP |

The OTA peak directions set(s) are the same as the EIRP accuracy directions set(s) used in the REL13 specification, the name has been changed as they are now used for other requirements. Examples of OTA peak directions sets remain the same as related EIRP accuracy directions set(s) in TR 37.842 [4].

Some further examples of the relationship between the OTA peak direction set(s) and the OTA coverage range are as follows:

Example 1.

This example shows the declarations that would be made for an OTA AAS BS with a passive antenna, the system hence has no beam steering capability (this is perhaps unlikely to be an OTA AAS BS however makes extreme example).

The declarations are: Vertical (θ) beam width = 10°, Horizontal (φ) beam width = 65°

Figure 5.1.1-1: Example OTA coverage range and OTA peak directions set for a passive antenna

Note as the beam cannot be steered the OTA peak directions set is a single point and the OTA coverage range is the same as the 3dB beam pattern of the passive antenna.

Example 2.

This example shows the declarations that would be made for an OTA AAS BS with multiple beam widths and beam steering capability. The declarations are defined as including an OTA peak directions set for both the widest beam width and the narrowest beam width in both dimensions. For simplicity it is assumed beam width in θ and ϕ are the same for this example.

-	For the minimum beam width case:  beam width (θ and ϕ) =10°, maximum steering (θ and ϕ) = ±32.5°

-	For the maximum beam width case: beam width (θ and ϕ) =35°, maximum steering (θ and ϕ) = ±25°

-	Note the larger beam width has a smaller declared extreme steering direction, but the potential coverage is greater.

The OTA peak directions sets for the 2 cases are different, however they require a common OTA coverage range, this can be seen in figure 5.1.1-2.

Figure 5.1.1-2: Example OTA coverage range and OTA peak directions set for a beam steering system with multiple beam widths

Note the OTA coverage range is the same for both cases as there is only a single OTA coverage range declared per OTA AAS BS, however there are multiple OTA peak directions sets allowed. In this case the OTA peak direction set for the minimum beam width case is the same as the OTA coverage range, for the maximum beam width case the OTA peak directions set is smaller than the OTA coverage range.

In both examples the OTA peak directions sets are a sub set of the OTA coverage range as required.

In both examples the beam widths for each of the maximum steering directions are shown for information, the beam width for each of the maximum steering direction is declared but it does not form part of the OTA coverage range. It can be seen in both examples that when the beam is steered to its maximum steering direction a portion of the beam is outside of the OTA coverage range. The maximum extent of the OTA coverage range on each axis is declared and used as test directions for the EVM requirement. The directions covered by this area on the directions diagram are not part of the OTA coverage range and hence are not part of the range of directions which are intended to meet the RF requirements, this is acceptable as performance beyond the OTA coverage range is not specified.

### 5.1.2	Co-location requirements

#### 5.1.2.1	Core requirements

There are a number of requirements which are based on the assumption of a co-located BS of the same specification as the BS under test.

These requirements include:

-	Transmitter spurious emissions- co-location with other base stations

-	Transmitter intermodulation

-	Tx OFF requirements

-	Co-location blocking

The non-AAS requirements at the antenna connector assume that the co-located BS have a coupling factor between them of 30 dB. This coupling factor is used to calculate the interferer level at a co-located victim in the case of the spurious emissions requirement, and the interferer level at the antenna connector from a co-located aggressor BS.

The figure of 30 dB is based on measurement of coupling levels between typical BS antennas in the same frequency band and is documented in [26] for the following scenarios:

d

d

I (90º)			II(120º)		III(180º)		IV(Horizontal)		V(Vertical)

Figure 5.1.2-1: The different scenarios used during the antenna coupling measurements. d denotes displacement

In most scenarios the reported coupling factor between antennas was greater than 30 dB. The figure of 30 dB comes from scenario IV when d is 0.25 m.

This figure agrees with the calculation for horizontal isolation coupling in [27].

There are a number of issues which make translating the existing conducted requirement into an OTA requirement difficult.

-	The required level of RX band spurious emissions is too low to measure in the far field.

-	The out-of-band antenna gain of the AAS BS is not known.

-	The interferer power level for the IMD requirement in the far field is very high.

However, OTA requirements have the advantage that the original co-location scenarios can be used to derive the requirements. The requirements can therefore be derived by defining both the type of co-located reference antenna and also how close it is to the AAS BS. This is referred to as the proximity method.

The co-location reference antenna can be regarded as having 2 interfaces, a proximity OTA interface which couples with the AAS BS and a conducted interface where the AAS BS requirements are set (and measured). For each of the co-location requirements the co-location power specified at the transceiver array boundary can be specified at the conducted output/input of the co-location reference antenna offset by 30 dB.

Some co-location requirements involve co-located BS of different frequency bands. As the coupling loss between antennas of different frequency ranges is higher than between antennas of the same frequency range, assuming antennas of the same frequency range can be considered worst case. This is consistent with the approach for the conducted co-location requirements where 30dB coupling loss was assumed for all antenna combinations.

For example, the co-located protection of other RX bands is based on the interfering power being low enough to not degrade the victim BS receiver by more than 0.5 dB, which equates to a power level in 100 kHz of -128 dBm. For the conducted requirement this is translated to the transceiver array boundary by adding the 30 dB isolation. So, the requirement at the output of the co-location reference antenna is -128 dBm (plus any scaling).

As the worst case is based on scenario IV in figure 5.1.2-1 the definition of the co-location reference antenna is based on this scenario. The simplest method for defining the co-location reference antenna is with a diagram as shown in figure 5.1.2-2.

The co-location reference antenna should be a single column passive antenna which has the same vertical radiating dimension (h), frequency range and polarization, as the composite antenna of AAS BS and nominal 65° horizontal half power beamwidth (suitable for 3-sector deployments). It is placed at a distance d from the edge of the AAS BS, as shown in figure 5.1.2-2.

Figure 5.1.2-2: Definition of the co-location reference antenna

Edge-to-edge separation d, between the AAS BS and the co-location reference antenna is set to 0.1 m.

The AAS BS and the co-location reference antenna are to be aligned in a common plane perpendicular to the mechanical bore-sight direction, as shown in figure 5.1.2-2.

The co-location reference antenna and the AAS can have different width.

The vertical radiating regions of the co-location reference antenna and the AAS composite antenna are aligned.

For co-location requirements where the frequency range of the signal at the co-location reference antenna is different from the AAS BS, a co-location reference antenna suitable for the frequency stated in the requirement is assumed.

OTA co-location requirements are based on the power at the conducted interface of a co-location reference antenna; depending on the requirement this interface is either an input or an output.

For AAS BS with dual polarization, the co-location reference antenna has two conducted interfaces each representing one polarization.

The area and dimensions of the radiating elements shown in figure 5.1.2-2 do not form part of the OTA core definition of the co-location reference antenna, they are included in the figure to demonstrate what is meant by the radiating dimension in the definition.

Based on the minimum separation of antennas used in the original scenarios in [26], the edge-to-edge separation d between the co-location reference antenna and the AAS BS is 0.10 m (this is derived from a 0.25 m centre to centre separation on 0.15 m wide antennas).

#### 5.1.2.2	Conformance requirements

##### 5.1.2.2.1	General

Co-location requirements are specified as power levels into or out of each conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of either a co-location test antenna (CLTA) or a standard test antenna (such as a dipole).

The CLTA is suitable for testing AAS BS implemented with a planar antenna array. The method for testing AAS BS with other antenna array implementations is FFS.

##### 5.1.2.2.2	Co-location test antenna

The co-location test antenna is a practical passive antenna based on the definition of the co-location reference antenna, and it is used for conformance testing of the co-location requirements. Thus the CLTA should be within a certain tolerance of the co-location reference antenna definition, so that commonly available passive BS antennas can be used to test the co-location requirements without having to use a large number of test antennas and without having to design specialized co-location test antennas.

For example, where the co-location reference antenna definition specifies that the co-location reference antenna, should be a single column passive antenna which has the same vertical radiating dimension (h) as the composite antenna of AAS, this would be very restrictive when selecting a test antenna. Hence the CLTA should be within a certain tolerance for the vertical radiating dimension (h) such, that a suitable practical BS antenna can be found for each AAS BS.

The CLTA is hence defined as follows:

The CLTA is a single column passive antenna which has a vertical radiating dimension (h) which is equal to the vertical radiating dimension of the AAS BS composite antenna ±30%.

For in-band requirements, the CLTA supports the same frequency range and polarizations as the AAS BS composite antenna.

For out-of-band co-location requirements, the half power vertical beamwidth of the CLTA equals the narrowest declared vertical beamwidth ±3°.

The polarization should match the in-band CLTA polarization

The CLTA has a horizontal half-power beamwidth of 65° ±10°.

The front faces of the AAS BS and the CLTA need to be aligned in a common plane perpendicular to the mechanical bore-sight direction, as shown in figure 5.1.2.2.2-1.

The centre of the vertical radiating regions of the CLTA and the AAS composite antenna need to be aligned.

This is summarised as follows:

Table 5.1.2.2.2-1: CLTA characteristics

| Parameter | in-band CLTA | out-of–band CLTAs |
| --- | --- | --- |
| Vertical radiating dimension (h) | Test object vertical radiating length ±30% | N/A |
| Horizontal beamwidth | 65° ± 10° | 65° ± 10° |
| Vertical beamwidth | N/A | The half-power vertical beamwidth of the CLTA equals the narrowest declared vertical beamwidth ± 3° |
| Polarization | match | match to in-band |
| Conducted interface return loss | > 10 dB | > 10 dB |
| NOTE 1: If a multi-column or multi-band antenna is used, the column closest to the AAS BS needs to be selected while other columns are terminated during testing. |  |  |

The alignment between the AAS BS under test and the co-location test antenna is described in Table 5.1.2.2.2-2 and Figure 5.1.2.2.2-1.

Table 5.1.2.2.2-2: CLTA alignment tolerances

| Parameter | in-band | out-of-band |
| --- | --- | --- |
| Edge-to-edge separation between the AAS BS and the CLTA (d) | 0.1 m ± 0.01 m | 0.1 m ± 0.01 m |
| Vertical alignment | centre ± 0.01 m | centre ± 0.01 m |
| Front to band alignment | Radome front ± 0.01 m | Radome front ± 0.01 m |

Figure 5.1.2.2.2-1: Alignment of AAS BS and CLTA

##### 5.1.2.2.3	Standard test antenna

It is captured in subclause 5.6.6.2 that co-location requirements may also be tested with a standardised test antenna such as a dipole. This has a number of advantages:

-	Standard test antennas are easily available and easier to specify.

-	Repeatability between different conformance testing runs will be greater.

-	Using standard test antennas, fewer antennas will be required for conformance testing.

-	A standard test antenna can have higher gain in the direction of the AAS BS than a CLTA, so that signal levels can be higher relative to the noise floor in emission measurements and lower test levels can be used in interference measurements.

In order to test with a standard test antenna, the translation between measured power levels in the standard test antenna and the co-location reference antenna must be shown. The method for translating the power levels in the co-location reference antenna to the standard test antenna and vice versa is FFS.

### 5.1.3	Transmit loss factor assumption

The transmitter loss factor is used to translate conducted power levels to OTA power levels

To align with the emissions requirements based on the recommendation in ITU-R SM.329 [18] the same limits are applied to both conducted and OTA, assuming the transmit loss factor accounting for antenna losses, distribution losses, integration losses etc. in the transmitter path(s) for the in-band, out-of-band and spurious region is assumed to be 0 dB.

## 5.2	Base station output power

### 5.2.1	Background information on the conducted requirement

The conducted single RAT specifications TS 25.104 [19], TS 25.105 [20] and TS 36.104 [8] as well as the MSR specification TS 37.104 [7] and the Rel-13 AAS BS specification TS 37.105 [3] all contain a set of manufacturer’s declarations and a requirement on conducted output power. The requirement is placed upon the rated carrier output power per carrier and applies to each antenna connector in non-AAS BS, and TAB connector in AAS BS. The rated carrier output power per carrier is declared, and the requirement is that the measured value is within ± 2dB (±2.5 dB in extreme conditions) of the declared value. In addition to the accuracy requirement on the carrier output power, a limit on the maximum declarable total rated output power per carrier, dependent on the BS class, is captured in the core specification TS 37.105 although not subject to any conformance test.

In the AAS BS conformance test specification TS 37.145-1 [23], the conducted power accuracy test is not carried out with a single carrier active, but with a set of carriers active such that the total output power at the TAB connector is at its maximum level and hence the power amplifier is operated at its maximum output power. Operation at maximum output power is an essential condition for a number of other transmitter tests, including in-band emissions, TX spurious emissions and EVM.

In the AAS BS specification TS 37.105, a requirement on EIRP accuracy was introduced. The requirement is based on a set of declarations that associate declared beams, beam steering ranges and EIRP achieved at certain points within the steering ranges. The declarations associated with the EIRP accuracy requirement are described in more detail in TR 37.842 [4]. The requirement itself states that the declared EIRP is achieved to within a ±2.2 dB accuracy interval at a number of (declared) directions.

### 5.2.2	OTA requirement

For the OTA requirements set, the possibility of complete OTA testing of output power is needed. This implies that where OTA testing is used, conformance to the conducted requirement cannot be verified. In order to decide on a suitable OTA solution, it is useful to review the purposes and uses of the radiated output power requirement.

-	Verifying the accuracy of the transmitter unit output power

-	The conducted output power requirement achieves this goal. The EIRP accuracy requirement verifies the accuracy of the transmitter unit output together with other factors relating to the antenna array. The EIRP accuracy requirement is a sufficient means to capture the accuracy of the whole system in delivering power.

-	A basis for the power limit for the BS classes

-	The BS class power limits have been derived based on co-existence simulations between different layers in heterogeneous network deployments. Due to the statistical nature of the co-existence simulations, it is to be expected to be the total power that will impact the co-existence statistics, independently of beamforming and the antenna pattern (as is the case for ACLR in general). This implies that an OTA based BS class power limit should be based on TRP.

-	A condition for other transmitter conformance requirements

During conformance testing, it is an important condition for other transmitter requirements that the OTA AAS BS is configured to and operating at maximum power.

### 5.2.3	Output power accuracy

#### 5.2.3.1	Core requirement

The existing EIRP accuracy requirements are adopted for the requirement in normal conditions. The accuracy requirement is hence ±2.2 dB.

As there are now a number of transmitter requirements which are specified as directional requirements the existing directions set name and definition over which the EIRP accuracy is met (i.e. EIRP accuracy directions set) is no longer suitable. EIRP accuracy is now be defined over the more general OTA peak directions set(s), the requirement is modified accordingly.

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within +2.2 dB and -2.2 dB of the claimed value.

EIRP accuracy was derived based on 3 methods outlined in sub-clause 7.2.3, TR 37.842 [4]:

1.	Impacts of accuracy on network performance,

2.	Non-AAS BS EIRP accuracy, and

3.	An estimate of the achievable accuracy by AAS BS.

Whilst much of the effort focused on the last of these the achievable accuracy of an AAS BS and the 3 error model used to estimate this, it was not the only basis for the final agreement.

The impact on network performance throughput is related to the wanted link between the AAS BS and the UE and hence is covered by the EIRP requirements so need not be considered when looking at TRP.

However, the BS class upper power limits are derived based on the interference the BS causes in co-existence scenarios. This was not considered when examining EIRP accuracy network performance. The upper power limits for the medium range and local area BS classes can be found in TR 25.951 [10] and are based on co-existence simulations with wide area networks and the macro layer downlink outage. From these simulations and the results it can be noted that:

-	The results for the medium range output power limit were between +20 to +30 dBm with +24 dBm agreed as a compromise.

-	The results for the local area output power limit were between +37 to +39 dBm with +38 dBm agreed as a compromise.

-	Simulations  in all cases did not take into account the BS output power variation, it can be assumed that this and the antenna network gain variation are assumed to be included as the channel and hence part of the slow fading (10 dB lognormal for the Macro layer and 6 dB for the micro/pico layers).

Considering the range of results found when deriving the power limit and the large variation considered in the slow fading and that the TRP accuracy to be a value between the conducted accuracy and the EIRP accuracy which are only 0.2 dB apart, it is unlikely that network simulation will provide a meaningful result. Hence methods 2 and 3 above were considered.

Unlike EIRP, TRP is not subject to the additional steering error caused by phase mismatch in between transceiver units when considering TRP. As there is no steering error, the other errors in the non AAS base station EIRP accuracy (method 2) and the estimate of the achievable accuracy by AAS BS (method 3) still applicable to TRP. For the latter, the TRP accuracy is expressed as:

TRPaccuracy  =σtx2+ σarray2              [dB]

Hence the TRP accuracy is bounded as follows:

conducted poweraccuracy≤TRPaccuracy≤ EIRPaccuracy

Finally RAN4 agreed to use ±2 dB as the TRP accuracy requirement with compromises by some vendors.

#### 5.2.3.2	Conformance requirement

The core requirement applies to any claimed power, however as the conducted requirement no longer exists it is important that the condition of all transceiver units operating at maximum power is defined for the conformance condition. The TRP accuracy requirement is valid for any operational condition, however if the condition for conformance is that all transceiver units are operating at full power, the TRP will not vary with beam steering. Hence it is only necessary to show conformance in a single beam direction.

#### 5.2.3.3	Extreme conditions

The only conducted transmitter requirement with an extreme requirement is conducted output power accuracy, in sub-clause 6.2.2.2.1 of TS 37.105 [3].

In normal conditions, Pmax,c,TABC shall remain within +2 dB and -2 dB of the configured carrier power for each TAB connector as declared by the manufacturer.

In extreme conditions, Pmax,c,TABC shall remain within +2.5 dB and -2.5 dB of the configured carrier power for each TAB connector as declared by the manufacturer.

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

For the OTA requirement set there are two out power accuracy requirements, one based on EIRP which is also in the hybrid requirement set, and one based on TRP.

The EIRP requirement in sub-clause 9.2.2 is currently only valid for nominal conditions:

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the EIRP accuracy directions set , a manufacturer  claimed EIRP level in the corresponding beam peak direction shall be achievable to within +2.2 dB and -2.2 dB of the claimed value.

The TRP accuracy requirement described in sub-clause 5.2.3.1 is the natural replacement of the conducted output power accuracy as they are directly related to each other. However the difficulty in measuring OTA at extreme temperatures means that practically extreme temperature measurements should be directional. This way the effect of the thermal chamber on the OTA accuracy and calibration can be limited to certain directions, this is not possible with a TRP measurement.

In addition any variation in TRP is included in the EIRP requirement, hence an extreme EIRP requirement will cover the TRP requirement.

The EIRP accuracy requirement will therefore have the extreme temperature requirement added to it for OTA AAS BS only.

The conducted allowance for extreme temperature is 0.5 dB, the same figure is added to the EIRP accuracy core and conformance requirements, for example:

Figure 5.2.3.3-1 example of EIRP accuracy core and conformance requirement, in nominal and extreme conditions

The core requirement for EIRP accuracy for an OTA AAS BS with therefore be ±2.7 dB. There is no extreme requirement for TRP accuracy.

The existing clarification stating that “In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal” also applies to the OTA requirement.

The EIRP accuracy requirement does not apply to the hybrid AAS BS.

### 5.2.4	BS class power limits

The AAS BS output power limits in existing requirements are based on the values derived for the non-AAS BS specifications [7][8][19][20] these power levels are specified as the rated output power per carrier at the antenna connector, i.e. per non-AAS transceiver. In the AAS BS specification for AAS BS with a conducted interface TS 37.105 [3] the requirement is based on both:

-	The rated system output power per carrier, PRated,c,sys

-	The rated output power per TAB connector, PRated,c,TABC

The limit per TAB connector is the same as that used in the non-AAS specifications, the system limit is the non-AAS limit scaled by the number of active transceiver units (capped at the appropriate number for E-UTRA or UTRA), NTXU,counted.

This ensures that the AAS BS does not exceed the power limit per carrier either for the system or per transceiver unit.

The equivalent output power limit for the OTA AAS BS specification is TRP. The AAS BS TRP is equivalent to the system output power (i.e. not the transceiver unit output power) by the following relationship:

PRated,c,TRP = PRated,c,sys – LTX

The OTA AAS BS output power limit can therefore be defined as:

PRated,c,TRP ≤ Non-AAS Power limit  – LTX + 10log(NTXU)

Where NTXU is the minimum number of active transceiver units, for an OTA AAS BS has ≥8 TRXU’s for E-UTRA or ≥4 TRXU’s for UTRA this number is fixed. As both LTX and NTXU have fixed values, the values in the power limit tables do not need to include variables and the adjusted fixed umber can be used.

For example for medium range E-UTRA

PRated,c,TRP ≤ 38 dBm  – 0 +10log(8) ≤ 47 dBm

Hence the power limits table is as follows:

Table 5.2.4-1: UTRA OTA AAS BS rated output power limits for BS classes

| AAS BS class |  | PRated,c,TRP |
| --- | --- | --- |
| Wide Area BS |  | (Note) |
| Medium Range BS |  | ≤ 44 dBm |
| Local Area BS |  | ≤ 30 dBm |
| NOTE: There is no upper limit for the PRated,c,TRP for the Wide Area Base Station. |  |  |

Table 5.2.4-2.: E-UTRA OTA AAS BS rated output power limits for BS classes

| AAS BS class |  | PRated,c,TRP |
| --- | --- | --- |
| Wide Area BS |  | (Note) |
| Medium Range BS |  | ≤ 47 dBm |
| Local Area BS |  | ≤ 33 dBm |
| NOTE: There is no upper limit for the PRated,c,TRP for the Wide Area Base Station. |  |  |

### 5.2.5	Control channel accuracy

#### 5.2.5.1	Background

There are a number of control channel power requirements in TS 37.105 [3]. The requirements are intended to confirm that power accuracy of the control channels is acceptable.

For UTRA

-	UTRA FDD primary CPICH power

-	UTRA TDD primary CCPCH power

-	UTRA FDD additional CPICH power for MIMO mode

And for E-UTRA

-	E-UTRA DL RS power

The UTRA FDD primary CPICH power, UTRA TDD primary CPICH power and the E-UTRA DL RS power are all very similar requirements. They all confirm the accuracy of the specified control channel is within x dB of the power indicated on the BCH (UTRA) or DL-SCH (E-UTRA), where x is different for each requirement and can be found in TS 37.105.

For hybrid AAS BS the possibility that these signals may be transmitted over a number of transceiver units so that the control channels could be beam formed was considered.  The requirement was modified so that the control channel power on a group of TAB connectors (those responsible for transmitting the control channel) was compared to the power indicted on the BCH (UTRA) or DL-SCH (E-UTRA).

The option to have a requirement per connector was also provided by using a declared set of beam forming weights for the TAB connector group and modifying the power indicated on the BCH (UTRA) or DL-SCH (E-UTRA) by the beam forming weight for each TAB connector.

The UTRA FDD addition CPICH power requirement for MIMO is slightly more complicated (both in AAS and non-AAS requirements) as it introduces the concept of “antenna 1”, “antenna 2”, “antenna 3” and “antenna 4”, P-CPICH is transmitted on “antenna 1” and S-CPICH on “antenna 2 - 3”. In non-AAS each “antenna” refers to an antenna connector, in AAS as the signals can be beam formed by a number of transceiver units the “antenna” applies to a declared group of TAB connectors. The requirement in this case is the difference between the P-CPICH power and the S-CPICH power.

As part of the specification simplification work, it was agreed that an UTRA OTA AAS BS would by design be capable of meeting the control channel accuracy requirements, these requirements are hence removed for UTRA OTA AAS BS.

#### 5.2.5.2	Core requirement

The E-UTRA DL RS power requirements is an accuracy requirements with respect to an absolute power indicted on the DL-SCH (E-UTRA).

The indicated power is provided by the BS, and clearly this is related to the BS rated output power. For non-AAS and hybrid AAS BS this power is a conducted power (PRated,c), however the OTA AAS BS does not have a conducted interface.

It is not clear if the power indicted on the DL_SCH for the OTA is related to the conducted power or to the OTA power, however the power indicted on the DL-SCH is controlled by the vendor and is effectively a declared parameter. In addition the control signal antenna gain may not be the same as the user signal beam forming gain hence it is necessary to declare the control signal antenna gain for the requirement. As both bits of information are variable depending on the AAS BS configuration and design the requirement is that the control signal power is within the required accuracy compared to the power level which is intended. The power level for the conformance test is declared.

As the core requirement E-UTRA DL RS power is an absolute requirement, the OTA accuracy requirement can be derived using the same methodology as the EIRP accuracy requirement. The EIRP accuracy requirement allows for an additional (non-correlated) 0.9 dB error.

Hence the control channel absolute accuracy requirement is given by:

For example for DL-RS the conduced requirement is 2.1 dB so the OTA requirement should be:

The core requirement for the E-UTRA DL RS power can hence be worded as:

The control channel power of each carrier shall be within dB of the configured control channel power.

#### 5.2.5.3	Conformance requirement

The control channel power accuracy requirement is intended to ensure that channels within the modulated signal maintain their power accuracy (essentially conforming that sufficient quantization is provided). The test condition for the almost all single direction requirements such as this is that the AAS BS is operating with the highest rated power, as such all the TRX units are operating at maximum power. In such a condition beam forming (for both steering and beam width) are done using phase manipulation and does not vary the accuracy of the power allocated to the control channels. Hence it is not necessary to show conformance in more than one direction.

Conformance testing of the control channel power accuracy is done at the reference steering direction when the system is operating at maximum power (PRated,c,TRP).

## 5.3	Output power dynamics

### 5.3.1	Background information on the conducted requirement

There are a number of output power dynamics requirements defined for AAS BS in TS 37.105 [3], in general they are RAT specific and are as follows:

UTRA FDD output power dynamics requirements are applicable to AAS BS in single RAT UTRA operation (TDD 1.28Mcps, or FDD) and to AAS BS in MSR operation using UTRA:

-	UTRA inner loop power control in the downlink: Inner loop power control in the downlink is the ability of the AAS BS to adjust the transmitted output power of a code channel in accordance with the corresponding TPC commands received in the uplink.

-	Power control dynamic range : The power control dynamic range is the difference between the maximum and the minimum code domain power of a code channel for a specified reference condition

-	Total power dynamic range: Total power dynamic range is the difference between the maximum and the minimum output power for a specified reference condition. The minimum power condition is modeled by using TM4 test model with fractional power allocation condition, while the maximum power condition is for the case of full power allocation specified by TM1.

-	UTRA FDD IPDL time mask: To support IPDL location method in UTRA FDD operation, the AAS BS interrupts all transmitted signals in the downlink (i.e. common and dedicated channels). The IPDL time mask specifies the limits of the TAB connector output power during these idle periods.

E-UTRA UTRA output power dynamics requirements are applicable to AAS BS in single RAT E-UTRA operation and to AAS BS in MSR operation using E-UTRA:

-	RE Power control dynamic range: The RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power (PRated,c,TABC) for a specified reference condition.

-	Total power dynamic range: Total power dynamic range is the difference between the maximum and the minimum output power for a specified reference condition. The minimum power condition is modeled by using E-TM2/2a test model with only one RB allocation, while the maximum power condition is for the case of all RB’s allocation (as per considered E-UTRA channel bandwidth) specified by E-TM3.1/3.1a.

All requirements are applied per each TAB connector and are differential between 2 measured powers.

### 5.3.2	Output power dynamics OTA: Core requirement

All the power dynamics requirements are in band and are concerned with controlling the power of modulated signals. These wanted signals are of interest between the BS and the UE. Such requirements should be applied as directional requirements and apply to the beam peak directions associated with the beam direction pairs.

As all the conducted output power dynamics requirements are differential they can be translated to OTA output power dynamics requirements with no further modification.

### 5.3.3	Output power dynamics OTA: Conformance requirement

As all output power dynamics requirements are relative between two measured power levels, any variation due to beam steering or beam shaping OTA is the same for both measured power values. The results are therefore unaffected by beam steering or beam shaping in case of OTA output power dynamics.

Hence it is only necessary to show conformance of the OTA output power dynamics requirements in a single direction, i.e. the reference steering direction for the narrowest (i.e. highest EIRP) beam, in a similar set up to the EIRP accuracy test in sub-clause 5.2.2 , i.e. to configure and operate at maximum transmit power during test. The narrowest beam can be determined based on the manufacturer declared beams as defined in declaration D9.3, in clause 9, where the narrowest intended BeWθ and BeWϕ are to be defined at the reference beam direction.

## 5.4	Transmit ON/OFF power

### 5.4.1	Background

The TDD TX OFF power ensures that receivers of other TDD system are not desensitized by the noise from an aggressor BS.

The TX OFF level requirement was first derived for the UTRA TDD requirement, the rationale for the UTRA TDD 1.28Mcps TX off level is given in TR 25.942

Assuming the Noise Figure(NF) of BS is 7dB and Minimum Couple Loss(MCL) is 30dB. For the victim receiver, the Tx OFF power should not exceed the thermal noise. If Tx OFF power is 6dB below thermal noise, it will introduce 1dB degradation. The proposal for Tx OFF power is as follows:

NOTE:	In 1.28 Mcps TDD option, The noise figure of BS is assumed to be 7dB, it is  based on that this parameter has been approved by CWTS yet and it is easier for implementation of BS.

For E-UTRA the power level is in 1MHz (rather than 1.28MHz) and the assumed NR is 5dB the requirement is hence:

And for E-UTRA (specified as -85dBm/MHz)

There are a number of different receivers which can be identified as being victims to the Tx OFF power:

-	Own receiver – this is possible but, it is unlikely a TDD system has separate Tx and Rx antennas so is identified by the RX sensitivity requirement even for a conducted system.

-	Co-located receivers – the requirement is measured for the Tx channel – however the same noise can be expected across the whole operational band. Other co-located receivers using other channels in the band are hence protected.

-	Same geographical area receivers – most co-location requirements in the same geographical area assume a reasonable separation between BS’s, hence the isolation is assumed to be greater.

For the release 13 AAS BS requirements the TX OFF level was applied to each TAB connector as the primary purpose of the requirement is to prevent desensitization of your own receiver. With a conducted interface the receiver sensitivity requirement alone is not sufficient to guarantee that, this is further documented in TR 37.842 [4].

### 5.4.2	OTA AAS BS – Core requirement

The OTA AAS BS has both radiated minimum sensitivity and radiated reference sensitivity requirements, hence any interference to your own receiver is fully covered by those requirements.

TX OFF level therefore ensures that co-located BS and BS co-existing in the same geographical area are protected. As the very low TX OFF levels cannot be measured accurately as a TRP the co-location method is used. The OTA TX OFF requirement is therefore specified a co-location requirement.

It is not possible to separate OTA emissions from separate transceiver units therefore OTA emissions requirements are specified as the sum of the emissions from all transceiver units in the AAS BS. For spurious emissions this has been the case for both conducted and OTA emissions. The sum of the total emissions from the AAS BS is intended to be no greater than that of a non-AAS system with equivalent number of MIMO branches and hence emission are capped at a level consistent with the maximum MIMO capability of the appropriate RAT (8 for E-UTRA, 4 for UTRA), however conducted TX OFF level is specified per TAB connector and as such is not subject to the same cap. However as the conducted requirement is primarily concerned with protection of your own receiver and OTA is concerned with co-location the parameters have somewhat changed, this was noted in the TR 37.842 when the conducted requirements were derived.

As a co-located TDD receiver should be offered the same protection as any other co-located receiver it is reasonable to apply the same cap on the TX OFF level as is applied to other unwanted emissions.

The TX OFF power level, from both polarisations, is less than:

Note: there is no requirement for UTRA TDD OTA AAS BS.

## 5.5	Transmitted signal quality

### 5.5.1	General

Unless otherwise stated, the requirements in sub-clause 5.5 apply during the transmitter(s) ON period.

### 5.5.2	Frequency error

#### 5.5.2.1	Background information on the conducted requirement

The frequency error requirement is defined to capture the maximum allowable difference between an assigned frequency and the actual generated frequency. The frequency error requirement is a regulatory requirement in some regions.

#### 5.5.2.2	Frequency error OTA: Core requirement

In order to meet the beam forming requirements of the AAS BS specifically EIRP accuracy, it is necessary that the transceiver units generating the beam have a known phase relationship between each other. To do this they need to be phase locked. There are two alternative ways to achieve this:

-	The RF LO originates from a common source and is distributed to each transceiver unit, or

-	The RF LO originates locally in each of the TRX units, where individual PLL’s use the same frequency reference, i.e. each individual RF source is locked to a common reference frequency.

In both cases, any frequency error is correlated among all TRX units forming the beam, hence the frequency error is coherent, has a ‘flat’ response in the spatial domain, i.e. OTA frequency error does not depend on the selection of the measurement point within beam’s compliance directions set.

As the intention of the eAAS WI is to provide the same performance and protection as the Rel-13 conducted AAS BS specification, the core requirement for the OTA frequency error requirement is reused from conducted frequency error requirement from TS 37.105 [3] specification, i.e. for the OTA frequency error requirement, the same frequency error values as defined for the conducted frequency error requirement in Rel-13 AAS BS specification is used.

Table 5.5.2.2-1: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

#### 5.5.2.3	Frequency error OTA: Conformance requirement

As the frequency error is flat in the spatial domain it is only necessary to show conformance in a single direction. The requirement needs to be defined so that all transmitter units are active and the system is operating at the declared maximum rated total radiated power.

For the sake of minimising the number of spatial declarations and as frequency error testing is generally done at the same time as EVM testing the ‘reference direction’ of the OTA peak directions set is the most suitable direction to define for the conformance testing. For conformance testing purposes, the frequency error is tested at the maximum and minimum power settings (together with the EVM test), in order to comply with the total power dynamic range requirement (refer to the Total power dynamic range requirement for E-UTRA in clause 5.3).

### 5.5.3	Time Alignment Error

# 5.5.3.1	Background information on the conducted requirement

Timing Alignment Error (TAE) is a signal quality requirement specified for transmit diversity, MIMO transmission and CA. TAE is defined as the largest timing difference (i.e. error) between two reference signals, and is only applicable for base stations transmitting from multiple antennas via transmit diversity, MIMO, CA, or some combinations of these three. In case of NR, there is no TX diversity transmission mode defined. As a result, no TAE tests need to be developed for TX diversity for NR operation.

In the conducted test setup, the differences between two antenna ports is measured, depending of the functionality of the signal analyser, TAE between the two or more signals can be measured simultaneously. To achieve precise measurements, the RF cables being used during the conducted test should be equal in electrical length.

For AAS BS the number of TAB connectors could be large. Therefore, a concept for reducing number of test combinations was introduced in TS 37.105, sub-clause 6.5.3.4 [3]. In TAE requirement for AAS BS is designed as:

The TAE between any two TAB connectors from different transmitter groups shall not exceed the specified minimum requirements below.

, where transmitter groups are associated with the TAB connectors in the transceiver unit array corresponding to TX diversity, MIMO transmission, CA, etc.

# 5.5.3.2	Time Alignment Error OTA: Core requirement

For OTA TAE, the DUT is placed in an OTA test environment and is configured to generate beams carrying reference signals (e.g. CRS0 and CRS1 in case of E-UTRA, DMRS ports in case of NR) associated to logical antenna ports and different transmitter groups. The OTA TAE requirement is defined as a time difference between two radiated signals.

# 5.5.3.3	Time Alignment Error OTA: Conformance requirement

The conformance testing of OTA TAE can be done in an OTA test environment similar to the one defined for radiated transmit power. In terms of testing effort it is beneficial, to coordinate testing of OTA TAE with testing of other transmitter parameters such as OTA frequency error and radiated transmit power.

### 5.5.4	Modulation Quality

#### 5.5.4.1	Background information on the conducted requirement

EVM captures a maximum allowed distortion allowed due to degradations in the BS transmitter.

For UTRA, EVM is defined as:

Where Pe is the power of the error signal between a perfect RRC filtered transmitted signal without distortion and the actual transmitted signal and Pt is the power of the perfect transmitted signal without distortion.

For E-UTRA, EVM has been defined slightly differently the measurement device contains a simple equalization algorithm and the EVM is assessed after equalization (The simple equalization algorithm accounts for the fact that some static amplitude and phase deviations incurred in the BS transmitter is removed at the UE receiver). The EVM result is defined as the square root of the ratio of the mean error vector power to the mean reference power expressed in percent.

EVM is calculated as a root mean square average over 10 sub-frames.

For AAS BS in Rel-13 TS 37.105 [3], the MSR, E-UTRA and UTRA, EVM requirements are retained and applied per TAB connector.

The conducted EVM requirement applied per TAB connector needs to be translated to an OTA requirement.

Although EVM is expressed in terms of a percentage it is a ratio of 2 powers, that of the error vector and the reference power. As with ACLR whilst the reference (or wanted) signal may be subjected to beam forming the error vector power has unknown correlation level between transmitter units and hence may be beam formed or not.

If the error vector power is correlated then the EVM is constant over the OTA angular range, however if it is not correlated then it varies, as shown in figure 5.5.4.1-1.

Figure 5.5.4.1-1. 10 Element array, beam pattern and non-correlated EVM distribution (8% EVM)

#### 5.5.4.2	Modulation Quality OTA: Core requirement

The EVM requirement however is important only between the BS and its intended UE. Hence the EVM is only of importance in directions where the BS intends to communicate with the UE’s. It is therefore in a set of specific directions from the AAS BS which is of interest rather than the average or total EVM over the entire sphere. In this respect the EVM requirement is different in nature to the ACLR requirement.

The range of directions which the AAS BS meets the EVM requirement is declared by the manufacturer as OTA coverage range.

Figure 5.5.4.2-1. Example of OTA coverage range and 5 conformance points

The EVM is required to be met across the OTA coverage range.

#### 5.5.4.3	Modulation Quality OTA: Conformance requirement

For conformance testing the centre direction and the 4 extreme directions in θ and ϕ of the OTA coverage range, as shown in figure 5.5.4.2-1 are declared and tested.

For conformance testing purposes, the EVM is tested at the maximum and minimum power settings, in order to comply with the total power dynamic range requirement (refer to the Total power dynamic range requirement for E-UTRA in clause 5.3).

### 5.5.5	Transmit pulse shape filter

#### 5.5.5.1	Background information on the conducted requirement

The transmit pulse shape filter for UTRA is defined as root-raised cosine (RRC) filter with the assumed roll-off factor. There is no minimum requirement directly defined for the transmit pulse shape filter in RAN4 specifications. Furthermore, there is no conformance requirement defined for the transmit pulse shape filter.

Transmit pulse shape filter requirement is applicable only for the AAS BS in single RAT UTRA operation (TDD, or FDD), or for AAS BS in MSR operation using UTRA (TDD, or FDD).

There is no transmit pulse shape filter requirement for an E-UTRA AAS BS.

#### 5.5.5.2	Transmit pulse shape filter OTA: Core requirement

For the transmit pulse shape filter OTA requirement, use the same approach as for the conducted transmit pulse shape filter requirement, by referring to the TS 25.104 [19] and TS 25.105 [20] specifications for UTRA FDD and UTRA TDD, respectively.

#### 5.5.5.3	Transmit pulse shape filter OTA: Conformance requirement

No conformance requirement to be defined for the transmit pulse shape filter OTA requirement.

## 5.6	Unwanted emissions

### 5.6.1	General

#### 5.6.1.1	Background information on the conducted requirement

In non-AAS BS, the unwanted emissions are specified in TS 25.104 [19], TS 25.105 [20], TS 36.104 [8] and TS 37.104 [7] and apply per antenna connector. The AAS BS has a TRXUA consisting of many transceiver units which is equal to, or potentially much higher than the number of non-AAS BS transmitter antenna connectors in a non-AAS BS with comparable functionality. As there is no upper limit on the number of transceiver units that an AAS BS system has, it is important to set the total unwanted emissions limits to an amount equivalent to Rel-13 non-AAS BS.

The total unwanted emissions from the AAS BS should be no higher than the unwanted emissions from a non-AAS BS conforming to the Rel-13 specifications with either the same number of transmitter units as the AAS BS or a maximum of 8 transmitter units (E-UTRA) or 4 transmitter units (UTRA).

#### 5.6.1.2	OTA unwanted emissions

The Rel-13 AAS BS specification which defined a hybrid AAS BS with a conducted interface applied a scaling factor which was based on the number of active transmitter units as counted at the transceiver array boundary. As the OTA AAS BS architecture is restricted to having a minimum number of transceiver units (8 for E-UTRA, 4 for UTRA) there is no need for a scaling variable as a fixed scaling factor (FSF) can be used based on the RAT-specific unwanted emissions requirements.

In Rel-13 requirements the emissions scaling was also multiplied by the minimum number of cells the AAS BS was declared to support (Ncells). This is necessary as if the system supports multiple cells at the conducted interface it is not always possible to directly map TAB connectors to cells. With an OTA AAS BS there is no conducted interface and requirements are applied radiated so no such mapping knowledge is required. The complication of scaling by the minimum number of cells therefore can also be removed as the requirement applies OTA per cell. It should be noted that the restriction on the minimum number of transceiver units should also apply per cell.

The OTA AAS BS emissions limits for E-UTRA therefore are applied per cell and are based on the basic limits used in the Rel-13 AAS BS requirements multiplied by the FSF equal to 8 (or plus 9dB).

The OTA AAS BS emissions limits for UTRA therefore are applied per cell and are based on the basic limits used in the Rel-13 AAS BS requirements multiplied by the FSF equal to 4 (or plus 6dB).

Some regional regulatory requirements for unwanted emissions are expressed as EIRP in the regulations. They implicitly assume a passive antenna. Since the gain between passive antenna systems does not vary significantly (i.e. only by a few dB, not 10s of dB), EIRP correlates directly to TRP in these cases.

For MSR, requirements are applied at an antenna connector. In order to transform the EIRP level stated in the regulations to a conducted level, vendors make a declaration of the passive antenna gain they assume.

In Rel-13 AAS these emissions requirements remain conducted. A declaration of the assumed passive antenna gain to relate the EIRP to a conducted level for the AAS is needed. It is important to note that what should be declared is the gain of an equivalent passive system.

OTA unwanted emissions requirements are defined as TRP. A means is needed to relate the EIRP values in the regulation to TRP. It is important to note that the EIRP does not correspond to the instantaneous EIRP of any beamforming scheme in the AAS, but was set based on the assumption of a passive antenna. In order to relate the EIRP values to OTA, a fixed assumption is made of the gain of a passive antenna system, in order that an OTA TRP requirement can be provided for AAS BS.

Since EIRP per antenna:  PEIRP = PTx + GAnt with GAnt equal to [17] dBi for both E-UTRA and UTRA.  To interpret existing EIRP regulations to TRP would be the follow:

UTRA: OTA AAS BS emissions limits = EIRP – [17] dBi + 6 dB

E-UTRA: OTA AAS BS emissions limits = EIRP – [17] dBi + 9 dB

In the above equations directivity value should be used in above equations, however as all antenna losses are assumed zero effective antenna gain can be used.

Since a fixed passive gain is assumed, the TRP limits can be stated directly as dBm numbers in the specifications.

### 5.6.2	Occupied bandwidth

#### 5.6.2.1	Background information on the conducted requirement

For UTRA and E-UTRA, if the ACLR requirements are met, then usually it is not possible for the system to fail the occupied bandwidth requirement, however the requirement still applies since some regulatory authorities expect an occupied bandwidth requirement for all systems.

According to SM.328 [9] occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. It defines the spectral properties of emission in a simple manner. The value of /2 is taken as 0.5%. This results in a power bandwidth of 99%, meaning that the occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated mean power of the transmitted spectrum on the assigned channel.

#### 5.6.2.2	Occupied bandwidth OTA: Core requirement

For occupied bandwidth, the beam characteristics are not important. The requirement should however cover the fact that all transmitter is active and the system is operating at the maximum declared rated total radiated power.

For the OTA requirement, the same occupied bandwidth value (i.e. /2= 0.5%) as for the conducted occupied bandwidth requirement for Rel-13 AAS BS is to be used.

As the occupied bandwidth is a measure of the wanted signal its performance is flat in space and can be completely characterized as a directional requirement or a TRP requirement. As performance of the wanted signal is generally characterized on the main beam as a directional requirement the same is done for occupied bandwidth.

Occupied bandwidth is therefore specified as a directional requirement valid over the OTA coverage range.

#### 5.6.2.3	Occupied bandwidth OTA: Conformance requirement

For conformance a beam should be identified which fulfils the core conditions.

### 5.6.3	Adjacent Channel Leakage Radio

#### 5.6.3.1	Background information on the conducted requirement

The Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency, expressed in dB. ACLR is defined for both the first adjacent channel and the second adjacent channel.

The Wide Area BS ACLR was originally derived by means of co-existence simulations as documented in the TR 36.942 [5]. The coexistence simulations captured DL network throughput loss due to interference from an aggressor network in the downlink vs. so-called ACIR. ACIR incorporates power in the UE receive band due to the transmitter unwanted emissions (ACLR) and the receiver (ACS), and is calculated as:

(assuming linear terms)

The coexistence simulation result for ACIR was split into ACLR and ACS, resulting in for wide area BS ACLR of 45dB and a UE ACS of 33dB.

An AAS BS may consist of multiple transmitters that perform beamforming. If the adjacent channel interference is not necessarily beam formed in the same manner as the wanted channel, hence ACLR may vary in space. Before setting a conducted requirement in the Release 13 AAS BS specification TS 37.105 [3], further co-existence simulations were performed in the AAS SI TR 37.840 [6] and AAS WI TR 37.842 [4]. These simulations revealed that it is the total power and not the spatial distribution of unwanted emissions (i.e. the correlation of adjacent channel noise between transmitter units) that impacts co-existence KPIs. Hence an ACLR based on total wanted channel and adjacent channel power was defined at TAB connectors.

In addition to the ACLR ratio of 45 dB, an absolute level of adjacent channel emissions is defined. The base station fulfils whichever is the less stringent of the ACLR ratio or the absolute level. The definition of an absolute level avoids specifying unnecessarily low levels on any BS that operate with a low wanted carrier transmit power. For wide area BS, the absolute level is defined in TS 37.104 [7] as either -13 dBm/MHz for category A, or -15dBm/MHz for category B emissions requirements, relating to regulatory frameworks.

In the AAS BS specification TS 37.105 [3] the ACLR requirement is set on the ratio of the total wanted signal power at all TAB connectors to the total power at all connectors of adjacent channel power. For the absolute requirement, the non-AAS requirement forms a basic limit, which is scaled up to an AAS BS requirement according to the number of active transceiver units, up to a maximum 8 times for E-UTRA (4 for UTRA), in the same manner as the scaling applied to other unwanted emissions requirements.

For medium range BS and local area BS, the ACLR requirement of 45 dB was retained in TS 37.104 [7] without any further simulation investigations. For TS 37.105 [3], the 45 dB ACLR was also retained for the other base station classes.

In the non-AAS specifications, the absolute limits for adjacent channel emissions are adjusted for the medium range and local area BS classes. For the local area BS class, the absolute limit are adjusted to -32 dBm/MHz. The limit of -32 dBm/MHz corresponds to an ACLR of roughly 45 dB for a local area BS with a 10 MHz carrier operating at the maximum permitted output power. For the medium range BS, the absolute limit are also adjusted. The absolute limit is set to -25 dBm/MHz. This again roughly corresponds to an ACLR of 45 dB for a medium range BS transmitting a 10MHz carrier near to maximum allowable output power, and also matches to the WCDMA SEM.

The absolute unwanted emissions requirements for medium range BS and local area BS were also used as basic limits in TS 37.105 [3], with the scaling mechanism applying in the same way as for wide area BS.

In addition to ACLR, a requirement on so-called Cumulative ACLR (CACLR) is applied for multicarrier and multiband BS. The CACLR requirement considers adjacent channel emissions falling onto a carrier that is adjacent to 2 active carriers; one at a lower frequency offset and one a higher frequency offset. The CACLR requirement is also 45 dB.

The CACLR requirements also apply in TS 37.105 [3] in the same manner as ACLR.

#### 5.6.3.2	ACLR OTA: Core requirement

Based on the findings of the simulation work done in TR 37.842 [4] the effect of adjacent channel interference on the throughput of adjacent networks is dependent on the total radiated adjacent channel power rather than the power in the main beam.

As the number of active transceiver units generating the beam and the coherence level of the adjacent channel noise from each active transceiver unit is unknown it is not possible to derive a relationship between the ACLR of a single point in the centre of the main beam (i.e. as ratio of 2 EIRP values) and the ACLR of the total radiated power.

The ACLR requirement is hence based on a ratio of the total radiated filtered mean power centred on the assigned channel frequency to the total radiated filtered mean power centred on an adjacent channel frequency:

.

The OTA ACLR requirement can be derived as:

OTA ACLR limit = Conducted ACLR limit + 10*log10(FSF)

Where:

-	Conducted ACLR limit: the AAS limit as defined in TS 37.105 [3] specification,

-	FSF: Fixed Scaling Factors for the OTA AAS BS. For UTRA the FSF = 4, for E-UTRA the FSF = 8,

Therefore the resulting OTA ACLR limit can be derived as:

-	UTRA: OTA ACLR limit = Conducted ACLR limit + 6 dB

-	E-UTRA: OTA ACLR limit = Conducted ACLR limit + 9 dB

#### 5.6.3.3	ACLR OTA: Conformance requirement

In order that the requirement is measurable the total radiated power may be defined as the sum of the EIRP on a grid of discrete directions around the sphere.The ACLR is calculated as described in sub-clause 10.8.

To properly capture the EIRP, two orthogonal polarizations must be summed together.

The points required for conformance of the ACLR OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion in sub-clause 10.8.

### 5.6.4	Spectrum emission mask

#### 5.6.4.1	Background information on the conducted requirement

Based on the conducted spectrum emission mask (SEM) requirement specified in TS 37.105 [3] for single RAT UTRA operation, the SEM limits are defined based on the basic limit definition over the frequency range starting with the carrier frequency offset of 2.5 MHz up to the fmax as defined in TS 37.105 [3].

There is no spectrum emission mask requirement for AAS BS in single RAT E-UTRA operation, nor for AAS BS in MSR operation using E-UTRA.

#### 5.6.4.2	Spectrum emission mask OTA: Core requirement

The metric used to capture SEM OTA is TRP.

The spectrum emission mask level for the OTA requirement can be derived as:

OTA SEM limit = Conducted SEM limit  + 10*log10(FSF)

Where:

-	Conducted SEM limit: the AAS limit as defined in TS 37.105 [3] specification,

-	FSF: Fixed Scaling Factors for the OTA AAS BS. For UTRA the FSF = 4,

Therefore the resulting OTA SEM limit can be derived as:

OTA SEM limit = Conducted SEM limit + 6 dB

#### 5.6.4.3	Spectrum emission mask OTA: Conformance requirement

Using TRP as a figure of merit to provide equivalent OTA SEM performance and protection to that of the existing conducted requirement, the conducted requirements can be adopted by translating the test procedure and requirement from TS 37.145-1 [23], sub clause 6.6.4 into the spatial domain. The TRP assessment grids and methods are described in sub-clause 10.8.

The points required for conformance of the SEM OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion in sub-clause 10.8.

### 5.6.5	Operating band unwanted emission

#### 5.6.5.1	Background information on the conducted requirement

Based on the conducted operating band unwanted emission (OBUE) requirement specified in TS 37.105 [3] for single RAT E-UTRA or for MSR operation, the OBUE limits are defined from 10 MHz below the lowest frequency of each supported downlink operating band to the lower Base Station RF Bandwidth edge located at FBW RF,low and from the upper Base Station RF Bandwidth edge located at FBW RF,high  up to 10 MHz above the highest frequency of each supported downlink operating band.

There is no operating band unwanted emission requirement for a single RAT UTRA FDD AAS BS, single RAT UTRA TDD AAS BS, nor MSR AAS BS operating in UTRA (FDD or TDD).

#### 5.6.5.2	Operating band unwanted emission OTA: Core requirement

The operating band unwanted emissions level for the OTA requirement can be derived as:

OTA OBUE limit = Conducted OBUE limit

, where conducted OBUE limit is the AAS limit as defined in TS 37.105 [3] specification.

Based on the discussion on the boundary between in-band and out of band region for NR FR1 operation, the ΔfUEM values for the boundary between OBUE and TX spurious emissions were agreed for AAS BS as presented in table 5.6.5.2-1. Those ΔfUEM values are applicable to conducted and radiated requirements for AAS BS in single RAT E-UTRA operation and to AAS BS in MSR operation using E-UTRA, and are applicable for all E-UTRA operating bands defined in TS 36.104 [7].

Table 5.6.5.2-1: ΔfUEM values for the boundary between OBUE and TX spurious emission regions

|  |  | E-UTRA AAS BS (Single-RAT, MSR) |
| --- | --- | --- |
| ΔfUEM | Operating band <100 MHz | 10 MHz |
|  | 100 MHz ≦ Operating band ≦ 200 MHz | 40 MHz |

#### 5.6.5.3	Operating band unwanted emission OTA: Conformance requirement

Using TRP as a figure of merit to provide equivalent OBUE performance and protection to that of the existing conducted requirement, the conducted requirements can be adopted by translating the test procedure and requirement from TS 37.145-1 [23], sub-clause 6.6.5 into the spatial domain. The TRP assessment grids and methods are described in sub-clause 10.8.

Thepoints required for conformance of the OBUE OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion in sub-clause 10.8.

### 5.6.6	Transmitter spurious emissions

#### 5.6.6.1	Mandatory Spurious emission requirements

##### 5.6.6.1.1	Background information on the conducted requirement

According to TS 37.105 [3], the transmitter spurious emission limits for the hybrid AAS BS operating bands apply in the frequency range from 9 kHz to 12.75 GHz, with consideration of the following exceptions:

-	Unwanted emissions: the spurious requirements apply at frequencies within the above specified frequency range with the exception of the UTRA spectrum emission mask (SEM) and E-UTRA operating band unwanted emissions (OBUE) ranges as specified below:

-	UTRA TDD BS 1.28 Mcps option as specified in TS 25.105 [20]: from 4 MHz below the lowest frequency of each operating band to 4 MHz above the highest frequency of each operating band.

-	UTRA TDD BS 3.84 Mcps option as specified in TS 25.105 [20]: from more than 12.5 MHz under the lowest carrier frequency used or more than 12.5 MHz above the highest carrier frequency used.

-	UTRA TDD BS 7.68 Mcps option as specified in TS 25.105 [20]: from more than 25 MHz under the lowest carrier frequency used or more than 25 MHz above the highest carrier frequency used.

-	UTRA FDD BS as specified in TS 25.104 [19]: from 12.5MHz below the lowest carrier frequency used up to 12.5MHz above the highest carrier frequency used.

-	E-UTRA BS as specified in TS 36.104 [8]: from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band.

-	MSR BS as specified in TS 37.104 [7]: from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band.

-	Upper frequency limit of the spurious range: in order to comply with the SM.329 recommendation [18], in case of spurious emissions requirement for certain operating bands, the upper limit of the spurious range is extended beyond 12.75GHz to the limit of 5th harmonic (but not higher than 26GHz). For more details on the upper limit setting in those cases, refer to SM.329 [18].

For AAS BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, this exclusion applies for each supported operating band.

##### 5.6.6.1.2	Transmitter spurious emissions OTA: Core requirement

The metric used to capture transmitter spurious emissions OTA is TRP.

The emission levels and the measurement bandwidths for the transmitter spurious emissions OTA requirement are based on the conducted requirements specified in TS 37.105 [3].

In case of OTA AAS BS requirement, emission levels are subject to fixed scaling factor (FSF), as described in sub-clause 5.6.1.2. The Tx spurious emissions level for the OTA requirement can be derived as:

OTA Tx spurious emissions limit = Conducted Tx spurious emissions limit + 10*log10(FSF)

Where:

-	Conducted Tx spurious emissions limit: the AAS limit as defined in TS 37.105 [3] specification,

-	FSF: Fixed Scaling Factors for the OTA AAS BS. For UTRA the FSF = 4, for E-UTRA the FSF = 8,

Therefore the resulting OTA Tx spurious emissions limit can be derived as:

UTRA: OTA Tx spurious emissions limit = Conducted Tx spurious emissions limit + 6 dB

E-UTRA: OTA Tx spurious emissions limit = Conducted Tx spurious emissions limit + 9 dB

The core requirement for the transmitter spurious emissions OTA is applied over 30 MHz to 12.75 GHz range, with consideration of the SEM and OBUE exceptions, and the operating band specific upper limit extensions of the spurious range,

Based on the discussion on the boundary between in-band and out of band region for NR FR1 operation, the ΔfUEM values for the boundary between OBUE and TX spurious emissions were agreed for AAS BS as presented in table 5.6.5.2-1. Those ΔfUEM values are applicable to conducted and radiated requirements for AAS BS in single RAT E-UTRA operation and to AAS BS in MSR operation using E-UTRA, and are applicable for all E-UTRA operating bands, except Band 46.

Where requirements for co-existence with other systems apply, the metric is TRP and the OTA requirement is based on the conducted requirements specified in TS 37.105 [3] for co-existence, subjected to fixed scaling as described in sub-clause 5.6.1.2. TRP has been identified as the most suitable metric for spurious emissions. This is due to the fact that decorrelation between emissions of different transmitter units, different antenna response in the spurious domain and time varying beamforming averages out the spatial pattern of the unwanted emissions. Coexistence studies indicate that degradation to other systems is generally related to the total amount of radiated interference. Furthermore the existing conducted requirement captures TRP.

##### 5.6.6.1.3	Transmitter spurious emissions OTA: Conformance requirement

In order that the requirement is measurable the total radiated power as defined in sub-clause 5.1.1 may be approximated as the sum of the EIRP on a grid of discrete directions around the sphere as described in sub-clause 10.8.

For the out-of-band region where the far field cannot be easily achieved other approximations based on power density should be considered,. Other measurement techniques are not precluded. For the upper part of the spurious region, the path-loss in the test chamber needs to be conserved, therefore power density should be considered.

An approximation of TRP measurement can be achieved by sampling discrete EIRP measurements surrounding the AAS BS.  The use of discrete sampling grid for the TRP approximation is motivated by the observation that in the spurious domain there is no coherent beam forming, as the AAS BS are designed to form beams based on superposition of signals from individual transceivers in TRXU array in their in-band frequency ranges, while in the spurious domain those signals are expected to be sufficiently uncorrelated. The points required for conformance of the SEM OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion in sub-clause 10.8.

Conformance requirement for the TX spurious requirement OTA is composed as a single requirement together with the EMC RE requirement OTA, as there is no possibility to separate those requirements in OTA test setup. The resulting test requirement will be subject to single test setup and single measurement procedure. Definition of the conformance requirement will be subject to TRP MU, where the MU definition is FFS. For more details on the EMC RE requirement, refer to clause 8.

{editors note: to be completed as part of conformance work}

#### 5.6.6.2	Co-location requirements

##### 5.6.6.2.1	Background

Spurious emission co-location requirements are applied for the protection of other BS receivers co-located with another BS.

The conducted requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

The levels are specified so that the co-located victim BS receiver sensitivity does not degrade when co-located.

For example an E-UTRA wide area BS has a sensitivity of -101.5dBm for 5 MHz channel bandwidth.

The co-location noise requirement level is -96dBm/100kHz so this would cause a sensitivity degradation of:

Or a 0.6dB desensitization.

##### 5.6.6.2.2	Core requirements

The OTA spurious emission co-location requirements are specified as a co-location requirement as described in sub-clause 5.1.2.

The output of the co-location reference antenna is therefore the conducted requirement level minus the 30dB co-location isolation assumption.

The conducted spurious emission co-location requirements are specified in non-AAS per antenna connector and as a sum of all the power at the transceiver array boundary in the hybrid AAS requirements. As such the level specified in the non-AAS requirements is subject to scaling when applied to the AAS system requirements. As the OTA AAS BS architecture is limited to systems with 8 or more transceiver units (4 for UTRA) a fixed scaling factor is added to the non-AAS limit.

As the requirement is based on the total output from multiple transceivers if the AAS BS is radiating on two orthogonal polarisations then the power level out of the co-location reference antenna should be the sum of both polarisations.

The total power level, from both polarizations, at the output of the co-location reference antenna and is:

where X = 9dB for E-UTRA and X=6dB for UTRA

##### 5.6.6.2.3	Conformance requirements

During conformance testing the co-location reference antenna may be replaced with a measurement antenna with a known performance. The core requirement level is translated to a conformance requirement level considering the difference in OTA performance between the co-location reference antenna and the measurement antenna.

It is expected that as the measurement antenna has higher gain in the direction of the DUT than the co-location reference antenna the conformance power level is higher than the core level. This makes measurement more practical as the levels are close to the thermal noise floor.

The translation from the co-location reference antenna to the measurement antenna are FFS.

As the translation from the co-location reference antenna to a measurement antenna with higher gain requires detailed analysis and experimentation, it is probably that initial measurements are done with test antennas which are co-location reference antenna as described in subclause 5.1.2.

In sub-clause 5.1.2 the co-location reference antenna is described as a passive single column BS antenna suitable for the specified frequency range. As such the signal level measured during the test will be the same as the core requirement, which is very low:

-	OTA Transmit ON/OFF power (E-UTRA only),	-106 dBm/MHz

-	Protection of the BS receiver of own or different BS ,	-119dBm/100kHz (E-UTRA BC2), -120 dBm/100kHz (UTRA)

-	Co-location with other BS, -122dBm/100kHz (WA, GSM bands),  -120dBm/100kHz (WA UTRA and E-UTRA bands)

The worst case is the GSM co-location requirement which is only 2dB above the thermal noise floor at -124 dBm/100kHz.

As the emission received at the measurement receiver PUEM for co-location requirements are very low, it is suggested to measure the noise change. The relations between measured noise change 1, noise floor N0 and the relation to PUEM with respect to the noise floor denoted 2 is visualized in linear scale in Figure 5.6.6.2.3-1.

Figure 5.6.6.2.3-1: Relative noise measurement

The absolute emission level in decibel scale is calculated as:

, where N0 is the noise floor of the measurement receiver and 2 is found in Figure 5.6.6.2.3-2.

Figure 5.6.6.2.3-2: Measured noise change to relative noise level

The absolute noise floor of the measurement receiver, including probe antenna, cables, filter and LNA is determined by a calibration procedure. The calibration will determine the absolute emission level (N0) accuracy of measuring out-of-band unwanted emission close to the thermal noise floor.

## 5.7	Transmitter intermodulation

### 5.7.1	General

The transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array. The requirement applies during the transmitter ON period and the transmitter transient period. The interfering signal level is derived based on the co-location assumption that the co-located BS is of the same class and output power level as the BS under test. The OTA TX IMD interferer level can hence be regarded as a co-location requirement.

### 5.7.2	Background information on the conducted requirement

In Rel-13 AAS base station specification, TS 37.105 [3], sub-clause 6.7.1 there are two types of transmitter intermodulation cases captured by the transmitter intermodulation requirement:

1.	Co-location transmitter intermodulation in which the interfering signal is from a co-located base station.

2.	Intra-system transmitter intermodulation in which the interfering signal is from other transmitter units within the AAS BS.

# 5.7.3	Transmitter intermodulation OTA: Core requirement

The minimum requirement for the transmitter intermodulation requirement is that the emissions output level does not exceed the unwanted emission limits specified for operating band unwanted emission in sub-clause 5.6.5.1, transmitter spurious emission in sub-clause 5.6.6.2, and ACLR in sub-clause 5.6.3.2 in the presence of a wanted signal and an interfering signal in an OTA environment.

For the conducted requirement the transmitter spurious emissions requirement includes the mandatory emissions requirements, protection of own or different BS , additional spurious emission requirements (co-existence in same geographical area) and co-location requirements. As the protection of own or different BS and the co-location requirements are specified as an output of the co-location reference antenna and the TX IMD requirement uses the co-location reference antenna as an input these emissions requirements are not included as part of the OTA TX IMD requirement.

The interfering signal level is the same power level as the AAS BS (PRated,t,TRP) fed into a co-location reference antenna.

If the AAS BS has dual polarised antennas then the co-location reference antenna also needs to have dual polarised antennas and PRated,t,TRP / 2 is input to each polarisation.

When OTA ACLR, OTA OBUE and OTA spurious emission is defined, the relevance for having a separate intra-system requirement disappears. The intra-system case is covered by OTA unwanted emission requirement including the unwanted emission for the whole system, hence a separate intra-system IMD requirement is not needed when considering OTA.

# 5.7.4	Transmitter intermodulation OTA: Conformance requirement

[Text to be added]

# 6	Radiated receiver characteristics

## 6.1	General

### 6.1.1	Spatial requirements

Overview of the radiated Rx requirements of the AAS BS is presented in table 6.1.1-1.

Table 6.1.1-1: Overview of radiated Rx requirements

| AAS BS requirement | OTA requirement type | Applicability levels | Coverage range | Number of conformance directions |
| --- | --- | --- | --- | --- |
| OTA sensitivity | Single direction | - | Receiver target redirection range (D10.8) | 5 |
| OTA reference sensitivity | Single direction | OTA REFSENS | OTA REFSENS RoAoA | 5 |
| Dynamic range | Single direction | OTA REFSENS | OTA REFSENS RoAoA | 1 |
| In-band selectivity and blocking | Single direction | OTA REFSENS and minSENS | OTA REFSENS RoAoA and minSENS RoAoA | 5 |
| ACS and narrowband blocking | Single direction | OTA REFSENS (NB blocking only) and minSENS | OTA REFSENS RoAoA (NB blocking only) minSENS RoAoA (NB blocking and ACS) | 5 (blocking) 1 (ACS) |
| Out-of-band blocking | Single direction | minSENS | minSENS RoAoA | 1 |
| Receiver spurious emissions | TRP | - | n/a | - |
| Receiver intermodulation | Single direction | OTA REFSENS and minSENS | OTA REFSENS RoAoA and minSENS RoAoA | 1 |
| In-channel selectivity | Single direction | minSENS | minSENS RoAoA | 1 |

Note: Single direction implies a single direction requirement, i.e. the requirement is applied in a single direction at a time, not that the requirement is only specified in 1 direction.

For conformance testing of the radiated RX requirements, FRCs defined in annex A of UTRA FDD specification in TS 25.104 [19], or E-UTRA specification in TS 36.104 [8] is reused, where applicable.

### 6.1.2	Receive loss factor assumption

The receiver loss factor is used to translate conducted power levels to OTA power levels..

The receiver loss factor accounting for antenna losses, distribution losses, integration losses etc. in the receiver path(s) for the in-band, out-of-band and spurious region is assumed to be 0dB.

For emissions the same reasoning as the transmitter that assuming the same conducted and radiated power level as assumed in ITU-R SM.329 [18].

### 6.1.3	OTA offsets

Most of the conducted receiver requirements (with the exception of receiver spurious emissions) are based on wanted and interferer power levels being applied to the antenna connector. The OTA requirements are based on these conducted levels and are based on an OTA power level from a single direction where the AoA is within the specified RoAoA.

As there are 2 OTA sensitivity requirements described in sub-clauses 6.2 there are effectively 2 antenna gain offsets which are used for each of the receiver requirements:

OTA sensitivity requirement is declared, a number of declarations may be made with different EIS values and different RoAoA and receiver target redirection range(s). The lowest declared EIS value from all the OSDD declarations is used to represent the system minimum sensitivity (minSENS).

If receiver requirements include the effect of the receiver noise figure then  the OTA levels should be based on minSENS or offset by ΔminSENS where,

ΔminSENS = REFSENSConducted – minSENS

OTA REFSENS is based on the OTA REFSENS RoAoA which is intended to estimate the element antenna gain, assuming a low gain results in higher OTA power levels. If the receiver requirements are intended include the effect of receiver non-linearity’s due to high signal levels the OTA levels should be based on OTA REFSENS or offset by ΔREFSENS. ΔREFSENS is derived in sub-clause 6.2.2.2.

In some cases where receiver performance includes both the effect of receiver noise figure and of non-linearity the requirements are specified using both offsets.

## 6.2	Reference sensitivity level

### 6.2.1	Background information on the conducted requirement

The conducted reference sensitivity serves two purposes. The first purpose is as a minimum RF performance requirement for sensitivity. The second purpose is as a reference level from which other RX requirements are defined.

The wide area sensitivity requirement has been derived based on estimation and assumption of the achievable noise figure in the BS receiver and the SNR requirement for the reference measurement channel.

For the medium range BS class, system simulations were conducted based on 2 network layers of which one was a macro layer and a second a micro layer. The micro sensitivity is set such that the macro layer is not significantly degraded due to large power UL transmissions in the micro layer, whilst the micro layer sensitivity is somewhat relaxed due to the fact that interference from the macro layer renders achieving the same sensitivity level not useful. Based on the simulations, a different sensitivity level than for the wide area BS class was decided.

For the local area BS class, system simulations and calculations of interference to the pico cells due to other network layers was calculated. The presences of potentially strong interference due to UEs transmitting to micros or macros means that a tight sensitivity for a pico cell does not make sense from a performance or cost perspective.

Thus in effect the wide area sensitivity is based on achievable performance, whilst the sensitivity for the other layers is based on interference conditions in a heterogeneous network.

### 6.2.2	OTA sensitivity requirements

#### 6.2.2.1	General

In the Rel-13 AAS BS specification TS 37.105 [3] there are 2 requirements related to receiver sensitivity:

-	Reference sensitivity part of the conducted receiver requirements; applies per TAB connector.

-	Sub-cause 10.2 OTA sensitivity: Part of the radiated receiver requirements; applies in the far field region over the declared OSDD.

The Rel-13 OTA sensitivity requirement currently is based on meeting a declared EIS value over a declared RoAoA. This declared requirement is maintained for Rel-15 AAS BS OTA sensitivity requirement. In addition, in order to provide a minimum EIS requirement level which can replace the conducted reference sensitivity level, additional minimum requirement is set based on the existing conducted minimum requirement in Rel-13.

As the declared RoAoA for the Rel-13 OTA sensitivity requirement is not bounded by specific requirements (such as the 3dB contour) it can capture a much larger RoAoA and hence demonstrate the spatial performance of the receiver including the antenna. One important characteristic which can be captured is the scan loss of the system due to large steering angles and antenna geometry.

The OTA reference sensitivity (OTA REFSENS) requirement is the OTA equivalent of the conducted reference sensitivity, its intention is not to specifically capture antenna performance but rather to estimate an equivalent passive antenna gain if it were being used in the same deployment scenario and show equivalence to the conducted requirement.

The OTA REFSENS requirement does not replace the existing OTA sensitivity requirement in sub-clause 9.2 of TS 37.105 [3] but it is an additional new OTA requirement.

As OTA reference sensitivity is used as a reference for many of the interference requirements where it is important that the interferer is polarization matched to the receiver antenna it is also important that the wanted signal is polarization matched. Hence OTA reference sensitivity is specifically declared per polarization and the EIS values are under the assumption of polarisation match.

#### 6.2.2.2	Core requirement

##### 6.2.2.2.1	OTA Sensitivity

The declared OTA sensitivity requirement is based upon declaration of one or more OSDD, following the concept defined for Rel-13 AAS specification TS 37.105 [3], sub-clause 10.2.

##### 6.2.2.2.2	OTA reference sensitivity

OTA REFSENS is specified over the RoAoA which define a contour equivalent to the 3dB beam width of a non-AAS passive antenna covering the same RoAoA. Using the 3dB contour allows for simple estimation of equivalent directivity by use of standard formula.

The OTA REFSENS level (EISREFSENS) depends on the equivalent directivity of a passive antenna pattern which has the same beam width(s) as the AAS element or sub-array (declared via OTA REFSENS RoAoA) and can be calculated in dBm as follows:

Where:

-	DRX_OTA_MARGIN is an implementation margin to allow for errors associated with beam forming in the UL. The DRX_OTA_MARGIN value is 1dB.

-	PREFSENS is the conducted reference sensitivity level in dBm in the Rel-13 TS 37.105 [3].

-	D0 is the estimated antenna peak directivity in dBi of a non-AAS BS, which has a beam pattern related to the AAS BS OTA REFSENS RoAoA region.  D0 is approximated by the Elliot’s formula and expressed as,

Where,

-	BeWθ,REFSENS is the beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees.

-	BeWφ,REFSENS is the beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees.OTA REFSENS RoAoA is declared by the vendor.

-	Doff-peak is the peak directivity off-peak margin, it is defined as follows:

-	The OTA REFSENS RoAoA is defined as the contour where the EIS is 3dB higher than in the reference direction, therefore by definition Doff-peak in the reference direction is 0dB. In all other directions within the OTA REFSENS RoAoA Doff-peak is 3dB.

Redirection capability is a spatial performance characteristic of the antenna and hence is part of the OTA sensitivity requirement. The OTA REFSENS requirement demonstrates the minimum sensitivity of the OTA AAS BS receiver and hence requires only a single OTA REFSENS RoAoA to be declared per operating band.

DRX_OTA_MARGIN allows for degradation in receiver performance due to beam forming aspects, including (but not excluded to):

-	Steering error

-	Correlation of noise sources in RX units.

-	Baseband combining efficiency/implementation margin

Steering error is primarily due to errors in phase alignment of the receiver units causing a steering error, this is similar to the beam steering error estimated for the EIRP accuracy value. The average EIRP beam steering  error component of the EIRP accuracy documented in TR 37.842 [4] was 0.85dB.

Correlation of noise sources between the receivers may occur due to mutual coupling of the antenna elements resulting in correlated noise being present in each of the receiver units, correlation of common noise sources such as LO’s. With a mutual coupling of 15dB it is estimated that the effect is approximately a 0.4dB degradation in system noise figure.

As the contributors are independent the rms sum of the factors is approximately 1dB, hence the value of DRX_OTA_MARGIN is1dB.

The offset between conducted reference sensitivity and OTA REFSENS (ΔREFSENS) is used for a number of the receiver requirements and can be defined as:

In the reference direction:

In all other directions in the OTA REFSENS RoAoA:

So OTA REFSENS can be written as:

#### 6.2.2.3	Conformance requirement

##### 6.2.2.3.1	OTA Sensitivity

Conformance testing for OTA sensitivity is performed for the five directions same as the Rel-13 OTA sensitivity requirements as described in TR 37.941 [36]. Same requirements for Rel-13 OTA sensitivity apply.

The conformance directions in minSENS RoAoA can be defined as the following five directions.

-	The receiver target reference direction.

-	The direction determined by the maximum  value achievable inside the minSENS RoAoA maintaining the receiver target reference direction  value.

-	The direction determined by the minimum  value achievable inside the minSENS RoAoA maintaining the receiver target reference direction  value.

-	The direction determined by the maximum  value achievable inside the minSENS RoAoA maintaining the receiver target reference direction  value.

-	The direction determined by the minimum  value achievable inside the minSENS RoAoA maintaining the receiver target reference direction  value.

##### 6.2.2.3.2	OTA reference sensitivity

Conformance testing for OTA reference sensitivity is performed for the following five directions.

-	The receiver target reference direction.

-	The direction determined by the maximum  value achievable inside the OTA REFSENS RoAoA maintaining	the receiver target reference direction  value.

-	The direction determined by the minimum  value achievable inside the OTA REFSENS RoAoA maintaining	the receiver target reference direction  value.

-	The direction determined by the maximum  value achievable inside the OTA REFSENS RoAoA	maintaining the receiver target reference direction  value.

-	The direction determined by the minimum  value achievable inside the OTA REFSENS RoAoA maintaining	the receiver target reference direction  value.

The manufacturer declares the five directions for conformance testing.

The measured EIS should be less than or equal to the EISREFSENS + Test Tolerance value.

#### 6.2.2.4	Extreme conditions

The exiting core conducted requirements in TS 37.105 [3] do not explicitly identify an extreme conditions receiver requirement, however in the conformance requirement TS 37.145 [24, 25] conducted reference sensitivity has a conformance requirement over the extreme conditions. The same is true for the 25, 36 and 37 series non-AAS requirements.

It is not cleat the reason why the core requirement is not captured, however to keep the specifications as similar is possible the same approach is used in the OTA part of the AAS specification.

Extreme temperature requirements for sensitivity could be applied to either OTA sensitivity or to OTA REFSENS, however as OTA REFSENS is intended primarily as the reference performance level for the interference requirements and it is larger or equal to the OTA sensitivity level, the extreme requirements are best applied to OTA sensitivity rather than OAT REFSENS. By passing OTA sensitivity requirements the extreme performance of OTA REFSENS is also demonstrated.

As there may be multiple OSDD’s declared for OTA sensitivity (OTA REFSENS has a single OTA EFSENS RoAoA) and hence a greater number of conformance directions. Extreme condition conformance directions should be minimised and restricted to reference directions only.

## 6.3	Dynamic range

### 6.3.1	Background information on the conducted requirement

The dynamic range requirement is related to the maximum co-channel input power than the receiver is expected to experience during operation. The receiver is exposed to a wanted signal and an interfering signal. The interfering level is set at around 20 dB above the receiver noise floor (assuming 5 dB NF in the receiver) and represents an estimated inter-cell interference scenario in an interference limited uplink (that was derived through simulations). The wanted signal level is set to a level at which 95% throughput is obtainable for a 16QAM based FRC. In effect, the RF level is that expected when the receiver is achieving a high SINR despite a large interference level.

Part of the design of the requirement involves deciding on an estimate for a worst-case level of inter-cell interference that is experienced by the BS receiver. The existing simulations of inter-cell interference performed for the existing requirement are based on a fixed passive BS antenna.

### 6.3.2	Core requirement: Receiver dynamic range

Under the assumption that there is no spatial rejection of the interference, in order to provide a reasonable conformance test, it was agreed that the requirement assumes that the wanted signal and interfering signal come from the same direction.

It is important to consider with this requirement that both wanted signal and AWGN interferer is transmitted from the test equipment during the OTA test. Since the AWGN interferer is much higher than the receiver noise floor, the AWGN interferer dominates the SINR. Both RF and in baseband provides the same combining gain for both the wanted signal and the AWGN, since both are co-channel and come from the same direction.

Thus, there is no impact of combining gain to the requirement. For this reason, a 2-level requirement, such as is discussed for the receiver in band blocking requirement is not necessary for receiver dynamic range.

As the wanted signal and the AWGN interferer are both above the receiver noise floor the core requirement is based on OTA REFSENS assumptions and both the wanted and interferer conducted power levels are offset by ΔREFSENS.

### 6.3.3	Conformance requirement: Receiver dynamic range

Testing is defined in the receiver target reference direction. The wanted signal and interfering signal are both aligned in space.

## 6.4	In-band selectivity and blocking

### 6.4.1	Background information on the conducted requirement

The in band receiver blocking requirement captures minimum requirements on several aspects of receiver behaviour. The conducted in band blocking requirement is based upon placing a high power interferer within the band on another carrier to a wanted signal. The Several types of interferer and several frequency relationships are defined in different blocking tests.

Figure 6.4.1-1 Example of an RX blocking requirement

The basic principle of the blocking test is that when the blocker is applied, a throughput target on the wanted signal should continue to be achieved. The wanted signal is set at 6 dB above reference sensitivity with the same RMC as for reference sensitivity. Thus in principle distortion arising from the blocker is allowed to desensitize a receiver operating at exactly reference sensitivity by around 6 dB.

It should be noted that the wanted signal level is set with respect to reference sensitivity, which is a minimum requirement and not the actual sensitivity level of the receiver, which may be lower.

Two signal levels are required for the blocking requirement; the blocker interferer level and the wanted signal level. To derive the blocker interferer level for wide area BS, uplink system level simulations were performed as documented in the TR 36.942 [5]. The blocker interferer level was decided partially by considering the 99.99th percentile of RX power observed from a non co-located aggressor LTE system on a victim system but mainly based on the UE transmit power and MCL. Later on, the AAS SI in 37.840 [6] investigated the dependency of the conducted blocker interferer level on antenna architecture and concluded that for the investigated array types, the conducted blocker interferer level was similar both for a receiver attached to a passive array and one attached to an individual antenna element. Based on this, in 37.105 [3] a per TAB connector conducted blocking interferer level is defined that is the same as the blocker interferer level defined in 37.104 [7].

For UTRA single RAT, blocker interferer levels were derived using system level simulations for the medium range and local area BS classes [10]. Higher blocker interferer levels are defined that for the wide area scenario. The wanted signal level is kept at 6dB above the reference sensitivity.

For E-UTRA single RAT, the blocker interferer level for the local area BS class was derived from system level simulations [12], and the wanted signal set at 6 dB above reference sensitivity. For the medium range BS class, the wanted signal was set at 6 dB above reference sensitivity, whilst the blocker interferer level was adjusted such that the difference between the blocker interferer level and the wanted signal level is equalized between E-UTRA wide area and medium range (The adjustment is needed because the reference sensitivity level differs between the two classes) [11].

Comparing UTRA and E-UTRA single RAT blocking requirements, there are differences in the blocker interferer levels (UTRA being larger). For MSR, there is a need for a single set of in band blocking requirements. To achieve this, the UTRA blocker interferer levels, which are more stringent than the E-UTRA were used for the MSR blocking requirement. However the difference between the blocker interferer level and the wanted signal level needed for the wide area BS class to pass blocking is also maintained for the other BS classes. To achieve this, the offset of the wanted signal from the reference sensitivity is adjusted and different to 6 dB for E-UTRA. Also the UTRA wanted signal levels are adjusted to keep a common difference between the blocker interferer level and the wanted signal level.

### 6.4.2	In-band selectivity and blocking: Core requirement

The conducted blocking level simulations assumptions use 3 UE’s in the interfering network, the interferer level recorded for the statistical analysis is the total of all 3 interfering UE’s. When translating this to an OTA requirement where the direction of the interferer also needs to be specified this can cause an added complication.

Directional information is distorted within the existing coexistence simulation framework due to the fact that multipath propagation is approximated as lognormal shadow fading. Whereas in the real world, several multipaths arrive from different directions in to the antenna, in the simulation the signal arrives from the line of sight direction, but a random fading margin is added. Nonetheless, coarse observations on likely locations of blocking UEs can be considered. Thus, using the same simulation parameters as the conducted blocking simulation but recording the data on the power level and location of the UE’s which defined the 99.99% point it was however found that the total power is dominated by a single UE.

Figure 6.4.2-1.Sample of 99.99% Blocking interference level and individual UE power

Therefore only a single interference direction is required to represent the 99.99% blocking case.

The same 99.99% blocking UE’s were investigated to see if they came from a predictable direction.

Figure 6.4.2-2 99.99% Blocking interfering UE location

The 99.99% UE’s direction was found in both azimuth and elevation to be approximately within the element beam width. Whether this is true when multipath propagation is considered is not clear from this type of simulation.

The simulation does not take into account the wanted signal and so it is not possible to find the direction of the wanted signal by the same analysis. It was judged to be feasible however to make a worst case assumption that the wanted UE and the aggressor UE are in the same direction. The assumption may have caused some over dimensioning of the blocking requirement, since the probability of the wanted signal and blocker arriving in the same direction is not 100%, but the potential over dimensioning is seen as acceptable for E-UTRA/UTRA requirements.

In sub-clause 6.2.2.2.2 the OTA reference sensitivity has been defined, this is based on the OTA REFSENS RoAoA which represents the average element/sub-array radiation pattern beam width and is intended to be the OTA equivalent of the conducted reference sensitivity. As such it provides a suitable basis for the OTA wanted signal power level when considering the maximum interference level at the input of each receiver unit.

As the wanted signal and the interferer are in the same location for the purpose of the requirement, and the purpose of the OTA requirements is to offer the same protection and performance as the conducted requirements the difference between the wanted signal and the interfering signal should be maintained. The conducted blocking interferer level can hence be translated to an OTA level using the same method:

Where:

-	PConducted_blocker is the conducted blocking interferer level is rel13

-	PREFSENS, D0 , Doff-peak, and DRX_OTA+MARGIN are the same as defined in sub-clause 6.2.2.2.2.

Or more simply

So

As OTA reference sensitivity and the OTA blocker level are both translated to OTA levels using the same method they retain the same difference and hence the requirement on the receiver unit hardware is similar to the existing conducted specification.

D0 is based on the OTA REFSENS RoAoA which is based on the element/sub array pattern as such is the minimum possible antenna gain for the AAS BS. The blocker interfere level is hence the largest level possible. This ensures that the linearity of the receiver for high signal levels at each of the receiver unit inputs.

However as the conducted requirements are per receiver unit and the OTA level is for the composite receive function the OTA REFSENS value does not include all of the beam forming gain the composite antenna provides. As such the receiver minimum sensitivity as not accuracy represented by the OTA REFSENS value.

When considering the conducted blocking requirement the receiver noise contributes to the receiver performance under blocking conditions as such the interference contribution due to the blocking interferer is only 4.7 dB rather than 6 dB. When only the element gain is considered the lower antenna gain results in higher blocker and wanted signal levels and the noise floor does not contribute significantly to the noise floor so the interference contribution due to the blocking interferer may be the full 6 dB.

For example consider an element of 65° by 65°, in a 10x1 array (0.9λ spacing) with the lowest declared EIS of -116 dBm.

For E-UTRA wide area

For the OTA REFSENS as a wanted power:

For the minSENS case:

As in both cases the system noise floor for the wanted signal is the same and is based on the composite receiver gain (i.e. -117dBm in this example), for the minimum gain case the interference contribution due to the blocking interferer may be:

or a contribution of 4.7dB

For the OTA REFSENS case the interference contribution due to the blocking interferer may be

or a contribution of 5.9dB

Although the OTA REFSENS case has a higher interferer level it may also generate a higher contribution of interference, hence it is difficult to define an absolute worst case as the 2 conditions cover 2 different cases:

-	OTA REFSENS covers the maximum interference level at the RX unit input

-	minSENS covers the case where the effect of the system noise figure is considered.

It is important to bear in mind that the ratio of wanted signal level to blocking level is related to the probability of users and RX power levels occurring in adjacent networks. The probabilities of the signal levels do not change as a result of BS architecture, and thus changing the blocking to wanted signal level would lead to a change in the blocking probability provided by the network.

As both these cases are important the OTA blocking requirement is set at 2 levels, in both cases the delta between the wanted signal and the blocking interferer (Δwanted-blocker) is the same as the difference between the two in the conducted requirement (e.g. 52.5 dB for wide area E-UTRA). This delta value is then applied at 2 OTA wanted signal EIS levels

1.	EISREFSENS + 6 dB

2.	minSENS + 6 dB

### 6.4.3	In-band selectivity and blocking: Conformance requirement

The OTA blocking requirement is tested as follows:

-	In the reference direction of the minSENS OSDD using the minSENS based requirement level

-	In each of the 4 conformance directions at the extremities of the OTA REFSENS RoAoA using the REFSENS based requirement level.

## 6.5	Out-of-band blocking

### 6.5.1	Background

The existing conducted out of band blocking requirement is based on a wanted signal level at PREFSENS + 6 dB, and a CW interfere at -15 dBm. It is valid from 1 MHz to 12.75 GHz excluding the operating band plus a guard (usually 20 MHz either side).

The original derivation of the -15 dBm level was done for the first release of the UTRA specification. The level of -15 dBm is based on analysis done on interference from a number of systems which are close in frequency to the 3GPP bands, notable other 3GPP bands in particular the TDD band 2010 to 2025 MHz and 2400 MHz ISM bands. In both cases the level of -15 dBm was derived by considering the interferer power level, the MCL and assuming that the BS antenna has the same gain for wanted and interfere signals. In some cases the figures were calculated for the UE and then applied to the BS at the same level.

The simple calculations were as follows:

TDD band 2010 to 2025 MHz:	BS to UE case	43 dBm - 60 = -17 dBm

WLAN: max emitted power = 30 dBm, requirement approx. -15 dBm

Based on the available documented information, no consideration was taken for systems operating at vastly different frequencies from the operating band. For a conducted requirement for a single transceiver it is reasonable to apply the same requirement level across the whole out of band spectrum as it likely the requirement is toughest close to the operational band.

### 6.5.2	Out-of-band blocking OTA core requirement

#### 6.5.2.1	Interferer level

There are a number of considerations which make translating the conducted requirement to an OTA requirement challenging:

-	The gain characteristics of an antenna is not known in the out-of-band frequency region.

-	The free space path-loss at high frequencies (i.e. up to 12.75 GHz) means that the radiated power levels required to provide -15 dBm at conducted point are unfeasibly high.

Clearly the interferers are present irrespective of the victim antenna gain, the original levels assumed that the victim antenna gain was the same as the in band gain, but no analysis was done at very high (or low) frequencies.  This was acceptable considering the conducted requirement in previous specifications and the fact it is difficult to estimate out of band antenna gain.

However, with the out-of-band blocking OTA requirement it is not necessary to estimate out-of-band antenna gain so instead another approach can be considered.

For the purposes of the OTA requirement it is useful to consider that the interferer power level is specified at a fixed distance from the AAS BS. This distance does not have to be representative of either the actual minimum distance of the interferer source or be at the same power level (of the interferer). The distance and the level can be adjusted to ensure the requirement presents a worst case requirement for the worst case interferer levels.

For frequencies outside the operating band for which the antenna is designed for the exact antenna gain is unknown, however the victim antenna aperture area is constant over the whole frequency range 30 MHz to 12.75 GHz.

In general, the power gain of an antenna is defined as:

, where Aeff is the effective antenna area in m2 and  is the wavelength in m.

Free space path loss for a given distance is calculated by

, where d is the distance in meters.

The resultant of the antenna gain and the path loss is given by:

means that electromagnetic radiation does not depend on frequency. In practice the antenna aperture Aeff is not constant over frequency, as antennas in general antennas are frequency tuned devices. But here it is reasonable to assume a worst-case scenario where the antenna aperture is constant. Hence it can be noted that the wavelength dependency of the path loss and the antenna gain cancel each out so that the worst-case for a fixed interferer power at a fixed distance results in a fixed equivalent conducted level.

Therefore the interferer power level (EIRP) can be determined at an arbitrary distance of 30m.

The assumed distance however it does not indicate that the conformance testing is done at 30m. The distance and power level may be adjusted considering the variation in FSPL and characteristics of the test facility.

Analysis shows that the worst-case interferers are other 3GPP BS and WLAN systems

For BS to BS the OTA interference power level is given by the co-existence scenario at 288m as:

EIRP = Ptx + Gtx = 43 dBm + 13 dBi - 6dB = 50 dBm, assuming 6 dB due to down-tilt.

The WLAN interference can be calculated based on BS to UE minimum distance and the WLAN Tx power, WLAN is specified in EIRP rather than total power, the value is 30 dB EIRP hence the interference power level can be calculated as 30 dBm (EIRP) at a distance of 42 m.

The historic analysis of the worst-case blocking does not provide the conducted interferer level currently in the specification.

i.e.

Prx = Pout + Gant_agressor – LDT – FSPL + Gant_victim - LDT = 43dBm + 13dBi  - 3dB - 87.65dB + 13dBi – 3dB = -24.65dBm

It could be argued that the existing conducted requirement is to high however, the conducted interferer blocking level of -15dB is more reasonable when it is considered that the antenna gain assumptions are low. Assuming an antenna again of 17dBi  is a more reasonable antenna again assumption.

i.e.

Prx = Pout + Gant_agressor – LDT – FSPL + Gant_victim - LDT = 43dBm + 17dBi  - 3dB - 87.65dB + 17dBi – 3dB = -16.65dBm

Using these assumptions as a basis for the OTA requirement gives:

EIRP(30m) = Prx – Gant + FSPL(30m) = -15dBm – 17dBi + 68dB(frequency=2 GHz, FSPL 30m) = 36dBm

The interferer EIRP level is hence specified as EIRP = 36dBm at 30m.

Polarization between the interferer and the AAS BS need to be taken into account.  As a pragmatic approach, at frequencies at several gigahertz away from the wanted signal polarization matching may be difficult to achieve.  However, polarization alignment where consideration of the physical AAS BS orientation and the interferer antenna polarization aligned need to be handled.  One approach is the requirement and test should use polarization matching in band and keep the same alignment even when performing the OOB measurement, even though the antenna may become detuned at the out of band frequency.

To take into account both polarizations of the AAS BS need to be excited equally, as the interferer is CW only a single polarization can be radiated hence the polarization of the interferer signal is split between the cross polarized victim antennas and therefore the input power needs to be doubled.

The interferer signal level at the aggressor can be converted to a RMS field-strength level at the victim base station as:

in V/m, where EIRP is in W and r is the distance in m.

The aggressor interferer level described as EIRP=36 dBm at a distance of 30m corresponds to a RMS field-strength level of 0.36 V/m. Using field-strength the interferer level the RF core requirement can be described at the base station, without assumptions related to the aggressor.

#### 6.5.2.2	Wanted signal level and direction

As with the in-band blocking requirements whist during operational conditions it is unlikely that the wanted signal and the interferer are in the same direction for the purposes of defining the RF requirement they can be considered to be in the same direction.

The impact of the blocking interfere on the system sensitivity is largest when the wanted signal is at its lowest level. The wanted signal is therefore defined at minSENS + 6 dB.

For practical purposes the OTA wanted signal level is only specified within the declared RoAoA, the direction of the RoAoA coincides with the direction of the antenna maximum gain, this can be assumed to apply for out of band performance as well as in band performance. The out of band blocking requirements are hence only specified over the receiver RoAoA associated with minSENS.

The wanted signal levels apply to all supported polarizations, under the assumption of polarization matching.

#### 6.5.2.3	Out of band region

The out of band requirement is applicable in the frequency range described in clause 6.5.2.4, with the exception of the frequency range covered by the operating band plus/minus an additional ΔfOBUE MHz on either side. Based on the discussion on the boundary between in-band and out of band region for NR FR1 operation, the ΔfOBUE values for the out of band blocking were agreed for AAS BS as presented in table 6.5.2.3-1. Those ΔfOBUE values are applicable to conducted and radiated requirements for AAS BS in single RAT E-UTRA operation and to AAS BS in MSR operation using E-UTRA, and are applicable for all E-UTRA operating bands defined in TS 36.104 [7].

Table 6.5.2.3-1: ΔfOBUE values the out of band blocking

|  |  | E-UTRA AAS BS (Single-RAT, MSR) |
| --- | --- | --- |
| ΔfOBUE | Operating band <100 MHz | 20 MHz |
|  | 100 MHz ≦ Operating band ≦ 200 MHz | 60 MHz |

#### 6.5.2.4	Frequency range

The frequency range for the OTA out of band blocking requirement is 30MHz to 12.75GHz (excluding the operating band plus/minus ΔfOBUE as described in clause 6.5.2.3). The lower limit of the frequency range is adjusted from 1MHz for the conducted requirement to 30MHz in order to be consistent with the out of band spurious emissions requirement, and considering that measurement at frequencies below 30MHz becomes unfeasible due to the large required chamber size.

### 6.5.3	Out-of-band blocking OTA conformance requirement

Out of band blocking is a long test and hence it is optimum to minimize the number of conformance test directions. Even though the antenna gain is not necessarily maximum at the reference direction above the 2nd harmonic of the designed antenna element center frequency, the practical measurement becomes excessively complex if wanted and interferer signals would be applied from different directions at different frequencies. Therefore it is considered sufficient to show conformance at the reference direction only.

It has been recognized that above 2nd harmonic frequency the antenna element response may be stronger in some other direction than in the reference direction. This means that there is a risk that testing in reference direction does not necessarily test the worst case, at least for some antenna types. Increasing the interferer level was considered to compensate for this effect. However, defining the breakpoint frequency and change in interferer level in a way which would treat different antenna implementations equally was not deemed feasible. Therefore, it was decided to apply a constant interferer level according to the core specification throughout the whole tested frequency range.

## 6.6	Receiver spurious emissions

### 6.6.1	Background information on the conducted requirement

Referring to the TS 37.105 [3], the RX spurious emissions power is defined as power of emissions generated or amplified in a receiver unit that appear at the TAB connector. The RX spurious emissions test purpose is to verify the ability of the AAS BS to limit the interference caused by receiver spurious emissions to other systems. Number of constraints are applicable for the RX spurious requirement depending on the TAB connectors capabilities:

1.	The requirements apply to all AAS BS with separate RX and TX TAB connectors. In this case for FDD AAS BS the test is performed when both TX and RX are ON, with the TX TAB connector terminated,

2.	For a TAB connector supporting both RX and TX in TDD, the requirements apply during the transmitter OFF period,

3.	For a TAB connector supporting both RX and TX in FDD, the receiver spurious requirements are superseded by the TX spurious requirements.

Based on the above limitations, it can be observed that the conducted RX spurious emissions requirement applicability is limited to the following cases:

1.	TDD case: applicable for RX TAB connectors and TX/RX TAB connectors, during TX OFF period,

2.	FDD case: applicable for RX TAB connectors, during TX ON period, when TX TAB connectors are terminated.

According to TS 37.105, the RX spurious emission limits for AAS BS apply in the frequency range from 30 MHz to 12.75 GHz, with consideration of the RAT-specific frequency exclusions for the OBUE/SEM, as well as the spurious range upper frequency limit extensions for selected operating bands, as defined in sub-clause 5.6.6.1 for the TX spurious emissions requirement.

In terms of emission levels, the basic limits and its scaling was also applied to the RX spurious emissions, similar to the TX spurious emissions.

### 6.6.2	Receiver spurious emissions OTA: Core requirement

The metric used to capture the receiver spurious emissions OTA is TRP, defined in the same way as for the transmitter spurious emissions OTA, in sub-clause 5.6.6.2.

The receiver spurious emission requirement is subject to the following limitations:

1.	For OTA AAS BS operating in FDD: OTA RX spurious emissions requirement do not apply to FDD duplex as being superseded by the OTA TX spurious emissions requirement. This is due to the fact that TX and RX spurious emissions cannot be distinguished in OTA domain.

- NOTE:	The OTA receiver spurious emission requirement applicability for the AAS BS with the RX-only capabilities is FFS.

2.	For OTA AAS BS operating in TDD: the OTA receiver spurious emissions requirement applies during the transmitter OFF period only.

The core requirement for the RX spurious emissions OTA is applied over the same spurious range as defined for the conducted RX spurious requirement in sub-clause 6.6.1.

Derivation of the emission levels for the OTA RX spurious emissions requirement is aligned with the methodology used for the derivation of TX spurious emission levels with the emissions limit, as captured in sub-clause 5.6.6.2.

### 6.6.3	Receiver spurious emissions OTA: Conformance requirement

In general, derivation of the OTA conformance requirement for the RX spurious emissions, uses the same principles as for the TX spurious emissions requirement OTA, as in sub-clause 5.6.6.3.

Definition of the emissions measurements for the conformance requirement is be based on the core requirement, subject to the TRP MU, where the MU definition should capture the whole spurious frequency range.

NOTE:	The OTA RX spurious emission requirement for the AAS BS with the RX-only capabilities is FFS.

For OTA AAS BS operating in TDD: applicability the OTA RX spurious emissions conformance requirement when all transmitters are OFF, is FFS.

{editors note: to be completed as part of conformance work}

## 6.7	Receiver intermodulation

### 6.7.1	Background information on the conducted requirement

The receiver intermodulation requirement relates to the ability of the receiver to avoid mixing (i.e. intermodulation) between 2 strong interferers leading to a significant noise floor rise in the receiver.

A scenario in which IM would arise would be one in which two UEs/transmitters from two interfering systems provide strong enough interference to the base station simultaneously. The interferer levels have been set to be the same in the EUTRA requirement for simplicity reasons; it is not necessarily the case that the interferer levels would be the same in real conditions. Neither is equal interferer levels necessarily the worst case.

### 6.7.2	Receiver intermodulation: Core requirement

Differing antenna architectures could have an impact to RX IM scenarios. However, simulation based investigation would be a highly complex and multidimensional problem for which there was insufficient time within the WI. Fortunately, several factors can be considered which to some extent mitigate the need for such an investigation:

-	It may be assumed that the two interfering systems are statistically independent

-	If it is also assumed that the two interfering systems operate the same RAT in the same deployment scenario (but on different frequencies), then their interference characteristics would be identical

-	For the blocking requirement, an assumption has been made that the conducted blocking level does not depend on the antenna element or module pattern. If the same assumption should be true for each of the interfering systems for RX intermodulation

-	The RX IM levels should be lower than the blocking level, since the probability of two interferers both having a high level simultaneously is lower than the probability of only one interferer having a high level

-	The justification in terms of simplification in assuming equal levels for the conducted requirements can be re-applied for OTA requirements

Taking these factors into account, the OTA RX IM requirement is defined based on the conducted RX IM requirement, with the conducted signal levels translated to OTA levels. Considering in particular the reasoning that the independent conducted levels should not depend on antenna element or module pattern, then a similar approach is adopted for receiver intermodulation as for receiver blocking; i.e. apply the requirement assuming all signals come from the same direction.

Since RX intermodulation relates to receiver non-linearity’s, it is important that the correct absolute conducted level is achieved by the OTA requirement for the receiver intermodulation. This is achieved by using the gain estimated from the OTA REFSENS RoAoA.

For the receiver blocking test, the requirement is also applied at lower absolute signal levels relating to the OTA minimum sensitivity. For RX IM, the signal levels are reduced then the IM effects are likely to reduce due to the LNA operating well within its linearity region. However in order to be consistent with the in band blocking requirement, RX IM is also at both OTA REFSENS and minSENS levels.

### 6.7.3	Receiver intermodulation: Conformance requirement

Since RX sensitivity and blocking already test at all conformance directions, it is sufficient to test RX IM only in a single direction.

## 6.8	In-channel selectivity

### 6.8.1	Background information on the conducted requirement

The in-channel selectivity requirement relates to the ability of the receiver to receive a low power signal whilst receiving a high-power signal in channel.

The selectivity is around 20 dB in 36.104 [8]; this happens to be roughly equal to the SINR range of the demodulation requirements.

### 6.8.2	In-channel selectivity: Core requirement

For setting the requirement, the same ICS as for the conducted requirements is assumed.

In regard to how to set the absolute levels it is worthwhile to consider that aspects of the receiver design that impact the selectivity include linearity, ADC range, phase noise, frequency error and others. The applicable aspects of receiver design divides into those which become of significance at high absolute power level and those that do not (e.g. phase noise). Other requirements, such as receiver blocking and dynamic range already capture the behavior of the receiver at high power level. It is important that the selectivity is achieved after combining. Thus, the most important receiver level to consider for this requirement is the level based on OTA minimum sensitivity.

Thus the receiver ICS requirement is specified at minimum sensitivity level:

-	Wanted signal level = Conducted wanted signal level + ΔminSENS Interferer level = Conducted interferer level + ΔminSENS

### 6.8.3	In-channel selectivity: Conformance requirement

The in channel selectivity requirement is tested in a single direction.

## 6.9	ACS and NB blocking

### 6.9.1	Background information on the conducted requirement

Adjacent channel selectivity (ACS) is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

ACS is derived by considering the results of coexistence system simulations that examine the impact of an aggressor system on the throughput attainable in a victim system. The ACIR, ACLR and ACS levels are set such that the average and 5th percentile throughput do not suffer a significant degradation in a victim network (usually no more than 5%), when adjacent channel interference from an aggressor network is modeled. The simulations are Monte Carlo simulations in which in effect all possible combinations of receiver SINR and interfering signal are modeled to find an ACIR requirement for the up-link and then the ACIR is shared between the BS ACS and the UR ACLR. Both these figures are relative.

It is worthwhile to note that the blocking requirement considers a particular situation in which the interfering signal is received with a very high level, whilst the wanted signal is low. However, the ACS may lead to degradation in many other situations, quite often in situations in which the wanted signal has high SINR and the interfering signal is received at a significantly lower power level than the blocking level. If the selectivity against the interfering signal is not sufficient, then high SINR wanted signals may suffer significant degradation.

The relative ACS for the BS requirement is 45 dBc.

The ACS requirement is in the form of an absolute interferer level and an absolute wanted signal level. The wanted signal level and the ACS relative requirement is used to find the ACS absolute value as follows:

For UTRA wide area BS:

And For E-UTRA wide area BS, 10MHz channel BW:

Note: The actual E-UTRA requirement is -52 dBm the same as UTRA.

The wanted power is an offset from REFSENS, generally it is REFSENS +6 dB however for E-UTRA as the REFSENS is different for different channel band widths the wanted power the wanted signal is fixed based on the 5-20 MHz channel band width requirement.

### 6.9.2	ACS and NB blocking: Core requirement

The ACS requirement is intended to specify the selectivity of the receiver, not the ability of the receiver to process a large signal level.

The conducted ACS has been derived based on co-existence simulations that take into account antenna selectivity considering a passive column. Thus, assuming that the AAS has the same type of antenna as the simulated scenario, then the task of the requirement definition and test is to re-create the conducted requirement. This implies that the test can be carried out with the wanted and interfering signals aligned in direction; there is no need to consider different directions for the signals, since selectivity has already been taken into account in the simulations used to derive the requirement.

For a beamforming system, the beamforming pattern is likely to provide additional spatial selectivity. During the eAAS WI, the impact of this additional spatial selectivity on ACS was not investigated. It is possible that for beamforming systems, the required ACS may be lower than 45 dBc; not higher.

The same 45 dBc is re-used for the OTA requirement.

It is agreed that the blocking requirements are tested using two OTA levels; one based on the OTA REFSENS RoAoA declaration and the second based on the OTA minimum sensitivity which is derived from the OSDD declarations. The purpose of the second level is to avoid that digital combining leads to the blocking being tested with the wanted signal at a large offset against the noise floor in the receiver.

For the ACS requirement, if due to combining the wanted signal would be significantly above the noise floor, then the selectivity would be relaxed somewhat. Thus, testing ACS using a 6 dB degradation against the minimum sensitivity level is a better requirement. On the other hand, testing of ACS based on the OTA REFSENS is not needed, since the blocking requirements tests the ability of the receiver to correctly manage absolute signal levels.

As the conducted absolute ACS level is an offset from REFSENS, the OTA ACS absolute value can be easily found as an offset from OTA minimum sensitivity value.

The requirement is valid over the OTA sensitivity OSDD.

The NB blocking requirement is defined using both the OTA REFSENS RoAoA and the OTA minimum sensitivity RoAoA in the same manner as the in band blocking requirement.

### 6.9.3	ACS and NB blocking: Conformance requirement

Conformance of the ACS requirements can be shown in the reference direction.

Conformance of the NB blocking requirement is shown in the same directions as OTA REFSENS.

# 7	Performance requirements

## 7.1	General

The objective of the performance part of the eAAS WID [2] for the AAS BS demodulation requirements is to clarify, which of the conducted BS demodulation requirements could be applied as radiated requirements for the OTA AAS BS and what methods could be used to perform conformance testing for such requirements. For OTA AAS, no new demodulation requirements are expected to be added beyond the set of AAS BS conducted demodulation requirements referred from single RAT UTRA FDD and E-UTRA specifications in TS 25.104 [19] and TS 36.104 [8] respectively.

NOTE: UTRA TDD BS demodulation requirements are out of scope of the work for OTA AAS BS.

## 7.2	Core requirements for OTA tests

In Rel-15 TS 37.105 [3], the AAS BS demodulation requirements for both hybrid AAS BS and OTA AAS BS have been specified, reusing the single RAT specifications:

1.	For single RAT UTRA FDD, demodulation requirements specified in TS 25.104 [19] for DCH and RACH apply.

2.	For single RAT E-UTRA, demodulation requirements specified in TS 36.104 [8] for PUSCH, PUCCH and PRACH apply. , except the PUSCH requirements for FS3, and except NPUSCH and NPRACH requirements.

Core requirements listed in the following clauses are subject for the discussion on which of the demodulation requirements could be applied as radiated requirements for the OTA AAS BS.

## 7.3	UTRA FDD requirements

In TS 25.104 [19], the following conducted BS demodulation requirements were defined:

Table 7.3-1: Overview of the conducted UTRA FDD demodulation requirements

|  | Tx (UE emulator) | Rx (AAS BS) | Propagation conditions |
| --- | --- | --- | --- |
| DCH |  |  |  |
| Demodulation in static propagation conditions | 1Tx | 1Rx, 2Rx | AWGN |
| Demodulation of DCH in multipath fading conditions | 1Tx | 1Rx, 2Rx | multipath fading (multiple cases defined) |
| Demodulation of DCH in moving propagation conditions | 1Tx | 1Rx, 2Rx | non-fading channel models with two equal power taps with pre-defined time difference between them |
| Demodulation of DCH in birth/death propagation conditions | 1Tx | 1Rx, 2Rx | non-fading propagation channel with two taps; which alternate between 'birth' and 'death'. The positions the paths appear are randomly selected with an equal probability rate. For BS with receiver diversity, the same path positions are applied to both Rx antenna connectors, and the path switching times are synchronized on the two receiver antenna connectors, but the AWGN signals applied to the two receiver antenna connectors are uncorrelated. |
| Demodulation of DCH in high speed train conditions | 1Tx | 1Rx, 2Rx | HST conditions: #1: open space: non-fading propagation channels #2: tunnel: fading propagation channel with one tap #3: tunnel: non-fading propagation channels |
| RACH |  |  |  |
| Performance requirement for RACH preamble detection | 1Tx | 1Rx, 2Rx | AWGN, multipath fading, HST conditions |
| Demodulation of RACH message | 1Tx | 1Rx, 2Rx | AWGN, multipath fading, HST conditions |
| HS-DPCCH |  |  |  |
| ACK false alarm | 1Tx | 2Rx | AWGN, multipath fading (multiple cases defined) |
| ACK mis-detection | 1Tx | 2Rx | AWGN, multipath fading (multiple cases defined) |
| ACK/NACK detection for 4C-HSDPA HS-DPCCH (ACK false alarm, ACK mis-detection) | 1Tx | 2Rx | AWGN, multipath fading |
| ACK/NACK detection for 8C-HSDPA HS-DPCCH | 1Tx | 2Rx | AWGN, multipath fading |
| E-DPDCH |  |  |  |
| E-DPDCH in multipath fading condition | 1Tx | 1Rx, 2Rx | multipath fading |
| E-DPDCH and S-E-DPDCH in multipath fading condition for UL MIMO | 1Tx | 2Rx | multipath fading |
| Signalling detection for E-DPCCH in multipath fading condition | 1Tx | 1Rx, 2Rx | multipath fading |

## 7.4	E-UTRA requirements

In TS 36.104 [8], the following conducted BS demodulation requirements were defined as listed in table 7.4-1.

NOTE: PUSCH requirements for FS3, NPUSCH and NPRACH requirements are not considered for OTA AAS BS.

Table 7.4-1: Overview of the conducted E-UTRA BS demodulation requirements

|  | Tx (UE emulator) | Rx (OTA AAS BS) | Propagation conditions | Notes |
| --- | --- | --- | --- | --- |
| PUSCH |  |  |  |  |
| PUSCH in multipath fading propagation conditions transmission | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA, ETU |  |
|  | 2Tx | 2Rx, 4Rx, 8Rx | EPA |  |
| UL timing adjustment | 1Tx | 2Rx | Moving propagation conditions: AWGN, ETU | 1 stationary UE, 1 moving UE |
| HARQ-ACK multiplexed on PUSCH | 1Tx | 2Rx | EVA, ETU |  |
| High Speed Train conditions | 1Tx | 1Rx, 2Rx | HST scenario#1: two non-fading propagation channels |  |
| PUSCH with TTI bundling and enhanced HARQ pattern | 1Tx | 2Rx, 4Rx, 8Rx | EVA, ETU |  |
| Enhanced performance requirement type A in multipath fading propagation conditions, synchronous interference | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA, ETU | 1 wanted UE, 1 interferer (2 Rx case) or 2 interfering UEs (4Rx or 8Rx case) |
| Enhanced performance requirement type A in multipath fading propagation conditions, asynchronous interference | 1Tx | 2Rx, 4Rx, 8Rx | EPA, ETU | 1 wanted UE, 2 interfering UEs |
| Enhanced performance requirement type B in multipath fading propagation conditions | 1Tx | 2Rx, 4Rx | EPA, EVA, ETU | 2 wanted UEs (2Rx case) or 4 wanted UEs (4Rx case); 1 interfering UE |
| Requirements for PUSCH supporting coverage enhancement | 1Tx | 2Rx | EPA, ETU |  |
| Requirements for PUSCH supporting Cat-M1 UEs | 1Tx | 2Rx | EPA, ETU |  |
| PUCCH |  |  |  |  |
| ACK missed detection for single user PUCCH format 1a transmission | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA, ETU |  |
|  | 2Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| CQI performance requirements for PUCCH format 2 transmission | 1Tx | 2Rx | EVA, ETU |  |
|  | 2Tx | 2Rx | EVA |  |
| ACK missed detection for multi user PUCCH format 1a | 1Tx | 2Rx | ETU | 1 wanted UE, 3 interfering UEs |
| ACK missed detection for PUCCH format 1b with Channel Selection | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| ACK missed detection for PUCCH format 3 | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| NACK to ACK detection for PUCCH format 3 | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| CQI performance requirements for PUCCH format 2 with DTX detection | 1Tx | 2Rx | EVA, ETU |  |
|  | 2Tx | 2Rx | EVA |  |
| PUCCH performance requirements for supporting Cat-M1 UEs | 1Tx | 2Rx | EPA, EVA |  |
| ACK missed detection requirements for PUCCH format 4 | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| ACK missed detection requirements for PUCCH format 5 | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| PRACH |  |  |  |  |
| PRACH missed detection, normal mode | 1Tx | 2Rx, 4Rx, 8Rx | AWGN, ETU |  |
| PRACH missed detection, high speed mode restricted set type A | 1Tx | 2Rx, 4Rx, 8Rx | AWGN, ETU |  |
| PRACH missed detection, Cat-M1 mode | 1Tx | 2Rx | AWGN, EPA |  |
| PRACH missed detection, high speed mode restricted set type B | 1Tx | 2Rx, 4Rx, 8Rx | AWGN, ETU |  |

For most of the requirements, multiple test cases were defined considering various number or Rx antennas at the BS.

## 7.5	Propagation conditions

The following propagation conditions were used in the BS demodulation requirements in TS 25.141 [21] and TS 36.141 [22]:

-	Static propagation conditions: static propagation consitions rely on AWGN channel with no fading or multi-paths. In case of the OTA test setup no channel emulators are required, and the test setup relies on test signal source and AWGN noise generator

-	Multipath fading propagation conditions: number of multipath channels were defined. In order to translate the fading profiles into OTA test setup, consideration of channel emulators is required on the transmitter side. Use of multiple instances of channel emulators over the same radio channel to model the conducted test setup is not precluded.

-	High Speed Train: three different scenarios were considered for the HST scenarios, using fading and non-fading profiles.

-	Moving propagation conditions: two UEs are modeled in this scenario, one being static, the other moving.

## 7.6	Conformance requirements

The OTA related parts of the conformance testing test procedures for the demodulation requirements are based upon the RX dynamic range requirement OTA procedure, as described in subclause 10.3.4. The setup and operation of the signal generator and measurement instruments are the same as for the corresponding conducted requirements.

Measurement Uncertainties for the OTA requirements are the same as for the conducted requirements, as described in subclause 10.11.

It is noted, that the possibility of OTA testing is considered also for those BS demodulation requirements, which were declared as Optional in TS 25.141 [21] and TS 36.141 [22].

For OTA test setup for the BS demodulation requirements and for description of the BS demodulation requirements feasible OTA, refer to TR 37.941 [36].

## 7.7	OTA test setup

NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

Figure 7.7-1: Void

Figure 7.7-2: Void

Figure 7.7-3: Void

Figure 7.7-4: Void

## 7.8	BS demodulation requirements feasible OTA

## NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

Table 7.8-1: Void

Table 7.8-2: Void

# 8	EMC requirements

## 8.1	General

The Rel-13 AAS EMC specification [13] was drafted with a simple principle of applying the existing EMC requirements to the AAS BS architecture hence it was possible to refer to the existing UTRA , E-UTRA and MSR EMC specifications [16][15][14]. It was possible to use this approach since release 13 of AAS BS specification [3] only allows for AAS BS architectures with a conducted interface, which means that the antenna array can be disconnected and TAB connectors can be terminated. Hence the system can be treated in the same way as in the existing specifications and therefore the EMC requirements remained applicable in exactly the same way.

The OTA AAS BS architecture does not have a conducted interface and so the antennas cannot be disconnected. This means that the conducted methods where the EMC antenna ports are terminated are no longer be possible and hence new methods for EMC testing are needed.

Figure 8.1-1 shows the prime difference in architecture from an EMC view point when comparing the ports on a conventional non-AAS BS to the ports on an AAS BS.

Figure 8.1-1: Comparison between BS architectures for EMC testing

Since the antenna array for the OTA AAS BS is integrated within the EUT, the transmissions from the EUT within the test chamber have antenna gain included in them.

Figure 8.1-2 illustrates the status of EMC antenna ports during EMC testing where it is noted that the ports are connected to a terminating load and the hybrid AAS BS is then set to transmit at full power.

Figure 8.1-2: EMC Antenna port status during EMC testing

NOTE:	In Rel-15 version of this TR, multiple clauses related to the OTA measurements of the EMC requirements were shifted to the OTA BS testing TR 37.941 [36].

## 8.2	Regulatory EMC requirements

The following table provides a summary of the existing EMC regulatory requirements in EU and US market. These requirements should serve as a reference when specifying the EMC requirements for the AAS BS.

Table 8.2-1: Overview of the regulatory EMC requirements for EU and US markets

|  | EU market | US market |
| --- | --- | --- |
| RF Radiated Spurious Emission (RSE) requirements | -36dBm/100kHz ERP below 1 GHz and -30dBm/1MHz ERP above 1 GHz Requirements specified in ETSI EN 301 908-1, and are tested with the antenna port connected in a terminating load. No TX exclusion bands defined for BS equipment today. | General requirement of -13 dBm/MHz ERP is applicable Requirements specified in FCC Part 2. FCC does not define any TX exclusion bands for BS equipment today. |
| EMC Radiated Emission (RE) requirements | -50dBm/MHz ERP general requirement. Not applicable for BS equipment Defined in ETSI EN 55032. | -50dBm/MHz above 1 GHz mandatory requirement. Specified in FCC Part 15, §15.35. |
| Radiated Immunity (RF electromagnetic field) requirements | Two type of requirements defined: 3 V/m, 80 MHz – 690 MHz, and 10 V/m 690 MHz – 6 GHz. Defined in ETSI EN 301 489-1 and ENSI EN 301 489-50 RX Exclusion bands defined 20 MHz above and below the operating band | No requirements today |

Note:	From the above regulatory requirements, the radiated spurious emission requirements (-36 dBm/100kHz and -30 dBm/1MHz) are reflected in the existing EMC specifications for AAS BS [13] and non-AAS BS [14]. Similarly, the 3 V/m immunity requirement is captured in the existing EMC specifications.

## 8.3	Field strength in EMC chamber

## NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

### 8.3.1	Void

### 8.3.2	Void

## 8.4	Protection of measurement equipment

## NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

## 8.5	EMC radiated emission requirement

## NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

Figure 8.5-1: Void

## 8.6	Radiated Immunity

## NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

### 8.6.1	Void

### 8.6.2	Void

### 8.6.3	Void

# 9	Manufacturer declarations

## 9.1	General

The Rel-15 AAS BS manufacturer’s declarations are defined in TS 37.145-1 [23] for the conducted testing of the hybrid AAS BS over its conducted RF interface, and in TS 37.145-2 [24] for the radiated testing of the hybrid AAS BS or OTA AAS BS over their radiated RF interfaces.

The following groups of the manufacturer’s declarations were considered for the hybrid AAS BS conformance requirements:

1.	For conducted requirements:

the already existingRel-13/14 manufacturers declarations as defined in the conducted conformance testing specification TS 37.145-1 [23] are reused for Rel-15 hybrid AAS BS. Those declaration were subject to multiple corrections: some declarations were found redundant (e.g. CA related declarations), additional missing declarations were introduced (e.g. BS class), multiple corrections were implemented for consistence.

2.	For the OTA requirements:

the already existing Rel-13/14 manufacturers declarations as defined for the radiated EIS and EIRP requirements in the radiated conformance testing specification TS 37.145-2 [24] table 4.10-1, were reused for Rel-15 hybrid AAS BS.

The following groups of the manufacturer’s declarations were considered for the OTA AAS BS conformance requirements:

1.	The Rel-13/14 set of OTA manufacturers declarations as defined for EIRP and EIS requirements for the hybrid AAS BS (2a above) in the radiated conformance testing specification TS 37.145-2 [24], table 4.10-1.

2.	Set of radiated testing declarations for OTA AAS BS, which were mirrored from the conducted specifications in Rel-15, and captured in TS 37.145-2 [24], table 4.10-2. Conducted declarations mirroring was performed in order to obtain independent declaration sets for hybrid AAS BS and the OTA AAS BS.

3.	Newly defined Rel-15 manufacturers declarations, for OTA REFSENS test requirement, captured in TS 37.145-2 [24], table 4.10-2.

## 9.2	Void

# 10	Conformance testing aspects

# NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

## 10.1	Void

## 10.2	Void

10log1-ΓSGH2

LSGHcal

10log1-ΓSGH2

LSGHcal

PIsotropic Radiator=Preference antenna+Greference antenna

Lpath loss=Preference antenna+ Greference antenna- Ptest equipment

Ltest equipment-receiver

Preceiver Lpath loss

## 10.3	Void

ΓSGH

LC↔SGH

10log1-ΓSGH2

LSGHcal

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

| MUwanted_conducted |  |  |  |  |
| --- | --- | --- | --- | --- |
| MUinterfer_conducted |  |  |  |  |
| MUmatching_conducted |  |  |  |  |
| MUwanted |  |  |  | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
| MUinterfer |  |  |  | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
| MUEIS_OTA |  |  |  |  |
| MUTestEquipment_OTA |  |  |  |  |
| MUmatching_OTA |  |  |  |  |
| MUchamber_OTA |  |  |  | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
| MUACS_OTA |  |  |  | MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

MUmatching_conducted= MUmatching_OTA

|  | MUwanted_conducted |  |  |  |
| --- | --- | --- | --- | --- |
|  | MUinterfer_conducted |  |  |  |
|  | MUmatching_conducted |  |  |  |
|  | MUwanted |  |  | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUinterfer |  |  | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA |  |  |  |
|  | MUTestEquipiment_OTA |  |  |  |
|  | MUmatching_OTA |  |  |  |
|  | MUchamber_OTA |  |  | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2 |
|  |  |  |  | MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |
|  | MUACS_OTA |  |  |  |

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

| MUwanted_conducted |  |  |  |
| --- | --- | --- | --- |
| MUCWinterfer_conducted |  |  |  |
| MUMODinterfer_conducted |  |  |  |
| MUmatching_conducted |  |  |  |
| MUwanted |  |  | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
| MUCWinterfer |  |  | MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2 |
| MUMODinterfer |  |  | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
| MUEIS_OTA |  |  |  |
| MUTestEquipment_OTA |  |  |  |
| MUmatching_OTA |  |  |  |
| MUchamber_OTA |  |  | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
| MURXIM_OTA |  |  | MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect |

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUMODinterfer=MUMODinterfer_conducted2-MUmatching_conducted2

MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

MUmatching_conducted= MUmatching_OTA

|  | MUwanted_conducted |  |  |  |
| --- | --- | --- | --- | --- |
|  | MUCWinterfer_conducted |  |  |  |
|  | MUMODinterfer_conducted |  |  |  |
|  | MUmatching_conducted |  |  |  |
|  | MUwanted |  |  | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUCWinterfer |  |  | MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2 |
|  | MUMODinterfer |  |  | MUMODinterfer=MUMODinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA |  |  |  |
|  | MUTestEquipiment_OTA |  |  |  |
|  | MUmatching_OTA |  |  |  |
|  | MUchamber_OTA |  |  | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2 |
|  | MURXIM_OTA |  |  | MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect |

## 10.4	Void

MUper-point2+SE2

f≤3GHz

[MUTRP=0.592+0.752

3GHz≤f≤4.2GHz

[MUTRP=0.702+0.752

MUTRP=MUper-point2+SE2

f≤3GHz

MUTRP=1.152+0.752

3GHz≤f≤4.2GHz

MUTRP=1.372+0.752

MUTRP=MUper-point2+SE2

f≤3GHz

[MUTRP=1.152+0.752

3GHz≤f≤4.2GHz

[MUTRP=1.372+0.752

## 10.5	Void

klim≤0.37(1-7.22/N0.64)

klim klimr(k)≤klimrk≤rlim

f

Pn=S212

Pn  = Ur250 PtMtLs

Mt= 1-S112

Pn

Pn

1/Nref

Pr=S21r2

UambPamb=Uamb250

TRPamb=ηM1rPrPamb

PEUT=UEUT250

TRPEUTTRPamb

## 10.6	Void

## 10.7	Void

AAS BS

Test system Calibrated point

θ

ϕ

0

AAS declared coordinate reference point and orientation

Test system enclosure

Test antenna (out of band)

Signal Generator for the interfering signal

Signal Generator for the wanted signal

Test antenna (in band)

Test antenna polarisation can be adjusted

## 10.8	Void

U(θ,ϕ)

Uθ,ϕ=EIRP(θ,ϕ)4π

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ

Etr,θ,ϕ,TRPReference

PDr,θ,ϕ=Etr,θ,ϕ2ηη=μ0ε0≈377 Ω

(θ,ϕ)

θϕθNϕMθϕ

Δθ= πN∆ϕ= 2πM

(N+1)×M

nmnθmϕθn, ϕmEIRPp1(θn,ϕm)EIRPp2(θn,ϕm)θϕ

TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn

sinθ0sinθN

NM∞

ΔϕrefΔθref,

15∘λ

EIRPθ,ϕ=sin2θcos2kcosθL2

sin2θcos2(kcosθ L/2)Δθref=λ/Lϕϕ

x,y,z=(0,0,±L/2)

ΔθλL=14,18,112,116,132

θϕθ

ϕ

Δθref=Δϕref=λ/L

Δϕref=λ/LΔθref=λ/L2+h2

Δθref=λL2+h2=λDΔϕref=λL=λDcyl

DDcyl

D=d2+w2+h2

Dcyl=d2+w2

Δθref=min⁡(180∘πarcsinλDZ, 15∘ )

Δϕref=min180∘ πarcsinλDY , 15∘

∆ϕref≅BeWϕ≅FNBWϕ2Δθref≅BeWθ≅FNBWθ2

FNBWϕFNBWθ

Δϕref=λλoBeWϕ=λλo FNBWϕ2Δθref=λλoBeWθ=λλo FNBWθ2

Dcyl=λoBeWϕ

Nθn,ϕn

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

Nsin θθϕ

N

N=4π∆θref∆ϕref

ΔθrefΔϕref

I

TRPEstimate=1Ii=0I-1EIRP(θi,ϕi)

I-1

θi=cos-11-2i+1I

ϕi=2πiΨ

Ψ=1+52

sin θ

EIRPav,cut-n=1Pj=1PEIRP(j)

TRPEstimate=1Nn=1NEIRPav,cut-n

xz

EIRPcut1ϕθ=θH EIRPcut2θϕ=0

u=sinθsinϕv=cosθ

EIRPcut1u vH=cosθHEIRPcut2vu=0u2+v2≤1

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

vH=0

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

u2+v2=1

u=sinθsinϕ, v=cos⁡(θ)

dudv=∂u,v∂θ,ϕdθdϕ=sin2θcosϕdθdϕ

TRP=14πfwdEIRPθ,ϕsinθcosϕdudv+14πbwdEIRPθ,ϕsinθcosϕdudv

TRP=ΔugridΔvgrid4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

Δugrid=λDy

Δvgrid=λDz

um vnumvn

ΔθrefΔϕref

TRPEstimate=EIRPpeakDEUT

DEUT

DEUT,BSOutputpower= Prated,c,EIRPPrated,c,TRP

ACLR=TRPdTRPe

K

ΔϕΔθ

SF=maxΔϕΔϕref,ΔθΔθref

SF-1SFmax-1 dB,

SFmaxΔϕrefΔθref

4πd2

maximum EIRP-measured EIRP

| HPBW2 |  |
| --- | --- |
| HPBW3 |  |
| HPBW4 |  |

HPBW4HPBW3

HPBW2

## 10.11	Void

Annex A:	Void

Annex B:
OTA sensitivity measurement error contribution descriptions

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

# B.1	Void

# B.2	Void

# B.3	Void

# B.4	Void

Annex C:
Beam sweeping

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

# C.1	Void

0.55λ

D=5λ=>Δϕref=λ/DΔϕref

# C.2	Void

# C.3	Void

Annex D:
Sparse sampling for spurious emissions

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

ΔϕΔθΔϕrefΔθref

Figure D-1: Void

Figure D-2: Void

Figure D-3: Void

Figure D-4: Void

Figure D-5: Void

ρmax =0.5

Table D-1: Void

Annex E:
Radiated transmit measurement error contribution descriptions

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

# E.1	Void

# E.2	Void

# E.3	Void

# E.4	Void

# E.5	Void

N σ/Nσ

Nσ/Nσ

S212

# E.6	Void

Annex F:
Test equipment uncertainty values

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

# F.1	Void

|  |  | - |  |  |  |
| --- | --- | --- | --- | --- | --- |

# F.2	Void

Annex G:
Power density measurements close to EUT

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

r0≥2DMA2/λ

Figure G-1: Void

Figure G-2: Void

Annex H:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-08 | RAN4#80bis | R4-168871 |  |  |  | Skeleton | 0.0.1 |
| 2017-02 | RAN4#81 | R4-1701717 |  |  |  | Approved text proposals in the following documents were implemented: R4-1609938, TP to 37.843: Background information on conducted RX sensitivity requirement R4-1610393, TP to TR37.843 on EVM R4-1610400, TP to TR 37.843 definitions and abbreviations R4-1610799, TP to TR 37.843: Remaining OTA requirements for eAAS BS R4-1610802, TP to 37.843: Background information on conducted ACLR requirement R4-1610803, TP to TR 37.843 on ACLR R4-1610808, TP to TR 37.843 on min EIS | 0.1.0 |
| 2017-02 | RAN4#82 | R4-1703913 |  |  |  | R4-1701717 TR 37.843 v0.1.0 Cleanup TP in R4-1701873 R4-1701873 TP to the TR 37.843: cleanup R4-1701061 TP for TR 37.843: TR structure update R4-1701730 TP to TR37.843 Updating reference architecture diagrams R4-1702261 TP to 37.843: Coordinate system for eAAS R4-1702262 TP to TR37.843 Hybrid requirements structure Huawei R4-1702264 TP for TR 37.843: Adding text for Operating band unwanted emissions OTA requirement R4-1702265 TP for TR 37.843: Adding background and OTA information for spurious emissions R4-1702267 TP to 37.843. EVM background information R4-1702268 TP for TR 37.843: Adding background information on OTA frequency error requirement R4-1702269 TP for TR 37.843: Adding background information about occupied bandwidth requirement R4-1702272 TP to 37.843: Background information on conducted RX blocking requirement R4-1702273 TP on Spurious Emission & EMC Radiated Emission requirements for AAS R4-1702274 "TP on the protection of measurement devices for EMC testing Decision: The document was not treated. " R4-1702275 TP on Regulatory EMC requirements for AAS R4-1702276 TP on EMC field strength estimation for AAS BS R4-1702277 TP on general section for EMC requirements R4-1702280 TP to TR 37.843 – maximum rated output power limits for BS classes R4-1702282 TP to TR 37.843: Overview of the Tx and Rx requirements and their compliance ranges | 0.2.0 |
| 2017-04 | RAN4#82bis | R4-1705637 |  |  |  | R4-1703914 TP to TR37.843 – definition of gain R4-1703942 TP to TR 37.843: Minimum power requirements applicability R4-1704322 TP for TR 37.843: Adding background information for TAE requirement R4-1704324 TP for TR 37.843: Addition of background information about OTA transmitter intermodulation requirement R4- 1704329 TP for TR 37.843: General Section to Sub-clause 5.6 R4-1704330 TP to TR 37.843: Spurious frequency range correction for the RF spurious emissions requirement in OTA setup R4-1704331 TP to TR 37.843: Capturing agreements on the OBUE requirement in OTA setup R4- 1704332 TP Minimum EIS for eAAS OTA Sensitivity Requirements R4-1704334 TP to TR 37.843 – Blocking R4-1704335 TP to TR 37.843 Receiver ACS and NB blocking R4-1704338 TP to TR 37.843 - Transmitter direction range definitions and declarations R4-1704339 TP to TR 37.843 – Capturing EIRP accuracy requirement R4-1704341 TP for 37.843 - TX output power dynamics requirements R4-1704342 TP for TR 37.843 On exclusion band for the protection of measurement devices for spurious emission testing R4-1704343 TP for TR 37.843 On summing Sp Emission & EMC RE requirements for AAS R4-1704344 TP to TR 37.843: OTA EMC requirements structure R4-1704345 TP to TR 37.843: Frequency range of the EMC RE and RF RSE measurements in OTA setup R4-1704346 TP to TR 37.843: Conformance testing aspects for OTA requirements R4-1704347 TP for TR 37.843: OTA testing of BS demodulation requirements | 0.3.0 |
| 2017-05 | RAN4#83 | R4-1708390 |  |  |  | R4-1705806 TP to TR 37.843: Alignment of the requirements structure in the eAAS TR R4-1704879 TP for TR 37.843: Adding Annex for background information common for co-location requirements R4-1705315 Operating band unwanted emission for eAAS R4-1705802 TP to TR 37.843: Total power dynamic range requirement R4-1705803 TP to TR 37.843: Rx spurious requirement OTA R4-1705804 TP to TR 37.843: Further details on the frequency error requirement OTA R4-1705807 TP to TR 37.843: UTRA demodulation requirements analysis R4-1706124 TP for TR 37.843: Improvements of definitions of spatial angles in sub-clause 4.6 R4-1706125 TP to TR 37.843: BS classification for eAAS BS R4-1706131 TP to TR 37.843 – TRP accuracy R4-1706135 TP for TR 37.843 - TX Power requirements for control channels R4-1703138 TP for TR 37.843: Section 4.4 R4-1706139 TP to TR 37.843: Consideration of missing aspects for core and conformance Tx spurious requirement OTA R4-1706140 TP to TR 37.843 – Clarify example in sub-clause 5.1 R4-1706141 TP to TR 37.843: OTA sensitivity requirements for eAAS R4-1706144 TP for 37.843 – Clarifying Declarations for OTA sensitivity and OTA REFSENS R4-1706150 TP to TR 37.843: Consideration of the manufacturer's declarations for Hybrid AAS BS and OTA AAS BS R4-1706151 TP to TR 37.843: Additional considerations for the OTA test setups R4-1706152 TP to TR 37.843: Further details on the definition of core and conformance requirements for BS demodulation requirements R4-1706153 TP to TR 37.843: Placeholder for the BS performance requirements doable OTA R4-1706229 TP for TR 37.843 EMC immunity for OTA AAS BS – general Section R4-1706230 TP for TR 37.843 EMC immunity for OTA AAS BS – Measurement setup | 0.4.0 |
| 2017-08 | RAN4#84 | R4-1701784 |  |  |  | R4-1708878 TP to TR37.843: cleanup R4-1707481 TP for TR 37.843 on RI spatial exclusion R4-1707906 TP to TR37.843: new High Speed requirements for PRACH R4-1708401 TP to TR 37.843 - capture agreements on TX IMD R4-1708407 TP to TR 37.843 – Correcting BS power class limits R4-1708408 TP to TR 37.843 – Occupied BW OTA requirement type R4-1708879 TP to TR 37.843: General architecture update for hybrid and OTA AAS BS: RIB interface R4-1708891 TP to TR 37.843 - capture agreements on co-location requirements R4-1708950 TP for TR 37.843: Implementation of agreements on loss factor and emissions scaling R4-1708951 TP for TR 37.843 on co-existence in the same geographical area R4-1708953 TP to TR 37.843: fixed scaling and losses for the unwanted emissions R4-1708954 Transmit pulse shape filter requirement for OTA AAS BS R4-1708958 TP to TR 37.843 - Capturing agreements on OTA REFSENS R4-1709178 TP to TR 37.843: SEM for UTRA | 0.5.0 |
| 2017-10 | RAN4#84bis | R4-1713228 |  |  |  | R4-1711333 TP to TR37.843: correction of the RF RSE and EMC RE requirement R4-1711791 TP to TR 37.843: EIRP type Requirements R4-1711793 TP for TR 37.843: OTA modulation quality requirement range (5.5.4) R4-1711797 TP to TR 37.843 – Co-location requirements R4-1711801 TP to TR 37.843: OTA receiver blocking R4-1711802 TP to TR 37.843: OTA receiver ACS R4-1711803 TP to TR 37.843: OTA RX in channel selectivity R4-1711804 TP to TR 37.843: OTA Receiver Intermodulation R4-1711805 TP to TR 37.843: OTA receiver dynamic range R4-1711855 TP to TR 37.843 - extreme temperature power accuracy limits R4-1711862 TP to TR 37.843 - Out of band blocking | 0.6.0 |
| 2017-11 | RAN4#85 | R4-1713230 |  |  |  | R4-1713229 TP to TR 37.843 - clean up R4-1713251 TP to TR 37.843 update WI scope based on spec simplification agreements R4-1714236 TP to TR 37.843: in-band and out-of-band boundary for AAS BS R4-1714218 TP to TR 37.843 - TX IMD - co-location requirements R4-1714219 TP to TR 37.843 - co-location spurious emissions requirements R4-1714220 TP to TR 37.843 - co-location requirements definitions R4-1713255 TP to TR 37.843 - update transmitter radiated requirements table in sub-clause 5.1 R4-1713673 TP to TR 37.843: BS class power limits (5.2.4) R4-1713277 TP to TR 37.843: Out of band blocking frequency range R4-1712635 TP to TR 37.843: Reuse of the FRC’s for OTA Rx requirements R4-1713247 TP to TR 37.843 - Add agreements on receiver requirements R4-1714214 TP for TR 37.843: Extension to RIB definition in clause 3 and sub-clause 4.4 R4-1714223 TP to TR 37.843 - TX OFF level R4-1714225 TP to TR 37.843 - on out of band blocking. R4-1714226 TP for TR 37.843: Section 6.5.2 Out of band blocking OTA core requirement R4-1714229 TP to TR 37.843 - Add agreements on receiver sensitivity to sub-clause 6.2 R4-1714237 TP for TR 37.843: Addition of TRP in terminology in clause 3 and 5 R4-1714238 TP for TR 37.843: CATR Test Method Procedure for EVM R4-1714239 TP to TR 37.843: further refinements for OTA BS demodulation requirements R4-1714280 TP to TR 37.843 - update receiver radiated requirements table in sub-clause 6.1 R4-1714231 TP to TR 37.843 Introduction of Radiated Immunity and Spatial Exclusion | 0.7.0 |
| 2017-12 | RAN#78 | RP-172548 |  |  |  | Presented to RAN#78 for Approval | 1.0.0 |
| 2017-12 | RAN#78 |  |  |  |  | Approved by plenary – Rel-15 under change control | 15.0.0 |
| 2018-09 | RAN#81 | RP-181907 | 0001 |  | B | CR to TR 37.843 | 15.1.0 |
| 2018-12 | RAN#82 | RP-182387 | 0005 |  | F | CR to TR 37.843 v15.1.0 | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0006 | 2 | B | New Annex to TR 37.843: Power density measurements close to EUT | 15.2.0 |
| 2019-03 | RAN#83 | RP-190418 | 0008 |  | F | CR to TR 37.843: Addition of uncertainty sources relevant for RC in Annex E | 15.3.0 |
| 2019-03 | RAN#83 | RP-190419 | 0009 | 1 | F | CR to TR 37.843: Addition of RC MU evaluation and description in clause 2 and sub-clause 10.5 | 15.3.0 |
| 2019-03 | RAN#83 | RP-190419 | 0010 | 1 | F | CR to TR 37.843: Editorial clean-up of TRP measurement section in sub-clause 10.8 | 15.3.0 |
| 2019-03 | RAN#83 | RP-190419 | 0011 | 2 | F | CR to TR 37.843: Wave vector spacing sampling grid (Section 10.8.2.6) | 15.3.0 |
| 2019-06 | RAN#84 | RP-191263 | 0017 |  | F | CR to TR37.843, correct references to annex E | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0018 |  | F | CR to TR 37.843: Addition of RC MU evaluation for spurious emission in subclause 10.5.2.3A.5.2 | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0019 |  | F | CR to TR 37.843: Update of RC description in subclause 10.5.2.3A | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0020 |  | F | CR to TR 37.843: Addition of MU evaluation for testing output power, ACLR and OBUE in RC test method in subclause 10.4 | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0021 |  | F | CR to TR 37.843: Removal of information in Annex A | 15.4.0 |
| 2019-06 | RAN#84 | RP-191262 | 0022 |  | F | CR to TR 37.843: Upper limit for angular step for TRP calculation | 15.4.0 |
| 2019-06 | RAN#84 | RP-191262 | 0023 | 2 | F | CR to TP 37.843: Correction on usage of term EIRP | 15.4.0 |
| 2019-06 | RAN#84 | RP-191262 | 0024 | 2 | F | CR to TR 37.843 removal of Tx Diversity for TAE testing | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0025 | 1 | F | CR to TR 37.843: editorial corrections to spherical angle formula in subclause 10.8.2.4 | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0026 | 2 | F | CR to TR37.843 Correction on beam based direcrtions | 15.4.0 |
| 2019-09 | RAN#85 | RP-192053 | 0027 | 1 | F | CR to TR 37.843: Improvement of error contribution decsription for RC in Annex E.5 | 15.5.0 |
| 2019-09 | RAN#85 | RP-192053 | 0028 | 1 | F | CR to TR 37.843: Improvement of RC technical background in clause 2 and subclause 10.5.2.3A | 15.5.0 |
| 2019-09 | RAN#85 | RP-192053 | 0029 | 1 | F | CR to TR 37.843: Addition of 2D Compact Range. MU, calibration and test procedure for EIRP and EIS. | 15.5.0 |
| 2019-12 | RAN#86 | RP-193048 | 0033 |  | F | CR to TR 37.843: Improvement of RC technical background in subclause 10.5.2.3A | 15.6.0 |
| 2019-12 | RAN#86 | RP-193048 | 0034 | 1 | F | CR to TR 37.843 Addition of Plane Wave Synthesizer MU calibration and test procedure for In-band TRP requirements-2. | 15.6.0 |
| 2019-12 | RAN#86 | RP-193048 | 0035 |  | F | CR to TR 37.843: Completion of MU terms for PWS. | 15.6.0 |
| 2019-12 | RAN#86 | RP-193048 | 0037 | 1 | F | CR to TR 37.843: adding background information on angular alignment in subclause 10.8.4 | 15.6.0 |
| 2019-12 | RAN#86 | RP-193048 | 0039 |  | F | CR to TR 37.843: Power Density limitation for PWS. | 15.6.0 |
| 2020-06 | RAN#88 | RP-201005 | 0040 | 1 | F | CR to TR 37.843: internal TR references corrections and content redundancy removal (wrt. TR 37.941 for OTA BS testing), Rel-15 | 15.7.0 |