| 3GPP TS 36.133 V16.13.0 (2022-03) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	92

1	Scope	94

2	References	94

3	Definitions, symbols and abbreviations	96

3.1	Definitions	96

3.2	Symbols	98

3.3	Abbreviations	99

3.4	Test tolerances	102

3.5	Additional notation	102

3.5.1	Groups of bands	102

3.6	General	104

3.6.1	Applicability of requirements in this specification version	104

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	110

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	112

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	113

3.6.2	Applicability of requirements for EN-DC operation	114

3.6.3	Applicability of requirements for NE-DC operation	115

3.6.4	Applicability of requirements for NGEN-DC operation	116

4	E-UTRAN RRC_IDLE state mobility	116

4.1	Cell Selection	116

4.2	Cell Re-selection	116

4.2.1	Introduction	116

4.2.2	Requirements	117

4.2.2.1	Measurement and evaluation of serving cell	118

4.2.2.2	Void	119

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	119

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	121

4.2.2.5	Measurements of inter-RAT cells	123

4.2.2.5.1	Measurements of UTRAN FDD cells	123

4.2.2.5.2	Measurements of UTRAN TDD cells	125

4.2.2.5.3	Measurements of GSM cells	126

4.2.2.5.4	Measurements of HRPD cells	127

4.2.2.5.5	Measurements of cdma2000 1X	128

4.2.2.5.6	Measurements of NR cells	129

4.2.2.5.7	Measurements of NR cells subject to CCA	131

4.2.2.6	Evaluation of cell re-selection criteria	132

4.2.2.7	Maximum interruption in paging reception	133

4.2.2.8	void	133

4.2.2.9	UE measurement capability	133

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	134

4.2.2.10	Reselection to CSG cells	134

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	135

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	135

4.2.2.11	Void	136

4.2.2.12	Void	136

4.2.2.13	Void	136

4.3	Minimization of Drive Tests (MDT)	136

4.3.1	Introduction	137

4.3.2	Measurements	137

4.3.2.1	Requirements	137

4.3.3	Relative Time Stamp Accuracy	137

4.3.3.1	Requirements	137

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	137

4.3.4.1	 Requirements	137

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	138

4.3.5.1	Requirements for timeSinceFailure	138

4.4	MBSFN Measurements	138

4.4.1	Introduction	138

4.4.2	MBSFN RSRP measurements	138

4.4.3	MBSFN RSRQ measurements	138

4.4.4	MCH BLER measurements	139

4.5	Proximity-based Services	139

4.5.1	Introduction	139

4.5.2	Requirements	139

4.5.2.1	Interruptions with ProSe Direct Discovery	139

4.5.2.2	Interruptions with ProSe Direct Communication	139

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	139

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	140

4.6	Cell Selection and Re-selection Requirements for UE category NB1	140

4.6.1	Cell Selection	140

4.6.2	Cell Re-selection	140

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	141

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	142

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	143

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	144

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	146

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	146

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	148

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	149

4.6.2.7	Maximum interruption in paging reception in normal coverage	151

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	151

4.6.2.8	UE measurement capability	151

4.6.2.9	WUS receptions for NB1	151

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	152

4.6.3.1	Introduction	152

4.6.3.2	Requirements on UE synchronization for transmission using PUR	152

4.6.3.3	Requirements on TA validation for transmission using PUR	152

4.7	Cell Selection and Re-selection Requirements for UE category M1	153

4.7.1	Cell Selection	153

4.7.2	Cell Re-selection	153

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	153

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	153

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	154

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	155

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	157

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	158

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	159

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	159

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	159

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	160

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	161

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	164

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	165

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	166

4.7.2.3	WUS receptions for UE category M1	166

4.7.3	Channel quality report for UE Category M1 in idle mode	166

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	167

4.7.4.1	Introduction	167

4.7.4.2	Requirements on UE synchronization for transmission using PUR	167

4.7.4.3	Requirements on TA validation for transmission using PUR	167

4.8	Idle State Positioning Measurement Requirements for UE category NB1	168

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	168

4.8.1.1	RSTD Measurement Reporting Delay	170

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	170

4.8.2.1	RSTD Measurement Reporting Delay	172

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	172

4.8.3.1	RSTD Measurement Reporting Delay	173

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	174

4.8.4.1	RSTD Measurement Reporting Delay	175

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	176

4.8.5.1	Measurement Reporting Delay	177

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	177

4.8.6.1	Measurement Reporting Delay	178

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	179

4.8.7.1	Measurement Reporting Delay	180

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	180

4.8.8.1	Measurement Reporting Delay	181

4.9	Idle Mode CA Measurement	182

4.9.1	Introduction	182

4.9.2	Requirement	182

4.9.2.1	Detected cell requirement during state transition and Idle mode	182

4.9.2.2	Measurements of inter-frequency CA candidate cells	183

4.9.2.3	Measurements on serving cell	183

4A	E-UTRAN RRC_INACTIVE state mobility	184

4A.1	Cell Re-selection	184

4A.1.1	Introduction	184

4A.1.2	Requirements	185

4A.1.2.1	UE measurement capability	185

4A.1.2.2	Measurement and evaluation of serving cell	185

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	185

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	185

4A.1.2.5	Evaluation of cell re-selection criteria	185

4A.1.2.6	Maximum interruption in paging reception	185

4A.1.2.7	Measurements of inter-RAT NR cells	185

4A.2	Requirements for UE Category M1	185

4A.2.1	Introduction	185

4A.2.2	Cell Selection	185

4A.2.3	Cell Reselection	185

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	185

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	185

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	186

A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	186

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	186

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	186

4A.2.4	Channel quality report for UE Category M1 in idle mode	188

5	E-UTRAN RRC_CONNECTED state mobility	188

5.1	E-UTRAN Handover	189

5.1.1	Introduction	189

5.1.2	Requirements	189

5.1.2.1	E-UTRAN FDD – FDD	189

5.1.2.1.1	Handover delay	189

5.1.2.1.2	Interruption time	189

5.1.2.2	E-UTRAN FDD – TDD	190

5.1.2.2.1	(Void)	191

5.1.2.2.2	(Void)	191

5.1.2.3	E-UTRAN TDD – FDD	191

5.1.2.3.1	(Void)	191

5.1.2.3.2	(Void)	191

5.1.2.4	E-UTRAN TDD – TDD	191

5.1.2.4.1	Handover delay	191

5.1.2.4.2	Interruption time	191

5.1.2.5	E-UTRAN HD–FDD	193

5.1.2.5.1	Handover delay	193

5.1.2.5.2	Interruption time	193

5.1.2.6	E-UTRAN FDD – FDD conditional handover	194

5.1.2.6.1	Handover delay	194

5.1.2.6.2	Measurement time	195

5.1.2.6.3	Preparation time	195

5.1.2.6.4	Interruption time	195

5.1.2.7	E-UTRAN FDD – TDD conditional handover	195

5.1.2.8	E-UTRAN TDD – FDD conditional handover	196

5.1.2.9	E-UTRAN TDD – TDD conditional handover	196

5.2	Void	196

5.3	Handover to other RATs	196

5.3.1	E-UTRAN - UTRAN FDD Handover	196

5.3.1.1	Introduction	196

5.3.1.1.1	Handover delay	196

5.3.1.1.2	Interruption time	196

5.3.2	E-UTRAN - UTRAN TDD Handover	197

5.3.2.1	Introduction	197

5.3.2.2	Requirements	197

5.3.2.2.1	Handover delay	197

5.3.2.2.2	Interruption time	197

5.3.3	E-UTRAN - GSM Handover	198

5.3.3.1	Introduction	198

5.3.3.2	Requirements	198

5.3.3.2.1	Handover delay	198

5.3.3.2.2	Interruption time	198

5.3.4	E-UTRAN - NR FR1 Handover	198

5.3.4.1	Introduction	198

5.3.4.2	Handover delay	199

5.3.4.3	Interruption time	199

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	199

5.3.4A.1	Introduction	199

5.3.4A.2	Handover delay	200

5.3.4A.3	Interruption time	200

5.3.5	E-UTRAN - NR FR2 Handover	201

5.3.5.1	Introduction	201

5.3.5.2	Handover delay	201

5.3.5.3	Interruption time	201

5.4	Handover to Non-3GPP RATs	202

5.4.1	E-UTRAN – HRPD Handover	202

5.4.1.1	Introduction	202

5.4.1.1.1	Handover delay	202

5.4.1.1.2	Interruption time	202

5.4.2	E-UTRAN – cdma2000 1X Handover	203

5.4.2.1	Introduction	203

5.4.2.1.1	Handover delay	203

5.4.2.1.2	Interruption time	203

5.5	E-UTRAN Handover for Cat-M1 UEs	203

5.5.1	Introduction	203

5.5.2	Requirements in CEModeA	203

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	203

5.5.2.1.1	Handover delay	203

5.5.2.1.2	Interruption time	204

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	204

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	204

5.5.2.3.1	Void	205

5.5.2.3.2	Void	205

5.5.3	Requirements in CEModeB	205

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	205

5.5.3.1.1	Handover delay	205

5.5.3.1.2	Interruption time	205

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	206

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	206

5.6	Void	206

5.7	E-UTRAN DAPS Handover	206

5.7.1	Introduction	206

5.7.2	Requirements	206

5.7.2.1	E-UTRAN FDD – FDD	206

5.7.2.1.1	DAPS Handover delay	206

5.7.2.1.2	Interruption time	207

5.7.2.2	E-UTRAN FDD – TDD	207

5.7.2.3	E-UTRAN TDD – FDD	207

5.7.2.4	E-UTRAN TDD – TDD	208

6	RRC Connection Mobility Control	208

6.1	RRC Re-establishment	208

6.1.1	Introduction	208

6.1.2	Requirements	208

6.1.2.1	UE Re-establishment delay requirement	208

6.2	Random Access	209

6.2.1	Introduction	209

6.2.2	Requirements	209

6.2.2.1	Contention based random access	209

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	209

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	209

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	209

6.2.2.1.4	Void	209

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	209

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	209

6.2.2.2	Non-Contention based random access	210

6.2.2.2.1	Correct behaviour when receiving Random Access Response	210

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	210

6.2.3	Requirements for Cat-M1 UEs	210

6.3	RRC Connection Release with Redirection	210

6.3.1	Introduction	210

6.3.2	Requirements	210

6.3.2.1	RRC connection release with redirection to UTRAN FDD	210

6.3.2.2	RRC connection release with redirection to GERAN	211

6.3.2.3	RRC connection release with redirection to UTRAN TDD	211

6.3.2.4	RRC connection release with redirection to NR	212

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	212

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	214

6.4.1	Introduction	214

6.4.2	Requirements	214

6.5	RRC Re-establishment for NB-IoT UEs	214

6.5.1	Introduction	214

6.5.2	Requirements	214

6.5.2.1	UE Re-establishment delay requirement in normal coverage	214

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	215

6.6	Random Access for UE category NB1	215

6.6.1	Introduction	215

6.6.2	Requirements	215

6.6.2.1	Correct behaviour when receiving Random Access Response reception	216

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	216

6.6.2.3	Correct behaviour when receiving a NACK on msg3	216

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	216

6.6.2.5	Correct behaviour when contention Resolution timer expires	216

6.6.2.6	MSG3-based channel quality report for UE Category NB1	216

6.6.3	Requirements for NPRACH configuration	217

6.7	RRC Re-establishment for Cat-M1 UEs	217

6.7.1	Introduction	217

6.7.2	Requirements	217

6.7.2.1	UE Re-establishment delay requirement for CEModeA	217

6.7.2.2	UE Re-establishment delay requirement for CEModeB	218

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	218

6.8.1	Introduction	218

6.8.2	Requirements	219

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	219

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	219

6.9.1	Introduction	219

6.9.2	Requirements	219

7	Timing and signalling characteristics	220

7.1	UE transmit timing	220

7.1.1	Introduction	220

7.1.2	Requirements	221

7.2	UE timer accuracy	222

7.2.1	Introduction	222

7.2.2	Requirements	222

7.3	Timing Advance	222

7.3.1	Introduction	222

7.3.2	Requirements	222

7.3.2.1	Timing Advance adjustment delay	222

7.3.2.2	Timing Advance adjustment accuracy	223

7.4	Cell phase synchronization accuracy (TDD)	223

7.4.1	Definition	223

7.4.2	Minimum requirements	223

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	224

7.5.1	Introduction	224

7.5.2	eNodeB Synchronization Requirements	224

7.5.2.1	Synchronized E-UTRAN	224

7.5.2.2	Non-Synchronized E-UTRAN	224

7.6	Radio Link Monitoring	224

7.6.1	Introduction	224

7.6.2	Requirements	226

7.6.2.1	Minimum requirement when no DRX is used	226

7.6.2.2	Minimum requirement when DRX is used	226

7.6.2.3	Minimum requirement at transitions	228

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	228

7.6.2.5	Minimum requirement under IDC Interference	228

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	228

7.7.1	Introduction	228

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	229

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	230

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	231

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	232

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	233

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	234

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	235

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	235

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	235

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	236

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	237

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	238

7.7.14	SCell Activation Delay Requirement for Dormant SCell	238

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	240

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	240

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	242

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	242

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	244

7.8	Interruptions with Carrier Aggregation	246

7.8.1	Introduction	246

7.8.2	Requirements	246

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	246

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	246

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	246

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	247

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	247

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	247

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	247

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	247

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	248

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	249

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	249

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	249

7.8.2.13	Interruptions at SRS carrier based switching	250

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	251

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	251

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	251

7.8.2.17	Interruptions during CQI measurement on dormant SCell	251

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	252

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	252

7.8.2.20	Interruptions at SCell hibernation	252

7.8.2.21	Interruptions at direct SCell activation and hibernation	252

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	253

7.9.1	Introduction	253

7.9.2	Minimum Requirements for Interband Carrier Aggregation	253

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	253

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	254

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	254

7.10.1	Introduction	254

7.10.2	Requirements	254

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	254

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	254

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	255

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	255

7.11	Radio Link Monitoring for UE Category 0	256

7.11.1	Introduction	256

7.11.2	Requirements for FD-FDD and TDD	257

7.11.2.1	Minimum requirement when no DRX is used	257

7.11.2.2	Minimum requirement when DRX is used	258

7.11.2.3	Minimum requirement at transitions	258

7.11.3	Requirements for HD-FDD	259

7.11.3.1	Minimum requirement when no DRX is used	259

7.11.3.2	Minimum requirement when DRX is used	259

7.11.3.3	Minimum requirement at transitions	260

7.12	Interruptions with Dual Connectivity	260

7.12.1	Introduction	260

7.12.2	Requirements	260

7.12.2.1	Interruptions at PSCell addition/release	260

7.12.2.2	Interruptions at transitions between active and non-active during DRX	260

7.12.2.3	Interruptions at transitions from non-DRX to DRX	261

7.12.2.4	Interruptions at SCell addition/release	261

7.12.2.5	Interruptions at SCell activation/deactivation	261

7.12.2.6	Interruptions during measurements on SCC	262

7.12.2.7	Interruptions at SRS carrier based switching	262

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	263

7.13.1	Definition	263

7.13.2	Minimum requirements	263

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	263

7.14.1	Introduction	263

7.14.2	PSCell Addition Delay Requirement	263

7.14.3	PSCell Release Delay Requirement	264

7.15	Maximum Receive Timing Difference in Dual Connectivity	264

7.15.1	Introduction	264

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	264

7.16	Proximity-based Services	264

7.16.1	Introduction	264

7.16.2	Requirements	264

7.16.2.1	ProSe UE transmission timing	264

7.16.2.1.1	Serving cell or PCell as timing reference	265

7.16.2.1.2	SCell or non-serving cell as timing reference	265

7.16.3	Interruptions with ProSe	265

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	265

7.16.3.2	Interruptions at ProSe Direct Communication configuration	266

7.16.3.3	Interruptions during ProSe Direct Discovery	266

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	266

7.16.3.5	Interruptions during ProSe Direct Communication	267

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	267

7.16.4.1	Measurement and evaluation of selected cell	267

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	267

7.16.5	Selection / Reselection of ProSe relay UE	268

7.16.6	ProSe operation under deactivated SCell	268

7.17	Maximum Transmission Timing Difference in Dual Connectivity	268

7.17.1	Introduction	268

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	269

7.18.1	Introduction	269

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	269

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	269

7.19	Radio Link Monitoring for UE Category M1	269

7.19.1	Introduction	269

7.19.2	Requirements for FD-FDD and TDD CE mode A	270

7.19.2.1	Minimum requirement when no DRX is used	271

7.19.2.2	Minimum requirement when DRX is used	272

7.19.2.3	Minimum requirement at transitions	273

7.19.3	Requirements for HD-FDD with CE mode A	273

7.19.3.1	Minimum requirement when no DRX is used	274

7.19.3.2	Minimum requirement when DRX is used	274

7.19.3.3	Minimum requirement at transitions	275

7.19.4	Requirements for FD-FDD and TDD with CE mode B	275

7.19.4.1	Minimum requirement when no DRX is used	277

7.19.4.2	Minimum requirement when DRX is used	278

7.19.4.3	Minimum requirement at transitions	279

7.19.5	Requirements for HD-FDD with CE mode B	279

7.19.5.1	Minimum requirement when no DRX is used	279

7.19.5.2	Minimum requirement when DRX is used	280

7.19.5.3	Minimum requirement at transitions	281

7.20	UE transmit timing for NB-IoT	281

7.20.1	Introduction	281

7.20.2	Requirements	281

7.21	 UE timer accuracy for NB-IoT	282

7.21.1	Introduction	282

7.21.2	Requirements	282

7.22	Timing Advance for NB-IoT	282

7.22.1	Introduction	282

7.22.2	Requirements	282

7.22.2.1	Timing Advance adjustment delay	282

7.22.2.2	Timing Advance adjustment accuracy	282

7.23	Radio Link Monitoring for Category NB1 UE	282

7.23.1	Introduction	282

7.23.2	Requirements for Category NB1 UE	283

7.23.2.1	Minimum requirement when no DRX is used	283

7.23.2.2	Minimum requirement when DRX is used	284

7.23.2.3	Minimum requirement at transitions	284

7.24	UE transmit timing for Category M1	284

7.24.1	Introduction	284

7.24.2	Requirements	285

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	285

7.25.1	Definition	285

7.25.2	Minimum requirements	286

7.26	UE transmit timing for Category M2	286

7.26.1	Introduction	286

7.26.2	Requirements	286

7.27	UE timer accuracy for category M1	286

7.27.1	Introduction	286

7.27.2	Requirements	286

7.28	Timing Advance for Category M1	287

7.28.1	Introduction	287

7.28.2	Requirements	287

7.29	Interruptions requirements with FeMBMS	287

7.29.1	Introduction	287

7.29.2	Requirements	287

7.30	Numerology switching delay requirements with FeMBMS	287

7.30.1	Introduction	287

7.30.2	Requirements	287

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	287

7.31.1	Introduction	287

7.31.2	NR PSCell Addition Delay Requirement	288

7.31.3	NR PSCell Release Delay Requirement	288

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	289

7.31A.1	Introduction	289

7.31A.2	NR PSCell Addition Delay Requirement	289

7.31A.3	NR PSCell Release Delay Requirement	290

7.32	Interruptions with EN-DC	290

7.32.1	Introduction	290

7.32.2	Requirements	291

7.32.2.1	Interruptions at PSCell addition/release	291

7.32.2.2	Interruptions at transitions between active and non-active during DRX	291

7.32.2.3	Interruptions at transitions from non-DRX to DRX	291

7.32.2.4	Interruptions at SCell addition/release	291

7.32.2.5	Interruptions at SCell activation/deactivation	291

7.32.2.6	Interruptions during measurements on SCC	292

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	292

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	292

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	292

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	292

7.32.2.7	Interruptions at active BWP switching	292

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	293

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	293

7.32.2.10	Interruptions at SCell hibernation	294

7.32.2.11	Interruptions at direct SCell activation and hibernation	294

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	294

7.32.2.13	Interruptions at NR SRS carrier based switching	294

7.32.2.14	Interruptions at NR SCell dormancy	295

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	295

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	295

7.32.2.15	Interruption during NR measurement with autonomous gaps	296

7.32.2.16	Interruptions at SRS carrier based switching	296

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	296

7.33.1	Introduction	296

7.33.2	Requirements	297

7.34	Void	297

7.35	Interruptions with SFTD measurements	297

7.35.1	Introduction	297

7.35.2	Requirements	297

7.36	Interruptions with NE-DC	297

7.36.1	Introduction	297

7.36.2	Requirements	298

7.36.2.1	Interruptions at transitions between active and non-active during DRX	298

7.36.2.2	Interruptions at transitions from non-DRX to DRX	298

7.36.2.3	Interruptions at SCell addition/release	298

7.36.2.4	Interruptions at SCell activation/deactivation	298

7.36.2.5	Interruptions during measurements on SCC	299

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	299

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	299

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	299

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	299

7.36.2.6	Interruptions at active BWP switching	299

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	300

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	300

7.36.2.9	Interruptions at SCell hibernation	300

7.36.2.10	Interruptions at direct SCell activation and hibernation	300

7.36.2.11	Interruptions at NR SRS carrier based switching	301

7.36.2.12	Interruptions at NR SCell dormancy	301

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	301

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	302

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	302

7.36.2.14	Interruption during NR measurement with autonomous gaps	302

7.36.2.15	Interruptions at SRS carrier based switching	303

7.37	Interruptions during NR measurement with autonomous gaps	303

7.37.1	Introduction	303

7.37.2	Requirements	303

8	UE Measurements Procedures in RRC_CONNECTED State	304

8.1	General Measurement Requirements	304

8.1.1	Introduction	304

8.1.2	Requirements	304

8.1.2.1	UE measurement capability	304

8.1.2.1.1	Monitoring of multiple layers using gaps	312

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	313

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	314

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	316

8.1.2.1.2	Network controlled small gap	317

8.1.2.2	E-UTRAN intra frequency measurements	318

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	318

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	323

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	327

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	328

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	329

8.1.2.3	E-UTRAN inter frequency measurements	330

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	330

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	336

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	342

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	342

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	343

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	344

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	345

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	346

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	347

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	354

8.1.2.4	Inter RAT measurements	354

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	354

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	359

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	359

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	363

8.1.2.4.5	E-UTRAN FDD – GSM measurements	363

8.1.2.4.6	E-UTRAN TDD – GSM measurements	368

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	368

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	370

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	370

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	370

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	371

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	371

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	371

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	371

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	373

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	373

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	373

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	373

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	374

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	374

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	376

8.1.2.4.21	E-UTRAN FDD – NR measurements	376

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	380

8.1.2.4.22	E-UTRAN TDD – NR measurements	384

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	384

8.1.2.4.23	Void	384

8.1.2.4.24	Void	384

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	384

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	386

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	387

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	388

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	388

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	388

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	390

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	392

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	393

8.1.2.6.5	Void	395

8.1.2.6.6	Void	395

8.1.2.6.7	Void	395

8.1.2.6.8	Void	395

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	395

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	396

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	397

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	399

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	401

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	402

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	404

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	406

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	408

8.1.2.7	E-UTRAN E-CID Measurements	409

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	409

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	411

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	412

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	413

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	413

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	413

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	416

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	419

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	422

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	426

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	426

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	426

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	427

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	427

8.1.2.10	Void	428

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	428

8.2.1	Introduction	428

8.2.2	Requirements	428

8.3	Measurements for E-UTRA carrier aggregation	432

8.3.1	Introduction	432

8.3.2	Measurements of the primary component carrier	432

8.3.3	Measurements of a secondary component carrier	432

8.3.3.1	Measurements of a secondary component carrier with active SCell	432

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	433

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	433

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	434

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	436

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	436

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	436

8.4.1	Introduction	436

8.4.2	Measurements on the primary component carrier	437

8.4.3	Measurements on a secondary component carrier	438

8.4.4	Measurements on both primary component carrier and a secondary component carrier	438

8.4.5	Measurements on different secondary component carriers	439

8.5	Measurements for UE category 0	440

8.5.1	Introduction	440

8.5.2 	Requirements	441

8.5.2.1 	E-UTRAN intra frequency measurements	441

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	441

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	444

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	446

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	450

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	450

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	451

8.6	Discovery signal measurements	452

8.6.1	Introduction	452

8.6.2	Requirements for CRS based discovery signal measurements	452

8.6.2.1	E-UTRAN intra frequency measurements	452

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	452

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	455

8.6.2.2	E-UTRAN inter frequency measurements	457

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	458

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	460

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	463

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	463

8.6.3	Requirements for CSI-RS based discovery signal measurements	463

8.6.3.1	E-UTRAN intra frequency measurements	463

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	464

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	466

8.6.3.2	E-UTRAN inter frequency measurements	468

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	469

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	471

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	474

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	474

8.7	Discovery signal measurements for E-UTRA carrier aggregation	474

8.7.1	Introduction	474

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	475

8.7.2.1	Measurements of the primary component carrier	475

8.7.2.2	Measurements of a secondary component carrier	475

8.7.2.3	Measurements of a secondary component carrier with active SCell	475

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	475

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	475

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	476

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	478

8.7.3.1	Measurements of the primary component carrier	478

8.7.3.2	Measurements of a secondary component carrier	478

8.7.3.3	Measurements of a secondary component carrier with active SCell	478

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	478

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	478

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	480

8.8	Measurements for E-UTRA dual connectivity	481

8.8.1	Introduction	481

8.8.2	Intra-frequency measurements requirements on PCell	481

8.8.3	Intra-frequency measurements requirements on PSCell	482

8.8.4	Inter-frequency and inter-RAT measurement requirements	482

8.8.5	Intra-frequency measurements with autonomous gaps	482

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	482

8.8.5.2	ECGI reporting delay	483

8.8.6	Inter-frequency measurements with autonomous gaps	483

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	483

8.8.6.2	ECGI reporting delay	484

8.8.7	SSTD Measurements	484

8.8.7.1	Introduction	484

8.8.7.2	SSTD Measurement requirements	484

8.8.7.3	SSTD Measurement Reporting Delay	485

8.8.8	Intra-frequency measurements requirements on SCell	485

8.9	MBSFN Measurements	485

8.9.1	Introduction	485

8.9.2	MBSFN RSRP Measurements	485

8.9.3	MBSFN RSRQ Measurements	486

8.9.4	MCH BLER Measurements	486

8.10	Proximity-based Services	486

8.10.1	Introduction	486

8.10.2	Requirements	486

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	486

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	487

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	487

8.11.1	Introduction	487

8.11.2	CRS based discovery signal measurements	488

8.11.2.1	E-UTRAN intra-frequency measurements	488

8.11.2.1.1	Requirements	488

8.11.2.1.1.1	Requirements when no DRX is used	488

8.11.2.1.1.1.1	Measurement Reporting Requirements	490

8.11.2.1.1.2	Requirements when DRX is used	491

8.11.2.1.1.2.1	Measurement Reporting Requirements	493

8.11.2.2	E-UTRAN inter-frequency measurements	494

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	494

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	498

8.11.3	CSI-RS based discovery signal measurements	498

8.11.3.1	E-UTRAN intra-frequency measurements	498

8.11.3.1.1	Requirements	498

8.11.3.1.1.1	Requirements when no DRX is used	498

8.11.3.1.1.1.1	Measurement Reporting Requirements	500

8.11.3.1.1.2	Requirements when DRX is used	500

8.11.3.1.1.2.1	Measurement Reporting Requirements	501

8.11.3.2	E-UTRAN inter-frequency measurements	502

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	502

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	506

8.11.4	RSSI measurements	506

8.11.4.1	E-UTRAN intra-frequency measurements	506

8.11.4.2	E-UTRAN inter-frequency measurements	506

8.11.5	Channel occupancy measurements	507

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	507

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	507

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	507

8.12.1	Introduction	507

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	507

8.12.2.1	Introduction	507

8.12.2.2	Measurements of a secondary component carrier	507

8.12.2.3	Measurements of a secondary component carrier with active SCell	507

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	508

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	508

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	510

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	513

8.12.3.1	Introduction	513

8.12.3.2	Measurements of a secondary component carrier	513

8.12.3.3	Measurements of a secondary component carrier with active SCell	513

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	513

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	513

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	515

8.13	Measurements for UE Category M1	517

8.13.1	Introduction	517

8.13.2	Requirements for UE category M1 with CE mode A	517

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	518

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	518

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	523

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	525

8.13.2.2	Void	529

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	529

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	529

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	532

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	534

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	535

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	535

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	537

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	539

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	540

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	540

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	540

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	541

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	541

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	541

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	542

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	542

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	543

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	544

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	544

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	545

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	549

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	551

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	555

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	555

8.13.3	Requirements for UE category M1 with CE mode B	556

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	556

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	557

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	561

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	564

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	569

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	570

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	570

8.13.3.2	Void	571

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	571

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	571

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	574

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	576

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	577

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	577

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	577

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	578

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	578

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	578

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	579

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	579

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	579

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	584

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	586

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	590

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	591

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	591

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	593

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	595

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	596

8.14	Measurements for UE category NB1	596

8.14.1	Introduction	596

8.14.2	NB-IoT intra frequency measurements under normal coverage	597

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	597

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	597

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	597

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	597

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	597

8.14.4	Connected mode channel quality report for UE Category NB1	597

8.15	Void	598

8.16	Measurements for UE Category M2	598

8.16.1	Introduction	598

8.16.2	Requirements for UE category M2 with CE mode A	598

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	598

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	599

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	599

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	600

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	600

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	600

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	601

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	601

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	603

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	606

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	606

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	606

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	609

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	611

8.16.3	Requirements for UE category M2 with CE mode B	612

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	612

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	612

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	614

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	617

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	617

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	618

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	620

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	622

8.17	Measurements for E-UTRA – NR Dual Connectivity	623

8.17.1	Introduction	623

8.17.1.1	Measurement Gap Sharing	623

8.17.1A	Intrafrequency Measurements	623

8.17.2	SFTD Measurements	624

8.17.2.1	Introduction	624

8.17.2.2	SFTD Measurement requirements	624

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	624

8.17.2.3	SFTD Measurement Reporting Delay	625

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	626

8.17.3.1	Introduction	626

8.17.3.2	E-UTRAN FDD inter frequency measurements	626

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	626

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	628

8.17.3.3	E-UTRAN TDD inter frequency measurements	630

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	630

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	631

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	633

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	633

8.17.4.1.1	NR Inter-RAT cell identification	634

8.17.4.1.2	NR Inter-RAT measurement	635

8.17.4.1.3	NR Inter-RAT measurement reporting	636

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	637

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	637

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	637

8.17.4A.1.1	NR Inter-RAT cell identification	638

8.17.4A.1.2	NR Inter-RAT measurement	639

8.17.4A.1.3	NR Inter-RAT measurement reporting	640

8.17.4A.1.4	NR inter-RAT RSSI measurements	640

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	641

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	641

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	641

8.17.5.1	Introduction	641

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	642

8.17.5.2.1	Identification of a new UTRA FDD cell	642

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	642

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	642

8.17.5.2.4	Periodic Reporting	643

8.17.5.2.5	Event Triggered Reporting	643

8.17.5.2.6	Event-triggered Periodic Reporting	643

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	643

8.17.5.3.1	Periodic Reporting	645

8.17.5.3.2	Event Triggered Reporting	645

8.17.5.3.3	Event-triggered Periodic Reporting	645

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	645

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	646

8.17.7.1	Introduction	646

8.17.7.2	Identification of a new UTRA FDD cell for SON	646

8.17.7.2.1	Requirements when no DRX is used	646

8.17.7.2.2	Requirements when DRX is used	646

8.17.7.2.3	Reporting Delay	647

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	648

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	648

8.17.9.1	Introduction	648

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	648

8.17.9.2.1	Identification of a new UTRA FDD cell	648

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	648

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	649

8.17.9.2.4	Periodic Reporting	649

8.17.9.2.5	Event Triggered Reporting	649

8.17.9.2.6	Event-triggered Periodic Reporting	650

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	650

8.17.9.3.1	Periodic Reporting	651

8.17.9.3.2	Event Triggered Reporting	651

8.17.9.3.3	Event-triggered Periodic Reporting	651

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	652

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	652

8.17.11.1	Introduction	652

8.17.11.2	Identification of a new UTRA TDD cell for SON	652

8.17.11.2.1	Requirements when no DRX is used	652

8.17.11.2.2	Requirements when DRX is used	652

8.17.11.2.3	Reporting Delay	653

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	653

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	654

8.17.13.1	Introduction	654

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	654

8.17.13.2.1	GSM carrier RSSI	654

8.17.13.2.2	BSIC verification	654

8.17.13.2.3	Enhanced BSIC verification	656

8.17.13.2.4	Periodic Reporting	656

8.17.13.2.5	Event Triggered Reporting	656

8.17.13.2.6	Event-triggered Periodic Reporting	657

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	657

8.17.13.3.1	GSM carrier RSSI	657

8.17.13.3.2	BSIC verification	657

8.17.13.3.3	Periodic Reporting	659

8.17.13.3.4	Event Triggered Reporting	659

8.17.13.3.5	Event-triggered Periodic Reporting	659

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	659

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	659

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	659

8.17.15.1.1	RSTD Measurement Reporting Delay	660

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	660

8.17.15.2.1	RSTD Measurement Reporting Delay	660

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	660

8.17.15.3.1	RSTD Measurement Reporting Delay	661

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	661

8.17.15.4.1	RSTD Measurement Reporting Delay	661

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	662

8.17.16.1	Introduction	662

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	662

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	662

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	662

8.17.17.1	Introduction	662

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	662

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	662

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	662

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	662

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	662

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	662

8.18	Measurements for non-BL/CE UE	662

8.18.1	Introduction	662

8.18.2	Requirements for non-BL/CE UE with CE Mode B	663

8.18.2.1	E-UTRAN intra frequency measurements	663

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	663

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	663

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	664

8.19	Measurements for NR – E-UTRA Dual Connectivity	665

8.19.1	Introduction	665

8.19.2	Intra-frequency Measurements	665

8.19.3	Inter-frequency Measurements	665

8.19.4	Void	665

8.19.5	Intra-frequency E-CID Measurements	665

8.19.6	Intra-frequency measurements with autonomous gaps	666

8.19.6.1	Introduction	666

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	666

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	666

8.19.7	Inter-frequency measurements with autonomous gaps	666

8.19.7.1	Introduction	666

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	666

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	666

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	666

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	666

9	Measurements performance requirements for UE	666

9.1	E-UTRAN measurements	666

9.1.1	Introduction	666

9.1.2	Intra-frequency RSRP Accuracy Requirements	667

9.1.2.1	Absolute RSRP Accuracy	667

9.1.2.2	Relative Accuracy of RSRP	667

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	668

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	669

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	670

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	671

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	672

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	673

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	674

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	674

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	674

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	675

9.1.2B.1	Introduction	675

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	675

9.1.3	Inter-frequency RSRP Accuracy Requirements	676

9.1.3.1	Absolute RSRP Accuracy	676

9.1.3.2	Relative Accuracy of RSRP	677

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	678

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	678

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	679

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	679

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	680

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	681

9.1.3B.1	Introduction	681

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	681

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	681

9.1.4	RSRP Measurement Report Mapping	682

9.1.5	Intra-frequency RSRQ Accuracy Requirements	682

9.1.5.1	Absolute RSRQ Accuracy	682

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	683

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	684

9.1.5.4	Absolute WB-RSRQ Accuracy	685

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	686

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	687

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	687

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	687

9.1.5B.1	Introduction	687

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	687

9.1.6	Inter-frequency RSRQ Accuracy Requirements	688

9.1.6.1	Absolute RSRQ Accuracy	688

9.1.6.2	Relative Accuracy of RSRQ	689

9.1.6.3	Absolute WB-RSRQ Accuracy	690

9.1.6.4	Relative WB-RSRQ Accuracy	690

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	691

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	692

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	692

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	692

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	693

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	694

9.1.6B.1	Introduction	694

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	694

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	694

9.1.7	RSRQ Measurement Report Mapping	695

9.1.8	Power Headroom	695

9.1.8.1	Period	696

9.1.8.2	Reporting Delay	696

9.1.8.3	Void	696

9.1.8.4	Report Mapping	696

9.1.9	UE Rx – Tx time difference	696

9.1.9.1	Measurement Requirement	696

9.1.9.2	Measurement Report mapping	697

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	698

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	699

9.1.10	Reference Signal Time Difference (RSTD)	700

9.1.10.1	Intra-Frequency Accuracy Requirement	700

9.1.10.2	Inter-Frequency Accuracy Requirement	701

9.1.10.3	RSTD Measurement Report Mapping	703

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	703

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	704

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	705

9.1.11	Carrier aggregation measurement accuracy	706

9.1.11.1	Primary component carrier accuracy requirement	707

9.1.11.2	Secondary component carrier accuracy requirement	707

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	707

9.1.11.4	Secondary component carrier relative accuracy requirement	707

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	707

9.1.13	Measurement accuracy for UE category 0	708

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	708

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	708

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	709

9.1.14	Accuracy requirements for Discovery Signal Measurements	710

9.1.14.1	Introduction	710

9.1.14.2	RSRP measurements in discovery signal occasions	710

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	710

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	710

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	710

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	711

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	712

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	712

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	712

9.1.14.3.3	CSI-RSRP measurement report mapping	713

9.1.14.4	RSRQ measurements in discovery signal occasions	713

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	713

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	714

9.1.15.1.1	Primary component carrier accuracy requirement	714

9.1.15.1.2	Secondary component carrier accuracy requirement	714

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	714

9.1.15.1.4	Secondary component carrier relative accuracy requirement	714

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	714

9.1.15.2.1	Primary component carrier accuracy requirement	714

9.1.15.2.2	Secondary component carrier accuracy requirement	714

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	714

9.1.15.2.4	Secondary component carrier relative accuracy requirement	714

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	715

9.1.17	RS-SINR Measurements	715

9.1.17.1	Measurement Report Mapping	715

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	716

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	716

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	716

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	716

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	717

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	718

9.1.18.1	Introduction	718

9.1.18.2	RSRP measurements	718

9.1.18.2.1	RSRP measurement report mapping	718

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	718

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	719

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	719

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	720

9.1.18.3	RSRQ measurements	720

9.1.18.3.1	RSRQ measurement report mapping	720

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	720

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	721

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	721

9.1.18.4	CSI-RSRP measurements	722

9.1.18.4.1	CSI-RSRP measurement report mapping	722

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	722

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	722

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	723

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	723

9.1.18.5	RSSI measurements	724

9.1.18.5.1	RSSI measurement report mapping	724

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	724

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	725

9.1.18.6	Channel occupancy measurements	725

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	725

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	725

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	726

9.1.19.1	Introduction	726

9.1.19.2	Accuracy requirements for measurements on SCC	726

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	726

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	726

9.1.20	SFN and Subframe Time Difference (SSTD)	727

9.1.20.1	SSTD Accuracy Requirement	727

9.1.20.2	SSTD Measurement Report Mapping	727

9.1.21	Measurement accuracy for UE category M1	728

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	728

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	730

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	731

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	733

9.1.21.5	RSRP Measurement Report Mapping	734

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	734

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	735

9.1.21.8	RSRQ Measurement Report Mapping	736

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	736

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	737

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	738

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	739

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	740

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	741

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	742

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	743

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	744

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	745

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	746

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	747

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	750

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	753

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	753

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	756

9.1.22	Measurement accuracy for UE Category NB1	757

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	757

9.1.22.2	Void	758

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	758

9.1.22.4	Void	759

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	759

9.1.22.6	Void	760

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	760

9.1.22.8	Void	761

9.1.22.9	NRSRP Measurement Report Mapping	761

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	761

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	762

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	763

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	764

9.1.22.14	NRSRQ Measurement Report Mapping	765

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	766

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	766

9.1.23	Power Headroom for UE Category NB1	767

9.1.23.1	Period	767

9.1.23.2	Reporting Delay	767

9.1.23.3	Report Mapping for UE Category NB1	767

9.1.23.3.1	Void	769

9.1.23.3.2	Void	769

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	769

9.1.24	Void	770

9.1.25	Measurement accuracy for UE category M2	770

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	770

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	771

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	772

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	773

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	775

9.1.26	Measurement Accuracy for non-BL CE UE	776

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	777

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	779

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	779

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	781

9.1.26.5	RSRP Measurement Report Mapping	782

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	782

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	782

9.1.26.8	RSRQ Measurement Report Mapping	783

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	783

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	784

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	785

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	786

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	787

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	787

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	787

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	788

9.1.27	SFN and frame Timing Difference (SFTD)	789

9.1.27.1	SFTD Accuracy Requirement	789

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	791

9.1.28.1	SFTD Accuracy Requirement under CCA	791

9.2	UTRAN FDD Measurements	792

9.2.1	UTRAN FDD CPICH RSCP	792

9.2.2	Void	793

9.2.3	UTRAN FDD CPICH Ec/No	793

9.3	UTRAN TDD Measurements	793

9.3.1	UTRAN TDD P-CCPCH RSCP	794

9.3.2	Void	794

9.3.3	Void	794

9.4	GSM Measurements	794

9.4.1	GSM carrier RSSI	794

9.5	CDMA2000 1x RTT Measurements	794

9.5.1	CDMA2000 1x RTT Pilot Strength	794

9.6	PCMAX,c	795

9.6.1	Report Mapping	795

9.6.2 	Estimation Period	795

9.6.3	Reporting Delay	795

9.7	IEEE802.11 Measurements	795

9.7.1	WLAN RSSI	795

9.7.2	WLAN RSSI Measurement Report Mapping	795

9.8	MBSFN Measurements	796

9.8.1	Introduction	796

9.8.2	MBSFN RSRP	796

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	796

9.8.2.2	MBSFN RSRP measurement report mapping	797

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	797

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	798

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	798

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	798

9.8.3	MBSFN RSRQ	799

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	799

9.8.3.2	MBSFN RSRQ measurement report mapping	799

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	800

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	800

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	800

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	801

9.8.4	MCH BLER	801

9.8.4.1	Measurement report mapping for MCH BLER	801

9.8.4.2	Measurement report mapping for MCH Block Number	802

9.9	ProSe Measurements	803

9.9.1	Introduction	803

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	803

9.9.2.1	Absolute S-RSRP Accuracy	803

9.9.2.2	Relative Accuracy of S-RSRP	804

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	805

9.9.3.1	Absolute SD-RSRP Accuracy	805

9.9.3.2	Relative Accuracy of SD-RSRP	805

9.10	V2X Measurements	806

9.10.1	Introduction	806

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	806

9.10.2.1	Absolute S-RSRP Accuracy	806

9.10.2.2	Relative Accuracy of S-RSRP	807

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	807

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	807

9.10.4	S-RSSI Measurement Accuracy Requirements	808

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	808

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	808

9.11	NR Measurements	809

9.11.1	NR SS-RSRP Measurements	809

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	809

9.11.2	NR SS-RSRQ Measurements	810

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	810

9.11.3	NR SS-SINR Measurements	810

9.11.4	NR SS-RSRP Measurements under CCA	811

9.11.5	NR SS-RSRQ Measurements under CCA	811

9.11.6	NR SS-SINR Measurements under CCA	811

9.11.7	NR RSSI Measurements under CCA	811

9.11.8	NR Channel Occupancy Measurements under CCA	812

10	Measurements Performance Requirements for E-UTRAN	812

10.1	Received Interference Power	812

10.1.1	Absolute accuracy requirement	812

10.1.2	Relative accuracy requirement	812

10.1.3	Received Interference Power measurement report mapping	813

10.2	Angle of Arrival (AOA)	813

10.2.1	Range/mapping	813

10.3	Timing Advance (TADV)	813

10.3.1	Report mapping	813

11	ProSe Requirements in Any Cell Selection state	814

11.1	Introduction	814

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	814

11.2.1	Introduction	814

11.2.2	ProSe UE transmission timing	814

11.3	Initiation/Cease of SLSS Transmissions	815

11.3.1	Introduction	815

11.3.2	Requirements	815

11.4	Measurements for ProSe in Any Cell Selection State	815

11.4.1	Introduction	815

11.4.2	Requirements	815

11.4.2.1	E-UTRA FDD	815

11.4.2.2	E-UTRA TDD	816

11.5	Selection / Reselection of ProSe Synchronization Reference	816

11.5.1	Introduction	816

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	816

11.5.2.1	Introduction	816

11.5.2.2	Requirements	816

11.6	Void	817

11.7	Selection / Reselection of ProSe relay UE	817

11.7.1	Introduction	817

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	817

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	817

12.1	Introduction	817

12.2	Transmit Timing	818

12.2.1	GNSS as timing reference	818

12.3	Interruption	818

12.4	Reliability of GNSS signal	818

13	V2X Requirements	818

13.1	Introduction	818

13.2	UE Transmit Timing	819

13.2.1	Introduction	819

13.2.2	GNSS as synchronization reference source	819

13.2.3	Serving cell/PCell as synchronization reference source	819

13.2.4	SyncRef UE as synchronization reference source	819

13.3	Initiation/Cease of SLSS Transmissions	819

13.3.1	Introduction	819

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	819

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	820

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	820

13.4	Selection / Reselection of V2X Synchronization Reference Source	821

13.5	Autonomous Resource Selection/Reselection measurements	822

13.5.1	Introduction	822

13.5.2	PSSCH-RSRP measurements	822

13.5.3	S-RSSI measurements	822

13.6	Congestion Control measurements	822

13.7	Interruption	822

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	822

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	822

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	823

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	823

13.8	Reliability of GNSS signal	823

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	824

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	824

Annex A (normative): Test Cases	818

A.1	Purpose of annex	818

A.2	Requirement classification for statistical testing	818

A.2.1	Types of requirements in TS 36.133	818

A.2.1.1	Time and delay requirements on UE higher layer actions	818

A.2.1.2	Measurements of power levels, relative powers and time	818

A.2.1.3	Implementation requirements	819

A.2.1.4	Physical layer timing requirements	819

A.3	RRM test configurations	820

A.3.1	Reference Measurement Channels	820

A.3.1.1	PDSCH	820

A.3.1.1.1	FDD	820

A.3.1.1.2	TDD	825

A.3.1.1.3	FDD for UE category 0	828

A.3.1.1.4	HD-FDD for UE category 0	829

A.3.1.1.5	TDD for UE category 0	830

A.3.1.1.6	Frame Structure 3	831

A.3.1.2	PCFICH/PDCCH/PHICH	832

A.3.1.2.1	FDD	832

A.3.1.2.2	TDD	832

A.3.1.2.3	HD-FDD for UE category 0	833

A.3.1.2.4	FS 3	833

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	833

A.3.1.3.1	FDD in CEModeA	834

A.3.1.3.2	HD-FDD in CEModeA	834

A.3.1.3.3	TDD in CEModeA	835

A.3.1.3.4	FDD in CEModeB	835

A.3.1.3.5	HD-FDD in CEModeB	836

A.3.1.3.6	TDD in CEModeB	836

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	837

A.3.1.4.1	FDD in CEModeA	837

A.3.1.4.2	HD-FDD in CEModeA	838

A.3.1.4.3	TDD in CEModeA	839

A.3.1.4.4	FDD in CEModeB	840

A.3.1.4.5	 HD-FDD in CEModeB	841

A.3.1.4.6	TDD in CEModeB	841

A.3.1.5	NPDSCH Reference Channel for UE category NB1	842

A.3.1.5.1	HD-FDD in-band operation	842

A.3.1.5.2	Void	843

A.3.1.5.3	HD-FDD standalone operation	843

A.3.1.5.4	Void	844

A.3.1.5.5	HD-FDD guard band operation	844

A.3.1.5.6	Void	845

A.3.1.5.7	TDD in-band operation	845

A.3.1.5.8	TDD standalone operation	845

A.3.1.5.9	TDD guard band operation	846

A.3.1.6	NPDCCH Reference Channel for UE category NB1	846

A.3.1.6.1	In-band operation	846

A.3.1.6.2	Void	847

A.3.1.6.3	Standalone operation	847

A.3.1.6.4	Void	847

A.3.1.6.5	Guard band operation	847

A.3.1.6.6	Void	848

A.3.2	OFDMA Channel Noise Generator (OCNG)	848

A.3.2.1	OCNG Patterns for FDD	848

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	849

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	849

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	850

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	850

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	851

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	852

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	852

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	852

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	853

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	854

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	854

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	855

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	855

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	856

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	856

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	857

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	857

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	858

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	858

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	859

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	859

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	860

A.3.2.2	OCNG Patterns for TDD	860

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	861

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	861

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	862

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	862

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	863

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	864

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	865

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	866

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	866

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	867

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	867

A.3.2.3	OCNG Patterns for Narrowband IoT	868

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	869

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	870

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	870

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	871

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	872

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	873

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	874

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	875

A.3.2.4	OCNG Patterns for V2X sidelink	876

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	876

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	877

A.3.3	Reference DRX Configurations	877

A.3.4	ABS Transmission Configurations	878

A.3.4.1	Non-MBSFN ABS Transmission Configurations	878

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	878

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	878

A.3.4.2	MBSFN ABS Transmission Configurations	879

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	879

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	880

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	880

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	880

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	881

A.3.6.1	Introduction	881

A.3.7	Test Cases with Different Channel Bandwidths	881

A.3.7.1	Introduction	881

A.3.7.2	Principle of testing	881

A.3.8	Antenna Configuration	881

A.3.8.1	Antenna connection for 4 Rx capable UEs	881

A.3.8.1.1 Introduction	881

A.3.8.1.2 Principle of testing	881

A.3.8.1.2.1 Single carrier tests	881

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	882

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	883

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	883

A.3.8.2	Antenna connection for 8 Rx capable UEs	883

A.3.8.2.1	Introduction	883

A.3.8.2.2	Principle of testing	883

A.3.8.2.2.1	Single carrier tests	883

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	884

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	884

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	884

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	884

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	884

A.3.9.1	Introduction	884

A.3.9.2	Principle of testing	884

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	885

A.3.10.1	Introduction	885

A.3.10.2	Principle of testing	885

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	885

A.3.11.1	Introduction	885

A.3.11.2	Principle of Testing	885

A.3.12	Proximity-based Services	885

A.3.12.1	Introduction	885

A.3.12.2	Reference DRX configurations for ProSe tests	885

A.3.12.3	Test Cases with Different Channel Bandwidths	886

A.3.12.3.1	Introduction	886

A.3.12.3.2	Principle of testing	886

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	886

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	893

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	896

A.3.12.6.1	FDD	896

A.3.12.7	Reference measurement channels for ProSe Direct Communication	896

A.3.12.7.1	FDD	896

A.3.12.8	ProSe Receive Traffic Generator	897

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	897

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	897

A.3.13	Time Offset between Cells	898

A.3.13.1	Introduction	898

A.3.13.2	Definition	898

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	898

A.3.14.1	Introduction	898

A.3.14.2	Principle of testing	898

A.3.15	Dual connectivity test cases with different combination of duplex mode	898

A.3.15.1	Introduction	898

A.3.15.2	Principle of testing	898

A.3.16	Reference PRACH Configurations	899

A.3.17	Listen before talk model	899

A.3.17.1	Introduction	899

A.3.17.2	Definition	899

A.3.18	Reference NPRACH Configurations	900

A.3.19	Dual connectivity test cases with different bandwidth combinations	901

A.3.19.1	Introduction	901

A.3.19.2	Principle of testing	901

A.3.20	Category M1 UE Test Cases	902

A.3.20.1	Introduction	902

A.3.20.2	Principle of Cat-M1 UE Testing	902

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	903

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	904

A.3.21.1	Introduction	904

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	904

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	905

A.3.22	Category 1bis UE Test Cases	906

A.3.22.1	Introduction	906

A.3.22.2	Principle of Category 1bis UE Testing	906

A.3.23	Category NB2 UE Test Cases	910

A.3.23.1	Introduction	910

A.3.23.2	Principle of Category NB2 UE Testing	910

A.3.24	V2X sidelink communication	913

A.3.24.1	Introduction	913

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	914

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	918

A.3.25	Category M2 UE Test Cases	919

A.3.25.1	Introduction	919

A.3.25.2	Principle of Cat-M2 UE Testing	919

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	920

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	921

A.3.26.1	Introduction	921

A.3.26.2	Principle of testing	921

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	921

A.3.27.1	Introduction	921

A.3.27.2	Principle of INACTIVE cell re-selection Testing	921

A.4	E-UTRAN RRC_IDLE state	921

A.4.2	Cell Re-Selection	921

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	921

A.4.2.1.1	Test Purpose and Environment	921

A.4.2.1.2	Test Requirements	923

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	924

A.4.2.2.1	Test Purpose and Environment	924

A.4.2.2.2	Test Requirements	925

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	926

A.4.2.3.1	Test Purpose and Environment	926

A.4.2.3.2	Test Requirements	927

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	928

A.4.2.4.1	Test Purpose and Environment	928

A.4.2.4.2	Test Requirements	929

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	930

A.4.2.5.1	Test Purpose and Environment	930

A.4.2.5.2	Test Requirements	931

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	932

A.4.2.6.1	Test Purpose and Environment	932

A.4.2.6.2	Test Requirements	933

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	934

A.4.2.7.1	Test Purpose and Environment	934

A.4.2.7.2	Test Requirements	935

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	936

A.4.2.8.1	Test Purpose and Environment	936

A.4.2.8.2	Test Requirements	937

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	938

A.4.2.9.1	Test Purpose and Environment	938

A.4.2.9.2	Test Requirements	938

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	938

A.4.2.10.1	Test Purpose and Environment	938

A.4.2.10.2	Test Requirements	942

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	942

A.4.2.11.1	Test Purpose and Environment	942

A.4.2.11.2	Test Requirements	946

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	946

A.4.2.12.1	Test Purpose and Environment	946

A.4.2.12.2	Test Requirements	948

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	949

A.4.2.13.1	Test Purpose and Environment	949

A.4.2.13.2	Test Requirements	950

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	951

A.4.2.14.1	Test Purpose and Environment	951

A.4.2.14.2	Test Requirements	952

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	953

A.4.2.15.1	Test Purpose and Environment	953

A.4.2.15.2	Test Requirements	954

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	955

A.4.2.16.1	Test Purpose and Environment	955

A.4.2.16.2	Test Requirements	956

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	957

A.4.2.17.1	Test Purpose and Environment	957

A.4.2.17.2	Test Requirements	958

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	959

A.4.2.18.1	Test Purpose and Environment	959

A.4.2.18.2	Test Requirements	961

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	962

A.4.2.19.1	Test Purpose and Environment	962

A.4.2.19.2	Test Requirements	964

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	965

A.4.2.20.1	Test Purpose and Environment	965

A.4.2.20.2	Test Requirements	966

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	967

A.4.2.21.1	Test Purpose and Environment	967

A.4.2.21.2	Test Requirements	968

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	969

A.4.2.22.1	Test Purpose and Environment	969

A.4.2.22.2	Test Requirements	970

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	971

A.4.2.23.1	Test Purpose and Environment	971

A.4.2.23.2	Test Requirements	972

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	973

A.4.2.24.1	Test Purpose and Environment	973

A.4.2.24.2	Test Requirements	976

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	977

A.4.2.25.1	Test Purpose and Environment	977

A.4.2.25.2	Test Requirements	978

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	979

A.4.2.26.1	Test Purpose and Environment	979

A.4.2.26.2	Test Requirements	980

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	981

A.4.2.27.1	Test Purpose and Environment	981

A.4.2.27.2	Test Requirements	982

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	983

A.4.2.28.1	Test Purpose and Environment	983

A.4.2.28.2	Test Requirements	984

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	985

A.4.2.29.1	Test Purpose and Environment	985

A.4.2.29.2	Test Requirements	986

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	987

A.4.2.30.1	Test Purpose and Environment	987

A.4.2.30.2	Test Requirements	988

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	989

A.4.2.31.1	Test Purpose and Environment	989

A.4.2.31.2	Test Requirements	990

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	991

A.4.2.32.1	Test Purpose and Environment	991

A.4.2.32.2	Test Requirements	992

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	993

A.4.2.33.1	Test Purpose and Environment	993

A.4.2.33.2	Test Requirements	994

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	995

A.4.2.34.1	Test Purpose and Environment	995

A.4.2.34.2	Test Requirements	996

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	997

A.4.2.35.1	Test Purpose and Environment	997

A.4.2.35.2	Test Requirements	999

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1000

A.4.2.36.1	Test Purpose and Environment	1000

A.4.2.36.2	Test Requirements	1002

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1003

A.4.2.37.1	Test Purpose and Environment	1003

A.4.2.37.2	Test Requirements	1005

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1006

A.4.2.38.1	Test Purpose and Environment	1006

A.4.2.38.2	Test Requirements	1008

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1009

A.4.2.39.1	Test Purpose and Environment	1009

A.4.2.39.2	Test Requirements	1010

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1011

A.4.2.40.1	Test Purpose and Environment	1011

A.4.2.40.2	Test Requirements	1012

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1013

A.4.2.41.1	Test Purpose and Environment	1013

A.4.2.41.2	Test Requirements	1016

A.4.2.42.2	Test Requirements	1020

A.4.2.43.2	Test Requirements	1023

A.4.2.44.2	Test Requirements	1027

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1027

A.4.2.45.1	Test Purpose and Environment	1027

A.4.2.45.2	Test Requirements	1030

A.4.2.46.2	Test Requirements	1034

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1034

A.4.2.47.1	Test Purpose and Environment	1034

A.4.2.47.2	Test Requirements	1038

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1038

A.4.2.48.1	Test Purpose and Environment	1038

A.4.2.48.2	Test Requirements	1040

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1041

A.4.2.49.1	Test Purpose and Environment	1041

A.4.2.49.2	Test Requirements	1043

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1044

A.4.2.50.1	Test Purpose and Environment	1044

A.4.2.50.2	Test Requirements	1046

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1047

A.4.2.51.1	Test Purpose and Environment	1047

A.4.2.51.2	Test Requirements	1049

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1050

A.4.2.52.1	Test Purpose and Environment	1050

A.4.2.52.2	Test Requirements	1052

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1053

A.4.2.53.1	Test Purpose and Environment	1053

A.4.2.53.2	Test Requirements	1055

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1056

A.4.2.54.1	Test Purpose and Environment	1056

A.4.2.54.2	Test Requirements	1058

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1059

A.4.2.55.1	Test Purpose and Environment	1059

A.4.2.55.2	Test Requirements	1061

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1062

A.4.2.56.1	Test Purpose and Environment	1062

A.4.2.56.2	Test Requirements	1063

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1064

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1064

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1064

A.4.3.1.1.1	Test Purpose and Environment	1064

A.4.3.1.1.2	Test Requirements	1066

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1066

A.4.3.1.2.1	Test Purpose and Environment	1066

A.4.3.1.2.2	Test Requirements	1069

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1069

A.4.3.1.3.1	Test Purpose and Environment	1069

A.4.3.1.3.2	Test Requirements	1072

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1072

A.4.3.1.4.1	Test Purpose and Environment	1072

A.4.3.1.4.2	Test Requirements	1073

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1073

A.4.3.1.5.1	Test Purpose and Environment	1073

A.4.3.1.5.2	Test Requirements	1076

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1077

A.4.3.2.1	Test Purpose and Environment	1077

A.4.3.2.1.1	Void	1077

A.4.3.2.1.2	1.28Mcps TDD option	1077

A.4.3.2.1.3	Void	1079

A.4.3.2.2	Test Requirements	1079

A.4.3.2.2.1	1.28Mcps TDD option	1079

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1079

A.4.3.2A.1	Test Purpose and Environment	1079

A.4.3.2A.2	Test Requirements	1083

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1083

A.4.3.3.1	Test Purpose and Environment	1083

A.4.3.3.2	Test Requirements	1086

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1086

A.4.3.3A.1	Test Purpose and Environment	1086

A.4.3.3A.2	Test Requirements	1091

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1092

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1092

A.4.3.4.1.1	Test Purpose and Environment	1092

A.4.3.4.1.2	Test Requirements	1094

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1094

A.4.3.4.2.1	Test Purpose and Environment	1094

A.4.3.4.2.2	Test Requirements	1096

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1096

A.4.3.4.3.1	Test Purpose and Environment	1096

A.4.3.4.3.2	Test Requirements	1099

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1099

A.4.3.4.4.1	Test Purpose and Environment	1099

A.4.3.4.4.2	Test Requirements	1103

A.4.4	E-UTRAN to GSM Cell Re-Selection	1103

A.4.4.1	E-UTRAN FDD – GSM:	1103

A.4.4.1.1	Test Purpose and Environment	1103

A.4.4.1.2	Test Requirements	1105

A.4.4.2	E-UTRAN TDD – GSM:	1105

A.4.4.2.1	Test Purpose and Environment	1105

A.4.4.2.2	Test Requirements	1107

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1108

A.4.5.1	E-UTRAN FDD – HRPD	1108

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1108

A.4.5.1.1.1	Test Purpose and Environment	1108

A.4.5.1.1.2	Test Requirements	1110

A.4.5.2	E-UTRAN TDD – HRPD	1110

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1110

A.4.5.2.1.1	Test Purpose and Environment	1110

A.4.5.2.1.2	Test Requirements	1113

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1113

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1113

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1113

A.4.6.1.1.1	Test Purpose and Environment	1113

A.4.6.1.1.2	Test Requirements	1116

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1116

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1116

A.4.6.2.1.1	Test Purpose and Environment	1116

A.4.6.2.1.2	Test Requirements	1119

A.4.7	Idle State Positioning Measurement for UE category NB1	1119

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1119

A.4.7.1.1	Test Purpose and Environment	1119

A.4.7.1.2	Test Requirements	1124

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1125

A.4.7.2.1	Test Purpose and Environment	1125

A.4.7.4.2	Test Requirements	1129

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1130

A.4.7.3.1	Test Purpose and Environment	1130

A.4.7.3.2	Test Requirements	1134

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1135

A.4.7.4.1	Test Purpose and Environment	1135

A.4.7.4.2	Test Requirements	1139

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1140

A.5.1	E-UTRAN Handover	1140

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1140

A.5.1.1.1	Test Purpose and Environment	1140

A.5.1.1.2	Test Requirements	1142

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1142

A.5.1.2.1	Test Purpose and Environment	1142

A.5.1.2.2	Test Requirements	1144

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1144

A.5.1.3.1	Test Purpose and Environment	1144

A.5.1.3.2	Test Requirements	1146

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1146

A.5.1.4.1	Test Purpose and Environment	1146

A.5.1.4.2	Test Requirements	1148

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1148

A.5.1.5.1	Test Purpose and Environment	1148

A.5.1.5.2	Test Requirements	1150

A.5.1.6	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1150

A.5.1.6.1	Test Purpose and Environment	1150

A.5.1.6.2	Test Requirements	1152

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1152

A.5.1.7.1	Test Purpose and Environment	1152

A.5.1.7.2	Test Requirements	1155

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1155

A.5.1.8.1	Test Purpose and Environment	1155

A.5.1.8.2	Test Requirements	1158

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1158

A.5.1.9.1	Test Purpose and Environment	1158

A.5.1.9.2	Test Requirements	1159

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1159

A.5.1.10.1	Test Purpose and Environment	1159

A.5.1.10.2	Test Requirements	1161

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1161

A.5.1.11.1	Test Purpose and Environment	1161

A.5.1.11.2	Test Requirements	1163

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1163

A.5.1.12.1	Test Purpose and Environment	1163

A.5.1.12.2	Test Requirements	1165

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1165

A.5.1.13.1	Test Purpose and Environment	1165

A.5.1.13.2	Test Requirements	1167

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1168

A.5.1.14.1	Test Purpose and Environment	1168

A.5.1.14.2	Test Requirements	1169

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1170

A.5.1.15.1	Test Purpose and Environment	1170

A.5.1.15.2	Test Requirements	1171

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1172

A.5.1.16.1	Test Purpose and Environment	1172

A.5.1.16.2	Test Requirements	1173

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1174

A.5.1.17.1	Test Purpose and Environment	1174

A.5.1.17.2	Test Requirements	1175

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1176

A.5.1.18.1	Test Purpose and Environment	1176

A.5.1.18.2	Test Requirements	1177

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1178

A.5.1.19.1	Test Purpose and Environment	1178

A.5.1.19.2	Test Requirements	1179

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1180

A.5.1.20.1	Test Purpose and Environment	1180

A.5.1.20.2	Test Requirements	1181

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1182

A.5.1.21.1	Test Purpose and Environment	1182

A.5.1.21.2	Test Requirements	1183

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1183

A.5.1.22.1	Test Purpose and Environment	1183

A.5.1.22.2	Test Requirements	1185

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1185

A.5.1.23.1	Test Purpose and Environment	1185

A.5.1.23.2	Test Requirements	1187

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1187

A.5.1.24.1	Test Purpose and Environment	1187

A.5.1.24.2	Test Requirements	1189

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1189

A.5.1.25.1	Test Purpose and Environment	1189

A.5.1.25.2	Test Requirements	1191

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1192

A.5.1.26.1	Test Purpose and Environment	1192

A.5.1.26.2	Test Requirements	1193

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1194

A.5.1.27.1	Test Purpose and Environment	1194

A.5.1.27.2	Test Requirements	1195

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1196

A.5.1.28.1	Test Purpose and Environment	1196

A.5.1.28.2	Test Requirements	1197

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1198

A.5.1.29.1	Test Purpose and Environment	1198

A.5.1.29.2	Test Requirements	1199

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1200

A.5.1.30.1	Test Purpose and Environment	1200

A.5.1.30.2	Test Requirements	1201

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1202

A.5.1.31.1	Test Purpose and Environment	1202

A.5.1.31.2	Test Requirements	1203

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1204

A.5.1.32.1	Test Purpose and Environment	1204

A.5.1.32.2	Test Requirements	1205

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1206

A.5.1.33.1	Test Purpose and Environment	1206

A.5.1.33.2	Test Requirements	1207

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1208

A.5.1.34.1	Test Purpose and Environment	1208

A.5.1.34.2	Test Requirements	1209

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1210

A.5.1.35.1	Test Purpose and Environment	1210

A.5.1.35.2	Test Requirements	1211

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1212

A.5.1.36.1	Test Purpose and Environment	1212

A.5.1.36.2	Test Requirements	1213

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1214

A.5.1.37.1	Test Purpose and Environment	1214

A.5.1.37.2	Test Requirements	1215

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1216

A.5.1.38.1	Test Purpose and Environment	1216

A.5.1.38.2	Test Requirements	1217

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1218

A.5.1.39.1	Test Purpose and Environment	1218

A.5.1.39.2	Test Requirements	1219

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1220

A.5.1.40.1	Test Purpose and Environment	1220

A.5.1.40.2	Test Requirements	1221

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1222

A.5.1.41.1	Test Purpose and Environment	1222

A.5.1.41.2	Test Requirements	1224

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1225

A.5.1.42.1	Test Purpose and Environment	1225

A.5.1.42.2	Test Requirements	1226

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1227

A.5.1.43.1	Test Purpose and Environment	1227

A.5.1.43.2	Test Requirements	1228

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1229

A.5.1.44.1	Test Purpose and Environment	1229

A.5.1.44.2	Test Requirements	1230

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1231

A.5.1.45.1	Test Purpose and Environment	1231

A.5.1.45.1	Test Requirements	1232

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1233

A.5.1.46.1	Test Purpose and Environment	1233

A.5.1.46.2	Test Requirements	1234

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1235

A.5.1.47.1	Test Purpose and Environment	1235

A.5.1.47.2	Test Requirements	1236

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1237

A.5.1.48.1	Test Purpose and Environment	1237

A.5.1.48.2	Test Requirements	1238

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1239

A.5.1.49.1	Test Purpose and Environment	1239

A.5.1.49.2	Test Requirements	1240

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1241

A.5.1.50.1	Test Purpose and Environment	1241

A.5.1.50.2	Test Requirements	1242

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1243

A.5.1.51.1	Test Purpose and Environment	1243

A.5.1.51.2	Test Requirements	1245

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1245

A.5.152.1	Test Purpose and Environment	1245

A.5.1.52.2	Test Requirements	1248

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1249

A.5.1.53.1	Test Purpose and Environment	1249

A.5.1.53.2	Test Requirements	1251

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1252

A.5.1.54.1	Test Purpose and Environment	1252

A.5.1.54.2	Test Requirements	1254

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1255

A.5.1.55.1	Test Purpose and Environment	1255

A.5.1.55.2	Test Requirements	1258

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1259

A.5.1.56.1	Test Purpose and Environment	1259

A.5.1.56.2	Test Requirements	1262

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1263

A.5.1.57.1	Test Purpose and Environment	1263

A.5.1.57.2	Test Requirements	1265

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1265

A.5.1.58.1	Test Purpose and Environment	1265

A.5.1.58.2	Test Requirements	1268

A.5.2	E-UTRAN Handover to other RATs	1268

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1268

A.5.2.1.1	Test Purpose and Environment	1268

A.5.2.1.2	Test Requirements	1270

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1271

A.5.2.2.1	Test Purpose and Environment	1271

A.5.2.2.2	Test Requirements	1274

A.5.2.3 E-UTRAN FDD- GSM Handover	1274

A.5.2.3.1	Test Purpose and Environment	1274

A.5.2.3.2	Test Requirements	1275

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1276

A.5.2.4.1	Test Purpose and Environment	1276

A.5.2.4.1.1	Void	1276

A.5.2.4.1.2	1.28 Mcps TDD option	1276

A.5.2.4.1.3	Void	1278

A.5.2.4.2	Test Requirements	1278

A.5.2.4.2.1	Void	1278

A.5.2.4.2.2	1.28 Mcps TDD option	1278

A.5.2.4.2.3	Void	1278

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1278

A.5.2.5.1	Test Purpose and Environment	1278

A.5.2.5.1.3	Void	1281

A.5.2.5.2	Test Requirements	1281

A.5.2.5.2.1	Void	1281

A.5.2.5.2.2	1.28 Mcps TDD option	1281

A.5.2.5.2.3	Void	1281

A.5.2.6	E-UTRAN TDD - GSM Handover	1281

A.5.2.6.1	Test Purpose and Environment	1281

A.5.2.6.2	Test Requirements	1283

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1284

A.5.2.7.1	Test Purpose and Environment	1284

A.5.2.7.2	Test Requirements	1286

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1286

A.5.2.8.1	Test Purpose and Environment	1286

A.5.2.8.2	Test Requirements	1287

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1288

A.5.2.9.1	Test Purpose and Environment	1288

A.5.2.9.2	Test Requirements	1289

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1290

A.5.2.10.1	Test Purpose and Environment	1290

A.5.2.10.2	Test Requirements	1292

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1292

A.5.2.10A.1	Test Purpose and Environment	1292

A.5.2.10A.2	Test Requirements	1295

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1295

A.5.2.10B.1	Test Purpose and Environment	1295

A.5.2.10B.2	Test Requirements	1298

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1298

A.5.2.11.1	Test Purpose and Environment	1298

A.5.2.11.2	Test Requirements	1298

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1299

A.5.3.1	E-UTRAN FDD – HRPD Handover	1299

A.5.3.1.1	Test Purpose and Environment	1299

A.5.3.1.2	Test Requirements	1301

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1301

A.5.3.2.1	Test Purpose and Environment	1301

A.5.3.2.2	Test Requirements	1304

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1304

A.5.3.3.1	Test Purpose and Environment	1304

A.5.3.3.2	Test Requirements	1306

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1307

A.5.3.4.1	Test Purpose and Environment	1307

A.5.3.4.2	Test Requirements	1308

A.5.3.5	E-UTRAN TDD – HRPD Handover	1309

A.5.3.5.1	Test Purpose and Environment	1309

A.5.3.5.2	Test Requirements	1311

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1312

A.5.3.6.1	Test Purpose and Environment	1312

A.5.3.6.2	Test Requirements	1315

A.6	RRC Connection Control	1315

A.6.1	RRC Re-establishment	1315

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1315

A.6.1.1.1	Test Purpose and Environment	1315

A.6.1.1.2	Test Requirements	1316

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1317

A.6.1.2.1	Test Purpose and Environment	1317

A.6.1.2.2	Test Requirements	1318

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1319

A.6.1.3.1	Test Purpose and Environment	1319

A.6.1.3.2	Test Requirements	1320

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1321

A.6.1.4.1	Test Purpose and Environment	1321

A.6.1.4.2	Test Requirements	1322

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1323

A.6.1.5.1	Test Purpose and Environment	1323

A.6.1.5.2	Test Requirements	1323

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1323

A.6.1.6.1	Test Purpose and Environment	1323

A.6.1.6.2	Test Requirements	1325

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1326

A.6.1.7.1	Test Purpose and Environment	1326

A.6.1.7.2	Test Requirements	1327

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1328

A.6.1.8.1	Test Purpose and Environment	1328

A.6.1.8.2	Test Requirements	1329

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1330

A.6.1.9.1	Test Purpose and Environment	1330

A.6.1.9.2	Test Requirements	1331

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1332

A.6.1.10.1	Test Purpose and Environment	1332

A.6.1.10.2	Test Requirements	1333

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1334

A.6.1.11.1	Test Purpose and Environment	1334

A.6.1.11.2	Test Requirements	1335

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1336

A.6.1.12.1	Test Purpose and Environment	1336

A.6.1.12.2	Test Requirements	1337

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1338

A.6.1.13.1	Test Purpose and Environment	1338

A.6.1.13.2	Test Requirements	1339

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1340

A.6.1.14.1	Test Purpose and Environment	1340

A.6.1.14.2	Test Requirements	1341

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1342

A.6.1.15.1	Test Purpose and Environment	1342

A.6.1.15.2	Test Requirements	1344

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1345

A.6.1.16.1	Test Purpose and Environment	1345

A.6.1.16.2	Test Requirements	1347

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1348

A.6.1.17.1	Test Purpose and Environment	1348

A.6.1.17.2	Test Requirements	1349

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1350

A.6.1.18.1	Test Purpose and Environment	1350

A.6.1.18.2	Test Requirements	1351

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1352

A.6.1.19.1	Test Purpose and Environment	1352

A.6.1.19.2	Test Requirements	1353

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1354

A.6.1.20.1	Test Purpose and Environment	1354

A.6.1.20.2	Test Requirements	1355

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1356

A.6.1.21.1	Test Purpose and Environment	1356

A.6.1.21.2	Test Requirements	1357

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1358

A.6.1.22.1	Test Purpose and Environment	1358

A.6.1.22.2	Test Requirements	1359

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1360

A.6.1.23.1	Test Purpose and Environment	1360

A.6.1.23.2	Test Requirements	1362

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1363

A.6.1.24.1	Test Purpose and Environment	1363

A.6.1.24.2	Test Requirements	1365

A.6.2	Random Access	1366

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1366

A.6.2.1.1	Test Purpose and Environment	1366

A.6.2.1.2	Test Requirements	1367

A.6.2.1.2.1	Random Access Response Reception	1367

A.6.2.1.2.2	No Random Access Response Reception	1367

A.6.2.1.2.3	Receiving a NACK on msg3	1367

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1368

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1368

A.6.2.1.2.6	Contention Resolution Timer expiry	1368

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1368

A.6.2.2.1	Test Purpose and Environment	1368

A.6.2.2.2	Test Requirements	1369

A.6.2.2.2.1	Random Access Response Reception	1370

A.6.2.2.2.2	No Random Access Response Reception	1370

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1370

A.6.2.3.1	Test Purpose and Environment	1370

A.6.2.3.2	Test Requirements	1372

A.6.2.3.2.1	Random Access Response Reception	1372

A.6.2.3.2.2	No Random Access Response reception	1372

A.6.2.3.2.3	Receiving a NACK on msg3	1372

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1373

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1373

A.6.2.3.2.6	Contention Resolution Timer expiry	1373

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1373

A.6.2.4.1	Test Purpose and Environment	1373

A.6.2.4.2	Test Requirements	1375

A.6.2.4.2.1	Random Access Response Reception	1375

A.6.2.4.2.2	No Random Access Response Reception	1375

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1375

A.6.2.5.1	Test Purpose and Environment	1375

A.6.2.5.2	Test Requirements	1376

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1376

A.6.2.6.1	Test Purpose and Environment	1376

A.6.2.6.2	Test Requirements	1376

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1377

A.6.2.7.1	Test Purpose and Environment	1377

A.6.2.7.2	Test Requirements	1379

A.6.2.7.2.1	Random Access Response Reception	1379

A.6.2.7.2.2	No Random Access Response Reception	1379

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1379

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1380

A.6.2.8.1	Test Purpose and Environment	1380

A.6.2.8.2	Test Requirements	1382

A.6.2.8.2.1	Random Access Response Reception	1382

A.6.2.8.2.2	No Random Access Response Reception	1382

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1382

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1383

A.6.2.9.1	Test Purpose and Environment	1383

A.6.2.9.2	Test Requirements	1386

A.6.2.9.2.1	Random Access Response Reception	1386

A.6.2.9.2.2	No Random Access Response Reception	1387

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1387

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1388

A.6.2.10.1	Test Purpose and Environment	1388

A.6.2.10.2	Test Requirements	1390

A.6.2.10.2.1	Random Access Response Reception	1390

A.6.2.10.2.2	No Random Access Response Reception	1391

A.6.2.10.2.3	Receiving a NACK on msg3	1391

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1391

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1391

A.6.2.10.2.6	Contention Resolution Timer expiry	1391

A.6.2.10.2.7	PRACH Resource Selection	1392

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1392

A.6.2.11.1	Test Purpose and Environment	1392

A.6.2.11.2	Test Requirements	1394

A.6.2.11.2.1	Random Access Response Reception	1394

A.6.2.11.2.2	No Random Access Response Reception	1395

A.6.2.11.2.3	Receiving a NACK on msg3	1395

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1395

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1395

A.6.2.11.2.6	Contention Resolution Timer expiry	1395

A.6.2.11.2.7	PRACH Resource Selection	1396

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1396

A.6.2.12.1	Test Purpose and Environment	1396

A.6.2.12.2	Test Requirements	1398

A.6.2.12.2.1	Random Access Response Reception	1398

A.6.2.12.2.2	No Random Access Response Reception	1399

A.6.2.12.2.3	Receiving a NACK on msg3	1399

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1399

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1399

A.6.2.12.2.6	Contention Resolution Timer expiry	1399

A.6.2.12.2.7	PRACH Resource Selection	1400

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1400

A.6.2.13.1	Test Purpose and Environment	1400

A.6.2.13.2	Test Requirements	1402

A.6.2.13.2.1	Random Access Response Reception	1402

A.6.2.13.2.2	No Random Access Response Reception	1403

A.6.2.13.2.3	Receiving a NACK on msg3	1403

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1403

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1403

A.6.2.13.2.6	Contention Resolution Timer expiry	1403

A.6.2.13.2.7	PRACH Resource Selection	1404

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1404

A.6.2.14.1	Test Purpose and Environment	1404

A.6.2.14.2	Test Requirements	1406

A.6.2.14.2.1	Random Access Response Reception	1406

A.6.2.14.2.2	No Random Access Response Reception	1407

A.6.2.14.2.3	Receiving a NACK on msg3	1407

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1407

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1407

A.6.2.14.2.6	Contention Resolution Timer expiry	1407

A.6.2.14.2.7	PRACH Resource Selection	1408

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1408

A.6.2.15.1	Test Purpose and Environment	1408

A.6.2.15.2	Test Requirements	1410

A.6.2.15.2.1	Random Access Response Reception	1410

A.6.2.15.2.2	No Random Access Response Reception	1411

A.6.2.15.2.3	Receiving a NACK on msg3	1411

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1411

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1411

A.6.2.15.2.6	Contention Resolution Timer expiry	1411

A.6.2.15.2.7	PRACH Resource Selection	1412

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1412

A.6.2.16.1	Test Purpose and Environment	1412

A.6.2.16.2	Test Requirements	1415

A.6.2.16.2.1	Random Access Response Reception	1415

A.6.2.16.2.2	No Random Access Response Reception	1416

A.6.2.16.2.3	Receiving a NACK on msg3	1416

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1416

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1416

A.6.2.16.2.6	Contention Resolution Timer expiry	1416

A.6.2.16.2.7	NPRACH Resource Selection	1416

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1417

A.6.2.17.1	Test Purpose and Environment	1417

A.6.2.17.2	Test Requirements	1420

A.6.2.17.2.1	Random Access Response Reception	1420

A.6.2.17.2.2	No Random Access Response Reception	1421

A.6.2.17.2.3	Receiving a NACK on msg3	1421

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1421

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1421

A.6.2.17.2.6	Contention Resolution Timer expiry	1421

A.6.2.17.2.7	NPRACH Resource Selection	1421

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1422

A.6.2.18.1	Test Purpose and Environment	1422

A.6.2.18.2	Test Requirements	1425

A.6.2.18.2.1	Random Access Response Reception	1425

A.6.2.18.2.2	No Random Access Response Reception	1426

A.6.2.18.2.3	Receiving a NACK on msg3	1426

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1426

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1426

A.6.2.18.2.6	Contention Resolution Timer expiry	1426

A.6.2.18.2.7	NPRACH Resource Selection	1426

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1427

A.6.2.19.1	Test Purpose and Environment	1427

A.6.2.19.2	Test Requirements	1429

A.6.2.19.2.1	Random Access Response Reception	1429

A.6.2.19.2.2	No Random Access Response Reception	1430

A.6.2.19.2.3	Receiving a NACK on msg3	1430

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1430

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1430

A.6.2.19.2.6	Contention Resolution Timer expiry	1430

A.6.2.19.2.7	NPRACH Resource Selection	1430

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1431

A.6.2.20.1	Test Purpose and Environment	1431

A.6.2.20.2	Test Requirements	1433

A.6.2.20.2.1	Random Access Response Reception	1433

A.6.2.20.2.2	No Random Access Response Reception	1434

A.6.2.20.2.3	Receiving a NACK on msg3	1434

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1434

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1434

A.6.2.20.2.6	Contention Resolution Timer expiry	1434

A.6.2.20.2.7	NPRACH Resource Selection	1434

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1435

A.6.2.21.1	Test Purpose and Environment	1435

A.6.2.21.2	Test Requirements	1437

A.6.2.21.2.1	Random Access Response Reception	1437

A.6.2.21.2.2	No Random Access Response Reception	1438

A.6.2.21.2.3	Receiving a NACK on msg3	1438

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1438

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1438

A.6.2.21.2.6	Contention Resolution Timer expiry	1438

A.6.2.21.2.7	NPRACH Resource Selection	1438

A.6.3	RRC Connection Release with Redirection	1439

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1439

A.6.3.1.1	Test Purpose and Environment	1439

A.6.3.1.2	Test Requirements	1441

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1441

A.6.3.2.1	Test Purpose and Environment	1441

A.6.3.2.2	Test Requirements	1443

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1443

A.6.3.3.1	Test Purpose and Environment	1443

A.6.3.3.2	Test Requirements	1444

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1445

A.6.3.4.1	Test Purpose and Environment	1445

A.6.3.4.2	Test Requirements	1446

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1447

A.6.3.5.1	Test Purpose and Environment	1447

A.6.3.5.2	Test Requirements	1449

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1449

A.6.3.6.1	Test Purpose and Environment	1449

A. 6.3.6.2	Test Requirements	1452

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1452

A.6.3.7.1	Test Purpose and Environment	1452

A.6.3.7.2	Test Requirements	1455

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1455

A.6.3.8.1	Test Purpose and Environment	1455

A.6.3.8.2	Test Requirements	1458

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1458

A.6.3.9.1	Test Purpose and Environment	1458

A.6.3.9.2	Test Requirements	1460

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1460

A.6.3.10.1	Test Purpose and Environment	1460

A.6.3.10.2	Test Requirements	1462

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1462

A.6.3.11.1	Test Purpose and Environment	1462

A.6.3.11.2	Test Requirements	1464

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1464

A.6.3.12.1	Test Purpose and Environment	1464

A.6.3.12.2	Test Requirements	1467

A.7	Timing and Signalling Characteristics	1447

A.7.1	UE Transmit Timing	1447

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1447

A.7.1.1.1	Test Purpose and Environment	1447

A.7.1.1.2	Test Requirements	1448

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1449

A.7.1.2.1	Test Purpose and Environment	1449

A.7.1.2.2	Test Requirements	1451

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1452

A.7.1.3.1	Test Purpose and Environment	1452

A.7.1.3.2	Test Requirements	1454

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1455

A.7.1.4.1	Test Purpose and Environment	1455

A.7.1.4.2	Test Requirements	1456

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1457

A.7.1.4A.1	Test Purpose and Environment	1457

A.7.1.4A.2	Test Requirements	1457

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1457

A.7.1.5.1	Test Purpose and Environment	1457

A.7.1.5.2	Test Requirements	1458

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1458

A.7.1.6.1	Test Purpose and Environment	1458

A.7.1.6.2	Test Requirements	1460

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1461

A.7.1.7.1	Test Purpose and Environment	1461

A.7.1.7.2	Test Requirements	1462

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1463

A.7.1.7A.1	Test Purpose and Environment	1463

A.7.1.7A.2	Test Requirements	1463

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1463

A.7.1.7B.1	Test Purpose and Environment	1463

A.7.1.7B.2	Test Requirements	1463

A.7.1.8	Void	1464

A.7.1.8.1	Void	1464

A.7.1.8.2	Void	1464

A.7.1.9	Void	1464

A.7.1.9.1	Void	1464

A.7.1.9.2	Void	1464

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1464

A.7.1.10.1	Test Purpose and Environment	1464

A.7.1.10.2	Test Requirements	1466

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1466

A.7.1.11.1	Test Purpose and Environment	1466

A.7.1.11.2	Test Requirements	1468

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1468

A.7.1.12.1	Test Purpose and Environment	1468

A.7.1.12.2	Test Requirements	1470

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1470

A.7.1.13.1	Test Purpose and Environment	1470

A.7.1.13.2	Test Requirements	1472

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1473

A.7.1.14.1	Test Purpose and Environment	1473

A.7.1.14.2	Test Requirements	1474

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1474

A.7.1.15.1	Test Purpose and Environment	1474

A.7.1.15.2	Test Requirements	1475

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1476

A.7.1.16.1	Test Purpose and Environment	1476

A.7.1.16.2	Test Requirements	1477

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1478

A.7.1.17.1	Test Purpose and Environment	1478

A.7.1.17.2	Test Requirements	1480

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1481

A.7.1.18.1	Test Purpose and Environment	1481

A.7.1.18.2	Test Requirements	1483

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1484

A.7.1.19.1	Test Purpose and Environment	1484

A.7.1.19.2	Test Requirements	1485

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1485

A.7.1.20.1	Test Purpose and Environment	1485

A.7.1.20.2	Test Requirements	1487

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1487

A.7.1.21.1	Test Purpose and Environment	1487

A.7.1.21.2	Test Requirements	1489

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1489

A.7.1.22.1	Test Purpose and Environment	1489

A.7.1.22.2	Test Requirements	1491

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1491

A.7.1.23.1	Test Purpose and Environment	1491

A.7.1.23.2	Test Requirements	1493

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1493

A.7.1.24.1	Test Purpose and Environment	1493

A.7.1.24.2	Test Requirements	1494

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1495

A.7.1.25.1	Test Purpose and Environment	1495

A.7.1.25.2	Test Requirements	1496

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1497

A.7.1.26.1	Test Purpose and Environment	1497

A.7.1.26.2	Test Requirements	1498

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1499

A.7.1.27.1	Test Purpose and Environment	1499

A.7.1.27.2	Test Requirements	1501

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1501

A.7.1.28.1	Test Purpose and Environment	1501

A.7.1.28.2	Test Requirements	1503

A.7.2	UE Timing Advance	1504

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1504

A.7.2.1.1	Test Purpose and Environment	1504

A.7.2.1.2	Test Requirements	1505

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1506

A.7.2.2.1	Test Purpose and Environment	1506

A.7.2.2.2	Test Requirements	1508

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1508

A.7.2.3.1	Test Purpose and Environment	1508

A.7.2.3.2	Test Requirements	1508

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1508

A.7.2.4.1	Test Purpose and Environment	1508

A.7.2.4.2	Test Requirements	1511

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1511

A.7.2.5.1	Test Purpose and Environment	1511

A.7.2.5.2	Test Requirements	1513

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1513

A.7.2.5A.1	Test Purpose and Environment	1513

A.7.2.5A.2	Test Requirements	1514

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1514

A.7.2.5B.1	Test Purpose and Environment	1514

A.7.2.5B.2	Test Requirements	1514

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1514

A.7.2.6.1	Test Purpose and Environment	1514

A.7.2.6.2	Test Requirements	1517

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1517

A.7.2.7.1	Test Purpose and Environment	1517

A.7.2.7.2	Test Requirements	1519

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1519

A.7.2.8.1	Test Purpose and Environment	1519

A.7.2.8.2	Test Requirements	1521

A.7.2.9.2	Test Requirements	1522

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1523

A.7.2.10.1	Test Purpose and Environment	1523

A.7.2.10.2	Test Requirements	1524

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1524

A.7.2.11.1	Test Purpose and Environment	1524

A.7.2.11.2	Test Requirements	1526

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1526

A.7.2.12.1	Test Purpose and Environment	1526

A.7.2.12.2	Test Requirements	1528

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1529

A.7.2.13.1	Test Purpose and Environment	1529

A.7.2.13.2	Test Requirements	1530

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1531

A.7.2.14.1	Test Purpose and Environment	1531

A.7.2.14.2	Test Requirements	1533

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1533

A.7.2.15.1	Test Purpose and Environment	1533

A.7.2.15.2	Test Requirements	1535

A.7.3	Radio Link Monitoring	1535

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1535

A.7.3.1.1	Test Purpose and Environment	1535

A.7.3.1.2	Test Requirements	1539

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1539

A.7.3.2.1	Test Purpose and Environment	1539

A.7.3.2.2	Test Requirements	1543

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1543

A.7.3.3.1	Test Purpose and Environment	1543

A.7.3.3.2	Test Requirements	1547

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1547

A.7.3.4.1	Test Purpose and Environment	1547

A.7.3.4.2	Test Requirements	1551

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1551

A.7.3.5.1	Test Purpose and Environment	1551

A.7.3.5.2	Test Requirements	1554

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1555

A.7.3.6.1	Test Purpose and Environment	1555

A.7.3.6.2	Test Requirements	1558

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1558

A.7.3.7.1	Test Purpose and Environment	1558

A.7.3.7.2	Test Requirements	1561

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1561

A.7.3.8.1	Test Purpose and Environment	1561

A.7.3.8.2	Test Requirements	1564

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1564

A.7.3.9.2	Test Requirements	1567

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1567

A.7.3.10.1	Test Purpose and Environment	1567

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1571

A.7.3.11.1	Test Purpose and Environment	1571

A.7.3.11.2	Test Requirements	1576

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1576

A.7.3.12.1	Test Purpose and Environment	1576

A.7.3.12.2	Test Requirements	1581

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1581

A.7.3.13.1	Test Purpose and Environment	1581

A.7.3.13.2	Test Requirements	1584

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1584

A.7.3.14.1	Test Purpose and Environment	1584

A.7.3.14.2	Test Requirements	1587

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1587

A.7.3.15.1	Test Purpose and Environment	1587

A.7.3.15.2	Test Requirements	1591

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1591

A.7.3.16.1	Test Purpose and Environment	1591

A.7.3.16.2	Test Requirements	1596

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1596

A.7.3.17.1	Test Purpose and Environment	1596

A.7.3.17.2	Test Requirements	1600

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1600

A.7.3.18.1	Test Purpose and Environment	1600

A.7.3.18.2	Test Requirements	1604

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1604

A.7.3.19.1	Test Purpose and Environment	1604

A.7.3.19.2	Test Requirements	1609

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1609

A.7.3.20.1	Test Purpose and Environment	1609

A.7.3.20.2	Test Requirements	1614

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1614

A.7.3.21.1	Test Purpose and Environment	1614

A.7.3.21.2	Test Requirements	1619

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1619

A.7.3.22.1	Test Purpose and Environment	1619

A.7.3.22.2	Test Requirements	1624

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1624

A.7.3.23.1	Test Purpose and Environment	1624

A.7.3.23.2	Test Requirements	1625

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1625

A.7.3.24.1	Test Purpose and Environment	1625

A.7.3.24.2	Test Requirements	1626

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1626

A.7.3.25.1	Test Purpose and Environment	1626

A.7.3.25.2	Test Requirements	1627

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1627

A.7.3.26.1	Test Purpose and Environment	1627

A.7.3.26.2	Test Requirements	1630

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1630

A.7.3.27.1	Test Purpose and Environment	1630

A.7.3.27.2	Test Requirements	1633

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1633

A.7.3.28.1	Test Purpose and Environment	1633

A.7.3.28.2	Test Requirements	1636

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1636

A.7.3.29.1	Test Purpose and Environment	1636

A.7.3.29.2	Test Requirements	1639

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1639

A.7.3.30.1	Test Purpose and Environment	1639

A.7.3.30.2	Test Requirements	1642

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1642

A.7.3.31.1	Test Purpose and Environment	1642

A.7.3.31.2	Test Requirements	1645

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1645

A.7.3.32.1	Test Purpose and Environment	1645

A.7.3.32.2	Test Requirements	1648

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1648

A.7.3.33.1	Test Purpose and Environment	1648

A.7.3.33.2	Test Requirements	1651

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1651

A.7.3.34.1	Test Purpose and Environment	1651

A.7.3.34.2	Test Requirements	1654

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1654

A.7.3.35.1	Test Purpose and Environment	1654

A.7.3.35.2	Test Requirements	1657

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1657

A.7.3.36.1	Test Purpose and Environment	1657

A.7.3.36.2	Test Requirements	1660

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1660

A.7.3.37.1	Test Purpose and Environment	1660

A.7.3.37.2	Test Requirements	1663

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1663

A.7.3.38.1	Test Purpose and Environment	1663

A.7.3.38.2	Test Requirements	1666

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1666

A.7.3.39.1	Test Purpose and Environment	1666

A.7.3.39.2	Test Requirements	1669

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1670

A.7.3.40.1	Test Purpose and Environment	1670

A.7.3.40.2	Test Requirements	1672

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1673

A.7.3.41.1	Test Purpose and Environment	1673

A.7.3.41.2	Test Requirements	1676

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1676

A.7.3.42.1	Test Purpose and Environment	1676

A.7.3.42.2	Test Requirements	1679

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1679

A.7.3.43.1	Test Purpose and Environment	1679

A.7.3.43.2	Test Requirements	1682

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1682

A.7.3.44.1	Test Purpose and Environment	1682

A.7.3.44.2	Test Requirements	1685

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1686

A.7.3.45.1	Test Purpose and Environment	1686

A.7.3.45.2	Test Requirements	1688

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1689

A.7.3.46.1	Test Purpose and Environment	1689

A.7.3.46.2	Test Requirements	1692

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1692

A.7.3.47.1	Test Purpose and Environment	1692

A.7.3.47.2	Test Requirements	1695

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1695

A.7.3.48.1	Test Purpose and Environment	1695

A.7.3.48.2	Test Requirements	1698

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1698

A.7.3.49.1	Test Purpose and Environment	1698

A.7.3.49.2	Test Requirements	1701

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1701

A.7.3.50.1	Test Purpose and Environment	1701

A.7.3.50.2	Test Requirements	1704

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1704

A.7.3.51.1	Test Purpose and Environment	1704

A.7.3.51.2	Test Requirements	1707

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1707

A.7.3.52.1	Test Purpose and Environment	1707

A.7.3.52.2	Test Requirements	1710

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1710

A.7.3.53.1	Test Purpose and Environment	1710

A.7.3.53.2	Test Requirements	1713

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1713

A.7.3.54.1	Test Purpose and Environment	1713

A.7.3.54.2	Test Requirements	1716

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1716

A.7.3.55.1	Test Purpose and Environment	1716

A.7.3.55.2	Test Requirements	1719

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1719

A.7.3.56.1	Test Purpose and Environment	1719

A.7.3.56.2	Test Requirements	1722

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1722

A.7.3.57.1	Test Purpose and Environment	1722

A.7.3.57.2	Test Requirements	1725

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1725

A.7.3.58.1	Test Purpose and Environment	1725

A.7.3.58.2	Test Requirements	1728

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1728

A.7.3.59.1	Test Purpose and Environment	1728

A.7.3.59.2	Test Requirements	1731

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1731

A.7.3.60.1	Test Purpose and Environment	1731

A.7.3.60.2	Test Requirements	1735

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1735

A.7.3.61.1	Test Purpose and Environment	1735

A.7.3.61.2	Test Requirements	1739

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1739

A.7.3.62.1	Test Purpose and Environment	1739

A.7.3.62.2	Test Requirements	1743

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1743

A.7.3.63.1	Test Purpose and Environment	1743

A.7.3.63.2	Test Requirements	1747

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1747

A.7.3.64.1	Test Purpose and Environment	1747

A.7.3.64.2	Test Requirements	1750

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1751

A.7.3.65.1	Test Purpose and Environment	1751

A.7.3.65.2	Test Requirements	1754

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1755

A.7.3.66.1	Test Purpose and Environment	1755

A.7.3.66.2	Test Requirements	1757

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1758

A.7.3.67.1	Test Purpose and Environment	1758

A.7.3.67.2	Test Requirements	1761

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1762

A.7.3.68.1	Test Purpose and Environment	1762

A.7.3.68.2	Test Requirements	1764

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1764

A.7.3.69.1	Test Purpose and Environment	1764

A.7.3.69.2	Test Requirements	1766

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1766

A.7.3.70.1	Test Purpose and Environment	1766

A.7.3.70.2	Test Requirements	1768

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1768

A.7.3.71.1	Test Purpose and Environment	1768

A.7.3.71.2	Test Requirements	1771

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1771

A.7.3.72.1	Test Purpose and Environment	1771

A.7.3.72.2	Test Requirements	1773

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1773

A.7.3.73.1	Test Purpose and Environment	1773

A.7.3.73.2	Test Requirements	1775

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1775

A.7.3.74.1	Test Purpose and Environment	1775

A.7.3.74.2	Test Requirements	1778

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1778

A.7.3.75.1	Test Purpose and Environment	1778

A.7.3.75.2	Test Requirements	1780

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1781

A.7.3.76.1	Test Purpose and Environment	1781

A.7.3.76.2	Test Requirements	1784

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1784

A.7.3.77.1	Test Purpose and Environment	1784

A.7.3.77.2	Test Requirements	1787

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1787

A.7.3.78.1	Test Purpose and Environment	1787

A.7.3.78.2	Test Requirements	1790

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1790

A.7.3.79.1	Test Purpose and Environment	1790

A.7.3.79.2	Test Requirements	1793

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1793

A.7.3.80.1	Test Purpose and Environment	1793

A.7.3.80.2	Test Requirements	1796

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1796

A.7.3.81.1	Test Purpose and Environment	1796

A.7.3.81.2	Test Requirements	1799

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1799

A.7.3.82.1	Test Purpose and Environment	1799

A.7.3.82.2	Test Requirements	1802

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1802

A.7.3.83.1	Test Purpose and Environment	1802

A.7.3.83.2	Test Requirements	1803

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1804

A.7.3.84.1	Test Purpose and Environment	1804

A.7.3.84.2	Test Requirements	1806

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1806

A.7.3.85.1	Test Purpose and Environment	1806

A.7.3.85.2	Test Requirements	1807

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1808

A.7.3.86.1	Test Purpose and Environment	1808

A.7.3.86.2	Test Requirements	1810

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1810

A.7.3.87.1	Test Purpose and Environment	1810

A.7.3.87.2	Test Requirements	1812

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1813

A.7.3.88.1	Test Purpose and Environment	1813

A.7.3.88.2	Test Requirements	1817

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1817

A.7.3.89.1	Test Purpose and Environment	1817

A.7.3.89.2	Test Requirements	1821

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1821

A.7.3.90.1	Test Purpose and Environment	1821

A.7.3.90.2	Test Requirements	1825

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1825

A.7.3.91.1	Test Purpose and Environment	1825

A.7.3.91.2	Test Requirements	1829

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1829

A.7.3.92.1	Test Purpose and Environment	1829

A.7.3.92.2	Test Requirements	1832

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1833

A.7.3.93.1	Test Purpose and Environment	1833

A.7.3.93.2	Test Requirements	1836

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1837

A.7.3.94.1	Test Purpose and Environment	1837

A.7.3.94.2	Test Requirements	1839

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1840

A.7.3.95.1	Test Purpose and Environment	1840

A.7.3.95.2	Test Requirements	1843

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1843

A.7.3.96.1	Test Purpose and Environment	1843

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1845

A.7.3.97.1	Test Purpose and Environment	1845

A.7.3.97.2	Test Requirements	1848

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1848

A.7.3.98.1	Test Purpose and Environment	1848

A.7.3.98.2	Test Requirements	1851

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1851

A.7.3.99.1	Test Purpose and Environment	1851

A.7.3.99.2	Test Requirements	1854

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1854

A.7.3.100.1	Test Purpose and Environment	1854

A.7.3.100.2	Test Requirements	1856

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1856

A.7.3.101.1	Test Purpose and Environment	1856

A.7.3.101.2	Test Requirements	1858

A.7.4	Interruption for Dual Connectivity	1858

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1858

A.7.4.1.1	Test Purpose and Environment	1858

A.7.4.1.2	Test Requirements	1861

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1861

A.7.4.2.1	Test Purpose and Environment	1861

A.7.4.2.2	Test Requirements	1863

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1863

A.7.4.3.1	Test Purpose and Environment	1863

A.7.4.3.2	Test Requirements	1865

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1865

A.7.4.4.1	Test Purpose and Environment	1865

A.7.4.4.2	Test Requirements	1868

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1868

A.7.4.5.1	Test Purpose and Environment	1868

A.7.4.5.2	Test Requirements	1870

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1870

A.7.4.6.1	Test Purpose and Environment	1870

A.7.4.6.2	Test Requirements	1872

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1872

A.7.4.7.1	Test Purpose and Environment	1872

A.7.4.7.2	Test Requirements	1875

A.7.5	Proximity-based Services	1875

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1875

A.7.5.1.1	Test Purpose and Environment	1875

A.7.5.1.2	Test Requirements	1876

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1877

A.7.5.2.1	Test Purpose and Environment	1877

A.7.5.1.2	Test Requirements	1877

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1878

A.7.5.3.1	Test Purpose and Environment	1878

A.7.5.3.2	Test Requirements	1879

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1880

A.7.5.4.1	Test Purpose and Environment	1880

A.7.5.4.2	Test Requirements	1881

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1882

A.7.5.5.1	Test Purpose and Environment	1882

A.7.5.5.2	Test Requirements	1884

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1885

A.7.5.6.1	Test Purpose and Environment	1885

A.7.5.6.2	Test Requirements	1886

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1887

A.7.5.7.1	Test Purpose and Environment	1887

A.7.5.7.2	Test Requirements	1889

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1889

A.7.5.8.1	Test Purpose and Environment	1889

A.7.5.8.2	Test Requirements	1891

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1892

A.7.5.9.1	Test Purpose and Environment	1892

A.7.5.9.2	Test Requirements	1894

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1895

A.7.5.10.1	Test Purpose and Environment	1895

A.7.5.10.2	Test Requirements	1897

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1898

A.7.5.11.1	Test Purpose and Environment	1898

A.7.5.11.2	Test Requirements	1900

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1900

A.7.5.12.1	Test Purpose and Environment	1900

A.7.5.12.2	Test Requirements	1904

A.7.6	Interruption for carrier aggregation	1905

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1905

A.7.6.1.1	Test Purpose and Environment	1905

A.7.6.1.2	Test Requirements	1908

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1908

A.7.6.2.1	Test Purpose and Environment	1908

A.7.6.2.2	Test Requirements	1911

A.8	UE Measurements Procedures	1911

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1911

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1911

A.8.1.1.1	Test Purpose and Environment	1911

A.8.1.1.2	Test Requirements	1913

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1913

A.8.1.2.1	Test Purpose and Environment	1913

A.8.1.2.2	Test Requirements	1915

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1915

A.8.1.3.1	Test Purpose and Environment	1915

A.8.1.3.2	Test Requirements	1918

A.8.1.4	Void	1918

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1918

A.8.1.5.1	Test Purpose and Environment	1918

A.8.1.5.2	Test Requirements	1920

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1920

A.8.1.6.1	Test Purpose and Environment	1920

A.8.1.6.2	Test Requirements	1922

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1922

A.8.1.7.1	Test Purpose and Environment	1922

A.8.1.7.2	Test Requirements	1924

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1925

A.8.1.8.1	Test Purpose and Environment	1925

A.8.1.8.2	Test Requirements	1927

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1928

A.8.1.9.1	Test Purpose and Environment	1928

A.8.1.9.2	Test Requirements	1928

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1928

A.8.1.10.1	Test Purpose and Environment	1928

A.8.1.10.2	Test Requirements	1929

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1929

A.8.1.11.1	Test Purpose and Environment	1929

A.8.1.11.2	Test Requirements	1931

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1932

A.8.1.12.1	Test Purpose and Environment	1932

A.8.1.12.2	Test Requirements	1933

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1934

A.8.1.13.1	Test Purpose and Environment	1934

A.8.1.13.2	Test Requirements	1936

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1936

A.8.1.14.1	Test Purpose and Environment	1936

A.8.1.14.2	Test Requirements	1938

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1939

A.8.1.15.1	Test Purpose and Environment	1939

A.8.1.15.2	Test Requirements	1940

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1941

A.8.1.16.1	Test Purpose and Environment	1941

A.8.1.16.2	Test Requirements	1943

A.8.1.17	Void	1943

A.8.1.18	Void	1943

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1943

A.8.1.19.1	Test Purpose and Environment	1943

A.8.1.19.2	Test Requirements	1945

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1946

A.8.1.20.1	Test Purpose and Environment	1946

A.8.1.20.2	Test Requirements	1948

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1948

A.8.1.21.1	Test Purpose and Environment	1948

A.8.1.21.2	Test Requirements	1950

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1951

A.8.1.22.1	Test Purpose and Environment	1951

A.8.1.22.2	Test Requirements	1953

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1953

A.8.1.23.1	Test Purpose and Environment	1953

A.8.1.23.2	Test Requirements	1954

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1955

A.8.1.24.1	Test Purpose and Environment	1955

A.8.1.24.2	Test Requirements	1957

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1957

A.8.1.25.1	Test Purpose and Environment	1957

A.8.1.25.2	Test Requirements	1959

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1959

A.8.1.26.1	Test Purpose and Environment	1959

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1962

A.8.1.27.1	Test Purpose and Environment	1962

A.8.1.27.2	Test Requirements	1964

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1964

A.8.1.28.1	Test Purpose and Environment	1964

A.8.1.28.2	Test Requirements	1966

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1966

A.8.1.29.1	Test Purpose and Environment	1966

A.8.1.29.2	Test Requirements	1968

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1969

A.8.1.30.1	Test Purpose and Environment	1969

A.8.1.30.2	Test Requirements	1971

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1971

A.8.1.31.1	Test Purpose and Environment	1971

A.8.1.31.2	Test Requirements	1973

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1974

A.8.1.32.1	Test Purpose and Environment	1974

A.8.1.32.2	Test Requirements	1975

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1976

A.8.1.33.1	Test Purpose and Environment	1976

A.8.1.33.2	Test Requirements	1977

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1978

A.8.1.34.1	Test Purpose and Environment	1978

A.8.1.34.2	Test Requirements	1979

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1980

A.8.1.35.1	Test Purpose and Environment	1980

A.8.1.35.2	Test Requirements	1981

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1982

A.8.1.36.1	Test Purpose and Environment	1982

A.8.1.36.2	Test Requirements	1983

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1984

A.8.1.37.1	Test Purpose and Environment	1984

A.8.1.37.2	Test Requirements	1986

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1986

A.8.1.38.1	Test Purpose and Environment	1986

A.8.1.38.2	Test Requirements	1987

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1988

A.8.1.39.1	Test Purpose and Environment	1988

A.8.1.39.2	Test Requirements	1990

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1990

A.8.1.40.1	Test Purpose and Environment	1990

A.8.1.40.2	Test Requirements	1992

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1992

A.8.1.41.1	Test Purpose and Environment	1992

A.8.1.41.2	Test Requirement	1994

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1994

A.8.1.42.1	Test Purpose and Environment	1994

A.8.1.42.2	Test Requirement	1995

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1995

A.8.1.43.1	Test Purpose and Environment	1995

A.8.1.43.2	Test Requirements	1998

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1998

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1998

A.8.2.1.1	Test Purpose and Environment	1998

A.8.2.1.2	Test Requirements	2000

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2001

A.8.2.2.1	Test Purpose and Environment	2001

A.8.2.2.2	Test Requirements	2003

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2003

A.8.2.3.1	Test Purpose and Environment	2003

A.8.2.3.2	Test Requirements	2005

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2006

A.8.2.4.1	Test Purpose and Environment	2006

A.8.2.4.2	Test Requirements	2008

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2008

A.8.2.5.1	Test Purpose and Environment	2008

A.8.2.5.2	Test Requirements	2010

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2011

A.8.2.6.1	Test Purpose and Environment	2011

A.8.2.6.2	Test Requirements	2014

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2015

A.8.2.7.1	Test Purpose and Environment	2015

A.8.2.7.2	Test Requirements	2016

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2017

A.8.2.8.1	Test Purpose and Environment	2017

A.8.2.8.2	Test Requirements	2019

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2019

A.8.2.9.1	Test Purpose and Environment	2019

A.8.2.9.2	Test Requirements	2021

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2022

A.8.2.10.1	Test Purpose and Environment	2022

A.8.2.10.2	Test Requirements	2024

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2024

A.8.2.11.1	Test Purpose and Environment	2024

A.8.2.11.2	Test Requirements	2026

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2026

A.8.2.12.1	Test Purpose and Environment	2026

A.8.2.12.2	Test Requirements	2028

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2029

A.8.2.13.1	Test Purpose and Environment	2029

A.8.2.13.2	Test Requirements	2031

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2031

A.8.2.14.1	Test Purpose and Environment	2031

A.8.2.14.2	Test Requirement	2033

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2033

A.8.2.15.1	Test Purpose and Environment	2033

A.8.2.15.2	Test Requirements	2035

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2035

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2035

A.8.3.1.1	Test Purpose and Environment	2035

A.8.3.1.2	Test Requirements	2037

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2037

A.8.3.2.1	Test Purpose and Environment	2037

A.8.3.2.2	Test Requirements	2040

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2040

A.8.3.3.1	Test Purpose and Environment	2040

A.8.3.3.2	Test Requirements	2043

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2043

A.8.3.4.1	Test Purpose and Environment	2043

A.8.3.4.2	Test Requirements	2045

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2045

A.8.3.5.2	Test Requirements	2047

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2047

A.8.3.6.1	Test Purpose and Environment	2047

A.8.3.6.2	Test Requirements	2049

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2050

A.8.3.7.1	Test Purpose and Environment	2050

A.8.3.7.2	Test Requirements	2052

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2053

A.8.3.8.1	Test Purpose and Environment	2053

A.8.3.8.2	Test Requirements	2056

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2056

A.8.3.9.1	Test Purpose and Environment	2056

A.8.3.9.2	Test Requirements	2060

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2061

A.8.3.10.1	Test Purpose and Environment	2061

A.8.3.10.2	Test Requirements	2062

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2062

A.8.3.11.1	Test Purpose and Environment	2062

A.8.3.11.2	Test Requirement	2064

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2065

A.8.3.12.1	Test Purpose and Environment	2065

A.8.3.12.2	Test Requirement	2066

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2067

A.8.3.13.1	Test Purpose and Environment	2067

A.8.3.13.2	Test Requirement	2068

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2069

A.8.3.14.1	Test Purpose and Environment	2069

A.8.3.14.2	Test Requirement	2070

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2071

A.8.3.15.1	Test Purpose and Environment	2071

A.8.3.15.2	Test Requirement	2072

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2073

A.8.3.16.1	Test Purpose and Environment	2073

A.8.3.16.2	Test Requirement	2075

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2075

A.8.3.17.1	Test Purpose and Environment	2075

A.8.3.17.2	Test Requirement	2078

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2078

A.8.3.18.1	Test Purpose and Environment	2078

A.8.3.18.2	Test Requirement	2081

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2081

A.8.3.19.1	Test Purpose and Environment	2081

A.8.3.19.2	Test Requirement	2084

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2084

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2084

A.8.4.1.1	Test Purpose and Environment	2084

A.8.4.1.2	Test Requirements	2086

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2086

A.8.4.2.1	Test Purpose and Environment	2086

A.8.4.2.2	Test Requirements	2089

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2089

A.8.4.3.1	Test Purpose and Environment	2089

A.8.4.3.2	Test Requirements	2091

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2092

A.8.4.4.1	Test Purpose and Environment	2092

A.8.4.4.2	Test Requirements	2094

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2095

A.8.4.5.1	Test Purpose and Environment	2095

A.8.4.5.2	Test Requirements	2097

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2097

A.8.4.6.1	Test Purpose and Environment	2097

A.8.4.6.2	Test Requirements	2098

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2098

A.8.4.7.1	Test Purpose and Environment	2098

A.8.4.7.2	Test Requirements	2102

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2102

A.8.4.8.1	Test Purpose and Environment	2102

A.8.4.8.2	Test Requirements	2105

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2105

A.8.4.9.1	Test Purpose and Environment	2105

A.8.4.9.2	Test Requirements	2110

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2110

A.8.4.10.1	Test Purpose and Environment	2110

A.8.4.10.2	Test Requirements	2112

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2112

A.8.4.11.1	Test Purpose and Environment	2112

A.8.4.11.2	Test Requirement	2114

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2114

A.8.4.12.1	Test Purpose and Environment	2114

A.8.4.12.2	Test Requirement	2116

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2117

A.8.4.13.1	Test Purpose and Environment	2117

A.8.4.13.2	Test Requirement	2118

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2119

A.8.4.14.1	Test Purpose and Environment	2119

A.8.4.14.2	Test Requirement	2121

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2121

A.8.4.15.1	Test Purpose and Environment	2121

A.8.4.15.2	Test Requirement	2124

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2124

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2124

A.8.5.1.1	Test Purpose and Environment	2124

A.8.5.1.2	Test Requirements	2126

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2126

A.8.5.2.1	Test Purpose and Environment	2126

A.8.5.2.2	Test Requirements	2128

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2128

A.8.5.3.1	Test Purpose and Environment	2128

A.8.5.3.2	Test Requirements	2131

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2131

A.8.5.4.1	Test Purpose and Environment	2131

A.8.5.4.2	Test Requirements	2133

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2133

A.8.5.5.1	Test Purpose and Environment	2133

A.8.5.5.2	Test Requirements	2136

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2136

A.8.5.6.1	Test Purpose and Environment	2136

A.8.5.6.2	Test Requirements	2137

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2138

A.8.5.7.1	Test Purpose and Environment	2138

A.8.5.7.2	Test Requirements	2138

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2138

A.8.5.8.1	Test Purpose and Environment	2138

A.8.5.8.2	Test Requirements	2141

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2141

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2141

A.8.6.1.1	Test Purpose and Environment	2141

A.8.6.1.2	Test Requirements	2143

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2143

A.8.6.2.1	Test Purpose and Environment	2143

A.8.6.2.2	Test Requirements	2146

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2146

A.8.6.3.1	Test Purpose and Environment	2146

A.8.6.3.2	Test Requirements	2149

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2149

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2149

A.8.7.1.1	Test Purpose and Environment	2149

A.8.7.1.1.1	Void	2149

A.8.7.1.1.2	1.28 Mcps TDD option	2149

A.8.7.1.1.3	Void	2151

A.8.7.1.2	Test Requirements	2151

A.8.7.1.2.1	Void	2151

A.8.7.1.2.2	1.28 Mcps TDD option	2151

A.8.7.1.2.3	Void	2151

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2151

A.8.7.2.1	Test Purpose and Environment	2151

A.8.7.2.2	Test Requirements	2154

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2155

A.8.7.3.1	Test Purpose and Environment	2155

A.8.7.3.2	Test Parameters	2155

A.8.7.3.3	Test Requirements	2156

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2157

A.8.7.4.1	Test Purpose and Environment	2157

A.8.7.4.2	Test Requirements	2159

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2159

A.8.7.5.1	Test Purpose and Environment	2159

A.8.7.5.2	Test Requirements	2161

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2161

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2161

A.8.7A.1.1	Test Purpose and Environment	2161

A.8.7A.1.2	Test Requirements	2164

A.8.8	E-UTRAN FDD – GSM Measurements	2164

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2164

A.8.8.1.1	Test Purpose and Environment	2164

A.8.8.1.2	Test Requirements	2166

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2166

A.8.8.2.1	Test Purpose and Environment	2166

A.8.8.2.2	Test Requirements	2168

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2169

A.8.8.3.1	Test Purpose and Environment	2169

A.8.8.3.2	Test Requirements	2170

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2171

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2171

A.8.9.1.1	Test Purpose and Environment	2171

A.8.9.1.2	Test Requirements	2172

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2173

A.8.9.2.1	Test Purpose and Environment	2173

A.8.9.2.2	Test Requirements	2175

A.8.10	E-UTRAN TDD – GSM Measurements	2175

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2175

A.8.10.1.1	Test Purpose and Environment	2175

A.8.10.1.2	Test Requirements	2176

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2177

A.8.10.2.1	Test Purpose and Environment	2177

A.8.10.2.2	Test Requirements	2179

A.8.11	Monitoring of Multiple Layers	2179

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2179

A.8.11.1.1	Test Purpose and Environment	2179

A.8.11.1.2	Test Requirements	2181

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2182

A.8.11.2.1	Test Purpose and Environment	2182

A.8.11.2.2	Test Requirements	2183

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2184

A.8.11.3.1	Test Purpose and Environment	2184

A.8.11.3.2	Test Requirements	2186

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2186

A.8.11.4.1	Test Purpose and Environment	2186

A.8.11.4.2	Test Requirements	2189

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2189

A.8.11.5.1	Test Purpose and Environment	2189

A.8.11.5.2	Test Requirements	2191

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2192

A.8.11.6.1	Test Purpose and Environment	2192

A.8.11.6.2	Test Requirements	2194

A.8.12	RSTD Intra-frequency Measurements	2195

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2195

A.8.12.1.1	Test Purpose and Environment	2195

A.8.12.1.2	Test Requirements	2199

A.8.12.1.2A	Test Requirements for UE Category 1bis	2199

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2199

A.8.12.2.1	Test Purpose and Environment	2199

A.8.12.2.2	Test Requirements	2204

A.8.12.2.2A	Test Requirements for UE Category 1bis	2204

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2204

A.8.12.3.1	Test Purpose and Environment	2204

A.8.12.3.2	Test Requirements	2210

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2210

A.8.12.4.1	Test Purpose and Environment	2210

A.8.12.4.2	Test Requirements	2215

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2215

A.8.12.5.1	Test Purpose and Environment	2215

A.8.12.5.2	Test Requirements	2220

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2220

A.8.12.6.1	Test Purpose and Environment	2220

A.8.12.6.2	Test Requirements	2225

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2225

A.8.12.7.1	Test Purpose and Environment	2225

A.8.12.7.2	Test Requirements	2230

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2230

A.8.12.8.1	Test Purpose and Environment	2230

A.8.12.8.2	Test Requirements	2235

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2235

A.8.12.9.1	Test Purpose and Environment	2235

A.8.12.9.2	Test Requirements	2240

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2240

A.8.12.10.1	Test Purpose and Environment	2240

A.8.12.10.2	Test Requirements	2245

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2245

A.8.12.11.1	Test Purpose and Environment	2245

A.8.12.11.2	Test Requirements	2250

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2250

A.8.12.12.1	Test Purpose and Environment	2250

A.8.12.12.2	Test Requirements	2255

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2255

A.8.12.13.1	Test Purpose and Environment	2255

A.8.12.13.2	Test Requirements	2260

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2260

A.8.12.14.1	Test Purpose and Environment	2260

A.8.12.14.2	Test Requirements	2265

A.8.13	RSTD Inter-frequency Measurements	2265

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2265

A.8.13.1.1	Test Purpose and Environment	2265

A.8.13.1.2	Test Requirements	2270

A.8.13.1.2A	Test Requirements for UE Category 1bis	2270

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2270

A.8.13.2.1	Test Purpose and Environment	2270

A.8.13.2.2	Test Requirements	2276

A.8.13.2.2A	Test Requirements for UE Category 1bis	2276

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2276

A.8.13.3.1	Test Purpose and Environment	2276

A.8.13.3.2	Test Requirements	2283

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2283

A.8.13.4.1	Test Purpose and Environment	2283

A.8.13.4.2	Test Requirements	2289

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2289

A.8.13.5.1	Test Purpose and Environment	2289

A.8.13.5.2	Test Requirements	2295

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2295

A.8.13.6.1	Test Purpose and Environment	2295

A.8.13.6.2	Test Requirements	2301

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2301

A.8.13.7.1	Test Purpose and Environment	2301

A.8.13.7.2	Test Requirements	2307

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2307

A.8.13.8.1	Test Purpose and Environment	2307

A.8.13.8.2	Test Requirements	2313

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2313

A.8.13.9.1	Test Purpose and Environment	2313

A.8.13.9.2	Test Requirements	2319

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2319

A.8.13.10.1	Test Purpose and Environment	2319

A.8.13.10.2	Test Requirements	2325

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2325

A.8.13.11.1	Test Purpose and Environment	2325

A.8.13.11.2	Test Requirements	2331

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2331

A.8.13.12.1	Test Purpose and Environment	2331

A.8.13.12.2	Test Requirements	2337

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2337

A.8.13.13.1	Test Purpose and Environment	2337

A.8.13.13.2	Test Requirements	2343

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2343

A.8.13.14.1	Test Purpose and Environment	2343

A.8.13.14.2	Test Requirements	2349

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2349

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2349

A.8.14.1.1	Test Purpose and Environment	2349

A.8.14.1.2	Test Requirements	2351

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2351

A.8.14.2.1	Test Purpose and Environment	2351

A.8.14.2.2	Test Requirements	2354

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2354

A.8.14.3.1	Test Purpose and Environment	2354

A.8.14.3.2	Test Requirements	2356

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2357

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2357

A.8.15.1.1	Test Purpose and Environment	2357

A.8.15.1.2	Test Requirements	2358

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2358

A.8.15.2.1	Test Purpose and Environment	2358

A.8.15.2.2	Test Requirements	2361

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2361

A.8.15.3.1	Test Purpose and Environment	2361

A.8.15.3.2	Test Requirements	2363

A.8.16	E-UTRAN Carrier Aggregation Measurements	2364

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2364

A.8.16.1.1	Test Purpose and Environment	2364

A.8.16.1.2	Test Requirements	2366

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2366

A.8.16.2.1	Test Purpose and Environment	2366

A.8.16.2.2	Test Requirements	2368

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2368

A.8.16.3.1	Test Purpose and Environment	2368

A.8.16.3.2	Test Requirements	2370

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2371

A.8.16.3A.1	Test Purpose and Environment	2371

A.8.16.3A.2	Test Requirements	2372

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2373

A.8.16.4.1	Test Purpose and Environment	2373

A.8.16.4.2	Test Requirements	2374

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2375

A.8.16.4A.1	Test Purpose and Environment	2375

A.8.16.4A.2	Test Requirements	2377

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2378

A.8.16.5.1	Test Purpose and Environment	2378

A.8.16.5.2	Test Requirements	2378

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2378

A.8.16.6.1	Test Purpose and Environment	2378

A.8.16.6.2	Test Requirements	2379

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2379

A.8.16.7.1	 Test Purpose and Environment	2379

A.8.16.7.2	 Test Requirements	2380

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2380

A.8.16.8.1	 Test Purpose and Environment	2380

A.8.16.8.2	 Test Requirements	2381

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2381

A.8.16.9.1	Test Purpose and Environment	2381

A.8.16.9.2	Test Requirements	2382

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2382

A.8.16.10.1	Test Purpose and Environment	2382

A.8.16.10.2	Test Requirements	2383

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2383

A.8.16.11.1	 Test Purpose and Environment	2383

A.8.16.11.2	 Test Requirements	2384

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2384

A.8.16.12.1	 Test Purpose and Environment	2384

A.8.16.12.2	 Test Requirements	2384

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2384

A.8.16.13.1	Test Purpose and Environment	2384

A.8.16.13.2	Test Requirements	2385

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2385

A.8.16.14.1	Test Purpose and Environment	2385

A.8.16.14.2	Test Requirements	2386

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2386

A.8.16.15.1	 Test Purpose and Environment	2386

A.8.16.7.2	 Test Requirements	2386

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2386

A.8.16.16.1	 Test Purpose and Environment	2386

A.8.16.16.2	 Test Requirements	2387

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2387

A.8.16.17.1	Test Purpose and Environment	2387

A.8.16.17.2	Test Requirements	2389

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2390

A.8.16.17A.1	Test Purpose and Environment	2390

A.8.16.17A.2	Test Requirements	2390

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2390

A.8.16.17B.1	Test Purpose and Environment	2390

A.8.16.17B.2	Test Requirements	2391

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2391

A.8.16.17C.1	Test Purpose and Environment	2391

A.8.16.17C.2	Test Requirements	2391

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2391

A.8.16.18.1	Test Purpose and Environment	2391

A.8.16.18.2	Test Requirements	2393

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2394

A.8.16.18A.1	Test Purpose and Environment	2394

A.8.16.18A.2	Test Requirements	2394

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2394

A.8.16.18B.1	Test Purpose and Environment	2394

A.8.16.18B.2	Test Requirements	2395

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2395

A.8.16.18C.1	Test Purpose and Environment	2395

A.8.16.18C.2	Test Requirements	2395

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2395

A.8.16.18D.1	Test Purpose and Environment	2395

A.8.16.18D.2	Test Requirements	2396

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2396

A.8.16.19.1	Test Purpose and Environment	2396

A.8.16.19.2	Test Requirements	2398

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2398

A.8.16.19A.1	Test Purpose and Environment	2398

A.8.16.19A.2	Test Requirements	2399

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2399

A.8.16.19B.1	Test Purpose and Environment	2399

A.8.16.19B.2	Test Requirements	2399

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2399

A.8.16.19C.1	Test Purpose and Environment	2399

A.8.16.19C.2	Test Requirements	2400

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2400

A.8.16.20.1	Test Purpose and Environment	2400

A.8.16.20.2	Test Requirements	2402

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2403

A.8.16.20A.1	Test Purpose and Environment	2403

A.8.16.20A.2	Test Requirements	2403

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2403

A.8.16.20B.1	Test Purpose and Environment	2403

A.8.16.20B.2	Test Requirements	2404

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2404

A.8.16.20C.1	Test Purpose and Environment	2404

A.8.16.20C.2	Test Requirements	2404

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2404

A.8.16.20D.1	Test Purpose and Environment	2404

A.8.16.20D.2	Test Requirements	2405

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2405

A.8.16.21.1	Test Purpose and Environment	2405

A.8.16.21.2	Test Requirements	2407

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2407

A.8.16.22.1	Test Purpose and Environment	2407

A.8.16.22.2	Test Requirements	2409

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2409

A.8.16.23.1	Test Purpose and Environment	2409

A.8.16.23.2	Test Requirements	2412

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2412

A.8.16.24.1	Test Purpose and Environment	2412

A.8.16.24.2	Test Requirements	2415

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2415

A.8.16.25.1	Test Purpose and Environment	2415

A.8.16.25.2	Test Requirements	2418

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2418

A.8.16.26.1	Test Purpose and Environment	2418

A.8.16.26.2	Test Requirements	2421

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2421

A.8.16.27.1	Test Purpose and Environment	2421

A.8.16.27.2	Test Requirements	2426

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2426

A.8.16.28.1	Test Purpose and Environment	2426

A.8.16.28.2	Test Requirements	2431

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2431

A.8.16.29.1	Test Purpose and Environment	2431

A.8.16.29.2	Test Requirements	2436

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2436

A.8.16.30.1	Test Purpose and Environment	2436

A.8.16.30.2	Test Requirements	2441

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2441

A.8.16.31.1	Test Purpose and Environment	2441

A.8.16.31.2	Test Requirements	2446

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2446

A.8.16.32.1	Test Purpose and Environment	2446

A.8.16.32.2	Test Requirements	2451

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2451

A.8.16.33.1	Test Purpose and Environment	2451

A.8.16.33.2	Test Requirements	2456

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2456

A.8.16.34.1	Test Purpose and Environment	2456

A.8.16.34.2	Test Requirements	2461

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2461

A.8.16.35.1	Test Purpose and Environment	2461

A.8.16.35.2	Test Requirements	2463

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2464

A.8.16.36.1	Test Purpose and Environment	2464

A.8.16.36.2	Test Requirements	2466

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2467

A.8.16.37.1	Test Purpose and Environment	2467

A.8.16.37.2	Test Requirements	2469

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2470

A.8.16.38.1	Test Purpose and Environment	2470

A.8.16.38.2	Test Requirements	2472

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2473

A.8.16.39.1	Test Purpose and Environment	2473

A.8.16.39.2	Test Requirements	2476

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2476

A.8.16.40.1	Test Purpose and Environment	2476

A.8.16.40.2	Test Requirements	2479

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2479

A.8.16.41.1	Test Purpose and Environment	2479

A.8.16.41.2	Test Requirements	2481

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2482

A.8.16.42.1	Test Purpose and Environment	2482

A.8.16.42.2	Test Requirements	2484

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2485

A.8.16.43.1	Test Purpose and Environment	2485

A.8.16.43.2	Test Requirements	2488

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2488

A.8.16.44.1	Test Purpose and Environment	2488

A.8.16.44.2	Test Requirements	2491

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2491

A.8.16.45.1	Test Purpose and Environment	2491

A.8.16.45.2	Test Requirements	2494

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2494

A.8.16.46.1	Test Purpose and Environment	2494

A.8.16.46.2	Test Requirements	2496

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2497

A.8.16.47.1	Test Purpose and Environment	2497

A.8.16.47.2	Test Requirements	2499

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2500

A.8.16.48.1	Test Purpose and Environment	2500

A.8.16.48.2	Test Requirements	2502

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2503

A.8.16.49.1	Test Purpose and Environment	2503

A.8.16.49.2	Test Requirements	2506

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2506

A.8.16.50.1	Test Purpose and Environment	2506

A.8.16.50.2	Test Requirements	2508

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2509

A.8.16.51.1	Test Purpose and Environment	2509

A.8.16.51.2	Test Requirements	2514

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2514

A.8.16.52.1	Test Purpose and Environment	2514

A.8.16.52.2	Test Requirements	2519

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2519

A.8.16.53.1	Test Purpose and Environment	2519

A.8.16.53.2	Test Requirements	2523

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2523

A.8.16.54.1	Test Purpose and Environment	2523

A.8.16.54.2	Test Requirements	2527

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2527

A.8.16.55.1	Test Purpose and Environment	2527

A.8.16.55.2	Test Requirements	2533

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2533

A.8.16.56.1	Test Purpose and Environment	2533

A.8.16.56.2	Test Requirements	2539

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2539

A.8.16.57.1	Test Purpose and Environment	2539

A.8.16.57.2	Test Requirements	2541

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2542

A.8.16.58.1	Test Purpose and Environment	2542

A.8.16.58.2	Test Requirements	2544

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2545

A.8.16.59.1	Test Purpose and Environment	2545

A.8.16.59.2	Test Requirements	2549

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2550

A.8.16.60.1	Test Purpose and Environment	2550

A.8.16.60.2	Test Requirements	2553

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2554

A.8.16.61.1	Test Purpose and Environment	2554

A.8.16.61.2	Test Requirements	2557

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2557

A.8.16.62.1	Test Purpose and Environment	2557

A.8.16.62.2	Test Requirements	2559

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2560

A.8.16.63.1	Test Purpose and Environment	2560

A.8.16.63.2	Test Requirements	2563

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2564

A.8.16.64.1	Test Purpose and Environment	2564

A.8.16.64.2	Test Requirements	2568

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2568

A.8.16.65.1	Test Purpose and Environment	2568

A.8.16.65.2	Test Requirements	2573

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2573

A.8.16.66.1	Test Purpose and Environment	2573

A.8.16.66.2	Test Requirements	2578

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2578

A.8.16.67.1	Test Purpose and Environment	2578

A.8.16.67.2	Test Requirements	2583

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2583

A.8.16.68.1	Test Purpose and Environment	2583

A.8.16.68.2	Test Requirements	2588

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2588

A.8.16.69.1	Test Purpose and Environment	2588

A.8.16.69.2	Test Requirements	2592

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2592

A.8.16.70.1	Test Purpose and Environment	2592

A.8.16.70.2	Test Requirements	2596

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2596

A.8.16.71.1	Test Purpose and Environment	2596

A.8.16.71.2	Test Requirements	2601

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2601

A.8.16.72.1	Test Purpose and Environment	2601

A.8.16.72.2	Test Requirements	2606

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2606

A.8.16.73.1	Test Purpose and Environment	2606

A.8.16.73.2	Test Requirements	2612

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2612

A.8.16.74.1	Test Purpose and Environment	2612

A.8.16.74.2	Test Requirements	2618

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2618

A.8.16.75.1	Test Purpose and Environment	2618

A.8.16.75.2	Test Requirements	2624

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2624

A.8.16.76.1	Test Purpose and Environment	2624

A.8.16.76.2	Test Requirements	2630

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2630

A.8.16.77.1	Test Purpose and Environment	2630

A.8.16.77.2	Test Requirements	2634

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2634

A.8.16.78.1	Test Purpose and Environment	2634

A.8.16.78.2	Test Requirements	2636

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2637

A.8.16.79.1	Test Purpose and Environment	2637

A.8.16.79.2	Test Requirements	2644

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2644

A.8.16.80.1	Test Purpose and Environment	2644

A.8.16.80.2	Test Requirements	2651

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2651

A.8.16.81.1	Test Purpose and Environment	2651

A.8.16.81.2	Test Requirements	2657

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2657

A.8.16.82.1	Test Purpose and Environment	2657

A.8.16.82.2	Test Requirements	2663

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2663

A.8.16.83.1	Test Purpose and Environment	2663

A.8.16.83.2	Test Requirements	2668

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2668

A.8.16.84.1	Test Purpose and Environment	2668

A.8.16.84.2	Test Requirements	2674

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2674

A.8.16.85.1	Test Purpose and Environment	2674

A.8.16.85.2	Test Requirements	2678

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2678

A.8.16.86.1	Test Purpose and Environment	2678

A.8.16.86.2	Test Requirements	2681

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2681

A.8.16.87.1	Test Purpose and Environment	2681

A.8.16.87.2	Test Requirements	2687

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2687

A.8.16.88.1	Test Purpose and Environment	2687

A.8.16.88.2	Test Requirements	2695

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2695

A.8.16.89.1	Test Purpose and Environment	2695

A.8.16.89.2	Test Requirements	2702

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2702

A.8.16.90.1	Test Purpose and Environment	2702

A.8.16.90.2	Test Requirements	2709

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2709

A.8.16.91.1	Test Purpose and Environment	2709

A.8.16.91.2	Test Requirements	2715

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2715

A.8.16.92.1	Test Purpose and Environment	2715

A.8.16.92.2	Test Requirements	2723

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2723

A.8.16.93.1	Test Purpose and Environment	2723

A.8.16.93.2	Test Requirements	2730

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2730

A.8.16.94.1	Test Purpose and Environment	2730

A.8.16.94.2	Test Requirements	2737

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2737

A.8.16.95.1	Test Purpose and Environment	2737

A.8.16.95.2	Test Requirements	2745

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2745

A.8.16.96.1	Test Purpose and Environment	2745

A.8.16.96.2	Test Requirements	2754

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2754

A.8.16.97.1	Test Purpose and Environment	2754

A.8.16.97.2	Test Requirements	2761

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2761

A.8.16.98.1	Test Purpose and Environment	2761

A.8.16.98.2	Test Requirements	2769

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2769

A.8.16.99.1	Test Purpose and Environment	2769

A.8.16.99.2	Test Requirements	2779

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2779

A.8.16.100.1	Test Purpose and Environment	2779

A.8.16.100.2	Test Requirements	2789

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2789

A.8.16.101.1	Test Purpose and Environment	2789

A.8.16.101.2	Test Requirements	2799

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2799

A.8.16.102.1	Test Purpose and Environment	2799

A.8.16.102.2	Test Requirements	2810

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2810

A.8.16.103.1	Test Purpose and Environment	2810

A.8.16.103.2	Test Requirements	2815

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2816

A.8.16.104.1	Test Purpose and Environment	2816

A.8.16.104.2	Test Requirements	2820

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2821

A.8.16.105.1	Test Purpose and Environment	2821

A.8.16.105.2	Test Requirements	2825

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2826

A.8.16.106.1	Test Purpose and Environment	2826

A.8.16.106.2	Test Requirements	2830

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2830

A.8.16.107.1	Test Purpose and Environment	2830

A.8.16.107.2	Test Requirements	2832

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2833

A.8.16.108.1	Test Purpose and Environment	2833

A.8.16.108.2	Test Requirements	2835

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2836

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2836

A.8.17.1.1	Test Purpose and Environment	2836

A.8.17.1.2	Test Requirements	2842

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2842

A.8.17.2.1	Test Purpose and Environment	2842

A.8.17.2.2	Test Requirements	2849

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2849

A.8.17.3.1	Test Purpose and Environment	2849

A.8.17.3.2	Test Requirements	2850

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2850

A.8.17.4.1	Test Purpose and Environment	2850

A.8.17.4.2	Test Requirements	2851

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2851

A.8.17.5.1	Test Purpose and Environment	2851

A.8.17.5.2	Test Requirements	2852

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2852

A.8.17.6.1	Test Purpose and Environment	2852

A.8.17.6.2	Test Requirements	2853

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2854

A.8.17.7.1	Test Purpose and Environment	2854

A.8.17.7.2	Test Requirements	2854

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2855

A.8.17.8.1	Test Purpose and Environment	2855

A.8.17.8.2	Test Requirements	2855

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2856

A.8.17.9.1	Test Purpose and Environment	2856

A.8.17.9.2	Test Requirements	2857

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2857

A.8.17.10.1	Test Purpose and Environment	2857

A.8.17.10.2	Test Requirements	2864

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2865

A.8.17.11.1	Test Purpose and Environment	2865

A.8.17.11.2	Test Requirements	2871

A.8.18	E-UTRAN TDD – HRPD Measurements	2872

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2872

A.8.18.1.1	Test Purpose and Environment	2872

A.8.18.1.2	Test Requirements	2874

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2874

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2874

A.8.19.1.1	Test Purpose and Environment	2874

A.8.19.1.2	Test Requirements	2875

A.8.20	Inter-frequency/RAT Measurements in CA mode	2876

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2876

A.8.20.1.1	Test Purpose and Environment	2876

A.8.20.1.2	Test Requirements	2877

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2877

A.8.20.2.1	Test Purpose and Environment	2878

A.8.20.2.2	Test Requirements	2879

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2880

A.8.20.2A.1	Test Purpose and Environment	2880

A.8.20.2A.2	Test Requirements	2880

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2880

A.8.20.2B.1	Test Purpose and Environment	2880

A.8.20.2B.2	Test Requirements	2883

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2883

A.8.20.3.1	Test Purpose and Environment	2883

A.8.20.3.2	Test Requirements	2885

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2885

A.8.20.4.1	Test Purpose and Environment	2885

A.8.20.4.1.1	1.28 Mcps TDD option	2885

A.8.20.4.2	Test Requirements	2887

A.8.20.4.2.1	1.28 Mcps TDD option	2887

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2887

A.8.20.4A.1	Test Purpose and Environment	2887

A.8.20.4A.1.1	1.28 Mcps TDD option	2887

A.8.20.4A.2	Test Requirements	2887

A.8.20.4A.2.1	1.28 Mcps TDD option	2887

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2888

A.8.20.4B.1	Test Purpose and Environment	2888

A.8.20.4B.1.1	1.28 Mcps TDD option	2888

A.8.20.4B.2	Test Requirements	2890

A.8.20.4B.2.1	1.28 Mcps TDD option	2890

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2890

A.8.21.1	Test Purpose and Environment	2890

A.8.21.2 Test Requirements	2894

A.8.22	E-UTRAN Discovery Signal Measurements	2894

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2894

A.8.22.1.1	Test Purpose and Environment	2894

A.8.22.1.2	Test Requirements	2897

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2897

A.8.22.2.1	Test Purpose and Environment	2897

A.8.22.2.2	Test Requirements	2900

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2900

A.8.22.3.1	Test Purpose and Environment	2900

A.8.22.3.2	Test Requirements	2903

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2903

A.8.22.4.1	Test Purpose and Environment	2903

A.8.22.4.2	Test Requirements	2906

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2906

A.8.22.5.1	Test Purpose and Environment	2906

A.8.22.5.2	Test Requirements	2910

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2910

A.8.22.6.1	Test Purpose and Environment	2910

A.8.22.6.2	Test Requirements	2914

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2914

A.8.22.7.1	Test Purpose and Environment	2914

A.8.22.7.2	Test Requirements	2918

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2918

A.8.22.8.1	Test Purpose and Environment	2918

A.8.22.8.2	Test Requirements	2922

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2922

A.8.22.9.1	Test Purpose and Environment	2922

A.8.22.9.2	Test Requirements	2924

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2925

A.8.22.10.1	Test Purpose and Environment	2925

A.8.22.10.2	Test Requirements	2927

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2928

A.8.22.11.1	Test Purpose and Environment	2928

A.8.22.11.2	Test Requirements	2931

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2931

A.8.22.12.1	Test Purpose and Environment	2931

A.8.22.12.2	Test Requirements	2935

A.8.23	E-UTRAN Dual Connectivity Measurements	2935

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2935

A.8.23.1.1	Test Purpose and Environment	2935

A.8.23.1.2	Test Requirements	2938

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2938

A.8.23.2.1	Test Purpose and Environment	2938

A.8.23.2.2	Test Requirements	2941

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2941

A.8.23.3.1	Test Purpose and Environment	2941

A.8.23.3.2	Test Requirements	2944

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2944

A.8.23.4.1	Test Purpose and Environment	2944

A.8.23.4.2	Test Requirements	2947

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2947

A.8.23.5.1	Test Purpose and Environment	2947

A.8.23.5.2	Test Requirements	2950

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2950

A.8.23.6.1	Test Purpose and Environment	2950

A.8.23.6.2	Test Requirements	2953

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2953

A.8.23.7.1	Test Purpose and Environment	2953

A.8.23.7.2	Test Requirements	2955

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2956

A.8.23.8.1	Test Purpose and Environment	2956

A.8.23.8.2	Test Requirements	2958

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2959

A.8.23.9.1	Test Purpose and Environment	2959

A.8.23.9.2	Test Requirements	2962

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2962

A.8.23.10.1	Test Purpose and Environment	2962

A.8.23.10.2	Test Requirements	2965

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2965

A.8.23.11.1	Test Purpose and Environment	2965

A.8.23.11.2	Test Requirements	2968

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2968

A.8.23.12.1	Test Purpose and Environment	2968

A.8.23.12.2	Test Requirements	2971

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2971

A.8.23.13.1	Test Purpose and Environment	2971

A.8.23.13.2	Test Requirements	2974

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2974

A.8.23.14.1	Test Purpose and Environment	2974

A.8.23.14.2	Test Requirements	2977

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2977

A.8.23.15.1	Test Purpose and Environment	2977

A.8.23.15.2	Test Requirements	2980

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2980

A.8.23.16.1	Test Purpose and Environment	2980

A.8.23.16.2	Test Requirements	2981

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2982

A.8.23.17.1	Test Purpose and Environment	2982

A.8.23.17.2	Test Requirements	2984

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2984

A.8.23.18.1	Test Purpose and Environment	2984

A.8.23.18.2	Test Requirements	2985

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2986

A.8.23.19.1	Test Purpose and Environment	2986

A.8.23.19.2	Test Requirements	2987

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2988

A.8.23.20.1	Test Purpose and Environment	2988

A.8.23.20.2	Test Requirements	2989

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2990

A.8.23.21.1	Test Purpose and Environment	2990

A.8.23.21.2	Test Requirements	2991

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2992

A.8.23.22.1	Test Purpose and Environment	2992

A.8.23.22.2	Test Requirements	2993

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2994

A.8.23.23.1	Test Purpose and Environment	2994

A.8.23.23.2	Test Requirements	2995

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2996

A.8.23.24.1	Test Purpose and Environment	2996

A.8.23.24.2	Test Requirements	2998

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2999

A.8.23.25.1	Test Purpose and Environment	2999

A.8.23.25.2	Test Requirements	3001

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3002

A.8.23.26.1	Test Purpose and Environment	3002

A.8.23.26.2	Test Requirements	3004

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3005

A.8.23.27.1	Test Purpose and Environment	3005

A.8.23.27.2	Test Requirements	3008

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3008

A.8.23.28.1	Test Purpose and Environment	3008

A.8.23.28.2	Test Requirements	3012

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3012

A.8.23.29.1	Test Purpose and Environment	3012

A.8.23.29.2	Test Requirements	3016

A.8.24	Proximity-based Services	3016

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3016

A.8.24.1.1	Test Purpose and Environment	3016

A.8.24.1.2	Test Requirements	3017

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3018

A.8.24.2.1	Test Purpose and Environment	3018

A.8.24.2.2	Test Requirements	3019

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3019

A.8.24.3.1	Test Purpose and Environment	3020

A.8.24.3.2	Test Requirements	3021

A.8.25	E-UTRAN-WLAN Measurements	3022

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3022

A.8.25.1.1	Test Purpose and Environment	3022

A.8.25.1.2	Test Requirements	3024

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3024

A.8.25.2.1	Test Purpose and Environment	3024

A.8.25.2.2	Test Requirements	3026

A.8.26 Frame Structure 3 (FS3)	3026

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3026

A.8.26.1.1	Test Purpose and Environment	3026

A.8.26.1.2	Test Requirements	3028

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3029

A.8.26.2.1	Test Purpose and Environment	3029

A.8.26.2.2	Test Requirements	3031

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3032

A.8.26.3.1	Test Purpose and Environment	3032

A.8.26.3.2	Test Requirements	3035

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3035

A.8.26.3A.1	Test Purpose and Environment	3035

A.8.26.3A.2	Test Requirements	3038

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3038

A.8.26.4.1	Test Purpose and Environment	3038

A.8.26.4.2	Test Requirements	3042

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3042

A.8.26.4A.1	Test Purpose and Environment	3042

A.8.26.4A.2	Test Requirements	3046

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3046

A.8.26.5.1	Test Purpose and Environment	3046

A.8.26.5.2	Test Requirements	3049

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3049

A.8.26.5A.1	Test Purpose and Environment	3049

A.8.26.5A.2	Test Requirements	3053

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3053

A.8.26.6.1	Test Purpose and Environment	3053

A.8.26.6.2	Test Requirements	3057

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3057

A.8.26.6A.1	Test Purpose and Environment	3057

A.8.26.6A.2	Test Requirements	3061

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3061

A.8.26.7.1	Test Purpose and Environment	3061

A.8.26.7.2	Test Requirements	3064

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3064

A.8.26.8.1	Test Purpose and Environment	3064

A.8.26.8.2	Test Requirements	3068

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3068

A.8.26.9.1	Test Purpose and Environment	3068

A.8.26.9.2	Test Requirements	3071

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3071

A.8.26.10.1	Test Purpose and Environment	3071

A.8.26.10.2	Test Requirements	3074

A.9	Measurement Performance Requirements	3076

A.9.1	RSRP	3076

A.9.1.1	FDD Intra frequency case	3076

A.9.1.1.1	Test Purpose and Environment	3076

A.9.1.1.2	Test parameters	3076

A.9.1.1.3	Test Requirements	3079

A.9.1.2	TDD Intra frequency case	3079

A.9.1.2.1	Test Purpose and Environment	3079

A.9.1.2.2	Test parameters	3079

A.9.1.2.3	Test Requirements	3080

A.9.1.3	FDD—FDD Inter frequency case	3081

A.9.1.3.1	Test Purpose and Environment	3081

A.9.1.3.2	Test parameters	3081

A.9.1.3.3	Test Requirements	3084

A.9.1.4	TDD—TDD Inter frequency case	3084

A.9.1.4.1	Test Purpose and Environment	3084

A.9.1.4.2	Test parameters	3084

A.9.1.4.3	Test Requirements	3086

A.9.1.5	FDD—TDD Inter frequency case	3087

A.9.1.5.1	Test Purpose and Environment	3087

A.9.1.5.2	Test parameters	3087

A.9.1.5.3	Test Requirements	3088

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3089

A.9.1.6.1	Test Purpose and Environment	3089

A.9.1.6.2	Test parameters	3089

A.9.1.6.3	Test Requirements	3092

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3092

A.9.1.7.1	Test Purpose and Environment	3092

A.9.1.7.2	Test parameters	3092

A.9.1.7.3	Test Requirements	3095

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3095

A.9.1.8.1	Test Purpose and Environment	3095

A.9.1.8.2	Test parameters	3095

A.9.1.8.3	Test Requirements	3099

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3099

A.9.1.9.1	Test Purpose and Environment	3099

A.9.1.9.2	Test parameters	3099

A.9.1.9.3	Test Requirements	3101

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3102

A.9.1.10.1	Test Purpose and Environment	3102

A.9.1.10.2	Test parameters	3102

A.9.1.10.3	Test Requirements	3105

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3105

A.9.1.11.1	Test Purpose and Environment	3105

A.9.1.11.2	Test parameters	3105

A.9.1.11.3	Test Requirements	3109

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3109

A.9.1.12.1	Test Purpose and Environment	3109

A.9.1.12.2	Test parameters	3109

A.9.1.12.3	Test Requirements	3110

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3110

A.9.1.13.1	Test Purpose and Environment	3110

A.9.1.13.2	Test parameters	3110

A.9.1.13.3	Test Requirements	3111

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3111

A.9.1.14.1	Test Purpose and Environment	3111

A.9.1.14.2	Test parameters	3111

A.9.1.14.3	Test Requirements	3115

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3115

A.9.1.15.1	Test Purpose and Environment	3115

A.9.1.15.2	Test parameters	3115

A.9.1.15.3	Test Requirements	3117

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3118

A.9.1.16.1	Test Purpose and Environment	3118

A.9.1.16.2	Test parameters	3118

A.9.1.16.3	Test Requirements	3119

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3119

A.9.1.17.1	Test Purpose and Environment	3119

A.9.1.17.2	Test parameters	3120

A.9.1.17.3	Test Requirements	3121

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3121

A.9.1.18.1	Test Purpose and Environment	3121

A.9.1.18.2	Test parameters	3121

A.9.1.18.3	Test Requirements	3122

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3123

A.9.1.19.1	Test Purpose and Environment	3123

A.9.1.19.2	Test parameters	3123

A.9.1.19.3	Test Requirements	3123

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3123

A.9.1.20.1	Test Purpose and Environment	3123

A.9.1.20.2	Test parameters	3123

A.9.1.20.3	Test Requirements	3125

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3125

A.9.1.21.1	Test Purpose and Environment	3125

A.9.1.21.2	Test parameters	3125

A.9.1.21.3	Test Requirements	3125

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3125

A.9.1.22.1	Test Purpose and Environment	3125

A.9.1.22.2	Test parameters	3126

A.9.1.22.3	Test Requirements	3130

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3130

A.9.1.23.1	Test Purpose and Environment	3130

A.9.1.23.2	Test parameters	3130

A.9.1.23.3	Test Requirements	3134

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3134

A.9.1.24.1	Test Purpose and Environment	3134

A.9.1.24.2	Test parameters	3134

A.9.1.24.3	Test Requirements	3135

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3135

A.9.1.25.1	Test Purpose and Environment	3135

A.9.1.25.2	Test parameters	3135

A.9.1.25.3	Test Requirements	3138

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3138

A.9.1.26.1	Test Purpose and Environment	3138

A.9.1.26.2	Test parameters	3138

A.9.1.26.3	Test Requirements	3141

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3141

A.9.1.27.1	Test Purpose and Environment	3141

A.9.1.27.2	Test parameters	3141

A.9.1.27.3	Test Requirements	3144

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3144

A.9.1.28.1	Test Purpose and Environment	3144

A.9.1.28.2	Test parameters	3144

A.9.1.28.3	Test Requirements	3147

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3147

A.9.1.29.1	Test Purpose and Environment	3147

A.9.1.29.2	Test parameters	3147

A.9.1.29.3	Test Requirements	3150

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3150

A.9.1.30.1	Test Purpose and Environment	3150

A.9.1.30.2	Test parameters	3151

A.9.1.30.3	Test Requirements	3154

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3154

A.9.1.31.1	Test Purpose and Environment	3154

A.9.1.31.2	Test parameters	3154

A.9.1.31.3	Test Requirements	3157

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3158

A.9.1.32.1	Test Purpose and Environment	3158

A.9.1.32.2	Test parameters	3158

A.9.1.32.3	Test Requirements	3161

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3161

A.9.1.33.1	Test Purpose and Environment	3161

A.9.1.33.2	Test parameters	3161

A.9.1.33.3	Test Requirements	3164

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3164

A.9.1.34.1	Test Purpose and Environment	3164

A.9.1.34.2	Test parameters	3164

A.9.1.34.3	Test Requirements	3167

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3167

A.9.1.35.1	Test Purpose and Environment	3167

A.9.1.35.2	Test parameters	3167

A.9.1.35.3	Test Requirements	3170

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3171

A.9.1.36.1	Test Purpose and Environment	3171

A.9.1.36.2	Test parameters	3171

A.9.1.36.3	Test Requirements	3174

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3174

A.9.1.37.1	Test Purpose and Environment	3174

A.9.1.37.2	Test parameters	3174

A.9.1.37.3	Test Requirements	3177

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3178

A.9.1.38.1	Test Purpose and Environment	3178

A.9.1.38.2	Test parameters	3178

A.9.1.38.3	Test Requirements	3181

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3182

A.9.1.39.1	Test Purpose and Environment	3182

A.9.1.39.2	Test parameters	3182

A.9.1.39.3	Test Requirements	3188

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3188

A.9.1.40.1	Test Purpose and Environment	3188

A.9.1.40.2	Test parameters	3189

A.9.1.40.3	Test Requirements	3195

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3195

A.9.1.41.1	Test Purpose and Environment	3195

A.9.1.41.2	Test parameters	3195

A.9.1.41.3	Test Requirements	3198

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3198

A.9.1.42.1	Test Purpose and Environment	3198

A.9.1.42.2	Test parameters	3198

A.9.1.42.3	Test Requirements	3201

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3201

A.9.1.43.1	Test Purpose and Environment	3201

A.9.1.43.2	Test parameters	3201

A.9.1.43.3	Test Requirements	3202

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3203

A.9.1.44.1	Test Purpose and Environment	3203

A.9.1.44.2	Test parameters	3203

A.9.1.44.3	Test Requirements	3208

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3208

A.9.1.45.1	Test Purpose and Environment	3208

A.9.1.45.2	Test parameters	3208

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2483

A.9.1.54.1	Test Purpose and Environment	2483

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2486

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2486

A.9.1.55.1	Test Purpose and Environment	2486

A.9.1.55.2	Test parameters	2486

A.9.1.55.3	Test Requirements	2490

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2490

A.9.1.56.1	Test Purpose and Environment	2490

A.9.1.56.2	Test parameters	2490

A.9.1.56.3	Test Requirements	2493

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2494

A.9.1.57.1	Test Purpose and Environment	2494

A.9.1.57.2	Test parameters	2494

A.9.1.57.3	Test Requirements	2497

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2497

A.9.1.57A.1	Test Purpose and Environment	2497

A.9.1.57A.2	Test parameters	2497

A.9.1.57A.3	Test Requirements	2498

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2498

A.9.1.58.1	Test Purpose and Environment	2498

A.9.1.58.2	Test parameters	2499

A.9.1.58.3	Test Requirements	2502

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2502

A.9.1.58A.1	Test Purpose and Environment	2502

A.9.1.58A.2	Test parameters	2502

A.9.1.58A.3	Test Requirements	2503

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2503

A.9.1.59.1	Test Purpose and Environment	2503

A.9.1.59.2	Test parameters	2504

A.9.1.59.3	Test Requirements	2506

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2506

A.9.1.60.1	Test Purpose and Environment	2506

A.9.1.60.2	Test parameters	2506

A.9.1.60.3	Test Requirements	2509

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2510

A.9.1.61.1	Test Purpose and Environment	2510

A.9.1.61.2	Test parameters	2510

A.9.1.61.3	Test Requirements	2513

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2513

A.9.1.62.1	Test Purpose and Environment	2513

A.9.1.62.2	Test parameters	2513

A.9.1.62.3	Test Requirements	2516

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2516

A.9.1.53.1	Test Purpose and Environment	2516

A.9.1.53.2	Test parameters	2516

A.9.1.63.3	Test Requirements	2519

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2519

A.9.1.64.1	Test Purpose and Environment	2519

A.9.1.64.2	Test parameters	2519

A.9.1.64.3	Test Requirements	2520

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2521

A.9.1.65.1	Test Purpose and Environment	2521

A.9.1.65.2	Test parameters	2521

A.9.1.65.3	Test Requirements	2527

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2527

A.9.1.66.1	Test Purpose and Environment	2527

A.9.1.66.2	Test parameters	2527

A.9.1.66.3	Test Requirements	2533

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2533

A.9.1.67.1	Test Purpose and Environment	2533

A.9.1.67.2	Test parameters	2533

A.9.1.67.3	Test Requirements	2534

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2535

A.9.1.68.1	Test Purpose and Environment	2535

A.9.1.68.2	Test parameters	2535

A.9.1.68.3	Test Requirements	2539

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2539

A.9.1.69.1	Test Purpose and Environment	2539

A.9.1.69.2	Test parameters	2539

A.9.1.69.3	Test Requirements	2543

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2544

A.9.1.70.1	Test Purpose and Environment	2544

A.9.1.70.2	Test parameters	2544

A.9.1.70.3	Test Requirements	2556

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2557

A.9.1.71.1	Test Purpose and Environment	2557

A.9.1.71.2	Test parameters	2557

A.9.1.71.3	Test Requirements	2569

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2570

A.9.1.72.1	Test Purpose and Environment	2570

A.9.1.72.2	Test parameters	2570

A.9.1.72.3	Test Requirements	2582

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	2583

A.9.1.73.1	Test Purpose and Environment	2583

A.9.1.73.2	Test parameters	2583

A.9.1.73.3	Test Requirements	2586

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	2586

A.9.1.74.1	Test Purpose and Environment	2586

A.9.1.74.2	Test parameters	2586

A.9.1.74.3	Test Requirements	2587

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	2587

A.9.1.75.1	Test Purpose and Environment	2587

A.9.1.75.2	Test parameters	2588

A.9.1.74.3	Test Requirements	2589

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2589

A.9.1.76.1	Test Purpose and Environment	2589

A.9.1.76.2	Test parameters	2589

A.9.1.76.3	Test Requirements	2592

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2592

A.9.1.77.1	Test Purpose and Environment	2592

A.9.1.77.2	Test parameters	2592

A.9.1.77.3	Test Requirements	2595

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2595

A.9.1.78.1	Test Purpose and Environment	2595

A.9.1.78.2	Test parameters	2595

A.9.1.78.3	Test Requirements	2598

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2598

A.9.1.79.1	Test Purpose and Environment	2598

A.9.1.79.2	Test parameters	2598

A.9.1.79.3	Test Requirements	2601

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2601

A.9.1.80.1	Test Purpose and Environment	2601

A.9.1.80.2	Test parameters	2601

A.9.1.80.3	Test Requirements	2604

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2604

A.9.1.81.1	Test Purpose and Environment	2604

A.9.1.78.2	Test parameters	2604

A.9.1.81.3	Test Requirements	2607

A.9.2	RSRQ	2607

A.9.2.1	FDD Intra frequency case	2607

A.9.2.1.1	Test Purpose and Environment	2607

A.9.2.1.2	Test parameters	2607

A.9.2.1.3	Test Requirements	2610

A.9.2.2	TDD Intra frequency case	2610

A.9.2.2.1	Test Purpose and Environment	2610

A.9.2.2.2	Test parameters	2610

A.9.2.2.3	Test Requirements	2612

A.9.2.3	FDD—FDD Inter frequency case	2612

A.9.2.3.1	Test Purpose and Environment	2612

A.9.2.3.2	Test parameters	2612

A.9.2.3.3	Test Requirements	2615

A.9.2.4	TDD—TDD Inter frequency case	2615

A.9.2.4.1	Test Purpose and Environment	2615

A.9.2.4.2	Test parameters	2615

A.9.2.4.3	Test Requirements	2617

A.9.2.4A	FDD—TDD Inter frequency case	2618

A.9.2.4A.1	Test Purpose and Environment	2618

A.9.2.4A.2	Test parameters	2618

A.9.2.4A.3	Test Requirements	2620

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2620

A.9.2.5.1	Test Purpose and Environment	2620

A.9.2.5.2	Test parameters	2621

A.9.2.5.3	Test Requirements	2624

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2624

A.9.2.6.1	Test Purpose and Environment	2624

A.9.2.6.2	Test parameters	2625

A.9.2.6.3	Test Requirements	2628

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2628

A.9.2.7.1	Test Purpose and Environment	2628

A.9.2.7.2	Test parameters	2628

A.9.2.7.3	Test Requirements	2632

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2632

A.9.2.8.1	Test Purpose and Environment	2632

A.9.2.8.2	Test parameters	2632

A.9.2.8.3	Test Requirements	2636

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2636

A.9.2.9.1	Test Purpose and Environment	2636

A.9.2.9.2	Test parameters	2636

A.9.2.9.3	Test Requirements	2640

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2640

A.9.2.10.1	Test Purpose and Environment	2640

A.9.2.10.2	Test parameters	2640

A.9.2.10.3	Test Requirements	2644

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2644

A.9.2.11.1	Test Purpose and Environment	2644

A.9.2.11.2	Test parameters	2644

A.9.2.11.3	Test Requirements	2645

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2645

A.9.2.12.1	Test Purpose and Environment	2645

A.9.2.12.2	Test parameters	2646

A.9.2.12.3	Test Requirements	2646

A.9.2.13	Void	2647

A.9.2.13.1	Void	2647

A.9.2.13.2	Void	2647

A.9.2.13.3	Void	2647

A.9.2.14	Void	2647

A.9.2.14.1	Void	2647

A.9.2.14.2	Void	2647

A.9.2.14.3	Void	2647

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2647

A.9.2.15.1	Test Purpose and Environment	2647

A.9.2.15.2	Test parameters	2647

A.9.2.15.3	Test Requirements	2651

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2651

A.9.2.16.1	Test Purpose and Environment	2651

A.9.2.16.2	Test parameters	2651

A.9.2.16.3	Test Requirements	2656

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2656

A.9.2.17.1	Test Purpose and Environment	2656

A.9.2.17.2	Test parameters	2656

A.9.2.17.3	Test Requirements	2658

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2658

A.9.2.18.1	Test Purpose and Environment	2658

A.9.2.18.2	Test parameters	2658

A.9.2.18.3	Test Requirements	2661

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2661

A.9.2.19.1	Test Purpose and Environment	2661

A.9.2.19.2	Test parameters	2661

A.9.2.19.3	Test Requirements	2663

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2663

A.9.2.20.1	Test Purpose and Environment	2663

A.9.2.20.2	Test parameters	2663

A.9.2.20.3	Test Requirements	2666

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2666

A.9.2.21.1	Test Purpose and Environment	2666

A.9.2.21.2	Test parameters	2666

A.9.2.21.3	Test Requirements	2669

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2669

A.9.2.22.1	Test Purpose and Environment	2670

A.9.2.22.2	Test parameters	2670

A.9.2.22.3	Test Requirements	2670

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2670

A.9.2.23.1	Test Purpose and Environment	2670

A.9.2.23.2	Test parameters	2670

A.9.2.23.3	Test Requirements	2673

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2673

A.9.2.24.1	Test Purpose and Environment	2673

A.9.2.24.2	Test parameters	2673

A.9.2.24.3	Test Requirements	2674

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2674

A.9.2.25.1	Test Purpose and Environment	2674

A.9.2.25.2	Test parameters	2674

A.9.2.25.3	Test Requirements	2679

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2679

A.9.2.26.1	Test Purpose and Environment	2679

A.9.2.26.2	Test parameters	2679

A.9.2.26.3	Test Requirements	2683

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2683

A.9.2.27.1	Test Purpose and Environment	2683

A.9.2.27.2	Test parameters	2683

A.9.2.27.3	Test Requirements	2684

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2684

A.9.2.28.1	Test Purpose and Environment	2684

A.9.2.28.2	Test parameters	2684

A.9.2.28.3	Test Requirements	2687

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2687

A.9.2.29.1	Test Purpose and Environment	2687

A.9.2.29.2	Test parameters	2687

A.9.2.29.3	Test Requirements	2690

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2690

A.9.2.30.1	Test Purpose and Environment	2690

A.9.2.30.2	Test parameters	2690

A.9.2.30.3	Test Requirements	2693

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2693

A.9.2.31.1	Test Purpose and Environment	2693

A.9.2.31.2	Test parameters	2693

A.9.2.31.3	Test Requirements	2696

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2696

A.9.2.32.1	Test Purpose and Environment	2696

A.9.2.32.2	Test parameters	2696

A.9.2.32.3	Test Requirements	2699

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2699

A.9.2.33.1	Test Purpose and Environment	2699

A.9.2.33.2	Test parameters	2699

A.9.2.33.3	Test Requirements	2703

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2703

A.9.2.34.1	Test Purpose and Environment	2703

A.9.2.34.2	Test parameters	2703

A.9.2.34.3	Test Requirements	2706

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2706

A.9.2.35.1	Test Purpose and Environment	2706

A.9.2.35.2	Test parameters	2706

A.9.2.35.3	Test Requirements	2709

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2709

A.9.2.36.1	Test Purpose and Environment	2709

A.9.2.36.2	Test parameters	2709

A.9.2.36.3	Test Requirements	2710

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2711

A.9.2.37.1	Test Purpose and Environment	2711

A.9.2.37.2	Test parameters	2711

A.9.2.37.3	Test Requirements	2712

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2713

A.9.2.38.1	Test Purpose and Environment	2713

A.9.2.38.2	Test parameters	2713

A.9.2.38.3	Test Requirements	2717

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2717

A.9.2.39.1	Test Purpose and Environment	2717

A.9.2.39.2	Test parameters	2717

A.9.2.39.3	Test Requirements	2721

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2721

A.9.2.40.1	Test Purpose and Environment	2721

A.9.2.40.2	Test parameters	2721

A.9.2.40.3	Test Requirements	2724

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2725

A.9.2.41.1	Test Purpose and Environment	2725

A.9.2.41.2	Test parameters	2725

A.9.2.41.3	Test Requirements	2728

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2728

A.9.2.42.1	Test Purpose and Environment	2728

A.9.2.42.2	Test parameters	2728

A.9.2.42.3	Test Requirements	2731

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2731

A.9.2.43.1	Test Purpose and Environment	2731

A.9.2.43.2	Test parameters	2731

A.9.2.43.3	Test Requirements	2734

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2734

A.9.2.44.1	Test Purpose and Environment	2734

A.9.2.44.2	Test parameters	2734

A.9.2.44.3	Test Requirements	2736

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2736

A.9.2.45.1	Test Purpose and Environment	2736

A.9.2.45.2	Test parameters	2736

A.9.2.45.3	Test Requirements	2740

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2741

A.9.2.46.1	Test Purpose and Environment	2741

A.9.2.46.2	Test parameters	2741

A.9.2.46.3	Test Requirements	2745

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2746

A.9.2.47.1	Test Purpose and Environment	2746

A.9.2.47.2	Test parameters	2746

A.9.2.47.3	Test Requirements	2750

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2750

A.9.2.48.1	Test Purpose and Environment	2750

A.9.2.48.2	Test parameters	2751

A.9.2.48.3	Test Requirements	2755

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2756

A.9.2.49.1	Test Purpose and Environment	2756

A.9.2.49.2	Test parameters	2756

A.9.2.49.3	Test Requirements	2762

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2763

A.9.2.50.1	Test Purpose and Environment	2763

A.9.2.50.2	Test parameters	2763

A.9.2.50.3	Test Requirements	2769

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2769

A.9.2.51.1	Test Purpose and Environment	2769

A.9.2.51.2	Test parameters	2770

A.9.2.51.3	Test Requirements	2774

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2774

A.9.2.52.1	Test Purpose and Environment	2774

A.9.2.52.2	Test parameters	2774

A.9.2.52.3	Test Requirements	2777

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2778

A.9.2.53.1	Test Purpose and Environment	2778

A.9.2.53.2	Test parameters	2778

A.9.2.53.3	Test Requirements	2784

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2785

A.9.2.54.1	Test Purpose and Environment	2785

A.9.2.54.2	Test parameters	2785

A.9.2.54.3	Test Requirements	2791

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2791

A.9.2.55.1	Test Purpose and Environment	2791

A.9.2.55.2	Test parameters	2792

A.9.2.55.3	Test Requirements	2797

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2797

A.9.2.56.1	Test Purpose and Environment	2797

A.9.2.56.2	Test parameters	2797

A.9.2.56.3	Test Requirements	2802

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2802

A.9.2.57.1	Test Purpose and Environment	2802

A.9.2.57.2	Test parameters	2803

A.9.2.57.3	Test Requirements	2807

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2808

A.9.2.58.1	Test Purpose and Environment	2808

A.9.2.58.2	Test parameters	2808

A.9.2.58.3	Test Requirements	2812

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2813

A.9.2.59.1	Test Purpose and Environment	2813

A.9.2.59.2	Test parameters	2813

A.9.2.59.3	Test Requirements	2818

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	2819

A.9.2.60.1	Test Purpose and Environment	2819

A.9.2.60.2	Test parameters	2819

A.9.2.60.3	Test Requirements	2822

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	2822

A.9.2.61.1	Test Purpose and Environment	2822

A.9.2.61.2	Test parameters	2822

A.9.2.61.3	Test Requirements	2825

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	2825

A.9.2.62.1	Test Purpose and Environment	2825

A.9.2.62.2	Test parameters	2825

A.9.2.62.3	Test Requirements	2828

A.9.3	UTRAN FDD CPICH RSCP	2828

A.9.3.1	E-UTRAN FDD	2828

A.9.3.1.1	Test Purpose and Environment	2828

A.9.3.1.2	Parameters	2828

A.9.3.1.3	Test Requirements	2831

A.9.3.2 E-UTRAN TDD	2831

A.9.3.2.1	Test Purpose and Environment	2831

A.9.3.2.2	Parameters	2831

A.9.3.2.3	Test Requirements	2834

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2834

A.9.3.3.1	Test Purpose and Environment	2834

A.9.3.3.2	Parameters	2834

A.9.3.3.3	Test Requirements	2835

A.9.4	UTRAN FDD CPICH Ec/No	2836

A.9.4.1	E-UTRAN FDD	2836

A.9.4.1.1	Test Purpose and Environment	2836

A.9.4.1.2	Parameters	2836

A.9.4.1.3	Test Requirements	2838

A.9.4.2	E-UTRAN TDD	2839

A.9.4.2.1	Test Purpose and Environment	2839

A.9.4.2.2	Parameters	2839

A.9.4.2.3	Test Requirements	2841

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2842

A.9.4.3.1	Test Purpose and Environment	2842

A.9.4.3.2	Parameters	2842

A.9.4.3.3	Test Requirements	2843

A.9.5	UTRAN TDD measurement	2843

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2843

A.9.5.1.1	Test Purpose and Environment	2843

A.9.5.1.2	Test parameters	2843

A.9.5.1.3	Test Requirements	2845

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2845

A.9.5.2.1	Test Purpose and Environment	2845

A.9.5.2.2	Test parameters	2845

A.9.5.2.3	Test Requirements	2847

A.9.6	GSM Carrier RSSI	2848

A.9.6.1	E-UTRAN FDD	2848

A.9.6.1.1	Test Purpose and Environment	2848

A.9.6.1.2	Test Requirements	2849

A.9.6.2	E-UTRAN TDD	2849

A.9.6.2.1	Test Purpose and Environment	2849

A.9.6.2.2	Test Requirements	2851

A.9.7	UE Rx – Tx Time Difference	2852

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2852

A.9.7.1.1	Test Purpose and Environment	2852

A.9.7.1.2	Test parameters	2852

A.9.7.1.3	Test Requirements	2853

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2853

A.9.7.2.1	Test Purpose and Environment	2853

A.9.7.2.2	Test parameters	2853

A.9.7.2.3	Test Requirements	2855

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2855

A.9.7.3.1	Test Purpose and Environment	2855

A.9.7.3.2	Test parameters	2855

A.9.7.3.3	Test Requirements	2858

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2858

A.9.7.4.1	Test Purpose and Environment	2858

A.9.7.4.2	Test Parameters	2858

A.9.7.4.3	Test Requirements	2861

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2861

A.9.7.5.1	Test Purpose and Environment	2861

A.9.7.5.2	Test parameters	2861

A.9.7.5.3	Test Requirements	2864

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2864

A.9.7.6.1	Test Purpose and Environment	2864

A.9.7.6.2	Test Parameters	2864

A.9.7.6.3	Test Requirements	2867

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2867

A.9.7.7.1	Test Purpose and Environment	2867

A.9.7.7.2	Test parameters	2867

A.9.7.7.3	Test Requirements	2868

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2869

A.9.7.8.1	Test Purpose and Environment	2869

A.9.7.8.2	Test parameters	2869

A.9.7.8.3	Test Requirements	2870

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2870

A.9.7.9.1	Test Purpose and Environment	2870

A.9.7.9.2	Test parameters	2870

A.9.7.9.3	Test Requirements	2871

A.9.8	RSTD	2872

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2872

A.9.8.1.1	Test Purpose and Environment	2872

A.9.8.1.2	Test Requirements	2874

A.9.8.1.2A	Test Requirements for UE Category 1bis	2874

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2875

A.9.8.2.1	Test Purpose and Environment	2875

A.9.8.2.2	Test Requirements	2878

A.9.8.2.2A	Test Requirements for UE Category 1bis	2878

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2879

A.9.8.3.1	Test Purpose and Environment	2879

A.9.8.3.2	Test Requirements	2881

A.9.8.3.2A	Test Requirements for UE Category 1bis	2881

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2882

A.9.8.4.1	Test Purpose and Environment	2882

A.9.8.4.2	Test Requirements	2884

A.9.8.4.2A	Test Requirements for UE Category 1bis	2884

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2885

A.9.8.5.1	Test Purpose and Environment	2885

A.9.8.5.2	Test Requirements	2887

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2887

A.9.8.6.1	Test Purpose and Environment	2887

A.9.8.6.2	Test Requirements	2890

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2890

A.9.8.7.1	Test Purpose and Environment	2890

A.9.8.7.2	Test Requirements	2891

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2891

A.9.8.8.1	Test Purpose and Environment	2891

A.9.8.8.2	Test Requirements	2892

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2892

A.9.8.9.1	Test Purpose and Environment	2892

A.9.8.9.2	Test Requirements	2893

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2893

A.9.8.10.1	Test Purpose and Environment	2893

A.9.8.10.2	Test Requirements	2894

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2894

A.9.8.11.1	Test Purpose and Environment	2894

A.9.8.11.2	Test Requirements	2895

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2895

A.9.8.12.1	Test Purpose and Environment	2895

A.9.8.12.2	Test Requirements	2896

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2896

A.9.8.13.1	Test Purpose and Environment	2896

A.9.8.13.2	Test Requirements	2897

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2897

A.9.8.14.1	Test Purpose and Environment	2897

A.9.8.14.2	Test Requirements	2904

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2904

A.9.8.15.1	Test Purpose and Environment	2904

A.9.8.15.2	Test Requirements	2910

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2910

A.9.8.16.1	Test Purpose and Environment	2910

A.9.8.16.2	Test Requirements	2913

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2913

A.9.8.17.1	Test Purpose and Environment	2913

A.9.8.17.2	Test Requirements	2916

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2916

A.9.8.18.1	Test Purpose and Environment	2916

A.9.8.18.2	Test Requirements	2919

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2919

A.9.8.19.1	Test Purpose and Environment	2919

A.9.8.19.2	Test Requirements	2922

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2922

A.9.8.20.1	Test Purpose and Environment	2922

A.9.8.20.2	Test Requirements	2925

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2925

A.9.8.21.1	Test Purpose and Environment	2925

A.9.8.21.2	Test Requirements	2929

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2930

A.9.8.22.1	Test Purpose and Environment	2930

A.9.8.22.2	Test Requirements	2933

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2934

A.9.8.23.1	Test Purpose and Environment	2934

A.9.8.23.2	Test Requirements	2937

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2937

A.9.8.24.1	Test Purpose and Environment	2937

A.9.8.24.2	Test Requirements	2941

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2941

A.9.8.25.1	Test Purpose and Environment	2941

A.9.8.25.2	Test Requirements	2945

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2946

A.9.8.26.1	Test Purpose and Environment	2946

A.9.8.26.2	Test Requirements	2948

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2948

A.9.8.27.1	Test Purpose and Environment	2948

A.9.8.27.2	Test Requirements	2951

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2951

A.9.8.28.1	Test Purpose and Environment	2951

A.9.8.28.2	Test Requirements	2954

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2954

A.9.8.29.1	Test Purpose and Environment	2954

A.9.8.29.2	Test Requirements	2958

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2959

A.9.8.30.1	Test Purpose and Environment	2959

A.9.8.30.2	Test Requirements	2962

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2963

A.9.8.31.1	Test Purpose and Environment	2963

A.9.8.31.2	Test Requirements	2967

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2967

A.9.8.32.1	Test Purpose and Environment	2967

A.9.8.32.2	Test Requirements	2970

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2970

A.9.8.33.1	Test Purpose and Environment	2970

A.9.8.33.2	Test Requirements	2973

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2973

A.9.8.34.1	Test Purpose and Environment	2973

A.9.8.34.2	Test Requirements	2976

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2976

A.9.8.35.1	Test Purpose and Environment	2976

A.9.8.35.2	Test Requirements	2979

A.9.9	RSRP and RSRQ on the serving cell	2979

A.9.9.1	FDD Intra frequency serving cell case	2979

A.9.9.1.1	Test Purpose and Environment	2979

A.9.9.1.2	Test parameters	2979

A.9.9.1.3	Test Requirements	2982

A.9.9.2	TDD Intra frequency serving cell case	2982

A.9.9.2.1	Test Purpose and Environment	2982

A.9.9.2.2	Test parameters	2982

A.9.9.2.3	Test Requirements	2984

A.9.10	SSTD	2984

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2984

A.9.10.1.1	Test Purpose and Environment	2984

A.9.10.1.2	Test parameters	2984

A.9.10.1.3	Test Requirements	2986

A.9.10.2	Void	2986

A.9.10.3	Void	2986

A.9.10.4	Void	2986

A.9.11 RSSI	2986

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2986

A.9.11.1.1	Test Purpose and Environment	2986

A.9.11.1.2	Test parameters	2986

A.9.11.1.3	Test Requirements	2989

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2989

A.9.11.2.1	Test Purpose and Environment	2989

A.9.11.2.2	Test parameters	2989

A.9.12	Channel occupancy	2992

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2992

A.9.12.1.1	Test Purpose and Environment	2992

A.9.12.1.2	Test parameters	2992

A.9.12.1.3	Test Requirements	2995

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2995

A.9.12.2.1	Test Purpose and Environment	2995

A.9.12.2.2	Test parameters	2995

A.9.12.2.3	Test Requirements	2998

A.9.13	RS-SINR	2998

A.9.13.1	FDD Intra-Frequency Case	2998

A.9.13.1.1	Test Purpose and Environment	2998

A.9.13.1.2	Test parameters	2998

A.9.13.1.3	Test Requirements	3001

A.9.13.2	TDD Intra-Frequency Case	3001

A.9.13.2.1	Test Purpose and Environment	3001

A.9.13.2.2	Test parameters	3001

A.9.13.2.3	Test Requirements	3004

A.9.13.3	FDD—FDD Inter frequency case	3004

A.9.13.3.1	Test Purpose and Environment	3004

A.9.13.3.2	Test parameters	3004

A.9.13.3.3	Test Requirements	3007

A.9.13.4	TDD—TDD Inter frequency case	3007

A.9.13.4.1	Test Purpose and Environment	3007

A.9.13.4.2	Test parameters	3007

A.9.13.4.3	Test Requirements	3011

A.9.13.5	FDD—TDD Inter frequency case	3012

A.9.13.5.1	Test Purpose and Environment	3012

A.9.13.5.2	Test parameters	3012

A.9.13.5.3	Test Requirements	3018

A.9.13.6	TDD—FDD Inter frequency case	3018

A.9.13.6.1	Test Purpose and Environment	3018

A.9.13.6.2	Test parameters	3018

A.9.13.6.3	Test Requirements	3022

A.9.14	Channel quality reporting accuracy	3022

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3022

A.9.14.1.1	Test Purpose and Environment	3022

A.9.14.1.2	Test parameters	3023

A.9.14.1.3	Test Requirements	3023

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3024

A.9.14.2.1	Test Purpose and Environment	3024

A.9.14.2.2	Test parameters	3024

A.9.14.2.3	Test Requirements	3025

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3025

A.9.14.3.1	Test Purpose and Environment	3025

A.9.14.3.2	Test parameters	3025

A.9.14.3.3	Test Requirements	3026

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3026

A.9.14.4.1	Test Purpose and Environment	3026

A.9.14.4.2	Test parameters	3026

A.9.14.4.3	Test Requirements	3027

A.10	Proximity-based Services in Any Cell Selection State	3050

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3050

A.10.1.1	Test Purpose and Environment	3050

A.10.1.2	Test Requirements	3051

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3052

A.10.2.1	Test Purpose and Environment	3052

A.10.2.2	Test Requirements	3053

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3053

A.10.3.1	Test Purpose and Environment	3053

A.10.3.2	Test Requirements	3055

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3056

A.10.4.1	Test Purpose and Environment	3056

A.10.4.2	Test Requirements	3058

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3058

A.11.1	V2V UE Transmission Timing Accuracy Test	3058

A.11.1.1	Test Purpose and Environment	3058

A.11.1.2	Test requirements	3059

A.11.2	Interruptions due to V2V sidelink communication	3059

A.11.2.1	Test Purpose and Environment	3059

A.11.2.2	Test Requirements	3061

A.12	3061

A.12.1	V2X UE Transmission Timing Accuracy Test	3061

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3061

A.12.1.1.1	Test Purpose and Environment	3061

A.12.1.1.2	Test requirements	3062

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3063

A.12.1.2.1	Test Purpose and Environment	3063

A.12.1.2.2	Test Requirements	3063

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3064

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3064

A.12.2.1.1	Test Purpose and Environment	3064

A.12.2.1.2	Test Requirements	3065

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3066

A.12.2.2.1	Test Purpose and Environment	3066

A.12.2.2.2	Test Requirements	3067

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3067

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3067

A.12.3.1.1	Test Purpose and Environment	3067

A.12.3.1.2	Test Requirements	3070

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3071

A.12.3.2.1	Test Purpose and Environment	3071

A.12.3.1.2	Test Requirements	3073

A.12.4	Congestion Control Measurement Test for V2X UE	3073

A.12.4.1	Test Purpose and Environment	3073

A.12.4.2	Test Requirements	3075

A.12.5	Interruptions due to V2X Sidelink Communication	3075

A.12.5.1	Test Purpose and Environment	3075

A.12.5.2	Test Requirements	3077

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3077

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3077

A.12.6.1.1	Test Purpose and Environment	3077

A.12.6.1.2	Test Requirements	3079

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3079

A.12.6.2.1	Test Purpose and Environment	3079

A.12.6.1.2	Test Requirements	3081

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3081

A.12.7.1	Test Purpose and Environment	3081

A.12.7.2	Test Requirements	3084

A.12.8	Interruptions due to V2X Carrier Aggregation	3085

A.12.8.1	Interruptions on a FDD PCell	3085

A.12.8.1.1	Test Purpose and Environment	3085

A.12.8.1.2	Test Requirements	3086

A.12.8.2	Interruptions on a TDD PCell	3086

A.12.8.2.1	Test Purpose and Environment	3086

A.12.8.2.2	Test Requirements	3088

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3089

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3089

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3089

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3089

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3089

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3091

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3092

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3092

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3092

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3092

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3094

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3095

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3095

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3095

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3096

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3096

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3097

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3097

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3097

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3098

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3098

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3099

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3099

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3099

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3099

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3100

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3100

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3100

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3101

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3101

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3101

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3102

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3102

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3103

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3104

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3105

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3106

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3107

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3107

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3107

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3107

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3108

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3108

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3110

B.3	Conditions for measurements performance requirements for UE	3110

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3110

B.3.2	Void	3110

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3110

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3110

B.3.5	Conditions for UE Rx – Tx time difference	3110

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3111

B.3.7	Conditions for inter-frequency RSTD measurements	3111

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3111

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3111

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3111

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3112

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3112

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3112

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3112

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3112

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3112

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3113

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3113

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3113

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3114

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3114

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3114

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3114

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3114

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3114

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3114

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3114

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3114

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3115

B.3.21.1	Conditions for RSRP measurements	3115

B.3.21.2	Conditions for RSRQ measurements	3115

B.3.21.3	Conditions for CSI-RSRP measurements	3115

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3115

B.3.22.1	Conditions for RSRP measurements	3115

B.3.22.2	Void	3116

B.3.22.3	Conditions for CSI-RSRP measurements	3116

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3116

B.3.23.1	Conditions for RSRP measurements	3116

B.3.23.2	Conditions for RSRQ measurements	3116

B.3.23.3	Conditions for CSI-RSRP measurements	3116

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3117

B.3.24.1	Conditions for RSRP measurements	3117

B.3.24.2	Conditions for RSRQ measurements	3117

B.3.24.3	Conditions for CSI-RSRP measurements	3117

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3117

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3117

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3117

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3118

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3118

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3118

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3118

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3118

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3119

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3119

B.4	RRM Requirements Exceptions	3119

B.4.1	General	3119

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3119

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3119

B.4.3.1	Inter-band carrier aggregation	3119

B.4.3.2	Intra-band non-contiguous carrier aggregation	3119

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3120

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3120

B.5.1	Conditions for S-RSRP Accuracy Requirements	3120

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3120

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3121

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3121

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3122

B.6	Conditions for V2X	3122

B.6.1	Test parameters for GNSS signals	3122

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3122

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3122

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3123

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3123

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3123

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3123

Annex C (informative): Change history:	3125