| 3GPP TS 38.133 V17.5.0 (2022-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	52

3.6.1	RRC connected state requirements in DRX	53

3.6.2	Number of serving carriers	53

3.6.2.1	Number of serving carriers for SA	53

3.6.2.2	Number of serving carriers for EN-DC	53

3.6.2.3	Number of serving carriers for NE-DC	53

3.6.2.4	Number of serving carriers for NR-DC	54

3.6.3	Applicability for intra-band FR2	54

3.6.4	Applicability for FR2 UE power classes	54

3.6.5	Applicability for SDL bands	54

3.6.6	Applicability of requirements for NGEN-DC operation	54

3.6.7	Applicability of QCL	54

3.6.9	Applicability of requirements for scheduling availability	55

3.6.10	Applicability of requirements for measurement restrictions	55

4	SA: RRC_IDLE state mobility	55

4.1	Cell Selection	55

4.2	Cell Re-selection	55

4.2.1	Introduction	55

4.2.2	Requirements	56

4.2.2.1	UE measurement capability	56

4.2.2.2	Measurement and evaluation of serving cell	56

4.2.2.3	Measurements of intra-frequency NR cells	57

4.2.2.4	Measurements of inter-frequency NR cells	58

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	60

4.2.2.6	Maximum interruption in paging reception	62

4.2.2.7	General requirements	62

4.2.2.8	Minimum requirement at transitions	63

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	63

4.2.2.9.1	Introduction	63

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	63

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	64

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	65

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	65

4.2.2.10.1	Introduction	65

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	65

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	66

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	67

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	68

4.2.2.11.1	Introduction	68

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	68

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	69

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	70

4.2A	Cell Re-selection when subject to CCA	70

4.2A.1	Introduction	70

4.2A.2	Requirements	71

4.2A.2.1	UE measurement capability	71

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	71

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	72

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	73

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	75

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	75

4.2A.2.7	General requirements	75

4.3	Minimization of Drive Tests (MDT)	76

4.3.1	Introduction	76

4.3.2	Measurement Requirements	76

4.3.3	Requirements for Relative Time Stamp Accuracy	76

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	77

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	77

4.4	Idle Mode CA/DC Measurements	77

4.4.1	Introduction	77

4.4.2	Measurement Requirements	77

4.4.2.1	Detected cell requirement during state transition and Idle mode	77

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	78

4.4.2.3	Measurements on serving cell	79

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	79

5	SA: RRC_INACTIVE state mobility	81

5.1	Cell Re-selection	81

5.1.1	Introduction	81

5.1.2	Requirements	81

5.1.2.1	UE measurement capability	81

5.1.2.2	Measurement and evaluation of serving cell	81

5.1.2.3	Measurements of intra-frequency NR cells	81

5.1.2.4	Measurements of inter-frequency NR cells	81

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	81

5.1.2.6	Maximum interruption in paging reception	81

5.1.2.7	General requirements	81

5.1A	Cell Re-selection with CCA	81

5.1A.1	Introduction	81

5.1A.2	Requirements	82

5.1A.2.1	UE measurement capability	82

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	82

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	82

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	82

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	82

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	82

5.1A.2.7	General requirements	82

5.2	Void	82

5.3	Minimization of Drive Tests (MDT)	82

5.3.1	Introduction	82

5.3.2	Measurement Requirements	83

5.3.3	Requirements for Relative Time Stamp Accuracy	83

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	83

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	83

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	83

5.4	Idle Mode CA/DC Measurements	83

5.4.1	Introduction	83

5.4.2	Measurement Requirements	83

5.4.2.1	Detected cell requirement during state transition and Idle mode	84

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	84

5.4.2.3	Measurements on serving cell	84

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	84

6	RRC_CONNECTED state mobility	84

6.1	Handover	84

6.1.1	NR Handover	84

6.1.1.1	Introduction	84

6.1.1.2	NR FR1 - NR FR1 Handover	84

6.1.1.2.1	Handover delay	84

6.1.1.2.2	Interruption time	84

6.1.1.3	NR FR2- NR FR1 Handover	85

6.1.1.3.1	Handover delay	85

6.1.1.3.2	Interruption time	85

6.1.1.4	NR FR2- NR FR2 Handover	86

6.1.1.4.1	Handover delay	86

6.1.1.4.2	Interruption time	86

6.1.1.5	NR FR1- NR FR2 Handover	87

6.1.1.5.1	Handover delay	87

6.1.1.5.2	Interruption time	87

6.1.2	NR Handover to other RATs	88

6.1.2.1	NR – E-UTRAN Handover	88

6.1.2.1.1	Introduction	88

6.1.2.1.2	Handover delay	88

6.1.2.1.3	Interruption time	88

6.1.2.2	NR – UTRAN Handover	89

6.1.2.2.1	Introduction	89

6.1.2.2.2	Handover delay	89

6.1.2.2.3	Interruption time	89

6.1.3	NR DAPS Handover	90

6.1.3.1	Introduction	90

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	90

6.1.3.2.1	DAPS handover delay	90

6.1.3.2.2	Interruption time	91

6.1.3.3	NR FR2- NR FR1 DAPS Handover	93

6.1.3.3.1	DAPS handover delay	93

6.1.3.3.2	Interruption time	93

6.1.3.4	NR FR1- NR FR2 DAPS Handover	94

6.1.3.4.1	DAPS handover delay	94

6.1.3.4.2	Interruption time	94

6.1.4	NR Conditional Handover	95

6.1.4.1	Introduction	95

6.1.4.2	NR FR1 – NR FR1 conditional handover	95

6.1.4.3	NR FR2 – NR FR1 conditional handover	96

6.1.4.4	NR FR2 – NR FR2 conditional handover	97

6.1.4.4.1	Handover delay	97

6.1.4.4.2	Measurement time	97

6.1.4.4.3	Preparation time	97

6.1.4.4.4	Interruption time	97

6.1.4.5	NR FR1 – NR FR2 conditional handover	98

6.1A	Void	98

6.1A.1	Void	98

6.1A.1.1	Void	98

6.1A.1.2	Void	98

6.1A.1.2.1	Void	98

6.1A.1.2.2	Void	98

6.1B	Handover to target cell using CCA	98

6.1B.1	NR Handover	98

6.1B.1.1	Introduction	98

6.1B.1.2	NR FR1 - NR FR1 Handover	99

6.1B.1.2.1	Handover delay	99

6.1B.1.2.2	Interruption time	99

6.2	RRC Connection Mobility Control	100

6.2.1	SA: RRC Re-establishment	100

6.2.1.1	Introduction	100

6.2.1.2	Requirements	100

6.2.1.2.1	UE Re-establishment delay requirement	100

6.2.1A	RRC Re-establishment with CCA	101

6.2.1A.1	Introduction	101

6.2.1A.2	Requirements	102

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	102

6.2.2	Random access	103

6.2.2.1	Introduction	103

6.2.2.2	Requirements for 4-step RA type	104

6.2.2.2.1	Contention based random access	104

6.2.2.2.2	Non-Contention based random access	105

6.2.2.2.3	UE behaviour when configured with supplementary UL	106

6.2.2.3	Requirements for 2-step RA type	106

6.2.2.3.1	Contention based random access	106

6.2.2.3.2	Non-Contention based random access	107

6.2.2.3.3	UE behaviour when configured with supplementary UL	108

6.2.2A	Random access when CCA is used on target frequency	108

6.2.2A.1	Introduction	108

6.2.2A.2	Requirements for 4-step RA type	108

6.2.2A.2.1	Contention based random access	108

6.2.2A.2.2	Non-Contention based random access	109

6.2.2A.3	Requirements for 2-step RA type	110

6.2.2A.3.1	Contention based random access	111

6.2.2A.3.2	Non-Contention based random access	112

6.2.3	SA: RRC Connection Release with Redirection	113

6.2.3.1	Introduction	113

6.2.3.2	Requirements	113

6.2.3.2.1	RRC connection release with redirection to NR	113

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	114

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	114

7	Timing	115

7.1	UE transmit timing	115

7.1.1	Introduction	115

7.1.2	Requirements	116

7.1.2.1	Gradual timing adjustment	117

7.1.2.2	Void	117

7.2	UE timer accuracy	117

7.2.1	Introduction	117

7.2.2	Requirements	118

7.3	Timing advance	118

7.3.1	Introduction	118

7.3.2	Requirements	118

7.3.2.1	Timing Advance adjustment delay	118

7.3.2.2	Timing Advance adjustment accuracy	118

7.4	Cell phase synchronization accuracy	118

7.4.1	Definition	118

7.4.2	Minimum requirements	118

7.5	Maximum Transmission Timing Difference	119

7.5.1	Introduction	119

7.5.2	Minimum Requirements for inter-band EN-DC	119

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	119

7.5.3	Minimum Requirements for intra-band EN-DC	120

7.5.4	Minimum Requirements for NR Carrier Aggregation	120

7.5.5	Minimum Requirements for inter-band NE-DC	120

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	121

7.5.6	Minimum Requirements for inter-band NR DC	121

7.6	Maximum Receive Timing Difference	122

7.6.1	Introduction	122

7.6.2	Minimum Requirements for inter-band EN-DC	122

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	122

7.6.3	Minimum Requirements for intra-band EN-DC	123

7.6.4	Minimum Requirements for NR Carrier Aggregation	123

7.6.5	Minimum Requirements for inter-band NE-DC	124

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	124

7.6.6	Minimum Requirements for inter-band NR DC	125

7.7	deriveSSB-IndexFromCell tolerance	125

7.7.1	Minimum requirements	125

7.8	Void	126

8	Signalling characteristics	126

8.1	Radio Link Monitoring	126

8.1.1	Introduction	126

8.1.2	Requirements for SSB based radio link monitoring	127

8.1.2.1	Introduction	127

8.1.2.2	Minimum requirement	128

8.1.2.3	Measurement restrictions for SSB based RLM	129

8.1.3	Requirements for CSI-RS based radio link monitoring	130

8.1.3.1	Introduction	130

8.1.3.2	Minimum requirement	131

8.1.3.3	Measurement restrictions for CSI-RS based RLM	133

8.1.4	Minimum requirement at transitions	133

8.1.5	Minimum requirement for UE turning off the transmitter	134

8.1.6	Minimum requirement for L1 indication	134

8.1.7	Scheduling availability of UE during radio link monitoring	134

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	134

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	134

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	135

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	135

8.1A	Radio Link Monitoring with CCA on Target Frequency	135

8.1A.1	Introduction	135

8.1A.2	Requirements for SSB Based Radio Link Monitoring	136

8.1A.2.1	Introduction	136

8.1A.2.2	Minimum Requirement	137

8.1A.3	Minimum requirement at transitions	138

8.1A.4	Minimum requirement for UE turning off the transmitter	139

8.1A.5	Minimum requirement for L1 indication	139

8.1A.6	Scheduling availability of UE during radio link monitoring	139

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	139

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	139

8.2	Interruption	140

8.2.1	EN-DC Interruption	140

8.2.1.1	Introduction	140

8.2.1.2	Requirements	140

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	140

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	141

8.2.1.2.3	Interruptions at SCell addition/release	141

8.2.1.2.4	Interruptions at SCell activation/deactivation	142

8.2.1.2.5	Interruptions during measurements on SCC	143

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	144

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	145

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	146

8.2.1.2.9	Interruptions at SCell hibernation	146

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	147

8.2.1.2.11	Interruptions due to UE-specific CBW change	147

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	147

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	148

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	149

8.2.1.2.15	Interruptions due to SCell dormancy	150

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps	150

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	151

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	151

8.2.2.1	Introduction	151

8.2.2.2	Requirements	152

8.2.2.2.1	Interruptions at SCell addition/release	152

8.2.2.2.2	Interruptions at SCell activation/deactivation	153

8.2.2.2.3	Interruptions during measurements on deactivated SCC	154

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	155

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	155

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	156

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	157

8.2.2.2.8	Interruptions due to UE-specific CBW change	157

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	158

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	159

8.2.2.2.11	Interruptions at direct SCell activation	160

8.2.2.2.12	Interruptions due to SCell dormancy	160

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	160

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	161

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	161

8.2.3	NE-DC Interruptions	162

8.2.3.1	Introduction	162

8.2.3.2	Requirements	162

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	162

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	163

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	163

8.2.3.2.4	Interruptions at SCell activation/deactivation	164

8.2.3.2.5	Interruptions during measurements on SCC	165

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	166

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	167

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	167

8.2.3.2.9	Interruptions at SCell hibernation	167

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	168

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	168

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	169

8.2.3.2.13	Interruptions due to SCell dormancy	170

8.2.3.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	171

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	171

8.2.4	NR-DC: Interruptions	172

8.2.4.1	Introduction	172

8.2.4.2	Requirements	172

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	172

8.2.4.2.2	Interruptions at SCell activation/deactivation	173

8.2.4.2.3	Interruptions during measurements on SCC	174

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	174

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	175

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	175

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	175

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	175

8.2.4.2.9	Interruptions at NR SRS carrier based switching	176

8.2.4.2.10	Interruptions at direct SCell activation	177

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps	177

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	178

8.2.4.2.13	 Interruptions due to SCell dormancy	179

8.2.4.2A	Void	179

8.2.4.2A.1	Void	179

8.2.4.2A.2	Void	179

8.2.4.2A.3	Void	179

8.3	SCell Activation and Deactivation Delay	179

8.3.1	Introduction	179

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	179

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	184

8.3.4	Direct SCell Activation at SCell addition	184

8.3.5	Direct SCell Activation at Handover	186

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	187

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	191

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	191

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	193

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	194

8.3A.1	Introduction	194

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	194

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	197

8.4	UE UL carrier RRC reconfiguration delay	198

8.4.1	Introduction	198

8.4.2	UE UL carrier configuration delay requirement	198

8.4.3	UE UL carrier deconfiguration delay requirement	198

8.5	Link Recovery Procedures	198

8.5.1	Introduction	198

8.5.2	Requirements for SSB based beam failure detection	199

8.5.2.1	Introduction	199

8.5.2.2	Minimum requirement	199

8.5.2.3	Measurement restriction for SSB based beam failure detection	201

8.5.3	Requirements for CSI-RS based beam failure detection	201

8.5.3.1	Introduction	201

8.5.3.2	Minimum requirement	202

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	204

8.5.4	Minimum requirement for L1 indication	205

8.5.5	Requirements for SSB based candidate beam detection	205

8.5.5.1	Introduction	205

8.5.5.2	Minimum requirement	205

8.5.5.3	Measurement restriction for SSB based candidate beam detection	207

8.5.6	Requirements for CSI-RS based candidate beam detection	208

8.5.6.1	Introduction	208

8.5.6.2	Minimum requirement	208

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	210

8.5.7	Scheduling availability of UE during beam failure detection	211

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	211

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	211

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	211

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	212

8.5.8	Scheduling availability of UE during candidate beam detection	212

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	212

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	212

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	212

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	213

8.5.9	Requirements for Beam Failure Recovery in SCell	213

8.5.9.1	Introduction	213

8.5.9.2	Requirement	213

8.5.10	Minimum requirement at transitions for beam failure detection	214

8.5A	Link Recovery Procedures when CCA is used on target frequency	214

8.5A.1	Introduction	214

8.5A.2	Requirements for SSB based beam failure detection	215

8.5A.2.1	Introduction	215

8.5A.2.2	Minimum requirement	215

8.5A.2.3	Measurement restriction for SSB based beam failure detection	216

8.5A.4	Minimum requirement for L1 indication	216

8.5A.5	Requirements for SSB based candidate beam detection	216

8.5A.5.1	Introduction	216

8.5A.5.2	Minimum requirement	216

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	217

8.5A.7	Scheduling availability of UE during beam failure detection	217

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	217

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	217

8.5A.8	Scheduling availability of UE during candidate beam detection	217

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	218

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	218

8.6	Active BWP switch delay	218

8.6.1	Introduction	218

8.6.2	DCI and timer based BWP switch delay on a single CC	218

8.6.2A	DCI based BWP switch delay on multiple CCs	219

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	219

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	221

8.6.2B	Timer based BWP switch delay on multiple CCs	221

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	221

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	222

8.6.3	RRC based BWP switch delay on a single CC	222

8.6.3A	RRC based BWP switch delay on multiple CCs	223

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	223

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	223

8.6.4	BWP switch delay on Consistent UL CCA recovery	224

8.7	Void	224

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	224

8.8.1	Introduction	224

8.8.2	E-UTRAN PSCell Addition Delay Requirement	224

8.8.3	E-UTRAN PSCell Release Delay Requirement	225

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure [2]8.9	NR-DC: PSCell Addition and Release Delay	225

8.9.1	Introduction	225

8.9.2	PSCell Addition Delay Requirement	225

8.9.3	PSCell Release Delay Requirement	226

8.10	Active TCI state switching delay	226

8.10.4	DCI based TCI state switch delay	228

8.10.5	RRC based TCI state switch delay	228

8.10.6	Active TCI state list update delay	228

8.10A	Active TCI state switching delay with CCA	229

8.10A.1	Introduction	229

8.10A.2	Known conditions for TCI state	229

8.10A.3	MAC-CE based TCI state switch delay	229

8.10A.4	DCI based TCI state switch delay	230

8.10A.5	RRC based TCI state switch delay	230

8.10A.6	Active TCI state list update delay	231

8.11	PSCell Change	231

8.11A	void	232

8.11B	Conditional PSCell Change	232

8.11B.1	Introduction	232

8.11B.2	Conditoinal PSCell Change delay	232

8.11B.2.1	Measurement time	232

8.12	Uplink spatial relation switch delay	233

8.12.1	Introduction	233

8.12.2	Known conditions for spatial relation when associated with DL-RS	233

8.12.3	MAC-CE based spatial relation switch delay	233

8.12.4	DCI based spatial relation switch delay	234

8.12.5	RRC based spatial relation switch delay	234

8.13	UE-specific CBW change	235

8.13.1	Introduction	235

8.13.2	UE-specific CBW change delay	235

8.14	Pathloss reference signal switching delay	235

8.14.1	Introduction	235

8.14.2	Known conditions for pathloss reference signal	235

8.14.3	MAC-CE based pathloss reference signal switch delay	236

9	Measurement Procedure	236

9.1	General measurement requirement	236

9.1.1	Introduction	236

9.1.2	Measurement gap	237

9.1.2.1	EN-DC: Measurement Gap Sharing	248

9.1.2.1a	SA: Measurement Gap Sharing	248

9.1.2.1b	NE-DC: Measurement Gap Sharing	249

9.1.2.1c	NR-DC: Measurement Gap Sharing	250

9.1.3	UE Measurement capability	251

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	251

9.1.3.1a	SA: Monitoring of multiple layers using gaps	251

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	252

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	252

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	253

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	254

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	254

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	255

9.1A.3.2	Void	256

9.1.3A	UE Measurement capability under operation mode with CCA	256

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	256

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	256

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	256

9.1A.3.2a	Void	257

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	257

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	257

9.1.4.1	Introduction	257

9.1.4.2	Requirements	257

9.1.5	Carrier-specific scaling factor	261

9.1.5.1	Monitoring of multiple layers outside gaps	261

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	263

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	264

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	265

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	266

9.1.5.2	Monitoring of multiple layers within gaps	267

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	268

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	270

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	271

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	273

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	275

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	275

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	275

9.1.6	Minimum requirement at transitions	275

9.2	NR intra-frequency measurements	276

9.2.1	Introduction	276

9.2.2	Requirements applicability	276

9.2.3	Number of cells and number of SSB	277

9.2.3.1	Requirements for FR1	277

9.2.3.2	Requirements for FR2	277

9.2.4	Measurement Reporting Requirements	277

9.2.4.1	Periodic Reporting	277

9.2.4.2	Event-triggered Periodic Reporting	277

9.2.4.3	Event Triggered Reporting	277

9.2.5	Intrafrequency measurements without measurement gaps	278

9.2.5.1	Intrafrequency cell identification	278

9.2.5.2	Measurement period	281

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	282

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	283

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	283

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	283

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	284

9.2.5.4	SFTD Measurements between PCell and PSCell	284

9.2.5.4.1	Introduction	284

9.2.5.4.2	SFTD Measurement delay	285

9.2.5.4.3	SFTD Measurement Reporting Delay	285

9.2.6	Intra-frequency measurements with measurement gaps	285

9.2.6.1	Void	285

9.2.6.2	Intra-frequency cell identification	285

9.2.6.3	Intrafrequency Measurement Period	287

9.2A	NR intra-frequency measurements with CCA	288

9.2A.1	Introduction	288

9.2A.2	Requirements applicability	289

9.2A.3	Number of cells and number of SSB	289

9.2A.4	Measurement Reporting Requirements	289

9.2A.5	Intra-frequency measurements without measurement gaps	290

9.2A.5.2	Measurement period	292

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	293

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	293

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	294

9.2A.6	Intra-frequency measurements with measurement gaps	294

9.2A.6.1	Intra-frequency cell identification	294

9.2A.6.2	Intra-frequency Measurement Period	295

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	296

9.2A.7.1	Intra-frequency RSSI measurements	296

9.2A.7.2	Intra-frequency Channel occupancy measurements	297

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	298

9.3	NR inter-frequency measurements	298

9.3.1	Introduction	298

9.3.2	Requirements applicability	299

9.3.2.1	Void	299

9.3.2.2	Void	299

9.3.3	Number of cells and number of SSB	299

9.3.3.1	Requirements for FR1	299

9.3.3.2	Requirements for FR2	299

9.3.4	Inter-frequency measurement with measurement gaps	299

9.3.4.1	Void	301

9.3.4.2	Void	301

9.3.5	Inter-frequency measurements	301

9.3.5.1	Void	301

9.3.5.2	Void	301

9.3.5.3	Void	301

9.3.6	Inter-frequency measurements reporting requirements	301

9.3.6.1	Periodic Reporting	301

9.3.6.2	Event-triggered Periodic Reporting	302

9.3.6.3	Event-triggered Reporting	302

9.3.7	Void	302

9.3.8	Inter-frequency SFTD measurement requirements	302

9.3.8.1	Introduction	302

9.3.8.2	SFTD Measurement delay	302

9.3.8.3	SFTD Measurement reporting delay	303

9.3.9	Inter frequency measurements without measurement gaps	304

9.3.9.1	Inter frequency Cell identification	304

9.3.9.2	Measurement period	305

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	306

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	306

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	307

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	307

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	307

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	308

9.3A.1	Introduction	308

9.3A.2	Requirements applicability	308

9.3A.3	Number of cells and number of SSB	308

9.3A.3.1	Requirements	308

9.3A.4	Inter-frequency cell identification	309

9.3A.5	Inter-frequency measurements	310

9.3A.6	NR Inter-frequency measurements reporting requirements	311

9.3A.6.1	Periodic Reporting	311

9.3A.6.2	Event-triggered Periodic Reporting	311

9.3A.6.3	Event-triggered Reporting	311

9.3A.8	Inter-frequency RSSI measurements	311

9.3A.9	Inter-frequency channel occupancy measurements	312

9.4	Inter-RAT measurements	312

9.4.1	Introduction	312

9.4.2	NR − E-UTRAN FDD measurements	314

9.4.2.1	Introduction	314

9.4.2.2	Requirements when no DRX is used	314

9.4.2.3	Requirements when DRX is used	315

9.4.2.4	Measurement reporting requirements	316

9.4.2.4.1	Periodic Reporting	316

9.4.2.4.2	Event-Triggered Periodic Reporting	316

9.4.2.4.3	Event-Triggered Reporting	316

9.4.3	NR − E-UTRAN TDD measurements	316

9.4.3.1	Introduction	316

9.4.3.2	Requirements when no DRX is used	316

9.4.3.3	Requirements when DRX is used	317

9.4.3.4	Measurement reporting requirements	319

9.4.3.4.1	Periodic Reporting	319

9.4.3.4.2	Event-Triggered Periodic Reporting	319

9.4.3.4.3	Event-Triggered Reporting	319

9.4.4	Inter-RAT RSTD measurements	319

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	319

9.4.4.1.1	Introduction	319

9.4.4.1.2	Requirements	320

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	323

9.4.4.2.1	Introduction	323

9.4.4.2.2	Requirements	324

9.4.5	Inter-RAT E-CID measurements	327

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	327

9.4.5.1.1	Introduction	327

9.4.5.1.2	Requirements	327

9.4.5.1.3	Measurement Reporting Delay	327

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	327

9.4.5.2.1	Introduction	327

9.4.5.2.2	Requirements	327

9.4.5.2.3	Measurement Reporting Delay	328

9.4.6	NR − UTRAN FDD measurements	328

9.4.6.1	Introduction	328

9.4.6.2	Requirements when no DRX is used	328

9.4.6.3	Requirements when DRX is used	329

9.4.7	NR – E-UTRAN measurements with autonomous gaps	331

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	331

The requirement for identifying a new CGI of an E-UTRA cell within Tidentify_CGI, E-UTRAN is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [2] is used.9.4.7.2	CGI reporting delay	331

9.5	L1-RSRP measurements for Reporting	332

9.5.1	Introduction	332

9.5.2	Requirements applicability	332

9.5.3	Measurement Reporting Requirements	332

9.5.3.1	Periodic Reporting	333

9.5.3.2	Semi-Persistent Reporting	333

9.5.3.3	Aperiodic Reporting	333

9.5.4	L1-RSRP measurement requirements	333

9.5.4.1	SSB based L1-RSRP Reporting	333

9.5.4.2	CSI-RS based L1-RSRP Reporting	335

9.5.4A	Void	338

9.5.4A.1	Void	338

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	338

9.5.5.1	Measurement restriction for SSB based L1-RSRP	338

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	339

9.5.6	Scheduling availability of UE during L1-RSRP measurement	339

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	339

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	339

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	340

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	341

9.5A	L1-RSRP measurements for Reporting under CCA	341

9.5A.1	Introduction	341

9.5A.2	Requirements applicability	341

9.5A.3	Measurement Reporting Requirements	341

9.5A.3.1	Periodic Reporting	342

9.5A.3.2	Semi-Persistent Reporting	342

9.5A.3.3	Aperiodic Reporting	342

9.5A.4	L1-RSRP measurement requirements	342

9.5A.4.1	SSB based L1-RSRP Reporting	342

9.5A.5	Measurement restriction for L1-RSRP measurement	343

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	343

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	344

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	344

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	344

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	344

9.6	NE-DC: Measurements	344

9.6.1	Introduction	344

9.6.2	SFTD Measurements	344

9.6.2.1	Introduction	344

9.6.2.2	SFTD Measurement requirements	344

9.7	Cross Link Interference measurements	345

9.7.1	Introduction	345

9.7.2	SRS-RSRP measurements	345

9.7.2.1	Introduction	345

9.7.2.2	Requirements applicability	346

9.7.2.3	Measurement Reporting Requirements	346

9.7.2.3.1	Periodic Reporting	346

9.7.2.3.2	Event-triggered Periodic Reporting	346

9.7.2.3.3	Event Triggered Reporting	346

9.7.2.4	Measurement capability	346

9.7.2.5	SRS-RSRP measurement period	346

9.7.3	CLI-RSSI measurements	347

9.7.3.1	Introduction	347

9.7.3.2	Requirements applicability	347

9.7.3.3	Measurement Reporting Requirements	347

9.7.3.3.1	Periodic Reporting	347

9.7.3.3.2	Event-triggered Periodic Reporting	347

9.7.3.3.3	Event Triggered Reporting	347

9.7.3.4	Measurement capability	348

9.7.3.5	CLI-RSSI measurement period	348

9.7.4	Scheduling availability of UE during CLI measurements	348

9.7.4.1	Scheduling availability of UE performing measurement on FR1	348

9.7.4.2	Scheduling availability of UE performing measurement on FR2	349

9.8	L1-SINR measurements for Reporting	349

9.8.1	Introduction	349

9.8.2	Requirements applicability	350

9.8.3	Measurement Reporting Requirements	350

9.8.3.1	Periodic Reporting	350

9.8.3.2	Semi-Persistent Reporting	351

9.8.4	L1-SINR measurement requirements	351

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	351

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	354

9.8.5	Measurement restriction for L1-SINR measurement	356

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	356

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	357

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	357

9.8.6	Scheduling availability of UE during L1-SINR measurement	358

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	358

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	358

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	359

9.9	 NR measurements for positioning	359

9.9.1	Introduction	359

9.9.2	RSTD measurements	360

9.9.2.1	Introduction	360

9.9.2.2	Requirements Applicability	360

9.9.2.3	Measurement Capability	360

9.9.2.4	Measurement Reporting Requirements	360

9.9.2.4.1	Void	360

9.9.2.4.2	Void	360

9.9.2.4.3	Void	360

9.9.2.5	Measurements Period Requirements	360

9.9.2.6	Void	362

9.9.3	PRS-RSRP measurements	362

9.9.3.1	Introduction	362

9.9.3.2	Requirements applicability	362

9.9.3.3	Measurement Capability	363

9.9.3.4	Measurement Reporting Requirements	363

9.9.3.5	Measurement Period Requirements	363

9.9.4	UE Rx-Tx time difference measurements	365

9.9.4.1 Introduction	365

9.9.4.2 Requirements Applicability	365

9.9.4.3 Measurement Capability	365

9.9.4.4 Measurement Reporting Requirements	365

9.9.4.5	Measurement Period Requirements	365

9.9.5.1	Introduction	367

9.9.5.2	Measurement Requirements	367

9.9.5.2.1	Intra-frequency Measurement Requirements	367

9.9.5.2.2	Inter-frequency Measurement Requirements	368

9.9.5.2.3	Measurement Reporting Delay	368

9.10	CSI-RS based L3 measurements	368

9.10.1	Introduction	368

9.10.2	CSI-RS based intra-frequency measurements	369

9.10.2.1	Introduction	369

9.10.2.2	Requirements applicability	369

9.10.2.3	Number of cells and number of CSI-RS	370

9.10.2.3.1	Requirements for FR1	370

9.10.2.3.2	Requirements for FR2	370

9.10.2.4	Measurement Reporting Requirements	371

9.10.2.4.1	Periodic Reporting	371

9.10.2.4.2	Event-triggered Periodic Reporting	371

9.10.2.4.3	Event Triggered Reporting	371

9.10.2.5	Intra-frequency measurements without measurement gaps	371

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	372

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	373

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	373

9.10.3	CSI-RS based Inter-frequency measurements	373

9.10.3.1	Introduction	373

9.10.3.2	Requirements applicability	373

9.10.3.3	Number of cells and number of CSI-RS resources	374

9.10.3.3.1	Requirements for FR1	374

9.10.3.3.2	Requirements for FR2	374

9.10.3.4	Measurements reporting requirements	374

9.10.3.4.1	Periodic Reporting	374

9.10.3.4.2	Event-triggered Periodic Reporting	374

9.10.3.4.3	Event-triggered Reporting	375

9.10.3.5	Inter frequency measurements with measurement gaps	375

9.11	NR measurements with autonomous gaps	376

9.11.1	Introduction	376

9.11.2	CGI identification of an NR cell with autonomous gaps	376

9.11.3	CGI reporting delay	377

10	Measurement Performance requirements	377

10.1	NR measurements	377

10.1.1	Introduction	377

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	378

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	378

10.1.2.1.1	Absolute SS-RSRP Accuracy	378

10.1.2.1.2	Relative SS-RSRP Accuracy	379

10.1.2.2	Void	380

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	380

10.1.2.3.1	Absolute CSI-RSRP Accuracy	380

10.1.2.3.2	Relative CSI-RSRP Accuracy	381

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	382

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	382

10.1.2B.1.1	Absolute SS-RSRP Accuracy	382

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	383

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	383

10.1.3.1.1	Absolute SS-RSRP Accuracy	383

10.1.3.1.2	Relative SS-RSRP Accuracy	384

10.1.3.2	Void	385

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	385

10.1.3.3.1	Absolute CSI-RSRP Accuracy	385

10.1.3.3.2	Relative CSI-RSRP Accuracy	385

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	386

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	386

10.1.3B.1.1	Absolute SS-RSRP Accuracy	386

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	387

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	387

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	387

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	388

10.1.4.2	Void	389

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	389

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	389

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	390

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	391

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	391

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	391

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	392

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	392

10.1.5.1.1	Absolute SS-RSRP Accuracy	392

10.1.5.1.2	Relative SS-RSRP Accuracy	393

10.1.5.2	Void	394

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	394

10.1.5.3.1	Absolute CSI-RSRP Accuracy	394

10.1.5.3.2	Relative CSI-RSRP Accuracy	394

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	395

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	395

10.1.5B.1.1	Absolute SS-RSRP Accuracy	395

10.1.6 RSRP Measurement Report Mapping	396

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	398

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	398

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	398

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	399

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	399

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	400

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	400

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	400

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	401

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	401

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	401

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	402

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	402

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	403

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	403

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	403

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	404

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	404

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	404

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	405

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	406

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	406

10.1.9.2.2	Relative CSI-RSRQ Accuracy	407

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	408

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	408

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	408

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	409

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	411

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	411

10.1.10.2.2	Relative CSI-RSRQ Accuracy	411

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	412

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	412

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	412

10.1.11	RSRQ report mapping	413

10.1.12	Intra-frequency SINR accuracy requirements for FR1	413

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	414

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	414

10.1.13	Intra-frequency SINR accuracy requirements for FR2	415

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	416

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	416

10.1.14	Inter-frequency SINR accuracy requirements for FR1	417

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	419

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	419

10.1.15	Inter-frequency SINR accuracy requirements for FR2	421

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	423

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	423

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	423

10.1.16	SINR report mapping	424

10.1.17	Power Headroom	425

10.1.18	PCMAX,c,f	426

10.1.19	L1-RSRP accuracy requirements for FR1	426

10.1.20	L1-RSRP accuracy requirements for FR2	430

10.1.21	SFTD accuracy requirements	433

10.1.22	CLI measurement accuracy requirements	437

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	441

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	441

10.1.23	RSTD Measurements	443

10.1.23.1	Introduction	443

10.1.23.2	Measurement Accuracy Requirements	444

10.1.23.3	Report mapping	448

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	448

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	450

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	452

10.1.24	PRS-RSRP Measurements	454

10.1.24.1	Introduction	454

10.1.24.2	Measurement Accuracy Requirements	454

10.1.24.2.1 Absolute PRS RSRP accuracy	454

10.1.24.2.2 Relative PRS RSRP accuracy	456

10.1.24.3	Report mapping	458

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	458

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	459

10.1.25	UE Rx-Tx Time Difference Measurements	461

10.1.25.1	Introduction	461

10.1.25.2	Measurement Accuracy Requirements	462

10.1.25.3	Report mapping	469

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	469

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	471

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	473

10.1.26	FR2 P-MPR report	475

10.1.26.1	Report mapping	475

10.1.27	L1-SINR accuracy requirements for FR1	475

10.1.28	L1-SINR accuracy requirements for FR2	483

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	490

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	490

10.1.29.1.1	Absolute SS-RSRQ Accuracy	490

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	490

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	490

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	490

10.1.30.1.2	Relative Accuracy of SS-RSRQ	491

10.1.31	Intra-frequency SINR accuracy requirements under CCA	492

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	492

10.1.31.1.1	Absolute SS-SINR Accuracy	492

10.1.32	Inter-frequency SINR accuracy requirements under CCA	492

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	492

10.1.32.1.1	Aboslute Accuracy of SS-SINR	492

10.1.32.1.2	Relative Accuracy of SS-SINR	493

10.1.33	L1-RSRP accuracy requirements under CCA	494

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	494

10.1.33.1.1	Absolute Accuracy	494

10.1.33.1.2	Relative Accuracy	494

10.1.34	RSSI measurements under CCA	495

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	495

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	495

10.1.34.3	RSSI measurement report mapping	495

10.1.35	Channel occupancy measurements under CCA	496

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	496

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	496

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	496

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	496

10.1.36.1.1	Absolute SS-RSRP Accuracy	496

10.1.36.1.2	Relative SS-RSRP Accuracy	497

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	498

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	498

10.1.37.1.1	Absolute Accuracy of SS-RSRP	498

10.1.37.1.2	Relative Accuracy of SS-RSRP	498

10.2	E-UTRAN measurements	499

10.2.1	Introduction	499

10.2.2	E-UTRAN RSRP measurements	499

10.2.3	E-UTRAN RSRQ measurements	499

10.2.4	E-UTRAN RSTD measurements	500

10.2.5	E-UTRAN RS-SINR measurements	500

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	500

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	501

10.3	UTRAN FDD Measurements	501

10.3.1	UTRAN FDD CPICH RSCP	501

10.3.2	UTRAN FDD CPICH Ec/No	502

10.4	V2X measurements	502

10.4.1	Introduction	502

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	502

10.4.2.1	PSBCH-RSRP Absolute Accuracy	502

10.4.2.2	PSBCH-RSRP Relative Accuracy	503

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	504

10.4.3.1	Absolute SL-RSSI Accuracy	504

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	504

10.4.4.1	Absolute L1 SL-RSRP Accuracy	504

11	Void	505

12	V2X Requirements	505

12.1	Introduction	505

12.2	UE Transmit Timing	505

12.2.1	Introduction	505

12.2.2	GNSS as synchronization reference source	506

12.2.3	NR Cell as synchronization reference source	506

12.2.4	E-URTAN Cell as synchronization reference source	506

12.2.5	SyncRef UE as synchronization reference source	507

12.3	Initiation/Cease of SLSS Transmissions	507

12.3.1	Introduction	507

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	507

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	508

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	509

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	509

12.4	Selection / Reselection of V2X Synchronization Reference Source	510

12.5	L1 SL-RSRP measurements	511

12.5.1	Introduction	511

12.5.2	SL-RSRP measurements	511

12.6	Congestion Control measurements	511

12.7	Interruption	511

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	511

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	512

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	514

12.8	Reliability of GNSS signal	514

12.9	Scheduling availability	514

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	514

13	Measurement Performance Requirements for NR gNB	515

13.1	UL-RTOA	515

13.1.1	Report mapping	515

13.2	gNB Rx-Tx time difference	517

13.2.1	Report mapping	517

13.2.2	Measurement Accuracy Requirements	519

13.2.2.1	Introduction	519

13.3	UL SRS RSRP measurement	520

13.3.1	Report mapping	520

13.3.2	Measurement accuracy requirements	521

13.3.2.1	Introduction	521

13.3.2.2	Requirements	521

13.4	AoA/ZoA	522

13.4.1	Report mapping	522

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.2.1.5	Requirements under CCA	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1.2	CORESET for RMSI scheduling	429

A.3.1.2.1	FDD	429

A.3.1.2.2	TDD	430

A.3.1.3	CORESET for RMC scheduling	433

A.3.1.3.1	FDD	433

A.3.1.3.2	TDD	434

A.3.1.4	TDD UL/DL configuration	437

A.3.1A	Reference measurement channels under CCA	439

A.3.1A.1	PDSCH	439

A.3.1A.1.1	TDD	439

A.3.1A.2	CORESET for RMSI scheduling	440

A.3.1A.2.1	TDD	440

A.3.1A.3	CORESET for RMC scheduling	441

A.3.1A.3.1	TDD	441

A.3.1A.4	TDD UL/DL configuration	442

A.3.1A.5	RMC burst transmission model	442

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	442

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	443

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	443

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	444

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	444

A.3.2.2	Void	445

A.3.3	Reference DRX configurations	445

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	445

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	446

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	446

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	446

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	447

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	447

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	447

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	448

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	448

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	448

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	449

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	449

A.3.4	Test Cases with Different Channel Bandwidths	449

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	449

A.3.4.1.1	Introduction	449

A.3.4.1.2	Principle of testing	449

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	450

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	450

A.3.5.1.1	Introduction	450

A.3.5.1.2	Principle of Testing	450

A.3.6	Antenna configurations	450

A.3.6.1	Antenna configurations for FR1	450

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	450

A.3.6.1.1.1	Introduction	450

A.3.6.1.1.2	Principle of testing	450

A.3.6.2	Antenna configurations for FR2	453

A.3.6A	Antenna configurations with unlicensed bands	453

A.3.6A.1	Antenna configurations for FR1	453

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	453

A.3.6A.1.1.1	Introduction	453

A.3.6A.1.1.2	Principle of testing	453

A.3.7	EN-DC test setup	455

A.3.7.1	Introduction	455

A.3.7.2	E-UTRAN Serving Cell Parameters	455

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	455

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	456

A.3.7A	NR FR1-FR2 test setup	457

A.3.7B	EN-DC test setup with unlicensed bands	458

A.3.7B.1	Introduction	458

A.3.7B.2	E-UTRAN Serving Cell Parameters	458

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	458

A.3.7C	LTE-FR1/FR2 test setup	459

A.3.7D	NE-DC test setup	459

A.3.7D.1	Introduction	459

A.3.7D.2	E-UTRAN Serving Cell Parameters	459

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	459

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	459

A.3.8	PRACH configurations	459

A.3.8.1	Introduction	459

A.3.8.2	PRACH configurations in FR1	460

A.3.8.2.1	FR1 PRACH configuration 1	460

A.3.8.2.2	FR1 PRACH configuration 2	460

A.3.8.2.3	FR1 PRACH configuration 3	461

A.3.8.2.4	FR1 PRACH configuration 4	462

A.3.8.3	PRACH configurations in FR2	463

A.3.8.3.1	FR2 PRACH configuration 1	463

A.3.8.3.2	FR2 PRACH configuration 2	464

A.3.8.3.3	FR2 PRACH configuration 3	465

A.3.8.3.4	FR2 PRACH configuration 4	466

A.3.8A	PRACH configurations under CCA	467

A.3.8A.1	Introduction	467

A.3.8A.2	PRACH configurations in FR1	467

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	467

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	468

A.3.9	BWP configurations	469

A.3.9.1	Introduction	469

A.3.9.2	Downlink BWP configurations	470

A.3.9.2.1	Initial BWP	470

A.3.9.2.2	Dedicated BWP	470

A.3.9.3	Uplink BWP configurations	470

A.3.9.3.1	Initial BWP	470

A.3.9.3.2	Dedicated BWP	471

A.3.10	SSB Configurations	471

A.3.10.1	SSB Configurations for FR1	471

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	471

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	473

A.3.10A	SSB Configurations under CCA	478

A.3.10A.1	SSB Configurations under CCA for FR1	478

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	478

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	479

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	479

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	480

A.3.11	SMTC Configurations	480

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	480

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	481

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	481

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	481

A.3.12	Test Cases with Different CC Configurations	481

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	481

A.3.12.1.1	Introduction	481

A.3.12.1.2	Principle of testing	481

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	482

A.3.12.2.1	Introduction	482

A.3.12.2.2	Principle of testing	482

A.3.13	Test Cases in SA and EN-DC Operations	482

A.3.13.1	Introduction	482

A.3.13.2	Principle of Testing	482

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	482

A.3.13A.1	Introduction	482

A.3.13A.2	Principle of Testing in EN-DC	482

A.3.13A.3	Principle of Testing in SA	483

A.3.13A.4	Principle of Testing in E-UTRA	483

A.3.13B	Test Cases for EN-DC and NE-DC Operations	484

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	484

A.3.13B.1.1	Introduction	484

A.3.13B.1.2	Principle of Testing	484

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	484

A.3.13B.2.1	Introduction	484

A.3.13B.2.2	Principle of Testing	484

A.3.14	CSI-RS configurations	484

A.3.14.1	FDD	484

A.3.14.2	TDD	487

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	493

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	493

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	493

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	493

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	494

A.3.15.3	Setup 3: 2 AoAs	494

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	494

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	494

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	494

A.3.16	TCI State Configuration	495

A.3.16.1	Introduction	495

A.3.16.2	TCI states	495

A.3.17	Configurations of CSI-RS for tracking	495

A.3.17.1	Configuration of CSI-RS for tracking for FR1	495

A.3.17.1.1	FDD	495

A.3.17.1.2	TDD	496

A.3.17.2	Configuration of CSI-RS for tracking for FR2	497

A.3.17.2.1	TDD	497

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	498

A.3.18.1	Introduction	498

A.3.18.2	PRACH Power Measurement	498

A.3.19	Test applicability for DAPS handover	498

A.3.19.1	Introduction	498

A.3.19.2	Principle of testing	498

A.3.20	MsgA configurations	499

A.3.20.1	Introduction	499

A.3.20.2	MsgA configurations in FR1	499

A.3.20.2.1	FR1 MsgA configuration 1	499

A.3.20.2.2	FR1 MsgA configuration 2	500

A.3.20.3	MsgA configurations in FR2	502

A.3.20.3.1	FR2 MsgA configuration 1	502

A.3.20.3.2	FR2 MsgA configuration 2	503

A.3.20A	MsgA configurations under CCA	505

A.3.20A.1	Introduction	505

A.3.20A.2	MsgA configurations in FR1	505

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	505

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	506

A.3.21	V2X sidelink communication	509

A.3.21.1	Introduction	509

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	509

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	515

A.3.22	CSI-IM configurations	515

A.3.22.1	FDD	515

A.3.22.2	TDD	516

A.3.23	Spatial Relation Configuration	518

A.3.23.1	Introduction	518

A.3.23.2	Spatial Relation	518

A.3.24	SRS configuration	519

A.3.25	Channel bandwidth (CBW) configurations	521

A.3.25.1	DL UE specific CBW	521

A.3.25.2	UL UE specific CBW	521

A.3.26	CCA model	521

A.3.26.1	Introduction	521

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	521

A.3.26.2.1	DL CCA model	521

A.3.26.2.2	UL CCA model	523

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	523

A.3.27.1	Introduction	524

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	524

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	524

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	524

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	524

A.3.28	Discovery Burst Transmission Window configuration under CCA	524

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	524

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	524

A.3.29.1	Introduction	524

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	524

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	525

A.3.30	CSI-RS configurations for RRM	526

A.3.30.1	FDD	526

A.3.30.2	TDD	527

A.3.31	PRS Configurations	529

A.3.31.1.	PRS Configurations for FR1	529

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	529

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	530

A.3.31.2.	PRS Configurations for FR2	530

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	530

A.4	EN-DC tests with all NR cells in FR1	531

A.4.1	Void	531

A.4.2	Void	531

A.4.3	RRC_CONNECTED state mobility	531

A.4.3.1	Void	531

A.4.3.2	RRC Connection Mobility Control	531

A.4.3.2.1	Void	531

A.4.3.2.2	Random Access	531

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	531

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	535

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	539

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	543

A.4.3.2.3	Void	547

A.4.4	Timing	547

A.4.4.1	UE transmit timing	547

A.4.4.1.1	NR UE Transmit Timing Test for FR1	547

A.4.4.1.1.1	Test Purpose and environment	547

A.4.4.1.1.2	Test requirements	551

A.4.4.2	UE timer accuracy	551

A.4.4.3	Timing advance	551

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	551

A.4.4.3.1.1	Test Purpose and Environment	551

A.4.4.3.1.2	Test Parameters	551

A.4.4.3.1.3	Test Requirements	555

A.4.5	Signaling characteristics	555

A.4.5.1	Radio link Monitoring	555

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	556

A.4.5.1.1.1	Test Purpose and Environment	556

A.4.5.1.1.2	Test Requirements	560

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	560

A.4.5.1.2.1	Test Purpose and Environment	560

A.4.5.1.2.2	Test Requirements	566

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	566

A.4.5.1.3.1	Test Purpose and Environment	566

A.4.5.1.3.2	Test Requirements	571

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	571

A.4.5.1.4.1	Test Purpose and Environment	571

A.4.5.1.4.2	Test Requirements	577

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	577

A.4.5.1.5.1	Test Purpose and Environment	577

A.4.5.1.5.2	Test Requirements	582

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	582

A.4.5.1.6.1	Test Purpose and Environment	582

A.4.5.1.6.2	Test Requirements	588

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	588

A.4.5.1.7.1	Test Purpose and Environment	588

A.4.5.1.7.2	Test Requirements	593

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	594

A.4.5.1.8.1	Test Purpose and Environment	594

A.4.5.1.8.2	Test Requirements	599

A.4.5.2	Interruption	599

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	599

A.4.5.2.1.1	Test Purpose and Environment	599

A.4.5.2.1.2	Test Requirements	603

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	603

A.4.5.2.2.1	Test Purpose and Environment	603

A.4.5.2.2.2	Test Requirements	607

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	607

A.4.5.2.3.1	Test Purpose and Environment	607

A.4.5.2.3.2	Test Requirements	611

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	612

A.4.5.2.4.1	Test Purpose and Environment	612

A.4.5.2.4.2	Test Requirements	615

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	616

A.4.5.2.5.1	Test Purpose and Environment	616

A.4.5.2.5.2	Test Requirements	619

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	619

A.4.5.2.6.1	Test Purpose and Environment	619

A.4.5.2.6.2	Test Requirements	623

A.4.5.2.7	Void	623

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	623

A.4.5.2.8.1	Test Purpose and Environment	623

A.4.5.2.8.2	Test Requirements	626

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	627

A.4.5.2.9.1	Test Purpose and Environment	627

A.4.5.2.9.2	Test Requirements	630

A.4.5.3	SCell Activation and Deactivation Delay	631

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	631

A.4.5.3.1.1	Test Purpose and Environment	631

A.4.5.3.1.2	Test Requirements	636

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle	637

A.4.5.3.2.1	Test Purpose and Environment	637

A.4.5.3.2.2	Test Requirements	637

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	637

A.4.5.3.3.1	Test Purpose and Environment	637

A.4.5.3.3.2	Test Requirements	638

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	638

A.4.5.3.4.1	Test Purpose and Environment	638

A.4.5.3.3.2	Test Requirements	641

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	641

A.4.5.3.5.1	Test Purpose and Environment	641

A.4.5.3.5.2	Test Requirements	646

A.4.5.4	UE UL carrier RRC reconfiguration Delay	646

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	646

A.4.5.4.1.1 Test Purpose and Environment	646

A.4.5.4.1.2	Test Requirements	656

A.4.5.5	Beam Failure Detection and Link recovery procedures	656

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	656

A.4.5.5.1.1	Test Purpose and Environment	656

A.4.5.5.1.2	Test Requirements	662

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	662

A.4.5.5.2.1	Test Purpose and Environment	662

A.4.5.5.2.2	Test Requirements	668

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	668

A.4.5.5.3.1	Test Purpose and Environment	668

A.4.5.5.3.2	Test Requirements	674

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	674

A.4.5.5.4.1	Test Purpose and Environment	674

A.4.5.5.4.2	Test Requirements	680

A.4.5.6	Active BWP switch	694

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	694

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	694

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	699

A.4.5.6.2	RRC-based Active BWP Switch	704

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	708

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	708

A.4.5.6.3.1.1	Test Purpose and Environment	708

A.4.5.6.3.1.2	Test Requirements	713

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	714

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	714

A.4.5.6.4.1.1	Test Purpose and Environment	714

A.4.5.6.4.1.2	Test Requirements	718

A.4.5.6.5	SCell dormancy switch	718

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	718

A.4.5.6.5.1.1	Test Purpose and Environment	718

A.4.5.6.5.1.2	Test Requirements	724

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	724

A.4.5.6.5.2.1	Test Purpose and Environment	724

A.4.5.6.5.2.2	Test Requirements	729

A.4.5.7	PSCell addition and release delay	729

A.4.5.7.1	Addition and Release Delay of known NR PSCell	729

A.4.5.7.1.1	Test purpose and environment	729

A.4.5.7.1.2	Test Requirements	733

A.4.5.8	DL Interruptions at switching between two uplink carriers	734

A.4.5.8.1	Test Purpose and Environment	734

A.4.5.8.2	Test Requirements	737

A.4.6	Measurement procedure	737

A.4.6.1	Intra-frequency Measurements	737

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	737

A.4.6.1.1.1	Test purpose and Environment	737

A.4.6.1.1.2	Test parameters	738

A.4.6.1.1.3	Test Requirements	741

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	741

A.4.6.1.2.1	Test purpose and Environment	741

A.4.6.1.2.2	Test parameters	741

A.4.6.1.2.2	Test Requirements	745

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	745

A.4.6.1.3.1	Test purpose and Environment	745

A.4.6.1.3.2	Test parameters	745

A.4.6.1.3.3	Test Requirements	749

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	749

A.4.6.1.4.1	Test purpose and Environment	749

A.4.6.1.4.2	Test parameters	749

A.4.6.1.4.3	Test Requirements	753

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	753

A.4.6.1.5.1	Test purpose and Environment	753

A.4.6.1.5.2	Test parameters	753

A.4.6.1.5.3	Test Requirements	755

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	756

A.4.6.1.6.1	Test purpose and Environment	756

A.4.6.1.6.2	Test parameters	756

A.4.6.1.6.3	Test Requirements	758

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	759

A.4.6.1.7.1	Test purpose and Environment	759

A.4.6.1.7.2	Test parameters	759

A.4.6.1.7.3	Test Requirements	763

A.4.6.2	Inter-frequency Measurements	763

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	763

A.4.6.2.1.1	Test Purpose and Environment	763

A.4.6.2.1.2	Test Requirements	767

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	767

A.4.6.2.2.1	Test Purpose and Environment	767

A.4.6.2.2.2	Test Requirements	771

A.4.6.2.3	Void	772

A.4.6.2.4	Void	772

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	772

A.4.6.2.5.1	Test Purpose and Environment	772

A.4.6.2.5.2	Test Requirements	776

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	776

A.4.6.2.6.1	Test Purpose and Environment	776

A.4.6.2.6.2	Test Requirements	781

A.4.6.2.7	Void	782

A.4.6.2.8	Void	782

A.4.6.3	Void	782

A.4.6.4	L1-RSRP measurement for beam reporting	782

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	782

A.4.6.4.1.1	Test Purpose and Environment	782

A.4.6.4.1.2	Test parameters	782

A.4.6.4.1.3	Test Requirements	785

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	785

A.4.6.4.2.1	Test Purpose and Environment	785

A.4.6.4.2.2	Test parameters	786

A.4.6.4.2.3	Test Requirements	789

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	789

A.4.6.4.3.1	Test Purpose and Environment	789

A.4.6.4.3.2	Test parameters	790

A.4.6.4.3.3	Test Requirements	793

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	793

A.4.6.4.4.1	Test Purpose and Environment	793

A.4.6.4.4.2	Test parameters	794

A.4.6.4.4.3	Test Requirements	796

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	796

A.4.6.4.5.1	Test Purpose and Environment	796

A.4.6.4.5.2	Test parameters	797

A.4.6.4.2.3	Test Requirements	800

A.4.6.5	CLI measurements	800

A.4.6.5.1	SRS-RSRP measurement with non-DRX	800

A.4.6.5.1.1	Test Purpose and Environment	800

A.4.6.5.1.2	Test Parameters	801

A.4.6.5.1.3	Test Requirements	804

A.4.6.5.2	CLI-RSSI measurement with non-DRX	804

A.4.6.5.2.1	Test Purpose and Environment	804

A.4.6.5.2.2	Test Parameters	805

A.4.6.5.2.3	Test Requirements	806

A.4.6.6.1.2	Test Requirements	811

A.4.6.7	L1-SINR measurement for beam reporting	811

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	814

A.4.6.7.2.1	Test Purpose and Environment	814

A.4.6.7.2.2	Test parameters	814

A.4.6.7.2.3	Test Requirements	817

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	817

A.4.6.7.3.1	Test Purpose and Environment	817

A.4.6.7.3.2	Test parameters	817

A.4.6.7.3.3	Test Requirements	819

A.4.6.8	CSI-RS based intra-frequency Measurement	820

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	820

A.4.6.8.1.1	Test purpose and Environment	820

A.4.6.8.1.2	Test Requirements	824

A.4.6.9	CSI-RS based inter-frequency Measurement	824

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	824

A.4.6.9.1.1	Test Purpose and Environment	824

A.4.6.9.1.2	Test Requirements	828

A.4.7	Measurement Performance requirements	829

A.4.7.1	SS-RSRP	829

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	829

A.4.7.1.1.1	Test Purpose and Environment	829

A.4.7.1.1.2	Test parameters	829

A.4.7.1.1.3	Test Requirements	833

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	833

A.4.7.1.2.1	Test Purpose and Environment	833

A.4.7.1.2.2	Test parameters	834

A.4.7.1.2.3	Test Requirements	838

A.4.7.1.3	Void	838

A.4.7.2	SS-RSRQ	838

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	838

A.4.7.2.1.1	Test Purpose and Environment	838

A.4.7.2.1.2	Test Parameters	839

A.4.7.2.1.3	Test Requirements	844

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	844

A.4.7.2.2.1	Test Purpose and Environment	844

A.4.7.2.2.2	Test Parameters	844

A.4.7.2.2.3	Test Requirements	849

A.4.7.3	SS-SINR	849

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	849

A.4.7.3.1.1	Test Purpose and Environment	849

A.4.7.3.1.2	Test Parameters	849

A.4.7.3.1.3	Test Requirements	853

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	853

A.4.7.3.2.1	Test Purpose and Environment	853

A.4.7.3.2.2	Test Parameters	853

A.4.7.3.2.3	Test Requirements	858

A.4.7.4	L1-RSRP measurement for beam reporting	858

A.4.7.4.1	SSB based L1-RSRP measurement	858

A.4.7.4.1.1	Test Purpose and Environment	858

A.4.7.4.1.2	Test parameters	859

A.4.7.4.1.3	Test Requirements	862

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	862

A.4.7.4.2.1	Test Purpose and Environment	862

A.4.7.4.2.2	Test parameters	863

A.4.7.4.2.3	Test Requirements	866

A.4.7.5	SFTD accuracy	866

A.4.7.5.1	SFTD accuracy	866

A.4.7.5.1.1	Test Purpose and Environment	866

A.4.7.5.1.2	Test Parameters	866

A.4.7.5.1.3	Test Requirements	871

A.4.7.5.2	Void	871

A.4.7.5.3	Void	871

A.4.7.6	CLI measurements	871

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	871

A.4.7.6.1.1	Test Purpose and Environment	871

A.4.7.6.1.2	Test parameters	872

A.4.7.6.1.3	Test Requirements	877

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	877

A.4.7.6.2.1	Test Purpose and Environment	877

A.4.7.6.2.2	Test parameters	878

A.4.7.6.2.3	Test Requirements	881

A.4.7.7	L1-SINR measurement for beam reporting	881

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	885

A.4.7.7.2.1	Test Purpose and Environment	885

A.4.7.7.2.2	Test parameters	885

A.4.7.7.2.3	Test Requirements	889

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	889

A.4.7.7.3.1	Test Purpose and Environment	889

A.4.7.7.3.2	Test parameters	889

A.4.7.7.3.3	Test Requirements	892

A.4.7.8	CSI-RSRP	892

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	892

A.4.7.8.1.1	Test Purpose and Environment	892

A.4.7.8.1.2	Test parameters	893

A.4.7.8.1.3	Test Requirements	898

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	898

A.4.7.8.2.1	Test Purpose and Environment	898

A.4.7.8.2.2	Test parameters	898

A.4.7.8.2.3	Test Requirements	903

A.4.7.9	CSI-RSRQ	903

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	903

A.4.7.9.1.1	Test Purpose and Environment	903

A.4.7.9.1.2	Test Parameters	903

A.4.7.9.1.3	Test Requirements	908

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	908

A.4.7.9.2.1	Test Purpose and Environment	908

A.4.7.9.2.2	Test Parameters	908

A.4.7.9.2.3	Test Requirements	913

A.4.7.10	CSI-SINR	913

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	913

A.4.7.10.1.1	Test Purpose and Environment	913

A.4.7.10.1.2	Test Parameters	914

A.4.7.10.1.3	Test Requirements	918

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	918

A.4.7.10.2.1	Test Purpose and Environment	918

A.4.7.10.2.2	Test Parameters	919

A.4.7.10.2.3	Test Requirements	923

A.4.8	Void	923

A.4A	NE-DC test with all NR cells in FR1	924

A.4A.1	Signaling characteristics	924

A.4A.1.1	E-UTRAN PSCell addition	924

A.4A.1.1.1	Test purpose and environment	924

A.4A.1.1.2	Test Requirements	928

A.4A.1.2	Active BWP switch	929

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	929

A.4A.1.2.1.1	Test Purpose and Environment	929

A.4A.1.2.1.2	Test Requirements	933

A.4A.2	Measurement performance	934

A.4A.2.1	SFTD accuracy	934

A.4A.2.1.1	SFTD accuracy	934

A.4A.2.1.1.1	Test Purpose	934

A.4A.2.1.1.2	Test Environment	934

A.4A.2.1.1.3	Test Requirements	937

A.5	EN-DC tests with one or more NR cells in FR2	938

A.5.1	Void	938

A.5.2	Void	938

A.5.3	RRC_CONNECTED state mobility	938

A.5.3.1	Void	938

A.5.3.2	RRC Connection Mobility Control	938

A.5.3.2.1	Void	938

A.5.3.2.2	Random Access	938

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	938

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	941

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	947

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	950

A.5.3.2.3	Void	954

A.5.4	Timing	954

A.5.4.1	UE transmit timing	954

A.5.4.1.1	NR UE Transmit Timing Test for FR2	954

A.5.4.1.1.1	Test Purpose and environment	954

A.5.4.1.1.2	Test requirements	957

A.5.4.2	UE timer accuracy	958

A.5.4.3	Timing advance	958

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	958

A.5.4.3.1.1 Test Purpose and Environment	958

A.5.4.3.1.2 Test Parameters	958

A.5.4.3.1.3	Test Requirements	962

A.5.5	Signaling characteristics	962

A.5.5.1	Radio link Monitoring	962

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	962

A.5.5.1.1.1	Test Purpose and Environment	962

A.5.5.1.1.2	Test Requirements	965

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	966

A.5.5.1.2.1	Test Purpose and Environment	966

A.5.5.1.2.2	Test Requirements	969

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	970

A.5.5.1.3.1	Test Purpose and Environment	970

A.5.5.1.3.2	Test Requirements	973

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	973

A.5.5.1.4.1	Test Purpose and Environment	973

A.5.5.1.4.2	Test Requirements	977

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	977

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	983

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	988

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	994

A.5.5.1.8.2	Test Requirements	1000

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1001

A.5.5.1.9.1	Test Purpose and Environment	1001

A.5.5.1.9.2	Test Requirements	1003

A.5.5.2	Interruption	1003

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1003

A.5.5.2.1.1	Test Purpose and Environment	1003

A.5.5.2.1.2	Test Requirements	1006

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1006

A.5.5.2.2.1	Test Purpose and Environment	1006

A.5.5.2.2.2	Test Requirements	1009

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1009

A.5.5.2.3.1	Test Purpose and Environment	1009

A.5.5.2.3.2	Test Requirements	1013

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1013

A.5.5.2.4.1	Test Purpose and Environment	1013

A.5.5.2.4.2	Test Requirements	1016

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1017

A.5.5.2.5.1	Test Purpose and Environment	1017

A.5.5.2.5.2	Test Requirements	1020

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1020

A.5.5.2.6.1	Test Purpose and Environment	1020

A.5.5.2.6.2	Test Requirements	1023

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1023

A.5.5.2.7.1	Test Purpose and Environment	1023

A.5.5.2.7.2	Test Requirements	1026

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1027

A.5.5.2.8.1 Test Purpose and Environment	1027

A.5.5.2.8.3	Test Requirements	1029

A.5.5.3	SCell Activation and Deactivation Delay	1029

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1029

A.5.5.3.1.1	Test Purpose and Environment	1029

A.5.5.3.1.2	Test Requirements	1031

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1031

A.5.5.3.2.1	Test Purpose and Environment	1031

A.5.5.3.2.2	Test Requirements	1035

A.5.5.3.3	Void	1035

A.5.5.3.4	Void	1035

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1035

A.5.5.3.5.1	Test Purpose and Environment	1035

A.5.5.3.5.2	Test Requirements	1040

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1041

A.5.5.3.6.1	Test Purpose and Environment	1041

A.5.5.3.6.2	Test Requirements	1044

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1045

A.5.5.3.7.1	Test Purpose and Environment	1045

A.5.5.3.7.2	Test Requirements	1048

A.5.5.4	Void	1048

A.5.5.5	Beam Failure Detection and Link recovery procedures	1048

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1048

A.5.5.5.1.1	Test Purpose and Environment	1048

A.5.5.5.1.2	Test Requirements	1054

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1054

A.5.5.5.2.1	Test Purpose and Environment	1054

A.5.5.5.2.2	Test Requirements	1060

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1060

A.5.5.5.3.1	Test Purpose and Environment	1060

A.5.5.5.3.2	Test Requirements	1066

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1066

A.5.5.5.4.1	Test Purpose and Environment	1066

A.5.5.5.4.2	Test Requirements	1072

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1072

A.5.5.5.5.1	Test Purpose and Environment	1072

A.5.5.5.5.2	Test Requirements	1078

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1078

A.5.5.5.6.1	Test Purpose and Environment	1078

A.5.5.5.6.2	Test Requirements	1083

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1083

A.5.5.5.7.1	Test Purpose and Environment	1083

A.5.5.5.7.2	Test Requirements	1089

A.5.5.6	Active BWP switch	1089

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1089

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1089

A.5.5.6.1.1.1	Test Purpose and Environment	1089

A.5.5.6.1.1.2	Test Requirements	1093

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1093

A.5.5.6.2	RRC-based Active BWP Switch	1098

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1098

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1102

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1102

A.5.5.6.3.1.1	Test Purpose and Environment	1102

A.5.5.6.3.1.2	Test Requirements	1106

A.5.5.6.4	SCell dormancy switch	1106

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1106

A.5.5.6.4.1.1	Test Purpose and Environment	1106

A.5.5.6.4.1.2	Test Requirements	1110

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1110

A.5.5.6.4.2.1	Test Purpose and Environment	1110

A.5.5.6.4.2.2	Test Requirements	1117

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1117

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1117

A.5.5.7	PSCell addition and release delay	1120

A.5.5.7.1	Addition and Release Delay of NR PSCell	1120

A.5.5.7.1.1	Test purpose and environment	1120

A.5.5.7.1.2	Test Requirements	1123

A.5.5.8	Active TCI state switch delay	1124

A.5.5.8.1	MAC-CE based active TCI state switch	1124

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1124

A.5.5.8.1.1.1	Test Purpose and Environment	1124

A.5.5.8.1.1.2	Test Requirements	1126

A.5.5.8.2	RRC based active TCI state switch	1126

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1126

A.5.5.8.2.1.1	Test Purpose and Environment	1126

A.5.5.8.2.1.2	Test Requirements	1129

A.5.5.9	Uplink spatial relation switch delay	1129

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1129

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1129

A.5.5.9.1.1.1	Test Purpose and Environment	1129

A.5.5.9.1.1.2	Test Requirements	1132

A.5.5.9.2	RRC based spatial relation switch	1132

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1132

A.5.5.9.2.1.1	Test Purpose and Environment	1132

A.5.5.9.2.1.2	Test Requirements	1135

A.5.5.10	UE specific CBW change	1135

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1135

A.5.5.10.1.1	Test Purpose and Environment	1135

A.5.5.10.1.2	Test Requirements	1138

A.5.6	Measurement procedure	1139

A.5.6.1	Intra-frequency Measurements	1139

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1139

A.5.6.1.1.1	Test purpose and Environment	1139

A.5.6.1.1.2	Test Requirements	1143

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1143

A.5.6.1.2.1	Test purpose and Environment	1143

A.5.6.1.2.2	Test Requirements	1145

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1146

A.5.6.1.3.1	Test purpose and Environment	1146

A.5.6.1.3.2	Test Requirements	1150

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1150

A.5.6.1.4.1	Test purpose and Environment	1150

A.5.6.1.4.2	Test Requirements	1153

A.5.6.2	Inter-frequency Measurements	1154

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1154

A.5.6.2.1.1	Test Purpose and Environment	1154

A.5.6.2.1.2	Test Requirements	1157

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1157

A.5.6.2.2.1	Test Purpose and Environment	1157

A.5.6.2.2.2	Test Requirements	1161

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1161

A.5.6.2.3.1	Test Purpose and Environment	1161

A.5.6.2.3.2	Test Requirements	1165

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1165

A.5.6.2.4.1	Test Purpose and Environment	1165

A.5.6.2.4.2	Test Requirements	1169

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1169

A.5.6.2.5.1	Test Purpose and Environment	1169

A.5.6.2.5.2	Test Requirements	1174

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1174

A.5.6.2.6.1	Test Purpose and Environment	1174

A.5.6.2.6.2	Test Requirements	1178

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1178

A.5.6.2.7.1	Test Purpose and Environment	1178

A.5.6.2.7.2	Test Requirements	1183

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1183

A.5.6.2.8.1	Test Purpose and Environment	1183

A.5.6.2.8.2	Test Requirements	1188

A.5.6.3	L1-RSRP measurement for beam reporting	1189

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1189

A.5.6.3.1.1	Test Purpose and Environment	1189

A.5.6.3.1.2	Test parameters	1189

A.5.6.3.1.3	Test Requirements	1191

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1191

A.5.6.3.2.1	Test Purpose and Environment	1191

A.5.6.3.2.2	Test parameters	1192

A.5.6.3.2.3	Test Requirements	1194

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1194

A.5.6.3.3.1	Test Purpose and Environment	1194

A.5.6.3.3.2	Test parameters	1195

A.5.6.3.3.3	Test Requirements	1197

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1197

A.5.6.3.4.1	Test Purpose and Environment	1197

A.5.6.3.4.2	Test parameters	1198

A.5.6.3.4.3	Test Requirements	1200

A.5.6.4	CLI measurements	1201

A.5.6.4.1	SRS-RSRP measurement with DRX	1201

A.5.6.4.1.1	Test Purpose and Environment	1201

A.5.6.4.1.2	Test Parameters	1201

A.5.6.4.1.3	Test Requirements	1203

A.5.6.4.2	CLI-RSSI measurement with DRX	1203

A.5.6.4.2.1	Test Purpose and Environment	1203

A.5.6.4.2.2	Test Parameters	1204

A.5.6.4.2.3	Test Requirements	1206

A.5.6.5	Measurements with autonomous gaps	1206

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1206

A.5.6.5.1.1	Test Purpose and Environment	1206

A.5.6.5.1.2	Test Requirements	1209

A.5.6.6	L1-SINR measurement for beam reporting	1210

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1212

A.5.6.6.2.1	Test Purpose and Environment	1212

A.5.6.6.2.2	Test parameters	1213

A.5.6.6.2.3	Test Requirements	1216

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1216

A.5.6.6.3.1	Test Purpose and Environment	1216

A.5.6.6.3.2	Test parameters	1216

A.5.6.6.3.3	Test Requirements	1218

A.5.6.7	CSI-RS based Intra-frequency Measurements	1218

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1218

A.5.6.7.1.1	Test purpose and Environment	1218

A.5.6.7.1.2	Test Requirements	1221

A.5.6.8	CSI-RS based Inter-frequency Measurements	1221

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1221

A.5.6.8.1.1	Test Purpose and Environment	1221

A.5.6.8.1.2	Test Requirements	1226

A.5.7	Measurement Performance requirements	1226

A.5.7.1	SS-RSRP	1226

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1226

A.5.7.1.1.1	Test Purpose and Environment	1226

A.5.7.1.1.2	Test parameters	1227

A.5.7.1.1.3	Test Requirements	1229

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1230

A.5.7.1.2.1	Test Purpose and Environment	1230

A.5.7.1.2.2	Test parameters	1230

A.5.7.1.2.3	Test Requirements	1234

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1235

A.5.7.1.3.1	Test Purpose and Environment	1235

A.5.7.1.3.2	Test parameters	1235

A.5.7.1.3.3	Test Requirements	1238

A.5.7.2	SS-RSRQ	1239

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1239

A.5.7.2.1.1	Test Purpose and Environment	1239

A.5.7.2.1.2	Test Parameters	1239

A.5.7.2.1.3	Test Requirements	1242

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1242

A.5.7.2.2.1	Test Purpose and Environment	1242

A.5.7.2.2.2	Test Parameters	1242

A.5.7.2.2.3	Test Requirements	1244

A.5.7.3	SS-SINR	1245

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1245

A.5.7.3.1.1	Test Purpose and Environment	1245

A.5.7.3.1.2	Test Parameters	1245

A.5.7.3.1.3	Test Requirements	1248

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1248

A.5.7.3.2.1	Test Purpose and Environment	1248

A.5.7.3.2.2	Test Parameters	1248

A.5.7.3.2.3	Test Requirements	1250

A.5.7.4	L1-RSRP measurement for beam reporting	1250

A.5.7.4.1	SSB based L1-RSRP measurement	1250

A.5.7.4.1.1	Test Purpose and Environment	1250

A.5.7.4.1.2	Test parameters	1251

A.5.7.4.1.3	Test Requirements	1253

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1254

A.5.7.4.2.1	Test Purpose and Environment	1254

A.5.7.4.2.2	Test parameters	1254

A.5.7.4.2.3	Test Requirements	1256

A.5.7.5	CLI measurements	1257

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1257

A.5.7.5.1.1	Test Purpose and Environment	1257

A.5.7.5.1.2	Test parameters	1257

A.5.7.5.1.3	Test Requirements	1260

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1261

A.5.7.5.2.1	Test Purpose and Environment	1261

A.5.7.5.2.2	Test parameters	1261

A.5.7.5.2.3	Test Requirements	1263

A.5.7.6	L1-SINR measurement for beam reporting	1264

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1266

A.5.7.6.2.1	Test Purpose and Environment	1266

A.5.7.6.2.2	Test parameters	1267

A.5.7.6.2.3	Test Requirements	1269

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1270

A.5.7.6.3.1	Test Purpose and Environment	1270

A.5.7.6.3.2	Test parameters	1270

A.5.7.6.3.3	Test Requirements	1272

A.5.7.7	CSI-RSRP	1272

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1272

A.5.7.7.1.1	Test Purpose and Environment	1272

A.5.7.7.1.2	Test parameters	1273

A.5.7.7.1.3	Test Requirements	1275

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1276

A.5.7.7.2.1	Test Purpose and Environment	1276

A.5.7.7.2.2	Test parameters	1276

A.5.7.7.2.3	Test Requirements	1280

A.5.7.8	CSI-RSRQ	1281

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1281

A.5.7.8.1.1	Test Purpose and Environment	1281

A.5.7.8.1.2	Test Parameters	1281

A.5.7.8.1.3	Test Requirements	1283

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1283

A.5.7.8.2.1	Test Purpose and Environment	1283

A.5.7.8.2.2	Test Parameters	1283

A.5.7.8.2.3	Test Requirements	1285

A.5.7.9	CSI-SINR	1285

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1285

A.5.7.9.1.1	Test Purpose and Environment	1285

A.5.7.9.1.2	Test Parameters	1285

A.5.7.9.1.3	Test Requirements	1288

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1288

A.5.7.9.2.1	Test Purpose and Environment	1288

A.5.7.9.2.2	Test Parameters	1288

A.5.7.9.2.3	Test Requirements	1291

A.5.8	Void	1291

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1027

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1027

A.6.1.1.5.1	Test Purpose and Environment	1027

A.6.1.1.5.2	Test Parameters	1027

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1032

A.6.1.1.6.1	Test Purpose and Environment	1032

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1041

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1041

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1041

A.6.1.2.1.1	Test Purpose and Environment	1041

A.6.1.2.1.2	Test Parameters	1041

A.6.1.2.1.3	Test Requirements	1044

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1045

A.6.1.2.2.1	Test Purpose and Environment	1045

A.6.1.2.2.2	Test Parameters	1045

A.6.1.2.2.3	Test Requirements	1048

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1049

A.6.1.2.3.1	Test Purpose and Environment	1049

A.6.1.2.3.2	Test Parameters	1049

A.6.1.2.3.3	Test Requirements	1052

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1053

A.6.1.2.4.1	Test Purpose and Environment	1053

A.6.1.2.4.2	Test Parameters	1053

A.6.1.2.4.3	Test Requirements	1056

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	1057

A.6.1.2.5.1	Test Purpose and Environment	1057

A.6.1.2.5.2	Test Parameters	1057

A.6.1.2.5.3	Test Requirements	1061

A.6.1.1.7	Void	1062

A.6.2	SA: RRC_INACTIVE state mobility	1062

A.6.3	RRC_CONNECTED state mobility	1062

A.6.3.1	Handover	1062

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1062

A.6.3.1.1.1	Test Purpose and Environment	1062

A.6.3.1.1.2	Test Parameters	1062

A.6.3.1.1.3 Test Requirements	1066

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1066

A.6.3.1.2.1	Test Purpose and Environment	1066

A.6.3.1.2.2	Test Parameters	1066

A.6.3.1.2.3	Test Requirements	1070

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1070

A.6.3.1.3.1	Test Purpose and Environment	1070

A.6.3.1.3.2	Test Parameters	1070

A.6.3.1.3.3 Test Requirements	1074

A.6.3.1.4	 SA NR - E-UTRAN handover	1074

A.6.3.1.4.1	Test Purpose and Environment	1074

A.6.3.1.4.2	Test Requirements	1080

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1080

A.6.3.1.5.1	Test Purpose and Environment	1080

A.6.3.1.5.2	Test Requirements	1086

A.6.3.1.6	 SA NR - UTRAN FDD handover	1086

A.6.3.1.6.1	Test Purpose and Environment	1086

A.6.3.1.6.2	Test Requirements	1090

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	1090

A.6.3.1.7.1	Test Purpose and Environment	1090

A.6.3.1.7.2	Test Parameters	1090

A.6.3.1.7.3 Test Requirements	1094

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	1095

A.6.3.1.8.1	Test Purpose and Environment	1095

A.6.3.1.8.2	Test Parameters	1095

A.6.3.1.8.3 Test Requirements	1099

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1100

A.6.3.1.9.1	Test Purpose and Environment	1100

A.6.3.1.9.2	Test Parameters	1100

A.6.3.1.9.3 Test Requirements	1104

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1104

A.6.3.1.10.1	Test Purpose and Environment	1104

A.6.3.1.10.2	Test Parameters	1104

A.6.3.1.10.3 Test Requirements	1108

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1108

A.6.3.1.11.1	Test Purpose and Environment	1108

A.6.3.1.11.2	Test Parameters	1108

A.6.3.1.11.3 Test Requirements	1115

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1115

A.6.3.1.12.1	Test Purpose and Environment	1115

A.6.3.1.12.2	Test Parameters	1115

A.6.3.1.12.3 Test Requirements	1122

A.6.3.2	RRC Connection Mobility Control	1122

A.6.3.2.1	SA: RRC Re-establishment	1122

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1122

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1126

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1130

A.6.3.2.2	Random Access	1133

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1133

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1137

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1142

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1147

A.6.3.2.3	SA: RRC Connection Release with Redirection	1151

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1151

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1155

A.6.3.3 Conditional handover	1162

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1162

A.6.3.3.1.1	Test Purpose and Environment	1162

A.6.3.3.1.2	Test Parameters	1162

A.6.3.3.1.3 Test Requirements	1166

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1166

A.6.3.3.2.1	Test Purpose and Environment	1166

A.6.3.3.2.2	Test Parameters	1166

A.6.3.3.2.3 Test Requirements	1170

A.6.4	Timing	1170

A.6.4.1	UE transmit timing	1170

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1170

A.6.4.1.1.1	Test Purpose and environment	1170

A.6.4.1.1.2	Test requirements	1174

A.6.4.2	UE timer accuracy	1174

A.6.4.3	Timing advance	1174

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1174

A.6.4.3.1.1	Test Purpose and Environment	1174

A.6.4.3.1.2	Test Parameters	1174

A.6.4.3.1.3	Test Requirements	1178

A.6.5	Signalling characteristics	1178

A.6.5.1	Radio link Monitoring	1178

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1179

A.6.5.1.1.1	Test Purpose and Environment	1179

A.6.5.1.1.2	Test Requirements	1184

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1184

A.6.5.1.2.1	Test Purpose and Environment	1184

A.6.5.1.2.2	Test Requirements	1190

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1190

A.6.5.1.3.1	Test Purpose and Environment	1190

A.6.5.1.3.2	Test Requirements	1196

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1196

A.6.5.1.4.1	Test Purpose and Environment	1196

A.6.5.1.4.2	Test Requirements	1202

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1202

A.6.5.1.5.1	Test Purpose and Environment	1202

A.6.5.1.5.2	Test Requirements	1208

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1208

A.6.5.1.6.1	Test Purpose and Environment	1208

A.6.5.1.6.2	Test Requirements	1214

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1214

A.6.5.1.7.1	Test Purpose and Environment	1214

A.6.5.1.7.2	Test Requirements	1219

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1219

A.6.5.1.8.1	Test Purpose and Environment	1219

A.6.5.1.8.2	Test Requirements	1225

A.6.5.2	Interruption	1225

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1225

A.6.5.2.1.2	Test Requirements	1229

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1230

A.6.5.2.2.1	Test Purpose and Environment	1230

A.6.5.2.2.2	Test Parameters	1230

A.6.5.2.2.3	Test Requirements	1232

A.6.5.3	SCell Activation and Deactivation Delay	1233

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1233

A.6.5.3.1.1	Test Purpose and Environment	1233

A.6.5.3.1.2	Test Requirements	1236

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle	1237

A.6.5.3.2.1	Test Purpose and Environment	1237

A.6.5.3.2.2	Test Requirements	1237

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1237

A.6.5.3.3.1	Test Purpose and Environment	1237

A.6.5.3.3.2	Test Requirements	1238

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1238

A.6.5.3.4.1	Test Purpose and Environment	1238

A.6.5.3.4.2	Test Requirements	1242

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1242

A.6.5.3.5.1	Test Purpose and Environment	1242

A.6.5.3.5.2	Test Requirements	1246

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1247

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1247

A.6.5.4.1.1	Test Purpose and Environment	1247

A.6.5.4.1.2	Test Requirements	1257

A.6.5.4.2	Void	1258

A.6.5.5	Beam Failure Detection and Link recovery procedures	1258

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1258

A.6.5.5.1.1	Test Purpose and Environment	1258

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1263

A.6.5.5.2.1	Test Purpose and Environment	1263

A.6.5.5.2.2	Test Requirements	1270

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1270

A.6.5.5.3.1	Test Purpose and Environment	1270

A.6.5.5.3.2	Test Requirements	1276

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1276

A.6.5.5.4.1	Test Purpose and Environment	1276

A.6.5.5.4.2	Test Requirements	1282

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1282

A.6.5.5.5.1	Test Purpose and Environment	1282

A.6.5.5.5.2	Test Requirements	1286

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1287

A.6.5.5.6.1	Test Purpose and Environment	1287

A.6.5.5.6.2	Test Requirements	1291

A.6.5.6	Active BWP switch	1291

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1291

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	1291

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1297

A.6.5.6.2	RRC-based Active BWP Switch	1301

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1301

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1305

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1305

A.6.5.6.3.1.1	Test Purpose and Environment	1305

A.6.5.6.3.1.2	Test Requirements	1310

A.6.5.6.4	SCell dormancy switch	1311

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1311

A.6.5.6.4.1.1	Test Purpose and Environment	1311

A.6.5.6.4.1.2	Test Requirements	1316

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1317

A.6.5.6.4.2.1	 Test Purpose and Environment	1317

A.6.5.6.4.2.2	 Test Requirements	1321

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1321

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1321

A.6.5.7	DL interruptions at switching between two uplink carriers	1325

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1325

A.6.5.7.1.1	Test Purpose and Environment	1325

A.6.5.7.1.2	Test Requirements	1329

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1329

A.6.5.7.2.1	Test Purpose and Environment	1329

A.6.5.7.2.2	Test Requirements	1333

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1333

A.6.5.8.1.1	Test Purpose and Environment	1333

A.6.5.8.1.2	Test Requirements	1337

A.6.6	Measurement procedure	1337

A.6.6.1	Intra-frequency Measurements	1337

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1337

A.6.6.1.1.1	Test purpose and Environment	1337

A.6.6.1.1.2	Test parameters	1337

A.6.6.1.1.3	Test Requirements	1341

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1341

A.6.6.1.2.1	Test purpose and Environment	1341

A.6.6.1.2.2	Test parameters	1341

A.6.6.1.2.3	Test Requirements	1345

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1345

A.6.6.1.3.1	Test purpose and Environment	1345

A.6.6.1.3.2	Test parameters	1345

A.6.6.1.3.3	Test Requirements	1349

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1349

A.6.6.1.4.1	Test purpose and Environment	1349

A.6.6.1.4.2	Test parameters	1349

A.6.6.1.4.3	Test Requirements	1353

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1353

A.6.6.1.5.1	Test purpose and Environment	1353

A.6.6.1.5.2	Test parameters	1353

A.6.6.1.5.3	Test Requirements	1355

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1356

A.6.6.1.6.1	Test purpose and Environment	1356

A.6.6.1.6.2	Test parameters	1356

A.6.6.1.6.3	Test Requirements	1357

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1358

A.6.6.1.7.1	Test purpose and Environment	1358

A.6.6.1.7.2	Test parameters	1358

A.6.6.1.7.3	Test Requirements	1362

A.6.6.2	Inter-frequency Measurements	1362

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1362

A.6.6.2.1.1	Test Purpose and Environment	1362

A.6.6.2.1.2	Test Requirements	1366

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1366

A.6.6.2.2.1	Test Purpose and Environment	1366

A.6.6.2.2.2	Test Requirements	1370

A.6.6.2.3	Void	1371

A.6.6.2.4	Void	1371

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1371

A.6.6.2.5.1	Test Purpose and Environment	1371

A.6.6.2.5.2	Test Requirements	1375

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1375

A.6.6.2.6.1	Test Purpose and Environment	1375

A.6.6.2.6.2	Test Requirements	1379

A.6.6.2.7	Void	1380

A.6.6.2.8	Void	1380

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1380

A.6.6.2.9.1	Test Purpose and Environment	1380

A.6.6.2.9.2	Test Requirements	1383

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1383

A.6.6.2.10.1	Test Purpose and Environment	1383

A.6.6.2.10.2	Test Requirements	1387

A.6.6.3	Inter-RAT Measurements	1391

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1391

A.6.6.3.1.1	Test Purpose and Environment	1391

A.6.6.3.1.2	Test Requirements	1397

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1397

A.6.6.3.2.1	Test Purpose and Environment	1397

A.6.6.3.2.2	Test Requirements	1404

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1404

A.6.6.3.3.1	Test Purpose and Environment	1404

A.6.6.3.3.2	Test Requirements	1408

A.6.6.4	L1-RSRP measurement for beam reporting	1408

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1408

A.6.6.4.1.1	Test Purpose and Environment	1408

A.6.6.4.1.2	Test parameters	1409

A.6.6.4.1.3	Test Requirements	1412

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1412

A.6.6.4.2.1	Test Purpose and Environment	1412

A.6.6.4.2.2	Test parameters	1413

A.6.6.4.2.3	Test Requirements	1416

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1416

A.6.6.4.3.1	Test Purpose and Environment	1416

A.6.6.4.3.2	Test parameters	1417

A.6.6.4.3.3	Test Requirements	1420

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1420

A.6.6.4.4.1	Test Purpose and Environment	1420

A.6.6.4.4.2	Test parameters	1421

A.6.6.4.4.3	Test Requirements	1424

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1424

A.6.6.4.5.1	Test Purpose and Environment	1424

A.6.6.4.5.2	Test parameters	1425

A.6.6.4.5.3	Test Requirements	1427

A.6.6.5	1427

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1427

A.6.6.5.1.1	Test Purpose and Environment	1427

A.6.6.5.1.2	Test Requirements	1431

A.6.6.6	CLI measurements	1431

A.6.6.6.1	SRS-RSRP measurement with DRX	1431

A.6.6.6.1.1	Test Purpose and Environment	1431

A.6.6.6.1.2	Test Parameters	1432

A.6.6.6.1.3	Test Requirements	1435

A.6.6.6.2	CLI-RSSI measurement with DRX	1435

A.6.6.6.2.1	Test Purpose and Environment	1435

A.6.6.6.2.2	Test Parameters	1436

A.6.6.6.2.3	Test Requirements	1438

A.6.6.7	NR measurements with autonomous gaps	1438

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1438

A.6.6.7.1.1	Test Purpose and Environment	1438

A.6.6.7.1.2	Test Parameters	1438

A.6.6.7.1.3	Test Requirements	1441

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1441

A.6.6.7.2.1	Test Purpose and Environment	1441

A.6.6.7.2.2	Test Requirements	1444

A.6.6.8	L1-SINR measurement for beam reporting	1445

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1447

A.6.6.8.2.1	Test Purpose and Environment	1447

A.6.6.8.2.2	Test parameters	1447

A.6.6.8.2.3	Test Requirements	1452

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1452

A.6.6.8.3.1	Test Purpose and Environment	1452

A.6.6.8.3.2	Test parameters	1453

A.6.6.8.3.3	Test Requirements	1455

A.6.6.9	Idle Mode CA/DC Measurements	1455

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1455

A.6.6.9.1.1	Test Purpose and Environment	1455

A.6.6.9.1.2	Test Requirements	1462

A.6.6.10	CSI-RS based intra-frequency Measurements	1462

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1462

A.6.6.10.1.1	Test purpose and Environment	1462

A.6.6.10.1.2	Test Requirements	1465

A.6.6.11	CSI-RS based inter-frequency Measurements	1465

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1465

A.6.6.11.1.1	Test Purpose and Environment	1465

A.6.6.11.1.2	Test Requirements	1469

A.6.6.12	RSTD measurements	1469

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1469

A.6.6.12.1.1	Test Purpose and Environment	1469

A.6.6.12.1.2	Test Requirements	1475

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1476

A.6.6.12.2.1	Test Purpose and Environment	1476

A.6.6.12.2.2	Test Requirements	1481

A.6.6.13 PRS-RSRP measurements	1482

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1482

A.6.6.13.1.1	Test purpose and Environment	1482

A.6.6.13.1.2	Test Requirements	1486

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1486

A.6.6.13.2.1	Test purpose and Environment	1486

A.6.6.13.2.2	Test Requirements	1490

A.6.6.14	UE Rx-Tx time difference measurements	1490

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1490

A.6.6.14.1.1	Test purpose and environment	1490

A.6.6.14.1.2	Test requirements	1494

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1494

A.6.6.14.2.1	Test purpose and environment	1494

A.6.6.14.2.2	Test requirements	1498

A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting	1498

A.6.6.15.1	Test Purpose and Environment	1498

A.6.6.15.2	Test Requirements	1505

A.6.7	Measurement Performance requirements	1506

A.6.7.1	SS-RSRP	1506

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1506

A.6.7.1.1.1	Test Purpose and Environment	1506

A.6.7.1.1.2	Test parameters	1506

A.6.7.1.1.3	Test Requirements	1511

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1511

A.6.7.1.2.1	Test Purpose and Environment	1511

A.6.7.1.2.2	Test parameters	1511

A.6.7.1.2.3	Test Requirements	1515

A.6.7.1.3	Void	1515

A.6.7.2	SS-RSRQ	1515

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1515

A.6.7.2.1.1	Test Purpose and Environment	1515

A.6.7.2.1.2	Test Parameters	1516

A.6.7.2.1.3	Test Requirements	1521

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1521

A.6.7.2.2.1	Test Purpose and Environment	1521

A.6.7.2.2.2	Test Parameters	1521

A.6.7.2.2.3	Test Requirements	1526

A.6.7.3	SS-SINR	1526

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1526

A.6.7.3.1.1	Test Purpose and Environment	1526

A.6.7.3.1.2	Test Parameters	1527

A.6.7.3.1.3	Test Requirements	1531

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1531

A.6.7.3.2.1	Test Purpose and Environment	1531

A.6.7.3.2.2	Test Parameters	1532

A.6.7.3.2.3	Test Requirements	1536

A.6.7.4	L1-RSRP measurement for beam reporting	1537

A.6.7.4.1	SSB based L1-RSRP measurement	1537

A.6.7.4.1.1	Test Purpose and Environment	1537

A.6.7.4.1.2	Test parameters	1537

A.6.7.4.1.3	Test Requirements	1541

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1541

A.6.7.4.2.1	Test Purpose and Environment	1541

A.6.7.4.2.2	Test parameters	1542

A.6.7.4.2.3	Test Requirements	1546

A.6.7.5	E-UTRAN RSRP	1546

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1546

A.6.7.5.1.1	Test Purpose and Environment	1546

A.6.7.5.1.2	Test parameters	1547

A.6.7.5.1.3	Test Requirements	1552

A.6.7.6	E-UTRAN RSRQ	1553

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1553

A.6.7.6.1.1	Test Purpose and Environment	1553

A.6.7.6.1.2	Test parameters	1553

A.6.7.6.1.3	Test Requirements	1558

A.6.7.7	E-UTRAN RS-SINR	1559

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1559

A.6.7.7.1.1	Test Purpose and Environment	1559

A.6.7.7.1.2	Test parameters	1559

A.6.7.7.1.3	Test Requirements	1564

A.6.7.8	CLI measurements	1565

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1565

A.6.7.8.1.1	Test Purpose and Environment	1565

A.6.7.8.1.2	Test parameters	1565

A.6.7.8.1.3	Test Requirements	1571

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1571

A.6.7.8.2.1	Test Purpose and Environment	1571

A.6.7.8.2.2	Test parameters	1572

A.6.7.8.2.3	Test Requirements	1575

A.6.7.9	L1-SINR measurement for beam reporting	1576

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1580

A.6.7.9.2.1	Test Purpose and Environment	1580

A.6.7.9.2.2	Test parameters	1581

A.6.7.9.2.3	Test Requirements	1586

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1586

A.6.7.9.3.1	Test Purpose and Environment	1586

A.6.7.9.3.2	Test parameters	1586

A.6.7.9.3.3	Test Requirements	1591

A.6.7.10	CSI-RSRP	1591

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1591

A.6.7.9.10.1	Test Purpose and Environment	1591

A.6.7.9.10.2	Test parameters	1592

A.6.7.10.1.3	Test Requirements	1597

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1597

A.6.7.9.10.1	Test Purpose and Environment	1597

A.6.7.10.2.2	Test parameters	1597

A.6.7.10.2.3	Test Requirements	1602

A.6.7.11	CSI-RSRQ	1602

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1602

A.6.7.11.1.1	Test Purpose and Environment	1602

A.6.7.11.1.2	Test Parameters	1602

A.6.7.11.1.3	Test Requirements	1607

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1607

A.6.7.11.2.1	Test Purpose and Environment	1607

A.6.7.11.2.2	Test Parameters	1607

A.6.7.11.2.3	Test Requirements	1613

A.6.7.12	CSI-SINR	1614

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1614

A.6.7.12.1.1	Test Purpose and Environment	1614

A.6.7.12.1.2	Test Parameters	1614

A.6.7.12.1.3	Test Requirements	1618

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1619

A.6.7.12.2.1	Test Purpose and Environment	1619

A.6.7.12.2.2	Test Parameters	1619

A.6.7.12.2.3	Test Requirements	1625

A.6.7.13	RSTD measurements	1625

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	1625

A.6.7.13.1.1	Test purpose and Environment	1625

A.6.7.13.1.2	Test Requirements	1628

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	1628

A.6.7.13.2.1	Test purpose and Environment	1628

A.6.7.13.2.2	Test Requirements	1632

A.6.7.14	PRS-RSRP measurements	1632

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	1632

A.6.7.14.1.1	Test Purpose and Environment	1632

A.6.7.14.1.2	Test parameters	1632

A.6.7.14.1.3	Test Requirements	1637

A.6.7.15	UE Rx-Tx time difference measurements	1637

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1637

A.6.7.15.1.1	Test purpose and environment	1637

A.6.7.15.1.2	Test parameters	1638

A.6.7.15.1.2	Test parameters	1638

A.6.7.15.1.3	Test requirements	1641

A.7	NR standalone tests with one or more NR cells in FR2	1641

A.7.1	SA: RRC_IDLE state mobility	1641

A.7.1.1	Cell re-selection to NR	1641

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1641

A.7.1.1.1.1	Test Purpose and Environment	1641

A.7.1.1.1.2	Test Parameters	1641

A.7.1.1.1.3	Test Requirements	1645

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1645

A.7.1.1.2.1	Test Purpose and Environment	1645

A.7.1.1.2.2	Test Parameters	1645

A.7.1.1.2.3	Test Requirements	1649

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1649

A.7.1.1.3.1	Test Purpose and Environment	1649

A.7.1.1.3.2	Test Parameters	1649

A.7.1.1.3.3	Test Requirements	1652

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1652

A.7.1.1.4.1	Test Purpose and Environment	1652

A.7.1.1.4.2	Test Parameters	1652

A.7.1.1.4.3	Test Requirements	1655

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1655

A.7.1.1.5.1	Test Purpose and Environment	1655

A.7.1.1.5.2	Test Parameters	1655

A.7.1.1.5.3	Test Requirements	1658

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1658

A.7.1.1.6.1	Test Purpose and Environment	1658

A.7.1.1.6.2	Test Parameters	1658

A.7.1.1.6.3	Test Requirements	1660

A.7.2	SA: RRC_INACTIVE state mobility	1661

A.7.3	RRC_CONNECTED state mobility	1661

A.7.3.1	Handover	1661

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1661

A.7.3.1.1.1	Test Purpose and Environment	1661

A.7.3.1.1.2	Test Parameters	1661

A.7.3.1.1.3	Test Requirements	1665

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1665

A.7.3.1.2.1	Test Purpose and Environment	1665

A.7.3.1.2.2	Test Parameters	1665

A.7.3.1.2.3	Test Requirements	1668

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1668

A.7.3.1.3.1	Test Purpose and Environment	1668

A.7.3.1.3.2	Test Parameters	1668

A.7.3.1.3.3 Test Requirements	1670

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1671

A.7.3.1.4.1	Test Purpose and Environment	1671

A.7.3.1.4.2	Test Parameters	1671

A.7.3.1.4.3 Test Requirements	1678

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1678

A.7.3.1.5.1	Test Purpose and Environment	1678

A.7.3.1.5.2	Test Parameters	1678

A.7.3.1.5.3 Test Requirements	1685

A.7.3.2	RRC Connection Mobility Control	1685

A.7.3.2.1	SA: RRC Re-establishment	1685

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1685

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1688

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1691

A.7.3.2.1.3.1	Test Purpose and Environment	1691

A.7.3.2.1.3.2	Test Requirements	1693

A.7.3.2.2	Random Access	1694

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	1694

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	1698

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1702

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	1705

A.7.3.2.3	SA: RRC Connection Release with Redirection	1708

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1708

A.7.3.3 Conditional Handover	1712

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1712

A.7.3.3.1.1	Test Purpose and Environment	1712

A.7.3.3.1.2	Test Parameters	1712

A.7.3.3.1.2.3	Test Requirements	1715

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	1715

A.7.3.3.2.1	Test Purpose and Environment	1715

A.7.3.3.2.2	Test Parameters	1715

A.7.3.3.2.3 Test Requirements	1717

A.7.4	Timing	1717

A.7.4.1	UE transmit timing	1717

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1717

A.7.4.1.1.1	Test Purpose and environment	1717

A.7.4.1.1.2	Test requirements	1720

A.7.4.2	UE timer accuracy	1721

A.7.4.3	Timing advance	1721

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1721

A.7.4.3.1.1	Test Purpose and Environment	1721

A.7.4.3.1.2	Test Parameters	1721

A.7.4.3.1.3 Test Requirements	1724

A.7.5	Signaling characteristics	1725

A.7.5.1	Radio link Monitoring	1725

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1725

A.7.5.1.1.1	Test Purpose and Environment	1725

A.7.5.1.1.2	Test Requirements	1728

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1729

A.7.5.1.2.1	Test Purpose and Environment	1729

A.7.5.1.2.2	Test Requirements	1733

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1734

A.7.5.1.3.1	Test Purpose and Environment	1734

A.7.5.1.3.2	Test Requirements	1737

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1737

A.7.5.1.4.1	Test Purpose and Environment	1737

A.7.5.1.4.2	Test Requirements	1741

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1741

A.7.5.1.5.1	Test Purpose and Environment	1741

A.7.5.1.5.2	Test Requirements	1746

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1746

A.7.5.1.6.1	Test Purpose and Environment	1746

A.7.5.1.6.2	Test Requirements	1750

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1750

A.7.5.1.7.1	Test Purpose and Environment	1750

A.7.5.1.7.2	Test Requirements	1754

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1754

A.7.5.1.8.1	Test Purpose and Environment	1754

A.7.5.1.8.2	Test Requirements	1759

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1759

A.7.5.1.9.1	Test Purpose and Environment	1759

A.7.5.1.9.2	Test Requirements	1762

A.7.5.2	Interruption	1762

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1762

A.7.5.2.1.1	Test Purpose and Environment	1762

A.7.5.2.1.2	Test Requirements	1765

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1766

A.7.5.2.2.1	Test Purpose and Environment	1766

A.7.5.2.2.2	Test Parameters	1766

A.7.5.2.2.3	Test Requirements	1768

A.7.5.3	SCell Activation and Deactivation Delay	1768

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1768

A.7.5.3.1.1	Test Purpose and Environment	1768

A.7.5.3.1.2	Test Requirements	1770

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1770

A.7.5.3.2.1	Test Purpose and Environment	1770

A.7.5.3.2.2	Test Requirements	1774

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1775

A.7.5.3.3.1	Test Purpose and Environment	1775

A.7.5.3.3.2	Test Requirements	1778

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	1779

A.7.5.3.4.1	Test Purpose and Environment	1779

A.7.5.3.4.2	Test Requirements	1782

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	1783

A.7.5.3.5.1	Test Purpose and Environment	1783

A.7.5.3.5.2	Test Requirements	1785

A.7.5.4	Void	1786

A.7.5.5	Beam Failure Detection and Link recovery procedures	1786

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1786

A.7.5.5.1.1	Test Purpose and Environment	1786

A.7.5.5.1.2	Test Requirements	1791

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1791

A.7.5.5.2.1	Test Purpose and Environment	1791

A.7.5.5.2.2	Test Requirements	1796

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1796

A.7.5.5.3.1	Test Purpose and Environment	1796

A.7.5.5.3.2	Test Requirements	1802

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1802

A.7.5.5.4.1	Test Purpose and Environment	1802

A.7.5.5.4.2	Test Requirements	1808

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1808

A.7.5.5.5.1	Test Purpose and Environment	1808

A.7.5.5.5.2	Test Requirements	1813

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1814

A.7.5.5.6.1	Test Purpose and Environment	1814

A.7.5.5.6.2	Test Requirements	1818

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1818

A.7.5.5.7.1	Test Purpose and Environment	1818

A.7.5.5.7.2	Test Requirements	1822

A.7.5.6	Active BWP switch	1822

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1822

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1822

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1827

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1832

A.7.5.6.1.3.1	Test Purpose and Environment	1832

A.7.5.6.1.3.2	Test Requirements	1835

A.7.5.6.2	RRC-based Active BWP Switch	1835

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1839

A.7.5.6.3.1.1	Test Purpose and Environment	1839

A.7.5.6.3.1.2	Test Requirements	1842

A.7.5.6.4	SCell dormancy switch	1842

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	1842

A.7.5.6.4.1.1	Test Purpose and Environment	1842

A.7.5.6.4.1.2	Test Requirements	1847

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	1847

A.7.5.6.4.2.1	 Test Purpose and Environment	1847

A.7.5.6.4.2.2	 Test Requirements	1852

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1852

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	1852

A.7.5.7	PSCell addition and release delay	1855

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1855

A.7.5.7.1.1	Test Purpose and Environment	1855

A.7.5.7.1.2	Test Requirements	1858

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1858

A.7.5.7.2.1	Test Purpose and Environment	1858

A.7.5.7.2.2	Test Requirements	1861

A.7.5.8	Active TCI state switch delay	1861

A.7.5.8.1	MAC-CE based active TCI state switch	1861

A.7.5.8.2	RRC based active TCI state switch	1864

A.7.5.9	Uplink spatial relation switch delay	1867

A.7.5.9.1.1.1	Test Purpose and Environment	1867

A.7.5.9.1.1.2	Test Requirements	1870

A.7.5.9.2	RRC based spatial relation switch	1870

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1870

A.7.5.9.2.1.1	Test Purpose and Environment	1870

A.7.5.9.2.1.2	Test Requirements	1873

A.7.5.10	UE specific CBW change	1873

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1873

A.7.5.10.1.1	Test Purpose and Environment	1873

A.7.5.10.1.2	Test Requirements	1876

A.7.6	Measurement procedure	1877

A.7.6.1	Intra-frequency Measurements	1877

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1877

A.7.6.1.1.1	Test purpose and Environment	1877

A.7.6.1.1.2	Test Requirements	1880

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1880

A.7.6.1.2.1	Test purpose and Environment	1880

A.7.6.1.2.2	Test Requirements	1883

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1884

A.7.6.1.3.1	Test purpose and Environment	1884

A.7.6.1.3.2	Test Requirements	1887

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1887

A.7.6.1.4.1	Test purpose and Environment	1887

A.7.6.1.4.2	Test Requirements	1890

A.7.6.2	Inter-frequency Measurements	1891

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1891

A.7.6.2.1.1	Test Purpose and Environment	1891

A.7.6.2.1.2	Test Requirements	1894

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1894

A.7.6.2.2.1	Test Purpose and Environment	1894

A.7.6.2.2.2	Test Requirements	1898

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1898

A.7.6.2.3.1	Test Purpose and Environment	1898

A.7.6.2.3.2	Test Requirements	1902

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1902

A.7.6.2.4.1	Test Purpose and Environment	1902

A.7.6.2.4.2	Test Requirements	1906

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1906

A.7.6.2.5.1	Test Purpose and Environment	1906

A.7.6.2.5.2	Test Requirements	1910

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1911

A.7.6.2.6.1	Test Purpose and Environment	1911

A.7.6.2.6.2	Test Requirements	1915

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1916

A.7.6.2.7.1	Test Purpose and Environment	1916

A.7.6.2.7.2	Test Requirements	1920

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1921

A.7.6.2.8.1	Test Purpose and Environment	1921

A.7.6.2.8.2	Test Requirements	1925

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1926

A.7.6.2.9.1	Test Purpose and Environment	1926

A.7.6.2.9.2	Test Requirements	1930

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1930

A.7.6.2.10.1	Test Purpose and Environment	1930

A.7.6.2.10.2	Test Requirements	1932

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1932

A.7.6.2.11.1	Test Purpose and Environment	1932

A.7.6.2.11.2	Test Requirements	1935

A.7.6.3	L1-RSRP measurement for beam reporting	1936

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1936

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1936

A.7.6.3.1.1	Test Purpose and Environment	1936

A.7.6.3.1.2	Test parameters	1936

A.7.6.3.1.3	Test Requirements	1938

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1938

A.7.6.3.2.1	Test Purpose and Environment	1938

A.7.6.3.2.2	Test parameters	1939

A.7.6.3.2.3	Test Requirements	1941

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1941

A.7.6.3.3.1	Test Purpose and Environment	1941

A.7.6.3.3.2	Test parameters	1942

A.7.6.3.3.3	Test Requirements	1944

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1945

A.7.6.3.4.1	Test Purpose and Environment	1945

A.7.6.3.4.2	Test parameters	1945

A.7.6.3.3.3	Test Requirements	1947

A.7.6.4	CLI measurements	1948

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1948

A.7.6.4.1.1	Test Purpose and Environment	1948

A.7.6.4.1.2	Test Parameters	1948

A.7.6.4.1.3	Test Requirements	1950

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1950

A.7.6.4.2.1	Test Purpose and Environment	1950

A.7.6.4.2.2	Test Parameters	1951

A.7.6.4.2.3	Test Requirements	1952

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1953

A.7.6.5.1.1	Test Purpose and Environment	1953

A.7.6.5.1.2	Test Requirements	1956

A.7.6.6	L1-SINR measurement for beam reporting	1956

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1958

A.7.6.6.2.1	Test Purpose and Environment	1958

A.7.6.6.2.2	Test parameters	1959

A.7.6.6.2.3	Test Requirements	1961

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1961

A.7.6.6.3.1	Test Purpose and Environment	1961

A.7.6.6.3.2	Test parameters	1962

A.7.6.6.3.3	Test Requirements	1964

A.7.6.7	CSI-RS based intra-frequency Measurements	1964

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	1964

A.7.6.7.1.1	Test purpose and Environment	1964

A.7.6.7.1.2	Test Requirements	1967

A.7.6.8	CSI-RS based inter-frequency Measurements	1967

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	1967

A.7.6.8.1.1	Test Purpose and Environment	1967

A.7.6.8.1.2	Test Requirements	1970

A.7.6.9	RSTD measurements	1970

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1970

A.7.6.9.1.1	Test Purpose and Environment	1970

A.7.6.9.1.2	Test Requirements	1975

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	1975

A.7.6.9.2.1	Test Purpose and Environment	1975

A.7.6.9.2.2	Test Requirements	1981

A.7.6.10 PRS-RSRP measurements	1981

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	1981

A.7.6.10.1.1	Test Purpose and Environment	1981

A.7.6.10.1.2	Test Requirements	1985

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1985

A.7.6.10.2.1	Test Purpose and Environment	1985

A.7.6.10.2.2	Test Requirements	1989

A.7.6.11	UE Rx-Tx time difference measurements	1989

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	1989

A.7.6.11.1.1	Test purpose and environment	1989

A.7.6.11.1.2	Test requirements	1991

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	1992

A.7.6.11.2.1	Test purpose and environment	1992

A.7.6.11.2.2	Test requirements	1994

A.7.7	Measurement Performance requirements	1995

A.7.7.1	SS-RSRP	1995

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1995

A.7.7.1.1.1	Test Purpose and Environment	1995

A.7.7.1.1.2	Test parameters	1995

A.7.7.1.1.3	Test Requirements	1999

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1999

A.7.7.1.2.1	Test Purpose and Environment	1999

A.7.7.1.2.2	Test parameters	2000

A.7.7.1.2.3	Test Requirements	2004

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2005

A.7.7.1.3.1	Test Purpose and Environment	2005

A.7.7.1.3.2	Test parameters	2005

A.7.7.1.3.3	Test Requirements	2009

A.7.7.2	SS-RSRQ	2009

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2009

A.7.7.2.1.1	Test Purpose and Environment	2009

A.7.7.2.1.2	Test Parameters	2009

A.7.7.2.1.3	Test Requirements	2011

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2011

A.7.7.2.2.1	Test Purpose and Environment	2011

A.7.7.2.2.2	Test Parameters	2011

A.7.7.2.2.3	Test Requirements	2013

A.7.7.3	SS-SINR	2013

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2013

A.7.7.3.1.1	Test Purpose and Environment	2013

A.7.7.3.1.2	Test Parameters	2013

A.7.7.3.1.3	Test Requirements	2016

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2016

A.7.7.3.2.1	Test Purpose and Environment	2016

A.7.7.3.2.2	Test Parameters	2016

A.7.7.3.2.3	Test Requirements	2018

A.7.7.4	L1-RSRP measurement for beam reporting	2018

A.7.7.4.1	SSB based L1-RSRP measurement	2018

A.7.7.4.1.1	Test Purpose and Environment	2018

A.7.7.4.1.2	Test parameters	2019

A.7.7.4.1.3	Test Requirements	2021

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2022

A.7.7.4.2.1	Test Purpose and Environment	2022

A.7.7.4.2.2	Test parameters	2022

A.7.7.4.2.3	Test Requirements	2024

A.7.7.5	CLI measurements	2025

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2025

A.7.7.5.1.1	Test Purpose and Environment	2025

A.7.7.5.1.2	Test parameters	2025

A.7.7.5.1.3	Test Requirements	2028

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2029

A.7.7.5.2.1	Test Purpose and Environment	2029

A.7.7.5.2.2	Test parameters	2029

A.7.7.5.2.3	Test Requirements	2031

A.7.7.6	L1-SINR measurement for beam reporting	2032

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2034

A.7.7.6.2.1	Test Purpose and Environment	2034

A.7.7.6.2.2	Test parameters	2035

A.7.7.6.2.3	Test Requirements	2037

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2037

A.7.7.6.3.1	Test Purpose and Environment	2037

A.7.7.6.3.2	Test parameters	2038

A.7.7.6.3.3	Test Requirements	2040

A.7.7.7	CSI-RSRP	2040

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2040

A.7.7.7.1.1	Test Purpose and Environment	2040

A.7.7.7.1.2	Test parameters	2041

A.7.7.7.1.3	Test Requirements	2044

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2045

A.7.7.7.2.1	Test Purpose and Environment	2045

A.7.7.7.2.2	Test parameters	2045

A.7.7.7.2.3	Test Requirements	2048

A.7.7.8	CSI-RSRQ	2048

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2048

A.7.7.8.1.1	Test Purpose and Environment	2048

A.7.7.8.1.2	Test Parameters	2049

A.7.7.8.1.3	Test Requirements	2051

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2051

A.7.7.8.2.1	Test Purpose and Environment	2051

A.7.7.8.2.2	Test Parameters	2051

A.7.7.8.2.3	Test Requirements	2053

A.7.7.9	CSI-SINR	2053

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2053

A.7.7.9.1.1	Test Purpose and Environment	2053

A.7.7.9.1.2	Test Parameters	2053

A.7.7.9.1.3	Test Requirements	2055

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2055

A.7.7.9.2.1	Test Purpose and Environment	2055

A.7.7.9.2.2	Test Parameters	2056

A.7.7.9.2.3	Test Requirements	2058

A.7.7.10	RSTD measurements	2058

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2058

A.7.7.10.1.1	Test purpose and Environment	2058

A.7.7.10.1.2	Test Requirements	2061

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2061

A.7.7.10.2.1	Test purpose and Environment	2061

A.7.7.10.2.2	Test Requirements	2063

A.7.7.11	PRS-RSRP measurements	2063

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2063

A.7.7.11.1.1	Test Purpose and Environment	2063

A.7.7.11.1.2	Test parameters	2063

A.7.7.11.1.3	Test Requirements	2066

A.7.7.12	UE Rx-Tx time difference measurements	2067

A.7.7.12.1	UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA	2067

A.7.7.12.1.1	Test purpose and environment	2067

A.7.7.12.1.2	Test parameters	2067

A.7.7.12.1.2	Test parameters	2068

A.7.7.12.1.3	Test requirements	2069

A.8	E-UTRA standalone tests for NR RRM	2069

A.8.1	Void	2069

A.8.2	RRC_IDLE state mobility	2069

A.8.2.1	Inter-RAT NR Cell re-selection	2069

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2069

A.8.2.1.1.1	Test Purpose and Environment	2069

A.8.2.1.1.2	Test Requirements	2074

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2075

A.8.2.1.2.1	Test Purpose and Environment	2075

A.8.2.1.2.2	Test Requirements	2079

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2080

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	2080

A.8.2.2.1.1	Test Purpose and Environment	2080

A.8.2.2.1.2	Test Requirements	2083

A.8.3	RRC_CONNECTED state mobility	2084

A.8.3.1	Handover	2084

A.8.3.1.1	E-UTRAN - NR handover in FR1	2084

A.8.3.1.1.1	Test Purpose and Environment	2084

A.8.3.1.1.2	Test Requirements	2089

A.8.4	Measurement procedure	2089

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2089

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2089

A.8.4.1.1.1	Test Purpose and Environment	2089

A.8.4.1.1.2	Test Requirements	2091

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2092

A.8.4.1.2.1	Test Purpose and Environment	2092

A.8.4.1.2.2	Test Requirements	2093

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2093

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2093

A.8.4.2.1.1	Test Purpose and Environment	2093

A.8.4.2.1.2	Test Requirements	2098

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2098

A.8.4.2.2.1	Test Purpose and Environment	2098

A.8.4.2.2.2	Test Requirements	2102

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2102

A.8.4.2.3.1	Test Purpose and Environment	2102

A.8.4.2.3.2	Test Requirements	2107

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2107

A.8.4.2.4.1	Test Purpose and Environment	2107

A.8.4.2.4.2	Test Requirements	2111

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2111

A.8.4.2.5.1	Test Purpose and Environment	2111

A.8.4.2.5.2	Test Requirements	2113

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2114

A.8.4.2.6.1	Test Purpose and Environment	2114

A.8.4.2.6.2	Test Requirements	2116

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2117

A.8.4.2.7.1	Test Purpose and Environment	2117

A.8.4.2.7.2	Test Requirements	2119

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2120

A.8.4.2.8.1	Test Purpose and Environment	2120

A.8.4.2.8.2	Test Requirements	2122

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2123

A.8.4.2.9.1	Test Purpose and Environment	2123

A.8.4.2.9.2	Test Requirements	2127

A.8.5	Measurement performance	2127

A.8.5.1	SFTD accuracy	2127

A.8.5.1.1	SFTD accuracy	2127

A.8.5.1.1.1	Test Purpose	2127

A.8.5.1.1.2	Test Environment	2128

A.8.5.1.1.3	Test Requirements	2134

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2134

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2134

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2139

A.8.5.2.1.2.1	Test Purpose and Environment	2139

A.8.5.2.1.2.2	Test Parameters	2139

A.8.5.2.1.2.3	Test Requirements	2141

A.8.5.2.2	SS-RSRQ	2141

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2141

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2146

A.8.5.2.2.2.1	Test Purpose and Environment	2146

A.8.5.2.2.2.2	Test Parameters	2146

A.8.5.2.2.2.3	Test Requirements	2148

A.8.5.2.3	SS-SINR	2148

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2148

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2152

A.8.5.2.3.2.1	Test Purpose and Environment	2152

A.8.5.2.3.2.2	Test Parameters	2152

A.8.5.2.3.2.3	Test Requirements	2154

A.9	V2X Tests	2155

A.9.1	V2X Tests in FR1	2155

A.9.1.1	Test for V2X UE Transmit Timing	2155

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	2155

A.9.1.1.1.1	Test Purpose and Environment	2155

A.9.1.1.1.2	Test requirements	2155

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2155

A.9.1.1.2.1	Test Purpose and Environment	2155

A.9.1.1.2.2	Test requirements	2156

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2156

A.9.1.1.3.1	Test Purpose and Environment	2156

A.9.1.1.3.2	Test requirements	2159

A.9.1.2	Test for Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2159

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2159

A.9.1.2.1.1	Test Purpose and Environment	2159

A.9.1.2.1.2	Test Requirements	2162

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2162

A.9.1.2.2.1	Test Purpose and Environment	2162

A.9.1.2.2.2	Test Requirements	2163

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	2164

A.9.1.3.1	 Test for GNSS configured as the highest priority	2164

A.9.1.3.1.1	Test Purpose and Environment	2164

A.9.1.3.1.2	Test Requirements	2166

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2167

A.9.1.3.2.1	Test Purpose and Environment	2167

A.9.1.3.2.2	Test Requirements	2169

A.9.1.4	Test for L1 SL-RSRP Measurement	2170

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2170

A.9.1.4.1.1	Test Purpose and Environment	2170

A.9.1.4.1.2	Test Requirements	2172

A. 9.1.4.2 Test for V2X UE Resource Pre-emption	2172

A. 9.1.4.2.1	Test Purpose and Environment	2172

A.9.1.4.2.2	Test Requirements	2176

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	2176

A.9.1.4.3.1	Test Purpose and Environment	2176

A.9.1.4.3.2	Test Requirements	2180

A.9.1.5	Test for Congestion Control Measurement	2180

A.9.1.5.1	Test Purpose and Environment	2180

A.9.1.5.2	Test Requirements	2183

A.9.1.6	Test for Interruption	2183

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2183

A.9.1.6.1.1	Test Purpose and Environment	2183

A.9.1.6.1.2	Test Requirements	2187

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2187

A.10.1	RRC_CONNECTED state mobility	2187

A.10.1.1	RRC connection mobility control	2187

A.10.1.1.1	Random Access	2187

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2187

A.10.1.1.1.1.1	Test Purpose and Environment	2187

A.10.1.1.1.1.2	Test Requirements	2190

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2190

A.10.1.1.1.1.2.2	Random Access Response Reception	2190

A.10.1.1.1.1.2.3	No Random Access Response Reception	2191

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2191

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2191

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2192

A.10.1.1.1.2.1	Test Purpose and Environment	2192

A.10.1.1.1.2.2	Test Requirements	2195

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2195

A.10.1.1.1.2.2.2	Random Access Response Reception	2196

A.10.1.1.1.2.2.3	No Random Access Response Reception	2196

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2196

A.10.1.1.1.3.1	Test Purpose and Environment	2196

A.10.1.1.1.3.2	Test Requirements	2200

A.10.1.1.1.3.2.1	MsgA Transmission	2200

A.10.1.1.1.3.2.2	MsgB Reception	2200

A.10.1.1.1.3.2.3	No MsgB Reception	2201

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2201

A.10.1.1.1.4.1	Test Purpose and Environment	2201

A.10.1.1.1.4.2	Test Requirements	2205

A.10.1.1.1.4.2.1	MsgA Transmission	2205

A.10.1.1.1.4.2.2	MsgB Reception	2206

A.10.1.1.1.4.2.3	No MsgB Reception	2206

A.10.2	Timing	2207

A.10.2.1	UE transmit timing	2207

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	2207

A.10.2.1.1.1	Test Purpose and environment	2207

A.10.2.1.1.2	Test requirements	2210

A.10.2.2	UE timing advance	2211

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	2211

A.10.2.2.1.1	Test Purpose and Environment	2211

A.10.2.2.1.2	Test Parameters	2211

A.10.2.2.1.3	Test Requirements	2215

A.10.3	Signalling characteristics	2215

A.10.3.1	Radio link monitoring	2215

A.10.3.1.1	Introduction	2215

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2216

A.10.3.1.2.1	Test purpose and environment	2216

A.10.3.1.2.2	Test requirements	2220

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2220

A.10.3.1.3.1	Test purpose and environment	2220

A.10.3.1.3.2	Test requirements	2226

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2226

A.10.3.1.4.1	Test purpose and environment	2226

A.10.3.1.4.2	Test requirements	2226

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2226

A.10.3.1.5.1	Test purpose and environment	2226

A.10.3.1.5.2	Test requirements	2226

A.10.3.2	Interruption	2226

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2226

A.10.3.2.1.1	Test Purpose and Environment	2226

A.10.3.2.1.2	Test Requirements	2231

A.10.3.3	SCell activation and deactivation delay	2231

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	2231

A.10.3.3.1.1	Test Purpose and Environment	2231

A.10.3.3.1.2	Test Requirements	2236

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 320 ms SCell measurement cycle	2236

A.10.3.3.2.1	Test Purpose and Environment	2236

A.10.3.3.2.2	Test Requirements	2237

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	2237

A.10.3.3.3.1	Test Purpose and Environment	2237

A.10.3.3.3.2	Test Requirements	2238

A.10.3.4	Beam failure detection and link recovery procedures	2238

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2238

A.10.3.4.1.1	Test Purpose and Environment	2238

A.10.3.4.1.2	Test Requirements	2244

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2244

A.10.3.4.2.1	Test Purpose and Environment	2244

A.10.3.4.2.2	Test Requirements	2251

A.10.3.5	Active BWP switching	2251

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2251

A.10.3.5.1.2	Test Requirements	2256

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2257

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2257

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2260

A.10.3.5.3	RRC-based Active BWP Switch	2264

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2264

A.10.3.6	PSCell addition and release delay	2268

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2268

A.10.3.6.1.1	Test purpose and environment	2268

A.10.3.6.1.2	Test Requirements	2272

A.10.3.7	Active TCI state switching delay	2273

A.10.4	Measurement procedure	2273

A.10.4.1	Intra-frequency measurements	2273

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2273

A.10.4.1.1.1	Test purpose and environment	2273

A.10.4.1.1.2	Test parameters	2273

A.10.4.1.1.3	Test Requirements	2273

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	2274

A.10.4.1.2.1	Test purpose and environment	2274

A.10.4.1.2.2	Test parameters	2274

A.10.4.1.2.3	Test Requirements	2274

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	2274

A.10.4.1.3.1	Test purpose and environment	2274

A.10.4.1.3.2	Test parameters	2274

A.10.4.1.3.3	Test Requirements	2274

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2275

A.10.4.1.4.1	Test purpose and environment	2275

A.10.4.1.4.2	Test parameters	2275

A.10.4.1.4.3	Test Requirements	2275

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2275

A.10.4.1.5.1	Test purpose and environment	2275

A.10.4.1.5.2	Test parameters	2275

A.10.4.1.5.3	Test Requirements	2275

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2276

A.10.4.1.6.1	Test purpose and environment	2276

A.10.4.1.6.2	Test parameters	2276

A.10.4.1.6.3	Test Requirements	2276

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2276

A.10.4.1.7.1	Test purpose and environment	2276

A.10.4.1.7.2	Test parameters	2276

A.10.4.1.7.3	Test Requirements	2276

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2277

A.10.4.1.8.1	Test purpose and environment	2277

A.10.4.1.8.2	Test parameters	2277

A.10.4.1.8.3	Test Requirements	2277

A.10.4.1.9	RSSI measurement reporting on PSCC	2277

A.10.4.1.9.1	Test purpose and environment	2277

A.10.4.1.9.2	Test parameters	2277

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2278

A.10.4.1.10.1	Test purpose and environment	2278

A.10.4.1.10.2	Test parameters	2278

A.10.4.1.11	RSSI measurement reporting on SCC	2278

A.10.4.1.11.1	Test purpose and environment	2278

A.10.4.1.11.2	Test parameters	2278

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2279

A.10.4.1.12.1	Test purpose and environment	2279

A.10.4.1.12.2	Test parameters	2279

A.10.4.2	Inter-frequency measurements	2279

A.10.4.2.1	RSSI measurement reporting	2279

A.10.4.2.1.1	Test purpose and environment	2279

A.10.4.2.1.2	Test parameters	2279

A.10.4.2.2	Channel occupancy measurement reporting	2280

A.10.4.2.2.1	Test purpose and environment	2280

A.10.4.2.2.2	Test parameters	2280

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2280

A.10.4.2.3.1	Test Purpose and Environment	2280

A.10.4.2.3.2	Test Requirements	2285

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2286

A.10.4.2.4.1	Test Purpose and Environment	2286

A.10.4.2.4.2	Test Requirements	2290

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2291

A.10.4.2.5.1	Test Purpose and Environment	2291

A.10.4.2.5.2	Test Requirements	2295

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2296

A.10.4.2.6.1	Test Purpose and Environment	2296

A.10.4.2.6.2	Test Requirements	2300

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2301

A.10.4.2.7.1	Test Purpose and Environment	2301

A.10.4.2.7.2	Test Requirements	2307

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2307

A.10.4.2.8.1	Test Purpose and Environment	2307

A.10.4.2.8.2	Test Requirements	2313

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2314

A.10.4.2.9.1	Test Purpose and Environment	2314

A.10.4.2.9.2	Test Requirements	2319

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2319

A.10.4.2.10.1	Test Purpose and Environment	2319

A.10.4.2.10.2	Test Requirements	2325

A.10.4.3	L1-RSRP measurements for beam reporting	2326

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2326

A.10.4.3.1.1	Test Purpose and Environment	2326

A.10.4.3.1.2	Test parameters	2326

A.10.4.3.1.3	Test Requirements	2328

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2329

A.10.4.3.2.1	Test Purpose and Environment	2329

A.10.4.3.2.2	Test parameters	2329

A.10.4.3.2.3	Test Requirements	2331

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2332

A.10.4.3.3.1	Test Purpose and Environment	2332

A.10.4.3.3.2	Test parameters	2332

A.10.4.3.3.3	Test Requirements	2335

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2336

A.10.4.3.4.1	Test Purpose and Environment	2336

A.10.4.3.4.2	Test parameters	2336

A.10.4.3.4.3	Test Requirements	2339

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2340

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2340

A.10.4.4.1.1	Test Purpose and Environment	2340

A.10.4.4.1.2	Test Requirements	2344

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2345

A.10.4.4.2.1	Test Purpose and Environment	2345

A.10.4.4.2.2	Test Requirements	2350

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2351

A.10.4.4.3.1	Test Purpose and Environment	2351

A.10.4.4.3.2	Test Requirements	2355

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2356

A.10.4.4.4.1	Test Purpose and Environment	2356

A.10.4.4.4.2	Test Requirements	2361

A.10.5	Measurement performance	2362

A.10.5.1	SS-RSRP	2362

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2362

A.10.5.1.1.1	Test Purpose and Environment	2362

A.10.5.1.1.2	Test parameters	2362

A.10.5.1.1.3	Test Requirements	2365

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2365

A.10.5.1.2.1	Test Purpose and Environment	2365

A.10.5.1.2.2	Test parameters	2365

A.10.5.1.2.3	Test Requirements	2368

A.10.5.2	SS-RSRQ	2368

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2368

A.10.5.2.1.1	Test Purpose and Environment	2368

A.10.5.2.1.2	Test Parameters	2368

A.10.5.2.1.3	Test Requirements	2371

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2371

A.10.5.2.2.1	Test Purpose and Environment	2371

A.10.5.2.2.2	Test Parameters	2371

A.10.5.2.2.3	Test Requirements	2374

A.10.5.3	SS-SINR	2374

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2374

A.10.5.4.1	SSB based L1-RSRP measurement	2374

A.10.5.4.1.1	Test Purpose and Environment	2374

A.10.5.4.1.2	Test parameters	2374

A.10.5.4.1.3	Test Requirements	2377

A.10.5.5	RSSI	2377

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2377

A.10.5.5.1.1	Test Purpose and Environment	2377

A.10.5.5.1.2	Test parameters	2377

A.10.5.5.1.3	Test Requirements	2380

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2380

A.10.5.5.2.1	Test Purpose and Environment	2380

A.10.5.5.2.2	Test parameters	2380

A.10.5.5.2.3	Test Requirements	2383

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2383

A.10.5.5.3.1	Test Purpose and Environment	2383

A.10.5.5.3.2	Test parameters	2383

A.10.5.5.3.3	Test Requirements	2386

A.10.5.6	Channel occupancy	2386

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2386

A.10.5.6.1.1	Test Purpose and Environment	2386

A.10.5.6.1.2	Test parameters	2386

A.10.5.6.1.3	Test Requirements	2390

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2390

A.10.5.6.2.1	Test Purpose and Environment	2390

A.10.5.6.2.2	Test parameters	2390

A.10.5.6.2.3	Test Requirements	2393

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2393

A.10.5.6.3.1	Test Purpose and Environment	2393

A.10.5.6.3.2	Test parameters	2393

A.10.5.6.3.3	Test Requirements	2397

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2397

A.11.1	RRC_IDLE state mobility	2397

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2397

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2397

A.11.1.1.1.1	Test Purpose and Environment	2397

A.11.1.1.1.2	Test Parameters	2397

A.11.1.1.1.3	Test Requirements	2400

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2400

A.11.1.1.2.1	Test Purpose and Environment	2400

A.11.1.1.2.2	Test Parameters	2400

A.11.1.1.2.3	Test Requirements	2404

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2404

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2404

A.11.1.2.1.1	Test Purpose and Environment	2404

A.11.1.2.1.2	Test Parameters	2405

A.11.1.2.1.3	Test Requirements	2411

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2412

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2412

A.11.1.3.1.1	Test Purpose and Environment	2412

A.11.1.3.1.2	Test Parameters	2412

A.11.1.3.1.3	Test Requirements	2418

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2419

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2419

A.11.1.4.1.1	Test Purpose and Environment	2419

A.11.1.4.1.2	Test Parameters	2419

A.11.1.4.1.3	Test Requirements	2422

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2423

A.11.1.4.2.1	Test Purpose and Environment	2423

A.11.1.4.2.2	Test Requirements	2426

A.11.2	RRC_CONNECTED state mobility	2427

A.11.2.1	Handover	2427

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2427

A.11.2.1.1.1	Test Purpose and Environment	2427

A.11.2.1.1.2	Test Parameters	2427

A.11.2.1.1.3 Test Requirements	2430

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2430

A.11.2.1.2.1	Test Purpose and Environment	2430

A.11.2.1.2.2	Test Parameters	2430

A.11.2.1.2.3	Test Requirements	2433

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2433

A.11.2.1.3.1	Test Purpose and Environment	2433

A.11.2.1.3.2	Test Parameters	2433

A.11.2.1.3.3 Test Requirements	2436

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2436

A.11.2.1.4.1	Test Purpose and Environment	2436

A.11.2.1.4.2	Test Parameters	2436

A.11.2.1.4.3 Test Requirements	2440

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2441

A.11.2.1.5.1	Test Purpose and Environment	2441

A.11.2.1.5.2	Test Parameters	2441

A.11.2.1.5.3 Test Requirements	2444

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2445

A.11.2.1.6.1	Test Purpose and Environment	2445

A.11.2.1.6.2	Test Parameters	2445

A.11.2.1.6.3 Test Requirements	2448

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2449

A.11.2.1.7.1	Test Purpose and Environment	2449

A.11.2.1.7.2	Test Requirements	2455

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2455

A.11.2.1.8.1	Test Purpose and Environment	2455

A.11.2.1.8.2	Test Requirements	2460

A.11.2.2	RRC connection mobility control	2460

A.11.2.2.1	RRC re-establishment	2460

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2460

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2463

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2470

A.11.2.2.2	Random Access	2475

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2475

A.11.2.2.2.1.1	Test Purpose and Environment	2475

A.11.2.2.2.1.2	Test Requirements	2478

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2478

A.11.2.2.2.1.2.2	Random Access Response Reception	2478

A.11.2.2.2.1.2.3	No Random Access Response Reception	2479

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2479

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2479

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2480

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2480

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2480

A.11.2.2.2.2.1	Test Purpose and Environment	2480

A.11.2.2.2.2.2	Test Requirements	2483

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2483

A.11.2.2.2.2.2.2	Random Access Response Reception	2484

A.11.2.2.2.2.2.3	No Random Access Response Reception	2484

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2484

A.11.2.2.2.3.1	Test Purpose and Environment	2484

A.11.2.2.2.3.2	Test Requirements	2488

A.11.2.2.2.3.2.1	MsgA Transmission	2488

A.11.2.2.2.3.2.2	MsgB Reception	2488

A.11.2.2.2.3.2.3	No MsgB Reception	2489

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2489

A.11.2.2.2.4.1	Test Purpose and Environment	2489

A.11.2.2.2.4.2	Test Requirements	2493

A.11.2.2.2.4.2.1	MsgA Transmission	2493

A.11.2.2.2.4.2.2	MsgB Reception	2494

A.11.2.2.2.4.2.3	No MsgB Reception	2494

A.11.2.2.3	RRC connection release with redirection	2495

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2495

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	2499

A.11.3	Timing	2504

A.11.3.1	UE transmit timing	2504

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	2504

A.11.3.1.1.1	Test Purpose and environment	2504

A.11.3.1.1.2	Test requirements	2507

A.11.3.2	UE timing advance	2508

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2508

A.11.3.2.1.1	Test Purpose and Environment	2508

A.11.3.2.1.2	Test Parameters	2508

A.11.3.2.1.3	Test Requirements	2512

A.11.4	Signalling characteristics	2512

A.11.4.1	Radio link monitoring	2512

A.11.4.1.1	Introduction	2512

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2513

A.11.4.1.2.1	Test purpose and environment	2513

A.11.4.1.2.2	Test requirements	2517

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2517

A.11.4.1.3.1	Test purpose and environment	2517

A.11.4.1.3.2	Test requirements	2523

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	2523

A.11.4.1.4.1	Test purpose and environment	2523

A.11.4.1.4.2	Test requirements	2523

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	2523

A.11.4.1.5.1	Test purpose and environment	2523

A.11.4.1.5.2	Test requirements	2523

A.11.4.2	Interruption	2524

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	2524

A.11.4.2.1.1 Test Purpose and Environment	2524

A.11.4.2.1.2	Test Requirements	2528

A.11.4.3	SCell activation and deactivation delay	2528

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	2528

A.11.4.3.1.1	Test Purpose and Environment	2528

A.11.4.3.1.2	Test Requirements	2533

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 320 ms SCell measurement cycle	2533

A.11.4.3.2.1	Test Purpose and Environment	2533

A.11.4.3.2.2	Test Requirements	2534

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	2534

A.11.4.3.3.1	Test Purpose and Environment	2534

A.11.4.3.3.2	Test Requirements	2535

A.11.4.4	Beam failure detection and link recovery procedures	2535

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2535

A.11.4.4.1.1	Test Purpose and Environment	2535

A.11.4.4.1.2	Test Requirements	2541

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2541

A.11.4.4.2.1	Test Purpose and Environment	2541

A.11.4.4.2.2	Test Requirements	2548

A.11.4.5	Active BWP switching	2548

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	2548

A.11.4.5.1.1	Test Purpose and Environment	2548

A.11.4.5.1.2	Test Requirements	2553

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	2554

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	2554

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	2557

A.11.4.5.3	RRC-based Active BWP Switch	2560

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2560

A.11.4.6	Active TCI state switching delay	2563

A.11.5	Measurement procedure	2563

A.11.5.1	Intra-frequency measurements	2563

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	2563

A.11.5.1.1.1	Test purpose and environment	2563

A.11.5.1.1.2	Test parameters	2563

A.11.5.1.1.3	Test Requirements	2564

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	2564

A.11.5.1.2.1	Test purpose and environment	2564

A.11.5.1.2.2	Test parameters	2564

A.11.5.1.2.3	Test Requirements	2564

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	2565

A.11.5.1.3.1	Test purpose and environment	2565

A.11.5.1.3.2	Test parameters	2565

A.11.5.1.3.3	Test Requirements	2565

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	2565

A.11.5.1.4.1	Test purpose and environment	2565

A.11.5.1.4.2	Test parameters	2565

A.11.5.1.4.3	Test Requirements	2565

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2566

A.11.5.1.5.1	Test purpose and environment	2566

A.11.5.1.5.2	Test parameters	2566

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2566

A.11.5.1.6.1	Test purpose and environment	2566

A.11.5.1.6.2	Test parameters	2566

A.11.5.1.6.3	Test Requirements	2566

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2567

A.11.5.1.7.1	Test purpose and environment	2567

A.11.5.1.7.2	Test parameters	2567

A.11.5.1.7.3	Test Requirements	2567

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2567

A.11.5.1.8.1	Test purpose and environment	2567

A.11.5.1.8.2	Test parameters	2567

A.11.5.1.8.3	Test Requirements	2567

A.11.5.1.9	RSSI measurement reporting on PCC	2568

A.11.5.1.9.1	Test purpose and environment	2568

A.11.5.1.9.2	Test parameters	2568

A.11.5.1.10	Channel occupancy measurement reporting on PCC	2568

A.11.5.1.10.1	Test purpose and environment	2568

A.11.5.1.10.2	Test parameters	2568

A.11.5.1.11	RSSI measurement reporting on SCC	2568

A.11.5.1.11.1	Test purpose and environment	2568

A.11.5.1.11.2	Test parameters	2569

A.11.5.1.12	Channel occupancy measurement reporting on SCC	2569

A.11.5.1.12.1	Test purpose and environment	2569

A.11.5.1.12.2	Test parameters	2569

A.11.5.2	Inter-frequency measurements	2569

A.11.5.2.1	RSSI measurement reporting	2569

A.11.5.2.1.1	Test purpose and environment	2569

A.11.5.2.1.2	Test parameters	2570

A.11.5.2.2	Channel occupancy measurement reporting	2570

A.11.5.2.2.1	Test purpose and environment	2570

A.11.5.2.2.2	Test parameters	2570

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2570

A.11.5.2.3.1	Test Purpose and Environment	2570

A.11.5.2.3.2	Test Requirements	2574

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2575

A.11.5.2.4.1	Test Purpose and Environment	2575

A.11.5.2.4.2	Test Requirements	2580

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2580

A.11.5.2.5.1	Test Purpose and Environment	2580

A.11.5.2.5.2	Test Requirements	2584

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2585

A.11.5.2.6.1	Test Purpose and Environment	2585

A.11.5.2.6.2	Test Requirements	2590

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2590

A.11.5.2.7.1	Test Purpose and Environment	2590

A.11.5.2.7.2	Test Requirements	2594

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2595

A.11.5.2.8.1	Test Purpose and Environment	2595

A.11.5.2.8.2	Test Requirements	2600

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2600

A.11.5.2.9.1	Test Purpose and Environment	2600

A.11.5.2.9.2	Test Requirements	2605

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2606

A.11.5.2.10.1	Test Purpose and Environment	2606

A.11.5.2.10.2	Test Requirements	2611

A.11.5.3	Inter-RAT E-UTRAN measurements	2612

A.11.5.4	L1-RSRP measurements for beam reporting	2612

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	2612

A.11.5.4.1.1	Test Purpose and Environment	2612

A.11.5.4.1.2	Test parameters	2612

A.11.5.4.1.3	Test Requirements	2614

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	2614

A.11.5.4.2.1	Test Purpose and Environment	2614

A.11.5.4.2.2	Test parameters	2615

A.11.5.4.2.3	Test Requirements	2617

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2617

A.11.5.4.3.1	Test Purpose and Environment	2617

A.11.5.4.3.2	Test parameters	2618

A.11.5.4.3.3	Test Requirements	2621

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	2622

A.11.5.4.4.1	Test Purpose and Environment	2622

A.11.5.4.4.2	Test parameters	2622

A.11.5.4.4.3	Test Requirements	2625

A.11.6	Measurement performance	2626

A.11.6.1	SS-RSRP	2626

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2626

A.11.6.1.1.1	Test Purpose and Environment	2626

A.11.6.1.1.2	Test parameters	2626

A.11.6.1.1.3	Test Requirements	2628

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2628

A.11.6.1.2.1	Test Purpose and Environment	2628

A.11.6.1.2.2	Test parameters	2628

A.11.6.1.2.3	Test Requirements	2630

A.11.6.2	SS-RSRQ	2630

A.11.6.3	SS-SINR	2630

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	2630

A.11.6.4.1	SSB based L1-RSRP measurement	2630

A.11.6.4.1.1	Test Purpose and Environment	2630

A.11.6.4.1.2	Test parameters	2630

A.11.6.4.1.3	Test Requirements	2633

A.11.6.5	RSSI	2633

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	2633

A.11.6.5.1.1	Test Purpose and Environment	2633

A.11.6.5.1.2	Test parameters	2633

A.11.6.5.1.3	Test Requirements	2636

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	2636

A.11.6.5.2.1	Test Purpose and Environment	2636

A.11.6.5.2.2	Test parameters	2636

A.11.6.5.2.3	Test Requirements	2639

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2639

A.11.6.5.3.1	Test Purpose and Environment	2639

A.11.6.5.3.2	Test parameters	2639

A.11.6.5.3.3	Test Requirements	2642

A.11.6.6	Channel occupancy	2642

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	2642

A.11.6.6.1.1	Test Purpose and Environment	2642

A.11.6.6.1.2	Test parameters	2642

A.11.6.6.1.3	Test Requirements	2645

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2645

A.11.6.6.2.1	Test Purpose and Environment	2645

A.11.6.6.2.2	Test parameters	2645

A.11.6.6.2.3	Test Requirements	2648

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2648

A.11.6.6.3.1	Test Purpose and Environment	2648

A.11.6.6.3.2	Test parameters	2648

A.11.6.6.3.3	Test Requirements	2651

A.11.6.7	E-UTRAN RSRP	2651

A.11.6.8	E-UTRAN RSRQ	2651

A.11.6.9	E-UTRAN SINR	2651

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	2651

A.12.1	RRC_IDLE state mobility	2651

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	2651

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	2651

A.12.1.1.1.1	Test Purpose and Environment	2651

A.12.1.1.1.2	Test Requirements	2656

A.12.2	RRC_CONNECTED state mobility	2657

A.12.2.1	Handover	2657

A.12.2.1.1	E-UTRAN - NR with CCA handover	2657

A.12.2.1.1.1	Test Purpose and Environment	2657

A.12.2.1.1.2	Test Requirements	2662

A.12.3	Signalling characteristics	2663

A.12.3.1	Interruptions	2663

A.12.4	Measurement procedure	2663

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	2663

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	2663

A.12.4.1.1.1	Test Purpose and Environment	2663

A.12.4.1.1.2	Test Requirements	2667

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2667

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2667

A.12.4.2.1.1	Test Purpose and Environment	2667

A.12.4.2.1.2	Test Requirements	2671

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2672

A.12.4.2.2.1	Test Purpose and Environment	2672

A.12.4.2.2.2	Test Requirements	2676

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2676

A.12.4.2.3.1	Test Purpose and Environment	2676

A.12.4.2.3.2	Test Requirements	2680

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2681

A.12.4.2.4.1	Test Purpose and Environment	2681

A.12.4.2.4.2	Test Requirements	2685

A.12.4.2.5	RSSI measurement reporting	2685

A.12.4.2.5.1	Test purpose and environment	2685

A.12.4.2.5.2	Test parameters	2685

A.12.4.2.6	Channel occupancy measurement reporting	2686

A.12.4.2.6.1	Test purpose and environment	2686

A.12.4.2.6.2	Test parameters	2686

A.12.5	Measurement performance	2686

A.12.5.1	E-UTRANNR SFTD	2686

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	2686

A.12.5.1.1.1	Test Purpose	2686

A.12.5.1.1.2	Test Environment	2687

A.12.5.1.1.3	Test Requirements	2692

A.12.5.2	E-UTRANNR SS-RSRP	2692

A.12.5.3	E-UTRANNR SS-RSRQ	2692

A.12.5.4	E-UTRANNR SS-SINR	2692

A.12.5.5	E-UTRANNR RSSI	2692

A.12.5.6	E-UTRANNR channel occupancy	2692

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	2692

A.13.1	Timing	2692

A.13.1.1	UE transmit timing	2692

A.13.1.2	Timing advance	2692

A.13.2	Signalling characteristics	2692

A.13.2.1	Interruption	2692

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	2692

A.13.2.1.1.1	Test Purpose and Environment	2692

A.13.2.1.1.2	Test Requirements	2696

A.13.2.2	SCell activation and deactivation delay	2696

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	2696

A.13.2.2.1.1	Test Purpose and Environment	2696

A.13.2.2.1.2	Test Requirements	2701

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 320 ms SCell measurement cycle	2701

A.13.2.2.2.1	Test Purpose and Environment	2701

A.13.2.2.2.2	Test Requirements	2702

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	2702

A.13.2.2.3.1	Test Purpose and Environment	2702

A.13.2.2.3.2	Test Requirements	2703

A.13.2.3	Active TCI state switching delay	2703

A.13.3	Measurement procedure	2703

A.13.3.1	Intra-frequency measurements	2703

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	2703

A.13.3.1.1.1	Test purpose and environment	2703

A.13.3.1.1.2	Test parameters	2703

A.13.3.1.1.3	Test Requirements	2708

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	2708

A.13.3.1.2.1	Test purpose and environment	2708

A.13.3.1.2.2	Test parameters	2708

A.13.3.1.2.3	Test Requirements	2712

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2712

A.13.3.1.3.1	Test purpose and environment	2712

A.13.3.1.3.2	Test parameters	2712

A.13.3.1.3.3	Test Requirements	2717

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2717

A.13.3.1.4.1	Test purpose and environment	2717

A.13.3.1.4.2	Test parameters	2717

A.13.3.1.4.3	Test Requirements	2722

A.13.3.1.5	RSSI measurement reporting on SCC	2722

A.13.3.1.5.1	Test purpose and environment	2722

A.13.3.1.5.2	Test parameters	2722

A.13.3.1.6	Channel occupancy measurement reporting on SCC	2723

A.13.3.1.6.1	Test purpose and environment	2723

A.13.3.1.6.2	Test parameters	2724

A.13.3.2	Inter-frequency measurements	2724

A.13.3.2.1	RSSI measurement reporting	2724

A.13.3.2.1.1	Test purpose and environment	2724

A.13.3.2.1.2	Test parameters	2724

A.13.3.2.2	Channel occupancy measurement reporting	2725

A.13.3.2.2.1	Test purpose and environment	2725

A.13.3.2.2.2	Test parameters	2725

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2725

A.13.3.2.3.1	Test Purpose and Environment	2725

A.13.3.2.3.2	Test Requirements	2730

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2730

A.13.3.2.4.1	Test Purpose and Environment	2730

A.13.3.2.4.2	Test Requirements	2735

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2736

A.13.3.2.5.1	Test Purpose and Environment	2736

A.13.3.2.5.2	Test Requirements	2741

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2741

A.13.3.2.6.1	Test Purpose and Environment	2741

A.13.3.2.6.2	Test Requirements	2747

A.13.3.3	L1-RSRP measurements for beam reporting	2748

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	2748

A.13.3.3.1.1	Test Purpose and Environment	2748

A.13.3.3.1.2	Test parameters	2748

A.13.3.3.1.3	Test Requirements	2752

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	2752

A.13.3.3.2.1	Test Purpose and Environment	2752

A.13.3.3.2.2	Test parameters	2752

A.13.3.3.2.3	Test Requirements	2756

A.13.4	Measurement performance	2756

A.13.4.1	SS-RSRP	2756

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2756

A.13.4.1.1.1	Test Purpose and Environment	2756

A.13.4.1.1.2	Test parameters	2756

A.13.4.1.1.3	Test Requirements	2758

A.13.4.2	SS-RSRQ	2758

A.13.4.3	SS-SINR	2758

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	2758

A.13.4.4.1	SSB based L1-RSRP measurement	2758

A.13.4.4.1.1	Test Purpose and Environment	2758

A.13.4.4.1.2	Test parameters	2758

A.13.4.4.1.3	Test Requirements	2763

A.13.4.5	RSSI	2763

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	2763

A.13.4.5.1.1	Test Purpose and Environment	2763

A.13.4.5.1.2	Test parameters	2763

A.13.4.5.1.3	Test Requirements	2766

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2766

A.13.4.5.2.1	Test Purpose and Environment	2766

A.13.4.5.2.2	Test parameters	2766

A.13.4.5.2.3	Test Requirements	2770

A.13.4.6	Channel occupancy	2770

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2770

A.13.4.6.1.1	Test Purpose and Environment	2770

A.13.4.6.1.2	Test parameters	2770

A.13.4.6.1.3	Test Requirements	2774

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2774

A.13.4.6.2.1	Test Purpose and Environment	2774

A.13.4.6.2.2	Test parameters	2774

A.13.4.6.2.3	Test Requirements	2778

Annex B (normative): Conditions for RRM requirements applicability for operating bands	1787

B.1	Conditions for NR RRC_IDLE state mobility	1787

B.1.1	Introduction	1787

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1787

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1788

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1788

B.2.1	Introduction	1788

B.2.1.1	General	1788

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1789

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1789

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1789

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1790

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	1790

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	1790

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	1790

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	1791

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	1791

B.2.2	Conditions for NR intra-frequency measurements	1792

B.2.3	Conditions for NR inter-frequency measurements	1793

B.2.4	Conditions for NR L1-RSRP reporting	1794

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1794

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1795

B.2.5	Conditions for RRC connection release with redirection to NR	1796

B.2.6	Void	1798

B.2.6.1	Void	1798

B.2.6.2	Void	1798

B.2.8	Conditions for NR L1-SINR reporting	1799

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1799

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	1800

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	1800

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	1800

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1801

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	1801

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	1802

B.2.9	Conditions for NR intra-frequency measurements under CCA	1804

B.2.10	Conditions for NR inter-frequency measurements under CCA	1804

B.2.11	Conditions for NR L1-RSRP reporting under CCA	1804

B.2.11.1	Conditions for SSB based L1-RSRP reporting	1804

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	1805

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	1806

B.2.14	Conditions for NR PRS-based measurements	1808

B.3	RRM Requirements Exceptions	1809

B.3.1	Introduction	1809

B.3.2	Receiver sensitivity relaxation for CA	1809

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1809

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1810

B.3.2.2.1	Inter-band carrier aggregation	1810

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1810

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1810

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1810

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1810

B.3.2.4.1	Intra-band contiguous carrier aggregation	1810

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1810

B.3.3	Receiver sensitivity relaxation for DC	1811

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1811

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1811

B.3.4	Receiver sensitivity relaxation for SUL	1811

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1811

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1811

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1811

B.4	Conditions for V2X	1811

B.4.1	Test parameters for GNSS signals	1811

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	1812

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	1812

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	1812

Annex C (informative): Change history	1814