| 3GPP TS 38.133 V17.6.0 (2022-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	52

3.6.1	RRC connected state requirements in DRX	53

3.6.2	Number of serving carriers	53

3.6.2.1	Number of serving carriers for SA	53

3.6.2.2	Number of serving carriers for EN-DC	53

3.6.2.3	Number of serving carriers for NE-DC	53

3.6.2.4	Number of serving carriers for NR-DC	54

3.6.3	Applicability for intra-band FR2	54

3.6.4	Applicability for FR2 UE power classes	54

3.6.5	Applicability for SDL bands	54

3.6.6	Applicability of requirements for NGEN-DC operation	54

3.6.7	Applicability of QCL	54

3.6.9	Applicability of requirements for scheduling availability	55

3.6.10	Applicability of requirements for measurement restrictions	55

4	SA: RRC_IDLE state mobility	55

4.1	Cell Selection	55

4.2	Cell Re-selection	56

4.2.1	Introduction	56

4.2.2	Requirements	56

4.2.2.1	UE measurement capability	56

4.2.2.2	Measurement and evaluation of serving cell	56

4.2.2.3	Measurements of intra-frequency NR cells	57

4.2.2.4	Measurements of inter-frequency NR cells	58

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	61

4.2.2.6	Maximum interruption in paging reception	63

4.2.2.7	General requirements	63

4.2.2.8	Minimum requirement at transitions	63

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	64

4.2.2.9.1	Introduction	64

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	64

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	65

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	65

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	65

4.2.2.10.1	Introduction	65

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	66

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	67

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	68

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	68

4.2.2.11.1	Introduction	68

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	68

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	69

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	70

4.2A	Cell Re-selection when subject to CCA	71

4.2A.1	Introduction	71

4.2A.2	Requirements	71

4.2A.2.1	UE measurement capability	71

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	71

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	72

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	73

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	75

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	75

4.2A.2.7	General requirements	76

4.3	Minimization of Drive Tests (MDT)	76

4.3.1	Introduction	76

4.3.2	Measurement Requirements	76

4.3.3	Requirements for Relative Time Stamp Accuracy	77

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	77

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	77

4.4	Idle Mode CA/DC Measurements	77

4.4.1	Introduction	77

4.4.2	Measurement Requirements	77

4.4.2.1	Detected cell requirement during state transition and Idle mode	78

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	78

4.4.2.3	Measurements on serving cell	79

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	79

5	SA: RRC_INACTIVE state mobility	81

5.1	Cell Re-selection	81

5.1.1	Introduction	81

5.1.2	Requirements	81

5.1.2.1	UE measurement capability	81

5.1.2.2	Measurement and evaluation of serving cell	81

5.1.2.3	Measurements of intra-frequency NR cells	81

5.1.2.4	Measurements of inter-frequency NR cells	81

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	81

5.1.2.6	Maximum interruption in paging reception	81

5.1.2.7	General requirements	81

5.1A	Cell Re-selection with CCA	81

5.1A.1	Introduction	81

5.1A.2	Requirements	82

5.1A.2.1	UE measurement capability	82

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	82

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	82

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	82

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	82

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	82

5.1A.2.7	General requirements	82

5.2	Void	82

5.3	Minimization of Drive Tests (MDT)	82

5.3.1	Introduction	82

5.3.2	Measurement Requirements	83

5.3.3	Requirements for Relative Time Stamp Accuracy	83

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	83

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	83

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	83

5.4	Idle Mode CA/DC Measurements	83

5.4.1	Introduction	83

5.4.2	Measurement Requirements	83

5.4.2.1	Detected cell requirement during state transition and Idle mode	84

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	84

5.4.2.3	Measurements on serving cell	84

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	84

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	84

5.5.1 Introduction	84

5.5.2	Requirements on UE synchronization for small data transmissions	84

5.5.3	TA validation requirements	84

5.5.4	Scheduling restriction	85

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	85

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	85

5.5.4.3	Scheduling availability of UE performing measurements on FR2	86

5.6	NR measurements for positioning	86

5.6.1	Introduction	86

5.6.2	RSTD measurements	87

5.6.2.1	Introduction	87

5.6.2.2	Requirements Applicability	87

5.6.2.3	Measurement Capability	87

5.6.2.5	Measurements Period Requirements	88

5.6.3	PRS-RSRP measurements	90

5.6.3.1	Introduction	90

5.6.3.2	Requirements applicability	90

5.6.3.3	Measurement Capability	90

5.6.3.4	Measurement Reporting Requirements	90

5.6.3.5	Measurement Period Requirements	90

5.6.4	UE Rx-Tx time difference measurements	92

5.6.4.1	Introduction	92

5.6.4.2	Requirements Applicability	93

5.6.4.3	Measurement Capability	93

5.6.4.4	Measurement Reporting Requirements	93

5.6.4.5	Measurement Period Requirements	93

5.6.5	PRS-RSRPP measurements	96

5.6.5.1	Introduction	96

5.6.5.2	Requirements applicability	96

5.6.5.3	Measurement capability	96

5.6.5.4 Measurement reporting requirements	96

5.6.5.5	Measurement period requirements	96

6	RRC_CONNECTED state mobility	98

6.1	Handover	98

6.1.1	NR Handover	98

6.1.1.1	Introduction	98

6.1.1.2	NR FR1 - NR FR1 Handover	98

6.1.1.2.1	Handover delay	98

6.1.1.2.2	Interruption time	98

6.1.1.3	NR FR2- NR FR1 Handover	99

6.1.1.3.1	Handover delay	99

6.1.1.3.2	Interruption time	99

6.1.1.4	NR FR2- NR FR2 Handover	100

6.1.1.4.1	Handover delay	100

6.1.1.4.2	Interruption time	100

6.1.1.5	NR FR1- NR FR2 Handover	101

6.1.1.5.1	Handover delay	101

6.1.1.5.2	Interruption time	101

6.1.2	NR Handover to other RATs	102

6.1.2.1	NR – E-UTRAN Handover	102

6.1.2.1.1	Introduction	102

6.1.2.1.2	Handover delay	102

6.1.2.1.3	Interruption time	102

6.1.2.2	NR – UTRAN Handover	103

6.1.2.2.1	Introduction	103

6.1.2.2.2	Handover delay	103

6.1.2.2.3	Interruption time	103

6.1.3	NR DAPS Handover	104

6.1.3.1	Introduction	104

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	104

6.1.3.2.1	DAPS handover delay	104

6.1.3.2.2	Interruption time	105

6.1.3.3	NR FR2- NR FR1 DAPS Handover	107

6.1.3.3.1	DAPS handover delay	107

6.1.3.3.2	Interruption time	107

6.1.3.4	NR FR1- NR FR2 DAPS Handover	108

6.1.3.4.1	DAPS handover delay	108

6.1.3.4.2	Interruption time	108

6.1.4	NR Conditional Handover	109

6.1.4.1	Introduction	109

6.1.4.2	NR FR1 – NR FR1 conditional handover	109

6.1.4.3	NR FR2 – NR FR1 conditional handover	110

6.1.4.4	NR FR2 – NR FR2 conditional handover	111

6.1.4.4.1	Handover delay	111

6.1.4.4.2	Measurement time	111

6.1.4.4.3	Preparation time	111

6.1.4.4.4	Interruption time	111

6.1.4.5	NR FR1 – NR FR2 conditional handover	112

6.1.5	NR Handover with PSCell	112

6.1.5.1	Introduction	112

6.1.5.2	Handover with PSCell from NR SA to EN-DC	112

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	113

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	113

6.1.5.3	HO with PSCell from NE-DC to NE-DC	113

6.1.5.3.1	Handover delay	114

6.1.5.3.2	HO with PSCell - PCell Interruption time	114

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC  HO with PSCell	114

6.1.5.4	HO with PSCell from NR-DC to NR-DC	114

6.1A	Void	116

6.1A.1	Void	116

6.1A.1.1	Void	116

6.1A.1.2	Void	116

6.1A.1.2.1	Void	116

6.1A.1.2.2	Void	116

6.1B	Handover to target cell using CCA	116

6.1B.1	NR Handover	116

6.1B.1.1	Introduction	116

6.1B.1.2	NR FR1 - NR FR1 Handover	116

6.1B.1.2.1	Handover delay	116

6.1B.1.2.2	Interruption time	116

6.2	RRC Connection Mobility Control	117

6.2.1	SA: RRC Re-establishment	117

6.2.1.1	Introduction	117

6.2.1.2	Requirements	117

6.2.1.2.1	UE Re-establishment delay requirement	118

6.2.1A	RRC Re-establishment with CCA	119

6.2.1A.1	Introduction	119

6.2.1A.2	Requirements	120

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	120

6.2.2	Random access	121

6.2.2.1	Introduction	121

6.2.2.2	Requirements for 4-step RA type	121

6.2.2.2.1	Contention based random access	122

6.2.2.2.2	Non-Contention based random access	123

6.2.2.2.3	UE behaviour when configured with supplementary UL	124

6.2.2.3	Requirements for 2-step RA type	124

6.2.2.3.1	Contention based random access	124

6.2.2.3.2	Non-Contention based random access	125

6.2.2.3.3	UE behaviour when configured with supplementary UL	125

6.2.2A	Random access when CCA is used on target frequency	126

6.2.2A.1	Introduction	126

6.2.2A.2	Requirements for 4-step RA type	126

6.2.2A.2.1	Contention based random access	126

6.2.2A.2.2	Non-Contention based random access	127

6.2.2A.3	Requirements for 2-step RA type	128

6.2.2A.3.1	Contention based random access	129

6.2.2A.3.2	Non-Contention based random access	130

6.2.3	SA: RRC Connection Release with Redirection	130

6.2.3.1	Introduction	130

6.2.3.2	Requirements	131

6.2.3.2.1	RRC connection release with redirection to NR	131

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	132

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	132

7	Timing	133

7.1	UE transmit timing	133

7.1.1	Introduction	133

7.1.2	Requirements	134

7.1.2.1	Gradual timing adjustment	135

7.1.2.2	Void	135

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	135

7.2	UE timer accuracy	136

7.2.1	Introduction	136

7.2.2	Requirements	136

7.3	Timing advance	136

7.3.1	Introduction	136

7.3.2	Requirements	136

7.3.2.1	Timing Advance adjustment delay	136

7.3.2.2	Timing Advance adjustment accuracy	137

7.4	Cell phase synchronization accuracy	137

7.4.1	Definition	137

7.4.2	Minimum requirements	137

7.5	Maximum Transmission Timing Difference	137

7.5.1	Introduction	137

7.5.2	Minimum Requirements for inter-band EN-DC	137

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	138

7.5.3	Minimum Requirements for intra-band EN-DC	138

7.5.4	Minimum Requirements for NR Carrier Aggregation	138

7.5.5	Minimum Requirements for inter-band NE-DC	139

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	139

7.5.6	Minimum Requirements for inter-band NR DC	139

7.6	Maximum Receive Timing Difference	140

7.6.1	Introduction	140

7.6.2	Minimum Requirements for inter-band EN-DC	140

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	141

7.6.3	Minimum Requirements for intra-band EN-DC	141

7.6.4	Minimum Requirements for NR Carrier Aggregation	142

7.6.5	Minimum Requirements for inter-band NE-DC	142

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	143

7.6.6	Minimum Requirements for inter-band NR DC	143

7.7	deriveSSB-IndexFromCell tolerance	144

7.7.1	Minimum requirements	144

7.8	Void	144

7.9	deriveSSB-IndexFromCell tolerance	144

7.9.1	Minimum requirements	144

8	Signalling characteristics	144

8.1	Radio Link Monitoring	144

8.1.1	Introduction	144

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	145

8.1.2	Requirements for SSB based radio link monitoring	146

8.1.2.1	Introduction	146

8.1.2.2	Minimum requirement	147

8.1.2.3	Measurement restrictions for SSB based RLM	150

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	151

8.1.3	Requirements for CSI-RS based radio link monitoring	152

8.1.3.1	Introduction	152

8.1.3.2	Minimum requirement	152

8.1.3.3	Measurement restrictions for CSI-RS based RLM	156

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	156

8.1.4	Minimum requirement at transitions	157

8.1.5	Minimum requirement for UE turning off the transmitter	158

8.1.6	Minimum requirement for L1 indication	158

8.1.7	Scheduling availability of UE during radio link monitoring	158

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	158

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	158

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	159

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	159

8.1A	Radio Link Monitoring with CCA on Target Frequency	159

8.1A.1	Introduction	159

8.1A.2	Requirements for SSB Based Radio Link Monitoring	160

8.1A.2.1	Introduction	160

8.1A.2.2	Minimum Requirement	161

8.1A.3	Minimum requirement at transitions	163

8.1A.4	Minimum requirement for UE turning off the transmitter	163

8.1A.5	Minimum requirement for L1 indication	164

8.1A.6	Scheduling availability of UE during radio link monitoring	164

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	164

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	164

8.2	Interruption	164

8.2.1	EN-DC Interruption	164

8.2.1.1	Introduction	164

8.2.1.2	Requirements	165

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	165

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	166

8.2.1.2.3	Interruptions at SCell addition/release	166

8.2.1.2.4	Interruptions at SCell activation/deactivation	167

8.2.1.2.5	Interruptions during measurements on SCC	168

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	169

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	170

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	171

8.2.1.2.9	Interruptions at SCell hibernation	171

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	171

8.2.1.2.11	Interruptions due to UE-specific CBW change	172

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	172

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	173

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	174

8.2.1.2.15	Interruptions due to SCell dormancy	175

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps	175

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	176

8.2.1.2.18	 Interruptions at NR SRS antenna port switching	176

8.2.1.2.19	Interruptions at fast SCell activation	177

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	178

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	178

8.2.2.1	Introduction	178

8.2.2.2	Requirements	179

8.2.2.2.1	Interruptions at SCell addition/release	179

8.2.2.2.2	Interruptions at SCell activation/deactivation	180

8.2.2.2.3	Interruptions during measurements on deactivated SCC	181

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	181

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	182

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	183

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	184

8.2.2.2.8	Interruptions due to UE-specific CBW change	184

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	184

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	186

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	186

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	187

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	187

8.2.2.2.11	Interruptions at direct SCell activation	188

8.2.2.2.12	Interruptions due to SCell dormancy	188

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	188

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	188

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	189

8.2.2.2.16	 Interruptions at NR SRS antenna port switching	189

8.2.2.2.17	Interruptions at fast SCell activation	191

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	191

8.2.3	NE-DC Interruptions	191

8.2.3.1	Introduction	191

8.2.3.2	Requirements	192

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	192

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	192

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	193

8.2.3.2.4	Interruptions at SCell activation/deactivation	194

8.2.3.2.5	Interruptions during measurements on SCC	195

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	196

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	196

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	197

8.2.3.2.9	Interruptions at SCell hibernation	197

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	197

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	197

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	199

8.2.3.2.13	Interruptions due to SCell dormancy	200

8.2.3.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	200

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	201

8.2.3.2.17	Interruptions at fast SCell activation	203

8.2.3.2.18	Interruptions due to UE-specific CBW change	203

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	203

8.2.4	NR-DC: Interruptions	203

8.2.4.1	Introduction	203

8.2.4.2	Requirements	204

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	204

8.2.4.2.2	Interruptions at SCell activation/deactivation	205

8.2.4.2.3	Interruptions during measurements on SCC	206

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	206

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	207

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	207

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	207

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	207

8.2.4.2.9	Interruptions at NR SRS carrier based switching	208

8.2.4.2.10	Interruptions at direct SCell activation	209

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps	209

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	210

8.2.4.2.13	 Interruptions due to SCell dormancy	211

8.2.4.2.14	Interruptions at NR SRS antenna port switching	211

8.2.4.2.15	Interruptions at SCell activation	212

8.2.4.2.16	Interruptions at SCG activation/deactivation	213

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	213

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PScell	213

8.2.4.2A	Void	214

8.2.4.2A.1	Void	214

8.2.4.2A.2	Void	214

8.2.4.2A.3	Void	214

8.3	SCell Activation and Deactivation Delay	214

8.3.1	Introduction	214

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	214

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	218

8.3.4	Direct SCell Activation at SCell addition	219

8.3.5	Direct SCell Activation at Handover	220

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	222

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	226

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	226

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	227

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	229

8.3.13	SCell activation delay Requirement for Deactivated PUCCH Scell with Multiple SCells	231

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	231

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	231

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	232

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	234

8.3A.1	Introduction	234

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	235

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	237

8.4	UE UL carrier RRC reconfiguration delay	238

8.4.1	Introduction	238

8.4.2	UE UL carrier configuration delay requirement	238

8.4.3	UE UL carrier deconfiguration delay requirement	238

8.5	Link Recovery Procedures	238

8.5.1	Introduction	238

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	239

8.5.2	Requirements for SSB based beam failure detection	240

8.5.2.1	Introduction	240

8.5.2.2	Minimum requirement	240

8.5.2.3	Measurement restriction for SSB based beam failure detection	243

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	244

8.5.3	Requirements for CSI-RS based beam failure detection	245

8.5.3.1	Introduction	245

8.5.3.2	Minimum requirement	245

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	249

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	250

8.5.4	Minimum requirement for L1 indication	250

8.5.5	Requirements for SSB based candidate beam detection	251

8.5.5.1	Introduction	251

8.5.5.2	Minimum requirement	251

8.5.5.3	Measurement restriction for SSB based candidate beam detection	254

8.5.6	Requirements for CSI-RS based candidate beam detection	254

8.5.6.1	Introduction	254

8.5.6.2	Minimum requirement	254

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	258

8.5.7	Scheduling availability of UE during beam failure detection	258

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	258

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	258

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	259

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	259

8.5.8	Scheduling availability of UE during candidate beam detection	259

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	259

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	260

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	260

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	260

8.5.9	Requirements for Beam Failure Recovery in SCell	261

8.5.9.1	Introduction	261

8.5.9.2	Requirement	261

8.5.10	Minimum requirement at transitions for beam failure detection	261

8.5A	Link Recovery Procedures when CCA is used on target frequency	261

8.5A.1	Introduction	261

8.5A.2	Requirements for SSB based beam failure detection	262

8.5A.2.1	Introduction	262

8.5A.2.2	Minimum requirement	263

8.5A.2.3	Measurement restriction for SSB based beam failure detection	264

8.5A.4	Minimum requirement for L1 indication	264

8.5A.5	Requirements for SSB based candidate beam detection	264

8.5A.5.1	Introduction	264

8.5A.5.2	Minimum requirement	264

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	266

8.5A.7	Scheduling availability of UE during beam failure detection	266

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	266

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	266

8.5A.8	Scheduling availability of UE during candidate beam detection	266

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	267

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	267

8.6	Active BWP switch delay	267

8.6.1	Introduction	267

8.6.2	DCI and timer based BWP switch delay on a single CC	267

8.6.2A	DCI based BWP switch delay on multiple CCs	268

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	268

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	270

8.6.2B	Timer based BWP switch delay on multiple CCs	270

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	270

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	271

8.6.3	RRC based BWP switch delay on a single CC	271

8.6.3A	RRC based BWP switch delay on multiple CCs	272

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	272

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	272

8.6.4	BWP switch delay on Consistent UL CCA recovery	273

8.7	Void	273

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	273

8.8.1	Introduction	273

8.8.2	E-UTRAN PSCell Addition Delay Requirement	273

8.8.3	E-UTRAN PSCell Release Delay Requirement	274

8.9	NR-DC: PSCell Addition and Release Delay	274

8.9.1	Introduction	274

8.9.2	PSCell Addition Delay Requirement	274

8.9.3	PSCell Release Delay Requirement	275

8.9A	Conditional PSCell Addition Delay	275

8.9A.1	Introduction	275

8.9A.2	Conditional PSCell Addition Delay Requirement	275

8.9A.2.1	Measurement time	276

8.10	Active TCI state switching delay	276

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	278

8.10.4	DCI based TCI state switch delay	278

8.10.5	RRC based TCI state switch delay	278

8.10.6	Active TCI state list update delay	279

8.10A	Active TCI state switching delay with CCA	279

8.10A.1	Introduction	279

8.10A.2	Known conditions for TCI state	279

8.10A.3	MAC-CE based TCI state switch delay	280

8.10A.4	DCI based TCI state switch delay	280

8.10A.5	RRC based TCI state switch delay	281

8.10A.6	Active TCI state list update delay	281

8.11	PSCell Change	282

8.11A	void	282

8.11B	Conditional PSCell Change	282

8.11B.1	Introduction	282

8.11B.2	Conditoinal PSCell Change delay	282

8.11B.2.1	Measurement time	283

8.12	Uplink spatial relation switch delay	283

8.12.1	Introduction	283

8.12.2	Known conditions for spatial relation when associated with DL-RS	283

8.12.3	MAC-CE based spatial relation switch delay	284

8.12.4	DCI based spatial relation switch delay	285

8.12.5	RRC based spatial relation switch delay	285

8.13	UE-specific CBW change	285

8.13.1	Introduction	285

8.13.2	UE-specific CBW change delay	285

8.14	Pathloss reference signal switching delay	286

8.14.1	Introduction	286

8.14.2	Known conditions for pathloss reference signal	286

8.14.3	MAC-CE based pathloss reference signal switch delay	286

8.15	Active downlink TCI state switching delay for unified TCI	287

8.15.1	Introduction	287

8.15.4	DCI based downlink TCI state switch delay	288

8.15.5	Active Downlink TCI state list update delay	289

8.16	Active uplink TCI state switching delay for unified TCI	289

8.16.1	Introduction	289

8.16.4	DCI based uplink TCI state switch delay	291

8.16.5	Active Uplink TCI state list update delay	292

8.17	SCG Activation and Deactivation Delay	292

8.17.1	Introduction	292

8.17.2	SCG Activation Delay Requirement	292

8.17.2	SCG Deactivation Delay Requirement	293

8.18	TRP specific Link Recovery Procedures	294

8.18.1	Introduction	294

8.18.2	Requirements for TRP specific SSB based beam failure detection	294

8.18.2.1	Introduction	294

8.18.2.2	Minimum requirement	295

8.18.2.3	Measurement restriction for SSB based beam failure detection	297

8.18.3	Requirements for CSI-RS based beam failure detection	297

8.18.3.1	Introduction	297

8.18.3.2	Minimum requirement	298

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	300

8.19	Pre-configured measurement gap activation/deactivation delay	301

8.19.1	Introduction	301

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	301

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	301

819.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	301

8.19.3	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	301

9	Measurement Procedure	302

9.1	General measurement requirement	302

9.1.1	Introduction	302

9.1.2	Measurement gap	302

9.1.2.1	EN-DC: Measurement Gap Sharing	314

9.1.2.1a	SA: Measurement Gap Sharing	314

9.1.2.1b	NE-DC: Measurement Gap Sharing	315

9.1.2.1c	NR-DC: Measurement Gap Sharing	316

9.1.3	UE Measurement capability	317

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	317

9.1.3.1a	SA: Monitoring of multiple layers using gaps	317

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	318

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	318

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	319

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	320

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	320

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	321

9.1A.3.2	Void	322

9.1.3A	UE Measurement capability under operation mode with CCA	322

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	322

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	322

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	322

9.1A.3.2a	Void	323

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	323

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	323

9.1.4.1	Introduction	323

9.1.4.2	Requirements	323

9.1.5	Carrier-specific scaling factor	327

9.1.5.1	Monitoring of multiple layers outside gaps	328

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	329

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	330

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	331

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	332

9.1.5.2	Monitoring of multiple layers within gaps	333

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	335

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	336

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	338

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	340

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	341

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	341

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	342

9.1.5.3	Monitoring of multiple layers within NCSG	342

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	342

9.1.6	Minimum requirement at transitions	343

9.1.7	Pre-configured measurement gap	343

9.1.7.1	Introduction	343

9.1.7.2	Requirements applicability	344

9.1.7.3	Requirements	344

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	344

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	345

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	346

9.1.8	Concurrent measurement gaps	346

9.1.8.1	Introduction	346

9.1.8.2	Requirements	346

9.1.8.3	Collision between concurrent measurement gaps	347

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	348

9.1.9	Network controlled small gap	348

9.1.9.1	Introduction	348

9.1.9.2	Requirements applicability	349

9.1.10	MUSIM gaps	350

9.2	NR intra-frequency measurements	352

9.2.1	Introduction	352

9.2.2	Requirements applicability	353

9.2.3	Number of cells and number of SSB	353

9.2.3.1	Requirements for FR1	353

9.2.3.2	Requirements for FR2	353

9.2.4	Measurement Reporting Requirements	354

9.2.4.1	Periodic Reporting	354

9.2.4.2	Event-triggered Periodic Reporting	354

9.2.4.3	Event Triggered Reporting	354

9.2.5	Intrafrequency measurements without measurement gaps	355

9.2.5.1	Intrafrequency cell identification	355

9.2.5.2	Measurement period	359

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	362

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	362

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	363

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	363

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	364

9.2.5.4	SFTD Measurements between PCell and PSCell	364

9.2.5.4.1	Introduction	364

9.2.5.4.2	SFTD Measurement delay	364

9.2.5.4.3	SFTD Measurement Reporting Delay	365

9.2.6	Intra-frequency measurements with measurement gaps	365

9.2.6.1	Void	365

9.2.6.2	Intra-frequency cell identification	365

9.2.6.3	Intrafrequency Measurement Period	367

9.2.7	Intra-frequency measurements with NCSG	368

9.2.7.1	Intra-frequency cell identification	368

9.2.7.2	Measurement period	370

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	371

9.2A	NR intra-frequency measurements with CCA	371

9.2A.1	Introduction	371

9.2A.2	Requirements applicability	372

9.2A.3	Number of cells and number of SSB	372

9.2A.4	Measurement Reporting Requirements	373

9.2A.5	Intra-frequency measurements without measurement gaps	373

9.2A.5.2	Measurement period	376

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	377

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	377

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	378

9.2A.6	Intra-frequency measurements with measurement gaps	378

9.2A.6.1	Intra-frequency cell identification	378

9.2A.6.2	Intra-frequency Measurement Period	379

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	380

9.2A.7.1	Intra-frequency RSSI measurements	380

9.2A.7.2	Intra-frequency Channel occupancy measurements	381

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	382

9.3	NR inter-frequency measurements	382

9.3.1	Introduction	382

9.3.2	Requirements applicability	383

9.3.2.1	Void	383

9.3.2.2	Void	383

9.3.3	Number of cells and number of SSB	383

9.3.3.1	Requirements for FR1	383

9.3.3.2	Requirements for FR2	384

9.3.4	Inter-frequency measurement with measurement gaps	384

9.3.4.1	Void	386

9.3.4.2	Void	386

9.3.5	Inter-frequency measurements	386

9.3.5.1	Void	387

9.3.5.2	Void	387

9.3.5.3	Void	387

9.3.6	Inter-frequency measurements reporting requirements	387

9.3.6.1	Periodic Reporting	387

9.3.6.2	Event-triggered Periodic Reporting	388

9.3.6.3	Event-triggered Reporting	388

9.3.7	Void	388

9.3.8	Inter-frequency SFTD measurement requirements	388

9.3.8.1	Introduction	388

9.3.8.2	SFTD Measurement delay	388

9.3.8.3	SFTD Measurement reporting delay	389

9.3.9	Inter frequency measurements without measurement gaps	390

9.3.9.1	Inter frequency Cell identification	390

9.3.9.2	Measurement period	392

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	393

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	393

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	393

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	394

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	394

9.3.10	Inter-frequency measurement with NCSG	394

9.3.10.1	Inter-frequency cell identification	394

9.3.10.2	Measurement period	396

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	396

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	396

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	397

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	398

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	400

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	400

9.3A.1	Introduction	400

9.3A.2	Requirements applicability	401

9.3A.3	Number of cells and number of SSB	401

9.3A.3.1	Requirements	401

9.3A.4	Inter-frequency cell identification	401

9.3A.5	Inter-frequency measurements	402

9.3A.6	NR Inter-frequency measurements reporting requirements	403

9.3A.6.1	Periodic Reporting	403

9.3A.6.2	Event-triggered Periodic Reporting	403

9.3A.6.3	Event-triggered Reporting	403

9.3A.8	Inter-frequency RSSI measurements	404

9.3A.9	Inter-frequency channel occupancy measurements	404

9.4	Inter-RAT measurements	405

9.4.1	Introduction	405

9.4.2	NR − E-UTRAN FDD measurements	407

9.4.2.1	Introduction	407

9.4.2.2	Requirements when no DRX is used	407

9.4.2.3	Requirements when DRX is used	408

9.4.2.4	Measurement reporting requirements	410

9.4.2.4.1	Periodic Reporting	410

9.4.2.4.2	Event-Triggered Periodic Reporting	410

9.4.2.4.3	Event-Triggered Reporting	410

9.4.3	NR − E-UTRAN TDD measurements	410

9.4.3.1	Introduction	410

9.4.3.2	Requirements when no DRX is used	411

9.4.3.3	Requirements when DRX is used	412

9.4.3.4	Measurement reporting requirements	414

9.4.3.4.1	Periodic Reporting	414

9.4.3.4.2	Event-Triggered Periodic Reporting	414

9.4.3.4.3	Event-Triggered Reporting	414

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	415

9.4.4	Inter-RAT RSTD measurements	415

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	415

9.4.4.1.1	Introduction	415

9.4.4.1.2	Requirements	415

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	418

9.4.4.2.1	Introduction	418

9.4.4.2.2	Requirements	419

9.4.5	Inter-RAT E-CID measurements	422

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	422

9.4.5.1.1	Introduction	422

9.4.5.1.2	Requirements	422

9.4.5.1.3	Measurement Reporting Delay	423

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	423

9.4.5.2.1	Introduction	423

9.4.5.2.2	Requirements	423

9.4.5.2.3	Measurement Reporting Delay	423

9.4.6	NR − UTRAN FDD measurements	423

9.4.6.1	Introduction	423

9.4.6.2	Requirements when no DRX is used	423

9.4.6.3	Requirements when DRX is used	425

9.4.7	NR – E-UTRAN measurements with autonomous gaps	427

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	427

The requirement for identifying a new CGI of an E-UTRA cell within  is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [2] is used.9.4.7.2	CGI reporting delay	427

9.5	L1-RSRP measurements for Reporting	427

9.5.1	Introduction	427

9.5.2	Requirements applicability	427

9.5.3	Measurement Reporting Requirements	428

9.5.3.1	Periodic Reporting	428

9.5.3.2	Semi-Persistent Reporting	428

9.5.3.3	Aperiodic Reporting	429

9.5.4	L1-RSRP measurement requirements	429

9.5.4.1	SSB based L1-RSRP Reporting	429

9.5.4.2	CSI-RS based L1-RSRP Reporting	432

9.5.4A	Void	436

9.5.4A.1	Void	436

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	436

9.5.5.1	Measurement restriction for SSB based L1-RSRP	436

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	436

9.5.6	Scheduling availability of UE during L1-RSRP measurement	437

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	437

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	437

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	437

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	439

9.5A	L1-RSRP measurements for Reporting under CCA	439

9.5A.1	Introduction	439

9.5A.2	Requirements applicability	439

9.5A.3	Measurement Reporting Requirements	439

9.5A.3.1	Periodic Reporting	440

9.5A.3.2	Semi-Persistent Reporting	440

9.5A.3.3	Aperiodic Reporting	440

9.5A.4	L1-RSRP measurement requirements	440

9.5A.4.1	SSB based L1-RSRP Reporting	440

9.5A.5	Measurement restriction for L1-RSRP measurement	442

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	442

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	442

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	442

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	443

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	443

9.6	NE-DC: Measurements	443

9.6.1	Introduction	443

9.6.2	SFTD Measurements	443

9.6.2.1	Introduction	443

9.6.2.2	SFTD Measurement requirements	443

9.7	Cross Link Interference measurements	444

9.7.1	Introduction	444

9.7.2	SRS-RSRP measurements	444

9.7.2.1	Introduction	444

9.7.2.2	Requirements applicability	444

9.7.2.3	Measurement Reporting Requirements	445

9.7.2.3.1	Periodic Reporting	445

9.7.2.3.2	Event-triggered Periodic Reporting	445

9.7.2.3.3	Event Triggered Reporting	445

9.7.2.4	Measurement capability	445

9.7.2.5	SRS-RSRP measurement period	445

9.7.3	CLI-RSSI measurements	446

9.7.3.1	Introduction	446

9.7.3.2	Requirements applicability	446

9.7.3.3	Measurement Reporting Requirements	446

9.7.3.3.1	Periodic Reporting	446

9.7.3.3.2	Event-triggered Periodic Reporting	446

9.7.3.3.3	Event Triggered Reporting	446

9.7.3.4	Measurement capability	447

9.7.3.5	CLI-RSSI measurement period	447

9.7.4	Scheduling availability of UE during CLI measurements	447

9.7.4.1	Scheduling availability of UE performing measurement on FR1	447

9.7.4.2	Scheduling availability of UE performing measurement on FR2	447

9.8	L1-SINR measurements for Reporting	448

9.8.1	Introduction	448

9.8.2	Requirements applicability	448

9.8.3	Measurement Reporting Requirements	449

9.8.3.1	Periodic Reporting	449

9.8.3.2	Semi-Persistent Reporting	449

9.8.4	L1-SINR measurement requirements	450

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	450

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	454

9.8.5	Measurement restriction for L1-SINR measurement	456

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	456

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	457

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	457

9.8.6	Scheduling availability of UE during L1-SINR measurement	458

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	458

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	458

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	459

9.9	 NR measurements for positioning	459

9.9.1	Introduction	459

9.9.1.1	General Aspects of Gap-based Measurement	459

9.9.1.2	General Aspects of Gapless Measurement	460

9.9.2	RSTD measurements	461

9.9.2.1	Introduction	461

9.9.2.2	Requirements Applicability	461

9.9.2.3	Measurement Capability	461

9.9.2.4	Measurement Reporting Requirements	461

9.9.2.4.1	Void	461

9.9.2.4.2	Void	461

9.9.2.4.3	Void	461

9.9.2.5	Measurements Period Requirements	461

9.9.2.6	Void	464

9.9.2.7	Measurements Period Requirements without Measurement Gaps	464

9.9.2.8	Scheduling Availability of UE during RSTD Measurement	465

9.9.3	PRS-RSRP measurements	466

9.9.3.1	Introduction	466

9.9.3.2	Requirements applicability	466

9.9.3.3	Measurement Capability	466

9.9.3.4	Measurement Reporting Requirements	466

9.9.3.5	Measurement Period Requirements	466

9.9.3.6	Measurement Period Requirements without Measurement Gaps	469

9.9.3.7	Scheduling Availability of UE during PRS-RSRP Measurement	470

9.9.4	UE Rx-Tx time difference measurements	471

9.9.4.1 Introduction	471

9.9.4.2 Requirements Applicability	471

9.9.4.3 Measurement Capability	471

9.9.4.4 Measurement Reporting Requirements	471

9.9.4.5	Measurement Period Requirements	471

9.9.4.6	Measurement Period Requirements without Measurement Gaps	474

9.9.4.7	Scheduling Availability of UE during UE Rx-Tx Time Difference Measurement	476

9.9.5	E-CID measurements	476

9.9.5.1	Introduction	476

9.9.5.2	Measurement Requirements	477

9.9.5.2.1	Intra-frequency Measurement Requirements	477

9.9.5.2.2	Inter-frequency Measurement Requirements	477

9.9.5.2.3	Measurement Reporting Delay	477

9.9.6	PRS-RSRPP measurements	478

9.9.6.1	Introduction	478

9.9.6.2	Requirements applicability	478

9.9.6.3	Measurement capability	478

9.9.564	Measurement reporting requirements	478

9.9.6.5	Measurement period requirements	478

9.9.6.6	Measurement Period Requirements without Measurement Gaps	478

9.9.6.7	Scheduling Availability of UE during PRS-RSRPP Measurement	480

9.10	CSI-RS based L3 measurements	481

9.10.1	Introduction	481

9.10.2	CSI-RS based intra-frequency measurements	481

9.10.2.1	Introduction	481

9.10.2.2	Requirements applicability	481

9.10.2.3	Number of cells and number of CSI-RS	482

9.10.2.3.1	Requirements for FR1	482

9.10.2.3.2	Requirements for FR2	483

9.10.2.4	Measurement Reporting Requirements	483

9.10.2.4.1	Periodic Reporting	483

9.10.2.4.2	Event-triggered Periodic Reporting	483

9.10.2.4.3	Event Triggered Reporting	483

9.10.2.5	Intra-frequency measurements without measurement gaps	484

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	485

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	485

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	486

9.10.3	CSI-RS based Inter-frequency measurements	486

9.10.3.1	Introduction	486

9.10.3.2	Requirements applicability	486

9.10.3.3	Number of cells and number of CSI-RS resources	487

9.10.3.3.1	Requirements for FR1	487

9.10.3.3.2	Requirements for FR2	487

9.10.3.4	Measurements reporting requirements	487

9.10.3.4.1	Periodic Reporting	487

9.10.3.4.2	Event-triggered Periodic Reporting	487

9.10.3.4.3	Event-triggered Reporting	488

9.10.3.5	Inter frequency measurements with measurement gaps	488

9.11	NR measurements with autonomous gaps	489

9.11.1	Introduction	489

9.11.2	CGI identification of an NR cell with autonomous gaps	490

9.11.3	CGI reporting delay	491

9.12	Measurement for Propagation Delay Compensation	491

9.12.1	Introduction	491

9.12.2	Requirements Applicability	491

9.12.3	Measurement Capability	491

9.12.4	Measurement period requirements	491

9.12.4.1 PRS Measurement Period	491

9.12.4.2 TRS Measurement Period	492

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	492

9.13.1	Introduction	492

9.13.2	Requirements Applicability	493

9.13.3	Measurement Reporting Requirements	493

9.13.3.1	Periodic Reporting	494

9.13.3.2	Semi-Persistent Reporting	494

9.13.3.3	Aperiodic Reporting	494

9.13.4	L1-RSRP measurement requirements	494

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	494

9.13.5	Measurement restriction for L1-RSRP measurement	496

9.13.5.1	Measurement restriction for SSB based L1-RSRP	496

9.13.6	Scheduling availability of UE during L1-RSRP measurement	497

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	497

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	497

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	498

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	498

10	Measurement Performance requirements	499

10.1	NR measurements	499

10.1.1	Introduction	499

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	499

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	499

10.1.2.1.1	Absolute SS-RSRP Accuracy	499

10.1.2.1.2	Relative SS-RSRP Accuracy	500

10.1.2.2	Void	501

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	501

10.1.2.3.1	Absolute CSI-RSRP Accuracy	501

10.1.2.3.2	Relative CSI-RSRP Accuracy	502

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	503

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	503

10.1.2B.1.1	Absolute SS-RSRP Accuracy	503

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	504

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	504

10.1.3.1.1	Absolute SS-RSRP Accuracy	504

10.1.3.1.2	Relative SS-RSRP Accuracy	505

10.1.3.2	Void	506

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	506

10.1.3.3.1	Absolute CSI-RSRP Accuracy	506

10.1.3.3.2	Relative CSI-RSRP Accuracy	506

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	507

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	507

10.1.3B.1.1	Absolute SS-RSRP Accuracy	507

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	508

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	508

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	508

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	509

10.1.4.2	Void	510

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	510

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	510

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	511

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	512

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	512

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	512

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	513

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	513

10.1.5.1.1	Absolute SS-RSRP Accuracy	513

10.1.5.1.2	Relative SS-RSRP Accuracy	514

10.1.5.2	Void	515

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	515

10.1.5.3.1	Absolute CSI-RSRP Accuracy	515

10.1.5.3.2	Relative CSI-RSRP Accuracy	515

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	516

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	516

10.1.5B.1.1	Absolute SS-RSRP Accuracy	516

10.1.6 RSRP Measurement Report Mapping	517

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	519

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	519

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	519

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	520

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	520

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	521

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	521

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	521

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	522

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	522

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	522

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	523

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	523

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	524

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	524

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	524

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	525

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	525

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	525

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	526

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	527

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	527

10.1.9.2.2	Relative CSI-RSRQ Accuracy	528

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	529

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	529

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	529

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	530

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	532

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	532

10.1.10.2.2	Relative CSI-RSRQ Accuracy	532

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	533

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	533

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	533

10.1.11	RSRQ report mapping	534

10.1.12	Intra-frequency SINR accuracy requirements for FR1	534

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	535

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	535

10.1.13	Intra-frequency SINR accuracy requirements for FR2	536

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	537

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	537

10.1.14	Inter-frequency SINR accuracy requirements for FR1	538

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	540

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	540

10.1.15	Inter-frequency SINR accuracy requirements for FR2	542

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	544

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	544

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	544

10.1.16	SINR report mapping	545

10.1.17	Power Headroom	546

10.1.18	PCMAX,c,f	547

10.1.19	L1-RSRP accuracy requirements for FR1	547

10.1.20	L1-RSRP accuracy requirements for FR2	551

10.1.21	SFTD accuracy requirements	554

10.1.22	CLI measurement accuracy requirements	558

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	562

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	562

10.1.23	RSTD Measurements	564

10.1.23.1	Introduction	564

10.1.23.2	Measurement Accuracy Requirements	565

10.1.23.3	Report mapping	569

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	569

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	571

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	573

10.1.24	PRS-RSRP Measurements	575

10.1.24.1	Introduction	575

10.1.24.2	Measurement Accuracy Requirements	575

10.1.24.2.1 Absolute PRS RSRP accuracy	575

10.1.24.2.2 Relative PRS RSRP accuracy	577

10.1.24.3	Report mapping	579

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	579

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	580

10.1.25	UE Rx-Tx Time Difference Measurements	582

10.1.25.1	Introduction	582

10.1.25.2	Measurement Accuracy Requirements	583

10.1.25.3	Report mapping	590

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	590

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	592

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	594

10.1.26	FR2 P-MPR report	596

10.1.26.1	Report mapping	596

10.1.27	L1-SINR accuracy requirements for FR1	596

10.1.28	L1-SINR accuracy requirements for FR2	604

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	611

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	611

10.1.29.1.1	Absolute SS-RSRQ Accuracy	611

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	611

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	611

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	611

10.1.30.1.2	Relative Accuracy of SS-RSRQ	612

10.1.31	Intra-frequency SINR accuracy requirements under CCA	613

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	613

10.1.31.1.1	Absolute SS-SINR Accuracy	613

10.1.32	Inter-frequency SINR accuracy requirements under CCA	613

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	613

10.1.32.1.1	Aboslute Accuracy of SS-SINR	613

10.1.32.1.2	Relative Accuracy of SS-SINR	614

10.1.33	L1-RSRP accuracy requirements under CCA	615

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	615

10.1.33.1.1	Absolute Accuracy	615

10.1.33.1.2	Relative Accuracy	615

10.1.34	RSSI measurements under CCA	616

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	616

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	616

10.1.34.3	RSSI measurement report mapping	616

10.1.35	Channel occupancy measurements under CCA	617

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	617

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	617

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	617

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	617

10.1.36.1.1	Absolute SS-RSRP Accuracy	617

10.1.36.1.2	Relative SS-RSRP Accuracy	618

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	619

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	619

10.1.37.1.1	Absolute Accuracy of SS-RSRP	619

10.1.37.1.2	Relative Accuracy of SS-RSRP	619

10.2	E-UTRAN measurements	620

10.2.1	Introduction	620

10.2.2	E-UTRAN RSRP measurements	620

10.2.3	E-UTRAN RSRQ measurements	620

10.2.4	E-UTRAN RSTD measurements	621

10.2.5	E-UTRAN RS-SINR measurements	621

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	621

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	622

10.3	UTRAN FDD Measurements	622

10.3.1	UTRAN FDD CPICH RSCP	622

10.3.2	UTRAN FDD CPICH Ec/No	623

10.4	V2X measurements	623

10.4.1	Introduction	623

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	623

10.4.2.1	PSBCH-RSRP Absolute Accuracy	623

10.4.2.2	PSBCH-RSRP Relative Accuracy	624

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	625

10.4.3.1	Absolute SL-RSSI Accuracy	625

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	625

10.4.4.1	Absolute L1 SL-RSRP Accuracy	625

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	626

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	626

11	Void	627

12	V2X Requirements	627

12.1	Introduction	627

12.2	UE Transmit Timing	627

12.2.1	Introduction	627

12.2.2	GNSS as synchronization reference source	628

12.2.3	NR Cell as synchronization reference source	628

12.2.4	E-URTAN Cell as synchronization reference source	628

12.2.5	SyncRef UE as synchronization reference source	629

12.3	Initiation/Cease of SLSS Transmissions	629

12.3.1	Introduction	629

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	629

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	630

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	631

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	631

12.4	Selection / Reselection of V2X Synchronization Reference Source	632

12.5	L1 SL-RSRP measurements	633

12.5.1	Introduction	633

12.5.2	SL-RSRP measurements	634

12.6	Congestion Control measurements	634

12.7	Interruption	634

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	634

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	635

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	636

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	637

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	637

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	638

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	638

12.7.8	Interruptions at NR sidelink discovery configuration	638

12.8	Reliability of GNSS signal	638

12.9	Scheduling availability	639

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	639

12.9.2	Scheduling availability of UE switching between uplink transmission and V2X sidelink transmission	639

12.10	Selection / Reselection of relay UE	639

12.10.1	Introduction	639

12.10.2	Selection / Reselection of relay UE	640

13	Measurement Performance Requirements for NR gNB	640

13.1	UL-RTOA	640

13.1.1	Report mapping	640

13.2	gNB Rx-Tx time difference	642

13.2.1	Report mapping	642

13.2.2	Measurement Accuracy Requirements	644

13.2.2.1	Introduction	644

13.3	UL SRS RSRP measurement	646

13.3.1	Report mapping	646

13.3.2	Measurement accuracy requirements	646

13.3.2.1	Introduction	646

13.3.2.2	Requirements	647

13.4	AoA/ZoA	647

13.4.1	Report mapping	647

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.2.1.5	Requirements under CCA	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1.2	CORESET for RMSI scheduling	429

A.3.1.2.1	FDD	429

A.3.1.2.2	TDD	430

A.3.1.3	CORESET for RMC scheduling	433

A.3.1.3.1	FDD	433

A.3.1.3.2	TDD	434

A.3.1.4	TDD UL/DL configuration	437

A.3.1A	Reference measurement channels under CCA	439

A.3.1A.1	PDSCH	439

A.3.1A.1.1	TDD	439

A.3.1A.2	CORESET for RMSI scheduling	440

A.3.1A.2.1	TDD	440

A.3.1A.3	CORESET for RMC scheduling	441

A.3.1A.3.1	TDD	441

A.3.1A.4	TDD UL/DL configuration	442

A.3.1A.5	RMC burst transmission model	442

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	442

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	443

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	443

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	444

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	444

A.3.2.2	Void	445

A.3.3	Reference DRX configurations	445

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	445

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	446

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	446

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	446

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	447

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	447

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	447

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	448

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	448

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	448

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	449

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	449

A.3.4	Test Cases with Different Channel Bandwidths	449

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	449

A.3.4.1.1	Introduction	449

A.3.4.1.2	Principle of testing	449

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	450

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	450

A.3.5.1.1	Introduction	450

A.3.5.1.2	Principle of Testing	450

A.3.6	Antenna configurations	450

A.3.6.1	Antenna configurations for FR1	450

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	450

A.3.6.1.1.1	Introduction	450

A.3.6.1.1.2	Principle of testing	450

A.3.6.2	Antenna configurations for FR2	453

A.3.6A	Antenna configurations with unlicensed bands	453

A.3.6A.1	Antenna configurations for FR1	453

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	453

A.3.6A.1.1.1	Introduction	453

A.3.6A.1.1.2	Principle of testing	453

A.3.7	EN-DC test setup	455

A.3.7.1	Introduction	455

A.3.7.2	E-UTRAN Serving Cell Parameters	455

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	455

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	456

A.3.7A	NR FR1-FR2 test setup	457

A.3.7B	EN-DC test setup with unlicensed bands	458

A.3.7B.1	Introduction	458

A.3.7B.2	E-UTRAN Serving Cell Parameters	458

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	458

A.3.7C	LTE-FR1/FR2 test setup	459

A.3.7D	NE-DC test setup	459

A.3.7D.1	Introduction	459

A.3.7D.2	E-UTRAN Serving Cell Parameters	459

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	459

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	459

A.3.8	PRACH configurations	459

A.3.8.1	Introduction	459

A.3.8.2	PRACH configurations in FR1	460

A.3.8.2.1	FR1 PRACH configuration 1	460

A.3.8.2.2	FR1 PRACH configuration 2	460

A.3.8.2.3	FR1 PRACH configuration 3	461

A.3.8.2.4	FR1 PRACH configuration 4	462

A.3.8.3	PRACH configurations in FR2	463

A.3.8.3.1	FR2 PRACH configuration 1	463

A.3.8.3.2	FR2 PRACH configuration 2	464

A.3.8.3.3	FR2 PRACH configuration 3	465

A.3.8.3.4	FR2 PRACH configuration 4	466

A.3.8A	PRACH configurations under CCA	467

A.3.8A.1	Introduction	467

A.3.8A.2	PRACH configurations in FR1	467

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	467

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	468

A.3.9	BWP configurations	469

A.3.9.1	Introduction	469

A.3.9.2	Downlink BWP configurations	470

A.3.9.2.1	Initial BWP	470

A.3.9.2.2	Dedicated BWP	470

A.3.9.3	Uplink BWP configurations	470

A.3.9.3.1	Initial BWP	470

A.3.9.3.2	Dedicated BWP	471

A.3.10	SSB Configurations	471

A.3.10.1	SSB Configurations for FR1	471

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	471

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	473

A.3.10A	SSB Configurations under CCA	478

A.3.10A.1	SSB Configurations under CCA for FR1	478

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	478

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	479

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	479

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	480

A.3.11	SMTC Configurations	480

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	480

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	481

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	481

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	481

A.3.12	Test Cases with Different CC Configurations	481

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	481

A.3.12.1.1	Introduction	481

A.3.12.1.2	Principle of testing	481

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	482

A.3.12.2.1	Introduction	482

A.3.12.2.2	Principle of testing	482

A.3.13	Test Cases in SA and EN-DC Operations	482

A.3.13.1	Introduction	482

A.3.13.2	Principle of Testing	482

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	482

A.3.13A.1	Introduction	482

A.3.13A.2	Principle of Testing in EN-DC	482

A.3.13A.3	Principle of Testing in SA	483

A.3.13A.4	Principle of Testing in E-UTRA	483

A.3.13B	Test Cases for EN-DC and NE-DC Operations	484

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	484

A.3.13B.1.1	Introduction	484

A.3.13B.1.2	Principle of Testing	484

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	484

A.3.13B.2.1	Introduction	484

A.3.13B.2.2	Principle of Testing	484

A.3.14	CSI-RS configurations	484

A.3.14.1	FDD	484

A.3.14.2	TDD	487

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	493

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	493

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	493

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	493

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	494

A.3.15.3	Setup 3: 2 AoAs	494

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	494

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	494

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	494

A.3.16	TCI State Configuration	495

A.3.16.1	Introduction	495

A.3.16.2	TCI states	495

A.3.17	Configurations of CSI-RS for tracking	495

A.3.17.1	Configuration of CSI-RS for tracking for FR1	495

A.3.17.1.1	FDD	495

A.3.17.1.2	TDD	496

A.3.17.2	Configuration of CSI-RS for tracking for FR2	497

A.3.17.2.1	TDD	497

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	498

A.3.18.1	Introduction	498

A.3.18.2	PRACH Power Measurement	498

A.3.19	Test applicability for DAPS handover	498

A.3.19.1	Introduction	498

A.3.19.2	Principle of testing	498

A.3.20	MsgA configurations	499

A.3.20.1	Introduction	499

A.3.20.2	MsgA configurations in FR1	499

A.3.20.2.1	FR1 MsgA configuration 1	499

A.3.20.2.2	FR1 MsgA configuration 2	500

A.3.20.3	MsgA configurations in FR2	502

A.3.20.3.1	FR2 MsgA configuration 1	502

A.3.20.3.2	FR2 MsgA configuration 2	503

A.3.20A	MsgA configurations under CCA	505

A.3.20A.1	Introduction	505

A.3.20A.2	MsgA configurations in FR1	505

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	505

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	506

A.3.21	V2X sidelink communication	509

A.3.21.1	Introduction	509

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	509

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	513

A.3.22	CSI-IM configurations	513

A.3.22.1	FDD	513

A.3.22.2	TDD	514

A.3.23	Spatial Relation Configuration	516

A.3.23.1	Introduction	516

A.3.23.2	Spatial Relation	516

A.3.24	SRS configuration	517

A.3.25	Channel bandwidth (CBW) configurations	519

A.3.25.1	DL UE specific CBW	519

A.3.25.2	UL UE specific CBW	520

A.3.26	CCA model	520

A.3.26.1	Introduction	520

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	520

A.3.26.2.1	DL CCA model	520

A.3.26.2.2	UL CCA model	521

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	522

A.3.27.1	Introduction	522

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	522

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	522

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	522

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	522

A.3.28	Discovery Burst Transmission Window configuration under CCA	523

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	523

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	523

A.3.29.1	Introduction	523

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	523

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	523

A.3.30	CSI-RS configurations for RRM	524

A.3.30.1	FDD	524

A.3.30.2	TDD	525

A.3.31	PRS Configurations	527

A.3.31.1.	PRS Configurations for FR1	527

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	527

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	528

A.3.31.2.	PRS Configurations for FR2	528

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	528

A.4	EN-DC tests with all NR cells in FR1	529

A.4.1	Void	529

A.4.2	Void	529

A.4.3	RRC_CONNECTED state mobility	529

A.4.3.1	Void	529

A.4.3.2	RRC Connection Mobility Control	529

A.4.3.2.1	Void	529

A.4.3.2.2	Random Access	529

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	529

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	533

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	537

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	541

A.4.3.2.3	Void	545

A.4.4	Timing	545

A.4.4.1	UE transmit timing	545

A.4.4.1.1	NR UE Transmit Timing Test for FR1	545

A.4.4.1.1.1	Test Purpose and environment	545

A.4.4.1.1.2	Test requirements	549

A.4.4.2	UE timer accuracy	549

A.4.4.3	Timing advance	549

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	549

A.4.4.3.1.1	Test Purpose and Environment	549

A.4.4.3.1.2	Test Parameters	549

A.4.4.3.1.3	Test Requirements	553

A.4.5	Signaling characteristics	553

A.4.5.1	Radio link Monitoring	553

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	554

A.4.5.1.1.1	Test Purpose and Environment	554

A.4.5.1.1.2	Test Requirements	558

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	558

A.4.5.1.2.1	Test Purpose and Environment	558

A.4.5.1.2.2	Test Requirements	564

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	564

A.4.5.1.3.1	Test Purpose and Environment	564

A.4.5.1.3.2	Test Requirements	569

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	569

A.4.5.1.4.1	Test Purpose and Environment	569

A.4.5.1.4.2	Test Requirements	575

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	575

A.4.5.1.5.1	Test Purpose and Environment	575

A.4.5.1.5.2	Test Requirements	580

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	580

A.4.5.1.6.1	Test Purpose and Environment	580

A.4.5.1.6.2	Test Requirements	586

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	586

A.4.5.1.7.1	Test Purpose and Environment	586

A.4.5.1.7.2	Test Requirements	591

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	592

A.4.5.1.8.1	Test Purpose and Environment	592

A.4.5.1.8.2	Test Requirements	597

A.4.5.2	Interruption	597

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	597

A.4.5.2.1.1	Test Purpose and Environment	597

A.4.5.2.1.2	Test Requirements	601

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	601

A.4.5.2.2.1	Test Purpose and Environment	601

A.4.5.2.2.2	Test Requirements	605

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	605

A.4.5.2.3.1	Test Purpose and Environment	605

A.4.5.2.3.2	Test Requirements	609

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	610

A.4.5.2.4.1	Test Purpose and Environment	610

A.4.5.2.4.2	Test Requirements	613

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	614

A.4.5.2.5.1	Test Purpose and Environment	614

A.4.5.2.5.2	Test Requirements	617

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	617

A.4.5.2.6.1	Test Purpose and Environment	617

A.4.5.2.6.2	Test Requirements	621

A.4.5.2.7	Void	621

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	621

A.4.5.2.8.1	Test Purpose and Environment	621

A.4.5.2.8.2	Test Requirements	624

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	625

A.4.5.2.9.1	Test Purpose and Environment	625

A.4.5.2.9.2	Test Requirements	628

A.4.5.3	SCell Activation and Deactivation Delay	629

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	629

A.4.5.3.1.1	Test Purpose and Environment	629

A.4.5.3.1.2	Test Requirements	634

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640 ms SCell measurement cycle	635

A.4.5.3.2.1	Test Purpose and Environment	635

A.4.5.3.2.2	Test Requirements	635

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	635

A.4.5.3.3.1	Test Purpose and Environment	635

A.4.5.3.3.2	Test Requirements	636

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	636

A.4.5.3.4.1	Test Purpose and Environment	636

A.4.5.3.3.2	Test Requirements	639

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	639

A.4.5.3.5.1	Test Purpose and Environment	639

A.4.5.3.5.2	Test Requirements	644

A.4.5.4	UE UL carrier RRC reconfiguration Delay	644

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	644

A.4.5.4.1.1 Test Purpose and Environment	644

A.4.5.4.1.2	Test Requirements	654

A.4.5.5	Beam Failure Detection and Link recovery procedures	654

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	654

A.4.5.5.1.1	Test Purpose and Environment	654

A.4.5.5.1.2	Test Requirements	660

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	660

A.4.5.5.2.1	Test Purpose and Environment	660

A.4.5.5.2.2	Test Requirements	666

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	666

A.4.5.5.3.1	Test Purpose and Environment	666

A.4.5.5.3.2	Test Requirements	672

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	672

A.4.5.5.4.1	Test Purpose and Environment	672

A.4.5.5.4.2	Test Requirements	678

A.4.5.6	Active BWP switch	692

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	692

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	692

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	697

A.4.5.6.2	RRC-based Active BWP Switch	702

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	706

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	706

A.4.5.6.3.1.1	Test Purpose and Environment	706

A.4.5.6.3.1.2	Test Requirements	711

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	712

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	712

A.4.5.6.4.1.1	Test Purpose and Environment	712

A.4.5.6.4.1.2	Test Requirements	716

A.4.5.6.5	SCell dormancy switch	716

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	716

A.4.5.6.5.1.1	Test Purpose and Environment	716

A.4.5.6.5.1.2	Test Requirements	722

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	722

A.4.5.6.5.2.1	Test Purpose and Environment	722

A.4.5.6.5.2.2	Test Requirements	727

A.4.5.7	PSCell addition and release delay	727

A.4.5.7.1	Addition and Release Delay of known NR PSCell	727

A.4.5.7.1.1	Test purpose and environment	727

A.4.5.7.1.2	Test Requirements	731

A.4.5.8	DL Interruptions at switching between two uplink carriers	732

A.4.5.8.1	Test Purpose and Environment	732

A.4.5.8.2	Test Requirements	735

A.4.6	Measurement procedure	735

A.4.6.1	Intra-frequency Measurements	735

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	735

A.4.6.1.1.1	Test purpose and Environment	735

A.4.6.1.1.2	Test parameters	736

A.4.6.1.1.3	Test Requirements	739

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	739

A.4.6.1.2.1	Test purpose and Environment	739

A.4.6.1.2.2	Test parameters	739

A.4.6.1.2.2	Test Requirements	743

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	743

A.4.6.1.3.1	Test purpose and Environment	743

A.4.6.1.3.2	Test parameters	743

A.4.6.1.3.3	Test Requirements	747

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	747

A.4.6.1.4.1	Test purpose and Environment	747

A.4.6.1.4.2	Test parameters	747

A.4.6.1.4.3	Test Requirements	751

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	751

A.4.6.1.5.1	Test purpose and Environment	751

A.4.6.1.5.2	Test parameters	751

A.4.6.1.5.3	Test Requirements	753

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	754

A.4.6.1.6.1	Test purpose and Environment	754

A.4.6.1.6.2	Test parameters	754

A.4.6.1.6.3	Test Requirements	756

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	757

A.4.6.1.7.1	Test purpose and Environment	757

A.4.6.1.7.2	Test parameters	757

A.4.6.1.7.3	Test Requirements	761

A.4.6.2	Inter-frequency Measurements	761

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	761

A.4.6.2.1.1	Test Purpose and Environment	761

A.4.6.2.1.2	Test Requirements	765

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	765

A.4.6.2.2.1	Test Purpose and Environment	765

A.4.6.2.2.2	Test Requirements	769

A.4.6.2.3	Void	770

A.4.6.2.4	Void	770

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	770

A.4.6.2.5.1	Test Purpose and Environment	770

A.4.6.2.5.2	Test Requirements	774

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	774

A.4.6.2.6.1	Test Purpose and Environment	774

A.4.6.2.6.2	Test Requirements	779

A.4.6.2.7	Void	780

A.4.6.2.8	Void	780

A.4.6.3	Void	780

A.4.6.4	L1-RSRP measurement for beam reporting	780

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	780

A.4.6.4.1.1	Test Purpose and Environment	780

A.4.6.4.1.2	Test parameters	780

A.4.6.4.1.3	Test Requirements	783

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	783

A.4.6.4.2.1	Test Purpose and Environment	783

A.4.6.4.2.2	Test parameters	784

A.4.6.4.2.3	Test Requirements	787

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	787

A.4.6.4.3.1	Test Purpose and Environment	787

A.4.6.4.3.2	Test parameters	788

A.4.6.4.3.3	Test Requirements	791

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	791

A.4.6.4.4.1	Test Purpose and Environment	791

A.4.6.4.4.2	Test parameters	792

A.4.6.4.4.3	Test Requirements	794

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	794

A.4.6.4.5.1	Test Purpose and Environment	794

A.4.6.4.5.2	Test parameters	795

A.4.6.4.5.3	Test Requirements	798

A.4.6.5	CLI measurements	798

A.4.6.5.1	SRS-RSRP measurement with non-DRX	798

A.4.6.5.1.1	Test Purpose and Environment	798

A.4.6.5.1.2	Test Parameters	799

A.4.6.5.1.3	Test Requirements	802

A.4.6.5.2	CLI-RSSI measurement with non-DRX	802

A.4.6.5.2.1	Test Purpose and Environment	802

A.4.6.5.2.2	Test Parameters	803

A.4.6.5.2.3	Test Requirements	804

A.4.6.6.1.2	Test Requirements	809

A.4.6.7	L1-SINR measurement for beam reporting	809

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	812

A.4.6.7.2.1	Test Purpose and Environment	812

A.4.6.7.2.2	Test parameters	812

A.4.6.7.2.3	Test Requirements	815

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	815

A.4.6.7.3.1	Test Purpose and Environment	815

A.4.6.7.3.2	Test parameters	815

A.4.6.7.3.3	Test Requirements	817

A.4.6.8	CSI-RS based intra-frequency Measurement	818

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	818

A.4.6.8.1.1	Test purpose and Environment	818

A.4.6.8.1.2	Test Requirements	822

A.4.6.9	CSI-RS based inter-frequency Measurement	822

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	822

A.4.6.9.1.1	Test Purpose and Environment	822

A.4.6.9.1.2	Test Requirements	826

A.4.7	Measurement Performance requirements	827

A.4.7.1	SS-RSRP	827

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	827

A.4.7.1.1.1	Test Purpose and Environment	827

A.4.7.1.1.2	Test parameters	827

A.4.7.1.1.3	Test Requirements	831

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	831

A.4.7.1.2.1	Test Purpose and Environment	831

A.4.7.1.2.2	Test parameters	832

A.4.7.1.2.3	Test Requirements	836

A.4.7.1.3	Void	836

A.4.7.2	SS-RSRQ	836

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	836

A.4.7.2.1.1	Test Purpose and Environment	836

A.4.7.2.1.2	Test Parameters	837

A.4.7.2.1.3	Test Requirements	842

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	842

A.4.7.2.2.1	Test Purpose and Environment	842

A.4.7.2.2.2	Test Parameters	842

A.4.7.2.2.3	Test Requirements	847

A.4.7.3	SS-SINR	847

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	847

A.4.7.3.1.1	Test Purpose and Environment	847

A.4.7.3.1.2	Test Parameters	847

A.4.7.3.1.3	Test Requirements	851

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	851

A.4.7.3.2.1	Test Purpose and Environment	851

A.4.7.3.2.2	Test Parameters	851

A.4.7.3.2.3	Test Requirements	856

A.4.7.4	L1-RSRP measurement for beam reporting	856

A.4.7.4.1	SSB based L1-RSRP measurement	856

A.4.7.4.1.1	Test Purpose and Environment	856

A.4.7.4.1.2	Test parameters	857

A.4.7.4.1.3	Test Requirements	860

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	860

A.4.7.4.2.1	Test Purpose and Environment	860

A.4.7.4.2.2	Test parameters	861

A.4.7.4.2.3	Test Requirements	864

A.4.7.5	SFTD accuracy	864

A.4.7.5.1	SFTD accuracy	864

A.4.7.5.1.1	Test Purpose and Environment	864

A.4.7.5.1.2	Test Parameters	864

A.4.7.5.1.3	Test Requirements	869

A.4.7.5.2	Void	869

A.4.7.5.3	Void	869

A.4.7.6	CLI measurements	869

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	869

A.4.7.6.1.1	Test Purpose and Environment	869

A.4.7.6.1.2	Test parameters	870

A.4.7.6.1.3	Test Requirements	875

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	875

A.4.7.6.2.1	Test Purpose and Environment	875

A.4.7.6.2.2	Test parameters	876

A.4.7.6.2.3	Test Requirements	879

A.4.7.7	L1-SINR measurement for beam reporting	879

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	883

A.4.7.7.2.1	Test Purpose and Environment	883

A.4.7.7.2.2	Test parameters	883

A.4.7.7.2.3	Test Requirements	887

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	887

A.4.7.7.3.1	Test Purpose and Environment	887

A.4.7.7.3.2	Test parameters	887

A.4.7.7.3.3	Test Requirements	890

A.4.7.8	CSI-RSRP	890

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	890

A.4.7.8.1.1	Test Purpose and Environment	890

A.4.7.8.1.2	Test parameters	891

A.4.7.8.1.3	Test Requirements	896

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	896

A.4.7.8.2.1	Test Purpose and Environment	896

A.4.7.8.2.2	Test parameters	896

A.4.7.8.2.3	Test Requirements	901

A.4.7.9	CSI-RSRQ	901

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	901

A.4.7.9.1.1	Test Purpose and Environment	901

A.4.7.9.1.2	Test Parameters	901

A.4.7.9.1.3	Test Requirements	906

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	906

A.4.7.9.2.1	Test Purpose and Environment	906

A.4.7.9.2.2	Test Parameters	906

A.4.7.9.2.3	Test Requirements	911

A.4.7.10	CSI-SINR	911

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	911

A.4.7.10.1.1	Test Purpose and Environment	911

A.4.7.10.1.2	Test Parameters	912

A.4.7.10.1.3	Test Requirements	916

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	916

A.4.7.10.2.1	Test Purpose and Environment	916

A.4.7.10.2.2	Test Parameters	917

A.4.7.10.2.3	Test Requirements	921

A.4.8	Void	921

A.4A	NE-DC test with all NR cells in FR1	922

A.4A.1	Signaling characteristics	922

A.4A.1.1	E-UTRAN PSCell addition	922

A.4A.1.1.1	Test purpose and environment	922

A.4A.1.1.2	Test Requirements	926

A.4A.1.2	Active BWP switch	927

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	927

A.4A.1.2.1.1	Test Purpose and Environment	927

A.4A.1.2.1.2	Test Requirements	931

A.4A.2	Measurement performance	932

A.4A.2.1	SFTD accuracy	932

A.4A.2.1.1	SFTD accuracy	932

A.4A.2.1.1.1	Test Purpose	932

A.4A.2.1.1.2	Test Environment	932

A.4A.2.1.1.3	Test Requirements	935

A.5	EN-DC tests with one or more NR cells in FR2	936

A.5.1	Void	936

A.5.2	Void	936

A.5.3	RRC_CONNECTED state mobility	936

A.5.3.1	Void	936

A.5.3.2	RRC Connection Mobility Control	936

A.5.3.2.1	Void	936

A.5.3.2.2	Random Access	936

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	936

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	939

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	945

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	948

A.5.3.2.3	Void	952

A.5.4	Timing	952

A.5.4.1	UE transmit timing	952

A.5.4.1.1	NR UE Transmit Timing Test for FR2	952

A.5.4.1.1.1	Test Purpose and environment	952

A.5.4.1.1.2	Test requirements	955

A.5.4.2	UE timer accuracy	956

A.5.4.3	Timing advance	956

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	956

A.5.4.3.1.1 Test Purpose and Environment	956

A.5.4.3.1.2 Test Parameters	956

A.5.4.3.1.3	Test Requirements	960

A.5.5	Signaling characteristics	960

A.5.5.1	Radio link Monitoring	960

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	960

A.5.5.1.1.1	Test Purpose and Environment	960

A.5.5.1.1.2	Test Requirements	964

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	965

A.5.5.1.2.1	Test Purpose and Environment	965

A.5.5.1.2.2	Test Requirements	968

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	969

A.5.5.1.3.1	Test Purpose and Environment	969

A.5.5.1.3.2	Test Requirements	973

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	973

A.5.5.1.4.1	Test Purpose and Environment	973

A.5.5.1.4.2	Test Requirements	977

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	977

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	981

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	985

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	990

A.5.5.1.8.2	Test Requirements	994

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	995

A.5.5.1.9.1	Test Purpose and Environment	995

A.5.5.1.9.2	Test Requirements	997

A.5.5.2	Interruption	997

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	997

A.5.5.2.1.1	Test Purpose and Environment	997

A.5.5.2.1.2	Test Requirements	1000

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1000

A.5.5.2.2.1	Test Purpose and Environment	1000

A.5.5.2.2.2	Test Requirements	1003

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1003

A.5.5.2.3.1	Test Purpose and Environment	1003

A.5.5.2.3.2	Test Requirements	1007

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1007

A.5.5.2.4.1	Test Purpose and Environment	1007

A.5.5.2.4.2	Test Requirements	1010

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1011

A.5.5.2.5.1	Test Purpose and Environment	1011

A.5.5.2.5.2	Test Requirements	1014

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1014

A.5.5.2.6.1	Test Purpose and Environment	1014

A.5.5.2.6.2	Test Requirements	1017

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1017

A.5.5.2.7.1	Test Purpose and Environment	1017

A.5.5.2.7.2	Test Requirements	1020

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1021

A.5.5.2.8.1 Test Purpose and Environment	1021

A.5.5.2.8.3	Test Requirements	1023

A.5.5.3	SCell Activation and Deactivation Delay	1023

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1023

A.5.5.3.1.1	Test Purpose and Environment	1023

A.5.5.3.1.2	Test Requirements	1025

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1025

A.5.5.3.2.1	Test Purpose and Environment	1025

A.5.5.3.2.2	Test Requirements	1029

A.5.5.3.3	Void	1029

A.5.5.3.4	Void	1029

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1029

A.5.5.3.5.1	Test Purpose and Environment	1029

A.5.5.3.5.2	Test Requirements	1032

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1033

A.5.5.3.6.1	Test Purpose and Environment	1033

A.5.5.3.6.2	Test Requirements	1037

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1038

A.5.5.3.7.1	Test Purpose and Environment	1038

A.5.5.3.7.2	Test Requirements	1041

A.5.5.4	Void	1041

A.5.5.5	Beam Failure Detection and Link recovery procedures	1041

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1041

A.5.5.5.1.1	Test Purpose and Environment	1041

A.5.5.5.1.2	Test Requirements	1045

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1045

A.5.5.5.2.1	Test Purpose and Environment	1045

A.5.5.5.2.2	Test Requirements	1049

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1049

A.5.5.5.3.1	Test Purpose and Environment	1049

A.5.5.5.3.2	Test Requirements	1053

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1053

A.5.5.5.4.1	Test Purpose and Environment	1053

A.5.5.5.4.2	Test Requirements	1057

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1057

A.5.5.5.5.1	Test Purpose and Environment	1057

A.5.5.5.5.2	Test Requirements	1061

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1061

A.5.5.5.6.1	Test Purpose and Environment	1061

A.5.5.5.6.2	Test Requirements	1066

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1066

A.5.5.5.7.1	Test Purpose and Environment	1066

A.5.5.5.7.2	Test Requirements	1072

A.5.5.6	Active BWP switch	1072

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1072

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1072

A.5.5.6.1.1.1	Test Purpose and Environment	1072

A.5.5.6.1.1.2	Test Requirements	1076

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1076

A.5.5.6.2	RRC-based Active BWP Switch	1081

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1081

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1085

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1085

A.5.5.6.3.1.1	Test Purpose and Environment	1085

A.5.5.6.3.1.2	Test Requirements	1089

A.5.5.6.4	SCell dormancy switch	1089

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1089

A.5.5.6.4.1.1	Test Purpose and Environment	1089

A.5.5.6.4.1.2	Test Requirements	1093

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1093

A.5.5.6.4.2.1	Test Purpose and Environment	1093

A.5.5.6.4.2.2	Test Requirements	1100

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1100

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1100

A.5.5.7	PSCell addition and release delay	1103

A.5.5.7.1	Addition and Release Delay of NR PSCell	1103

A.5.5.7.1.1	Test purpose and environment	1103

A.5.5.7.1.2	Test Requirements	1106

A.5.5.8	Active TCI state switch delay	1107

A.5.5.8.1	MAC-CE based active TCI state switch	1107

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1107

A.5.5.8.1.1.1	Test Purpose and Environment	1107

A.5.5.8.1.1.2	Test Requirements	1110

A.5.5.8.2	RRC based active TCI state switch	1110

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1110

A.5.5.8.2.1.1	Test Purpose and Environment	1110

A.5.5.8.2.1.2	Test Requirements	1114

A.5.5.9	Uplink spatial relation switch delay	1114

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1114

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1114

A.5.5.9.1.1.1	Test Purpose and Environment	1114

A.5.5.9.1.1.2	Test Requirements	1117

A.5.5.9.2	RRC based spatial relation switch	1117

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1117

A.5.5.9.2.1.1	Test Purpose and Environment	1117

A.5.5.9.2.1.2	Test Requirements	1120

A.5.5.10	UE specific CBW change	1120

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1120

A.5.5.10.1.1	Test Purpose and Environment	1120

A.5.5.10.1.2	Test Requirements	1123

A.5.6	Measurement procedure	1124

A.5.6.1	Intra-frequency Measurements	1124

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1124

A.5.6.1.1.1	Test purpose and Environment	1124

A.5.6.1.1.2	Test Requirements	1127

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1127

A.5.6.1.2.1	Test purpose and Environment	1127

A.5.6.1.2.2	Test Requirements	1129

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1130

A.5.6.1.3.1	Test purpose and Environment	1130

A.5.6.1.3.2	Test Requirements	1134

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1134

A.5.6.1.4.1	Test purpose and Environment	1134

A.5.6.1.4.2	Test Requirements	1137

A.5.6.2	Inter-frequency Measurements	1138

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1138

A.5.6.2.1.1	Test Purpose and Environment	1138

A.5.6.2.1.2	Test Requirements	1141

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1141

A.5.6.2.2.1	Test Purpose and Environment	1141

A.5.6.2.2.2	Test Requirements	1145

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1145

A.5.6.2.3.1	Test Purpose and Environment	1145

A.5.6.2.3.2	Test Requirements	1149

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1149

A.5.6.2.4.1	Test Purpose and Environment	1149

A.5.6.2.4.2	Test Requirements	1153

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1153

A.5.6.2.5.1	Test Purpose and Environment	1153

A.5.6.2.5.2	Test Requirements	1158

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1158

A.5.6.2.6.1	Test Purpose and Environment	1158

A.5.6.2.6.2	Test Requirements	1162

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1162

A.5.6.2.7.1	Test Purpose and Environment	1162

A.5.6.2.7.2	Test Requirements	1167

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1167

A.5.6.2.8.1	Test Purpose and Environment	1167

A.5.6.2.8.2	Test Requirements	1172

A.5.6.3	L1-RSRP measurement for beam reporting	1173

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1173

A.5.6.3.1.1	Test Purpose and Environment	1173

A.5.6.3.1.2	Test parameters	1173

A.5.6.3.1.3	Test Requirements	1175

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1175

A.5.6.3.2.1	Test Purpose and Environment	1175

A.5.6.3.2.2	Test parameters	1176

A.5.6.3.2.3	Test Requirements	1178

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1178

A.5.6.3.3.1	Test Purpose and Environment	1178

A.5.6.3.3.2	Test parameters	1179

A.5.6.3.3.3	Test Requirements	1181

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1181

A.5.6.3.4.1	Test Purpose and Environment	1181

A.5.6.3.4.2	Test parameters	1182

A.5.6.3.4.3	Test Requirements	1184

A.5.6.4	CLI measurements	1185

A.5.6.4.1	SRS-RSRP measurement with DRX	1185

A.5.6.4.1.1	Test Purpose and Environment	1185

A.5.6.4.1.2	Test Parameters	1185

A.5.6.4.1.3	Test Requirements	1187

A.5.6.4.2	CLI-RSSI measurement with DRX	1187

A.5.6.4.2.1	Test Purpose and Environment	1187

A.5.6.4.2.2	Test Parameters	1188

A.5.6.4.2.3	Test Requirements	1190

A.5.6.5	Measurements with autonomous gaps	1190

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1190

A.5.6.5.1.1	Test Purpose and Environment	1190

A.5.6.5.1.2	Test Requirements	1193

A.5.6.6	L1-SINR measurement for beam reporting	1194

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1196

A.5.6.6.2.1	Test Purpose and Environment	1196

A.5.6.6.2.2	Test parameters	1197

A.5.6.6.2.3	Test Requirements	1200

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1200

A.5.6.6.3.1	Test Purpose and Environment	1200

A.5.6.6.3.2	Test parameters	1200

A.5.6.6.3.3	Test Requirements	1202

A.5.6.7	CSI-RS based Intra-frequency Measurements	1202

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1202

A.5.6.7.1.1	Test purpose and Environment	1202

A.5.6.7.1.2	Test Requirements	1205

A.5.6.8	CSI-RS based Inter-frequency Measurements	1205

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1205

A.5.6.8.1.1	Test Purpose and Environment	1205

A.5.6.8.1.2	Test Requirements	1210

A.5.7	Measurement Performance requirements	1210

A.5.7.1	SS-RSRP	1210

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1210

A.5.7.1.1.1	Test Purpose and Environment	1210

A.5.7.1.1.2	Test parameters	1211

A.5.7.1.1.3	Test Requirements	1213

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1214

A.5.7.1.2.1	Test Purpose and Environment	1214

A.5.7.1.2.2	Test parameters	1214

A.5.7.1.2.3	Test Requirements	1218

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1219

A.5.7.1.3.1	Test Purpose and Environment	1219

A.5.7.1.3.2	Test parameters	1219

A.5.7.1.3.3	Test Requirements	1222

A.5.7.2	SS-RSRQ	1223

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1223

A.5.7.2.1.1	Test Purpose and Environment	1223

A.5.7.2.1.2	Test Parameters	1223

A.5.7.2.1.3	Test Requirements	1226

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1226

A.5.7.2.2.1	Test Purpose and Environment	1226

A.5.7.2.2.2	Test Parameters	1226

A.5.7.2.2.3	Test Requirements	1228

A.5.7.3	SS-SINR	1229

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1229

A.5.7.3.1.1	Test Purpose and Environment	1229

A.5.7.3.1.2	Test Parameters	1229

A.5.7.3.1.3	Test Requirements	1232

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1232

A.5.7.3.2.1	Test Purpose and Environment	1232

A.5.7.3.2.2	Test Parameters	1232

A.5.7.3.2.3	Test Requirements	1234

A.5.7.4	L1-RSRP measurement for beam reporting	1234

A.5.7.4.1	SSB based L1-RSRP measurement	1234

A.5.7.4.1.1	Test Purpose and Environment	1234

A.5.7.4.1.2	Test parameters	1235

A.5.7.4.1.3	Test Requirements	1237

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1238

A.5.7.4.2.1	Test Purpose and Environment	1238

A.5.7.4.2.2	Test parameters	1238

A.5.7.4.2.3	Test Requirements	1240

A.5.7.5	CLI measurements	1241

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1241

A.5.7.5.1.1	Test Purpose and Environment	1241

A.5.7.5.1.2	Test parameters	1241

A.5.7.5.1.3	Test Requirements	1244

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1245

A.5.7.5.2.1	Test Purpose and Environment	1245

A.5.7.5.2.2	Test parameters	1245

A.5.7.5.2.3	Test Requirements	1247

A.5.7.6	L1-SINR measurement for beam reporting	1248

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1250

A.5.7.6.2.1	Test Purpose and Environment	1250

A.5.7.6.2.2	Test parameters	1251

A.5.7.6.2.3	Test Requirements	1253

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1254

A.5.7.6.3.1	Test Purpose and Environment	1254

A.5.7.6.3.2	Test parameters	1254

A.5.7.6.3.3	Test Requirements	1256

A.5.7.7	CSI-RSRP	1256

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1256

A.5.7.7.1.1	Test Purpose and Environment	1256

A.5.7.7.1.2	Test parameters	1257

A.5.7.7.1.3	Test Requirements	1259

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1260

A.5.7.7.2.1	Test Purpose and Environment	1260

A.5.7.7.2.2	Test parameters	1260

A.5.7.7.2.3	Test Requirements	1264

A.5.7.8	CSI-RSRQ	1265

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1265

A.5.7.8.1.1	Test Purpose and Environment	1265

A.5.7.8.1.2	Test Parameters	1265

A.5.7.8.1.3	Test Requirements	1267

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1267

A.5.7.8.2.1	Test Purpose and Environment	1267

A.5.7.8.2.2	Test Parameters	1267

A.5.7.8.2.3	Test Requirements	1269

A.5.7.9	CSI-SINR	1269

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1269

A.5.7.9.1.1	Test Purpose and Environment	1269

A.5.7.9.1.2	Test Parameters	1269

A.5.7.9.1.3	Test Requirements	1272

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1272

A.5.7.9.2.1	Test Purpose and Environment	1272

A.5.7.9.2.2	Test Parameters	1272

A.5.7.9.2.3	Test Requirements	1275

A.5.8	Void	1275

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1027

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1027

A.6.1.1.5.1	Test Purpose and Environment	1027

A.6.1.1.5.2	Test Parameters	1027

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1032

A.6.1.1.6.1	Test Purpose and Environment	1032

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1041

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1041

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1041

A.6.1.2.1.1	Test Purpose and Environment	1041

A.6.1.2.1.2	Test Parameters	1041

A.6.1.2.1.3	Test Requirements	1044

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1045

A.6.1.2.2.1	Test Purpose and Environment	1045

A.6.1.2.2.2	Test Parameters	1045

A.6.1.2.2.3	Test Requirements	1048

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1049

A.6.1.2.3.1	Test Purpose and Environment	1049

A.6.1.2.3.2	Test Parameters	1049

A.6.1.2.3.3	Test Requirements	1052

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1053

A.6.1.2.4.1	Test Purpose and Environment	1053

A.6.1.2.4.2	Test Parameters	1053

A.6.1.2.4.3	Test Requirements	1056

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	1057

A.6.1.2.5.1	Test Purpose and Environment	1057

A.6.1.2.5.2	Test Parameters	1057

A.6.1.2.5.3	Test Requirements	1061

A.6.1.1.7	Void	1062

A.6.2	SA: RRC_INACTIVE state mobility	1062

A.6.3	RRC_CONNECTED state mobility	1062

A.6.3.1	Handover	1062

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1062

A.6.3.1.1.1	Test Purpose and Environment	1062

A.6.3.1.1.2	Test Parameters	1062

A.6.3.1.1.3 Test Requirements	1066

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1066

A.6.3.1.2.1	Test Purpose and Environment	1066

A.6.3.1.2.2	Test Parameters	1066

A.6.3.1.2.3	Test Requirements	1070

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1070

A.6.3.1.3.1	Test Purpose and Environment	1070

A.6.3.1.3.2	Test Parameters	1070

A.6.3.1.3.3 Test Requirements	1074

A.6.3.1.4	 SA NR - E-UTRAN handover	1074

A.6.3.1.4.1	Test Purpose and Environment	1074

A.6.3.1.4.2	Test Requirements	1080

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1080

A.6.3.1.5.1	Test Purpose and Environment	1080

A.6.3.1.5.2	Test Requirements	1086

A.6.3.1.6	 SA NR - UTRAN FDD handover	1086

A.6.3.1.6.1	Test Purpose and Environment	1086

A.6.3.1.6.2	Test Requirements	1090

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	1090

A.6.3.1.7.1	Test Purpose and Environment	1090

A.6.3.1.7.2	Test Parameters	1090

A.6.3.1.7.3 Test Requirements	1094

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	1095

A.6.3.1.8.1	Test Purpose and Environment	1095

A.6.3.1.8.2	Test Parameters	1095

A.6.3.1.8.3 Test Requirements	1099

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1100

A.6.3.1.9.1	Test Purpose and Environment	1100

A.6.3.1.9.2	Test Parameters	1100

A.6.3.1.9.3 Test Requirements	1104

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1104

A.6.3.1.10.1	Test Purpose and Environment	1104

A.6.3.1.10.2	Test Parameters	1104

A.6.3.1.10.3 Test Requirements	1108

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1108

A.6.3.1.11.1	Test Purpose and Environment	1108

A.6.3.1.11.2	Test Parameters	1108

A.6.3.1.11.3 Test Requirements	1115

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1115

A.6.3.1.12.1	Test Purpose and Environment	1115

A.6.3.1.12.2	Test Parameters	1115

A.6.3.1.12.3 Test Requirements	1122

A.6.3.2	RRC Connection Mobility Control	1122

A.6.3.2.1	SA: RRC Re-establishment	1122

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1122

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1126

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1130

A.6.3.2.2	Random Access	1133

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1133

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1137

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1142

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1147

A.6.3.2.3	SA: RRC Connection Release with Redirection	1151

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1151

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1155

A.6.3.3 Conditional handover	1162

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1162

A.6.3.3.1.1	Test Purpose and Environment	1162

A.6.3.3.1.2	Test Parameters	1162

A.6.3.3.1.3 Test Requirements	1166

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1166

A.6.3.3.2.1	Test Purpose and Environment	1166

A.6.3.3.2.2	Test Parameters	1166

A.6.3.3.2.3 Test Requirements	1170

A.6.4	Timing	1170

A.6.4.1	UE transmit timing	1170

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1170

A.6.4.1.1.1	Test Purpose and environment	1170

A.6.4.1.1.2	Test requirements	1174

A.6.4.2	UE timer accuracy	1174

A.6.4.3	Timing advance	1174

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1174

A.6.4.3.1.1	Test Purpose and Environment	1174

A.6.4.3.1.2	Test Parameters	1174

A.6.4.3.1.3	Test Requirements	1178

A.6.5	Signalling characteristics	1178

A.6.5.1	Radio link Monitoring	1178

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1179

A.6.5.1.1.1	Test Purpose and Environment	1179

A.6.5.1.1.2	Test Requirements	1184

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1184

A.6.5.1.2.1	Test Purpose and Environment	1184

A.6.5.1.2.2	Test Requirements	1190

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1190

A.6.5.1.3.1	Test Purpose and Environment	1190

A.6.5.1.3.2	Test Requirements	1196

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1196

A.6.5.1.4.1	Test Purpose and Environment	1196

A.6.5.1.4.2	Test Requirements	1202

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1202

A.6.5.1.5.1	Test Purpose and Environment	1202

A.6.5.1.5.2	Test Requirements	1208

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1208

A.6.5.1.6.1	Test Purpose and Environment	1208

A.6.5.1.6.2	Test Requirements	1214

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1214

A.6.5.1.7.1	Test Purpose and Environment	1214

A.6.5.1.7.2	Test Requirements	1219

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1219

A.6.5.1.8.1	Test Purpose and Environment	1219

A.6.5.1.8.2	Test Requirements	1225

A.6.5.2	Interruption	1225

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1225

A.6.5.2.1.2	Test Requirements	1229

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1230

A.6.5.2.2.1	Test Purpose and Environment	1230

A.6.5.2.2.2	Test Parameters	1230

A.6.5.2.2.3	Test Requirements	1232

A.6.5.3	SCell Activation and Deactivation Delay	1233

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1233

A.6.5.3.1.1	Test Purpose and Environment	1233

A.6.5.3.1.2	Test Requirements	1237

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	1237

A.6.5.3.2.1	Test Purpose and Environment	1237

A.6.5.3.2.2	Test Requirements	1237

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1238

A.6.5.3.3.1	Test Purpose and Environment	1238

A.6.5.3.3.2	Test Requirements	1238

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1239

A.6.5.3.4.1	Test Purpose and Environment	1239

A.6.5.3.4.2	Test Requirements	1243

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1243

A.6.5.3.5.1	Test Purpose and Environment	1243

A.6.5.3.5.2	Test Requirements	1247

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1248

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1248

A.6.5.4.1.1	Test Purpose and Environment	1248

A.6.5.4.1.2	Test Requirements	1258

A.6.5.4.2	Void	1259

A.6.5.5	Beam Failure Detection and Link recovery procedures	1259

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1259

A.6.5.5.1.1	Test Purpose and Environment	1259

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1264

A.6.5.5.2.1	Test Purpose and Environment	1264

A.6.5.5.2.2	Test Requirements	1271

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1271

A.6.5.5.3.1	Test Purpose and Environment	1271

A.6.5.5.3.2	Test Requirements	1277

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1277

A.6.5.5.4.1	Test Purpose and Environment	1277

A.6.5.5.4.2	Test Requirements	1283

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1283

A.6.5.5.5.1	Test Purpose and Environment	1283

A.6.5.5.5.2	Test Requirements	1287

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1288

A.6.5.5.6.1	Test Purpose and Environment	1288

A.6.5.5.6.2	Test Requirements	1292

A.6.5.6	Active BWP switch	1292

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1292

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	1292

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1298

A.6.5.6.2	RRC-based Active BWP Switch	1302

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1302

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1306

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1306

A.6.5.6.3.1.1	Test Purpose and Environment	1306

A.6.5.6.3.1.2	Test Requirements	1311

A.6.5.6.4	SCell dormancy switch	1312

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1312

A.6.5.6.4.1.1	Test Purpose and Environment	1312

A.6.5.6.4.1.2	Test Requirements	1317

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1318

A.6.5.6.4.2.1	 Test Purpose and Environment	1318

A.6.5.6.4.2.2	 Test Requirements	1322

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1322

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1322

A.6.5.7	DL interruptions at switching between two uplink carriers	1326

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1326

A.6.5.7.1.1	Test Purpose and Environment	1326

A.6.5.7.1.2	Test Requirements	1330

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1330

A.6.5.7.2.1	Test Purpose and Environment	1330

A.6.5.7.2.2	Test Requirements	1334

A.6.5.8	UE specific CBW change	1334

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1334

A.6.5.8.1.1	Test Purpose and Environment	1334

A.6.5.8.1.2	Test Requirements	1338

A.6.5.9	Pathloss reference signal switching delay	1338

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	1338

A.6.5.9.1.1	Test Purpose and Environment	1338

A.6.5.9.1.2	Test Requirements	1341

A.6.6	Measurement procedure	1342

A.6.6.1	Intra-frequency Measurements	1342

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1342

A.6.6.1.1.1	Test purpose and Environment	1342

A.6.6.1.1.2	Test parameters	1342

A.6.6.1.1.3	Test Requirements	1346

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1346

A.6.6.1.2.1	Test purpose and Environment	1346

A.6.6.1.2.2	Test parameters	1346

A.6.6.1.2.3	Test Requirements	1350

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1350

A.6.6.1.3.1	Test purpose and Environment	1350

A.6.6.1.3.2	Test parameters	1350

A.6.6.1.3.3	Test Requirements	1354

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1354

A.6.6.1.4.1	Test purpose and Environment	1354

A.6.6.1.4.2	Test parameters	1354

A.6.6.1.4.3	Test Requirements	1358

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1358

A.6.6.1.5.1	Test purpose and Environment	1358

A.6.6.1.5.2	Test parameters	1358

A.6.6.1.5.3	Test Requirements	1360

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1361

A.6.6.1.6.1	Test purpose and Environment	1361

A.6.6.1.6.2	Test parameters	1361

A.6.6.1.6.3	Test Requirements	1362

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1363

A.6.6.1.7.1	Test purpose and Environment	1363

A.6.6.1.7.2	Test parameters	1363

A.6.6.1.7.3	Test Requirements	1367

A.6.6.2	Inter-frequency Measurements	1367

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1367

A.6.6.2.1.1	Test Purpose and Environment	1367

A.6.6.2.1.2	Test Requirements	1371

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1371

A.6.6.2.2.1	Test Purpose and Environment	1371

A.6.6.2.2.2	Test Requirements	1375

A.6.6.2.3	Void	1376

A.6.6.2.4	Void	1376

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1376

A.6.6.2.5.1	Test Purpose and Environment	1376

A.6.6.2.5.2	Test Requirements	1380

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1380

A.6.6.2.6.1	Test Purpose and Environment	1380

A.6.6.2.6.2	Test Requirements	1384

A.6.6.2.7	Void	1385

A.6.6.2.8	Void	1385

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1385

A.6.6.2.9.1	Test Purpose and Environment	1385

A.6.6.2.9.2	Test Requirements	1388

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1388

A.6.6.2.10.1	Test Purpose and Environment	1388

A.6.6.2.10.2	Test Requirements	1392

A.6.6.3	Inter-RAT Measurements	1396

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1396

A.6.6.3.1.1	Test Purpose and Environment	1396

A.6.6.3.1.2	Test Requirements	1402

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1402

A.6.6.3.2.1	Test Purpose and Environment	1402

A.6.6.3.2.2	Test Requirements	1409

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1409

A.6.6.3.3.1	Test Purpose and Environment	1409

A.6.6.3.3.2	Test Requirements	1413

A.6.6.4	L1-RSRP measurement for beam reporting	1413

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1413

A.6.6.4.1.1	Test Purpose and Environment	1413

A.6.6.4.1.2	Test parameters	1414

A.6.6.4.1.3	Test Requirements	1417

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1417

A.6.6.4.2.1	Test Purpose and Environment	1417

A.6.6.4.2.2	Test parameters	1418

A.6.6.4.2.3	Test Requirements	1421

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1421

A.6.6.4.3.1	Test Purpose and Environment	1421

A.6.6.4.3.2	Test parameters	1422

A.6.6.4.3.3	Test Requirements	1425

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1425

A.6.6.4.4.1	Test Purpose and Environment	1425

A.6.6.4.4.2	Test parameters	1426

A.6.6.4.4.3	Test Requirements	1429

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1429

A.6.6.4.5.1	Test Purpose and Environment	1429

A.6.6.4.5.2	Test parameters	1430

A.6.6.4.5.3	Test Requirements	1432

A.6.6.5	1432

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1432

A.6.6.5.1.1	Test Purpose and Environment	1432

A.6.6.5.1.2	Test Requirements	1436

A.6.6.6	CLI measurements	1436

A.6.6.6.1	SRS-RSRP measurement with DRX	1436

A.6.6.6.1.1	Test Purpose and Environment	1436

A.6.6.6.1.2	Test Parameters	1437

A.6.6.6.1.3	Test Requirements	1440

A.6.6.6.2	CLI-RSSI measurement with DRX	1440

A.6.6.6.2.1	Test Purpose and Environment	1440

A.6.6.6.2.2	Test Parameters	1441

A.6.6.6.2.3	Test Requirements	1443

A.6.6.7	NR measurements with autonomous gaps	1443

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1443

A.6.6.7.1.1	Test Purpose and Environment	1443

A.6.6.7.1.2	Test Parameters	1443

A.6.6.7.1.3	Test Requirements	1446

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1446

A.6.6.7.2.1	Test Purpose and Environment	1446

A.6.6.7.2.2	Test Requirements	1449

A.6.6.8	L1-SINR measurement for beam reporting	1450

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1452

A.6.6.8.2.1	Test Purpose and Environment	1452

A.6.6.8.2.2	Test parameters	1452

A.6.6.8.2.3	Test Requirements	1456

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1456

A.6.6.8.3.1	Test Purpose and Environment	1456

A.6.6.8.3.2	Test parameters	1457

A.6.6.8.3.3	Test Requirements	1459

A.6.6.9	Idle Mode CA/DC Measurements	1459

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1459

A.6.6.9.1.1	Test Purpose and Environment	1459

A.6.6.9.1.2	Test Requirements	1466

A.6.6.10	CSI-RS based intra-frequency Measurements	1466

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1466

A.6.6.10.1.1	Test purpose and Environment	1466

A.6.6.10.1.2	Test Requirements	1469

A.6.6.11	CSI-RS based inter-frequency Measurements	1469

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1469

A.6.6.11.1.1	Test Purpose and Environment	1469

A.6.6.11.1.2	Test Requirements	1473

A.6.6.12	RSTD measurements	1473

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1473

A.6.6.12.1.1	Test Purpose and Environment	1473

A.6.6.12.1.2	Test Requirements	1479

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1480

A.6.6.12.2.1	Test Purpose and Environment	1480

A.6.6.12.2.2	Test Requirements	1485

A.6.6.13 PRS-RSRP measurements	1486

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1486

A.6.6.13.1.1	Test purpose and Environment	1486

A.6.6.13.1.2	Test Requirements	1490

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1490

A.6.6.13.2.1	Test purpose and Environment	1490

A.6.6.13.2.2	Test Requirements	1494

A.6.6.14	UE Rx-Tx time difference measurements	1494

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1494

A.6.6.14.1.1	Test purpose and environment	1494

A.6.6.14.1.2	Test requirements	1498

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1498

A.6.6.14.2.1	Test purpose and environment	1498

A.6.6.14.2.2	Test requirements	1502

A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting	1502

A.6.6.15.1	Test Purpose and Environment	1502

A.6.6.15.2	Test Requirements	1509

A.6.7	Measurement Performance requirements	1510

A.6.7.1	SS-RSRP	1510

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1510

A.6.7.1.1.1	Test Purpose and Environment	1510

A.6.7.1.1.2	Test parameters	1510

A.6.7.1.1.3	Test Requirements	1515

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1515

A.6.7.1.2.1	Test Purpose and Environment	1515

A.6.7.1.2.2	Test parameters	1515

A.6.7.1.2.3	Test Requirements	1519

A.6.7.1.3	Void	1519

A.6.7.2	SS-RSRQ	1519

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1519

A.6.7.2.1.1	Test Purpose and Environment	1519

A.6.7.2.1.2	Test Parameters	1520

A.6.7.2.1.3	Test Requirements	1525

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1525

A.6.7.2.2.1	Test Purpose and Environment	1525

A.6.7.2.2.2	Test Parameters	1525

A.6.7.2.2.3	Test Requirements	1530

A.6.7.3	SS-SINR	1530

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1530

A.6.7.3.1.1	Test Purpose and Environment	1530

A.6.7.3.1.2	Test Parameters	1531

A.6.7.3.1.3	Test Requirements	1535

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1535

A.6.7.3.2.1	Test Purpose and Environment	1535

A.6.7.3.2.2	Test Parameters	1536

A.6.7.3.2.3	Test Requirements	1540

A.6.7.4	L1-RSRP measurement for beam reporting	1541

A.6.7.4.1	SSB based L1-RSRP measurement	1541

A.6.7.4.1.1	Test Purpose and Environment	1541

A.6.7.4.1.2	Test parameters	1541

A.6.7.4.1.3	Test Requirements	1545

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1545

A.6.7.4.2.1	Test Purpose and Environment	1545

A.6.7.4.2.2	Test parameters	1546

A.6.7.4.2.3	Test Requirements	1550

A.6.7.5	E-UTRAN RSRP	1550

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1550

A.6.7.5.1.1	Test Purpose and Environment	1550

A.6.7.5.1.2	Test parameters	1551

A.6.7.5.1.3	Test Requirements	1557

A.6.7.6	E-UTRAN RSRQ	1558

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1558

A.6.7.6.1.1	Test Purpose and Environment	1558

A.6.7.6.1.2	Test parameters	1558

A.6.7.6.1.3	Test Requirements	1563

A.6.7.7	E-UTRAN RS-SINR	1564

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1564

A.6.7.7.1.1	Test Purpose and Environment	1564

A.6.7.7.1.2	Test parameters	1564

A.6.7.7.1.3	Test Requirements	1570

A.6.7.8	CLI measurements	1571

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1571

A.6.7.8.1.1	Test Purpose and Environment	1571

A.6.7.8.1.2	Test parameters	1571

A.6.7.8.1.3	Test Requirements	1577

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1577

A.6.7.8.2.1	Test Purpose and Environment	1577

A.6.7.8.2.2	Test parameters	1578

A.6.7.8.2.3	Test Requirements	1581

A.6.7.9	L1-SINR measurement for beam reporting	1582

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1586

A.6.7.9.2.1	Test Purpose and Environment	1586

A.6.7.9.2.2	Test parameters	1587

A.6.7.9.2.3	Test Requirements	1592

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1592

A.6.7.9.3.1	Test Purpose and Environment	1592

A.6.7.9.3.2	Test parameters	1592

A.6.7.9.3.3	Test Requirements	1597

A.6.7.10	CSI-RSRP	1597

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1597

A.6.7.9.10.1	Test Purpose and Environment	1597

A.6.7.9.10.2	Test parameters	1598

A.6.7.10.1.3	Test Requirements	1603

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1603

A.6.7.9.10.1	Test Purpose and Environment	1603

A.6.7.10.2.2	Test parameters	1603

A.6.7.10.2.3	Test Requirements	1608

A.6.7.11	CSI-RSRQ	1608

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1608

A.6.7.11.1.1	Test Purpose and Environment	1608

A.6.7.11.1.2	Test Parameters	1608

A.6.7.11.1.3	Test Requirements	1613

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1613

A.6.7.11.2.1	Test Purpose and Environment	1613

A.6.7.11.2.2	Test Parameters	1613

A.6.7.11.2.3	Test Requirements	1619

A.6.7.12	CSI-SINR	1620

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1620

A.6.7.12.1.1	Test Purpose and Environment	1620

A.6.7.12.1.2	Test Parameters	1620

A.6.7.12.1.3	Test Requirements	1624

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1625

A.6.7.12.2.1	Test Purpose and Environment	1625

A.6.7.12.2.2	Test Parameters	1625

A.6.7.12.2.3	Test Requirements	1631

A.6.7.13	RSTD measurements	1631

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	1631

A.6.7.13.1.1	Test purpose and Environment	1631

A.6.7.13.1.2	Test Requirements	1634

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	1634

A.6.7.13.2.1	Test purpose and Environment	1634

A.6.7.13.2.2	Test Requirements	1638

A.6.7.14	PRS-RSRP measurements	1638

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	1638

A.6.7.14.1.1	Test Purpose and Environment	1638

A.6.7.14.1.2	Test parameters	1638

A.6.7.14.1.3	Test Requirements	1643

A.6.7.15	UE Rx-Tx time difference measurements	1643

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1643

A.6.7.15.1.1	Test purpose and environment	1643

A.6.7.15.1.2	Test parameters	1644

A.6.7.15.1.2	Test parameters	1644

A.6.7.15.1.3	Test requirements	1647

A.7	NR standalone tests with one or more NR cells in FR2	1647

A.7.1	SA: RRC_IDLE state mobility	1647

A.7.1.1	Cell re-selection to NR	1647

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1647

A.7.1.1.1.1	Test Purpose and Environment	1647

A.7.1.1.1.2	Test Parameters	1647

A.7.1.1.1.3	Test Requirements	1651

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1651

A.7.1.1.2.1	Test Purpose and Environment	1651

A.7.1.1.2.2	Test Parameters	1651

A.7.1.1.2.3	Test Requirements	1655

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1655

A.7.1.1.3.1	Test Purpose and Environment	1655

A.7.1.1.3.2	Test Parameters	1655

A.7.1.1.3.3	Test Requirements	1658

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1658

A.7.1.1.4.1	Test Purpose and Environment	1658

A.7.1.1.4.2	Test Parameters	1658

A.7.1.1.4.3	Test Requirements	1661

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1661

A.7.1.1.5.1	Test Purpose and Environment	1661

A.7.1.1.5.2	Test Parameters	1661

A.7.1.1.5.3	Test Requirements	1664

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1664

A.7.1.1.6.1	Test Purpose and Environment	1664

A.7.1.1.6.2	Test Parameters	1664

A.7.1.1.6.3	Test Requirements	1666

A.7.2	SA: RRC_INACTIVE state mobility	1667

A.7.3	RRC_CONNECTED state mobility	1667

A.7.3.1	Handover	1667

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1667

A.7.3.1.1.1	Test Purpose and Environment	1667

A.7.3.1.1.2	Test Parameters	1667

A.7.3.1.1.3	Test Requirements	1671

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1671

A.7.3.1.2.1	Test Purpose and Environment	1671

A.7.3.1.2.2	Test Parameters	1671

A.7.3.1.2.3	Test Requirements	1674

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1674

A.7.3.1.3.1	Test Purpose and Environment	1674

A.7.3.1.3.2	Test Parameters	1674

A.7.3.1.3.3 Test Requirements	1676

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1677

A.7.3.1.4.1	Test Purpose and Environment	1677

A.7.3.1.4.2	Test Parameters	1677

A.7.3.1.4.3 Test Requirements	1684

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1684

A.7.3.1.5.1	Test Purpose and Environment	1684

A.7.3.1.5.2	Test Parameters	1684

A.7.3.1.5.3 Test Requirements	1691

A.7.3.2	RRC Connection Mobility Control	1691

A.7.3.2.1	SA: RRC Re-establishment	1691

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1691

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1694

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1697

A.7.3.2.1.3.1	Test Purpose and Environment	1697

A.7.3.2.1.3.2	Test Requirements	1699

A.7.3.2.2	Random Access	1700

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	1700

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	1704

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1708

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	1711

A.7.3.2.3	SA: RRC Connection Release with Redirection	1714

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1714

A.7.3.3 Conditional Handover	1718

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1718

A.7.3.3.1.1	Test Purpose and Environment	1718

A.7.3.3.1.2	Test Parameters	1718

A.7.3.3.1.2.3	Test Requirements	1721

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	1721

A.7.3.3.2.1	Test Purpose and Environment	1721

A.7.3.3.2.2	Test Parameters	1721

A.7.3.3.2.3 Test Requirements	1723

A.7.4	Timing	1723

A.7.4.1	UE transmit timing	1723

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1723

A.7.4.1.1.1	Test Purpose and environment	1723

A.7.4.1.1.2	Test requirements	1726

A.7.4.2	UE timer accuracy	1727

A.7.4.3	Timing advance	1727

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1727

A.7.4.3.1.1	Test Purpose and Environment	1727

A.7.4.3.1.2	Test Parameters	1727

A.7.4.3.1.3 Test Requirements	1730

A.7.5	Signaling characteristics	1731

A.7.5.1	Radio link Monitoring	1731

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1731

A.7.5.1.1.1	Test Purpose and Environment	1731

A.7.5.1.1.2	Test Requirements	1734

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1735

A.7.5.1.2.1	Test Purpose and Environment	1735

A.7.5.1.2.2	Test Requirements	1739

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1740

A.7.5.1.3.1	Test Purpose and Environment	1740

A.7.5.1.3.2	Test Requirements	1744

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1744

A.7.5.1.4.1	Test Purpose and Environment	1744

A.7.5.1.4.2	Test Requirements	1749

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1749

A.7.5.1.5.1	Test Purpose and Environment	1749

A.7.5.1.5.2	Test Requirements	1754

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1754

A.7.5.1.6.1	Test Purpose and Environment	1754

A.7.5.1.6.2	Test Requirements	1758

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1758

A.7.5.1.7.1	Test Purpose and Environment	1758

A.7.5.1.7.2	Test Requirements	1762

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1762

A.7.5.1.8.1	Test Purpose and Environment	1762

A.7.5.1.8.2	Test Requirements	1767

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1767

A.7.5.1.9.1	Test Purpose and Environment	1767

A.7.5.1.9.2	Test Requirements	1770

A.7.5.2	Interruption	1770

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1770

A.7.5.2.1.1	Test Purpose and Environment	1770

A.7.5.2.1.2	Test Requirements	1773

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1774

A.7.5.2.2.1	Test Purpose and Environment	1774

A.7.5.2.2.2	Test Parameters	1774

A.7.5.2.2.3	Test Requirements	1776

A.7.5.3	SCell Activation and Deactivation Delay	1776

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1776

A.7.5.3.1.1	Test Purpose and Environment	1776

A.7.5.3.1.2	Test Requirements	1778

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1778

A.7.5.3.2.1	Test Purpose and Environment	1778

A.7.5.3.2.2	Test Requirements	1782

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1783

A.7.5.3.3.1	Test Purpose and Environment	1783

A.7.5.3.3.2	Test Requirements	1786

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	1787

A.7.5.3.4.1	Test Purpose and Environment	1787

A.7.5.3.4.2	Test Requirements	1790

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	1791

A.7.5.3.5.1	Test Purpose and Environment	1791

A.7.5.3.5.2	Test Requirements	1793

A.7.5.4	Void	1794

A.7.5.5	Beam Failure Detection and Link recovery procedures	1794

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1794

A.7.5.5.1.1	Test Purpose and Environment	1794

A.7.5.5.1.2	Test Requirements	1798

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1798

A.7.5.5.2.1	Test Purpose and Environment	1798

A.7.5.5.2.2	Test Requirements	1802

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1802

A.7.5.5.3.1	Test Purpose and Environment	1802

A.7.5.5.3.2	Test Requirements	1806

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1806

A.7.5.5.4.1	Test Purpose and Environment	1806

A.7.5.5.4.2	Test Requirements	1810

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1810

A.7.5.5.5.1	Test Purpose and Environment	1810

A.7.5.5.5.2	Test Requirements	1814

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1814

A.7.5.5.6.1	Test Purpose and Environment	1814

A.7.5.5.6.2	Test Requirements	1818

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1818

A.7.5.5.7.1	Test Purpose and Environment	1818

A.7.5.5.7.2	Test Requirements	1822

A.7.5.6	Active BWP switch	1822

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1822

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1822

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1827

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1832

A.7.5.6.1.3.1	Test Purpose and Environment	1832

A.7.5.6.1.3.2	Test Requirements	1835

A.7.5.6.2	RRC-based Active BWP Switch	1835

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1839

A.7.5.6.3.1.1	Test Purpose and Environment	1839

A.7.5.6.3.1.2	Test Requirements	1842

A.7.5.6.4	SCell dormancy switch	1842

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	1842

A.7.5.6.4.1.1	Test Purpose and Environment	1842

A.7.5.6.4.1.2	Test Requirements	1847

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	1847

A.7.5.6.4.2.1	 Test Purpose and Environment	1847

A.7.5.6.4.2.2	 Test Requirements	1852

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1852

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	1852

A.7.5.7	PSCell addition and release delay	1855

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1855

A.7.5.7.1.1	Test Purpose and Environment	1855

A.7.5.7.1.2	Test Requirements	1858

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1858

A.7.5.7.2.1	Test Purpose and Environment	1858

A.7.5.7.2.2	Test Requirements	1861

A.7.5.8	Active TCI state switch delay	1861

A.7.5.8.1	MAC-CE based active TCI state switch	1861

A.7.5.8.2	RRC based active TCI state switch	1865

A.7.5.9	Uplink spatial relation switch delay	1869

A.7.5.9.1.1.1	Test Purpose and Environment	1869

A.7.5.9.1.1.2	Test Requirements	1872

A.7.5.9.2	RRC based spatial relation switch	1872

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1872

A.7.5.9.2.1.1	Test Purpose and Environment	1872

A.7.5.9.2.1.2	Test Requirements	1875

A.7.5.10	UE specific CBW change	1875

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1875

A.7.5.10.1.1	Test Purpose and Environment	1875

A.7.5.10.1.2	Test Requirements	1878

A.7.6	Measurement procedure	1879

A.7.6.1	Intra-frequency Measurements	1879

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1879

A.7.6.1.1.1	Test purpose and Environment	1879

A.7.6.1.1.2	Test Requirements	1882

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1882

A.7.6.1.2.1	Test purpose and Environment	1882

A.7.6.1.2.2	Test Requirements	1885

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1886

A.7.6.1.3.1	Test purpose and Environment	1886

A.7.6.1.3.2	Test Requirements	1889

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1889

A.7.6.1.4.1	Test purpose and Environment	1889

A.7.6.1.4.2	Test Requirements	1892

A.7.6.2	Inter-frequency Measurements	1893

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1893

A.7.6.2.1.1	Test Purpose and Environment	1893

A.7.6.2.1.2	Test Requirements	1896

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1896

A.7.6.2.2.1	Test Purpose and Environment	1896

A.7.6.2.2.2	Test Requirements	1900

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1900

A.7.6.2.3.1	Test Purpose and Environment	1900

A.7.6.2.3.2	Test Requirements	1904

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1904

A.7.6.2.4.1	Test Purpose and Environment	1904

A.7.6.2.4.2	Test Requirements	1908

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1908

A.7.6.2.5.1	Test Purpose and Environment	1908

A.7.6.2.5.2	Test Requirements	1912

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1913

A.7.6.2.6.1	Test Purpose and Environment	1913

A.7.6.2.6.2	Test Requirements	1917

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1918

A.7.6.2.7.1	Test Purpose and Environment	1918

A.7.6.2.7.2	Test Requirements	1922

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1923

A.7.6.2.8.1	Test Purpose and Environment	1923

A.7.6.2.8.2	Test Requirements	1927

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1928

A.7.6.2.9.1	Test Purpose and Environment	1928

A.7.6.2.9.2	Test Requirements	1932

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1932

A.7.6.2.10.1	Test Purpose and Environment	1932

A.7.6.2.10.2	Test Requirements	1934

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1934

A.7.6.2.11.1	Test Purpose and Environment	1934

A.7.6.2.11.2	Test Requirements	1937

A.7.6.3	L1-RSRP measurement for beam reporting	1938

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1938

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1938

A.7.6.3.1.1	Test Purpose and Environment	1938

A.7.6.3.1.2	Test parameters	1938

A.7.6.3.1.3	Test Requirements	1940

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1940

A.7.6.3.2.1	Test Purpose and Environment	1940

A.7.6.3.2.2	Test parameters	1941

A.7.6.3.2.3	Test Requirements	1943

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1943

A.7.6.3.3.1	Test Purpose and Environment	1943

A.7.6.3.3.2	Test parameters	1944

A.7.6.3.3.3	Test Requirements	1946

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1947

A.7.6.3.4.1	Test Purpose and Environment	1947

A.7.6.3.4.2	Test parameters	1947

A.7.6.3.3.3	Test Requirements	1949

A.7.6.4	CLI measurements	1950

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1950

A.7.6.4.1.1	Test Purpose and Environment	1950

A.7.6.4.1.2	Test Parameters	1950

A.7.6.4.1.3	Test Requirements	1952

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1952

A.7.6.4.2.1	Test Purpose and Environment	1952

A.7.6.4.2.2	Test Parameters	1953

A.7.6.4.2.3	Test Requirements	1954

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1955

A.7.6.5.1.1	Test Purpose and Environment	1955

A.7.6.5.1.2	Test Requirements	1958

A.7.6.6	L1-SINR measurement for beam reporting	1958

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1960

A.7.6.6.2.1	Test Purpose and Environment	1960

A.7.6.6.2.2	Test parameters	1961

A.7.6.6.2.3	Test Requirements	1963

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1963

A.7.6.6.3.1	Test Purpose and Environment	1963

A.7.6.6.3.2	Test parameters	1964

A.7.6.6.3.3	Test Requirements	1966

A.7.6.7	CSI-RS based intra-frequency Measurements	1966

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	1966

A.7.6.7.1.1	Test purpose and Environment	1966

A.7.6.7.1.2	Test Requirements	1969

A.7.6.8	CSI-RS based inter-frequency Measurements	1969

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	1969

A.7.6.8.1.1	Test Purpose and Environment	1969

A.7.6.8.1.2	Test Requirements	1972

A.7.6.9	RSTD measurements	1972

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1972

A.7.6.9.1.1	Test Purpose and Environment	1972

A.7.6.9.1.2	Test Requirements	1977

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	1977

A.7.6.9.2.1	Test Purpose and Environment	1977

A.7.6.9.2.2	Test Requirements	1983

A.7.6.10 PRS-RSRP measurements	1983

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	1983

A.7.6.10.1.1	Test Purpose and Environment	1983

A.7.6.10.1.2	Test Requirements	1987

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1987

A.7.6.10.2.1	Test Purpose and Environment	1987

A.7.6.10.2.2	Test Requirements	1991

A.7.6.11	UE Rx-Tx time difference measurements	1991

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	1991

A.7.6.11.1.1	Test purpose and environment	1991

A.7.6.11.1.2	Test requirements	1993

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	1994

A.7.6.11.2.1	Test purpose and environment	1994

A.7.6.11.2.2	Test requirements	1996

A.7.7	Measurement Performance requirements	1997

A.7.7.1	SS-RSRP	1997

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1997

A.7.7.1.1.1	Test Purpose and Environment	1997

A.7.7.1.1.2	Test parameters	1997

A.7.7.1.1.3	Test Requirements	2001

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2001

A.7.7.1.2.1	Test Purpose and Environment	2001

A.7.7.1.2.2	Test parameters	2002

A.7.7.1.2.3	Test Requirements	2006

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2007

A.7.7.1.3.1	Test Purpose and Environment	2007

A.7.7.1.3.2	Test parameters	2007

A.7.7.1.3.3	Test Requirements	2011

A.7.7.2	SS-RSRQ	2011

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2011

A.7.7.2.1.1	Test Purpose and Environment	2011

A.7.7.2.1.2	Test Parameters	2011

A.7.7.2.1.3	Test Requirements	2013

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2013

A.7.7.2.2.1	Test Purpose and Environment	2013

A.7.7.2.2.2	Test Parameters	2013

A.7.7.2.2.3	Test Requirements	2015

A.7.7.3	SS-SINR	2015

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2015

A.7.7.3.1.1	Test Purpose and Environment	2015

A.7.7.3.1.2	Test Parameters	2015

A.7.7.3.1.3	Test Requirements	2018

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2018

A.7.7.3.2.1	Test Purpose and Environment	2018

A.7.7.3.2.2	Test Parameters	2018

A.7.7.3.2.3	Test Requirements	2020

A.7.7.4	L1-RSRP measurement for beam reporting	2020

A.7.7.4.1	SSB based L1-RSRP measurement	2020

A.7.7.4.1.1	Test Purpose and Environment	2020

A.7.7.4.1.2	Test parameters	2021

A.7.7.4.1.3	Test Requirements	2023

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2024

A.7.7.4.2.1	Test Purpose and Environment	2024

A.7.7.4.2.2	Test parameters	2024

A.7.7.4.2.3	Test Requirements	2026

A.7.7.5	CLI measurements	2027

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2027

A.7.7.5.1.1	Test Purpose and Environment	2027

A.7.7.5.1.2	Test parameters	2027

A.7.7.5.1.3	Test Requirements	2030

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2031

A.7.7.5.2.1	Test Purpose and Environment	2031

A.7.7.5.2.2	Test parameters	2031

A.7.7.5.2.3	Test Requirements	2033

A.7.7.6	L1-SINR measurement for beam reporting	2034

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2036

A.7.7.6.2.1	Test Purpose and Environment	2036

A.7.7.6.2.2	Test parameters	2037

A.7.7.6.2.3	Test Requirements	2039

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2039

A.7.7.6.3.1	Test Purpose and Environment	2039

A.7.7.6.3.2	Test parameters	2040

A.7.7.6.3.3	Test Requirements	2042

A.7.7.7	CSI-RSRP	2042

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2042

A.7.7.7.1.1	Test Purpose and Environment	2042

A.7.7.7.1.2	Test parameters	2043

A.7.7.7.1.3	Test Requirements	2046

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2047

A.7.7.7.2.1	Test Purpose and Environment	2047

A.7.7.7.2.2	Test parameters	2047

A.7.7.7.2.3	Test Requirements	2050

A.7.7.8	CSI-RSRQ	2050

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2050

A.7.7.8.1.1	Test Purpose and Environment	2050

A.7.7.8.1.2	Test Parameters	2051

A.7.7.8.1.3	Test Requirements	2053

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2053

A.7.7.8.2.1	Test Purpose and Environment	2053

A.7.7.8.2.2	Test Parameters	2053

A.7.7.8.2.3	Test Requirements	2055

A.7.7.9	CSI-SINR	2055

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2055

A.7.7.9.1.1	Test Purpose and Environment	2055

A.7.7.9.1.2	Test Parameters	2055

A.7.7.9.1.3	Test Requirements	2057

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2057

A.7.7.9.2.1	Test Purpose and Environment	2057

A.7.7.9.2.2	Test Parameters	2058

A.7.7.9.2.3	Test Requirements	2060

A.7.7.10	RSTD measurements	2060

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2060

A.7.7.10.1.1	Test purpose and Environment	2060

A.7.7.10.1.2	Test Requirements	2063

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2063

A.7.7.10.2.1	Test purpose and Environment	2063

A.7.7.10.2.2	Test Requirements	2065

A.7.7.11	PRS-RSRP measurements	2065

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2065

A.7.7.11.1.1	Test Purpose and Environment	2065

A.7.7.11.1.2	Test parameters	2065

A.7.7.11.1.3	Test Requirements	2068

A.7.7.12	UE Rx-Tx time difference measurements	2069

A.7.7.12.1	UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA	2069

A.7.7.12.1.1	Test purpose and environment	2069

A.7.7.12.1.2	Test parameters	2069

A.7.7.12.1.2	Test parameters	2070

A.7.7.12.1.3	Test requirements	2071

A.8	E-UTRA standalone tests for NR RRM	2071

A.8.1	Void	2071

A.8.2	RRC_IDLE state mobility	2071

A.8.2.1	Inter-RAT NR Cell re-selection	2071

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2071

A.8.2.1.1.1	Test Purpose and Environment	2071

A.8.2.1.1.2	Test Requirements	2076

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2077

A.8.2.1.2.1	Test Purpose and Environment	2077

A.8.2.1.2.2	Test Requirements	2081

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2082

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	2082

A.8.2.2.1.1	Test Purpose and Environment	2082

A.8.2.2.1.2	Test Requirements	2085

A.8.2.2.2	E-UTRA – NR Early Measruement Reporting for NR in FR2	2086

A.8.2.2.2.1	Test Purpose and Environment	2086

A.8.2.2.2.2	Test Requirements	2088

A.8.3	RRC_CONNECTED state mobility	2089

A.8.3.1	Handover	2089

A.8.3.1.1	E-UTRAN - NR handover in FR1	2089

A.8.3.1.1.1	Test Purpose and Environment	2089

A.8.3.1.1.2	Test Requirements	2094

A.8.4	Measurement procedure	2094

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2094

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2094

A.8.4.1.1.1	Test Purpose and Environment	2094

A.8.4.1.1.2	Test Requirements	2096

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2097

A.8.4.1.2.1	Test Purpose and Environment	2097

A.8.4.1.2.2	Test Requirements	2098

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2098

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2098

A.8.4.2.1.1	Test Purpose and Environment	2098

A.8.4.2.1.2	Test Requirements	2103

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2103

A.8.4.2.2.1	Test Purpose and Environment	2103

A.8.4.2.2.2	Test Requirements	2107

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2107

A.8.4.2.3.1	Test Purpose and Environment	2107

A.8.4.2.3.2	Test Requirements	2112

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2112

A.8.4.2.4.1	Test Purpose and Environment	2112

A.8.4.2.4.2	Test Requirements	2116

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2116

A.8.4.2.5.1	Test Purpose and Environment	2116

A.8.4.2.5.2	Test Requirements	2118

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2119

A.8.4.2.6.1	Test Purpose and Environment	2119

A.8.4.2.6.2	Test Requirements	2121

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2122

A.8.4.2.7.1	Test Purpose and Environment	2122

A.8.4.2.7.2	Test Requirements	2124

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2125

A.8.4.2.8.1	Test Purpose and Environment	2125

A.8.4.2.8.2	Test Requirements	2127

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2128

A.8.4.2.9.1	Test Purpose and Environment	2128

A.8.4.2.9.2	Test Requirements	2132

A.8.5	Measurement performance	2132

A.8.5.1	SFTD accuracy	2132

A.8.5.1.1	SFTD accuracy	2132

A.8.5.1.1.1	Test Purpose	2132

A.8.5.1.1.2	Test Environment	2133

A.8.5.1.1.3	Test Requirements	2139

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2139

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2139

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2144

A.8.5.2.1.2.1	Test Purpose and Environment	2144

A.8.5.2.1.2.2	Test Parameters	2144

A.8.5.2.1.2.3	Test Requirements	2146

A.8.5.2.2	SS-RSRQ	2146

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2146

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2151

A.8.5.2.2.2.1	Test Purpose and Environment	2151

A.8.5.2.2.2.2	Test Parameters	2151

A.8.5.2.2.2.3	Test Requirements	2153

A.8.5.2.3	SS-SINR	2153

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2153

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2157

A.8.5.2.3.2.1	Test Purpose and Environment	2157

A.8.5.2.3.2.2	Test Parameters	2157

A.8.5.2.3.2.3	Test Requirements	2159

A.9	V2X Tests	2160

A.9.1	V2X Tests in FR1	2160

A.9.1.1	Test for V2X UE Transmit Timing	2160

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	2160

A.9.1.1.1.1	Test Purpose and Environment	2160

A.9.1.1.1.2	Test requirements	2160

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2160

A.9.1.1.2.1	Test Purpose and Environment	2160

A.9.1.1.2.2	Test requirements	2161

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2161

A.9.1.1.3.1	Test Purpose and Environment	2161

A.9.1.1.3.2	Test requirements	2164

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	2164

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2164

A.9.1.2.1.1	Test Purpose and Environment	2164

A.9.1.2.1.2	Test Requirements	2167

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2167

A.9.1.2.2.1	Test Purpose and Environment	2167

A.9.1.2.2.2	Test Requirements	2168

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	2169

A.9.1.3.1	 Test for GNSS configured as the highest priority	2169

A.9.1.3.1.1	Test Purpose and Environment	2169

A.9.1.3.1.2	Test Requirements	2171

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2172

A.9.1.3.2.1	Test Purpose and Environment	2172

A.9.1.3.2.2	Test Requirements	2174

A.9.1.4	Test for L1 SL-RSRP Measurement	2175

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2175

A.9.1.4.1.1	Test Purpose and Environment	2175

A.9.1.4.1.2	Test Requirements	2178

A.9.1.4.2	Test for V2X UE Resource Pre-emption	2178

A.9.1.4.2.1	Test Purpose and Environment	2178

A.9.1.4.2.2	Test Requirements	2182

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	2182

A.9.1.4.3.1	Test Purpose and Environment	2182

A.9.1.4.3.2	Test Requirements	2187

A.9.1.5	Test for Congestion Control Measurement	2187

A.9.1.5.1	Test Purpose and Environment	2187

A.9.1.5.2	Test Requirements	2193

A.9.1.6	Test for Interruption	2193

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2193

A.9.1.6.1.1	Test Purpose and Environment	2193

A.9.1.6.1.2	Test Requirements	2196

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2196

A.10.1	RRC_CONNECTED state mobility	2196

A.10.1.1	RRC connection mobility control	2196

A.10.1.1.1	Random Access	2196

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2196

A.10.1.1.1.1.1	Test Purpose and Environment	2196

A.10.1.1.1.1.2	Test Requirements	2199

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2199

A.10.1.1.1.1.2.2	Random Access Response Reception	2199

A.10.1.1.1.1.2.3	No Random Access Response Reception	2200

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2200

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2200

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2201

A.10.1.1.1.2.1	Test Purpose and Environment	2201

A.10.1.1.1.2.2	Test Requirements	2204

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2204

A.10.1.1.1.2.2.2	Random Access Response Reception	2205

A.10.1.1.1.2.2.3	No Random Access Response Reception	2205

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2205

A.10.1.1.1.3.1	Test Purpose and Environment	2205

A.10.1.1.1.3.2	Test Requirements	2209

A.10.1.1.1.3.2.1	MsgA Transmission	2209

A.10.1.1.1.3.2.2	MsgB Reception	2209

A.10.1.1.1.3.2.3	No MsgB Reception	2210

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2210

A.10.1.1.1.4.1	Test Purpose and Environment	2210

A.10.1.1.1.4.2	Test Requirements	2214

A.10.1.1.1.4.2.1	MsgA Transmission	2214

A.10.1.1.1.4.2.2	MsgB Reception	2215

A.10.1.1.1.4.2.3	No MsgB Reception	2215

A.10.2	Timing	2216

A.10.2.1	UE transmit timing	2216

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	2216

A.10.2.1.1.1	Test Purpose and environment	2216

A.10.2.1.1.2	Test requirements	2219

A.10.2.2	UE timing advance	2220

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	2220

A.10.2.2.1.1	Test Purpose and Environment	2220

A.10.2.2.1.2	Test Parameters	2220

A.10.2.2.1.3	Test Requirements	2224

A.10.3	Signalling characteristics	2224

A.10.3.1	Radio link monitoring	2224

A.10.3.1.1	Introduction	2224

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2225

A.10.3.1.2.1	Test purpose and environment	2225

A.10.3.1.2.2	Test requirements	2229

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2230

A.10.3.1.3.1	Test purpose and environment	2230

A.10.3.1.3.2	Test requirements	2235

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2235

A.10.3.1.4.1	Test purpose and environment	2235

A.10.3.1.4.2	Test requirements	2235

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2235

A.10.3.1.5.1	Test purpose and environment	2235

A.10.3.1.5.2	Test requirements	2235

A.10.3.2	Interruption	2235

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2235

A.10.3.2.1.1	Test Purpose and Environment	2235

A.10.3.2.1.2	Test Requirements	2240

A.10.3.3	SCell activation and deactivation delay	2240

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	2240

A.10.3.3.1.1	Test Purpose and Environment	2240

A.10.3.3.1.2	Test Requirements	2245

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 320 ms SCell measurement cycle	2245

A.10.3.3.2.1	Test Purpose and Environment	2245

A.10.3.3.2.2	Test Requirements	2246

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	2246

A.10.3.3.3.1	Test Purpose and Environment	2246

A.10.3.3.3.2	Test Requirements	2247

A.10.3.4	Beam failure detection and link recovery procedures	2247

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2247

A.10.3.4.1.1	Test Purpose and Environment	2247

A.10.3.4.1.2	Test Requirements	2252

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2252

A.10.3.4.2.1	Test Purpose and Environment	2252

A.10.3.4.2.2	Test Requirements	2258

A.10.3.5	Active BWP switching	2258

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2258

A.10.3.5.1.2	Test Requirements	2263

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2264

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2264

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2267

A.10.3.5.3	RRC-based Active BWP Switch	2271

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2271

A.10.3.6	PSCell addition and release delay	2275

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2275

A.10.3.6.1.1	Test purpose and environment	2275

A.10.3.6.1.2	Test Requirements	2279

A.10.3.7	Void	2280

A.10.4	Measurement procedure	2280

A.10.4.1	Intra-frequency measurements	2280

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2280

A.10.4.1.1.1	Test purpose and environment	2280

A.10.4.1.1.2	Test parameters	2280

A.10.4.1.1.3	Test Requirements	2280

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	2281

A.10.4.1.2.1	Test purpose and environment	2281

A.10.4.1.2.2	Test parameters	2281

A.10.4.1.2.3	Test Requirements	2281

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	2281

A.10.4.1.3.1	Test purpose and environment	2281

A.10.4.1.3.2	Test parameters	2281

A.10.4.1.3.3	Test Requirements	2281

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2282

A.10.4.1.4.1	Test purpose and environment	2282

A.10.4.1.4.2	Test parameters	2282

A.10.4.1.4.3	Test Requirements	2282

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2282

A.10.4.1.5.1	Test purpose and environment	2282

A.10.4.1.5.2	Test parameters	2282

A.10.4.1.5.3	Test Requirements	2282

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2283

A.10.4.1.6.1	Test purpose and environment	2283

A.10.4.1.6.2	Test parameters	2283

A.10.4.1.6.3	Test Requirements	2283

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2283

A.10.4.1.7.1	Test purpose and environment	2283

A.10.4.1.7.2	Test parameters	2283

A.10.4.1.7.3	Test Requirements	2283

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2284

A.10.4.1.8.1	Test purpose and environment	2284

A.10.4.1.8.2	Test parameters	2284

A.10.4.1.8.3	Test Requirements	2284

A.10.4.1.9	RSSI measurement reporting on PSCC	2284

A.10.4.1.9.1	Test purpose and environment	2284

A.10.4.1.9.2	Test parameters	2284

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2285

A.10.4.1.10.1	Test purpose and environment	2285

A.10.4.1.10.2	Test parameters	2285

A.10.4.1.11	RSSI measurement reporting on SCC	2285

A.10.4.1.11.1	Test purpose and environment	2285

A.10.4.1.11.2	Test parameters	2285

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2286

A.10.4.1.12.1	Test purpose and environment	2286

A.10.4.1.12.2	Test parameters	2286

A.10.4.2	Inter-frequency measurements	2286

A.10.4.2.1	RSSI measurement reporting	2286

A.10.4.2.1.1	Test purpose and environment	2286

A.10.4.2.1.2	Test parameters	2286

A.10.4.2.2	Channel occupancy measurement reporting	2287

A.10.4.2.2.1	Test purpose and environment	2287

A.10.4.2.2.2	Test parameters	2287

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2287

A.10.4.2.3.1	Test Purpose and Environment	2287

A.10.4.2.3.2	Test Requirements	2292

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2293

A.10.4.2.4.1	Test Purpose and Environment	2293

A.10.4.2.4.2	Test Requirements	2297

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2298

A.10.4.2.5.1	Test Purpose and Environment	2298

A.10.4.2.5.2	Test Requirements	2302

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2303

A.10.4.2.6.1	Test Purpose and Environment	2303

A.10.4.2.6.2	Test Requirements	2307

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2308

A.10.4.2.7.1	Test Purpose and Environment	2308

A.10.4.2.7.2	Test Requirements	2314

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2314

A.10.4.2.8.1	Test Purpose and Environment	2314

A.10.4.2.8.2	Test Requirements	2320

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2321

A.10.4.2.9.1	Test Purpose and Environment	2321

A.10.4.2.9.2	Test Requirements	2326

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2326

A.10.4.2.10.1	Test Purpose and Environment	2326

A.10.4.2.10.2	Test Requirements	2332

A.10.4.3	L1-RSRP measurements for beam reporting	2333

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2333

A.10.4.3.1.1	Test Purpose and Environment	2333

A.10.4.3.1.2	Test parameters	2333

A.10.4.3.1.3	Test Requirements	2335

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2336

A.10.4.3.2.1	Test Purpose and Environment	2336

A.10.4.3.2.2	Test parameters	2336

A.10.4.3.2.3	Test Requirements	2338

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2339

A.10.4.3.3.1	Test Purpose and Environment	2339

A.10.4.3.3.2	Test parameters	2339

A.10.4.3.3.3	Test Requirements	2342

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2343

A.10.4.3.4.1	Test Purpose and Environment	2343

A.10.4.3.4.2	Test parameters	2343

A.10.4.3.4.3	Test Requirements	2346

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2347

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2347

A.10.4.4.1.1	Test Purpose and Environment	2347

A.10.4.4.1.2	Test Requirements	2351

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2352

A.10.4.4.2.1	Test Purpose and Environment	2352

A.10.4.4.2.2	Test Requirements	2357

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2358

A.10.4.4.3.1	Test Purpose and Environment	2358

A.10.4.4.3.2	Test Requirements	2362

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2363

A.10.4.4.4.1	Test Purpose and Environment	2363

A.10.4.4.4.2	Test Requirements	2368

A.10.5	Measurement performance	2369

A.10.5.1	SS-RSRP	2369

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2369

A.10.5.1.1.1	Test Purpose and Environment	2369

A.10.5.1.1.2	Test parameters	2369

A.10.5.1.1.3	Test Requirements	2372

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2372

A.10.5.1.2.1	Test Purpose and Environment	2372

A.10.5.1.2.2	Test parameters	2372

A.10.5.1.2.3	Test Requirements	2375

A.10.5.2	SS-RSRQ	2375

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2375

A.10.5.2.1.1	Test Purpose and Environment	2375

A.10.5.2.1.2	Test Parameters	2375

A.10.5.2.1.3	Test Requirements	2378

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2378

A.10.5.2.2.1	Test Purpose and Environment	2378

A.10.5.2.2.2	Test Parameters	2378

A.10.5.2.2.3	Test Requirements	2381

A.10.5.3	SS-SINR	2381

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	2381

A.10.5.3.1.1	Test Purpose and Environment	2381

A.10.5.3.1.2	Test Parameters	2381

A.10.5.3.1.3	Test Requirements	2384

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	2384

A.10.5.3.2.1	Test Purpose and Environment	2384

A.10.5.3.2.2	Test Parameters	2384

A.10.5.3.2.3	Test Requirements	2387

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	2387

A.10.5.3.3.1	Test Purpose and Environment	2387

A.10.5.3.3.2	Test Parameters	2387

A.10.5.3.3.3	Test Requirements	2390

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2390

A.10.5.4.1	SSB based L1-RSRP measurement	2390

A.10.5.4.1.1	Test Purpose and Environment	2390

A.10.5.4.1.2	Test parameters	2391

A.10.5.4.1.3	Test Requirements	2393

A.10.5.5	RSSI	2393

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2393

A.10.5.5.1.1	Test Purpose and Environment	2393

A.10.5.5.1.2	Test parameters	2393

A.10.5.5.1.3	Test Requirements	2396

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2396

A.10.5.5.2.1	Test Purpose and Environment	2396

A.10.5.5.2.2	Test parameters	2396

A.10.5.5.2.3	Test Requirements	2399

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2399

A.10.5.5.3.1	Test Purpose and Environment	2399

A.10.5.5.3.2	Test parameters	2399

A.10.5.5.3.3	Test Requirements	2402

A.10.5.6	Channel occupancy	2402

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2402

A.10.5.6.1.1	Test Purpose and Environment	2402

A.10.5.6.1.2	Test parameters	2402

A.10.5.6.1.3	Test Requirements	2406

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2406

A.10.5.6.2.1	Test Purpose and Environment	2406

A.10.5.6.2.2	Test parameters	2406

A.10.5.6.2.3	Test Requirements	2409

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2409

A.10.5.6.3.1	Test Purpose and Environment	2409

A.10.5.6.3.2	Test parameters	2409

A.10.5.6.3.3	Test Requirements	2413

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2413

A.11.1	RRC_IDLE state mobility	2413

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2413

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2413

A.11.1.1.1.1	Test Purpose and Environment	2413

A.11.1.1.1.2	Test Parameters	2413

A.11.1.1.1.3	Test Requirements	2416

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2416

A.11.1.1.2.1	Test Purpose and Environment	2416

A.11.1.1.2.2	Test Parameters	2416

A.11.1.1.2.3	Test Requirements	2420

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2420

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2420

A.11.1.2.1.1	Test Purpose and Environment	2420

A.11.1.2.1.2	Test Parameters	2421

A.11.1.2.1.3	Test Requirements	2427

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2428

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2428

A.11.1.3.1.1	Test Purpose and Environment	2428

A.11.1.3.1.2	Test Parameters	2428

A.11.1.3.1.3	Test Requirements	2434

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2435

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2435

A.11.1.4.1.1	Test Purpose and Environment	2435

A.11.1.4.1.2	Test Parameters	2435

A.11.1.4.1.3	Test Requirements	2438

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2439

A.11.1.4.2.1	Test Purpose and Environment	2439

A.11.1.4.2.2	Test Requirements	2442

A.11.2	RRC_CONNECTED state mobility	2443

A.11.2.1	Handover	2443

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2443

A.11.2.1.1.1	Test Purpose and Environment	2443

A.11.2.1.1.2	Test Parameters	2443

A.11.2.1.1.3 Test Requirements	2446

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2446

A.11.2.1.2.1	Test Purpose and Environment	2446

A.11.2.1.2.2	Test Parameters	2446

A.11.2.1.2.3	Test Requirements	2449

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2449

A.11.2.1.3.1	Test Purpose and Environment	2449

A.11.2.1.3.2	Test Parameters	2449

A.11.2.1.3.3 Test Requirements	2452

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2452

A.11.2.1.4.1	Test Purpose and Environment	2452

A.11.2.1.4.2	Test Parameters	2452

A.11.2.1.4.3 Test Requirements	2456

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2457

A.11.2.1.5.1	Test Purpose and Environment	2457

A.11.2.1.5.2	Test Parameters	2457

A.11.2.1.5.3 Test Requirements	2460

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2461

A.11.2.1.6.1	Test Purpose and Environment	2461

A.11.2.1.6.2	Test Parameters	2461

A.11.2.1.6.3 Test Requirements	2464

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2465

A.11.2.1.7.1	Test Purpose and Environment	2465

A.11.2.1.7.2	Test Requirements	2471

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2471

A.11.2.1.8.1	Test Purpose and Environment	2471

A.11.2.1.8.2	Test Requirements	2476

A.11.2.2	RRC connection mobility control	2476

A.11.2.2.1	RRC re-establishment	2476

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2476

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2479

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2486

A.11.2.2.2	Random Access	2491

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2491

A.11.2.2.2.1.1	Test Purpose and Environment	2491

A.11.2.2.2.1.2	Test Requirements	2494

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2494

A.11.2.2.2.1.2.2	Random Access Response Reception	2494

A.11.2.2.2.1.2.3	No Random Access Response Reception	2495

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2495

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2495

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2496

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2496

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2496

A.11.2.2.2.2.1	Test Purpose and Environment	2496

A.11.2.2.2.2.2	Test Requirements	2499

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2499

A.11.2.2.2.2.2.2	Random Access Response Reception	2500

A.11.2.2.2.2.2.3	No Random Access Response Reception	2500

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2500

A.11.2.2.2.3.1	Test Purpose and Environment	2500

A.11.2.2.2.3.2	Test Requirements	2504

A.11.2.2.2.3.2.1	MsgA Transmission	2504

A.11.2.2.2.3.2.2	MsgB Reception	2504

A.11.2.2.2.3.2.3	No MsgB Reception	2505

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2505

A.11.2.2.2.4.1	Test Purpose and Environment	2505

A.11.2.2.2.4.2	Test Requirements	2509

A.11.2.2.2.4.2.1	MsgA Transmission	2509

A.11.2.2.2.4.2.2	MsgB Reception	2510

A.11.2.2.2.4.2.3	No MsgB Reception	2510

A.11.2.2.3	RRC connection release with redirection	2511

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2511

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	2515

A.11.3	Timing	2520

A.11.3.1	UE transmit timing	2520

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	2520

A.11.3.1.1.1	Test Purpose and environment	2520

A.11.3.1.1.2	Test requirements	2523

A.11.3.2	UE timing advance	2524

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2524

A.11.3.2.1.1	Test Purpose and Environment	2524

A.11.3.2.1.2	Test Parameters	2524

A.11.3.2.1.3	Test Requirements	2528

A.11.4	Signalling characteristics	2528

A.11.4.1	Radio link monitoring	2528

A.11.4.1.1	Introduction	2528

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2529

A.11.4.1.2.1	Test purpose and environment	2529

A.11.4.1.2.2	Test requirements	2533

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2533

A.11.4.1.3.1	Test purpose and environment	2533

A.11.4.1.3.2	Test requirements	2539

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	2539

A.11.4.1.4.1	Test purpose and environment	2539

A.11.4.1.4.2	Test requirements	2539

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	2539

A.11.4.1.5.1	Test purpose and environment	2539

A.11.4.1.5.2	Test requirements	2539

A.11.4.2	Interruption	2540

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	2540

A.11.4.2.1.1 Test Purpose and Environment	2540

A.11.4.2.1.2	Test Requirements	2544

A.11.4.3	SCell activation and deactivation delay	2544

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	2544

A.11.4.3.1.1	Test Purpose and Environment	2544

A.11.4.3.1.2	Test Requirements	2549

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 320 ms SCell measurement cycle	2549

A.11.4.3.2.1	Test Purpose and Environment	2549

A.11.4.3.2.2	Test Requirements	2550

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	2550

A.11.4.3.3.1	Test Purpose and Environment	2550

A.11.4.3.3.2	Test Requirements	2551

A.11.4.4	Beam failure detection and link recovery procedures	2551

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2551

A.11.4.4.1.1	Test Purpose and Environment	2551

A.11.4.4.1.2	Test Requirements	2556

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2556

A.11.4.4.2.1	Test Purpose and Environment	2556

A.11.4.4.2.2	Test Requirements	2562

A.11.4.5	Active BWP switching	2562

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	2562

A.11.4.5.1.1	Test Purpose and Environment	2562

A.11.4.5.1.2	Test Requirements	2566

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	2567

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	2567

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	2570

A.11.4.5.3	RRC-based Active BWP Switch	2573

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2573

A.11.4.6	Void	2576

A.11.5	Measurement procedure	2576

A.11.5.1	Intra-frequency measurements	2576

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	2576

A.11.5.1.1.1	Test purpose and environment	2576

A.11.5.1.1.2	Test parameters	2576

A.11.5.1.1.3	Test Requirements	2577

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	2577

A.11.5.1.2.1	Test purpose and environment	2577

A.11.5.1.2.2	Test parameters	2577

A.11.5.1.2.3	Test Requirements	2577

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	2578

A.11.5.1.3.1	Test purpose and environment	2578

A.11.5.1.3.2	Test parameters	2578

A.11.5.1.3.3	Test Requirements	2578

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	2578

A.11.5.1.4.1	Test purpose and environment	2578

A.11.5.1.4.2	Test parameters	2578

A.11.5.1.4.3	Test Requirements	2578

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2579

A.11.5.1.5.1	Test purpose and environment	2579

A.11.5.1.5.2	Test parameters	2579

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2579

A.11.5.1.6.1	Test purpose and environment	2579

A.11.5.1.6.2	Test parameters	2579

A.11.5.1.6.3	Test Requirements	2579

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2580

A.11.5.1.7.1	Test purpose and environment	2580

A.11.5.1.7.2	Test parameters	2580

A.11.5.1.7.3	Test Requirements	2580

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2580

A.11.5.1.8.1	Test purpose and environment	2580

A.11.5.1.8.2	Test parameters	2580

A.11.5.1.8.3	Test Requirements	2580

A.11.5.1.9	RSSI measurement reporting on PCC	2581

A.11.5.1.9.1	Test purpose and environment	2581

A.11.5.1.9.2	Test parameters	2581

A.11.5.1.10	Channel occupancy measurement reporting on PCC	2581

A.11.5.1.10.1	Test purpose and environment	2581

A.11.5.1.10.2	Test parameters	2581

A.11.5.1.11	RSSI measurement reporting on SCC	2581

A.11.5.1.11.1	Test purpose and environment	2581

A.11.5.1.11.2	Test parameters	2582

A.11.5.1.12	Channel occupancy measurement reporting on SCC	2582

A.11.5.1.12.1	Test purpose and environment	2582

A.11.5.1.12.2	Test parameters	2582

A.11.5.2	Inter-frequency measurements	2582

A.11.5.2.1	RSSI measurement reporting	2582

A.11.5.2.1.1	Test purpose and environment	2582

A.11.5.2.1.2	Test parameters	2583

A.11.5.2.2	Channel occupancy measurement reporting	2583

A.11.5.2.2.1	Test purpose and environment	2583

A.11.5.2.2.2	Test parameters	2583

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2583

A.11.5.2.3.1	Test Purpose and Environment	2583

A.11.5.2.3.2	Test Requirements	2587

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2588

A.11.5.2.4.1	Test Purpose and Environment	2588

A.11.5.2.4.2	Test Requirements	2593

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2593

A.11.5.2.5.1	Test Purpose and Environment	2593

A.11.5.2.5.2	Test Requirements	2597

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2598

A.11.5.2.6.1	Test Purpose and Environment	2598

A.11.5.2.6.2	Test Requirements	2603

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2603

A.11.5.2.7.1	Test Purpose and Environment	2603

A.11.5.2.7.2	Test Requirements	2607

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2608

A.11.5.2.8.1	Test Purpose and Environment	2608

A.11.5.2.8.2	Test Requirements	2613

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2613

A.11.5.2.9.1	Test Purpose and Environment	2613

A.11.5.2.9.2	Test Requirements	2618

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2619

A.11.5.2.10.1	Test Purpose and Environment	2619

A.11.5.2.10.2	Test Requirements	2624

A.11.5.3	Inter-RAT E-UTRAN measurements	2625

A.11.5.4	L1-RSRP measurements for beam reporting	2625

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	2625

A.11.5.4.1.1	Test Purpose and Environment	2625

A.11.5.4.1.2	Test parameters	2625

A.11.5.4.1.3	Test Requirements	2627

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	2627

A.11.5.4.2.1	Test Purpose and Environment	2627

A.11.5.4.2.2	Test parameters	2628

A.11.5.4.2.3	Test Requirements	2630

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2630

A.11.5.4.3.1	Test Purpose and Environment	2630

A.11.5.4.3.2	Test parameters	2631

A.11.5.4.3.3	Test Requirements	2634

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	2635

A.11.5.4.4.1	Test Purpose and Environment	2635

A.11.5.4.4.2	Test parameters	2635

A.11.5.4.4.3	Test Requirements	2638

A.11.6	Measurement performance	2639

A.11.6.1	SS-RSRP	2639

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2639

A.11.6.1.1.1	Test Purpose and Environment	2639

A.11.6.1.1.2	Test parameters	2639

A.11.6.1.1.3	Test Requirements	2641

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2641

A.11.6.1.2.1	Test Purpose and Environment	2641

A.11.6.1.2.2	Test parameters	2641

A.11.6.1.2.3	Test Requirements	2643

A.11.6.2	SS-RSRQ	2643

A.11.6.2.1	Intra-frequency measurement accuracy	2643

A.11.6.2.1.1	Test Purpose and Environment	2643

A.11.6.2.1.2	Test Parameters	2643

A.11.6.2.1.3	Test Requirements	2646

A.11.6.2.2	Inter-frequency measurement accuracy	2646

A.11.6.2.2.1	Test Purpose and Environment	2646

A.11.6.2.2.2	Test Parameters	2646

A.11.6.2.2.3	Test Requirements	2649

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	2649

A.11.6.2.3.1	Test Purpose and Environment	2649

A.11.6.2.3.2	Test Parameters	2649

A.11.6.2.3.3	Test Requirements	2652

A.11.6.2.4	Inter-frequency measurement accuracy	2652

A.11.6.2.4.1	Test Purpose and Environment	2652

A.11.6.2.4.2	Test Parameters	2652

A.11.6.2.4.3	Test Requirements	2661

A.11.6.3	SS-SINR	2661

A.11.6.3.1	Intra-frequency measurement accuracy	2661

A.11.6.3.1.1	Test Purpose and Environment	2661

A.11.6.3.1.2	Test Parameters	2661

A.11.6.3.1.3	Test Requirements	2664

A.11.6.3.2	Inter-frequency measurement accuracy	2664

A.11.6.3.2.1	Test Purpose and Environment	2664

A.11.6.3.2.2	Test Parameters	2664

A.11.6.3.2.3	Test Requirements	2667

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	2667

A.11.6.3.3.1	Test Purpose and Environment	2667

A.11.6.3.3.2	Test Parameters	2667

A.11.6.3.3.3	Test Requirements	2670

A.11.6.3.4	Inter-frequency measurement accuracy	2670

A.11.6.3.4.1	Test Purpose and Environment	2670

A.11.6.3.4.2	Test Parameters	2670

A.11.6.3.4.3	Test Requirements	2678

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	2678

A.11.6.4.1	SSB based L1-RSRP measurement	2678

A.11.6.4.1.1	Test Purpose and Environment	2678

A.11.6.4.1.2	Test parameters	2679

A.11.6.4.1.3	Test Requirements	2682

A.11.6.5	RSSI	2682

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	2682

A.11.6.5.1.1	Test Purpose and Environment	2682

A.11.6.5.1.2	Test parameters	2682

A.11.6.5.1.3	Test Requirements	2685

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	2685

A.11.6.5.2.1	Test Purpose and Environment	2685

A.11.6.5.2.2	Test parameters	2685

A.11.6.5.2.3	Test Requirements	2688

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2688

A.11.6.5.3.1	Test Purpose and Environment	2688

A.11.6.5.3.2	Test parameters	2688

A.11.6.5.3.3	Test Requirements	2691

A.11.6.6	Channel occupancy	2691

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	2691

A.11.6.6.1.1	Test Purpose and Environment	2691

A.11.6.6.1.2	Test parameters	2691

A.11.6.6.1.3	Test Requirements	2694

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2694

A.11.6.6.2.1	Test Purpose and Environment	2694

A.11.6.6.2.2	Test parameters	2694

A.11.6.6.2.3	Test Requirements	2697

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2697

A.11.6.6.3.1	Test Purpose and Environment	2697

A.11.6.6.3.2	Test parameters	2697

A.11.6.6.3.3	Test Requirements	2700

A.11.6.7	E-UTRAN RSRP	2700

A.11.6.8	E-UTRAN RSRQ	2700

A.11.6.9	E-UTRAN SINR	2700

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	2700

A.12.1	RRC_IDLE state mobility	2700

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	2700

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	2700

A.12.1.1.1.1	Test Purpose and Environment	2700

A.12.1.1.1.2	Test Requirements	2705

A.12.2	RRC_CONNECTED state mobility	2706

A.12.2.1	Handover	2706

A.12.2.1.1	E-UTRAN - NR with CCA handover	2706

A.12.2.1.1.1	Test Purpose and Environment	2706

A.12.2.1.1.2	Test Requirements	2711

A.12.3	Signalling characteristics	2712

A.12.3.1	Interruptions	2712

A.12.4	Measurement procedure	2712

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	2712

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	2712

A.12.4.1.1.1	Test Purpose and Environment	2712

A.12.4.1.1.2	Test Requirements	2716

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2716

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2716

A.12.4.2.1.1	Test Purpose and Environment	2716

A.12.4.2.1.2	Test Requirements	2720

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2721

A.12.4.2.2.1	Test Purpose and Environment	2721

A.12.4.2.2.2	Test Requirements	2725

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2725

A.12.4.2.3.1	Test Purpose and Environment	2725

A.12.4.2.3.2	Test Requirements	2729

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2730

A.12.4.2.4.1	Test Purpose and Environment	2730

A.12.4.2.4.2	Test Requirements	2734

A.12.4.2.5	RSSI measurement reporting	2734

A.12.4.2.5.1	Test purpose and environment	2734

A.12.4.2.5.2	Test parameters	2734

A.12.4.2.6	Channel occupancy measurement reporting	2735

A.12.4.2.6.1	Test purpose and environment	2735

A.12.4.2.6.2	Test parameters	2735

A.12.5	Measurement performance	2735

A.12.5.1	E-UTRANNR SFTD	2735

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	2735

A.12.5.1.1.1	Test Purpose	2735

A.12.5.1.1.2	Test Environment	2736

A.12.5.1.1.3	Test Requirements	2741

A.12.5.2	E-UTRANNR SS-RSRP	2741

A.12.5.3	E-UTRANNR SS-RSRQ	2741

A.12.5.4	E-UTRANNR SS-SINR	2741

A.12.5.5	E-UTRANNR RSSI	2741

A.12.5.6	E-UTRANNR channel occupancy	2741

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	2741

A.13.1	Timing	2741

A.13.1.1	UE transmit timing	2741

A.13.1.2	Timing advance	2741

A.13.2	Signalling characteristics	2741

A.13.2.1	Interruption	2741

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	2741

A.13.2.1.1.1	Test Purpose and Environment	2741

A.13.2.1.1.2	Test Requirements	2745

A.13.2.2	SCell activation and deactivation delay	2745

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	2745

A.13.2.2.1.1	Test Purpose and Environment	2745

A.13.2.2.1.2	Test Requirements	2750

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 320 ms SCell measurement cycle	2750

A.13.2.2.2.1	Test Purpose and Environment	2750

A.13.2.2.2.2	Test Requirements	2751

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	2751

A.13.2.2.3.1	Test Purpose and Environment	2751

A.13.2.2.3.2	Test Requirements	2752

A.13.2.3	Void	2752

A.13.3	Measurement procedure	2752

A.13.3.1	Intra-frequency measurements	2752

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	2752

A.13.3.1.1.1	Test purpose and environment	2752

A.13.3.1.1.2	Test parameters	2752

A.13.3.1.1.3	Test Requirements	2757

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	2757

A.13.3.1.2.1	Test purpose and environment	2757

A.13.3.1.2.2	Test parameters	2757

A.13.3.1.2.3	Test Requirements	2761

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2761

A.13.3.1.3.1	Test purpose and environment	2761

A.13.3.1.3.2	Test parameters	2761

A.13.3.1.3.3	Test Requirements	2766

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2766

A.13.3.1.4.1	Test purpose and environment	2766

A.13.3.1.4.2	Test parameters	2766

A.13.3.1.4.3	Test Requirements	2771

A.13.3.1.5	RSSI measurement reporting on SCC	2771

A.13.3.1.5.1	Test purpose and environment	2771

A.13.3.1.5.2	Test parameters	2771

A.13.3.1.6	Channel occupancy measurement reporting on SCC	2772

A.13.3.1.6.1	Test purpose and environment	2772

A.13.3.1.6.2	Test parameters	2773

A.13.3.2	Inter-frequency measurements	2773

A.13.3.2.1	RSSI measurement reporting	2773

A.13.3.2.1.1	Test purpose and environment	2773

A.13.3.2.1.2	Test parameters	2773

A.13.3.2.2	Channel occupancy measurement reporting	2774

A.13.3.2.2.1	Test purpose and environment	2774

A.13.3.2.2.2	Test parameters	2774

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2774

A.13.3.2.3.1	Test Purpose and Environment	2774

A.13.3.2.3.2	Test Requirements	2779

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2779

A.13.3.2.4.1	Test Purpose and Environment	2779

A.13.3.2.4.2	Test Requirements	2784

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2785

A.13.3.2.5.1	Test Purpose and Environment	2785

A.13.3.2.5.2	Test Requirements	2790

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2790

A.13.3.2.6.1	Test Purpose and Environment	2790

A.13.3.2.6.2	Test Requirements	2796

A.13.3.3	L1-RSRP measurements for beam reporting	2797

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	2797

A.13.3.3.1.1	Test Purpose and Environment	2797

A.13.3.3.1.2	Test parameters	2797

A.13.3.3.1.3	Test Requirements	2801

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	2801

A.13.3.3.2.1	Test Purpose and Environment	2801

A.13.3.3.2.2	Test parameters	2801

A.13.3.3.2.3	Test Requirements	2805

A.13.4	Measurement performance	2805

A.13.4.1	SS-RSRP	2805

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2805

A.13.4.1.1.1	Test Purpose and Environment	2805

A.13.4.1.1.2	Test parameters	2805

A.13.4.1.1.3	Test Requirements	2807

A.13.4.2	SS-RSRQ	2807

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	2807

A.13.4.2.1.1	Test Purpose and Environment	2807

A.13.4.2.1.2	Test Parameters	2807

A.13.4.2.1.3	Test Requirements	2815

A.13.4.3	SS-SINR	2815

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	2815

A.13.4.3.1.1	Test Purpose and Environment	2815

A.13.4.3.1.2	Test Parameters	2815

A.13.4.3.1.3	Test Requirements	2821

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	2822

A.13.4.4.1	SSB based L1-RSRP measurement	2822

A.13.4.4.1.1	Test Purpose and Environment	2822

A.13.4.4.1.2	Test parameters	2822

A.13.4.4.1.3	Test Requirements	2826

A.13.4.5	RSSI	2826

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	2826

A.13.4.5.1.1	Test Purpose and Environment	2826

A.13.4.5.1.2	Test parameters	2826

A.13.4.5.1.3	Test Requirements	2829

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2829

A.13.4.5.2.1	Test Purpose and Environment	2829

A.13.4.5.2.2	Test parameters	2829

A.13.4.5.2.3	Test Requirements	2833

A.13.4.6	Channel occupancy	2833

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2833

A.13.4.6.1.1	Test Purpose and Environment	2833

A.13.4.6.1.2	Test parameters	2833

A.13.4.6.1.3	Test Requirements	2837

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2837

A.13.4.6.2.1	Test Purpose and Environment	2837

A.13.4.6.2.2	Test parameters	2837

A.13.4.6.2.3	Test Requirements	2841

Annex B (normative): Conditions for RRM requirements applicability for operating bands	1787

B.1	Conditions for NR RRC_IDLE state mobility	1787

B.1.1	Introduction	1787

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1787

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1788

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1788

B.2.1	Introduction	1788

B.2.1.1	General	1788

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1789

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1789

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1789

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1790

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	1790

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	1790

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	1790

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	1791

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	1791

B.2.2	Conditions for NR intra-frequency measurements	1792

B.2.3	Conditions for NR inter-frequency measurements	1793

B.2.4	Conditions for NR L1-RSRP reporting	1794

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1794

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1795

B.2.5	Conditions for RRC connection release with redirection to NR	1796

B.2.6	Void	1798

B.2.6.1	Void	1798

B.2.6.2	Void	1798

B.2.8	Conditions for NR L1-SINR reporting	1799

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1799

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	1800

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	1800

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	1800

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1801

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	1801

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	1802

B.2.9	Conditions for NR intra-frequency measurements under CCA	1804

B.2.10	Conditions for NR inter-frequency measurements under CCA	1804

B.2.11	Conditions for NR L1-RSRP reporting under CCA	1804

B.2.11.1	Conditions for SSB based L1-RSRP reporting	1804

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	1805

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	1806

B.2.14	Conditions for NR PRS-based measurements	1808

B.3	RRM Requirements Exceptions	1809

B.3.1	Introduction	1809

B.3.2	Receiver sensitivity relaxation for CA	1809

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1809

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1810

B.3.2.2.1	Inter-band carrier aggregation	1810

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1810

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1810

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1810

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1810

B.3.2.4.1	Intra-band contiguous carrier aggregation	1810

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1810

B.3.3	Receiver sensitivity relaxation for DC	1811

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1811

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1811

B.3.4	Receiver sensitivity relaxation for SUL	1811

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1811

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1811

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1811

B.4	Conditions for V2X	1811

B.4.1	Test parameters for GNSS signals	1811

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	1812

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	1812

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	1812

Annex C (informative): Change history	1814