3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112341

# Electronic Meeting, Aug. 16-27, 2021

Source:	LG Electronics

Title:	TP on MPR for NR V2X intra-band con-current operation with Uu

Agenda item:	9.15.5.4

Document for:	Approval

Introduction

With the agreed WF [1], the corresponding TP for TR 38.785 is derived as below.

- Reference

[1] R4-2107870, “WF on MPR for intra-band V2X con-current operation”, LG Electronics

< START OF TP >

### 5.2.4	Intra-band V2X con-current UE RF requirements with adjacent channel for FDM operation

#### 5.2.4.1 Tx requirements for NR intra-band V2X con-current operation with adjacent channel

##### 5.2.4.1.1	Maximum output power

The following NR intra-band V2X con-current operating UE’ Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 5.2.4.1.1-1: intra-band V2X contiguous con-current V2X UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| V2X_n79B |  |  | 26 | ±2 | 23 | ±2 |  |  |
| NOTE 1: The intra-band contiguous con-current NR V2X operating band is used for NR V2X and NR Uu Service. Bandwidth class B is that the aggregated channel bandwidth 20 MHz ≤ BWChannel_SL&UL ≤ 100 MHz. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

##### 5.2.4.1.2	UE maximum output power reduction

The following assumption can serve as a starting point for MPR simulation assumptions as shown in Table 5.2.4.1.2-1 and Table 5.2.4.1.2-2.

Table 5.2.4.1.2-1: MPR simulation assumption for NR V2X intra-band contiguous con-current operation

| parameter | Assumption |
| --- | --- |
| center frequency | 4.5GHz (n79) |
| Bandwidth | 10/20/30/40MHz for NR SL 10/20/40/60/80MHz for NR Uu |
| Maximum output power | 26 dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM for NR SL CP-OFDM or DFT-S-OFDM for NR Uu |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. |

For NR V2X intra-band contiguous con-current operation, simultaneous transmission of PUSCH/PUCCH and NR V2X’s PSCCH and PSSCH in the same sub-frame is supported. The following constraints in Table 5.2.4.1.2-2 can be assumed based on current RAN1’s agreement.

Table 5.2.4.1.2-2: NR V2X intra-band contiguous con-current operation UE’ MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes for NR SL | • Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation for NR SL | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216. No restriction of LCRB for NR Uu |
| Regarding PSCCH / PSSCH multiplexing for NR SL |  |
| PSCCH size for NR SL | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH for NR SL | 0dB |
| NOTE 1: If there is not indicate the detail parameters, then follow TR38.886 for NR SL operation. Also follow TS38.211/TS38.212/TS38.101-1 for NR Uu operation. |  |

For simultaneous transmission of PSFCH transmission for V2X UE and PUSCH/PUCCH for NR Uu, RAN4 need further discussion how to apply the MPR requirements.

For simultaneous transmission of S-SSB transmission for V2X UE and PUSCH/PUCCH for NR Uu, RAN4 need further discussion how to apply the MPR requirements.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with contiguous RB allocation is specified in Table 5.2.4.1.2-3 for UE power class 3 with bandwidth class B.

Table 5.2.4.1.2-3: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] |
|  | 256QAM | ≤ [1.5] | ≤ [1.5] |

For bandwidth class B with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for SL CC1, RBStart2, LCRB2, and NRB2 are for UL CC2. SL CC1 is the component carrier with lower frequency.

In contiguous NR V2X intra-band con-current operation, a contiguous allocation is an inner allocation if

RBStart,Low  ≤  RBStart_SL&UL  ≤  RBStart,High, and NRB_alloc  ≤  ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2))

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= LCRB1 ∙ 2µ1 + LCRB2 ∙ 2µ2

NRB_alloc= (NRB1 - RBStart1)∙ 2µ1 + (RBStart2 + LCRB2 ) ∙ 2µ2,

NRB,agg=NRB1∙2µ1+ NRB2∙2µ2.

If LCRB1 =0, RBStart_SL&UL = NRB1∙2µ1+ RBStart2∙2µ2,

if LCRB1 > 0, RBStart_SL&UL = RBStart1∙2µ1.

Where, µ1 and µ2 is 0, 1 and 2 for SCS of 15kHz, 30kHz and 60kHz respectively.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with non-contiguous RB allocation is specified in Table 5.2.4.1.2-4 for UE power class 3 with bandwidth class B.

Table 5.2.4.1.2-4: Non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 256QAM | ≤ [1.5] | ≤ [2.0] | ≤ [3.0] |

For bandwidth classes B with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0, when both SL CC and UL CC are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for SL CC1, RBStart2, LCRB2, and NRB2 are for UL CC2. SL CC1 is the component carrier with lower frequency.

In contiguous NR V2X intra-band con-current operation, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((BWChannel_SL&UL / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2µ1 + (RBStart2 + LCRB2 ) ∙ 2µ2, RBStart_SL&UL = RBStart1∙21

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_SL&UL – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of SL CC1 and UL CC2 respectively.

In contiguous NR V2X intra-band con-current operation, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_SL&UL  ≤  RBStart,High and NRB_alloc ≤  ceil((3 BWChannel_SL&UL / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_SL&UL – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_SL&UL – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc , RBStart_SL&UL , BWgap and BWGB,low are as defined for the Inner region.

In contiguous NR V2X intra-band con-current operation, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with contiguous RB allocation is specified in Table 5.2.4.1.2-5 for UE power class 2 with bandwidth class B.

Table 5.2.4.1.2-5: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [3.5] |
|  | 16QAM | ≤ [2.0] | ≤ [3.5] |
|  | 64QAM | ≤ [3.0] | ≤ [3.5] |
|  | 256QAM | ≤ [5.0] | ≤ [5.0] |

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with non-contiguous RB allocation is specified in Table 5.2.4.1.2-6 for UE power class 2 with bandwidth class B.

Table 5.2.4.1.2-6: Non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [4.0] | ≤ [6.0] |
|  | 16QAM | ≤ [2.5] | ≤ [4.0] | ≤ [6.0] |
|  | 64QAM | ≤ [3.5] | ≤ [4.5] | ≤ [6.0] |
|  | 256QAM | ≤ [4.5] | ≤ [5.0] | ≤ [6.0] |

< END OF TP >