3GPP TSG-RAN WG4 Meeting #100-e	R4-2113461

Electronic Meeting, 16 August – 27 August 2021

Agenda Item:	6.1.1.5.5

Source:	Ericsson

Title:	Availability of SSB monitoring occasions

Document for:	Discussion

# 1	Introduction

RAN4#99-e agreed with the way forward on the availability of SSB monitoring occasions for CBD [1], and the remaining open issue is whether the same rules are applicable for RLM/BFD/L1-RSRP.

Availability of SSB occasions for CBD - For CBD, the UE, which is configured in DRX, is not required to determine the availability of SSB occasions more frequent than: - Once per Max(25ms, P* TSSB) if TDRX ≤ 320ms - Once per P* TDRX if TDRX > 320ms. Availability of SSB occasions for RLM/BFD/L1-RSRP - FFS: whether to consider P factor for RLM/BFD/L1-RSRP

This contribution discusses the remaining open issue for the availability of SSB occasions.

# 2	Discussion

Table 1, Table 2, Table 3, and Table 4 show the evaluation periods of RLM, BFD, CBD, and L1-RSRP for NR-U. According to the agreements so far [1], the following evaluation periods depend on the number of available SSB occasions:

- RLM Qin

- CBD

- L1-RSRP

This means RLM Qout and BFD evaluation periods do not depend on the available SSB occasions.

Observation 1: Evaluation periods of RLM Qin, CBD, and L1-RSRP depend on the available SSB occasions.

Table 1	Evaluation period of RLM Qin/Qout for NR-U (TS38.133 Table 8.1A.2.2-1).

| Configuration | TEvaluate_out_SSB,CCA (ms) |  | TEvaluate_in_SSB,CCA (ms) |
| --- | --- | --- | --- |
|  | RLM-RS SSB Es/IotNote4 ≥-7 dB | RLM-RS SSB Es/Iot Note4 <-7 dB |  |
| no DRX | Max(200, Ceil(17*P)*TSSB) | Max(200, Ceil(24*P)*TSSB) | Max(100, Ceil((5+Lin)*P)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*15*P)*Max(TDRX,TSSB)) | Max(200, Ceil(1.5*20*P)*Max(TDRX,TSSB)) | Max(100, Ceil(1.5*(5+Lin)*P)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil(13*P)*TDRX | Ceil(16*P)*TDRX | Ceil((5+Lin)*P)*TDRX |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: Lin is the number of RLM-RS SSB occasions which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. [The UE is not required to determine the availability of SSB occasions more frequent than once per DRX cycle length, when configured with DRX.] NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. NOTE 4: RLM-RS SSB Es/Iot is the averaged Es/Iot over the most recent previous out-of-sync evaluation period. |  |  |  |

Table 2	Evaluation period of BFD for NR-U (TS 38.133 Table 8.5A.2.2-1).

| Configuration | TEvaluate_BFD_SSB_CCA (ms) |  |
| --- | --- | --- |
|  | BFD-RS SSB Es/Iot Note2 ≥ -7 dB | BFD-RS SSB Es/Iot Note2 < -7 dB |
| no DRX | Max(50, Ceil((10 P) TSSB)) | Max(50, Ceil((12 P) TSSB)) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 8 P) Max(TDRX,TSSB)) | Max(50, Ceil(1.5 10 P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(7 P) TDRX | Ceil(8 P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length. Note 2: BFD-RS SSB Es/Iot is the averaged BFD-RS SSB Es/Iot over the most recent previous evaluation period. |  |  |

Table 3	Evaluation period of CBD for NR-U (TS 38.133 8.5A.5.2-1).

| Configuration | TEvaluate_CBD_SSB_CCA (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil((3 + LCBD) P) TSSB) |
| DRX cycle > 320ms | Ceil((3 + LCBD) P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q1. TDRX is the DRX cycle length. Note 2: LCBD is the number of CBD-RS SSB occasions not available at the UE during TEvaluate_CBD_SSB_CCA where LCBD ≤ LCBD,max. [The UE is not required to determine the availability of SSB occasions more frequent than once per DRX cycle length, when configured with DRX.] Note 3: LCBD,max=7 for Max(TDRX, TSSB) ≤ 40 assuming TDRX=0 for non-DRX, LCBD,max=5 for 40 < Max(TDRX, TSSB) ≤ 320, LCBD,max=3 for TDRX > 320. Note 4 If LCBD>LCBD,max, the UE shall assume no new candidate beams are found for this evaluation period. |  |

Table 4	Evaluation period of L1-RSRP for NR-U (TS 38.133 Table 9.5A.4.1-1).

| Configuration | TL1-RSRP_Measurement_Period_SSB_CCA (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil((M+L1)*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: L1=0 if higher layer parameter timeRestrictionForChannelMeasurement is configured. Otherwise L1 is the number of SSBs not available at the UE during TL1-RSRP_Measurement_Period_SSB_CCA where L1 ≤ L1max. Note 3: L1max =7 for Max(TDRX,TSSB) ≤ 40ms assuming TDRX=0 for non-DRX, L1max =5 for 40ms < Max(TDRX, TSSB) ≤ 320ms, L1max =3 for TDRX > 320ms. |  |

In [1], RAN4 agreed that UE is not required to determine the availability of SSB occasions for CBD evaluation more frequent than:

- Once per Max(25ms, P* TSSB) if TDRX ≤ 320ms

- Once per P* TDRX if TDRX > 320ms.

From the consistency of requirements, we propose to specify the monitoring of SSB availability for RLM Qin and L1-RSRP as follows:

Proposal 1: For RLM Qin, the UE, which is configured in DRX, is not required to determine the availability of SSB occasions more frequent than:

- Once per Max(100ms, 1.5 x P x Max(TDRX, TSSB)) if TDRX ≤ 320ms

- Once per P x TDRX if TDRX > 320ms

Proposal 2: For L1-RSRP measurement, the UE, which is configured in DRX, is not required to determine the availability of SSB occasions more frequent than:

- Once per Max(TReport, 1.5 x P x Max(TDRX, TSSB)) if TDRX ≤ 320ms

- Once per P x TDRX if TDRX > 320ms

# 3	Summary

Observation 1: Evaluation periods of RLM Qin, CBD, and L1-RSRP depend on the available SSB occasions.

Proposal 1: For RLM Qin, the UE, which is configured in DRX, is not required to determine the availability of SSB occasions more frequent than:

- Once per Max(100ms, 1.5 x P x Max(TDRX, TSSB)) if TDRX ≤ 320ms

- Once per P x TDRX if TDRX > 320ms

Proposal 2: For L1-RSRP measurement, the UE, which is configured in DRX, is not required to determine the availability of SSB occasions more frequent than:

- Once per Max(TReport, 1.5 x P x Max(TDRX, TSSB)) if TDRX ≤ 320ms

- Once per P x TDRX if TDRX > 320ms

# 4	References

- R4-2108253, “WF on NR-U RRM Core Requirements”, Ericsson.