3GPP TSG-RAN WG4 Meeting # 100 	R4-2113521

# Electronic Meeting, Aug. 16-27, 2021

Source:	Ericsson

Title:	Timing requirements

Agenda item:	9.13.5.4

Document for:	Approval

# 1	Introduction

In RAN4#99-e a WF was approved [1]. We analyze the questions in the WF and continue to discuss timing requirements for NTN WI.

# 2	Discussion

## 2.1	UE initial transmit timing error

In the approved WF, it is stated [1]:

… WF on UE specific TA estimation error: - Whether to define a separate accuracy requirement for UE specific TA estimation - Option 1: Yes (Intel, NEC, THALES, Ericsson) - Option 2: No (QC, CATT, Xiaomi, CMCC, LGE, Apple, Huawei, MTK, Ericsson, ZTE) - FFS the UE specific TA estimation accuracy - Whether to define the update rate for UE specific TA estimation - Option 1: Yes (Intel, CMCC, Ericsson, LGE) - Option 2: No (CATT, Xiaomi, Apple, Huawei, MTK, Qualcomm, ZTE, THALES) - Option 3: Under discussion in RAN1 (LGE) - Whether to define UE behaviour related to UE specific TA estimation - Option 1: Defer discussion for specifying UE behaviour related to UE specific TA estimation, and wait RAN1 conclusion (CATT, CMCC) - Option 2: Specify UE behaviour related to the combination of UE specific TA estimation (NTA,UE-specific) and self-estimated TA common (NTA,common) (THALES, Ericsson, Intel) - Option 3: No need to define UE behavior for UE specific TA estimation as a requirement, as long as UE can meet the timing requirement, i.e., Te/Tq/Tp (Apple, Xiaomi, Huawei, Qualcomm, ZTE) - Wait RAN1/RAN2 conclusions on UE specific TA pre-compensation reporting to determine whether we need to define separate UE specific TA estimation requirement or not - Whether to define a separate accuracy requirement for self-estimated TA common (NTA,common) - Option 1: Yes (THALES, Ericsson) - Option 2: No (Apple, Xiaomi, Huawei, Qualcomm, CATT, CMCC, LGE) - Whether to define a separate accuracy requirement for the combination of NTA,UE-specific+NTA,common - Option 1: Yes (THALES, Ericsson) - Option 2: No (Apple, Xiaomi, Huawei, Qualcomm, CATT, LGE) - It is up to RAN1/RAN2 decision on whether UE should use the referenceTimeInfo-R16 and GNSS-provided time reference to calculate TA at the UE WF on UE initial transmit timing requirements: - The composites should be considered for initial transmit timing requirement in NTN (Te_NTN) - Option 1: (QC, Xiaomi, Huawei, LGE, ZTE, NEC, CMCC) - UE position estimation error - Serving-satellite position estimation error - The current UE transmit timing error requirement - Option 1a: (LGE, MTK, Huawei, ZTE) - GNSS inaccuracy - The current UE transmit timing error requirement - Option 1b: (CATT) - UE position estimation error - Error calculated by extrapolation from ephemeris data - The current UE transmit timing error requirement - Option 2: (Apple) - legacy Te - UE specific TA estimation error (without ephemeris uncertainty) - Option 3: (THALES, Ericsson) - The accuracy of UE specific TA estimation (N_(TA,UE-specific)) and self-estimated TA common (N_(TA,common)) is counted into the UE transmit timing error requirement. - GNSS position error assumption for Te_NTN - Option 1: (QC, LGE) - at least 50m, and further relax up to 100m - Option 2: (Xiaomi, CATT, THALES, Apple) - 50m - Option 3: (CMCC, CATT) - 50m as the worst case and 20m as the typical case - Option 4: (MTK, Xiaomi, THALES, NEC, Intel) - For UL SCS of 15/30 kHz: <= 50 m - For UL SCS of 60/120 kHz: <= 30 m - Option 5: (Apple, LGE, Nokia) - The worst case: 100m - Whether to define general GNSS positioning accuracy requirements? - Option 1: (Huawei, Ericsson, Nokia, THALES) - Yes, it is suggested to define general GNSS positioning accuracy requirements which can be referred for deriving other RRM requirements. - Option 2: No (Apple, Xiaomi, QC, CATT, CMCC, Intel) - FFS - FFS the reference timing for UE initial transmission ...

Our main point is that it is the total NTN UE Te error that decides UL performance, no matter the source of inaccuracy. If a UE is subject to a positioning error ΔUE-pos (in time), the error in satellite position is ΔSat-pos (in time) and the UE SSB timing estimation accuracy is ΔUE_timing_estimate then we get, where Te is the initial timing error:

ΔUE-pos + ΔSat-pos  + ΔUE_timing_estimate  < Te

Te is needed in existing specification to safeguard against Inter Symbol interference and maintain a desired UL throughput and capacity. The existing requirements can be found in Table 1.

Table 1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Observation 1: The UE initial transmit timing error is needed to make sure we avoid Inter Symbol Interference and loose UL throughput and capacity.

Observation 2: The existing requirements make sure we fulfil intended UL throughput and capacity in NR.

The existing requirements are the conclusion of analysis in 3GPP RAN4 for an NR network for a large set of time dispersive channels. This makes sure that minimum system performance is guaranteed also for difficult radio environments.

Another way to analyse the question is to try and assess maximum allowed fraction of uncertainties of CP from UE to gNB. In [2], the existing uncertainties for a terrestrial UE transmitting to a gNB is summarised as table 2 below:

Table 2: Relation between CP length and existing timing errors at the BS reception

| SSB SCS (kHz) | UL SCS (kHz) | UE timing error (Te) [Tc] | TA command resolution error (section 4.2, TS 38.213) [Tc] | TA adjustment accuracy (section 7.3.2, TS 38.133) [Tc] | BS error (TBSE) [Tc] | BS error (TBSE) as percentage of UL CP length (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 15 | 768 | 512 | 256 | 1536 | 17 |
|  | 30 | 640 | 256 | 256 | 1152 | 25 |
|  | 60 | 640 | 128 | 128 | 896 | 39 |
| 30 | 15 | 512 | 512 | 256 | 1280 | 14 |
|  | 30 | 512 | 256 | 256 | 1024 | 22 |
|  | 60 | 448 | 128 | 128 | 704 | 31 |
| 120 | 60 | 224 | 128 | 128 | 480 | 21 |
|  | 120 | 224 | 64 | 32 | 320 | 28 |
| 240 | 60 | 192 | 128 | 128 | 448 | 19 |
|  | 120 | 192 | 64 | 32 | 288 | 25 |

If we add NTN specific factor we need to consider the TA mechanism, as currently discussed in RAN1:

Agreement RAN1#104-bis-e: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

In [3] it is estimated that with a reasonable balance between update frequency and amount of data to signal for a good prediction, the NTA,common factor will have an uncertainty of 10 % of CP. For a fair and balanced budget we allocate 10 % of CP to uncertainties in NTA,UE-specific as well. If we summarize we get, using one sided values for all factors:

Table 3: BS error (TBSE) + margin

| SSB SCS (kHz) | UL SCS (kHz) | BS error (TBSE) [Tc] | Feeder link margin [Tc] (±5 % CP) | UE margin [Tc] (±5% CP) | BS error (TBSE) + margins [Tc] | BS error (TBSE) + margin as percentage of UL CP length (%) | Channel Delay spread margin (µs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 15 | 1536 | 461 | 461 | 2458 | 27% | 2.25 |
|  | 30 | 1152 | 231 | 231 | 1613 | 35% | 0.7 |
|  | 60 | 896 | 115 | 115 | 1126 | 49% | 0.023 |
| 30 | 15 | 1280 | 461 | 461 | 2202 | 24% | 2.4 |
|  | 30 | 1024 | 231 | 231 | 1485 | 32% | 0.84 |
|  | 60 | 704 | 115 | 115 | 934 | 41% | 0.234 |
| 120 | 60 | 480 | 115 | 115 | 710 | 31% | 0.468 |
|  | 120 | 320 | 58 | 58 | 435 | 38% | 0.140 |
| 240 | 60 | 448 | 115 | 115 | 678 | 29% | 0.490 |
|  | 120 | 288 | 58 | 58 | 403 | 35% | 0.175 |

It is difficult to find exact references in 3GPP as to how large a fraction of CP we can tolerate. In [2] to was proposed that one existing Te could be used as a margin. The result is summarised in Table 4.

Table 4: BS error (TBSE) + margin [2]

| SSB SCS (kHz) | UL SCS (kHz) | BS error (TBSE) [Tc] | UE margin [Tc] | BS error + margin [Tc] | BS error (TBSE) + margin as percentage of UL CP length (%) |
| --- | --- | --- | --- | --- | --- |
| 15 | 15 | 1536 | 768 | 2304 | 25 |
|  | 30 | 1152 | 640 | 1792 | 39 |
|  | 60 | 896 | 640 | 1536 | 67 |
| 30 | 15 | 1280 | 512 | 1792 | 19 |
|  | 30 | 1024 | 512 | 1536 | 33 |
|  | 60 | 704 | 448 | 1152 | 50 |
| 120 | 60 | 480 | 224 | 704 | 31 |
|  | 120 | 320 | 224 | 544 | 47 |
| 240 | 60 | 448 | 192 | 640 | 28 |
|  | 120 | 288 | 192 | 480 | 42 |

If we compare Table 3 and Table 4 we see that the NTN budget in some cases require a larger fraction that what was proposed in [2], but most of the time less.

In the release 15 study report [4], section 6.7.2 the Delay Spread (DS) worst case LOS scenario for 20 degrees or higher elevation has a  Delay spread (DS) lgDS = log10(DS/1s) with lgDS = -7.28 and lgDS = 0.67. This means that lgDS +lgDS = -7.28 + 0.67 = -6.61 and DS < 0.245 µs with 68 % probability, if we assume Gaussian statistics [4, Table 6.7.2-1a].

Observation 3: The Delay Spread (DS) is listed as < 245 ns NTN across scenarios in the release 15 study report if we use on standard deviation as bound.

This could further motivate the allowed UE margin in Table 4,since almost all NTN radio channels are strong line of sight channels with only a weak fraction of delayed signal components. However if we take the single sided %-value of CP = x% for sum of the single sided uncertainties, we can have one UE + x% CP early and another -x% CP late. The radio channel dispersion margin is indicated in the rightmost column as CP*(1 - 2*x%). The red cases cannot handle the worst case radio channel from the release 15 study report [4].

Proposal 1: Allocate 10% (±5 %) of CP for UE position estimation error and Serving-satellite position estimation error in order to define a new Te requirement for NTN. The feasibility of UL SCS = 120 kHz and the combination UL SCS = 60 kHz and SSB SCS = 15 kHz has to be further investigated.

## 2.2	TA adjustment accuracy

A TA adjustment accuracy requirement is needed to safeguard against Inter Symbol interference and maintain a desired UL throughput and capacity. The needed TA adjustment accuracy is a function of the step size. Based on the assumption that RAN1 keeps the existing TA step size of 16*64/2µ then we should maintain the existing step sizes in Table 2:

Table 2: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc |

Observation 4: The UE Timing Advance adjustment accuracy is needed to make sure we avoid Inter Symbol Interference and loose UL throughput and capacity.

Proposal 2: Use existing TA adjustment accuracy also for NTN.

## 2.3 	Gradual Timing Adjustment

The parameter Te has further use to regulate gradual timing adjustment. For the gradual timing adjustment, TS 38.133 offers the following definition:

When the transmission timing error between the UE and the reference timing exceeds ±Te then the UE is required to adjust its timing to within ±Te. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2.1-1.

Table 7.1.2.1-1 in TS 38.133 is repeated here as Table 1.

Table 1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

The Tq and Tp requirements are essentially a function of Te, with an aim to make sure that the UL timing follow the DL timing reference. If we take the case of LEO, then the rate of change of round-trip time can be as high as dRTT/dt = 40 µs/s ≈ 79 Tc/ms [from Table 7.1-1 in [5]].

At higher speed there is larger time variation or time drift (Td) of the DL reference signal received at the UE. This will require the UE to apply larger maximum adjustment step or aggregated maximum adjustment over 200 ms period. The time drift over a given time period (T0) can be estimated as follows:

Td = dRTT/dt * T0

Where dRTT/dt is the maximum delay variation as seen by the UE.

Based on the above expression the speed dependent time drift (Td) at the TBD UE speed over a duration of 200 ms is about 246 * 64 Tc, if we assume a worst case dRTT/dt = ± 40 µs/s. from Table 7.1-1 in [5]. This is very high.

Observation 5: The parameter Tq will have to be modified. For a period of 200 ms we could have a worst case delay variation of 246 * 64 Tc.

This observation is similar to Table 1, Maximum timing drift in NTN scenario [4].

Observation: 6: Either the period has to be shortened from 200 ms to something smaller, or we need to increase Tq.

# 3	Summary

Observation 1: The UE initial transmit timing error is needed to make sure we avoid Inter Symbol Interference and loose UL throughput and capacity.

Observation 2: The existing requirements make sure we fulfil intended UL throughput and capacity in NR.

Observation 3: The Delay Spread (DS) is listed as < 245 ns NTN across scenarios in the release 15 study report if we use on standard deviation as bound.

Proposal 1: Allocate 10% (±5 %) of CP for UE position estimation error and Serving-satellite position estimation error in order to define a new Te requirement for NTN. The feasibility of UL SCS = 120 kHz and the combination UL SCS = 60 kHz and SSB SCS = 15 kHz has to be further investigated.

Observation 4: The UE Timing Advance adjustment accuracy is needed to make sure we avoid Inter Symbol Interference and loose UL throughput and capacity.

Proposal 2: Use existing TA adjustment accuracy also for NTN.

Observation 5: The parameter Tq will have to be modified. For a period of 200 ms we could have a worst case delay variation of 246 * 64 Tc.

Observation: 6: Either the period has to be shortened from 200 ms to something smaller, or we need to increase Tq.

# References

- R4-2108350, WF on timing requirements for NR NTN, Xiaomi

- R4-1912197, Further analysis of one-shot timing adjustment requirements, Ericsson

- R1-2107637, On UL time and frequency synchronization enhancements for NTN, Ericsson

- TR 38.811, Study on New Radio (NR) to support non-terrestrial networks (Release 15)

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), V16.0.0

- R4-2104604, Discussion on NTN timing requirements, CMCC