TSG-RAN Working Group 4 meeting #100-E	R4-2114978

Electronic Meeting, 16th - 27th August 2021

Source:	Ericsson

Title:	WF on n14 coexistence evaluation for NR PS UE

Agenda item:	9.15.1

Document for:	Approval

# Introduction

During RAN4#100-E meeting a way-forward on n14 coexistence evaluation for NR PS UE is created based on the discussion in 1st round [1].

# Way-Forward

## 2.1	Protection of B13/n13 UE by n14 PS operation with PC1/PC3

- Option 1: RAN4 does not see the need for the additional coexistence evaluation to protect B13/n13 UE based on evaluation of the difference of coexisting simulation difference between NR SL UE and LTE SL UE

- Always enable the OLPC in NR SL UE in-coverage: Network always configures the in-coverage NR SL with association to a network cell (e.g (a) and (b) in Figure 1 in Annex) and avoid the configuration where no network cell is associated NR SL UE (e.g the configuration of the (c) and (d) in Figure 1 of the annex)

- NR SL SINR to throughput table is similar with LTE V2X SNIR to throughput table.

- Option 2: RAN4 can not exclude the additional coexistence evaluation to protect B13/n13 UE based on evaluation of the difference of coexisting simulation difference between NR SL UE and LTE SL UE.

- OLPC in NR SL UE may not be guaranteed: Network cannot exclude the configuration for the in-coverage NR SL where no network cell is not associated NR SL UE (e.g the configuration of the (c) and (d) in Figure 1 of the annex)

- Option 3: FFS

WF on Protection of legacy n14 Uu system in in-coverage NW scenarios:

Option 1

## 2.2 Protection of legacy n14 Uu system in in-coverage NW scenarios

- Option 1: RAN4 does not see the need for the additional coexistence evaluation for based on evaluation of the difference of coexisting simulation difference between NR SL UE and LTE SL UE

- NR SL SINR to throughput table is similar with LTE V2X SNIR to throughput table

- RB allocations difference between NR SL and LTE V2X is not key factor for Coexisting simulation result. LTE 2 RB is worst case from interference point of view compared with the at least 10RB allocation for NR SL.

- Option 2: RAN4 cannot exclude the additional coexistence evaluation based on evaluation of the difference of coexisting simulation difference between NR SL UE and LTE SL UE

- NR SL SINR to throughput table is significant different with LTE V2X SNIR to throughput table

- RB allocations difference between NR SL and LTE V2X play significant role in Coexisting simulation result.

- Option 3: FFS

WF on Protection of legacy n14 Uu system in in-coverage NW scenarios:

- Option 1

# References

- R4-2114734	Email discussion summary for [100-e][134] NRSL_enh_Part_1, LGE

- R4-2114335, coexisting simulation assumption for public safety UC and protection of B13, Ericsson

- TR 36.837, Public safety broadband high power User Equipment (UE) for band 14

- TR 36.877, LTE Device to Device Proximity Services; User Equipment (UE) radio transmission and reception

# Annex

Figure 1: Network configuration for the sidelink carrier/BWP and Uu carrier/BWP [2]