3GPP TSG-RAN WG4 Meeting # 101-bis-e 	R4-2200139

# Electronic Meeting, January 17-25, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1, Correction on MOP requirements for inter-band V2X con-current operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2021-12-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was agreed no upper power bound is considered for inter-band V2X con-current operation. The MOP requirements specified in clause 6.2E.1.2 are not appropriate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the description to indicate the respective MOP requirements specified for single carrier of Uu and SL apply for uplink transmission and sidelink transmission of inter-band con-current operation. - Leave table 6.2E.1.2-1 as void. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MOP requirements specified for inter-band con-current operation would cause confusion and contradiction with configured transmitted power. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.2E.1.2	UE maximum output power for V2X con-current operation

For the inter-band con-current NR V2X operation, the maximum output power specified in clause 6.2.1 shall apply for the uplink in licensed and the maximum output power specified in clause 6.2E.1 shall apply for the sidelink in licensed band or Band n47. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.2-1: void

<End of Change 1>