3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200996

# Electronic Meeting, Jan.17-25,2022

Title: 	Discussion on BS demodulation requirements for additional enhancements of NB-IOT.

Source: 	Huawei, HiSilicon

Agenda item:	8.9.6.2.2

Document for:	Discussion

Background

WI of Rel-17 additional enhancements of NB-IOT has been almost closed from RAN 1’s part and it’s time for RAN 4 to define the corresponding performance requirements. In this paper, we give our summary of RAN 1’s feature on additional enhancements of NB-IOT firstly and provide our discussions on how to define the corresponding BS performance requirements.

Summary of RAN 1’s feature

The summary of RAN 1’s feature of additional enhancement of NB-IOT for BS side has been included in Table 1:

Table 1: Summary of RAN 1’s feature of additional enhancement of NB-IOT

| RAN 1 feature | RAN 1 sub-feature | Description | Impact on RAN 4 performance requirements |
| --- | --- | --- | --- |
| Introduce 16 QAM for unicast in UL for NB-IOT | New modulation order | - Introduce 16QAM for unicast UL link | - NPUSCH format 1 with16QAM should be verified |
|  | TBS design | - For uplink, TBS table defined in clause 16.4.1.5.1 of TS 36.213 is extended with additional rows (Row 14~21) are used for 16QAM/ without increasing of maximum TBS | - No impact since no new TBS is introduced |
|  | DCI design | - Reserved state “1111” in the “Modulation and coding scheme” field in DCI Format N0 is utilized to indicate the use of 16QAM. - The “Repetition number” field in DCI Format N0 is utilized to indicate the TBS indices (i.e., I_TBS indices from 14 to 21) for 16-QAM in UL. | - No impact since there is no change of NPDCCH structure |
|  | Uplink Power control | - In order to increase the coverage, RAN 1 agree to introduce parameter∆TF,c with function of spectrum efficiency following the same way of LTE to the current power control formula | - No impact since it belongs to power control part |
|  | Others | - Repetition is not used for 16-QAM in uplink. - 16QAM can be used at least for multi-tone transmission with 12 subcarriers,3 subcarriers. - Support 16-QAM for multi-TB scheduling. - Support 16-QAM for NPDSCH and NPUSCH in PUR procedure, - 16-QAM can be enabled/disabled by UE specific RRC signaling for NPDSCH and NPUSCH separately | - PUSCH repetition is not configured in the test - Multi-TB is covered in Rel-16 - Multi-tones can be configured for NPUSCH format 1 performance for 16QAM |

Different from UE side, only 16QAM is introduced for uplink, therefore, we propose to verify the BS demodulation requirements with 16QAM

Proposal 1:  Define NPUSCH format 1 requirements to verify the 16 QAM.

Discussion on BS performance requirements

Based on our understanding, we can reuse the test parameters in Table 8.5.1.1.1-3 of TS.36.104 with some change of repetition, modulation and tbSize. One issue is how to select TBS, the corresponding code rate with different ITBS and IRU for 16QAM are included in Table 2

Table 2: Code rate with different ITBS and IRU for 16-QAM

| Modulation Scheme | ITBS | IRU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 16-QAM only | 14 | 0.53 | 0.54 | 0.54 | 0.54 | 0.54 | 0.53 | 0.54 | - |
|  | 15 | 0.61 | 0.57 | 0.57 | 0.57 | 0.57 | 0.56 | - | - |
|  | 16 | 0.63 | 0.63 | 0.63 | 0.63 | 0.63 | 0.63 | - | - |
|  | 17 | 0.69 | 0.69 | 0.69 | 0.68 | 0.70 | 0.69 | - | - |
|  | 18 | 0.75 | 0.75 | 0.76 | 0.76 | 0.76 | - | - | - |
|  | 19 | 0.81 | 0.81 | 0.81 | 0.82 | 0.82 | - | - | - |

We can observe that coderate for different IRU are closed to 0.5 when ITBS =14 which are common use cases, therefore, we propose to use ITBS =14. Therefore, based on the other simulation assumptions of test case in Table 8.5.1.1.1-3 of TS.36.104, we propose to use Table 3 as simulation assumptions for additional enhancement of NB-IOT BS requirements

Table 3: Proposed simulation assumptions

| Parameters | Value |
| --- | --- |
| Number of tones | 12 |
| SCS | 15kHz |
| Antenna configuration | 1T2R |
| Channel model | ETU 1Hz |
| Frequency error | 0Hz |
| Timing error | 0μs |
| Modulation scheme | 16 QAM |
| Performance target | SNR@ 70% of maximum throughput |
| Repetition number | Not configured |
| ITBS / IRU | 14/ 0(TBS=256 bits, Number of RUs=1) |

Proposal 2: Use Table 3 as simulation assumptions for NPUSCH format 1 with 16QAM performance requirements definition

Conclusion

In this contribution, we give our summary of RAN 1’s feature on additional enhancements of NB-IOT firstly and provide our discussions on how to define the corresponding BS performance requirements. The proposals are:

Proposal 1:  Define the BS requirements to verify the 16 QAM reception.

Proposal 2: Use Table 3 as simulation assumptions

Table 3: Proposed simulation assumptions

| Parameters | Value |
| --- | --- |
| Number of tones | 12 |
| Modulation order | 4 |
| SCS | 15kHz |
| Antenna configuration | 1T2R |
| Channel model | ETU 1Hz |
| Frequency error | 0Hz |
| Timing error | 0μs |
| Modulation scheme | 16 QAM |
| Performance target | SNR@ 70% of maximum throughput |
| NRU | 5 |
| Repetition number | Not configured |
| ITBS / IRU | 14/ 0 |