3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201845

Electronic Meeting, January 17-25, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On HST FR2 PUSCH UL Timing Adjustment Requirements

Agenda item:	6.9.5.3.2 PUSCH with UL timing adjustment requirements

Document for:	Discussion

Introduction

In the previous RAN4 meetings all of the open issues regarding the HST FR2 PUSCH UL timing adjustment testing setup were resolved. In particular, at RAN4#100-e it was agreed that only MCS16 shall be tested [1]. Additionally, at RAN4#101-e it was agreed that existing PUSCH UL timing adjustment demodulation requirement specified in TS 38.104 shall be reused in HST FR2 [2]:

Agreements: - GTW agreement on channel model for UL timing adjustment requirement: - Follow same approach as existing PUSCH UL timing adjustment requirement demodulation requirement specified in TS 38.104

In this contribution, following the agreements achieved, we are presenting the simulation results that will be used for the definition of the performance requirements.

Discussion

We are following the simulation assumptions for UL timing adjustment requirements that were summarized in [2] to parametrize link-level simulations:

Table 1: Simulation assumptions for UL timing adjustment requirements.

| Parameters | Value |
| --- | --- |
| Channel Model | Stationary UE: AWGN, Moving UE: AWGN |
| UE speed | 350km/h |
| Waveform | CP-OFDM |
| CP length | Normal |
| A | AWGN |
| Δω | 1T2R |
| CBW | 50MHz and 200MHz |
| DM-RS type | Type 1 |
| Mapping type | B |
| Length of PUSCH data | 10 |
| RS configuration | 1DMRS+PT-RS(L=1,K=2), 2DMRS+PT-RS(L=1,K=2), 3DMRS+PT-RS(L=1,K=2) |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| DM-RS port | {0} |
| DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| PUSCH resource allocation | 50MHz: 0 to 15 RB for moving UE, 16 to 31 for stationary UE (if 100MHz 200MHz: 0 to 65RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | 50MHz CBW (32RBs)~C_SRS =9, B_SRS =0 200MHz CBW (132RBs)~ C_SRS=33, B_SRS=0 |

Additionally, we take into account the agreements from RAN4#100-e [1]:

- RS configuration - Align RS configuration for UL timing adjustment requirement and PUSCH requirement - SRS transmission - The transmission of SRS in UL timing adjustment requirement is optional - MCS: Only with MC16

and adopt the parameters of the channel model from TS 38.104 to 120 kHz SCS as it was agreed at RAN4#101-e [2]:

- On test parameters for timing offset: - Use A=1.25us, Δw= 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements - On Timing different between moving UE and stationary UE: - Apply the timing difference between moving UE and stationary UE as Δτ-(TA-31)x16*8Tc for UL timing adjustment requirement - On SRS transmission configuration: - The SRS transmission location is the last symbol in slot#3 in radio frame with TDD pattern as DDDSU, S=10:2:2

The results of link-level simulations are summarized in Table 2 below.

Table 2: PUSCH UL Timing Adjustment demodulation performance results.

| SCS and BW | MCS | Channel | DM-RS mapping | Length of data | RS configuration | SNR, dB |
| --- | --- | --- | --- | --- | --- | --- |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+0 | 6.75 |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+1 | 6.46 |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+1+1 | 6.73 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+0 | 6.60 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+1 | 6.47 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+1+1 | 6.47 |

Use the results in the table above for the alignment of PUSCH UL Timing Adjustment demodulation performance.

Conclusion

In the contribution, we are providing the link-level simulations results for alignment of PUSCH UL Timing Adjustment demodulation performance requirements. The following proposal is made:

| SCS and BW | MCS | Channel | DM-RS mapping | Length of data | RS configuration | SNR, dB |
| --- | --- | --- | --- | --- | --- | --- |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+0 | 6.75 |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+1 | 6.46 |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+1+1 | 6.73 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+0 | 6.60 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+1 | 6.47 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+1+1 | 6.47 |

- Use the results in the table above for the alignment of PUSCH UL Timing Adjustment demodulation performance.

- References

- R4-2115726, WF on FR2 HST demodulation, Samsung, RAN4#100-e.

- R4-2120703, WF on BS demodulation requirement for FR2 HST, Nokia, Nokia Shanghai Bell, Samsung, RAN4#101-e.