3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118102

Electronic Meeting, November 01-12, 2021

Agenda Item: 8.8.2.2

Source: MediaTek Inc.

Title: Discussion on Rel-17 HST in FR1 for inter-frequency measurement

Document for: Discussion

- Introduction

In the previous meeting, there are some issues are still under discussion, e.g., inter frequency measurement with MG in CONNECTED mode and inter frequency measurement in IDLE mode. Our views are provided in this paper.

- Discussion

2.1 inter-frequency measurement with MG in CONNECTED mode

RAN4 has agreed the requirement for TSSB_time_index_inter and TSSB_measurement_period_inter but not yet for TPSS/SSS_sync_inter and in last meeting. Thus, our view on TPSS/SSS_sync_inter is provided as follows. For convenience of reference, the requirement of non-HST for TPSS/SSS_sync_inter is provided as follows.

The measurement period for inter-frequency measurements with gaps in clause 9.3.4 of TS 38.133

Table 9.3.4-1: Time period for PSS/SSS detection (Frequency range FR1) Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(8*1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 8 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group..

In our understanding, the inter-RAT measurement (from LTE to NR; defined in clause 8.1.2.4.21 of TS 36.133) is similar to inter-frequency measurement because both two are measuring on different frequency points in NR. Thus, we propose to follow the similar logic as in the R16 HST inter-RAT measurement (from LTE to NR) to enhance inter-frequency enhancement for R17 HST, i.e., the scaling factor of 1.5 can be replaced by the scaling factor of M2.

Proposal 1: In CONNECTED mode, for the inter-frequency measurements with MGs for Rel-17 HST,  introduce the scaling factor M2, by following the similar logic as the R16 HST inter-RAT measurement (from LTE to NR) specified in clause 8.1.2.4.21 of TS 36.133.

Thus, according to Proposal 1, the TPSS/SSS_sync_inter or R17 HST inter-frequency measurements with MGs are provided as follows:

Proposal 2: In CONNECTED mode, for the inter-frequency measurements with MGs in Rel-17 HST, the TPSS/SSS_sync_inter should be:

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8* M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

2.2 inter-frequency measurement in IDLE mode

To specify the enhanced RRC_IDLE mode inter-frequency measurement for R17 HST, we can take the RRC_IDLE inter-RAT measurement (from LTE to NR; defined in clause 4.2.2.5.6 of TS 36.133) as reference.

Proposal 3: In IDLE mode, for the inter-frequency measurements for Rel-17 HST, introduce the scaling factor M2, M3 and M4 to define the requirement, by following the similar logic as the R16 HST inter-RAT measurement (from LTE to NR) defined in clause 4.2.2.5.6 of TS 36.133.

The requirements of the RRC_IDLE inter-RAT measurement (from LTE to NR; defined in clause 4.2.2.5.6 of TS 36.133) for non-HST and R16 HST are provided as follows for reference.

The measurement period for RRC_IDLE inter-RAT measurements (from LTE to NR; defined in clause 4.2.2.5.7 of TS 36.133) Table 4.2.2.5.6-1: Tdetect,NR, TmeasureNR, and Tevaluate,NR DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR [s] (number of DRX cycles); Tmeasure,NR [s] (number of DRX cycles); Tevaluate,NR [s] (number of DRX cycles). FR1; FR2Note1. 0.32; 1; 8; 11.52 x 1.5 x N1 (36 x 1.5 x N1); 1.28 x 1.5 x N1 (4 x 1.5 x N1); 5.12 x 1.5 x N1 (16 x 1.5 x N1). 0.64; 5; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length.. Table 4.2.2.5.6-2: Tdetect,NR_HST, TmeasureNR_HST, and Tevaluate,NR_HST for UE configured with highSpeedInterRAT-NR-r16 DRX cycle length [s]; Tdetect,NR_HST [s] (number of DRX cycles); Tmeasure,NR_HST [s] (number of DRX cycles); Tevaluate,NR_HST [s] (number of DRX cycles). 0.32; 4.16 x M2 (13 x M2)Note 2; 0.64 x M3 (2 x M3)Note 2; 0.96 x M4 (3 x M4) Note 2. 0.64; 7.68 (12)); 1.28 (2); 1.92 (3). 1.28; 12.8(10); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: FR2 high speed requirements are not specified. Note 2: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=1..

Besides, for convenience of reference, the RRC_IDLE inter-frequency measurement for non-HST in TS 38.133 is provided as follows.

The measurement period for RRC_IDLE inter-frequency measurements in clause 4.2.2.4 of TS 38.133 Table 4.2.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). FR1; FR2Note1. 0.32; 1; 8; 11.52 x N1 x 1.5 (36 x N1 x 1.5); 1.28 x N1 x 1.5 (4 x N1 x 1.5); 5.12 x N1 x 1.5 (16 x N1 x 1.5). 0.64; 5; 17.92x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length..

According to Proposal 3, our views on the Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for R17 HST RRC_IDLE inter-frequency measurements are provided as follows:

Proposal 4: In IDLE mode, for the inter-frequency measurements in Rel-17 HST, the Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter should be:

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 1 | 0.64 x M3 (2 x M3)Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 7.68 (12)) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1. |  |  |  |

- Summary

In this paper, the discussion of R17 HST in FR1 is provided. We have the following proposal:

Proposal 1: In CONNECTED mode, for the inter-frequency measurements with MGs for Rel-17 HST,  introduce the scaling factor M2, by following the similar logic as the R16 HST inter-RAT measurement (from LTE to NR) specified in clause 8.1.2.4.21 of TS 36.133.

Proposal 2: In CONNECTED mode, for the inter-frequency measurements with MGs in Rel-17 HST, the TPSS/SSS_sync_inter should be:

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8* M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

Proposal 3: In IDLE mode, for the inter-frequency measurements for Rel-17 HST, introduce the scaling factor M2, M3 and M4 to define the requirement, by following the similar logic as the R16 HST inter-RAT measurement (from LTE to NR) defined in clause 4.2.2.5.6 of TS 36.133

Proposal 4: In IDLE mode, for the inter-frequency measurements in Rel-17 HST, the Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter should be:

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 1 | 0.64 x M3 (2 x M3)Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 7.68 (12)) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1. |  |  |  |