3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118106

# Electronic Meeting, November 01-12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for CGI reading test case in R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-Perf |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The event A3 is not applicable for inter-RAT scenario because it is for NR neighbouring cell. - The IE si-RequestForHO is used for E-UTRAN. According to TS 38.331, si-RequestForHO should be replaced by useAutonomousGaps. ReportCGI-EUTRA field descriptions. useAutonomousGaps Indicates whether or not the UE is allowed to use autonomous gaps in acquiring system information from the E-UTRAN neighbour cell. When the field is included, the UE applies the corresponding value for T321.. - For Table A.6.6.7.2.1-3, the Es/Noc, Es/Iot, SS_RSRP and SSB_RP are ambiguous during T2 and T3. |
| Summary of change: |  | - In the test purpose and environment, change triggering event from A3 to B2 - In Table A.6.6.7.2.1-2 and the test purpose and environment, replace si-RequestForHO by useAutonomousGaps. - In Table A.6.6.7.2.1-2, remove the field “A3-offset” - In Table A.6.6.7.2.1-3, add a new column for T3 and add b2-Threshold2EUTRA = -109dB/15Hz for event B2 (LTE neighboring cell) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test cell configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

A.6.6.7.2.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in NR SA in clause 9.4.7.

The test scenario comprises of one NR carrier and an E-UTRA carrier and two cells as given in tables A.6.6.7.2.1-1, A.6.6.7.2.1-2, A.6.6.7.2.1-3 and A.6.6.7.2.1-4. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event B2. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.6.6.7.2.1-1: Supported test configurations of inter-RAT E-UTRAN cell using autonomous gap in SA

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.7.2.1-2: General test parameters for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| LTE Channel Bandwidth | MHz | 10 |  |
| LTE PDSCH/PCFICH/PDCCH/PHICH parameters |  |  | As specified in clause A.3.7.2.1 |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| useAutonomousGaps |  | TRUE | As specified in clause 5.5.3.1 in TS 38.331. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.6.6.7.2.1-3: PCell specific test parameters for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR

| Parameter |  |  | Unit | Configuration | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 |  | T2 | T3 |  |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |  |  |  |
|  |  |  |  | 4, 5, 6 | TDD |  |  |  |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |  |  |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |  |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |  |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |  |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |  |  |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |  |  |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |  |  |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |  |  |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |  |  |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |  |  |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |  |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |  |  |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |  |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |  |  |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |  |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |  |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |  |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |  |  |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |  |  |  |
|  |  |  |  | 3, 6 | -95 |  |  |  |  |
| b2-Threshold2EUTRA |  |  | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -109 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |  |  |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |  |  |  |
|  |  |  |  | 3, 6 | -103 |  |  |  |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 |  | -2 |  | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 |  | -2 |  | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 |  | -108 |  | -108 |
|  |  |  |  | 3, 6 | -85 |  | -105 |  | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 |  | -108 |  | -108 |
|  |  |  |  | 3, 6 | -85 |  | -105 |  | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 |  | -75.92 |  | -75.92 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 |  | -69.82 |  | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |  |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

Table A.6.6.7.2.1-4: Cell specific test parameters for inter-RAT E-UTRAN cell for identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 10 MHz: NRB,c = 50 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 10 MHz: R.3 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 10 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 10 MHz: OP.10 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 | 7 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 | 7 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -99 | -99 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -99 | -99 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

<End of Change 1>