3GPP TSG-RAN4 WG4 Meeting #101-e	R4-2118367

# Electronic meeting, Nov.1-12, 2021

Source: 	OPPO

Title: 	Interruption requirements for 52.6-71GHz

Agenda Item:	8.16.7.3

Document for:	Discussion

Introduction

In last meeting, RAN4 has discussed and agreed on the interruptions at SCell addition/release, at SCell activation/deactivation, due to active BWP switching requirement and at NR SRS carrier-based switching. A WF [1] related to interruption was agreed.

In this paper, we further provide our view on the remaining issues on interruption at SCell addition/release and at SCell activation/deactivation for inter-band CA.

Discussion

Issue 3-3-1: Interruptions at SCell addition/release (inter-band CA) Agreements: Update Table 8.2.2.2.1-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X1 for SCell addition/release for inter-band CA NR Slot length (ms) of victim cell; Interruption length X1 (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; Both aggressor cell and victim cell are on FR2; 4. Either aggressor cell or victim cell is on FR1; 5. 3; 0.125; Aggressor cell is on FR2; 8. Aggressor cell is on FR1; 9. 5; 0.03125; Aggressor cell is on FR2-2; 32. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS. 6; 0.015625; Aggressor cell is on FR2-2; 64. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS. Issue 3-3-3: Interruptions at SCell activation/deactivation (inter-band CA) Agreements: - Update Table 8.2.2.2.2-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X2 for SCell activation/deactivation for inter-band CA NR Slot length (ms) of victim cell; Interruption length X2 (slots). 0; 1; 1. 1; 0.5; 1. 2; 0.25; Both aggressor cell and victim cell are on FR2; 2. Either aggressor cell or victim cell is on FR1; 3. 3; 0.125; Aggressor cell is on FR2; 4. Aggressor cell is on FR1; 5. 5; 0.03125; Aggressor cell is on FR2-2; 16. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS. 6; 0.015625; Aggressor cell is on FR2-2; 32. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS.

As agreed in RAN4, the requirements for the following deployment scenarios with equal priority are to be defined.

- Standalone single carrier and CA in FR2-2 - FR2-2 CA and DC with anchor on FR1 - Note: the scenario may be further adjusted pending further discussion in the RF session

Firstly, whether single or multiple bands to be defined for FR2-2 depends on RF session. From our side, we prefer to not preclude multiple bands being introduced in Rel-17 or later.

- Thus, two inter bands CA in FR2-2 are not precluded. Keeping the case of both victim cell and aggressor cell on FR2-2 is fine, where interruption length X = 18 when SCS = 480 kHz, and 32 when SCS = 960 kHz.

For the case of more than 3 inter bands CA/DC across FR1, FR2-1 and FR2-2, we think it is also reasonable because UE can support FR1+FR2-1 and FR1+FR2-2 CA/DC.

- If UE has FR1+FR2-2 inter-band CA, and another SCell is being added in FR2-1, it may cause an interruption on FR2-2 cell. Therefore, in our view, if aggressor cell is on FR2-1, the interruption length can be 18 slots when SCS = 480 kHz, and 34 slots when SCS = 960 kHz.

- If UE has FR1+FR2-2 inter-band CA, and another SCell is being added in FR1, it may cause an interruption on FR2-2 cell as well. Therefore, if aggressor cell is on FR1, the interruption length can be 20 slots when SCS = 480 kHz, and 36 slots when SCS = 960 kHz.

The above conclusion can be applied for both interruption length X1 for SCell addition/release for inter-band CA, and interruption length X2 for SCell activation/deactivation for inter-band CA.

Table X: interruption at SCell addition/release and at SCell activation/deactivation for inter-band CA.

|  | NR Slot length (ms) of victim cell | Interruption lengthX1/X2 (slots) |  |
| --- | --- | --- | --- |
| 5 | 0.03125 | Aggressor cell is on FR2-2 | 16 |
|  |  | Aggressor cell is on FR2-1 | 18 |
|  |  | Aggressor cell is on FR1 | 20 |
| 6 | 0.015625 | Aggressor cell is on FR2-2 | 32 |
|  |  | Aggressor cell is on FR2-1 | 34 |
|  |  | Aggressor cell is on FR1 | 36 |

Proposal 1: RAN4 to consider the case of more than 3 inter bands CA/DC across FR1, FR2-1 and FR2-2.

Proposal 2: If Proposal 1 was agreed, RAN4 to define the interruption requirements for the case of victim cell on FR2-2 and aggressor cell is on FR2-2, FR2-1 or FR1, as the table below.

Table X: interruption at SCell addition/release and at SCell activation/deactivation for inter-band CA.

|  | NR Slot length (ms) of victim cell | Interruption lengthX1/X2 (slots) |  |
| --- | --- | --- | --- |
| 5 | 0.03125 | Aggressor cell is on FR2-2 | 16 |
|  |  | Aggressor cell is on FR2-1 | 18 |
|  |  | Aggressor cell is on FR1 | 20 |
| 6 | 0.015625 | Aggressor cell is on FR2-2 | 32 |
|  |  | Aggressor cell is on FR2-1 | 34 |
|  |  | Aggressor cell is on FR1 | 36 |

Conclusion

In this paper, we further provide our view on the remaining issues on interruption at SCell addition/release and at SCell activation/deactivation for inter-band CA.

Proposal 1: RAN4 to consider the case of more than 3 inter bands CA/DC across FR1, FR2-1 and FR2-2.

Proposal 2: If Proposal 1 was agreed, RAN4 to define the interruption requirements for the case of victim cell on FR2-2 and aggressor cell is on FR2-2, FR2-1 or FR1, as the table below.

Table X: interruption at SCell addition/release and at SCell activation/deactivation for inter-band CA.

|  | NR Slot length (ms) of victim cell | Interruption lengthX1/X2 (slots) |  |
| --- | --- | --- | --- |
| 5 | 0.03125 | Aggressor cell is on FR2-2 | 16 |
|  |  | Aggressor cell is on FR2-1 | 18 |
|  |  | Aggressor cell is on FR1 | 20 |
| 6 | 0.015625 | Aggressor cell is on FR2-2 | 32 |
|  |  | Aggressor cell is on FR2-1 | 34 |
|  |  | Aggressor cell is on FR1 | 36 |

- References

[1] R4-2115351, WF on R17 NR_ext_to_71GHz_RRM_1, Qualcomm