3GPP TSG-RAN WG4 Meeting #101-e	R4-2118442

Online, 1 Nov – 12 Nov, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRM requirements for HO with PSCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Draft CR for HO with PSCell RRM requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RRM requirements for HO with PSCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM requirements for HO with PSCell are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

## 6.x	Handover with PSCell

### 6.x.1	Introduction

This clause defines requirements for the delay within which the UE shall be able to perform inter-RAT or intra-RAT handover with PSCell. The handover with PSCell procedure is initiated with a RRC message that implies a handover with PSCell as described in TS 38.331 [2].

### 6.x.2	HO with PSCell from SA to EN-DC

The requirements in this clause are applicable to handover with PSCell from NR SA to EN-DC.

This clause defines requirements for the delay within which the UE shall be able to handover from NR cell to E-UTRAN cell and add NR PSCell in the meantime.

When the UE receives a RRC message implying handover with PSCell,

- The UE shall be ready to start the transmission of the new uplink PRACH channel of the target E-UTRAN PCell within Dhandover ms from the end of the last TTI containing the RRC command where Dhandover is specified in clause 6.1.2.1, and

- The UE shall be capable of transmitting PRACH preamble towards the target PSCell no later than specified in clause 7.31.2 of TS 36.133 [15].

### 6.x.3	HO with PSCell from EN-DC to EN-DC

The requirements in this clause are applicable to handover with PSCell from EN-DC to EN-DC.

This clause defines requirements for the delay within which the UE shall be able to handover from E-UTRAN cell to E-UTRAN cell and add NR PSCell in the meantime.

When the UE receives a RRC message implying handover with PSCell,

- The UE shall be ready to start the transmission of the new uplink PRACH channel of the target E-UTRAN PCell within Dhandover ms from the end of the last TTI containing the RRC command where Dhandover is specified in clause 5.1.2.2 ~ 5.1.2.4 of TS 36.133 [15], and

- The UE shall be capable of transmitting PRACH preamble towards the target PSCell no later than specified in clause 7.31.2 of TS 36.133 [15].

### 6.x.4	HO with PSCell from NE-DC to NE-DC

The requirements in this clause are applicable to handover with PSCell from NE-DC to NE-DC.

This clause defines requirements for the delay within which the UE shall be able to handover from NR cell to NR cell and add E-UTRAN PSCell in the meantime.

When the UE receives a RRC message implying handover with PSCell,

- The UE shall be ready to start the transmission of the new uplink PRACH channel of the target PCell within Dhandover ms from the end of the last TTI containing the RRC command where Dhandover is specified in clause 6.1.1, and

- The UE shall be capable of transmitting PRACH preamble towards the target E-UTRAN PSCell no later than specified in clause 8.8.2.

### 6.x.5	HO with PSCell from NR-DC to NR-DC

The requirements in this clause are applicable to handover with PSCell from NR-DC to NR-DC.

This clause defines requirements for the delay within which the UE shall be able to handover from NR cell to NR cell and add NR PSCell in the meantime.

When the UE receives a RRC message implying handover with PSCell,

- The UE shall be ready to start the transmission of the new uplink PRACH channel of the target PCell within Dhandover ms from the end of the last TTI containing the RRC command where Dhandover is specified in clause 6.1.1, and

- The UE shall be capable of transmitting PRACH preamble towards the target PSCell no later than specified in clause 8.9.2.

# < End of change 1>