3GPP TSG-RAN WG4 Meeting # 101-e                              R4-2118709

Electronic Meeting, 1– 12 November, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on how to move forward for simplifying extended MSD table

Agenda Item:	7.26.2

Document for:	Approval

Introduction

Based on the WF [1] in RAN4#101e meeting, working group focused on two potential ways for simplifying extended MSD table due to harmonic interference and cross band isolation.

Option 1: Only one MSD test point is specified per aggressor/victim pair of bands with following configuration template for REFSENS exceptions due to harmonic interference or cross band isolation.

| UL band | DL band | UL Fc | UL BW | UL RB Allocation | DL Fc | DL BW | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | LCRB | (MHz) | (MHz) | (dB) |

Option 2: Based on current existing table, to investigate some additional method that the table will not be horizontally expanded in next meeting.

Since this topic didn’t make much progress in previous meetings, I’d like to provide our views to move forward for simplifying extended MSD table.

Discussion

As we discussed in the paper [2], for inter-band ENDC, NR CA and SUL band combinations, the reference sensitivity exceptions due to harmonic interference/harmonic mixing/cross band isolation are specified as bandwidth-specific requirements. As more and more kinds of bandwidth for single carrier are specified in the spec, the following issues are brought into specifications for MSD exceptions.

•	After the completion of new bandwidth for single carrier, RAN4 have to check the MSD exceptions for all the relative band combinations with BCS4 and fill up the missing requirements.

•	In Rel-17, RAN4 will introduce 15 channel bandwidths (5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100) for FR1. Considering that the existing UE RF MSD exception tables with channel bandwidth dependency is already fairly crowded.

•	It’s inconvenient to maintain the MSD exception tables as the channel bandwidths are increasing.

•	There are still some missing requirements and errors in the MSD exception tables.

Observation 1: As the channel bandwidths are increasing, it’s impossible to specify all kinds of channel bandwidth combinations as test cases.

Since we need to consider both the victim channel bandwidths and aggressor UL allocations, the situation seems to be more complex due to the increase of channel bandwidths. So many cases need to be considered as below.

|  |  | n1/n2/n3/n5/n8/n20/n25/n28/n66/n71 Aggressor Bands |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| n1 n25 n41 n48 n50 n70 n75 n77 n78 n79 Victim bands | 5 | * | * | * | * | * | * | * | * |
|  | 10 | * | * | * | * | * | * | * | * |
|  | 15 | * | * | * | * | * | * | * | * |
|  | 20 | * | * | * | * | * | * | * | * |
|  | 25 | * | * | * | * | * | * | * | * |
|  | 30 | * | * | * | * | * | * | * | * |
|  | 40 | * | * | * | * | * | * | * | * |
|  | 50 | * | * | * | * | * | * | * | * |
|  | 60 | * | * | * | * | * | * | * | * |
|  | 70 | * | * | * | * | * | * | * | * |
|  | 80 | * | * | * | * | * | * | * | * |
|  | 90 | * | * | * | * | * | * | * | * |
|  | 100 | * | * | * | * | * | * | * | * |

Furthermore, not all test cases will be tested. RAN5 just choose one of these configurations when they define the test cases. Thus, it’s better for RAN4 to limit test configurations for REFSENS exceptions instead of extending them infinitely.

Observation 2: it’s better for RAN4 to limit test configurations for REFSENS exceptions instead of extending them infinitely.

Based on the WF [1], there are two options as below.

Option 1: Only one MSD test point is specified per aggressor/victim pair of bands with following configuration template for REFSENS exceptions due to harmonic interference or cross band isolation.

| UL band | DL band | UL Fc | UL BW | UL RB Allocation | DL Fc | DL BW | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | LCRB | (MHz) | (MHz) | (dB) |

Option 2: Based on current existing table, to investigate some additional method that the table will not be horizontally expanded in next meeting.

In this paper, we provided our analysis about these two options.

For option 1, RAN4 can specify the MSD requirements accurately based on the specific UL/DL configurations. As the channel bandwidths are increasing, RAN4 don’t need to maintenance MSD table for saving efforts. However, this will result some big changes in current specification.

For option 2, one sentence or note may be clarified in the current specification in order to keep current existing table stable, but not to expand the MSD table horizontally. For the band combinations which have REFSENS exceptions due to harmonic interference or cross band isolation, the corresponding MSD requirements can be verified using one of specified test configurations even if REFSENS exceptions are not specified for some bandwidth combination configurations.

Observation 3: For the band combinations which have REFSENS exceptions due to harmonic interference or cross band isolation, the corresponding MSD requirements can be verified using one of specified test configurations even if REFSENS exceptions are not specified for some bandwidth combination configurations.

Proposal: RAN4 should make final decision to choose option 1 or 2 in this meeting in order to move forward.

Summary

Based on the analysis and discussion above, all the proposals are listed below:

Observation 1: As the channel bandwidths are increasing, it’s impossible to specify all kinds of channel bandwidth combinations as test cases.

Observation 2: it’s better for RAN4 to limit test configurations for REFSENS exceptions instead of extending them infinitely.

Observation 3: For the band combinations which have REFSENS exceptions due to harmonic interference or cross band isolation, the corresponding MSD requirements can be verified using one of specified test configurations even if REFSENS exceptions are not specified for some bandwidth combination configurations.

Proposal: RAN4 should make final decision to choose option 1 or 2 in this meeting in order to move forward.

- References

- R4-2114920, WF on MSD improvement, Huawei, HiSilicon, Skyworks Solutions Inc.

- R4-2110405, Discussion on how to simplify MSD due to harmonic interference using bandwidth-agnostic approach, Huawei, HiSilicon