3GPP TSG-RAN WG4 Meeting #101-e  		                                  R4-2119505

Electronic Meeting, 1st-12th of November, 2021

# Source:	THALES

# Title:	On the NTN UL Timing Accuracy

# Type:	discussion

# Document for:	discussion

# Agenda Item:	8.13.5.4

Release:	Rel-17

# Introduction

Previous agreements in RAN1#104-bis-e with respect to NTN UE timing estimation are as follows:

Agreement RAN1#104-bis-e: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

Moreover, during RAN4 #98-bis-e, it was also agreed:

RAN4#98-bis-e Agreements:

- The UE specific TA estimation accuracy is counted into the UE transmit timing error requirement

- UE specific TA estimation accuracy is FFS

- FFS whether the UE specific TA estimation accuracy shall be also defined as a separate accuracy requirement

- Specify UE behavior related to UE specific TA estimation and the detailed behavior is FFS

Moreover, during RAN4 #100-e, it was also agreed:

RAN4#100-e Agreements:

- Te_NTN = Te + Te_GNSS + Te_SAT

  - Te is the legacy timing error

  - Te_GNSS is the GNSS accuracy

    - Note: Te_GNSS shall include the total RTT error

    - FFS how to derive Te_GNSS from the GNSS positioning accuracy

  - Te_SAT is the serving-satellite position estimation error

    - Note: Te_SAT shall include the total RTT error

  - FFS if the equation shall be included into the specification or only Te_NTN values shall be included

- GNSS accuracy assumption for timing requirements

  - For UL SCS = 15 kHz and 30 kHz: 2-D position error is 50m

  - For UL SCS = 60kHz in FR1: FFS

The goal of this contribution is therefore to further clarify NTN UL timing synchronization requirements to be considered by NTN RAN4 work.

# UE Specific Timing Estimation Proposals

The UE initial transmission timing error shall be less than or equal to ±Te where the timing error limit value Te is specified in Table 7.1.2-1 from TS 38.133. Still according to TS 38.133, this requirement applies when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS or it is the PRACH transmission.

RAN4 should consider the NTN UE transmit timing error requirements to be the same as the ones already specified for TN UEs. This will allow reusing the same set of requirements for both terrestrial UE and non-terrestrial UE and therefore reduce NTN specification impact.

Proposal 1: The NTN UE initial transmission timing error requirement should apply when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS or it is the PRACH transmission.

The following proposal indicating how the NTN UE shall apply its TA has been agreed in RAN1#104-bis-e:

RAN1#104-bis-e Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

The following proposal indicating how the NTN UE shall apply its TA has been agreed in latest RAN1 (Oct 19th 2021):

RAN1 Agreement: Confirm the working assumption: Common TA may include parameter(s) indicating timing drift. - The UE will apply common TA according to the parameters provided by the network (if any). No offset between the common TA according to the parameters provided by the network and the actual feeder link RTT is considered when defining UE UL timing error requirements.

From our perspective, the time reference for the UE transmit timing control requirement shall be the downlink timing of the reference cell minus TTA= NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc.

The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0.

Based on the above, Option 3 is the only valid option to define the requirements related to the NTN UE self-estimated TA accuracy.

Proposal 2: The time reference for the UE transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc. Therefore, the UE transmit timing error requirement does not cover the self-TA estimation errors.

# Initial Transmit Timing Error (Te_NTN) for SCS = 15/30 kHz

From TS 38.133 we have the legacy Te (Timing Error Limit):

Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

It can be found that the proposed Te_NTN can be obtained from legacy legacy Te. If we consider just the GNSS errors (30-50m) and the NTN satellite position error (30m), the total error would correspond to maximum 50+30m=80m. If we neglect the impact of the Delay Spread, the specific timing NTN error would be 80m/(3*108m/s)=0.2(6)µs equivalent to 8.186*64*Tc, which can be added to the legacy Timing Error Limit. Therefore we obtain the following Te_NTN (Timing Error Limit Proposal):

Te_NTN Timing Error Limit Proposal

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | Te_NTN | % of CP | % of CP/2 | CP/2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 15 | (8.186+12)*64*Tc | [20.186]*64*Tc | 20.186/144 =14,02% | 20.186/72 =28,03% | 4.69/2µs or 144/2*64*Tc |
|  |  | 30 | (8.186+10)*64*Tc | [18.186]*64*Tc | 18.186/144 =12,63% | 18.186/72 =25,26% | 4.69/2µs or 144/2*64*Tc |
|  |  | 60 | (8.186+10)*64*Tc | [18.186]*64*Tc | 18.186/144 =25,26% | 18.186/72 =25,26% | 4.69/2µs or 144/2*64*Tc |
|  | 30 | 15 | (8.186+8)*64*Tc | [16.186]*64*Tc | 16.186/72 =22,48% | 16.186/36 =44,96% | 2.34/2µs or 144/4*64*Tc |
|  |  | 30 | (8.186+8)*64*Tc | [16.186]*64*Tc | 16.186/72 =22,48% | 16.186/36 =44,96% | 2.34/2µs or 144/4*64*Tc |
|  |  | 60 | (8.186+7)*64*Tc | [15.186]*64*Tc | 15.186/72 =21,09% | 15.186/36 =42,18% | 2.34/2µs or 144/4*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |  | - | - | - |
|  |  | 120 | 3.5*64*Tc |  | - | - | - |
|  | 240 | 60 | 3*64*Tc |  | - | - | - |
|  |  | 120 | 3*64*Tc |  | - | - | - |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |  |  |

Which are comparable with other values proposed by other companies in RAN4#100-e (see R4-2115402 and R4-2115346):

| SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Xiaomi | Apple | Ericsson C | QC |
| 15 | 15 | [18]*64*Tc | 32.5*64*Tc | [17.4]*64*Tc | 24*64*Tc |
|  | 30 | [16]*64*Tc | 22*64*Tc | [15.4]*64*Tc | 22*64*Tc |
|  | 60 | [16]*64*Tc | 10*64*Tc | FFS | FFS |
| 30 | 15 | [12]*64*Tc | 28.5*64*Tc | [13.4]*64*Tc | 20*64*Tc |
|  | 30 | [12]*64*Tc | 22*64*Tc | [13.4]*64*Tc | 20*64*Tc |
|  | 60 | [11]*64*Tc | 10*64*Tc | FFS | FFS |

Proposal 3:

Te_NTN Timing Error Limit Proposal

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | Te_NTN | % of CP | % of CP/2 | CP/2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 15 | (8.186+12)*64*Tc | [20.186]*64*Tc | 20.186/144 =14,02% | 20.186/72 =28,03% | 4.69/2µs or 144/2*64*Tc |
|  |  | 30 | (8.186+10)*64*Tc | [18.186]*64*Tc | 18.186/144 =12,63% | 18.186/72 =25,26% | 4.69/2µs or 144/2*64*Tc |
|  |  | 60 | (8.186+10)*64*Tc | [18.186]*64*Tc | 18.186/144 =25,26% | 18.186/72 =25,26% | 4.69/2µs or 144/2*64*Tc |
|  | 30 | 15 | (8.186+8)*64*Tc | [16.186]*64*Tc | 16.186/72 =22,48% | 16.186/36 =44,96% | 2.34/2µs or 144/4*64*Tc |
|  |  | 30 | (8.186+8)*64*Tc | [16.186]*64*Tc | 16.186/72 =22,48% | 16.186/36 =44,96% | 2.34/2µs or 144/4*64*Tc |
|  |  | 60 | (8.186+7)*64*Tc | [15.186]*64*Tc | 15.186/72 =21,09% | 15.186/36 =42,18% | 2.34/2µs or 144/4*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |  | - | - | - |
|  |  | 120 | 3.5*64*Tc |  | - | - | - |
|  | 240 | 60 | 3*64*Tc |  | - | - | - |
|  |  | 120 | 3*64*Tc |  | - | - | - |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |  |  |

Observation 1: We therefore confirm the values, but the values have to be < CP/2 because the CP has to absorb the propagation delay.

# APPENDIX - Satellite Orbit Determination and Prediction

RAN1 is currently considering different potential methods for synchronization and timing advance. More precisely, RAN1 is currently considering UE pre-compensation using GNSS and ephemeris data e.g. Position Velocity and Time (PVT) and/or orbital information.

The aim of this section is to provide some insights on how and with which accuracy satellite position and velocity can be determined and predicted.

## Satellite orbit determination

The satellite Orbit Determination (OD) consists in estimating the most likely track or orbit taken by a satellite, based on past and noisy measurements of its position and velocity. The satellite OD is useful for several reasons. It helps estimating what was the satellite past trajectory so one can identify whether the satellite is diverging from a reference orbit, and if needed, schedule satellite station keeping maneuvers to get it back on track.

An overview of a typical satellite system architecture is given in Figure 1, where the locations of each OD step are explicitly represented.

Figure 1. System Architecture Overview

In general, the satellite orbit determination is performed as follows:

Step 1: Measurements of the satellite position and velocity are made and dated. Most of the time, these measurements are GNSS-based measurements performed on-board. The accuracy of GNSS measurements depends on several factors such as:

- The GNSS receiver characteristics (mono-frequency vs. bi-frequency, single vs. multi-constellation, quality of the GNSS receiver);

- The satellite orbit, and the number of GNSS satellites in view.

Step 2: The measurements are collected in the NTN Control Center (NCC) where the satellite OD will be performed. In general, the satellite GNSS measurements are delivered to the ground via dedicated telemetry channels between the satellite and the NTN GW network. The reporting period of new GNSS measurements is implementation specific but it is directly related to the accuracy target associated to the instantaneous knowledge of the satellite position and velocity in the system.

Step 3: The satellite OD is performed in the NCC. This operation can be more or less complex depending on the models considered, the quantity of measurements available and the algorithms used. Traditionally, two filtering techniques are used on input measurements: Kalman filtering or LS (least square) filtering. Least square filtering usually provides more accurate results but requires more computing power.

Hereafter, we provide several examples of orders of magnitude of precision, with values that can be expected for orbit determination. This performance of orbit determination shall not be confused with orbit prediction performance which concern the future satellite trajectory (and are discussed in the next section).

- Observation 2: One shall distinguish between orbit determination performance based on past measurements of the satellite trajectory and orbit prediction performance that concerns the future satellite trajectory.

Case 1: “Best” Precision Orbit Determination

For satellite systems designed for navigation purposes delivering very accurate and abundant GNSS measurements, it is possible to achieve the following performance:

- 3D Position RMS Error = 0.05 m

- 3D Velocity RMS Error = 0.05 mm/s

However, in commercial telecommunication satellites such performance cannot be met because the cost would be prohibitive and such OD precision is not required for the system to work.

Case 2: “Typical” Precision Orbit Determination

For telecommunication satellites, it is more typical to consider the following values:

- 3D Position RMS Error = 0.5 m

- 3D Velocity RMS Error = 0.5 mm/s

Case 3: “Low Quality” Precision Orbit Determination

Finally, for low cost system the following performance can be considered:

- 3D Position RMS Error = 5 m

- 3D Velocity RMS Error =5 mm/s

As a rule of thumb, it can be assumed that there is a factor of 1000 between the position error (in [m]) and the velocity error (in [m/s]). This is important to keep in mind when defining accuracy targets for satellite position and velocity.

- Observation 3: As a rule of thumb, it can be assumed that there is a factor of 1000 between the position error (in [m]) and the velocity error (in [m/s]). Is important to keep in mind this rule when allocating an error budget for satellite position and velocity estimations.

## Satellite orbit prediction

The satellite orbit determination can also be used to derive updated satellite ephemeris. The satellite ephemeris are useful to predict future satellite positions and velocity vectors. This can be useful for various reasons such as:

- Determine when the satellite is going to fly by over a specific zone of interest;

- Anticipate collision-avoidance operations;

- Anticipate future feeder-link switchover operations within the NTN gateways network;

- Perform Doppler compensation (for instance on the feeder links between the satellite and the serving NTN GW).

Of course, the accuracy of the orbit prediction required to perform such operations can be significantly different from an application to another.

Finally, the NCC can share the satellite ephemeris (under various formats) with other entities in the system responsible for performing such operations (e.g. with the NTN GW).

Then, one can wonder with which accuracy one can predict the future satellite position and velocity vectors and over what time horizon.

The orbit prediction accuracy may depend on several factors:

- The accuracy of the orbit determination used to derive the satellite ephemeris: this will drive the initial errors of the propagation model.

- The accuracy of the orbit propagation model: there are many propagation models which take into account more or less precisely the physical effects on the satellite movement (J2/4 perturbations, aero-dynamical drag, and anticipated station keeping maneuvers).

- The time horizon over which the prediction is made: the initial errors combined with the propagation model errors tend to increase the prediction errors when the prediction period increases.

- Observation 4: The orbit prediction accuracy depends on:

- The accuracy of the orbit determination used to derive the satellite ephemeris;

- The accuracy of the orbit propagation model;

- The time horizon over which the prediction is made.

## Orbit prediction error at NTN Control Center

In this section, we tried to quantify the propagation error evolution depending on the initial PV errors. The main assumptions and results of the study are summarized in Table 1.

A simplified relative position-velocity model for circular orbits (Clohessy-Wiltshire or CW) has been considered. The CW equations are given in Figure 2 where the z-axis is along the radius vector of the target spacecraft, the y-axis is along the angular momentum vector of the target spacecraft, and the x-axis completes the right-handed system. These equations are usually used for “rendez-vous” or docking relative position evaluations.

However, the same equations are used here to evaluate the propagation error evolution between the reference orbit and the predicted orbit, which progressively moves away from the reference orbit due to propagation errors.

Figure 2. Clohessy-Wiltshire equations of motion

This study characterizes only the impact of the initial PV error on the prediction errors. The additional errors related to the orbit propagation model limits are not simulated here. However, when considering short prediction period (i.e. inferior to the orbital period), the propagation errors induced by the propagator models limitations (J2/J4 perturbation, air drag, etc.) may be considered negligible w.r.t. to the prediction errors induced by the initial PV errors.

Effect of initial errors on tangential (X), normal (Y), radial (Z) axes in position and velocity gives very different results. The worst case effects are observed when the initial error is applied on Z axis for position and X axis for velocity.

For illustrative purpose, the propagation errors over 2 revolution orbits for each worst case initial errors on position (dz0 = 1 m) and on velocity (dvx0 = 1mm/s) have been illustrated in Figure 3 and Figure 4 respectively. It can be observed that the velocity error is not an increasing function of time.

Figure 3. Propagation errors – dZ0 = 1 m

Figure 4. Propagation errors – dvX0 = 1 mm/s

For initial PV errors, the POD assumptions presented in the previous section have been considered.

Finally, one shall define a PV accuracy target to be guaranteed. When the propagation errors exceed the PV accuracy target then it means that a new orbit determination shall be performed and updated ephemeris shall be provided. The PV accuracy target is going to be overshoot after a more or less long prediction period depending on the initial PV error assumption.

In this study, the target PV accuracy has been driven by the accuracy expected to perform Doppler pre/post-compensation on feeder links. The Doppler compensation residual errors experienced on the feeder link shall be almost transparent to the gNB/UE. Then, it shall be an order of magnitude lower than the frequency error budget allocated at UE side (~0.1 ppm). Moreover, the carrier frequencies used on the feeder links may be significantly higher than the one used on the service link so additional margin shall be considered to counter shift this effect. Therefore, it is proposed to assume that the satellite radial velocity w.r.t the NTN GW should be known with an accuracy better than +/- 0.3 m/s when performing Doppler compensation on the feeder link. It has been demonstrated by simulation (see Figure 5 and Table 2) that when the following assumptions on the PV errors are met the previous requirement is guaranteed:

- Gaussian position error distribution with 3D Position RMS Error = 10 m => 3sigma = 30 m

- Gaussian velocity error distribution with 3D Velocity RMS Error = 10 mm/s => 3sigma = 30 mm/s

Therefore, it is proposed to consider the following PV accuracy target:

- Position error < 30 m

- Velocity error < 30 mm/s

Observation 5: The PV accuracy target reference hypothesis could use Position error < 30 m and Velocity error < 30 mm/s.

The simulation results are summarized in Table 2. It can be observed that the maximal propagation time before overshooting the PV accuracy target is significantly driven by the initial error. More precisely, depending on the quality of the orbit determination, the maximal propagation time can go from 8 hours to less than 10 minutes. The less accurate is the orbit determination, the more often orbit determination shall be performed based on more recent measurements and updated satellite ephemeris shall be distributed.

It can be also observed that the velocity error degradation is negligible w.r.t the position error. As a consequence, the radial velocity (or Doppler) error is mainly driven by the position error degradation.

In conclusion, it has been demonstrated that even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably. The limiting condition is that the system should be designed such that new orbit determination and satellite ephemeris update can be performed frequently enough.

- Observation 6: Even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably.

Figure 5: Monte Carlo simulation to evaluate radial velocity error [m/s] distribution assuming a PV Gaussian errors with (3D Position RMS Error = 10 m / 3D Velocity RMS Error = 10 mm/s)

Table 1. Monte Carlo simulation parameters

| Satellite orbit | Polar orbit (i = 90°) |
| --- | --- |
| GW position | Equatorial ground station Min elevation = 30° |
| Error on position/velocity | Gaussian distribution on norm - 3D Position RMS Error = 10 m - 3D Velocity RMS Error = 10 mm/s Uniform distribution on direction |

Table 2. Results of propagation error study

| Model | Kepler Model Clohessy-Wiltshire equations |
| --- | --- |
| Orbit | Circular @ 600 km Period = 5801 s |
| Initial PV Errors | Case 1 : “Best” Precision Orbit Determination - 3D Position RMS Error = 0.05 m - 3D Velocity RMS Error = 0.05 mm/s Case 2 : “Typical” Precision Orbit Determination - 3D Position RMS Error = 0.5 m - 3D Velocity RMS Error = 0.5 mm/s Case 3 : “Low Quality” Precision Orbit Determination - 3D Position RMS Error = 5 m - 3D Velocity RMS Error =5 mm/s |
| PV accuracy target | Position error < 30 m Velocity error < 30 mm/s |
| Prediction period before overshooting PV accuracy target | Case 1 : “Best” Precision Orbit Determination Propagation to 5 orbital periods before overshooting PV accuracy target ~ 8 h - Statistics on max position error - Mean = 4.35 m - Std dev = 4.10 m - Max = 35.9 m - Statistics on max velocity error - Mean = 0.3 mm/s - Std dev = 0.3 mm/s - Max = 2.6 mm/s Case 2 : “Typical” Precision Orbit Determination Propagation to 0.5 period before overshooting PV accuracy target ~ 48 mn - Statistics on max position error - Mean = 4.63 m - Std dev = 4.23 m - Max = 36.7 m - Statistics on max velocity error - Mean = 0.3 mm/s - Std dev = 0.3 mm/s - Max = 2.6 mm/s Case 3 : “Low Quality” Precision Orbit Determination Propagation to 0.1 period before overshooting PV accuracy target ~ 10 mn - Statistics on max position error - Mean = 5.65 m - Std dev = 3.61 m - Max = 28.9 m - Statistics on max velocity error - Mean = 7.5 mm/s - Std dev = 5.1 mm/s - Max = 46 mm/s |

In above results the PV accuracy targets are Position error < 30 m and Velocity error < 30 mm/s. Thereby, in case of “Low Quality” Precision Orbit Determination, propagation to 0.1 period before overshooting PV accuracy target is needed. That is, in this case an OD shall be performed every 10 minutes to achieve such requirement on PV accuracy. In case of “Typical” Precision Orbit Determination, propagation to 0.5 period before overshooting PV accuracy target is enough and then an OD shall be performed every 48 minutes.

Therefore, higher performance can be achieved when the Orbit Determination (OD) is performed frequently and propagation period is shortened to a fraction of orbit period (up to few of minutes).

As can be seen in the following Table 3 and Table 4: By taking into account propagation model errors, satellite position can be predicted 60 seconds ahead with only max error of 1.47m and 5 minutes ahead with max error of 3.87m in case of  initial 3D Position RMS Error = 0.5 m and 3D Velocity RMS Error = 0.5 mm/s (Typical” Precision Orbit Determination used for telecommunication satellites).

Table 3 Satellite PV errors - propagation model errors not taken into account

Table 4 Satellite PV errors - propagation model errors taken into account

- Observation 7: Typical Precision Orbit Determination (initial 3D Position RMS Error = 0.5 m and 3D Velocity RMS Error = 0.5 mm/s) allows Satellite position prediction 60 seconds ahead with max error of 1.47m and 5 minutes ahead with max error of 3.87m.

# Conclusions

Proposal 1: The NTN UE initial transmission timing error requirement should apply when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS or it is the PRACH transmission.

Proposal 2: The time reference for the UE transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc. Therefore, the UE transmit timing error requirement does not cover the self-TA estimation errors.

Proposal 3:

Te_NTN Timing Error Limit Proposal

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | Te_NTN | % of CP | % of CP/2 | CP/2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 15 | (8.186+12)*64*Tc | [20.186]*64*Tc | 20.186/144 =14,02% | 20.186/72 =28,03% | 4.69/2µs or 144/2*64*Tc |
|  |  | 30 | (8.186+10)*64*Tc | [18.186]*64*Tc | 18.186/144 =12,63% | 18.186/72 =25,26% | 4.69/2µs or 144/2*64*Tc |
|  |  | 60 | (8.186+10)*64*Tc | [18.186]*64*Tc | 18.186/144 =25,26% | 18.186/72 =25,26% | 4.69/2µs or 144/2*64*Tc |
|  | 30 | 15 | (8.186+8)*64*Tc | [16.186]*64*Tc | 16.186/72 =22,48% | 16.186/36 =44,96% | 2.34/2µs or 144/4*64*Tc |
|  |  | 30 | (8.186+8)*64*Tc | [16.186]*64*Tc | 16.186/72 =22,48% | 16.186/36 =44,96% | 2.34/2µs or 144/4*64*Tc |
|  |  | 60 | (8.186+7)*64*Tc | [15.186]*64*Tc | 15.186/72 =21,09% | 15.186/36 =42,18% | 2.34/2µs or 144/4*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |  | - | - | - |
|  |  | 120 | 3.5*64*Tc |  | - | - | - |
|  | 240 | 60 | 3*64*Tc |  | - | - | - |
|  |  | 120 | 3*64*Tc |  | - | - | - |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |  |  |

Observation 1: We therefore confirm the values, but the values have to be < CP/2 because the CP has to absorb the propagation delay.

Observation 2: One shall distinguish between orbit determination performance based on past measurements of the satellite trajectory and orbit prediction performance that concerns the future satellite trajectory.

Observation 3: As a rule of thumb, it can be assumed that there is a factor of 1000 between the position error (in [m]) and the velocity error (in [m/s]). Is important to keep in mind this rule when allocating an error budget for satellite position and velocity estimations.

Observation 4: The orbit prediction accuracy depends on:

- The accuracy of the orbit determination used to derive the satellite ephemeris;

- The accuracy of the orbit propagation model;

- The time horizon over which the prediction is made.

Observation 5: The PV accuracy target reference hypothesis could use Position error < 30 m and Velocity error < 30 mm/s.

Observation 6: Even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably.

- Observation 7: Typical Precision Orbit Determination (initial 3D Position RMS Error = 0.5 m and 3D Velocity RMS Error = 0.5 mm/s) allows Satellite position prediction 60 seconds ahead with max error of 1.47m and 5 minutes ahead with max error of 3.87m.

END