3GPP TSG-RAN WG4 Meeting #101-e                         	         R4-2119540

# Electronic meeting, 1st November – 12th November, 2021

Agenda item:	8.1.2.3

Source:	Huawei, HiSilicon

Title:	On measurement uncertainty of NR UE FR1 MIMO OTA

Document for:	Approval

# Introduction

In RAN4 #100e meeting, it was agreed that in order to facilitate the requirements discussion, RAN4 should discuss the preliminary MU assessment for NR UE FR1 MIMO OTA [1]. This document discusses our view on this topic.

Issue 2-4: FR1 MU assessment

- Agreements

  - RAN4 should discuss the preliminary MU assessment for FR1 MIMO OTA including example expanded uncertainty, final MU and TT will be decided by RAN5.

  - WID is suggested to update in Sep RAN plenary meeting, to reflect this working scope extension, for easy tracing the progress in the status report of WI.

# Discussion

During previous study of radiated multi-antenna reception performance for E-UTRA UE, Maximum Test System uncertainties of Radiated Performance of Multiple-antenna Receivers was accessed as ±2.65 dB for MPAC. Test tolerance was specified as 1 dB accordingly. (Annex D of TS 37.544). The details of Measurement uncertainty budget contributors were described in Annex N of TS 37.544, cited in the annex of this document.

For the current study of NR UE FR1 MIMO OTA, measurement uncertainty budget contributors are listed in TR 38.827 [4] as below. Compared with E-UTRA study mentioned above, all non-zero contributors have been considered in NR, as shown in last column of below table.

Observation 1: all non-zero MU contributors of E-UTRA MPAC have been considered in NR UE FR1 MIMO OTA.

It is the fact that NR FR1 MPAC setting is similar as E-UTRA except 16 probes vs. 8 probes. Therefore it is expected that at least for frequency range of 410MHz<f≤3GHz, MU of NR FR1 MPAC is close to E-UTRA. While the impact of higher frequency range (3GHz <f≤7.125GHz) needs further study.

Proposal 1: for frequency range of 410MHz<f≤3GHz, use previous MU study in E-UTRA as reference for preliminary MU assessment of NR UE FR1 MIMO OTA, i.e. ±2.65 dB. While the impact of higher frequency range (3GHz <f≤7.125GHz) needs further study.

Table B.1.1-1: Measurement uncertainty budget for MPAC (from TR 38.827)

| UID | Description of uncertainty contribution | Example value (410MHz<f≤3GHz) | Example value (3GHz <f≤7.125GHz) | Distribution of the probability | Details in | Comments (E-UTRA Std values in TS 37.544) |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process |  |  | U-Shaped | B.1.2.1 | 0.00 |
| 2 | Measure distance uncertainty |  |  | Normal | B.1.2.2 | 0.00 |
| 3 | Quality of quiet zone |  |  | Rectangular | B.1.2.3 | 0.50 |
| 4 | Base Station simulator |  |  | Rectangular | B.1.2.4 | 0.58 |
| 5 | Channel Emulator -absolute value -stability |  |  | Normal | B.1.2.5 | 0.84 |
| 6 | Amplifier uncertainties |  |  | Rectangular | B.1.2.6 | 0.17+0.06+0.17 |
| 7 | Random uncertainty |  |  | Normal | B.1.2.7 | 0.12 |
| 8 | Throughput measurement: output level step resolution |  |  | Rectangular | B.1.2.8 | 0.14 |
| 9 | DUT sensitivity drift |  |  | Rectangular | B.1.2.9 | 0.12 |
| 10 | Signal flatness |  |  | Normal | B.1.2.10 | FFS |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna |  |  | U-Shaped | B.1.2.11 | 0.14 |
| 12 | Reference antenna positioning misalignment |  |  | Normal | B.1.2.12 |  |
| 13 | Quality of quiet zone |  |  | Rectangular | B.1.2.3 | 0.29 |
| 14 | Total uncertainty of the Network Analyzer |  |  | Rectangular | B.1.2.13 | 0.29 |
| 15 | Uncertainty of an absolute gain of the calibration antenna |  |  | Normal | B.1.2.14 | 0.50 |
| 16 | Offset of the Phase Center of the Reference Antenna |  |  | Normal | B.1.2.15 | 0.00 |
| Note: E-UTRA expanded uncertainty at 95% confidence level = 1.96 * Combine all the standard uncertainties by the Root of the Sum of the Squares (RSS) method = ±2.65 dB |  |  |  |  |  |  |

# Conclusions

This document discusses the open issues of EN-DC test methodologies identified in the WF [2].

Observation 1: all non-zero MU contributors of E-UTRA MPAC have been considered in NR UE FR1 MIMO OTA.

Proposal 1: for frequency range of 410MHz<f≤3GHz, use previous MU study in E-UTRA as reference for preliminary MU assessment of NR UE FR1 MIMO OTA, i.e. ±2.65 dB. While the impact of higher frequency range (3GHz <f≤7.125GHz) needs further study.

# Reference

- R4-2115756 WF on NR MIMO OTA vivo, CAICT

- TR 37.977 V16.0.0

- TS 37.544 V16.0.0

- TR 38.827 V16.1.0

# ANNEX

Table N.1-1: Measurement uncertainty budget for Multiprobe Anechoic method (from TS 37.544)

|  | Description of uncertainty contribution | Details in | MPAC |  |  |
| --- | --- | --- | --- | --- | --- |
| # | Stage 1-DUT measurement |  | Example Value [dB] | Prob Distr | Std Uncertainty [dB] |
| 1 | Mismatch of transmitter chain (i.e. between fixed measurement antenna and base station simulator) | TS 34.114, E.1-E.2 | 0.00 | u-shape | 0.00 |
| 2 | Insertion loss of transmitter chain | TS 34.114, E.3-E.5 | 0.00 | rect | 0.00 |
| 3 | Influence of the fixed measurement antenna cable | TS 34.114, E.6 | 0.00 | rect | 0.00 |
| 4 | Uncertainty of the absolute antenna gain of the fixed measurement antenna | TS 34.114, E.7 | 0.00 | rect | 0.00 |
| 5 | Base station simulator: uncertainty of the absolute output level | TS 34.114, E.17 [TS 36.521-1 F.1.3] | 1.00 | rect | 0.58 |
| 6 | Throughput measurement: output level step resolution | TS 34.114, E.18 | 0.25 | rect | 0.14 |
| 7 | Statistical uncertainty of throughput measurement | TS34.114, E.19 | FFS (negligible and partially included in repeatability) |  |  |
| 8 | Fading channel emulator output uncertainty (if used) - absolute output power - output signal stability - output stability with temperature | N.2 | 1.5dB 0.5dB 0.4dB | normal (output power) rect (stability) | 0.84 |
| 9 | AWGN flatness within LTE band | TBD (NOTE 4) |  |  | FFS |
| 10 | Signal-to noise ratio uncertainty, averaged over downlink transmission Bandwidth | TBD (NOTE 4) |  |  | FFS |
| 11 | Channel model implementation (NOTE 2) | TBD | FFS |  | FFS |
| 12 | Chamber statistical ripple and repeatability | TS 34.114, E.26.A | N/A |  | 0.00 |
| 13 | Additional power loss in EUT chassis | TS 34.114, E.26.B | N/A |  | 0.00 |
| 14 | Quality of the quiet zone | TS 34.114, E.10 | 0.50 | std | 0.50 |
| 15 | Measurement Distance - VSWR - Chamber Standing Wave | TS 34.114, E.9 | 0.00 |  | 0.00 |
| 16 | DUT sensitivity drift | TS 34.114, E.21 | 0.20 | rect | 0.12 |
| 17 | Uncertainty related to the use of the phantoms: |  |  |  | 0.00 |
|  | a) Uncertainty of dielectric properties and shape of the hand phantom | TR 25.914, A.12.3 |  |  | 0.00 |
|  | b) Uncertainty related to the use of laptop ground plane phantom | TR 25.914, A.12.4 |  |  | 0.00 |
| 18 | Random uncertainty (repeatability) | TS 34.114, E.14 | 0.20 | rect | 0.12 |
| 19 | Uncertainty associated with the stirring method and number of subframes (NOTE 3) |  | N/A |  | 0.00 |
|  | Stage 2-Calibration measurement |  |  |  | 0.00 |
| 20 | Uncertainty of network analyzer - Receiver and Source VNA - Receiver VNA Calibration prior to measurement | TS 34.114, E.15 | 0.50 | rect | 0.29 |
| 21 | Mismatch of transmitter chain | TS 34.114, E.1-E.2 | 0.20 | u-shape | 0.14 |
| 22 | Insertion loss of transmitter chain | TS 34.114, E.3-E.5 | 0.00 |  | 0.00 |
| 23 | Mismatch in the connection of calibration antenna | TS 34.114, E.1 | 0.00 | rect | 0.00 |
| 24 | Influence of the calibration antenna feed cable | TS 34.114, E.6 |  |  | 0.00 |
| 25 | Influence of the transmitter antennas/probes cables | TS 34.114, E.6 | 0.00 | rect | 0.00 |
| 26 | Uncertainty of the absolute gain of the transmitter antennas/probes | TS 34.114, E.7 | 0.00 | rect | 0.00 |
| 27 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | TS 34.114, E.16 | 0.50 | std | 0.50 |
| 28 | Chamber statistical ripple and repeatability | TS 34.114, E.26.A |  |  | 0.00 |
| 29 | Phase Centre Offset (when using horn to calibrate) | TS 34.114, E.9 | 0.00 | rect | 0.00 |
| 30 | Quality of the quiet zone (Range Ref. Antenna) | TS 34.114, E.10 | 0.50 | rect | 0.29 |
|  | External Amplifiers |  |  |  | 0.00 |
| 31 | Stability | N.3.1 | 0.30 | rect | 0.17 |
| 32 | Linearity | N.3.2 | 0.10 | rect | 0.06 |
| 33 | Noise Figure | N.3.3 | 0.30 | rect | 0.17 |
| 34 | Mismatch | N.3.4 | 0.00 | rect | 0.00 |
| 35 | Gain | N.3.5 | 0.00 | rect | 0.00 |