3GPP TSG-RAN WG4 Meeting#101e      	R4-2119786

E-meeting, 1st – 12th Nov, 2021

Title:	TP to TR 37.717-11-21: DC_8A_n1A-n28A

Source:	Nokia, Telefonica

Agenda item:	7.18.2

Document for:	Approval

# 1	Introduction

This contribution introduces DC_8A_n1A-n28A into TP to TR 37.717-11-21.

******************************** Start of the TP ********************************************

## 6.X	DC_8_n1-n28

### 6.X.1	 Operating bands for DC

Table 6.X.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 6.X.2 	Configuration for DC

Table 6.X.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n1A-n28A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |

### 6.X.3	 Co-existence studies

When Uplink EN-DC configuration is DC_8_n28, 5th IMD will fall into Rx band of Band 1. However, when consider the 5MHz UL channel BW for DC_8_n28, the IMD5 does not fall into the B1 own Rx band. This was decision made for DC_1-8_n28 in 37.716-21-11.

IMD4 caused by UL DC_8_n1 may fall into Rx band n28.

### 6.X.4	 ∆TIB and ∆RIB values

Relaxation values of DC_1A-8A_n28A are re-used

Table 6.X.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n28 | 8 | 0.6 |
|  | n1 | 0.3 |
|  | n28 | 0.6 |

Table 6.X.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n28 | 8 | 0.2 |
|  | n28 | 0.2 |

### 6.X.5	MSD

IMD4 MSD is copied from DC_8A_n28A-n78A and IMD5 treatment from DC_2A-5A_n12A is reused

Table 6.X.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A_n1A-n28A1 | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

******************************** End of the TP ********************************************