3GPP TSG RAN WG4 Meeting #101-e	R4-2119816

Online, 1st – 12th Nov. 2021

Agenda item:	7.14.2

Source:	SoftBank Corp., LG Electronics

Title:	TP for TR 37.717-21-11: EN-DC_8-41_n77

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_8-41_n77 was approved in RAN#93[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 2-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-41A_n77A DC_8A-41C_n77A | DC_8A_n77A DC_41A_n77A DC_41C_n77A |

3.  Co-existence

When Uplink EN-DC configuration is DC_8A_n77A, (1) IMD2 of (B8 - n77) will fall into Rx band of Band 41. When Uplink EN-DC configuration is DC_41A_n77A, (2) IMD2/3/5 of (B41 - n77) will fall into Rx band of Band 8.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 4-1: ΔTIB of constituent 2 band combo.

| ΔTIB | B8+B41 | B8+n77 | B41+n77 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0.3 | 0.6 |  | 0.6 |
| B41 | 0.3 |  | 0.3 | 0.3 |
| n77 |  | 0.8 | 0.8 | 0.8 |

Table 4-2: ΔRIB of constituent 2 band combo.

| ΔRIB | B8+B41 | B8+n77 | B41+n77 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0 | 0.2 |  | 0.2 |
| B41 | 0 |  | 0 | 0 |
| n77 |  | 0.5 | 0.5 | 0.5 |

Then the proposal for this 2LTE-1NR EN-DC is:

Table 4-3: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n77 | 8 | 0.6 |
|  | 41 | 0.3 |
|  | n77 | 0.8 |

Table 4-4: ΔRIB,c

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n77 | 8 | 0.2 |
|  | 41 | 0 |
|  | n77 | 0.5 |

5.  Reference sensitivity exceptions

As mentioned in section 3, REFSENS exceptions due to IMD2/3/5 are expected. The following values are proposed.

Table 5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-41A_n77A DC_8A-41C_n77A | 41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 29.1 | FDD | IMD21, 4 |
| DC_8A-41A_n77A DC_8A-41C_n77A | 8 | 895 | 5 | 25 | 940 | N/A | FDD | N/A |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |
|  | 41 | 2650 | 5 | 25 | 2650 | 28.0 | TDD | IMD2 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

6.  Conclusion

This paper proposes study results of EN-DC 8-41_n77.

TP is shown in Annex.

7.  Reference

[1]  RP-212174  WID revision: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL); rapporteur: Huawei

Annex.   Proposed text for TR37.717-21-11:

[Unchanged Parts Skipped]

5.x	DC_8-41_n77

### 5.x.1	Configurations for DC_8-41_n77

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-41A_n77A DC_8A-41C_n77A | DC_8A_n77A DC_41A_n77A DC_41C_n77A |

### 5.x.2	Co-existence studies

When Uplink EN-DC configuration is DC_8A_n77A, (1) IMD2 of (B8 - n77) will fall into Rx band of Band 41. When Uplink EN-DC configuration is DC_41A_n77A, (2) IMD2/3/5 of (B41 - n77) will fall into Rx band of Band 8. But the synchronous operation is considered to DC_41A_n77A, (1) do not need to be considered.

### 5.x.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n77 | 8 | 0.6 |
|  | 41 | 0.3 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n77 | 8 | 0.2 |
|  | 41 | 0 |
|  | n77 | 0.5 |

### 5.x.4	Reference sensitivity exceptions

As mentioned above, IMD2/3/5 of B41 and n77 to Band8 Rx and IMD2 of B8 and n77 to Band41 need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.x.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-41A_n77A DC_8A-41C_n77A | 41 | 2630 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 29.1 | FDD | IMD21, 4 |
| DC_8A-41A_n77A DC_8A-41C_n77A | 8 | 895 | 5 | 25 | 940 | N/A | FDD | N/A |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |
|  | 41 | 2650 | 5 | 25 | 2650 | 28.0 | TDD | IMD2 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

[Unchanged Parts Skipped]