3GPP TSG-RAN WG4 Meeting # 101-e 					     R4-2120320

Electronic Meeting, 1th – 12th Nov., 2021

Agenda item:			8.19.3.1

Source:	Intel

Title:	WF on RRM impact for unified TCI in FeMIMO

Document for:	Approval

# Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FeMIMO RRM in RAN4 #101 meeting.

# Agreements in GTW session

In GTW session, the following bullets have been agreed

  - Define the following requirements

    - DCI and MAC-CE based TCI switching delay requirements

    - DL/UL TCI switching requirements for both joint TCI and separate TCI.

    - TCI switching delay requirements for the case when target TCI is associated with one of serving cell or non-serving cell (i.e., cell with different PCI)

      - Define requirements at least for Rel-17 TCI state switching framework

    - UL TCI state switching delay requirements when target TCI is associated with DL-RS

    - Pathloss RS switch delay requirement for beam alignment case

# WF

### Issue 1: PL-RS switching delay requirement for beam alignment case

  - Don’t need to specify additional PL-RS switching delay requirement.

  - RAN4 will further discuss whether UL TCI state switching requirements can be re-used for PL-RS switching delay if UL TCI state switching can trigger PL-RS switch, how to capture this in the spec is FFS.

### Issue 2: MAC-CE based DL/UL TCI switching delay in separate TCI mode for serving cell

  - Re-use existing known conditions for associated DL-RS

  - Re-use existing MAC-CE based TCI switching delay requirements for DL TCI switching delay requirements for PDCCH and PDSCH

  - Reuse existing MAC-CE based uplink spatial relation switching requirements for UL TCI switching delay as starting points

  - FFS: Whether and how to specify requirements for MAC-CE based TCI state-pair indication, considering such one TCI state-pair containing 1 DL TCI and 1 UL TCI, and the source RSs for the 2 TCIs are different.

### Issue 3: MAC-CE based DL/UL TCI switching delay in Joint TCI mode for serving cell

  - Re-use existing known conditions for associated DL-RS

  - RAN4 will further discuss whether to define a total switching delay requirement or re-use the legacy requirements or DL/UL TCI switching delay defined in separate mode

### Issue 4: DCI based TCI state switching delay for serving cell

  - RAN4 will further discuss DCI based TCI switching delay requirements in RAN4 based on the clear decision in RAN1

  - RAN4 will further discuss the below conditions for DCI based switching requirement

  - DCI based TCI switch (for joint or UL TCI) requirements are defined for known TCI, target TCI in active TCI list (for joint only) and PL-RS is maintained

  - DCI-based TCI state-pair switch is not precluded, considering such one TCI state-pair containing 1 DL TCI and 1 UL TCI, and the source RSs for the 2 TCIs are different.

### Issue 5: Multiple TCI switching delay

  - RAN4 will deprioritize the discussion about whether to specify the requirements for multiple TCI switching delay

### Issue 6: Common TCI switching delay for CA case

  - RAN4 will further discuss whether to specify the requirements for common TCI switching delay for CA case

### Issue 7: TCI switch delay requirements for target TCI is associated with non-serving cell (i.e., cell with different PCI)

  - Re-use existing known conditions for associated DL-RS

  - FFS: Re-use existing MAC-CE based TCI switching delay requirements for DL TCI switching delay requirements for PDCCH and PDSCH

  - Reuse existing MAC-CE based uplink spatial relation switching requirements for UL TCI switching delay as starting points.

  - RAN4 further discuss how to elaborate the wording for “non-serving cell”