3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120700

Electronic Meeting, November 01-12, 2021

Agenda item:			8.8.3

Source:	CMCC

Title:	WF on FR1 HST demodulation

Document for:	Approval

Introduction

This is the WF to capture all agreements and open issues in [101-e][319] NR_HST_FR1_Demod for RAN4#101e meeting

Topic #1: PDSCH requirements for CA scenarios

Issue 1-1: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme

- Agreements in GTW:

| Scenario | UE is capable of: | Applicability for UE capable of HST-SFN advanced receiver | Applicability for UE not capable of HST-SFN advanced receiver |
| --- | --- | --- | --- |
| 1 | (Test 1) FDD 15 + TDD 30 and (Tet 3) TDD 30 + TDD 30 | HST-SFN for (Test 1) HST-DPS for (Test 3) | HST-DPS for (Test 1) and (Test 3) |
| 2 | (Test 2) FDD 15 + FDD 15 and (Test 3) TDD 30 + TDD 30 | HST-SFN for (Test 2) HST-DPS for (Test 3) | HST-DPS for (Test 2) and (Test 3) |
| 3 | Only (Test 1) FDD 15 + TDD 30 | HST-SFN and HST-DPS | HST-DPS |
| 4 | Only (Test 2) FDD 15 + FDD 15 only | HST-SFN and HST-DPS | HST-DPS |
| 5 | Only (Test 3) TDD 30 + TDD 30 | HST-SFN and HST-DPS | HST-DPS |

Issue 1-2: UE capability for HST-SFN CA

- Option 1: define a new Rel-17 UE capability to indicate the support of HST-SFN CA

  - Option 1a: the granularity of the capability is a per-UE

  - Option 1b: the granularity of the capability is per band combination

  - Option 1c: the granularity of the capability is per band

- Option 2: Do not introduce additional UE capability for HST-SFN CA.

Issue 1-3: Applicability rule between single carrier and CA

- Agreement:

  - Define the applicability rule between single carrier and CA as UE can skip single carrier test case if it explicitly passes the corresponding CA test case for the same SCS. The existing applicability rules for single carrier requirements would still apply.

- If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

- UE need to pass Rel-16 FDD HST single tap test.

- UE can skip Rel-16 TDD HST single tap and Rel-15 HST single tap tests

- If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

- UE need to pass Rel-16 FDD HST single tap test.

- UE can skip Rel-16 TDD HST single tap and Rel-15 HST single tap tests

Issue 1-4: Release independent

- Option 1: Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15.

- Option 2:

  - HST-DPS CA requirements are release independent from Rel-15

  - HST-SFN CA requirements are applicable from Rel-17

- Option 3:

  - Align the release number of demodulation requirements with RRM requirements.

  - No more release independent discussion for demodulation requirements.

Issue 1-5: Simulation

- Agreements in 1st round:

  - RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes [2.5 dB] or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within [2.5 dB]

CR work split

| CR | Responsible company |
| --- | --- |
| Applicability rule | Qualcomm |
| HST-SFN CA requirements for 2Rx | Intel |
| HST-SFN CA requirements for 4Rx | CMCC |
| HST DPS CA requirements for 2Rx | Huawei |
| HST DPS CA requirements for 4Rx | Apple |
| FRC | Ericsson |

- Reference

R4-2103876 WF on UE demodulation for FR1 HST, RAN4#98-e

R4-2106098 WF on UE demodulation for FR1 HST, RAN4#98-bis-e

R4-2108635 WF on UE demodulation for FR1 HST, RAN4#99-e

R4-2115724 WF on UE demodulation for FR1 HST, RAN4#100-e