3GPP TSG-RAN WG4 Meeting # 101-e 												R4-21xxxxx

Electronic Meeting, 1– 12 Nov, 2021

Agenda item:			5.1.1.2, 5.1.6.2.1, 5.1.6.2.2, 5.1.6.2.3, 5.2.2

Source:	Moderator (OPPO)

Title:	Email discussion summary for [101-e][102] R16_Maintenance

Document for:	Information

Introduction

This summary covers the papers submitted in agenda 5.1.1.2, 5.1.6.2.1, 5.1.6.2.2, 5.1.6.2.3, 5.2.2 which are targeting R16 maintenance for 38.307, 38.101-1, 38.101-2, 38.101-3 and 36.101.

Topic #1: 38.307

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117552 | Nokia | Release independence information for shared spectrum access is added |
| R4-2117534 | Nokia | draftCR 38.307: Addition of release independence information for FR2 PC5 R15 |
| R4-2117535 | Nokia | draftCR 38.307: Addition of release independence information for FR2 PC5 R16 |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117552 R4-2117553 | draftCR 38.307: Addition of release independence information for shared spectrum access R16 CATB |
| R4-2117534 | draftCR 38.307: Addition of release independence information for FR2 PC5 R15 |
| R4-2117535 | draftCR 38.307: Addition of release independence information for FR2 PC5 R16 |

Summary for 1st round

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

Topic #2: 38.101-1

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117861 R4-2117862 | MediaTek | Draft CR for TS 38.101-1: Missing MOP for NR DC |
| R4-2117512 | Qualcomm | Observation 1: If RSS-195 is used, then the link performance will suffer from the extra back-off required for the 5MHz channel BW in Canada and not in the US. The Canadian and US regulatory requirements are usually aligned. Proposal 1: Further clarify from the Canadian authorities as to the intention of RSS-195 to follow the FCC requirement for WCS 2300MHz band. |
| R4-2117960 | Apple | Observation 1: Measurement bandwidth for the first for the first one MHz directly adjacent to the channel edge is equal to one MHz but the resolution bandwidth is close to 1% of the channel bandwidth. This requirement is tighter than NR NS_21 SEM and leads to the issue that power backoff requirements are not correctly reflected for all modulation types with 5MHz CBW. Observation 2: Complying to the adjusted emission limit from Observation 1 is especially challenging for PI/2 BPSK due to low MPR allowance. Proposal 1: Introduce separate SEM table for NS_21 and update the measurement bandwidth of the first row (ΔfOOB = 0-1) from “1 % of channel BW” to “1MHz”. - Proposal 2: Introduce A-MPR for NS_21 with 5MHz CBW according to the proposed CR. |
| R4-2117961 R4-2117962 | Apple | draftCR: Rel-16 Additional requirements and A-MPR for NS_21 and n30 |
| R4-2117956 | Apple | Observation: Out-of-band emissions can provide challenges for implementation when inter-band CA combinations are used with bands featuring low frequency separation between each other. Due to this issue some combinations specify the minimum requirements only for non-simultaneous Rx/Tx operation. Proposal 1: Due to low frequency separation between band n40 and n41, explicitly capture that CA_n40-n41 is only for non-simultaneous Rx/Tx. Proposal 2: Due to low minimum frequency separation between band n39, n40 and n41 the combinations CA_n39-n40 and CA_n39-n41 should only be specified for non-simultaneous Rx/Tx. |
| R4-2117957 | Apple | draftCR: Rel-16 Inter-band CA Operating Bands |
| R4-2117959 | Apple | Proposal: Agree on option 1 from WF of RAN4#101-e, which means that the transient and EVM requirements are kept as is and only the [] are removed. |
| R4-2118783 | Qualcomm | Proposal: tpstart=[-0.6] for 2us capability (to be verified with both 15kHz and 30kHz SCS) and tpstart=[-2.7]us for 7us capability(to be verified with 15kHz SCS). Tighten EVM to [6%] for 256QAM. |
| R4-2117977 | Apple | Draft CR for TS 38.101-1: MSD test configurations modification for US inter-band CA combinations with n77 |
| R4-2118120 | Ericsson | Observation 1: - for operations with 2 UL symbols in special slot, AS or AS+FH cannot be used at all (i.e., for any of 1T2R, 2T4R, 1T4R) since there is no room for a guard period G; - for operations with 3-4 UL symbols in special slot, AS+FH cannot be used at all. AS only (without FH) can be used for 1T2R and 2T4R, 1T4R cannot be used in a single slot at all (the latter for periodic/semi-persistent SRS) we make the following Proposal 1: remove the guard period between the SRS resources of the SRS set used for antenna switching in the SRS time mask for SCS = 15k and 30k with a view to solve the problematic cases with AS use in the special slot. A guard period is only motivated for accommodating transients for SCS = 60k for UEs not supporting the transient-period capability. RAN1 to be informed accordingly to align specifications. Proposal 2: send the LS in the attached to RAN1. |
| R4-2118121 R4-2118122 | Ericsson | Correction to SRS time mask for SRS usage set to antenna switching |
| R4-2118455 R4-2118456 | Xiaomi | Draft CR for 38.101-1 to correct the note in table 5.3.5-1 for Rel-16 |
| R4-2118704 R4-2118705 | Huawei | Draft CR for 38.101-1 to clarify the ASE requirements for NS_52 (Rel-16) |
| R4-2118880 | OPPO | Draft R16 CR on SRS IL |
| R4-2119081 R4-2119082 | ZTE | Draft CR to TS 38.101-1 on UE maximum output power reduction (Rel-16) |
| R4-2119291 R4-2119292 | Apple | draftCR: Rel-16 Correction on Channel Raster |
| R4-2119435 R4-2119436 | Qualcomm | DeltaT_RxSRS for PC5 |
| R4-2119567 | Huawei | draft CR for TS 38.101-1 correction of IE for DC location for CA (R16) |
| R4-2119497 R4-2119498 | Qualcomm | V2X pcmax corrections |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 NS_21 Regulatory Requirement

Issue 2-1-1: Whether it is needed to further clarify from the Canadian authorities as to the intention of RSS-195 to follow the FCC requirement for WCS 2300MHz band. (R4-2117152)

  - Option 1: Yes

  - Option 2: No, and alternative is

| Company | Comments |
| --- | --- |

Issue 2-1-2: Whether it is acceptable to introduce separate SEM table for NS_21 and update the measurement bandwidth of the first row (ΔfOOB =  0-1) from “1 % of channel BW” to “1MHz”. (R4-2117960)

  - Option 1: Yes

  - Option 2: No, and alternative is

| Company | Comments |
| --- | --- |

Issue 2-1-3: Whether it is acceptable to introduce A-MPR for NS_21 with 5MHz CBW according to the proposed CR R4-2117961.

  - Option 1: Yes

  - Option 2: No, and alternative is

| Company | Comments |
| --- | --- |

Sub-topic 2-2 non-simultaneous Rx/Tx

Issue 2-2-1: Whether it is acceptable to capture that CA_n40-n41 is only for non-simultaneous Rx/Tx. (R4-2117956)

  - Option 1: Yes

  - Option 2: No, and alternative is

| Company | Comments |
| --- | --- |

Issue 2-2-2: Whether it is acceptable to capture that CA_n39-n40 and CA_n39-n41 are only for non-simultaneous Rx/Tx. (R4-2117956)

  - Option 1: Yes

  - Option 2: No, and alternative is

| Company | Comments |
| --- | --- |

Sub-topic 2-3 Transient period capability

Issue 2-3-1: Which option is acceptable for transient period definition?

  - Option 1: Remove [] for EVM metric, keep requirements for shorter transient as they are

  - Option 2: tpstart=[-0.6] for 2us capability (to be verified with both 15kHz and 30kHz SCS) and tpstart=[-2.7]us for 7us capability(to be verified with 15kHz SCS). Tighten EVM to [6%] for 256QAM.

| Company | Comments |
| --- | --- |

Sub-topic 2-4 Guard period between the SRS resources

Issue 2-4-1: Whether it is acceptable to remove the guard period between the SRS resources of the SRS set used for antenna switching in the SRS time mask for SCS = 15k and 30k?

  - Option 1: Yes

  - Option 2: No, and alternative is

| Company | Comments |
| --- | --- |

Issue 2-4-2: Whether it is acceptable to send LS to RAN1 as R4-2118120?

  - Option 1: Yes

  - Option 2: No, and alternative is

| Company | Comments |
| --- | --- |

Companies views’ collection for 1st round

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117861 R4-2117862 | Draft CR for TS 38.101-1: Missing MOP for NR DC |
| R4-2117961 R4-2117962 | draftCR: Rel-17 Additional requirements and A-MPR for NS_21 and n30 Moderator note: rely on the outcome of Issue 2-1-3 |
| R4-2117957 | draftCR: Rel-16 Inter-band CA Operating Bands Moderator note: rely on the outcome of sub-topic 2-2 |
| R4-2117977 | Draft CR for TS 38.101-1: MSD test configurations modification for US inter-band CA combinations with n77 |
| R4-2118121 R4-2118122 | Correction to SRS time mask for SRS usage set to antenna switching Moderator note: rely on the outcome of sub-topic 2-4 |
| R4-2118455 R4-2118456 | Draft CR for 38.101-1 to correct the note in table 5.3.5-1 for Rel-16 |
| R4-2118704 R4-2118705 | Draft CR for 38.101-1 to clarify the ASE requirements for NS_52 (Rel-16) |
| R4-2118880 | Draft R16 CR on SRS IL |
| R4-2119081 R4-2119082 | Draft CR to TS 38.101-1 on UE maximum output power reduction (Rel-16) |
| R4-2119291 R4-2119292 | draftCR: Rel-16 Correction on Channel Raster |
| R4-2119435 R4-2119436 | DeltaT_RxSRS for PC5 |
| R4-2119567 | draft CR for TS 38.101-1 correction of IE for DC location for CA (R16) |
| R4-2119497 R4-2119498 | V2X pcmax corrections |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| CR/TP number | Comments collection |

Discussion on 2nd round

Topic #3: 38.101-2

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117422 | Apple | Propose two options to address the inconsistency between RAN4 R16 beam correspondence requirements and RAN2 UE capability. It is recommended RAN4 adopt either option. - Option 1: RAN4 sends an LS to RAN2 asking RAN2 to correct the capabilities - Option 2: This inconsistency is captured in RAN4 chairman’s note and companies can submit a CR in RAN2 with a reference to RAN4 chairman’s note. |
| R4-2117423 | Apple | Correction of UE enhanced beam correspondence requirements |
| R4-2117424 | Apple | Correction of UE enhanced beam correspondence requirements |
| R4-2117546 R4-2117547 | Nokia | draft CR removal of FR2 MPR brackets REL16 CATF |
| R4-2117978 | Apple | FR2 A-MPR requirements for intra-band non-contiguous UL CA |
| R4-2117979 R4-2117980 | Apple | Draft CR for TS 38.101-2: FR2 CA_NS_202 and CA_NS_203 A-MPR requirements for intra-band non-contiguous UL CA |
| R4-2119083 R4-2119084 | ZTE | Draft CR to TS 38.101-2 on configurations for intra-band contiguous CA (Rel-16) |
| R4-2119538 R4-2119539 | Huawei | draft CR for TS 38.101-2: Alignment of description of mpr-PowerBoost-Fr2-r16 (R16) |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1 Beam correspondence capability

Issue 3-1-1: Which option below is acceptable to solve the beam correspondence capability inconsistency between RAN4 R16 requirements and RAN2 UE capability. (R4-2117422)

  - Option 1: RAN4 sends an LS to RAN2 asking RAN2 to correct the capabilities

  - Option 2: This inconsistency is captured in RAN4 chairman’s note and companies can submit a CR in RAN2 with a reference to RAN4 chairman’s note.

| Company | Comments |
| --- | --- |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117423 | Correction of UE enhanced beam correspondence requirements |
| R4-2117424 | Correction of UE enhanced beam correspondence requirements |
| R4-2117546 R4-2117547 | draft CR removal of FR2 MPR brackets REL16 CATF |
| R4-2117979 R4-2117980 | Draft CR for TS 38.101-2: FR2 CA_NS_202 and CA_NS_203 A-MPR requirements for intra-band non-contiguous UL CA |
| R4-2119083 R4-2119084 | Draft CR to TS 38.101-2 on configurations for intra-band contiguous CA (Rel-16) |
| R4-2119538 R4-2119539 | draft CR for TS 38.101-2: Alignment of description of mpr-PowerBoost-Fr2-r16 (R16) |

Summary for 1st round

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round

Topic #4: 38.101-3

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117666 | SoftBank | Observation 1: For DC_42_n77/78 and DC_48_n77, the Option 1a/1b indicates that two DL carriers are placed contiguously. Observation 2: For DC_42_n77/78 and DC_48_n77, the UE supports intra-band non-contiguous EN-DC requirements as a default considering the UE capability signaling interBandContiguousMRDC. Proposal: The concept of Option 2, two DL carriers are placed non-contiguously, should be included in the requirements. |
| R4-2117855 | Qualcomm | Proposal 1: Focus on option 3 in WF and choose an imbalance that covers all frequency offsets. Observation 1: REFSENS is tested with limited UL configuration. REFSENS should also be tested with a limited power imbalance at the closest frequency offset. Observation 2: At the worst-case power imbalance of 30dB, the ACS 1 test case is no longer reflective of the UE to be tested at the edge of cell since all UE RX power level and ACS jammer are raised by 14dB. A lower REFSENS relaxation of 1dB retains the ability to test at edge of cell. Proposal 2: Specify a power imbalance limit of 25dB, which is consistent to allow UE to be tested according for RX requirements at the cell edge case. Proposal 3: Choose the power imbalance and frequency offset relationship as shown in Table 2.3-1. Observation 3: No significant impact on RX requirements if the power imbalance is limited to 25dB due to OOB blocking range 3 requirement. |
| R4-2118698 | Huawei | Observation 1: At least 25dB power imbalance should be considered for type 2 UE Rx requirements considering the network deployment. Observation 2: Both option 1a and option 1b can be met by the UE with 33dB ACS implementation based on the link budget evaluation. Observation 3: Option 2 can be met by UE with 33dB ACS implementation. Proposal 3: To specify the power imbalance requirements for Type 2 UE as below. |
| R4-2118699 R4-2118700 | Huawei | DraftCR for 38.101-3 to specify type 2 UE requirements(Rel-16) |
| R4-2118540 | NTT DOCOMO | Observation 1: The advantage of option 1 is that it can cover the frequency allocation of any operators since it defines the frequency separation as worst case. Observation 2: The advantage of option 2 is to optimize the value of Rx power imbalance considering actual spectrum allocation. Observation 3: Option 2 does not cover the frequency allocation of some operators. It is better to avoid such situation to enhance the size of market as large as possible. Observation 4: If interBandContiguousMRDC is absent, it means that the UE does not support intra-band contiguous requirements, and thus we do not need to test under the contiguous CCs placement. Observation 5: Applicability of Rx power imbalance requirements for EN-DC in TS 38.101-4 is based on whether or not UE indicate interBandContiguousMRDC Proposal: For inter-band EN-DC which is subject to interBandContiguousMRDC capability: - If interBandContgiuousMRDC is indicated, place two DL carriers as close as possible - If interBandContiguousMRDC is not indicated, define frequency offset from the edge of wanted carrier to the center frequency of another carrier as “DL CBW of another carrier”. For inter-band EN-DC which is not subject to interBandContiguousMRDC capability: - [Define frequency separation as placing two DL carriers as close as possible] |
| R4-2117981 R4-2117982 | Apple | Draft CR for TS 38.101-3: Corrections for intra-band EN-DC configurations |
| R4-2118450 | Xiaomi | Observation: If a UE is capable of supporting contiguous configuration in DL, it can support contiguous or non-contiguous configuration in UL; but if a UE is capable of supporting non-contiguous configuration in DL, it only supports non-contiguous configuration in UL. Proposal 1: RAN4 should define the contiguous and non-contiguous intra-band ENDC based on the aggregated status of DL intra-band ENDC. Proposal 2: Move DL DC_48A-(n)48AA with UL DC_(n)48AA and DC_48A_n48A from Table 5.3B.1.3-1 to Table 5.3B.1.2-1 in TS 38.101-3. Proposal 3: Apply Option 2, IE IntraBandENDC-Support should be indicated in UL and DL separately per band combination. Send LS to RAN2 to introduce new UE capability on distinguish intra-band ENDC UL and DL contiguous/non-contiguous support. |
| R4-2119318 | Google | Proposal 1: To introduce the new UE capability signaling from Rel-16 for intra-band EN-DC UL and DL configuration. Proposal 2: If proposal 1 is not agreed, it is proposed that the contiguous or non-contiguous intra-band EN-DC is determined by the configuration between primary cell in each cell group - Redefine the following intra-band EN-DC combinations - DC_(n)48CA and DC_(n)48DA with UL DC_48A_n48A are intra-band non-contiguous EN-DC combination - DC_48A_(n)48AA with UL DC_(n)48AA is intra-band contiguous EN-DC combination |

Open issues summary

Sub-topic 4-1 Type 2 UE RX Imbalance Requirement

Sub-topic description: Below options are from RAN4#100e agreed WF R4-2114905

- Alt 1a: Carriers; Power in transmission bandwidth configuration (dBm); channel bandwidth; Frequency relationship. Wanted carrier; REFSENS + 14 dB; BWwanted ≤ BWanother; Place two DL carriers as close as possible. Another carrier with overlapping DL bands; Power of wanted carrier + 31.5 dB. Wanted carrier; REFSENS + 14 dB; BWwanted > BWanother. Another carrier with overlapping DL bands; Power of wanted carrier + 31.5 – 10*log10(BWwanted /BWanother) dB. - Alt 1b: Carriers; Power in transmission bandwidth configuration (dBm); channel bandwidth; Frequency relationship. Wanted carrier; REFSENS + 1 dB; BWwanted ≤ BWanother; Place two DL carriers as close as possible. Another carrier with overlapping DL bands; Power of wanted carrier + 25 dB. Wanted carrier; REFSENS + 1 dB; BWwanted > BWanother. Another carrier with overlapping DL bands; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) dB. - Alt 2 (Gap between DL carriers ≥ 50MHz): Carriers; Power in transmission bandwidth configuration (dBm); channel bandwidth; Frequency relationship. Wanted carrier; REFSENS + 1 dB; BWwanted ≤ BWanother; max (5/2*another DL BW, 50MHz). Another carrier with overlapping DL bands; Power of wanted carrier + 25 dB. Wanted carrier; REFSENS + 1 dB; BWwanted > BWanother. Another carrier with overlapping DL bands; Power of wanted carrier + 25 dB – 10*log10(BWwanted /(5*min(BWanother, 20MHz))). - Alt 3: Combination of option 1 and option 2

Issue 4-1-1: Whether it is acceptable to specify 25 dB power imbalance for type 2 UE Rx requirements (R4-2117855)

  - Option 1: Yes

  - Option 2: No, and alternative is

| Company | Comments |
| --- | --- |

Issue 4-1-2: Which Alt is acceptable for the power imbalance testing

  - Alt 2 in RAN4#100e agreed WF R4-2114905

  - Alt 3 in RAN4#100e agreed WF R4-2114905

  - Alt 4 (R4-2117855):

| Carriers | Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship (Center of BWanother Relative to edge of BWwanted) |
| --- | --- | --- | --- |
| Wanted carrier | REFSENS + 1 dB | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz) |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 dB |  |  |
| Wanted carrier | REFSENS + 1 dB | BWwanted > BWanother |  |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) dB |  |  |
| Wanted carrier | REFSENS + 1 dB | N/A | ≥ max (5/2* BWanother, 50MHz) |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 dB |  |  |

  - Alt 5 (R4-2118698)

| Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship |
| --- | --- | --- | --- |
| Wanted carrier | REFSENS + 14 dB | BWwanted ≤ BWanother | Place two DL carriers as close as possible |
| Another carrier with overlapping DL bands | Power of wanted carrier + 31.5 dB |  |  |
| Wanted carrier | REFSENS + 14 dB | BWwanted > BWanother |  |
| Another carrier with overlapping DL bands | Power of wanted carrier + 31.5 – 10*log10(BWwanted /BWanother) dB |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |

| Company | Comments |
| --- | --- |

Issue 4-1-3: Whether it is acceptable to define frequency offset from the edge of wanted carrier to the center frequency of another carrier as “DL CBW of another carrier” if interBandContiguousMRDC is not indicated (R4-2118540)

  - Option 1: Yes

  - Option 2: No, and alternative is

| Company | Comments |
| --- | --- |

Issue 4-1-4: For inter-band EN-DC which is not subject to interBandContiguousMRDC capability, whether it is acceptable to “Define frequency separation as placing two DL carriers as close as possible” (R4-2118540)

  - Option 1: Yes

  - Option 2: No, and alternative is

| Company | Comments |
| --- | --- |

Sub-topic 4-2 IntraBandENDC-Support

Issue 4-2-1: Which option can be used to determine the contiguous or non-contiguous intra-band EN-DC

  - Option 1: based on the aggregated status of DL intra-band ENDC and (R4-2118450)

    - Move DL DC_48A-(n)48AA with UL DC_(n)48AA and DC_48A_n48A from Table 5.3B.1.3-1 to Table 5.3B.1.2-1 in TS 38.101-3

  - Option 2: determined by the configuration between primary cell in each cell group and Redefine the following intra-band EN-DC combinations  (R4-2119318)

    - DC_(n)48CA and DC_(n)48DA with UL DC_48A_n48A are intra-band non-contiguous EN-DC combination

    - DC_48A_(n)48AA with UL DC_(n)48AA is intra-band contiguous EN-DC combination

| Company | Comments |
| --- | --- |

Issue 4-2-2: Whether it is acceptable to indicate IE IntraBandENDC-Support in UL and DL separately per band combination？

  - Option 1: Yes

  - Option 2: No

| Company | Comments |
| --- | --- |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118699 R4-2118700 | DraftCR for 38.101-3 to specify type 2 UE requirements(Rel-16) Moderator note: rely on the outcome of Issue 4-1-2 |
| R4-2117981 R4-2117982 | Draft CR for TS 38.101-3: Corrections for intra-band EN-DC configurations |

Summary for 1st round

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round

Topic #5: 36.101

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117965 | Apple | draftCR: Rel-16 36.101 Corrections on spurious emission band UE co-existence |
| R4-2119422 | Qualcomm | Alignment of out-of-band blocking between LAA and NR-U |
| R4-2119423 R4-2119424 | Qualcomm | Out-of-band blocking for Band 46 |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117965 | draftCR: Rel-16 36.101 Corrections on spurious emission band UE co-existence |
| R4-2119423 R4-2119424 | Out-of-band blocking for Band 46 |

Summary for 1st round

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round

Recommendations for Tdocs

1st round

- New tdocs

| Title | Source | Comments |
| --- | --- | --- |

- Existing tdocs for 38.307

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

- Existing tdocs for 38.101-1

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

- Existing tdocs for 38.101-2

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

- Existing tdocs for 38.101-3

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

- Existing tdocs for 36.101

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

2nd round

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)