3GPP TSG-RAN WG4 Meeting #101-e	R4-2119099

E-meeting, 1 – 12 November 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.114 – applicability overview

Agenda item:	8.5.4

Document for:	Approval

1	Introduction

At the RAN4#100-e meeting, a work split for TP to TS 38.114 was agreed. In this contribution, we provide the TP to TS 38.114 for Section 7 as shown below.

TS38.114 work split for next meeting (i.e. #101-e meeting) --agreed

| 7 | Applicability overview |  |
| --- | --- | --- |
| 7.1 | Emission | Nokia, Nokia Shanghai Bell |
| 7.2 | Immunity | Nokia, Nokia Shanghai Bell |

Constructive comments are encouraged.

4	References

- TS 38.114 (v0.0.1)

5	Text proposal

TS 38.114 v0.0.1 (2021-xx)

--------------Start of text proposal-------------

7	Applicability overview

7.1	Emission

Table 7.1-1: Emission requirements applicability

| Phenomenon | Application | Equipment test requirement |  | Reference clause in the | Reference standard |
| --- | --- | --- | --- | --- | --- |
|  |  | NR repeater equipment | Ancillary equipment | present document |  |
| Radiated emission | Enclosure | applicable for [repeater TBD] (Note) | not applicable | 8.2.1 | ITU-R SM.329 [x] |
| Radiated emission | Enclosure of ancillary equipment | not applicable | applicable | 8.2.2 | CISPR 32 [x] |
| Conducted emission | DC power input/output port | applicable | applicable | 8.3 | CISPR 32 [x] |
| Conducted emission | AC mains input/output port | applicable | applicable | 8.4 | CISPR 32 [x] |
| Conducted emission | Telecommunication port | applicable | applicable | 8.5 | CISPR 32 [x] |
| Harmonic current emissions | AC mains input port | applicable | applicable | 8.6 | IEC 61000-3-2 [x] or IEC 61000-3-12 [x] (NOTE 2) |
| Voltage fluctuations and flicker | AC mains input port | applicable | applicable | 8.7 | IEC 61000-3-3 [x] or IEC 61000-3-11 [x] (NOTE 2) |
| NOTE 1: Radiated emission requirements for [repeater TBD] are described in subclause 8.2.1. NOTE 2: Selection of the reference IEC specification is based on the rated input current of the EUT’s power supply. |  |  |  |  |  |

7.2	Immunity

Table 7.2-1: Immunity requirements applicability

| Phenomenon | Application | Equipment test requirement |  | Reference | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  | NR repeater equipment | Ancillary equipment | clause in the present document | standard |
| RF electromagnetic field (80 – 6000 MHz) | Enclosure | applicable | applicable | 9.2 | IEC 61000-4-3 [x] |
| Electrostatic discharge | Enclosure | applicable | applicable | 9.3 | IEC 61000-4-2 [x] |
| Fast transients common mode | Signal, telecommunications and control ports, DC and AC power input ports | applicable | applicable | 9.4 | IEC 61000-4-4 [x] |
| RF common mode 0.15 - 80 MHz | Signal, telecommunications and control ports, DC and AC power input ports | applicable | applicable | 9.5 | IEC 61000-4-6 [x] |
| Voltage dips and interruptions | AC mains power input ports | applicable | applicable | 9.6 | IEC 61000-4-11 [x] |
| Surges, common and differential mode | AC power input ports and telecommunications port | applicable | applicable | 9.7 | IEC 61000-4-5 [x] |

--------------end of text proposal-------------