3GPP TSG-RAN WG4 Meeting #102-e                                                             R4-2203972

Electronic Meeting, February 21 – March 3, 2022

Source: 	CATT

Title: 	Simulation results for PUSCH demodulation requirements for FR2 HST

Agenda item:	10.9.4.3.1

Document for:	Approval

- Introduction

In RAN4#101-bis-e, a way forward on FR2 HST demodulation was approved [1].

In this contribution, we provide our simulation results for PUSCH demodulation following latest agreements.

- Simulation results

As per the WF [1], the simulation assumptions for PUSCH requirement are shown in Table A-1.

The simulation results for PUSCH requirement for FR2 HST are shown in Table2-1.

Table 2-1, Simulation results for PUSCH requirement for FR2 HST

| PUSCH |  |  |  |  |  |  |  | SNR@70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Channel | Frequency offset | DMRS mapping type | PUSCH data length | SCS and BW | MCS | RS configuration | Ideal | Impairment |
| Bi-directional scenario B | AWGN | 1944Hz | B | 10 | 120kHz, 50MHz | MCS16 | DMRS 1+0 | 6.88 | 9.38 |
|  |  |  |  |  |  | MCS17 |  | 8.26 | 10.76 |
|  |  |  |  |  |  | MCS18 |  | 8.47 | 10.97 |
|  |  |  |  |  |  | MCS19 |  | 9.52 | 12.02 |
|  |  |  |  |  |  | MCS20 |  | 11.40 | 13.90 |
|  |  |  |  |  |  | MCS16 | DMRS 1+1 | 6.73 | 9.23 |
|  |  |  |  |  |  | MCS17 |  | 8.02 | 10.52 |
|  |  |  |  |  |  | MCS18 |  | 8.59 | 11.09 |
|  |  |  |  |  |  | MCS19 |  | 9.34 | 11.84 |
|  |  |  |  |  |  | MCS20 |  | 11.44 | 13.94 |
|  |  |  |  |  |  | MCS16 | DMRS 1+1+1 | 6.62 | 9.12 |
|  |  |  |  |  |  | MCS17 |  | 8.04 | 10.54 |
|  |  |  |  |  |  | MCS18 |  | 8.26 | 10.76 |
|  |  |  |  |  |  | MCS19 |  | 9.25 | 11.75 |
|  |  |  |  |  |  | MCS20 |  | 10.19 | 12.69 |
|  |  |  |  |  | 120kHz, 200MHz | MCS16 | DMRS 1+0 | 7.52 | 10.02 |
|  |  |  |  |  |  | MCS17 |  | 9.11 | 11.61 |
|  |  |  |  |  |  | MCS18 |  | 9.3 | 11.8 |
|  |  |  |  |  |  | MCS19 |  | 10.34 | 12.84 |
|  |  |  |  |  |  | MCS20 |  | 11.13 | 13.63 |
|  |  |  |  |  |  | MCS16 | DMRS 1+1 | 7.37 | 9.87 |
|  |  |  |  |  |  | MCS17 |  | 8.87 | 11.37 |
|  |  |  |  |  |  | MCS18 |  | 9.42 | 11.92 |
|  |  |  |  |  |  | MCS19 |  | 10.16 | 12.66 |
|  |  |  |  |  |  | MCS20 |  | 11.14 | 13.64 |
|  |  |  |  |  |  | MCS16 | DMRS 1+1+1 | 7.36 | 9.86 |
|  |  |  |  |  |  | MCS17 |  | 9 | 11.5 |
|  |  |  |  |  |  | MCS18 |  | 9.26 | 11.76 |
|  |  |  |  |  |  | MCS19 |  | 10.39 | 12.89 |
|  |  |  |  |  |  | MCS20 |  | 12.21 | 14.71 |

- Conclusion

This contribution provides our simulation results for PUSCH demodulation requirements for FR2 HST for alignment.

- References

- R4-2203006, WF on BS demodulation requirement for FR2 HST, Nokia, Nokia Shanghai Bell, RAN4-bis-e

- R4-2120703, WF on BS demodulation requirement for FR2 HST, Nokia, Nokia Shanghai Bell, Samsung, RAN4#101-e

- Annex

Table A-1, Simulation assumption for PUSCH requirement for FR2 HST

| Parameter |  | Value |
| --- | --- | --- |
| Channel Model |  | HST FR2 mode based on Bi-directional scenario B, the details can be referred as R4-2115726 |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos0, Pos1, Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | 50MHz and 200MHz |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |