3GPP TSG-RAN WG4 Meeting #102-e	R4-2204717

Electronic Meeting, 21 February – 3 March 2022

Source: 	Ericsson

Title:  	Measurement procedure requirements for HST FR2

Agenda Item:	10.9.3.5

Document for:	Discussion

# Introduction

Work on RRM requirements for train-mounted UE in high speed train scenario in FR2 [1] continued during the RAN4#101-bis-e meeting, with outcome in terms of agreements and open issues captured in WFs [2].

In this contribution, we focus on issues measurement procedure specifications due to HST FR2 deployment.

# Disucssion

## Sub-topic 2-2: Measurement procedure requirements

Issue 2-2-3: Time period for PSS/SSS detection and Measurement period for intra-frequency measurements

Agreement: PSS/SSS detection Set 1: Table 1: Time period for PSS/SSS detection when [flag1] is configured, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil([6] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 80 ms; max(600ms, ceil([6] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms< DRX cycle≤ 320ms; max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. Set 2: Table 2: Time period for PSS/SSS detection when [flag2] is configured, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil([18] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 80ms; max(600ms, ceil([18] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms< DRX cycle≤ 320ms; max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. SSB measurement period Set 1: Table 3: Measurement period for intra-frequency measurements without gaps when [flag1] is configured (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil([6] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 80ms; max(400ms, ceil([6] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms< DRX cycle≤ 320ms; max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. Set 2: Table 4: Measurement period for intra-frequency measurements without gaps when [flag2] is configured (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil([18] x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ [80ms]; max(400ms, ceil([18] x M2Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 80ms< DRX cycle≤ 320ms; max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. Way forward: - Discuss the following options to define the scaling factors: - Option 1: Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 6 for Set 1 and 18 for Set 2 - Option 2: Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 10 for Set 1 and 24 for Set 2 Companies are encouraged to check, whether 600ms lower bound is makes sense when 80ms< DRX cycle≤ 320ms

Following same rule to present specification, Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps)= 3*RX beam.

Proposal 1: support Option1. Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 6 for Set 1 and 18 for Set 2.

Issue 2-2-5: Applicability of requirements to non-PC6 UE

Way forward: FFS, whether enhanced requirement should be applied to other than PC6 UEs even when HST FR2 flags are configured.

The studies only address PC6 UEs in demod and RRM sessions, other UEs may fail to meet enhanced requirements even HST FR2 flags are configured.

Proposal 2: No enhanced requirement should be applied to other than PC6 UEs even when HST FR2 flags are configured.

# Conclustion

Proposal 1: support Option1. Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 6 for Set 1 and 18 for Set 2.

Proposal 2: No enhanced requirement should be applied to other than PC6 UEs even when HST FR2 flags are configured.

# References

- RP-202118 “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia

- R4-2202594 “WF on FR2 HST RRM (part 1)”, Nokia, Nokia Shanghai Bell