3GPP TSG-RAN WG4 Meeting #102-e 	R4-2204834

Electronic Meeting, 21 Feb- 3 Mar., 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On ext_to_71GHz UE Demodulation and CSI requirements

Agenda item:	10.16.10.2

Document for:	Discussion

Introduction

This paper presents Nokia’s view on UE demodulation aspects related to the operation between 52.6 GHz and 71 GHz (FR2-2). As part of this discussion, we analyze the impacts of new features introduced for FR2-2 that might have an impact on UE demodulation requirements such as new subcarrier spacings and deployment scenarios that need to be considered for the work.

The work item “extending current NR operation to 71GHz” defines the following performance objectives related to UE demodulation performance requirements [2]:

Specify the following requirements [RAN4] […] - UE demodulation performance requirements […]

From the WI description [2], we list below the objectives that possibility impact the UE demodulation performance requirements:

According to the outcome of the study item on Supporting NR above 52.6GHz and leveraging FR2 design to the extent possible, this WI extends NR operation up to 71GHz considering, both, licensed and unlicensed operation, with the following objectives: - Physical layer aspects including [RAN1]: - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz […] - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued […] Similar to regular NR and NR-U operations below 52.6GHz, NR/NR-U operation in the 52.6GHz to 71GHz can be in stand-alone or aggregated via CA or DC with an anchor carrier.

In this paper we provide our view on the impact to the existing UE demodulation requirements as well as where new UE demodulation is required when introducing FR2-2 with 120kHz/480kHz and 960kHz SCS.

UE Demodulation Requirements

New sub-carrier spacings and bandwidths

Some of the most important changes introduced for the operation up to 71 GHz is the introduction of new sub-carrier spacings. As part of that work, 120 kHz SCS is reused for FR2-2 and 480 kHz and 960 kHz are introduced with channel bandwidths up to 2 GHz. The latest status of the RF discussion includes the following combinations of SCS and BW [5]:

Table 5.3.5-1: Channel bandwidths for each NR band Operating band; SCS (kHz); UE channel bandwidth (MHz). 50; 100; 200; 400; 800; 1600; 2000. n257; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n258; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n259; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n260; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n261; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n262; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n263; 120; 100; 400. 4802; 400; 800; 1600. 9602; 400; 800; 1600; 2000. NOTE 1: This UE channel bandwidth is optional in this release of the specification. NOTE 2: This SCS is optional in this release of the specification. Editor’s note: Optional/mandatory CHBW discussion is still on-going and further modifications are possible..

From the latest RAN1 agreements on the UE capabilities, we should also consider that in order to support FR2-2, the UE is only mandated to support 120 kHz, where 480 kHz and 960 kHz are included as optional features [1].

Operation in FR2-2 includes support of 120, 480, and 960 kHz SCS.

Channel bandwidths of up to 2000 MHz are supported in FR2-2.

In order to support FR2-2 operation, a UE has to support at least 120 kHz SCS.

Table 2-1 show how the FR2-2 SCS impacts the slot duration and reduced CP length. The reduced CP length will have an impact on UE demodulation performance.

Table 2-1: Slot duration and CP length of FR2-2 numerologies

| Numerology | Slot duration | CP length | Maximum FFT size |
| --- | --- | --- | --- |
| 120 kHz | 125 us | 585.9 ns | 4096 |
| 480 kHz | 31.25 us | 146.5 ns |  |
| 960 kHz | 15.625 us | 73.24 ns |  |

Reduced slot duration and CP length will have an impact on UE demodulation performance.

120kHz SRS can be found in both existing FR2-1 as well as the new FR2-2 and is mandatory for FR2-2. The 480/960kHz SCS are not mandatory for FR2-2 [1].

For FR2-2 the 120kHz SCS is mandatory.

For FR2-2 the 480/960kHz SCS are optional.

As the current UE performance specification only covers frequencies in FR2 up to 52.6GHz (FR2-1), new UE demodulation performance requirements are required for the introduced SCS options of 480kHz and 960kHz for PDSCH, PDCCH and PBCH.

Specify new UE demodulation and CSI reporting performance requirements for SCS 480kHz and 960kHz, including PDSCH, PDCCH and PBCH.

Additionally, for 120 kHz SCS, there are only 2 channel bandwidths specified for far, 100 MHz and 400 MHz. The existing UE demodulation requirements for 120 kHz in FR2-1 include 400 MHz requirements only for CA aggregation scenarios. Since in FR2-2 the support of larger channel bandwidths is important in order to enable the full benefit of the operation in such bands, we believe that specification of UE demodulation requirements for 100 MHz and 400 MHz is needed.

In FR2-2 there is support for operation of 120 kHz SCS with 100 MHz and 400 MHz channel bandwidth.

In order to take advantage of the benefits of the operation above 52.6 GHz, the support of 400 MHz channel bandwidth for 120 kHz SCS might become more important when comparing to operation in FR2-1.

Specify new UE demodulation and CSI reporting performance requirements for SCS 120 kHz with 400 MHz channel bandwidth for PDSCH, PDCCH and PBCH.

When the support for 120 kHz SCS with 100 MHz channel bandwidth is considered, we can verify if the existing FR2-1 requirements with 100 MHz channel bandwidth might be reused for the operation above 52.6 GHz.

Study if new UE demodulation and CSI reporting performance requirements for SCS 120 kHz with 100 MHz channel bandwidth are needed for the operation in FR2-2.

When the larger SCSs are considered, the channel bandwidths start at 400 MHz, and go up to 1600 MHz for 480 kHz SCS and 2000 MHz for 960 kHz SCS.

Support of 480 kHz SCS includes channel bandwidths from 400 MHz to 1600 MHz.

Support of 960 kHz SCS includes channel bandwidths from 400 MHz to 2000 MHz.

RAN4 to specify new UE demodulation and CSI reporting requirements for PDSCH, PDCCH and PBCH with the following SCS and channel bandwidth combinations: 
-480 kHz SCS with 400 MHz, 800 MHz, and 1600 MHz channel bandwidth
-960 kHz SCS with 400 MHz, 800 MHz, 1600 MHz, and 2000 MHz channel bandwidth

In Rel.16 FLD_high was defined to not exceed 40.000 MHz. For Rel.17 this was increased to 48200 MHz [1]:

The minimum performance requirements are applicable to the FR2 operating bands defined in TS 38.101-2 [7] with FDL_high not exceeding 48200 MHz.

FR2-2 is adding band n263 which needs to be covered in the requirements [7]:

NR is designed to operate in the FR2 operating bands defined in Table 5.2-1. Table 5.2-1: NR operating bands in FR2 Operating Band; Uplink (UL) operating band BS receive UE transmit; Downlink (DL) operating band BS transmit UE receive; Duplex Mode. FUL_low – FUL_high; FDL_low – FDL_high. n257; 26500 MHz; –; 29500 MHz; 26500 MHz; –; 29500 MHz; TDD. n258; 24250 MHz; –; 27500 MHz; 24250 MHz; –; 27500 MHz; TDD. n259; 39500 MHz; –; 43500 MHz; 39500 MHz; –; 43500 MHz; TDD. n260; 37000 MHz; –; 40000 MHz; 37000 MHz; –; 40000 MHz; TDD. n261; 27500 MHz; –; 28350 MHz; 27500 MHz; –; 28350 MHz; TDD. n262; 47200 MHz; –; 48200 MHz; 47200 MHz; –; 48200 MHz; TDD. n263; 57000 MHz; –; 71000 MHz; 57000 MHz; –; 71000 MHz; TDD1. NOTE 1: This band is restricted to operation with shared spectrum channel access as defined in [TS 37.213]..

UE Demodulation requirements are independent of center frequency but is dependent on phase noise. If FDL_high max value is increased to cover FR2-2 the validity of the existing UE demodulation requirements would need to be confirmed. In case the UE demodulation requirements cannot be confirmed, they need to be reconsidered with the new phase noise requirements for the higher bands.

Currently FDL_high may not exceeding 48200 MHz as defined in 38.101-4. This value must be increased to cover the FR2-2 frequency range.

Increase the FLD_high max frequency to cover FR2-2 frequency bands, i.e. “FDL_high may not exceed 71000 MHz” and reconsider existing requirements if needed.

Phase noise

The use of higher carrier frequencies in FR2-2 will result in increased phase error in comparison to FR2-1. This increase needs to be analyzed with relation to the currently selected configuration of the Phase Tracking Reference Signal (PT-RS).

Higher frequencies used in FR2-2 will result in increased phase error which needs to be considered when defining the performance requirements for FR2-2 for all SCS defined in FR2-2.

Current PTRS configuration used in TS38.101-4 [1] provides the most time dense configuration with the highest frequency density.

PTRS configuration; Frequency density (KPT-RS); 2. Time density (LPT-RS); 1. Resource Element Offset; 2.

As it is not possible to further improve the PTRS configuration for better phase tracking, it would be required to adjust the requirements to capture the increased phase error for FR2-2.

Current configuration for PTRS provides the maximum PTRS REs possible in the standard.

Rederive performance requirements with the maximally dense PTRS configurations for FR2-2 below:

| PTRS configuration | Frequency density (KPT-RS) |  | 2 |
| --- | --- | --- | --- |
|  | Time density (LPT-RS) |  | 1 |
|  | Resource Element Offset |  | 2 |

Channel model

As part of the study phase for operation between 52.6 and 71 GHz, extensive system level evaluations were performed in order to identify the delay spread in typical scenarios. The results are summarized in [10] as shown below:

## 6.3 Summary of delay spread evaluations The following are observations on the delay spread distribution: - One source [60] observed that for the delay spread distributions for the typical indoor scenarios evaluated, the delay spread of almost 80% of the users are less than 30 nsec. - One source [18] observed that Factory Scenario A (InF-DH) results in post-beamforming delay spreads that are a significant fraction of the CP duration for 960 kHz SCS. - One source [63] observed that 85% of the UE experience r.m.s delay spread small than CP length of 1.92 MHz subcarrier spacing (i.e. 36.6ns) in indoor, outdoor, and factory scenarios. - One source [30] observed that for small range indoor hotspot deployment, the channel delay spread is not an issue with normal CP. For outdoor scenarios with larger ISD and at moderate to high SNR (this may be produced by higher EIRP or smaller BW), normal CP demonstrates SINR degradation compared to extended CP. However, for such large coverage, high EIRP, and small BW use cases, we can choose to use a small SCS, e.g., 120kHz, with NCP. - One source [38] observed that while each scenario experiences different amounts of r.m.s. delay spread, regardless of scenarios, most of UEs experience smaller r.m.s. delay spreads than normal CP of 960 kHz.

As part of these results, it can be identified that typically the delay spread is contained within 30 ns in the scenarios that were evaluated. As an example, [8] and [9] show system level evaluation with pre- and post-beamforming RMS delay spread. The results show that BF results in a significant reduction of delay spread in this frequency range. Additionally, in Indoor factory scenarios, a significant portion of the UEs might experience delay spread between 10 and 20 ns. Considering that type of distribution, it is reasonable to assume that a gNB would adapt the SCS and MCS depending on the UE propagation conditions that would also include the delay spread.

Scheduling and link adaptation typically allocates large SCS and MCS in situations with low delay spread.

Distribution of the delay spread in typical scenarios for operation in FR2-2 are mostly concentrated below 30 ns, with a large part of the samples experiencing delay spread between 10 and 20 ns.

Existing RAN4 requirements include TDLA30 as the minimum delay spread for fading channels.

RAN4 to study the use of TDLA10 and TDLA20 for demodulation requirements with large SCS and high MCS.

Doppler

The doppler shift for FR2 is given in TS 38.104 in Table G.2.2-2 shown below

Doppler shift normally is calculated using the following equation:

fd=±vfcc

where v is the relative speed between the UE and the BS. fc is the carrier frequency. fd is the doppler shift. c is the speed of light.

Since doppler shift is directly proportional to the carrier frequency, it is expected to be higher in FR2-2 compared to FR2-1. Hence, we propose to increase the doppler shifts for FR2-2 over the one considered in Table G.2.2-2.

Doppler shift is directly proportional to the carrier frequency; Hence, it should be higher in FR2-2 compared to FR2-1.

Simulation assumption for FR2 was 30 GHz.

We propose to double the doppler shifts in FR2-2 than the ones already used for FR2-1. i.e., TDLAxx-150, and TDLAxx-300.

FR2-1 and FR2-2 differences for 120 kHz SCS

The existing requirements for SCS 120kHz might not be sufficient when extending the frequency range to 71GHz (FR2-2) which means analysis should be done on the existing requirements for SCS 120kHz to determine if they are sufficient for covering the full FR2 frequency range.

The existing UE demodulation requirements for 120kHz SCS has been defined based on FR2-1 frequency range. Depending on outcome from phase noise, channel model, doppler and channel bandwidth analysis it shall be considered if an update to existing UE demodulation requirement or creating new UE demodulation requirements for 120kHz SCS is needed.

SCS of 120kHz is extended to also be part of FR2-2, hence the current requirements for SCS 120kHz might not be sufficient to cover the full FR2 frequency range.

Based on analysis of impact related to phase noise, channel model doppler and channel bandwidth it can be decided if new requirements for 120kHz SCS in FR2-2 is required or if the existing UE demodulation requirements can be adjusted to cover the full FR2 range (i.e., FR2-1 + FR2-2).

Analyse the impact of the FR2-2 frequency range on the existing UE demodulation and CSI reporting performance requirements for SCS of 120kHz. If needed, add new performance requirements for PDSCH, PDCCH and PBCH with FR2-2 using 120kHz SCS.

In case new UE demodulation and CSI reporting requirements are needed for 120kHz SCS in FR2-2, update the existing requirements to be valid for FR2-1 only and create separate requirements for FR2-2.

multi-PDSCH scheduling by a single DCI

In the WID, one of objectives is to support multi-PDSCH scheduling by a single DCI. One of the main motivations is to reduce the need for monitoring PDCCH in every slot for high SCS values. Multi-slot scheduling for PUSCH was introduced in Rel. 16 NR-U, however multi-slot scheduling for PDSCH where a single DCI can schedule multiple DL TBs across multiple slots is new in Rel.17.

Multi-PDSCH scheduling by a single DCI has been introduced in Rel.17. However, the scheduled PDSCH resources are independent and as such the same demodulation requirements should apply to each PDSCH instance.

RAN4 to not include new UE demodulation performance requirements for the feature of Multi-PDSCH scheduled by a single DCI.

For FR2-2, RAN1#107bis-e has agreed to increase the number of HARQ processes from 16 to 32 [4].

### Agreement: - In NR FR2-2, a UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL (or for UL) shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL (or UL), subject to UE capability.

The number of HARQ processes has been increase to 32 [4]. There are enough HARQ processes to run the UE demodulation tests without running out of processes and thus starving the transmission pipeline.

RAN4 to not have explicit new demod requirements for increased number of HARQ processes. However, RAN4 to take care to specify sufficient HARQ processes for other PDSCH requirements.

CSI reporting requirements

Previous observations on show that there might be possible impact on which channel models shall be used for FR2-2 The changes to the models would mean PMI calculations might be different, hence the possible changes to the existing CSI reporting requirements would need to be discussed.

If the channel model is changed for FR2-2 it should be discussed in RAN4 if changes to the CSI reporting requirements will be needed.

RAN4 to discuss how changes to the channel models would impact the CSI reporting requirements.

Deployment scenarios

As a part of the RAN4 discussion in RRM and RF it was decided to prioritize scenarios as the ones described below [11]:

- Core specifications for UE, gNB and RRM requirements [RAN4]: - Specify new band(s) for the frequency range from 52.6GHz-71GHz. The band(s) definition should include UL/DL operation and excludes ITS spectrum in this frequency range. - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1). - For the case of FR2-2 DC or CA with an anchor in FR1 the following three example band combinations shall be considered: - n79 + Nx - n77 + Nx - n41 + Nx - where Nx is the 57-71 GHz band for unlicensed operation and the [66-71] GHz for licensed operation. - RAN4 to further discuss the need for single or multiple bands relevant for FR2-2 licensed/unlicensed operation.

As part of these objectives, it can be assumed that the extension to 71 kHz will support at least the following scenarios:

- Standalone FR2-2 only

- CA and NR-DC between FR2-2 an anchor in FR1

Deployment scenarios agreed in RF and RRM already include FR2-2 only standalone and CA/NR-DC with FR1 anchor.

Additionally, the latest RAN1 agreement on UE capabilities does not include mandatory support of UL in FR2-2. Therefore, it is expected that some UEs might support DL only operation in FR2-2.

When the existing UE demodulation requirements are considered, separate requirements are defined for carrier aggregation and dual connectivity. As an example, NR DC between FR1 and FR2 is defined as [6]:

### 9.2B.2 NR DC between FR1 and FR2 The test setup for FR1 PCell is specified in Table 5.5A-1 with antenna configuration 1x2. The PDSCH demodulation performance requirements for NR FR2 cell(s) are specified in Clause 7.2. During the test, only the PDSCH performance on FR2 NR cell(s) shall be verified.

RAN4 to consider UE demodulation and CSI reporting requirements including the following scenarios:
    -Standalone FR2-2 only
    -CA and NR-DC with FR1 anchor and FR2-2.

Conclusion

In this contribution we have presented Nokia’s point of view concerning the UE demodulation requirements for the extension to 71 GHz. As a result of the analysis we provided, we would like to highlight the following observations and proposals.

New sub-carrier spacings and bandwidths

- Operation in FR2-2 includes support of 120, 480, and 960 kHz SCS.

Channel bandwidths of up to 2000 MHz are supported in FR2-2.

In order to support FR2-2 operation, a UE has to support at least 120 kHz SCS.

Reduced slot duration and CP length will have an impact on UE demodulation performance.

For FR2-2 the 120kHz SCS is mandatory.

For FR2-2 the 480/960kHz SCS are optional.

- Specify new UE demodulation and CSI reporting performance requirements for SCS 480kHz and 960kHz, including PDSCH, PDCCH and PBCH.

In FR2-2 there is support for operation of 120 kHz SCS with 100 MHz and 400 MHz channel bandwidth.

In order to take advantage of the benefits of the operation above 52.6 GHz, the support of 400 MHz channel bandwidth for 120 kHz SCS might become more important when comparing to operation in FR2-1.

Specify new UE demodulation and CSI reporting performance requirements for SCS 120 kHz with 400 MHz channel bandwidth for PDSCH, PDCCH and PBCH.

Study if new UE demodulation and CSI reporting performance requirements for SCS 120 kHz with 100 MHz channel bandwidth are needed for the operation in FR2-2.

Support of 480 kHz SCS includes channel bandwidths from 400 MHz to 1600 MHz.

Support of 960 kHz SCS includes channel bandwidths from 400 MHz to 2000 MHz.

RAN4 to specify new UE demodulation and CSI reporting requirements for PDSCH, PDCCH and PBCH with the following SCS and channel bandwidth combinations: 
-480 kHz SCS with 400 MHz, 800 MHz, and 1600 MHz channel bandwidth
-960 kHz SCS with 400 MHz, 800 MHz, 1600 MHz, and 2000 MHz channel bandwidth

Currently FDL_high may not exceeding 48200 MHz as defined in 38.101-4. This value must be increased to cover the FR2-2 frequency range.

Increase the FLD_high max frequency to cover FR2-2 frequency bands, i.e. “FDL_high may not exceed 71000 MHz” and reconsider existing requirements if needed.

Phase noise

Higher frequencies used in FR2-2 will result in increased phase error which needs to be considered when defining the performance requirements for FR2-2 for all SCS defined in FR2-2.

Current configuration for PTRS provides the maximum PTRS REs possible in the standard.

Rederive performance requirements with the maximally dense PTRS configurations for FR2-2 below:

| PTRS configuration | Frequency density (KPT-RS) |  | 2 |
| --- | --- | --- | --- |
|  | Time density (LPT-RS) |  | 1 |
|  | Resource Element Offset |  | 2 |

Channel model

Scheduling and link adaptation typically allocates large SCS and MCS in situations with low delay spread.

Distribution of the delay spread in typical scenarios for operation in FR2-2 are mostly concentrated below 30 ns, with a large part of the samples experiencing delay spread between 10 and 20 ns.

Existing RAN4 requirements include TDLA30 as the minimum delay spread for fading channels.

RAN4 to study the use of TDLA10 and TDLA20 for demodulation requirements with large SCS and high MCS.

Doppler shift is directly proportional to the carrier frequency; Hence, it should be higher in FR2-2 compared to FR2-1.

Simulation assumption for FR2 was 30 GHz.

We propose to double the doppler shifts in FR2-2 than the ones already used for FR2-1. i.e., TDLAxx-150, and TDLAxx-300.

FR2-1 and FR2-2 differences for 120 kHz SCS

SCS of 120kHz is extended to also be part of FR2-2, hence the current requirements for SCS 120kHz might not be sufficient to cover the full FR2 frequency range.

Based on analysis of impact related to phase noise, channel model doppler and channel bandwidth it can be decided if new requirements for 120kHz SCS in FR2-2 is required or if the existing UE demodulation requirements can be adjusted to cover the full FR2 range (i.e., FR2-1 + FR2-2).

Analyse the impact of the FR2-2 frequency range on the existing UE demodulation and CSI reporting performance requirements for SCS of 120kHz. If needed, add new performance requirements for PDSCH, PDCCH and PBCH with FR2-2 using 120kHz SCS.

In case new UE demodulation and CSI reporting requirements are needed for 120kHz SCS in FR2-2, update the existing requirements to be valid for FR2-1 only and create separate requirements for FR2-2.

multi-PDSCH scheduling by a single DCI

Multi-PDSCH scheduling by a single DCI has been introduced in Rel.17. However, the scheduled PDSCH resources are independent and as such the same demodulation requirements should apply to each PDSCH instance.

RAN4 to not include new UE demodulation performance requirements for the feature of Multi-PDSCH scheduled by a single DCI.

The number of HARQ processes has been increase to 32 [4]. There are enough HARQ processes to run the UE demodulation tests without running out of processes and thus starving the transmission pipeline.

RAN4 to not have explicit new demod requirements for increased number of HARQ processes. However, RAN4 to take care to specify sufficient HARQ processes for other PDSCH requirements.

CSI reporting requirements

If the channel model is changed for FR2-2 it should be discussed in RAN4 if changes to the CSI reporting requirements will be needed.

RAN4 to discuss how changes to the channel models would impact the CSI reporting requirements.

Deployment scenarios

Deployment scenarios agreed in RF and RRM already include FR2-2 only standalone and CA/NR-DC with FR1 anchor.

RAN4 to consider UE demodulation and CSI reporting requirements including the following scenarios:
    -Standalone FR2-2 only
    -CA and NR-DC with FR1 anchor and FR2-2.

- References

- 3GPP TS 38.101-4 V17.3.0 (2021-12)

- RP-212637 -  “Extending current NR operation to 71GHz”.

- R1-2112482 Change Request for 38.214

- R1-2200744	Summary #2 of PDSCH/PUSCH enhancements (Scheduling/HARQ)

- R4-2202364, “Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2”

- TS 38.101-4, NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

- R4-2202364 - Draft CR for TS 38.101-2 Introduction of system parameters for FR2-2

- R1-2007982 "On NR operations in 52.6 to 71 GHz" Ericsson

- R1-2008615 "NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz" Qualcomm Incorporated.

- TR 38.808 v17.0.0, Study on supporting NR from 52.6 GHz to 71 GHz

- RP-213540, Revised WID:  Extending current NR operation to 71 GHz, Qualcomm

- R4-2203530 - Discussion on general demodulation requirements for the extension to 71 GHz