3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204907

Electronic Meeting, February 21 – March 3, 2022

Title: 	Discussion on	signaling characteristics due to UE complexity reduction

Source: 	Huawei, HiSilicon

Agenda item:	10.20.3.1.4

Document for:	Discussion

Introduction

The revised WID on support of reduced capability (RedCap) NR devices has been approved in [1]. RAN4 has started work on the support of RedCap NR devices, where the related study item has been concluded in TR 38.875. The WID has the following objectives on UE complexity reduction:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz. - Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE. - Maximum number of DL MIMO layers: - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. - Relaxed maximum modulation order: - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE. - No other relaxations of maximum modulation order are specified for a RedCap UE. - Duplex operation: - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

It is concluded in RAN#100e meeting that there is no RRM impact due to Maximum number of DL MIMO layers and Relaxed maximum modulation order [2]. This contribution further provides analysis on the signaling characteristics due to reduced UE BW, reduced RX branch.

Discussion

RLM

There are good progress on RLM in last meeting. The only left open issue [3] is,

If lower bound is extended for RLM evaluation period, how much to extend: - Option 1 (MTK): By factor 2

It is agreed in last meeting that for RedCap UE with 1RX, sample number for both SSB and CSI-RS based RLM OOS evaluation period are doubled.

SSB-based RLM : evaluation period for Qout and Qin - Double the evaluation period for Qout - keep the legacy evaluation period for Qin - Further discuss in performance part to relax test case SINR by X db to compensate for loss in SINR accuracy, where X is TBD. CSI-RS-based RLM : evaluation period for Qout and Qin Follow the agreement from corresponding issue for SSB-based RLM evaluation period.

In the current RLM evaluation period, the lower bound is 200ms considering 10 samples and 20ms SSB/CSI-RS periodicity. It is straight forward to extend the lower bound of RLM OOS evaluation period by factor 2 as the evaluation samples are doubled.

Proposal 1: Double the lower bound for SSB and CSI-RS based RLM OOS evaluation period.

BFD

There are good progress on BFD in last meeting. The left open issues [3] are,

SSB-based based BFD: evaluation period - Option 1 (Apple, vivo, Oppo, HW, E///, QC): No need to extend the evaluation period for BFD in FR1 and FR2. - Option 2 (MTK): Follow the agreements from RLM. NOTE: It is unclear from option 2 whether it refers to the corresponding agreement from RLM Qin or Qout. CSI-RS-based BFD: evaluation period - Option 1 (Apple, vivo, Oppo, HW, E///, QC): No need to extend the evaluation period for BFD for FR1 and FR2. - Option 2 (MTK): Follow the agreements from RLM. NOTE: It is unclear from option 2 whether it refers to the corresponding agreement from RLM Qin or Qout.

The legacy evaluation period for SSB based and CSI-RS based BFD are specified as below. The sample numbers are in yellow highlight. For CSI-RS based BFD, MBFD = 10.

Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 P) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.2.2-2: Evaluation period TEvaluate_BFD_SSB for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P N) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 P N) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 P N) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, [MBFD P PBFD] TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, [1.5 × MBFD P PBFD] Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD P PBFD] TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Table 8.5.3.2-2: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, [MBFD P N PBFD] TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, [1.5 × MBFD P N PBFD] Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD P N PBFD] TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Regarding BFD, during defining out-of-sync evaluation period for BFD in R15 (considering 2RX), Qout is observed around side condition -6dB (note: Qout for RLM is observed @-10dB). From the simulation results in [4], there are some observations:

- for SSB based BFD evaluation

  - It is up to 0.5 dB delta-SINR difference between 2RX@ -6dB and 1RX@ -6dB with 5 sample numbers;

  - The delta-SINR accuracy for 1RX@ -6dB can already satisfy the L1-SINR accuracy.

- for CSI-RS based BFD evaluation

  - It is up to 0.5 dB delta-SINR difference between 2RX@ -6dB and 1RX@ -6dB with 10 sample numbers;

  - The delta-SINR accuracy for 1RX@ -6dB can already satisfy the L1-SINR accuracy.

Therefore the BFD evaluation sample number for both SSB and CSI-RS based BFD can be kept unchanged.

Proposal 2: Legacy SSB and CSI-RS based BFD evaluation period can be reused for 1 RX Redcap.

CBD including L1-RSRP measurement

It is agreed in last meeting [3],

SSB-based L1-RSRP: measurement period without measurement restriction For RedCap UE with 1RX SSB-based L1-RSRP measurement period can be unchanged for FR1 and FFS for FR2. CSI-RS based L1-RSRP (excluding 60 kHz SCS): measurement period without measurement restriction For RedCap UE with 1RX CSI-RS based L1-RSRP measurement period can be unchanged for FR1 and FFS for FR2

Based on the SSB based and CSI-RS based L1-RSRP simulation results shown in [4], the following observations were identified:

- when side condition is -3dB and L1 physical sample is 1 (without measurement restriction),

  - the absolute measurement accuracy with 1RX is about 3dB worse than that of 2 Rx for both FR1 and FR2;

  - the relative measurement accuracy with 1RX is about 3dB worse than that of 2 Rx for both FR1 and FR2;

- when side condition is -3dB and L1 physical sample is 3, the measurement accuracy with 1RX is 1dB worse than that of 2 Rx for both FR1 and FR2;

  - the absolute measurement accuracy with 1RX is about 3dB worse than that of 2 Rx for both FR1 and FR2;

  - the relative measurement accuracy with 1RX is about 3dB worse than that of 2 Rx for both FR1 and FR2;

Observation 1: For SSB and CSI-RS based L1-RSRP,

- when side condition is -3dB and L1 physical sample is 1 (without measurement restriction),

  - the absolute measurement accuracy with 1RX is about 3dB worse than that of 2 Rx for both FR1 and FR2;

  - the relative measurement accuracy with 1RX is about 3dB worse than that of 2 Rx for both FR1 and FR2.

- when side condition is -3dB and L1 physical sample is 3, the measurement accuracy with 1RX is 1dB worse than that of 2 Rx for both FR1 and FR2;

  - the absolute measurement accuracy with 1RX is about 3dB worse than that of 2 Rx for both FR1 and FR2;

  - the relative measurement accuracy with 1RX is about 3dB worse than that of 2 Rx for both FR1 and FR2.

As the legacy accuracy requirement for L1-RSRP are based on one sample, the measurement accuracy of L1-RSRP for RedCap UE with 1RX can follow the same principle.

Proposal 3: For RedCap UE with 1RX, the measurement accuracy shall be based on one sample.

- The absolute accuracy for both SSB based and CSI-RS based L1-RSRP can be relaxed by 3dB;

- The relative accuracy for both SSB based and CSI-RS based L1-RSRP can be relaxed by 3dB.

Conclusions

This contribution provides analysis on the signaling characteristics due to complexity reduction for RedCap UE. The following proposals are provided:

Proposal 1: Double the lower bound for SSB and CSI-RS based RLM OOS evaluation period.

Proposal 2: Legacy SSB and CSI-RS based BFD evaluation period can be reused for 1 RX Redcap.

Proposal 3: For RedCap UE with 1RX, the measurement accuracy shall be based on one sample.

- The absolute accuracy for both SSB based and CSI-RS based L1-RSRP can be relaxed by 3dB;

- The relative accuracy for both SSB based and CSI-RS based L1-RSRP can be relaxed by 3dB.

Reference

- RP-211038, Revised WID on support of reduced capability NR devices, Ericsson

- R4-2115358, WF on RedCap RRM requirements, Ericsson

- R4-2202670, WF on RedCap RRM requirements, Ericsson

- R4-2201189, Simulation results for RLM and BFD due to complexity reduction for RedCap UE, Huawei, HiSilicon