3GPP TSG-RAN WG4 Meeting #102-e	R4-2205118

Online meeting, 21 Feb- 3 Mar, 2022

Agenda item:			9.27.2

Source:	Xiaomi

Title:	TP for TR 38.862 to correct the maximum aggregated bandwidth for intra-band C CA with BCS4/BCS5

Document for:	Approval

1. Introduction

This contribution is a text proposal for TR 38.862 v0.5.0 to modify the rule of the maximum aggregated bandwidth for intra-band CA with BCS4/BCS5.

2. Text Proposal

---Start of changes---

5.5.3	The maximum aggregated bandwidth for intra-band CA with BCS4/BCS5

To guarantee the BCS4/BCS5 can cover all the possible bandwidth configurations for intra-band CA, the maximum aggregated bandwidth chosen for BCS4/BCS5 should equal to

-	min{n*max channel bandwidth of each carrier, BWChannel_CA of each CA bandwidth class, floor(Maximum frequency range of each band/5MHz)*5MHz} for intra-band contiguous CA.

-	min{n*max channel bandwidth of each carrier, Maximum frequency range of each band - Minimum sub-block gaps} for intra-band non-contiguous CA.

where n is the number of aggregated CCs, minimum sub-block gaps indicates the sum of the min sub-block gap between the upper edge of lower component carrier and lower edge of higher component carrier that UE can support per band combination in two adjacent non-contiguous component carriers.

The value of min sub-block gaps could be clarified by the request operator but it should try to cover the needs of all possible operators.

---End of changes---