3GPP TSG-RAN4 Meeting #102-e	R4-2205292

# Online, , February 21 – March 4, 2022

Agenda item:			10.16.3.2

Source:	Qualcomm Inc

Title:	60 GHz UE RX

Document for:	Approval

# Introduction

Here we discuss REFSENS and EIS requirements for both PC1 and PC3 UEs in n263

# Reference sensitivity

Our proposal continues to be based on rel. 15 assumptions for REFSENS. Note that REFSENS estimate is agnostic of misalignment between UE and TE axes. For reference, REFSENS requirement for 400MHz CC and -1 dB target SNR for n262 is -83.5 dBm (47.2-48.2 GHz).  For FR2-2 the array size is 8 elements for PC1 and 64 elements for PC3. Our preliminary budget for FR2-2 is below.

Power class 1 REFSENS budget

| Parameter | Value | Units |
| --- | --- | --- |
| number of elements per polarization | 64.0 |  |
| element antenna gain | 4.1 | dBi |
| element LNA Noise Figure (assumed same for all elements) | 11 | dB |
| Total of all losses | 12.1 | dB |
| Minimum SNR (note 3GPP REFSENS is -1 dB) | -1 | dB |
| Noise bandwidth | 380 | MHz |
| Rqd EIS (effective Isotropic Signal) -with no statistical losses | -88.3 | dBm |

Proposal: REFSENS for PC1 based on 64 array elements

Proposal: n263 PC1 REFSENS is -88.3 dBm under the assumption of 95% spectral occupancy, 64 elements, and 400 MHz.

For PC3 the REFSENS is based on 8 elements per polarization

FR2-2 Power class 3 REFSENS budget

| Parameter | Value | Units |
| --- | --- | --- |
| number of elements per polarization | 8.0 |  |
| element antenna gain | 4.1 | dBi |
| element LNA Noise Figure (assumed same for all elements) | 11 | dB |
| Total of all losses | 12.1 | dB |
| Minimum SNR (note 3GPP REFSENS is -1 dB) | -1 | dB |
| Noise bandwidth | 380 | MHz |
| Rqd EIS (effective Isotropic Signal) -with no statistical losses | -79.2 | dBm |

Proposal: REFSENS for PC3 based on 8 array elements

Proposal: n263 PC3 REFSENS is -79.2 under the assumption of 95% spectral occupancy, 8 elements, and 400 MHz.

# EIS spherical coverage

RAN4 has agreed to use a single panel assumption for PC1 spec development. Further propose to use the 85%ile for power class 1, and in [1] we find that the 85 %ile can be specified as 13.5 dB below the peak value.

Proposal: For PC1 specify the 85%ile at 14 dB down from the peak sensitivity value, based on 64 elements and 400MHz CBW/95% spectral occupancy.

Antenna array pattern is one factor involved in establishing spherical coverage metrics. The other major factor is the number of antenna panels. Here we consider the antenna gain CDF for a 2x4 antenna array, and 2 panels positioned to fire in opposite directions, front and back. In [1] we find that the 50 %ile can be specified as 16.1 dB below the peak value.

Proposal: For PC3 specify the 50%ile at 16.1 dB down from the peak sensitivity value, based on 8 elements and 400MHz CBW/95% spectral occupancy.

# Conclusions

REFSENS

Proposal: REFSENS for PC1 based on 64 array elements

Proposal: n263 PC1 REFSENS is -88.3 dBm under the assumption of 95% spectral occupancy, 64 elements, and 400 MHz.

Proposal: REFSENS for PC3 based on 8 array elements

Proposal: n263 PC3 REFSENS is -79.2 under the assumption of 95% spectral occupancy, 8 elements, and 400 MHz.

EIS Spherical coverage

Proposal: For PC1 specify the 85%ile at 14 dB down from the peak sensitivity value, based on 64 elements and 400MHz CBW/95% spectral occupancy.

Proposal: For PC3 specify the 50%ile at 16.1 dB down from the peak sensitivity value, based on 8 elements and 400MHz CBW/95% spectral occupancy.

# References

[1] – R4-2205246, 60GHz UE TX, Qualcomm Incorporated