3GPP TSG-RAN WG4 Meeting #102-e	R4-2205505

Electronic Meeting, Feb. 21- Mar. 3, 2022

Agenda Item:	10.12.2.1.2

Source:	Ericsson

Title:	Simulation results on CSI reporting for inter-cell interference

Document for:	Information

# 1	Introduction

In this contribution, the MMSE-IRC for suppressing inter-cells’ interference performance simulation results.

| Parameter |  |  | Unit | FDD | TDD | Interferer cell |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 40 |  |
| Duplex Mode |  |  |  | FDD | TDD |  |
| Subcarrier spacing |  |  | kHz | 15 | 30 |  |
| SINR |  |  | dB | [-6: 2 :10] |  | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |  | 1 |
|  | CDM Type |  |  | FD-CDM2 |  | noCDM |
|  | Density (ρ) |  |  | 1 |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3(6, -) |  | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  | Periodic |
|  | CSI-IM RE pattern |  |  | 0 |  | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  | (6,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 10/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic |  | Not configured |
| CQI-table |  |  |  | Table 2 |  | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |  | Wideband |
| Sub-band Size |  |  | RB | 8 | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 10/9 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured |  | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  | Not configured |
|  |  | CodebookSubsetRestriction |  | 010000 |  | Not configured |
|  |  | RI Restriction |  | N/A |  | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH |  | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 | 9.5 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 |  | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-2 |  |  |
| INR |  |  | dB | N/A |  | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 |  | AWGN |
| Antenna configuration |  |  |  | 2×2, 2×4 |  | 1×2, 1×4 |
| Correlation configuration |  |  |  | ULA Low |  |  |
| Note: Note 1: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Intefering cell is fully loaded. Note 2: Both cells are time-synchronous. Note 3: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. |  |  |  |  |  |  |

| Serving cell configuration | Antenna configuration |
| --- | --- |
| #1 | 2*2 |
| #2 | 2*4 |

# 2	Simulation Results

The relative throughput ratio γ is defined as follow.

γ= Tfollow with interfTfollow with AWGN

The simulation results are summarized as follow.

Table 1. simulation results for FR1 FDD

| Test | Receiver Type | γ @ SINR [dB] |  |  |
| --- | --- | --- | --- | --- |
|  |  | -4 | -3 | -2 |
| #1 | MRC | 1.5 | 1.6 | 1.5 |
|  | IRC | 3.4 | 3.1 | 2.8 |
| #2 | MRC | 1.4 | 1.4 | 1.4 |
|  | IRC | 2.9 | 2.7 | 2.6 |

Observation 1: Compared with MRC-based CQI reporting, IRC-based CQI reporting receiver can have obvious gain for relative throughput ratio in low SINR.

Observation 2: The BLER is greater than or equal to 2% which implies a reasonable CQI reporting to NW.

Detail simulation results are shown in the appendix.

# References

- R4-2203009, “WF on CSI requirements for inter-cell interference MMSE-IRC”, Ericsson

# Appendix

Test 1

Figure 1 TP and BLER comparison for test 1

Test 2

Figure 2 TP and BLER comparison for Test 2